3GPP TSG-RAN WG1  Meeting #120	R1-2500596

# Athens, Greece, February 17th - 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on CSI parameters in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Misaligned parameter names in sections 5.2.2.2.8, 5.2.2.2.9, 5.2.2.2.10 and 5.2.2.2.11, based on TS 38.331 v18.4.0 and the RAN2 LS R1-2407592(R2-2407827) - There is no parameter numberOfSDCombinations and numberOfSDCombinations-PS to explicitly configure the value of NL, which can be obtained based on number of elements in the higher layer parameter paramCombination-CJT-L-r18 and paramCombination-CJT-PS-alpha-r18 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the misaligned paramters in sections 5.2.1.4.1, 5.2.1.4.2, 5.2.1.6, 5.2.2.2.8, 5.2.2.2.9, 5.2.2.2.10 and 5.2.2.2.11 - Revise the description to configure the value of NL in sections 5.2.2.2.8, 5.2.2.2.9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misaligned parameter names - Ambiguous UE behaviour refering to non-exist parameter numberOfSDCombinations and numberOfSDCombinations-PS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.1, 5.2.1.4.2, 5.2.1.6, 5.2.2.2.8, 5.2.2.2.9, 5.2.2.2.10, 5.2.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.2.1.4.1	Resource Setting configuration

<Unchanged part omitted>

A UE configured with a CSI-ReportConfig with the higher layer parameter vectorLengthDD and reportQuantity set to 'cri-RI-PMI-CQI', is expected to be configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement. For an aperiodic CSI-RS resource set for channel measurement, the K CSI-RS resources are triggered by the same triggering instance and the separation between two consecutive CSI-RS resources is m∈{1,2} slots, which is configured by higher layer parameter in the NZP-CSI-RS-ResourceSet. The K aperiodic CSI-RS resources are transmitted following the order of the CSI-RS resource IDs configured in the CSI-RS resource set. The UE shall assume that the antenna port with the same port index of the K aperiodic CSI-RS resources is the same. If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet. If interference measurement is performed on NZP CSI-RS, only one resource is configured in the corresponding NZP-CSI-RS-ResourceSet for interference measurement.

<Unchanged part omitted>

5.2.1.4.2	Report quantity configurations

<Unchanged part omitted>

Subject to UE capability, a UE configured with a CSI-ReportConfig with the higher layer parameter vectorLengthDD  and reportQuantity set to 'cri-RI-PMI-CQI' is assumed to support UE-side CSI prediction. The reported PMI indicates predicted precoder matrices associated with N4 consecutive slot intervals, each with duration of d slots, where the value of N4∈{1,2,4,8} is configured by higher layer parameter vectorLengthDD. If the UE is configured with an aperiodic CSI-RS resource set for channel measurement, the value, in number of slots, of the time unit d∈{1,m} is configured by higher layer parameter unitDurationDD, where m is defined in Clause 5.2.1.4.1. If the UE is configured with a periodic or semi-persistent CSI-RS resource set for channel measurement, the value of d is equal to the periodicity of the CSI-RS resource. The earliest of the N4 slot intervals starts at slot l=n+δ, where n is the uplink slot in which the CSI is reported and the slot offset δ∈{-nCSI_ref,0,1,2} is configured by higher layer parameter predictionDelay, where nCSI_ref defined in Clause 5.2.2.5 and the value δ=-nCSI_ref can be configured subject to UE capability.

<Unchanged part omitted>

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', ' cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI' or 'cri-RI-LI-PMI-CQI', then the UE is not expected to be configured with more than 8 CSI-RS resources in a CSI-RS resource set contained within a resource setting that is linked to the CSI-ReportConfig, except when the UE is configured with a CSI-ReportConfig with the higher layer parameter vectorLengthDD, reportQuantity set to 'cri-RI-PMI-CQI' and the corresponding CSI-RS resource set for channel measurement is aperiodic with K=12 resources.

5.2.1.6	CSI processing criteria

-	if a CSI-ReportConfig is configured with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI' and with codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18',

-	if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with K CSI-RS resources,  OCPU=8 for K=12 and OCPU=Y1⋅K for K<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

-	if the corresponding CSI-RS Resource Set for channel measurement is periodic or semi-persistent and configured with a single CSI-RS resource, OCPU=4 for N4=1 and OCPU=max(Y2⋅N4,4) for N4>1, where the value of N4 is configured by the higher layer parameter vectorLengthDD, and Y2∈{1, 2, 3} is reported by UE capability indication,

5.2.2.2.8	Enhanced Type II codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, and the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-r18'

-	the values of N1, N2  and O1, O2 are the same for all NTRP CSI-RS resources and are configured by n1-n2-codebookSubsetRestrictionList-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2 are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2 for each of the NTRP CSI-RS resources.

A set of NL∈{1,2,4} combinations of values of {L1,…,LNTRP} is configured by the higher layer parameter paramCombination-CJT-L-r18, where the value of NL is based on the number of elements configured by the higher layer parameter paramCombination-CJT-L-r18 and the mapping is given in Table 5.2.2.2.8-1, with the value of Ln corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of pν and β is configured by the higher layer parameter paramCombination-CJT-r18, where the mapping is given in Table 5.2.2.2.8-2. The configurable combinations of {L1,…,LNTRP} and {pυ,β} are marked with 'x' in Table 5.2.2.2.8-3.

-	The UE is not expected to be configured with paramCombination-CJT-L-r18 equal to

-	2, 3 for NTRP=1; 2, 3, 4 for NTRP=2; 2, 3, 4, 5 for NTRP=3; 2, 3 or 4 for NTRP=4; when PCSI-RS=4,

-	3 for NTRP=1, when PCSI-RS<32,

-	3 for NTRP=1, when higher layer parameter typeII-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	3 for NTRP=1, when R=2.

<Unchanged part omitted>

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-r18, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index.

<Unchanged part omitted>

5.2.2.2.9	Further enhanced Type II port selection codebook for CJT

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with NTRP∈{1,2,3,4} CSI-RS resources in a resource set for channel measurement and with higher layer parameter codebookType set to 'typeII-CJT-PortSelection-r18'

-	the number of CSI-RS ports for each CSI-RS resource, PCSI-RS, is configured as in clause 5.2.2.2.4.

A set of NL∈{1,2,4} combinations of values of {α1,…,αNTRP} is configured by the higher layer parameter paramCombination-CJT-PS-alpha-r18, where the value of NL is based on the number of elements configured by the higher layer parameter paramCombination-CJT-PS-alpha-r18 and the mapping is given in Table 5.2.2.2.9-1, with the value of αn corresponding to CSI-RS resource n, for n=1,…,NTRP. A single value of M and β is configured by the higher layer parameter paramCombination-CJT-PS-r18, where the mapping is given in Table 5.2.2.2.9-2. The configurable combinations of {α1,…,αNTRP} and {M,β} are marked with 'x' in Table 5.2.2.2.9-3.

-	The UE is not expected to be configured with paramCombination-CJT-PS-alpha-r18 equal to

-	2 for NTRP=1; 4 for NTRP=2; 2, 3 or 4 for NTRP=3; when PCSI-RS∈{4,12},

-	3 for NTRP=1; 5 for NTRP=2; 8 for NTRP=3; 4 for NTRP=4; when paramCombination-CJT-PS-r18 is configured to 4 or 5 and PCSI-RS=32,

-	1 for NTRP=1; 1, 2, 3 for NTRP=2; 1, 2, 3, 4, 5, 6,7 for NTRP=3; 1, 2, 3 for NTRP=4, when PCSI-RS=4 and higher layer parameter typeII-PortSelectionRI-Restriction-r18 is configured with ri=1 for any i>1.

-	If NL>1, the UE is expected to select one of the NL configured combinations of {α1,…,αNTRP} and report the index of the selected combination, where the index value 0 corresponds to the first configured combination and the index value NL-1 corresponds to the NL-th configured combination. If NL=1, a single combination of {α1,…,αNTRP} is configured and the selection is not reported.

<Unchanged part omitted>

-	The value of N∈{2,4} is configured with the higher-layer parameter valueOfN-CJT-r18, when M=2.

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-r18, when M=2, and R=1 when M=1, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-PortSelectionRI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-PortSelectionRI-Restriction-r18 forms the bit sequence r3,r2,r1,r0, where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…, 3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N0 selected CSI-RS resources corresponds to the nonzero bit with lowest index.

<Unchanged part omitted>

5.2.2.2.10	Enhanced Type II codebook for predicted PMI

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-r18'

-	The values of N1 and N2 are configured with the higher layer parameter n1-n2‑codebookSubsetRestriction-r18. The supported configurations of N1,N2 for a given number of CSI-RS ports and the corresponding values of O1,O2  are given in Table 5.2.2.2.1-2. The number of CSI-RS ports, PCSI-RS, is 2N1N2.

-	The values of L, β and pυ are determined by the higher layer parameter paramCombination-Doppler-r18, where the mapping is given in Table 5.2.2.2.10-1.

-	The UE is not expected to be configured with paramCombination-Doppler-r18 equal to

-	4, 5, 6, 7, 8, or 9 when PCSI-RS=4,

-	8 or 9 when PCSI-RS<32

-	8 or 9 when higher layer parameter typeII-RI-Restriction-r18 is configured with ri=1 for any i>1.

-	8 or 9 when R=2.

<Unchanged part omitted>

-	The value of R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-r18, where R and the corresponding value of N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-RI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-RI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The value of N4∈{1,2,4,8} is configured by the higher layer parameter vectorLengthDD, such that the PMI indicates N3 precoder matrices for each of the N4 consecutive slot intervals of duration d slots, as defined in Clause 5.2.1.4.2.

<Unchanged part omitted>

5.2.2.2.11	Further enhanced Type II port selection codebook for predicted PMI

For 4 antenna ports {3000, 3001, …, 3003}, 8 antenna ports {3000, 3001, …, 3007}, 12 antenna ports {3000, 3001, …, 3011}, 16 antenna ports {3000, 3001, …, 3015}, 24 antenna ports {3000, 3001, …, 3023}, and 32 antenna ports {3000, 3001, …, 3031} per CSI-RS resource, the UE configured with K∈{4,8,12} aperiodic CSI-RS resources or with a single periodic or semi-persistent CSI-RS resource in the resource set for channel measurement and with codebookType set to 'typeII-Doppler-PortSelection-r18'

-	the number of CSI-RS ports, PCSI-RS, is configured as in clause 5.2.2.2.4.

-	The values α, M and β are determined by the higher layer parameter paramCombinationDoppler-PS-r18, where the mapping is given in Table 5.2.2.2.7-1 and the applicable configuration restrictions are described in Clause 5.2.2.2.7.

-	The parameter N∈{2,4} is configured with the higher-layer parameter valueOfN-Doppler-r18, when M=2.

-	The parameter R∈{1,2} is configured with the higher-layer parameter numberOfPMI-SubbandsPerCQI-Subband-r18, when M=2, and R=1 when M=1, where R and the corresponding value of parameter N3 are defined as in clause 5.2.2.2.5.

-	The UE shall report the RI value υ according to the configured higher layer parameter typeII-PortSelectionRI-Restriction-r18. The UE shall not report υ>4. The bitmap parameter typeII-PortSelectionRI-Restriction-r18 forms the bit sequence r3,r2,r1,r0 where r0 is the LSB and r3 is the MSB. When ri is zero, i∈0,1,…,3, PMI and RI reporting are not allowed to correspond to any precoder associated with υ=i+1 layers.

-	The value of N4=1 is configured by the higher layer parameter vectorLengthDD, such that the PMI indicates N3 precoder matrices for one slot interval of duration d slots, as defined in Clause 5.2.1.4.2.

The PMI value corresponds to the codebook indices of i1 and i2 as described in Clause 5.2.2.2.7 and the precoder matrices for 1-4 layers are obtained from the PMI codebook as in Table 5.2.2.2.7-3.

<Unchanged part omitted>