3GPP TSG RAN WG1 #120		R1-2500604

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.6.3

Source:	NEC

Title:              	Discussion on LP-WUS operation in RRC CONNECTED mode

Document for:	Discussion and Decision

Introduction

In the RAN1#119, the following agreements on LP-WUS operation in RRC CONNECTED mode were achieved:

Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable. Agreement LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication. - Note: LP-WUS indication does not impact SCell dormancy behavior. Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell. Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. Agreement For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask the feasibility of the above potential solution. Conclusion For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask whether RAN2 has same understanding or not. Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

In this contribution, we provide our views on LP-WUS operation in RRC CONNECTED mode.

Discussion

First of all, in RRC_IDLE/INACTIVE, it is expected that a LP-WUS configuration applied to a group of RRC_IDLE/INACTIVE UEs will be introduced, e.g., the LP-WUS configuration is provided in system information for all UEs, and each LP-WUS capable UE may, based on its specific parameter such as UE-ID to determine which LP-WUS resource it will apply. It is noted that in this way, multiple UEs will be associated with the same LP-WUS resource, which causes false alarm issues (i.e., unnecessary wake-ups by indications intended for the other UEs). However, for LP-WUS operation in RRC_CONNECTED, the UE can receive dedicated RRC signaling and has UE-specific monitoring resources, so besides common LP-WUS configuration, the UE-specific LP-WUS configuration is possible for RRC_CONNECTED UE.

For UE-specific LP-WUS configuration, the network could use dedicated RRC signaling such as RRCSetup, RRCResume during RRC connection establishment, or RRCReconfiguration during RRC_CONNECTED to provide the UE-specific LP-WUS configuration to the UE. Since the LP-WUS monitoring resource is dedicated to each individual UE, no false alarm issue will be introduced then.

On the other hand, considering a LP-WUS may occupy relatively large time-frequency resources, e.g., ~ 5MHz bandwidth and several OFDM symbols, supporting UE-specific LP-WUS in RRC connected mode may introduce considerable overhead. Therefore, group based LP-WUS can also be considered for RRC_CONNCTED to improve network resource efficiency. The group based LP-WUS can be jointly designed with the legacy DCP mechanism (i.e., wake up signal for C-DRX) to improve the false alarm of wake-up. For example, the UE could firstly monitor a group-common LP-WUS, after a wake-up indication is received, UE may monitor a DCP to further check whether a wake-up indication for it is comprised, and UE may fallback to LP-WUS monitoring if no wake-up indication is received. Through this method, a better tradeoff between power saving and false wake-up rate can be achieved. .

In RAN1#116b, several options for case 1 for the LP-WUS procedures to trigger PDCCH monitoring were agreed for further study. In our view, case 1, i.e., LP-WUS combined with C-DRX, can be supported to further improve the power saving of C-DRX. The following figure gives an example for option 1-1 of case 1, where an LP-WUS monitoring occasion is configured for each drx-onDurationTimer based on e.g., offset in every C-DRX cycle, once LP-WUS is detected, the UE starts drx-onDurationTimer to enter Active Time for PDCCH monitoring. Otherwise the UE keeps in inactive mode until the next C-DRX cycle. Since the power consumption of LP-WUS monitoring is less than DCP monitoring by MR, further power saving gain can be achieved by replacing DCP with LP-WUS.

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead.

For LP-WUS MOs in connected mode, a common or similar way for both option 1-1 and option 1-2 is preferred. For option 1-1, the design should follow the legacy DRX on-duration timer configuration, therefore, approach 2 is a more straightforward choice, i.e., on-duration timer is configured first, and LP-WUS MO is determined based on an offset value. Option 1-2 may follow a similar method as Option 1-1, i.e., approach 2.

Proposal 2: For LP-WUS MOs in connected mode for Option 1-1, support approach 2.

Proposal 3: For LP-WUS MOs in connected mode for Option 1-2, support approach 2.

Conclusion

In this contribution, we discuss the LP-WUS operation in RRC CONNECTED mode, and the following proposals are made:

Proposal 1: support LP-WUS in combination with DCP to further reduce power consumption without introducing too much overhead.

Proposal 2: For LP-WUS MOs in connected mode for Option 1-1, support approach 2.

Proposal 3: For LP-WUS MOs in connected mode for Option 1-2, support approach 2.

References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN meeting #102, Dec. 11-15, 2023