3GPP TSG-RAN WG1 Meeting #120	R1- 2500686

Athens, Greece, February 17th – February 21st, 2025

Agenda Item:	9.1.1

Source:	NVIDIA

Title:	Specification support for AI-enabled beam management

Document for:	Discussion

# 1	Introduction

The approval of the Rel-19 work package marks the next wave of 5G Advanced evolution [1]. The package includes a work item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface and the latest version of the work item description can be found in [2].

The work item consists of multiple objectives, including specification support for AI/ML-enabled beam management:

- Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

At the previous RAN1 meeting, several agreements on specification support for AI/ML-enabled beam management were made (see Appendix).

In this contribution, we discuss the remaining issues of specification support for AI/ML-enabled beam management.

# 2	Background

5G NR can operate at a wide range of frequencies, ranging from sub-6 GHz to millimeter wave frequencies. To support operation over such a wide range of carrier frequencies, NR has been designed to utilize beam-based operation, where gNB and UE may use transmit and receive beamforming for all channels and signals.

Initial transmit and receive beam finding is used in initial access and connected UEs on, for example, a non-standalone NR carrier in MR-DC. The gNB transmits multiple SSBs that are beamformed in different directions. Up to a maximum of 64 such SSBs can be transmitted in a burst of 5 ms within a period of 20 ms. The UE detects an SSB and stores the information related to the corresponding receive beam. After detecting the SSB and decoding the MIB, the UE knows which SSB it has detected.

In initial access, the UE may use the same receive beam to receive SIB1 as it used to receive the detected SSB. SIB1 conveys the association information which links the index of a SSB to a corresponding set of PRACH preambles in time and frequency. In a non-standalone NR carrier in MR-DC, the UE does not need to read SIB1 and instead may get the information from dedicated RRC signaling. The UE would perform PRACH transmission in the PRACH resource associated with its detected SSB.

The gNB can detect the PRACH using a beam direction same as the beam direction it used to transmit the SSB. When the gNB detects a PRACH from the UE, it knows which SSB the UE detected and can determine its transmit beam for communicating with the UE. The transmit and receive beams established in initial transmit and receive beam finding are used to complete random access procedure and the subsequent procedures before dedicated RRC configuration is set up.

In general, the downlink beam management procedures consist of three steps: Procedure 1 (P-1): SSB-based beam sweeping; Procedure 2 (P-2): CSI-RS based transmit-end beam refinement; Procedure 3 (P-3): CSI-RS based receive-end beam refinement. There are also uplink beam management procedures.

AI/ML based algorithms may find applications in many steps of beam management. Beam prediction in time and/or spatial domain can help reduce overhead and latency, as well as improving beam selection accuracy.

Specifically, Rel-19 will focus on specification support for the following two beam management use cases:

- BM-Case 1: Spatial-domain DL beam prediction for Set A of beams based on measurement results of Set B of beams

- BM-Case 2: Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams

For each of these two use cases, the AI/ML model training can be at network side or at UE side. The AI/ML training process can be online, where the model being used for inference is (typically continuously) trained in (near) real-time with the arrival of new training samples. The AI/ML training process can also be offline, where the AI/ML model is trained based on collected dataset and the trained model is later used or delivered for inference. Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

Observation 1: Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

For AI/ML model inference, it is natural to focus on inference at network (resp. UE) side when training is performed at network (resp. UE) side. The specification support of other cases where training and inference reside at two different sides (e.g., training at network side and inference at UE side, or vice versa) can be postponed.

## 2.1 	Spatial-domain DL beam prediction

In BM-Case 1, spatial-domain DL beam prediction for Set A of beams is performed based on measurement results of Set B of beams, i.e., Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

There can be multiple options for Set A of beams and Set B of beams.

In one alternative, Set B of beams is a subset of Set A of beams. In another alternative, Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams). In either case, the underlying assumption is that the measurement effort on Set B of beams is less than the measurement effort on Set A of beams, and AI/ML based algorithms based on e.g., super resolution models are applied to predict the quality of the full set of beams in Set A.

Proposal 1: For BM-Case 1, introduce specification support for associating Set A of beams with Set B of beams.

When it comes to AI/ML model input for spatial-domain DL beam prediction, there are many different possibilities. The simplest option would be L1-RSRP measurement based on Set B of beams.

Proposal 2: For BM-Case 1, at least introduce specification support for using L1-RSRP measurement based on Set B of beams as AI/ML model input.

Additional assistance information besides L1-RSRP measurement based on Set B of beams may be used as AI/ML input for the spatial-domain DL beam prediction. Examples brought up include Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.), expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction), UE position information, UE direction information, Tx beam usage information, UE orientation information, etc. Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the spatial-domain DL beam prediction.

For UE-sided spatial-domain DL beam prediction, UE needs to report its inference results to the network. To this end, two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report. UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference. Besides reporting beam information, UE may report L1-RSRP.

Proposal 3: For BM-Case 1, introduce specification support for UE to report inference results on L1-RSRP and beam information in the resource set for Set A.

## 2.2 	Temporal DL beam prediction

In BM-Case 2, temporal DL beam prediction for Set A of beams is performed based on the historic measurement results of Set B of beams, i.e., Set B is a set of beams whose measurements are taken as inputs of the AI/ML model.

There can be multiple options for Set A of beams and Set B of beams.

In one alternative, Set B of beams is a subset of Set A of beams. In another alternative, Set A and Set B are different (e.g., Set A consists of narrow beams and Set B consists of wide beams). In yet another alternative, Set A and Set B are the same. In either of these cases, the underlying assumption is that the historical measurement results on Set B of beams are used as input to AI/ML based algorithms to predict future beam quality of the set of beams in Set A.

Specifically, in BM-Case 2, the measurement results of K (K>=1) latest measurement instances are used for AI/ML model input, and the AI/ML model output includes F (F>=1) predictions for F future time instances, where each prediction is for each time instance.

Proposal 4: For BM-Case 2, introduce specification support for associating Set A of beams with Set B of beams.

When it comes to AI/ML model input for temporal DL beam prediction, there are many different possibilities. The simplest option would be the historical optimal beam index based on Set B of beams. Another option would be L1-RSRP measurement based on Set B of beams.

Proposal 5: For BM-Case 2, at least introduce specification support for using historical optimal beam index and/or L1-RSPR measurement based on Set B of beams as AI/ML model input.

Additional assistance information besides L1-RSRP measurement based on Set B of beams may also be used as AI/ML input for the temporal DL beam prediction. Examples brought up include Tx and/or Rx beam angle, position information, UE direction information, positioning-related measurement (such as Multi-RTT), expected Tx and/or Rx beam/occasion for the prediction (e.g., expected Tx and/or Rx beam angle for the prediction, expected occasions of the prediction), Tx and/or Rx  beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight directions (azimuth and elevation), 3dB beamwidth, etc.), increase ratio of L1-RSRP for best N beams, UE orientation information. Comprehensive evaluation results showing convincing performance gains is needed to nail down the essential assistance information needed for the temporal DL beam prediction.

For UE-sided temporal DL beam prediction, UE needs to report its inference results for a number of future time instances to the network. To this end, two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report. UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference. Besides reporting beam information, UE may report L1-RSRP.

Proposal 6: For BM-Case 1, introduce specification support for UE to report inference results on L1-RSRP and beam information in the resource set for Set A for a number of future time instances.

# 3	Specification support

Although it is likely that most of the AI/ML algorithms for beam prediction can be up to implementation, there are several aspects that have potential specification impact.

AI/ML model training can occur at gNB or UE side. In either case, the training entity can benefit from assistance from the other entity for training data collection. Relevant areas for discussion include training data type/size, training data source determination, and assistance signalling and procedure for training data collection.

Regarding the data collection for AI/ML model training at UE side, it is important to study the potential specification impact by considering aspects including whether and how to initiate data collection, configurations (e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B), assistance information from network to UE, etc.

It is not always possible to collect sufficient beam management data effectively and efficiently in real deployments. One promising solution is to utilize a digital twin network – a digital replica of a corresponding physical network – to generate synthetic data, which is annotated information that a simulation running in a digital twin network generates as an alternative to real-world data. Put another way, synthetic data is created in a cyber sibling of the physical world rather than being measured or collected from the real world. It may be artificial, but synthetic data reflects real-world data in a statistical sense. Digital twin networks capture the precise geometry and material properties of objects in the environment to produce datasets for training AI/ML models used for beam management.

Similar to many other functionalities in 3GPP systems, the usage of AI/ML model for a certain functionality should be under network control, if the functionality at one side cannot be made transparent to the other side. Therefore, assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, model selection, etc. should be investigated.

Proposal 7: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

AI/ML models are data driving. They are trained to learn patterns from data. But the environment of a mobile communication system is dynamic and changes over time, and thus the data also keeps changing. As the data from the environment changes, the AI/ML model performance may be degraded. Therefore, it is essential to monitor the AI/ML model performance and regularly update the model to maintain satisfactory model performance. To this end, assistance signalling and procedure for model performance monitoring and model update/tuning should be investigated.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, the model monitoring can be mainly performed at the network-side. To facilitate the network-side model monitoring, it is important to study the necessity and the potential specification impacts of several aspects, including UE reporting of beam measurement(s) based on a set of beams indicated by gNB, signaling (e.g., RRC-based, L1-based).

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, there are different alternatives for model monitoring.

- UE-side model monitoring, where UE monitors the performance metric(s) and UE makes decision(s) of model selection/activation/deactivation/switching/fallback operation. Potential specification impacts include configuration/signaling from gNB to UE for performance monitoring, as well as support of indication/request/report from UE to gNB for performance monitoring. Dedicated resource set for monitoring and monitoring report configuration can be configured in a dedicated CSI report configuration used for monitoring. The ID of an inference report configuration can be configured in the monitoring report configuration to link the inference report configuration to the monitoring report configuration.

- Network-side model monitoring, where network monitors the performance metric(s) and network makes decision(s) of model selection/activation/deactivation/switching/ fallback operation. Potential specification impacts include configuration/signaling from gNB to UE for measurement and/or reporting, UE reporting to network (e.g., for the calculation of performance metric), and indication from network for UE to perform life cycle management operations.

- Hybrid model monitoring (i.e., UE-assisted performance monitoring), where UE calculates the performance metric(s) and reports to the network which then makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation.

Different performance metrics can be considered for AI/ML model monitoring for BM-Case1 and BM-Case2. Examples include beam prediction accuracy related KPIs (e.g., Top-K/1 beam prediction accuracy), link quality related KPIs (e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER), performance metric based on input/output data distribution of AI/ML, L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP, etc.

The potential specification impact of utilizing the aforementioned alternatives and the performance metrics for model monitoring requires investigation.

Proposal 8: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning.

When it comes to AI/ML model inference input, report/feedback of model input for inference (e.g., UE feedback as input for network side model inference) may be needed. In general, the type of model input, and model input acquisition and pre-processing may have potential specification impact. Similarly, when it comes to AI/ML model inference output, outputs generated by an AI/ML model may need to be delivered from gNB to UE or from UE to gNB. The post-processing of AI/ML model inference output may have potential specification impact as well.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, it is beneficial to study potential specification impact of L1 reporting enhancements for AI/ML model inference, such as UE reporting of the measurement results of more than 4 beams in one reporting instance.

For BM-Case1 with a UE-side AI/ML model, the information of AI/ML model inference such as (1) the beam(s) that is based on the output of AI/ML model inference and (2) predicted L1-RSRP corresponding to the beam(s) should be reported to network. Besides, confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) can also be reported from UE to network. The potential specification impact of L1 signaling to support the reporting requires investigation.

For BM-Case2 with a UE-side AI/ML model, the information of AI/ML model inference such as (1) the beam(s) of N future time instance(s) that is based on the output of AI/ML model inference, (2) predicted L1-RSRP corresponding to the beam(s), and (3) information about the timestamp corresponding the reported beam(s) should be reported to network. Besides, confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) can also be reported from UE to network. The potential specification impact of L1 signaling to support the reporting requires investigation.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable, can be beneficial. Also, beam indication from network for UE reception is needed.

Different UEs may have different capabilities when it comes to the support of AI/ML algorithms for beam prediction in spatial/time domain. Therefore, UE capability for AI/ML based beam prediction including model training, model inference and model monitoring needs to be investigated and defined.

Proposal 9: For AI/ML based beam prediction in spatial/time domain, introduce specification support for UE capability signaling for AI/ML based beam prediction including model training, model inference and model monitoring.

Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. Which aspects should be specified as conditions of a Feature/FG available for functionality will need to be discussed specifically for AI/ML-based beam prediction in spatial/time domain.

Proposal 10: For AI/ML based beam prediction in spatial/time domain, introduce specification support for conditions of a Feature/FG available for functionality.

Model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will need to be discussed specifically for AI/ML-based beam prediction in spatial/time domain.

For UE-sided model, associated ID may be configured within CSI framework. UE may assume the properties of a DL Tx beam or beam set are similar to those of a different DL Tx beam or beam set assigned with the same associated ID.

Proposal 11: For AI/ML based beam prediction in spatial/time domain, introduce specification support for additional conditions to include them in model description information during model identification.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: Offline training may be more feasible for the near future. But in the long run, it is vital that the AI/ML models can learn continuously to adapt to varying environments, site-specific conditions, and heterogenous configurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1: For BM-Case 1, introduce specification support for associating Set A of beams with Set B of beams.

Proposal 2: For BM-Case 1, at least introduce specification support for using L1-RSRP measurement based on Set B of beams as AI/ML model input.

Proposal 3: For BM-Case 1, introduce specification support for UE to report inference results on L1-RSRP and beam information in the resource set for Set A.

Proposal 4: For BM-Case 2, introduce specification support for associating Set A of beams with Set B of beams.

Proposal 5: For BM-Case 2, at least introduce specification support for using historical optimal beam index and/or L1-RSPR measurement based on Set B of beams as AI/ML model input.

Proposal 6: For BM-Case 1, introduce specification support for UE to report inference results on L1-RSRP and beam information in the resource set for Set A for a number of future time instances.

Proposal 7: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection.

Proposal 8: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning.

Proposal 9: For AI/ML based beam prediction in spatial/time domain, introduce specification support for UE capability signaling for AI/ML based beam prediction including model training, model inference and model monitoring.

Proposal 10: For AI/ML based beam prediction in spatial/time domain, introduce specification support for conditions of a Feature/FG available for functionality.

Proposal 11: For AI/ML based beam prediction in spatial/time domain, introduce specification support for additional conditions to include them in model description information during model identification.

# Reference

X. Lin, “The Bridge Toward 6G: 5G-Advanced Evolution in 3GPP Release 19,” arXiv preprint arXiv:2312.15174, December 2023.

RP-243244, “Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,” 3GPP TSG RAN #106, December 2024.

# Appendix A

## A.1	RAN1#119 agreements

Agreement

- For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Agreement

Send LS to RAN2 with below information.

RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

RAN 1 made the following agreements related to the Questions from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848:

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Q3: Is NW-side additional condition functionality specific?

Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2.

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

Answer to Q4: And please refer to the agreements related to the Questions from RAN 2.

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2.

Q5: What is the content of applicable functionality reporting in Step 4?

Answer to Q5: Please refer to the agreements related to the Questions from RAN 2.

Q6: What is the content of inference configuration in Step 5?

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

Answer to Q7: Please refer to the agreements related to the Questions from RAN 2.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2.

Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

Answer to Q9: Please refer to the agreements related to the Questions from RAN 2.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

Answer to Q10:  Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.