3GPP TSG RAN WG1 #120                                                       R1-2500727

Athens, Greece, November 15th – 22nd, 2025

Agenda Item:	9.2.3

Source:	Xiaomi

Title:	Discussion on the support of 3-antenna-port CB based transmissions

Document for:	Discussion and Decision

Introduction

The Rel-19 WID on NR MIMO Phase 5 is approved [1], which includes the following objective:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

In RANP#105 meeting, the scope expansion on Rel-19 MIMO was agreed and the scope for 3Tx transmission is as below [2],

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource

Note: UL full power transmission mode 1 and 2 are not supported.

Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

In RANP#106 meeting, a clarification was endorsed on the scope of the WID for 3Tx transmission as below [3],

- Proposal: RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1.

  - where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports.

  - No RAN4 impact.

In this contribution, we discuss the leftover issues related to the enhancements for 3Tx UE.

SRS for Antenna Switching

In order to acquire the DL CSI accurately, SRS for antenna switching should be enhanced for 3Tx UE too, otherwise the DL CSI would be sub-optimal and leads directly to a loss in the DL throughput.

At present, mobile phone with 4 receive antennas is already supported in spec and real implementation. In order to support the SRS for antenna switching for a 3 Tx UE, 3T4R should be supported in our view. For 3T3R, since antenna switching is not really needed, the SRS ports can be transmitted in one symbol.

Currently, 6Rx is supported in RAN4 spec for hand-held UEs in Rel-19. Thus 3T6R should also be naturally supported in RAN1.

Considering the spec efforts, SRS antenna switching configurations 3T3R and 3T6R are supported for Rel-19 3Tx transmission in the RAN plenary meeting, and at least two issues can be discussed on how to support these configurations.

- SRS configuration for 3T6R

- SRS configuration for 3T3R;

- UE capability signaling;

In RAN1#118bis meeting, the agreement has already been made on the support of 3T6R antenna switching configuration [4],

Agreement

To support 3T6R antenna switching for a 3TX UE,

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’,

- Reuse the principles used for 4T8R, i.e.,

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources, transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports, or

  - For 4T8R 3T6R, zero or one or two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'aperiodic', where in the case of one resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports with port 1003 disabled, and the SRS ports of the resource in the set are associated with a different UE antenna ports. In the case of two resource sets a total of two SRS resources are transmitted in different symbols of two different slots, and where the SRS ports of each SRS resource in the given two sets are associated with a different UE antenna ports.

Note: This is not meant to be a text proposal to be captured for the specifications.

In RAN1#119 meeting, the following conclusion and agreement have been made on the support of SRS antenna switching configurations [5],

Conclusion

To support 3T6R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e., when two aperiodic SRS resource sets are configured,

  - if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Agreement

The following capabilities are introduced for Rel-19

- 't3r3' for 3T3R.

- 't3r6' for 3T6R.

Issue#1: SRS configuration for 3T3R

Since the SRS for codebook can be configured with 4-port SRS resource with the last port muted, it would be natural to apply the same SRS configuration for the case “T=R” of SRS for antenna switching. Also, if 3T6R can be downgraded to 3T3R, the antenna switching configuration of 3T3R has to be supported first.

Similar to 3T6R, the following can be supported for SRS configuration of 3T3R.

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’.

- Reuse the principles used for xTyR, where x=y.

- The SRS resource configured is a 4-port SRS resource with port1003 disabled.

Proposal 1: For 3T3R, the SRS antenna switching configuration is as below.

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’.

- Reuse the principles used for xTyR, where x=y.

- The SRS resource configured is a 4-port SRS resource with port1003 disabled.

Issue#2: UE capability signaling

As usual, 3T3R and 3T6R can be supported up to UE capability reporting. An issue that worth discussing is that which fallback SRS configurations for antenna switching can be supported. In our view,

3T3R can be supported as fallback configuration of 3T6R.  Thus,

For 3T3R configuration, the following downgraded antenna switching configurations can be considered:

- 3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

For 3T6R configuration, the following downgraded antenna switching configurations can be considered:

- 3T6R-2T6R-1T6R-3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

Proposal 2: Further study the fallback antenna switching configurations for 3T3R and 3T6R.

Proposal 3: For 3T3R configuration, the following downgraded antenna switching configurations can be considered:

- 3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

Proposal 4: For 3T6R configuration, the following downgraded antenna switching configurations can be considered:

- 3T6R-2T6R-1T6R-3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

Conclusion

In this contribution, we provide our views on the following issues.

Proposal 1: For 3T3R, the SRS antenna switching configuration is as below.

- The resourceType for a configured SRS resource set can be one of ‘periodic’, ‘semi-persistent’ and ‘aperiodic’.

- Reuse the principles used for xTyR, where x=y.

- The SRS resource configured is a 4-port SRS resource with port1003 disabled.

Proposal 2: Further study the fallback antenna switching configurations for 3T3R and 3T6R.

Proposal 3: For 3T3R configuration, the following downgraded antenna switching configurations can be considered:

- 3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

Proposal 4: For 3T6R configuration, the following downgraded antenna switching configurations can be considered:

- 3T6R-2T6R-1T6R-3T3R-2T4R-1T4R-1T2R-2T2R-1T1R

References

- RP-234007, New WID: NR MIMO Phase 5 , Samsung, RAN#102, December 2023

- RP-242394 WID revision: NR MIMO Phase5, Samsung, RAN#105, September 2024

- RP-243265 Way forward on REL-19 MIMO 3T3R, RAN#106, December 2024

- 3GPP Chairman’s notes, RAN1#118bis

- 3GPP Chairman’s notes, RAN1#119