3GPP TSG RAN WG1 #120	                                         	           R1-2500792

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.6.2

Source:	Moderator (Apple)

Title:	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode

Document for:	Discussion/Decision

# Introduction

One of the objectives in Rel-19 WI on LP-WUS/WUR (RP-241824) is to support LP-WUS operation in idle/inactive mode:

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

This contribution summarizes the discussions on LP-WUS operation in IDLE/INACTIVE modes in RAN1#120.

Section 2 provides a summary of the agreements as the outcome of RAN1#120 discussions. Section 3 captures the proposals for online sessions. Section 4 documents the detailed discussions. Section 5 provides guidance for RAN1#120bis discussions. Agreements from previous meetings and companies’ proposals from the contributions are captured in the Appendix.

# RAN1#120 Agreements

# Proposals for Online Sessions

## Proposals for Monday Online

Proposal 5-2r1

The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS

Proposal 2-2r1

For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided.

- The reference point for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO.

- FFS other offset value(s) to determine the MOs of the LO

Proposal 4-1r1

The previous agreement in RAN1#119 is updated as follows:

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Proposal 3-1r1

For the UE capability report on the wake-up delay, adopt Alt 4:

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

- Send an LS to RAN4

Proposal 4-2

The mapping between LP-WUS/LP-SS beams and SSB beams is determined implicitly based on the resource configuration of LP-WUS/LP-SS.

Proposal 6-1

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- It is left to RAN4 to decide whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

## Proposals for Tuesday Online

Proposal 5-1r1

Confirm the following working assumption with the modification:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 31 32.

Proposal 1-1r1

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO).

- This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1.

- FFS conditions/restrictions for mapping multiple POs to one LO

- FFS the maximum number of POs per LO

Proposal 2-1r1

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1.

- gNB can configure up to [2 or 3] offset values.

- If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports,

  - Alt 1: the UE monitors the LO associated with the smallest offset that has a gap between the LO and the PO no less than the wake-up delay.

  - Alt 2: the UE monitors the LO(s) associated with all the offset value(s) that has a gap between the LO and the PO no less than the wake-up delay.

Proposal 3-1

For the UE capability report on the wake-up delay, down-select between the following:

- One of Alt 1/2/3

  - FFS Alt 1 or 2 or 3

- Alt 4

From RAN1#119:

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

# LP-WUS Operation in Idle/Inactive Mode

## LP-WUS configuration and monitoring

### LP-WUS MO configuration

The following was agreed on the multiple MOs per beam per LO:

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

For LO-to-PO mapping, we had the following agreements:

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Regarding the support of Options 2 on the mapping between LO and PO:

- Option 2:

Yes: [1] Futurewei, [2] TCL, [3] Huawei/HiSi, [7] CMCC, [12] Nokia (2 POs), [13] NEC, [17] Samsung, [21] Sharp, [25] Ericsson (up to 2 POs per LO)

No: [14] Sony, [16] Apple, [26] Nordic

There are still a few companies proposing to support Option 3 and/or Option B:

- Option 3: [2] TCL, [11] InterDigital (and Option B), [13] NEC, [24] DCM (or Option B)

[11] InterDigital: Revert the working assumption on not supporting Option 3 and Option B in case of supporting 32 subgroups within one MO.

- Option B: [11] InterDigital, [18] ETRI, [19] LG, [24] DCM

R=1: [18] ETRI

Whether to support repetition across multiple MOs (R=1 or R>=1):

- R>=1: [3] Huawei/HiSi, [4] ZTE, [7] CMCC, [20] MediaTek (<=5), [21] LG

- R=1: [8] vivo, [17] Samsung, [25] Ericsson, [26] Nordic

One MO confined within one slot and contiguous in time

- Confined within one slot and contiguous in time: [3] Huawei/HiSi, [20] MediaTek

- Can be multiple slots: [14] Sony (contiguous), [16] Apple (FFS contiguous), [17] Samsung

[3] Huawei/HiSi: >4 MOs per PO if one LO maps to multiple POs

Some more detailed proposals are included at the end of this sub-section.

There is good support for Option 2.

However, it seems that no strong arguments have been provided to justify the reconsideration of Option 3 or Option B if 32 subgroups is supported. Therefore, this discussion is deprioritized in this meeting.

Proposal 1-1

For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO), where UEs corresponding to 2 POs can monitor the same LO.

- This is supported if the number of subgroups per PO is no more than 16.

- [FFS other conditions]

| Company | Comments |
| --- | --- |
| Qualcomm | Support the proposal. |
| Spreadtrum | Y |
| Xiaomi | If the working assumption that maximum 32 subgroups per LO is supported, we think it is OK to support this Proposal 1-1, since the overall number of subgroups of the 2 PO does not exceed 32 subgroups. But by the same logic, why to restrict the number of Pos is 2? For example, if the number of PO is 4, and the number of subgroups per PO is no more than 4, would this case be also OK to support? |
| CMCC | Support |
| ZTE, Sanechips | It is not preferred to restrict the number of subgroups when multiple POs are mapped to one LO. Otherwise, in this case, the UE can not benefit from 32 subgroups per PO when multiple POs are mapped to one LO. Also, I do not know why this bullet comes out. |
| CATT | We had discussed the options for LO-to-PO mapping a few meetings, and we think it is the time to have the decision. We do not support Option 2 and Option 3 due to the additional specification complexity even the maximum subgroups is 32 among multiple POs for Option2. |
| Huawei, HiSilicon | We support Option 2 in general. And we share the similar question as Xiaomi on why PO number is restricted to 2. We also share the similar view as ZTE that the sub-bullet is unnecessary. No justification is provided on the reason to restrict 16 subgroups per PO. |
| Sharp | Since the maximum POs(Ns) for a PF is 4, We prefer the maximum number of POs associated with the same LO can be up to 4. |
| vivo | If this proposal assumes to support that a LP-WUS/LP-WUS MO is associated with up to 32 subgroups, we can go with this proposal. Besides, in my mind, the subbullet is reasonable which should be kept. Otherwise, we cannot agree with this proposal. |
| LG | Support the proposal. |
| Samsung | We share a same understanding with Xiaomi. In our understanding, the key point is that we should limit the subgroups associated with one LP-WUS to 32 to avoid to increase the resource overhead. And support one-to-multiple and multiple-to-multiple for the mapping relationship between LO and PO can help to reduce the resource overhead for LP-WUS. If the number of subgroups is X for one PO based on configuration, if N*X <=32, the N PO can be associated with one PO, and follow a same logic, if N*X <= M*32 (M<N), the N PO can be associated with M PO. |
| Nokia1 | We support the configuration for mapping at least two POs to one LO. We are open to consider if we can map more, e.g. 4 POs to one LO, depending on the deployment. |
| DCM | Support with single offset. |
| Nordic | Do not support or support with restrictions: - POs of the same PF - no increase in number of MO UE has to monitor (i.e. 4) - no increase in number of sub-groups supported by LP-WUS |
| FW | We prefer to support Option 2 even if the number of subgroups per PO is 32 along with an updated definition of Option B as follows “A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its group or subgroup ID.”. The reason is that if the LO is determined based on an offset with respect to the reference PO, then for 2 POs that are spaced less than 10ms apart, an LO shall most likely not span more than the spacing between the two POs which can be limiting. |
| OPPO | We a fine to support option 2, if the number of sub-group is no more than 16. |
| Panasonic | We are open to discuss the possibility of one LO covering multiple POs. If there are multiple cases and conditions to support this, we prefer to clarify together. |
| Moderator | Proposal 1-1r1 For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - FFS the maximum number of POs per LO |

Question 1-2

A few issues are related: whether one MO should be confined within a slot or can span across multiple slots, whether the time domain resource for an MO should be contiguous in time, whether to support repetitions across MOs (R>1).  Please share your views on these issues and explain why.

| Company | Support R>1? (Y/N) | Comments |
| --- | --- | --- |
| Qualcomm | Y | R=1 or >1 can be configured by gNB. In the basic LP-WUS design, we analyzed that repetition within MO is not efficient for performance. Repetition across MOs can be considered for cases that basic design of LP-WUS covers a small portion of the cell. |
| Xiaomi |  | For “whether one MO should be confined within a slot or can span across multiple slots”, we think this issue should not be discussed now, since the MO resource is highly dependent by the payload, M value for OOK-4 waveform, it is consequence of the LP-WUS signal design being discussed in 9.6.1, rather than a design target. So we suggest to wait for LP-WUS signal design being discussed in 9.6.1. For “whether the time domain resource for an MO should be contiguous in time,” we think it is better for a simple LR implementation if an MO is contiguous in time. we also think this issue is related to collision handling between LP- WUS and legacy NR channels. If we aim to keep an MO contiguous in time, this may need to dropping of overlaid legacy NR channels. For “whether to support repetitions across MOs (R>1)”, we think it is highly related to whether coverage target can be met with a single repetition. Based on simulation results in 9.6.1, in some cases coverage enhancement is needed. So we think repetitions across MOs (R>1) can be supported. |
| CMCC | Y | Based on the current evaluation results in AI 9.6.1, the LP-WUS design for both OOK-receiver and OFDM-receiver has a certain gap in coverage requirements (i.e. align with Msg3 coverage performance). Thus, R>1 can be supported to enhance the performance on LP-WUS detection. Furthermore, whether R is equal or larger than 1 can be configured by gNB. |
| CATT | Y | A LP-WUS MO should be configured and set at a given slot with contiguous in time. And the repetition should be supported for coverage enhancement. |
| Huawei, HiSilicon |  | According to current specification, the transmission of signals is usually slot based. Therefore, from gNB perspective, it is better to define a MO to be confined within a slot, which is more friendly to gNB implementation. Also, from UE perspective, having the time domain resource for an MO to be contiguous in time domain is beneficial for timeline design. On the repetition, we think it is better to wait for some discussion in 9.6.1. |
| Sharp |  | “confined within a slot” may be too limiting for the LP-WUS payload design. And we support R>1 for LP-WUS repetition in different LP-WUS MOs. |
| vivo | N | We want to support LP-WUS repetition within a MO. In our view, the difference for the LP-WUS repetition across different MOs i.e., inter-MO repetition, and LP-WUS repetition within a MO i.e., intra-MO repetition, is whether to define certain gap between nominal LP-WUS repetitions. The benefit to define gap with value larger than 0 between nominal LP-WUS repetitions is unclear. And some drawbacks for inter-LP-WUS MO repetition is the required number of LP-WUS MOs of a LO will increase which brings the risk of the fragmentation of a LO in time domain resource due to existing legacy signal/channels e.g., PO and SSB. Besides, multiple MO lengths may need to be configured if a preamble before LP-WUS signal is considered which increases the configuration complexity. For other issues, we think the length of a LP-WUS MO is configured by the network. And at least one complete LP-WUS signal should be carried in a LP-WUS MO. |
| LG | Y | We think one MO should be confined within a slot. For coverage, combining between MOs and repetition can be supported with UE implementation. |
| Samsung |  | For OOK-1 M=1, if one LP-WUS has 6 information bits and Manchester coding is applied, 12 OFDM symbols are used to transmit it. According to the previous agreements, we cannot decide whether these symbols are enough or not. If CRC bits are needed, it seems that we cannot avoid to transmit one LP-WUS span across multiple slots. |
| Nokia1 |  | Firstly, in order to facilitate the multiplexing with other NR signals, containing the LP-WUS to a slot would be useful. Also accounting that legacy devices are not aware of WUS, having MOs non-continuous in time would seem necessary intra or inter-MO continuity would need to be considered. Also in our understanding the gain of plain repetitions is limited for non-coherent receivers. It might be more beneficial to consider enabling increasing the redundancy by considering channel coding to generate the code points. |
| OPPO | Generally, Y | We consider one LP-WUS would be not longer than a slot. That will also contain the MO, there could be multiple MO and some possible repetition in time if the slots are available. |
| Panasonic |  | As we discussed in the contribution, depending on the value of M, number of information bits carried by LP-WUS in one MO and whether/how channel encoding, repetition and CRC is supported, the length of one MO can potentially be longer than one slot. |

Some detailed proposals:

| [1] Futurewei | Proposal 5: Support Option 2 for LO to PO mapping to alleviate the constraint on the total LO duration and to avoid the need to explicitly account for the potential overlap between LOs. Proposal 6: Support Option B (K LP-WUS MOs for a beam are divided into G groups of R*M LP-WUS MOs) and R>=1 when K>1 LP-WUS MOs are configured for each beam in an LO. |
| --- | --- |
| [2] TCL | Proposal 2: Consider the maximum number of information bits (including CRC) in an LP-WUS as 8 bits for a single LP-WUS subgroup. |
| [3] Huawei/HiSi | Proposal 9: For LP-WUS in IDLE/inactive mode, to minimize the number of codepoints per MO for a UE to monitor, the codepoint corresponding to all the subgroups is only monitored in some MOs. • In the MOs above, the maximum number of codepoints for a UE to monitor is 1 or no larger than the number of codepoints monitored in the other MOs. Proposal 10: To reduce the signalling/resource overhead, configure a MO/LO which is monitored by all the UEs to detect the codepoint corresponding to all the subgroups. Proposal 12: The power offset between SSB and LP-WUS/LP-SS should be specified and can be configured by gNB in system information. |
| [4] ZTE | Proposal 5: Clarify that if multiple POs associate to one LO,  from the UE perspective, one LO associates to one PO in a cycle  from the system perspective, one LO associates all the POs in a cycle Proposal 6: Discuss and decide using LP-SS or LP-WUS to achieve cell specific wake-up. • Slightly prefer to use LP-SS for cell specific wake-up • If LP-WUS is used to achieve cell specific wake-up, at least, 4POs would be needed to be associated to LO and LP-WUS design and coverage would be further impacted. Proposal 9: The LO and MO definition should be applicable for both FDD and TDD • FFS whether MO could be based on flexible symbols or slots in TDD Proposal 10: The QCL source for LP-SS/LP-WUS should be CD-SSB and the QCL type could be type C in FR1 and type D in FR2 Proposal 11: Consider the power offset between LP-SS and SSB • FFS the power control for LP-WUS |
| [6] CATT | Proposal 11: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO. Proposal 14: The UE falls back to legacy mechanism when the LP-WUS is in detection errors. |
| [7] CMCC | Proposal 6. If one LO is associated with multiple POs/PFs, support the reference PO/PF is the first PO/PF within the multiple POs/PFs. Proposal 8. RAN1 to discuss the UE behaviour when LP-SS/LP-WUS transmission overlap with existing pattern (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement), or when the time interval between LP-SS/LP-WUS transmission and existing pattern is smaller than the RF retuning time. |
| [8] vivo | Proposal 3: UE is required to monitor each of M groups per beam until its target LP-WUS is detected. Proposal 9: Support Option 2B-1: gNB configures one or two offset values. And if the configured offset(s) is smaller than the UE reported wake-up delay, the UE follows the legacy paging monitoring procedure. Proposal 10: When two offsets between LO and PO/PF are configured,  Each offset is configured corresponding to a wake-up delay category, i.e. deep sleep or ultra-deep sleep.  For UE with wake-up delay capability corresponding to ultra-deep sleep, it only monitors LP-WUS at a LO corresponding to the larger offset.  For UE with wake-up delay capability corresponding to deep sleep, it shall monitor LP-WUS in both two LOs corresponding to the two configured offsets. |
| [9] OPPO | Proposal 4: On the LO configuration for iDRX, if multiple offset values between a LO and a reference PO/PF are supported, similar definition as PEI could be considered, e.g., frame-level offset and symbol-level offset. Support different ranges of frame-level offsets from LO to PO. Proposal 5: In case of multiple offset values between an LO and reference PO/PF, down select Option 2B-1. |
| [10] Panasonic | Proposal 4: Depending on the supported number of beams, MO groups and repetitions within an LO, an LO is highly possible to span multiple slots. It should be further studied and discussed on whether an LO should always be contiguous or can be non-contiguous, considering the collision/interruption with other channels/signals. Proposal 5: To settle the LP-WUS design details first, before discussing whether there is any impact if the length of one MO spans multiple slots. |
| [11] InterDigital | Proposal 3: Support LO configuration with the following. • Frame offset from the start of a frame to a paging frame. • Symbol offset from the start of the frame to the start of the first MO. |
| [12] Nokia | Proposal: LO reference time location is defined in relation to PF. One common offset from PF to a reference frame before the PF is configured for all LOs, together with offsets for each PO-LO pair from the reference frame to indicate the start of a LO. Proposal: Define starting symbol for each the LP-WUS MOs in LO. |
| [13] NEC | Proposal 1: for RRC-IDLE/INACTIVE, the LP-WUS configuration indicates one or more LP-WUS frequency bands in a carrier, and for each LP-WUS frequency band, a periodicity and an offset of LOs are comprised in the configuration. |
| [14] Sony | Proposal 2 – RAN1 to investigate new schemes for the LP-WUS MO configuration that supports reduced/limited channel monitoring duration. Proposal 6 – Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space. Proposal 7 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme. |
| [16] Apple | Proposal 6: For multi-beam support of LP-WUS monitoring, the LP WUS MO(s) are defined in a similar way as for paging occasions and PEI occasions. |
| [17] Samsung | Proposal 6: For the relationship between LO and PO, support UEs corresponding to different POs, which are belong to multiple DRX cycle, monitor the same LO to further reduce the power consumption. Further clarified whether the proponent LO:PO association relationship belongs to Option 2 defined from UE perspective or should separate discussed. Proposal 8: For the mapping relationship between LO and PO, a revised option 3 can be considered to reduce the resource overhead from network perspective, which is • From network perspective, UEs monitoring the multiple POs are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Proposal 9: If the revised Option 3 can be supported, the number of LOs can be determined by configuration, and the maximum number of LOs associated with multiple POs should be limited to 2 to avoid excessive increase in paging latency. Proposal 10: The configuration of LO can be provided to the UE by system information for RRC_IDLE and RRC_INACTIVE mode. Proposal 11: The time location of LP-WUS occasion can be determined by a reference point and an offset, wherein the reference point is the start of a reference PF associated with the LO if LO associated with all the POs in a PF, or the reference point is the start of the one or more PO(s) associated with the LO if LO associated with one or more PO(s) in one DRX cycle. |
| [19] LG | Proposal #6: Use paging frame as reference point for offsets to allocate LP-WUS occasion. Proposal #8: The network shall determine the number of LP-WUS MOs for each beam in an LO using one of the following alternatives:  Alt. 1: Individual configuration of G, R, and M parameters  Network configures G, R, and M parameters  Total number of LMOs is determined as G * R * M  Alt. 2: Hybrid configuration with total LMO constraint  Network configures G and R parameters  Total number of LMOs K is also configured separately  M is derived from K/(G * R)  Alt. 3: Bounded configuration approach  Network configures G, R, and M parameters  Total number of LMOs K_max is determined by LO periodicity and LMO duration. Actual LMOs is bounded by K_max Proposal #9: For repetition and multiple information support, adopt one of the following approaches:  Alt. 1: Network ensures either R>1 with M=1, or M>1 with R=1  Alt. 2: It is up to UE capability to support values for R and M parameters that can be simultaneously greater than 1  Alt. 3: UE reports set of supported (R, M) value combinations Proposal #10: The LO-to-PO/PF offset indicates the starting point of an LP-WUS occasion. Within an LP-WUS occasion, LP-WUS monitoring occasions are allocated within LO in sequential order. Proposal #11: For allocation of N*K LP-WUS MOs, consider one of the following approaches:  Option 1: Configure only first LP-WUS MO position with sequential allocation  Option 2: Configure starting position per UE group (G)  Option 3: Configure starting position per beam (N)  Option 4: Configure starting position per information group (M) Proposal #12: For time-domain resource allocation of LMOs within an LO, the following sequential allocation methods shall be supported:  Option 1: LMOs are allocated in consecutive OFDM symbols  Option 2: LMOs are allocated sequentially within slot boundaries  If an LMO would cross a slot boundary, it starts at the beginning of the next slot  Option 3: Introduce pre-defined or configured LP-WUS starting points within each slot Proposal #13: For sequential allocation of N*K LP-WUS MOs, the following ordering approaches can be considered:  Option 1:  first, in increasing order of R  second, in increasing order of G  third, in increasing order of M  fourth, in increasing order of N  Option 2:  first, in increasing order of R  second, in increasing order of G  third, in increasing order of N  fourth, in increasing order of M  Option 3:  first, in increasing order of R  second, in increasing order of N  third, in increasing order of G  fourth, in increasing order of M Proposal #15: UEs with different wake-up delay should be assigned to different UE subgroups Proposal #16: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately  The number of those subgroups can be separately configured  Wake-up delays below a certain threshold are considered as shorter wake-up delay |
| [20] MediaTek | Proposal 1: Define LP-WUS MO is of X OFDM symbols, wherein X bits (X/4 bits) can be carried by a LPWUS of M = 2 or 4 (M = 1). Let X = 6 can be a baseline setting to accommodate 32 subgroups/codepoints Proposal 2: Given that a LO can include N×K MOs, let K≤5 for the repetition factor per beam. |
| [22] Lenovo | Proposal 1: Discuss LPWUS sub-band blocks FDMed with multiple MOs and how the LPWUS subgroup IDs in MOs are mapped to the FDMed LPWUS sub-band blocks to reduce network power consumption. Proposal 2: Support UEs corresponding to different POs monitor the different LO, LOs are separated according to the wakeup delay or resource offset from PO. Proposal 3: UEs monitoring the same or different PO are divided into multiple sets of subgroup according to the wakeup delay or resource offset from PO, with UEs within each set of subgroups monitoring the same LO. |
| [25] Ericsson | Proposal 8 The reference PF/PO is the start of a first paging frame associated with the LP-WUS LO. The UE determines the start of the reference frame for LP-WUS LO using the single offset value configured to the UE. Proposal 15 The number of POs addressable by LP-WUS monitored in a MO is up to 2. More than one PO is only configured if the number of configured UE subgroups per PO is less than 16 Proposal 16 When multiple POs are addressed by LP-WUS monitored in a MO, the total number of LP-WUS information bits is a sum of the LP-WUS information bits for subgroup indication per PO. |
| [26] Nordic | Proposal-6: Maximum number of information bits is log2(31+1)=5, i.e. 31 dedicated sub-groups and 1 sub-group assigned to “wake-up all”. • Consider one more codepoint/sub-group to be dedicated to early termination of monitoring for the UEs. |

### Offset between LO and PO/PF

For the offset value(s) between LO and PO/PF, the following was agreed:

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

This is a summary of companies’ views:

- Option 1-1: [3] Huawei/HiSi, [5] Spreadtrum, [6] CATT, [7] CMCC, [10] Panasonic, [12] Nokia, [17] Samsung, [18] ETRI, [25] Ericsson, [26] Nordic (modified Option 1-1)

- Configurability between Option 1-1 and Option 1-2: [13] NEC, [15] xiaomi

- Option 2B-1: [1] Futurewei, [2] TCL, [4] ZTE, [8] vivo, [9] OPPO (?), [14] Sony, [16] Apple, [19] LG, [20] MediaTek, [21] Sharp, [23] QC, [24] DCM

  - [1] Futurewei: support 3 candidate values for the offset, e.g. {80, 400, 720}

  - [1] Futurewei: If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE uses the smallest offset >= the reported wake-up delay.

- Option 2A: [11] InterDigital, [22] Lenovo

There are a good number of supporting companies for both Option 1-1 and Option 2B-1, with slightly more supporting companies for Option 2B-1. There is very little support for Configurability between Option 1-1 and Option 1-2. Based on the input, and considering that Option 1-1 is actually a subset of Option 2B-1 (with Option 2B-1, the gNB always has the choice to configure a single offset value), the proposal is to adopt Option 2B-1, with up to 2 offset values configured.

Proposal 2-1

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1.

- gNB can configure up to 2 offset values.

| Company | Comments |
| --- | --- |
| Qualcomm | We support the main bullet without the sub-bullet or with it as FFS. |
| Spreadtrum | Not support, multiple offset values will cause significant resource overhead. |
| Xiaomi | If multiple offsets are configured, UE has to additionally decide which offset to apply for LP WUS monitoring. And UE itself also has multiple UE wake up capabilities. How should UE determine where to monitor LP-WUS? We think it complicates UE behaviour. We are more inclined to support single offset, both option 1-1 and option 1-2 are OK. |
| CMCC | From our understanding, - Multiple offset values can be used to match different UE wake up capabilities, but will increase the LP-WUS overhead from network side. In comparison, a single offset value can be configured considering the largest UE wake up delay capability, and the gNB can only send one LP-WUS associated with one PO which is beneficial for NW energy consumption. - In addition, considering the large ramp-up time for MR in ultra-deep sleep mode, the waking up delay in RRC_IDLE/INACTIVE mode is not a critical issue. Therefore, we think one offset value is enough. |
| ZTE, Sanechips | Support. |
| CATT | Not support. If multiple offsets values need to be configured, the network needs to reserve multiple LP-WUS resources for one UE. The higher layer signaling in indicating multiple offset values associated with one UE or UE subgroup would be complicated. UE would need to prepare to monitor the LP-WUS based on the largest offset value among multiple offset values. The additional offset values are the additional UE power consumption without any benefit. When UE uses the largest offset value of multiple offset values to determine the sleeping type and the time in the preparation of the LP-WUS monitoring, it is equivalent to have one offset value. |
| Huawei, HiSilicon | We don’t support this proposal. Configuring multiple offsets makes the gNB implementation too complicated. According to the summary, most of the network vendors support Option 1-1, which justifies the complexity of supporting two offset values. Also, we share the similar view as Spreadtrum and CMCC that two offsets lead to higher resource overhead. Considering the drawbacks of configuring two offsets, even the spec allows to configure two offsets, in the future in the real deployment, network would likely to configure only a single offset, which makes the ‘two offset’ configuration a paper work. Therefore, it is better to have a simple and clear design, which is Option 1-1. |
| Sharp | ok |
| vivo | Support this proposal. First of all, we don’t think a UE will report more than one wake-up delay capabilities. And a UE capability of wake-up delay is uniquely certain. The method for UE to determine its associated LO is to choose the one which is large than UE’s reported wake-up delay and also nearest its wake-up delay. So, no UE complexity with two configured offsets. And also as FL mentioned, with option 2B-1, it is totally up to gNB’s decision to configure one or two offsets. In our view, option 2B-1 provide more flexible for the gNB to configure one or two offset values considering the reported wake-up delay capabilities from different sleep category UEs. Besides, for the resource overhead issue when two offsets are configured, we also provide a potential solution to address this i.e., UE with deep sleep type should monitor all two LOs for the sake of providing the network flexibility to decide whether to transmit multiple LP-WUSs or not. Detailed analyses can be found in our TDoc[R1-2500358]. |
| LG | Support the proposal. |
| Samsung | We support Option 1-1 because the benefit to support multiple offsets is not that obvious. Refer to Option 2B-1, the sub-bullet is needed to limit the resource overhead in our understanding if we talk about this proposal even though we do not prefer it. Besides, we do not think it is reasonable to limit UE to monitor the LO with the smallest offset subject to UE capability for Option 2B-1. |
| Nokia1 | Firstly we think system should be allowed to select the offset based on the deployment needs. Also the UE behavior should be clear and deterministic so that system (and desired service) performance metrics can be met with a configuration. Thus, firstly agreeing the UE behavior, as determined for options x-1 (1-1 or 2B-1) is most relevant for our perspective. Then we can discuss whether there is strong justification for enabling two different offsets to be supported, based on network configuration, or whether, single offset configuration can meet the need. |
| DCM | Support main bullet part. For sub-bullet part, why up to 2 offset values? Wake-up delay values are 3. For the reason of supporting option 2B-1, since wake-up delay capability report is already supported, multiple offset should be supported to take advantage of wake-up delay report by reducing paging latency of smaller wake-up delay UEs such as ultra deep sleep with eMBB and deep sleep UE. |
| Nordic | We would like to have a separate offset for RedCap UEs, gNB should already know now whether UE type is RedCap or eMBB Secondly 2B-1 should support extra delay of one eDRX cycle. |
| FW | Support with the sub-bullet as FFS as QC suggested. |
| OPPO | We can consider for Option2B-1. In that case, there should be at least 2 offset values. |
| Moderator | Proposal 2-1r1 For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure up to [2 or 3] offset values. - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, - Alt 1: the UE monitors the LO associated with the smallest offset that has a gap between the LO and the PO no less than the wake-up delay. - Alt 2: the UE monitors the LO(s) associated with all the offset value(s) that has a gap between the LO and the PO no less than the wake-up delay. |

There have been proposals on how to determine the reference PO/PF for the offset value.

[CLOSED] Proposal 2-2

For the offset value(s) between an LO and a reference PO/PF,

- The reference point for the offset value(s) is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO.

- The offset value is a frame-level offset.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. |
| Spreadtrum | Y(single offset) |
| Xiaomi | OK with the proposal. |
| CMCC | Support with single offset |
| ZTE, Sanechips | How to understand the start of the PF? In a paging cycle? Or relative to SFN0? |
| CATT | Support. |
| Huawei, HiSilicon | Generally OK. |
| vivo | OK. |
| ETRI | Support the proposal. |
| LG | Support the proposal. |
| Samsung | Support. |
| Nokia1 | To confirm that the intent is that, if multiple LOs are configured, each LO would be configured with an offset from PF? And then we will further discuss the configuration of (start of) MOs? |
| DCM | Support. Moreover, LP-WUS time duration should be also configured to UE. |
| Nordic | OK, values of 64 and 128 should be included |
| FW | Not sure how to handle the case when one LO is associated with one PO then. If the offset values are related to the wake-up delay values, then there can be collision of the LOs associated with different POs within the PF. |
| Moderator | Proposal 2-2r1 For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO |
| Moderator | A modified version was agreed during online session. Discussion closed. |

### Wake-up delay

For the UE capability report on the wake-up delay, the following was agreed:

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Here is a summary of companies’ views on the wake-up delay:

- Alt 1: [6] CATT (?), [7] CMCC, [15] xiaomi, [19] LG, [24] DCM

  - [6] CATT: {80ms, 500ms, 900ms}

  - [7] CMCC: {80ms, 600ms, 1000ms}

- Alt 2: [3] Huawei/HiSi, [16] Apple, [20] MediaTek, [24] DCM

  - [3] Huawei/HiSi: {(20ms, 2), (400ms, 2), (800ms, 2)}

  - [24] DCM: (20ms, 3), (400ms, 5), (800ms, 5)

- Alt 3: [10] Panasonic, [24] DCM

  - Modified Alt 3: [26] Nordic

- Alt 4: [4] ZTE, [7] CMCC, [8] vivo, [12] Nokia, [16] Apple, [23] QC

- [9] OPPO: {20, 400, 800} + 1 SSB

- [10] Panasonic: Alt 3 > Alt 2 > Alt 1 > Alt 4

- [11] InterDigital: original definition of wake-up delay?

The preferences are quite diverse among companies. Alt 1/2/3 essentially serve the same function through different ways, and the major difference between Alt 1/2/3 and Alt 4 is whether the resync time is decided now or decided by RAN4 requirements later. A good first step is to decide which way to go. If we decide to go with the direction of Alt 1/2/3, we can then discuss which one to select among Alt 1/2/3.

Proposal 3-1

For the UE capability report on the wake-up delay, down-select between the following:

- One of Alt 1/2/3

  - FFS Alt 1 or 2 or 3

- Alt 4

Please indicate your preference and explain the reason.

| Company | Comments |
| --- | --- |
| Qualcomm | Alt 4. The MR sync time is not within scope of LP-WUS study. RAN4 knows the MR sync performance requirement and can determine the number of SSBs for MR sync after LP-WUS is detected. Besides, for deep sleep, 20ms is the total ramp-up and ramp-down time. For MR wakeup, the ramp-up time in Alt 1/2/3/4 should be changed from 20ms to 10ms. |
| Xiaomi | Since we already have a previous agreement on wake-up delay, we think it is natural to still go with it, necessary flowing-up agreement can be made to guarantee it work properly. We support go with the first way : One of Alt 1/2/3 |
| CMCC | We support either Alt 1 or Alt 4. Moreover, if Alt 1 is supported, the exact value for candidates can be further discussed. From our understanding, since different UEs may have different capabilities and channel conditions, the offset value shall ensure the requirement for all UEs. For instance, the candidate values can be 80ms (with 3 SSB bursts), 600ms (with 10 SSB bursts) and 1000ms (with 10 SSB bursts). |
| ZTE, Sanechips | Why Alt1/Alt2/alt3 are merged into one selection? They should be separate at least for Alt1, we do not think it explicitly defined the sync time. We prefer alt4. No need to define the sync time in the spec, which would cause more complicated UE behavior, e.g., the UE may not monitor LP-WUS if the offset does not contain required number of SSBs. If it is defined in RAN4, the gNB would configure the offset by considering this sync time. |
| CATT | Support select among Alt 1/2/3. |
| Huawei, HiSilicon | We are OK with this direction, and we support One of Alt 1/2/3. Specifically, Alt 2=Alt 3>Alt 1. We don’t support Alt 4. Alt 4 simply put the responsibility and burden to RAN4 from RAN1, while RAN4 may not have a clear understanding on ramp-up time and sync time. If in RAN1 spec, e.g., in the procedure description in 38.213, the sync time is not included, it may lead to a wrong understanding of the timeline definition. |
| vivo | Alt 4. For UE wake-up delay capability which reflects the almost unchangeable characteristic of UE itself, it is more reasonable for UE to only report ramp up time without sync time which is various with the channel condition. On how long the sync/re-sync time is needed, we think RAN4 group will be more professional on this. So, the time needed for synchronization needs to be defined by RAN4 requirements. |
| LG | We also support Alt. 4. As FL commented, it is about same function with different ways, only for the taking into account MR sync up time. RAN4 has expertise for this over RAN1. |
| Samsung | Alt 4. How many SSB can be used for sync is up to UE implementation. We don’t think it is necessary to let NW know this kind of information for each UE. The time for synchronization can be decided by RAN4. |
| Nokia1 | We would prefer Alt4, i.e. leaving the synchronization time for RAN4. As indicated our paper, the number of SSBs considered seems somewhat pessimistic and does not account the expected MR and LR receiver performance difference. |
| DCM | We generally support Alt.1/2/3 to reduce discussion time. But from Qualcomm comment, if RAN4 does not fully discuss the number of SSBs, we are open to support Alt.4. Since synch time should be discussed by RAN4. |
| Nordic | Up to RAN4 |
| Moderator | There is a good support for Alt 4 based on the feedback. Proposal 3-1r1 For the UE capability report on the wake-up delay, adopt Alt 4: - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later. - Send an LS to RAN4 |
| OPPO | As discussed in online, the questions back to Alt1/2/3. My understanding is the Alt4 is not considered for the moment. |
| Xiaomi | Our preference is not to modify previous agreements. And we support Alt1 with additionally define how to update the synchronization delay when the periodicity of SSB is not 20ms. |
| Panasonic | We think it should be addressed in RAN1, as RAN4 would spend more additional effort to discuss the same thing. This is just for using SSB for synchronization (not other components of MR ramping up delay), which we have established a good framework since Rel.16 UE power saving. |

### QCL for multi-beam operation

On the QCL between LP-WUS/LP-SS and SSB, the following was agreed:

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

On the FFS for QCL Type A or Type C between LP-WUS/LP-SS and SSB:

- Type C: [3] Huawei/HiSi, [4] ZTE, [8] vivo, [11] InterDigital, [15] xiaomi, [16] Apple, [20] MediaTek (?), [25] Ericsson

  - [11] InterDigital, [15] xiaomi: Each LP-WUS is QCLed with LP-SS in QCL Type A and QCL Type D (when applicable)

- Type A: [6] CATT, [9] OPPO, [23] QC, [26] Nordic

- [7] CMCC: UE report Type A or Type C?

- [17] Samsung: Support each LP-WUS/LP-SS QCLed with SSB with Type C if LP-WUS/LP-SS is outside the initial DL BWP, support each LP-WUS/LP-SS QCLed with SSB with Type A if LP-WUS/LP-SS is inside the initial DL BWP and share a same carrier frequency with SSB.

| [3] Huawei/HiSi | Proposal 11: QCL assumptions between LP-WUS/LP-SS and SSB should be configured • The QCL type is type C with the associated SSB, and when applicable, type D with the same SSB. • Note: The subset of all SSB beams for LP-WUS/LP-SS transmission can be indicated to UE by configuring the QCL sources above to only a subset of SSB beams. |
| --- | --- |
| [8] vivo | Proposal 12: Support the network configures the LP-SS beams can map with a subset of SSB beams. Proposal 13: Explicit signaling for the QCL determination is needed to inform UE which subset of SSB beams are used for LP-SS/LP-WUS. |
| [12] Nokia | Proposal: Define that there is one-to-one association between LP-SS and LP-WUS MO time locations and SSB beams. Proposal: UE can assume that LP-SS/LP-WUS and the associated SSB are quasi co-located with respect to average gain, quasi co-location 'typeC' and 'typeD', where applicable. No signalling is need, association can be determined implicitly by time location association. |
| [21] Sharp | Proposal 5: Consider two options for providing LP-WUS QCL information: - Implicitly, using the beam index within the LP-WUS monitoring occasion - Configured LP-WUS resources with TCI state for each beam. |

QCL type

As majority companies proposed ‘typeC’ for QCL between LP-WUS/LP-SS and SSB, we will use the following proposal to start the discussion.

Proposal 4-1

The previous agreement in RAN1#119 is updated as follows:

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

For the companies supporting ‘typeA’, please explain why ‘typeA’ is necessary for the UE as opposed to ‘typeC’.

For the companies supporting ‘typeC’, please explain why ‘typeA’ cannot be automatically supported if LP-WUS/LP-SS and SSB are assumed to be transmitted from the same beam anyway.

| Company | Comments |
| --- | --- |
| Qualcomm | At least for the case that LP-WUS is in the initial DL BWP, the relationship should be typeA similar to paging PDCCH. We are open to discuss additional conditions for typeC otherwise. |
| Xiaomi | QCL type A is a very “strict” QCL type on delay spread/ average delay/ doppler shift/doppler spread. But considering different receiveing sensitivity of LR and MR, the multi-paths of LP WUS and SSB in propogation would be some how different. For example, some long-distance path can be detected by MR but not by LR, so the delay spread/ doppler spread that can be detected by LR would be different from that by MR. So QCL type C should be supported rather than QCL type A. |
| ZTE, Sanechips | Support. |
| CATT | Even the channel estimation and channel equalization operations are not expected in LP-WUS, Type A should be considered to make sure that the LP-WUS and the SSB are transmitted from the same gNB or TRP due to the same Doppler spread of LP-WUS and reference SSB is assumed. For QCL Type C, it does not guarantee the LP-WUS and SSB transmitted from the same gNB or TRP. |
| Huawei, HiSilicon | Support. In our view, typeA is usually used for channel estimation, while for LP-WUR the channel estimation is almost impossible. Thus, typeC is enough to serve for LP-WUR. |
| vivo | In our view, only the time information is helpful for LP-WUS processing in time domain with low power consumption, hence Type C seems sufficient. |
| Samsung | As Xiaomi said, our motivation to support ‘Type A’ is to restrict a same RF to receive LP-WUS and SSB for OFDM architecture. If the propagation is different, how to guarantee the sync accuracy for LP-WUS detection. Therefore, if LP-WUS is configured inside the initial BWP, type A is preferred. However, if LP-WUS is configured outside initial BWP, type A cannot be ensure. Therefore, type C can be used in this case. |
| Nokia1 | TypeC seems most generic from our perspective accounting different deployment options. We do not seen clear benefit of TypeA from assumed LR perspective either. |
| Nordic | NO, TYPE C cannot guarantee that channel gain is the same. |
| Moderator | Reading the comments, my understanding is that the supporters for type A do not mean to do complicated channel estimation/equalization based on the assumption. It is simply to guarantee that they are experiencing the same channel. Considering type C e.g. has been used for SSB for CSI-RS to accommodate the cases where different beam width may be used for the two, such a case is not intended to be considered here. Type A on the other hand is used when exactly the same channel is experienced. Therefore, the proposal is updated to type A instead: Proposal 4-1r1 The previous agreement in RAN1#119 is updated as follows: Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. |
| Moderator | ‘typeC’ was agreed during online session. |
| Nokia | From Jorma’s email: “Regarding the agreement for TypeA/C, we checked with Karol and I stand corrected. As (already) per Rel-15 specification the broadcast QCL is determined as follows: “The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], and if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET.” There is latter specification that refers only to these being quasi co-located. Hence from my perspective we are fine to revert the agreement and adopt TypeA.” |
| Nordic | From Karol’s email: “I believe that the part of ”average gain” is the most important part here.” |
| Moderator | I agree with Karol that “average gain” is missing from the agreement. Therefore I have a new proposal here. Please also comment on your view on Nokia’s comment regarding reverting the agreement. Proposal 4-1A Each LP-WUS or LP-SS is QCLed with an SSB w.r.t. average gain, in addition to QCL-TypeC, and QCL-TypeD properties, when applicable. |
| OPPO | A little confused, there are 2 proposals. Nokia would revert agreement to Type A. Nordic would add average gain. It seems average gain is more like specification language. We had already agree for TypeD, there is not problem. I think editor can capture that. Maybe we don’t need to revert agreement, either. Type C is OK. |
| Xiaomi | We are OK to add average gain. |
| Panasonic | With Nokia’s comments and clarification, we are okay to Type A. |

Mapping between LP-WUS/LP-SS and SSB beams

On how the mapping between LP-WUS/LP-SS and SSB beams is determined, there are mainly two approaches, one is implicit mapping (in order mapping), the other is explicit configuration. There was discussion in RAN1#119 on whether to support the case that LP-WUS/LP-SS are transmitted only on a subset of the SSB beams. No agreement was reached, and the guideline from the chairman was not to further discuss this issue. With this guideline in mind, the implicit mapping without explicit configuration can be considered as the starting point for discussion.

Proposal 4-2

The mapping between LP-WUS/LP-SS beams and SSB beams is determined implicitly based on the resource configuration of LP-WUS/LP-SS.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. |
| xiaomi | Considering the guideline from chairman, we think it is OK to support implicitly mapping between LP-WUS/LP-SS beams and SSB beams. |
| CMCC | Fine with the proposal |
| ZTE, Sanechips | We may need to further check how this work. |
| CATT | OK with the proposal. |
| Huawei, Hisilicon | Either implicit mapping or explicit mapping is OK for us. If explicit mapping is defined, then there will be explicit signaling on the related SSB beam for each LP-WUS/LP-SS beam. Then naturally it is possible to configure only a subset of SSB beams to LP-WUS/LP-SS beams. This is our understanding why Mr. chairman thought configuring a subset of SSB beams is naturally supported. If implicit mapping is defined, then the order of SSB beams and LP-WUS/LP-SS beams will be consistent. In this case, additional explicit signaling is needed to configure the subset of SSB beams. (like the signaling for actually transmitted SSB) From our view, no matter the explicit mapping or implicit mapping is used, supporting to only transmit LP-WUS/LP-SS via a subset of SSB beams is important. Since it helps to reduce the overhead of LP-WUS/LP-SS. Thus we suggest to discuss the proposal 4-2 together with the discussion on the subset issue. |
| LG | Support the proposal. |
| Samsung | Support |
| Nokia1 | In principle OK, though it would be good to clarify what we mean by resource configuration? |
| DCM | Support. |
| Nordic | OK |
| OPPO | That is OK, we need to move to the exact mapping in time relation. |
| Panasonic | We think the intention is to define the QCL between LP-WUS/LP-SS and SSB based on certain “rule” implicitly. We are supportive to the motivation and also think no explicit signaling is needed. In this regard, we share similar view with Nokia. In our current understanding, there should be explicit statement on the mapping, rather than “implicit”. |
| Moderator | Proposal 4-2r1 The mapping between LP-WUS/LP-SS beams and SSB beams is determined based on rules defined in the specifications. |

### Misc

Companies seem to be generally fine with the working assumption of supporting 32 subgroups per PO.

Proposal 5-1

Confirm the following working assumption:

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. |
| Xiaomi | Support to confirm the working assumption |
| ZTE, Sanechips | OK |
| CATT | Support. |
| Huawei, HiSililcon | OK |
| Sharp | OK |
| vivo | Ok. Moreover, we also think the number of subgroups can be indicated by a LP-WUS should also be determined which is related to derive the LP-WUS information bits and LP-WUS signal design. In our view, we want to support a LP-WUS/LP-WUS MO can associate with up to 32 subgroups. |
| Samsung | Support |
| DCM | Support. |
| Nordic | No, per LP-WUS |
| FW | OK |
| Moderator | During the offline discussion, Ericsson suggested to consider 31 subgroups, in order to reduce the number of information bits from 6 to 5 bits. Please provide comments on this proposal. The maximum number of subgroups per PO supported in Rel-19 is 31. |
| OPPO | We support the proposal as the payload of the LP-WUS is very limited. |
| Xiaomi | OK with the proposal. |
| Moderator | Proposal 5-1r1 Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32. |

A few companies continue to propose to make the power offset of LP-SS/LP-WUS w.r.t. the power of SSB configurable. The benefit includes (1) AGC and (2) make the RSRP/RSRQ of LP-SS based measurement and MR measurement comparable. From the feedback collected in RAN1#119, there was good support to configure the power offset. The following is proposed, with a sub-bullet stating that LP-WUS and LP-SS have the same transmit power.

Proposal 5-2

The [average] EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB.

- LP-WUS and LP-SS are assumed to have the same EPRE.

| Company | Comments |
| --- | --- |
| Qualcomm | Support. |
| Xiaomi | Support |
| ZTE, Sanechips | OK |
| CATT | OK. |
| Huawei, HiSilicon | Support. |
| ETRI | Support. |
| LG | Support. |
| Samsung | Support. |
| Nordic | Support |
| FW | Support |
| Moderator | Proposal 5-2r1 The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS |
| Moderator | The following was agreed during online session: Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported) Let us continue to discuss the first FFS bullet. The last FFS bullet can be discussed if we agree to support such a sequence for LP-SS. |
| OPPO | Maybe we can on hold until we have finalized LP-WUS and LP-SS signal. |
| xiaomi | Share similar view as OPPO. |
| Panasonic | We think it depends on whether OOK-1/OOK-4, M = 1 and OOK-4, M>1 are adopted by LP-WUS and LP-SS, respectively. |

## RRM measurement

This is a collection of companies’ proposals on the RRM measurement related issues:

| [3] Huawei/HiSi | Proposal 14: Confirm the working assumption that from RAN1 perspective for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. Proposal 15: The RRM metrics for OOK-based receiver, including LP-RSRP and LP-RSRQ can be also applied to OFDM-based receiver. |
| --- | --- |
| [4] ZTE | Proposal 1: - For OFDM WUR, - LP-SSS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. - LP-SSS-RSRQ is defined as the ratio of N×SS-RSRP / NR carrier RSSI - SMTC is not needed for OFDM WUR and OOK WUR for serving cell measurement - Serving cell measurement for OFDM-based WUR and OOK-based WUR can be applied for RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency |
| [8] vivo | Proposal 11: The network configures the measurement time resource for LP-SSS-RSSI, which can be different from NR SMTC. |
| [11] InterDigital | Proposal 11: For the entry condition, define procedures efficiently triggering LR based measurement for identifying whether LR-based thresholds are satisfied. Proposal 12: For the exit condition, the other conditions such as timer are supported to ensure proper UE behavior. Proposal 13: SS-RSRP and SS-RSRQ are reused for OFDM based LR. • LP-SSS-RSRP and LP-SSS-RSRQ are not defined in the specifications. Proposal 14: LP-SS based RRM measurement is supported for OFDM based LR. Proposal 15: Introduce definitions of LP-RSRP and LP-RSRQ for OFDM based LR to support LP-SS based RRM measurement. Proposal 16: Introduce the following definitions of LP-RSRP and LP-RSRQ for OFDM based LR. • LP-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry LP-SSs. • LP-RSRQ is defined as the ratio of N×LP-RSRP / LP-RSSI, where N is the number of resource blocks in the LP-RSSI measurement bandwidth. |
| [14] Sony | Proposal 8 – In order to avoid ping-pong behaviours, a time window can be used. The UE switches on its MR to evaluate the channel condition only if no LP-WUS/LP-SS received/detected by the UE within this time-window or after such evaluation period. |
| [15] xiaomi | Proposal 9: For OFDM-based LR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LR. |
| [24] DCM | Proposal 12:  For RRM metric based on OFDM-based LP-WUR,  No need to introduce LP-SSS-RSRP and LP-SSS-RSRQ in the spec  Confirm the working assumption with following update from legacy SS-RSRP/SS-RSRQ  The measurement time resources(s) restriction by SMTC window duration is not applicable.  The measurement time resource(s) for SS-RSRP/RSSI are confined within a duration, e.g., duration that the OFDM-based LP-WUR receiver is activated  If SS-RSRP or SS-RSRQ is measured by OFDM-based LP-WUR, following is applicable, - RRC_IDLE intra-frequency - RRC_INACTIVE intra-frequency |

The following working assumption was made for the RRM measurement metrics based on SSS for OFDM-based LP-WUR:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

There have been discussions mainly on the following two aspects:

- Whether LP-SSS-RSRP and LP-SSS-RSRQ should be separately defined in the specifications

- Whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement

It seems that majority think the existing metrics can be reused, and there is a good support to leave the SMTC window related discussion to RAN4.

Proposal 6-1

Confirm the following working assumption with the additional text in red:

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications.

- The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement.

- It is left to RAN4 to decide whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.

| Company | Comments |
| --- | --- |
| DCM | Support with red part description. |
| Moderator | A modified version was agreed during online session. The following FFS in the agreement can be further discussed in this meeting: “FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement.” |

A few companies are also proposing to reuse the LP-RSRP/LP-RSRQ definition for OFDM-based LR.

Proposal 6-2

For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR.

| Company | Comments |
| --- | --- |

## Other

Topics that are expected to be discussed in AI 9.6.1:

- Whether to have preamble for LP-WUS

- Bandwidth of LP-WUS

- Periodicity of LP-SS

- Overlaid OFDM sequence for LP-SS

Topics that are deprioritized in this meeting:

- LR and MR on the same or different carrier/band

#### Question 7-1:

Please provide any other comments/suggestions you may have, e.g., the topics that you think are important to be discussed now.

| Company | Comment |
| --- | --- |
| CMCC | We prefer to further consider the overlapping issue between LP-SS/LP-WUS transmission and existing NR transmission (e.g. UL slot/symbol, common signal transmission, configuration related to L1/L3 measurement). According to the current progress in AI 9.6.1, a single LP-WUS or LP-SS transmission may need several OFDM symbols (e.g. 8 symbols or 16 symbols). Meanwhile, LP-WUS or LP-SS transmission also need repetition and beam sweeping to ensure the coverage performance, which will extend a LO/LP-SS burst to frame or half-frame level duration. Given consideration on the existing pattern in NR system, LP-SS/LP-WUS transmission will overlap with the existing pattern, or the time interval between LP-SS/LP-WUS transmission and existing pattern may smaller than the RF retuning time. UE behaviour under such scenario needs to be clarified (enough it may related to HW design) to align gNB and UE. |
| Vodafone | We think the “LR and MR operation in different band” should not be deprioritized since we have already a Way Forward agreement coming from RAN Plenary in RP-243266: - For idle/inactive mode, consider a standardized solution for the intra-frequency case (e.g., solutions 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g., solution 4) will not be considered. - Detailed discussions and final solution selection are left to the working groups (WGs). In this sense we propose that RAN1 discusses this issue and the way forward to identify any RAN1 impacts for the intra-frequency case. [Moderator] My understanding is that these options will be discussed in RAN2/RAN4. RAN1 can wait for RAN2/RAN4 decisions and then discuss the potential RAN1 impact for the corresponding option(s). |

# Guidance for RAN1#120bis Discussions

For RAN1#120bis, some of the important issues to make progress include:

# References

- R1-2500049	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

- R1-2500064	Discussion on LP-WUS Operation in IDLE/INACTIVE state	TCL

- R1-2500074	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

- R1-2500132	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

- R1-2500177	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

- R1-2500232	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

- R1-2500295	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

- R1-2500358	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

- R1-2500438	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

- R1-2500492	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

- R1-2500531	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

- R1-2500574	LP-WUS operation in IDLE/Inactive mode	Nokia

- R1-2500603	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

- R1-2500657	LP-WUS operation in IDLE / INACTIVE modes	Sony

- R1-2500740	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

- R1-2500789	LP-WUS operation in IDLE/INACTIVE modes	Apple

- R1-2500858	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

- R1-2500916	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

- R1-2500957	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

- R1-2501024	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

- R1-2501097	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

- R1-2501101	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

- R1-2501164	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

- R1-2501210	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

- R1-2501255	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

- R1-2501293	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

# Appendix A: Agreements from previous meetings

## RAN1#116 agreements

Agreement

Multi-beam operations are supported for LP-WUS and LP-SS for idle mode

Agreement

LP-WUS occasions (LOs) are defined for LP-WUS monitoring.

- Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs.

  - Different LP-WUS MOs may correspond to different beams in multi-beam operation

  - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs

  - FFS details

- It is at least supported that a UE monitors LOs with a configured periodicity.

- Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period.

  - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time

  - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported)

- FFS eDRX, if supported

Agreement

For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not.

Agreement

It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up.

- FFS: support of UE monitoring dynamic PO

Conclusion

For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

## RAN1#116bis agreements

Agreement

For multi-beam operation of LP-WUS, UE assumes the same LP-WUS information payload is repeated in all transmitted beams corresponding to LP-WUS

- the selection of the beam(s) for the reception of the LP-WUS is up to UE implementation

Agreement

Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam.

- Option 1: K = 1

- Option 2: K can be larger than or equal to 1

  - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

Agreement

From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS:

- LP-RSRP

  - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols.

    - FFS: How to determine the received power of LP-SS in OOK ON symbols

- LP-RSRQ

  - LP-RSRQ = LP-RSRP/LP-RSSI

  - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options:

    - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols.

    - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols.

    - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols.

- FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol

Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Conclusion

LP-SINR is not considered further as a metric for RRM serving cell measurement for OOK-based receiver.

## RAN1#117 agreements

Conclusion

Regarding the “FFS: How to determine the received power of LP-SS in OOK ON symbols” for LP-RSRP, no additional work in RAN1.

Agreement

It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options:

- Option 1: UEs monitoring the same PO monitor the same LO

- Option 2: UEs corresponding to different POs monitor the same LO

- Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.

- Combinations of the above options can be considered.

Agreement

For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth.

Agreement

For LP-RSSI definition for LP-RSRQ, the following is agreed

- Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols.

Note: Above does not constrain LP-SS sequence design for OOK

Working Assumption

From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

- Above is applicable for both time-domain processing or frequency-domain processing

- Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Agreement

For idle/inactive mode, the maximum number of information bits (excluding CRC) in a LP-WUS is Z, where Z <= [8 or 16].

- FFS the exact value of Z

Agreement

For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256.

- FFS the exact value of X.

## RAN1#118 agreements

Agreement

The definitions of LP-RSRP and LP-RSSI for LP-RSRQ are updated as follows:

LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols over the frequency resources defined by the number of REs that carry LP-SS.

LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols over the frequency resources defined by the number of REs that carry LP-SS.

Agreement

At least support 1:1 association between LP-WUS MO(s)/LP-SS transmissions and SSB beams.

Agreement

For the wake-up delay, consider the following options:

Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting.

- FFS: X is 2, 3, 4

Above applies for IDLE/INACTIVE mode.

Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Agreement

For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis:

- Option 1: X = 8

- Option 2: X = 16

- Option 3: X = 32

- Option 4: X = 64

- Option 5: X = 128

- Option 6: X = 256

For decision in RAN1#118bis, companies are encouraged to provide the maximum number of information bits per LP-WUS (Z), the number of OFDM symbols occupied by LP-WUS per MO, the number of MOs for their preferred option.

Conclusion

There is no consensus in RAN1 on the support of dynamic PO.

Agreement

On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured.

- FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use

- FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported)

- (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle.

- If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives

  - Alt 1: it monitors the PO associated with the offset.

  - Alt 2: it monitors the first PO after its reported wake-up delay.

  - Other alternatives are not precluded

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

Send an LS to RAN2 and RAN4 to convey the following

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. Request RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Final LS in R1-2407559.

Conclusion

RAN1 will not initiate work on entry/exit conditions based on RRM measurement and RRM measurement offloading/relaxation conditions unless triggered by RAN2 and RAN4

## RAN1#118bis agreements

Agreement

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Agreement

For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO).

Agreement

From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if

- it does not detect a LP-WUS on the monitored LO

- or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup

Agreement

For the offset value(s) between an LO and a reference PO/PF, consider the following options:

- Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay.

- Option 1: gNB configures a single offset value.

  - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS.

  - Otherwise,

    - Option 1-1: the UE follows the legacy paging monitoring procedure.

    - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

- Option 2: gNB configures one or multiple offset values.

  - For the same PO, each offset corresponds to a LO.

    - This does not preclude the possibility that the same LO may correspond to different POs with different offset values.

  - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

    - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs.

    - FFS exactly how to choose the offset

  - Option 2B:

    - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay.

      - FFS exactly how to choose the offset

    - Otherwise,

      - Option 2B-1: the UE follows the legacy paging monitoring procedure.

      - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay.

        - FFS exactly how to choose the offset

  - FFS the UE shall monitor the LO associated with additional offset(s)

Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement

At least the following codepoints are supported for LP-WUS:

- One codepoint corresponding to each of the subgroups that can be indicated by LP-WUS

- One codepoint corresponding to all the subgroups that can be indicated by LP-WUS

- FFS: Additional codepoints

Agreement

Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state.

- FFS the values

- Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

Agreement

Each LP-WUS is QCLed with one SSB. Each LP-SS is QCLed with one SSB.

- FFS QCL Type A or Type C and/or Type D

- FFS implicit QCL determination or some signaling is required

Agreement

The number of beams for LP-SS is the same as the number of beams for the LP-WUS MOs in an LO.

Agreement

When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between

- Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs.

  - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

    - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

  - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

  - M = 1 and M > 1 is supported.

  - FFS: detailed UE monitoring behavior

  - FFS R=1 or R>= 1

- Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID.

  - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs.

    - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted.

      - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs

    - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs.

    - FFS: detailed UE monitoring behavior

  - M = 1 and M > 1 is supported.

  - FFS R=1 or R>=1

  - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

Working Assumption

The maximum number of subgroups per PO supported in Rel-19 is 32.

## RAN1#119 agreements

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

For future meetings:

Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.