3GPP TSG RAN WG1 #120                                                                     R1- 2500809

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.14

Source:	Apple Inc.

Title:	Rel-19 TEI proposal on remaining ambiguities with SRS carrier switching

Document for:	Discussion/Decision

# Introduction

UE sounding procedure for DL CSI acquisition is specified in NR since R15. The procedure is based on SRS carrier switching (SRS-CS) when UE switches to a carrier of the serving cell, without configured PUCCH/PUSCH transmission, to transmit SRS. Since R15, there has been many CR discussions in RAN1 to clarify specification ambiguities for SRS-CS. For instance, in RAN1#108-e and after couple of RAN1 meetings, a new UE capability (srs-SwitchingAffectedBandsListNR-r17) was defined based on which UE can indicate for a pair of source-target, which other bands in that BC are affected by the SRS-CS transmission on target (although the question remains on the band that is not impacted, what additional specification/requirements are needed to keep it “not impacted”). Consequently, the following agreement on prioiritzation rules for SRS carrier switching for intra-band and inter-band CA was reached in RAN1#108-e.

Agreement

For SRS carrier switching in Rel-17,

- Introduce prioritization rules for carriers that are in the same band as the source CC for intra-band CA

- Introduce prioritization rules for carriers that are in a different band as the source CC for inter-band CA

  - Introduce a new capability

Some aspects related to SRS-CS are still ambiguous for both UE and NW, as such issues have not been well discussed in RAN1 in the past. In this contribution, we share our views on required specification for the interaction between SRS-CS and uplink Tx switching.

# SRS-CS + uplink Tx switching

## Discussion

Uplink Tx switching between two uplink carriers was first specified in Rel-16, where the Tx chain would switch from one carrier to the other carrier when uplink switching is triggered. For the case of discussion in this subsection, let’s assume the following scenario, as shown in Fig. 1:

- UL Tx switching is configured between C1 and C2

  - For example, (C1, C2) represents (carrier1, carrier2), i.e. C2 supports 2Tx where 1Tx can switch to C1

- SRS-CarrierSwitching is configured between C2 and C3, where C3 is DL only.

For this case, specification is not clear for the following aspects:

- Switching gap: when all Tx chains are not available at the source CC, RF tuning time depends on whether UE is supposed to switch first from C1 to source (C2) and next switch from source to target (C3); or UE is capable to directly switch from C1 to C3. For this aspect, a new UE capability for switching gap may be needed.

- Prioritization rules for SRS-CS: following current specification, overlapping is determined based on required time for SRS transmission plus switching time to/from target CC.  Now the question is, if UE is not capable of simultaneous transmission of SRS-CS (including RF tuning times) and uplink transmission on C1 under Tx switching scenario, whether/how to update the prioritization rules in 38.214 Sec. 6.2.1.2 to consider uplink transmission on C1. Current prioritization rules do not consider the interaction between SRS-CS configuration on target and uplink transmission on a band configured with ulTxswitch with source. For example, UE may be capable to keep simultaneous transmission between C1 and C3, if total number of used Tx chains in both bands is not more than two. In our view, specifying such complicated procedure is not desired given that in addition to specification efforts, it limits UE flexibility on which antenna to be sounded and depending on overlapping scenario, it may be even infeasible at the end. For this issue, it is more convenient for both UE and NW to clarify that the prioritization rules are applied between C1 and C3 only based on srs-SwitchingAffectedBandsListNR-r17 and regardless of number of SRS antenna port configuration for transmission on target.

- ulTxswitch state: UE and NW need to have same understanding on Tx switching state, after SRS-CS is performed. In the example shown in Fig.1, the state of Tx chain can be back to that of before SRS-CS (1T+1T), or both Tx chains are returned back to the source (0T+2T).  This ambiguity needs to be discussed and resolved.

Fig. 1: SRS-CS + ulTxswitch

It should be noted that in theory, the required specification depends on the configuration of SRS antenna switching on target, i.e. 1TyR vs 2TyR. Given that when 2Tx changes are available 2TyR is a more practical configuration, the focus of the rest of this paper is on 2TyR.

## Proposal to resolve remaining ambiguities with SRS-CS

Based on the above discussion, the following is proposed:

Proposal: To resolve ambiguities with concurrent configuration of SRS-CS and ulTxswitch

- Confirm that the prioritization rules in 38.214 Sec. 6.2.1.2 are applied between target and C1, regardless of SRS-AS antenna port configuration on target CC, if UE indicates based on srs-SwitchingAffectedBandsListNR-r17 that SRS-CS on target impacts uplink transmission on C1, where C1 is the CC which may share Tx chains with source CC. No spec change is needed.

- If the UE is under the operation state in which all Tx chains are available at the source CC, the required switching time before the beginning of SRS-CS transmission on target CC is SRS-SwitchingTimeNR. Otherwise, the switching time before the beginning of SRS-CS transmission on target CC is the sum of uplinkTxSwitchingPeriod and SRS-SwitchingTimeNR.

- After SRS transmissions within an SRS resource set is done, if UE is indicated to transmit on C1, the switching gap between the end of SRS-CS transmission on target CC and start of UL transmission on C1 is the sum of uplinkTxSwitchingPeriod and SRS-SwitchingTimeNR. Otherwise, all Tx chains are returned to the source CC, and the RF tuning time to switch from target to source will be SRS-SwitchingTimeNR.

# Conclusion

In this contribution, we shared our views on remaining ambiguities with SRS-CS. The following proposal is made:

Proposal: To resolve ambiguities with concurrent configuration of SRS-CS and ulTxswitch

- Confirm that the prioritization rules in 38.214 Sec. 6.2.1.2 are applied between target and C1, regardless of SRS-AS antenna port configuration on target CC, if UE indicates based on srs-SwitchingAffectedBandsListNR-r17 that SRS-CS on target impacts uplink transmission on C1, where C1 is the CC which may share Tx chains with source CC. No spec change is needed.

- If the UE is under the operation state in which all Tx chains are available at the source CC, the required switching time before the beginning of SRS-CS transmission on target CC is SRS-SwitchingTimeNR. Otherwise, the switching time before the beginning of SRS-CS transmission on target CC is the sum of uplinkTxSwitchingPeriod and SRS-SwitchingTimeNR.

- After SRS transmissions within an SRS resource set is done, if UE is indicated to transmit on C1, the switching gap between the end of SRS-CS transmission on target CC and start of UL transmission on C1 is the sum of uplinkTxSwitchingPeriod and SRS-SwitchingTimeNR. Otherwise, all Tx chains are returned to the source CC, and the RF tuning time to switch from target to source will be SRS-SwitchingTimeNR.