3GPP TSG-RAN WG1 Meeting #120		R1-2500863

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.8.2

Title:	Discussion on channel model adaptation/extension of TR38.901 for 7-24GHz

Source:	Samsung

Document for:	Discussion and Decision

- Introduction

The Rel-19 study items on “Study on channel modelling enhancements for 7 – 24 GHz for NR” is endorsed in RAN#102 [1]. The objectives for this SI are shown below:

The objectives of this study are: - Validate using measurements the channel model of TR38.901 at least for 7-24 GHz - Note: Only stochastic channel model is considered for the validation. - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory. - Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured. Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this contribution, we discuss channel model adaptation/extension of TR 38.901 [2] related aspects for Rel-19 channel model SI.

- Near-field channel

2.1 Direct and non-direct path modelling

2.1.1 Direct path

Agreement For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE: - Option-1: The following formula is used to calculate the antenna element-wise delay of direct path: τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by: Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s - Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side. - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as: δτ-τ1,u,s, τ1,u,s=τ1-d3D-¦ru,s¦c where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side. - Option-3: Confirm the following working assumption made in RAN1#118BIS: - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.” - Note: τ1 is same as that is defined in 7.5-30 in TR38.901

- In the last meeting, RAN1 agreed upon a methodology based on Clause 7.6.2 of TR 38.901 regarding the calculation of delay terms in direct path modelling.

- On the other hand, the formular in Clause 7.6.2 reflects both the impact of large antenna size and channel characteristics in large bandwidth system. Specifically, it reflects both resolution in the time domain for evaluating wideband transmission and spatial domain for high directivity characteristics due to large antenna array. As a result, the delay term, which is the time of arrival in the time domain, is reflected for each ray of a cluster, and the wavelength of the phase term is modelled to have frequency bandwidth dependent property.

- Among them, extending the delay term to an antenna element wise perspective is an approach that well reflect the non-planar wavefront characteristics in the near field of a large antenna array. However, due to the frequency bandwidth dependent characteristics of the phase term, the bandwidth used may vary depending on the user’s preference. Therefore, it is necessary to assume that the center frequency (λ0) as the baseline for the parameter. And then, the necessity of modification for phase term can be discussed, separately.

In Clause 7.6.2 of TR 38.901, the delay term, which is the time of arrival in the time domain, is reflected for each ray of a cluster, and the wavelength of the phase term is modelled to have frequency bandwidth dependent property.

Extending the delay term to an antenna element wise perspective is an approach that well reflect the non-planar wavefront characteristics in the near field of a large antenna array. However, due to the frequency bandwidth dependent characteristics of the phase term, the bandwidth used may vary depending on the user’s preference.

RAN1 consider the Option 2 for delay term in direct path as a baseline

RAN1 discuss the necessity of the frequency bandwidth-dependent property in phase term

2.1.2 Non-direct path

Agreement For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120: - Option-1: The distance d1 is generated directly following a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution - FFS: The specific distribution, e.g., log-normal distribution for UMi - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. - Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution. - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1) - For other k2 clusters, the distance d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1) - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa - FFS: The values of k1 and k2 - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901 - FFS: The value for UMi, UMa, InH-Office. Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

RAN1 agreed to considerations and viable options for non-direct path modelling in near-field. Our understanding is that although implementation methods vary across options, they all share the commonality of creating/dropping clusters for non-direct paths modelling. Thus, before discussing the decision of one particular implementation method, one question pertains to the range of impacts of clusters. In cases where UEs reside in the near-field of the BS, most clusters generated will likely incorporate near-field parameters. On the contrary, for UEs located outside the near-field of the BS, the definition of parameters for generating clusters becomes difficult. Because those clusters can be generated inside and/or outside near-field. To address this, the impact of non-direct paths needs to account not only for the BS-UE interaction but also for the BS-cluster interactions.

The impact between BS and clusters may differ depending on whether the UE’s location is in the near-field or far-field region

RAN1 consider the impact not only between BS and UE but also between BS and cluster taking into account UE’ location

Distribution method

In the last meeting, RAN1 discussed about the methodology for generating the distance d1 between the BS and the first porting associated with a cluster in the non-direct path. Two options were proposed, both based on stochastic distribution approach. Both methodologies share the common feature of directly calculating the distance between the dominant cluster (e.g., specular reflection) k1 and the BS using the absolute delay of the cluster. Meanwhile, the derivation of the distance for non-dominant clusters, k2, differs between the two options. In Option 1, the distribution of k2 clusters is defined using a stochastic approach based on ray tracing or measurement results. In contrast, Option 2 defines the relative radius of curvature distribution, utilizing it as a scaling factor to multiply the distance between cluster and the BS (i.e., speed of light times absolute delay) to generate d1.

A key common factor in both options is the methodology for generating absolute delay, it was agreed to reuse the method outlines in Clause 7.6.9 of TR 38.901. However, this method is based on a lognormal distribution defined using ray tracing or measurement results in Indoor Factory scenario. Therefore, both options require sufficient data collection in UMa, UMi, and InH scenarios to define the distribution.

The absolute delay generating method in Clause 7.6.9 of TR 38.901 is based on a lognormal distribution defined using ray tracing or measurement results in Indoor Factory scenario

For both non-direct path generating methods, RAN1 collect sufficient data per deployment scenario to define the distribution of absolute delay

Option 1 has the advantage of intuitively generating the distribution of k2 clusters. However, a constraint exists where the distance d1 between k2 clusters and the BS should be smaller than the speed of light times absolute delay. Thus, it is necessary to determine whether to collect only data smaller than the speed of light times the absolute delay when defining the distribution or to apply a hard selection to include only d1 values smaller than the speed of light times the absolute delay.

Option 1 has the advantage of intuitively generating the distribution of k2 clusters

RAN1 discuss how to satisfy the restriction that distance d1 between k2 clusters and the BS is smaller than the speed of light times absolute delay

Option 2 allows for the incorporation of more realistic near-field non-planar wavefront characteristics by collecting data on the relative radius of curvature distribution. However, a criterion for selecting the radius of curvature need first be established. The relative radius of curvature (i.e., the scaling factor) is determined based on relationship with at least one of the two radii from the BS and the speed of light times the absolute delay. Meanwhile, discussions thus far have not defined how to generate these two radii from the BS or which value to select. Therefore, after the discussion and making this decision, data collection for the scaling factor should be conducted.

Option 2 allows for the incorporation of more realistic near-field non-planar wavefront characteristics by collecting data on the relative radius of curvature distribution

RAN1 discuss how to generate two radii from the BS

Additionally, both options require defining the number of dominant k1 clusters and non-dominant k2 clusters. While determining the distribution of these clusters may be the optimal approach, existing TR 38.901 adopts a methodology where sub-clusters are generated for dominant clusters, with the number of dominant clusters defined as two (referred from step11 in Clause 7.5 of TR 38.901). Considering consistency with existing channel models, it is necessary to assume that the number of k1 cluster is 2 and k2 is the rest of the clusters.

Existing TR 38.901 adopts a methodology where sub-clusters are generated for dominant clusters, with the number of dominant clusters defined as two (referred from step11 in Clause 7.5 of TR 38.901)

RAN1 consider that the number of k1 cluster is two and k2 is the rest of the clusters.

Further discussion is needed on the necessity of modelling from the UE perspective. The UE’s antenna array panel is significantly smaller than that of the BS, making near-field non-planar wavefront characteristics less dominant from the UE’s perspective. Consequently, modelling the distance d2 between the UE and the cluster need to be deprioritized.

The UE’s antenna array panel is significantly smaller than that of the BS, making near-field non-planar wavefront characteristics less dominant from the UE’s perspective

RAN1 deprioritize the modelling of distance d2 between the UE and the cluster

Derivation method

The important thing regarding near-field is whether the impact of non-planar wavefront exists, and if so, how to reflect these characteristics in the channel model. If non-planar wavefront modelling is required, the main considerations will include changes in channel parameters such as phase and field pattern, as well as how to accurately account for their effects. Especially, to calculate the phase term for each antenna element in a non-direct path, it is necessary to obtain the cluster location. Therefore, non-direct paths should encompass all these channel parameters, and a modelling approach based on this is necessary.

One way to specifically address the aforementioned things is by determining the location of clusters. To achieve this, RAN1 agreed to derive the distance between the BS/UE and a point associated with cluster. The method of determining the location of clusters has been widely discussed in various academic papers and documents [10]. A potential solution is considering the cluster as a point between the BS and the UE. Figure 1 illustrates the concept of single bounce based cluster approximation.

Figure 1 (a) represents the cluster as one entity between the BS and the UE, along with its corresponding vector and angular components. The ultimate goal of this model is to generate antenna element wise channel coefficients for both the BS and the UE. To achieve this, derivation of the distance between the clusters using the midpoint between the BS and the UE as the reference. Subsequently, vector components for each antenna element can be obtained. Figure 1 (b) presents a simplified version of (a) using the midpoint between the BS and the UE as the origin.

| (a) | (b) |
| --- | --- |
| Figure 1. Example for the single bounce modeling |  |

The total length of the path through a cluster is given by dtotal=τn,m∙c+rref, where τn,m represents the delay value of the mth ray in the nth cluster, c is the speed of light, and, rref is the distance between the centre of the BS and the UE. rrx,n,m and rtx,n,m represents the vectors towards the cluster from the Rx (UE) and Tx (BS), respectively. The direction of each vector is known information during cluster generation, namely azimuth and zenith angles. It can be represented as below:

rrx,n,m=sin(θn,m,ZOA)∙cos(ϕn,m,AOA)sin(θn,m,ZOA)∙sin(ϕn,m,AOA)cos(θn,m,ZOA)=rrx,n,mrrx,n,m

rtx,n,m=sin(θn,m,ZOD)∙cos(ϕn,m,AOD)sin(θn,m,ZOD)∙sin⁡(ϕn,m,AOD)cos(θn,m,ZOD)=rtx,n,mrtx,n,m

Next, the length of rtx,n,m is obtained.

As shown in Figure 1, when considering the cluster as a point, the BS/UE and the cluster from a triangle. Based on this, the length of rrx,n,m can be expressed using the law of cosines as below:

rrx,n,m2=rref2+rtx,n,m2-2rrefrtx,n,mcosx

x represents the angle between rtx,n,m and rref

x=-rrefTrrefrtx,n,m

Since rrx,n,m is equal to dtotal-rtx,n,m, it can be represented as follows:

dtotal-rtx,n,m2=rref2+rtx,n,m2+2rtx,n,mrrefTrtx,n,m

At the end, dtotal, rref and rtx,n,m are known, the length of rtx,n,m (i.e., the distance between the centre of the BS and the cluster) can be derived

rtx,n,m=dtotal-rref22∙(dtotal+rrefTrtx,n,m)

By applying the above method, rtx,n,m and rrx,n,m can be derived, allowing us to calculate the vectors between each antenna elements and cluster. For instance, the vector from the BS antenna element ‘s’ towards the cluster, rtx,n,ms, can be obtained. And, the vectors of the UE antenna elements can be obtained in the same way:

rtx,n,ms=rtx,n,m-dtx,s

rrx,n,mu=rrx,n,m-drx,u

Here, dtx,s is the vector from the centre of the BS antenna toward the antenna element ‘s’.

For derivation method, Considering the cluster between BS and UE as a single point, it is possible to compute the distance by utilizing the triangular relation among BS, UE and a cluster point.

Although the position of the cluster can be easily identified through this method, accuracy may decrease due to many approximations. This is because the current creation of clusters representing the multipath environment in TR38.901 is generated using statistical scheme without specifying birth and death of cluster. Therefore, it is difficult to determine whether a particular cluster is single or multiple bounces. If the distance between the BS and the UE is long and the size of the BS antenna is relatively small, the single bounce approximation mentioned above may be valid. However, if the distance between the BS and the UE is short and the size of the BS antenna is very large, the single bounce approximation may be inappropriate. Therefore, RAN1 need to discuss how much approximation is valid.

The current creation of clusters representing the multipath environment in TR38.901 is generated using statistical scheme without specifying birth and death of cluster

It is difficult to determine whether a particular cluster is single or multiple bounces. If the distance between the BS and the UE is long and the size of the BS antenna is relatively small, the single bounce approximation mentioned above may be valid. However, if the distance between the BS and the UE is short and the size of the BS antenna is very large, the single bounce approximation may be inappropriate.

Another method is to interpret the clusters between BS and UE from a multi bounce perspective. For one generated cluster, it can be assumed the first bounce at the BS side and the last bounce at the UE side, and calculate the location of the cluster based on these two points. Figure 2(a) shows a snapshot of the multipath between BS and UE considering multi bounce. In Figure 2(b), only necessary parameters for calculating the cluster location are represented with respect to the centres of BS and UE.

The first segment is the vector rtx,n,m from the centre of the BS to the first bounce cluster. The second segment is the vector rrx,n,m from the centre of the UE to the last bounce cluster. The third segment is an assumed direction from the first to last bounce cluster which is represented cn,m in figure 2.

dtotal=rtx,n,m+rrx,n,m+cn,m

| (a) | (b) |
| --- | --- |

Figure 2. Example for the multi bounce modeling

rrx,n,m and rtx,n,m represents the vectors towards the cluster from the Rx (UE) and Tx (BS), respectively. The direction of each vector is known information during cluster generation, namely azimuth and zenith angles. It can be represented as below:

rrx,n,m=sin(θn,m,ZOA)∙cos(ϕn,m,AOA)sin(θn,m,ZOA)∙sin(ϕn,m,AOA)cos(θn,m,ZOA)=rrx,n,mrrx,n,m

rtx,n,m=sin(θn,m,ZOD)∙cos(ϕn,m,AOD)sin(θn,m,ZOD)∙sin⁡(ϕn,m,AOD)cos(θn,m,ZOD)=rtx,n,mrtx,n,m

Based on the above segments, the vector rref from the center of the BS to that of the UE can be represented by the combination of vectors of each segment.

rref=rtx,n,m∙rtx,n,m+cn,m∙cn,m-rrx,n,m∙rrx,n,m

However, it is difficult to directly calculate the above vectors as shown in Figure 2(b). Thus, to form a triangular shape, the vector en,m can be added from the center of the UE to the first bounce cluster.

en,m=rref+rrx,n,m∙rrx,n,m

In addition, the length of the path excluding the vector rrx,n,m can be expressed as follows

dtotaltemp=τn,m∙c+rref-rrx,n,m

By means of the above vector en,m and the path length, the length of the vector rtx,n,m can be calculated

rtx,n,m=dtotaltemp2-en,m22∙(dtotaltemp-en,mTrtx,n,m)

Although the multi bounce approach aligns better with the existing channel modelling scheme than the single bounce approach, there are still issues to be resolved in the multi bounce based calculation. From a modelling perspective, the definition of the vector between the first bounce cluster and the last bounce cluster remains ambiguous. Considering the cluster generation aspect, since both clusters are part of the same cluster, it would be appropriate cn,m to equal to zero. However, this would lead to same result as the single bounce based calculation. Hence, a proper way to define cn,m needs to be established. Moreover, in order to complete the derived formula for the vector lengths, either rtx,n,m or rrx,n,m should be set as a fixed value, which requires further discussion.

Although the multi bounce approach aligns better with the existing channel modelling scheme than the single bounce approach, there are still issues to be resolved in the multi bounce based calculation such as definition between first and last bounce cluster and approximation for the length of vector to fixed value

2.2 Methodology of channel realization

Agreement For near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near- and existing properties of far-field under the structure of existing stochastic model TR 38.901. - FFS: whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization

RAN1 has agreed to design a unified model that explicitly reflect newly introduced near-field properties and existing far-field properties based on the structure of TR38.901. Accordingly, there is a need for discussion on whether the implementation of near-field and far-field should be handled in the same or different manner (procedure or equation).

The same implementation method will employ a single procedure/equation that integrates non-planar wavefront effects, application across all distances between BS and UE. The primary advantage is its consistency. With one procedure/equation handling both near-field and far-field scenarios, the model remains uniform. This consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions.

Alternatively, a different implementation method divides the channel model into near-field and far-field regions, each with its own tailored equation. For example, the model in near-field region captures the properties of non-planar wavefronts, while the model in far-field regions simplifies calculations by assuming a planar wavefront. This method enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near-field and far-field regions.

Therefore, the decision needs to be made taking into account trade-off between channel consistency and computational complexity.

For same implementation method, the consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions

For different implementation method, it enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near- and far-field regions

RAN1 discuss same/different implementation method taking into account channel consistency and computational complexity

2.3 Other channel parameter

Another discussion point of interest is the field pattern of antenna arrays. During the last meeting, RAN1 came to a consensus not to factor in the near-field properties of separate antenna elements. Nevertheless, exploring the subject of analogue beams in arrays continues to hold significance. When considering an antenna array, it is necessary to confirm phase control for tilting analogue beams. In general, the propagation characteristics of an antenna array in the far-field region is a pattern multiplication form that include a weighting term called an array factor for each antenna element’s phase control to the propagation characteristics of an antenna element [8], [9]. Unlike the far-field region’s plane wave approximation, spherical waves in the near field region require consideration of both angle and distance. This is because not all antenna elements maintain the same angle with respect to the wave front in the near-field area. Therefore, it is necessary to validate the array factor for array operation along with the propagation characteristics of an antenna element.

However, the existing channel models lack terms representing distinct phases of field patterns for each antenna element. Therefore, referring to [8], RAN1 needs to explicitly indicate the weighting vectors of each antenna element and consider modeling incorporating these factors.

RAN1 consider the updates of channel parameter with weighting vector for antenna array

| Configuration | Multiple columns (NV x NH elements) |
| --- | --- |
| Composite Array radiation pattern in dB | the steering matrix components are given by the weighting factor is given by wm,n=1NHNVexpi∙2πn-1dVλsinθetilt-m-1dHλcosθetiltsinφescan M=1,2,…NH; n=1,2,…NV;_ |

- Spatial non-stationarity

In the last RAN1 meeting, two options were agreed upon as candidates for modelling spatial non-stationarity. However, prior to full-fledged discussions on modelling, it is necessary to clarify when this phenomenon occurs. This is because clarifying the purpose of channel modelling can make it easier to determine the direction of the modelling.

Spatial non-stationarity’ means that the antenna elements or subarrays within the BS's whole antenna array experience different propagation environments. For instance, one end of an antenna element and/or subarray in whole antenna array may be in LOS status with the UE, while the opposite end of the whole antenna array is shadowed by obstacles and becomes NLOS status.

However, it seems this phenomenon occurs when a large-scale antenna array is introduced or the UE is close to the BS. Therefore, the occurrence condition of spatial non-stationarity depends on the configuration of the BS's antenna array and the distance between the BS and UE.

Based on this, we consider that it is important to clarify the conditions under when spatial non-stationarity occurs. In addition, discussions on how each antenna element within the BS whole antenna array determines its LOS state with the UE are needed. Finally, methods for generating and/or determining multipath clusters according to this phenomenon should also be discussed.

Spatial non-stationarity occurs when a large-scale antenna array in BS is introduced or the UE is very close to the BS.

RAN1 discuss the condition of occurrence for spatial non-stationarity

3.1 Modeling methodology

Discussions are ongoing regarding options for modeling methodologies for spatial non-stationarity. From our understanding, the main discussion point for spatial non-stationarity is based on the perspective of BS equipped with large antenna array. Hence, considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS. To maintain this perspective and channel modeling continuity, continuing discussion is prioritized using existing Blockage model B of blocker-based approach as the starting point.

Considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS

RAN1 consider the blockage model B of blocker-based approach for the modelling of spatial non-stationarity as starting point

Working Assumption - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected. - For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Building edge; Cartesian: w=50 m; h=20 m; Stationary. - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge: - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901. Agreement For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level.

RAN1 has discussed one method of spatial non-stationarity modelling, the physical blocker-based approach. The building dimensions and power attenuation formula considering the influence of building edges for blockage model B was agreed. Our understanding is that the characteristics of spatial non-stationarity at least includes the imbalance of power due to shadow effects per element within large antenna array at the BS. Therefore, the blockage model B needs to be extended to an antenna element-wise modelling.

The existing methodology in TR 38.901 calculates knife-edge diffraction for 4 edges of the blocker, taking reference to distance D1 and D2 between each Tx/Rx and blocker edge. To be more specific, ‘F’ in attenuation calculation is expressed as follow:

Fh1h2w1|w2=tan-1±π2πλD1h1h2w1|w2+D2h1h2w1|w2-r π, for direct path in LOStan-1±π2πλD1h1h2w1|w2-r' π, for all other paths

In here, D1h1h2w1|w2 and D2h1h2w1|w2 is defined as the projected (onto the side and top view planes) distances between the Rx and four edges and between the Tx and four edges of corresponding blocker, respectively. In addition, r is the distance between the Tx and Rx for the direct path in LOS, and r' is the distance between the blocker screen and Rx, projected onto the incoming sub-path direction, for all the NLOS paths.

In order to reflect the characteristics of spatial non-stationarity, antenna element wise modelling needs to be considered. To achieve this, distance calculations between the BS and the edge of blocker in the current modelling require individual calculation for each antenna element rather than the reference point of the BS. This allows us to simulate the power imbalance among antenna elements in a BS employing a large antenna array. Thus, for spatial non-stationarity modelling form the perspective of the BS, it is necessary to consider by calculating D2h1h2w1|w2 and r based on the location of each antenna element in the BS at least for the LOS path.

The characteristics of spatial non-stationarity at least includes the imbalance of power due to shadow effects per element within large antenna array at the BS

In the existing blockage mode B, the projected distance (D1h1h2w1|w2 and D2h1h2w1|w2) between Tx/Rx and the edge of blocker and distance (r) between Tx and Rx is calculated based on reference point of Tx and Rx

RAN1 consider the location of each element of antenna array in the BS when the projected distance and reference distance is calculated

For the blocker-based approach, from our understanding, Model A seems to focus on human and vehicular blocking, where blockers are generated statistically. However, from the perspective of analyzing the impact of blocking on each antenna element of a BS equipped with large antennas, Model A need to be considered more. If the need for Model A arises, it would be necessary to first discuss how the blocking areas for both the building edges and individual antenna elements will be established under non-self blocking, similar to what was mentioned during the discussions about Model B.

If the need for Model A arises, it would be necessary to first discuss how the blocking areas for both the building edges and individual antenna elements will be established under non-self blocking, similar to what was mentioned during the discussions about Model B

- Conclusion

- In Clause 7.6.2 of TR 38.901, the delay term, which is the time of arrival in the time domain, is reflected for each ray of a cluster, and the wavelength of the phase term is modelled to have frequency bandwidth dependent property.

Extending the delay term to an antenna element wise perspective is an approach that well reflect the non-planar wavefront characteristics in the near field of a large antenna array. However, due to the frequency bandwidth dependent characteristics of the phase term, the bandwidth used may vary depending on the user’s preference.

- RAN1 consider the Option 2 for delay term in direct path as a baseline

RAN1 discuss the necessity of the frequency bandwidth-dependent property in phase term

The impact between BS and clusters may differ depending on whether the UE’s location is in the near-field or far-field region

RAN1 consider the impact not only between BS and UE but also between BS and cluster taking into account UE’ location

The absolute delay generating method in Clause 7.6.9 of TR 38.901 is based on a lognormal distribution defined using ray tracing or measurement results in Indoor Factory scenario

For both non-direct path generating methods, RAN1 collect sufficient data per deployment scenario to define the distribution of absolute delay

Option 1 has the advantage of intuitively generating the distribution of k2 clusters

RAN1 discuss how to satisfy the restriction that distance d1 between k2 clusters and the BS is smaller than the speed of light times absolute delay

Option 2 allows for the incorporation of more realistic near-field non-planar wavefront characteristics by collecting data on the relative radius of curvature distribution

RAN1 discuss how to generate two radii from the BS

Existing TR 38.901 adopts a methodology where sub-clusters are generated for dominant clusters, with the number of dominant clusters defined as two (referred from step11 in Clause 7.5 of TR 38.901)

RAN1 consider that the number of k1 cluster is two and k2 is the rest of the clusters.

The UE’s antenna array panel is significantly smaller than that of the BS, making near-field non-planar wavefront characteristics less dominant from the UE’s perspective

RAN1 deprioritize the modelling of distance d2 between the UE and the cluster

For derivation method, Considering the cluster between BS and UE as a single point, it is possible to compute the distance by utilizing the triangular relation among BS, UE and a cluster point.

The current creation of clusters representing the multipath environment in TR38.901 is generated using statistical scheme without specifying birth and death of cluster

It is difficult to determine whether a particular cluster is single or multiple bounces. If the distance between the BS and the UE is long and the size of the BS antenna is relatively small, the single bounce approximation mentioned above may be valid. However, if the distance between the BS and the UE is short and the size of the BS antenna is very large, the single bounce approximation may be inappropriate.

Although the multi bounce approach aligns better with the existing channel modelling scheme than the single bounce approach, there are still issues to be resolved in the multi bounce based calculation such as definition between first and last bounce cluster and approximation for the length of vector to fixed value

For same implementation method, the consistency makes that the model can be universally applied without the need for transition points. However, this comes at the complexity for computation. Non-planar wavefronts are significant in near-field conditions, while far-field condition is adequately modelled using plane wavefront. So, by using a more complex equation across all field condition, the model may introduce unnecessary computational overhead in far-field conditions

For different implementation method, it enhances computational efficiency by applying the appropriate model to each region. However, it introduces challenges in managing the transition between near- and far-field regions

RAN1 discuss same/different implementation method taking into account channel consistency and computational complexity

RAN1 consider the updates of channel parameter with weighting vector for antenna array

Spatial non-stationarity occurs when a large-scale antenna array in BS is introduced or the UE is very close to the BS.

RAN1 discuss the condition of occurrence for spatial non-stationarity

Considering the presence of geographical blockers between the BS and UE, it is crucial to examine power attenuation per antenna element within the BS

RAN1 consider the blockage model B of blocker-based approach for the modelling of spatial non-stationarity as starting point

The characteristics of spatial non-stationarity at least includes the imbalance of power due to shadow effects per element within large antenna array at the BS

In the existing blockage mode B, the projected distance (D1h1h2w1|w2 and D2h1h2w1|w2) between Tx/Rx and the edge of blocker and distance (r) between Tx and Rx is calculated based on reference point of Tx and Rx

RAN1 consider the location of each element of antenna array in the BS when the projected distance and reference distance is calculated

If the need for Model A arises, it would be necessary to first discuss how the blocking areas for both the building edges and individual antenna elements will be established under non-self blocking, similar to what was mentioned during the discussions about Model B

References

RP-234018 “New SID: Study on channel modelling enhancements for 7-24GHz for NR,” RAN#102

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

R1-2402481 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2404130 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2406667 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2408660 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

R1-2409611 “Discussion on channel model adaptation extension of TR38.901 for 7-24GHz”, Samsung

TR37.840 “Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station (Release 12)”

Balanis, C. A. (2016). Antenna theory: analysis and design. John wiley & sons.

Jaeckel, Stephan, et al. "QuaDRiGa: A 3-D multi-cell channel model with time evolution for enabling virtual field trials." IEEE transactions on antennas and propagation 62.6 (2014): 3242-3256.