3GPP TSG RAN WG1 Meeting #120			    	R1-2500892

Athens, Greece, February 17th – 21st, 2025

Source 	: CAICT

Title 	: Considerations on ISAC channel modelling

Agenda Item	: 9.7.2

Document for	: Discussion / Decision

Introductions

In the meeting for the study on ISAC channel modelling in RAN1#119, the following agreements have been reached:

Agreement Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction. Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx Agreement The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model Agreement The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - the recommended five scattering points are located in front, left, back, right and roof side of the vehicle - the following RCS model is supported for each scattering point - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model. Agreement - If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled. - FFS whether/how blockage/forward scattering can be modelled in the target channel. Agreement - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, - rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft - Where, - rrx,n',m'Tis the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mTis the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'Tis the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mTis the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2 Agreement To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2 and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i Agreement The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2. Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k. Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model.

In this paper, we will present our views on ISAC channel modelling. Observations and proposals are put forward accordingly.

Discussions

Sensing Target Modelling

2.1.1 single or multiple scattering points for a target

It is reached an agreement that scattering of a sensing target can be modeled as single scattering point or multiple scattering points and the sensing target can be assumed in far field of sensing Tx/Rx. According to the scope of ISAC channel modelling described in the SID [1], there are five kinds of sensing targets:

- UAVs

- Humans indoors and outdoors

- Automotive vehicles (at least outdoors)

- Automated guided vehicles (e.g. in indoor factories)

- Objects creating hazards on roads/railways, with a minimum size dependent on frequency

There are many factors that may influence the selection of single or multiple scattering points for the target, e.g. depending on size/shape/materials of target, the distance between target and sensing Tx/Rx, sensing resolution, etc. If the shape and material of the target are not identical to the isotropic direction and the size of the target exceeds distance resolution or angle resolution, multiple scattering points can model the target more accurately.

The theoretical formula of sensing distance resolution and angle resolution is presented in [2]. According to the typical configuration for Sub 6GHz and mmWave, values of sensing distance resolution and angle resolution are listed in the table. Considering the typical size of targets and proper frequency bands for different scenario.

Table.1 Sensing resolution analysis

| Sensing resolution | Formula | Sub 6GHz | mmWave |
| --- | --- | --- | --- |
| Distance resolution (Mono-static) |  | 1.5m (B=100MHz) | 0.375m (B=400MHz) |
| Distance resolution (Bi-static) |  | >1.5m (B=100MHz) | >0.375m (B=400MHz) |
| Angle resolution |  | 0.2215 rad (D=8*0.5λ) | 0.111 rad (D=16*0.5λ) |

Proposal1: Model sensing target as single point or multiple point as follows:

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

2.1.2 Polarization matrix of the scattering point at target

The polarization matrix of the scattering point was agreed in RAN1#119, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ, in which the initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]. There are four options were proposed for αi,1,αi,2,βi,1,βi,2:

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

The impact of αi,1 can be moved to the large scale fading and small scale fading, and according to most measurement results shown in last meeting, co-polarization ratios αi,1 and αi,2 are similar value, while cross polarization ratios βi,1 and βi,2 are similar value. So, making αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is a simple and reasonable solution for CPMsp,i.

Proposal2: We suggest Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 for polarization matrix of the scattering point at target, κsp,i is randomly generated by log-normal distribution.

Environment Object Modelling

Modelling of environment object is a hot topic in last two RAN1 meetings and we have some concerns about the first three options to share. If EO channel is modeled, it may have overlap with stochastic clutters modeled by 38.901 for background, especially for EO type-2 having extremely large size. Some strategies should be proposed to reduce the impact, which can include power distribution and cluster number adjustment for target channel, EO channel and background channel. For Option 3, an additional process is brought in to get EO mapped from clusters in 38.901. If stochastic channel is generated for EO after getting EO, we think the gain for EO modelling is very limited.

Proposal3: Strategies should be studied to resolve the overlap between EO channel and background channel, such as, power distribution and cluster number adjustment.

ISAC channel modelling of slow fading

Simulation results based on ray tracing is provided to analyze large scale fading features of ISAC channel. The scenario is shown in Fig.1, where mono-static BS at 7GHz is deployed on top of the building in a campus. The upper parts are buildings and the lower parts is a street with a crosswalk. UAVs are distributed at different heights and distances. The links between BS and UAVs are LOS. Signals between UAVs are ignored.

Fig.1 Simulation scenario in campus

The trend that large scale fading varies with the height of UAV and the distance between UAV and BS is shown in Fig.2. We can observe that the distance factor has no effect on the background channel. Large scale fading of target channel increases with UAV height and distance between UAV and BS.

Fig.2 Large scale fading of background channel and target channel

2.3.1 Pathloss of target channel

The pathloss of the target channel is a combination of the pathloss from the sensing signal transmitter to the sensing target, the reflecting/scattering loss at the sensing target which usually is presented by RCS, and the pathloss from the sensing target to the sensing signal receiver. Therefore, we can formulate the pathloss of target channel as

PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS,

where PLd1 and PLd2 denote the pathloss from the sensing signal transmitter to the sensing target and the pathloss from the sensing target to the sensing signal receiver, respectively. λ is the wavelength of the sensing signal, and σRCS is the RCS of the sensing target. The pathloss PLd1 and PLd2 in different sensing scenarios like UMA-AVs, InH, Indoor-office, Indoor-factory, and highway, can use the corresponding pathloss formulations in TR 38.901, TR 37.885 and TR 36.777 as references. While when to apply current pathloss formulations in the literature, there are some constraints about the height of sensing target. Therefore, further modification and validation of the pathloss between sensing signal transmitter and sensing target, and the pathloss between sensing target and sensing signal receiver is necessary considering the sensing target height requirement.

Proposal4: It is suggested to adopt the formula PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS as a baseline to model the sensing pathloss.

Proposal5: When to apply the current pathloss formulations in the literature like TR 38.901, TR 37.885 and TR 36.777 for sensing pathloss, FFS the case when the sensing target height is not fulfilled the application constraints.

2.3.2 Large scale fading of background channel

Conventional large scale fading model consists of pathloss, shadow fading and penetration loss. These concepts still hold for target channel and background channel in bi-static. But whether pathloss still holds for background channel in mono-static should be reconsidered. Because Tx and Rx are too close to apply pathloss. Three options are agreed to study how to model the background channel for TRP and UE mono-static sensing mode in last meeting:

- Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario.

- Option 2: new channel model based on measurement results or ray-tracing model validated by experimental results or radar literature.

- Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

A large scale fading model based on ray-tracing simulation results is proposed in this Tdoc. The simulation scenario is shown in Fig.3. It is a campus similar to dense urban scenario. 200 mono-static TRPs are uniformed distributed in the campus with the height of 30m. Three sets of results in 7GHz, 26GHz and 100GHz are analyzed in Fig.4.

Fig.3 Ray-tracing simulation scenario for TRP mono-static

Fig.4 Large scale fading of background channel for TRP mono-static

Large scale fading of background channel for TRP mono-static can be model as a normal distribution N(μ,σ2). For 7GHz, large scale fading satisfies N(94.63dB,5.632). For 26GHz, large scale fading satisfies N(106.46dB,5.572). For 100GHz, large scale fading satisfies N(118.36dB,5.62). It can be observed that the mean value arises with frequency and variance keeps constant.

Proposal6: Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ.

ISAC channel modelling of fast fading

2.4.1 Target channel

Fast fading of target channel is modeled to describe the small scale characteristics of Tx-target link and target-Rx link for sensing scenario. There are 6 cases should be discussed.

- LOS Tx-target link and LOS target-Rx link with Mono-static sensing mode

- NLOS Tx-target link and NLOS target-Rx link with Mono-static sensing mode

- LOS Tx-target link and LOS target-Rx link with Bi-static sensing mode

- LOS Tx-target link and NLOS target-Rx link with Bi-static sensing mode

- NLOS Tx-target link and LOS target-Rx link with Bi-static sensing mode

- NLOS Tx-target link and NLOS target-Rx link with Bi-static sensing mode

In Mono-static sensing mode, there are only 2 cases with “LOS+LOS” and “NLOS+NLOS” for Tx-target link and target-Rx link, respectively. For LOS+LOS case, Type 1/2/3/4 propagation paths described in section 2.1 exist. For complexity reducing, we suggest the priority Type 1>2=3>4. For NLOS+NLOS case, only Type 4 exist. Due to channel reciprocity, we can consider that small scale characteristics of Tx-target link and target-Rx link are the same. Therefore, after generating Tx-target link’s small scale parameters, target-Rx link’s small scale parameters can also be derived.

In Bi-static sensing mode, there are 4 cases, that are “LOS+LOS”, “LOS+NLOS”, “NLOS+LOS”, and “NLOS+NLOS” for Tx-target link and target-Rx link, respectively. For LOS+LOS case, Type 1/2/3/4 exist. To reduce the complexity, we suggest the priority Type 1>2=3>4. For LOS+NLOS case, Type 2/4 exist. For NLOS+LOS case, Type 3/4 exist. For NLOS+NLOS case, only Type 4 exists. Small scale parameters of Tx-target link and target-Rx link should be generated separately.

The number of Tx-target link and target-Rx link clusters NTx-target, Ntarget-Rx can set to control the type of propagation paths of target channel as shown in Table.2. Other small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, can take steps 4-10 in TR 38.901 chapter 7.1 as reference. Specific formulas and numerical values need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Table.2 Propagation type of different link state

| Link State | NTx-target | Ntarget-Rx | Propagation Type | Sensing mode |
| --- | --- | --- | --- | --- |
| LOS+LOS | 1 | 1 | Type 1 | Mono-static/Bi-static |
|  | >1 | >1 | Type 1/2/3/4 | Mono-static/Bi-static |
|  | 1 | >1 | Type 1/2 | Bi-static |
|  | >1 | 1 | Type 1/3 | Bi-static |
| LOS+NLOS | 1 | ≥1 | Type 2 | Bi-static |
|  | >1 | ≥1 | Type 2/4 | Bi-static |
| NLOS+LOS | ≥1 | 1 | Type 3 | Bi-static |
|  | ≥1 | >1 | Type 3/4 | Bi-static |
| NLOS+NLOS | ≥1 | ≥1 | Type 4 | Mono-static/Bi-static |

Proposal7: We suggest to define the cluster number of Tx-target link and target-Rx link. The specific values require further study according to scenario and target features. One LOS ray and more than one NLOS cluster can be the starting point.

Proposal8: Small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Some simulation related to target channel is conducted. The scenario and configurations are consistent with Fig.1. We made some analyses and comparisons of the power ratio and loss of the LOS path and NLOS path in target channel, as shown in Table.3. From the results, we can see that as the height of the UAV increases, especially when the UAV is higher than the surrounding environmental objects(from No.3 to No.9), the power of the LOS path occupies almost the entire proportion of the received power signal because of empty surroundings. Therefore, K factor for target channel should be designed with consideration of the height of target and environments. And, the overall trend has certain fluctuations. It may be caused by different RCS. With the change of UAV height, different incident angle to UAV and the departure angle from UAV will change the RCS. No.1 and from No.10 to No.19 are UAVs at the same height with different distance to BS. The loss of LOS path increases monotonically as distance arises. However the loss of NLOS path has certain fluctuations that may be caused by environment.

Table.3 power ratio and loss of the LOS path and NLOS path in target channel

| No. | UAV height(m) | Horizontal distance between BS and UAV(m) | Pow ratio of LOS path(%) | Loss of LOS path(dB) | Power ratio of NLOS path(%) | Loss of NLOS path(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 20 | 40 | 45.79 | 123.78 | 54.21 | 123.04 |
| 2 | 25 | 40 | 32.35 | 124.59 | 67.65 | 121.39 |
| 3 | 30 | 40 | 97.2 | 124.69 | 2.8 | 140.08 |
| 4 | 35 | 40 | 99.96 | 112.33 | 0.04 | 145.94 |
| 5 | 40 | 40 | 97.58 | 117.97 | 2.42 | 134.03 |
| 6 | 45 | 40 | 97.36 | 125.69 | 2.64 | 141.37 |
| 7 | 50 | 40 | 96.93 | 129.46 | 3.07 | 144.44 |
| 8 | 55 | 40 | 98.71 | 126.25 | 1.29 | 145.11 |
| 9 | 60 | 40 | 98.93 | 125.09 | 1.07 | 144.73 |
| 10 | 20 | 45 | 20.88 | 126.92 | 79.12 | 121.14 |
| 11 | 20 | 50 | 41.27 | 129.30 | 58.73 | 127.77 |
| 12 | 20 | 55 | 8.65 | 131.21 | 91.35 | 120.97 |
| 13 | 20 | 60 | 12.59 | 132.78 | 87.41 | 124.36 |
| 14 | 20 | 65 | 13.58 | 134.11 | 86.42 | 126.07 |
| 15 | 20 | 70 | 15.01 | 135.06 | 84.99 | 127.75 |
| 16 | 20 | 75 | 7.65 | 136.25 | 92.35 | 125.44 |
| 17 | 20 | 80 | 8.46 | 137.06 | 91.54 | 126.71 |
| 18 | 20 | 85 | 8.96 | 137.89 | 91.04 | 127.82 |
| 19 | 20 | 90 | 30.25 | 138.46 | 69.75 | 134.83 |

Proposal9: K factor for target channel should be designed in two categories.

- Category 1 is for UAV above background environments, which define LOS power ratio larger than 95%.

- Category 2 is for UAV below background environments and other targets, which is similar to K factor in TR 38.901.

2.4.2 Background channel

Background components include interactions with the environment targets except sensing target on the sensing signal. For Bi-static sensing mode, we can reuse the stochastic model of TR 38.901 to generate its background components, with modification of the fading channel parameters for sensing purpose. For Mono-static sensing mode, two solutions was agreed in last meeting:

- Solution A: drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

- Solution B: drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

Considering TR 38.901 framework has been agreed for bi-static sensing mode for background channel modeling, we proposed to reuse TR 38.901 framework for mono-static sensing mode as well. For fast fading, the Solution A, i.e. randomly N reference points, and then the background channel is generated based on the fasting fading channel generated as TR 38.901 between the Tx and each reference point for a scenario can be the starting point.

Proposal10: For mono-static sensing mode, the stochastic model of TR 38.901 can be reused to generate its fasting fading background component, consider the previous Solution A as starting point, i.e. randomly dropping N reference points, and then the fast fading background channel is generated based on TR 38.901 between the Tx and reference points.

Conclusions

In this contribution, we have presented our considerations on the ISAC channel modelling of slow fading and fast fading, and put forward the following observations and proposals:

Proposal1: Model sensing target as single point or multiple point as follows:

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

Proposal2: We suggest Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 for polarization matrix of the scattering point at target, κsp,i is randomly generated by log-normal distribution.

Proposal3: Strategies should be studied to resolve the overlap between EO channel and background channel, such as, power distribution and cluster number adjustment.

Proposal4: It is suggested to adopt the formula PLsd1,d2=PLd1+PLd2+10log10λ24π-10log10σRCS as a baseline to model the sensing pathloss.

Proposal5: When to apply the current pathloss formulations in the literature like TR 38.901, TR 37.885 and TR 36.777 for sensing pathloss, FFS the case when the sensing target height is not fulfilled the application constraints.

Proposal6: Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ.

Proposal7: We suggest to define the cluster number of Tx-target link and target-Rx link. The specific values require further study according to scenario and target features. One LOS ray and more than one NLOS cluster can be the starting point.

Proposal8: Small scale parameters, such as delay spread, angel spread, number of rays, intra-cluster delay and angel spread, need further study and validation according to the characteristics of the target, such as target size, shape, material, etc.

Proposal9: K factor for target channel should be designed in two categories.

- Category 1 is for UAV above background environments, which define LOS power ratio larger than 95%.

- Category 2 is for UAV below background environments and other targets, which is similar to K factor in TR 38.901.

Proposal10: For mono-static sensing mode, the stochastic model of TR 38.901 can be reused to generate its fasting fading background component, consider the previous Solution A as starting point, i.e. randomly dropping N reference points, and then the fast fading background channel is generated based on TR 38.901 between the Tx and reference points.

References

- RP-234069, New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR, Nokia, Nokia Shanghai Bell, Dec. 2023.

- IMT-2020(5G) Promotion Group, “Study on 5G-Advanced ISAC Technologies in RAN”, April 2024.

- IMT-2020(5G) Promotion Group, “Study on Evaluation Methodology of 5G-Advanced Integrated Sensing and Communication”, June 2023.

- R1-2400573, Discussion on ISAC channel model, Xiaomi, BUPT, February 26th – March 1st, 2024.

- R1-2400899, Considerations on ISAC channel modelling, 3GPP RAN1#116, CAICT, Feb. 2024.

- R1-2403160, Considerations on ISAC channel modelling, 3GPP RAN1#116b, CAICT, April 2024.

- R1-2404724, Considerations on ISAC channel modelling, 3GPP RAN1#116b, CAICT, May 2024.

- R1- 2406897, Considerations on ISAC channel modelling, 3GPP RAN1#118, CAICT, August 2024.

- R1-2408810, Considerations on ISAC channel modelling, 3GPP RAN1#118bis, CAICT, October 2024.

- R1-2410370, Considerations on ISAC channel modelling, 3GPP RAN1#119, CAICT, Nov. 2024.