3GPP TSG RAN WG1 #120	R1-2500935

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.2

Source: 	Fujitsu

Title:	Discussion on SBFD random access operation

Document for:		Discussion and decision

# Introduction

The Rel-19 work item on evolution of NR duplex operation includes a list of objectives as in the WID[1], out of which the following objectives should be discussed under agenda item 9.3.2.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2]

In this contribution, we will share our views on how to support random access in SBFD symbols for SBFD-aware UEs.

# Discussion

## RACH configuration Option 1

Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

In the legacy, a UE determines a transmission power for a physical random access channel (PRACH) as

PPRACH,b,f,ci=minPCMAX,f,ci,PPRACH,target,f,c+PLb,f,c [dBm].

PCMAX,f,cis the UE configured maximum output power. To avoid severe UE-UE CLI in SBFD symbols, it is preferred to support lower maximum power in SBFD symbols than non-SBFD symbols. For example, the UE can be configured with a power offset （Poffset）relative to the maximum power in non-SBFD symbols (PCMAX,f,c). Then, the maximum output power for SBFD symbols PCMAX,f,c,sbfd  is determined as follows, PCMAX,f,c,sbfd=PCMAX,f,ci-Poffset.

PPRACH,target,f,c is the PRACH target reception power PREAMBLE_RECEIVED_TARGET_POWER determined based on preambleReceivedTargetPower and powerRampingStep. In the agreement above,  it has agreed to support separate preambleReceivedTargetPower. To support finer separate power control for SBFD symbols and non-SBFD symbols, powerRampingStep can also be separately provided for PRACH transmission in SBFD symbols and non-SBFD symbols.

Proposal 1: For RACH configuration Option 1, separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols additionally include:

- maximum output power PCMAX,f,ci

  - Introduce an RRC parameter to configure a power offset （Poffset）relative to the maximum output power in non-SBFD symbols (PCMAX,f,c). The maximum output power for SBFD symbols PCMAX,f,c,sbfd=PCMAX,f,c-Poffset.

- powerRampingStep.

## RACH configuration Option 2

Agreement For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives: - Alt-1: do not optimize this case - Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

To avoid adverse impact on non-SBFD-aware UEs (including legacy UEs), there should be no PRACH transmission using an additional RO overlapping with a legacy RO. Therefore, Alt-2 is preferred.

Proposal 2: For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt.2.

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

## PUSCH scheduled by RAR UL grant or DCI format 0_0 addressed to TC-RNTI

### Addressable PRBs

In the legacy, random access may be performed on the initial UL BWP or another UL BWP (non-initial UL BWP).

In the case of non-initial UL BWP, for the PUSCH scheduled by RAR and the Msg3 PUSCH retransmission scheduled by DCI format 0_0 with CRC scrambled with TC-RNTI, the FDRA is subject to the size of the initial UL BWP.

In details, for the PUSCH scheduled by RAR,

- If the non-initial UL BWP and the initial UL BWP have same SCS and same CP length and the non-initial UL BWP includes all RBs of the initial UL BWP, the initial UL BWP is used.

- else the RB numbering starts from the first RB of the non-initial UL BWP and the maximum number of RBs for FDRA equals the number of RBs in the initial UL BWP.

For the Msg3 PUSCH retransmission scheduled by DCI format 0_0, the RB numbering starts from the first RB of the non-initial UL BWP and the maximum number of RBs for FDRA equals the number of RBs in the initial UL BWP.

In SBFD operation, that may result in limited addressable usable PRBs for the PUSCH in SBFD symbols on the non-initial UL BWP. In the worst case, there is no addressable usable PRB. As shown in Figure 1 and Figure 2, in some cases, less/no PRBs in the UL usable PRBs in the non-initial UL BWP are addressable for FDRA for the PUSCH.

Figure 1. Examples of addressable PRBs for the FDRA in SBFD symbols on non-initial UL BWP, when the initial UL BWP is used

Figure 2. Examples of addressable PRBs for the FDRA in SBFD symbols on non-initial UL BWP, when the RB numbering starts from the first RB of the non-initial UL BWP and the maximum number of RBs for FDRA equals the number of RBs in the initial UL BWP

To lift the limit, we have the following proposal.

Proposal 3: For SBFD aware UEs, for the PUSCH scheduled by RAR and the Msg3 PUSCH retransmission scheduled by DCI format 0_0 with CRC scrambled with TC-RNTI on a non-initial UL BWP, to extend the range of addressable usable PRBs in SBFD symbols, the RB numbering for RIV provided in the FDRA field in the RAR/DCI starts from the first RB of UL usable PRBs for the active UL BWP.

- UE does not expect that the PRBs for the PUSCH transmission in SBFD symbols to be overlapped with PRBs outside UL usable PRBs for the active UL BWP.

In the legacy, for PUSCH scheduled by RAR, the UE would truncate/insert bits in the FDRA field in the RAR according to the size of initial UL BWP, as below.

For an initial UL BWP size of NBWPsize RBs, a UE processes the frequency domain resource assignment field as follows - if NBWPsize≤180, or for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided if NBWPsize≤90 - truncate the frequency domain resource assignment field to its log2NBWPsize∙NBWPsize+12 least significant bits and interpret the truncated frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212] - else - insert - log2NBWPsize∙NBWPsize+12-12 most significant bits, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided; - log2NBWPsize∙NBWPsize+12-14 most significant bits, otherwise; with value set to '0' after the NUL,hop bits to the frequency domain resource assignment field, where NUL,hop=0 if the frequency hopping flag is set to '0' and NUL,hop is provided in Table 8.3-1 if the hopping flag bit is set to '1', and interpret the expanded frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212] - end if

With the processing, the number of addressable PRBs may still be limited by the size of initial UL BWP when the number of UL usable PRBs for the non-initial UL BWP is larger than the size of initial UL BWP.

To address the issue, how to truncate/insert bits in the FDRA field in the RAR needs to be enhanced. For example, the UE could truncate/insert bits in the FDRA field in the RAR according to the size of UL usable PRBs instead of initial UL BWP in some cases.

Proposal 4: For SBFD aware UEs, for the PUSCH scheduled by RAR, to extend the range of addressable usable PRBs in SBFD symbols, truncate/insert bits in the FDRA field in the RAR according to the size of UL usable PRBs for the active UL BWP.

### Frequency hopping

Agreement For SBFD aware UEs, for Msg3 PUSCH transmission, reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop, with the update that the frequency offset for 2nd hop is based on the size of UL usable PRBs. - FFS the case when the value of NUL,hop= 11 - FFS the definition of UL usable PRBs for Msg3 PUSCH transmission Agreement For SBFD aware UEs, for Msg3 PUSCH transmission, - the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification. - the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification. Agreement Update the following agreement made in RAN1#118bis meeting in cyan: Agreement For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

In general, the same definition of UL usable PRBs should be used for determining the starting PRB index of UL usable PRBs RBUL SB start, the frequency offset for 2nd hop RBoffset and the number of UL usable PRBs NUL SBsize.

According to the agreement made in AI 9.3.1, RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of active UL BWP, and RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. It suggests that the UL usable PRBs mentioned refer to the UL usable PRBs for the active UL BWP.

Table 8.3-1: Frequency offset for second hop of PUSCH transmission with frequency hopping scheduled by RAR UL grant or of Msg3 PUSCH retransmission

| Number of PRBs in initial UL BWP | Value of NUL,hop Hopping Bits | Frequency offset for 2nd hop |
| --- | --- | --- |
| NBWPsize<50 | 0 | NBWPsize2 |
|  | 1 | NBWPsize4 |
| NBWPsize≥50 | 00 | NBWPsize2 |
|  | 01 | NBWPsize4 |
|  | 10 | -NBWPsize4 |
|  | 11 | Reserved |

In addition, Table 8.3-1 is used not only for Msg3 PUSCH, but also PUSCH scheduled by RAR in CFRA. Following the principle, for SBFD aware UEs, the updated interpretation should also apply to PUSCH scheduled by RAR in CFRA.

Proposal 5: For SBFD aware UEs, for Msg3 PUSCH transmission and PUSCH scheduled by RAR in CFRA (i.e. PUSCH scheduled by RAR UL grant and Msg3 PUSCH retransmission), reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop in SBFD symbols based on the size of UL usable PRBs for the active UL BWP.

## Msg3 power control

In the RAN1#119 meeting, the following agreement was achieved for separate power control for Msg3 PUSCH on non-SBFD symbols and SBFD symbols.

Agreement [119] Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters. - preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR

For the listed open-loop power control parameters including preambleReceivedTargetPower, msg3-DeltaPreambl and deltaPreamble, they can be separately configured for Msg3 PUSCH on non-SBFD symbols and SBFD symbols. In addition to the listed parameters, it can be also supported that msg3-Alpha is separately configured for non-SBFD symbols and SBFD symbols to achieve more flexibility. For PUSCH transmission, it has been agreed in AI 9.3.1 that a TCI state can be associated with two sets of power control parameters for non-SBFD symbols and SBFD symbols respectively. Since both P0 and alpha are included in a set of power control parameters, alpha can be separately configured for PUSCH as agreed in AI 9.3.1. Similarly, it should be supported that alpha is separately configured for Msg3 PUSCH. If one of the aforementioned parameters is not configured for SBFD symbols, the parameter configured for non-SBFD symbols will apply to both non-SBFD and SBFD symbols, i.e., the same parameter is used by default. As for the RRC IE structure, a parameter for SBFD symbols can be configured under the same parent RRC IE of the corresponding parameter for non-SBFD symbols.

Proposal 6: For separate power control for Msg3 PUSCH on non-SBFD symbols and SBFD symbols, the following separate power control parameters are supported.

- The first set of parameters including preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, and msg3-Alpha is configured for Msg3 PUSCH on non-SBFD symbols. The second set of parameters including preambleReceivedTargetPower-SBFD, msg3-DeltaPreamble-SBFD/deltaPreamble-SBFD, and msg3-Alpha-SBFD is configured for Msg3 PUSCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for Msg3 PUSCH on SBFD symbols.

- A parameter in the second set is configured under the same parent RRC IE of the corresponding parameter in the first set.

## PUCCH power control

For PUCCH (HARQ-ACK) for Msg4, separate power control should be also supported for non-SBFD symbols and SBFD symbols. Since alpha is assumed to be always 1 for PUCCH power control, it is only considered to support separate P0 values. For PUCCH, P0 is determined by a cell-specific parameter p0-nominal and a UE-specific parameter p0-PUCCH-Value. To have more flexibility, both separate p0-nominal values and separate p0-PUCCH-Value values should be supported. If one of the aforementioned parameters is not configured for SBFD symbols, the parameter configured for non-SBFD symbols will apply to both non-SBFD and SBFD symbols, i.e., the same parameter is used by default. To support separate p0-nominal values, two p0-nominal values can be configured under the same parent RRC IE. Under the unified TCI framework, p0-PUCCH-Value is determined by the minimum p0-PUCCH-Id in p0-Set. Therefore, the RRC structure to configure separate p0-PUCCH-Value values may not be as straightforward as that of p0-nominal. As one alternative, two p0-PUCCH-Value values can both be associated with the minimum p0-PUCCH-Id. As another alternative, two p0-PUCCH-Value values can be associated with the minimum p0-PUCCH-Id and the second minimum p0-PUCCH-Id respectively.

Proposal 7: Separate power control is supported for PUCCH (conveying HARQ-ACK for Msg4) on non-SBFD symbols and SBFD symbols. Separate power control parameters are supported as follows.

- The first set of parameters including p0-nominal and p0-PUCCH-Value is configured for PUCCH on non-SBFD symbols. The second set of parameters including p0-nominal-SBFD and p0-PUCCH-Value-SBFD is configured for PUCCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for PUCCH on SBFD symbols.

- p0-nominal and p0-nominal-SBFD are configured under the same parent RRC IE.

- p0-PUCCH-Value and p0-PUCCH-Value-SBFD are both associated with the minimum p0-PUCCH-Id, or are associated with the minimum p0-PUCCH-Id and the second minimum p0-PUCCH-Id respectively.

## 2-step RACH

Agreement: For SBFD aware UEs in RRC CONNECTED state, support Type-1 random access procedure (4-step RACH) in SBFD symbols. - FFS Type-2 random access procedure (2-step RACH)

In Rel-16, 2-step RACH was introduced to reduce random access latency. SBFD-aware UEs can also benefit from 2-step RACH, and at least the legacy 2-step RACH can be supported.

Regarding whether to support 2-step RACH in SBFD symbols, more discussions are needed.

For 2-step RACH, operation in SBFD symbols is beneficial for reduction of latency and collision probability. However, because 2-step RACH is applied when RSRP is high enough, it may not have benefits in coverage improvement and cell range extension. Therefore, considering the benefits may be marginal, support of 2-step RACH in SBFD symbols can be deprioritized.

Proposal 8: For SBFD-aware UEs, at least support legacy 2-step RACH.

- FFS whether/how to support 2-step RACH in SBFD symbols.

# Conclusion

According to the discussions above, we have the following proposals. And proposals related to higher layer signaling are also summarized in Appendix.

RACH configuration Option 1

Proposal 1: For RACH configuration Option 1, separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols additionally include:

- maximum output power PCMAX,f,ci

  - Introduce an RRC parameter to configure a power offset （Poffset）relative to the maximum output power in non-SBFD symbols (PCMAX,f,c). The maximum output power for SBFD symbols PCMAX,f,c,sbfd=PCMAX,f,c-Poffset.

- powerRampingStep.

RACH configuration Option 2

Proposal 2: For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, support Alt.2.

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs

PUSCH scheduled by RAR UL grant or DCI format 0_0 addressed to TC-RNTI

(Addressable PRBs)

Proposal 3: For SBFD aware UEs, for the PUSCH scheduled by RAR and the Msg3 PUSCH retransmission scheduled by DCI format 0_0 with CRC scrambled with TC-RNTI on a non-initial UL BWP, to extend the range of addressable usable PRBs in SBFD symbols, the RB numbering for RIV provided in the FDRA field in the RAR/DCI starts from the first RB of UL usable PRBs for the active UL BWP.

- UE does not expect that the PRBs for the PUSCH transmission in SBFD symbols to be overlapped with PRBs outside UL usable PRBs for the active UL BWP.

Proposal 4: For SBFD aware UEs, for the PUSCH scheduled by RAR, to extend the range of addressable usable PRBs in SBFD symbols, truncate/insert bits in the FDRA field in the RAR according to the size of UL usable PRBs for the active UL BWP.

(Frequency hopping)

Proposal 5: For SBFD aware UEs, for Msg3 PUSCH transmission and PUSCH scheduled by RAR in CFRA (i.e. PUSCH scheduled by RAR UL grant and Msg3 PUSCH retransmission), reuse Table 8.3-1 in TS 38.213 for determination of the frequency offset for the 2nd hop in SBFD symbols based on the size of UL usable PRBs for the active UL BWP.

Msg3 power control

Proposal 6: For separate power control for Msg3 PUSCH on non-SBFD symbols and SBFD symbols, the following separate power control parameters are supported.

- The first set of parameters including preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, and msg3-Alpha is configured for Msg3 PUSCH on non-SBFD symbols. The second set of parameters including preambleReceivedTargetPower-SBFD, msg3-DeltaPreamble-SBFD/deltaPreamble-SBFD, and msg3-Alpha-SBFD is configured for Msg3 PUSCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for Msg3 PUSCH on SBFD symbols.

- A parameter in the second set is configured under the same parent RRC IE of the corresponding parameter in the first set.

PUCCH power control

Proposal 7: Separate power control is supported for PUCCH (conveying HARQ-ACK for Msg4) on non-SBFD symbols and SBFD symbols. Separate power control parameters are supported as follows.

- The first set of parameters including p0-nominal and p0-PUCCH-Value is configured for PUCCH on non-SBFD symbols. The second set of parameters including p0-nominal-SBFD and p0-PUCCH-Value-SBFD is configured for PUCCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for PUCCH on SBFD symbols.

- p0-nominal and p0-nominal-SBFD are configured under the same parent RRC IE.

- p0-PUCCH-Value and p0-PUCCH-Value-SBFD are both associated with the minimum p0-PUCCH-Id, or are associated with the minimum p0-PUCCH-Id and the second minimum p0-PUCCH-Id respectively.

2-step RACH

Proposal 8: For SBFD-aware UEs, at least support legacy 2-step RACH.

- FFS whether/how to support 2-step RACH in SBFD symbols.

# Appendix (proposals related to higher layer signaling)

Proposal 1: For RACH configuration Option 1, separate power control parameters for PRACH transmission in SBFD symbols and non-SBFD symbols additionally include:

- maximum output power PCMAX,f,ci

  - Introduce an RRC parameter to configure a power offset （Poffset）relative to the maximum output power in non-SBFD symbols (PCMAX,f,c). The maximum output power for SBFD symbols PCMAX,f,c,sbfd=PCMAX,f,c-Poffset.

- powerRampingStep.

Proposal 6: For separate power control for Msg3 PUSCH on non-SBFD symbols and SBFD symbols, the following separate power control parameters are supported.

- The first set of parameters including preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, and msg3-Alpha is configured for Msg3 PUSCH on non-SBFD symbols. The second set of parameters including preambleReceivedTargetPower-SBFD, msg3-DeltaPreamble-SBFD/deltaPreamble-SBFD, and msg3-Alpha-SBFD is configured for Msg3 PUSCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for Msg3 PUSCH on SBFD symbols.

- A parameter in the second set is configured under the same parent RRC IE of the corresponding parameter in the first set.

Proposal 7: Separate power control is supported for PUCCH (conveying HARQ-ACK for Msg4) on non-SBFD symbols and SBFD symbols. Separate power control parameters are supported as follows.

- The first set of parameters including p0-nominal and p0-PUCCH-Value is configured for PUCCH on non-SBFD symbols. The second set of parameters including p0-nominal-SBFD and p0-PUCCH-Value-SBFD is configured for PUCCH on SBFD symbols.

- If a parameter in the second set is not configured, the corresponding parameter in the first set is used for PUCCH on SBFD symbols.

- p0-nominal and p0-nominal-SBFD are configured under the same parent RRC IE.

- p0-PUCCH-Value and p0-PUCCH-Value-SBFD are both associated with the minimum p0-PUCCH-Id, or are associated with the minimum p0-PUCCH-Id and the second minimum p0-PUCCH-Id respectively.

# References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.