3GPP TSG RAN WG1#120                                 R1-2500962

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.1

Source:	Transsion Holdings

Title:	Discussion on specification support for AI/ML beam management

Document for:	Discussion and decision

- Introduction

In RAN#102[1], a new WID for Rel-19 AI/ML for the air interface was approved[1], with the objectives to enhance AI/ML beam management. In this contribution, we discuss enhancements on AI/ML beam management.

- 2 Potential specification impacts on LCM

- 2.1 Data collection

- 2.1.1 NW-side model

- In RAN1#119 meeting, the following proposal on the content for data collection for training was discussed[2]:

Proposal 6.2A:

For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case 1, support the following options:

- Opt 1: L1-RSRPs measured based on one resource set (for Set A and Set B) [FFS: two sets] configured to UE

- Opt 2: L1-RSRPs measured based on one resource set (for Set B) configured to UE, and beam information of Top K measured based on another resource set (for Set A) configured to UE

- Differential L1-RSRP reporting is supported with legacy quantization steps and ranges

  - FFS on whether other values for quantization steps and/or ranges can be configured by NW.

- FFS on whether/how to omit some of L1-RSRPs in the report,

  - Considering the following methods for omittion, including

    - Alt 1: Top M beam(s) is the beams with largest M measured values of L1-RSRPs, where M is configured by gNB

    - Alt 2: All beams within X dB gap to the largest measured value of L1-RSRP

Regarding content for data collection for training for NW-sided model, two options were discussed in the last meeting. Regarding Opt 1, L1-RSRPs of RS resources from one set can be reported, where  RS resources of Set A and Set B can be configured in one resource set. If Set A and Set B are different (i.e., Set B is not subset of Set A), RS resources of Set A and Set B need to be distinguished during RS resource configuration. In addition, L1-RSRPs of Set A and Set B need to be distinguished during L1-RSRP reporting. Regarding Opt 2, L1-RSRPs of RS resources from two sets can be reported, where  RS resources of Set A and Set B can be configured in a separate resource set. By configuring a separate resource set for Set A and Set B, a unified resource set framework for Set A and Set B can be configured as inference and monitoring. Therefore, regarding content for data collection for training for NW-sided model, support Opt 2.

Proposal 1: Regarding content for data collection for training for NW-sided model, support Opt2:

- Opt 2: L1-RSRPs measured based on one resource set (for Set B) configured to UE, and beam information of Top K measured based on another resource set (for Set A) configured to UE.

Regarding data collection for NW-side AI/ML model of BM-Case1 and BM-Case2, the following reporting signalling for beam-specific aspects have been achieved [3]:

-	L1 signalling to report the collected data

-	Higher-layer signalling to report the collected data

-	At least not applicable to AI/ML model inference

Regarding training data collection, if training data is reported via L1 signalling, high UCI payload size is required due to the large amount of training data. Hence, reporting training data via L1 signalling is not a proper solution. In the other hand, compared with inference data, the latency requirement of training data is relatively low, reporting training data via higher-layer signalling may be a proper solution. Therefore, regarding the signaling for training data collection, support higher-layer signalling.

Proposal 2: Regarding the signaling for training data collection, support higher-layer signalling to report the contents of training data.

- 2.1.2 UE-side model

- The following alternatives on trigger/initiating data collection have been achieved [3]:

-	Trigger/initiating data collection considering:

-	Option 1: data collection initiated/triggered by configuration from NW.

-	Option 2: request from UE for data collection.

Regarding Option 1, it provides centralized resource management and efficient resource allocation over different UEs. However, resource configurations may not be specific to individual UE. Regarding Option 2, it provides resource configurations flexibly and the resource configuration can be specific to individual UE. By this way, the resource configuration can satisfy preferred resource configuration or the request from UE. Therefore, regarding the trigger/initiating data collection for UE-side model, we support Option 2.

Proposal 3: Regarding the trigger/initiating data collection for UE-side model, support Option 2:

- Option 2: request from UE for data collection.

- 2.2 Model inference

- 2.2.1 NW-side model

- In RAN1#117 meeting, the following agreements on model inference for NW-side model have been achieved [4]:

Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

- In legacy, the reported L1-RSRP values include largest L1-RSRP and differential L1-RSRP, where the largest L1-RSRP is quantized to a 7-bit value with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value with 2 dB step size with a reference to the largest L1-RSRP. Regarding the quantization of a reported L1-RSRP value, legacy quantization step of differential L1-RSRP has been agreed in the last meeting. According to the evaluation in R18 SI, a larger quantization step of differential L1-RSRP has limited beam prediction accuracy degradation but the overhead of measurement results can be reduced significantly. Therefore, to further reduce the report overhead, support larger quantization step(s) for differential L1-RSRP and/or for absolute L1-RSRP.

Proposal 4: Regarding the quantization of a reported L1-RSRP value, support larger quantization step(s) than the legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP.

- 2.2.2 UE-side model

- In RAN1#119 meeting, the following agreements on model inference for UE-side model have been achieved [5]:

Agreement For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. Conclusion For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report. Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - two CSI-ResourceConfigId s are configured for Set A and Set B separately.

- Regarding UE-sided model for BM-Case 2, differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting were agreed in the RAN1#118 meeting, where differential RSRP can be reported among multiple beams over multiple time instances. However, the largest RSRP should be selected to determine differential RSRP among multiple beams over multiple time instances. If a set of absolute RSRP and differential RSRP are reported to NW, NW should identify the time instance information of largest RSRP so that NW can determine the absolute RSRP among multiple beams over multiple time instances. Therefore, for absolute RSRP and differential RSRP report over multiple time instances, time instance information of largest RSRP can be reported explicitly.

- Proposal 5: Regarding UE-sided model for BM Case-2, for absolute RSRP and differential RSRP report over multiple time instances, time instance information of largest RSRP can be reported explicitly.

- Regarding the reference time of the earliest time instance for the predicted results for BM-Case 2 of UE-side model, three options were achieved in the RAN1#118b meeting. The reference time for the time instance(s) should be determined first so that N future time instance(s) can be determined based on reference time and duration value of two adjacent time instances. For the Option 1, reference time is based on the time domain resource for the report, which reuse the similar rule in Rel-18 MIMO CSI prediction. For the Option 2, reference time is based on the CSI reference resource corresponding to the report. Since the CSI reference resource is determined based on the uplink slot for the report, it is not necessary to use a relative time instance of uplink slot for the report as the reference time of the earliest time instance for the predicted results. For the Option 3, reference time is based the last transmission occasion of the CSI-RS/SSB resource in Set B for the report and an duration value. Since the predicted time instance is highly related to the measurements of CSI-RS/SSB resource in Set B, Option 3 is a reasonable solution. In addition, Option 3 can provide a fixed time offset between the latest transmission occasion of the CSI-RS/SSB resource in Set B and the reference time of the earliest time instance, where time offset can be determined based on UE capability. Therefore, regarding the reference time of the earliest time instance for the predicted results for BM-Case 2 of UE-side model, we support Option 1(2nd) and Option 3(1st).

- Proposal 6: Regarding UE-sided model for BM Case-2, for the reference time to determine the earliest time instance from the future time instances, support Option 1(2nd) and Option 3(1st):

- Option 1: Based on the time domain resource for the report.

- Option 3: Based on the last transmission occasion of the CSI-RS/SSB resource in Set B for the report.

- Regarding CSI-RS resource types for CMR for Set B for BM-Case 2 of UE-side AI/ML model, Periodic (P) CSI-RS and Semi-persistent (SP) CSI-RS were agreed in RAN1#118b meeting. However, support of Aperiodic (AP) CSI-RS for Set B for BM-Case 2 is still FFS. Regarding BM-Case2, model inference is based on the measurement results of Set B for multiple past time instances and hence the observation window of Set B needs to be considered. Compared with periodic and semi-persistent CSI report, aperiodic CSI report has the advantage of reducing latency of CSI report.

- Regarding the UE-side CSI prediction in Rel-18 MIMO, UE can measure different CSI-RS resources in different time instances. By extending the similar mechanism in Rel-18 MIMO, CSI request field can trigger aperiodic resource set in multiple time instances. By this way, UE can be indicated with multiple time instances of Set B and hence measurement results of multiple time instances can be served as model input to obtain the prediction results. Therefore, regarding CSI-RS resource types for CMR for Set B for BM-Case 2 of UE-side AI/ML model, support to trigger Aperiodic (AP) CSI-RS in multiple time instances.

Proposal 7: Regarding CSI-RS resource types for CMR for Set B for BM-Case 2 of UE-side AI/ML model, support to trigger Aperiodic (AP) CSI-RS in multiple time instances.

- 2.3 Performance monitoring

- 2.3.1 NW-side model

Regarding performance monitoring of NW-side model, reporting the predicted information(e.g., beam ID and/or L1-RSRP) is not needed and except for data collection for performance monitoring, there is no need to specify the remaining monitoring procedure for NW-side model. According to the performance metric(s), measurement results of Set B and/or Set A should be reported by UE. Therefore, configuration of Set B and/or Set A and the reporting of measurement results of Set B and/or Set A may need to be specified.

- Proposal 8: For performance monitoring of NW-side model, configuration of Set B and/or Set A and the reporting of measurement results of Set B and/or Set A need to be specified.

- 2.3.2 UE-side model

- In RAN1#119 meeting, the following agreements on performance monitoring for UE-side model have been achieved [5]:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

- Regarding performance metric for Option 2 of UE-sided AI/ML model, Alt 1(i.e., Top 1 or Top K beam prediction accuracy) was agreed in RAN1#118b meeting. However, whether/how to configure or define a window for calculation is still FFS. To achieve Top 1 or Top K beam prediction accuracy, the reported metric can be calculated per sample (one-shot), or per set of samples (window). If the reported metric is calculated per sample, the measurement of CSI-RS/SSB resource in single time instance may not be sufficient in reflecting the link performance for the model. On the other hand, if the reported metric is calculated per set of samples (window), it can provide accurate monitoring results when the statistics of multiple samples are considered.

- To evaluate the beam prediction accuracy per set of samples, a monitoring window for calculating the metric should be configured to collect measurement results and prediction results. Regarding the window for calculating the metric, by reusing the principle of UE-side CSI prediction in Rel-18 MIMO, following two alternatives can be considered. For Alt-1, the window for calculating the metric is defined based on the uplink slot for the report. For Alt-2, the window for calculating the metric is defined based on the CSI reference resource corresponding to the report.

- Proposal 9: Regarding Option 2 of UE-sided AI/ML model, support to configure a window for calculating the metric and the following alternatives can be considered to define a window:

- Alt-1: The window for calculating the metric is defined based on the uplink slot for the report.

- Alt-2: The window for calculating the metric is defined based on the CSI reference resource corresponding to the report.

- Regarding performance metric for Option 2 of UE-sided AI/ML model, Alt 1(i.e., Top 1 or Top K beam prediction accuracy) was agreed in RAN1#118b meeting. However, the details on how to configure the resource set/resources for monitoring is still FFS. Regarding the resource set/resources for monitoring, full set of Set A or subset of Set A can be used as the resources for monitoring. If subset of Set A is used as the resources for monitoring, the actual measurement of Top 1 or K beam(s) may not be within the subset of Set A, which may fail to achieve the precise Top 1 or Top K beam prediction accuracy. On the other hand, full Set A can be configured to be measured to achieve precise monitoring results. Therefore, regarding the resource set/resources for monitoring for Option 2 of UE-sided AI/ML model, support to use full set of Set A for measurement.

- Proposal 10: Regarding the resource set/resources for monitoring for Option 2 of UE-sided AI/ML model, support to use full set of Set A for measurement.

- Regarding Option 2 (UE-assisted performance monitoring) of Type 1 performance monitoring, UE calculates and reports performance metric(s). However, how to report and what to report should be determined. Regarding the report content, following three alternatives can be considered. Firstly, UE calculates performance metric(e.g., Beam prediction accuracy) and the quantization of performance metric can be directly reported. It is a straightforward way but would introduce quantization error. Secondly, the statistic values of the performance metric in a period of time can be reported. By this way, it would not introduce quantization error. Thirdly, if the condition of a specific event is satisfied, the specific event can be reported. For event based reporting, the overhead of uplink resource can be further reduced.

- Proposal 11: For Option 2 of Type 1 performance monitoring, the following alternatives can be considered to report the metric:

- Alt-1: Quantization of performance metric.

- Alt-2: The statistic values of the performance metric.

- Alt-3: A specific event.

- Regarding the configuration for monitoring for Option 2 of UE-sided AI/ML model, dedicated resource set(s) for monitoring and dedicated CSI report configuration for monitoring were agreed in RAN1#119 meeting. However, the timing related issues of dedicated resource set(s) for monitoring is still FFS. To calculate the beam prediction accuracy, measurement of the resource set for monitoring should match the corresponding predicted beam(s) in time domain and following two alternatives can be considered. For Alt-1, time instance of resource set(s) for monitoring is determined based on the time instance of predicted beam(s). By Alt-1, UE can determine the resource set(s) for monitoring, where the resource set(s) for monitoring is located in or near the time instance of predicted beam(s). For Alt-2, time instance of resource set(s) for monitoring is determined based on the uplink slot for the report. By Alt-2, if time instance of predicted beam(s) is determined based on the uplink slot for the report, UE can indirectly determine the resource set(s) for monitoring.

- Proposal 12: For time instance of resource set(s) for monitoring, the following alternatives can be considered:

- Alt-1: Time instance of resource set(s) for monitoring is determined based on the time instance of predicted beam(s).

- Alt-2: Time instance of resource set(s) for monitoring is determined based on the uplink slot for the report.

- 3 NW-side additional condition

- In RAN1#118 meeting, the following agreements on the consistency of NW-side additional condition across training and inference for UE-side model have been achieved [6]:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

- For the consistency of NW-side additional condition across training and inference for UE-side model, two options were achieved in the previous meeting. Regarding Opt1, consistency information can be derived implicitly from the associated ID, where different associated IDs may be corresponding to different NW-side additional conditions. With the same associated ID, NW-side additional condition(e.g., spatial domain filter or beam index/order) are consistent between training and inference. Regarding Opt2, the consistency of NW-side additional conditions can be achieved by monitoring the performance of the AI/ML models. By this way, constantly monitoring is required to achieve better model performance to align the additional condition and hence it may require more monitoring overhead. Therefore, regarding the consistency of NW-side additional condition across training and inference for UE-side model, support to confirm the working assumption.

- Proposal 13: Regarding the consistency of NW-side additional condition across training and inference for UE-side model, support to confirm the working assumption:

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- 4 RAN2 LS handling

- In RAN1#119 meeting, the following agreements on the RAN2 LS handling have been achieved [5]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

- Regarding applicability for inference for UE-side model, the potential contents of Step 3 to Step 5 for inference of UE-side model were agree in RAN1#119 meeting. However, whether/what other information along with the applicability is needed is still FFS.

- For the UE-side model, gNB has no information on what the UE needs for training and/or inference. Therefore, the applicable information should be reported to gNB to support the UE-side model operation, where the applicable information include, for example, observation window related configurations for BM-Case 2, prediction window related configurations for BM-Case 2, configuration of Set A and Set B (e.g., Set A and Set B are different, Set B is a subset of Set A, Set A and Set B are the same), Set B pattern (e.g., fixed Set B pattern, variable Set B pattern), Rx beam (e.g., best Rx beam, specific Rx beam(s), random Rx beam(s), quasi-optimal Rx beam), applicable associated ID(s) of the functionality (if any). Based on the the applicable information reported from UE, gNB can accordingly configure CSI-ReportConfig for inference configuration in Step 5 to facilitate the UE-side model operation.

- Proposal 14: UE can report the following information in Step 4 along with an associated applicable functionality:

- Observation window related configurations for BM-Case 2

- Prediction window related configurations for BM-Case 2

- Configuration of Set A and Set B, e.g., Set A and Set B are different, Set B is a subset of Set A, Set A and Set B are the same

- Set B pattern, e.g., fixed Set B pattern and variable Set B pattern

- Rx beam, e.g., best Rx beam, specific Rx beam(s), random Rx beam(s), quasi-optimal Rx beam

- Applicable associated ID(s) of the functionality (if any).

- 5 Conclusion

In this contribution, we discuss potential enhancements on AI/ML beam management. Based on the discussion above, we provide the following proposals:

Proposal 1: Regarding content for data collection for training for NW-sided model, support Opt2:

- Opt 2: L1-RSRPs measured based on one resource set (for Set B) configured to UE, and beam information of Top K measured based on another resource set (for Set A) configured to UE.

Proposal 2: Regarding the signaling for training data collection, support higher-layer signalling to report the contents of training data.

Proposal 3: Regarding the trigger/initiating data collection for UE-side model, support Option 2:

- Option 2: request from UE for data collection.

Proposal 4: Regarding the quantization of a reported L1-RSRP value, support larger quantization step(s) than the legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP.

- Proposal 5: Regarding UE-sided model for BM Case-2, for absolute RSRP and differential RSRP report over multiple time instances, time instance information of largest RSRP can be reported explicitly.

- Proposal 6: Regarding UE-sided model for BM Case-2, for the reference time to determine the earliest time instance from the future time instances, support Option 1(2nd) and Option 3(1st):

- Option 1: Based on the time domain resource for the report.

- Option 3: Based on the last transmission occasion of the CSI-RS/SSB resource in Set B for the report.

Proposal 7: Regarding CSI-RS resource types for CMR for Set B for BM-Case 2 of UE-side AI/ML model, support to trigger Aperiodic (AP) CSI-RS in multiple time instances.

- Proposal 8: For performance monitoring of NW-side model, configuration of Set B and/or Set A and the reporting of measurement results of Set B and/or Set A need to be specified.

- Proposal 9: Regarding Option 2 of UE-sided AI/ML model, support to configure a window for calculating the metric and the following alternatives can be considered to define a window:

- Alt-1: The window for calculating the metric is defined based on the uplink slot for the report.

- Alt-2: The window for calculating the metric is defined based on the CSI reference resource corresponding to the report.

- Proposal 10: Regarding the resource set/resources for monitoring for Option 2 of UE-sided AI/ML model, support to use full set of Set A for measurement.

- Proposal 11: For Option 2 of Type 1 performance monitoring, the following alternatives can be considered to report the metric:

- Alt-1: Quantization of performance metric.

- Alt-2: The statistic values of the performance metric.

- Alt-3: A specific event.

- Proposal 12: For time instance of resource set(s) for monitoring, the following alternatives can be considered:

- Alt-1: Time instance of resource set(s) for monitoring is determined based on the time instance of predicted beam(s).

- Alt-2: Time instance of resource set(s) for monitoring is determined based on the uplink slot for the report.

- Proposal 13: Regarding the consistency of NW-side additional condition across training and inference for UE-side model, support to confirm the working assumption:

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- Proposal 14: UE can report the following information in Step 4 along with an associated applicable functionality:

- Observation window related configurations for BM-Case 2

- Prediction window related configurations for BM-Case 2

- Configuration of Set A and Set B, e.g., Set A and Set B are different, Set B is a subset of Set A, Set A and Set B are the same

- Set B pattern, e.g., fixed Set B pattern and variable Set B pattern

- Rx beam, e.g., best Rx beam, specific Rx beam(s), random Rx beam(s), quasi-optimal Rx beam

- Applicable associated ID(s) of the functionality (if any).

- 6 Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN#102.

- R1-2410736, FL summary #3 for AI/ML in beam management, Samsung (Moderator), RAN1#119, November 18th–22nd

- Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 3GPP TR 38.843 v18.0.0

- Draft Report of 3GPP TSG RAN WG1 #117 v0.2.0, Chairman, RAN1#117, May 20th-24th, 2024

- Draft Report of 3GPP TSG RAN WG1 #119 v0.2.0, Chairman, RAN1#119, November 18th–22nd, 2024

- Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0, Chairman, RAN1#118, August 19th–23rd, 2024