3GPP TSG RAN WG1 #120                                                                                                 R1-25000983

Athens, Greece, February 17th – 21st, 2025

Source:               TCL

Title:                    Discussion on NR-NTN Downlink Coverage Enhancement

Agenda item:      9.11.1

Document for:    Discussion and Decision

- Introduction

In the RAN #106 meeting, a revised WID was approved for Rel-19 NR NTN [1], one of the objectives of the WID is related to downlink coverage enhancement with NR NTN operating, the details are shown below:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

In RAN1#119 meeting [2], the following agreements have been achieved.

Agreement

For PDSCH with Msg4 Link level enhancement:

- Support PDSCH repetition

  - FFS: signalling design including number of repetitions

  - FFS: impact on UE capability

- Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

Agreement

For link level enhancement of PDSCH with SIB1:

- Support PDSCH repetitions within 20 ms duration

  - The number of repetitions is fixed to 2 repetitions

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling (enabling repetitions, associated time resource determination, etc.)

- Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size.

- Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

Agreement

For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN.

- FFS: intra-slot and/or inter-slot

In this contribution, we share our views on NR-NTN downlink coverage enhancements based on system level and link level.

- Discussion

System-level enhancement

It is claimed in Rel-19 NTN WI [1] that system-level enhancements are discussed to increase the system-level coverage of FR1-NTN and/or FR2-NTN allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprints, this section will be focused on system level enhancements to improve DL coverage.

    - SSB periodicity

As discussed in the RAN1#116 meeting, the additional satellite parameter sets, i.e., LEO600km Set 1-1/1-2/1-3 have been agreed, it is observed that only 1.5% or 10.02% beam footprints are served, this limits the coverage area that the satellite can serve. In current spec, a default 20ms SSB periodicity is assumed to be a UE for cell discovery, as agreed for satellite parameters set1-1/1-2/1-3, a satellite beam footprint can be up to 1058, it’s very hard to cover such a large beam footprint within 20 ms, within an SSB burst, 4 footprints can be served by SSB, such, for set 1-1/1-3, the total number of  4*106 =424  footprint beams can be served, it’s more serious for set 1-2, only 4*16 =64 footprint beam can be served. Thus, the periodicity of SSB to a UE for cell discovery needs to be enlarged, and the extended SSB periodicity of 160ms has been agreed in the previous meeting. In addition, when the period of SSB is extended, to cover the whole footprint of a satellite, TDM-ed SSB transmission within a satellite footprint can be used. However, in the current specification, for random access procedure, there is a strict time relationship between a UL signal and a corresponding DL signal, TDM-ed SSB transmission will impact the random access procedure, thus, the impacts of TDM-ed SSB transmission on random access need to study.

Proposal 1: The impacts of TDM-ed SSB transmission on random access procedure need to be studied.

2.1.2 Potential solution to improve DL coverage based on system level

As defined in TR 38.821[2], satellite beams or satellites are not considered to be visible from the UE perspective in NTN. Both options a) the same PCI (Physical Cell Identifier) for several satellite beams and b) one PCI per satellite beam, can be considered in NTN, a satellite beam can consist of one SSB or more SSB beams. One cell (PCI) can have a maximum of L SSB beams, where L can be 4, 8, or 64 depending on the band, which means, mapping one SSB beam/cell to a/multiple satellite beams or multiple SSB/cells to a/multiple satellite beams are based on implementation way.

In Rel-18, for power saving at the gNB side, cell level-based DTX/DRX has been introduced, and cell DTX/DRX was introduced in the time domain to allow the network to configure non-active periods, e.g., cell DTX/DRX pattern. The pattern configuration for cell DTX/DRX is common for the UEs in the cell, and the cell DTX and cell DRX patterns can be configured and activated separately. When cell DTX is configured, the UE does not need to monitor PDCCH or SPS occasions during the cell DTX idle period. When cell DRX is configured and activated for the concerned cell, the UE does not transmit on CG resources or SR during the cell DRX idle period. This feature is only applicable to UEs in the RRC_CONNECTED state and it does not impact random access procedure, SSB transmission, paging, and system information broadcasting.

It is proposed that Rel-18 network energy-saving techniques should be considered as a baseline in the system-level study for NTN, in our view, for one NR cell/beam mapping to one satellite beam scenario, the same rules of Rel-18 NES techniques can be re-used, however, for another case, the flexibility will be limited if cell-based DTX/DRX is re-used, thus, some modifies of Rel-18 NES techniques will be needed.

Proposal 2: Modified Rel-18 network energy-saving techniques of cell DTX/DRX to align with the different scenarios encountered in the mapping of NR cell/SSB and satellite beams can be considered.

Link level enhancement

System level-based enhancements to DL coverage improvement are analyzed in section 2.1, this section, however, will concentrate on link-level enhancements designed to enhance DL coverage, proposing potential coverage improvements specifically for the CSS PDCCH and SIB1/Msg4 PDSCH channels.

    - Link level enhancements for common PDCCH

For DL coverage improvement for common PDCCH, enabling the common PDCCH with repetition can improve coverage performance due to time-domain diversity gain can be achieved, and a single PDCCH can be repeated across multiple PDCCH monitoring occasions in the time domain. A set of additional PDCCH monitoring occasions can be configured and UE can first monitor PDCCH in a set of legacy PDCCH monitoring occasions, if not decoding PDCCH correctly, then monitoring the PDCCH repetition within the additional PDCCH monitoring occasions. In addition, for Type-0 and others CSS with repetition, the additional PDCCH monitoring occasions indication needs to take into consideration of impact on legacy UEs.

Proposal 3: For common PDCCH coverage enhancement, enabling PDCCH with repetition in the time domain can be considered.

In addition, for type 0 PDCCH CSS is utilized for scheduling SIB1, to enable type0 PDCCH with repetition, the most straightforward way is to indicate/trigger type 0 PDCCH CSS with repetition via MIB. For other type CSSs, SIB1 can be used for enabling the corresponding PDCCHs with repetition.

Proposal 4: For enabling type 0 PDCCH with repetition, explicit indication via  MIB can be considered.

Proposal 5: For enabling other types of CSSs PDCCH with repetition, except for type 0, explicit indication via SIB1 can be considered.

To further improve the coverage performance for common PDCCH, an enlarged aggregation level can be considered, thus, additional gain of coding can be achieved. For instance, aggregation levels of 32 and 64 can be supported. However, the potential impact of the enlarged aggregation level for common PDCCH will be increased the blocking probability, to handle this, one of a straightforward way is to enlarge the symbol number for CORESET.

Proposal 6: For common PDCCH coverage enhancement, enlarging aggregation level can be considered.

2.2.2 Link level enhancements for PDSCH with Msg4/SIB1

In terms of enhancement for PDSCH conveying SIB1/Msg4, PDSCH with repetition is the most straightforward way, such repetition could be applied in the time domain. To enable PDSCH to convey SIB1/Msg4 with repetition, at least the repetition number and/or the repetition pattern need to be indicated.

Proposal 7: For PDSCH conveying SIB1/Msg4 coverage enhancement, repetition in the time domain can be considered.

In the current specification, an additional column associated with the repetition number into the TDRA table for PUSCH has been standardized. Similarly, for enabling Msg4 PDSCH repetition, the mechanism of PUSCH repetition indicator can be re-used. In actual, under the current specifications, for DCI formats containing TC-RNTI, the TDRA table will either be a default table or a table within PDSCH-ConfigCommon, thus, a column associated with Msg4 repetition number can be added into the corresponding table(s).  In addition, for the UE capability of Msg4 with repetition reporting, Msg3 PUSCH can be used.

Proposal 8: For enabling Msg4 PDSCH repetition, the repetition number can be joint coding with TDRA.

For enabling SIB1 PDSCH repetition, as agreed in previous meeting, the number of repetitions is fixed to 2 repetitions. Thus, to trigger SIB1 PDSCH repetition, 1 bit within the scheduling DCI can be considered.

Proposal 9: For enabling SIB1 PDSCH repetition,  the scheduling DCI of SIB1 can be considered.

In addition, downlink reference signals such as DMRS are configured for PDSCH use to channel estimation, and channel estimation accuracy has a large impact on PDSCH demodulation performance especially for the scenario with low SNR. Increasing the channel estimation accuracy can also increase the MCL of coverage. Generally, more DMRS will contribute to a more accurate channel estimation at the cost of a larger overhead, to ensure accurate channel estimation without more overhead, joint utilization of the DMRS among different slots or transmissions is straightforward. In Rel-17 PUSCH with repetition, DMRS bundling for cross-slot channel estimation has been adopted to enhance the coverage performance, to support this, the important premises include: (1) The power is consistent across the repetition slots, and (2) The phase is continuous between the repetition slots. When PDSCH conveys SIB1/Msg4 with repetition, a similar way of joint channel estimation in Rel-17 can be re-used, and the power consistent and phase continuous across the repetition slots can be based on gNB implementation.

Proposal 10: When PDSCH conveys SIB1/Msg4 with repetition, DMRS bundling for joint channel estimation across the repetition slots can be considered.

- Conclusion

In this contribution, the following proposals have been made:

Proposal 1: The impacts of TMD-ed SSB transmission on random access procedure need to be studied.

Proposal 2: Modified Rel-18 network energy-saving techniques of cell DTX/DRX to align with the different scenarios encountered in the mapping of NR cell/SSB and satellite beams can be considered.

Proposal 3: For common PDCCH coverage enhancement, enabling PDCCH with repetition in the time domain can be considered.

Proposal 4: For enabling type 0 PDCCH with repetition, explicit indication via  MIB can be considered.

Proposal 5: For enabling other types of CSSs PDCCH with repetition, except for type 0, explicit indication via SIB1 can be considered.

Proposal 6: For common PDCCH coverage enhancement, enlarging aggregation level can be considered.

Proposal 7: For PDSCH conveying SIB1/Msg4 coverage enhancement, repetition in the time domain can be considered.

Proposal 8: For enabling Msg4 PDSCH repetition, the repetition number can be joint coding with TDRA.

Proposal 9: For enabling SIB1 PDSCH repetition, the scheduling DCI of SIB1 can be considered.

Proposal 10: When PDSCH conveys SIB1/Msg4 with repetition, DMRS bundling for joint channel estimation across the repetition slots can be considered.

4. References

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] 3GPP RAN1 #119 meeting, Chairman notes.

[3] TR 38.821, Solutions for NR to support non-terrestrial networks (NTN).