3GPP TSG-RAN WG1 Meeting #120	Tdoc R1-2500992

Athens, Greece, February 17th – 21st 2025

Agenda Item:	9.9.1

Source:	Ericsson

Title:	Measurement related enhancements for LMT

Document for:	Discussion, Decision

# 1	Introduction

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1]

  - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

NOTE: 	RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

After revision at RAN#105, the WID for the Rel. 19 “NR mobility enhancements Phase 4” [3] states the following objective:

In this contribution we present our analysis and proposals for discussion on these topics.

# 2	Discussion

## 2.1	Event triggered L1 measurement reporting

### 2.1.1	Number of measurement results in a report

In RAN2#128, RAN2 agreed

7.	For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

8.	For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

9.	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

In the subsequent post meeting email discussion for the RRC running CR, [POST128][108][MOB] RRC running CR, many companies expressed that RAN1 input is needed for the question: “What should be the maximum number of beam measurement results can be reported?”

For the gNB scheduled L1 measurement reporting specified in Rel-18 and also agreed for Rel-19, the network configures nrOfReportedCells (L, up to four) nrOfReportedRS-PerCell (M, up to four), in total 16 beams. The network can additionally configure spCellInclusion. It is reasonable to keep this configurability of up to 16 beams and spCellInclusion for event triggered L1 measurement reports:

For event triggered L1 measurement reports, support UE to include up to 4 beams per cell, for up to 4 cells.

Note that RAN2 has brought up the issue in Proposal 1 as part of the discussion of the RRC running CR [4].

According to the agreements above, measurements on resources that do not meet the event conditions can be included in the report. By including multiple beams from the same LTM Candidate Cell, the network can learn whether the triggering beam is an outlier or not, if it is a single beam with much higher RSRP than the other beams of the cell or if there are multiple beams with high RSRP. Such information can be useful input to the decision about triggering cell switch.

By including multiple beams from the same LTM Candidate Cell, the network can learn if there are multiple beams with high RSRP in the cell or not.

In the same way, by including measurements from additional LTM candidate cells, the network can learn if for example there is another LTM candidate cell that has several beams with high RSRP.

By including measurements from additional LTM candidate cells, the network can learn if there is another LTM candidate cell that has several beams with high RSRP.

The LTM2 event does not involve LTM Candidate resources. Still, even with LTM2 the network can benefit from measurements on LTM candidates since it gives information about the mobility options that are available as the quality of the serving cell is going down. For example, if there is another LTM Candidate cell that is better than the serving cell, but not good enough to trigger an event itself.

For event-triggered L1 LTM measurement reporting, keep the option to configure up to four nrOfReportedCells, up to four nrOfReportedRS-PerCell and spCelInclusion.

### 2.1.2	Reference signal for event evaluation

In RAN1#119, the following agreement was made:

Agreement

For the identification of the serving cell RS for event evaluation,

If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell.

- UE does not expect the following configuration:

  - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and

  - CSI-RS is configured as measurement resource for the candidate cell(s).

This means that the UE determines the quality of the serving cell either based on the CSI-RS in the indicated TCI state, or on the SSB that is the QCL source of the CSI-RS in the indicated TCI state. The UE determines which option to choose based on the type of the RSs in the candidate configurations.

This approach always works for LTM3, LTM4 and LTM5, where the NW must configure RSs in the candidate configurations, since the event is triggered based on a comparison between the serving RS quality and candidate RS quality. For LTM2 (beam of serving cell becomes worse than absolute threshold), the event can trigger also without any configuration of the candidate beams, and in this case, the agreed rule would not work.

However, we note that also for LTM2, in most cases the NW would configure the UE to report measurements on beams. We also note that the NW can control how many beams are included in the report, using the RRC parameters nrOfReportedCells and nrOfReportedRS-PerCell. In the unlikely event that the NW does not want the UE to include any additional beams in the report, the NW would simply set nrOfReportedCells= nrOfReportedRS-PerCell=1. Hence, we can assume that the NW always configures candidate RSs, also for LTM2:

The NW configures at least one candidate RS for LTM2, and the UE determines the serving cell RS using the already agreed mechanism.

### 2.1.3	Filtering

Relevant agreements and background:

RAN2#126 agreements:

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Filtering of L1-RSRP has been discussed for the purpose of (1) event condition evaluation and (2) measurement reporting. Filtering is mainly needed for event condition evaluation, but we are also positive to filtering for measurement reporting.

Figure 1 illustrates the L1-RSRP measurements for three LTM Candidates measured by a UE in an Urban Micro FR2 scenario, see the Appendix for the detailed simulation assumptions. Notice that only a few seconds are captured. Although a UE implementation filter is applied, the variations are still large and fast over time. For example, at time t=3.9s LTM Candidate #1 RSRP increases dramatically. Then at t=4.2s it drops back only to increase again at t=4.4s. The fast variations make it difficult to select suitable LTM event thresholds and time-to-trigger: If TTT is long, temporary dips will reset the timer and the event triggering becomes significantly delayed. If TTT is short then there is a large risk for ping-pong handover and spurious TCI state activation/deactivation.

Large variance in L1-RSRP cannot be handled well by TTT mechanisms: If TTT is too long then temporary dips will reset the timer and delay the event. If TTT is too short then it leads to frequent ping-pong handovers.

Figure 1: L1-RSRP for three LTM Candidates. For each candidate, the SSB with highest SS-RSRP is used.

In the following we use the terminology:

- UE implementation filter: Internal layer 1 filtering of RSRP measurement which is up to UE implementation, see TS 38.300 Sec. 9.2.4.  As per the RAN4 requirements, for L3-RSRP, 5 samples are considered for layer 1 filtering. For L1-RSRP, when the UE is not configured with timeRestrictionForChannelMeasurements, 3 samples are considered for averaging of L1-RSRP.

- Network configurable filter: A filter that is configured by the network that operates on the output of the UE-implementation filter.

The L3 filtering is one example of a filter with parameters that can be configured by the network and tuned for performance. A similar network configurable filter can be used in the MAC layer. The current L3 filtering includes a first order IIR filter, as depicted in eq. (1), with a tunable filter coefficient, a, that is a forgetting factor: a large value means that only the most recent measurements have a significant weight, while older measurements are quickly “forgotten”. Hence, with a large filter coefficient, the filter quickly adapts but is sensitive to noisy samples. A small filter coefficient can on the other hand reduce the impact of noise at the expense of slower adaptation.

The main reason for including a network configurable filter in addition to a UE implementation filter is to reduce the variance of RSRP so that the trend becomes clear and TTT and event thresholds and offsets can be properly configured.

A network configurable filter can reduce the variance of RSRP, making the trend clear such that TTT and event thresholds and offsets can be properly configured.

Another important reason for including a network configurable filter is that the quality of measurements differs between UEs. As a result, the configured TTT value and the RSRP thresholds and offsets that are suitable for one UE will not work well with other UEs. A network configurable filter provides a consistent variance of the RSRP measurements across different UEs.

When the quality of measurements differs between UEs, the configured TTT value and the RSRP thresholds and offsets that are suitable for one UE will not work well with other UEs.

A network configurable filter can ensure consistent variance of the RSRP measurements across different UEs.

The network can tune the filter forgetting factor for different deployments to achieve optimal performance with respect to for example handover success rate or other KPIs. For example, some deployments may require fast dynamics (e.g. FR2 and UEs in cars) while other benefit from slower dynamics but lower variance (FR1, pedestrian road).

A network configurable filter can be tuned for requirements on fast dynamics and robustness to ping-pong handover.

To illustrate the effect of filtering on the measurement process, we have performed system simulations for an urban macro scenario, where the LTM3 event has been used. We have included a UE implementation filter and investigated the effect of including a network configurable filter, which operates on the output of that UE implementation filter, as described earlier in this section, i.e., the UE is allowed to average 3 samples when calculating the L1-RSRP. We have studied the following quantities:

- Number of times the LTM3 event was triggered

- Number of times the LTM switch was finished

- Number of ping-pong handovers. A ping-pong handover is a handover where the UE stays in the new cell for less than a second before going back to the source

We have used a first order IIR filter as a NW configurable filter:

Fn=1-a×Fn-1+a×Mn 					(1)

where Fk is the output of the NW-configurable filter at time instant k, Mk is the output of the UE-implementation filter at time instant k, and a is a configurable filter coefficient. More simulation parameters are collected in the appendix.

The results are depicted in Figure 2 for pedestrian speed and in Figure 3 for vehicular speed.

Figure 2: Results for pedestrian speed. Note that the filtering significantly reduces the number of ping-pong handovers.

Figure 3: Results for vehicular speed. Note that the filtering significantly reduces the number of ping-pong handovers.

The results in Figure 2 and Figure 3 show the benefits of the NW-configurable filtering. In particular, there is a reduction in the number of ping-pong handovers:

With NW-configurable filtering, the number of ping-pong handovers is reduced.

Based on the discussion and these results it is important to introduce a NW-configurable filter for Event-triggered LTM after the UE implementation filter. The output from the UE implementation filter is the input to the NW-configurable filter. We propose that it should be a first order IIR filter like the one used for L3 mobility since it is simple to configure, intuitive and has worked well for L3 mobility. A first order IIR only needs one configuration parameter for the forgetting-factor and the UE only needs to store one state-variable corresponding to the filtered RSRP, consequently the implementation can be very light-weight. Compared to L3 filtering, the LTM filter coefficient can be tuned differently if it is desirable to achieve for example faster dynamics, but the fundamental structure of the filter can be the same. This will minimize both the specification impact and implementation cost.

Note that the NW configurable filter will be implemented in MAC specification, based on the following agreement from RAN2:

MAC layer handles the event evaluation and measurement report triggering. (RAN2#127)

The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1. (RAN2#127bis)

Hence, we are not proposing any changes to the UE-implementation filter: the additional filter will operate at the output of the L1 filter.

A first-order IIR filter only requires the UE to store one state-variable corresponding to the filtered RSRP.

A first-order IIR filter is configured with a single filter coefficient.

Introduce a network configurable filter in the MAC layer.

The network configurable filter is a first order IIR filter.

LTM event conditions should be evaluated on RSRP measurements that are filtered with the network configurable filter.

## 2.2	CSI-RS measurements for LTM procedures and beam management

### 2.2.1	Candidate Cell CSI-RS configuration

Relevant background:

RAN1#118 Agreement

- Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported

RAN1#118 Agreement

For gNB scheduled reporting and event triggered reporting

- At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell

  - FFS: CSI-RS for mobility

RAN1#118-bis Agreement

The agreement “Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting” made in RAN#118 is further clarified for L1-RSRP as follows:

- UCI format defined in Table 6.3.1.1.2-8C of TS38.212 can be used by replacing SSBRI with CRI.

- Whether the L1-RSRP(s) of serving cell is always included is configurable (in line with Rel-18)

- The quantization method defined in clause 5.2.1.4.3 of TS38.214 and bit width defined in Table 6.3.1.1.2-6 of TS38.212 can be used

- No L1 specified filtering for time and spatial domain is introduced

- No enhancement on how to report L cells x M beams

- Periodic reporting on PUCCH is supported

  - FFS: semi-persistent reporting on PUCCH/PUSCH, and aperiodic reporting on PUSCH

RAN1#118-bis Agreement

For CSI-RS based L1-measurement report by gNB scheduled measurement reporting, semi-persistent reporting on PUCCH/PUSCH and aperiodic reporting on PUSCH are supported

RAN1#119 Conclusion

- No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM

- Note: From the actual gNB transmission viewpoint, CSI-RS for mobility and CSI-RS for BM may be the same

We note that the RRC spec. already support explicit configuration of CSI-RS resource(s) and resource sets per LTM-Candidate, in accordance with the RAN1#118 agreement above. The structure LTM-Candidate-r18   LTM-TCI-Info-r18 was introduced for the purpose of fine time synchronization with TRS, but the structure is flexible enough to support CSI-RS for beam management as well as other CSI-RS resources that may be considered too.

Reuse LTM-TCI-Info to configure candidate cell CSI-RS resources for beam management

Regarding the reporting framework, it is natural if the LTM CSI-RS measurements could be reported in the same way as the LTM SSB measurements. The SSBs of an LTM Candidate are firstly specified in LTM-Candidate  LTM-SSB-Config (this is done per LTM Candidate) and secondly in LTM-CSI-ResourceConfig. To align how the CSI-RS measurements are reported, there is a need to also align the measurement and resource configuration. Hence, we propose

Specify the CSI-RS for LTM measurements in the LTM-CSI-ResourceConfig.

In LTM-CSI-SSB-ResourceSet, SSBs of multiple LTM Candidate cells can be included. The LTM SSB measurement report includes an SSBRI and RSRP, where the SSBRI is the index into the LTM-CSI-SSB-ResourceSet. Similarly, the gNB scheduled LTM CSI-RS measurements should include a CRI and RSRP. This will allow  reuse of the report format detailed in TS 38.212, Table 6.3.1.1.2-8C (only replacing SSBRI with CRI). The CRI is the index in a resource set that includes CSI resources from multiple LTM Candidate cells. We propose to introduce a new LTM-CSI-NZP-CSI-RS-ResourceSet which is added under LTM-CSI-ResourceConfig and parallell to LTM-CSI-SSB-ResourceSet, see the ASN.1 example below.

Introduce a resource set with resources from multiple LTM candidate cells under LTM-CSI-ResourceConfig.

Note that in this example the LTM-CSI-NZP-CSI-RS-ResourceSet includes resources from different LTM candidate cells. We don’t see that NZP-CSI-RS-ResourceSet is necessary for LTM L1 measurements, on the contrary, if multiple NZP-CSI-RS-ResourceSet from different LTM candidate cells are combined into a “hyper-set”, then CRI cannot be used in the measurement report.

Aside from small differences in fieldnames, the example above is very similar to the current proposal in the RRC running CR [4]. The only real difference is that we have included the optional repetition field since CSI-RS for beam management is agreed to be supported. However, unlike the legacy NZP-CSI-RS-ResourceSet, there is no need for trs-info and no fields for aperiodic triggering or resource type since trs-info should not be configured, and aperiodic CSI-RS has not been agreed to be supported.

In the P3 procedure in legacy beam management, the NW transmits a CSI-RS resource set with repetition on, meaning that the UE can assume that the CSI-RS resources in the CSI-RS resource set is transmitted using the same beam. The UE can use this CSI-RS resource set to quickly evaluate the quality with different Rx beams. Unfortunately, the overhead on the NW side is large, since multiple CSI-RS resources must be transmitted in the same beam. Therefore, P3 is more suitable with aperiodic CSI-RS, where only a few Tx beams are probed:

The P3 procedure is more suitable with aperiodic CSI-RS.

However, for LTM it is difficult to use aperiodic reference signals due to the transport NW delay. Based on this, we propose

Do not support the P3 procedure, i.e., measurements on a CSI-RS resource set with repetition on.

If repetition ON is agreed to be supported (in contradiction to Proposal 10) then it should be interpreted per LTM Candidate cell in the in the resource set. That is, with repetition ON, all resources from Cell#1 are repetitions and all resources from Cell#2 are repetitions etc., but one resource from Cell#1 and another resource from Cell#2 are not repetitions.

If semi-persistent CSI-RS is agreed to be supported for LTM, the configuration should follow the legacy principles for SP-CSI-RS. The resourceType (semi-persistent) should be indicated in the resource configuration. The report configuration is separate. There is also a need to introduce a MAC CE indication when transmission is turned ON/OFF.

If semi-persistent CSI-RS is agreed to be supported for LTM, keep legacy principles for SP CSI-RS: the LTM-CSI-ResourceConfiguration should indicate the resourceType semi-persistent, the network indicates when transmission is turned ON/OFF with a MAC CE, reporting is a separate configuration.

### 2.2.2	Time domain properties

Semi-persistent CSI-RS is agreed to be supported for gNB scheduled reporting. It can potentially reduce the signaling overhead compared to periodic CSI-RS although it is not clear how the network can do that. One approach is to indicate additional SP CSI-RS to the UE when periodical SSB or CSI-RS measurements suggests that the UE is approaching the cell border. For event-triggered reporting on the other hand, one of the main motivations is that the UE takes care of the measurements itself. If the network must monitor periodical SSB or CSI-RS measurements to turn ON/OFF SP CSI-RS, then parts of the benefits are lost.

Parts of the benefits with event-triggered reporting are lost if the network must monitor periodical SSB or CSI-RS measurements to turn ON/OFF SP CSI-RS.

In line with this observation, we propose that SP CSI-RS is not supported for event-triggered reporting:

Semi-persistent CSI-RS is not supported for event-triggered reporting.

Regarding aperiodic CSI-RS, one concern is that AP CSI-RS necessitates additional signaling over the transport network, between the serving cell and the LTM candidate cells. This signaling comes with delays that in the worst case means that the UE will not know in time that e.g. an aperiodic CSI-RS is going to be transmitted. Transport network delays may not be as critical for SP CSI-RS: even if the UE misses the first transmission(s) it can still measure on later ones.

A UE may not be notified in time that an aperiodic CSI-RS is going to be transmitted due to transport network delays.

Aperiodic CSI-RS is not supported for event triggered or gNB scheduled reporting.

## 2.3	CSI acquisition

In RAN1#118bis and RAN1#119, the following was agreed:

Agreement (RAN1#118bis)

The following alternatives are further studied:

- Alt-1: CSI-RS measurement and CSI reporting operations are performed before reception of LTM Cell Switch Command (CSC) MAC CE.

  - The report is sent to the serving cell and transferred to the candidate/target cell(s)

- Alt-2: CSI-RS measurement can start before reception of LTM CSC MAC CE and CSI reporting operation is performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

- Alt-3: CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

  - The report is sent directly to target cell

Companies are requested to provide the details of exact report timing and triggering mechanism in the next meeting

Working Assumption (RAN1#119)

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

  - Other than UE capability, strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

### 2.3.1	Basics

CSI acquisition in the serving cell can be performed based on periodic, semi-persistent and aperiodic CSI-RS. With semi-persistent and aperiodic CSI-RS, the serving cell activates the transmission of the corresponding reference signals and instructs the UE to perform the measurements. For LTM, the CSI-RS should be transmitted by the candidate cell, which is not under the control of the serving cell. To activate CSI-RS transmission in a candidate cell, the serving cell would have to request the candidate cell to activate the transmission, and that would have to be signalled over a transport network with potentially varying and non-deterministic delay. Based on this reasoning, we feel that it is sufficient to specify early CSI acquisition based on periodic CSI-RS:

Support CSI acquisition on candidate cells based on periodic CSI-RS.

To perform link adaptation, the NW needs a CQI value, which is one part of CSI. In NR, there are many additional parameters that describe CSI, and the natural question is what parameters would be relevant to report also for a candidate cell. Since this would be used only during a short time, only the most important parameters should be reported. Here we propose to limit this to Type I codebook reporting, and only report CRI, CQI, PMI and RI:

Support reporting of CRI, CQI, PMI and RI for a Type I codebook for a candidate cell before or after LTM cell switch.

We thus exclude the Type II codebook, due to complexity and reporting overhead.

CSI acquisition on candidate cells should support all Type I codebooks in NR. This means that we support up to 128 ports. In case the candidate cell uses 128-port codebooks for intra-cell MIMO, it is vital that it is possible to reuse these CSI-RS resources also for candidate cell measurements:

Support Type I codebook with up to 128 ports for CSI acquisition on candidate cells.

### 2.3.2	Measurement procedure

For serving cell CSI measurements on periodic CSI-RS which is associated with an (active) CSI report configuration, the UE must always be prepared to report. The UE reports its limits when it comes to CSI reporting via the corresponding UE capabilities.

However, performing CSI measurements on a CSI-RS resource on a candidate cell is more complicated than on the serving cell, since the UE must perform fine time-frequency synchronize with the candidate cell before it can perform CSI acquisition. The UE typically does not synchronize to a candidate cell before a TCI state in the candidate cell is activated.

RAN1 has agreed that in the baseline solution, the CSI-RS measurement and CSI reporting operations are performed after reception of the LTM CSC MAC CE. Fundamentally, the time when the UE performs measurements on a CSI-RS is up to UE implementation. What must be specified is the time when the CSI report will be delivered to the NW, and how the CSI report is delivered to the NW. As the early CSI acquisition is only relevant for the initial time the UE is in candidate (before the normal CSI acquisition becomes active) it is relevant to consider some sort of aperiodic reporting:

It is not relevant to consider periodic or semi-persistent CSI reporting for early CSI reporting.

When discussing aperiodic CSI reporting, there are two time-constraints that are relevant (TS 38.214 Sec. 5.4):

- Z’ is with respect to the CSI-RS resource. This requirement describes how much time the UE needs to calculate the CSI.

- Z is with respect to the DCI requesting the report. This requirement describes how much time the UE needs to prepare the UL transmission.

For aperiodic CSI-RS, the UE is not required to report CSI if the time between the DCI and the report is less than Z, or if the time between the CSI-RS resource and the report is less than Z’. When these requirements are not fulfilled, the UE behaviour is undetermined: the UE can send any CSI report.

For periodic CSI-RS, only Z is strictly applicable: if the time between the CSI-RS resource and the report is smaller than Z’, the UE would simply use a previous CSI-RS resource occasion to calculate the CSI. However, for early CSI reporting for LTM, it is relevant to consider also Z’ for a periodic CSI-RS resource, due to the transient nature of the procedure: the UE may not have had time to measure on a previous CSI-RS transmission. Hence, the UE can only report a valid CSI if the time between the latest CSI-RS resource and the report is larger than Z’. The target knows when the CSI-RSs are transmitted, so under normal circumstances, this should not be a problem.

In the baseline solution in the working assumption agreed in RAN1#119, the CSI-RS measurement and CSI reporting operations are performed after reception of the LTM CSC MAC CE. The problem is that the target does not know when the UE receives the LTM CSC MAC CE: the target only knows when it receives the first UL message. The maximum time between the LTM CSC MAC CE and the first UL message is provided in section 6.3.1 in [1], but the UE can (and in many cases will) access the target before that.

Since the target does not know when the LTM CSC MAC CE was received, the target does not know when the UE performs the measurement, which in turn means that the target cannot determine when a valid CSI has been derived by the UE:

Since the target does not know when the UE received the LTM CSC MAC CE was received, the target does not know when the UE has calculated a valid CSI.

The situation is illustrated in Figure 4.

Figure 4: The timing between the CSI-RS transmission and the LTM CSC is unknown at the target. The target only knows when it receives the first UL message, and the time between the LTM CSC and the first UL message is unknown. In case 1, the UE has most likely computed valid CSI already when transmitting the first UL message. In case 2, the UE cannot compute valid CSI until after the reception of RS#3.

In legacy, the NW sends a CSI request to the UE, and the UE responds with the corresponding CSI report. In legacy, it is up to the NW to allow enough time for the UE to derive a valid CSI. However, as previously described, the target does not know when the UE starts measuring CSI, as it is determined by signaling from the source. If the legacy procedure is reused, the target can only assume that the UE measures CSI based on the first CSI-RS after the first UL message:

If legacy CSI reporting mechanisms are reused, the target can only request CSI based on the first CSI-RS after the first UL message.

With Observation 15, the benefits of early CSI acquisition become unclear, since CSI-RSs transmitted before the first UL message cannot be used.

The same problem was analysed in the TEI proposal [2]. The conclusion in [2] is to introduce a specific MAC CE that informs the target when CSI is available. The UE would include that MAC CE in any UL transmission, perhaps even the first UL transmission. As the reception of that MAC CE, the target could send a CSI request to the UE, which sends the corresponding CSI. Since the target knows that the CSI is available at the UE, only the restriction Z is applicable.

The solution proposed in [2] provides a solution to the timing uncertainty problem described above. Of course, the UE knows when it computed the CSI, and can notify the target as soon as the CSI measurement is available. However, an alternative solution is that the UE directly sends the CSI using a MAC CE. This would reduce the delay with another RTT. Hence, we propose

The UE reports the early CSI to target over MAC CE as soon as the CSI has been calculated.

Note that with Proposal 14, the UE can include the report as soon as it is available, in any PUSCH allocation, for example the PUSCH where the UE sends RRC reconfiguration complete. Also, the method makes it possible for the UE to perform the CSI measurements at any point in time after receiving the configuration. There is no need to specify a timeline in RAN1 for the baseline solution, nor is there a need to discuss a timeline for the faster solution. Strictly speaking, there is no need for a UE capability either: there is no need for the NW to adapt its behaviour, since the UE sends the report as soon as the CSI is available.

### 2.3.3	Configuration

Each LTM candidate needs to provide its CSI report configuration to the source cell in advance, so that the source cell can configure the UE with it.

Each LTM candidate provides its report configuration for CSI acquisition to the source cell.

The same configuration solution should be used to configure CSI-RS resources for CSI acquisition as for CSI-RS for L1-RSRP measurements, i.e. LTM-CSI-ResourceConfig (see Section 2.2.1). The proposed LTM-CSI-NZP-CSI-RS-ResourceSet can be used but with the field repetition absent.

CSI-RS for CSI acquisition should be configured in LTM-CSI-NZP-CSI-RS-ResourceSet under LTM-CSI-ResourceConfig similar to CSI-RS for L1-RSRP measurements but with the repetition field absent.

Moreover, we can reuse much of the existing framework for measurement and reporting. The LTM-CSI-ReportConfig can be extended to support report quantities like cri-RI-PMI-CQI.

Adapt LTM-CSI-ReportConfig to support relevant report quantities, including cri-RI-PMI-CQI.

Given that the measurements are triggered in the source cell and the reporting in the target cell, both the LTM-CSI-ResourceConfig and the LTM-CSI-ReportConfig need to be shared between the source cell and the target cell (and the UE). Then the UE can be configured to measure CSI while it is still in the source cell and send the CSI report to the target cell with reference to the same report and resource configurations. The LTM-CSI-ResourceConfig is already kept by the UE over cell switch execution, what remains is to also share the LTM-CSI-ReportConfiguration for CSI acquisition between the source cell, the target cell and the UE.

A UE configured with CSI measurements by the source cell can report the results to the target cell if the report configuration is shared between the source and target cells, just like LTM-CSI-Resource-Configuration is already shared.

The LTM-CSI-ReportConfig for CSI acquisition should be shared between the source cell, the target cell and the UE. The UE should keep it over cell switch execution.

## 2.4	Conditional LTM

In LS R1-2500011, RAN2 informs RAN1/3/4 about their agreements regarding Conditional LTM, in particular the following:

8.	The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

There is however nothing that RAN1 need to act upon yet, we can wait for further progress in RAN2 and more detailed requests/questions from RAN2. The impact of RAN2 agreements in RAN1 specifications need to be understood. Conditional LTM introduces a new mechanism to trigger the execution of LTM and the description in TS 38.213 Sec 21 need to be updated accordingly. The following aspects should be considered:

- The UE is provided with CLTM configurations including execution conditions. The UE need to measure the corresponding resources and evaluate the conditions, but not report the results.

- The LTM Cell Switch Command MAC CE can indicate a TCI state to be used for receptions and transmissions in the target cell. This does not apply for CLTM. Which TCI is used when CLTM triggers cell switch should be described. After CLTM execution is triggered, activated TCI states that are not used should be deactivated.

- The timeline from when a CLTM execution conditions for an CLTM candidate is met to when the UE should have applied the corresponding configuration and candidate TCI state.

- Updates in configured-grant PUSCH transmission in the target, pending RAN2 progress.

TS 38.213 Section 21 need to be updated to describe conditional LTM, including the following aspects:
- Configuration and evaluation of CLTM execution conditions in the UE,
- TCI state to be applied upon CLTM execution,
- The timeline for CLTM execution,
- CG PUSCH transmission in the target cell.

# Conclusion

In the previous sections we made the following observations:

Observation 1	By including multiple beams from the same LTM Candidate Cell, the network can learn if there are multiple beams with high RSRP in the cell or not.

Observation 2	By including measurements from additional LTM candidate cells, the network can learn if there is another LTM candidate cell that has several beams with high RSRP.

Observation 3	Large variance in L1-RSRP cannot be handled well by TTT mechanisms: If TTT is too long then temporary dips will reset the timer and delay the event. If TTT is too short then it leads to frequent ping-pong handovers.

Observation 4	A network configurable filter can reduce the variance of RSRP, making the trend clear such that TTT and event thresholds and offsets can be properly configured.

Observation 5	When the quality of measurements differs between UEs, the configured TTT value and the RSRP thresholds and offsets that are suitable for one UE will not work well with other UEs.

Observation 6	A network configurable filter can ensure consistent variance of the RSRP measurements across different UEs.

Observation 7	A network configurable filter can be tuned for requirements on fast dynamics and robustness to ping-pong handover.

Observation 8	With NW-configurable filtering, the number of ping-pong handovers is reduced.

Observation 9	A first-order IIR filter only requires the UE to store one state-variable corresponding to the filtered RSRP.

Observation 10	A first-order IIR filter is configured with a single filter coefficient.

Observation 11	The P3 procedure is more suitable with aperiodic CSI-RS.

Observation 12	Parts of the benefits with event-triggered reporting are lost if the network must monitor periodical SSB or CSI-RS measurements to turn ON/OFF SP CSI-RS.

Observation 13	A UE may not be notified in time that an aperiodic CSI-RS is going to be transmitted due to transport network delays.

Observation 14	It is not relevant to consider periodic or semi-persistent CSI reporting for early CSI reporting.

Observation 15	Since the target does not know when the UE received the LTM CSC MAC CE was received, the target does not know when the UE has calculated a valid CSI.

Observation 16	If legacy CSI reporting mechanisms are reused, the target can only request CSI based on the first CSI-RS after the first UL message.

Observation 17	A UE configured with CSI measurements by the source cell can report the results to the target cell if the report configuration is shared between the source and target cells, just like LTM-CSI-Resource-Configuration is already shared.

Observation 18	TS 38.213 Section 21 need to be updated to describe conditional LTM, including the following aspects: - Configuration and evaluation of CLTM execution conditions in the UE, - TCI state to be applied upon CLTM execution, - The timeline for CLTM execution, - CG PUSCH transmission in the target cell.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For event triggered L1 measurement reports, support UE to include up to 4 beams per cell, for up to 4 cells.

Proposal 2	For event-triggered L1 LTM measurement reporting, keep the option to configure up to four nrOfReportedCells, up to four nrOfReportedRS-PerCell and spCelInclusion.

Proposal 3	The NW configures at least one candidate RS for LTM2, and the UE determines the serving cell RS using the already agreed mechanism.

Proposal 4	Introduce a network configurable filter in the MAC layer.

Proposal 5	The network configurable filter is a first order IIR filter.

Proposal 6	LTM event conditions should be evaluated on RSRP measurements that are filtered with the network configurable filter.

Proposal 7	Reuse LTM-TCI-Info to configure candidate cell CSI-RS resources for beam management

Proposal 8	Specify the CSI-RS for LTM measurements in the LTM-CSI-ResourceConfig.

Proposal 9	Introduce a resource set with resources from multiple LTM candidate cells under LTM-CSI-ResourceConfig.

Proposal 10	Do not support the P3 procedure, i.e., measurements on a CSI-RS resource set with repetition on.

Proposal 11	If semi-persistent CSI-RS is agreed to be supported for LTM, keep legacy principles for SP CSI-RS: the LTM-CSI-ResourceConfiguration should indicate the resourceType semi-persistent, the network indicates when transmission is turned ON/OFF with a MAC CE,reporting is a separate configuration.

Proposal 12	Semi-persistent CSI-RS is not supported for event-triggered reporting.

Proposal 13	Aperiodic CSI-RS is not supported for event triggered or gNB scheduled reporting.

Proposal 14	Support CSI acquisition on candidate cells based on periodic CSI-RS.

Proposal 15	Support reporting of CRI, CQI, PMI and RI for a Type I codebook for a candidate cell before or after LTM cell switch.

Proposal 16	Support Type I codebook with up to 128 ports for CSI acquisition on candidate cells.

Proposal 17	The UE reports the early CSI to target over MAC CE as soon as the CSI has been calculated.

Proposal 18	Each LTM candidate provides its report configuration for CSI acquisition to the source cell.

Proposal 19	CSI-RS for CSI acquisition should be configured in LTM-CSI-NZP-CSI-RS-ResourceSet under LTM-CSI-ResourceConfig similar to CSI-RS for L1-RSRP measurements but with the repetition field absent.

Proposal 20	Adapt LTM-CSI-ReportConfig to support relevant report quantities, including cri-RI-PMI-CQI.

Proposal 21	The LTM-CSI-ReportConfig for CSI acquisition should be shared between the source cell, the target cell and the UE. The UE should keep it over cell switch execution.

# References

3GPP TS 38.133, NR; Requirements for support of radio resource management, RAN#103, Maastricht, The Netherlands, v18.5.0, March 2024

R2-2409931, Enhancement for resolving throughput degradation during handover, China Unicom, Huawei, HiSilicon, Sony, Turkcell, NTT Docomo, Meta, RAN2#128, Orlando, USA, November 2024

3GPP RP-242356, Revised Work Item: NR mobility enhancements Phase 4, 3GPP TSG RAN Meeting #105, September 2024.

R2-25xxxxx, Introduction of event-triggered measurement reporting for RRC spec, Huawei, RAN2#129, February 2025