3GPP TSG RAN WG1 #120			 R1-2501002

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.7.2

Source:	Moderator (Xiaomi)

Title:	Summary #4 on ISAC channel modelling

Document for:	Discussion/Decision

- Highlight on discussion plan for RAN1#120bis

Based on the progress in RAN1 #120, the following issues needs to be resolved in RAN1 #120bis meeting.

- Physical object model

  - Framework and values for bistatic/monostatic RCS

    - Values for RCS model with angular dependent B1 for large size UAV, human RCS model 2, vehicle (bistatic), AGV, …

    - Values for bistatic RCS model of small size UAV, human RCS model 1

  - XPR values of polarization matrix of target

- Channel model

  - Remaining issues of basic ISAC channel model, e.g.,

    - Polarization matrix normalization

    - Parameter values for monostatic background channel

    - Details on power normalization of target channel and background channel

  - Remaining issues on reference TRs to generate target channel and target channel

  - Remaining issues on channel model for target with multiple scattering points

  - Remaining issues on EO type-2, e.g.,

    - LOS condition determination

    - EO type-2 in background channel

- Remaining issues on spatial consistency

  - Correlation among the multiple scattering points

  - New model for spatial consistency for ‘link correlated’

    - Note: intention is to align companies’ understanding, but not to specify details in the CR to 38.901

- Link level channel model, if time allows

- Others

  - Angular correlation of RCS

  - Blockage model A/B

  - Micro-Doppler

  - New low power clusters in background channel

Introduction

In RAN #105, the updated SID on channel modelling for Integrated Sensing and Communication (ISAC) was agreed. The change is to remove the checkpoint for additional study beyond channel modelling [1].

The focus of the study is to define channel modelling aspects to support object detection and/or tracking (as per the SA1 meaning in TS 22.137). The study should aim at a common modelling framework capable of detecting and/or tracking the following example objects and to enable them to be distinguished from unintended objects: - UAVs - Humans indoors and outdoors - Automotive vehicles (at least outdoors) - Automated guided vehicles (e.g. in indoor factories) - Objects creating hazards on roads/railways, with a minimum size dependent on frequency All six sensing modes should be considered (i.e. TRP-TRP bistatic, TRP monostatic, TRP-UE bistatic, UE-TRP bistatic, UE-UE bistatic, UE monostatic). Frequencies from 0.5 to 52.6 GHz are the primary focus, with the assumption that the modelling approach should scale to 100 GHz. (If significant problems are identified with scaling above 52.6 GHz, the range above 52.6 GHz can be deprioritized.) For the above use cases, sensing modes and frequencies: - Identify details of the deployment scenarios corresponding to the above use cases. - Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including: - modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns; - spatial consistency.

This document summarizes the contributions and discussions on ISAC channel modelling in RAN1 #120 meeting.

- The proposals in this document are tagged and color coded respectively

  - [H] Proposal: High Priority, plan to handle it in offline and online session.

  - Proposal: Other proposals which can be handled in offline/online session if a clear consensus is observed from the companies’ inputs.

- The new/revised proposals in current round for discussion is further tagged with [FL3]. Note: Some proposals without update are still marked with early round number. If any company didn’t provide a comment yet, or prefer to update the comments, feel free to continue comment on them.

The following email thread is assigned for discussion of the study item (agenda 9.7):

[120-R19-ISAC] Email discussion on Rel-19 ISAC channel model – xiaomi (Yingyang)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

Key issues to be concluded in the RAN1 #120 meeting, from moderator’s understanding

- RCS is the most controversial issue

  - We must make agreement on the general principle (issue 4.2) in the first online session

  - Then, it is also urgent to agree on exact values for RCS model of targets

- Remaining issues on basic ISAC channel model

  - Background channel for monostatic sensing (issue 6.6) is the most critical issue, but we’d better avoiding long discussion in the offline/online session

  - All other remaining issues labelled with high priority should also be solved to start calibration

  - Need to avoid to spend much time on the tables for reference TRs (issue 5.1) in the offline session. I will go to each colleague for offline checking

- Design for target with multiple scattering points

  - At least all issues labelled with high priority

- Spatial consistency (cannot delay spatial consistency to last offline session, all high priority issues)

- Details on EO type-2

  - We will start from the polarization matrix generation from ZTE and Huawei (issue 8.1)

Proposed online proposals

Tuesday (2/18)

After Tuesday online session, the following agreements are made

Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

    - A is equal to a single value per target type

      - FFS: this allows different values for the same target type with different size, if needed

      - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

Wednesday (2/19)

After Wednesday online session, the following agreements are made

Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

Thursday (2/20)

After Thursday online session

Agreement

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

Agreement

The reference TR to generate a TRP-UE channel is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

Agreement

For mono-static, the following values of component A, B2 are agreed for UAV of small size

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.74 dB

Agreement

For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.94 dB

Working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

Agreement

When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

- For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

[H][FL3] Proposal 4.1-2

Study the following options for A*B1 of RCS for a scattering point of a target bistatic sensing:

- Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree

- Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1: derive the RCS based on monostatic RCS

    - This may or may not be Option 1

  - Region 2: derive the RCS around the angles near to the specular reflection angle

  - Region 3: RCS value including large effect of diffraction

- Option 3: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1+2:

σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2)

    - Where a is the length of a rectangle (cf R1-2501212)

    - The formula may be further updated

  - Region 3: RCS value including large effect of diffraction

- Distance dependent RCS is not considered in any incident/scattered angles

- Which option is selected/updated will be based on validation results

Agreement

To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point.

Agreement

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

Agreement

For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions.

Agreement

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

- FFS: Spatial consistency between multiple scattering points of the same target

Agreement

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

Agreement

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

Agreement

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target

  - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

    - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

  - FFS: spatial consistency of random phase when a scattering point moves

  - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

[H][FL1] Proposal 6.2-1 polarization matrix normalization

Normalization on the polarization of a direct/indirect path, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is not supported.

  - Note: for path dropping, CPM is also considered

Without normalization by: LG, SKT, Ericsson, Sony, CATT,

With normalization: ZTE, vivo, Samsung, Qualcom, OPPO,

Agreement

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Agreement

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted

[FL1] Proposal 8.2-1 LOS condition

When the EO type-2 is modelled in the target channel or in the background channel, to determine the LOS condition of the Tx-target link and target-Rx link

- If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, otherwise the link is determined as LOS condition

[H][FL1] Proposal 6.7-1 blockage model

The existing blockage model B is reused as an optional feature to model the blocking effect due to a target

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

Proposed conclusion

At least for the background channel of the target, the existing blockage model B can be used to model the blocking effect due to a target.

Proposed offline proposals

Monday (2/17)

After Monday offline session

[H][FL2] Proposal 4.1-1-rev1 bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic should be aligned with RCS for monostatic

[H][FL2] Proposal 4.2-1-rev1 RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - The following two options are supported for component A, expressed in dBsm scale. B1 is dependent on A*B1 and value of component A. Up to company to select the option used in ISAC channel generation

    - A=0 dBsm

Supported by: Ericsson, Huawei, OPPO, CMCC,

    - A is equal to a value per target type

Supported: ZTE, LGE, CATT, Apple, MTK, vivo, Sony, BUPT, Sharp, NIST, AT&T, Lenovo, Samsung, QC,

  - Component B2 follows log-normal distribution characterized by mean μB2 and variance σB22, subject to fixed relation μB2=-ln1020σB22, subject to B2 less than k*sigma

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

[H][FL2] Proposal 4.6-1 vehicle with single scattering point

- For mono-static, the RCS=A*B=A*B1*B2 for a vehicle with single scattering point is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-AVdBθ+AHdB φ,Amax

Where,

AVdBθ=-min12θ-θcenterθ3dB2, Amax,

AHdB φ=-min12φ-φcenterφ3dB2, Amax,

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90 | 33.48 | 77 | 62.20 | 19.66 | 10.91 | [25°,180°] | [45°,135°] |
| Back | 180 | 41.32 | 71 | 33.00 | 18.59 | 6.94 | [25°,180°] | [135°,225°] |
| Right | -90 | 33.48 | 77 | 62.20 | 19.43 | 10.93 | [25°,180°] | (225°,315°) |
| Front | 0 | 60.26 | 54 | 33.00 | 19.25 | 8.69 | [25°,180°] | [0°,45°] U [315°,360°] |
| Roof | / | / | 0 | 21.60 | 24.67 | 12.97 | [0°,25°) | [0°,360°] |

[H][FL2] Proposal 6.6-1 background channel for monostatic

Down-select one from the following 3 solutions on modelling background channel for monostatic within RAN1#120,

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

  - FFS the distribution and the mobility for reference points dropping

  - N is determined by validation results for each scenario, FFS value N

Supported by: ZTE, OPPO, Xiaomi, Samsung, BUPT, vivo, IDC, Lenovo, LGE, MTK, CMCC, CATT, Huawei

Solution B/C (Previously option 2)

- generate a grid with N bins, each of which associates with a delay and a direction, where a clutter locates in each bin or backscatters signal to sensing Rx in each bin, generate a direct path for each bin. The sum of the N paths generates the background channel

- The backscattered power of each path follows a distribution, depending on the physical characteristics and scenarios

  - FFS: other factors affecting backscattered power

Supported by: Ericsson, Nokia, QC, Lenovo, AT&T

Tuesday (2/18)

After Tuesday offline session

[H][FL2] Proposal 6.6-1-rev2 background channel for monostatic

Down-select one from the following 2 solutions on modelling background channel for monostatic within RAN1#120,

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop 1 reference point following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

- FFS: whether to add very low power clusters

Supported by: ZTE, OPPO, Xiaomi, Samsung, BUPT, vivo, IDC, Lenovo, LGE, MTK, CMCC, CATT, Huawei

Solution B/C (Previously option 2)

- generate a grid with N bins, each of which associates with a delay and a direction, where a clutter locates in each bin or backscatters signal to sensing Rx in each bin, generate a direct path for each bin. The sum of the N paths generates the background channel

- The backscattered power of each path follows a distribution, depending on the physical characteristics and scenarios

  - FFS: other factors affecting backscattered power

Supported by: Ericsson, Nokia, QC, AT&T

[H][FL2] Proposal 9.1-1 Cases when spatial consistency is not supported

Spatial consistency is not modelled for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the links that are generated referring to channel models with parameter values of different scenarios

  - E.g., between TRP-target link in one scenario and target-UT link in another scenario

  - E.g., between TRP-UT link in one scenario and UT-target link in another scenario

- Case 3: the two links with different LOS/NLOS condition

- Case 4: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

[H][FL2] Proposal 9.2-1-rev1

- The existing spatially consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs

[H][FL2] Proposal 9.3-1 Cases needing new spatial consistency model

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

[H][FL1] Proposal 9.3-2 New correlation types among ST/UT

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of dual-end Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | dual-end Correlated |
| Cluster powers | dual-end Correlated |
| AOA/ZOA/AOD/ZOD offset | dual-end Correlated |
| AOA/ZOA/AOD/ZOD sign | dual-end Correlated |
| Random coupling | dual-end Correlated |
| XPR | dual-end Correlated |
| Initial random phase | dual-end Correlated |
| LOS/NLOS states | dual-end Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

Wednesday (2/19)

After Wednesday offline session:

[H][FL2] Proposal 4.4-1 UAV of small size (i.e., RCS mode 1)

- For mono-static, the following values of component A, B2 are agreed for UAV of small size (option 2 for UAV size in UAV parameters table)

  - Component A: -13.40 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.02 dB

    - standard deviation: 2.93 dB

[H][FL2] Proposal 4.5-1 RCS mode 1 for human

- For mono-static, the following values of component A, B2 are agreed for human of small size  (option 2 for UAV size in UAV parameters table)

  - Component A: -1.02 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.62 dB

    - standard deviation: 3.49 dB

[H][FL2] Proposal 5.1-3 reducing options for reference TRs

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

[H][FL2] Proposal 5.1-4 reducing options for reference TRs

The reference TR to generate a TRP-UE chanenl is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

[FL2] Proposal 6.9-2 for working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

[FL2] Proposal 6.9-1-rev1 on absolute delay model Δτ

When absolute delay model Δτ is configured in the target channel, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

[H][FL1] Proposal 7.1-1 general

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and keeps unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

Thursday (2/20)

After Thursday offline session:

[H][FL3] Proposal 4.1-2 bistatic RCS

Down select or merge from the following options for A*B1 of RCS for a scattering point of a target bistatic sensing

- Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree

- Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1: derive the RCS based on monostatic RCS

    - This may or may not be Option 1

  - Region 2: derive the RCS around the angles near to the specular reflection angle

  - Region 3: RCS value including large effect of diffraction

- Option 3: for target with multiple scattering points

  - Region 1+2: formula from DOCOMO contribution

    - The formula may be further updated

  - Region 3: RCS value including large effect of diffraction

- Distance dependent RCS is not considered in any incident/scattered angles

- Which option is selected/updated will be based on validation results

[H][FL1] Proposal 7.3-1 shared/separated NLOS clusters

- To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point

[H][FL1] Proposal 7.4-1-rev1 LOS condition, pathloss

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

[H][FL2] Proposal 7.2-1-rev1 Coordinates of the scattering point(s)

- For a target with single/

- multiple scattering points, the relative 3D location of each scattering point is defined separately in the evaluation assumptions

[H][FL1] Proposal 6.1-1-rev1 Polarization of target

Solution A

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

  - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type. i.e.

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

Supported: ZTE, QC, CATT, SS, LGE, Nokia, OPPO, Xiaomi, SKT, Huawei, Apple, Ericsson (if CPM is defined in LCS), NIST

Solution B

polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target is defined per target type

- For UAV

  - Option 1

    - αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where  κsp,i is XPR ratio is randomly generated by log-normal distribution.

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

- For human, vehicle, and other target types

  - Option 3

    - αi,1,αi,2,βi,1,βi,2 are angular dependent variables generated for path i

    - Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ are initial random phase of path i

    - CPMsp,i defined in LCS

Supported: vivo, Ericsson

- Solution C

  - CPMsp,i=αi,1expjΦsp,iθθ00αi,2expjΦsp,iϕϕ

  - Φsp,iθθ, Φsp,iϕϕ are deterministic

Supported: IDC, Lenovo, vivo

[H][FL1] Proposal 6.2-1 polarization matrix normalization

- Normalization on the polarization of a direct/indirect path, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is not supported

Without normalization by: LG, SKT, Ericsson, Sony, CATT,

With normalization: ZTE, vivo, Samsung, Qualcom, OPPO,

[H][FL1] Proposal 6.3-1 on power threshold -25dB

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel, the power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reduced to [Y<-25] dB

  - Reuse -25dB: OPPO, vivo, CATT,

  - Reduced to Y<-25dB: ZTE, SKT, Apple, LG, Nokia

  - Reuse -25dB with update: LG

  - The reference power for removing cluster is the min (max. Tx-target link cluster power,  max. target-Rx link cluster power)

  - Reuse -25 to existing clusters + new low power NLOS clusters: Lenovo, Ericsson

RCS of target

Samsung: frequency-dependency of the RCS

Bistatic/monostatic RCS

Summary on company views

Bistatic RCS derived based on monostatic RCS

- LG: RCSBi≈RCSMo∙cosβ2

- Huawei: σn*cosβ2 where σn represents the maximum power of the scattering point for monostatic

- ZTE: Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2) where Gmax_RCS represents the maximum power of the scattering point for monostatic

- vivo: Based on elliptical model

- DCM: using transition formular

- No: E//

E//: Monostatic RCS modelling should follow the bistatic RCS modelling. Monostatic RCS is a special case of bistatic RCS, where incidence zenith angle and azimuth equal the scattering zenith angle and azimuth.

E//: Model near-field phenomena, such as specular reflections and shadowing, using other means than the radar cross-section.

[Moderator’s note] Monostatic RCS is a special case of bistatic RCS by setting equal incident/scattered angles. Therefore, one principle for bistatic RCS design is that it should be compatible with the current agreements on monostatic RCS, which sounds agreeable. Companies are encouraged to check if the following proposal is agreeable.

[H][FL1] Proposal 4.1-1 bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic should be aligned with early agreements on RCS for monostatic

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with the proposal |
| ZTE |  | Basically, we are fine with the proposed guidance. Because of limited time left in this Release, we suggest directly to discuss what the model itself is if this proposal cannot be agreed quickly. |
| Lenovo |  | Yes |
| LGE |  | We can agree with the first bullet. We need more clarification on the second bullet. Which early agreement is referred by the proposal? |
| Ericsson |  | Fine |
| Huawei, HiSilicon |  | What is the ‘early agreement on RCS for monostatic’ exactly it meant? Agree with ZTE to discuss the model itself is if this proposal cannot be agreed quickly. |
| Nokia, NSB |  | Agree with the 1st bullet. For the 2nd bullet, suggest wording: “RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic should be aligned with early agreements on RCS for monostatic” |
| CATT, CICTCI |  | Agree |
| NIST |  | We agree with the 1st item. We also felt confused about the 2nd bullet. What is exactly the early agreements on monostatic |
| Samsung |  | Support |
| CMCC |  | Support. We agree with FL’s assessment that monostatic RCS is a special case of bistatic RCS by setting equal incident/scattered angles. |
| vivo |  | We are fine with this proposal, but it seems not make too much progress on RCS compared to the previous agreements. Therefore, ZTE’s comment seems good. |
| Qualcomm |  | Prefer Nokia’s change |
| AT&T |  | Fine with the proposal, using Nokia’s suggested changes on bullet 2. |
| MediaTek |  | We agree with the proposals. Especially, we share the same view with the FL that the monostatic RCS is a special case of bistatic RCS case, e.g., RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic should be aligned with RCS for monostatic. |
| InterDigital | Yes |  |
| Xiaomi |  | Support. |
| DOCOMO |  | Agree, and using Nokia’s suggested changes on bullet 2. |
| Apple |  | We are fine with the proposal and Nokia’s update. |
| BUPT | Yes |  |
| Moderator | Thanks for all inputs. It seems Nokia version is better. So let’s try it. |  |

[H][FL2] Proposal 4.1-1-rev1 bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic should be aligned with RCS for monostatic

| Company | Yes/No | Comments |
| --- | --- | --- |

[Moderator’s note] Agreement after Tuesday online.

- Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

[Moderator’s note] One proposal on A*B1 for bistatic RCS discussed in the Thursday offline session.

[H][FL3] Proposal 4.1-2 bistatic RCS

Down select or merge from the following options for A*B1 of RCS for a scattering point of a target bistatic sensing

- Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree

- Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1: derive the RCS based on monostatic RCS

    - This may or may not be Option 1

  - Region 2: derive the RCS around the angles near to the specular reflection angle

  - Region 3: RCS value including large effect of diffraction

- Option 3: for target with multiple scattering points

  - Region 1+2: formula from DOCOMO contribution

σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2)

    - The formula may be further updated

  - Region 3: RCS value including large effect of diffraction

- Distance dependent RCS is not considered in any incident/scattered angles

- Which option is selected/updated will be based on validation results

[Moderator’s note] Revised in Thursday online but no agreement

- [H][FL3] Proposal 4.1-2

Study the following options for A*B1 of RCS for a scattering point of a target bistatic sensing:

- Option 1: RCSbi=RCSmo∙abscosβ2, β is bistatic angle [0, 360] degree

- Option 2: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1: derive the RCS based on monostatic RCS

    - This may or may not be Option 1

  - Region 2: derive the RCS around the angles near to the specular reflection angle

  - Region 3: RCS value including large effect of diffraction

- Option 3: bistatic RCS is constructed into 3 angular regions according to scattered angles

  - Region 1+2:

σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2)

    - Where a is the length of a rectangle (cf R1-2501212)

    - The formula may be further updated

  - Region 3: RCS value including large effect of diffraction

- Distance dependent RCS is not considered in any incident/scattered angles

- Which option is selected/updated will be based on validation results

RCS model and application in ISAC channel generation

Summary on company views

Component A

- 1m2: HW, E//

- mean RCS: Nokia, Apple, QC, ZTE, vivo, CATT, BUPT, Sony, CMCC, DCM, AT&T, EURECOM, NIST

- A is not used: E//

B1, A*B1(E//)

- σθ1,ϕ1,θ2,ϕ2=l1=0L1m1=-l1l1l2=0L2m2=-l2l2cl1,m1,l2,m2Yl1m1θ1, ϕ1Yl2m2θ2,ϕ22: E//

- similar as the antenna radiation power pattern in 38.901: HW, ZTE, BUPT, Xiaomi, QC

- a function: Nokia, SS, IDC, DCM

- order-1 or -2 polynomial: NIST

- piece-wise linear function: Apple

- Lookup table

- fixed value for A*B1: LG, AT&T

B2

- μB2=0dB: HW, BUPT(no constraint)

- μB2=-ln1020σB22: ZTE, Xiaomi, Nokia, QC, CATT, CMCC, EURECOM

A/B1/B2 in dBsm/dB scale

- NO:

DOCOMO

- σB(f,θ,θ+β)=4π λ2S2cos2(θ+β)∙sinc22πaλsin(θ+β/2)∙cos(β/2)

- σB(f,θ,θ+β)≈σM(f,θ+β/2),β≤20 degrees

OPPO: Adopt Option D in Observation 1 for modeling of RCS, in order to ensure the large-scale fading part 4πAλ2PL1∙PL2 by itself is the true reflection of total channel gain in the target channel; otherwise, 4πAλ2PL1∙PL2 should be given a terminology other than gain of “large-scale fading”, and RAN1 should clarify which channel modeling aspects would use this term, especially for the terms associated with Option B and Option C

[Moderator’s note] Based on the discussions in past RAN1 meetings, it seems hard to conclude whether component A is mean RCS value or equals to 1, for the case of angular dependent B1. One potential way forward is that we avoid splitting A and B1, i.e., A*B1 as a whole is captured in the RCS model.

As to component B2, we have an agreement that mean of linear B2 values equals 1, which is motivated by normalizing the power of small scale channel. Note: the small scale channel in existing TR 38.901 is normalized. Having linear mean equals 1 for component B2, companies could derive a mean RCS value in large scaling only from the merged A*B1.

Cited from agreement in RAN1 #118bis - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

On the other hand, the application of RCS component A in the generation of large scale scaling factor of ISAC channel model is still supported as compromise. Given it is hard to agree on an exact definition for component A, the exact way to determine value of component A can be left to company implementation.

Companies are encouraged to check if the following compromise proposal is agreeable. It is expected that any company may be happy with a part of the proposal, but not comfortable with other part. However, it is appreciated if colleagues can be flexible, otherwise, we may not be able to complete the study item on time. Thanks

[H][FL1] Proposal 4.2-1 RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target

  - The values/pattern of the product of component A and B1, i.e., A*B1 will be given per target type, expressed in dBsm scale

    - Up to company implementation to decide how to determine component A and B1 in the generation of ISAC target channel

  - Component B2 follows log-normal distribution characterized by mean μB2 and variance σB22, subject to fixed relation μB2=-ln1020σB22

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLdBd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with this proposal |
| ZTE | Yes without the first subbullet of the first bullet | We don’t understand why A cannot be concluded as mean value with the following two reasons: - A as mean value had been agreed for a part of cases of UAV and Human - A as mean value is supported by super majority The first sub-bullet of the first bullet reverts previous agreement for UAV and Human case. We prefer delete the following in the proposal: - The values/pattern of the product of component A and B1, i.e., A*B1 will be given per target type, expressed in dBsm scale - Up to company implementation to decide how to determine component A and B1 in the generation of ISAC target channel |
| Lenovo |  | The equivalence μB2=-ln1020σB22 holds for an untruncated log-normal distribution. However, we have agreed that the RCS is bounded from above, which contradicts the classic log-normal assumptions. |
| LGE |  | From Proposal 4.1-1, we’re trying to agree option 3 from the previous agreement. So, we need at least the mean value of B2 should be 1. Given above, A*B1 should represent the mean value of RCS as RCS=A*B1*B2. A*B1 as a whole can be a mean of RCS, or A alone can be a mean. We prefer A as the mean of RCS, which is aligned with ZTE. |
| Ericsson |  | Support. Note that step 14 in section 7.9.4.1 of the draft CR needs to be renamed to reflect the proposed term “power scaling factor”. |
| Huawei, HiSilicon |  | Fine. |
| Nokia, NSB |  | The proposed term “power scaling factor” is quite vague. Previous proposal has an adjective “large scale” before this term. Do we need to differentiate “large scale” and “small scale” fading in ISAC channel modeling? |
| CATT, CICTCI |  | By natural, three must be a mean value of RCS used for large scale estimation, which we think could be A. We have the same understanding as ZTE. For progress we are fine with ‘A*B1 will be given per target type, expressed in dBsm scale’; but how to derive a mean value of ‘A*B1’ for the purpose of large scale study should better be aligned. |
| NIST |  | As we agreed before, we prefer A, B1, and B2 to present mean RCS, angle-dependent RCS variations (fitted using a function), and RCS residual, respectively. Our observation is that A itself is useful in some cases, for example, for system designers who want to estimate the sensing link budget quickly based on SNR. However, we are willing to further discuss to support progress. |
| Samsung |  | We have concern on first sub- sub-bullet. If it is remained as a companies’ selection, the calibration seems to be difficult. We would like to define a concrete method for it. |
| CMCC |  | From channel measurement results, it may be hard to obtain a mean value of A from the whole part of A*B1, so we prefer to define A*B1 as a whole component for each target. While some companies suggest defining A and B1 as separate components, so FL’s suggestion that up to company implementation to decide how to determine component A and B1 is a compromised way to move forward. Additionally, for large scale calibration, the value of RCS component A should be aligned among companies. |
| vivo |  | Besides the reasons pointed by ZTE, how we can ensure the consistency in calibration if the values/pattern become the product of component A and B1? |
| Qualcomm |  | We also have a concern on what it is really proposed above. It seems there is an attempt to say that “A” is not the mean, but it is a power scaling, etc. Tend to agree with ZTE and NIST’s comments. |
| AT&T |  | Agree with Qualcomm’s and other companies comments. We need to be more precise in the definition of A. |
| MediaTek |  | For the first bullet, we share the similar view with ZTE, and we also have agreed the definition of mean RCS in previous meeting that “ Mean RCS value is defined as the mean value of the distribution of RCS”. For the second bullet, more clarification is needed. Since the wording of large scale has been deleted in the proposal, how to understand the calibration method of large scale calibration as discussed in AI 9.7.1? |
| Xiaomi |  | Support in principle. Also fine with the comments from ZTE. |
| DOCOMO |  | We agree with the comments of other companies regarding the refinement of the definition of A. |
| Apple |  | We agree with NIST’s comments i.e. prefer for A to represent the mean RCS value. Also, if we do not define and explain A explicitly, then how do we agree on the second bullet which depends on A ? |
| BUPT |  | In the agreement reached at the previous meeting, the modeling method using the RCS value of A * B1 * B2 has been approved. We still prefer to prioritize modeling in this way. To make the mean of B2 equal to 1 in the linear scale, we consider two ways to fit component B2. The first method is to fit the σRCS-AdB-B1dBpart directly according to the constraint condition μB2=-ln1020σB22. The second method is to fit B2dB~N(μB2, σB22) without constraints, and then adjust by subtracting a constant c from μB2, so that μB2'=μB2-c=-ln1020σB22. In this way, the RCS framework can be expressed as σRCS=AdB+B1dB+B2,newdB+c, - wherein, B2,newdB follows the normal distribution N(μB2-c,σB22 ). |
| Moderator | Thanks for all comments and offline discussions for potential way forward. The proposal is updated with following key notes - No intention to revert existing agreement. Therefore, I add “when the target type is vehicle, large size UAV, human with RCS model 2,” in first bullet - Two options for value A are proposed, i.e. A=0 and A is target dependent (likely to be non-zero). Up to company for the selection |  |

[H][FL2] Proposal 4.2-1-rev1 RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2,

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - The following two options are supported for component A, expressed in dBsm scale. Up to company to select the option(s) used in ISAC channel generation

    - A=0 dBsm

    - A is equal to a value per target type

  - Component B2 follows log-normal distribution characterized by mean μB2 and variance σB22, subject to fixed relation μB2=-ln1020σB22

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-ST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and target, where d1 is the distance between Tx and target

    - PLdBd2 is pathloss between Rx and target, where d2 is the distance between target and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx-target link and target-Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

[Moderator’s note] After Monday offline session, the updated version is as below, together with counting on supporting companies. Further offline comments on B2 is received too.

[H][FL2] Proposal 4.2-1-rev2 RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - The following two options are supported for component A, expressed in dBsm scale. B1 is dependent on A*B1 and value of component A. Up to company to select the option used in ISAC channel generation

    - A=0 dBsm

Supported by: Ericsson, Huawei, OPPO, CMCC,

    - A is equal to a value per target type

Supported: ZTE, LGE, CATT, Apple, MTK, vivo, Sony, BUPT, Sharp, NIST, AT&T, Lenovo, Samsung, QC,

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfies a fixed relation μB2_dB=-ln1020σB2_dB2, subject to B2 less than k*sigma

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

[Moderator’s note] Further updated based on comments from Qualcomm. The revision should be quite acceptable since it only clarify the application of B1/B2, which is in step 10 in the draft CR.

[H][FL2] Proposal 4.2-1-rev3 RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - The following two options are supported for component A, expressed in dBsm scale. B1 is dependent on A*B1 and value of component A. Up to company to select the option used in ISAC channel generation

    - A=0 dBsm

Supported by: Ericsson, Huawei, OPPO, CMCC,

    - A is equal to a value per target type

Supported: ZTE, LGE, CATT, Apple, MTK, vivo, Sony, BUPT, Sharp, NIST, AT&T, Lenovo, Samsung, QC,

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2, subject to B2 less than k*sigma

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

| Company | Yes/No | Comments |
| --- | --- | --- |

[Moderator’s note] Agreements after Tuesday online

- Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

    - A is equal to a single value per target type

      - FFS: this allows different values for the same target type with different size, if needed

      - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Correlation of RCS

Summary on company views

RCS B2 in time domain

spatial-temporal consistency of RCS B2: E//, NIST, Lenovo, Pansonic

E//: Model the stochastic B2 with random scattering centres

RCS as a random time-domain process: LGE

Sony: Consider correlation angle θcorr [º] on B1

The stochastic component B2​ in the RCS calculation can be modelled as a spatially and temporally correlated process.: NIST

Figure 2.1-6: Small-Scale RCS (B2) autocorrelation function as a function of aspect angle lag.

NIST: This correlation is achieved by passing independently and identically distributed log-normal random variables through a filter with an impulse response shaped by an exponential ACF.

[Moderator’s note] The existing validation results show that the RCS values fluctuate a lot with the change of incident/scattered angles. However, the RCS value of close/adjacent incident/scattered angles can still have some correlations as discussed in some contributions. On the other hand, some companies comment that the angle dependent pattern of B1 (or A*B1) is already a means to model correlation of adjacent incident/scattered angles.

- [FL1] Question 4.3-1

- Companies are encouraged to comment on whether/how to model correlation of RCS of a scattering point in adjacent incident/scattered angles?

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | We don’t observe any correlation from our results. For example, for a human, RCS values on nose and eye could be quite different even the corresponding angles are close to each other. |
| Lenovo |  | Suggest to use the correlation distance as defined in 38.901, with value fitted from measurements |
| LGE |  | At least for vehicle RCS, we think that the angular dependent modeling of RCS is needed, where some correlation is needed across the adjacent angles. |
| Ericsson |  | It is necessary to let B2 be continuous over the angles to ensure a continuous channel when the target moves or rotates. The problem occurs when using different stochastic values for B2 for different incident/outgoing rays. Instead of a complicated correlation procedure we propose to ensure continuity by representing B2 as a sum of sinusoids, where each sinusoid is determined from a scattering center or scattering point of the target. This will naturally lead to rapid but continuous variations over angle of B2. |
| Huawei, HiSilicon |  | In our view, the B1 has already modeled the correlation of the RCS related to the angles. We have to point out the B2 is the standard deviation between the measurement data and the fitted data(A*B1), which should not be modelled with angular dependency. |
| Nokia, NSB | No | We agree that the correlation among angles exists. But modeling this aspect should not be the 1st priority for ISAC channel modeling, given that we don’t have much time. |
| CATT, CICTCI | N | Correlation of RCS in adjacent incident/scattered angles should be reflected in B1, not B2. |
| NIST |  | In our measurements, we observed some local RC fluctuations that cannot be completely captured by the RCS angle-dependent fitting function. The angle-dependent fitting mainly focuses on a more large-scale RCS trend. |
| Samsung |  | There may or may not be a correlation between adjacent incident and scattered angles in RCS. For very large objects, the shape and size variations are not significant between adjacent angles, which could result in correlation. On the other hand, smaller objects may not exhibit such correlation. Moreover, the current RCS modeling approach in RAN1 is based on stochastic characteristics derived from companies' measurement and ray-tracing results. Introducing angular correlation in this context would not align with the philosophy of defining RCS as a stochastic characteristic. Therefore, we believe that this discussion should be deprioritized. |
| vivo |  | This correlation may not be needed because the variation between adjacent angles occurs in any experiment measurement with a white behavior. |
| MediaTek |  | Since the B1 modelling is angle dependent, we think it also has reflected the correlation of RCS of a scattering point in adjacent incident/scattered angles. |
| Xiaomi |  | Do not support. The correlation heavily depends on the target surface, and it may even be completely uncorrelated in some cases. Our RT simulation results also exhibit that large fluctuation occurs between adjacent angle, e.g., human monostatic RCS with 1° simulation step. |
| Apple |  | Although from our measurement and electromagnetic simulations, there is quite a lot of fluctuation in the value of the RCS across angle, we think this is a lower priority issue. |
| BUPT | No | This will lead to a more complex RCS modeling, which should be a low-priority issue. |
| Ericsson2 |  | It is essential that the radar cross-section, i.e B2 and the CPM, is spatially consistent, in the sense that it gives correlated values that change continuously when the angles are changed gradually with movement or rotation of the target. A lack of spatial consistency (correlation) will lead to unphysical behaviour (that does not conform to the laws of physics) with discontinuous RCS over time that will create very high Doppler frequencies and a nonlinear channel. A non-linear channel will among other things result in out of band emissions, which is clearly a highly undesirable outcome of a channel model. If companies don’t observe any continuity in B2 it is clear evidence that that their measurements are done with too coarse angular sampling. While it is common understanding that B1 captures slower variations of the RCS over angle, the variations captured by B2 are fast, but not infinitely fast. |

RCS model for UAV

Summary on company views

Small size UAV

- A=1: HW

- Results submitted: HW, QC, ZTE

Large size UAV

- p1β2+p2β+p3: NIST

- Results submitted: Samsung, BUPT

[Moderator’s note] It is urgent to collect values of RCS model for calibration after Feb. meeting. Let’s try the following way to collect RCS values from companies before a further guideline can be agreed.

- Up to each company to decide whether to separate report of A (dBsm) and B1 (dB), or report A*B1 (dBsm) as a whole

- Mean/std of B2 in dB scale are to be reported

- If A*B1 is finally agreed, the moderator will adding the inputted A and B1 to get values/pattern of A*B1

This note commonly apply to all remaining questions on collection of RCS values for all targets

- [FL1] Question 4.4-1 UAV of small size (i.e., RCS mode 1)

- Please provide your input on monostatic RCS model for UAV of small size

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| ZTE | -12.74 | B1=1 has been the agreement. We don’t agree to revert previous agreement | -1.45 | 3.6 |
| LGE | -10 | B1=0 dB (per agreement) | -0.85 dBsm | We only need μ as σ can be calculated from μ: σ2=-20ln10μ |
| CATT, CICTCI | -12.4 |  | -1.1064 | 3.1 |
| Samsung | -13.02 (if consider only one zenith angle (perpendicular with target)) | If we follow the previous agreement for small UAV, B1 = 1 | -1.25 | 3.3 |
| BUPT | Asmall UAV= 0.2133*frequency-22.04 (typical 15 GHz: -18.8405 dB) | B1=0 dB | If there is no relationship between μ and σ μ=-0.4605 if μ as σ is calculated from : μB2=-ln1020σB22 μ= -0.4331 | If there is no relationship between μ and σ σ=1.9467 if μ as σ is calculated from : μB2=-ln1020σB22 σ=1.9396 |

- Please provide your input on bistatic RCS model for UAV of small size

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| BUPT | Asmall UAV= 0.2133*frequency-22.04 (typical 15 GHz: -18.8405 dB ) | B1=0 dB | If there is no relationship between μ and σ μ=-0.4605 if μ as σ is calculated from : μB2=-ln1020σB22 μ= -0.4331 | If there is no relationship between μ and σ σ=1.9467 if μ as σ is calculated from : μB2=-ln1020σB22 σ=1.9396 |
| Ericsson2 |  | A*B1=spherical harmonics expansion, where c_{0,0,0,0} = 0.14 (-17dBsm in power scale) and c_{m1,l1,m2,l2} = 0 for all other m1, l1, m2, l2 | Use 5 scattering centres randomly placed in a volume of 2dm² each with a scattering gain of 1/5. |  |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |
| ZTE | We prefer not to revert agreement. For small size UAV, B1=1, A=mean value have been agreed. |
| vivo | A*B1 (dBsm) is not yet agreed, and we believe that, even proponents provide such a measurement result, the results will not be captured in TR. |
| BUPT | For the B2 component, we provide the fitting results without constraints and the results fitted according to the constraints. Which value will be adopted depends on the subsequent discussion. |

[Moderator’s note] Based on inputs, merged values of RCS model component A, B2 for UAV of small size UAV are obtained by averaging the values from all companies.

[H][FL2] Proposal 4.4-1 UAV of small size (i.e., RCS mode 1)

- For mono-static, the following values of component A, B2 are agreed for UAV of small size (option 2 for UAV size in UAV parameters table)

  - Component A: -13.40 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.02 dB

    - standard deviation: 2.93 dB

[Moderator’s note] Updated results with inputs from QC and HW

[H][FL2] Proposal 4.4-1-rev1 UAV of small size (i.e., RCS mode 1)

- For mono-static, the following values of component A, B2 are agreed for UAV of small size (option 2 for UAV size in UAV parameters table)

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.61 dB

    - standard deviation: 3.74 dB

[Moderator’s note] Agreement at Thursday online session

- Agreement

For mono-static, the following values of component A, B2 are agreed for UAV of small size

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.74 dB

- [FL1] Question 4.4-2 UAV of large size (i.e., RCS mode 2)

- Please provide your input on monostatic RCS model for UAV of large size

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| LGE |  | -6 for A*B1 (dBsm) | -1.5 dBsm | σ from μ |
| BUPT | AUAV = 0.3397*frequency-10.16 (typical 15 GHz: AUAV=-5.0645 dB) | Horizontal: B1dBθ=θn,φ=Gmax-min12φ-φkφ3dB2,Ymax Vertical: B1dBθ,φ=φn=Gmax-min12θ-θkθ3dB,k2,Ymax 3D: B1dBθ,φ=Gmax-min⁡{-B1dBθ=θn,φ+B1dBθ,φ=φn,Ymax} | If there is no relationship between μ and σ μ=-0.52 if μ as σ is calculated from : μB2=-ln1020σB22 μ=-0.5969 | If there is no relationship between μ and σ σ=2.3118 if μ as σ is calculated from : μB2=-ln1020σB22 σ=2.277 |

- Please provide your input on bistatic RCS model for UAV of large size

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| NIST | -12.85 | 2nd order polynomial | -2.04 | 4.63 |
| BUPT | -2.7436 | B1dBθ=θn,φ=Gmax-min12φ-φrφ3dB2,Ymax | If there is no relationship between μ and σ 10°:-1.7791 20°:-3.5432 30°:-2.3596 40°:-1.1319 50°:-2.2312 60°:0.7858 if μ as σ is calculated from : μB2=-ln1020σB22 10°:-1.5187 20°:-1.8240 30°:-1.8598 40°:-0.4694 50°:-0.7150 60°:-0.8533 | If there is no relationship between μ and σ 10°:3.5697 20°:2.6142 30°:3.8097 40°:1.7655 50°:1.3421 60°:2.7881 if μ as σ is calculated from : μB2=-ln1020σB22 10°:3.6319 20°:3.9803 30°:4.0192 40°:2.0192 50°:2.4920 60°:2.7224 |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |
| NIST | We are computing B2 mean and standard deviation in dB using the mean (dB)= 10*μ/ln(10) and standard deviation (dB) = 10*σ/ln(10), where μ and σ values are computed using the log-normal fitting. |
| BUPT | The RCS distribution of large UAV is directional and can be analyzed using modeling method similar to those of vehicles." |

RCS model for human

Summary on company views

On B1 or A*B1

- Alt 1: formulated similar as the antenna radiation power pattern in 38.901

Supported by: HW(only vertical)

- Alt 2: a function

- Alt 3: Lookup table

- 2nd order polynomial: OPPO

Results for RCS model 1 submitted: Apple, ZTE, AT&T

Results for RCS model 2 submitted: HW, OPPO, QC

- B1 dependent on evaluation: QC

- [FL1] Question 4.5-1 RCS model 1 for human

- Please provide your input on monostatic RCS model 1 for human

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| ZTE | -4 | B1=1 has been the agreement. We don’t agree to revert previous agreement | -1.96 | 4.13 |
| LGE | Adult: -3 Child: -6 | B1=0 dB (per agreement) | -1.2 dBsm | σ from μ |
| CATT, CICTCI | 3.6 | 0 dB | -0.6631 | 2.4 |
| vivo |  | Proponents can provide report A*B1 as a whole, that will not be captured in TR if without the agreement on Proposal 4.2-1. In addition, we have the measurement results, showing for σB1 = 0dB. |  |  |
| Xiaomi | 3.65 | B1=1 in linear scale | -3.47 | 5.49 |
| Apple | -2.1 (ray tracing) -3.1 (measurement) | 0 dB | 0 dB | 4.07 (ray tracing) 4.7 (measurements) |
| BUPT | Ahuman = 0.16*frequency-4.68 (typical 15 GHz: Ahuman=-2.28 dB) | B1=0 dB | If there is no relationship between μ and σ μ=-0.1737 if μ as σ is calculated from : μB2=-ln1020σB22 μ=-0.1903 | If there is no relationship between μ and σ σ=1.2964 if μ as σ is calculated from : μB2=-ln1020σB22 σ=1.2856 |
| NIST | -3.48 |  | -2.78 dB | 5.56 dB |

- Please provide your input on bistatic RCS model for human, corresponding to RCS model 1 for human for monostatic

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |

[Moderator’s note] Based on inputs, merged values of RCS model component A, B2 for RCS model 1 of human are obtained by averaging the values from all companies.

[H][FL2] Proposal 4.5-1 RCS mode 1 for human

- For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.02 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.62 dB

    - standard deviation: 3.49 dB

[Moderator’s note] Updated results with inputs from NIST

[H][FL2] Proposal 4.5-1-rev1 RCS mode 1 for human

- For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - Mean: -1.78 dB

    - standard deviation: 3.94 dB

[Moderator’s note] Agreement at Thursday online session

- Agreement

For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.94 dB

- [FL1] Question 4.5-2 RCS model 2 for human

- Please provide your input on monostatic RCS model 2 for human

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| Apple | -3.1 | Front: +6.6dB Side : -0.1 dB | 0 dB | 4.07 |
| BUPT | Ahuman = 0.2417*frequency-7.133 (typical 15 GHz: Ahuman=-3.5075 dB) | B1=0 dB | If there is no relationship between μ and σ μ=-0.1737 if μ as σ is calculated from : μB2=-ln1020σB22 μ=-0.1903 | If there is no relationship between μ and σ σ=1.2964 if μ as σ is calculated from : μB2=-ln1020σB22 σ=1.2856 |

- Please provide your input on bistatic RCS model for human, corresponding to RCS model 2 for human for monostatic

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| Xiaomi |  | polynomial function | -5.02 | 6.60 |
| DOCOMO | -9.4 | B1=0 dB | 0 | 4.2 |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |
| BUPT | We consider that Model 1 is a special case of Model 2. |

RCS model for vehicle

Summary on company views

Component A

- Mean RCS: QC, AT&T

Pattern for B1 or A*B1

- B1=1: AT&T

- Alt 1: formulated similar as the antenna radiation power pattern in 38.901

Supported by: HW, ZTE, Xiaomi, QC

- Alt 2: a function

- Alt 3: Lookup table

- B1θ,β,ϕ=p1β+p2,     0≤modθ+Δθ-θβ,ϕ,180≤90,p3β+p4,   90<modθ+Δθ-θβ,ϕ,180≤180,: NIST

For each of multiple scattering points

- Distinct coordinates: HW

- RCS in range θ^″∈[0°",180" °] and φ^″∈["-180" °",180" °]: HW

Results for single scattering point submitted: ZTE

Results for multiple scattering points submitted: HW, QC

- [FL1] Question 4.6-1 vehicle with single scattering point

- Please provide your input on monostatic RCS model for vehicle with single scattering point

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| Huawei, HiSilicon |  | Towards progress further, based on the available RCS values for vehicle in terms of tabulated pattern (Huawei, ZTE, Xiaomi, BUPT), we are trying to nail down the exact values for vehicle with single point and multiple points at the same time with the basic principle that the RCS value for a single point in general should be the sum of values from each scattering point. With that, the vehicle with single point is modelled as follows: The mono-static RCS model for vehicle with single point Parameter; Values. Vertical cut of the scattering power pattern for each scattering point (dB); σ″dBθ″, φ″=0°=-min12θ″-θcenterθ3dB,n2,σmax,. Horizontal cut of the scattering power pattern for each scattering point (dB); σ″dBθ″=90°, φ″=-min12φ″-φcenterφ3dB,n2,σmax,. 3D scattering power pattern(dB) for each scattering point; σ″dBθ″,φ″=Gmax-min-σ″dBθ″, φ″=0°+σ″dBθ″=90°, φ″,σmax. The RCS value for the mono-static vehicle RCS model with single point φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Left; 90; 33.48; 77; 62.20; 19.66; 10.91; [25°,180°]; [45°,135°]. Back; 180; 41.32; 71; 33.00; 18.59; 6.94; [25°,180°]; [135°,225°]. Right; -90; 33.48; 77; 62.20; 19.43; 10.93; [25°,180°]; (225°,315°). Front; 0; 60.26; 54; 33.00; 19.25; 8.69; [25°,180°]; [0°,45°] U [315°,360°]. Roof; /; /; 0; 21.60; 24.67; 12.97; [0°,25°); [0°,360°]. |  |  |
| ZTE | A= 10.69 | For mono-static, the RCS=A*B=A*B1*B2 of a scattering point for a passenger vehicle Type 1/2 can be generated by - B1 is deterministic based on incident/scattered angles B1dB = rcsdB(θ,φ)-A. Where, rcsdB(θ,φ)=Gmax-min-AVdBθ+AHdB φ,Amax Where, AVdBθ=-min12θ-θcenterθ3dB2, Amax, AHdB φ=-min12φ-φcenterφ3dB2, Amax, For the detailed values, besides our contribution, we are also fine with the following merged values with Huawei, xiaomi, etc.: φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Left; 90; 33.48; 77; 62.20; 19.66; 10.91; [25°,180°]; [45°,135°]. Back; 180; 41.32; 71; 33.00; 18.59; 6.94; [25°,180°]; [135°,225°]. Right; -90; 33.48; 77; 62.20; 19.43; 10.93; [25°,180°]; (225°,315°). Front; 0; 60.26; 54; 33.00; 19.25; 8.69; [25°,180°]; [0°,45°] U [315°,360°]. Roof; /; /; 0; 21.60; 24.67; 12.97; [0°,25°); [0°,360°]. | -1.24 | 3.28 |
| Xiaomi | 20.05 | Also fine with the parameters merged by Huawei (based on the value provided by Huawei, ZTE, Xiaomi, and BUPT) besides our contribution. As for value A, we are open to discuss whether A is separately captured. | -4.02 | 5.91 |
| BUPT | Avehicle =0.1889*frequency + 5.971 (typical 15 GHz: Avehicle=8.806 dB) | Horizontal: B1dBθ=θn,φ=-min12φ-φkφ3dB2,Ymax Vertical: B1dBθ,φ=φn=-min12θ-θkθ3dB,k2,Ymax 3D: B1dBθ,φ=Gmax-min⁡{-B1dBθ=θn,φ+B1dBθ,φ=φn,Ymax} | If there is no relationship between μ and σ μ=-0.5317 if μ as σ is calculated from : μB2=-ln1020σB22 μ=-0.7561 | If there is no relationship between μ and σ σ=2.6365 if μ as σ is calculated from : μB2=-ln1020σB22 σ=2.5627 |
| NIST | 9.98/13.36/ 20.88 for different sizes | 1st order polynomial |  |  |

- Please provide your input on bistatic RCS model for vehicle with single scattering point

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| NIST | 9.98/13.36/20.88 for different sizes | 1st order polynomial | 1.6/1.56/ 1.6 | 7.12/7.73/ 8.77 |
| BUPT | Horizontal Left:11.4146 Head:11.8391 Back:11.2909 Vertical Left:14.5232 Head:12.7879 Back:12.2971 | RCSdB(θ=θn,φ)=Gmax-min12φ-φrφ3dB2,Ymax. RCSdBθ,φ=φn=Gmax-min12θ-θrθ3dB2,Ymax. Horizontal Vertical | If there is no relationship between μ and σ Horizontal_left: -0.6001 Horizontal_head: 0.6062 Horizontal_back: -0.0834 Veritical_left: -4.8360 Veritical _head: -1.7974 Veritical _back: -3.1407 if μ as σ is calculated from : μB2=-ln1020σB22 Horizontal_left: -4.0444 Horizontal_head: -4.3155 Horizontal_back: -3.5221 Veritical_left: -6.4043 Veritical _head: -5.6016 Veritical _back: -4.2176 | If there is no relationship between μ and σ Horizontal_left: 7.1598 Horizontal_head: 7.4765 Horizontal_back: 6.5663 Veritical_left: 8.6062 Veritical _head: 8.8122 Veritical _back: 6.7118 if μ as σ is calculated from : μB2=-ln1020σB22 Horizontal_left: 5.9270 Horizontal_head: 6.1224 Horizontal_back: 5.5311 Veritical_left: 7.4583 Veritical _head: 6.9753 Veritical _back: 6.0526 |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |
| NIST | Three values are the results from small-size, mid-size, and large-size vehicles (sedan, SUV, and Van) |

[Moderator’s note] Thanks for offline work from Huawei, ZTE and Xiaomi, we have a common RCS angle dependent pattern from 3 companies now, which is based “Alt 1: formulated similar as the antenna radiation power pattern in 38.901” in the agreement from RAN1 #119. Some other companies are supportive to using antenna radiation pattern too. Therefore, the moderator propose the following proposal based on the majority view.

[H][FL2] Proposal 4.6-1 vehicle with single scattering point

- For mono-static, the RCS=A*B=A*B1*B2 for a vehicle with single scattering point is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-AVdBθ+AHdB φ,Amax

Where,

AVdBθ=-min12θ-θcenterθ3dB2, Amax,

AHdB φ=-min12φ-φcenterφ3dB2, Amax,

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90 | 33.48 | 77 | 62.20 | 19.66 | 10.91 | [25°,180°] | [45°,135°] |
| Back | 180 | 41.32 | 71 | 33.00 | 18.59 | 6.94 | [25°,180°] | [135°,225°] |
| Right | -90 | 33.48 | 77 | 62.20 | 19.43 | 10.93 | [25°,180°] | (225°,315°) |
| Front | 0 | 60.26 | 54 | 33.00 | 19.25 | 8.69 | [25°,180°] | [0°,45°] U [315°,360°] |
| Roof | / | / | 0 | 21.60 | 24.67 | 12.97 | [0°,25°) | [0°,360°] |

[Moderator’s note] After Monday offline session, one comment received from Christina that it is confusing to use many parameters with style of AXY  and RCS component A. So, the moderator updates the proposal by replacing ‘AXY’ with ‘σXY ’.

[H][FL2] Proposal 4.6-1-rev1 vehicle with single scattering point

- For mono-static, the RCS=A*B=A*B1*B2 for a vehicle with single scattering point is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | 90 | 33.48 | 77 | 62.20 | 19.66 | 10.91 | [25°,180°] | [45°,135°] |
| Back | 180 | 41.32 | 71 | 33.00 | 18.59 | 6.94 | [25°,180°] | [135°,225°] |
| Right | -90 | 33.48 | 77 | 62.20 | 19.43 | 10.93 | [25°,180°] | (225°,315°) |
| Front | 0 | 60.26 | 54 | 33.00 | 19.25 | 8.69 | [25°,180°] | [0°,45°] U [315°,360°] |
| Roof | / | / | 0 | 21.60 | 24.67 | 12.97 | [0°,25°) | [0°,360°] |

| Company | Yes/No | Comments |
| --- | --- | --- |
| BUPT |  | - We agree that this RCS modeling approach and tabular format for single scattering point are reasonable. However, regarding the specific data in this table, we believe that some additional offline time is needed to verify its correctness. - We recommend removing the specific numerical values from this table. Additionally, our measured results indicate that the A value shows strong frequency dependence, we think frequency should be considered here or discussed in the context of target size. |
| Moderator | Based on further offline, some more discussions are necessary to agree on the exact values. After the deletion of the values, the same framework of A*B1 applies to vehicle with single and multiple scattering points. |  |

[H][FL2] Proposal 4.6-1-rev2 vehicle with single/multiple scattering points

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

[Moderator’s note] Agreement after Wednesday online

- Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

- [FL1] Question 4.6-2 vehicle with multiple scattering points

- Please provide your input on monostatic RCS model for vehicle with multiple scattering points

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| Huawei, HiSilicon |  | Towards progress further, based on the available RCS values for vehicle in terms of tabulated pattern (Huawei, ZTE, Xiaomi, BUPT), we are trying to nail down the exact values for vehicle with single point and multiple points at the same time with the basic principle that the RCS value for a single point in general should be the sum of values from each scattering point. With that, the vehicle with single point is modelled as follows: The mono-static RCS model for vehicle with multiple points Parameter; Values. Vertical cut of the scattering power pattern for each scattering point (dB); σ″dBθ″, φ″=0°=-min12θ″-θcenterθ3dB,n2,σmax,. Horizontal cut of the scattering power pattern for each scattering point (dB); σ″dBθ″=90°, φ″=-min12φ″-φcenterφ3dB,n2,σmax,. 3D scattering power pattern(dB) for each scattering point; σ″dBθ″,φ″=Gmax-min-σ″dBθ″, φ″=0°+σ″dBθ″=90°, φ″,σmax. The RCS value for the mono-static vehicle RCS model with multiple points φcenter; φ3dB, n; θcenter; θ3dB,n; Gmax; σmax; Applicable Range of θ; Applicable Range of φ. Left; 90; 33.48; 77; 62.20; 19.41; 20.00; [0°,180°]; [-180°,180°]. Back; 180; 41.32; 71; 33.00; 18.26; 20.00; [0°,180°]; [-180°,180°]. Right; -90; 33.48; 77; 62.20; 19.17; 20.00; [0°,180°]; [-180°,180°]. Front; 0; 60.26; 54; 33.00; 18.98; 20.00; [0°,180°]; [-180°,180°]. Roof; /; /; 0; 21.60; 24.65; 20.00; [0°,180°]; [-180°,180°]. |  |  |
| ZTE | 10.69 | Please see the table listed in Huawei’s comment. B1dB = rcsdB(θ,φ)-A. | -1.24 | 3.28 |
| BUPT | Avehicle =0.1889*frequency + 5.971 (typical 15 GHz: Avehicle=8.806 dB) | Horizontal: B1dBθ=θn,φ=-min12φ-φkφ3dB2,Ymax Vertical: B1dBθ,φ=φn=-min12θ-θkθ3dB,k2,Ymax 3D: B1dBθ,φ=Gmax-min⁡{-B1dBθ=θn,φ+B1dBθ,φ=φn,Ymax} | If there is no relationship between μ and σ μ=-0.5317 if μ as σ is calculated from : μB2=-ln1020σB22 μ=-0.7561 | If there is no relationship between μ and σ σ=2.6365 if μ as σ is calculated from : μB2=-ln1020σB22 σ=2.5627 |

- Please provide your input on bistatic RCS model for vehicle with multiple scattering points

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| ZTE | 10.69 | - B1 is deterministic based on incident/scattered angles B1dB = rcsdB(θi,φi,θs,φs)-6.61. Where, rcsdB(θi,φi,θs,φs)=Greflection-min-AVdBθs+AHdB φs,Amax and, AVdBθs=-min12θs-θreflect(θi)θ3dB2, Amax, AHdB φs=-min12φs-φreflect(φi)φ3dB2, Amax, Greflection= Gmax_RCS*cos((φi-φreflect)/2)*cos((θi-θreflect)/2), | -1.24 | 3.28 |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |

RCS model for AGV

Summary on company views

On B1 or A*B1

- Alt 1: formulated similar as the antenna radiation power pattern in 38.901

Supported by: HW

- Alt 2: a function

- Alt 3: Lookup table

- p1α+p2: NIST

AGV has 4 scattering points, i.e., front, left, right, top: HW

For each of multiple scattering points

- Distinct coordinates: HW

- RCS in range θ^″∈[0°",180" °] and φ^″∈["-180" °",180" °]: HW

Results for single scattering point submitted:

Results for multiple scattering points submitted: HW

- [FL1] Question 4.7-1 AGV

- Please provide your input on monostatic RCS model for AGV

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| LGE |  | A*B1 (dBsm) 3 dBsm for AGV option 1 9 dBsm for AGV option 2 | -1.5 dBsm | σ from μ |
| NIST | 8.54 | 1st order polynomial | -3.21 | 4.90 |

- Please provide your input on bistatic RCS model for AGV

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- |
|  |  |  | μ | σ |
| NIST | 3.94 | 1st order polynomial | -3.21 | 4.90 |

| Company | Comments |
| --- | --- |
| Moderator | If any additional comments, please provide it in following table |

RCS model for other targets

Summary on company views

E//: Study RCS of more specific types of intended and unintended targets and use the list of objects in Table 1 as a starting point

| Physical characteristics (e.g., size) | Cyclist: 1.7m x 0.5m x 1.5m E-Scooter: 1m x 0.5m x 1.9m Motorcyclist: 2m x 0.8m x 1.5m Train: 24m x 3.5m x 3 m Bird Option 1: similar sizes as UAV Option 1 Bird Option 2: similar sizes as UAV Option 2 Tree: 5m x 5m x 15 m Other ground clutter: FFS |
| --- | --- |

NIST:

The RCS model parameters for robotic arm in dB scale across different bistatic angles (α) are summarized below. In particular, the mean RCS (A) value for the quasi-monostatic configuration (α  = 9°) was recorded as -5 dBsm.

Table 2.3-2. RCS model parameter values for robotic arm

| Company | polarization | frequency (GHz) | A (dBsm) | B1 (dB) | B2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | mean (dB) | standard deviation (dB) |
| NIST | VV | 28.5 | -5.14 | Polynomial | -2.38 | 5.12 |

- [FL1] Question 4.8-1 other targets

- Please provide your input on monostatic RCS model for other targets, if any

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | Target type | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | μ | σ |
| LGE |  |  | A*B1 0 dBsm for animal as a hazardous object on the road | -1.2 dBsm | σ from μ |

- Please provide your input on bistatic RCS model for other targets, if any

  - To report A*B1 as a whole, please leave the block of ‘A (dBsm)’ empty.

| Company | Target type | A (dBsm) | B1 (dB), or A*B1 (dBsm) | B2 in dB scale, i.e., normal distribution |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | μ | σ |

| Moderator | If any additional comments, please provide it in following table |
| --- | --- |

Reference TRs reused as starting point

Reference TRs

Summary on company views

E// observes that the channel models for the background TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 are inconsistent with each other.

Table 1	Comparison of K-factor in fast-fading models

| Model | K-factor |
| --- | --- |
| TR 38.858: TRP-TRP UMa | LOS:  = 9 dB,  = 3.5 dB NLOS: N/A |
| TR 36.777: UMa-UAV Alternative 1 | LOS:  = 20 dB,  = 0 dB NLOS:  = 10 dB,  = 0 dB |
| TR 36.777: UMa-UAV Alternative 2 | LOS:  = 4.217⋅log10hUT+5.787 = 11 dB at hUT = 25 m,  = 8.158⋅exp0.0046⋅hUT = 9.2 dB at hUT = 25 m NLOS: N/A |
| TR 36.777: UMa-UAV Alternative 3 | LOS:  = 15 dB,  = 0 dB NLOS: N/A |

The channel of TRP-TPR link refers to

- 36.777: vivo

- 38.858: CATT, Huawei

E//

- Reuse Option 2 (baseline) of UE-UE channel of InH, and InF in section A.3 of TR 38.858 for UE-target link, i.e., do not support Option 1: A.2.1.2 in TR 36.843 (NOTE 6).

- Do not reuse UE-UE link in section A.1 of TR 38.859 for UE-target channel in indoor sensing scenarios, because it only addresses two InF sub-scenarios, with two options, and it doesn’t revise BS’s angular spread in BS-UE channel model in 38.901.

E//

- Support the same height of the two BS for BS-BS background channel

- Reuse InF channel model in 38.901 for BS-BS by setting the same height for the two BS. For BS-BS below clutter, the InF-SL or InF-DL scenarios should be used, while for BS-BS above clutter, InF-SH, InF-DH, and InF-HH should be used.

QC

- For a vehicle UE – Vehicle UE: use TR 37.885

- For a non-vehicle UEs, reuse the SL Positioning TR solution:

  - Option 1: BS-to-UE channel model defined in TR 38.901 [11] is revised:

  - Option 2: D2D channel model from 36.843 A.2.1.2 is used.

[Moderator’s note] RAN1 already agreed that the existing TRs can be used to generate Tx-target link, target-Rx link and the background channel for sensing as starting point. However, the detailed mapping between reference TR and a pair of nodes (STX, SRX, target) are still missing. The following 3 tables are already proposed in past meetings (with updates). Since the calibration work will start after Feb. meeting, it urges the agreement on details of the mapping. Further, it is generally preferred to minimize the options for each pair of nodes.

- Table 1: Reference TRs

- The related sections in the following existing TRs are used as starting point to generate a channel between any two nodes from TRP, normal UE, vehicle UE and aerial UE.

| Case | Node 1 | Node 2 | Existing TRs as starting point |
| --- | --- | --- | --- |
| 1 | TRP | TRP | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: - TRP-TRP link of scenario UMi, UMa, InH, and InF in section A.3 of TR 38.858 - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hUE=35m - Option 2: - TRP-UE link of scenario UMi-AV, UMa-AV, and RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 for UMi, UMa, RMa, UMi-AV, UMa-AV, and RMa-AV - TRP-TRP link of scenario InH and InF in section A.3 of TR 38.858 for InH and InF Highway - Option 1: - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hUE=35m - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 in FR2 - Option 2: - TRP-UE link of scenario RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR1 - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR2 Urban grid - Option 1: - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 for Urban grid - Option 2: - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 for Urban grid HST - Option 1: - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1, e.g., hUE=35m - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 in FR2 - Option 2 - TRP-UE link of scenario RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR1 - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR2 |
| 2 | TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| 3 | TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 - Option 2: [only for UMa, RMa] V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 |
| 4 | TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | normal UE | normal UE | UMi, UMa, RMa, InH, and InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: UE-UE of UMi, UMa, InH, and InF in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901 for InH and InF; UE-UE of Umi and UMa in section A.3 of TR 38.858 for UMi, UMa, UMi-AV, and UMa-AV; - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 3: UE-UE link in section A.1 of TR 38.859 Highway and Urban grid - Option 1: P2P channel model in section 6 of TR 37.885 HST - Option 1: P2P channel model in section 6 of TR 37.885 - Option 2: Reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 in FR1, e.g., hBS=1.5m, UE-UE channel model for UMa in FR2 - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 6 | normal UE | vehicle UE | UMi, UMa, RMa - Option 1: UE-UE of UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2P in section 6 of TR 37.885 - Option 3: UE-UE link in section A.1 of TR 38.859 Highway and Urban grid - Option 1: V2P in section 6 of TR 37.885 |
| 7 | normal UE | aerial UE | UMi-AV, UMa-AV, and RMa-AV Option 1: FFS reuse TRP-aerial UE of UMi-AV in Annex A and B of TR 36.777 for UMi-AV, UMa-AV, and RMa-AV |
| 8 | vehicle UE | vehicle UE | Highway and Urban grid - Option 1: V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: UE-UE link of scenario UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 9 | aerial UE | aerial UE | UMi-AV, UMa-AV, RMa-AV - Option 1: FFS aerial UE-aerial UE link of scenarios UMi-AV, UMa-AV, RMa-AV in section Annex A and B of TR 36.777 |

- Table 2: on target channel

- To generate the Tx-target link and the target-Rx link in the target channel, the following mapping between STX/SRX, target and the proper case of existing TRs is adopted

  - UAV is considered as aerial UE

  - Vehicle is considered as vehicle UE

  - Human, AGV and objects creating hazard are considered as normal UE

| STX/SRX | Target | Case of existing TRs as starting point |
| --- | --- | --- |
| TRP | UAV | Case 4: TRP-Aerial UE link of same scenario |
|  | Human | Case 2: TRP-normal UE link of same scenario |
|  | Vehicle | Case 3: TRP-Vehicle UE link of same scenario |
|  | AGV | Case 2: TRP-normal UE link of same scenario |
|  | Object (Hazard) | Case 2: TRP-normal UE link of same scenario |
| Terrestrial UE | UAV | Case 7: normal UE - Aerial UE link of same scenario |
|  | Human | Case 5: normal UE-normal UE link of same scenario |
|  | Vehicle | Case 6: normal UE - Vehicle UE link of same scenario |
|  | AGV | Case 5: normal UE-normal UE link of same scenario |
|  | Object (Hazard) | Case 5: normal UE-normal UE link of same scenario |
| Vehicle UE | Human | Case 6: normal UE - Vehicle UE link of same scenario |
|  | Vehicle | Case 8: Vehicle UE - Vehicle UE link of same scenario |
|  | Object (Hazard) | Case 6: normal UE - Vehicle UE link of same scenario |
| Aerial UE | UAV | Case 9: Aerial UE - Aerial UE link of same scenario |
| AGV UE | AGV | Case 5: normal UE - normal UE link of same scenario |

- Table 3: on background channel

- To generate the background channel of bistatic sensing mode, the following mapping between STX/SRX and existing TRs is adopted

| STX/SRX | SRX/STX | Case of existing TRs as starting point |
| --- | --- | --- |
| TRP | TRP | Case 1: TRP-TRP link of same scenario |
|  | Terrestrial UE | Case 2: TRP-normal UE link of same scenario |
|  | Vehicle UE | Case 3: TRP-Vehicle UE link of same scenario |
|  | Aerial UE | Case 4: TRP-Aerial UE link of same scenario |
|  | AGV UE | Case 2: TRP-normal UE link of same scenario |
| Terrestrial UE | Terrestrial UE | Case 5: normal UE-normal UE link of same scenario |
|  | Vehicle UE | Case 6: normal UE - Vehicle UE link of same scenario |
|  | Aerial UE | Case 7: normal UE - Aerial UE link of same scenario |
|  | AGV UE | Case 5: normal UE -normal UE link of same scenario |
| Vehicle UE | Vehicle UE | Case 8: Vehicle UE - Vehicle UE link of same scenario |
| Aerial UE | Aerial UE | Case 9: Aerial UE - Aerial UE link of same scenario |
| AGV UE | AGV UE | Case 5: normal UE - normal UE link of same scenario |

[Moderator’s note] It is generally preferred to minimize the options for each pair of nodes. However, unless we can quickly converge on down-selection of the options, we may agree on the tables as starting point. Since we need to start calibration after Athens meeting, please each company be seriously on checking if the tables are agreeable.

- [H][FL1] Proposal 5.1-1 on reference TR

- In order to generate Tx-target link, target-Rx link and the background channel, the above Table 1/2/3 are adopted for the mapping between reference TRs and a pair of nodes (STX, SRX, target)

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Use these tables as a starting point with optional reduction some cases in future |
| ZTE | Yes |  |
| Lenovo | No | Based on our evaluation, in a large portion of the agreed deployment scenarios, the referenced TRs (38.901) do not generate/model clutters as large as 20-30 dB stronger than the sensing target channel (neglect clutters which are stronger than the target channel). This is the reason that the current TRs give a very sparse clutter pattern in the delay domain (only 20 delay taps contain clutter energy), which is unrealistic for an ISAC channel model. In our view, we can only use the current TRs if the are amended to generate/model clutters in the order of the target channel energy, please see an example analysis for UMa-ISD=500m |
| Ericsson |  | Please find our comments to Case 5 and Case 1 of Table 1. Case 5 UMi, UMa, RMa, InH, and InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: UE-UE of UMi, UMa, InH, and InF in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901 for InH and InF; UE-UE of Umi and UMa in section A.3 of TR 38.858 for UMi, UMa, UMi-AV, and UMa-AV; - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 3: UE-UE link in section A.1 of TR 38.859 Our comments are - Option 3 is not preferred. - Section A.3 of TR 38.858 is referred to in both Option 1 and Option 2. But it includes a baseline (38.901) option and another option of D2D channel mode from 36.843. The baseline 38.901 is preferred, rather than both are supported. Case 1: UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: - TRP-TRP link of scenario UMi, UMa, InH, and InF in section A.3 of TR 38.858 - FFS reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hUE=35m - Option 2: - TRP-UE link of scenario UMi-AV, UMa-AV, and RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 for UMi, UMa, RMa, UMi-AV, UMa-AV, and RMa-AV - TRP-TRP link of scenario InH and InF in section A.3 of TR 38.858 for InH and InF Our comments are - The previous comment to case 1 also applies here about the two options of section A.3 of TR 38.858. In both options, TRP-TRP link of scenario InH and InF in section A.3 of TR 38.858 is reused for InH and InF. Note that it supports BS height of 3m, which is not the BS height in InF scenario in 38.901. However, it is common to deploy BS of the same height for BS-BS background channel. Therefore, in InF scenario, for BS-BS below clutter, BS height should be consistent with InF-SL or InF-DL scenarios, while for BS-BS above clutter, InF-SH, InF-DH, and InF-HH. |
| CATT, CICTCI | Yes |  |
| Samsung |  | We would like to clarify the possibility of reusing the channel parameters from TR 36.777 in FR2. In the corresponding study, the evaluation was conducted with 2 GHz and 700 MHz. It is necessary to verify whether such channel characteristics apply to FR2. If not, the channel parameters should follow TR 38.901, in which case only Annex B.1.3 of TR 36.777 would be applicable. |
| Xiaomi |  | We are fine with the proposal. Down selection of a reference TR when several reference TRs are used for a link can be further discussed in the calibration in AI 9.7.1. |
| DOCOMO |  | Support |
| Apple |  | Is the TRP-to-UE /UE-to-TRP case captured here ? |
| SONY | Yes |  |
| Moderator | Based on the agreement of Proposal 5.1-2, considering the unified design, I just delete all 36.777 based solutions for TRP-TRP channel of highway, Urban grid and HST. Please note I add an option 4 which is based on comments from Ericsson which is also in the same principle of the agreement on TRP-TRP channel Please continue comments on the revised Table 1 |  |

- Revised Table 1: Reference TRs

- The related sections in the following existing TRs are used as starting point to generate a channel between any two nodes from TRP, normal UE, vehicle UE and aerial UE.

| Case | Node 1 | Node 2 | Existing TRs as starting point |
| --- | --- | --- | --- |
| 1 | TRP | TRP | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858 - For InF, hUE is changed to the same height as the BS - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m Highway - Option 1: - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 in FR2 - Option 2: - TRP-UE link of scenario RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR1 - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR2 Urban grid - Option 1: - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 for Urban grid - Option 2: - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 for Urban grid HST - Option 1: - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m for FR1 - TRP-TRP link of scenario UMa in section A.3 of TR 38.858 in FR2 - Option 2 - TRP-UE link of scenario RMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR1 - TRP-UE link of scenario UMa-AV in Annex B.1.2 and Annex B.1.3 of TR 36.777 in FR2 |
| 2 | TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| 3 | TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 - Option 2: [only for UMa, RMa] V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 |
| 4 | TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |
| 5 | normal UE | normal UE | UMi, UMa, RMa, InH, and InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: UE-UE of UMi, UMa, InH, and InF in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: FFS reuse TRP-UE link of scenario InH and InF in section 7 of TR 38.901 for InH and InF; UE-UE of Umi and UMa in section A.3 of TR 38.858 for UMi, UMa, UMi-AV, and UMa-AV; - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 3: UE-UE link in section A.1 of TR 38.859 - Option 4: - UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858 - TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS=1.5m Highway and Urban grid - Option 1: P2P channel model in section 6 of TR 37.885 HST - Option 1: P2P channel model in section 6 of TR 37.885 - Option 2: Reuse TRP-UE link of scenario RMa in section 7 of TR 38.901 in FR1, e.g., hBS=1.5m, UE-UE channel model for UMa in FR2 - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 6 | normal UE | vehicle UE | UMi, UMa, RMa - Option 1: UE-UE of UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2P in section 6 of TR 37.885 - Option 3: UE-UE link in section A.1 of TR 38.859 Highway and Urban grid - Option 1: V2P in section 6 of TR 37.885 |
| 7 | normal UE | aerial UE | UMi-AV, UMa-AV, and RMa-AV Option 1: FFS reuse TRP-aerial UE of UMi-AV in Annex A and B of TR 36.777 for UMi-AV, UMa-AV, and RMa-AV |
| 8 | vehicle UE | vehicle UE | Highway and Urban grid - Option 1: V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: UE-UE link of scenario UMi and UMa in section A.3 of TR 38.858 - For RMa, reuse TRP-UE link of scenario RMa in section 7 of TR 38.901, e.g., hBS=1.5m - Option 2: V2V link of scenario Highway and Urban grid in section 6 of TR 37.885 - Option 3: UE-UE link in section A.1 of TR 38.859 |
| 9 | aerial UE | aerial UE | UMi-AV, UMa-AV, RMa-AV - Option 1: FFS aerial UE-aerial UE link of scenarios UMi-AV, UMa-AV, RMa-AV in section Annex A and B of TR 36.777 |

| Company | Yes/No | Further comments |
| --- | --- | --- |

[Moderator’s note] Since there are still multiple options for a pair of nodes in the first table, and it is generally preferred to minimize options, companies are encouraged to provide any views on reducing the options. Especially for TRP-TRP channel as discussed in some contributions for several meetings, i.e., 36.777 vs. 38.858.

[H][FL1] Question 5.1-2 reducing options for reference TRs

- Companies are further encouraged to comment on how to down select the options for each pair of nodes.

  - Particularly, which TR between 36.777 and 38.858 is preferred to generate a TRP-TRP channel?

| Company | 36.777 or 38.858 | Further comments |
| --- | --- | --- |
| ZTE | 38.858 | 38.858 is preferred because it was for TRP-TRP channel. There is no description saying that 36.777 is applicable for TRP-TRP. |
| Huawei, HiSilicon | 38.858 |  |
| CATT, CICTCI | 38.858 | The antenna size and physical characteristics of the TRP and UAV are different, and the surrounding environment is also different (e.g. TRP has base, on top of roof… but UAV does not), which naturally leads to different statistical characteristics. Prefer 38.858 since the model is designed with TRP-TRP link. |
| Samsung | 38.858 |  |
| vivo | 36.777 | It is UAV case that only TR 36.777 can be applied. TRP-TRP link can be generated based on the parameters defined in TR36.777 and modify the hUE = 25m, 10m, and 35m in UMa, UMi, and RMa scenarios, respectively. We suggest taking two different ways for different scenarios. One is to refer to the scenario of UMi UMa RMa InH and InF, where the TRP-TRP link generation used the model defined in TR38.858. The other is that the TRP-TRP link in UMi-AV UMa-AV and RMa-AV can reused the model defined in TR36.777. We are not sure why the TRP-TRP link generation methods in different scenario need to be same? |
| Xiaomi | 38.858 | 38.858 is preferred since it can be used for TRP-TRP channel in more scenarios, i.e., UMa, UMi, InH, and InF. Besides, the TRP-TRP channels in 38.858 have been used in several TRs, i.e., TR 38.802, TR 38.828, TR 38.859, for evaluations. In our view, the down selection of options can be discussed in the calibration in AI 9.7.1. |
| DOCOMO | 38.858 |  |

[H][FL2] Proposal 5.1-2 reducing options for reference TRs

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP chanenl is

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

| Company | Yes/No | Comments |
| --- | --- | --- |

[Moderator’s note] Agreements after Tuesday online

- Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

[Moderator’s note] Following the principle in above agreement, the following proposal on UE-UE channel is made

[H][FL2] Proposal 5.1-3 reducing options for reference TRs

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE chanenl is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

[Moderator’s note] Agreement at Thursday online session

- Agreement

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

[Moderator’s note] For TRP-UE channel, the basic proposal is to reuse communication channel in TR 38.901

[H][FL2] Proposal 5.1-4 reducing options for reference TRs

The reference TR to generate a TRP-UE chanenl is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 - Option 2: [only for UMa, RMa] V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

[Moderator’s note] revised in Wed offline session

[H][FL3] Proposal 5.1-4-rev1 reducing options for reference TRs

The reference TR to generate a TRP-UE chanenl is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - Annex B.1.2 and Annex B.1.3 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

[Moderator’s note] agreement at Thursday online session

- Agreement

The reference TR to generate a TRP-UE channel is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

Remaining issues on a basic ISAC channel model

HW: For the coupling of the stochastic cluster in Tx-ST and ST-Rx link, the two strongest clusters are forcing to 1 by 1 couple. The N-2 weakest cluster are randomly 1 by 1 couple

Nokia: System-level simulations shall be applied to ensure that complexity reduction scheme of Option 3 maintain similar ISAC channel odelling performance of Option 0.

QC: Reuse the existing number of number of clusters and rays per cluster as in TS38.901

Samsung: Support a shared LOS cluster modelling method for mono-static sensing modes, as Tx-target link and Target-Rx link are likely to have significant similarities

ZTE: For single scattering point odelling, the number of NLOS clusters is 1/3 of that defined in 38.901

Polarization of target

Agreement To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path I, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

Summary on company views

RCS polarization VV, HH, VH, HV

Option 1: QC, Xiaomi, ZTE, Apple, CATT, BUPT, Sony, LG, CAICT, IDC, LG

Option 2: IDC, Sony, LG (future use)

Option 3: E//, vivo

Option 4: HW, Xiaomi

αi,1=αi,2=1, βi,1,βi,2 are separately generated: NIST

Φspθθ,Φspθϕ,Φspϕθ,Φspϕϕ uniformly distributed within -π,π: BUPT, ZTE, QC, NIST

E//: incidence and scattering directions have to be converted to the local coordinate system of the object prior to evaluating the RCS and the CPM

E//: CPMsp must be specified in a local coordinate system, in which the z-axis is parallel to the z-axis in the global coordinate system, zGCS,zLCS,target=1

E//: CPMsprotated target=R1⋅CPMsp⋅R2

Measurements on 4 polarization RCS values, i.e., HH, VV, HV, VH to derive XPR.

- Xiaomi, Apple, ZTE, BUPT: similar/same RCS for HH, VV; same value for RCS HV, VH; RCS VH is much lower than RCS VV

| Target type | Company | μXPR (dB) | σXPR (dB) | κ |
| --- | --- | --- | --- | --- |
| UAV | ZTE | 24.4 | 3.05 |  |
|  | BUPT | 9.7659 | 3.768 |  |
| Human | ZTE | 25.8 | 2.99 |  |
|  | BUPT | 11.5509 | 2.153 |  |
|  | Apple |  |  | 19.74dB |
|  | Xiaomi |  |  | 31.29dB |
| Vehicle | ZTE | 22.3 | 4.71 |  |
|  | BUPT | 9.456 | 2.51706 |  |
|  | Xiaomi |  |  | 34.33dB |

[Moderator’s note] From the companies’ inputs, Option 1 gets a majority support. Please check if the following proposal is acceptable. Please also input on the (mean, std) for XPR targeting agreement in RAN1 #120 in each parameter in your preferred options.

[H][FL1] Proposal 6.1-1 Polarization of target

To model polarization matrix of a direct/indirect path i of a scattering point of a target,

- αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where  κsp,i is XPR ratio is randomly generated by log-normal distribution.

- The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Ericsson | No | Spatial consistency needs to be considered, i.e. should the κsp,i for different times, positions, or combinations of incidence and outgoing angles be correlated? Furthermore, the proposal is incomplete without specifying which coordinate system it refers to. The natural choice would be to specify this polarization matrix in the local, target-oriented, coordinate system. Note that bi-static RCS was agreed to be obtained in LCS, and this should apply to mono-static RCS too. Rotation of RCS between LCS and GCS is needed for angle-dependent RCS. It is the same for polarization matrix, since the same measurements in LCS are divided into RCS values and polarization matrix, both in LCS. Agreement Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction. We suggest to revise the proposal as follows: To model polarization matrix of a direct/indirect path I of a scattering point of a target in the local coordinate system (LCS) of the target, - αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where κsp,i is XPR ratio is randomly generated by log-normal distribution. - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π |
| Huawei, HiSilicon |  | Is this proposal applied to any sensing target? |
| CATT, CICTCI | Yes |  |
| NIST | No | We are fine with “ αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where κsp,i is XPR ratio is randomly generated by log-normal distribution” when modelling the three types of targets as a single scattering point.” However, these relationships may not hold for every type of target, especially when the CPM modeling is performed in the antenna coordinate system. We recommend generalizing the polarization framework to prevent future modifications for different target types. |
| Samsung | Yes |  |
| vivo |  | If the target behaves a specular reflection with the size > 10 wavelength, αi,1=αi,2=1 is no longer properly assumed. |
| Qualcomm | Yes | OK with the change from Ericsson |
| MediaTek | Yes |  |
| InterDigital | No | The random initial phase should not be the only choice. We propose to have options other than the random initial phase. As we discussed in our contribution, in addition to random initial phase, we should incorporate at least one of the following options for Φsp,iθθ,Φsp,iϕϕ; - Option 1: Deterministic values for CPMsp considering specular reflection - Option 2: Stochastic CPMsp considering truncated Gaussian distribution The above options assume the following polarization matrix. CPMsp,i=αi,1expjΦsp,iθθ00αi,2expjΦsp,iϕϕ |
| Xiaomi | Yes | Agree. Also fine with Option 4 for further complexity reduction. |
| Apple | Yes |  |
| SONY | Yes |  |
| BUPT | Yes | Based on the measurement, we have provided the parameters for modeling the CPMsp of the three types of sensing targets using Option 1. The detailed values are shown in the table above. |
| Moderator | Based on offline discussion, the following two solutions are proposed for down selection. |  |

[H][FL3] Proposal 6.1-1-rev1 Polarization of target

Solution A

To model polarization matrix of a direct/indirect path i of a scattering point of a target, αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

  - κsp,i is XPR ratio is randomly generated by log-normal distribution. i.e.

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

Solution B

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target,

- For UAV

  - Option 1

    - αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where  κsp,i is XPR ratio is randomly generated by log-normal distribution.

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

- For human, vehicle

  - Option 1 of above

  - Option 3

    - αi,1,αi,2,βi,1,βi,2 are variables generated for path i

    - Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ are initial random phase of path i

    - CPMsp,i defined in LCS

[Moderator’s note] revised in Thu. Offline session

[H][FL3] Proposal 6.1-1-rev2 Polarization of target

Solution A

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

  - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type. i.e.

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

Supported: ZTE, QC, CATT, SS, LGE, Nokia, OPPO, Xiaomi, SKT, Huawei, Apple, Ericsson (if CPM is defined in LCS), NIST

Solution B

polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target is defined per target type

- For UAV

  - Option 1

    - αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, where  κsp,i is XPR ratio is randomly generated by log-normal distribution.

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

- For human, vehicle, and other target types

  - Option 3

    - αi,1,αi,2,βi,1,βi,2 are angular dependent variables generated for path i

    - Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ are initial random phase of path i

    - CPMsp,i defined in LCS

Supported: vivo, Ericsson

- Solution C

  - CPMsp,i=αi,1expjΦsp,iθθ00αi,2expjΦsp,iϕϕ

  - Φsp,iθθ, Φsp,iϕϕ are deterministic

Supported: IDC, Lenovo, vivo

[Moderator’s note] Agreed in Thursday online session

- Agreement

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target

  - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

    - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

  - FFS: spatial consistency of random phase when a scattering point moves

  - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

Polarization matrix normalization

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio κ and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

Summary on company views

Normalization of CPMtx-sp-rx: vivo, ZTE, Tejas, Apple

- CPMtx,sp,rxnorm=d11d12d21d2212d11+d22=2d11+d22d11d12d21d22: ZTE

The magnitude of the diagonal elements of each component CPMsp,rx , CPMsp , CPMtx,sp  should be set to 1: Apple

[Moderator’s note] The polarization matrix of a direct/indirect path is generated as the product of 3 component polarization matrixes. Each component polarization matrix is already normalized. Different views are observed regarding whether further normalization on the product matrix is necessary or not. Though the product matrix of each direct/indirect path may not be normalized, the average effects of all direct/indirect paths are likely still normalized since the number of paths will be from several thousands to hundreds of thousands.

[H][FL1] Proposal 6.2-1 polarization matrix normalization

- Normalization on the polarization of a direct/indirect path, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is not supported

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | Normalization on CPM has been agreed in our view. The FFS part is how to do that. For each path, if the power will be changed considering the impact from CPM if normalization is not done. However, path dropping based on the threshold such as -25dB will not consider CPM impact. This will lead higher power path is dropped. |
| LGE | Yes |  |
| Ericsson | Yes | Support |
| Huawei, HiSilicon | Yes |  |
| Nokia, NSB | Yes |  |
| CATT, CICTCI |  | OK for progress. |
| Samsung |  | CPM_tx-sp_rx needs to be normalized to ensure power conservation. |
| vivo | No | In principle, polarization does not generate any gain. |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| SONY | Yes |  |
| BUPT | Yes | The polarization matrix only facilitates power transfer without any power loss. The cascading of three polarization matrices effectively results in three power transfers. When each component polarization matrix is already normalized, CPMtx,sp,rx does not require additional normalization. |
| Moderator | Revised at Thursday online, but no agreement |  |

- [H][FL1] Proposal 6.2-1 polarization matrix normalization

Normalization on the polarization of a direct/indirect path, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp is not supported.

  - Note: for path dropping, CPM is also considered

Without normalization by: LG, SKT, Ericsson, Sony, CATT,

With normalization: ZTE, vivo, Samsung, Qualcom, OPPO,

Power threshold

Summary on company views

E//: Include spurious targets in the target channel to model spurious paths with weak power and Doppler shift.

Threshold in step 6 for Tx-target or target-Rx link

- Keep -25dB: Xiaomi

- Remove: Xiaomi

- Lower value:

Threshold in step 6 for background channel

- Keep -25dB: Apple, CATT

- Lower value: E//, Lenovo

Threshold for path dropping after concatenation of Tx-target or target-Rx link

- -25dB: QC, LG, vivo

- Lower value: Apple (-30dB), Xiaomi(-45dB)

- Power metric considering A and/or B1: Samsung

- further power normalization after link concatenation should be considered: Samsung

QC, Xiaomi, Apple, ZTE: No further power normalization of target channel is performed after path dropping

- Needed: SS, CATT, BUPT, Tejas

Lenovo

The stochastic clutter associated with an ISAC background channel can be modeled via the following steps:

- Generate a first set (S1) of cluster/rays according to the steps 1-10 of 38.901 (Section 7.5), utilizing N, M values given in Table 7.5.6 [this is the same as the current 901 procedure] - Generate a second set (S2) of all NLOS cluster/rays utilizing similar large scale parameters as for the first set, and by repeating the steps 5-10 of 38.901 (Section 7.5) where - N,M values set to [NLE,MLE] [different number of rays and/or clusters may be desired here, e.g., for low-energy clusters we may not need high number of per-cluster rays with angle spread, but more number of clusters] - The power of the S2 clusters are scaled by modifying (7.5-6) as PnS2=PnS2’n=1NLEPnS2’P1S110-25/10 where P1S1 is the power of the strongest (first) cluster belonging to set (S1). [the total generated power of the S2 clusters is set to the current 901 dropping threshold (25 dB below the strongest cluster power) and hence does not impact the statistics of original clusters] - Channel coefficient generation and addition of pathloss/shadowing term according to steps 11, 12 of 38.901 considering all of the generated cluster/rays. [same as the current 901 procedure].

[Moderator’s note] In the early agreements, 3 power threshold for cluster/path dropping need to be resolved

- Case 1: The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB to generate Tx-target link and target-Rx link

- Case 2: The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB to generated the background channel

- Case 3: The power threshold for path dropping after concatenation, i.e., X=[-25] dB

On Case 1, there sounds good support to keep the current behavior

On Case 2, it is controversial whether to reduce the threshold. Some companies have concern that a dropped path in background channel by -25dB threshold may have a power much higher than the target channel. On the other hand, some other companies worry about the inconsistency with existing communication channel, and think that the major part of background channel is already modeled, so not need to change the threshold.

On Case 3, evaluation results show that most indirect paths of NLOS ray + NLOS ray will be dropped with threshold -25dB which results in reduction of >10% power of the target channel.

[H][FL1] Proposal 6.3-1 on power threshold -25dB

- The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

- The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generated the background channel

- The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with the first and second bullets. The value of X in the third bullet is FFS |
| ZTE | Yes | We support the proposal. Just wondering how to determine X value in the future discussion. |
| Lenovo | No | For the background channel, the proposed procedure in 38.901 (with or without dropping) is not sufficient for ISAC background channel modeling. Based on our evaluation, in a large portion of the agreed deployment scenarios, the referenced TRs (38.901) do not generate/model clutters as large as 20-30 dB stronger than the sensing target channel (i.e., neglect clutters which are stronger than the target channel). This is the reason that the current TRs give a very sparse clutter pattern in the delay domain (only 20 delay taps contain clutter energy), which is unrealistic for an ISAC channel model. Actually, changing the dropping threshold usually only impacts 3 or less clusters, and does not change this issue. In our view, we can only use the current TRs if the are amended to generate/model clutters in the order of the target channel energy, please see an example analysis for UMa-ISD=500m for both cases with and without cluster dropping: |
| LGE |  | - For the 1st bullet, we support - For the 2nd bullet, we think that the same power level needs to be applied for both target and background for path dropping to get more realistic channel modeling. So we suggest the following modification. That is, any background cluster having power lower than -25dB of the max. cluster power of the target channel is dropped. - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused with reference to the max. cluster power of the target channel to generated the background channel - For the 3rd bullet, we don’t think that further path dropping is needed after target channel concatenation. But if majority supports it, we prefer to keep -25dB threshold to keep consistency. |
| Ericsson |  | The first bullet is fine. The second bullet is problematic, since all clusters in the target channel are likely to be significantly weaker than all clusters in the background channel. Therefore, the mere existence of cluster with power <25 dB of the strongest cluster is evidence that a target exists. In reality there is no such distinction as there will be many weaker multipaths also in the background channel. This artificial cluster power difference between the target channel and the background channel will lead to false conclusions on the performance of certain target detection algorithms, especially AI-based methods that will utilize such an artificial difference. |
| Huawei, HiSilicon | Yes |  |
| Nokia, NSB |  | Support the 1st bullet. The 2nd bullet should be discussed later after agreement of background channel. |
| CATT, CICTCI |  | Fine with the first 2 bullets. For the 3rd bullet, we think no need to drop paths after concatenation, assuming that a power normalization will be done eventually. |
| Samsung |  | Generally fine. If an update to a value other than 25 dB is necessary, we can discuss it along with the measurement and validation results. |
| vivo | Yes | From the complexity perspective, the lower the X value is, the less the rays are dropped, the more rays are kept, that is not friendly for the simulation complexity. |
| Qualcomm |  | Needs discussion together with the next question (6.3-2). We tend to acknowledge the issue of dropping low power NLOS clusters of the background channel. |
| AT&T |  | Agree with the first bullet. The 2 other bullets need further discussion in light of results shown by other companies on the discrepancy between target and background channels |
| MediaTek |  | We support the first 2 bullets, and regarding the third bullet, we need more discussion whether it is really needed. Per our understanding, the motivation for the third bullet is not clear. |
| InterDigital |  | We are ok to keep the value of X open. In our updated contribution (R1-2501368), we propose not to perform path dropping. |
| Xiaomi |  | We support the proposal. The method to determine the value of X in the third bullet should be aligned. |
| Apple |  | From our measurements in a conference room, this conclusion may be made. However, for an outdoor scenario in which the target power is much less than the channel power, some discussion may be needed. |
| SONY |  | Fine with the first two bullets For the third bullet, path dropping can reduce the computation complexity. But the value of the X need further discussed. |
| Moderator | The following summarizes the supporting companies on each bullet -25dB for Tx-target and target-Rx link - Yes: EURECOM, LGE, E//, HW, Nokia, CATT, SS, vivo, AT&T, MTK, XM, Sony - No -25dB for background channel - Yes: EURECOM, HW, CATT, SS, vivo, MTK, XM, Sony - No: Lenovo, E//, QC, LGE, Apple? [X=-40] for path dropping after concatenation - Yes: ZTE, SS, HW, vivo, XM - No: CATT, IDC - Value: EURECOM(FFS), LGE(-25), Sony(FFS) Continue discussions |  |
| Moderator | Revised in offline session and additional discussions |  |

[H][FL1] Proposal 6.3-1-rev1 on power threshold -25dB

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background chanel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power,  max. target-Rx link cluster power)

  - Reuse -25dB: OPPO, vivo, CATT,

  - Reduced to Y<-25dB: SKT, Apple, LG, Nokia

  - Reuse -25dB with update: LG

    - The reference power for removing cluster is the min (max. Tx-target link cluster power,  max. target-Rx link cluster power)

  - Reuse -25 to existing clusters + new low power NLOS clusters: Lenovo, Ericsson, ZTE

[Moderator’s note] Agreed in Thursday online session

- Agreement

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

[Moderator’s note] According to the observation from Lenovo, Ericsson, please comments whether/how to model some lower power NLOS clusters in background channel that are still higher than paths in target channel.

- [FL1] Question 6.3-2  further NLOS cluster power reduction

- Whether/how to model some lower power NLOS clusters in background channel that are still higher than paths in target channel?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Lenovo | Yes | We need to model the low-energy clutters as far as their energy is comparable with the target channel to have a realistic ISAC channel model, please see our analysis above. |
| LGE | Yes | As commented in Proposal 6.3-1, we think that the same power level needs to be applied for both target and background for path dropping to get more realistic channel modeling. So we suggest the following modification. That is, any background cluster having power lower than -25dB of the max. cluster power of the target channel is dropped. |
| Ericsson | Yes | This is one way of overcoming the problems associated with the second bullet in the previous proposal. Another way is to model a number of unwanted targets, i.e. EO type 1, that can lead to weak clusters which may allow more appropriate channel conditions for evaluating sensing. Type-1 EO may include trees moving in the wind. Though they don’t have comparable sizes to sensing targets, their Doppler signatures will challenge sensing algorithms in distinguishing an intended target from trees in the background. |
| Nokia, NSB | Yes |  |
| Xiaomi |  | In our view, this question is related to the threshold for cluster dropping in background channel, and should be determined before the discussion of proposal 6.3-1. |
| Apple |  | Yes |

RCS upper bound

- Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Summary on company views

k=1: ZTE

k=2: Apple

k=3: Nokia, Xiaomi, QC, LG, BUPT

k=4: QC

[Moderator’s note] k=1,2,3,4 are proposed by contributions to derive the upper bound kσ of RCS component B2. A value in the middle may be the compromise to move forward.

[H][FL1] Proposal 6.4-1 RCS upper bound

- k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Yes |  |
| Nokia, NSB | Yes |  |
| CATT, CICTCI | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes |  |
| AT&T | Yes |  |
| Xiaomi | Yes |  |
| DOCOMO | Yes |  |
| Apple | Yes |  |
| SONY | Yes |  |
| BUPT | Yes | The 3σ bound is based on the empirical rule of the Gaussian distribution, which covers 99.7% of the data, effectively avoiding extreme values while ensuring more reasonable and stable simulation results. |

[Moderator’s note] agreement after Tuesday online

- Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Pathloss

Summary on company views

Pathloss of the target channel

- PLtarget,dB=PLd1+PLd2+10lgc24πf2-10lgσRCS,A

  - 1m2: HW

  - Mean RCS: vivo, Nokia, QC, ZTE, Xiaomi, EURECOM, CATT, AT&T, CAICT, Apple, MTK, IDC, Lenovo, Tiami, Sony

- If a dielectric blocker to block the LoS path: PL2hop(d1,d2)=PL(d1+d2)+L

    - Supported by QC

E//:

- uses RCS components A, B1, and B2 in one step

- Apply the pathloss and shadow fading to each link before concatenating them with a target’s RCS.

Shadowing

Separate shadow fading for each link, σTx-tar-Rx = σ1+σ2: DOCOMO

Breaking point distance

HW: the path loss model in UMi, UMa and RMa scenario of TR38.901 is used for the target channel, only the PL1 is applied in the target channel without the breaking point distance.

[Moderator’s note] Since key controversial part is whether to include RCS component A (e.g., mean RCS, or 1 m2) in a concatenated pathloss formula or using all RCS components together, and we already discuss the issue multiple times, please directly check whether a proposal for compromise, i.e. Proposal 4.2-1 is acceptable.

Background channel for monostatic

[Moderator’s note] The following proposal was the outcome after online in RAN1 #119, though there is no agreement on it.

- Proposal 6.3-1-rev3 -RAN1 #119 Strive to down-select among the solutions below at RAN1#120: Solution A: (previous Option 1) On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point - drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario. - FFS the distribution and the mobility for reference points dropping - N is determined by validation results for each scenario, FFS value N Solution B (Previously option 2) - drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel - only direct path is modeled for each ground clutter - FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Summary on company views

Solution A: HW (3), ZTE (1), BUPT(3), OPPO, Samsung, Lenovo, CATT, CAICT, MTK, EURECOM, Panasonic, CALTTA, Tejas

Solution B: E//, QC, Tejas (lower priority)

Solution C: Nokia

ZTE

Proposal 15: For Hbackground modelling in the case of mono-static sensing mode, a virtual Rx location can be randomly generated, and then the stochastic mechanism of TR 38.901 between the real Tx and the virtual Rx is reused. - Gamma distributions can be used to generate the distance between Tx/Rx and the virtual Rx, and to generate the virtual Rx height - The virtual Rx is uniformly dropped in the horizontal domain for each TRP. - ASD parameter in TR 38.901 can be updated to make background clusters in any horizontal directions. - ASA generation is not needed where ASA is enforced to be the same as ASD Proposal 16: In UMa scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by The extended mean value of log-normal distribution for ASD could Proposal 17: In Indoor-Office scenario, the Gamma distribution of distance between Tx/Rx and the virtual Rx, and the virtual Rx height could be given by

HW

- If the mono-static background channel is modelled using the reference points as in solution A. - The generation and parameters can be modelled as - Step1：deploy the reference points as: Nrp=3 AODrp_n~Uniform{(n-1)*120°, n*120°} ,n=1,2,3 drp_2D_n~Γα=10.59, β=0.13+68.5 hrp_n~Γα=18.74, β=1.92+2.05 The 3D coordinates of the reference points are: (drp_2D_n*cos(AODrp_n),drp_2D_n*sin(AODrp_n),hrp_n) - Step 2: generate the clusters following the TR 38.901 generation steps under NLOS condition with modification as number of clusters = 8. SF = 2.5 the absolute delay for each cluster generated as log10(∆τn/1s)~N-6.62, 0.14 - Step 3: Combine the channels of each BS-RP link. Hmono-static background=i=1NrpHBS-RP(i)

E//

Step 1: Set environment, network layout, and antenna array parameters Step 2: Stochastically generate a grid of ground clutter RCS values (RCS per unit area x area of grid cell) Step 3: Calculate the path loss between BS and each grid cell Step 4: Determine the power of the backscattered path by using the radar equation (path loss x2 + RCS) Step 5: Generate a single ray for each grid cell. The azimuth and elevation angle and the delay are deterministically determined using the relative positions of the BS and the center point of the grid cell, while the power comes from step 4. Since this is a monostatic ray, the DoD is identical to the DoA while the total delay is two times the distance between the BS and the center point of the grid cell. The set of rays from all grid cells can then be combined into a set of n rays which can be used as input to step 9-12 in TR 38.901. An example model parameters are - RCS per unit area is Gaussian distributed with mean σ0=-20 dBm2m2 and standard deviation 20 dB - Grid size: 5 m

Nokia

Step 1: Choose an environment (e.g. indoor, Umi, UMa) and generate parameters: - Minimum distance to clutter is defined by nearest illuminated large clutter (wall/building): Radar location Parameter; center/wall of 6x6 room; center/wall of 20x20 room; Building wall near intersection (Umi); Rooftop (UMa). dmin; 3 m / 6 m; 10 m / 20 m; 30 m; 30 m (FFS, pending measurements). Reverberation time Trev; 10-8 sec; 3x10-8 (?)sec, FFS; FFS; FFS. Clutter reflection coefficient; 0.25 (dielectric environments, like concrete); 0.25 (dielectric environments, like concrete); 0.25 (dielectric environments, like concrete); 0.25 (dielectric environments, like concrete). Step 2 generate a grid of delays, azimuth and elevation angles: Delays at resolution defined by signal B, starting at minimum delay from nearest clutter, up to 10x reverberation time: Azimuth angles with 1 deg resolutionspanning 360 degrees Elevation angles with 1 deg resolution: spanning +/-, whereis the rms elevation spread proscribed by 3GPP 38.901 for the environment of interest. Step 3: Generate average backscatter power delay profile (linear, not dB): Using step function Step 4: Generate average power deviation (dB) using a normal distribution, once per simulated environment Step 5: Generate a normally distributed “clutter map” (dB) across the delay-azimuth-elevation grid Step 6: Generate random small-scale fading phase for each multipath component and each polarization combination in the angle-delay grid in :

Apple

- the locations of clusters in the target channel are deterministically generated: Apple,

- add a fixed or variable white Gaussian whose power reflects the interference suppression ability of the sensing receiver

CAICT

- Large scale fading of background channel for TRP mono-static can be modeled as normal distribution with μ varying along frequency and fixed σ

[Moderator’s note] In addition to the two solutions from last meeting, one new solution is proposed by Nokia with validation results (named as solution C). So let’s start our discussion using the 3 solutions.

Note: Since background channel for monostatic is necessary feature to start calibration work, we should conclude this issue in RAN1 #120.

[H][FL1] Question 6.6-1 background channel for monostatic

- Companies are encouraged to provide comments on the following 3 solutions on modelling background channel for monostatic, for down-selection within RAN1#120:

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

  - FFS the distribution and the mobility for reference points dropping

  - N is determined by validation results for each scenario, FFS value N

Solution B (Previously option 2)

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - only direct path is modeled for each ground clutter

- FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Solution C

Step 1: Choose an environment (e.g. indoor, Umi, UMa) and generate parameters: - Minimum distance to clutter is defined by nearest illuminated large clutter (wall/building): Step 2 generate a grid of delays, azimuth and elevation angles: Delays at resolution defined by signal B, starting at minimum delay from nearest clutter, up to 10x reverberation time: Azimuth angles with 1 deg resolutionspanning 360 degrees Elevation angles with 1 deg resolution: spanning +/-, whereis the rms elevation spread proscribed by 3GPP 38.901 for the environment of interest. Step 3: Generate average backscatter power delay profile (linear, not dB): , Using step function Step 4: Generate average power deviation (dB) using a normal distribution, once per simulated environment Step 5: Generate a normally distributed “clutter map” (dB) across the delay-azimuth-elevation grid. Step 6: Generate random small-scale fading phase for each multipath component and each polarization combination in the angle-delay grid in :

| Company | Solution A or B or C | Comments |
| --- | --- | --- |
| EURECOM | Solution A |  |
| ZTE | Solution A | To FL, option A has been super majority and validated by several companies, why cannot we go for it. Solution C may be feasible from our view, however, it is a new one and suddenly proposed in this meeting, we don’t have time to validate it. The concern on solution B is raised in our contribution. Since we don’t see the validation from the proponent, so the results are provided by ZTE based on some assumption. Finally, the results are not aligned either with RT result in UMa scenario or with measurement result in indoor as shown in the following figures respectively. The pathloss gap between the model and the real background channel is quite large. |
| Nokia, NSB | Solution C | The detailed proposal, with field measurement results, is in our contribution R1-2500680. In summary, the proposal is using statistical log-normal distribution to capture backscatter power along the grid of delay, azimuth angles, and elevation angles. These 6-step procedure follows: - Set an environment (indoor, Umi, or Uma), define minimum distance to clutter: dmin - For example: dmin=do=6m indoor, or dmin=do=25m in Umi - Generate a grid of delays, azimuth angles, and elevation angles (τi, ϕp, θq), from the Tx/Rx - One specific (τi, ϕp, θq) indicates one “bin” in the space associated with a range and a direction. - Calculate average backscatter power delay profile P0τi (power ratio, not dB) based on propagation model - (Optional) Calculate average power deviation Pv in dB as log-normal distribution, specified for each simulated environment - Calculate a “clutter map” with log-normal distribution across the delay-azimuth-elevation grid, as P(τi, ϕp, θq) in dB; - Calculate random small-scale fading phase for each multipath component and each polarization in the angle-delay grid, where Φp,q,iis a uniform distribution between 0 to 2π. With backscatter measurement campaign, the power variation across azimuth angles are shown as validation of the proposed scheme: Compared to the background channel modelling in Solution A, the scheme is based on dropping of N “virtual Rx” in the field to capture statistics of background channel. The number of dropped “virtual Rx”, N, can be as high as 1000, based on proposals from companies. The number and placement of “virtual Rx” are as yet undefined; and these must correspond to the environmental geometry to produce correct power. Compared with the backscatter model of Solution C, this “virtual Rx” approach is computationally complicated because of the large number of “virtual Rx”. Backscattered power depends directly on the number and location of “virtual Rx”, yet it must, in the end, match measured backscatter power as it depends on a large number N of ground clutters. We think that the backscatter model is a simple distribution model based on log-normal distribution observed in backscatter measurements. The model is calibrated with sensing channel measurements, as presented in Section 3.1 of R1-2500680. The background channel can be captured with a relatively simple model, backed by backscatter measurements, suitable for large scale system level simulations required for sensing performance evaluation. |
| Ericsson | Solution B | We did validate Solution B with measurements in our contribution R1-2500756, copied below. Figure 15 is the measurement results, and Figure 17 our realization of Solution B. Both show multipaths in a large delay range in the background channel. Figure 15 Measured impulse response of a monostatic UMa background channel at 3.5 GHz. Figure 17 Impulse response for a monostatic UMa background channel generated using the proposed model. We appreciate ZTE’s efforts to validate Solution B. Solution B’s parameters can be derived from measurements, so it is inherently validated. A bad choice of parameters may lead to poor fitting results. We used the following parameters in our realization. So RCS of each grid = -20+10*log10(25) = -6dBsm - RCS per unit area is Gaussian distributed with mean σ0=-20 dBm2m2 and standard deviation 20 dB - Grid size: 5 m Note the unit RCS -20 dBm2m2 value corresponds to moderate clutter: trees, fields, and choppy water [1]. - D. K. Barton, “Modern radar system analysis’, (Norwood, Artech House, 1988) In ZTE’s contribution R1-2500577, parameters used to model Solution B are quoted below. “the grid size is equal to 5m. In each grid point, it is supposed there is one point target, with RCS in dB value obeying the Gaussian distribution of .” Since there were no details on how the results were generated, we put our best guesses. @ZTE, please correct and clarify. The sentence above says RCS of the 5m x 5m grid cell has the mean value of -20dBsm. We suggest -20dBsm/sm as unit RCS, so RCS of the grid cell=-20+ 10*log10(25)= -6 dBsm. Does the missing 10*log10(25)=14dB explain the pathloss difference from RT results? Figure 13 CDF of pathloss Moreover, Figure 15 and Figure 23 in R1-2500577 show CDF curves of delay range with different grid sizes. For a grid size of 100m*100m, the largest delay is 0.2us, and for a grid size of 250x250m, the largest delay of 0.6us. Our speculation is that that the BS is dropped at the center of the square grid, the furthest ground clutter locates at the four points of the square, and therefore the largest BS-ground clutter distances are 100*sqrt(2)/2=71m and 250*sqrt(2)/2=177m respectively, corresponding to a one-way propagation delay of 0.2us and 0.6us. Note that with Solution B, delay of a path generated by ground clutter in mono-static background channel is two times of a one-way propagation delay. If our speculation is correct, delay values in the simulation should be doubled. Moreover, the two grid sizes for Solution B are much smaller than the area size of RT simulation. Note that in our measurements in Figure 15 above, the largest propagation delay is 12us, and in our simulation of Solution B in Figure 17 above, it is 9ms. Figure 15 CDF of Delay Figure 23 CDF of Delay Solution B only requires two parameters, unit RCS and grid size. Exemplary values are given as follows and can be further discussed. - RCS per unit area is Gaussian distributed with mean σ0=-20 dBm2m2 and standard deviation 20 dB - Grid size: 5 m As FL summarized above, ZTE pointed out that mean value of ASD in 38.901 has to be revised for UMa scenario to allow all azimuth departure/arrival angles. Actually, ZSD also needs revision, at least for indoor scenarios, to reflect received signals in a very large zenith range. HW suggested that number of clusters and SF should be revised from those in 38.901. Different alternatives of Solution A may end up with many changes of 38.901 parameters, which is one of our concerns. Our concerns of Solution A include - Delay spread, angular spread, and path loss cannot be reused from 38.901 - There is no outdoor measurement validation of the model, and no measurements to help determine suitable parameter values - It is very unclear how to generalize this model to other scenarios than UMa or other monostatic Tx/Rx such as UEs or indoor Office gNBs.. - It is unclear how to handle spatial consistency, including moving virtual Rx (Spatial consistency procedure A can only support movement up to 1m) and cell border between two BSs. Spatial consistency is mandatory for target tracking. - It seems overly complex to generate clusters of rays with similar delays and angles, since this will require very many rays and clusters to span the expected richness in delay and angle. The complexity increases with the number of virtual Rx. In contrast, benefits of Solution B are as follows. - Supported by 70+ years of radar research - Reproduces measurements very well - Can be generalized to e.g. monostatic UE-based sensing by using 3D distribution of clutter - Spatial consistency is inherently supported. Moving ground clutter is modelled in the same way as sensing target. - Simple to parameterize (only σ0 and grid size needed) - Easy to implement by reusing direct path modelling of target channel Moreover, Solutions B and C share many similarities and both will better represent the measured and simulated monostatic background channel. The complexity is likely to be much lower than solution A. |
| CATT, CICTCI | A (1st prefer) C (FFS) | We think a background channel model should achieve a good balance between accuracy and complexity. Making it more and more complicated does not help too much since it will be eliminated, canceled… at the end. To us Option A (N=1) is good enough. Option C may also be OK but need time to think about it. |
| CMCC |  | Support Solution A. |
| vivo | Solution A |  |
| ZTE2 | A | @Nokia with regards to the number of virtual Rx, as FL summarized, it is only 1 or 3 based on companies’ suggested. Hence, the complexity issues do not exist. @E/// Thanks for the further clarification. Accordingly, we update the simulation for solution C where the total grid size is 3000m*3000m for Uma scenarios. Also, the additional RCS 10*log10(25) is added. Because RCS i larger now, the pathloss based on solution B is even smaller. The gap between the solution and RT results is even larger than 30dB Also, the delay distribution is shown as follows where solution B cannot provide well matched results. Furthermore, for indoor office, we have measurement results to check the feasibility of solution B, however, the gap is still very large especially for pathloss. In addition, there are still a lot of issues to be researched in solution B, at least including: - The shape/size of grid - The distribution for dropping point target especially in vertical domain - CPM - How to distinguish UMa, UMi and RMa, only grid size ? - The difference of delay spread is too small in different TRP/drops, i.e. different TPRs have almost the same delay spread and other large scale parameters (the variation is just 0.19 based on solution B). Note: delay spread in current 38.901 is a log normal distribution, and the variation is 0.39 which is much larger than solution B |
| Qualcomm | Solution B |  |
| LGE | Solution A | We prefer Solution A for commonality with the bi-static background channel modelling, which is based on stochastic clutters. |
| BUPT | Solution A | We think the Solution A presents a more favorable approach, which maximal reuses the existing TR. This solution requires only the identification of appropriate reference points. Regarding the generation of mono-static background channels using N reference points, we propose N=3 as the configuration for 360° horizontal coverage. When N=3, the generated channel better matches the angular spread of the measured mono-static background channel, which has been validated in our proposal [R1-2501369]. |
| Moderator | Thanks for all inputs. Given there is support to each solution A/B/C, while it is generally preferred to have single option for a feature, it would be great if RAN1 could down-select to single solution in this meeting. |  |

[H][FL2] Proposal 6.6-1 background channel for monostatic

Down-select one from the following 3 solutions on modelling background channel for monostatic within RAN1#120,

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

  - FFS the distribution and the mobility for reference points dropping

  - N is determined by validation results for each scenario, FFS value N

Solution B (Previously option 2)

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - only direct path is modeled for each ground clutter

- FFS the number, the distribution, RCS, XPR, etc. for ground clutters

Solution C

Step 1: Choose an environment (e.g. indoor, Umi, UMa) and generate parameters: - Minimum distance to clutter is defined by nearest illuminated large clutter (wall/building): Step 2 generate a grid of delays, azimuth and elevation angles: Delays at resolution defined by signal B, starting at minimum delay from nearest clutter, up to 10x reverberation time: Azimuth angles with 1 deg resolutionspanning 360 degrees Elevation angles with 1 deg resolution: spanning +/-, whereis the rms elevation spread proscribed by 3GPP 38.901 for the environment of interest. Step 3: Generate average backscatter power delay profile (linear, not dB): , Using step function Step 4: Generate average power deviation (dB) using a normal distribution, once per simulated environment Step 5: Generate a normally distributed “clutter map” (dB) across the delay-azimuth-elevation grid. Step 6: Generate random small-scale fading phase for each multipath component and each polarization combination in the angle-delay grid in :

[Moderator’s note] After Monday offline session, the updated version is as below, together with counting on supporting companies.

[H][FL2] Proposal 6.6-1-rev1 background channel for monostatic

Down-select one from the following 3 solutions on modelling background channel for monostatic within RAN1#120,

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario.

  - FFS the distribution and the mobility for reference points dropping

  - N is determined by validation results for each scenario, FFS value N

Supported by: ZTE, OPPO, Xiaomi, Samsung, BUPT, vivo, IDC, Lenovo, LGE, MTK, CMCC, CATT, Huawei

Solution B/C (Previously option 2)

- generate a grid with N bins, each of which associates with a delay and a direction, where a clutter locates in each bin or backscatters signal to sensing Rx in each bin, generate a direct path for each bin. The sum of the N paths generates the background channel

- The backscattered power of each path follows a distribution, depending on the physical characteristics and scenarios

  - FFS: other factors affecting backscattered power

Supported by: Ericsson, Nokia, QC, Lenovo, AT&T

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes | After some offline offline discussion with some companies, to make solution A complete, the solution A can be updated with more details as follows. Considering we only have two meetings left, and need some time for calibration, we hope this updated solution can be adopted in this meeting. Solution A: (previous Option 1) On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point - drop 1 reference point following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario. - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution, drp_2D_n~Γα1, β1+c1 hrp_n~Γα2, β2+c2 - FFS: whether to add very low power clusters |

- [H][FL2] Proposal 6.6-1-rev2 background channel for monostatic

Down-select one from the following 3 solutions on modelling background channel for monostatic within RAN1#120,

Solution A: (previous Option 1)

On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point

- drop 1 reference point following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

- FFS: whether to add very low power clusters

Supported by: ZTE, OPPO, Xiaomi, Samsung, BUPT, vivo, IDC, Lenovo, LGE, MTK, CMCC, CATT, Huawei

Solution B/C (Previously option 2)

- generate a grid with N bins, each of which associates with a delay and a direction, where a clutter locates in each bin or backscatters signal to sensing Rx in each bin, generate a direct path for each bin. The sum of the N paths generates the background channel

- The backscattered power of each path follows a distribution, depending on the physical characteristics and scenarios

  - FFS: other factors affecting backscattered power

Supported by: Ericsson, Nokia, QC, Lenovo, AT&T

[Moderator’s note] Working assumption after Wed. online

- Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Diffraction / blockage model

Summary on company views

Diffraction

- the effect of diffraction at a scattering point can be included in the bi-static RCS modelling: Samsung, Apple

- Bistatic RCS derived from measurements excludes the forward scattering region: AT&T

- Forward scattering effect on RCS is optional: AT&T

E//: One option to model shadow is by forward scattering in target channel with Htarget≈-Hbackground so that HISAC=Hbackground+Htarget becomes small.

- Observation 3: Forward scattering option has multiple disadvantages. - With the very strong forward scattering component, it is difficult to tabulate RCS since it will vary by orders of magnitude as a function of angle. The forward scattering also has a strong frequency dependence, making it even more difficult to tabulate. - The amplitude and phase of the RCS needs to be very precisely modeled to cancel Hbackground - The RCS needs to be complex, otherwise it cannot have a negative phase. - The “mean value of the RCS” is totally dominated by the forward direction - The RCS is very large, but the received power is very small. Therefore, RCS cannot be used for link budget analysis! - The RCS needs to be dependent on both Tx-target distance and target-Rx distance to generate the correct “depth” of the shadow

E//, Lenovo: Another option of modelling the shadows is to apply the existing blockage model B to background channel so that Hbackground is reduced in the shadow region

E//: modelling interactions between the targets when needed by introducing blockage by target1 on the Tx-target2 link and/or the target2-Rx link

E//: Study the impact of the shadowing on the target channel, i.e., attenuation in forward scattering direction, either in tabulated RCS or by blockage model A

[Moderator’s note] Forward scattering was discussed already in the first meetings of the study item. However, there is no agreement on that due to the limited inputs from the companies.

Two effects were discussed by the companies

- Due to blocking effect of the target, e.g., in the direction of 180 degrees, the signal from transmitter is blocked at least in area near to the blocker

- Due to forward scattering, certain energy of signal from transmitter can still arrive at the receiver

Companies are encouraged to check if the following proposal is agreeable.

- [H][FL1] Proposal 6.7-1 blockage model

- The existing blockage model B is reused to model the blocking effect due to a target as an optional feature

  - Applicable to the LOS/NLOS rays in the background channel of the target

  - Applicable to the LOS/NLOS rays in the Tx-target and target-Rx link of the other target

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | No | More study is needed to check whether blockage model B can be completely reused. For example, the human sizes are different between blockage model B and what we proposed in agenda 9.7.1. For background, we think both blockage model A and B could be possible for stochastic method. It is not needed to emphasize mode B only. For target channel, blockage can be considered with RCS together. What proponent suggested seems the issue in proposal 7.5-1. |
| Lenovo | Yes | We can take Blockage model B as a starting point and improve further if needed. |
| Ericsson | Yes | @ZTE, it is not self-blockage in proposal 7.5-1. Two use cases are - A target in the LOS between Tx and Rx may lead to very weak power received by the Rx. In the figure, the human blocks the LOS from Tx to Rx in background channel. 2）A target in the LOS between Tx and a second target may lead to very weak power of incident rays toward the second target. Similar case is the first target is in LOS between a second target and Rx. The same as blockage in 38.901, such blocking is optional. For example, we don’t need to model a UAV far away from sensing Rx. Blockage model B is more proper. |
| Huawei, HiSilicon |  | For the bi-static background channel, it is used as in the communication channel, the blockage model has already been in the current TR38.901 as the other additional features. There is no need to duplicate the discussion on the blockage model in the background channel which is the same as communication channel. For the target channel, the proposal is not clear, the target itself is a blocker or another blocker as defined in the blockage B is reused. For the target itself, the forward scattering (the scattering angle minus incident angle equal to 180°) is already been modelled by the bi-static RCS value. |
| Nokia, NSB |  | Support this proposal, as long as blockage model is an optional feature. |
| CATT, CICTCI | N | We think the effect will be modelled naturally in B1 of RCS for bi-static sensing. It is already incorporated any impact (incl. blockage) when measuring RCS value of B1 (or A*B1, anyway). |
| Samsung |  | If it is optional feature (as additional modeling feature in TR), we are fine. |
| Qualcomm |  | If this proposal is to incorporate the forward scattering effect, it may be part of RCS already. For background channel, it is OK. |
| InterDigital | Yes |  |
| Apple |  | From measurements taken, this may be already part of the RCS if RCS is modelled across all bi-static angles. |
| SONY | Yes |  |
| BUPT | Yes | We have experimentally validated bullet 1 in an IoT scenario [R1-2501369]. When the target blocks the LOS/NLOS rays in the background channel, it causes significant variations in the original channel power. These variations can be preliminarily modeled using blockage B. |

Power normalization between target channel and background channel

- Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Summary on company views

OPPO:

- Option 1: the power normalization includes both LOS part and NLOS part in background channel.

- Option 2: the power normalization includes NLOS part but not LOS part in background channel, i.e., both the total power and LOS power in background channel remain unchanged due to power normalization.

Power normalization Option 2: vivo, Intel, BUPT AT&T, CAICT, IDC, EURECOM

- Alt 1: Power normalization on both target channel and background channel: OPPO, QC, Apple, IDC

- Alt 2: Power normalization on background channel only: OPPO

- Alt 3: the target channel of a target will replace one cluster in the background channel

[Moderator’s note] More inputs will be helpful to resolve Option 2 for power normalization.

- [FL1] Question 6.8-1 Power normalization

- Companies are encouraged to provide details on the preferred alternatives.

| Company | Alt /1/2/3 | Comments |
| --- | --- | --- |
| EURECOM | Alt 2 | Power normalization is needed on both target channel and background channel |
| ZTE | Alt 2 | The power for direct path of the target should not be changed before and after normalization. Otherwise, the power of the direct path of a UAV in UMa and UMi scenarios will be different. Hence, if Alt 1 is supported, power scaling is only needed for the indirect path of the target and background. |
| Lenovo | Alt 1 |  |
| LGE | Alt 1 | If applied, power normalization on both channels is needed to avoid unnecessary boosting of either channel. |
| Ericsson | Option 1, power normalization is not performed. | We sympathize with the intent behind Option 2, but we believe a better way is to ensure that the power in the target channel is always significantly smaller than in the background channel, i.e. by ensuring that RCS values and LOS probabilities to targets are not overly optimistic. Therefore our preference is Option 1. |
| CATT, CICTCI |  | Option 1 is preferred. Hard to find a suitable way to converge on Option 2. Anyway, Option 2 is an ‘additional component’ which may or may not be used. |
| Samsung |  | We would like to go with Option 1 in previous agreement. The motivation of power normalization between target and background channel is unclear. |
| MediaTek |  | Share the similar view with Samsung that the power normalization is not needed. |
| InterDigital | Alt. 1 |  |
| Apple | Option 1, Power normalization not needed. | If we do have power normalization, Option 2-Al-1 is selected. |
| BUPT | Alt 3 | We think that power normalization between the target and background channels is unnecessary. In simpler scenarios like UAV channel, direct superposition of target and background channels is sufficient. However, in more complex environments (e.g., indoor/outdoor scenarios) where the target may significantly impact the background channel, Alt 3 emerges as an appropriate method for combining these two channels. In the ISAC channel realization, the following implementation process can be considered [R1-2501369] - Step 1: The target channel htar(t,τ) is generated by the concatenation method. The angular region of the sensing target (θeod, ϕaod,θeoa, ϕaoa) is determined. - Step 2: The original environment channel henvt,τ is generated by TR 38.901. henvt,τ denotes the vector form of environment channel clusters with dimensions Nenv×1. (Nenv denote the total number of the clusters) - Step 3: Define a Boolean variable B with dimensions 1×Nenv, where the element Bn is 0 or 1. - Option 1: In UAV scenarios, directly superimpose the target and background channels, i.e., B=I. In indoor/outdoor scenarios, select one environmental cluster n* closest to the target in the angular domain, remove this cluster (Bn*=0) in the environment channel. - Option 2：Select the clusters n* in the environment channel that overlap with the target in the angular domain (the probability of overlap is high in indoor and outdoor scenarios but rare in UAV scenarios). Then, remove these clusters (Bn*=0). - Step 4: The background channel is calculated as hbact,τ=B∙henvt,τ. - Step 5: Combine the target and background channels: hisact,τ=htart,τ+hbact,τ. |

Absolute delay model Δτ

Agreement A single direct path is modeled for a scattering point of target - In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray. - FFS how to generate NLOS cluster when Δτ is applied

Summary on company views

Absolute delay is only specified for InF in the latest TR and using absolute delay for UMa and UMi is now being discussed in AI 9.8.

QC: Section 7.6.9 as a starting point for each Tx-target and target-Rx link.

Huawei, E//: Section7.6.9 in TR38.901 can be reused to model the absolute delay in target and background channel

OPPO: When the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is applied, LOS scattering can be modeled by splitting a single LOS ray after Tx-target and target-Rx concatenation into M rays

ZTE: The absolute time delay Δτ should be used to all the stochastic clusters including the first NLOS cluster

Apple: When Δτ is applied, it is not applied to the first NLOS cluster generated as it is effective part of the LOS link

[Moderator’s note] Absolute delay model Δτ is a featured introduced in NR positioning. If it is introduced for ISAC channel model, the application of Δτ in 3 links, i.e., Tx-target link, target-Rx link and the background channel need considerations.

[FL1] Proposal 6.9-1 on absolute delay model Δτ

When absolute delay model Δτ is configured in the target channel, it applies to all NLOS clusters in each of Tx-target and target-Rx links.

- Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes | For clarification, don’t we also need the addition of background channel to the main sentence? When absolute delay model Δτ is configured in the target channel, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel. |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Yes | The absolute delay is essential for the ISAC channel model and should be generated in the main procedure rather than as an additional component in our view. |
| Nokia, NSB | Yes |  |
| CATT, CICTCI | Yes |  |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| Moderator | @LGE, do you mean a single configuration to configure absolute delay to all Tx-target, target-Rx links and background channel? I delete ‘in the target channel’ and add ‘and background channel’. Hope it is OK for all Add a question as commented by Huawei |  |

[FL2] Proposal 6.9-1-rev1 on absolute delay model Δτ

When absolute delay model Δτ is configured in the target channel, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

[Moderator’s note] Agreement at Thursday online

- Agreement

When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

- For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

[FL2] Question 6.9-2 on absolute delay model Δτ

Should we define absolute delay model as a mandatory feature or optional feature for ISAC?

[Moderator’s note] Proposal after Wed. offline session

[FL2] Proposal 6.9-2 for working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

[Moderator’s note] Agreement at Thursday online

- Working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

Micro-Doppler

Summary on company views

Clutter

E//: same parameters vscatt and p both for the background and the Tx-target and target-Rx

OPPO: Maximum speed and ratio of moving scatters depends on scenarios

Micro-Doppler:

- based on the dual mobility odelling of Section 7.6.10 of TR38.901 as baseline: Nokia

- function for micro-Doppler should be specified: QC, vivo, OPPO

OPPO:

| Sensing targets | Micro-Doppler function f(t) |
| --- | --- |
| UAV | The kth blade of the dth rotor: ft=Lcosβcosφd,kt,L=L1+L2/2, L1 is the distance between the blade roots and the center of the rotation, and L2 as the distance between the blade tip and the center of the rotation, β is the elevation angle of the rotor to the radar line-of-sight (LOS), and φd,kt=φd,k0+ωdt is the phase of the kth blade of the dth rotor, withφd,k0 denoting the initial rotation angle, and ωd is the rotation radian frequency of the dth rotor [3]. |
| Birds | ft=lsinϑcosαcosϕt+lcosϑsinϕt α is azimuth angle, ϑ is pitch angle and flapping angle is ϕt ϕt=θmaxsin2πfbirdt=θmaxsinω0t θmax is maximum flapping angle, ω0=2πfbird is flapping frequency [3]. |
| Respiration rate and heartbeat rate | ft=mhsinωht+mrsinωrt mh and mrare the amplitudes of human heartbeat and respiration, and ωhand ωrare the angular frequencies of heartbeat and respiration [3]. |

Vivo

Proposal 31: 	RAN1 agrees on the formula of micro-Doppler frequency shift in Eq. 8 or Eq. 9 at least for the human arm/leg target with the vibration of a scatterer point and the mono-static transmitter/receiver.

Apple

- the maximum speed of moving scatterers should not exceed maximum speed of target, Tx or Rx

- the ratio of moving scatterers among all scatterers depends on evaluation goal.

[Moderator’s note] RAN1 already agreed on the placeholder for micro-Doppler. If there is remaining TU, it is nice to decide certain detailed functions to handle micro-Doppler

- [FL1] Proposal 6.10-1 function(s) for micro-Doppler

- Companies are encouraged to input on the proper functions/models for micro-Doppler

| Company | Comments |
| --- | --- |
| LGE | We compromised to leave a place holder for future development of the micro-Doppler issue. We don’t support to develop further details on the micro-Doppler in Rel-19. |
| vivo | Agreeing on a formula based on mature measurements as an example is very important. In addition, considering a human target, hand/leg vibration is a typical use case that should be captured in TR. As a high ratio of supporting companies, we suggest only agreeing on a use case for micro-Doppler. |
| InterDigital | We support further study of MD dependencies (e.g., target motion characteristics, carrier frequency) and formulations for different target types and corresponding motions, e.g., UAV (rotation, vibration), human (walking/running, heartbeat, respiration), AGV (rotation, vibration). |
| BUPT | We endorse the theoretical Micro-Doppler model proposed by vivo, with its validity being empirically verified through human subject experimental data. Our findings suggest that the Micro-Doppler effect should be incorporated into ISAC channel modeling frameworks. The derived formula fDt=2λωvDvsinωvt provides a fundamental basis for characterizing Micro-Doppler phenomena in such models. |

Target with multiple scattering points

General designs

Summary on company views

Single vs. multiple scattering points

ZTE: Open for multiple scattering points for UAV, human

CAICT

- single point for UAV in low altitude scene, human outdoors, etc.

- multiple point for automotive vehicles, automated guided vehicles, human indoor etc.

Sony

- vehicle and human: multiple scattering points

- UAV: single scattering point model only

Factors to decide single or multiple scattering points for a target?

- the distance between Tx/Rx and the target: Nokia, QC, vivo, Samsung, Apple, Xiaomi, AT&T, CATT, EUROCOM, CAICT, IDC, Tiami

- target type: Nokia, vivo, Xiaomi, EUROCOM, Sony, AT&T, DOCOMO, Apple, CAICT, Lenovo

- use case and deployment scenario: Nokia, vivo, Xiaomi, AT&T, Apple, MTK

- frequency: vivo, CATT

- target size: Huawei, vivo, Apple, Nokia, Samsung, MTK, DOCOMO, Xiaomi, CATT, EUROCOM, AT&T, TOYOTA, CAICT, Tiami, Lenovo

- target shape: Huawei, vivo, Nokia, Apple, CAICT,

- sensing resolution: MTK, CAICT,

- sensing service: MTK

- incident/scattered angle: Lenovo

Number of scattering point(s)

- varied with distance between Tx/Rx and target: Nokia, Samsung

  - different for different pairs of Tx/Rx: Samsung

- The same number and locations of scattering points in one drop regardless of sensing Tx/Rx: Nokia, QC, Xiaomi, ZTE, CATT, Apple, AT&T, EURECOM, vivo, E//

Location of the multiple points

- locations of the scattering points are common to each pair of sensing Tx/Rx: AT&T, ZTE, Panasonic, QC

- Relative location of the N points not changed: QC, CATT, Apple, Xiaomi, ZTE, MTK

  - No rotational motion: Apple

Interaction between the multiple points

- NO rays scattered from one point to another point of the same target: QC, Intel, CATT, CT, Apple, AT&T, MTK, EUROCOM, Xiaomi

RCS of the multiple scattering points

- RCS values are individually determined: Huawei, AT&T, QC, CATT, vivo, CT, Apple, Tiami, EUROCOM, Ericsson

  - Single/common RCS pattern: DOCOMO

- RCS of entire target equals to sum of RCS of each scattering points: vivo, Nokia, LG

Visibility of the multiple (N) scattering points

- N points always visible: ZTE

Spatial consistency are applied for channels of multiple scattering points: ZTE

[Moderator’s note] The issues in Proposal 7.1-1 had been discussed in past 5 meetings and should be much stable. Since they are actually quite basic design principles, it would be quite helpful to agree on the proposal to check further design details.

Regarding the number/location of the multiple scattering points. Two options were discussed and majority companies prefer Option 1, i.e., the simple one. Given TU for the Rel-19 study item is quite limited, it is generally preferred to adopt certain simplification for the modelling.

Option 1: The number and locations of the scattering points are common to each pair of sensing Tx/Rx

Option 2: The number and/or locations of the scattering points are dependent on the directions of sensing Tx/Rx

[H][FL1] Proposal 7.1-1 general

If a target is modelled with multiple scattering points,

- The number of scattering points of a target is generated in the beginning of the simulation and keeps unchanged in the whole simulation

- The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with the proposal |
| ZTE | Yes |  |
| LGE | Yes |  |
| Ericsson | Yes |  |
| Huawei, HiSilicon | Yes | Just wonder whether all of or which subullt(s) will be captured in the TR? |
| CATT, CICTCI | Yes |  |
| NIST | Yes |  |
| Samsung | Yes | Question to FL. Is there a plan to discuss the method for determining whether it is single or multiple scattering points at a later stage? |
| vivo |  | In case of modelling micro-Doppler with multiple scattering points, the location of each scattering point is unfixed, due to the mechanical vibration or rotation. Therefore, we believe that the number of scattering points can be kept the same, but their locations could be varying during the simulation. |
| Qualcomm | Yes |  |
| AT&T | yes |  |
| MediaTek | Yes |  |
| Xiaomi | Yes |  |
| DOCOMO | Yes |  |
| Apple | Yes |  |
| SONY | Yes |  |

[Moderator’s note] After Wednesday offline session

[H][FL1] Proposal 7.1-1-rev1 general

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and keeps unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

[Moderator’s note] After Thursday online session

- Agreement

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

Coordinates of the scattering point(s)

Summary on company views

Single scattering point

- the location of the sensing target is located in the physical centre of the object: HW, Xiaomi, Samsung

Location of the multiple points

- The location of scattering points of a target are not fixed: CMCC, Lenovo

- N points within size/shape of the target: ZTE, Huawei, vivo, Xiaomi

Lenovo: For modelling of a target via one or multiple scattering points, the scattering point locations shall be determined as part of the target channel modelling procedure.

[Moderator’s note] to generate the target channel, it is necessary to define the exact 3D location of the single/multiple scattering points of the target.

[H][FL1] Proposal 7.2-1 Coordinates of the scattering point(s)

- For a target with single scattering point, the 3D location of the scattering point is defined as the geometry center of the target

- For a target with multiple scattering points, the relative 3D location of each of multiple scattering points is defined separately in the evaluation assumptions

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| LGE | Yes |  |
| Ericsson |  | For the second bullet, our preference is to let the physical size of the target determine the locations of the multiple scattering points. For instance, by putting scattering points on the front/back/left/right/upper/lower sides of the target. This will ensure that the combined channel from all scattering points will have a signal strength that varies at a reasonable rate over time, movement, and rotation of the target. |
| Huawei, HiSilicon |  | What is the ‘relative’ referred to? The agreement about the vehicle modelled with multiple scattering points has already made it clear where the points are located. What does the second bullet exactly mean? |
| Nokia, NSB | Yes |  |
| CATT, CICTCI | Yes |  |
| NIST |  | We agree that multiple scattering points' 3D locations should not be always fixed to support certain use cases, such as human gestures or robotic arm motion detection. |
| Samsung | Yes |  |
| vivo | Yes |  |
| Qualcomm | Yes |  |
| AT&T | yes |  |
| MediaTek | Yes |  |
| Xiaomi | Yes |  |
| DOCOMO |  | Support the 1st bullet. For the second bullet, we have the same view with Ericsson |
| Apple |  | Yes for Bullet 1 |
| SONY | Yes |  |
| Moderator | Deleted ‘relative’ and ‘separate’ based on comments from Huawei since it doesn’t hurt the meaning of the sentence. |  |

[H][FL2] Proposal 7.2-1-rev1 Coordinates of the scattering point(s)

- For a target with single scattering point, the 3D location of the scattering point is defined as the geometry center of the target

- For a target with multiple scattering points, the relative 3D location of each of multiple scattering points is defined separately in the evaluation assumptions

[Moderator’s note] After Thursday offline session

[H][FL2] Proposal 7.2-1-rev2 Coordinates of the scattering point(s)

- For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions

[Moderator’s note] After Thursday online session

- Agreement

For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions.

Shared/separated NLOS clusters

Summary on company views

Each scattering point is separately handled as if a target with single scattering point: QC, ZTE, Lenovo?, Samsung, vivo, EURECOM, NVIDIA, OPPO, Nokia, Ericsson, CATT

  - The indirect path generation for the target with single scattering point applies to each scattering point

Lenovo: correction of paths of scattering points follows spatial consistency.

Shared K factor and stochastic clusters: HW, CATT

- LOS ray is generated per scattering point: HW

- the multiple scattering points share the same K-factor value and the stochastic clusters: HW

- the indirect path generation can be applied to one of the scattering points or the geometric centroid of the target: HW

DOCOMO: pseudo-scattering points

- Each LOS pseudo-scattering point is modeled with a direct path

- Total number of direct paths (LOS rays) nLOS_s is equal to summation of single SP and LOS pseudo-SPs

- Humans and UAVs cases: nLOS_r = 5

- AGVs and animals, vehicles cases: nLOS_r = 5/8/10 (dependent on positions of Tx, Rx and targets)

[Moderator’s note] For a target with multiple scattering points, it would be straightforward that each scattering point has its own direct path. On the other hand, different views are observed regarding whether the multiple scattering points would share same set of NLOS clusters or have individually generated NLOS clusters. A preference on simplied solution considering the limited remaining TU is observed too. Companies are encouraged to provide views on the following proposal.

[H][FL1] Proposal 7.3-1 shared/separated NLOS clusters

- To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree with this proposal |
| ZTE | Yes |  |
| LGE | Yes | We support multiple scattering points only for a vehicle ST. In this case, FL proposal is supported. |
| Ericsson | Yes | Spatial consistency modeling can ensure that the points have correlated LOS/NLOS states and correlated large- and small-scale parameters. |
| Huawei, HiSilicon | ok |  |
| Nokia, NSB |  | okay |
| CATT, CICTCI |  | Our first preference is to use a single set of NLOS clusters can be generated for the group of scattering points. If majority want to go with this proposed 7.3-1, we can live with it, though it is unnecessarily making the NLOS cluster complicated. |
| Samsung | Yes |  |
| vivo | Yes | Since the number of clusters are pretty high, the reduction mechanism should be taken into account. |
| Qualcomm | Yes |  |
| MediaTek | Yes |  |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| SONY | Yes |  |
| BUPT |  | We think that if the target establishes multiple points, these multi-point channels should share some parameters or NLOS clusters among them. |
| DOCOMO |  | Because the distances between scattering points are much smaller than correlation distances (RMa: 60m, UMa: 50m, UMi: 15m, InF: 10m), the multiple scattering points of a target should share NLOS clusters. Complexity is also reduced. |

[Moderator’s note] After Thursday online session

- Agreement

To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point.

LOS condition, pathloss

Summary on company views

LOS probability

- Each scattering point is separately handled as if a target with single scattering point: Samsung, ZTE, HW

- Common to all multiple scattering points: Xiaomi

Pathloss

- Each scattering point is separately handled as if a target with single scattering point: Samsung, Xiaomi, ZTE, HW

- Common to all multiple scattering points:

[Moderator’s note] LOS condition and pathloss for the multiple scattering points are the quite basic issue for decision. A most simple way is to enforce same LOS condition and pathloss. On the other hand, the multiple scattering points can have same LOS condition or pathloss in reality due to blockage of clutter and/or EO. Companies are encouraged to provide views on the following proposal.

[H][FL1] Proposal 7.4-1 LOS condition, pathloss

- The LOS condition between Tx/Rx and target are separately generated for each of the multiple scattering points.

- The pathloss between Tx/Rx and target are separately generated for each of the multiple scattering points.

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM |  | Agree. Each scattering point in multiple scattering points is handled as a single scattering point |
| ZTE | Yes |  |
| LGE | Yes |  |
| Ericsson | Yes | Same answer as to 7.3-1 |
| Huawei, HiSilicon | yes | Agree with E// the ‘Spatial consistency’ somehow should be ensured. |
| Nokia, NSB | Yes |  |
| CATT, CICTCI |  | Our first preference is to use a common LOS condition and common path loss for multiple scattering points. We believe the difference will be marginal. However, if majority want to go with this proposed 7.4-1, we can live with it. |
| Samsung | Yes |  |
| vivo |  | In most cases, the multiple scattering points are in vicinity from each other, the pathloss generation should be dependent on the use case target. For example, for vehicle, we are fine with this proposal, but for human or UAV, single point should be considered to generate the pathloss. |
| Qualcomm | Yes |  |
| MediaTek | Yes |  |
| Xiaomi | Yes |  |
| DOCOMO |  | We don’t understand the meaning of “separately” in the 1st bullet. In our understanding, there is a relationship regarding the LOS condition between scattering points. For example, from Tx or Rx, for a 5-scattering point model (front, back, left, right, up sides), the maximum number of scattering points can be seen is 3. In addition, both front and back points, left and right points cannot be seen together. Hence, it cannot separately generate. |
| Apple |  | Spatial consistency may need to be considered |
| SONY | Yes |  |
| Moderator | Revised following offline discussion |  |

[H][FL3] Proposal 7.4-1-rev1 LOS condition, pathloss

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

[Moderator’s note] After Thursday online session

- Agreement

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

Self-blocking

Summary on company views

- Modeled by angular dependent RCS component B1: vivo, CATT, ZTE, HW, E//, Nokia, OPPO

- Explicit self-blockage model considering shape of target:

[Moderator’s note] Self-blocking always happens among the multiple points of a target, i.e. vehicle. E.g., if the transmitter is in the left side, the signal cannot reach the right side of the vehicle, which means less or no signal radiated by the scattering point at the right side. On the other hand, it would be complicated to accurately model the self-blocking at the vehicle since the self-blocking is dependent on the incident/scattered angles. Thanks to the incident/scattered angle dependent RCS model, it provides a way to implicitly model the self-blocking effect.

- [FL1] Proposal 7.5-1 self-blocking

- The self-blocking effect at a target, i.e., a scattering point of the target that is invisible from an incident or scattered direction is modeled by angular dependent RCS component B1 (or A*B1)

| Company | Option 1 or 2 | Comments |
| --- | --- | --- |
| ZTE | agree |  |
| LGE |  | We can live with the proposal. RCS of the invisible side can be set as zero. |
| Ericsson |  | Prefer modeling any self-blocking via the angular dependent RCS component. |
| Huawei, HiSilicon |  | It is not only for ‘multiple scattering points’. self-blocking can be modelled via the angular dependent RCS component. |
| Nokia, NSB |  | Agreeable. |
| CATT, CICTCI | Yes | Support. |
| vivo |  | It the surface of target is self-blocked, it can be modeled with a constant number, e.g., -25dB. |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| BUPT | Yes | The modeling of component B1 needs to account for the self - blocking effect, which has been considered in the multipoint model we provided. |

EO type-2

E//: If the size of the EO is larger than the first Fresnel zone and the reflection point, for which the incidence angle and reflection angle are equal, is on the EO, Type-2 EO is modelled as a reflective surface with the Fresnel reflection coefficients. Otherwise, it is not modelled.

E//: Model arbitrary orientations of type 2 EOs

E//: Limit the number of reflections of each path in the two links tx—target and target--rx to one.

Nokia: In indoor scenarios, a limited number of walls, ceilings and floor may be modeled with as EO-type 2

Polarization

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio κ and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx - FFS CPMsp of a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present

Summary on company views

Text proposal or detailed model provided: HW, ZTE, LG

Only model specular reflection: vivo, CATT, LG, Tiami, NVIDIA

FFS specular reflection is sufficient: Samsung

Pathloss

PLEO=dEOd3DPLTx-STST-Rx: Huawei (LOS, NLOS)

FFS if NLOS condition: CATT

EO type-2 if NLOS condition?

QC, CATT: EO type-2 is only modelled when LOS ray exists in the same Tx-target or target-Rx link

CATT: FFS EO type-2 in NLOS condition

Ericsson: Model blocking of type 2 environment objects reusing the blockage model B in TR38.901, updating the model to account for the shape and orientation of the object.

[Moderator’s note] ZTE and Huawei both propose a procedure to derive the polarization matrix for EO type-2, e.g., a wall, which could be a good starting point for the review by other companies.

[H][FL1] Proposal 8.1-1 on polarization

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | Blocking by EO Type-2 also needs to be modeled. - For specular reflection, we propose to model arbitrary orientations of type 2 EOs. |
| Nokia, NSB | Yes | The blockage issue by EO type-2 can be modeled as an optional feature. |
| CATT, CICTCI | Yes |  |
| NIST |  | Do we have measurements to support the claim that only EO type-2 specular reflection needs to be modeled? |
| Qualcomm | Yes |  |
| Xiaomi | Yes |  |
| SONY | Yes |  |
| Moderator | Revised by offline discussions |  |

[H][FL1] Proposal 8.1-1-rev1 on polarization

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted

[Moderator’s note] After Thursday online session

- Agreement

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted

LOS condition of Tx-target/target-Rx link

- Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

Summary on company views

Option A: HW, Nokia, QC, Apple, SPRD, EURECOM, Panasonic

- The LOS probability equation defined in TR38.901: HW, Panasonic

Opiton B: E//, IDC

- Type-2 EO has no impact on LOS/NLOS condition

E//: Model blocking of Type 2 environment objects reusing the blockage model B in TR38.901, updating the model to account for the shape and orientation of the object.

Samsung

- Option #1:  Reuse the existing LOS probability scheme in TR 38.901 if type-2 EOs are considered as components in the scenario layout

- Option #2: Restrict type-2 EOs to specific areas or locations to avoid impacting the LOS probability

- Option #3: Consider the geometrical locations of EOs in the LOS probability calculation

[Moderator’s note] Companies are encouraged to provide views on the two options

- [FL1] Question 8.2-1 LOS condition

- Which option is preferred to generate the LOS condition for Tx-target link and target-Rx link, Option A or B?

| Company | Option A or B | Comments |
| --- | --- | --- |
| EURECOM | Option A |  |
| LGE | Option A | We slightly prefer option A for simplicity. |
| Ericsson | Option B | Option B is in line with how TR 38.901 treats blocking, i.e. that blocking does not alter the LOS/NLOS state. Concerns of Option A include - Option A is not in line with previous agreements to reuse 38.901 procedures. - It causes inconsistent LOS and NLOS conditions between communication and sensing, if a UE is close to a target. - Another concern of Option A is the LOS and NLOS condition of Tx-target link may change as the target moves, similar to the hard transition in the channel response due to UT mobility stated in clause 7.6.3.3 of 38.901. It increases the complexity of channel realizations. |
| Huawei, HiSilicon | Option A | Option B essentially is modelling the EO as a blockage model, which can be deemed as the existing additional Blockage modelling component as in TR38.901 and no change is needed to the TR presumably. Although both the NLOS and blockage cause a propagation attenuation in the channel, they model the different propagation characteristics. The NLOS is a status defined as a large-scale fading phenomenon in the propagation channel, for example, a building blocks the direct path. The blockage models an instantaneous attenuation in a path, for example, a bus blocks a path when it passes by a user. The blockage can apply in either direct path or indirect path. From the modelling perspective, the blockage is only modelled as an additional attenuation applied on a given path without changing the path loss and fast fading parameters. It is also noted that the modelling of the blockage does not change LOS/NLOS state of each link. Considering the EO type2 represents the wall which is a stationary object in the environment, it is proper to model the Rx as a NLOS condition when the EO type2 blocks the direct path between the Tx and the Rx. |
| Nokia, NSB | Option A |  |
| CATT, CICTCI | Option B | However, since the Rx could be far from EO type-2 and EO type-2 is significantly larger than the wavelength (resulting in negligible diffraction effects), the necessity of modeling additional blockage from EO type-2 needs further study. |
| Samsung | Option A |  |
| InterDigital | Option B |  |
| Xiaomi | Option A |  |
| DOCOMO | Option B |  |
| Apple | Option A |  |
| Moderator | Yes (7): EURECOM, LGE, HW, Nokia, SS, Xiaomi, Apple No (4): Ericsson, CATT, IDC, DOCOMO |  |

[FL1] Proposal 8.2-1 LOS condition

When the EO type-2 is modelled in the target channel, to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

[Moderator’s note] revised at Thursday online but no agreement

- [FL1] Proposal 8.2-1 LOS condition

When the EO type-2 is modelled in the target channel or in the background channel, to determine the LOS condition of the Tx-target link and target-Rx link

- If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, otherwise the link is determined as LOS condition

EO type-2 in background channel

Summary on company views

EO type-2 in background channel if it is modelled in target channel

- Yes: Huawei, CATT, AT&T, Tiami, Sony, ZTE

- NO: Ericsson, QC, DOCOMO

- Neutral: Xiaomi

[Moderator’s note] For EO type-2, companies are encouraged to provide views on the following proposal.

- [FL1] Question 8.4-1 EO type-2 in background channel

- EO type-2 can be modeled in background channel

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM | Yes |  |
| ZTE | Yes |  |
| LGE | Yes | Yes for consistency of channel modeling in geometric perspective. |
| Ericsson | No | Since stochastic clusters have been used to generate multipath propagation in UE-BS communication channel, the need to model Type-2 EO in background channel is not clear. Type-2 EO if modelled in background channel would lead to the consequence that the received power of the sensing Rx’s in the ISAC channel model in the absence of sensing targets is different from that in the legacy UE-BS communication channel. |
| Huawei, HiSilicon |  | Regarding the comment from E//: In our view, if EO is not modelled in the target channel, it would not be modelled in the background channel so to be consistent with the communication channel. |
| CATT, CICTCI | Tend to No | The impact of EO type-2 in background channel is already modeled in current stochastic NLOS path modeling (at least for bi-static sensing). No need to duplicate the modeling. |
| Samsung |  | For specular reflection model, it includes one additional path to the generated channel to reflect the ground reflection effect. In other words, there is no interaction between the reflection point and target, e.g., TxEO type-2targetRx. One concern is that such interaction seems necessary if we want to consider the specular reflection in the target channel; otherwise, EO type-2 can be included in the background channel where no target is involved. So, we want to clarify the EO type-2 modeling’s intention first. |
| InterDigital | No | We are sure why we need to duplicate EO type 2s in background channels. Modeling EO type 2 in the target channel should be sufficient. |
| DOCOMO | No |  |
| Moderator | 4 companies say Yes: EURECOM, ZTE, LGE, HW? 5 companies say No: Ericsson, CATT, CICTCI, IDC, DOCOMO |  |

Spatial consistency modelling

Nokia: Correlation of LOS condition for Tx-target and target-Rx links of the target component channel can use the spatial consistency procedure defined 7.6.3.3 of TR38.901 as a starting point

Samsung: Consider the spatial consistency modelling for the stationary Tx/Rx and moving sensing target first

Temporal consistency

- Spatial and temporal consistency of multiple instances of the same or different sensing modes: Lenovo

Cases spatial consistency is not supported

Summary on company views

spatial consistency for following links are not modelled:

- Links with different LOS and NLOS conditions, e.g., BS-ST1 is LOS, BS-ST2 is NLOS: HW, OPPO

- The BS-ST and ST-UT links: HW, Xiaomi, Apple

- Different BS-ST links, e.g., two BS to one ST: HW, Xiaomi, Apple

[Moderator’s note] In order to progress on spatial consistency discussions, we already spend time to clarify 1) the cases in that spatial consistency may be supported; 2) the cases that spatial consistency is not supported. Point 2) which may be more useful reference to simply spatial consistency discussion

- Same target but two different TRPs, since the two TRPs are far away, there is no need for spatial consistency

- As proposed in proposal 8.1-2 (RAN1 #119), if it happens that the Tx-target link and target-Rx link are generated using channel models of different scenarios (in fact, different statistic property of all/most parameters), spatial consistency are not modeled.

  - Note: it is not precluded we may define channel model of Tx-target link and target-Rx link belonging to the same scenario. If so, spatial consistency can be modeled

Correspondingly, companies are encouraged to check if the following proposal is agreeable

[H][FL2] Proposal 9.1-1 Cases when spatial consistency is not supported

Spatial consistency is not modelled for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the links that are generated referring to channel models with parameter values of different scenarios

  - E.g., between TRP-target link in one scenario and target-UT link in another scenario

  - E.g., between TRP-UT link in one scenario and UT-target link in another scenario

- Case 3: the two links with different LOS/NLOS condition

- Case 4: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Ericsson |  | The intention of Case 1 should be the two TRPs are not co-located. |
| Huawei, HiSilicon | Yes | Agree with E// comment. |
| Nokia, NSB | Yes |  |
| CATT, CICTCI |  | OK for simplicity |
| vivo | Yes |  |
| LGE | Yes | Generally fine. But, at least for UAV cases where the correlation distance may be larger than one of the terrestrial scenarios, the link between different TRPs to X (Case 1) may be considered. |
| InterDigital |  | What is the condition for Case 1 to hold? What is the distance threshold between TRP1 and TRP2 so we don’t consider spatial consistency? |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| SONY | [YES] | Does the 1st bullet mean ‘the link Links from/to two TRPs’? |
| BUPT | Yes | For the links from two TRPs (Case 1), the links in two different scenarios (Case 2), and the links with different LOS conditions (Case 3), the parameters exhibit no or weak correlation. Therefore, spatial consistency modeling is not supported. |
| Moderator | Adding non-co-located in the proposal directly |  |
| Moderator | Further revised in Tuesday offline session |  |

[Moderator’s note] Agreed in Wed. online

- Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

Legacy site-specific spatial consistency

Summary on company views

Spatial consistency of background channel follows existing 38.901: vivo, Lenovo, MTK

For links from one BS to different STs, spatial consistency procedure from TR38.901 can be reused

- HW, Xiaomi, ZTE, SPDR, Xiaomi, AT&T, EURECOM IDC, Sony

[Moderator’s note] ISAC channel is eventually designed for the performance evaluation of both communication and sensing. Therefore, it is necessary to maintain backward compatible as much as possible. Therefore, if it would be generally preferred, if possible, to reuse the existing spatial consistency for all links to/from TRPs.

Correspondingly, companies are encouraged to check if the following proposal is agreeable. Note: this proposal will be the default behaviour since the SID captures that “using 38.901 as a starting point”

[H][FL1] Proposal 9.2-1 legacy site specific spatial consistency

- The existing spatially consistency model in TR 38.901 is reused to model correlation of links from/to same/different TRPs

| Company | Yes/No | Comments |
| --- | --- | --- |
| EURECOM | Yes |  |
| ZTE | Yes | Can we change the wording clearer, e.g. The existing spatially consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs |
| Ericsson | Yes |  |
| Huawei, HiSilicon |  | Agree with ZTE’s comment. The existing spatially consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs without updates. |
| Nokia, NSB | Yes |  |
| CATT, CICTCI | Yes |  |
| vivo | Yes |  |
| c | Yes | Tend to prefer ZTE and Huawei’s updates for clarity |
| LGE | Yes | For links from one BS to different STs/UEs, the existing TR can be reused. But, we think the links from/to different TRPs as mentioned above appears to allow for an interpretation similar to Case 1 of Proposal 9.1-1 where the spatial consistency is not supported. |
| Xiaomi | Yes |  |
| Apple | Yes |  |
| SONY | Yes |  |
| BUPT | Yes | It is necessary for modelling correlation of links from/to TRPs to maintain compatible with TR 38.901. |
| Moderator | Revised based ZTE proposal |  |

[H][FL2] Proposal 9.2-1-rev1 legacy site specific spatial consistency

- The existing spatially consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs

[Moderator’s note] Agreed in Wed. online

- Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

New model for spatial consistency

Summary on company views

Cases needing new spatial consistency

- one ST to different UTs: HW, Xiaomi, OPPO, Apple

- one UT to different STs: HW, Xiaomi, OPPO, Apple

- different STs to different UTs: HW, Xiaomi, Apple

- ST-UT vs. UT-UT: Xiaomi, OPPO, Apple

New types on spatial consistency to be defined:

- Dual-end correlation: ST-UT links are correlated: HW

New spatial consistency model

- RcorrTxm,Txn⋅RcorrRxp,Rxq: HW

- UT specific grid (grid with correlations): HW, vivo, IDC, CATT, Sony

- UE specific grid (grid with correlations): HW, IDC, Sony

- Global grid: Xiaomi

ZTE: the correlation distance should be much less than that of TRP

EO type-2

HW, Vivo, CATT, MTK, NVIDIA: Naturally meet the spatial consistency, i.e., based on deterministic locations and movement (if any) of the environment objects

[Moderator’s note] In early discussions, we also clarified that only relying on legacy site specific spatial consistency without enhancements is not sufficient for ISAC channel model

- E.g., assuming the legacy site specific grid is generated for TX1 being a TRP while RX1 is a UE, the legacy spatial consistency can be reused for TX1-ST2 and TX1-ST1 link. However, it cannot model spatial consistency between ST1-RX1 and ST2-RX1

In early meetings, we generally discussed site specific grid, target specific grid or combinations. However, a balance for complexity and accuracy is hard to be converged.

[Moderator’s note] Please check if the following proposal is agreeable. Especially, please express your views on whether Case 7 needs to be supported or not.

[H][FL2] Proposal 9.3-1 Cases needing new spatial consistency model

New model for spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- FFS Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Huawei, HiSilicon |  | We agree with the principle. However, actually, the spatial consistency of the following four cases is the same. - one ST to different UTs: - one UT to different STs: - different STs to different UTs: - ST-UT vs. UT-UT The truth of these cases is how to ensure the spatial correlation between two links when the dual-end of the link can move. When modelling this dual-end correlation spatial consistency, the ST and UT are treated as the same kind of entity. Thus, there is no need to specify the cases. |
| vivo | Yes | Case 7 is also necessary. We suggest removing FFS as FFS Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes |
| LGE | Yes |  |
| InterDigital | Yes |  |
| Xiaomi | Yes |  |
| SONY | Yes |  |
| BUPT | Yes | Links within the same scenario are expected to exhibit correlation due to the similarity of their propagation environments. Therefore, we propose that Case 5, Case 6, and Case 7 should be included in the modeling. |
| Moderator | Based on input from Huawei, vivo and BUPT, I remove ‘FFS’ from the last bullet |  |
| Moderator | Updated in Tuesday offline session |  |

[H][FL2] Proposal 9.3-1-rev1 Cases needing new spatial consistency model

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

[Moderator’s note] Agreed at Thursday online

- Agreement

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

- FFS: Spatial consistency between multiple scattering points of the same target

[Moderator’s note] In the existing 38.901, spatial correlation types, i.e., site-specific and all-correlated are defined. However, the exact scheme to achieve such purpose, e.g., 2D grid is not specified. Similar situation happens in the current study. It may be difficult to agree on the exact solution, but we should at least introduce the new spatial correlation type which is motivated by correlation among ST/UT links.

[H][FL1] Proposal 9.3-2 New correlation types among ST/UT

New correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of Dual-end Correlated: parameters for links between STs/UTs are correlated, subjected to correlation distance. (definition may be dependent on whether Case 7 is supported or not)

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | Dual-end Correlated |
| Cluster powers | Dual-end Correlated |
| AOA/ZOA/AOD/ZOD offset | Dual-end Correlated |
| AOA/ZOA/AOD/ZOD sign | Dual-end Correlated |
| Random coupling | Dual-end Correlated |
| XPR | Dual-end Correlated |
| Initial random phase | Dual-end Correlated |
| LOS/NLOS states | Dual-end Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson |  | B2 and κsp,i of target polarization matrix should be added. |
| Huawei, HiSilicon | Yes |  |
| vivo | Yes |  |
| Xiaomi | Yes |  |
| SONY | Yes |  |
| Moderator | Based on the offline discussion, a new name for the new correlation type is suggested by companies |  |

[H][FL2] Proposal 9.3-2-rev1 New correlation types among ST/UT

New correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of [link correlated]: parameters for links between STs/UTs are correlated, subjected to correlation distance. (definition may be dependent on whether Case 7 is supported or not)

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

[Moderator’s note] Revised in Tuesday offline session

[H][FL2] Proposal 9.3-2-rev2 New correlation types among ST/UT

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of dual-end Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | dual-end Correlated |
| Cluster powers | dual-end Correlated |
| AOA/ZOA/AOD/ZOD offset | dual-end Correlated |
| AOA/ZOA/AOD/ZOD sign | dual-end Correlated |
| Random coupling | dual-end Correlated |
| XPR | dual-end Correlated |
| Initial random phase | dual-end Correlated |
| LOS/NLOS states | dual-end Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

[Moderator’s note] Further updated based on some offline comments

[H][FL2] Proposal 9.3-2-rev3 New correlation types among ST/UT

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of dual-end Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | dual-end Correlated |
| Cluster powers | dual-end Correlated |
| AOA/ZOA/AOD/ZOD offset | dual-end Correlated |
| AOA/ZOA/AOD/ZOD sign | dual-end Correlated |
| Random coupling | dual-end Correlated |
| XPR | dual-end Correlated |
| Initial random phase | dual-end Correlated |
| LOS/NLOS states | dual-end Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

[Moderator’s note] Agreed at Thursday online

- Agreement

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

[Moderator’s note] It is also important for us to try to align the understanding on exact solution to model spatial consistency for ST/UT links.

- [FL1] Question 9.3-3 New spatial consistency model

- What is your preferred spatial consistency model for the correlation of links between two nodes of STs and UTs?

  - Option 1: global grid (aka. Unified grid) + shifting UT/ST locations

  - Option 2: direct modelling on the correlation of grids of different UT/ST, e.g., Cholesky decomposition

  - Option 3: UT specific grid only in Option 2

  - Option 4: ST specific grid only in Option 2

- Please also provide accurate and compact descriptions on the preferred solution

| Company | UT specific grid, target specific grid, global grid, or others | Comments |
| --- | --- | --- |
| ZTE | Option 1 | Option 1 is simpler. A single global grid is needed as the reference grid. Then, UT or ST specific grid can be obtained by the shift from the reference grid. Cholesky decomposition is workable only in the case of small number of UT/STs. However, if we drop many UT/STs in the whole simulation area, it is hard to determine how many UT/STs should be involved together for one Cholesky decomposition. |
| Huawei, HiSilicon |  | We need to point out the spatial consistency implementation is up to each company in the current TR901. Although the discussion on how to implement the spatial consistency is helpful for understanding and exchanging the views among companies, the method will not be captured in the CR. |
| CATT, CICTCI | Option 1 | Option 1 achieves a balance between complexity and accuracy. Option 2/3/4 is too complicated. |
| vivo |  | We can keep those options so that companies can choose one of them. In addition, we suggest modifying the proposal as - What is your preferred spatial consistency model for the correlation of links between two nodes of STs and UTs? - Option 1: global grid (aka. Unified grid) + shifting UT/ST locations - Option 2: direct modelling on the correlation of grids of different UT/ST, e.g., Cholesky decomposition - Alt-1: UT specific grid only in Option 2 - Alt-2: ST specific grid only in Option 2 Please also provide accurate and compact descriptions on the preferred solution |
| LGE | Option 2 | We think a grid at each node is needed, because each node has a different background environment. |
| InterDigital | Option 1 |  |

Procedure A vs. B for spatial consistency

Summary on company views

Procedure A:

- SPRD: When the sensing target speed is taken into account, the change of distance can be expressed as:

rα,txTtk-1⋅vtxtk-1+rα,rxTtk-1∙vtk-1+rβ,txTtk-1∙vtk-1+rβ,rxT⋅vrxtk-1∙∆t

Procedure B:

- NO: SPDR

Mobility

Samsung, CATT proposes to consider the spatial consistency modelling for the stationary Tx/Rx and moving sensing target first

[Moderator’s note] There is limited discussions from the contributions. However, since procedure A/B are supported in the existing 38.901, it may be supported by default, subjected to RAN1 discussions. Companies are encouraged to comment on the following proposal.

- [FL1] Proposal 9.4-1 Procedure A vs. B

- Both spatial consistency procedures A and B in 7.6.3.2 in TR 38.901 are reused to support spatial consistency of ISAC channel STX/SRX/ST is/are moving

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | Yes |  |
| Nokia, NSB | No | There is no need to support both procedures of 38.901. We support of using procedure A. |
| vivo | Yes |  |
| BUPT | Yes | Procedure A focuses on updating the channel within stationary intervals due to the movement of STX/SRX/ST, while Procedure B updates the channel parameters based on the spatial positions of STX/SRX/ST. These two procedures complement each other, enabling the realization of spatial consistency for both stationary and non-stationary channels. Therefore, we propose that both Procedure A and Procedure B should be reused to support the spatial consistency of the ISAC channel. |

3D spatial consistency

Summary on company views

3D spatial consistency

- the existing 2D correlation distance in Table 7.6.3.1-2 in TR39.901 is extended to the vertical plane

HW, Xiaomi

- Model correction of target specific 3D grid by Cholesky decomposition of correlation matrix: vivo

[Moderator’s note] Companies are encouraged to comment on the following proposal.

- [FL1] Proposal 9.5-1 3D spatial consistency

- The same value of existing 2D correlation distance in Table 7.6.3.1-2 in TR39.901 is extended to the vertical plane

| Company | Yes/No | Comments |
| --- | --- | --- |
| ZTE | A |  |
| Huawei, HiSilicon |  | The procedure A and B here is unclear. The correlation distance in table 7.6.3.1-2 is unrelated to the procedure A and B. |
| LGE | Yes | We prefer procedure A to B. But, both procedures can be reused for spatial consistency. |
| Xiaomi | Yes |  |
| BUPT |  | 3D spatial consistency is primarily utilized for spatial consistency modeling in UAV scenarios. However, Table 7.6.3.1-2 in TR38.901 does not include the 2D coherence distance values for UAV scenarios. Therefore, whether it can be directly extended or how to extend it to the vertical plane requires further discussion regarding its feasibility. |

Link level channel model

[Moderator’s note] NO discussion at Feb. meeting. However, early inputs on ISAC channel model for link level simulation are appreciated.

| Company | Comments |
| --- | --- |

- Reference

- RP-242348, “Revised SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR”, Xiaomi, AT&T

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz, V18.0.0 (2024-03)”

- R1-2500072	Channel modelling for ISAC	Huawei, HiSilicon

- R1-2500179	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

- R1-2500235	Discussion on ISAC channel modelling	CATT, CICTCI

- R1-2500267	Discussion on ISAC channel modelling	China Telecom

- R1-2500298	Discussion on channel modeling methodology for ISAC	CMCC, BUPT, SEU, PML

- R1-2500313	Discussion on ISAC channel modelling	CALTTA

- R1-2500361	Views on Rel-19 ISAC channel modelling	vivo, BUPT

- R1-2500414	Discussion on ISAC channel modeling	EURECOM

- R1-2500418	Discussion on ISAC Channel Modeling	Tiami Networks

- R1-2500463	Study on ISAC channel modelling	OPPO

- R1-2500483	Discussion on ISAC channel modelling	Tejas Network Limited

- R1-2500577	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips

- R1-2501369	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO

- R1-2500660	Discussion on Channel Modelling for ISAC	Sony

- R1-2500680	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

- R1-2500681	Discussion on ISAC Channel Modeling	NIST

- R1-2500685	Discussion on ISAC channel modelling	SK Telecom

- R1-2500692	Channel modeling for integrated sensing and communication with NR	NVIDIA

- R1-2500743	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT

- R1-2501368	Discussion on ISAC channel modeling	InterDigital, Inc.

- R1-2500756	Discussion on ISAC Channel Modelling	Ericsson

- R1-2500797	Discussion on ISAC channel modelling	Apple

- R1-2500861	Discussion on ISAC channel modelling	Samsung

- R1-2500892	Considerations on ISAC channel modelling	CAICT

- R1-2500979	Discussion on ISAC Channel Modelling 	Panasonic

- R1-2501027	Discussion on ISAC channel modelling	MediaTek Inc.

- R1-2501046	Discussion on ISAC channel modelling	LG Electronics

- R1-2501060	Discussion on Channel Modelling for ISAC	Lenovo

- R1-2501082	Discussions on ISAC Channel Modeling	AT&T

- R1-2501167	Discussion on ISAC channel modelling	Qualcomm Incorporated

- R1-2501212	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

- R1-2500999	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2501000	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2501001	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

- R1-2501002	Summary #4 on ISAC channel modelling	Moderator (Xiaomi)

- ANNEX: All agreements

- RAN1 #116

Agreement

The common framework for ISAC channel model is composed of a component of target channel and a component of background channel,

HISAC=Htarget+Hbackground

- Target channel Htarget includes all [multipath] components impacted by the sensing target(s).

  - FFS details of the target channel

- Background channel Hbackground includes other [multipath] components not belonging to target channel

  - FFS details of the background channel

- FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s)

  - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s)

- FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s)

- Note: the notation HISAC can be revised later if needed

- RAN1 #116bis

Agreement

The following cases of radio propagation in the target channel are considered for the study

| Case | Tx-target | Target-Rx |
| --- | --- | --- |
| 1 | LOS condition | LOS condition |
| 2 | LOS condition | NLOS condition |
| 3 | NLOS condition | LOS condition |
| 4 | NLOS condition | NLOS condition |

- Case 1/2/3/4 can be considered for bistatic sensing mode

- At least Case 1/4 can be considered for monostatic sensing mode

- Note: It doesn’t imply the channel response for each link is separately generated then concatenated

- FFS how to determine LOS condition and NLOS condition, e.g., based on LOS probability, or determined based on geometrical locations of environment object (EO).

- In LOS condition, line of sight ray(s) are present between Tx/Rx and target, and there may or may not exist non-line of sight ray(s) between Tx/Rx and target too

- In NLOS condition, there only exist non-line of sight ray(s) between Tx/Rx and target

Agreement

- In the target channel between Tx and Rx, scattering of a sensing target can be modelled as single scattering point or multiple scattering points

- FFS one or multiple incoming/output rays corresponding to a scattering point

- FFS how to select single or multiple scattering points for the target, e.g. depending on the distance between target and Tx/Rx, size/shape of target, etc.

- Note: the sensing target can be assumed in far field of sensing Tx/Rx.

- FFS details to model the single or multiple scattering points

Agreement

RCS of a physical object shows dependency to at least the following factors:

- Type of the object

  - The size of the object

  - The material of the object

  - The shape of the object

- Orientation of the object

- FFS: Distance between Tx/Rx and the object

- The incident angle and scatter angle

- The carrier frequency

- polarization of the transmitter and receiver

- FFS Temporal or spatial consistency

- FFS antenna pattern

- FFS whether/how to model the above factors in the CR, e.g. with an RCS model with a scattering point

Agreement

EO is a non-target object with known location.

- FFS other known parameters of the EO

- FFS details on EO modelling

The following options for EO modelling are considered for further study

  - Option 1: EO is modelled different from a sensing target

    - Applicable at least for an EO having extremely large size (referred as EO type-2 for discussion purpose)

    - FFS modeled similar to section 7.6.8 ground reflection in TR 38.901

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 2: EO is modeled same/similar as a sensing target

    - Applicable for an EO having comparable physical characteristics as a sensing target, (referred as EO type-1 for discussion purpose)

    - FFS Applicable for EO type-2

    - FFS EO modelling impacts the target channel and/or the background channel

  - Option 3: EO is modeled and its location is determined from a stochastic clutter generated following the cluster generation in TR 38.901

    - FFS details

  - Option 4: EO is not modelled

  - Other options are not precluded

  - Note: it is not precluded that multiple options can be supported in the channel modelling

Agreement

The following options are considered for further study to model the target channel for a target

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry

- Option 3: combination of Option 1 and Option 2

Agreement

If a target is modelled with single scattering point, the following options to model RCS of the target are considered for further study.

  - Option 1: Random RCS value generated by a statistical distribution, depending on the factor(s) having impacts on the RCS modelling.

    - FFS the distribution.

    - FFS the factor(s)

  - Option 2: Deterministic RCS value is defined by a function and/or a table, depending on the factor(s) having impacts on the RCS modelling

    - Note: Constant RCS for a target type can be a special case of Option 2

    - FFS the factor(s)

    - FFS details of function and/or table

  - Option 3: combination of Option 1 & 2, e.g., RCS value is generated by combining a deterministic component and a randomly generated component.

  - FFS application of each option to large scale fading and/or small scale fading

  - FFS target with multiple scattering points

Agreement

- Interested companies are encouraged to submit validation results together with their proposal for ISAC channel modelling

- Up to each company to select the way for validation

  - Option 1: Experimental results

  - Option 2: Experimental results to validate a ray-tracing model, then the ray-tracing based results to validate the ISAC channel model

    - Note: the layout of the scenario used for validation is up to company choice

Agreement

ISAC channel model for link level simulation is to be discussed after the system level channel model is sufficiently stable with basic functionalities.

- RAN1 #117

Agreement

- Multiple sensing targets can be modelled in the ISAC channel of a pair of sensing Tx and sensing Rx

  - FFS whether to model a propagation path from Tx to Rx interacting with more than one sensing target

- The same sensing target can be modelled in the ISAC channels of multiple pairs of sensing Tx and Rx

Agreement

- For discussion purpose, the propagation paths in the target channel are classified

  - The direct path, i.e., LOS ray from Tx to target + LOS ray from target to Rx

  - The indirect paths, i.e., any propagation path other than the direct path, including

    - LOS ray from Tx to target + NLOS ray from target to Rx

    - NLOS ray from Tx to target + LOS ray from target to Rx

    - NLOS ray from Tx to target + NLOS ray from target to Rx

- For radio propagation Case 1,

  - For a direct path, the following parameters are [deterministically] generated at least based on the geometry location of Tx, target and Rx

    - AoA/ZoA at Rx

    - AoD/ZoD at Tx

    - AoA/ZoA/AoD/ZoD at target

    - delay

    - FFS initial phase

    - Doppler

    - FFS power/polarization including the impact of RCS

    - FFS the number of direct path(s) for a target

  - FFS on detailed modelling of indirect path(s)

- FFS on details of modelling of indirect paths in  radio propagation Case 2/3/4

- To generate the channel coefficients of direct/indirect path(s) in the target channel, the channel coefficient generation function in step 11 in section 7.5 of TR 38.901 (e.g., formula 7.5-22) is used as the start point

  - Note: modification to step 11 is deemed necessary

  - FFS adding impact of small scale RCS

  - FFS Doppler

Agreement

- Spatial consistency should be supported for ISAC channel

- Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx

  - FFS EO handling

Agreement

When the stochastic cluster is used to generate the indirect paths in the target channel of a target

  - The stochastic cluster generation in section 7, TR 38.901 is used as starting point.

    - FFS a stochastic cluster is generated between Tx and Rx satisfying Tx-target-Rx geometry, or between Tx/Rx and target

      - FFS modification to stochastic cluster generation in section 7, TR 38.901

      - FFS use of sub-cluster to model the indirect paths

Note: RAN1 continues studying using EO to generate the indirect paths in the target channel of a target

Agreement

When the stochastic cluster is used to model indirect path in the target channel

- For bistatic, the LOS condition from Tx to target and from target to Rx is determined separately for a target

  - FFS: The correlation of LOS condition of Tx-target and Rx-target links of a target

- For monostatic, a same LOS condition is determined for Tx to target and target to Rx

- The LOS condition from Tx to target and/or from target to Rx is determined with the LOS probability

  - The probability schemes in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, etc. are considered as start point

  - FFS: How to consider the impacts of target height on LOS probability.

Agreement

When stochastic cluster is used to model indirect path in the target channel, down-select between the following options

- Option 1: modelled by concatenation of path(s) from Tx to target and from target to Rx

  - For each of the Tx-target link and target-Rx link,

    - The parameters delay, power, angle, [initial phase], [Doppler] of NLOS ray(s) in the link Tx-to-Target or Target to RX are generated

      - FFS following cluster generation in section 7, 38.901

    - The target channel is generated by concatenating the parameters of the Tx-target link and target-Rx link.

      - FFS on Convolutional or 1-by-1 coupling or 1-to-many coupling

  - FFS how to combine the clusters in target channel and the clusters in background channel

- Option 2: modelled by Tx-to-Rx path(s) satisfying Tx-target-Rx geometry of the direct path

  - The parameters delay, power, angle, initial phase of a stochastic (sub-)cluster between Tx and Rx are generated following cluster generation in section 7, 38.901

    - The parameters [delay], [power], [angle], [Doppler] of the (sub-)cluster are updated by the target property

  - FFS how to combine the clusters in target channel and the clusters in background channel

- RAN1 #118

Agreement

if RCS related coefficient of a scattering point is included in small scale, the RCS related coefficients are separately determined for different pairs of incident/scattered ray(s) at the scattering point.

Agreement

For radio propagation Case 1, for modelling the target channel of a target with single scattering point,

- To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated

  - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system

  - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system

  - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c

  - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx

  - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS

  - FFS initial phase

  - FFS how to model RCS, polarization of target

- FFS number of direct paths

- FFS on detailed modelling of indirect path(s)

- FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points

Agreement

For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- An indirect path in small scale is modelled by concatenation of path(s) from Tx to target and from target to Rx, i.e., Option 1 in the agreement of RAN1 #117

  - AoD/ZoD/AoA/ZoA from Tx to target or from target to Rx are

    - generated for a LOS ray at least based on the 3D location of Tx/Rx and target in the global coordinate system

    - stochastically generated for a NLOS ray using section 7, 38.901 as starting point

  - Delay is sum of delay of LOS/NLOS ray from Tx to target and the LOS/NLOS ray from target to Rx

  - Doppler is generated by spherical unit vector by AoD/ZoD at Tx and velocity of Tx, by spherical unit vector by AoA/ZoA at Rx and velocity of Rx, and by spherical unit vectors by AoA/ZoA/AoD/ZoD at target and velocity of target

    - FFS The mobility of stochastic clutter

  - The power of the indirect path is generated as the product of the power of the LOS/NLOS ray from Tx to target, the power of the LOS/NLOS ray from target to Rx and the effect of RCS

  - FFS initial phase

  - FFS how to model effect of RCS at target

  - FFS whether/how to model polarization at target

  - FFS How to reduce complexity

  - FFS applicability of the indirect path generation to each/a single scattering point when the target is modelled as multiple scattering points

Working assumption

The RCS related coefficient of a scattering point can be modelled with two components, i.e., linear value RCS = A*B

- A first RCS component A (m2) is included in large scale

  - FFS the first RCS component is deterministic or stochastic

  - FFS The first component is dependent on incident and scattered directions at target

- A second RCS component B (unit ratio) is included in small scale

  - FFS The second component is dependent on incident and scattered directions at target

  - FFS the second RCS component is deterministic or stochastic or combination

- Note: RCS component A or B can be disabled by setting its linear value to 1

  - Whether to disable a component can be discussed per target type

- FFS how to determine A and B for each target

- FFS whether/how to model polarization impact at target

- FFS whether/how to normalize power accounting for target channel and background channel

Agreement

The impact of a scattering point of the target in the target channel is modelled by a scalar RCS value RCS times a complex-valued 2x2 polarization matrix CPMsp, i.e., RCS∙CPMsp

  - FFS whether CPMsp is angular/ray-dependent or independent

  - FFS whether polarization matrix CPMsp is modelled assuming specular reflection or random coefficient for diffraction or scattering

  - FFS whether polarization matrix CPMsp is explicitly modelled or merged with other polarization matrixes from Tx to target and/or from target to Rx

Agreement

For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode,

- The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point

In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode,

- RAN1 to study how to model background channel

  - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results

FFS whether/how to do power normalization between target channel and background channel

Agreement

In order to define the background channel for TRP and UE mono-static sensing mode,

- RAN1 to study how to model the background channel

  - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

    - FFS EO is modelled in the background channel

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures

    - FFS EO is modelled in the background channel

  - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - Other options are not precluded

Agreement

When EO type-2 is modelled, specular reflection is considered to model EO type-2 using section 7.6.8 of TR 38.901 as reference

- As starting point, the effect of type-2 EO (i.e., in the path node1-EO-node2) is modelled as bR∥specular00-R⊥specular, b is a scaling factor (e.g., c equals to d3DdGR relative to LOS ray in section 7.6.8 in TR 38.901)

  - FFS any update to b,R∥specular,R⊥specular

  - FFS any update taking EO orientation into account

- FFS any changes based on section 7.6.8 of TR 38.901 if EO type-2 has finite size

- FFS whether diffraction and scattering can be considered in addition to specular reflection

- EO type-2 is an optional modelling component if supported in a sensing scenario

  - FFS which deployment scenario(s) EO type-2 will apply

- RAN1 #118bis

Agreement

RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target.

- Option 2: The RCS=A*B of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - A is [mean] RCS value. FFS value(s) A

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B

    - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

- Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by

  - The component A is commonly applied to any incident/scattered angles at the scattering point

    - FFS: A = 1 m2 or [mean] RCS value

      - Note: Mean RCS value is defined as the mean value of the distribution of RCS

  - The component B is further split into B1, B2, i.e., B=B1*B2

    - B1 is deterministic based on incident/scattered angles

      - FFS: B1 is defined by a function or by a table

    - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2

      - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths

  - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS

Agreement

RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic

  - B2 of UAV is modelled using log-normal distribution for monostatic

  - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc.

  - For UAV of small size (option 2 for UAV size in UAV parameters table)

    - B1=1

    - A is mean RCS value

  - For UAV of large size (option 1 for UAV size in UAV parameters table)

    - B1 have dependency on incident/scattered angles

    - A is mean RCS value

Agreement

To model the effect of polarization for each direct/indirect path:

- Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link

  - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

    - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx

    - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx is generated by XPR ratio  and initial random phases referring to TR 38.901 as start point

    - FFS how to normalize on CPMtx,sp,rx

    - FFS CPMsp of a scattering point of the target

- FFS: how to model the effect of polarization when EO type-2 is present

Agreement

A single direct path is modeled for a scattering point of target

- In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray.

  - FFS how to generate NLOS cluster when Δτ is applied

Agreement

In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

Agreement

On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point

- Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model

  - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X

- FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

Agreement

3D spatial consistency needs to be studied for at least UAV scenario

Agreement

In LOS condition between sensing Tx/Rx and target, the power of LOS ray is generated following power of LOS ray in TR 38.901.

Agreement

The following options are to be studied for the concatenation of Tx-target and target-Rx link in the target channel

- Direct path (if present) is always kept

- Indirect paths of LOS+NLOS, NLOS+LOS (if present) are generated

- On other indirect paths of NLOS + NLOS

  - Option 0: ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 0A: ray level full convolution between Tx-target link and target-Rx link only for radio propagation Case 4

  - Option 1: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 1A: cluster level full convolution between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 2: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters for radio propagation Case 1/2/3/4

  - Option 2A: cluster level 1-by-1 coupling between Tx-target link and target-Rx link, then 1-by-1 coupling rays within each pair of clusters only for radio propagation Case 4

  - Option 3: ray level 1-by-1 coupling between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3A: ray level 1-by-1 coupling between Tx-target link and target-Rx link only for radio propagation Case 4

- Note: reducing the number of rays per cluster and/or reducing the number of clusters can be considered for the options above

- Any indirect path with power metric less than [threshold] is dropped

  - the power metric of a path is the product of power of a ray in Tx-target link, power of a ray in target-Rx link and RCS of the pair of rays

  - FFS power normalization of target channel after path dropping

  - FFS the set of remaining indirect paths can be updated during movement of Tx, target or Rx

- RAN1 #119

Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

Agreement

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

- Note: power normalization when target channel and background channel are combined can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

Agreement

- If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

Agreement

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, 
vspt=vmacrot+vmicro,pt

  - FFS: maximum speed of moving scatterers

  - FFS: ratio of moving scatterers among all scatterers

Agreement

- The following options are supported to generate the combined ISAC channel

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed.

  - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Agreement

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

  - FFS: CPM normalization

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

Agreement

The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2.

Agreement

Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Agreement

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

- RAN1 #120

Agreement

For bistatic/monostatic RCS

- RCS values/pattern for a scattering point of a target for bistatic sensing is generated by A*B1*B2 (i.e., Option 3 from the agreement in RAN1 #118bis)

- RCS values/pattern obtained by setting the same incident/scattered angle in the RCS model for bistatic sensing should be aligned with RCS for monostatic sensing

Agreement

RCS model and application in ISAC channel generation

- To define the RCS model (RCS=A*B1*B2) for a scattering point of a target, when the target type is vehicle, large size UAV, human with RCS model 2, AGV

  - The values/pattern of the product of component A and B1, i.e., A*B1 is given per target type, expressed in dBsm scale

  - Component A is expressed in dBsm scale. B1 is dependent on A*B1 and value of component A.

    - A is equal to a single value per target type

      - FFS: this allows different values for the same target type with different size, if needed

      - FFS: this allows different values for monostatic and bistatic sensing, if needed

  - Component B2 follows log-normal distribution. The mean μB2_dB and variance σB2_dB2 used to characterize 10lgB2 satisfied a fixed relation μB2_dB=-ln1020σB2_dB2.

- In the procedure of generating ISAC target channel, B1*B2 is applied after coupling of rays for a STX-SPST link and the corresponding SPST-SRX link before path dropping

- In the procedure of generating ISAC target channel, the following power scaling factor is applied in the last step in target channel generation (i.e., step 14 in the running CR).

LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2

Where,

    - PLdBd1 is pathloss between Tx and SPST, where d1 is the distance between Tx and SPST

    - PLdBd2 is pathloss between Rx and SPST, where d2 is the distance between SPST and Rx

    - σRCS,A is the value of RCS component A

    - SFdB,1,SFdB,2 are shadow fading respectively generated for the Tx- SPST link and SPST -Rx link referring to step 4 in section 7.5, TR 38.901

    - Note: for monostatic sensing, PLdBd1=PLdBd2,SFdB,1=SFdB,2

Agreement

RCS upper bound: k equals to 3 is adopted to derive the upper bound of RCS component B2, kσ, where σ is the standard deviation of B2 in dB

Agreement

For reducing options for reference TRs: for sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a TRP-TRP channel is:

- TRP-TRP link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

  - For InF, hUE is changed to the same height as the BS

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hUE=35m

- FFS: whether to add very low power clusters

Agreement

For vehicle with single/multiple scattering points:

- For mono-static, the RCS=A*B=A*B1*B2 for a scattering point of a vehicle is generated by

  - The values/pattern A*B1, i.e., rcsdB(θ,φ) is deterministic based on incident/scattered angles

rcsdB(θ,φ)=Gmax-min-σVdBθ+σHdB φ,σmax

Where,

σVdBθ=-min12θ-θcenterθ3dB2, σmax,

σHdB φ=-min12φ-φcenterφ3dB2, σmax,

For example, in case of vehicle with multiple scattering points:

|  | φcenter | φ3dB, n | θcenter | θ3dB,n | Gmax | σmax | Applicable Range of θ | Applicable Range of φ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Left | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Back | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Right | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Front | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |
| Roof | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] | [ ] |

  - Note: the applicable angular range is 360 degrees per row in horizontal domain in case of vehicle with multiple scattering points, and the applicable angular range is < 360 degrees per row in horizontal domain in case of vehicle with a single scattering point.

    - FFS: angular continuity

Working assumption

For modelling background channel for monostatic sensing:

Solution A: (previous Option 1)

- drop N reference point(s) following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and the reference point for a scenario.

  - The distance between Tx and the reference point, and the height of the reference point follow Gamma distribution,

drp_2D_n~Γα1, β1+c1

hrp_n~Γα2, β2+c2

  - FFS: value of N (N<4)

- FFS: whether to add additional very low power clusters

- FFS: any update to parameters e.g. angular distribution and delay spread

Agreement

The existing spatial consistency model in TR 38.901 is reused to model correlation of links between one TRP and different STs/UEs.

Agreement

Spatial consistency is not modelled at least for the following links

- Case 1: the links from/to two non-co-located TRPs

  - Link TRP1-X and link TRP2-X, where node X can be a target, a UE or another TRP

  - Link TRP1-X and link TRP2-Y, where node X, Y can be a target, a UE or another TRP

- Case 2: the two links with different LOS/NLOS condition

- Case 3: background channel for monostatic vs. any link (Tx-ST, ST-Rx, background channel) for bistatic

Agreement

For sensing scenario UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV, the reference TR to generate a UE-UE channel is

- UE-UE link of scenario UMi, UMa, InH, and InF following the option based on TR 38.901 defined in section A.3 of TR 38.858

- TRP-UE link of scenario RMa defined in section 7 of TR 38.901 by setting hBS =1.5m

- FFS: whether to add very low power clusters

Agreement

The reference TR to generate a TRP-UE channel is

| TRP | normal UE | UMi, UMa, RMa, InH, InF, UMi-AV, UMa-AV, and RMa-AV - Option 1: TRP-UE link of scenario UMi, UMa, RMa, InH, and InF in section 7 of TR 38.901 Highway and Urban grid - Option 1: P2B link of scenario Highway and Urban grid in section 6 of TR 37.885 HST - Option 1: TRP-UE link of scenario RMa in section 7 of TR 38.901 for FR1 and TRP-UE link of scenario UMa in section 7 of TR 38.901 for FR2 |
| --- | --- | --- |
| TRP | vehicle UE | Highway and Urban grid - Option 1: V2B link of scenario Highway and Urban grid in section 6 of TR 37.885 UMi, UMa, and RMa - Option 1: TRP-UE link of scenario UMi, UMa, and RMa in section 7 of TR 38.901 |
| TRP | aerial UE | UMa-AV, UMi-AV, and RMa-AV - Option 1: - TRP-aerial UE link of scenario UMa-AV, UMi-AV, and RMa-AV in section Annex A and B of TR 36.777 for FR1 - FFS reuse the channel model of scenario UMa-AV, UMi-AV, and RMa-AV of FR1 for FR2 |

Agreement

For mono-static, the following values of component A, B2 are agreed for UAV of small size

  - Component A: -12.81 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.74 dB

Agreement

For mono-static, the following values of component A, B2 are agreed for RCS model 1 of human

  - Component A: -1.37 dBsm

  - Component B1: 0 dB (already agreed in RAN1#118bis)

  - Component B2:

    - standard deviation: 3.94 dB

Working assumption

Absolute delay model (referring to 7.6.9 in TR 38.901 as starting point) is a mandatory feature for both target channel and background channel for ISAC for UMi, UMa, InH, InF

- Related model referring to Δτ values from 7-24GHz study item

Agreement

When absolute delay model Δτ is configured, it applies to all NLOS clusters in each of Tx-target and target-Rx links and background channel.

- For bistatic sensing: Different values of Δτ are separately generated for the Tx-target link, target-Rx link and the background channel

- For monostatic sensing: the same value of Δτ is used for Tx-target link and target-Rx link, and a different value of Δτ is separately generated for the background channel

Agreement

To generate the LOS ray and NLOS clusters for the multiple scattering points, each scattering point is separately handled as if a different target with single scattering point.

Agreement

- The LOS condition between Tx/Rx and each of the multiple scattering points of a same target are individually generated

- The pathloss between Tx/Rx and each of the multiple scattering points of a same target are individually generated

Agreement

For a target with single/multiple scattering points, the 3D location of each scattering point is defined in the evaluation assumptions.

Agreement

Spatial consistency is needed to model correlation of the following links from ST-UT links and UT-UT links

- Case 5: links between same UT and two nodes X/Y, subjected to correlation distance, i.e., link UT1-X and link UT1-Y, where nodes X/Y can be target or UT

- Case 6: links between same target and two nodes X/Y, subjected to correlation distance, i.e., link target1-X and link target1-Y, where nodes X, Y are different UTs

- Case 7: link X1-Y1 and link X2-Y2, subjected to correlation distance, where X1, X2, Y1, Y2 are 4 different nodes

- FFS: Spatial consistency between multiple scattering points of the same target

Agreement

Correlation type is introduced for large scale parameter, cluster specific parameter and ray specific parameter of ST-UT links and UT-UT links

- Definition of link Correlated: parameters for any two links between STs/UTs are correlated, subjected to correlation distance.

Table 4: Correlation type for links between STs/UTs

| Parameters | Correlation type |
| --- | --- |
| Delays | link Correlated |
| Cluster powers | link Correlated |
| AOA/ZOA/AOD/ZOD offset | link Correlated |
| AOA/ZOA/AOD/ZOD sign | link Correlated |
| Random coupling | link Correlated |
| XPR | link Correlated |
| Initial random phase | link Correlated |
| LOS/NLOS states | link Correlated |
| Blockage (Model A) | All-correlated |
| O2I penetration loss | All-correlated |
| Indoor distance | All-correlated |
| Indoor states | All-correlated |

- Note: it is not precluded more parameters for spatial consistency can be discussed and added in the table

Agreement

If a target is modelled with multiple scattering points,

- The number of scattering points of the target is generated in the beginning of the simulation and kept unchanged in the whole simulation

- The number and locations of the scattering points of the target (if it is a vehicle) are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- RCS values of the multiple scattering points are individually determined

Agreement

To model polarization matrix CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ of a direct/indirect path i of a scattering point of a target

  - in Rel-19 study item (e.g., UAV, human, vehicle, AGV), αi,1=αi,2=1, βi,1=βi,2=κsp,i-1, i.e.,

CPMsp,i=expjΦsp,iθθκsp,i-1expjΦsp,iθϕκsp,i-1expjΦsp,iϕθexpjΦsp,iϕϕ

Where,

    - κsp,i is XPR ratio is randomly generated by log-normal distribution per target type

    - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is uniformly distributed within -π,π

  - FFS: spatial consistency of random phase when a scattering point moves

  - FFS: whether the base station rotation procedure in 38.901 can be reused to support rotation of the target

Agreement

- For target channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB is reused to generate Tx-target link and target-Rx link

  - The power threshold for path dropping after concatenation is relaxed to [X=-40] dB

- For background channel

  - The power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., [-25 dB] is reused to generated the background channel

    - FFS: whether to add additional very low power clusters

    - FFS: The reference power for removing cluster is the min (max. Tx-target link cluster power, max. target-Rx link cluster power)

Agreement

If EO type-2 is modelled in an indirect path, only specular reflection is modeled for EO type-2

- polarization of the indirect path is product of polarization matrix of the target-Rx link, the target, and the Tx-target link, i.e., CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp

  - For the specular reflected ray generated by a EO type-2 in the Tx-target link or the target-Rx link (i.e., Tx-EO type-2-target, or target-EO type-2-Rx), CPMtx,sp or CPMsp,rx is the polarization matrix of EO type-2

  - To generate polarization matrix of EO type-2, the procedure in [R1-2409394, R1-2410648] is taken as starting point

    - FFS applicability if the surface of EO type-2 is tilted