3GPP TSG RAN WG1 #120	R1- 2501034

Athens, Greece, 17th – 21st February, 2025

Agenda item:		9.13

Source:	MediaTek Inc.

Title:	Proposals for TEI19

Document for:		Discussion and Decision

# Introduction

In this contribution the following TEI19 proposal is described:

- Counting one NZP-CSI-RS resource as multiple active NZP-CSI-RS resources

  - Supported by: MediaTek, Nokia, Apple, NTT DOCOMO, Ericsson

# 2		Counting one NZP-CSI-RS resource as multiple active NZP-CSI-RS resources

## Background and Proposal

The current specification defines rules how the number of the NZP CSI-RS resources are calculated for UE capability signaling, When the same NZP-CSI-RS resource is used in multiple CSI reporting configurations, that one resource is counted as multiple active CSI-RS resources. The different UE implementations may or may not be aligned with this assumption. The UEs may need to report conservative CSI-RS capability values per CC due high receive antenna and CSI-RS port requirement, and with CA the aggregate limit across all CCs may further limit what the UE can be configured with in a given CC. For other UE implementations, eliminating these active NZP-CSI-RS counting inefficiencies would allow for a larger number of CSI-RS configurations while maintaining the same capability value.

Therefore, at RAN1#118bis, a proposal was discussed in [1] to allow a new UE capability for a UE which may not need N times the UE processing capability for N CSI reports based on the same CSI-RS resources.

From the resulting discussion captured in [2], the main comment made by many companies was that counting simultaneous CSI resources as the same may cause a problem for some UE implementations in the case where certain reported contents are different across the N reports, which may not allow a common capability to be used. Different codebookType values (e,g, a mix of type I and type II) and reportFreqConfiguration values (e.g. some wideband some sub-band, or not all for same sub-band) were highlighted.

On the other hand, comments and support from other device vendors suggested that there are UE implementations that may not require such restriction, so it would seem reasonable to also be able to optimise for those UE implementations.

Concerns were also raised for NES R18 functionality, where different CSI reporting sub-configurations may lead to separate processing requirements in some UE implementations.

Therefore, the proponents have taken the feedback from other companies into account and developed what we hope to be an acceptable proposal below.

Proposal 1: For simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41 and 2-43, define 2 new UE capabilities:

- UE capability 1: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource, even if the CSI-RS resource is referred by N Report Settings.

- UE capability 2: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource even if the CSI-RS resource is referred by N Report Settings, but ONLY if the following reporting configuration parameters are set to the same value for each of the N reports:

  - codebookType

  - reportFreqConfiguration

Proposal 2: For simultaneous CSI-RS reception in Network Energy saving UE features 42-1/1a/1b/1c, and 42-2/2a/2b/2c, define the following new UE capability:

- UE capability 3: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource even if the CSI-RS resource is referred multiple times.

## Possible specification changes and new UE capabilities

Possible specification changes according to proposals 1 and 2 are shown below.

If a CSI-RS resource is referred N times by one or more CSI Reporting Settings not configured with higher layer parameter csi-ReportSubConfigToAddModList, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are - counted once, if the UE indicates [active NZP-CSI-RS resource counting UE capability 1], - counted once, if the UE indicates [active NZP-CSI-RS resource counting UE capability 2] and each of the N CSI Reporting settings fulfil the conditions applicable for that UE capability, specified in [TS38.306] - counted N times otherwise. For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, if a CSI-RS resource is referred X times by one of the M CSI-RS resources, where M is defined in clause 5.2.1.4.2, and/or one or two Resource Pairs, the CSI-RS resource and the CSI-RS ports within the CSI-RS resource are counted X times. For a CSI-ReportConfig containing a list of L sub-configuration(s) provided by higher layer parameter csi-ReportSubConfigToAddModList, if a CSI-RS resource is referred by M sub-configurations among N triggered sub-configurations for CSI reporting for aperiodic CSI-RS resource, or L configured sub-configurations for CSI reporting for periodic or semi-persistent CSI-RS resource, the CSI-RS resource is - counted once, if the UE indicates [active NZP-CSI-RS resource counting UE capability 3], - counted M times otherwise, and the CSI-RS ports within the CSI-RS resource are - counted once, if the UE indicates [active NZP-CSI-RS resource counting UE capability 3], - counted maxs=1MPs, P times otherwise, where P is the number of ports configured by nrofPorts and Ps is the number of CSI-RS ports in s-th sub-configuration from M sub-configurations derived from the corresponding antenna port subset indicator portSubsetIndicator according to clause 5.2.1.4.2 if configured, otherwise Ps=P .

Proposal 3: Consider the draft text proposal in this document as a starting point for the specification update.

# Conclusion

The following proposals are made:

Proposal 1: For simultaneous CSI-RS reception in UE features 2-33, 2-36, 2-40, 2-41 and 2-43, define 2 new UE capabilities:

- UE capability 1: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource, even if the CSI-RS resource is referred by N Report Settings.

- UE capability 2: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource even if the CSI-RS resource is referred by N Report Settings, but ONLY if the following reporting configuration parameters are set to the same value for each of the N reports:

  - codebookType

  - reportFreqConfiguration

Proposal 2: For simultaneous CSI-RS reception in Network Energy saving UE features 42-1/1a/1b/1c, and 42-2/2a/2b/2c, define the following new UE capability:

- UE capability 3: To allow the UE to indicate that CSI-RS ports within one CSI-RS resource, as well as the CSI-RS resource, are counted as one resource even if the CSI-RS resource is referred multiple times.

Proposal 3: Consider the draft text proposal in this document as a starting point for the specification update.

# 4	References

- R1-2408958, Proposals for TEI19, Nokia

- R1-2410830, [draft] FL Summary #2 on Rel-19 TEIs, Moderator (NTT DOCOMO, INC.)