3GPP TSG RAN WG1 #120                                   R1-2501092

Athens, Greece, February 17th - 21st, 2025

Source:	Sharp

Title:	Discussion on modulation aspects

Agenda Item:	9.4.1

Document for:	Discussion and Decision

Introduction

A new work item on Ambient IoT [1] for Rel-19 was approved in RAN#106, with objectives including the following:

- RAN1 scope: - [...] - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

In this document, we share our views on a few modulation aspects of the A-IoT physical layer channel design.

Discussion

R2D waveform

For the OFDM-based R2D waveform, the waveform generation as specified for legacy NR DL, UL and SL can be reused. For Rel-19 where UE Readers are not supported, the resource grid configuration should be up to the Reader. On the other hand, the resource grid configuration should be forward compatible with Rel-20 where UE Readers are expected to be supported, in which case, for example, there may need to be a restriction on the size of the resource grid. Since the exactly deployed topology (i.e. Topology 1 or Topology 2) should be transparent to the devices, in our view, any restriction on the resource grid should be already specified in Rel-19.

- For waveform generation of an R2D transmission (including preamble and PRDCH), OFDM baseband signal generation for legacy NR DL, UL and SL is reused.

- Configuration of the resource grid is up to the Reader in Rel-19.

Restriction(s) on the resource grid, if any, should be already specified in Rel-19.

- Considering forward compatibility with future evolution of A-IoT, e.g. deployment on other bands, the SCS of 15 kHz (which was the only SCS studied in Rel-19) should not be a fixed value for R2D waveform generation. In our view, SCS should be determined on a per-band basis.

For forward compatibility with future evolution of A-IoT e.g. deployment on other bands, SCS for R2D waveform generation should not be a fixed value.

- SCS for R2D waveform generation is determined on a per-band basis.

R2D bandwidth

The following alternatives were captured in the TR [2] for R2D bandwidths:

- Transmission bandwidth, Btx,R2D from a reader perspective: The frequency resources used for transmitting R2D. For an OFDM-based waveform with subcarrier spacing of 15 kHz, Btx,R2D ≤ twelve PRBs, and would be down-selected among: - Alt 1: Including 180 kHz, 360 kHz - Alt 2: Integer multiple(s) of 180 kHz - Alt 3: Integer multiple(s) of the subcarrier spacing

Considering the agreed minimum R2D bandwidths for each associated M values are in integer number of PRBs, we think Alt 3 above has been excluded. For a decision between Alt-1 and Alt-2, since in Rel-19 the device is not sensitive to the actual R2D bandwidth of an R2D transmission, we think it suffices to assume, from a Reader perspective, that Alt-2 is followed, leaving the actual R2D bandwidth value to the Reader implementation.

- Regarding R2D bandwidths from a Reader perspective, Alt-2 (i.e. integer multiple(s) of 180 kHz) is assumed in Rel-19.

Values of M

Regarding the values of M, at least the chip length of R2D start-indicator-part (SIP) does not need to follow that of the corresponding R2D clock-acquisition-part (CAP) or PRDCH, in order to create a R2D SIP with multiple rising and/or falling edges within a constant time length (which is independent of the chip length of R2D CAP or PRDCH). In that sense, in our view it is necessary to allow multiple values of M within one OFDM symbol.

R2D chip length for R2D start-indicator-part (SIP) should correspond to a relatively large value of M, even if the value of M for the corresponding clock-acquisition-part (CAP) or PRDCH is small.

- For R2D, different chips within one OFDM symbol can correspond to different values of M.

CP handling

Regarding CP handling, the following alternatives were captured in the TR [2],

Method Type 1: Removal of CP at device without specified transmit-side. Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality.

At least for small values of M (e.g. with chip lengths larger than CP duration), our observation of companies’ results captured in the TR [2] is that Method Type 1 is sufficient, and choice of Alt M1-1 or Alt M1-2, or any potential combination of Alt M1-1 and Alt M1-2, could be left to device implementation.

At least for small values of M (e.g. with chip lengths larger than CP duration), CP handling Method Type 1 is sufficient, and choice of Alt M1-1 or Alt M1-2, or any potential combination of Alt M1-1 and Alt M1-2, could be left to device implementation.

- At least for small values of M (e.g. with chip lengths larger than CP duration), adopt CP handling Method Type 1.

For large values of M (e.g. with chip lengths comparable to or smaller than CP duration), the results captured in the TR [2] are quite divergent, and we think some more discussions are necessary in RAN1 before making a decision.

For large values of M (e.g. with chip lengths comparable to or smaller than CP duration), more discussions are necessary on whether to adopt CP handling Method Type 1 or CP handling Method Type 2.

D2R waveform

In UHF RFID standards, a Tag can choose which of ASK modulation or PSK modulation to use for backscattering. In A-IoT D2R, RAN1 should discuss whether to allow the device to choose which of OOK modulation and BPSK modulation to use for backscattering, or to allow the Reader to fully control which modulation scheme to use.

- RAN1 to discussion the following two options for D2R modualtion:

- Option 1: the device can choose which of OOK modulation and BPSK modulation to use for D2R backscattering.

- Option 2: the Reader has full control of which of OOK modulation and BPSK modulation to use for D2R backscattering.

Since only double-side-band modulation is concluded in the study item phase to be feasible for D2R transmission for all devices, it should be the only option to be specified in Rel-19.

- Double-side-band modulation is used for D2R backscattering.

D2R bandwidth

Regarding D2R bandwidth, the following alternatives were discussed during the study item phase,

For Btx,D2R of the D2R transmissions associated with one/each single-tone of a carrier wave, it can be: Alt 1: An integer number of PRBs Alt 2: An integer multiple of SCS

In our view, Alt 2 (which includes Alt 1 as a special case) is more beneficial in terms of both spectrum efficiency, coverage, and flexibility in D2R data rates, and hence should be adopted for D2R bandwidth design. A set of D2R bandwidths should be pre-defined for backscattering transmissions in Rel-19. More D2R bandwidths may be considered (e.g. for Device 2) in future releases.

- A set of D2R bandwidths (including a number of values as integer multiple of SCS and other values as integer multiple of PRBs) is pre-defined for backscattering transmissions in Rel-19.

Conclusion

In this contribution, we discuss a few modulation aspects of the A-IoT physical layer channel design, and make the following observations and proposals.

- Restriction(s) on the resource grid, if any, should be already specified in Rel-19.

For forward compatibility with future evolution of A-IoT e.g. deployment on other bands, SCS for R2D waveform generation should not be a fixed value.

- R2D chip length for R2D start-indicator-part (SIP) should correspond to a relatively large value of M, even if the value of M for the corresponding clock-acquisition-part (CAP) or PRDCH is small.

- At least for small values of M (e.g. with chip lengths larger than CP duration), CP handling Method Type 1 is sufficient, and choice of Alt M1-1 or Alt M1-2, or any potential combination of Alt M1-1 and Alt M1-2, could be left to device implementation.

For large values of M (e.g. with chip lengths comparable to or smaller than CP duration), more discussions are necessary on whether to adopt CP handling Method Type 1 or CP handling Method Type 2.

- For waveform generation of an R2D transmission (including preamble and PRDCH), OFDM baseband signal generation for legacy NR DL, UL and SL is reused.

- Configuration of the resource grid is up to the Reader in Rel-19.

- SCS for R2D waveform generation is determined on a per-band basis.

- Regarding R2D bandwidths from a Reader perspective, Alt-2 (i.e. integer multiple(s) of 180 kHz) is assumed in Rel-19.

- For R2D, different chips within one OFDM symbol can correspond to different values of M.

- At least for small values of M (e.g. with chip lengths larger than CP duration), adopt CP handling Method Type 1.

- RAN1 to discussion the following two options for D2R modualtion:

- Option 1: the device can choose which of OOK modulation and BPSK modulation to use for D2R backscattering.

- Option 2: the Reader has full control of which of OOK modulation and BPSK modulation to use for D2R backscattering.

- Double-side-band modulation is used for D2R backscattering.

- A set of D2R bandwidths (including a number of values as integer multiple of SCS and other values as integer multiple of PRBs) is pre-defined for backscattering transmissions in Rel-19.

References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), RAN#106.

- TR 38.769, “Study on Solutions for Ambient IoT (Internet of Things)”, v19.0.0.