3GPP TSG RAN WG1 #120			R1-2501138

Athens, Greece, February 17th – 21st, 2025

Agenda item:	5

Source: 	Qualcomm Incorporated

Title: 	Discussion on response to RAN2 LS on PWS support in RRC_CONNECTED for NB-IoT NTN

Document for:	Discussion and Decision

# Background

In [1], RAN2 inquired whether an NB-IoT NTN UE can receive PWS notifications in RRC_CONNECTED mode:

RAN2 discussed whether to introduce support for reception of PWS (ETWS and CMAS) notifications in RRC_CONNECTED for NB-IoT but were not able to conclude.

PWS notifications are delivered via paging or direct indication to trigger acquiring ETWS system information and CMAS system information.

Currently NB-IoT UEs are not required to monitor for paging or receive system information triggered by the paging in RRC_CONNECTED, and RAN2 would like to understand from RAN1 perspective whether it is feasible to introduce this in Rel-19.

ACTION:	 Please respond with an indication of whether it is feasible from RAN1 perspective to introduce support in Rel-19 for NB-IoT UEs reception of PWS notifications (in paging, direct indication, and system information) while in RRC_CONNECTED.

In this contribution, we present our views on this issue.

# Discussion

As explained in the RAN2 LS, currently the UE is not required to monitor the NPDCCH search space and/or SIBs during RRC_CONNECTED mode. The main reason to do this back in Rel-13 was to reduce the UE complexity in terms of parallel monitoring of search spaces and number of decodes / DCI sizes to handle.

Supporting the monitoring of PWS notifications in RRC_CONNECTED would be beneficial for the following reasons::

- Currently, in the absence of monitoring PWS notifications during RRC_CONNECTED, the eNB would need to release all UEs to RRC_IDLE for them to receive the PWS message.

- Releasing all connected UEs may result in a very large overhead for the eNB (due to the need to send one release message to each individual UE), which will increase the delay in acquiring the PWS notification.

- Furthermore, due to releasing the RRC connection, the ongoing session may be terminated, impacting UE experience for some use cases and potentially becoming critical if voice support is introduced in Rel-20.

From RAN1 perspective, it would be feasible to specify monitoring of CSS to receive PWS during connected mode, as long as it does not increase the UE complexity. For example, if the USS and CSS are TDM'd (i.e., there is no simultaneous monitoring of both), the UE could monitor both without incurring additional UE complexity.

Proposal 1: RAN1 specifies mechanisms for the UE to monitor ETWS/CMAS notification in RRC_CONNECTED mode

- These mechanisms shall not increase UE complexity.

- As a baseline, the CSS for PWS notification and USS are TDM’d, with a minimum separation of 4ms between them.

Once the notification of PWS message is received in CSS, the UE is required to decode SIB to receive the message. For this, RAN2 can actually define similar mechanisms as in reacquisition of SIB31 to have the UE decode the PWS-SIB while in RRC_CONNECTED. Note that the PWS-SIB will only be sent in the anchor carrier; thus, in general, the UE will need to retune to receive the notification (and would, therefore, be unable to monitor CSS).

Proposal 2: For the decoding of SIBs in RRC_CONNECTED, RAN1 recommends RAN2 to follow a similar approach as with reacquisition of SIB31.

It is also proposed to send the above agreements in an LS to RAN2, as follows:

Proposal 3: RAN1 to reply to RAN2 LS in R2-2411194 as follows:

- In Rel-19, In Rel-19, RAN1 will specify mechanisms for monitoring CSS carrying ETWS/CMAS notifications in RRC_CONNECTED

- For the decoding of SIBs in RRC_CONNECTED, RAN1 recommends RAN2 to follow a similar approach as with reacquisition of SIB31

# Summary

In this contribution we presented our views on the LS in [1]. We made the following proposals:

Proposal 1: RAN1 specifies mechanisms for the UE to monitor ETWS/CMAS notification in RRC_CONNECTED mode

- These mechanisms shall not increase UE complexity.

- As a baseline, the CSS for PWS notification and USS are TDM’d, with a minimum separation of 4ms between them.

Proposal 2: For the decoding of SIBs in RRC_CONNECTED, RAN1 recommends RAN2 to follow a similar approach as with reacquisition of SIB31.

Proposal 3: RAN1 to reply to RAN2 LS in R2-2411194 as follows:

- In Rel-19, In Rel-19, RAN1 will specify mechanisms for monitoring CSS carrying ETWS/CMAS notifications in RRC_CONNECTED

- For the decoding of SIBs in RRC_CONNECTED, RAN1 recommends RAN2 to follow a similar approach as with reacquisition of SIB31

# References

[1] R2-2411194, “LS on PWS support in RRC_CONNECTED for NB-IoT NTN”