3GPP TSG RAN WG1 #120			R1-2501166

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.7.1

Source: 	Qualcomm Incorporated

Title: 	ISAC Deployment Scenarios

Document for:		Discussion

# Introduction

At RAN1 #119, the following agreements were made related to ISAC deployment scenarios:

Guidance for further work 1. Rapporteurs are encouraged to start providing a draft CR for both agendas to RAN1#120 2. Jerome to provide an initial proposal for calibrations discussions by the end of RAN1#119 3. RAN1 agenda will clarify that input on calibrations discussions is to be provided to agenda 9.7.1 starting at RAN1#120 Agreement For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-UAV Details on ISAC-UAV scenarios are listed in Table x. Table x. Evaluation parameters for UAV sensing scenarios Parameters; Value. Applicable communication scenarios; UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV. Sensing transmitters and receivers properties; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario.. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations.. 3D distribution; Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx. Orientation; Random in horizontal domain. Physical characteristics (e.g., size); Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s). Minimum 3D distances between pairs of Tx/Rx and sensing target; Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a target Option 2: 10 meters. [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.]; FFS. NOTE: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Automotive Details on ISAC-Automotive scenarios are listed in Table x. Table x. Evaluation parameters for Automotive sensing scenarios Parameters; Values. Applicable communication scenarios; Highway, Urban Grid. NOTE1. Sensing transmitters and receivers properties; Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m. Sensing target; LOS/NLOS; LOS and NLOS (including NLOSv). Outdoor/indoor; Outdoor. Mobility (horizontal plane only); Based on TR37.885 mobility for urban grid or highway scenario. Distribution (horizontal); Based on dropping in TR37.885 per urban grid or highway communication scenario. Orientation; Lane direction in horizontal plane. Physical characteristics (e.g., size); Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc.. NOTE1: calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Human Details on ISAC-Human scenarios are listed in Table x. Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios Parameters; Indoor Values; Outdoor Values. Applicable communication scenarios NOTE1; Indoor office, indoor factory [TR38.901] Indoor room [TR38.808]; UMi, Uma, RMa [TR38.901]. Sensing transmitters and receivers properties; Rx/Tx Locations NOTE 2; Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario; Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario. Rx/Tx Mobility for UEs; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr. Sensing target; LOS/NLOS; LOS and NLOS; LOS and NLOS. Outdoor/indoor; Indoor; Outdoor. 3D mobility; Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory); Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory). 3D distribution; N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm; Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm. Orientation; Random over the horizontal area; Random over the horizontal area. Physical characteristics (e.g., size); Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m; Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS. Environment Objects, e.g., types, characteristics, mobility, distribution, etc.; FFS, based on outcome for AI 9.7.2; FFS, based on outcome for AI 9.7.2. NOTE1: For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. Agreement For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-AGV Details on ISAC-AGV are listed in Table x. Table x. Evaluation parameters for Automated Guided Vehicles Parameters; Value. Applicable communication scenarios NOTE1; InF (TR38.901 including Table 7.8-7). Sensing transmitters and receivers properties NOTE2; Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Indoor. 3D mobility; Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h. 3D distribution; Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane. Orientation; Horizontal plane only. Physical characteristics (e.g., size); Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS. Environment objects, e.g., types, characteristics, mobility, distribution, etc.; FFS. NOTE1: For the AGV sensing targets, additional communication scenarios can be considered for future evaluations. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. NOTE3: RAN1 can further discuss narrowing down the number of sub-scenarios of InF Agreement For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline: The detailed scenario description in this clause can be used for channel model calibration. ISAC-Hazards Details on ISAC-Hazards are listed in Table x. Table x. Evaluation parameters for objects creating hazards Parameters; Value. Applicable communication scenarios NOTE1; Highway, Urban grid, HST (High Speed Train). Sensing transmitters and receivers properties NOTE2; Rx/Tx Locations; Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m. Sensing target; LOS/NLOS; LOS and NLOS. Outdoor/indoor; Outdoor. 3D mobility; Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory. 3D distribution; Uniformly distributed in horizontal plane. Orientation; Random distribution in horizontal plane. Physical characteristics (e.g., size); For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations. Minimum 3D distances between pairs of Tx/Rx and sensing target; Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx. Minimum 3D distance between sensing targets; Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS. Environment objects, e.g., types, characteristics, mobility, distribution, etc.; EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc.. NOTE1: For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations. NOTE2: A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations. [Post-119-ISAC-01] – Jerome (AT&T) Email discussion on simulation assumptions for channel model calibration, from January 8th-17th: - Companies are encouraged to provide initial channel model calibration assumptions, parameters, and values - Jerome to collect inputs, provide potential consolidated tables, proposals for consideration by January 17th Decision: The discussions are to be carried over to RAN1#120.

In this contribution, we summarize our views on the next steps on ISAC deployment scenarios.

# UAV scenarios

With regards to the Evaluation parameter UAV sensing scenarios, we propose the following updates:

| Parameter | Preferred value |
| --- | --- |
| 3D distribution in the horizontal plane | Option A: N targets uniformly distributed within one cell |
| 3D distribution in the vertical plane | Option A: Uniform between 1.5m and 300m |
| sizes of UAV | make Option 2 (0.3m x 0.4m x 0.2m) as baseline |
| minimum 3D distances between sensing target | Support Option 1 (At least larger than the physical size of a target) |

Proposal 1: Support the following updates in the Evaluation parameter table for UAV sensing scenarios:

- 3D distribution in the horizontal plane, support:

  - Option A: N targets uniformly distributed within one cell

- 3D distribution in the vertical plane, support:

  - Option A: Uniform between 1.5m and 300m

- sizes of UAV:

  - make Option 2 (0.3m x 0.4m x 0.2m) as baseline

- minimum 3D distances between sensing target:

  - Support Option 1 (At least larger than the physical size of a target)

Proposal 2: For the UAV Evaluation parameter table, with regards to the Unintended objects we propose the following:

- For the purpose of sensing the i-th target, the N-1 remaining uniformly distributed targets in the cell are considered an unintended objects

- Do not introduce Environment Objects Type 1 or Type 2

# Humans scenarios

With regards to the Evaluation parameter for humans sensing scenarios, we propose the following updates:

| Parameter | Preferred value |
| --- | --- |
| Rx/Tx mobility | Option 3 |
| 3D mobility | Option 3 |
| 3D distribution in indoor | N=[1, 5] targets |
| Environment Objects in outdoor | Do not include any Environment object |
| Environment Objects in indoor | Include as an optional scenario, one or more of the: - 4 outside walls, - the ceiling and - the ground floor FFS how many EOs are needed depending on the scenario of the indoor office, factory, indoor room |

Proposal 3: Support the following updates in the Evaluation parameter table for humans sensing scenarios:

- For the Rx/Tx mobility,

  - support Option 3

- For the 3D mobility,

  - support Option 3

- For the 3D distribution in indoor,

  - support N=[1, 5] targets

- For the 3D distribution in outdoor,

  - Option A: N targets uniformly distributed within one cell

- For the EO in outdoor

  - Do not include any Environment object

- For the EO in indoor

  - Include as an optional scenario, one or more of the:

    - 4 outside walls,

    - the ceiling and

    - the ground floor

# Automotive vehicles scenarios

With regards to the Evaluation parameter for automotive vehicles scenarios, we propose the following updates:

| Parameter | Preferred value |
| --- | --- |
| minimum 3D distances between targets | Option 1 |
| Environment Objects | - Add as an optional component Precipitation modelling according to the LS (R1-2405964) - Add EO Type-2 according to the urban grid layout of 37.885 with the following details: - 4 walls (as EO type 2) per building of size 413m X 230m X Hm (FR1) and 186 m X 72 X Hm (FR2), where H > BS Height - EO Type 2 should be an optional modelling component in urban grid (blockage & pathloss is still modelled) |

Proposal 4: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- With regards to the minimum 3D distances between targets:

  - Support Option 1

- With regards to the environment objects:

  - EO Type 2 should be an optional modelling component in urban grid (blockage & pathloss is still modelled)

  - 4 walls (as EO type 2) per building of size 413m X 230m X Hm, where H > BS Height

# Automated guided vehicles scenarios

With regards to the Evaluation parameter for automated guided vehicle scenarios, we propose the following updates:

| Parameter | Preferred value |
| --- | --- |
| Rx/Tx Mobility for UEs | Option 3: Uniform distribution between 0km/h and 3km/h |
| 3D mobility | Option 1: Uniform distribution in the range of up to 30 km/h |
| 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment |
| Minimum 3D distance between sensing targets | Option A: At least larger than the physical size of a target |
| Environment Objects | Include as an optional scenario, one or more of the: - 4 outside walls, - the ceiling and - the ground floor FFS how many EOs are needed |

Proposal 5: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- Rx/Tx Mobility for UEs

  - Option 3: Uniform distribution between 0km/h and 3km/h

- 3D mobility

  - Option 1: Uniform distribution in the range of up to 30 km/h

- 3D distribution

  - Option A: Uniformly distributed in the convex hull of the horizontal BS deployment

- Minimum 3D distance between sensing targets

  - Option A: At least larger than the physical size of a target

- Environment Objects

  - Include as an optional scenario, one or more of the:

    - 4 outside walls,

    - the ceiling and

    - the ground floor

# Objects creating hazards on roads/railways scenarios

With regards to the Evaluation parameter for automated guided vehicle scenarios, we propose the following updates:

| Parameter | Preferred value |
| --- | --- |
| 3D mobility | Horizontal velocity: up to 10 km/h for humans and animals |
| Minimum 3D distance between sensing targets | Option 1: At least larger than the physical size of a target |
| Environment Objects | - Add EO Type-2 according to the urban grid layout of 37.885 with the following details: - 4 walls (as EO type 2) per building of size 413m x 230m x Hm, where H > BS Height |

Proposal 6: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- 3D mobility

  - Horizontal velocity: up to 10 km/h for humans and animals

- Minimum 3D distance between sensing targets

  - Option 1: At least larger than the physical size of a target

- Environment Objects

  - Add EO Type-2 according to the urban grid layout of 37.885 with the following details:

  - 4 walls (as EO type 2) per building of size 413m X 230m X Hm (FR1) and 186m X 72m X Hm (FR2), where H > BS Height

# Channel Model Calibration

The following high-level channel model calibration considerations are made related to ISAC channel modelling calibration:

- Support both large scale calibration and full calibration

- Spatial consistency calibration can be an optional channel model calibration effort.

- All 5 sensing target types identified in the SID should be represented in new ISAC channel model calibration clause(s) similar to TR 38.901 section 7.8.

Proposal 7: Support the following high level principles for channel model calibration:

- Support both large scale calibration and full calibration

- All 5 sensing target types identified in the SID should be represented in new ISAC channel model calibration clause(s) similar to TR 38.901 section 7.8.

# Conclusion

We have discussed ISAC deployment scenarios and made the following proposals:

Proposal 1: Support the following updates in the Evaluation parameter table for UAV sensing scenarios:

- 3D distribution in the horizontal plane, support:

  - Option A: N targets uniformly distributed within one cell

- 3D distribution in the vertical plane, support:

  - Option A: Uniform between 1.5m and 300m

- sizes of UAV:

  - make Option 2 (0.3m x 0.4m x 0.2m) as baseline

- minimum 3D distances between sensing target:

  - Support Option 1 (At least larger than the physical size of a target)

Proposal 2: For the UAV Evaluation parameter table, with regards to the Unintended objects we propose the following:

- For the purpose of sensing the i-th target, the N-1 remaining uniformly distributed targets in the cell are considered an unintended objects

- Do not introduce Environment Objects Type 1 or Type 2

Proposal 3: Support the following updates in the Evaluation parameter table for humans sensing scenarios:

- For the Rx/Tx mobility,

  - support Option 3

- For the 3D mobility,

  - support Option 3

- For the 3D distribution in indoor,

  - support N=[1, 5] targets

- For the 3D distribution in outdoor,

  - Option A: N targets uniformly distributed within one cell

- For the EO in outdoor

  - Do not include any Environment object

- For the EO in indoor

  - Include as an optional scenario, one or more of the:

    - 4 outside walls,

    - the ceiling and

    - the ground floor

Proposal 4: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- With regards to the minimum 3D distances between targets:

  - Support Option 1

- With regards to the environment objects:

  - EO Type 2 should be an optional modelling component in urban grid (blockage & pathloss is still modelled)

  - 4 walls (as EO type 2) per building of size 413m X 230m X Hm, where H > BS Height

Proposal 5: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- Rx/Tx Mobility for UEs

  - Option 3: Uniform distribution between 0km/h and 3km/h

- 3D mobility

  - Option 1: Uniform distribution in the range of up to 30 km/h

- 3D distribution

  - Option A: Uniformly distributed in the convex hull of the horizontal BS deployment

- Minimum 3D distance between sensing targets

  - Option A: At least larger than the physical size of a target

- Environment Objects

  - Include as an optional scenario, one or more of the:

    - 4 outside walls,

    - the ceiling and

    - the ground floor

Proposal 6: Support the following updates in the evaluation parameter table for automotive vehicle sensing scenarios:

- 3D mobility

  - Horizontal velocity: up to 10 km/h for humans and animals

- Minimum 3D distance between sensing targets

  - Option 1: At least larger than the physical size of a target

- Environment Objects

  - Add EO Type-2 according to the urban grid layout of 37.885 with the following details:

  - 4 walls (as EO type 2) per building of size 413m X 230m X Hm (FR1) and 186m X 72m X Hm (FR2), where H > BS Height

Proposal 7: Support the following high level principles for channel model calibration:

- Support both large scale calibration and full calibration

- All 5 sensing target types identified in the SID should be represented in new ISAC channel model calibration clause(s) similar to TR 38.901 section 7.8.

# References

- R1-2101456, “ISAC Channel modeling”, Qualcomm Incorporated, RAN1#116

- A. Herschfelt et al., "Consumer-grade drone radar crosssection and microDoppler phenomenology," 2017 IEEE Radar Conference (RadarConf), 2017, pp. 0981-0985.

- O. Bshara, Y. Liu and K. R. Dandekar, "Radar Cross Section Measurement Comparison of UAVs at C-band and V-band," 2019 IEEE 20th Wireless and Microwave Technology Conference (WAMICON), 2019, pp. 1-6

- R1-2405964, “LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC”, 5GAA LS Working Group 4