3GPP TSG-RAN WG1 Meeting #120	R1- 2501302

Athens, Greece, February  17th –21nd, 2025

Agenda Item:	5

Source:	Ericsson

Title:	Discussion for SA2 LS on LMF-based AI/ML Positioning for Case 2b

Document for:	Discussion

# Introduction

In the SA2 LS “LS on LMF-based AI/ML Positioning for Case 2b” SA2 requests the following to RAN1:

LS from R1-2500388 (S2-2501133) # 1 Overall description WID AI/ML_CN in SA2 has defined the new feature that LMF-based AI/ML Positioning in R19, covering Case 2b as studied in TR 38.843 by RAN WGs (Case 3b is also covered). The work done by SA2 includes at least the following aspects: - As defined in TS 23.273, the LMF may perform location calculation based on ML model, which was trained by NWDAF containing MTLF or the LMF. - AS defined in TS 23.288, the LMF may retrieve ML model for AI/ML Positioning from NWDAF containing MTLF. - AS defined in TS 23.273, the LMF collects input data from RAN and UE and may also expose to NWDAF to train or monitor the performance of the ML models for LMF-based AI/ML positioning. To RAN 1 and RAN 2: For case 2b, SA2 is working on some procedures in corresponding R19 specifications, but has also captured the following Editor’s Notes on the interaction between the LMF and UE which depend on RAN progress: Editor's note: The input data used for AI/ML based positioning will be decided by RAN WGs. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs. Q1: SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference. # 2 Actions ACTION: To RAN 1, RAN2 - SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

# discussion

Regarding SA2 inquiry in the LS, RAN1 can reply to SA2 LS by providing the existing agreements made on training data collection, model inference and monitoring for Case 2b. Additionally, RAN1 should point out that Case 2b is considered 2nd priority in RAN Rel-19 work item NR_AIML_air (see RP-243244), and it is unlikely that the work on Case 2b will be completed within Rel-19 timeline.

RAN1 reply to SA2 LS S2-2501133 by providing the existing agreements made on training data collection, model inference and monitoring for Case 2b.

In RAN1 response to SA2 LS S2-2501133, add a note: “Case 2b is considered 2nd priority in RAN Rel-19 work item NR_AIML_air (see RP-243244). It is unlikely that the work on Case 2b will be completed within Rel-19 timeline.”

RAN1 agreements on model input which is applicable to both training data collection and model inference:

Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. Agreement - For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF: - timing information; - paired timing information and power information. Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

RAN1 agreements on training data collection for Case 2b:

- Entities for training data generation:

Working Assumption For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE. Working Assumption For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope.

- Content of training data samples:

Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

RAN1 agreements on monitoring for Case 2b:

Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model.

# References

R1-2500388  LS on LMF-based AI/ML Positioning for Case 2b,  SA2