3GPP TSG-RAN WG1 Meeting #120	R1-2501516

Athens, Greece, February 17th - 21st, 2025

Agenda item:		7

Source:		Moderator (Huawei)

Title:	Moderator summary on discussion of p0-nominal

Document for:		Discussion and Decision

- Introduction

This document is a summary of discussions related to [1][2]. The discussion baseline is the summary for the issue from previous FL summary in [3].

Please Note that in this meeting, the proposed CR is from Rel-17 instead of from Rel-16 to address the potential concern on impact on UE implementation.

TS 38.213 v16.17.0 ### 7.1.1 UE behaviour If a UE transmits a PUSCH on active UL BWP of carrier of serving cell using parameter set configuration with index and PUSCH power control adjustment state with index , the UE determines the PUSCH transmission power in PUSCH transmission occasion as [dBm] where, - is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3] for carrier of serving cell in PUSCH transmission occasion . - is a parameter composed of the sum of a component and a component where . - If a UE established dedicated RRC connection using a Type-1 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet or for a PUSCH (re)transmission corresponding to a RAR UL grant as described in clause 8.3, , , and , where PO_PRE is provided by preambleReceivedTargetPower [11, TS 38.321] and ΔPREAMBLE_Msg3 is provided by msg3-DeltaPreamble, or dB if msg3-DeltaPreamble is not provided, for carrier of serving cell - If a UE established dedicated RRC connection using a Type-2 random access procedure, as described in clause 8, and is not provided P0-PUSCH-AlphaSet, or for a PUSCH transmission for Type-2 random access procedure as described in clause 8.1A, j=0, PO_UE_PUSCH,b,f,c(0)=0, and PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH, where PO_PRE is provided by msgA-preambleReceivedTargetPower, or by preambleReceivedTargetPower if msgA-preambleReceivedTargetPower is not provided and ΔMsgA_PUSCH is provided by msgA-DeltaPreamble, or ΔMsgA_PUSCH=ΔPREAMBLE_Msg3 dB if msgA-DeltaPreamble is not provided, for carrier f of serving cell c - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, , is provided by p0-NominalWithoutGrant, or if p0-NominalWithoutGrant is not provided, and is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet for active UL BWP of carrier of serving cell - For , a value, applicable for all , is provided by p0-NominalWithGrant, or if p0-NominalWithGrant is not provided, for each carrier of serving cell and a set of values are provided by a set of p0 in P0-PUSCH-AlphaSet indicated by a respective set of p0-PUSCH-AlphaSetId for active UL BWP of carrier of serving cell

According to the text highlighted in cyan above, a UE needs to apply PO_NOMINAL_PUSCH,f,c(0) to determine the transmit power of CG PUSCH/DG PUSCH in some cases e.g. P0-PUSCH-AlphaSet is not configured or p0-NominalWithoutGrant/p0-NominalWithGrant is not configured.

According to the text highlighted in yellow above, when 2-step RACH is configured in SIB1, there are two values PO_NOMINAL,PUSCH,f,c0, i.e. for 4-step RACH PO_NOMINAL,PUSCH,f,c0=PO_PRE+∆PREAMBLE,Msg3 while for 2-step RACH PO_NOMINAL_PUSCH,f,c(0)=PO_PRE+ΔMsgA_PUSCH.

Issue 1

According to S7.1.1 of TS 38.213, the open-loop power control of DG or CG PUSCH relies on the value of PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithGrant/p0-NominalWithoutGrant is not configured in a cell but 2-step RACH resources are configured. For EN-DC, the UE established dedicated RRC connection by LTE RACH which does not belong to NR type 1 or type 2 RACH. In this case, it is unclear how to determine PO_NOMINAL,PUSCH,f,c0 for a NR PSCell with the highlight texts above, e.g. “If a UE established dedicated RRC connection using a Type-1 random access procedure”. As a result, there is no PO_NOMINAL,PUSCH,f,c0 to be applied for CG/DG PUSCH if P0-PUSCH-AlphaSet is not configured in the Cell e.g. NR PSCell.

- In the current specification, it is unclear how to determine PO_NOMINAL,PUSCH,f,c0 for a cell of SCG in EN-DC case. As a result, if parameter P0-PUSCH-AlphaSet is not configured in the cell, it is unclear how to determine PO_NOMINAL,PUSCH,f,cj for CG/DG PUSCH which is equal to PO_NOMINAL,PUSCH,f,c0 in this case.

Issue 2:

For a PCell configured without CA/DC but with 2-step RACH feature, a similar issue as above occurs if P0-PUSCH-AlphaSet, p0-NominalWithoutGrant or p0-NominalWithGrant is not configured.

As shown in Figure 1 as an example, a UE may trigger 2-step RACH procedure first for e.g. RRC connection setup, and later use 4-step RACH for multiple procedures, including RRC reconfiguration, procedures of SR failure or UL data arrival during RRC_CONNECTED mode when there are no PUCCH resources for SR available. It is not clear whether the PO_NOMINAL,PUSCH,f,c0 for 4-step RACH or the PO_NOMINAL,PUSCH,f,c0 for 2-step RACH should be used for CG/DG PUSCH.

For the transmit power of the 1st CG/DG PUSCH shown in Figure 1 below, PO_NOMINAL,PUSCH,f,c0 for 2-step RACH (i.e. msgA PUSCH PO_NOMINAL,PUSCH,f,c0) should be used to determine its transmit power, because the UE has not performed 4-step RACH yet and it does not make sense to use PO_NOMINAL,PUSCH,f,c0 for 4-step RACH (i.e. msg3 PUSCH PO_NOMINAL,PUSCH,f,c0) in this case.

However, for the transmit power of the 2nd CG/DG PUSCH shown in Figure 1 below, it is not clear whether to reuse the same PO_NOMINAL,PUSCH,f,c0 as the previous 1st CG/DG PUSCH (i.e. msgA PUSCH PO_NOMINAL,PUSCH,f,c0), or to use the new  PO_NOMINAL,PUSCH,f,c0 (i.e. msg3 PUSCH PO_NOMINAL,PUSCH,f,c0).

Figure 1 one example for  PO_NOMINAL,PUSCH,f,c0 determination for CG/DG PUSCH if P0-PUSCH-AlphaSet is not configured or p0-NominalWithoutGrant/p0-NominalWithGrant is not configured

In summary, when 2-step RACH is configured and PO_NOMINAL,PUSCH,f,c0 of 2-step RACH is different from PO_NOMINAL,PUSCH,f,c0 of 4-step RACH, PO_NOMINAL,PUSCH,f,c0 may be updated from 2-step RACH to 4-step RACH or vice versa. Once the PO_NOMINAL,PUSCH,f,c0 is updated, it is not clear whether CG/DG PUSCH use the old PO_NOMINAL,PUSCH,f,c0 or the new PO_NOMINAL,PUSCH,f,c0.

- if p0-NominalWithoutGrant/p0-NominalWithGrant is not configured, once the PO_NOMINAL,PUSCH,f,c0 is updated from 2-step RACH to 4-step RACH or vice versa, it is not clear which one PO_NOMINAL,PUSCH,f,c0 (previous one or the updated one) should be used for CG/DG PUSCH.

To address these two issues, two alternative solutions can be,

- For a cell configured with 2-step RACH feature, adopt one of the following options:

- Option 1: the most recent PO_NOMINAL,PUSCH,f,c0 is used for CG/DG PUSCH if P0-PUSCH-AlphaSet is not configured or p0-NominalWithoutGrant/p0-NominalWithGrant is not configured.

- Option 2: When UE dedicated RRC configuration is provided to a UE for the cell, all the three parameters P0-PUSCH-AlphaSet, p0-NominalWithoutGrant and p0-NominalWithGrant are expected to be configured.

- For a cell configured with 2-step RACH feature, adopt either one of CRs in [2].

- Discussion

According to the previous discussion, it seems the following are discussion points:

Question 1

Whether the text “established dedicated RRC connection” in S7.1.1 of TS 38.213 includes NR RRC connection establishment in S5.3.3 of TS 38.331 (see below), or the texts in S7.1.1 of TS 38.213 does not account for EN-DC case.

TS 38.331 ### 5.3.3 RRC connection establishment #### 5.3.3.1 General Figure 5.3.3.1-1: RRC connection establishment, successful Figure 5.3.3.1-2: RRC connection establishment, network reject The purpose of this procedure is to establish an RRC connection. RRC connection establishment involves SRB1 establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to the network. The network applies the procedure e.g.as follows: - When establishing an RRC connection; - When UE is resuming or re-establishing an RRC connection, and the network is not able to retrieve or verify the UE context. In this case, UE receives RRCSetup and responds with RRCSetupComplete.

Option 1-1: the text “established dedicated RRC connection” in S7.1.1 of TS 38.213 includes NR RRC connection establishment in S5.3.3 of TS 38.331.

Option 1-2: the texts in S7.1.1 of TS 38.213 does not account for EN-DC case and no further RAN1 spec update to be pursued for EN-DC.

| Company | View |
| --- | --- |
| MTK | We prefer Option 1-2 as we tend to think EN-DC was considered when developing this paragraph of 38.212 spec. |

Question 2

Whether the selected parameters can be up to UE implementation, or up to gNB (in the latter case, some conclusion or spec change is preferred)

Moderator understanding is UE implementation does not work since the gNB does not know which parameter to use. In the example below for the 2nd CG/DG PUSCH, when up to UE implementation, UE determines the P0 according to 4-step RACH configuration while gNB assumes the configuration of 2-step RACH (which is the one used for establishing dedicated RRC connection) applies.

Instead, by gNB implementation it may work by mandating gNB to configure all the three parameters P0-PUSCH-AlphaSet, p0-NominalWithoutGrant and p0-NominalWithGrant. Our preference in this case is to clarify this by spec change or at least by a conclusion.

More reasonable if flexibility is preferred, it is expected that always the most recent RA configuration applies since UE and gNB has the aligned latest configuration at both sides.

| Company | View |
| --- | --- |
| MTK | We prefer by gNB implementation to work by mandating gNB to configure all the three parameters P0-PUSCH-AlphaSet, p0-NominalWithoutGrant and p0-NominalWithGrant. Find to have spec change or draw RAN1 conclusion. |

- Conclusion

Tbd.

- References

- R1-2501067	, Discussion on p0-nominal			, 		Huawei, HiSilicon

- R1-2501320, 	Draft CRs for p0-nominal	, Huawei, HiSilicon

- R1-2410829, Summary#2 of discussion on power control for 2-step RACH, Moderator(Huawei )