3GPP TSG-RAN WG1 Meeting #120	R1-2500073

Athens, Greece, February 17th-21st, 2025

Agenda Item:	9.6.1

Source:	Huawei, HiSilicon

Title:	Signal Design of LP-WUS and LP-SS

Document for:	Discussion and Decision

# Introduction

In this contribution, various aspects on physical layer design of LP-WUS are discussed. Firstly, fundamental waveform design for LP-WUS is discussed in section 2. Several coverage recovery schemes are evaluated and compared in section 3. The information carried by LP-WUS is discussed in section 4. LP-SS designs are addressed in section 5. The RRC parameters are discussed in Chapter 6.

# Waveform design of LP-WUS

## Discussion on OOK modulation

Regarding the waveform design for LP-WUS, the following were agreed in RAN1#116[8] and RAN1#118[11].

Agreement: Support both OOK-1 and OOK-4 for LP-WUS. - FFS how OOK-1 and OOK-4 are specified - For OOK-4, M<=4, FFS supported values - The SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as one of the SCS(s) used for other NR transmissions in the same CP-OFDM symbol - FFS different SCS.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) FFS M=1 for OOK-4

According to our evaluation during SI phase[17], M=4 for 30kHz SCS can also tolerant to up to 2us timing error. Therefore, the working assumption of M=4 for 30KHz SCS should be confirmed.

- For OOK-4, confirm the working assumption of M=4 is confirmed for 30kHz SCS.

Agreement For the SCS used for LP-WUS and LP-SS, further discuss the following options: - The single SCS is configured by gNB - The single SCS is determined by pre-defined rule

Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap

In RAN1#119, it was agreed that for IDLE/INACTIVE mode the single SCS for LP-WUS/LP-SS is same as both the associated CD-SSB and initial DL BWPif associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier [13].  When associated CD-SSB and initial DL BWP have different SCSs, in our view, the SCS of LP-WUS/LP-SS should be the same as initial DL BWP for the following reasons,

- Interference to legacy NR UEs caused by mixed numerologies of LP-WUS requires not only Base-station transmission filter but also additional UE receiver filter to mitigate, which is infeasible for legacy NR UEs.

- Additional RAN4 specification impacts for guard band are needed.

For the case of RedCap in IDLE/INACTIVE mode, according to the following conclusion and its corresponding CRs [19] made in RAN1#118, the RedCap-dedicated initial DL BWP, if configured, must have the same SCS as the legacy initial DL BWP. Therefore, the SCS of LP-WUS should also be the same as that of the initial DL BWP specified in S12 of TS 38.213.  In other words, the SCS is the same as the DL BWP provided by initialDownlinkBWP, if provided, or as that of CORESET#0 if initialDownlinkBWP is not provided.

RAN1#118 Conclusion - There is no intent to introduce Rel-17 Redcap UE capability to support the configuration with a separate Initial DL BWP with different SCS and CP compared to legacy initial DL BWP. - Rel-17 RedCap UE is not expected to be configured with a separate Initial DL BWP with different SCS and CP compared to legacy initial DL BWP.

For CONNECTED mode, SSB-based measurement is not expected to be performed by LP-WUR. In shorts, the SCS of LP-WUS is the same as the SCS of a BWP, where the LP-WUS/LP-SS is located in the BWP. With respect to RedCap UEs, it is agreed in CR [19] that the SCS of active DL BWP must be the same as that of initial DL BWP of RedCap. Therefore, the same solution here can apply to both non-RedCap UEs and RedCap/eRedCap UEs.

- For RRC IDLE/INACTIVE state of both non-RedCap and RedCap/eRedCap UEs, in the case where associated CD-SSB and initial DL BWP have different SCSs, the SCS of LP-WUS is the same as that of the legacy initial DL BWP determined in subclause 12 of TS 38.213.

- For RRC CONNECTED state, the SCS of LP-WUS is the same as the SCS of active BWP where the triggered PDCCH monitoring is configured .

## Framework to carry information by LP-WUS

In TR 38.869, the diagrams for LP-WUS waveform generation are provided [1]. Particularly, the procedures for OOK-4 generation and transmission is shown in Figure 1. In this paper, the following terminologies are used for the signal of different procedures shown in Figure 1.

- Bit stream: all the bits to be transmitted via an LP-WUS. If channel coding is used, the bit stream includes the encoded bits. If CRC is used, the bit stream also includes the CRC bits.

- Bits per OFDM symbol: the bits to be carried by OOK symbols in an OFDM symbol, which is a segment of the bit stream.

- OOK symbols/OOK sequence: the ON/OFF pattern determined based on the bit stream. Manchester coding impacts the ON/OFF pattern of the OOK symbols/OOK sequence.

- Pre-DFT sequence: a complex-valued sequence, which is generated based on the OOK symbols/OOK sequence. For example, if DFT is applied, an OFF symbol can be mapped to an all-zero sequence, an ON symbol can be mapped to a particular short complex-valued sequence, and the sequences are concatenated to get the pre-DFT sequence. It is noted that, the short complex-valued sequence can be selected from a set of candidates to carry information as shown by the red arrow in Figure 1. More details can be found in Section 2.3.

- Frequency sequence: the complex-valued modulated symbols to be mapped to REs of LP-WUS.

- OFDM symbol: the time domain signal of LP-WUS. One LP-WUS can contain serval OFDM symbols.

Figure 1 Procedures for OOK-4 waveform generation

With respect to the number of information bits (bit stream) to be carried in one LP-WUS, as discussed in our companion paper [2], it can depend on network deployment (e.g. how many cells in a tracking area), UE density, traffic arrival, and etc. Thus, it is preferred that spec can support configurable length of information bits in LP-WUS. By the framework shown in Figure 1, if one LP-WUS can be comprised by multiple OFDM symbols, no matter how long the bit stream is, it can always be processed and transmitted OFDM symbol by OFDM symbol. Therefore, the framework can be easily adapted to different bit length.

At receiver side, the receiving process of LP-WUR is the same for each OFDM symbol, i.e. demodulating OOK signal or processing the same set of sequence(s) which are used to generate the Pre-DFT sequence. Also, LP-WUR does not need to buffer the whole resource of a LP-WUS. Instead, LP-WUR can perform a pipeline processing, i.e. process the signal in previous LP-WUS occasion while buffering signal in current LP-WUS occasion. Since the subgroup ID indicated by LP-WUS is pre-known by the UE, thus if the UE finds the received bits in first several LP-WUS occasions do not match its own subgroup ID, UE can stop receiving the subsequent LP-WUS occasions to save power. Additionally, one interesting discussion point is whether the series of pre-DFT sequences to wake up a UE only depends on UE-specific predetermined information. For example, if the series of pre-DFT sequences to wake up a UE depends on nothing but the UE ID information, then the number of pre-DFT sequences per OFDM symbol for the UE correlation processing is reduced to only one pre-DFT sequence per OFDM symbol, which may be beneficial for UE complexity.

- For LP-WUS, UEs are configured to monitor one or multiple LP-WUS occasions and each occasion can convey a block of information bits.

- The bit length of the block of information is configurable or determined only from RRC configurations.

- One LP-WUS occasion comprises of one or multiple OFDM symbols.

  - Note: The OFDM symbol refers to the symbols after the processing “iFFT+CP” in S7.2.1.1 of TR 38.869

- FFS details of the pre-DFT sequences that refers to the input to the DFT/LS processing block in S7.2.1.1 of TR 38.869

  - The size of pre-DFT sequence set

  - Sequence generation/selection

- FFS the mapping from a block of information bits to pre-DFT sequences and OFDM symbols

  - FFS: whether the series of pre-DFT sequences to wake up a UE only depends on UE-specific predetermined information

## Details for overlaid sequences on top of OOK symbol

### How to carry information by OFDM overlaid sequence(s)

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

In RAN1 #118bis, option 1 was agreed when the overlaid OFDM sequence does not carry information, and option 2 was agreed when the overlaid OFDM sequence carry information.

Discussion on Option 1

Figure 2 illustration of LP-WUS design with one overlaid sequence

Figure 2 illustrates the LP-WUS transmission when overlaid OFDM sequence does not carry information. It was agreed that gNB can configure a single overlaid OFDM sequence for LP-WUS generation, but it is still unclear what are the candidate overlaid OFDM sequences that can be configured. For example, assume that for Option 2-2, 4 sets of candidate overlaid OFDM sequences are specified where in each set there are 4 candidate overlaid OFDM sequences, which results in 16 candidate overlaid OFDM sequences in total. Then for Option 1, there can be two alternatives:

- Alt 1: gNB can configure any one from the 16 candidate overlaid OFDM sequences. By this alternative, gNB has the full flexibility, and the impact of interference from neighbor cell depends on gNB implementation.

- Alt 2: gNB can only configure a subset of the 16 candidate overlaid OFDM sequences. For example, only one candidate overlaid OFDM sequence from each set can be configured by gNB, i.e., gNB can configure one from 4 candidate overlaid OFDM sequences. By this alternative, the impact of interference can somehow be guaranteed by specification design.

- Further discuss what are the candidate overlaid OFDM sequences that can be configured by gNB when overlaid OFDM sequence does not carry information.

Discussion on Option 2-2

For Option 2-2, The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s).

In the case where the overlaid OFDM sequence(s) can carry information bits, for OFDM-based LP-WUR, information can be obtained both through OOK modulation and overlaid sequence-based modulation. This allows the OFDM-based LP-WUR to terminate LP-WUS reception earlier than OOK-based receiver. It is beneficial to design the mapping order from information bits to overlaid sequences, so that OFDM-based LP-WUR can terminate reception as early as possible, thereby gaining more energy savings.

An illustration of Option 2-2 is shown in Figure 3, in each OFDM symbol, 2 bits can be carried by OOK ON/OFF pattern and 4 bits can be carried by overlaid sequences on two OOK ON symbols, where N bits information is assumed, M=4 is used for OOK-4 modulation with Manchester coding and one overlaid sequence is selected from 4 candidate overlaid sequences on each OOK ON symbol. In each OFDM symbol, 2 bits can be carried by OOK ON/OFF pattern and 4 bits can be carried by overlaid sequences on two OOK ON symbols. As shown in Figure 3, two ways of mapping the LP-WUS information bits to the overlaid OFDM sequences in OOK ON symbols can be considered. In Figure 3 (i) the information bits carried by the overlaid OFDM sequence(s) is in the same order as the information bits carried by the ON/OFF pattern, while in Figure 3 (ii) the information bits carried by the overlaid OFDM sequence(s) is in a revised order as the information bits carried by the ON/OFF pattern. Assuming that LP-WUS carries 12-bit information, to get the whole LP_WUS information, the OFDM receiver requires a unit of 3 OFDM symbols in Figure 3 (i) and a unit of 2 OFDM symbols in Figure 3 (ii) are needed for OFDM based receiver.

(i) Sequential order in overlaid sequence-based modulation

(ii) Reversed order in overlaid sequence-based modulation

Figure 3 illustration of LP-WUS design with multiple overlaid sequences

- For the mapping from information bits to overlaid sequences, reversed order is better than forward order because of potential earlier termination of detection if the mapping order for OOK sequences is forward order.

- For the mapping from information bits to overlaid sequences, support a mapping order different from the mapping order of OOK sequences-based modulation, to enable OFDM based receiver to terminate LP-WUS reception as early as possible.

To support envelope-detection based receiver, more than one unit of OFDM symbols needs to be used for LP-WUS transmission. For example, as shown in Figure 3 (ii), three units of 2 OFDM symbols (i.e., in total 6 OFDM symbols) are needed for LP-WUS transmission. However, if a LP-WUS is targeted to wake up OFDM based receiver, gNB can transmit only one unit of OFDM symbols (e.g., the first two OFDM symbols), since OFDM based receiver can get the whole LP_WUS information from both OOK modulation and overlaid OFDM sequences. Therefore, to reduce the resource overhead of LP-WUS, transmission duration of a LP-WUS targeting to wake up OFDM based receiver should be allowed to be shorter than the transmission duration required for envelope-detection based receiver.

- In order to reduce resource overhead, transmission duration of a LP-WUS targeting to wake up OFDM based receiver can be shorter than the transmission duration required for ED based receiver.

### How to design overlaid sequence(s)

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

Agreement For overlaid OFDM sequence at least for LP-WUS, - For OOK-4 with M>1, support ZC sequence for overlaid sequence. - FFS details

Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling

Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4

In RAN1 #118bis, ZC sequence was agreed to be the sequence type of the overlaid OFDM sequence for LP-WUS. And in RAN1 #119, it was agreed that different cyclic shifts and different roots can be used to determine the overlaid OFDM sequence. It is under further discussion on the set of values of CS and root for configuration.

#### How to select the ZC root sequences

When frequency offset is large, the performance of ZC sequences generated by some particular root would be degraded significantly.

Figure 4 The impact of frequency error on auto-correlation of ZC sequences

At the receiver side, sliding correlation can be employed to counteract the impact of timing error on auto-correlation. However, the sliding window cannot be too large, otherwise the ZC sequence with different cyclic shift may be falsely detected. In Figure 4, The vertical axis represents the maximum value of the auto-correlation of ZC sequence within the sliding window at a specific frequency offset. As shown in Figure 4, under the condition of OOK4 M=1, ZC sequences with q=2 and q=3 are sensitive to the frequency error, while ZC sequences with q=1 and q=32 are robust to frequency error. The reason is analyzed below.

The ZC sequence xqm is defined in the time domain as follows:

xqm=e-jπqmm+1L

where q is the root of ZC sequence, and L is the length of ZC sequence.

When there is frequency offset, the sequence with frequency offset xqm,∆f can be expressed as:

xqm,∆f=e-jπqmm+1Lej2π∆fmL

=e-jπqmm+1 -2∆fmL

where ∆f is the value of frequency offset, and it is normalized to the SCS.

When there is time offset, the sequence with frequency offset xqm,∆t can be expressed as:

xqm,∆t=e-jπqm+∆tm+1+∆tL

where ∆t is the value of time offset.

It can be observed that when the following equation is satisfied:

q∆t-∆fmod L=0

Then it follows that:

e-jπqmm+1 -2∆fmL=e-jπqm2+qm+2q∆tmL

=ce-jπqm2+qm+2q∆tm+q∆t2L

=ce-jπqm+∆tm+1+∆tL

Where c is a constant.

Thus:

xqm,∆f=xqm,∆t

In other words, a frequency offset equivalently introduces a timing offset for ZC sequence, which further leads to an offset of the correlation peak. The amount of this timing offset for a given frequency offset is impacted by the value of root q of the ZC sequence. If the value of the root is properly selected (e.g., q=1 and q=32), the timing offset caused by the frequency offset can be controlled to be small, which minimize the miss-detection of two different cyclic shifts. However, if the value of the root is NOT properly selected (e.g., q=2 and q=3), the timing offset caused by frequency offset can be large, which may require a much larger minimum cyclic shift and restrict the number of candidate cyclic shifts provided by the same root.

Figure 5 Auto-correlation of ZC sequence with frequency offset

Figure 5 shows that when there is a frequency offset, different roots can lead to the presence of correlation peaks at different time offsets. For q=1 and q=32, the positions of the peak values are close to the position of time offset equals 0us. If sliding correlation is adopted, where the sliding window is smaller than the minimum cyclic shift, q=1 and q=32 can be detected while q=2 and q=3 cannot.

Also, in RAN1 #119, it was agreed that the number of sequences per ON symbol is 4 to carry 2-bit information for M=4. As discussed in Section 2.3.2.2, different cyclic shifts are recommended to be used in the same cell, then up to 4 cyclic shifts are supposed to be supported in an OOK symbol. To makes the different sequences detectable, the candidate value of roots for LP-WUS should have the property that the ZC sequence is robust to frequency error.

Therefore, optimal ZC root sequences should be selected, which are very robust to frequency error. Examples of some optimal ZC sequences are shown in Table 1.

Table 1 Optimal ZC root sequences

| The length of ZC | Optimal ZC root sequences |
| --- | --- |
| L = 127 | q = 1, 32, 42, 63, 64, 85, 95, 126 |
| L = 61 | q = 1, 15,20, 30, 31, 41, 46, 60 |
| L = 59 | q = 1, 15, 20, 29, 30, 39, 44, 58 |
| L = 53 | q = 1, 13, 18, 26, 27, 35, 40, 52 |
| L = 31 | q = 1, 8, 10, 15, 16, 21,23, 30 |
| L = 29 | q = 1, 7, 10, 14 ,15, 19, 22, 28 |
| L = 23 | q = 1, 6, 8, 11, 12, 15, 17, 22 |

- A frequency offset of a ZC sequence will cause a shift of the correlation peak location of the ZC sequence in time domain, and the extent of this shift is determined by the value of root q and should be minimized by selection of root q.

  - With minimized range of the shift of correlation peak, optimized ZC root selection for LP-WUS allows UEs to have smaller sliding window and thus is beneficial to reduce UE implementation complexity.

- In order to make ZC sequences robust to timing/frequency error, optimized roots of ZC sequences are selected.

#### How to select the ZC sequence for the same cell and different cell

Typically, different ZC sequences can be generated by different cyclic shifts and different roots. In the following, how to select the sets of candidate overlaid OFDM sequences is discussed.

Figure 6 Aperiodic cross-correlation between different roots of ZC sequence

Figure 6 shows the aperiodic cross-correlation between different roots(e.g., q =1 and q =15) of ZC sequence for OOK-4 M=1 and M=4 with 30kHz SCS. Generally, regardless of how large the timing offset or frequency offset is, ZC sequences with different roots show good orthogonality between each other. Specifically, the cross-correlation value is less than -20dB for OOK-4 M=1, and less than -15dB for OOK-4 M=4. The difference comes from different length of ZC sequence.

Figure 7  Aperiodic cross-correlation between different CS of ZC sequence with the same root

Figure 7 shows the aperiodic cross-correlation between different cyclic shifts of ZC sequences with the same root for OOK-4 M=1 and M=4 with 30kHz SCS. It is observed that at most of the time offset locations the cross-correlation value can be less than -30dB for OOK-4 M=1, and less than -20dB for OOK-4 M=4, which means better orthogonality than ZC sequences with different roots. However, when the time offset equals to the cyclic shift, there is a ‘fake peak’, which may strongly impact the detection if the timing error is not small enough.

From the simulation results shown in Figure 6 and Figure 7, it can be concluded that if timing error is smaller than half of the minimum cyclic shift, ZC sequences with the same root but different cyclic shift may be better. "Minimum cyclic shift" refers to the minimum cyclic shift between any two ZC sequences that have the same root within the candidate overlaid sequence set. While if timing error is larger than the minimum cyclic shift, ZC sequences with the different roots may be better.

- Based on the property of ZC sequence

- If timing error is smaller than half of the minimum cyclic shift, ZC sequences with the same root but different cyclic shift is the best choice because of smaller cross-correlation.

- If timing error is larger than half of the minimum cyclic shift, ZC sequences with the different roots may be the best choice

- Based on the property of ZC sequence

- When different cyclic shifts of a ZC sequence with the same root are used as the candidate overlaid sequences, the timing error and frequency error need to be controlled within a target range when UE is detecting LP-WUS;

- When ZC sequences with different roots are used as the candidate overlaid sequences, the timing error and frequency error can be relaxed when UE is detecting LP-WUS;

With the above observation, specifically for cellular system design, if different roots are used in different cells, no matter how large the timing difference among different neighboring cells is, the aperiodic cross-correlation is still good enough. If different cyclic shifts are used in different cells, depending on the timing difference, there may be strong interference from neighboring cells. In order to minimize the interference between different cells, it is preferred to use different roots for different cells to generate ZC sequences.

- ZC sequences generated by the same root but difference cyclic shifts are not preferred to be used in different cells, since the timing difference between different cells may be larger than the minimum cyclic shift which results in strong interference.

- If different roots for ZC sequences are used in different cells, no matter how large the timing difference of different cells is, the aperiodic cross-correlation is still good enough.

In case of the same cell, the timing error can be estimated and corrected based on LP-SS and/or SSB for OFDM based receiver. With careful selection of the cyclic shift values, the timing error can be controlled to be smaller than the timing error, so that the benefit of lower cross correlation can be obtained for LP-WUS. Therefore, it is feasible to employ different cyclic shifts or different roots to carry information in the same cell.

- ZC sequences generated by the same root but different cyclic shifts are preferred to be used in the same cell to carry information for the following reasons,

- With the aid of LP-SS/SSB, the timing error and frequency error can be estimated and corrected to be smaller than a target range, thus LP-WUS can enjoy the benefit of lower cross correlation

Based on the above, we propose the following

- Different overlaid ZC sequences can be used to minimize the interference between different cells.

  - Prefer to use different roots for different cells to minimize the interference.

  - Prefer to use different cyclic shifts to carry information.

In RAN1#119, It was agreed that support maximum 4 candidates overlaid sequences for M=4. If only different cyclic shifts are used to carry information, up to 4 cyclic shifts can be supported in one OOK symbol for M=4. Then for M=2 and M=1, with the same tolerance of timing error, up to 8 and 16 cyclic shifts can be supported in one OOK symbol.

- Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, support maximum 8 candidates overlaid sequences for M=2 and 16 candidates overlaid sequences for M=1.

Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

For M>1, the length of overlaid OFDM sequence was also discussed and the above two alternatives were agreed in RAN1 #119. From our view, Alt1 is recommended. If Alt1 is applied, gNB can enable fast DFT implementation, which allows the gNB to dynamically generate overlaid OFDM sequences. From UE perspective, it makes the UE's sampling rate is an integer multiple of LZC, which ensures the performance of the reception detection.

- For M>1, Support the sequence length, with ,.

#### The maximum number of candidate overlaid sequences for a cell

Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells

In RAN1#118bis, the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell was discussed, and several values are listed for further down-selection. Per our understanding, if the number of candidate overlaid sequence is too large, the orthogonality between sequences cannot be guaranteed, which shall further impact the detection performance. For example, if different cyclic shifts are used to carry information, as shown in Figure 7, there will be a ‘fake peak’ of correlation at the location corresponding to the cyclic shift value. For OOK-4 M=4 with SCS=30kHz, the length of an OOK ON chip is about 8us. If 4 cyclic shifts are supported, then the nearest ‘fake peak’ appears at about ±2us, meaning that it can only tolerate up to 2us timing error. Similarly, if 8 cyclic shifts are supported, it can only tolerate up to 1us timing error. More than 8 cyclic shifts require finer timing error correct, which may increase the complexity and power consumption of UE. To leave enough margin for UE implementation, up to 2us timing error tolerance can be a starting point, which requires at most 4 cyclic shifts.

- The maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell is mainly impacted by the orthogonality between sequences.

- Assuming that only cyclic shifts are used to differentiate candidate overlaid sequences for a cell, the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for the cell is 4.

#### How to map the frequency sequence to REs

Since it was agreed that the DFT size is 2^N, there will be some spare REs. For example, for M=1, 11PRB corresponds to 132 REs. The length of overlaid sequence before DFT shall be 128 points, and it is better to map the sequence after DFT to center REs, thus there can be 2 REs at each side used as guard REs.

- The frequency sequence after DFT is mapped to the center of 132 REs with 2 guard REs at each side.

### Pulse shape of OOK with overlaid sequence(s)

Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption

In RAN1 #119, two alternatives were agreed to be further discussed for the sequence generation for M=2 and 4, where in Alt2 is given by the largest prime number such that, which is the so-called  concentrated waveform.

Figure 8 Illustration of concentrated OOK waveform providing robustness against ISI and timing error among OOK symbols

Figure 9 Performance of concentrated waveform

Figure 9 shows that the concentrated waveform can improve the robustness with respect to timing error, for envelope-detection based receiver. It can be observed that the concentrated waveform provides 0.9 gain for ideal timing case. This is because that the ISI can be better resolved by using the concentrated OOK.  Thus, Alt2 should be supported when M=2,4.

- For the overlaid OFDM sequence in time domain, support Alt2 for M=2,4, i.e., ,, whereis given by the largest prime number such that,  is the overlaid OFDM sequence length.

In Section 5.3.1 of [4], the CP-OFDM baseband signal generation includes a cyclic shifting of the signal spectrum (so-called fftshift) such that it is centered around its DC component. In Section 6.3.1.4 of [7], the DFT-s-OFDM generation is however without such cyclic shifting of the signal spectrum after DFT-precoding, and thus the DC component outputted by the DFT precoder is mapped to the spectrum edge. In legacy NR uplink DFT-s-OFDM with corresponding legacy receiver, cyclic shifting the spectrum would be meaningless while just reverted at the receiver without impacting data symbol detection performance. Moreover, a given uplink signal is not necessarily in the middle of the gNB baseband spectrum, and thus its DC component does not necessarily map to the DC of the overall received uplink baseband signal.

When considering OOK waveform generation for LP-WUS, the situation is different and cyclic shifting the spectrum is not a meaningless operation anymore; so to make sure that the LP-WUS has the necessary spectrum adjustment while being multiplexed along with other legacy NR signal/channel by the gNB using a single IFFT, an equivalent cyclic shifting of the signal spectrum needs also to be done after DFT precoding.

This cyclic shifting is necessary in order to map the DC Fourier coefficient of the input signal to the DC component of the output signal, and so for the output signal to be a proper interpolation of the input signal; otherwise it is not. In other words, spectrum shifting changes the signal’s envelope and phase and thus OOK detection performance.  This is discussed in more details in Appendix A.

In addition, with Manchester coding, the DC of the input signal is constant. Therefore, with proper spectrum adjustment mapping the input DC to the output DC, the LP- WUS can be guaranteed to have a constant DC which will ease signal processing for the LP-WUR. Otherwise, the DC of LP-WUS may be data dependent and fluctuates from one OFDM symbol to another OFDM symbol which require more dynamic signal adaptation.

Finally, this necessary spectrum shifting adjustment can be directly embedded into overlaid sequence(s) as the OOK input by using an alternating “1” and “-1” as a cover code of other sequence (e.g. “1” and “-1” multiplied with ZC sequence) without changing legacy DFT-s-OFDM to get the benefit of both. Apply an alternating “1” and “-1” as a cover code on overlaid sequence(s) for the expected pulse shape of LP-WUS signal to guarantee the detection performance of envelope and phase detection.

# Coverage

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

Regarding the coverage of LPWUS, the following is captured in the WID “Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3” [18]. From evaluations in study phase, it can be observed that such target is challenging for LPWUS due to limitations such as high noise figure and single antenna reception. In this section, we provide analysis and evaluations on several potential coverage recovery schemes.

There are two types of time domain repetition. One is bit-level time-domain repetition, and the other is signal-level time-domain repetition, which are shown in Figure 10. In the following, detailed analyses of the three types of repetition are provided.

Figure 10 Time domain repetition

Bit-level time domain repetition

Basically, bit-level time-domain repetition can provide time domain diversity. In addition, it can be used to handle CP issue. As shown in Section 2.2, CP is added after the processing ‘IFFT’. For the OOK-4 M=1, CP has no impact, since the whole OFDM symbol is used as a single OOK symbol. However, for OOK-4 M>1, CP can’t be used to combat multipath effects if there is no ‘FFT’ operation at receiver side. When the envelope of CP is different from the envelope of the first OOK symbol in the OFDM symbol, CP introduces unnecessary signal ON/OFF transition, which may affect the detection performance of the first OOK symbol, especially when there is timing error.

In RAN1 118, Manchester coding was agreed to be used for LP-WUS. If direct repetition of information bits is adopted, it will inevitably lead to a mismatch between the envelope of the CP and the envelope of the first OOK symbol. An example is shown in Figure 11, where OOK-4 M=4 with Manchester coding is used.

Figure 11 Mismatch of envelope between CP and the first OOK symbol when Manchester coding is used

In order to solve CP issue, bit inversion repetition can be adopted, i.e., the information bit ‘1’ is ‘repeated ’ as ‘10’, and the information bit ‘0’ is ‘repeated ’ as ‘01’. After Manchester coding, the information bit ‘1’ is mapped to ON-OFF-OFF-ON, and the information bit ‘0’ is mapped OFF-ON-ON-OFF. It is illustrated in Figure 12 where the envelope of CP is always the same as the envelope of the first OOK symbol when Manchester coding and bit inversion repetition are adopted. in this way, the detection performance of the first OOK symbol would not be affected by CP.

Figure 12 Manchester coding and bit inversion repetition are adopted

- When Manchester coding is used for OOK-4 M=4, bit inversion repetition can ensure the envelope of the CP and the first OOK symbol is always the same, which is beneficial to reduce the performance impact due to CP.

Furthermore, the length of the first OOK symbol can be shortened so that the combined length of the first OOK symbol and the CP is equal to the length of the other OOK symbols. Figure 13 provides an example when the IFFT size is 2048.

Figure 13 Shorten the length of the first OOK symbol

At this point, the CP can be combined with the first OOK symbol to form a new OOK symbol, where the CP becomes a part of the useful signal instead of being a redundant signal. Thus, it enhances the detection performance of the LP-WUS.

Figure 14 CP is used to enhance the detection performance of the LP-WUS

For envelope-detection based receiver, if CP is used as a part of the first OOK symbol, the detection performance of the LP-WUS can improved. The reason is that in this case, CP is also utilized for signal processing instead of simply dropped, which provide more energy of desired signal and/or more time domain diversity. Simulation results are shown in Figure 14.

- If CP is used as a part of the first OOK symbol, the detection performance of the LP-WUS is improved when Manchester coding and bit inversion repetition is adopted.

- Bit-level time domain repetition is considered to improve the detection performance of LP-WUS.

  - Manchester coding and  bit inversion repetition can be used to solve CP issue.

  - CP can be used as a part of signal.

Spatial diversity and time domain repetition

For OFDM symbole-level and signal-level time-domain repetition, it can be used together with transmit diversity. One possible way to achieve transmit diversity is precoder cycling. For example, for each repeated transmission, a different precoder is used at gNB for LPWUS transmission to get spatial diversity gain in addition to combination gain. A benefit of precoder cycling is that it can be transparent to LPWUS receiver, and thus no spec impact. Based on the simulation results shown in Figure 15, 2dB or more performance gain can be observed where two antenna ports with precoder cycling is adopted.

Figure 15 Simulation results of spatial diversity and time domain repetition

- For 4x time domain repetition, transmit diversity by precoder cycling with two antenna ports can provide 2dB or more performance gain.

- Signal-level time domain repetition, OFDM symbol-level time domain repetition and transmit diversity by precoder cycling are considered to improve the performance of LP-WUS.

Frequency domain diversity and time domain diversity

In addition to simple time domain repetition, further performance gain can be provided by exploiting frequency/time domain diversity.

- Frequency domain diversity can be harvested by frequency hopping, i.e. LP-WUS is transmitted on different resources at different time. At each time, even the BW of LP-WUS is small, when accounting all the transmission time of a LP-WUS, the spanned BW of LP-WUS can be larger, thus providing higher frequency diversity gain. Frequency hopping may be used in conjunction with repetition, e.g. same data is transmitted over different frequencies, as illustrated in Figure 16(c). At the receiver side, UE can receive LP-WUS by retuning its LO and combine multiple repetitions.

- Time domain diversity can be obtained by multiplexing multiple LP-WUSs in an interleaved way, as shown in Figure 16 (d). Consider LPWUS transmission may has a delay budget on the order of hundreds of ms, time domain diversity can be exploited. For example, assume that a LP-WUS targeted to a particular UE/UE group occupies N occasions, where one occasion is a time domain resource element for LP-WUS transmission. Instead of transmitting N occasions contiguously, they can be transmitted in a distributed way so that transmission are over different time / channel conditions. .

Figure 16 illustration of coverage recovery schemes which exploit time / frequency diversities

Figure 17 Simulation results of frequency hopping

The simulation results of frequency hopping and time domain interleaving schemes can be found in Figure 17, where the simulation assumptions are provided in Appendix B. It can be observed that:

- Compared to repeated LP-WUS transmission over contiguous occasions, 1.5-2 dB gain can be obtained when the transmissions are over non-contiguous occasions. This is because more time diversity can be harvested.

- Compared to repeated LP-WUS transmission over contiguous occasions, 2.3 dB performance gain can be obtained with frequency hopping, when the signal bandwidth is 4.32MHz.

In general, LPWUS with narrower bandwidth benefit more from diversity schemes. This is understandable as narrower bandwidth brings less frequency diversity.

- Coverage recovery schemes that exploit time / frequency diversities are considered.

Time domain spreading code

Figure 18 Block diagram of a transmitter using binary spreading sequences, ci, to multiplex bits, bi, from M UEs before the OOK modulator. Each UE receiver performs energy detection to give the binary vector y, which is used as input to demultiplex the bits.

With OOK modulation and energy detection, the receiver cannot detect any phase value of the signal. Hence, classical CDM with BPSK spreading sequences cannot be used to multiplex WUSs on the same time-frequency resource. However, with OOK it is possible to use binary spreading sequences, for which the WUS multiplexing is performed in the binary domain, before the OOK modulator. The binary spreading sequences could be constructed to offer redundancy of the transmitted bits, which improves the performance. We demonstrated in [4] that binary spreading sequences could be used to achieve lower BLER than for TDM.

Figure 18 shows a block scheme of transmitter and receiver. The binary spreading sequences ci,  are N×1 vectors with N≥M and the binary vector y=Cb mod 2, where the columns of the N×M matrix C consists of the spreading sequences. UE receiver i produces bits yi and from C it can determine bi. The multiplexing in the transmitter can be made by modulo-2 addition as in Figure 18. If the message size of a UE is L bits, spreading can be performed L times (i.e., transmitting L vectors y of length N) or by  inputting LM bits which are spread by codes of length  LN giving one vector y of length LN.

The demultiplexing is made in the binary domain and has a complexity comparable to hard decoding of block codes. The spreading sequences can, e.g., be constructed from a Hamming code, for which there are decoding algorithms with very small complexity [5][6], or from the Reed-Muller code which is already specified in NR. If additionally FEC is applied, the bits bi are taken from the FEC encoder output and the demultiplexed bits bi are fed to the FEC decoder.

The gain is around 1.5 dB at 1% BLER when using spreading sequences from a Hamming (7,4) code compared to a TDM solution without coding, according to Figure 19. Compared to a TDM solution with repetition code, the gain is around 1 dB. The ID size is  L=8 bits or L=16 bits and 2 UEs are multiplexed, giving N=28 (if L=8) or N=56 (if L=16) transmitted bits. For the uncoded TDM scheme, L bits are transmitted. A pseudo-random interleaver is applied after the encoder, to the N transmitted coded bits.  Either 1 bit (M=2 with Manchester coding) or 2 bits (M=4 with Manchester coding) is/are transmitted per OFDM symbol. The bit repetition pattern is set to maximize the performance of the repetition code given N coded bits. The simulation assumptions are given in Table 5 in Appendix B.

Figure 19 BLER for multiplexing 2 WUSs either by spreading sequences from a Hamming code, by a repetition code or by uncoded transmission, for a packet size of 8 bits and 16 bits, using either M=2 or M=4 bits per OFDM symbol with Manchester coding.

- Binary spreading sequences are considered to multiplex WUSs on the same time-frequency resource to improve the BLER.

# LP-WUS information

Agreement For RRC idle/inactive state, support the following option for at least indicating subgroup information using LP-WUS: - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs - UE monitoring one or more MOs (up to X MOs) for the same beam within an LO is supported - Value of X is larger than 1. FFS on additional details of X.

Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

In RAN1#118, it was already agreed that codepoint based information is supported for at least indicating subgroup information using LP-WUS in RRCIDLE/INACTIVE state. It is still under discussion how to define the LP-WUS information in RRC CONNECTED state.

Based on our analysis, for CONNECTED state, codepoint based information has the following advantages than bitmap based information

- The average overhead of codepoint based information is smaller than bitmap based information. Numerical analyses on the average overhead (the way to calculate the average overhead for LP-WUS information is shown in Appendix ) are shown in Figure 20, where total subgroup number are 16 and 32. respectively. Typically, in a cell, there are tens of UEs in CONNECTED mode. Considering that different UE/subgroup can monitor different set of MOs, assuming 16 or 32 subgroups per MO is reasonable. It is shown in Figure 20 that when the traffic arrival rate is lower than  ~30%/40%/45% (for 8/16/32 subgroups per MO, respectively), codepoint based information requires smaller overhead. Based on our understanding, LP-WUS can only provide attractive power saving gain when the traffic is not too dense. Thus, the target scenario for LP-WUS would be the one where traffic arrival rate is lower than  30%~45%. Therefore, the average overhead of codepoint based information is smaller than bitmap based information.

- Codepoint based information is beneficial for local detection. A UE only needs to detect the codepoint corresponding to itself. Early termination of detection can be feasible for 2)	Codepoint based information.

- Codepoint based information is beneficial for reducing FAR. As analyzed in our companion paper [2], if the codepoint is long enough, the FAR is naturally low enough.

- Codepoint based information is beneficial for the unified design. It is good for UE implementation to have the unified design in IDLE/INACTIVE mode and CONNECTED mode.

- Codepoint based information is beneficial for reducing MDR. Based on our evaluation, when the same number of UEs/subgroups are support, the MDR performance for codepoint based information is better than that for bitmap based information. It results in that less number of repetitions is required by codepoint based information than bitmap based information to achieve Msg3 coverage. For example, to achieve Msg3 coverage, no matter for 8 or 16 or 32 UEs/subgroups, 7-bit codepoint is needed to meet the FAR requirement, for which 4 repetitions are needed based on our evaluation. For bitmap based information, to achieve Msg3 coverage, 8-bit bitmap with 6-bit CRC requires 5 repetitions 16-bit bitmap with 6-bit CRC requires 8 repetitions, and 32-bit bitmap with 6-bit CRC requires 10 repetitions. Different number of repetitions leads to different resource overhead, and the impact is also reflected in Figure 20.

Figure 20 The average overhead of codepoint and bitmap

In summary, for CONNECTED mode, codepoint based information has more advantages than bitmap based information. Therefore, codepoint based information is also preferred.

- Regarding the LP-WUS information for connected UEs, support the codepoint mapping method, i.e. option 3.

# Synchronization and/or RRM for serving cell

Based on the WID scope and agreement in RAN1#118bis, the following will be specified for LP-SS [18].

- Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

As determined by objective of LP-WUS signal, the LP-WUS modulation is OOK with overlaid sequence(s). Therefore, both OFDM based receiver and envelope detection based receiver are considered in Rel-19 WID.

For OFDM based receiver, the time and frequency synchronization are performed before the monitoring of each potential LP-WUS occasion, using either SSB or LP-SS. As mentioned by the note as above, existing PSS/SSS with typically 20ms periodicity can be used for time/frequency synchronization. Based on the agreement achieved in RAN1#118bis, overlaid sequence is supported for LP-SS. Considering the ZC sequence has been agreed for LP-WUS, and ZC sequence has good auto-correlation and cross-correlation properties, therefore, it is naturally that overlaid sequence over the LP-SS signal can be used for synchronization and RRM measurement. Based on the analysis in Section 2.3.2, ZC sequence has a characteristic where a frequency offset and a time offset are equivalent. This can lead to aliasing when using ZC sequences for synchronization. To mitigate this issue, different roots of ZC sequences can be used for different OOK on symbols.

For envelope-detection based receiver, only LP-SS can be used for synchronization and serving cell measurement.

- For LP-SS, using different roots of ZC sequences on different OOK "on" symbols can improve receivingperformance.

## Waveform of LP-SS

### Consideration of the common aspects for LP-SS and LP-WUS

Regarding waveform of LP-SS, the following was agreed in RAN1#118bis.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

In our view, the waveform of LP-SS should have similar design as LP-WUS. From performance perspective, the enhancements of LP-WUS can also improve the performance of LP-SS, since the UE uses the same LP-WUR to receive both LP-SS and LP-WUS. On the other hand, if the enhancements are already discussed for LP-WUS design, utilizing the same enhancements to LP-SS requires little effort.

The following aspects, which are discussed in Section 2, can be considered to be used to LP-SS:

- Pulse shaping. The pulse shaping methods can at least include the concentrated waveform to improve the performance of LP-WUS, and the spectrum adjustment for compatibility with CP-OFDM generation. With larger M value, the synchronization performance for envelope detection-based receiver gets better, since shorter OOK symbol length provide finer granularity of the correlation result. Smaller residual timing offset after LP-SS based synchronization is also beneficial to LP-WUS detection, so it is reasonable to use a larger M value for LP-SS than LP-WUS. Actually, the effect to get shorter OOK symbol length can also be provided by the concentrated waveform. Therefore, concentrated waveform is useful and important for LP-SS design. An example is shown in Figure 21 with M=4 and L=32, in which  X% concentration means that the overlaid sequence has X% of zeros samples split equally at its beginning and  end. As it can be seen on the figure, concentrating the waveform can provide significant timing accuracy gain at the 90% CDF target.

Figure 21 Effect of pulse shaping on LP-SS timing accuracy

- Overlaid sequence(s). It was agreed that overlaid OFDM sequence(s) is supported for LP-SS, and LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. It was agreed that the LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. However, the details regarding how to select overlaid sequences within the set of overlaid OFDM sequences specified for LP-WUS needs to be further discussed. Another issue is whether transmitting LP-SS by using a specified overlaid OFDM sequence is configurable for OOK-4 M>1. Different from OOK-4 M=1, it is almost impossible to reuse legacy frequency-domain NR signals to represent ON/OFF sequences of LP-SS in time domain. Thus, it is impossible to share the resource of LP-SS with legacy NR UEs. In this case, it is always beneficial to let UE know the transmitted overlaid sequence, while it does no harm to gNB.

- For the M value relationship between LP-WUS and LP-SS, support Alt2.

- For LP-SS with OOK-4 M>1, the OFDM sequence(s) that are overlaid on the LP-SS are always indicated to UEs.

- LP-SS has similar design as LP-WUS, including at least the following aspects：

  - pulse shaping methods, including the concentrated waveform and the spectrum adjustment

### Consideration of the LP-SS specific aspects

Beside the above common design aspects between LP-WUS and LP-SS, there may be other LP-SS specific consideration.

- Consider the CP impact. Differently from other synchronization signals in NR, if LP-SS needs to be transmitted along concurrent data, it needs to be designed by modulating several CP-OFDM symbols. Due to the insertion of CPs for LP-SS before each OFDM symbol, the auto- and cross- correlation properties of the LP-SS signal may differ from the correlation properties of the original binary-valued sequence since the CP shall introduce some ‘narrow ON/OFF chip within the LP-SS, and will impact performance. The larger M, the closer the CP duration gets to an OOK symbol, and the larger is its impact in performance.

- Good auto-correlation property. For the purpose of timing synchronization with envelope detection, the LP-SS should have an envelope with good auto-correlation property so that the LR can get a peak location to find the timing by correlation operation. Since timing estimation is performed with a small search window, main lobe of the autocorrelation should notably be minimized. Reasonably good auto-correlation property can be obtained by first considering binary OOK sequences with good auto-correlation property Meanwhile, binary sequences with same auto-correlation property can lead to different LP-SS auto-correlation property after modulation and CP addition, notably the width of the main lobe, and thus timing performance.

- Good cross-correlation property. For the purpose of mitigating inter-cell interference, it was agreed to support 4 LP-SS binary sequences. In order to minimize interference, the LP-SS set should minimize its maximum cross-correlation. Again, this can be largely controlled from the cross-correlation property of the LP-SS binary sequence set, but the true cross-correlation is altered by modulation and notably CP since the CP shall introduce some ‘narrow ON/OFF chip within the LP-SS. Therefore, complete cross-correlation properties of the modulated signal should be considered. As interfering signal can arrive with any different delay, the cross-correlation should be considered over the complete correlation window.

- Balanced '0's and '1's in the whole sequence. The '0's and '1's inside the binary LP-SS sequence should better be balanced such that both the OOK sequence itself and the envelope of the modulated LP-SS signal are centred around their mean value, which is friendly to AGC tracking and can be used for better detection. The LR may maintain AGC based on the input signal, so the length of consecutive OFF segments should not be too long to avoid losing AGC. Balanced ‘0’s and ‘1’s are also helpful to guarantee the performance of LP-RSRP, LP-RSRQ.

- Balanced ‘0’ and ‘1’within OFDM symbols. For M>1, in order to fully utilize the gNB’s power, the same power level in each OFDM symbol of LP-SS should be strictly maintained. For LP-SS binary sequence design, this translates in enforcing to not have a segment of M consecutive zeros that is allocated to a single OFDM symbol.

  - For M=4, another special case to avoid transmit power wasting is to transmit only one OOK ON symbol in each OFDM symbol, which results in a low-density binary sequence. By low-density binary sequence, the power of one OFDM symbol can be more concentrated, which is beneficial for the timing performance of LP-SS. However, low-density binary sequence has some drawbacks. OFDM based receiver can estimate the frequency error based on the overlaid sequences in OOK ON symbols. For low-density binary sequence, it contains consecutive OOK OFF symbols, which impacts the detectable range of frequency error. Another drawback is that due to higher power level of OOK ON symbol for LP-SS than that for LP-WUS, AGC can be impacted.

- Low-density binary sequence to ensure one OOK ON symbol in each OFDM symbol for M=4 can also avoid transmit power wasting, but it impacts the frequency error estimation (for OFDM based receiver) and AGC.

- The design of LP-SS should follow the guidelines:

  - Good auto-correlation and cross-correlation property taking into account modulation with CP

  - Balanced '0's and '1's in the whole sequence

  - Balanced ‘0’ and ‘1’ within OFDM symbols for M>1.

In RAN1#118bis, it was agreed that to determine the binary sequence of LP-SS, sync accuracy and cross-correlation can be used as the metrics. Also, in the agreed template for collecting LP-SS binary sequence results [14], companies shall report whether CP is considered or not for sequence correlation in column P, which applies to both auto-correlation and cross-correlation. Per our understanding, the sync accuracy generally reflects the auto-correlation property of the LP-SS sequence where the CP impact is already mandatorily accounted in the LLS. Therefore, to report LP-SS binary sequence results by the template, companies have to report ‘yes’ in column P as required by the auto-correlation, which further requires companies to consider CP for cross-correlation operation. On the other hand, technically speaking, to reflect the impact of transmitter and receiver (i.e., to simulate the signal that really received by the UE), the cross-correlation should be computed in the way the LR perform correlation for detection, i.e. between a modulated LP-SS sequence and LR local sequence taking into account the CP impact.

- The correlation property is different depending on whether CP impact is included as the metric.

- To reflect the impact of transmitter/receiver, modulation with CP should be included in the calculation of the cross-correlation, which is also implicitly required by the template for collecting LP-SS binary sequence results.

One possible way to reduce the impact of CP is to use the concentrated waveform. When concentrated waveform is used, there will be ‘zero’-points at each edge of an ON symbol. If the ON symbol is concentrated enough, the CP will always be constructed by ‘zero’-points, and thus the impact of CP is eliminated.

- The impact of CP can be reduced by concentrated OOK.

### Consideration of the time-domain resources of LP-SS

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4 - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified - The frequency error: up to company report - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption

Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32}

Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value - FFS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference

Agreement To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy.

The synchronization performance of LP-SS is mainly determined by the time-domain resources occupied by the LP-SS, and then after by the specific sequence. Therefore, before deciding on a specific LP-SS sequence type, the resources occupied by LP-SS should be determined first.

The time-domain resources required for LP-SS can be determined through the sync accuracy requirements in the RAN1 #118[11]. In Figure 22, the simulation results of the timing estimation performance for LP-SS are provided, where a M-sequence is used to generate the LP-SS.

| a) OOK-4 M=4 | b) OOK-4 M=2 | c) OOK-4 M=1 |
| --- | --- | --- |

Figure 22 Timing accuracy based on LP-SS for envelope detection-based receiver

Based on the simulation results, 8 OFDM symbols are needed to meet the sync accuracy requirements at an SNR of -3dB for M=1/2/4. Therefore, we propose that at least 8 OFDM symbols should be supported for LP-SS for OOK-4 with M=1/2/4.

- For LP-SS, support at least 8 OFDM symbols for OOK-4 with M=1/2/4.

  - M=1, L= 8

  - M=2, L= 16

  - M=4, L= 32

## Design principle for the binary sequence of LP-SS

The following was agreed in RAN1#117 and RAN1#118 regarding binary sequence of LP-SS [10][11].

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

Agreement For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence

Agreement The number of binary LP-SS sequences is 4.

### Analysis of binary LP-SS sequence type

It was agreed to further down-select the binary LP-SS sequence type from Gold sequence, M sequence, and computer searched sequences. As one way, designs based on M- and Gold sequences can be used along with Manchester encoding, and after optimization, they can meet the design guidelines in section 5.1.2. A corresponding construction framework and correlation analysis is provided in Appendix . Sequence generated based on M or Gold sequences may help to reduce the workload of the standard.

Meanwhile, as only 4 LP-SS sequences are needed with rather small sequence length, computer searching on sequences without restrictive analytical structure is another possible way, which is tractable for a complete optimization of the performance. Computer searched sequences includes as subsets M- and Gold based sequences and may incidentally select some of them. Further details can be found in Section 5.2.3.

### Analysis of LP-SS reception methods

As LRs with envelope detector will have to perform synchronization with the received OOK-modulated LP-SS. The best correlation metric for designing LP-SS sequence set needs to be considered as it is not legacy synchronization with coherent signals as PSS/SSS. For low complexity purpose, it is better that the LR uses a binary, either unipolar (0 and 1) or bipolar (-1 and +1) sequence to correlate with the receive envelope. Remark if the local sequence is bipolar, the output of the correlator may be negative, which cannot be the result from an unknown random phase, so correlation samples should better not be converted in absolute value. Otherwise it would just convert negative out-of-phase peaks into false positive peaks and thus increases the timing error probability. Three types of correlator could be envisioned:

- The LR shifts the received LP-SS envelope (i.e., removing the DC) and correlate with a bipolar local sequence. In this way, it could retrieve familiar correlation properties of bipolar sequence types, e.g. (+/-1)-modulated sequences. However, since with sliding window, the LR does not know if it integrates totally or partly a noise sequence, it should recompute the mean at every single time instant to perform well. Moreover, this mean computation and shifting is highly prone to error as it is done with low precision after ADC.

- For complexity reduction, the LR can correlate the received LP-SS envelope without shifting directly with a bipolar local sequence. This is known as mismatched detection [15] and was the detection method considered for design the WUS preamble of IEEE 802.11ba Wake-Up[16].

- The LR correlates the received LP-SS envelope without shifting directly with a unipolar local sequence. This has less background in the scientific literature, but our observation from primarily simulation results is that it could also provide good detection performance for some sequences.

At least for complexity purpose, the LR receivers (ii) or (iii) discussed above are preferred and thus corresponding correlation properties needs to be considered for LP-SS design.

It can be observed from simulation that timing performance of unipolar correlation and mismatched correlation are very close to each other, with slightly better timing for unipolar correlation. However, the cross-correlation is much better with mismatched correlation than unipolar correlation. In real deployment, to minimize the interference impact, mismatched correlation is expected to be prioritized over unipolar correlation. Therefore, in the simulation, it is suggested to use mismatched correlation

- For designing set of binary LP-SS sequences, mis-matched correlation or unipolar correlation should be considered as the detection algorithm for binary sequence design, where mis-matched correlation is preferred than unipolar.

### Binary LP-SS sequence selection

Among all possible sequences, we searched sequences with similar autocorrelation characteristics in order to provide a uniform timing performance among cells. Since timing estimation is performed within a small search window, we first selected sequences with smaller main lobe width, and then minimized the peak side lobe in the rest of the auto-correlation profile.  Further selection was done to minimize the maximum cross correlation. Finally, we optimized the LLS timing performance among multiple candidate sets to account for the modulation and CP impact.  The optimized LP-SS binary sequence sets for (M=1, L=8), (M=2, L=16), and (M=4, L=32), are provided in Table 2.  It is noted that two sets of sequences are provided for (M=4, L=32), where set#1 is with balanced ‘0’ and ‘1’ in each OFDM symbol, and set#2 is with a single ‘1’ in each OFDM symbol. As discussed in Section 5.1.2, set#1 is friendlier for frequency error estimation (for OFDM receiver) and AGC, while set#2 can provide better timing estimation performance.

Additionally, for all the sequences proposed by companies, their timing estimation performances are evaluated and provided in a separate Excel attachment file with columns F and G. Our simulation assumptions are provided in columns I to R.

Table 2 Optimized LP-SS binary sequence sets

| M = 1, L = 8 | [0 1 1 0 0 1 0 1] [1 0 0 0 1 1 0 1] [1 0 1 0 0 1 1 0] [1 0 1 1 0 0 0 1] |
| --- | --- |
| M = 2, L = 16 | [0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 1] [0 1 0 1 1 0 1 0 0 1 1 0 0 1 0 1] [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] |
| M = 4, L = 32 Set#1: balanced ‘0’ and ‘1’ in each OFDM symbol | [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] |
| M = 4, L = 32 Set#2: a single ‘1’ in each OFDM symbol | [0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0 0 1 0 0 0 0 1 0 0 1 0 0 1 0 0] [0 0 0 1 0 1 0 0 0 0 0 1 0 0 0 1 0 1 0 0 0 1 0 0 0 1 0 0 1 0 0 0] [0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0 0 1 0 0 1 0 0 1 0 0 0] [0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 1 0 0] |

- Support LP-SS binary sequence sets in the following table, where for (M=4, L=32) a set can be down-selected between set#1 and set#2.

| M = 1, L = 8 | [0 1 1 0 0 1 0 1] [1 0 0 0 1 1 0 1] [1 0 1 0 0 1 1 0] [1 0 1 1 0 0 0 1] |
| --- | --- |
| M = 2, L = 16 | [0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1] [0 1 1 0 1 0 0 1 0 1 0 1 1 0 0 1] [0 1 0 1 1 0 1 0 0 1 1 0 0 1 0 1] [1 0 0 1 1 0 1 0 0 1 0 1 1 0 0 1] |
| M = 4, L = 32 Set#1: balanced ‘0’ and ‘1’ in each OFDM symbol | [0 1 0 1 1 0 1 0 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0 0 1 1 0 0 1 0 1] [0 1 1 0 0 1 0 1 0 1 1 0 0 1 0 1 1 0 0 1 1 0 1 0 1 0 1 0 0 1 0 1] [0 1 0 1 0 1 0 1 1 0 1 0 1 0 0 1 1 0 1 0 1 0 0 1 1 0 1 0 0 1 1 0] [0 1 0 1 0 1 1 0 0 1 0 1 1 0 1 0 0 1 1 0 0 1 1 0 1 0 1 0 0 1 0 1] |
| M = 4, L = 32 Set#2: a single ‘1’ in each OFDM symbol | [0 0 0 1 0 1 0 0 0 0 1 0 0 0 0 1 0 0 1 0 0 0 0 1 0 0 1 0 0 1 0 0] [0 0 0 1 0 1 0 0 0 0 0 1 0 0 0 1 0 1 0 0 0 1 0 0 0 1 0 0 1 0 0 0] [0 0 0 1 0 1 0 0 0 0 1 0 0 1 0 0 0 0 0 1 0 0 1 0 0 1 0 0 1 0 0 0] [0 0 0 1 0 1 0 0 0 0 0 1 0 1 0 0 0 1 0 0 0 0 1 0 0 0 1 0 0 1 0 0] |

## The feasibility of using LP-SS for frequency sync

The envelope-based LP-WUS receiver is expected to both time and frequency synchronized to the received signal, even the requirement can be relaxed compare to MR. For the agreed waveform OOK, it is shown in SI that it is robust against frequency error but sensitive to timing error. Nevertheless, if frequency error is not corrected, timing drift is accumulated accordingly. For example, a LO with frequency error of 50ppm will result in timing drift of 50μs within one second. To avoid frequent timing correction, it is also important to keep the frequency error low.

If a single branch architecture is used, it is unclear how frequency correction can be performed, since envelope signal is insensitive to frequency offset. In this case, it may rely on other means for frequency correction. For example, MR can be used to assist the initial time/frequency calibration of LP-WUR.

- MR can be used to calibrate the frequency of LP-WUR to control the initial frequency error.

In RAN1 #116bis, the following were agreed.

Agreement For timing error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of RTC/oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction/clock calibration by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type. Agreement For frequency error evaluation purpose, the following two options for residual frequency error are considered: - Option 1: The maximum frequency error (Fe) of oscillator is assumed, companies report Fe value and the applied LP-WUR type. - Option 2: The residual frequency error (Fr) after frequency error correction by LR or after assistance from MR is assumed, companies report Fr value, how to achieve it and the applied LP-WUR type.

Based on the above analyses, it is feasible to correct frequency error by LR or calibrate the frequency error based on MR. According to our analysis, the residual frequency error (Fr) can be <= 5ppm.

- For both timing and frequency error evaluation purpose, the residual frequency error (Fr) can be <= 5ppm after assistance from MR.

## Periodicity of LP-SS and preamble

Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

According to current RAN4 specification, UE shall obtain one RRM measurement result every DRX cycle, i.e., every 1.28s. When the serving cell measurement is offloaded to LP-WUR, to ensure that no larger latency of RRM measurement is introduced, the periodicity of LP-SS cannot be larger than 1.28s. Otherwise, large latency for RRM measurement results obtaining may lead to a restriction on UE moving speed. For example, if UE moves fast, and the periodicity of LP-SS is too long, UE may move out of the serving cell before UE notices it.

- ,If the periodicity of LP-SS is larger than 1280ms, the latency of obtaining RRM measurement results may be too large, which may restrict the UE max moving speed.

As discussed in our companion paper [2], it is essential for latency and paging reliability to support the mechanism that UE fallbacks to MR when LP-WUS coverage is not good enough. To ensure UE knows the coverage status in time, the LP-SS cannot be too sparse. As shown in Figure 23, in the best case, UE can know the coverage status based on LP-SS#1, and after the rampup time it is just its PO. But in the worst case, the gap between LP-SS#1 and PO is shorter than the rampup time, UE has to get the coverage status based on LP-SS#2, which is the former LP-SS of LP-SS#1. Therefore, ‘periodicity + rampup time’ should be no longer than an I-DRX cycle. Considering the rampup time can be 800ms and the I-DRX cycle length is 1.28s, the periodicity of LP-SS should be no longer than 480ms.

Figure 23 UE timeline for falling back to MR

- To support the functionality of falling back to MR and avoid too large latency of paging, the periodicity of LP-SS cannot be larger than 320ms.

- For LP-SS periodicity, to support the functionality of RRM measurement by LP-WUR and the functionality of fallback operation, values larger than 320ms are NOT supported.

As discussed and observed in section 5.1, for OFDM based receiver, the existing PSS and SSS can be used for time and frequency synchronization. And optionally, if overlaid sequence is used for LP-SS, LP-SS can also be used for time/frequency synchronization.

For envelope detection-based receiver, it is essential to use LP-SS for time synchronization. One issue to be discussed is what could be the required periodicity of LP-SS for performing time/frequency error estimation and correction. Obviously, large periodicity will deteriorate the performance of time and frequency tracking, while too small periodicity will lead to increased resource overhead.

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1 - T=1us(optional), 2us for OOK-4 with M=2 - T=0.5us(optional), 1us for OOK-4 with M=4

In RAN1 118, it was agreed that after a synchronization signal the residual time error is 1/2/5 us.The periodicity can be determined by the time error and frequency error requirement. The target time error for envelope detection is 2us and the target frequency error is 10ppm based on our LLS results in [3]. The required periodicity is analyzed in Table 3 following the agreed Model 1 [1] for time/frequency error. The result shows that to satisfy the time error requirement the periodicity of LP-SS can be up to ~200ms, and to satisfy the frequency error requirement the periodicity of LP-SS can be 50s. if a single LP-SS is designed for both time and frequency estimation/correction, the smaller value (i.e. ~200ms) should be used. Therefore, it is observed that the requirement of periodicity is dominated by the time error, since according to the model time error grows linear to frequency error even with fixed frequency error. And for the time error, residual frequency error contributes much more than the frequency drift since the drift value is small.

Table 3 Required periodicity of LP-SS for envelope detection

|  | Assumed residual time and frequency error | Target time or frequency error | Required periodicity of LP-SS |
| --- | --- | --- | --- |
| OOK4 M=4 | Time error = 1us Frequency error = 5ppm | Time error = 2us | 200 ms |
|  |  | Frequency error = 10ppm | 50 s |
| OOK4 M=2 | Time error = 2us Frequency error = 5ppm | Time error = 4us | 400 ms |
|  |  | Frequency error = 10ppm | 50 s |
| OOK4 M=1 | Time error = 5us Frequency error = 5ppm | Time error = 8us | 600 ms |
|  |  | Frequency error = 10ppm | 50 s |

- The requirement of periodicity is usually dominated by the time error, where the residual frequency error contributes most of the time error.

For the receiver of sequence detection, similar methods can be used to perform time and frequency synchronization, i.e., either based on MR calibration to assist LP-WUR or based on periodical synchronization signal. For example, if the receiver of sequence detection is capable to receive legacy SSS, legacy SSS can be utilized. In this case, to save the power, the receiver can receive only part bandwidth of SSS (e.g., 11 RBs, which is the same as the assumption for LP-WUS) via time domain correlator, and the periodicity to perform synchronization can also be as long as e.g., 160ms.

On the other hand, if the sequence(s) utilized to generate LP-SS is also known by UE, the receiver can also perform sequence correlation for synchronization. The target time and frequency error are similar as that for receiver with envelope detection [3], so the periodicity of 160ms is also enough for receiver with sequence detection.

- To support the functionality of synchronization by periodic LP-SS only, the configured periodicity of LP-SS can be less or equal to 160ms.

In RAN1#116, preamble was proposed by some companies to assist LP-WUR to perform time/frequency synchronization in addition to periodic LP-SS. It was argued that by adopting preamble, the periodicity of LP-SS can be longer, and thus the resource overhead can be reduced. However, per our understanding, introducing preamble can even increase the resource consumption of LP-WUS. An analysis is provided in Table 4, where the length of one LP-SS is assumed to be 8 OFDM symbols, and the length of one preamble is assumed to be 8 OFDM symbols, 4 OFDM symbols and 2 OFDM symbols, respectively. Beam sweeping of 4 is assumed for all the cases. As agreed in RAN1#118bis, the maximum number of subgroups per PO is 32. And in typical real NR deployment, there can be 64 POs per I-DRX cycle. Thus, in total there can be 32*64=2048subgroups. Assuming 3% group paging rate (which is the highest value to get power saving gain), about 60 LP-WUSs can be transmitted per I-DRX cycle. Therefore, in Table 4 we analyze the resource overhead for transmitting 30, 50 and 100 LP-WUSs per I-DRX cycle. It is noted that based on our analyses in Section 5.4.1, a periodicity that is no larger than 160ms is needed to satisfy the requirement of sync, RRM and fallback operation. Therefore, in Table 4 the overhead of 320ms is taken as the baseline for comparison, which is marked in green. From Table 4, it is observed that, on average the overhead will be increased by introducing preamble.

Table 4 Resource overhead of LP-SS and/or preamble

| Total overhead (OFDM symbols) |  | periodicity of LP-SS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 160 ms | 320 ms | 640 ms | 1280 ms |
| no preamble |  | 256 | 128 | 64 | 32 |
| with preamble= 8 OFDM symbols # of LP-WUS per 1.28s | 30 | 1232 (=30*4*8+256) | 1104 (=30*4*8+128) | 1024 (=30*4*8+64) | 992 (=30*4*8+32) |
|  | 50 | 1856 | 1728 | 1664 | 1632 |
|  | 100 | 3456 | 3328 | 3264 | 3232 |
| with preamble= 4 OFDM symbols # of LP-WUS per 1.28s | 30 | 736 | 608 | 544 | 512 |
|  | 50 | 1056 | 928 | 864 | 832 |
|  | 100 | 1856 | 1728 | 1664 | 1632 |

- Introducing preamble in addition to periodic LP-SS will increase the resource overhead.

- For LP-SS periodicity and preamble, support Option 1A, i.e., additionally support 160ms and does not support preamble.

# RRC parameters

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells.

Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

Based on the agreements, the RRC parameters configuration needs to consider the overlaid OFDM sequence, M value, and the LP-SS sequence.

- Capture the parameter in Appendix G in the RRC parameter list.

# Conclusions

In this contribution, the design of LP-WUS and LP-SS are discussed. The following observations and proposals are made:

- For the mapping from information bits to overlaid sequences, reversed order is better than forward order because of potential earlier termination of detection if the mapping order for OOK sequences is forward order.

- A frequency offset of a ZC sequence will cause a shift of the correlation peak location of the ZC sequence in time domain, and the extent of this shift is determined by the value of root q and should be minimized by selection of root q.

  - With minimized range of the shift of correlation peak, optimized ZC root selection for LP-WUS allows UEs to have smaller sliding window and thus is beneficial to reduce UE implementation complexity.

- Based on the property of ZC sequence

- If timing error is smaller than half of the minimum cyclic shift, ZC sequences with the same root but different cyclic shift is the best choice because of smaller cross-correlation.

- If timing error is larger than half of the minimum cyclic shift, ZC sequences with the different roots may be the best choice

- Based on the property of ZC sequence

- When different cyclic shifts of a ZC sequence with the same root are used as the candidate overlaid sequences, the timing error and frequency error need to be controlled within a target range when UE is detecting LP-WUS;

- When ZC sequences with different roots are used as the candidate overlaid sequences, the timing error and frequency error can be relaxed when UE is detecting LP-WUS;

- ZC sequences generated by the same root but difference cyclic shifts are not preferred to be used in different cells, since the timing difference between different cells may be larger than the minimum cyclic shift which results in strong interference.

- If different roots for ZC sequences are used in different cells, no matter how large the timing difference of different cells is, the aperiodic cross-correlation is still good enough.

- ZC sequences generated by the same root but different cyclic shifts are preferred to be used in the same cell to carry information for the following reasons,

- With the aid of LP-SS/SSB, the timing error and frequency error can be estimated and corrected to be smaller than a target range, thus LP-WUS can enjoy the benefit of lower cross correlation

- The maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell is mainly impacted by the orthogonality between sequences.

- When Manchester coding is used for OOK-4 M=4, bit inversion repetition can ensure the envelope of the CP and the first OOK symbol is always the same, which is beneficial to reduce the performance impact due to CP.

- If CP is used as a part of the first OOK symbol, the detection performance of the LP-WUS is improved when Manchester coding and bit inversion repetition is adopted.

- For 4x time domain repetition, transmit diversity by precoder cycling with two antenna ports can provide 2dB or more performance gain.

- Compared to repeated LP-WUS transmission over contiguous occasions, 1.5-2 dB gain can be obtained when the transmissions are over non-contiguous occasions. This is because more time diversity can be harvested.

- Compared to repeated LP-WUS transmission over contiguous occasions, 2.3 dB performance gain can be obtained with frequency hopping, when the signal bandwidth is 4.32MHz.

- For LP-SS, using different roots of ZC sequences on different OOK "on" symbols can improve receivingperformance.

- Low-density binary sequence to ensure one OOK ON symbol in each OFDM symbol for M=4 can also avoid transmit power wasting, but it impacts the frequency error estimation (for OFDM based receiver) and AGC.

- The correlation property is different depending on whether CP impact is included as the metric.

- The impact of CP can be reduced by concentrated OOK.

- MR can be used to calibrate the frequency of LP-WUR to control the initial frequency error.

- ,If the periodicity of LP-SS is larger than 1280ms, the latency of obtaining RRM measurement results may be too large, which may restrict the UE max moving speed.

- To support the functionality of falling back to MR and avoid too large latency of paging, the periodicity of LP-SS cannot be larger than 320ms.

- The requirement of periodicity is usually dominated by the time error, where the residual frequency error contributes most of the time error.

- To support the functionality of synchronization by periodic LP-SS only, the configured periodicity of LP-SS can be less or equal to 160ms.

- Introducing preamble in addition to periodic LP-SS will increase the resource overhead.

- For OOK-4, confirm the working assumption of M=4 is confirmed for 30kHz SCS.

- For RRC IDLE/INACTIVE state of both non-RedCap and RedCap/eRedCap UEs, in the case where associated CD-SSB and initial DL BWP have different SCSs, the SCS of LP-WUS is the same as that of the legacy initial DL BWP determined in subclause 12 of TS 38.213.

- For RRC CONNECTED state, the SCS of LP-WUS is the same as the SCS of active BWP where the triggered PDCCH monitoring is configured .

- For LP-WUS, UEs are configured to monitor one or multiple LP-WUS occasions and each occasion can convey a block of information bits.

- The bit length of the block of information is configurable or determined only from RRC configurations.

- One LP-WUS occasion comprises of one or multiple OFDM symbols.

  - Note: The OFDM symbol refers to the symbols after the processing “iFFT+CP” in S7.2.1.1 of TR 38.869

- FFS details of the pre-DFT sequences that refers to the input to the DFT/LS processing block in S7.2.1.1 of TR 38.869

  - The size of pre-DFT sequence set

  - Sequence generation/selection

- FFS the mapping from a block of information bits to pre-DFT sequences and OFDM symbols

  - FFS: whether the series of pre-DFT sequences to wake up a UE only depends on UE-specific predetermined information

- Further discuss what are the candidate overlaid OFDM sequences that can be configured by gNB when overlaid OFDM sequence does not carry information.

- For the mapping from information bits to overlaid sequences, support a mapping order different from the mapping order of OOK sequences-based modulation, to enable OFDM based receiver to terminate LP-WUS reception as early as possible.

- In order to reduce resource overhead, transmission duration of a LP-WUS targeting to wake up OFDM based receiver can be shorter than the transmission duration required for ED based receiver.

- In order to make ZC sequences robust to timing/frequency error, optimized roots of ZC sequences are selected.

- Different overlaid ZC sequences can be used to minimize the interference between different cells.

  - Prefer to use different roots for different cells to minimize the interference.

  - Prefer to use different cyclic shifts to carry information.

- Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, support maximum 8 candidates overlaid sequences for M=2 and 16 candidates overlaid sequences for M=1.

- For M>1, Support the sequence length, with ,.

- Assuming that only cyclic shifts are used to differentiate candidate overlaid sequences for a cell, the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for the cell is 4.

- The frequency sequence after DFT is mapped to the center of 132 REs with 2 guard REs at each side.

- For the overlaid OFDM sequence in time domain, support Alt2 for M=2,4, i.e., ,, whereis given by the largest prime number such that,  is the overlaid OFDM sequence length.

- Bit-level time domain repetition is considered to improve the detection performance of LP-WUS.

  - Manchester coding and  bit inversion repetition can be used to solve CP issue.

  - CP can be used as a part of signal.

- Signal-level time domain repetition, OFDM symbol-level time domain repetition and transmit diversity by precoder cycling are considered to improve the performance of LP-WUS.

- Coverage recovery schemes that exploit time / frequency diversities are considered.

- Binary spreading sequences are considered to multiplex WUSs on the same time-frequency resource to improve the BLER.

- Regarding the LP-WUS information for connected UEs, support the codepoint mapping method, i.e. option 3.

- For the M value relationship between LP-WUS and LP-SS, support Alt2.

- For LP-SS with OOK-4 M>1, the OFDM sequence(s) that are overlaid on the LP-SS are always indicated to UEs.

- LP-SS has similar design as LP-WUS, including at least the following aspects：

  - pulse shaping methods, including the concentrated waveform and the spectrum adjustment

- The design of LP-SS should follow the guidelines:

  - Good auto-correlation and cross-correlation property taking into account modulation with CP

  - Balanced '0's and '1's in the whole sequence

  - Balanced ‘0’ and ‘1’ within OFDM symbols for M>1.

- To reflect the impact of transmitter/receiver, modulation with CP should be included in the calculation of the cross-correlation, which is also implicitly required by the template for collecting LP-SS binary sequence results.

- For LP-SS, support at least 8 OFDM symbols for OOK-4 with M=1/2/4.

  - M=1, L= 8

  - M=2, L= 16

  - M=4, L= 32

- For designing set of binary LP-SS sequences, mis-matched correlation or unipolar correlation should be considered as the detection algorithm for binary sequence design, where mis-matched correlation is preferred than unipolar.

- Support LP-SS binary sequence sets in the following table, where for (M=4, L=32) a set can be down-selected between set#1 and set#2.

- For both timing and frequency error evaluation purpose, the residual frequency error (Fr) can be <= 5ppm after assistance from MR.

- For LP-SS periodicity, to support the functionality of RRM measurement by LP-WUR and the functionality of fallback operation, values larger than 320ms are NOT supported.

- For LP-SS periodicity and preamble, support Option 1A, i.e., additionally support 160ms and does not support preamble.

- Capture the parameter in Appendix G  in the RRC parameter list.

# References

TR 38.869, Study on low-power wake up signal and receiver for NR

R1-2403949, Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode, Huawei, HiSilicon.

R1-2308647, LLS results for LP WUS in RAN1#114, Moderator (Nordic Semiconductor ASA)

R1-2306550, “Further details on signal design and procedure for LP-WUS”, Huawei, HiSilicon, RAN1#114, Toulouse, France, August 21 – 25, 2023.

S. Lin and D. J. Costello, Error control coding, 2nd edition, Pearson Prentice Hall, NJ, 2004.

T. Mano, K. Yamashita, T. Nagasato and C. -J. Ahn, “Sum-product decoding based simple Hamming code for low power mobile devices,” in Proc. IEEE Int. Conf. on Consumer Electronics-Asia (ICCE-Asia), Yeosu, Korea, Republic of, 2022, pp. 1-4.

TS38.211 NR; Physical channels and modulation.

Chairman’s notes RAN1#116.

Chairman’s notes RAN1#116bis.

Chairman’s notes RAN1#117.

Chairman’s notes RAN1#118.

Chairman’s notes RAN1#118bis.

Chairman’s notes RAN1#119.

R1-2410925, Template for collecting LP-SS binary sequence results, vivo.

N. Levanon, "Noncoherent pulse compression," in IEEE Transactions on Aerospace and Electronic Systems, vol. 42, no. 2, pp. 756-765, April 2006.

D. -J. Deng, S. -Y. Lien, C. -C. Lin, M. Gan and H. -C. Chen, "IEEE 802.11ba Wake-Up Radio: Performance Evaluation and Practical Designs," in IEEE Access, vol. 8, pp. 141547-141557, 2020.

R1-2306548, Evaluations for LP-WUS, Huawei, HiSilicon.

RP-240801, Revised WID Low-power wake-up signal and receiver for NR, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.

R1-2407556, Rel-17 RedCap UE behavior correction on initial DL BWP, ZTE, etc., RAN1#118, August 2024.

# Appendix A

The cascade of DFT-precoding, fftshit operation, and IFFT results in a sinc-interpolation of the signal input.  However, if this operation is removed, the input signal is interpolated with different phase functions which result in a mismatch between the input signal and interpolated output signal. Figure 24 shows the sliding crosscorrelation (or aperiodic correlation) between the ZC sequence used for bit spreading, and the output CP-OFDM symbol with corresponding downsampling rate. If there is no fftshift operation at the input, then the cross-correlation does not display any clear peak as the legacy DFT-s-OFDM modulation changes the enveloped and phase of the interpolated signal. If the fftshift operation is applied, the output signal is a proper interpolated version of the input signal, and then as expected a clear correlation peak appears when the correlation window is shifted of about the CP length of 36 samples.

Figure 24 Impact of fftshift operation at OOK-4 signal generation  no  on the correlation between input and output signals.

# Appendix B

Table 4 Simulation assumptions for Figure 17

| Parameters | Assumptions |
| --- | --- |
| Packet size | 8 bits |
| Waveform | OOK4 M=4 |
| Signal bandwidth | 11RB (4.32MHz) |
| Subcarrier spacing | 30kHz |
| Channel model | TDL-C 300ns |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Manchester coding | 1/2 Manchester code applied |
| Time / frequency synchronization | Ideal |
| ADC | 3.84MHz, with 8 bit resolution |
| Others | - For FH with 11RB BW, frequency hopping step size is assumed to be 7.68MHz - For repeated transmission over non-contiguous occasions, the interval between two repetitions is assumed to be 8.6 ms, i.e. each LPWUS transmission lasts for 24 OFDM symbols 0.86 ms. And 10 LPWUS from different users are time multiplexed, which accounts for 8.6ms interval between repetitions. |

Table 5. Simulation parameters for Figure 19

| Parameters | Assumptions |
| --- | --- |
| Packet size | 8 or 16 bits |
| Waveform | OOK4, M=2 and 4, with Zadoff-Chu sequence |
| Signal bandwidth | 4 RB (1.44MHz) |
| Subcarrier spacing | 30 kHz |
| Channel model | TDL-C, 300ns |
| UE speed | 3 km/h |
| Number of RX antennas | 1 |
| Manchester coding | Rate 1/2 |
| Repetition code | If L=8, 3 bits are repeated 3 times, 5 bits are not repeated If L=16, 6 bits are repeated 3 times, 10 bits are not repeated |
| Time / frequency synchronization | Ideal |
| ADC | Ideal |

# Appendix C

The simulation result of sequence detection on OOK-4 signal considering interference from neighbor cell are shown in Figure 25, where the x-axis is SINR accounting both the noise power and the interference power. In this simulation, we assume 1/8 of the interference power contributed by a random OOK-4 signal from a neighbor cell in the same bandwidth. Different ZC sequence (e.g. ZC sequence with different root) is used for different cell.

It can be observed that sequence detection is more robust against interference than envelope detection. The performance loss is about 1 dB for 1/8 LP-WUS interference respectively for sequence detection, while more than 2.5dB performance degradation is observed for envelope detection for the same cases.

Figure 25  Link level performance of sequence detection on OOK-4 signal considering interference from neighbor cell

To compare the coverage performance for different modulation types, the MIL for different modulation types and MIL target are drawn in Figure 26 to show the coverage margin that LP-WUS can provide for different modulation types in a straight forward way. In Figure 26, modulations (OOK-4 and sequence detection of OOK-4) are shown as example. 56 kbps corresponds to the case where neither repetition nor precoder cycling is used, 28k bps corresponds to the case where 2 repetitions and precoder cycling is used, and 14 kbps corresponds to the case where 4 repetitions and precoder cycling used. For OOK-4 and FSK-2 the FAR is <10-11, and for sequence detection of OOK-4 the FAR is ~10-3. Under the ideal time and frequency error, and compared with Msg3 MIL target:

-	OOK has 1dB coverage gap under 14 kbps data rate;

-	Sequence on top of OOK can achieve Msg3 MIL under 56 kbps with ~3 dB margin, which can be made up by power boosting feasible from gNB.

Therefore, to obtain the same coverage performance, repetition 4 times is needed for envelop detection than sequence detection.

Figure 26  Coverage performance for some schemes

# Appendix D

During previous meetings benefits and motivations of using Manchester encoding for LP-SS was also discussed. We see the following pros and cons of Manchester encoding for LP-SS.

- Pros:

  - Manchester encoding is an easy solution to provide balance sequences with equal number of 0s and 1s, both, inside the whole sequence and within OFDM symbol for M>1 , guaranteeing to fully utilize the gNB power and help AGC tracking.

  - The mismatched autocorrelation function of Manchester encoded sequence is a function of the autocorrelation function of the bipolar sequence before Manchester encoding [15] as explicitly given in Appendix E, and from this property it can be observed that the main lobe will systematically be surrounded by deep negative values. A similar effect can be observed for unipolar aut-correlation. Thus Manchester encoding have a systematic tendency to create sequences whose auto-correlation has narrow main lobe, which  is beneficial to provide good timing estimation performance of LP-SS.

- Cons: The design dimension is halved reducing the search space for finding the best set. Nevertheless, for the purpose of designing a small sequence set, it does not preclude neither to provide a close to optimum solution; and in practice for long sequences, a smaller search space can often lead to a better solution than with an intractable large search space.

Considering the aboveManchester encoded binary sequence for M>1  provide tractable way to design sequences with good auto-correlation properties with useful balanced  0/1 properties.

In our previous contributions, we considered a generic framework for any set size and sequence lengths based on simple modification M-sequence with cyclic shifting, extension/truncation, binary complement, decimation, to obtain multiple binary LP-SS sequences with good correlations, and still enable if desired low-complexity generation. Specifically, a set of binary sequences of length L  is constructed based on a single M-sequence. Precisely, an L’-long sequence can be obtained as,

s(f,cb,ca,d)n=f ⊕ mdn+cbmod Lm+ca,

for n= 0,…,L'-1 and defined from the tuples (f,cb,ca,d)  where

-       mn is an m-sequence of length Lm

-       L'=L/2 If the sequence is Manchester encoded, if not then L'=L

- f∈0,1 either returns the sequence or its binary complementary version,

- ca∈0,…,Lm-1 is a shift before extension/truncation

- cb∈{0,…,L'-1} is a shift after extension/truncation

- and d>0 is a decimation, where it is satisfied that  gcd(d,L’)=1.

Existing sequences from analytical construction may incidentally be selected by computer search on all sequences for complete optimization. In Table 2, the set for (M=1, L=8),  is made of two sequence pairs where for each pair one sequence is the time-reversed of the other, and modifications of the cyclically extended 7-length M-sequence m=1 0 0 1 0 1 1 where f,cb,ca,d = (0, 2, 6, 3), (1, 4, 0, 1), (1 2 0 3), (1 4 5 1). For (M=2, L= 16), the set is made of Manchester encoding of modifications of the same 7-length M-sequence m=[1 0 0 1 0 1 1] where f,cb,ca,d = (1,0,2,1), (0,3,3,3), (0,6,0,1), (1,2,3,3). For (M=4, L= 32), the set is made of Manchester encoded sequences without being simple modifications of M-sequences.

# Appendix E

In [15], it is shown through several examples that the mismatched autocorrelation of a Manchester encoded sequence is related to the autocorrelation function of the bipolar sequence before Manchester encoding. This property is provided in details here. In general, the aperiodic correlation of two sequences s1n and s2n of length L is defined as

Cs1,s2d=n=0L-1+ds1ns2[n-d]-L<d<0n=0L-1-ds2ns1n+d0≤d<L0d≥L

Consider a bipolar sequence b0n∈{-1,1}  with aperiodic autocorrelation Cb0d. Consider its unipolar equivalent u0n=(b0n-1)/2∈{0,1} which is then Manchester encoded into un∈{0,1}. The mismatched correlation between  un∈{0,1} with its bipolar version bn=2 un-1∈{-1,1}   is given by

Cu, bd= Cb0δif d=2δ even -1/2Cb0δ+ Cb0δ+1-b0[N-δ-1]if   d=2δ+1 odd,  d≥0-1/2Cb0δ+ Cb0δ+1+b0[-δ-1]if   d=2δ+1 odd,  d<0

Therefore, the mismatched auto-correlation of a Manchester encoded signal for every even samples correspond to the autocorrelation of the original bipolar sequence, while for the odd sample it is the negative of the averaging of two values of the autocorrelation of the original bipolar sequence  (with a +/-1/2 offset that depends of the sequence itself).

# Appendix F

The expectation of average number of bits for LP-WUS information by codepoint based indication is

EAn=max log2N,7*pg*N*Repcodepoint

Where

- N is the total number of subgrougs.

- pg is the per subgroup paging rate (e.g. 1%).

- log2N is the number of bits for codepoint based indication.

- Repcodepoint is the number of retransmissions required for the codepoint to meet the MDR requirements. When the number of subgroups is 8 , 16 and 32, Repcodepoint is 4.

In order to make sure that the FAR is smaller than 1%, the bit number for a codepoint should not be smaller than 7.

For bitmap based indication, in the case that LP_WUS information would still be sent even if no subgroup is paged ,the expectation of average number of bits for LP-WUS information by bitmap based indication is

EAn=N+BCRC

For bitmap based indication, in the case that LP_WUS information is no need to be sent when no subgroup is paged ,the expectation of average number of bits for LP-WUS information by bitmap based indication is

EAn=N+BCRC1-1-pgN*Repbitmap

=N1-1-pgN+X*Repbitmap

Where

- N is the total number of subgrougs.

- pg is the per subgroup paging rate (e.g. 1%).

- BCRC is the number of CRC bits.

- Repbitmap is the number of retransmissions required for the bitmap to meet the MDR requirements. . When the number of subgroups is 8, 16 and 32, Repbitmap is 5 , 8 and 10.

In order to make sure that the FAR is smaller than 1%, BCRC should not be smaller than 6.

# Appendix G

| WI code | Sub-feature group | RAN1 specification | Section | RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | Required for initial access or IDLE/INACTIVE | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New |  | Config ZC root(s) for LP-WUS |  |  |  |  | 38.331 | Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New |  | Config ZC root(s) for LP-SS |  |  |  |  | 38.331 |  |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New |  | Config cyclic shift(s) of ZC for LP-WUS |  |  |  |  | 38.331 |  |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New |  | Config M value for LP-WUS |  |  |  |  | 38.331 | Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New |  | Config M value for LP-SS |  |  |  |  | 38.331 |  |
| NR_LPWUS | Signal Design of LP-WUS and LP-SS |  |  |  |  |  | New or existing |  | Config the information to determine binary LP-SS sequence |  |  |  |  | 38.331 | Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration |