3GPP TSG RAN WG1 #120                                                      R1-2500177

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.6.2

Source:	Spreadtrum, UNISOC

Title: 	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Document for:	Discussion and decision

Introduction

It was agreed in WID [1] to specify LP-WUS procedures and configurations in idle/inactive state.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In this contribution, we will discuss procedure and configurations of LP-WUS

Definitions of LO and LMO

LP-WUS occasion (LO) and LP-WUS monitoring occasion (LMO) were agreed to be introduced in RAN1#116 [2].

Agreement LP-WUS occasions (LOs) are defined for LP-WUS monitoring. - Each LO has one or more LP-WUS monitoring occasions (MOs), where UE can monitors for LP-WUS transmission in each of the LP-WUS MOs. - Different LP-WUS MOs may correspond to different beams in multi-beam operation - It is not precluded that FFS whether or not each LO is defined as a time window that covers the corresponding LP-WUS MOs - FFS details - It is at least supported that a UE monitors LOs with a configured periodicity. - Each UE has a periodicity for LO monitoring, and it is at least supported that a UE monitors one LO per period. - FFS: A UE does not expect its LP-WUS monitoring occasions overlapping in time - FFS: monitoring of multiple more than one LOs per period e.g. if LP-WUS common to all UEs is supported or in case of eDRX (if supported) - FFS eDRX, if supported

In RAN1#116bis, it was agreed the number of LMO includes N (the number of transmitted beams) and K (the number of monitoring occasion per transmitted beam) [3].

Agreement Each LO consists of N * K LP-WUS MOs, where N is the number of beams corresponding to LP-WUS, and K is the number of LP-WUS MOs for each beam. - Option 1: K = 1 - Option 2: K can be larger than or equal to 1 - FFS if more than 1 LP-WUS is transmitted from the same beam, whether the information in these multiple LP-WUS is always the same or can be different

For parameter K, when K>1, it can be interpreted as different meanings.

- Interpretation-1: K means K repetitions within a transmitted beam.

- Interpretation-2: K means K sets of information bits which are different.

For Interpretation-1, the purpose is coverage enhancement. However, coverage enhancement is not officially agreed so far, and repetition across transmitted beams may be enough for coverage improvement.

For Interpretation-2, the purpose is more information bits carried by an LP-WUS occasion. Nevertheless, more information bits in an LP-WUS monitoring occasion may be enough, which is being discussed in agenda item 9.6.1, or multiple bit blocks in an LP-WUS monitoring occasion like PEI is not exclude in agenda item 9.6.1. Using K for more information bits purpose is too redundant.

In the later discussion, both Interpretation 1 and 2 are valid, i.e. K has both two purposes including repetition and subgroup information container.

In RAN1#118bis, these two functions (repetition and subgroup information container) is supported and can be configured flexibly by gNB [4].

Agreement When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between - Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - M = 1 and M > 1 is supported. - FFS: detailed UE monitoring behavior - FFS R=1 or R>= 1 - Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID. - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - FFS: detailed UE monitoring behavior - M = 1 and M > 1 is supported. - FFS R=1 or R>=1 - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO.”

In RAN1#119, the futher agreement on LO and LMO had been achived as following [5].

Agreement At least for the 1:1 LO to PO mapping, - The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

For Option A, an LO can be divided into M sub-LO. A sub-LO can correspond to multiple (depending on the payload Size of the LP-WUS) UE groups. In this case, it is possible to reduce the LP-WUS payload size while allowing an LO to support more UE groups.

For Option B, it is a more complex LMO partitioning method compared to Option B

From UE perspective, we prefer Option A.

Proposal 1: Option A should be supported for definition of LO and LMO.

Offset between LO and PO

Offset issue is more complicated for LP-WUS compared with PEI, since offset and wakeup delay are mutually coupled.

In RAN1#118bis, there are some options initially agreed.

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

In RAN1#119, further agreement on the offset value(s) between an LO and a reference PO/PF had been achieved as following.

Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1

For Option 1, gNB configures a single offset value for UE to determine the location of LO. From UE respective, one PO corresponds to one LO. For Option 2, gNB configures one or multiple offset values for UE to determine the location of LO. From UE respective, one PO corresponds to one or multiple LO(s). In our view, Option 2 presents a serious resource overhead problem

Therefore, we prefer Option-1.

Proposal 2: gNB configures a single offset value for UE to determine the location of LO (i.e., Option 1) should be supported.

For Option 1, if the gap between an LO and the PO associated with the offset is less than the wake-up delay a UE supports, we prefer that the UE follows the legacy paging monitoring procedure(i.e., Option 1-1).

Proposal 3: For Option 1, if the gap between an LO and the PO associated with the offset is less than the wake-up delay a UE supports, we prefer that the UE follows the legacy paging monitoring procedure(i.e., Option 1-1).

In addition, for UE capability report on the wake-up delay, the following alternatives had been discussed in last meeting.

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

For UE capability report on the wake-up delay, we prefer Alt 1.

The maximum number of subgroups per PO

In RAN1#117, it was agreed that the maximum number of subgroups per PO is X, and value of X is FFS.

Agreement For idle/inactive mode, the maximum number of subgroups per PO is X, where 8 <= X <= 256. - FFS the exact value of X.

In RAN1#118, it was further listed for candidate X value.

Agreement For the maximum number of subgroups per PO (X), down-select from the following options in RAN1#118bis: - Option 1: X = 8 - Option 2: X = 16 - Option 3: X = 32 - Option 4: X = 64 - Option 5: X = 128 - Option 6: X = 256

In RAN1#118bis, it was working assumption that the maximum number of subgroups per PO is 32.

Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

In this way, if one LO is mapping to one PO, in typical case, the information bits is 5 (code-point based) for each LP-WUS MO in each beam. However, we have no complete idea for K value, which is also related to overhead for each LP-WUS MO in each beam. Therefore, we can wait for the details for K value before confirmation of the working assumption.

Proposal 4: We can wait for the details for K value before confirmation of the working assumption.

Conclusion

We have the following proposals.

LP-WUS occasion and monitoring occasion

Proposal 1: Option A should be supported for definition of LO and LMO.

Proposal 2: gNB configures a single offset value for UE to determine the location of LO (i.e., Option 1) should be supported.

Proposal 3: For Option 1, if the gap between an LO and the PO associated with the offset is less than the wake-up delay a UE supports, we prefer that the UE follows the legacy paging monitoring procedure(i.e., Option 1-1).

Proposal 4: We can wait for the details for K value before confirmation of the working assumption.

Reference

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, CMCC, RAN#102, Dec. 11th - 15th, 2023.

- 3GPP RAN1#116 Chair’s Notes, Feb. 26th – Mar. 1st, 2024.

- 3GPP RAN1#116bis Chair’s Notes, Apr. 15th – 19th, 2024.

- 3GPP RAN1#118bis Chair’s Notes, Oct. 14th – 18th, 2024.

- 3GPP RAN1#119 Chair’s Notes, Orlando, US, November 18th – 22nd, 2024.