3GPP TSG RAN WG1 #120                                                  R1-2500239

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.2.4

Source:		TCL

Title:			Discussion on asymmetric DL sTRP/UL mTRP scenarios

Document for:	Discussion and Decision

1 Introduction

In RAN#102 meeting, the Rel-19 WID of NR MIMO Phase 5 was approved [1]. The following objective is related to this AI.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In this contribution, our opinions on asymmetric DL sTRP/UL mTRP scenarios are presented.

2 Discussion

2.1 Deployment scenarios

Within the NR system, the UL performance has always been significantly constrained due to the limitations of UE transmission power and the number of antennas. Particularly for UEs at the edge of the cell, power control often employs partial PL compensation, which further diminishes the UL performance of these UEs. Moreover, the currently commercially used networks generally employ TDD patterns with a large number of DL resources, which further restricts UL performance.

To enhance UL performance, a rudimentary idea would be to increase the deployment density of base stations. However, given that the DL performance is sufficient, it may be considered to only increase the base stations only with UL nodes. Such base stations only need to receive signals, without the need to transmit signals, thus conserving energy consumption. Furthermore, these base stations only require a receiving system, without the need for a transmitting system, thereby saving on hardware costs.

In WID, the deployment scenario with less DL nodes and more UL nodes is called as an asymmetric DL sTRP/UL mTRP deployment scenario. These TRPs are non-co-located and ideal-backhaul. Meanwhile, these TRPs utilize the same bandwidth part. Figure 1 illustrates two deployment modes. In Figure 1 (a), UL reception and DL transmission are completely separated; the DL-only TRP only transmits DL signals, while the UL-only TRP only receives signals. In Figure 1 (b), UL reception and DL transmission are partially separated; the DL and UL TRP can transmit signals and receive signals, while the UL-only TRP only receives signals.

Figure 1. Two modes of asymmetric DL sTRP/UL mTRP deployment scenario.

- The asymmetric DL sTRP/UL mTRP deployment scenario have two modes, complete separation and partial separation.

2.2 TCI framework

The following conclusion on unified TCI framework for the asymmetric DL sTRP/UL mTRP deployment scenarios was made in RAN1#117 meeting:

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In current specifications, in unified TCI framework, TCI state modes are joint TCI state mode and separate TCI state mode. If UL and DL channels are reciprocal, the former is applicable. If UL and DL channels are not reciprocal, the latter is applicable. When unifiedTCI-StateType-r17 is configured as joint, the UE employs a joint TCI state mode. When unifiedTCI-StateType-r17 is configured as separate, the UE employs a separate TCI state mode. The network cannot configure two different TCI state modes for the UE simultaneously. If a joint TCI state mode is employed, the network configures a joint/DL TCI state list for the UE, activating and indicating one or more joint/DL TCI states. The indicated joint/DL TCI state is used for both UL and DL transmission. If a separate TCI state mode is employed, the network not only configures a joint/DL TCI state list for the UE, but also configures an UL TCI state list. The network activates and indicates one or more joint/DL TCI states from the joint/DL TCI state list, as well as one or more UL TCI states from the UL TCI state list. The indicated UL TCI state(s) is used for UL transmission, while the indicated joint/DL TCI state(s) is used for DL transmission.

For FR2 with asymmetric DL sTRP/UL mTRP deployment scenario, if a joint TCI state mode is configured, the network configures a joint/DL TCI state list for the UE but does not configure an UL TCI state list. However, as the UL-only TRP cannot transmit DL signals and can only receive UL signals, the UE must use an UL TCI state when communicating with the UL-only TRP. Therefore, the joint TCI state mode is not suitable for this deployment scenario. If a separate TCI state mode is configured, the network configures DL TCI state and UL TCI state lists for the UE. However, the separate TCI state mode means that UL and DL channels are not reciprocal, even for the DL and UL TRP. Even if UL and DL channels are reciprocal between UE and DL and UL TRP, UE should also perform UL beam management. This results in unnecessary resource and energy consumption.

In order to make better use of the UL and DL reciprocity of DL and UL TRP, introduce a new unified TCI state mode (e.g. named as partialJoint) which is used to indicate partial joint TCI state mode. When unifiedTCI-StateType-r17 is configured as partialJoint, the UE employs a partial joint TCI state mode, wherein the DL TCI state and UL TCI state corresponding to DL and UL TRP are identical, and the UL TCI state corresponding to UL-only TRP is different from the TCI states corresponding to DL and UL TRP. If a partial joint TCI state mode is employed, the network configures a joint/DL TCI state list and an UL TCI state list for the UE. The network activates and indicates one or more joint/DL TCI states from the joint/DL TCI state list, as well as one or more UL TCI states from the UL TCI state list. The indicated UL TCI state(s) is used for UL transmission, while the indicated joint/DL TCI state(s) is used for DL and UL transmission. In fact, the configured joint/DL TCI state list, the activated joint/DL TCI states and the indicated joint/DL TCI state are corresponding to the DL and UL TRP. The configured UL TCI state list, the activated UL TCI states and the indicated UL TCI state are corresponding to the UL-only TRP. Therefore, UL beam management is not need to be perform between UE and DL and UL TRP. This can save resource and energy consumption.

- Introduce a new unified TCI state mode (e.g. named as partialJoint) which is used to indicate partial joint TCI state mode.

2.3 Power control

2.3.1 Path loss acquisition

In current specifications, when UE performs initial access, the UE will identify a SSB with superior signal quality. If the UE adopts 4-step random access, in subsequent transmission of msg1 and msg3 and receptions of msg2 and msg4, the UE employs the transmission and reception beams corresponding to the identified SSB beam. If the UE adopts 2-step random access, in subsequent transmission of msgA and receptions of msgB, the UE also employs the transmission and reception beams corresponding to the identified SSB beam.

For asymmetric DL sTRP/UL mTRP deployment scenario, if the UE is close to the DL and UL TRP, the UE employs the access method in the current specifications to connect to the network via the DL and UL TRP. However, if the UE is close to the UL-only TRP, utilizing the access method in the current specifications to connect to the network via the DL and UL TRP could result in significant interference and large latency. In such instances, it is preferable for the UE to receive DL signals of initial access from the DL and UL TRP and transmit UL signals of initial access to the UL-only TRP. This procedure can reduce interference and latency.

A straightforward method is to include the PL information in system information (e.g. SIB1). When UE performs initial access, UE use this PL information to transmit msg1 and msgA. For msg3, the UE can also use the PL in system information. Besides, because msg3 is scheduled by msg2, the PL information for msg3 can be carried by msg2.

- SIB1 can carry PL information for msg1, msgA and msg3.

- Msg2 can carry PL information for msg3.

The asymmetric DL sTRP/UL mTRP deployment scenario have two modes, complete separation and partial separation. In partial separation scenario, UE acquires the PL of the UL-only TRP according to the corresponding PL of the DL and UL TRP and the PL offset. At UE side, the PL offset is configured by RRC parameters and updated by MAC CE. At network side, the PL offset is acquired according to the difference value between UL measurement of the UL-only TRP and the DL and UL TRP. However, in complete separation scenario, there is not a DL and UL TRP and there are a DL-only TRP and an UL-only TRP. At network side, the PL offset cannot be acquired because the DL-only TRP cannot perform UL reception.

- In complete separation scenario, network side cannot acquire PL offset.

It is necessary to find a new mechanism to measure PL between UE and the UL-only TRP in complete separation scenario. A straightforward way to acquire PL of the UL-only TRP at network side is to measure the UL signals, such as SRS. Then network configures and/or indicates the measured PL value to UE. This mechanism can also apply to partial separation scenario. It is simpler and more universal than the mechanism of PL offset.

- Network can acquire PL of the UL-only TRP according to the measurement of UL signals and indicates to UE.

2.3.2 Closed loop power control

The following agreement on the indication of independent closed-loop power control for SRS for the asymmetric DL sTRP/UL mTRP deployment scenarios was made in RAN1#119 meeting:

Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

In current specification, DCI format 2_3 with several TPC command fields is used for SRS CLPC adjustment. DCI format 2_3 is a group common DCI format, which can be used for multiple CCs and multiple UEs. The main purpose of this design can reduce overhead of UE-specific DCI formats such as DCI format 0_1 and 1_1. The main usage of UE-specific DCI formats is to schedule PUSCH or PDSCH, on the other words, to guarantee the data transmission. While in this agreement, a TPC command filed with 2 bits for one of separate SRS CLPC adjustment states is introduced in DCI format 1_1. And to distinguish the separate SRS CLPC adjustment states, additional 1 bit is also introduced. This increase overhead of UE-specific DCI format. If multiple CCs and UEs need to adjust separate SRS CLPC simultaneously, a number of DCI format 1_1 need to transmit. This causes frequently PDCCH blocking.

- Do not support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH.

Agreement For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

In current specifications, if an SRS resource set is not configured with followUnifiedTCI-StateSRS, the PL estimation and CLPC adjustment state of an SRS resource in the SRS resource set is determined by the TCI state of the SRS resource with the lowest ID. This is the rule specified in Rel-17.

If the SRS resource with the lowest ID is not provided the TCI state, current specifications imply that it is an error case and gNB can avoid this case by implementation. Therefore, extra enhancement is not necessary.

- Support Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement.

3 Conclusion

In these contributions, our opinions on asymmetric DL sTRP/UL mTRP scenarios are listed as follows.

- The asymmetric DL sTRP/UL mTRP deployment scenario have two modes, complete separation and partial separation.

- Introduce a new unified TCI state mode (e.g. named as partialJoint) which is used to indicate partial joint TCI state mode.

- SIB1 can carry PL information for msg1, msgA and msg3.

- Msg2 can carry PL information for msg3.

- In complete separation scenario, network side cannot acquire PL offset.

- Network can acquire PL of the UL-only TRP according to the measurement of UL signals and indicates to UE.

- Do not support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH.

- Support Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement.

4 Reference

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung.