3GPP TSG RAN WG1 #120			R1-2500249

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.3.1

Source: 	LG Electronics

Title: 	Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion and decision

Introduction

In RAN#102 meeting, a work item on Evolution of NR duplex operation is approved and the corresponding description is provided in [1].

In this contribution, the following are discussed for SBFD Tx/Rx/measurement procedures based on the agreements and discussions in the last meetings [2][4][6][8][10][12].

- Semi-static indication of time location of SBFD subbands

- Semi-static indication of frequency location of SBFD subbands

- Transmission and reception behaviors on SBFD subbands

- Resource allocation in frequency domain in SBFD symbols

- Physical channels/signals and procedure across SBFD and non-SBFD symbols

- Configuration for SRS, PUCCH, and PUSCH on SBFD and non-SBFD symbols

- Collision handling between DL reception in DL subband(s) and UL transmission in UL subband

Time/Frequency location indication of SBFD subbands

- 2.1	Semi-static indication of time location of SBFD subbands

The following agreements regarding time location indication of SBFD subband were made in the previous meetings [2][4][6][8].

RAN1#116 (2024.02) [2] Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement: For RRC connected mode UEs, SBFD subband time locations are configured within a period. At least when only one TDD-UL-DL pattern is configured, the period is down-selected from one of the following options. - Option 1: The period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - Option 2: The period is integer multiple of TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - FFS: Further details FFS: Details when two TDD-UL-DL patterns are configured Agreement A slot can consist of SBFD symbols and non-SBFD symbols. Agreement A slot can consist of SBFD symbols and non-SBFD symbols. For semi-static indication of SBFD subband time location, - When only one TDD-UL-DL pattern is configured, SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. - When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for only one of the patterns, SBFD symbols are configured in consecutive manner within the TDD-UL-DL pattern period. - When two TDD-UL-DL patterns are configured and if SBFD symbols are configured for both patterns, SBFD symbols are configured in consecutive manner within each TDD-UL-DL pattern period. - SBFD symbols are configured in DL and/or flexible symbols configured in TDD-UL-DL-ConfigCommon - The configured SBFD symbols can start from any symbol within a slot and can end in any symbol within a slot. - referenceSubcarrierSpacing in TDD-UL-DL-ConfigCommon is used as reference SCS. - FFS details Agreement For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL usable PRBs are allowed - FFS SSB symbols - DL receptions within DL usable PRBs are allowed - UL transmissions outside UL usable PRBs are not allowed - DL receptions outside DL usable PRBs are not allowed - This restriction is not applicable for CLI measurement CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3. RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 - DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

RAN1#116bis (2024.04) [4] Agreement A symbol configured as SBFD symbol via cell-specific configuration cannot be reverted to a non-SBFD symbol via any UE-specific configuration or group-common signaling. A symbol not configured as SBFD symbol via cell-specific configuration cannot be reverted to an SBFD symbol via any UE-specific configuration or group-common signaling.

RAN1#117 (2024.05) [6] Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

RAN1#117 (2024.08) [8] Agreement For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot

In this section, following topics regarding semi-static indication of time location of SBFD subbands are discussed.

- Details of indication of SBFD symbol location within a TDD-UL-DL pattern

- Transition time from non-SBFD symbol to non-SBFD symbols

Details of indication of SBFD symbol location within a TDD-UL-DL pattern

In RAN1#118 meeting, it was agreed that for configuration of SBFD symbols within a TDD-UL-DL pattern period, the parameters (i.e., a starting slot index, a starting symbol index within the starting slot, an ending slot index, an ending symbol index within ending slot) are supported. The remaining issue is to determine the reference slot/symbol for indicating the starting/ending slot/symbol of the SBFD symbols.

SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. In this case, SBFD symbols should be indicated within symbols designated as DL or Flexible configured by TDD-UL-DL-ConfigCommon. To prevent the indication of UL symbols indicated by TDD-UL-DL-ConfigCommon as SBFD symbols, it is necessary to discuss methods for ensuring that the network correctly designates SBFD symbols within DL or Flexible symbols configured by TDD-UL-DL-ConfigCommon. As a method for this, it can be expected that the network will configure the SBFD symbols not to be located in UL symbols according to the TDD-UL-DL-ConfigCommon. Alternatively, a more proactive approach would be to define SBFD symbol indication methods to prevent UL symbols configured by TDD-UL-DL-ConfigCommon from being designated as SBFD symbols, thereby fundamentally blocking the possibility of such symbols being indicated as SBFD symbols.

Proposal 1: Determine the configuration method of SBFD symbol location in a way that prevents UL symbols configured by TDD-UL-DL-ConfigCommon from being designated as SBFD symbols.

Considering above aspects to prevent UL symbols within TDD-UL-DL pattern configured by TDD-UL-DL-ConfigCommon, it is proposed to determine the reference slot/symbol position for indicating the staring slot/symbol position of SBFD symbols and the ending slot/symbol position of SBFD symbol as follows:

- For the starting slot/symbol position of SBFD symbols, the reference position for indicating the starting symbol of the SBFD symbols is the slot of the TDD-UL-DL pattern period. If the position of the starting slot/symbol position of the SBFD symbols are not configured, the default position of the starting symbol is the same as the first symbol within the TDD-UL-DL pattern period.

- For the ending slot/symbol position of SBFD symbols, the reference position for indicating the ending symbol of the SBFD symbols is the slot where the last symbol among the DL or Flexible symbols configured by TDD-UL-DL-ConfigCommon is contained. If the position of the ending slot/symbol of the SBFD symbols are not configured, the default position of the last symbol is the same as the position of the last symbol among the DL and/or Flexible symbols indicated by TDD-UL-DL-ConfigCommon.

By using this method, there is an advantage in preventing the assignment of UL symbols by TDD-UL-DL-ConfigCommon as SBFD symbols.

Proposal 2: For the starting slot/symbol and/or the ending slot/symbol of SBFD symbols within TDD-UL-DL pattern period,

- The reference position for indicating the starting slot of the SBFD symbols is the starting slot of the TDD-UL-DL pattern period. If the starting slot/symbol of SBFD symbols are not configured, the default position of the starting slot/symbol is the same as the first symbol within the TDD-UL-DL pattern period.

- The reference position for indicating the ending slot of the SBFD symbols is the slot where the last symbol among the DL or Flexible symbols configured by TDD-UL-DL-ConfigCommon is contained. If the ending slot/symbol of SBFD symbols are not configured, the default position of the ending slot/ symbol is the same as the position of the last symbol among the DL and/or Flexible symbols indicated by TDD-UL-DL-ConfigCommon.

Transition time from non-SBFD symbol to non-SBFD symbols

For transition from non-SBFD symbols to SBFD symbols and at gNB side, a transition time (e.g., 10us for FR1, 3us for FR2) is required. The length of the transition time from non-SBFD symbol to SBFD symbol can be decided depending on the implementation of gNB. On the hands, for transition from SBFD symbol to non-SBFD symbol, a transition time is not required.

During the transition from non-SBFD symbol to SBFD symbol, normal transmission and reception operations may not be possible at least at the gNB side. Therefore, it is desirable for the UE to refrain from performing any transmission or reception operations during the interval. Considering these aspects, it will be necessary to decide whether to define the position of the guard period and its behavior from the UE perspective or to resolve these issues through network scheduling without specification impact.

(a) Case 1: From DL in non-SBFD symbol to SBFD symbol

(b) Case 2: From UL in non-SBFD symbol to SBFD symbol

Figure 2. Example of transition time from non-SBFD symbol to SBFD symbol

Proposal 3: Explicitly configure a guard period for the transition from non-SBFD to SBFD symbol and the related behavior of an SBFD-aware UE.

- 2.2	Semi-static indication of frequency location of SBFD subbands

For frequency location indication of SBFD subband and DL/UL usable PRB determination, the following agreements and a working assumption were made in the previous meetings [2][4][6][8][10][12].

- Frequency location indication of SBFD subband

RAN1#116 (2024.02) [2] Agreement For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands Agreement: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. The UL subband can be located at one side of the carrier or can be located at the middle part of the carrier. For semi-static indication of SBFD subband frequency location, down-select from the following options. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). - Option 2: Frequency location of UL subband and the number of RBs for guardband(s), if any, are explicitly configured. DL subband(s) are implicitly derived as RBs which are not within UL subband or guardband(s). Agreement The subband frequency-domain resources are same across different SBFD symbols within a TDD carrier. Frequency location of cell specific UL subband, and DL subband(s) if explicitly indicated, are indicated with reference to CRB grid. - RB-level granularity is supported for semi-static indication of SBFD subband frequency location. - Subject to RAN4 guidance on the size of subband/guardband, if any - FFS reference starting RB and reference SCS

RAN1#116bis (2024.04) [4] Agreement: For cell-specific configuration of frequency locations of SBFD subbands, - Option 1: Cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. - For each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

RAN1#117 (2024.05) [6] Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#117 (2024.08) [8] Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

RAN1#119 (2024.11) [12] Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

- DL/UL usable PRB determination

RAN1#116 (2024.02) [2] Agreement For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

RAN1#118bis (2024.10) [10] Agreement If UE-specific configuration on frequency locations of SBFD subbands is not supported nor provided, support Option 1 for determining UL/DL usable PRBs. - Option 1: For a UL BWP, UL usable PRBs are determined as intersection between cell-specific UL subband and the UL BWP in SBFD symbols. For a DL BWP, DL usable PRBs are determined as intersection between cell-specific DL subband(s) and the DL BWP in SBFD symbols.

- PRB size for PDSCH/PUSCH

RAN1#118bis (2024.10) [10] Conclusion Nominal RBG size for PDSCH/PUSCH in SBFD symbols is determined based on the size of DL/UL BWP as legacy. Nominal CSI reporting subband size in SBFD symbols is determined based on the size of DL BWP as legacy.

In this section, following topics regarding semi-static indication of time location of SBFD subbands are discussed.

- UE specific SBFD frequency location Indication

UE specific SBFD frequency location Indication

In RAN1#116 meeting, it was agreed that for RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier. One remaining issue point is whether/how to additionally support of UE-specific configuration on time and/or frequency location of SBFD subbands. In this section, UE specific configuration on frequency location of SBFD subband is discussed.

The main motivation of supporting UE specific configuration on frequency location of SBFD subband on top of cell specific configuration on frequency location of SBFD subbands is to support different guardbands due to different UE capabilities and/or UE-to-UE CLI mitigation.

In macro cell deployment scenario, UE-to-UE CLI from aggressor UE with higher transmission power could affect to DL performance degradation if DL frequency resources adjacent UL subband are assigned. If severe CLI level is reported, gNB may indicate additional UE specific configuration on frequency location to reduce DL usable PRB. If UE determines the size of DL usable PRB according to the additional UE specific configuration, UE may apply the size of DL usable PRB depending on the type of DL signal/channels (e.g., CSI-RS, TRS) within a BWP.

Proposal 4:  Support additional UE specific configuration on frequency location of SBFD subband.

SBFD Tx/Rx/Measurement procedure

- 3.1	Transmission and reception behaviors on SBFD subbands

The following agreement regarding Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot was made in the previous meeting [12].

RAN1#119 (2024.11) [12] Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.

In this section, further details of Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot is discussed.

Tx/Rx occasion mapped to SBFD and non-SBFD symbols within a slot

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, considering that it is hard to assume that phase continuity is maintained between SBFD and non-SBFD symbols, it is desirable that the symbols for transmitting a DL/UL signal/channel should be either all SBFD symbols or all non-SBFD symbols within a slot.

In RAN1#119 meeting, it was agreed that for a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot. Two remaining issues need to be discussed.

- Determination of valid slots for PUSCH repetition type A with available slot based counting and PUCCH repetition

- Selection of one option for PUSCH repetition type B

For PUSCH with repetition type A based on available slot counting and for PUCCH repetition, if a physical channel occasion mapped to SBFD and non-SBFD symbols within a slot, it seems reasonable that the UE postpones transmissions in the slot.

For PUSCH with repetition type B, considering it is used for latency purpose, it is desirable to use as many symbols as possible for PUSCH transmission. In this case, if the symbol resources allocated for a specific nominal repetition contain both SBFD symbols and non-SBFD symbols, it can be considered that the UE divides the nominal repetition into actual repetitions based on the boundary between SBFD symbol and non-SBFD symbol.

- In case Configuration 1, OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition.

- In case Configuration 2, both OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition.

Proposal 5: For PUSCH with repetition type A based on available slot counting and for PUCCH repetition, if a physical channel occasion mapped to SBFD and non-SBFD symbols within a slot, the UE postpones transmissions in the slot.

Proposal 6: For PUSCH repetition type B, if a particular nominal repetition contains SBFD and non-SBFD symbols, divide the actual repetition based on the boundary between SBFD symbol and non-SBFD symbol.

- In case Configuration 1, OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition. Support option 2 (i.e., A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.)

- In case Configuration 2, both OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition. Support option 1 (i.e., A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.)

- 3.2	Resource allocation in frequency domain in SBFD symbols

Regarding the Resource allocation in frequency domain in SBFD symbols, the following was agreed in the previous meetings.[4][6][8][10][12]

- Frequency domain resource allocation Type 0 for PDSCH or PUSCH

RAN1#116bis (2024.04) [4] Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

RAN1#117 (2024.05) [6] Agreement: For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

RAN1#118bis (2024.10) [10] Agreement: For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SPS PDSCH, CG PUSCH, PDSCH/PUSCH repetition, multi-PDSCH/PUSCH scheduled by a single DCI and TBoMS is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs. - FFS: The above applies to SP-CSI on PUSCH when applicable

RAN1#119 (2024.11) [12] Agreement For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

- Frequency domain resource allocation Type 1 for PDSCH

RAN1#116bis (2024.04) [4] Agreement: For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, discuss and decide whether/which of the following options is supported. - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - Option 1-2: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs as legacy - FFS: DMRS sequence mapping - Option 2: Introduce new RB indexing/PRB bundle indexing to ensure VRBs are mapped to DL usable PRBs only. - Existing VRB-to-PRB mapping is reused - Legacy TBS determination method is used - FFS: DMRS sequence mapping - Option 3: Modify VRB-to-PRB mapping interleaver to ensure VRBs are mapped to DL usable PRBs only. - Existing RB indexing/PRB bundle indexing is reused - If the interleaver is not enabled, Option 1-1 or Option 1-2 is used - Legacy TBS determination method is used - FFS: DMRS sequence mapping

RAN1#117 (2024.05) [6] Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG

RAN1#118 (2024.08) [8] Conclusion If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

RAN1#118bis (2024.10) [10] Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols. Agreement For a PDSCH/PUSCH with assigned PRBs overlapping with SBFD subband boundary and only the PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH reception/transmission, legacy DMRS sequence mapping is applied to assigned PRBs within DL/UL usable PRBs only (effectively, this is same as the case when the DMRS sequence mapped to the RBs outside the DL/UL usable PRBs are punctured).

RAN1#118bis (2024.10) [10] Agreement: For configuration 1 and frequency resource allocation Type 1, if the valid symbol type for SPS PDSCH, PDSCH repetition and multi-PDSCH scheduled by a single DCI is SBFD symbols, then - Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. - Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused. - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only.

- CSI-RS across downlink subbands

RAN1#117 (2024.05) [6] Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

RAN1#118bis (2024.10) [10] Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

- CSI reporting subband

RAN1#117 (2024.05) [6] Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only.

In this section, following topics regarding enhancements related to frequency domain resource allocation in SBFD symbols are discussed.

- Frequency domain resource allocation Type 1 for PUSCH

- Resource allocation in frequency domain for CSI-RS across two DL subbands in SBFD symbol

Frequency domain resource allocation Type 1 for PUSCH

So far, agreements have been reached on RA type 0 for PDSCH and PUSCH, as well as RA type 1 for PDSCH. However, there is no agreement yet on RA type 1 for PUSCH. For PUSCH, it should suffice to indicate the frequency resources within the bandwidth of the UL subband. For RA type 1 for PUSCH, the case where the RBG overlaps with the subband boundary should be addressed. Therefore, it seems reasonable to apply the approach used for RA type 0 for PUSCH to RA type 1 for PUSCH as well.

Proposal 7: For frequency resource allocation Type 1 for PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary,

- Only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

- The number of PRBs for TBS determination is based on the assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Resource allocation in frequency domain for CSI-RS across two DL subbands in SBFD symbol

In RAN1#117bis meeting, for resource allocation in frequency domain for CSI-RS across two DL subbands in SBFD symbol, supporting one contiguous CSI-RS resource allocation with non-contiguous CSI-RS derived by excluding frequency resources outside DL usable PRBs was agreed as working assumption. Also, for contiguous CSI-RS resource which overlaps with SBFD subband boundaries, it was agreed that only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware UE.

In the working assumption and agreement, ‘CSI-RS’ is mentioned in general. But, in NR system, the various type of CSI-RS is defined depending on the purpose. That is, the working assumption and agreement can be applied to the CSI-RS used for NZP-CSI-RS, CSI-IM and CSI-RS for tracking. For CSI-RS for mobility, it needs to be clarified whether the working assumption and agreement are applicable.

Proposal 8: The working assumption and agreement are applied to the CSI-RS used for NZP-CSI-RS, CSI-IM and CSI-RS for tracking. For CSI-RS for mobility, discuss to clarify whether the working assumption and agreement are applicable.

- 3.3	Physical channels/signals and procedure across SBFD and non-SBFD symbols

Regarding the Physical channels/signals and procedure across SBFD and non-SBFD symbols, the following was agreed in the previous meetings. [4][6][8][10][12]

- An SPS PDSCH configuration without repetitions

RAN1#116bis (2024.04) [4] Agreement: For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid - Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH - Other options are not precluded Agreement For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped - Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid - Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs - Other options are not precluded - FFS: Applicable conditions Agreement: For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols - FFS type 2 CG PUSCH - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid - Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH - Other options are not precluded Agreement: For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed - Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid - Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs - Other options are not precluded - FFS: Applicable conditions

- Configuration 1, Configuration 2

RAN1#117 (2024.05) [6] Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

RAN1#118bis (2024.10) [10] Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with Configuration 1 or Configuration 2 via RRC configuration. Select from the following options - Option 1: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per serving cell basis - Option 2: SBFD-aware UE is configured with Configuration 1 or Configuration 2 on a per UE per BWP per channel/signal basis - Separate configurations for configured and dynamic transmission/receptions for a given channel/signal are not precluded. - Option 3: Configuration 1 or Configuration 2 is on a per cell basis. - No separate UE capabilities for Configuration 1 and 2.

RAN1#119 (2024.11) [12] Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

- The valid symbol type (Configuration1)

RAN1#118 (2024.08) [8] Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. - For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC. - It is not applicable to PDCCH - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. - For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options: - Option 1: - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig. - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig. - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config. - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource]. - Option 2: - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation. - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

RAN1#119 (2024.11) [12] Agreement: For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - For type 2 CG PUSCH, SPS PDSCH, down-select from the following options: - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

RAN1#119 (2024.11) [12] Agreement: It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted>

- With/without available slot counting (Configuration1)

RAN1#118 (2024.08) [8] Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1, - For PUSCH repetition type A with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

RAN1#119 (2024.11) [12] Agreement Update the following agreement made in RAN1#118 meeting: Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) with Configuration 1, - For PUSCH repetition type A with available slot counting, A-SRS with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH with neither TBoMS nor PUSCH repetition type A with available slot counting,and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

- An SPS PDSCH configuration, PDSCH repetitions, multi-PDSCH scheduled by a single DCI (Configuration2)

RAN1#118 (2024.08) [8] Working Assumption For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

RAN1#118bis (2024.10) [10] Agreement - For multi-PDSCH scheduled by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs.

RAN1#119 (2024.11) [12] Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

- CG PUSCH configuration, PUSCH repetition type-A, multi-PUSCH scheduled by a single DCI, TBoMS (Configuration2), and frequency hopping

RAN1#117 (2024.05) [6] Agreement Support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively. - FFS: How to indicate/determine the FH offsets for PUSCH transmissions in SBFD and non-SBFD symbols, respectively. - FFS: Whether/how to update FH calculation to only consider the UL usable PRBs

RAN1#118 (2024.08) [8] Agreement Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2. - Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols. Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH. - Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols. - Support separate configurations for DCI format 0_2 and other DCI formats as legacy - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions. FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

RAN1#118 (2024.08) [8] Agreement For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols

RAN1#118bis (2024.10) [10] Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, - The number of PRBs for PUSCH transmissions in SBFD and non-SBFD symbols is determined as legacy. - The PRBs for PUSCH transmissions in non-SBFD symbols are determined as legacy. - Consider the following options for determining starting PRB for PUSCH transmissions in SBFD symbols: - Option 1: One or multiple RBoffsetSBFD are configured and the starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 1-A: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFD - Negative values of RBoffsetSBFD are not precluded. - Equation 1-B: RBstartSBFD=RBstartnon-SBFD+RBoffsetSBFDmodNBWPsize - Equation 1-C: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - Equation 1-D: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD-RBstartUL SB+RBoffsetSBFDmodNUL SBsize - Option 2: The starting PRB for PUSCH in SBFD symbol is determined according to one of the following equation: - Equation 2-A: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize -NPUSCHsize) - Equation 2-B: RBstartSBFD = RBstartUL SB+mod(RBstartnon-SBFD,NUL SBsize ) - Equation 2-C: RBstartSBFD = round(RBstartnon-SBFD* NUL SBsizeNBWPsize) + RBstartUL SB - Equation 2-D: RBstartSBFD = RBstartUL SB+RBstartnon-SBFD - The variables are defined as follows: - RBstartSBFD is the starting PRB index of PUSCH in SBFD symbol with reference to the start of UL active BWP - RBstartnon-SBFD is the starting PRB index of PUSCH in non-SBFD symbol with reference to the start of UL active BWP - RBstartUL SB is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP - NBWPsize is the number of PRBs of UL BWP - NUL SBsize is the number of PRBs of UL usable PRBs - NPUSCHsize is the number of PRBs for PUSCH transmissions - No additional configuration/indication to enable/disable whether to apply RBSBFDoffset in SBFD symbols. - FFS: Condition to enable/disable - UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols after applying RBSBFDoffset to be overlapped with PRBs outside UL usable PRBs. - It applies at least to RA type 1. FFS for RA type 0. - Note: Other equations are not precluded.

RAN1#118bis (2024.10) [10] Agreement The number of configured FH offsets of FH offset list for SBFD symbols is the same as the number of FH offsets of FH offset list for non-SBFD symbols. - The number of configured FH offsets of FH offset list is determined based on the UL BWP size as legacy. The number of bits used to indicate FH offset in DCI is determined as legacy. Agreement For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

RAN1#119 (2024.11) [12] Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero

RAN1#119 (2024.11) [12] Agreement Update the following agreement made in RAN1#118bis meeting in cyan: Agreement: For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Decide in RAN1#119, one of the following options: - Alt1: Remove -RBUL SB start in the above equations - Alt2: Remove square brackets from [-RBUL SB start] in the above equations

- CSI report associated with periodic/semi-persistent CSI-RS

RAN1#117 (2024.05) [6] Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

RAN1#117 (2024.10) [10] Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation.

In this section, following topics regarding physical channel/signals and procedure across SBFD and non-SBFD symbols are discussed.

- Configuration 1, Configuration 2

- The valid symbol type (Configuration1)

- An SPS PDSCH configuration, PDSCH repetitions (Configuration2)

- CG PUSCH configuration, PUSCH repetition type-A, multi-PUSCH scheduled by a single DCI, TBoMS (Configuration2) and frequency hopping

- CSI report for SBFD and non-SBFD symbols

- Joint channel estimation across SBFD and non-SBFD symbols

An SPS PDSCH configuration, PDSCH repetitions, multi-PDSCH scheduled by a single DCI (Configuration2)

In RAN1#118 meeting, for An SPS PDSCH configuration, PDSCH repetitions, multi-PDSCH scheduled by a single DCI (Configuration2), it was agreed that only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. In addition, in RAN1#118bis meeting, it was agreed that for multi-PDSCH scheduling by a single DCI with Configuration 2, the number of PRBs for TBS determination for a PDSCH in SBFD symbols is based on assigned PRBs within DL usable PRBs only. The number of PRBs for TBS determination for a PDSCH in non-SBFD symbols is based on assigned PRBs. In RAN1#119, for TBS determination for dynamically scheduled PDSCH repetitions with Configuration 2, it was agreed that if the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs, and if the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

The remain issue is the number of PRBs for TBS determination for SPS PDSCH configuration.

Consideration point is that the number of assigned PRB in SBFD symbol and non-SBFD symbols is not same. If the TBS is determined based on assigned PRBs in non-SBFD symbol, the coding rate of the symbol is increased in SBFD symbols due to the reduced PRBs. If the TBS is determined based on assigned PRBs within DL usable PRBs in SBFD symbol, the coding rate is decreased in non-SBFD symbols. It appears preferable to determine the TBS based on the symbol type that has more opportunities for DL reception. In cases where SBFD is operational, it is expected that the DL reception opportunities in SBFD symbols will be greater than those in non-SBFD symbols. Therefore, TBS determination should ideally be based on the SBFD symbols.

Proposal 9: In case of configuration 2, for an SPS PDSCH configuration, support the number of PRBs for TBS determination is based assigned PRBs within DL usable PRBs only.

Joint channel estimation across SBFD and non-SBFD symbols

In order to enhance the coverage of PUSCH and PUCCH in Rel-17, joint channel estimation using DMRS in multiple slots has been introduced.

When PUSCH/PUCCH are repeated and transmitted across SBFD symbols and non-SBFD symbols in different slots, there is a possibility that both SBFD symbols and non-SBFD symbols can be included within the time resources for joint channel estimation. In such cases, it may be difficult for the UE to maintain power consistency and phase continuity for PUSCH/PUCCH transmission across these two resources.

Considering this, it would be necessary to include the boundary between SBFD symbols and non-SBFD symbols in the semi-static event used for determining the actual TDW. Therefore, the actual TDW could be divided at the boundary between SBFD symbols and non-SBFD symbols. This would allow the UE to potentially not maintain power consistency and phase continuity across SBFD symbols and non-SBFD symbols.

Proposal 10: To support joint channel estimation when PUSCH/PUCCH repetition is transmitted across SBFD symbols and non-SBFD symbols in different slots, the boundary between SBFD symbols and non-SBFD symbols is included in the semi-static event used for determining the actual TDW.

CSI report for SBFD and non-SBFD symbols

In RAN1#117 meeting, it was agreed that for CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the two options:

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Two options are applicable for separate CSI report on SBFD symbol and non-SBFD symbol, above two options are applicable for SBFD operation. But, depending on the UE capability, one option can be operated depending on UE capability. Also, for each option, one type of CSI report (i.e., CSI report on SBFD or CSI report on non-SBFD) can be configured depending on UE capability.

Proposal 11: Two options are applicable for separate CSI report on SBFD symbol and non-SBFD symbol.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

In Option A, where CSI-ReportConfig is independently configured for measuring and reporting SBFD symbols and non-SBFD symbols, the CSI reporting subband can also be configured independently. However, in Option B, where the same CSI-ReportConfig is used for instructing the measurement and report of SBFD symbols and non-SBFD symbols, the same CSI reporting subband configuration applies to the measurement and report of both types of symbols.

For example, to perform measurements for SBFD symbols, the UE can consider only the CSI reporting subbands located within the DL usable PRBs as valid. Alternatively, all CSI reporting subbands could be considered valid subbands regardless of their inclusion in DL usable PRBs, and a specific measurement reporting method could be defined for subbands located outside of the DL usable PRBs.

Proposal 12: When measurement and report of SBFD symbols and non-SBFD symbols are configured by the same CSI-ReportConfig (Option B), discuss the issue of CSI reporting subbands in SBFD symbols being located outside of DL usable PRBs.

3.4	Configuration for SRS, PUCCH and PUSCH on SBFD and non-SBFD symbols

For the discussion of configuration for SRS, PUCCH and PUSCH on SBFD and non-SBFD symbols, followings were agreed in the previous meetings. [4][6][8][10][12]

- Power control, Spatial relation

RAN1#116bis (2024.04) [4] Agreement Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

RAN1#117 (2024.05) [6] Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

RAN1#118 (2024.08) [8] Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters. - Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively - FFS: No RRC impact - FFS: New MAC CE or reuse existing MAC CE - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively - FFS: Impact on MAC CE - Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS - FFS: Same offset or different offsets for all channels - FFS: RRC or MAC CE introduced to convey power offset - FFS whether the maximum number of closed power control loops is increased to be more than 2. - FFS: Whether/How to handle the case where UL power control is not associated within TCI state

RAN1#119 (2024.11) [12] Conclusion There is no RAN1 consensus to support mTRP scenario for SBFD in Rel-19

RAN1#119 (2024.11) [12] Agreement The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19. Agreement: For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

- SRS

RAN1#118bis (2024.10) [10] Agreement Support separate SRS configurations for SBFD and non-SBFD symbols. Select one from the following options. - Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS applicable usages - Option 2: Support separate configurations for SBFD and non-SBFD symbols in the same SRS-Resource. - At least freqDomainPosition, freqDomainShift and freqHopping are separately configured. - FFS separate configurations of other parameters - FFS applicable usages

RAN1#119 (2024.11) [12] Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

- Frequency Configuration for PUCCH

RAN1#118 (2024.08) [8] Agreement Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

In this section, following topics regarding configuration for PUCCH and PUSCH on SBFD and non-SBFD symbols are discussed:

- Separate resource configuration for PUCCH

- UCI multiplexing parameters

Frequency Resource Configuration for PUCCH

In RAN1#118 meeting, it was agreed to support separate frequency configuration for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource, and to introduce new RRC parameters of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols.

The one of remaining issues is how to configure the PRI for SBFD symbol and non-SBFD symbol. Instead of independently indicating PRI for SBFD symbol and non-SBFD symbol, a single PRI in DCI can be used. The starting PRB and second hop PRB for the PRI indicated by DCI can then be applied with values corresponding to the symbol type, i.e., non-SBFD symbols and SBFD symbols, respectively.

Proposal 13: A single PRI in DCI can be used to indicate PRI for SBFD symbol and non-SBFD symbol. The starting PRB and second hop PRB for the PRI indicated by DCI can then be applied with values corresponding to the symbol type, i.e., non-SBFD symbols and SBFD symbols, respectively.

The other remaining issue is a UE behaviour when no separate configuration is provided for SBFD symbols. The simple and clear method is that PUCCH transmissions in SBFD symbols or non-SBFD symbols for this pucch-ResourceId is not expected.

For example, in case of configuration 1, UE may transmit PUCCH in SBFD symbols or non-SBFD symbols. For this case, when only RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbol needs to be configured, UE can assume the PUCCH-Resource is applicable for SBFD symbol only. In the same way, when only RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for non-SBFD symbol needs to be configured, UE can assume the PUCCH-Resource is applicable for non-SBFD symbol only.

For example, in case of configuration 2, UE may transmit PUCCH in both SBFD symbol and non-SBFD symbols. When only RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for non-SBFD symbol needs to be configured, UE can assume the PUCCH-Resource is applicable for non-SBFD symbol only. For PUCCH repetition, the UE may postpone transmissions of PUCCH in SBFD symbol.

Proposal 14: When no separate configuration of starting PRB and second hop PRB for SBFD symbol is provided, PUCCH transmissions for this pucch-ResourceId is not expected in SBFD symbols.

UCI multiplexing parameters

In SBFD symbols, the channel environment may not be as favorable compared to non-SBFD symbols due to cross-link interference (CLI) and residual self-interference (SI). Therefore, if UCI multiplexing on PUSCH is performed in SBFD symbols based on the channel environment in non-SBFD symbols, the UCI may not be transmitted properly.

Considering this, it could be beneficial to adjust the amount of resources used for UCI transmission when SBFD-aware UE multiplexes UCI on PUSCH in SBFD symbols, allowing for more resources to be allocated for UCI transmission compared to when it is transmitted in non-SBFD symbols.

For this purpose, the values of betaOffsets used to determine the number of transmission REs for UCI could be independently configured for SBFD symbols and non-SBFD symbols.

Proposal 15: For UCI multiplexing on PUSCH, separate configuration of betaOffsets is supported for SBFD and non-SBFD symbols.

3.5	Collision handling between DL reception in DL subband(s) and UL transmission in UL subband

For the discussion of collision handling between DL reception in DL subband(s) and UL transmission in UL subband, followings were agreed in the previous meetings. [2][4][6][8][10][12]

- Link Direction in SBFD symbol

RAN1#116 (2024.02) [2] Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

RAN1#119 (2024.11) [12] Agreement For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols - Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

- Collision cases (1,2,3,4,6)

RAN1#116 (2024.02) [2] Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

RAN1#116bis (2024.04) [4] Agreement: If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition Agreement: If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

RAN1#117 (2024.05) [6] Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions

RAN1#118 (2024.08) [8] Agreement For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

- Collision Case 5

RAN1#117 (2024.05) [6] Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

RAN1#118bis (2024.10) [10] Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

In this section, following topics regarding collision handling between DL reception in DL subband(s) and UL transmission in UL subband are discussed.

- Link direction determination of SBFD aware UE

- Collision handling for case 5

Link direction determination of SBFD aware UE

In the existing TDD system, a UE can perform DL reception and UL transmission in symbols determined as DL and UL, respectively. However, in symbols determined as flexible, it can perform either DL reception or UL transmission depending on the gNB scheduling.

On the other hand, for SBFD-aware UEs, if a symbol indicated as DL by TDD-UL-DL-ConfigCommon is configured as an SBFD symbol, it should be able to perform DL reception or UL transmission in such a symbol. To support this behavior, a SBFD-aware UEs needs to treat the DL symbol indicated by TDD-UL-DL-ConfigCommon as a flexible symbol if it is configured as an SBFD symbol.

Proposal 16: If a DL symbol by TDD-UL-DL-ConfigCommon is configured as a SBFD symbol, the SBFD-aware UE treats it as a flexible symbol by TDD-UL-DL-ConfigCommon.

In RAN1#116 meeting, for SBFD-aware UE transmission and reception in an SBFD symbol, following two options to determine link direction were identified [2].

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

In the case where only Option 1 is applied for the determination of the link direction in the SBFD symbol of the SBFD-aware UE, if TDD-UL-DL-ConfigDedicated or SFI is indicated, this information should be used to indicate DL or UL for flexible symbols that are non-SBFD symbols, but not for SBFD symbols.

Proposal 17: If it is agreed that only Option 1 (i.e., SBFD-aware UE determines link direction in a SBFD symbol based on configured/scheduled transmissions/receptions and collision handling) is supported in Rel-19, it can conclude that the legacy TDD-UL-DL-ConfigDedicated and/or SFI are not applicable for the SBFD symbol.

Also, in RAN1#119 meeting, it was agreed that for a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols [12].

When Option 2 is applied, no new signal to indicate the link direction is required, and the legacy configuration/indicator (i.e., the legacy TDD-UL-DL-ConfigDedicated) can be used to indicate the link direction of the SBFD-aware UE in the SBFD symbols. Because the legacy TDD-UL-DL-ConfigDedicated can be configured as UE specific manner and the configuration(s) are applied separately for legacy UE and SBFD-aware UE, legacy UE(s) may not impact from the configuration for SBFD-aware UE.

For SBFD-aware UE, the legacy TDD-UL-DL-ConfigDedicated can be used in non-SBFD symbols to indicate whether the UE performs DL reception or UL transmission on the entire frequency resource, and the legacy TDD-UL-DL-ConfigDedicated is applicable for SBFD symbols to indicate whether the UE performs DL reception on DL usable PRBs or UL transmission on UL usable PRBs. Also, Option 1 may be applied for SBFD symbols where the link direction is not specified as DL or UL by TDD-UL-DL-ConfigDedicated and SFI information.

Proposal 18: If it is agreed that both Option 1 (i.e., SBFD-aware UE determines link direction in a SBFD symbol based on configured/scheduled transmissions/receptions and collision handling) and Option 2 (i.e., link direction of SBFD-aware UE in SBFD symbol is explicitly indicated by gNB) are supported in Rel-19, for the SBFD-aware UE, the legacy TDD-UL-DL-ConfigDedicated can be used to indicate the link direction in the SBFD symbol. Also, for the SBFD-aware UE, the legacy TDD-UL-DL-ConfigDedicated can be used in non-SBFD symbols to indicate whether the UE performs DL reception or UL transmission on the entire frequency resource.

- For a SBFD symbol, if it is directed to DL by TDD-UL-DL-ConfigDedicated, the UE can perform DL reception within DL usable PRB resources in the symbol.

- For a SBFD symbol, if it is directed to UL by TDD-UL-DL-ConfigDedicated, the UE can perform UL transmission within UL usable PRB resources in the symbol.

- If a SBFD symbol is not directed to DL/UL by TDD-UL-DL-ConfigDedicated, link direction in the SBFD symbol is determined based on configured/scheduled transmissions/receptions and collision handling.

Collision handling for case 5

In RAN1#118bis meeting, it was agreed as working assumption that for SSB symbols configured with SBFD subbands, the SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. That is, the collision between SSB symbols and UL signal/channel can be avoided by this agreement.

On top of the agreement, the case a UE can transmit UL signal/channel before and after reception of SSB symbols should be discussed. In this case, Rx-Tx switching time and Tx-Rx switching time between UL signal/channel and SSB symbols should be handled to avoid the collision between UL transmission and UL/DL switching time before and after reception of SSB symbols. For an SBFD-aware UE, following specified for a HD-UE can be adopted.

If a HD-UE would transmit a PUSCH, or PUCCH, or SRS based on a configuration by higher layers and the HD-UE is indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon or by NonCellDefiningSSB, the HD-UE does not transmit - PUSCH or PUCCH if a last symbol of the PUSCH or PUCCH transmission would not be at least NTx-Rx⋅Tc [4, TS 38.211] prior to a first symbol of the next earliest SS/PBCH block - PUSCH or PUCCH if a first symbol of the PUSCH or PUCCH transmission would not be at least NRx-Tx⋅Tc [4, TS 38.211] after a last symbol of the previous latest SS/PBCH block - SRS in symbols that would not be at least NTx-Rx⋅Tc prior to a first symbol of the next earliest SS/PBCH block - SRS in symbols that would not be at least NRx-Tx⋅Tc after a last symbol of the previous latest SS/PBCH block

Proposal 19: To avoid the collision between UL transmission and UL/DL switching time before and after reception of SSB symbols, specify a UE behavior considering on Rx-Tx switching time and Tx-Rx switching time between UL signal/channel and SSB symbols

Summary

In this contribution, we discussed on the subband non-overlapping full duplex. Based on the discussion, we obtained following proposals.

1. Time/Frequency location indication of SBFD subbands

- 1.1) Semi-static indication of time location of SBFD subbands

Proposal 1: Determine the configuration method of SBFD symbol location in a way that prevents UL symbols configured by TDD-UL-DL-ConfigCommon from being designated as SBFD symbols.

Proposal 2: For the starting slot/symbol and/or the ending slot/symbol of SBFD symbols within TDD-UL-DL pattern period,

- The reference position for indicating the starting slot of the SBFD symbols is the starting slot of the TDD-UL-DL pattern period. If the starting slot/symbol of SBFD symbols are not configured, the default position of the starting slot/symbol is the same as the first symbol within the TDD-UL-DL pattern period.

- The reference position for indicating the ending slot of the SBFD symbols is the slot where the last symbol among the DL or Flexible symbols configured by TDD-UL-DL-ConfigCommon is contained. If the ending slot/symbol of SBFD symbols are not configured, the default position of the ending slot/ symbol is the same as the position of the last symbol among the DL and/or Flexible symbols indicated by TDD-UL-DL-ConfigCommon.

Proposal 3: Explicitly configure a guard period for the transition from non-SBFD to SBFD symbol and the related behavior of an SBFD-aware UE.

- 1.2) Semi-static indication of frequency location of SBFD subbands

Proposal 4:  Support additional UE specific configuration on frequency location of SBFD subband.

2. SBFD Tx/Rx/Measurement procedure

- 2.1) Transmission and reception behaviors on SBFD subbands

Proposal 5: For PUSCH with repetition type A based on available slot counting and for PUCCH repetition, if a physical channel occasion mapped to SBFD and non-SBFD symbols within a slot, the UE postpones transmissions in the slot.

Proposal 6: For PUSCH repetition type B, if a particular nominal repetition contains SBFD and non-SBFD symbols, divide the actual repetition based on the boundary between SBFD symbol and non-SBFD symbol.

- In case Configuration 1, OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition. Support option 2 (i.e., A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped.)

- In case Configuration 2, both OFDM symbols in SBFD symbols only or OFDM symbols in non-SBFD symbols can be determined as valid for actual repetition. Support option 1 (i.e., A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.)

- 2.2) Resource allocation in frequency domain in SBFD symbols

Proposal 7: For frequency resource allocation Type 1 for PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary,

- Only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

- The number of PRBs for TBS determination is based on the assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Proposal 8: The working assumption and agreement are applied to the CSI-RS used for NZP-CSI-RS, CSI-IM and CSI-RS for tracking. For CSI-RS for mobility, discuss to clarify whether the working assumption and agreement are applicable.

- 2.3) Physical channels/signals and procedure across SBFD and non-SBFD symbols

Proposal 9: In case of configuration 2, for an SPS PDSCH configuration, support the number of PRBs for TBS determination is based assigned PRBs within DL usable PRBs only.

Proposal 10: To support joint channel estimation when PUSCH/PUCCH repetition is transmitted across SBFD symbols and non-SBFD symbols in different slots, the boundary between SBFD symbols and non-SBFD symbols is included in the semi-static event used for determining the actual TDW.

Proposal 11: Two options are applicable for separate CSI report on SBFD symbol and non-SBFD symbol.

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Proposal 12: When measurement and report of SBFD symbols and non-SBFD symbols are configured by the same CSI-ReportConfig (Option B), discuss the issue of CSI reporting subbands in SBFD symbols being located outside of DL usable PRBs.

2.4) Configuration for SRS, PUCCH and PUSCH on SBFD and non-SBFD symbols

Proposal 13: A single PRI in DCI can be used to indicate PRI for SBFD symbol and non-SBFD symbol. The starting PRB and second hop PRB for the PRI indicated by DCI can then be applied with values corresponding to the symbol type, i.e., non-SBFD symbols and SBFD symbols, respectively.

Proposal 14: When no separate configuration of starting PRB and second hop PRB for SBFD symbol is provided, PUCCH transmissions for this pucch-ResourceId is not expected in SBFD symbols.

Proposal 15: For UCI multiplexing on PUSCH, separate configuration of betaOffsets is supported for SBFD and non-SBFD symbols.

2.5) Collision handling between DL reception in DL subband(s) and UL transmission in UL subband

Proposal 16: If a DL symbol by TDD-UL-DL-ConfigCommon is configured as a SBFD symbol, the SBFD-aware UE treats it as a flexible symbol by TDD-UL-DL-ConfigCommon.

Proposal 17: If it is agreed that only Option 1 (i.e., SBFD-aware UE determines link direction in a SBFD symbol based on configured/scheduled transmissions/receptions and collision handling) is supported in Rel-19, it can conclude that the legacy TDD-UL-DL-ConfigDedicated and/or SFI are not applicable for the SBFD symbol.

Proposal 18: If it is agreed that both Option 1 (i.e., SBFD-aware UE determines link direction in a SBFD symbol based on configured/scheduled transmissions/receptions and collision handling) and Option 2 (i.e., link direction of SBFD-aware UE in SBFD symbol is explicitly indicated by gNB) are supported in Rel-19, for the SBFD-aware UE, the legacy TDD-UL-DL-ConfigDedicated can be used to indicate the link direction in the SBFD symbol. Also, for the SBFD-aware UE, the legacy TDD-UL-DL-ConfigDedicated can be used in non-SBFD symbols to indicate whether the UE performs DL reception or UL transmission on the entire frequency resource.

- For a SBFD symbol, if it is directed to DL by TDD-UL-DL-ConfigDedicated, the UE can perform DL reception within DL usable PRB resources in the symbol.

- For a SBFD symbol, if it is directed to UL by TDD-UL-DL-ConfigDedicated, the UE can perform UL transmission within UL usable PRB resources in the symbol.

- If a SBFD symbol is not directed to DL/UL by TDD-UL-DL-ConfigDedicated, link direction in the SBFD symbol is determined based on configured/scheduled transmissions/receptions and collision handling.

Proposal 19: To avoid the collision between UL transmission and UL/DL switching time before and after reception of SSB symbols, specify a UE behavior considering on Rx-Tx switching time and Tx-Rx switching time between UL signal/channel and SSB symbols

Reference

- Huawei, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RP-241614, RAN#104, June 17-20, 2024

- 3GPP RAN1 Chair’s note in RAN1#116 meeting

- Moderator (CATT), “Summary #3 of 	SBFD TX/RX/measurement procedures,” R1-2401654, RAN1#116, February 26th – March 1st, 2024

- 3GPP RAN1 Chair’s note in RAN1#116bis meeting

- Moderator (CATT), “Summary #3 of 	SBFD TX/RX/measurement procedures,” R1-2403474, RAN1#116bis, April 15th – 19th, 2024

- 3GPP RAN1 Chair’s note in RAN1#117 meeting

- Moderator (CATT), “Summary #4 of 	SBFD TX/RX/measurement procedures,” R1-2405425, RAN1#117, May 20th – 24th, 2024

- 3GPP RAN1 Chair’s note in RAN1#118 meeting

- Moderator (CATT), “Summary #3 of 	SBFD TX/RX/measurement procedures,” R1-2407220, RAN1#118, August 19th – 23th, 2024

- 3GPP RAN1 Chair’s note in RAN1#118bis meeting

- Moderator (CATT), “Summary #3 of 	SBFD TX/RX/measurement procedures,” R1-2409088, RAN1#118bis, October, 14th – 18th, 2024

- 3GPP RAN1 Chair’s note in RAN1#119 meeting

- Moderator (CATT), “Summary #3 of SBFD TX/RX/measurement procedures,” R1-2410694, RAN1#119, November 18th – 22nd, 2024

- 3GPP TR 38.858 V18.0.0