3GPP TSG RAN WG1 #120																			R1-2500373

Athens, Greece, February 17th – 21st, 2025

Source:	vivo

Title:	Discussion on UE features for evolution of NR duplex operation

Agenda Item:	9.15.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on the UE features for Rel-19 SBFD. The objectives of NR duplex operation: Sub-band full duplex (SBFD) is illustrated in [1].

- Discussion

  - SBFD TX/RX/measurement procedures

  - For SBFD TX/RX/measurement procedures, following agreements related to UE capabilities were made.

Agreement (@RAN1#118b) For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols. Agreement (@RAN1#118b) For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Agreement (@RAN1#119) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

  - Regarding SBFD TX/RX operation, a general UE capability is defined, which includes a component that UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots are supported with Configuration 1. For Configuration 2, it involves more UE complexity, and can be defined as an enhanced UE capability.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-1 | Basic SBFD operation | Support basic SBFD operation 1. SBFD subband time/frequency configuration 2. UL transmissions within UL usable PRBs and DL receptions within DL usable PRBs in SBFD symbols 3. Enhancements for frequency domain resource allocation in SBFD symbols 4. UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1 5. Collision handling between UL transmissions and DL receptions in SBFD symbols 6. Maximum number of supported DL partial PRGs in SBFD symbols |  | Yes | No | UE does not support basic SBFD operation | Per UE | N/A (TDD only) | No | N/A | For component 6, candidate values for maximum number of supported DL partial PRGs in SBFD symbols: {2,3,4] | Optional with capability signalling |
| 60. NR_duplex_evo | 60-2 | Tx/Rx behavior with Configuration 2 | Support UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 2 | 60-1 | Yes | No | UE does not support SBFD operation with Configuration 2 | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-3 | CSI processing timeline in SBFD symbols | Support CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands. 1. UE supports either of following { CSI processing timeline and CPU counting are the same as legacy; Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy } | 60-1 | Yes | No | UE does not support CSI processing in SBFD symbols to process the CSI-RS across two DL subbands. | Per UE | N/A (TDD only) | No | N/A | Note: If this new indication is missing, the UE defaults to { Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy }. | Optional with capability signalling |

  - SBFD random access operation

  - Regarding the feature of SBFD random access operation, a UE capability of supporting SBFD RACH operation should be defined.

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-4 | SBFD RACH with single RACH configuration | Support RACH with SBFD operation for idle/inactive and connected states. 1. Support RACH resources in SBFD symbols - RACH configuration Option 1 (i.e., use one single RACH configuration) |  | Yes | No | UE does not support RACH with SBFD operation | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-5 | SBFD RACH with additional RACH configuration | Support RACH with SBFD operation for idle/inactive and connected states. 1. Support RACH resources in SBFD symbols - RACH configuration; Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration)} |  | Yes | No | UE does not support RACH with SBFD operation | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-6 | RACH occasion across SBFD and non-SBFD symbol | Support additional RO across SBFD symbols and non-SBFD symbols in the same slot or across slots | 60-6 | Yes | No | UE does not support RACH occasion across SBFD and non-SBFD symbol | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |

  - CLI handling

- For enhancements for CLI handling, following agreements related to UE capabilities were made.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity. - FFS: configuration of number of reported CLI resources in the reportConfig - FFS: reporting criteria Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - Note: No new usage needs to be defined for SRS resource sets Agreement Update the following agreement Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI. Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols. Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled. Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS. Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

  - Regarding the feature of CLI handling, following UE capabilities should be defined.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-7 | UL resource muting for DFT-s-OFDM | Support UL resource muting for a PUSCH when transform precoding is enabled - Support of semi-statically configured time location of each of one or two UL muting symbols, and dynamically turn on/off all of the semi-statically configured time location of UL muting symbol(s) by TDRA field in DCI. - Support of configuration of comb offset for the frequency location of UL resource muting for a PUSCH |  | Yes | No | UE does not support UL resource muting for DFT-s-OFDM | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-8 | UL resource muting for CP-OFDM | Support UL resource muting for a PUSCH when transform precoding is not enabled - Support of semi-statically configured time location of each of one or two UL muting symbols, and dynamically turn on/off all of the semi-statically configured time location of UL muting symbol(s) by TDRA field in DCI. - Support of configuration of comb offset for the frequency location of UL resource muting for a PUSCH - Support of 3dB power boosting for REs in the PUSCH symbol not containing PT-RS with UL resource muting. |  | Yes | No | UE does not support UL resource muting for CP-OFDM | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-9 | Aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type #1 and reporting | Support aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type #1 and reporting |  | Yes | No | UE does not support aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type#1 and reporting | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-10 | Aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type #2 and reporting | Support aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type #2 and reporting |  | Yes | No | UE does not support aperiodic L1 CLI-RSSI measurement in DL subband with measurement resource type#2 and reporting | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-11 | Aperiodic L1 CLI-RSSI measurement in UL subband and reporting | Support aperiodic L1 CLI-RSSI measurement in UL subband and reporting |  | Yes | No | UE does not support aperiodic L1 CLI-RSSI measurement in UL subband and reporting | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |
| 60. NR_duplex_evo | 60-12 | Aperiodic L1 CLI-SRS-RSRP measurement and reporting | Support aperiodic L1 CLI-SRS-RSRP measurement and reporting |  | Yes | No | UE does not support aperiodic L1 CLI-SRS-RSRP measurement and reporting | Per UE | N/A (TDD only) | No | N/A |  | Optional with capability signalling |

# References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), RAN #104, June 17-20, 2024.