3GPP TSG RAN WG1 #120          						R1-2500481

Athens, Greece, Feb 17th – 21st, 2025

Source:	Tejas Networks Ltd.

Title:	Discussion on multiple access, scheduling and timing aspects for A-IoT

Agenda item:	9.4.4

Document for:        Discussion and Decision

# Introduction

As discussed in TR 38.769, the Ambient IoT (A-IoT) communication system model includes a base station or reader communicating with multiple A-IoT devices available in the vicinity of the reader. The reader always transmits broadcast messages to all the devices. The devices decode the downlink (DL) message signals and decide whether to respond to that message or not. The reader to device (R2D) transmission is single message signal or multiple message signals which can be the repetition of the same message signal or the same service request. Thus, the R2D message signals can use either Time Division Multiplexing or Time Division Multiple Access (TDMA). Whereas, multiple A-IoT devices respond to the DL broadcast signals. For efficient resource allocation and efficient system performance both TDMA and Frequency Division Multiple Access (FDMA) can be used.

Fig. 1 presents the timing diagram of UL and DL message exchange between reader and device in multiple cycles. Inventory cycle is considered as the time required to complete the inventory process when 99% devices available in the vicinity successfully transmits the data to the reader using multiple communication slots.

Fig. 1: Timing diagram of UL and DL message signals between the device and the reader.

# L1 Control Information

  - L1 R2D control information

For R2D, the only physical channel is PRDCH, which carries any higher-layer payload (including system information, if defined), and L1 R2D control information, if defined. The following design options have been studied for PRDCH:

Option 1: 	If there is no L1 R2D control information carried by PRDCH, then PRDCH transmission with only R2D data is considered.

Note: In this option, R2D control information (if any) maybe carried by higher-layer signalling.

Option 2: 	If any L1 R2D control information is supported & carried by PRDCH, then for the CRC attachment (appended if there is non-zero length CRC), following two options have been studied.

Option 2-1: Joint CRC is attached to L1 R2D control information and R2D data.

Option 2-2: Separate CRC is attached to L1 R2D control information and R2D data, respectively.

The following is discussed in TR 38.769, section 6.1.1 on L1 R2D control information [1]:

We propose to include L1 R2D control information, as few information below can be sent as a part of L1 control information which are required to decode the L1 data.

The R2D control information may include:

    - Modulation scheme (OOK-1/OOK-4), M value

    - Coding scheme

    - Receiving method (coherent/non-coherent)

    - Length of PRDCH frame

    - Midamble overhead if any

    - Time, frequency, and phase accuracy

    - Chip length

    - Repetitions (bit level/chip level/block level)

In A-IoT communication, the control information length and data length are comparatively smaller and hence single CRC is sufficient to decode the error. Therefore, we propose to attach joint CRC on the L1 R2D control information and data or the control information can be sent without CRC and Manchester coding.

Proposal 1: We propose to include L1 R2D control information to send the following:

    - Modulation scheme (OOK-1/OOK-4), M value

    - Coding scheme

    - Receiving method (coherent/non-coherent)

    - Length of PRDCH frame

    - Midambles overhead if any

    - Time, frequency, and phase accuracy

    - Chip length

    - Repetitions (bit level/chip level/block level)

Proposal 2: We propose the following options to transmit the L1 control information:

Option 1: Attach joint CRC on the L1 R2D control information and data

Option 2: The control information can be sent without CRC and Manchester coding.

  - L1 D2R control information

For D2R, a physical channel PDRCH carries any higher-layer payload, the response transmitted from device to reader during the contention-based access procedure, and L1 D2R control information, if defined.

The following is discussed in TR 38.769, section 6.1.2.5 on L1 D2R control information [1]:

The D2R control information may include the D2R modulation and coding scheme, and other parameters required to decode the PDRCH frame.

Proposal 3: We propose to include the following as L1 D2R control information:

    - Modulation scheme (OOK/BPSK)

    - Coding scheme

    - Receiving method (coherent/non-coherent)

    - Length of PDRCH frame

    - Midambles overhead if any

    - Time, frequency, and phase accuracy

    - Time domain resource information of the PDRCH signal

    - Frequency domain resource information of the PDRCH signal

    - Chip length

    - Repetitions (bit level/chip level/block level)

# Estimate Optimal Value of M for R2D Transmission

The following are discussed in RP#106 on R2D modulation [2]:

The following objectives are set, within the General Scope:

- RAN1 scope:

  - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

  - R2D and D2R signal(s)

  - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

  - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

According to [2], only OOK-4 modulation is considered for R2D transmission. Thus, it is important to define the M-value for OOK-4 modulation for all DL message signals.

The following is discussed in TR 38.769, section 6.1.2 on L1 R2D control information [1]:

Reference signals including DMRS, PTRS, SRS, are not considered for D2R. CSI feedback and autonomous SR are not considered for L1 D2R control information.

Fig. 2a: Device estimates the M-value and send to reader through msg1.

Fig. 2b: Reader estimates the M-value from the known preambles received through msg1.

According to [1], reference signals are not considered for D2R to provide CSI feedback to the reader. Therefore, we propose the following three options to decide the optimum M value.

Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg1 (Fig. 2a).

Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions (Fig. 2b).

Option 3: An optimum value can be decided based on the following parameters:

- Received signal strength of the UL message signals received form the device

- Block error rate

- Data rate and latency

Observation 1: As R2D signals are transmitted using OOK-4 generated by OFDM modulator, it is important to define the M-value for OOK-4 modulation for all DL message signals.

Proposal 4: we propose the following three options to decide the optimum M value.

    - Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg.

    - Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions.

    - Option 3: An optimum value can be decided based on the following parameters:

- Received signal strength of the UL message signals received form the device

- Block error rate

- Data rate and latency

# Scheduling Information and Timing Relationships of A-IoT

The agreements made in RAN2 #127-bis [3] on TDMA and FDMA for Msg1 are as follows:

From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

The devices can be allocated few frequency domain resources along with time domain resources based on the transmission bandwidth of the devices and the total communication bandwidth of A-IoT.

Fig. 3: Multiple Msg1 transmission from multiple devices at different time and frequency.

When multiple devices transmit Mgs1 at the same time in response to Msg0 from the reader, there is a chance of collision. To minimize collision and maximize throughput, the device can use slotted Aloha based random access method to transmit Msg1 frame. The D2R Msg1 transmission time is denoted as T1 which is bounded by [T1_min, T1_max]. T1_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min, is the maximum time available to the device to transmit Msg1. After receiving the Msg0, the devices randomly select a time slot t between 1 to X and a frequency f between 1 to Y and transmit Msg1 at that time slot using that frequency. The signal propagation time Tp << T1_min. Thus, T1_min is considered as the slot duration of slotted Aloha. If the device does not get acknowledgement, it randomly chooses a time slot from the remaining available slots and retransmit Msg1. The flow diagram of slotted Aloha mechanism to transmit Msg1 is given in Fig. 4. If any device is not able to access the channel to send Msg1 within the time [T1_min, T1_max], it goes to sleep mode and reattempts in the next communication slot.

Fig. 4: Slotted Aloha-based multiaccess communication.

Observation 2: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 5: RAN1 should define the maximum and minimum time TD2R_min and TD2R_max for transmitting D2R signal Msg1. T1_min = TD2R_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the maximum time available to the device to transmit Msg1. T1_min is considered as the slot duration of slotted Aloha.

# Multiplexing/multiple access in D2R Msg1 by TDMA and FDMA

The R2D transmission bandwidth agreed on RAN1#118 is [4]:

Agreement The following table is a starting point for the study of M values and the associated minimum Btx,R2D value - Reader can use any R2D transmission bandwidth >= minimum Btx,R2D - FFS: Impacts, if any, of CP handling solutions, and other impacts - Note: depending on further study, the maximum value of M may be less than 32 M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 2. 32; 3.

Although R2D transmission bandwidth is studied, the D2R chip length and its range are not studied for the D2R bandwidth. The frequency domain resources available for Msg1 transmission from multiple devices depend on the transmission bandwidth required for each device and the D2R receiver bandwidth. If the device maintains the OFDM symbol duration chip length of 71 µs.

Considering channel bandwidth is same as the receiver bandwidth of the reader, the guard band is considered as minimum 20% of the receiver bandwidth. Thus, the available frequency domain resources are calculated from the effective bandwidth after deducting the guard band from the receiver bandwidth.

| Chip rate (kcps) | Chip length (µs) | D2R transmission bandwidth (kHz) | No. of frequency domain resources for D2R receiver bandwidth without guard band |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 180 kHz | 360 kHz | 1.08 MHz |
| 14 | 71.4 | 15 | 9 | 19 | 57 |
| 28 | 35.7 | 30 | 4 | 9 | 28 |
| 56 | 17.85 | 60 | 2 | 4 | 14 |
| 112 | 8.92 | 120 | 1 | 2 | 7 |

Agreement

RAN1 studies following:

- A R2D transmission triggering random access determines X time domain resource(s) for D2R transmission(s) for Msg1, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s).

- The study includes

  - Study X=1 and X>1 and X>=1, the maximum value of X>1 should be set considering the device implementation complexity, device power consumption, the resource usage efficiency affected at least by SFO, and inventory latency.

  - Size(s) for resource allocation in the time domain

  - Determination of the X time domain resource(s) by the device

  - Addressing timing errors for adjacent time domain resources due to residual SFO of the device

The following agreements were made in RAN1#118-bis on the time domain resources for Msg1 transmission [5]:

The time domain resources or the time domain slots of slotted aloha should be decided based on the number of devices. Increasing the time domain slots increases the overall Msg1 transmission time. Thereby increases the single device latency. However, decreasing the time domain slots increases the overall latency or the inventory completion time as the re-attempt probability of devices increases due to the unavailability of slots.

Let, X = Number of time domain resources

N = Number of available devices to transmit Msg1

LD = Average value of single device latency; where LD=T0+T1+T2+T3+T4+TGAP

L = Overall latency or the inventory completion time

P = Power consumption of device

ED = Energy consumption of the device

Pre-attempt = Re-attempt probability of device assuming one attempt from each device at one communication slot

Thus, the optimum number of time domain resources are to be chosen such that it minimizes the single device latency as well as the overall latency, as given in (1).

X*=argminX N×LD+L              (1)

s.t.  Lmin≤L≤Lmax

P×LD<ED

Pre-attempt=0.1

Proposal 6: The available frequency domain resources (Y) depends on the transmission bandwidth of devices, receiver bandwidth of the reader, and the guard band. Considering channel bandwidth is same as the receiver bandwidth of the reader and the guard band is minimum 20% of the receiver bandwidth, the frequency domain resources for different values of D2R chip length are:

| Chip rate (kcps) | Chip length (µs) | D2R transmission bandwidth (kHz) | No. of frequency domain resources for D2R receiver bandwidth without guard band |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 180 kHz | 360 kHz | 1.08 MHz |
| 14 | 71.4 | 15 | 9 | 19 | 57 |
| 28 | 35.7 | 30 | 4 | 9 | 28 |
| 56 | 17.85 | 60 | 2 | 4 | 14 |
| 112 | 8.92 | 120 | 1 | 2 | 7 |

Proposal 7: The number of time domain resources X primarily depends on the total number of devices available in the vicinity of the reader and the system deployment scenarios.

The optimum number of time domain resources are to be chosen such that it minimizes the single device latency as well as the overall latency as follows:

X*=argminX N×LD+L

s.t.  Lmin≤L≤Lmax

P×LD<ED

Pre-attempt=0.1

X = Number of time domain resources

N = Number of available devices to transmit Msg1

LD = Average value of single device latency

L = Overall latency or the inventory completion time

P = Power consumption of device

ED = Energy consumption of the device

Pre-attempt = Re-attempt probability of device assuming one attempt from each device at a communication slot

# Msg0 content for scheduling and timing related information

The timing related information form msg0, msg1, and msg2 and scheduling related information for msg1 transmission from multiple devices can be explicitly or implicitly conveyed to device through initial paging signal msg0. The information includes:

- 1 bit information for 2-step/4-step CBRA: msg0 signal can carry one bit information to convey the 2-step or 4-step CBRA process to the device.

- Msg0 related information:

- The reader can inform the duration of msg0 transmission (T0) to the reader through msg0 signal. As the distance between the reader and the devices are very small (few meters) in Ambient IoT communication, it is assumed that all the devices will receive msg0 at the same time. Knowing T0 the device can calculate the start time of T1.

- Msg1 related information:

- The duration of time domain slot (T1_min) for slotted aloha based multi-access communication to transmit msg1.

- Total number of time domain slots or occasions available to the device to randomly choose one slot and transmit msg1.

- Total number of frequency domain resources such as, the number of subcarriers available to the device to randomly choose one frequency domain resource and transmit msg1.

- Msg2 related information:

- The msg0 signal can carry 1 bit to convey the information to the device that the reader will send one common msg2 signal or multiple msg2 signals in response to multiple msg1 signals received from the devices available in the vicinity.

- The duration of msg2 signals transmission (T2). One or multiple msg2 signals can be transmitted within the duration T2.

Proposal 8: The following information can be explicitly or implicitly conveyed to the device through msg0:

- 1 bit information for 2-step/4-step CBRA

- Msg0 timing related information

- Msg1 timing related information

- Msg1 scheduling related information

- Msg2 related information

- One or multiple msg2 transmission

# Msg1 content for scheduling related information

The energy status of the device may consist of one or more bits to indicate the energy available at the device for receiving Msg1, followed by executing the further communication with the reader. If the energy available at the device is below certain threshold, reader may choose not to allocate resource to that device. The device can get more energy form the CW signal received in the next slot and try again to communicate with reader at that slot. The D2R control information of Msg1 and Msg3 are same.

The following information can be explicitly or implicitly conveyed to the BS/reader from the device through msg1 for scheduling the device in 4-step CBRA process:

    - Msg ID

    - Unique random ID associated with the device

    - Energy status of the device

    - Msg3 size

    - Transmission bandwidth of device

Proposal 9: The following information can be explicitly or implicitly conveyed to the BS/reader from the device through msg1 for scheduling the device in 4-step CBRA process:

    - Msg ID

    - Unique random ID associated with the device

    - Energy status of the device

    - Msg3 size

    - Transmission bandwidth of device

Proposal 10: Information regarding the expected time gap between each message signal such as, TGapA, TGapB, etc. can be handled in two ways:

    - Option 1: The device already has the TGap information stored in the memory, which can reduce the number of bits transmitted through DL signals. In this case, the TGap values are fixed.

    - Option 2: The reader sends the TGap information through DL message signals to the device. In this case, the TGap values can be variable.

# Conclusion

This work includes the L1 control information, multiplexing/multiple access of UL signals, estimation of optimum M value for R2D signals, and scheduling and timing relationships. The work also discusses the msg0 and msg1 content to carry the scheduling and timing information. We have made the following observations and proposals related to the above-mentioned aspects of A-IoT:

Proposal 1: We propose to include L1 R2D control information to send the following:

    - Modulation scheme (OOK-1/OOK-4), M value

    - Coding scheme

    - Receiving method (coherent/non-coherent)

    - Length of PRDCH frame

    - Midambles overhead if any

    - Time, frequency, and phase accuracy

    - Chip length

    - Repetitions (bit level/chip level/block level)

Proposal 2: We propose the following options to transmit the L1 control information:

Option 1: Attach joint CRC on the L1 R2D control information and data

Option 2: The control information can be sent without CRC and Manchester coding.

Proposal 3: We propose to include the following as L1 D2R control information:

    - Modulation scheme (OOK/BPSK)

    - Coding scheme

    - Receiving method (coherent/non-coherent)

    - Length of PDRCH frame

    - Midambles overhead if any

    - Time, frequency, and phase accuracy

    - Time domain resource information of the PDRCH signal

    - Frequency domain resource information of the PDRCH signal

    - Chip length

    - Repetitions (bit level/chip level/block level)

Observation 1: As R2D signals are transmitted using OOK-4 generated by OFDM modulator, it is important to define the M-value for OOK-4 modulation for all DL message signals.

Proposal 4: we propose the following three options to decide the optimum M value.

    - Option 1: Msg0 contains known preamble from which the device can estimate the channel condition and choose an optimum M value from a lookup table stored in the device and convey the same through msg.

    - Option 2: Msg1 contains known preamble from which the reader can estimate the channel condition and choose an optimum M value from a lookup table stored in the reader and the M value will be used for subsequent DL transmissions.

    - Option 3: An optimum value can be decided based on the following parameters:

            - Received signal strength of the UL message signals received form the device

            - Block error rate

            - Data rate and latency

Observation 2: The maximum time duration for Msg1, considered as T1_max depends on the number of devices available, transmission bandwidth of device, and the receiver bandwidth of the reader which indicates the number of frequency domain resources available to the devices to transmit Msg1.

Proposal 5: RAN1 should define the maximum and minimum time TD2R_min and TD2R_max for transmitting D2R signal Msg1. T1_min = TD2R_min is the Msg1 frame transmission time, which is the minimum time required to transmit Msg1. T1_max = X × T1_min = TD2R_max is the maximum time available to the device to transmit Msg1. T1_min is considered as the slot duration of slotted Aloha.

Proposal 6: The available frequency domain resources (Y) depends on the transmission bandwidth of devices, receiver bandwidth of the reader, and the guard band. Considering channel bandwidth is same as the receiver bandwidth of the reader and the guard band is minimum 20% of the receiver bandwidth, the frequency domain resources for different values of D2R chip length are:

| Chip rate (kcps) | Chip length (µs) | D2R transmission bandwidth (kHz) | No. of frequency domain resources for D2R receiver bandwidth without guard band |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 180 kHz | 360 kHz | 1.08 MHz |
| 14 | 71.4 | 15 | 9 | 19 | 57 |
| 28 | 35.7 | 30 | 4 | 9 | 28 |
| 56 | 17.85 | 60 | 2 | 4 | 14 |
| 112 | 8.92 | 120 | 1 | 2 | 7 |

Proposal 7: The number of time domain resources X primarily depends on the total number of devices available in the vicinity of the reader and the system deployment scenarios.

The optimum number of time domain resources are to be chosen such that it minimizes the single device latency as well as the overall latency as follows:

X*=argminX N×LD+L

s.t.  Lmin≤L≤Lmax

P×LD<ED

Pre-attempt=0.1

X = Number of time domain resources

N = Number of available devices to transmit Msg1

LD = Average value of single device latency

L = Overall latency or the inventory completion time

P = Power consumption of device

ED = Energy consumption of the device

Pre-attempt = Re-attempt probability of device assuming one attempt from each device at a communication slot

Proposal 8: The following information can be explicitly or implicitly conveyed to the device through msg0:

- 1 bit information for 2-step/4-step CBRA

- Msg0 timing related information

- Msg1 timing related information

- Msg1 scheduling related information

- Msg2 related information

- One or multiple msg2 transmission

Proposal 9: The following information can be explicitly or implicitly conveyed to the BS/reader from the device through msg1 for scheduling the device in 4-step CBRA process:

    - Msg ID

    - Unique random ID associated with the device

    - Energy status of the device

    - Msg3 size

    - Transmission bandwidth of device

Proposal 10: Information regarding the expected time gap between each message signal such as, TGapA, TGapB, etc. can be handled in two ways:

    - Option 1: The device already has the TGap information stored in the memory, which can reduce the number of bits transmitted through DL signals. In this case, the TGap values are fixed.

    - Option 2: The reader sends the TGap information through DL message signals to the device. In this case, the TGap values can be variable.

# 4. References

[1] TR 38.769, Study on solutions for ambient IoT (Internet of Things) in RAN, V2.0.0.

[2] RP-243326, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

[3] 3GPP RAN2 Chairman’s Notes, 3GPP TSG RAN2 meeting #127-bis.

[4] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #118, Maastricht, Netherlands, August 19th – 23rd, 2024.

[5] 3GPP RAN1 Chairman’s Notes, 3GPP TSG RAN1 meeting #118-bis, Hefei, China, Oct 14th – 18th, 2024.