3GPP TSG-RAN WG1 #120	R1-2500533

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.3

Source:	InterDigital, Inc.

Title:	On AI/ML-based CSI prediction

Document for:	Discussion and Decision

# Introduction

In RAN #102, normative work on AI/ML for NR Air Interface has been approved with some additional studies which were not finalized during the study phase [1].  For the CSI enhancement use case, the objective of the study was to address some of the outstanding issues identified during the Rel-18 Study Item, as summarized in TR 38.843 [2].

The work item was updated at the RAN #105 checkpoint and subsequently in RAN #106 to provide specification support for CSI prediction (UE-sided model), as shown below [3], [10].

Provide specification support for the following aspects: ----------------------------------------------------------Text omitted ------------------------------------------------------ - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In RAN1 #118b [8], the following agreement regarding consistency between training and inference has been made.

Agreement For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance. Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

The consistency between training and inference was discussed in RAN1 #119 ([11] and [12]), but no consensus was reached whether associated ID is needed or performance monitoring is good enough to support consistency for the CSI prediction use case.

This contribution discusses aspects of consistency of training/inference and the necessity of the associated ID for the CSI prediction use case. We provide intermediate KPI evaluation results for various NW-side additional conditions including down tilt angles, as well as for localized and generalized models and compare their relative SGCS gains over benchmark #1. We also discuss specification impacts of performance monitoring for UE-side CSI prediction models.

# Performance evaluation

## AI/ML Model considerations

### Model architecture

Transformers based model

The CNN-Xformer model exploits both spatial and temporal correlation of L input (NRx×NTx) MIMO channels, where prediction for each RB is performed independently.

The convolutional block, consisting of 8 Conv2D layers, residual connections and batch normalization, is leveraged to extract spatial features. Subsequently, a transformer encoder block is utilized to learn the temporal features of the channel, followed by a fully connected layer at the output of the model. The model has a total of 162 k parameters and is trained using a weighted combination of SGCS and NMSE as the loss function.

### Dataset for model training

In the context of CSI prediction, let K denote the number of historical CSI to be fed to the CSI predictor (either the AI/ML model or the baselines), and let N denote the time slot index of the future CSI to be predicted. In the construction of the dataset, Nsamples channel samples are generated, where each two consecutive channel samples are 1 ms apart. The size of channel sample is NRx×NTx×Nsb×NUEs, where NRx, NTx, Nsb, and NUEs denote the number of receive antenna ports, the number of transmit antenna ports, the number of sub-bands, and the number of UEs, respectively. The time samples of each channel element in the 4D tensor are then grouped together to create NRx×NTx×Nsb×NUEs sequences of size Nsamples each. Afterwards, for each channel coefficient, Nseq= Nsamples-K+N+1 pair of sequences are constructed from its initial sequence of size Nsamples time samples. Specifically, for all i ∈ 1, Nseq: the two sequences are defined as follows:

- The first sequence contains the time samples of the channel element from i-1+1 to i-1+K, which is the sequence of features (or sequence of historical CSI) that will be fed to the CSI predictor,

- The second sequence contains the time samples of the same channel element from i-1+K+1 to i-1+N+1, which represents the sequence of labels (or sequence of future CSI) to be predicted by the CSI predictor.

Finally, the dataset consists of the NRx×NTx×Nsb×NUEs×Nseq distinct sequences of features, where each sequence has K features, and their associated NRx×NTx×Nsb×NUEs×Nseq sequences of labels, where each sequence has N labels.

## Impact of NW-side additional condition: Antenna down-tilt

To identify potential NW-side additional conditions, if any, that may impact the consistency between training and inference for UE-sided CSI prediction, RAN1 #118b reached the following agreement.

Agreement For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated: - [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees) - [Optional evaluation] Various TXRU mapping - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements) - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively) - Other configurations are not precluded Note: for other evaluation assumptions, reuse baseline of TR 38.843. Note: report how to map antenna port to antenna elements. Note: report the backbone/structure of AI/ML model used.

We evaluated the generalization performance, in terms of SGCS, for the UE-sided CSI prediction performance under antenna down tilt angles of 102 and 110 degrees ([9]).  For generalization Case 1 ([2]), the model is trained with a dataset using antenna down tilt configuration #B and tested with the same down tilt configuration #B, and for generalization Case #2 the model is trained with a dataset using down tilt configuration #A, and tested with a dataset using NW-side antenna down tilt configuration #B.  The evaluation results for a UE speed of 30 km/h, Doppler only, are shown in Table 1, for generalization Cases 1 and 2.

Table 1 Generalization performance of UE-sided CSI prediction over various antenna down-tilt

|  |  | Test Configuration #B: 102 deg down tilt | Test Configuration #B: 110 deg down tilt |
| --- | --- | --- | --- |
| S&H | SGCS L1 | 0.755 | 0.747 |
| Generalization Case1 | Train Configuration #B | 102 deg down tilt | 110 deg down tilt |
|  | SGCS L1 | 0.961 | 0.961 |
| Generalization Case2 | Train Configuration #A | 110 deg down tilt | 102 deg down tilt |
|  | SGCS L1 (absolute) | 0.961 | 0.960 |
|  | SGCS L1 (% relative to Case1) | 0.0 | -0.1 |

For generalization Case 2, in terms of SGCS, the results in Table 1 show that generalization performance with respect to antenna down tilt is achieved, at least for the considered test scenarios, as follows:

- When down tilt configuration #B is 102 degrees, and down tilt configuration #A is 110 degrees, there is no SGCS degradation with respect to Case 1.

- When down tilt configuration #B is 110 degrees, and down tilt configuration #A is 102 degrees, there is a very slight (negligible) SGCS degradation of 0.1% with respect to Case 1.

Observation 1: Case 2 generalization performance of the UE-sided CSI prediction over various antenna down tilts shows negligible SGCS change relative to Case 1.

## Intermediate KPI performance of localized models

The evaluation assumptions for training localized models for CSI prediction were agreed in RAN1 #117 [6], as shown below.

Agreement For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding training, - The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N. - The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach. - Region #A is the same as the union of regions #B_1, …, #B_N. - Region #A is a proper superset of the union of regions #B_1, …, #B_N. - Region #A is generated separately from regions #B_1, …, #B_N. - Note: companies to report which method was used. For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding testing, - The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N. - In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

In this section, we present intermediate KPIs for localized models, for an initial configuration of N=1 local regions (Region #B_1), and a generalized Region #A which is generated separately from Region #B_1.

For the local dataset (Region B#1), 360 UEs were dropped in 1 site (3 sectors/site) and the simulation length of 3000 slots. For the generalized dataset (Region #A), a total of 210 UEs were dropped across 7 sites (3 sectors/site), for 30 drops with simulation length of 300 slots for each drop. Spatial consistency was enabled for both the localized and the generalized datasets. We used the baseline 100% outdoor distribution as agreed in RAN1 #116b [5], with UE speed of 30 km/h.

The intermediate KPI (specifically, SGCS of Layer 1) performance relative to benchmark #1 (sample-and-hold) for the generalized and local model are presented in Table 2.

Table 2 SGCS performance of generalized/local AI/ML CSI prediction models relative to Benchmark#1

|  | Test on generalized data With spatial consistency (Region #A) | Test on local data With spatial consistency (Region #B1) |
| --- | --- | --- |
| SGCS Baseline #1 (Sample and hold) | 0.486 | 0.419 |
| AI/ML CSI prediction | Trained on generalized data (Region #A) | Trained on local data (Region #B1) |
| Percent SGCS improvement of AI/ML CSI model over benchmark #1 | 17.5% | 19.8% |

The results in Table 2 show that – at least for the scenario considered here – the localized AI/ML CSI prediction model provides a minor improvement of the SGCS gain over benchmark #1, as compared to the generalized model.

Observation 2: For the simulated scenario (one local region and a generalized region generated separately from the local) the localized AI/ML CSI prediction model provides a minor increase of the SGCS gain over benchmark #1, as compared to the generalized model.

# Considerations on AI/ML-based CSI Prediction

## Discussion on consistency of training/inference for UE-side CSI prediction

During the Rel-18 SI, it was identified that a ML model may be associated with “specific configurations/conditions associated with UE capability capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side“ (section 4.2.1 of TR 38.843 [2]).  The additional conditions can be NW-side additional conditions, and UE-side additional conditions, where the additional conditions may not necessarily be specified.

For UE-side models, the SI further identified potential approaches to ensure consistency of training and inference regarding NW-side additional conditions, as follows (section 4.2.3 of TR 38.843 [2]).

For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary): - Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side - Model training at NW and transfer to UE, where the model has been trained under the additional condition - Information and/or indication on NW-side additional conditions is provided to UE - Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality) - Other approaches are not precluded - Note: the possibility that different approaches can achieve the same function is not denied

In RAN1 #116 [4], it has been agreed to study options for model identification, including “MI-Option 1: Model identification with data collection related configuration(s) and/or indication(s) “.

The Associated ID is part of an example of MI-Option 1, agreed to be studied at RAN1 #116b [5], as follows:

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed - Alt.4: Model ID is determined by pre-defined rule(s) in the specification - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Lastly, the following assumption was agreed in RAN1 #118 [7].

Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

The Associated ID may be used to determine the consistency of training and inference for UE-sided models; for example, when the data collection and inference configurations match, the UE may determine that the NW-side additional conditions are consistent between training and inference if the associated ID signalled by the NW for data collection matches the associated ID signalled by the NW at inference time.

However, at least for UE-sided CSI prediction models, the associated ID based approach to determining the consistency of training/inference (for NW-side additional conditions) may be unnecessarily high complexity. For example, from a NW perspective, while it is understood that the associated ID is generated by the NW to correspond to proprietary or implementation specific NW-side additional conditions, it is not clear what resolution of NW-side additional conditions warrants a different associated ID. This may potentially result in a large number of associated IDs. From a UE perspective, managing models corresponding to different associated IDs across different cells and potentially different NW-vendors may result in increased UE complexity.

Observation 3: At least for UE-sided CSI prediction models, the associated ID based approach for determining the consistency of training/inference (for NW-side additional conditions) may be of high complexity and is not essential.

For UE-sided CSI prediction, when the associated ID does not match between data collection and inference, the UE may assume that there is no consistency between the NW-side additional conditions; however, this may not necessarily result in performance degradation, so model performance monitoring and/or out-of-distribution (OOD) detection would be needed to further determine if the lack of consistency between training and inference results in poor performance.  Given that model performance monitoring is needed as part of LCM, it is not clear that the use of associated ID provides additional benefit for determining the consistency.

From a CSI prediction model generalization perspective, the evaluations performed during the first part of the Rel-19 WI have shown that for generalization Case3 (where the training dataset is constructed with data samples subject to multiple scenarios/configurations including the inference scenario/configuration), generalized performance of the AI/ML CSI prediction model can be achieved ([7]).  For example, for generalization Case3, negligible degradation is observed over different aspects known at the NW-side (but not available at the UE-side), such as UMi/UMa, or different UE outdoor/indoor distributions. Moreover, our evaluation of intermediate KPI for localized and generalized models (Section 2.3), shows a negligible difference between localized and generalized models, in terms of the SGCS gain over benchmark #1.  Lastly, our evaluation of generalization performance of UE-sided CSI prediction over various antenna down-tilt angles (Section 2.2) shows that the model generalizes well (as the generalization Case 2 SGCS performance degradation relative to Case 1 is negligible).

For CSI prediction, given the findings on the generalization performance of the AI/ML models, and the fact that model performance monitoring may be needed when the training and inference associated ID are mismatched, it appears that the associated ID does not provide additional benefits. When the UE determines that the inference configuration matches the data collection configuration for a given AI/ML model and if the UE-side additional conditions match between data collection (training) and inference (e.g., UE speed), the UE may assume that the AI/ML model is applicable to the current conditions. The UE may use metrics such as OOD metrics as well as model monitoring metrics (e.g., intermediate KPIs) to ensure consistency between training and inference regarding NW-side additional conditions. If based on the OOD and/or model monitoring metrics, the UE determines that the NW-side additional conditions are not consistent between data collection and inference, the UE may use model switching and/or fallback to legacy.

In view of the considerations above as well as the complexity of managing the associated ID, we believe that at least for CSI prediction, the associated ID is not needed to ensure training/inference consistency regarding NW-side additional conditions.

Proposal 1: For CSI prediction, the associated ID is not needed to ensure training/inference consistency regarding NW-side additional conditions.

## UE-side CSI prediction model performance monitoring

Regarding model performance monitoring, in Rel-18, the TR 38.843 captured the following types of UE-side CSI prediction model monitoring for functionality-based LCM:

- Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

For Type 2 model performance monitoring, reporting the predicted CSI and/or the corresponding ground truth incurs excessive feedback overhead (e.g., the overhead may be even larger in case of CSI prediction where N4>1) which may not be justified for checking prediction performance since a UE can simply do the same at the UE side and report only the outcome of the performance monitoring occasion. Therefore, the Type 2 performance monitoring should be ruled out from the options.

Proposal 2: Type 2 model performance monitoring is not further considered.

During the Rel-19 work item, the following agreement was made in RAN1 #117 [6] regarding the boundary between Type 3 and Type 1 performance monitoring:

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured.

Regarding the model monitoring types, RAN1 #117 [6] identified at least the following specification impacts.

Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

For Type 1, for monitoring output, the UE may report the binary outcome of the CSI prediction model performance at the UE against the performance threshold set by the gNB. Type 1 can be considered as UE-side monitoring, and it may have lower feedback overhead compared to Type 3 monitoring, at the expense of less granular monitoring information provided to the gNB.

For Type 3, for performance metrics, the UE may report for example the CSI prediction accuracy, e.g., NMSE or SGCS between predicted CSI and measured CSI; the gNB may determine whether the prediction model performs above the required performance set by gNB, and the UE may not need to know about the threshold. In that sense, Type 3 seems to be more appropriate for the cases where the gNB monitors the UE-side CSI prediction model and the UE provides assistance information. Moreover, Type 3 may provide more granular information to the network to enable the determination of cause of error.

Proposal 3: Type 3 is supported for UE-side model performance monitoring.

Regarding the performance metric for Type 3 (and for Type 1 if selected), one option is to use the intermediate KPI (e.g., SCGS and NMSE).  The evaluation part of the Rel-18 and Rel-19 studies found that intermediate KPIs (such as SGCS and/or NMSE) may not be the best indicators of the overall end-to-end performance. For example, for CSI prediction, a lower SGCS value may not result in a significant throughput loss; moreover, a lower SGCS value may not necessarily indicate poor performance of the model. For example, when the prediction window size is larger than the coherence time of the channel, poor performance is due to incorrect prediction configuration rather than generalized poor model performance. Inaccurate indicators of the model performance may result in increased overhead for model switching, activation/deactivation, or model parameter configuration. To avoid such overhead, it is important to use metrics for monitoring the UE-side CSI prediction model that are more indicative of the end-to-end performance.

Observation 4: 	Metrics for monitoring the UE-side CSI prediction model that are more indicative of end-to-end performance should be used to reduce the model switching, activation/deactivation overhead.

UE-side performance monitoring based on data distribution may help improve the model performance, where metrics related to the distribution of input data, determined by the UE, could be reported in addition to the intermediate KPI metric. For example, the UE may be configured to perform out-of-distribution detection for the input data, or to provide out-of-distribution metrics. While these metrics may not directly reflect the end-to-end system performance, when used in conjunction with intermediate KPIs and end-to-end metric, they can help improve the model monitoring performance.

Observation 5:	Using out-of-distribution metrics in conjunction with intermediate KPIs may be beneficial for UE-side model monitoring.

Proposal 4:	For UE-side model monitoring, out-of-distribution metrics should be used in conjunction with intermediate KPIs.

# Conclusion

In this contribution, we discussed aspects of CSI prediction accuracy, model complexity, and LCM aspects related to model monitoring, and provided a revised set of evaluation results for CSI prediction.

We provide the following observations and proposals.

Observation 1: Case 2 generalization performance of the UE-sided CSI prediction over various antenna down tilts shows negligible SGCS change relative to Case 1.

Observation 2: For the simulated scenario (one local region and a generalized region generated separately from the local) the localized AI/ML CSI prediction model provides a minor increase of the SGCS gain over benchmark #1, as compared to the generalized model.

Observation 3: At least for UE-sided CSI prediction models, the associated ID based approach for determining the consistency of training/inference (for NW-side additional conditions) may be of high complexity and is not essential.

Observation 4: 	Metrics for monitoring the UE-side CSI prediction model that are more indicative of end-to-end performance should be used to reduce the model switching, activation/deactivation overhead.

Observation 5:	Using out-of-distribution metrics in conjunction with intermediate KPIs may be beneficial for UE-side model monitoring.

Proposal 1: For CSI prediction, the associated ID is not needed to ensure training/inference consistency regarding NW-side additional conditions.

Proposal 2: Type 2 model performance monitoring is not further considered.

Proposal 3: Type 3 is supported for UE-side model performance monitoring.

Proposal 4:	For UE-side model monitoring, out-of-distribution metrics should be used in conjunction with intermediate KPIs.

# References

RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

3GPP TR38.843 V18.0.0 (2023-12) Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description.

RAN1 #116 Chairman’s note

RAN1 #116b Chairman’s note

RAN1 #117 Chairman’s note

RAN1 #118 Chairman’s note

RAN1 #118b Chairman’s note

3GPP TR 38.901 v18.0.0 (2024-03) Study on channel model for frequencies from 0.5 to 100 GHz (Release 18)

RP-243244, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description

RAN1 #119 Chairman’s note

R1-2410899, “Summary #3 of CSI prediction”