3GPP TSG RAN WG1 #120		R1-2500554

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.5.3

Source:	Google

Title:	Adaptation of Common Signals

Document for:	Discussion/Decision

# Introduction

In RAN1 #119 meeting, the following agreements on adaptation of common signal have been achieved.

Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Agreement At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH. Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16] - FFS: whether the subset of the additional PRACH resources is in - Alt 2-1: RO level per SSB - Alt 2-2: SSB-to-RO mapping cycle level - Alt 2-3: PRACH association period level - Alt 2-4: PRACH association pattern period level - Alt 2-5: SFN level - Alt 2-6: Network configured time period Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Working Assumption For DCI-based adaptation for additional PRACH resources, - Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI - Option 1: Use reserved bits in the DCI format - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format - Option 2: Use Bits 5-8 within the Short Message (from upper layers) - Note: Availability should be confirmed by checking with RAN2. - Option 3: Use bits available for both Option 1 and Option 2 FFS: Payload size for adaptation for additional PRACH resources Agreement For DCI-based adaptation for additional PRACH resources, select only from the following alternatives: - Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - [DCI payload size = 1 bit] - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources - FFS: details Agreement Confirm the following working assumption from RAN1#118bis. Working Assumption For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode. - P-RNTI is used

In this contribution, we provide some discussion on adaptation of common signal/channel transmissions.

# Adaptation of SSB in time domain

## Control signal to update the SSB periodicity

For NES, the NW transmit some SSBs associated with active TCI state frequently but other SSBs less frequently as shown in Figure 1. Since the active TCI state can be dynamically changed, it is necessary to consider dynamic update of periodicities for some SSBs.

Figure 1: SSBs with different usages

Since the SSBs are shared by multiple UEs, it is better to dynamically update the periodicities for SSBs by a group-cast signal, e.g., group-cast DCI. With regard to the idle mode UEs that need to decode MIB, it is better to update the SSB periodicity based on the MIB periodicity. Then, the UEs can update the periodicity for the SSBs at the next MIB periodicity, e.g., 80ms, after receiving the group-cast DCI.

Proposal 1: Support to update the periodicity for SSBs by group-cast DCI

## RLM/BFD based on updated periodicity

After update of the periodicity of SSB for RLM/BFD, UE may further update the RLM/BFD interval. If the NW increases the periodicity of SSB for RLM/BFD, to update the RLM/BFD interval may increase the latency for RLM/BFD. Therefore, one possible way is to update the RLM/BFD RS based on a CSI-RS with the same periodicity as previous periodicity for the SSB. Therefore, it is necessary to support a MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

Proposal 2: Support MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

## Rate matching based on updated periodicity

For UE-dedicated signals, the rate matching can be based on the SSB with the updated periodicity. For non-UE dedicated signals, since legacy UE cannot receive the periodicity update signal, the rate matching pattern can be based on the SSB periodicity configured by RRC signaling as shown in Figure 2.

Figure 2: Rate matching pattern for different type of signals

Proposal 3: For UE-dedicated signal, the rate matching pattern should be based on the updated periodicity for the SSB.

Proposal 4: For UE-dedicated signal, the rate matching pattern should be based on the initial periodicity for the SSB configured by RRC.

## Collision handling between SSB and PRACH

Since the RO validation is based on the RRC configured SSB pattern, if the SSB configuration is dynamically changed, some valid RO may collide with SSB as shown in Figure 3. Then it is necessary to define certain collision handling rule between SSB and valid RO. One possible way is to determine such valid RO as unavailable RO for PRACH transmission. Then the UE can select the next available valid RO for PRACH transmission.

Figure 3: SSB and RO collision due to SSB time-domain adaptation

Proposal 5: With regard to SSB time-domain adaptation, a valid RO that overlaps with SSB should be considered as an unavailable RO

- UE transmits the PRACH on the available valid RO

# Adaptation of PRACH

## PDCCH ordered PRACH

For PRACH time-domain adaptation, it has been agreed that additional PRACH resources for NES are configured and separate SSB-RO mapping for the additional PRACH resources for NES are supported as follows. Then for the PDCCH ordered PRACH, it is necessary to introduce an indicator indicating whether the triggered PRACH is based on legacy PRACH resources or additional PRACH resources for NES.

Agreement For adaptation of PRACH in time-domain, support the following: - SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any) - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources - FFS: SSB-RO mapping for the additional PRACH resources

Proposal 6: Support to introduce one DCI field for the PDCCH used to trigger PRACH indicating whether the triggered PRACH is from the legacy PRACH resources or the additional PRACH resources for NES.

## Configuration for the additional PRACH resources for NES

For PRACH time-domain adaptation, the following on the PRACH configuration has been agreed.

Agreement For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain, - For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211) - For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s), - Opt 1-2a: up to N1 additional value(s) of (x, y) - FFS: value of N1(>=1) - Opt 1-2b: up to N2 additional value(s) of y - FFS: value of N2 (>=1) - Opt 1-3: Muting/masking ROs - Opt 1-4: - up to N4_1 additional timing offset(s) at frame-level - up to N4_2 additional timing offset(s) at slot-level - FFS: values of N4_1(>=1) and N4_2(>=1) - Opt 1-5: No additional mechanism is selected. - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

For Alt1 (same PRACH configuration index), one possible way is to configure multiple y value, e.g., a value set Y, then the NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y. Therefore, option 1-2a should be supported.

For Alt2 (different PRACH configuration index), to configure multiple values of (x, y) can also help to create more the orthogonal ROs. However, some collision cannot be avoided. Therefore, on top of option 1-2a, option 1-3 (muting/masking ROs) should be supported.

Proposal 7: Support the followings for PRACH configuration for the additional PRACH resources

- For Alt1 (Same PRACH configuration index) and Alt2 (different PRACH configuration index), support opt 1-2a, i.e., to configure multiple y value, e.g., a value set Y, for additional PRACH resources for NES

  - NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y

- For Alt2, support Opt 1-3 (muting/masking ROs) additionally

## RO selection

Since the NW can configure two sets of PRACH resources – additional PRACH resources for NES and legacy PRACH resources. Then how to select the RO for initial transmission and retransmission could be one open issue.

For CFRA, the NW can configure whether UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

For CBRA, to reduce the probability of collision, one simple way is that the NES-capable UEs select the additional PRACH resource and other UEs select the legacy PRACH resources. However, as the number of NES-capable UE increases, such simple way could lead to high probability of collision for NES-capable UEs. Therefore, one possible way is to select the PRACH resources based on the measured RSRP. If the NES-capable UEs measures a RSRP for an SSB that is higher than a threshold, it can select the RO from the additional PRACH resources, otherwise it selects the RO from the legacy PRACH resources.

For retransmission of CBRA, one simple way is that the UE selects the RO from the same set of PRACH resources. This could simplify the power ramping procedure. However, if the UE can select the RO from a different set of PRACH resources, the probability of collision for retransmission can be reduced. Then how to perform the power ramping when the UE performs retransmission by RO from a different set of PRACH resources could be studied.

Proposal 8: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 9: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

- If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Proposal 10: For retransmission of CBRA, support the UE to select the RO either from the additional PRACH resources for NES or legacy PRACH resources

- FFS: how to perform the power ramping for the retransmission of PRACH

## Interaction with SBFD

For SBFD, the NW can also configure additional PRACH resources for SBFD symbols/slots. Then for PRACH adaptation for NES, it should be clarified that whether the additional PRACH resources can be applicable to SBFD symbols/slots. One possible way is to configure two additional PRACH resource sets for NES, where one is not applicable to the SBFD symbols/slots and the other one is applicable to the SBFD symbols/slots, with regard to UEs that support NES only and UEs that support both SBFD and NES.

Proposal 11: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is not applicable to the SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots

# Adaptation of PO

## SSB-specific PO

Currently one PO includes the PDCCH MOs associated all the actually transmitted SSBs. However, since the load for different beams can be different, it is unnecessary to include all the PDCCH MOs within one PO. With regard to NES, one possible way is to divide the SSBs into several groups, and configures different PO for different SSB groups. There are similar issues for the PEI-O. Thus, the PEI-O can also be configured in a SSB group specific manner. Then as shown in Figure 4, the NW can configure more POs/PEI-Os for SSB group with high load and less POs/PEI-Os for SSB group with low load.

Figure 4: SSB group specific PO/PEI-O configuration

Proposal 12: Support SSB-group-specific PO/PEI-O configuration

- One PO/PEI-O may include the MOs associated one group of SSBs

## PO activation/deactivation

With regard to NES, the NE may dynamically activate/de-activate some POs or shift some POs. The NW may update the MOs corresponding to one PO. For example, the NW may update the first MO for a PO. Similarly, the NW can also activate/deactivate some PEI-Os or updates the MOs corresponding to one PEI-O. The NW can transmit the dynamic update of PO/PEI-O configuration by L1/L2 signaling.

Proposal 13: Support dynamic update of PO/PEI-O configuration, including

- Activation/deactivation of PO/PEI-O

- Activation/deactivation of MOs of a PO/PEI-O

# Conclusion

In this contribution, we provided discussion on adaptation of common signals. Based on the discussion, the following proposals are provided.

SSB time-domain adaptation

Proposal 1: Support to update the periodicity for SSBs by group-cast DCI

Proposal 2: Support MAC CE based RLM/BFD RS configuration with regard to dynamic periodicity update of SSB.

Proposal 3: For UE-dedicated signal, the rate matching pattern should be based on the updated periodicity for the SSB.

Proposal 4: For UE-dedicated signal, the rate matching pattern should be based on the initial periodicity for the SSB configured by RRC.

Proposal 5: With regard to SSB time-domain adaptation, a valid RO that overlaps with SSB should be considered as an unavailable RO

- UE transmits the PRACH on the available valid RO

PRACH time-domain adaptation

Proposal 6: Support to introduce one DCI field for the PDCCH used to trigger PRACH indicating whether the triggered PRACH is from the legacy PRACH resources or the additional PRACH resources for NES.

Proposal 7: Support the followings for PRACH configuration for the additional PRACH resources

- For Alt1 (Same PRACH configuration index) and Alt2 (different PRACH configuration index), support opt 1-2a, i.e., to configure multiple y value, e.g., a value set Y, for additional PRACH resources for NES

  - NW and UE can determine PRACH can be present in frame nf if it can meet the condition of nf mod x ∈Y

- For Alt2, support Opt 1-3 (muting/masking ROs) additionally

Proposal 8: For CFRA, support the NW to configure whether the UE should select the RO from additional PRACH resources for NES or legacy PRACH resources.

Proposal 9: For initial transmission of CBRA, support the UE to select the PRACH resource set based on the measured RSRP

- If the measured RSRP is above a threshold, UE selects the RO from the additional PRACH resources for NES; otherwise, UE selects the RO from legacy PRACH resources

Proposal 10: For retransmission of CBRA, support the UE to select the RO either from the additional PRACH resources for NES or legacy PRACH resources

- FFS: how to perform the power ramping for the retransmission of PRACH

Proposal 11: With regard to UEs that support NES only and UEs that support both SBFD and NES, support to configure two additional PRACH resource sets for NES

- The first set is not applicable to the SBFD symbols/slots and the second set is applicable to the SBFD symbols/slots

PO/PEI-O adaptation

Proposal 12: Support SSB-group-specific PO/PEI-O configuration

- One PO/PEI-O may include the MOs associated one group of SSBs

Proposal 13: Support dynamic update of PO/PEI-O configuration, including

- Activation/deactivation of PO/PEI-O

- Activation/deactivation of MOs of a PO/PEI-O