3GPP TSG RAN WG1 #120										R1-2500593

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.2.4

Source:	NEC

Title: 	Discussion on enhancement for asymmetric DL sTRP and UL mTRP scenarios

Document for:	Discussion and Decision

- Introduction

NR MIMO evolution Rel-19 is going to support asymmetric DL sTRP/UL mTRP deployment scenarios and to extend Rel-18 2TA for multi-DCI-based mTRP [1]. In this scenario, some TRP may reduce or turn off its DL transmission to save power and it may cause issues like pathloss estimation, closed-loop power control and so on. The following agreements have been achieved in the last meeting [2]. In this contribution, we provide our views on enhancements for asymmetric DL sTRP/UL mTRP scenarios.

Agreement Support to apply PL offset on PDCCH-order PRACH in FR2 - The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here. Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19. Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured. Agreement For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA: - The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs. Agreement For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts: - Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement. - Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional. - Alt3: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources. - Alt4: - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17) - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources. - FFS how to define the PL offset and CLPC adjustment state

- Discussion

  - Scenarios and configurations

Note that we don’t have an absolute UL-only cell without any DL transmission and the micro node may only turn off or reduce its DL transmission to save power, it suggests that switching between symmetric DL/UL and asymmetric DL sTRP/UL mTRP may be considered. In Rel-18 NES, cell DTX (discontinuous transmission) is introduced and similar mechanism may be extended as TRP DTX that the micro node may enter DTX mode with MAC CE signalling to notify the UE.

Micro node may not always turn off its DL transmission.

Consider TRP DTX configuration to support switching between symmetric DL/UL and asymmetric DL sTRP/UL mTRP.

  - PL offset calculation and configurations

It is NW’s responsibility to calculate the pathloss offset from UE to different TRPs, since UE may not be able to assess the channel from the TRP with no DL transmission. While as one factor for determining transmit power is pathloss measured by UE, which is unknown to the NW, to make it possible for NW measuring on the pathloss, one way is that NW may configure SRS transmission with the same transmit power to different TRPs and calculate the pathloss offset by comparing the receive power, another way is that UE may provide the information of transmit power to NW.

PL offset can be calculated at NW by configuring SRS transmissions to different TRPs with the same Tx power or UE reporting information of transmit power.

In addition, for the node with no DL, the source to determine spatial Tx filter in a separated UL TCI state would only be SRS, and the separate UL TCI state can be regarded for UL only if it’s configured with PL offset value(s).

The separate UL TCI state configured with PL offset value(s) can be regarded for UL only, and the source RS in the separate UL TCI state should only be SRS resource.

It could be NW implementation to determine when the pathloss offset value needs update, for example, via triggering SRS transmission for pathloss offset estimation and followed by pathloss offset indication or update. On the other hand, considering that UE movement and the change that UE can detect via the measurement of the PL RS configured, it is also possible for UE to request the pathloss offset update, for example, when the measurement pathloss is becoming smaller, which suggests that UE may move towards the macro node with DL PL RS transmission and the configured PL offset value may not be suitable anymore.

Support UE request to update the configured pathloss offset value.

It was agreed that a new MAC CE would be introduced to update the PL offset value(s) for joint or UL TCI state(s). Further discussions are needed on the content of the MAC CE and the detailed mechanism for updating. There could be multiple options, for example, the MAC CE only provide one value, or multiple values, the MAC CE can update the PL offset values for all configured TCI states, or only activated TCI states, the value can be an absolute value, or an accumulative value, and so on. For simplicity, it is preferred that the MAC CE provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

Support the PL offset value update MAC CE to provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

This new MAC CE would cause the UL power change in the end, together with other power control factors. In particular, NW can use both TPC and this new MAC CE to control the UL power. When NW has to signal this new MAC CE, it considers the PL difference change between UE to two TRPs. In this case, the previous TPC command may be not valid anymore because a more significant part, i.e., PL, is changed and UE needs to reset the TPC accumulation as 0. For example, when a PL offset update MAC CE is received or applied, the UE resets accumulation of a PUSCH power control adjustment state 𝑙 for active UL BWP 𝑏 of carrier 𝑓 of serving cell 𝑐 to fb,f,ck,l=0, k=0,1,…,i.

Support UE to reset TPC accumulation as 0 upon reception of the PL offset value update MAC CE

Meanwhile, it could be NW implementation to determine the value of update, which may not be very accurate. If the value is too large, it may suggest that UE relative positions to two TRPs change a lot, e.g., the UE is moving much closer to the micro node or the macro node, compared to the positions when the PL offset value is configured originally. In this case, it is useful to trigger PHR report to let the NW know more timely information about UE transmit power.

PHR is triggered when the PL offset value update MAC CE provides a PL offset variation larger than a threshold.

- Conclusion

In this contribution, we provided our views on Rel-19 MIMO work on enhancements for asymmetric DL sTRP/UL mTRP scenarios, and we have the following observations and proposals:

Observation 1:	Micro node may not always turn off its DL transmission.

Proposal 1:	Consider TRP DTX configuration to support switching between symmetric DL/UL and asymmetric DL sTRP/UL mTRP.

Proposal 2:	PL offset can be calculated at NW by configuring SRS transmissions to different TRPs with the same Tx power or UE reporting information of transmit power.

Proposal 3:	The separate UL TCI state configured with PL offset value(s) can be regarded for UL only, and the source RS in the separate UL TCI state should only be SRS resource.

Proposal 4:	Support UE request to update the configured pathloss offset value.

Proposal 5:	Support the PL offset value update MAC CE to provide one accumulative value which is applied to the configured/applied PL offset values for all configured TCI states.

Proposal 6:	Support UE to reset TPC accumulation as 0 upon reception of the PL offset value update MAC CE

Proposal 7:	PHR is triggered when the PL offset value update MAC CE provides a PL offset variation larger than a threshold.

- References

- RP-242394, WID revision: NR MIMO Phase 5

- Chair’s Notes, 3GPP TSG RAN WG1 #119, Orlando, US, November 18th – 22nd, 2024