3GPP TSG RAN WG1#120	        	R1-2500654

Athens, Greece, February 17th – 21st, 2025

Agenda Item:					    9.5.2

Source:	Sony

Title:	On-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion

Introduction

According to the revised WID for Rel-19 enhancements of NES [1], procedures and signaling methods to support on-demand SIB1 for UEs in idle/inactive mode would be specified.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3]

  - Specify procedures and signaling method(s) for Case 2 [RAN1/2]

    - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

    - Triggering method by UL WUS using PRACH

  - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3]

  - Note 1: No modification of SSB will be discussed under this objective

  - Note 2:

    - UL WUS: Uplink wake-up signal

    - Cell A: A cell that is periodically transmitting at least its own SIB1

    - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE

  - Note 3:

    - RAN1 strives to minimize impact to legacy UE

    - RAN1 specification impact to support this feature should be minimized

In this contribution, our considerations on on-demand SIB1 are made.

Discussion

Identification of NES cell

Agreement (RAN1#116bis meeting @Changsha)

RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options:

- Option 1: By WUS configuration

- Option 2: By PBCH payload of NES cell

Agreement (RAN1#118bis meeting @Hefei)

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

- SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

- UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

Agreement (RAN1#119 meeting @Orlando)

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options:

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

In RAN1#116bis meeting [4], RAN1#118bis meeting [3], and in RAN1#119 meeting [2], the following agreements were made.

Fig. 1 shows a block diagram of the procedure for a Rel-19 NES capable UE to obtain OD-SIB1 as we envision it, and Fig. 2 shows a block diagram of the procedure for a legacy UE in the same scenario. Depending on whether a UE firstly detects the SSB of the NES cell or Cell A, the following two scenarios can be considered.

Scenario 1: If the SSB which is firstly detected by the UE through initial cell search is the SSB of the NES cell, the following three cases can be further considered:

- Case 1-1: If the K_SSB of the NES cell's SSB is ≤ 23 for FR1and ≤ 11 for FR2, as the SIB1 of the NES cell has already been transmitted (i.e., OD-SIB1 has already started transmission), the UE can obtain SIB1 transmitted from the NES cell by referring to the pdcch-ConfigSIB1 field in MIB of the NES cell.

- Case 1-2: If the K_SSB of the NES cell's SSB is < 30 and > 23 for FR1 and < 14 and > 11 for FR2, the pdcch-ConfigSIB1 field is used for the indication of GSCN (Global Synchronization Channel Number), and the UE searches for other CD-SSBs of the NES cell by referring to this GSCN value. If another detected SSB indicated by this GSCN value is an SSB with K_SSB ≤ 23 for FR1 and ≤ 11 for FR2 (a.k.a. CD-SSB), the procedure goes to case 1-1, while if another detected SSB indicated by this GSCN value is an SSB with K_SSB = 30 for FR1 and K_SSB = 14 for FR2, the procedure goes to case 1-3.

- Case 1-3: If the K_SSB of the NES cell's SSB is equal to 30 for FR1 and 14 for FR2, since ‘No SIB1’ is notified to the UE and the pdcch-ConfigSIB1 field is indicated as a reserved state, the UE performs cell re-selection and searches for the SSB of another cell.

  - For the Rel-19 NES capable UE, to help the UE to access Cell A as quickly as possible, the reserved state of pdcch-ConfigSIB1 field could be used to indicate frequency assistance information for detecting the SSB of Cell A (e.g., the frequency location of Cell A's SSB). Without the assistance information for Cell A, the UE has to blindly search for the SSB of Cell A, which causes more access latency as well as UE energy consumption. Furthermore, to indicate the frequency assistance information for Cell A, the Rel-19 NES capable UE needs to identify that the detected cell is an NES cell, and this identification can be performed with K_SSB = 30 for FR1 and K_SSB = 14 for FR2.

  - The legacy UE performs cell re-selection and searches for SSB of another cell without the frequency assistance information.

Scenario 2: If the UE detects the SSB of Cell A through initial cell search or cell re-selection, the following two cases can be further considered:

- Case 2-1: If the UE firstly detects the SSB of Cell A through initial cell search, the UE performs initial access to camp on Cell A because the Cell A provides the SSB with the best RSRP.

- Case 2-2: If the UE detects the SSB of Cell A through cell re-selection after detection of the NES cell,

  - The Rel-19 NES capable UE attempts to obtain the UL WUS configuration to request the OD-SIB1 of the NES cell from Cell A.

  - The legacy UE performs initial access to camp on Cell A.

In our view, since these two scenarios for NES cell identification can be considered, NES cell identification should be performed using K_SSB (K_SSB = 30 for FR1 and K_SSB = 14 for FR2) in addition to UL WUS configuration.

In the case where the K_SSB of the NES cell's SSB is equal to 30 for FR1 and 14 for FR2, the subsequent action of the UE, after obtaining the MIB of the NES cell,  is to obtain the UL WUS configuration of OD-SIB1 from Cell A. Utilizing the pdcch-ConfigSIB1 field to indicate frequency assistance information enables a reduction in access latency as well as power consumption of the UE compared to using it for indicating searchSpaceZero and controlResourceSetZero of OD-SIB1 (Option 2) or leaving it as a reserved value (Option 3). Therefore, in our view, the pdcch-ConfigSIB1 field should be used to indicate frequency assistance information for detecting the SSB of Cell A, and the searchSpaceZero and controlResourceSetZero of OD-SIB1 can be indicated through the UL WUS configuration.

Additionally, in the case of indicating frequency assistance information in Option 1, we consider two possible methods for such indication:

- Option 1-1: Indicate the frequency location of the SSB of Cell A.

- Option 1-2: Indicate a range of frequency location that includes the frequency location of the SSB for Cell A.

Fig. 3 shows the example of the values indicated by the NES cell and the behavior of the UE in Option 1-1 and Option 1-2.

For instance, the legacy frequency assistance information, GSCN, is represented by combining the pdcch-ConfigSIB1 field and the value of K_SSB. In the case of Option 1-1, the number of indicatable frequency locations is smaller compared to legacy method, being 1/6 in FR1 and 1/2 in FR2. When attempting to indicate the frequency location of SSB of Cell A using the pdcch-ConfigSIB1 field, it may not be possible to cover the entire bandwidth depending on the band being used. Furthermore, if the SSB of Cell A is transmitted within such a band, the UE will need to blindly search for the SSB of Cell A, which prevents the benefits of applying Option 1.

On the other hand, in the case of Option 1-2, by indicating a range of frequency location that includes frequency locations with a granularity larger than GSCN, the number of values required to be indicated to cover the bandwidth is smaller compared to legacy GSCN. As a result, it is sufficient to indicate the frequency assisted information using only the pdcch-ConfigSIB1 field. However, compared to Option 1-1, UE needs to perform a blind search within the indicated range of frequency location. Nevertheless, compared to Option 2 or Option 3, which do not indicate frequency assistance information, the range of frequencies for blind search can be narrowed, thereby reducing access latency and UE power consumption.

Fig. 1 Block diagram for Rel-19 NES capable UE to obtain OD-SIB1

Fig.2 Block diagram for legacy UE

Fig.3 Example of the values indicated by the NES cell and the behavior of the UE

Proposal 1: For indication of NES cell with on-demand SIB1, K_SSB (K_SSB = 30 for FR1 and K_SSB = 14 for FR2) should be supported in addition to UL WUS configuration.

Proposal 2: If SSB on NES cell has the value of K_SSB = 30 in FR1 or K_SSB = 14 in FR2, frequency assistance information which indicates a frequency range that includes the frequency location of the SSB for Cell A should be indicated by using pdcch-ConfigSIB1 field.

On-demand SIB1 monitoring occasions

Agreement (RAN1#118 meeting @Maastricht)

At least for Case-2: For further work on type 0 PDCCH monitoring occasions for on demand SIB1, on the starting time and duration of the time window of type 0 PDCCH monitoring occasions, RAN1 to down select from the following two options:

- Option 1: starting time and duration are indicated in RAR of the UL-WUS transmission

- Option 2: starting time and duration are indicated in the UL WUS configuration

Agreement (RAN1#119 meeting @Orlando)

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

In RAN1#118 meeting [5] and RAN1#119 meeting [2], the following agreements for further study on the on-demand SIB1 monitoring occasions were made.

In RAN1#119 meeting, it was agreed that the duration of the time window for on-demand SIB1 would be indicated in the UL WUS configuration, while the method for indication of the starting time was left as FFS. The difference between indicating the starting time in the RAR of UL WUS transmission and indicating it in the UL WUS configuration lies in the flexibility of the time gap between the reference time point used to determine the starting time of the SIB1 monitoring window and the starting time of the window.

If the starting time for the SIB1 monitoring window is indicated to the UE in the UL WUS configuration, the gNB indicates a fixed time gap to all UEs because the gNB must indicate the time gap before receiving the SIB1 transmission request from each UE. If the gNB starts transmitting on-demand SIB1 immediately after transmitting the RAR of the UL WUS transmission to each UE, each UE can receive on-demand SIB1 in a short monitoring window. However, it is not energy efficient for the network to transmit on-demand SIB1 dedicatedly for each UE in response to the UL WUS reception from each UE, compared to broadcasting on-demand SIB1 to all UEs that requested it at the same time. The same applies even when the starting time of the SIB1 monitoring window is pre-determined.

On the other hand, if the starting time is indicated to the UE in the RAR of the UL WUS transmission, the gNB can indicate different time gaps to each UE that transmitted UL WUS after receiving each SIB1 transmission request. If the gNB coordinates the starting time of the monitoring window for each UE, the gNB can efficiently transmit on-demand SIB1 at the same transmission occasion to each UE, and each UE can receive the on-demand SIB1 without unnecessary extension of the SIB1 monitoring window.

Proposal 3: The starting time for SIB1 monitoring should be indicated by the RAR of the UL WUS.

How to transmit on-demand SIB1

FL Proposal 5-5-2

RAN1 assumes the transmission time of the (updated) on-demand SIB1 on the NES cell should be restricted into one time window when the NES cell is not periodically broadcasting SIB1.

- Note:

  - As agreed in RAN1 #117, there can be one or more type 0 PDCCH monitoring occasions for on demand SIB1 within the time window.

  - In RAN1 #127-bis, it is agreed that

    - A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

In the feature lead summary in RAN1#119 [6], it was discussed about the transmission time of the on-demand SIB1 on the NES cell as follows.

Agreement

At least for Case-2: For further study of type 0 PDCCH monitoring occasions for on demand SIB1, after UE transmits the UL WUS in idle/inactive mode, RAN1 assumes following as a starting point:

- Option 1: One or more type 0 PDCCH monitoring occasions for on demand SIB1 within a time window

  - FFS: How the search space zero configuration is provided (e.g. from  searchSpaceZero  in  MIB  or  from a new search space that is indicated by UL-WUS configuration)

  - FFS: Details of the time window, including at least the starting time and duration

  - FFS: Whether/how to support transmission of on-demand SIB1 with the association with SSB(s) based on a received UL-WUS

In addition, the following agreement was made in RAN1#117 [7].

Based on the agreement at RAN1#117, one or more on-demand SIB1 are transmitted within a time window. If on-demand SIB1 is transmitted across multiple time windows, the motivation might be to ensure a longer transmission time window. However, even if a longer transmission time window is necessary, it would be sufficient to extend the duration of the transmission time window. To avoid unnecessary complexity in indication and transmission methods introduced by using multiple transmission time windows, it is preferable for the NES cell to restrict the transmission time of on-demand SIB1 into one time window.

Proposal 4: The transmission time of the on-demand SIB1 on the NES cell should be restricted into one time window.

Conclusions

The following proposals were made in this contribution.

Proposal 1: For indication of NES cell with on-demand SIB1, K_SSB (K_SSB = 30 for FR1 and K_SSB = 14 for FR2) should be supported in addition to UL WUS configuration.

Proposal 2: If SSB on NES cell has the value of K_SSB = 30 in FR1 or K_SSB = 14 in FR2, frequency assistance information which indicates a frequency range that includes the frequency location of the SSB for Cell A should be indicated by using pdcch-ConfigSIB1 field.

Proposal 3: The starting time for SIB1 monitoring should be indicated by the RAR of the UL WUS.

Proposal 4: The transmission time of the on-demand SIB1 on the NES cell should be restricted into one time window.

References

- RP-241650, “Revised WID: Enhancements of network energy savings for NR,” 3GPP TSG RAN Meeting #104, June 2024.

- RAN1#119 meeting chairman’s note

- RAN1#118bis meeting chairman’s note

- RAN1#116bis meeting chairman’s note

- RAN1#118 meeting chairman’s note

- R1-2410680, MediaTek, “	FL summary 4 for on-demand SIB1 in idle/inactive mode	,” RAN1#119 meeting, Oct. 2024.

- RAN1#117 meeting chairman’s note