3GPP TSG RAN WG1 Meeting #120		R1-2500663

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	9.10.1

Source: 	Sony

Title: 	Discussion on enabling TX/RX for XR during RRM measurements

Document for: 	Discussion & Decision

# Introduction

The work item (WI) on eXtended Reality (XR) for NR phase 3 was revised in RANP#105 [1]. The WI is basically based on the outcome of of the previous study item phase [2]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited. The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In RAN1#119 meeting, the following agreements were made [3]:

Agreement For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”. - FFS: Interpretation of bit value equal to “0”. A component of an FG can have Option 1 as a first value and Option 2 as a second value: - Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI. - Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour. Note: UE indicates only one value of this component. For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3. - Explicit indication can be configured for each DCI format individually by higher layer. - The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field) Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

This contribution provides our views on enabling transmission/reception in measurement gaps / scheduling restrictions period to support XR application.

# Discussion

To support XR application, the UE may require continuing the transmission and skip the RRM measurement. Several proposals / solutions have been proposed in the last few meetings. In RAN1#118, it was agreed as a working assumption on the solution based on the dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is provided from gNB to UE via downlink control information (DCI) carried out by PDCCH. This approach is illustrated in Figure 2.

Figure 1: Illustration of Skip / Release Measurement to support data Tx/Rx

The skip measurement here means the UE can continue CG transmission during the affected measurement gap. The other solution is to provide the indication semi-statically. We consider this solution may not be able to support XR characteristics. The gNB may not be known the UE traffic far in advance. Hence, the working assumption to support the dynamic indication made in RAN1#118 as shown below should be confirmed.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Proposal 1: Confirm the working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

The dynamic indication by DCI to skip a particular gap/restriction is carried out within DCI formats 0_1/1_1, DCI formats 0_2/1_2, and DCI formats 0_3/1_3. As agreed earlier, one-bit bit-field size in each DCI format is required to support this operation (i.e. enable Tx/Rx during gap/restriction) . We consider the gNB may have a flexibility to enable whether a specific DCI format may carry such information. In this case, it requires a new high layer parameter (e.g., RRC signalling) to enable that one-bit information in each DCI format.

Proposal 2: Support higher layer signalling (i.e., RRC signalling) to enable one-bit information (to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements )in each DCI format.

There may be further new RRC parameters pending to the progress of other WGs (e.g., RAN2, and RAN4). For example on UE Assistance information (UAI) aspect and partial RRM measurement aspect. In RAN1#117, RAN1 could not reach consensus to support UE assistance information (UAI) related to measurements occasions. The reason is due to lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). Hence, RAN1 sends an LS to RAN4 requesting their views on UAI [5]. Here, we express our observation related to UAI.

For the case of uplink transmission, UE may have better information than gNB in terms of the uplink transmission characteristics. The UE has direct interaction with the upper layer (e.g., application layer) so that the UE can predict the upcoming uplink transmission characteristics. The uplink transmission can be in a form of an aperiodic transmission that require spontaneous transmission from the UE. It can also be in a form of semi-static / periodic transmission. Furthermore, the UE may have various types of uplink transmission. Some uplink transmission can be compromised, such that the legacy gap/restriction can still be applied. In this case, the UE can prioritize the measurement over the transmission. Some other uplink transmission (e.g., XR traffic for video, control, data) has higher priority over RRM measurement. In this case, the UE skips RRM measurement and performs UL transmission instead. Based on these reasonings, UE assistance information to serving gNB on indicating an upcoming temporary measurement gap modification (e.g., skipping) shall be supported. The gNB can take that information into account and decide whether to grant the temporary measurement gap modification or not. This mechanism would also ensure that the gNB is still in full control of the UE behaviour. Hence, any potential system performance degradation (e.g., due to the measurement gap skipping) can be avoided.

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 4: Support UE Assistance Information (UAI) indicating the gNB on the MG(s) to be skipped by the UE.

In case of the RRM measurement is affected by the temporary measurement gap modification, for example when one or more of the measurement gaps has been skipped, then the UE can also indicate that the reported RRM measurement has been affected. Skipping one or more measurement gaps is expected to reduce the quality of the reported RRM measurement from the UE, such as reducing the mobility performance. Alternatively, the UE is actually postponing the measurement. Hence, it may affect the latency in reporting the RRM measurement. This may require further study in other WG(s), such as RAN2 and RAN4.

Observation 3: Skipping RRM measurement may affect the quality and/or latency of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Proposal 4: Pending to the progress of other WGs, there may be further RRC signalling related to UAI and partial RRM measurement. If time permits, RAN1 can continue to work on these aspects.

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The observations and proposals are listed below.

Observation 1: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 2: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Observation 3: Skipping RRM measurement may affect the quality and/or latency of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Proposal 1: Confirm the working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

Proposal 2: Support higher layer signalling (i.e., RRC signalling) to enable one-bit information (to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements )in each DCI format.

Proposal 4: Support UE Assistance Information (UAI) indicating the gNB on the MG(s) to be skipped by the UE.

Proposal 4: Pending to the progress of other WGs, there may be further RRC signalling related to UAI and partial RRM measurement. If time permits, RAN1 can continue to work on these aspects.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- TR 38.838, “Study on XR (Extended Reality) Evaluations for NR”, v17.0.0, December 2021

- RAN1 #118bis Chairman’s notes, Hefei, China, October 2024

- R1-2409306 “Moderator summary #4 - Enabling TX/RX for XR during RRM measurements”, Nokia

- R1-2405736 “LS on UE assistance information related to measurement occasions”, Nokia