3GPP TSG-RAN WG1 Meeting #119		R1-2500862

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.8.1

Title:	Discussion on channel model validation of TR38.901 for 7-24 GHz

Source:	Samsung

Document for:	Discussion and Decision

- Introduction

The Rel-19 study items on “Study on channel modelling enhancements for 7 – 24 GHz for NR” is endorsed in RAN#102 [1]. The objectives for this SI are shown below:

The objectives of this study are: - Validate using measurements the channel model of TR38.901 at least for 7-24 GHz - Note: Only stochastic channel model is considered for the validation. - Note: The validation may consider all existing scenarios: UMi-street canyon, UMa, Indoor-Office, RMa and Indoor-Factory. - Adapt/extend as necessary the channel model of TR38.901 at least for 7-24 GHz, including at least the following aspects for applicable scenarios: - Near-field propagation (with consideration being given to consistency between near-field and far-field) - Spatial non-stationarity Note 1: Continuity of the channel model in the frequency domain below 7 GHz and above 24 GHz shall be ensured. Note 2: Mathematical and/or theoretical aspects (if any) may be studied before results of measurement campaigns are available. While measurement results may be available and submitted at any time, the study of measurement results may start later (e.g., Q3 2024).

In this contribution, we discuss channel mode validation of TR 38.901 [2] related aspects for Rel-19 channel model SI.

- Initial measurement results and observations

2.1 Measurement set-up and scenario

| (a) | (b) |
| --- | --- |

Figure 1. Measurement set-up and scenario: (a) Tx positioned atop a building; (b) Measurement scenario

Continuing from the previous our contirubtion [3], an outdoor measurement campaign was conducted. Figure 1 shows the measurement setup and scenario. The measurement was performed with a frequency-domain channel sounder, which was built based on a Vector Network Analyzer (VNA). As mentioned earlier, the VNA covers the frequency band of interest, 6 - 14 GHz. As shown in Figure 1(a), a horn antenna operating at 6 - 18 GHz was used as the Tx antenna, and the same antenna was used for Rx. As shown in Figure 1(b), the Tx is located above 20 meters on the rooftop of the building (red dot in the figure), and the Rx is 1.7 meters height and moved along the road located in front of the Tx (blue dots and yellow lines in the figure). The overall measurement environment is similar with urban micro (UMi) scenario in TR38.901 taking into account a mix of buildings, streets and open spaces. Rx is located whith can be ensured with a clear LOS environment at several points close to Tx and measurements were performed in an obstructed LOS environments caued by foilage and buidings for most other points.

Measurements were taken by mechanically scanning the Tx and Rx at regular angles to collect multipath components (MPCs) from all directions. Specifically, the azimuth of the Tx scanned a 120° area and ranged from -60° to 60° with a 10° azimuthal resolution. The Rx performed a complete scan ranging from 0° to 360° with a 10° azimuthal resolution. In addition, for more accurate measurements, the Rx additionally performed an elevation scan ranging from -20° to 20° with a 10° resolution.

2.2 Initial measurement results

In this contribution, measurements were conducted at both 6.5 GHz and 13.5 GHz. The collected channel frequency response is converted into the response in delay domain through inverse FFT, which can be represented as a direction power delay profile (PDP). Subsequently, the collected PDPs underwent postprocessing according to each channel parameter. For measurement result analysis, the dynamic range was selected based on the result from the furthest measured point at each frequency band, which was then applied to all PDPs to derive the channel parameters.

2.2.1 Pathloss

Figure 2 presents the measured pathloss results at different frequency bands. For comparison, the LOS and NLOS pathloss of the UMi scenario in TR38.901 are overlaid. As mentioned before, most measurement points except for some points were located in obstructed LOS or NLOS environments due to vegetation and buildings. Considering UMi NLOS as the upper bound of pathloss and UMi LOS as the lower bound and measurement environments, the measurement results have marginal differences. Thus, it can be adequately covered by the existing equations in TR38.901.

| (a) | (b) |
| --- | --- |

Figure 2. Pathloss: (a) 6.5 GHz and (b) 13.5 GHz

The marginal difference compared to pathloss for UMi scenario in existing channel model is confirmed

RAN1 discuss whether the updates of pathloss in UMi scenario is needed

2.2.2 Delay spread

Figure 3 presents the measured results for delay spread at 6.5 GHz and 13.5 GHz. The measurement environment was mostly LOS and for comparison, the delay spread values in the TR38.901 UMi LOS scenario were referenced. According to the measurement results, the mean delay spread in the LOS environment was found to be lower compared to the existing channel model parameter. In particular, a lower delay spread was observed in relatively clear LOS environments (e.g., below the 70%-tile CDF). On the other hand, in LOS environments where reflectors such as buildings are present around the Rx, a higher delay spread was observed. Furthermore, when comparing the measurement results at 6.5 GHz and 13.5 GHz, the mean delay spread in the 13.5 GHz band tends to be slightly lower than that in the 6.5 GHz. As reviewed above, even in a visually LOS environment between Tx and Rx, the delay spread can vary depending on the surrounding environment. In particular, if an update of delay spread in the LOS environment is required, it will be necessary to collect and analyze sufficient measurement data from various LOS environment to reflect actual conditions.

| (a) | (b) |
| --- | --- |

Figure 3. Delay spread: (a) 6.5 GHz and (b) 13.5 GHz

As shown in below table, in the UMi scenario, the mean delay spread in the LOS environment was found to be lower compared to the existing channel model parameter

| Delay spread | UMi_LOS (TR38.901) @ 6.5 GHz | Measured DS @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured DS @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μDS (log_10⁡〖DS/1s〗 ) | -7.35 | -8.16 | -7.41 | -8.19 |
| (ns) | 44.67 | 6.91 | 38.13 | 6.32 |
| σDS | 0.38 | 0.83 | 0.38 | 0.91 |

Even in a visually LOS environment between Tx and Rx, the delay spread can vary depending on the surrounding environment. In particular, if an update of delay spread in the LOS environment is required, it will be necessary to collect and analyze sufficient measurement data from various LOS environment to reflect actual conditions

RAN1 discuss whether the updates of delay spread in UMi LOS scenario is needed

2.2.3 Angular spread

Figure 4 and 5 shows the measured results of azimuth spread of departure angles and arrival angles respectively through CDF. For comparison, the LOS UMi channel parameters from TR38.901 are also presented. Regarding Figure 4, at the 6.5 GHz band, the mean value of LOS (UMi) is 14.66 degree, and the mean value of measured value is 11.18 degree. It was observed that the measurement results were slightly lower than the mean value of existing LOS parameters. Moreover, a disparity of variance was also confirmed. At the 13.5 GHz band, the mean value of LOS (UMi) is 14.18 degree and the mean value of measurement result is 9.6 degree, indicating a similar trend to the 6.5 GHz band. Based on the measurement results, slight difference compared to mean and standard deviation of ASD in the existing channel model were confirmed.

| (a) | (b) |
| --- | --- |

Figure 4. Azimuth spread of departure angles: (a) 6.5 GHz and (b) 13.5 GHz

As shown in below table, slight difference compared to mean and standard deviation of ASD for UMi LOS scenario in the existing channel model were confirmed

| Azimuth spread of departure angles | UMi_LOS (TR38.901) @ 6.5 GHz | Measured ASD @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured ASD @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μASD (log_10⁡〖ASD/1°〗 ) | 1.16 | 1.04 | 1.15 | 0.98 |
| (degree) | 14.66 | 11.18 | 14.18 | 9.6 |
| σASD | 0.41 | 0.21 | 0.41 | 0.20 |

Regarding Figure 4, at the 6.5 GHz band, the mean value of LOS (UMi) is 45.71 degree, and the mean value of measured value is 14.26 degree. It was observed that the measurement results were significantly lower than the mean value of existing LOS ASA parameters in UMi scenario. Moreover, a disparity of variance was also confirmed. At the 13.5 GHz band, the mean value of LOS (UMi) is 43.36 degree, and the mean value of measured value is 12.7 degree, indicating a similar trend to the 6.5 GHz band. Based on the measurement results, significant differences compared to mean and standard deviation of ASD in the existing channel model were confirmed.

| (a) | (b) |
| --- | --- |

Figure 5. Azimuth spread of arrival angles: (a) 6.5 GHz and (b) 13.5 GHz

As shown in below table, significant difference compared to mean and standard deviation of ASA for UMi scenario in the existing channel model were confirmed

| Azimuth spread of arrival angles | UMi_LOS (TR38.901) @ 6.5 GHz | Measured ASA @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured ASA @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μASA (log_10⁡〖ASA/1°〗 ) | 1.65 | 1.15 | 1.64 | 1.11 |
| (degree) | 45.71 | 14.26 | 43.36 | 12.7 |
| σASA | 0.29 | 0.28 | 0.29 | 0.33 |

RAN1 discuss whether the updates of azimuth spread of departure/arrival angles in UMi LOS scenario is needed

On the other hand, the weak point of the measurement campaign could be its low EIPR characteristic. Therefore, additional measurement using a channel sounder with antenna array are planned to supplement the data from measurement campaigns using a horn antenna and secure deeper insights. After completing these measurements, it is expected the discussion on the statistical properties of the parameters according to the LOS/NLOS conditions.

2.3 Other consideration

2.3.1 Parameter updates

Conclusion For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19. - Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

In the last meeting, RAN1 conclude to replace existing parameters in TR with updated values/model equation were necessary. This approach allows newly updated values to integrate seamlessly with other values ensuring versatility. However, one challenges lies in maintain continuity with existing channel parameters. Currently available channel parameters reflect stochastic properties of measurement results focused on both Below 6 GHz band around 3.5 GHz and mmWave bands around 28 GHz. New measurements across 7 – 24 GHz band may exhibit nonlinear traits inconsistent with prior values. One method to address this imbalance involves collecting older measurement results to extract fresh statistical results. Therefore, RAN1 need to discuss regarding how to replace channel parameter values/model equations accordingly.

Currently available channel parameters reflect stochastic properties of measurement results focused on both Below 6 GHz band around 3.5 GHz and mmWave bands around 28 GHz. New measurements across 7 – 24 GHz band may exhibit nonlinear traits inconsistent with prior values

RAN1 need to discuss regarding how to replace channel parameter values/model equations (e.g., by aggregating old measurement results and new measurement results, extracting new statistical outcomes)

While examining the frequency dependence of each channel parameter, we had one question in mind. To extract meaningful channel parameters from measurement data, we defined a dynamic range between the maximum power and noise-thresholded values in the PDP. We found that dynamic range varies with the distance depending on factor such as distance between Tx and Rx, as well as the measured frequency band (i.e., it decreases as the distance increase and the frequency band increases.) Among the factor which is impacted to dynamic range, for frequency band, the different dependency trends were observed when applying different corresponding dynamic ranges across each frequency band versus when applying same dynamic ranges cross all frequency band. In this study, the validation is important not only the validity of the channel parameter values but also the validity of the frequency dependent function for channel parameter. Therefore, it would be beneficial to discuss how to properly apply the dynamic range for each frequency band while examining the frequency dependency properties.

When examining frequency dependent properties of channel parameter, dynamic range of the measured data affect to results

RAN1 discuss whether/how to apply dynamic range when it comes to frequency dependent properties for channel parameter

2.3.2 Vegetation effects

The 3GPP channel model defines the channel parameters based on LOS and NLOS conditions. However, we confirmed that it is difficult to clearly differentiate between these two categories due to the presence of vegetation. Even when a visual LOS condition exists between the Tx (BS) and the Rx (UE), most paths can be obstructed by vegetation, resulting in a significant decrease in pathloss compared to an environment with LOS. This phenomenon will serve as an important indicator not only for analyzing path loss but also for small-scale parameters. In this context, clarification regarding how to handle the impact of vegetation is necessary. As starting point for discussion, we would like to propose clarifying whether the path obstruction caused by vegetation falls under LOS or NLOS

Even when a visual LOS condition exists between the Tx (BS) and the Rx (UE), most paths can be obstructed by vegetation, resulting in a significant decrease in path loss compared to an environment with pure LOS

RAN1 discuss whether/how to handle the vegetation impact

2.3.3 Pathloss on point beyond breakpoint

One point we would like to propose to discuss is the LOS pathloss equation. Taking the UMa pathloss equation as an example, there are two equations for before and after the breakpoint in LOS pathloss. Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge. As shown in Figure 6, as distance greater than about 1 km, the pathloss for the 3.5 GHz band converges with other band’s pathlosses.

Certainly, consideration of the LOS probability is crucial while using the current channel model in system level simulation, and this phenomenon might be regarded as insignificant beyond 1 km. However, when analyzing link budget or pure coverage, the pathloss equation can be used. For example, coverage can be analyzed under only LOS conditions. In this case, considering only the LOS condition, accurate analysis becomes difficult beyond a specific point due to the aforementioned problem. Therefore, we would like to suggest a clarification discussion on how RAN1 addresses such situations.

Figure 6. LOS UMa pathloss considering breakpoint

Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge

RAN1 discuss whether/how to handle the phenomena that converges regardless of the frequency band after the breakpoint in LOS pathloss equation

- Other channel parameters

3.1 Suburban Macro scenario

Agreement For suburban scenario, adopt the following assumption for calibration purposes: - Building type ratio: 90% residential, 10% commercial Agreement For suburban scenario, adopt the following assumption for calibration purposes: - UT Vertical Distribution Option 2: uniform distribution across all floors for a building type Agreement For suburban scenario, adopt the following assumption for calibration purposes: - UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h Agreement For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes: - Option 1) BS height = 25m - Option 2) BS height = 35m Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m. Agreement For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes: - Option 1) ISD = 1299 m - Option 2) ISD = 1732 m Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m. Agreement Capture channel modeling updates regarding suburban macrocell scenario as follows: - Define a new scenario, e.g., SMa - The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

RAN1 continues discussion on Suburban Macro (SMa) scenario as new scenario introduced. SMa scenario has already been studied in ITU-R M.2135-1[8]. Referring to the guideline and ongoing discussion in RAN1, major characteristics of SMa compared to UMa are wider area and surrounding building environment with lower heights. Thus, channel parameters also need to be defined reflecting these features well. In this context, remaining issues among channel parameters are BS height and ISD setting. As mentioned above, SMa may show intermediate feature between UMa and RMa. Therefore, for parameter selection for calibration purpose, BS height similar to UMa which is 25 m and ISD of 1299 m would be suitable.

Major characteristics of SMa compared to UMa are wider area and surrounding building environment with lower heights. Thus, channel parameters also need to be defined reflecting these features well

RAN1 consider that BS height is 25 m (Option 1) and ISD is 1299 m (Option 1) at least for calibration purpose

3.2 CDL channels

Agreement - To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles. - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A. - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180]. - Down-select among options. - Option 1) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel). - Option 2a) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - s=minx≥0{x: ASx=ASdesired}. - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn - Option 2b) - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model, - s is chosen such that AS(ϕn,intermediate) = AS_desired - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn - Option 3) - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model, - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread. - FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

At last few meetings, there has been raised an issue that the formulation within Clause 7.7.5.1 of TR 38.901 might cause discontinuity in azimuth angles. Although this issue had been addressed through Notes in TR 38.901, thanks to the consensus reached at the previous meeting, a more intuitive formulation became possible

Remaining issue is related to scaling factor which is now conducted by linear operation, but this operation does not exactly result in the desired angular spread, i.e., ASdesired. However, our understanding is that the intention of scaling factor by linear operation, which was an outcome during the discussion on channel model in TR, is not matching an exact desired angular spread. In addition, the difference between the outcome of current equation and the actual desired angular spread is large only when the large value of scaling factor is used which is not an usual case.

For one of examples, the comparison between Option 2a and Option 3 across azimuth angle spread of departure is conducted as shown in Figure 7. The scaling factor of Option 3 is ASdesired/ASmodel same with existing channel model and Option 2a use description in agreement. As a result, when applying large value for scaling factors in Option 3, it shows that desired angular spread can come out differently in CDL-D and -E only. On the other hand, if Option 2a is applied, the accurate results can be confirmed.

The point here needs to be highlighted is the necessity of using large scaling factors and deriving exact values. From our understanding, the primary intention of the MIMO extension to CDL is to avoid fixed PMI usage and not to have exact scaling targets. In this context, Option 3 with prohibiting the large values of scaling factor would be appropriate. If updates for existing one are needed, discussions on motivation such as how often large value of scaling will be used under what scenarios along with the necessity of changes in scaling operations based on Option 2a should precede.

| Figure 7. Azimuth angle spread of departure after scaling across CDL channel types |  |  |
| --- | --- | --- |

The difference between the outcome of current equation and the actual desired angular spread is large only when the large value of scaling factor is used which is not an usual case

When applying large value for scaling factors in Option 3, it shows that desired angular spread can come out differently in CDL-D and -E only

From our understanding, the primary intention of the MIMO extension to CDL is to avoid fixed PMI usage and not to have exact scaling targets

RAN1 consider Option 3 with prohibiting the large value of scaling factor

If exact result for scaling factor is needed, RAN1 clarify the motivation such as how often large value of scaling will be used and what scenarios along with the necessity of changes in scaling operation is considered based on Option 2a, first

3.2 UE antenna modeling

Agreement For handheld devices adopt the following device dimensions for UE antenna modeling: - 15 cm x 7 cm x 0 cm Agreement For UE antenna modeling of handheld devices, - Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2. - FFS on handling of antenna array panels

Mapping to antenna location candidates depend on the context of the evaluation and preference of readers of the channel model. For instance, someone might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown. Hence, instead of suggesting guidelines regarding antenna placement under specific frequency band, it can be allowed to determine how the antenna elements and modules are mapped to antenna location candidates.

Some readers of channel model might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown.

RAN1 do not consider the detailed guidelines for how antenna elements and antenna modules are mapped to antenna location candidates

Agreement Further refine previous agreement (made in RAN1#118bis) on UE antenna radiation pattern modeling as follows: - For UE antenna radiation pattern modeling, consider the following options for further study: - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element Parameter; Values. Antenna element vertical radiation pattern (dB); AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB. Antenna element horizontal radiation pattern (dB); AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB. Combining method for 3D antenna element pattern (dB); FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am. Maximum directional gain of an antenna element GE,max; FFS dBi. Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi.. - Option 2) half-wavelength dipole. - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis - FFS: range of θ - Option 2b) half-wavelength dipole - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ)) - range of θ = -90° to +90° - Option 3) omni-directional antenna pattern Agreement For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations. - For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

UE antennas have many challenges in implementation compared to BS antennas, so even antennas in the same band can be degraded in terms of performance. For example, the gain of the antenna may decrease and it may have omnidirectional antenna characteristics rather than directivity. In addition, it is necessary to consider the shielding performance of the back radiation of the front-back ratio.

UE antennas have many challenges in implementation compared to BS antennas, so even antennas in the same band can be degraded in terms of performance such as gain and radiation pattern

- In the last meeting, RAN1 discussed several options for UE antenna modelling.

- Option 1

- In the last agreement, Option 1 was to reuse existing antenna radiation pattern modelling. At this time, the UE antenna’s radiation pattern needs to reflect the realistic situation. Regarding each parameter’s role in the radiation pattern modelling, θ3dB and ϕ3dB representing half power beamwidth (HPBW) can indicate the directivity of the antenna. In addition, SLAV and Am representing side lobe suppression and front-back power ratio, respectively. It can serve as a lower bound for the antenna radiation pattern.

- Figure 8 shows the radiation pattern with varying HPBW while maintaining SLAV and Am. It tends that the larger the HPBW becomes, the weaker the directional properties get in both horizontal and vertical planes.

| - AE,Hϕ″ | - AE,Vθ″ |
| --- | --- |
| Figure 8. Radiation pattern with different half power beamwidth (θ3dB and ϕ3dB) |  |

- Figure 9 shows the radiation pattern with varying SLAV and Am while maintaining HPBW. The lower bound boundary increases in both horizontal and vertical planes.

| - AE,Hϕ″ | - AE,Vθ″ |
| --- | --- |
| Figure 9. Radiation pattern with different side lobe suppression and front-back power ratio (SLAV and Am) |  |

With varying the value of HPBW, it tends that the larger the HPBW becomes, the weaker the directional properties get in both horizontal and vertical planes.

With varying SLAv and Am_while maintaining HPBW, the lower bound boundary increases in both horizontal and vertical planes.

- Option 2a

- The radiation pattern of the dipole antenna pattern, which is option 2a, may not be ideal. This is because there is a singularity where the value becomes very large if θ=0 or θ=π in sin⁡(θ). Therefore, it seems need to optimization method at θ=0 and θ=π. As an alternative, Eθ=sin⁡(π2*cos⁡(θ)), which is the radiation pattern of a half wavelength dipole, can be suggested as shown in Figure 10 (a).

- Option 2b

- On the other hand, a common consideration in both radiation pattern modelling is the back radiation component. When the antenna is mounted on the device (i.e., handheld device), factors such as structure, frame material and placement can reduce the gain in the back lobe direction. Then, the back side radiation component will not radiate as much as the front side radiation component. Therefore, ignoring the back lobe radiation component can be a reasonable way. In this context, it is necessary to limit the range of θ from -90° to 90° when it comes to Eθ=sinπ2*cosθ as shown in Figure 10 (b).

| - sinπ2*cosθ, θ∈{-180, 180} | - sinπ2*cosθ, θ∈{-90, 90} |
| --- | --- |
| - Figure 10. Example for the normalized dipole antenna radiation pattern |  |

The radiation pattern of the dipole antenna pattern, which is option 2a (E(θ) = cos(cos(θ)*π /2) / sin(θ)), may not be ideal. This is because there is a singularity where the value becomes very large if θ=0 or θ=π in sin⁡(θ).

When the antenna is mounted on the device (i.e., handheld device), factors such as structure, frame material and placement can reduce the gain in the back lobe direction. Then, the back side radiation component will not radiate as much as the front side radiation component.

- As another option, there is option 3. This option is suitable for simple evaluation at low frequency bands such as FR1 and has been widely used. However, this does not align with the purpose of this study for modelling realistic UE antenna radiation pattern. In addition, while dipole antennas (Option 2) have patterns similar to omnidirectionality, they cannot guarantee representing all types of antenna radiation patterns.

- Therefore, considering the diversity of radiation patterns corresponding to each antenna applied across multiple frequency bands and the continuity of existing channel models, RAN1 need to consider Option1 as a baseline.

- Furthermore, the proper parameter selection in Option 1 is needed based on the analysis above. Therefore, for better discussion, we would like to suggest the following ranges as starting points:

- θ3dB, ϕ3dB: 90 – 120 degrees

- SLAV: 25 – 15 dB

- Am: 25 – 15 dB

- GE,max: 0 – 5 dB

- Combining method for 3D antenna element pattern: same with TR38.901

Option 3 is suitable for simple evaluation at low frequency bands such as FR1 and has been widely used. However, this does not align with the purpose of this study for modelling realistic UE antenna radiation pattern

Option 2 have patterns similar to omnidirectionality, but they cannot guarantee representing all radiation patterns for various types of antennas

RAN1 consider Option1 as a baseline for UE antenna modelling with the range of parameter values as below:

- θ3dB, ϕ3dB: 90 – 120 degrees

- SLAV: 25 – 15 dB

- Am: 25 – 15 dB

- GE,max: 0 – 5 dB

- Combining method for 3D antenna element pattern: same with TR38.901

- In addition, it is necessary to define the orientation of radiation pattern for each antenna element candidate location. As explained in the agreement, the direction of the radiation pattern of each antenna element can be aligned so that the directionality of the element’s maximum gain is directed towards the direction of the antenna element candidate element location.

- On the other hand, UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle. Therefore, the order of overall UE antenna modelling may be as follows; UE distribution  assume the corresponding the location as reference for orientation of UE  set the radiation pattern orientation for each antenna element candidate location according to the relative direction of the UE’s center orientation  apply same bearing angle for each antenna element candidate location for rotation.

UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle

RAN1 confirm that maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location

RAN1 consider that same bearing angle is applied to all antenna element candidates

- Conclusion

- The marginal difference compared to pathloss for UMi scenario in existing channel model is confirmed

- RAN1 discuss whether the updates of pathloss in UMi scenario is needed

- As shown in below table, in the UMi scenario, the mean delay spread in the LOS environment was found to be lower compared to the existing channel model parameter

| Delay spread | UMi_LOS (TR38.901) @ 6.5 GHz | Measured DS @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured DS @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μDS (log_10⁡〖DS/1s〗 ) | -7.35 | -8.16 | -7.41 | -8.19 |
| (ns) | 44.67 | 6.91 | 38.13 | 6.32 |
| σDS | 0.38 | 0.83 | 0.38 | 0.91 |

- Even in a visually LOS environment between Tx and Rx, the delay spread can vary depending on the surrounding environment. In particular, if an update of delay spread in the LOS environment is required, it will be necessary to collect and analyze sufficient measurement data from various LOS environment to reflect actual conditions

- RAN1 discuss whether the updates of delay spread in UMi LOS scenario is needed

- As shown in below table, slight difference compared to mean and standard deviation of ASD for UMi LOS scenario in the existing channel model were confirmed

| Azimuth spread of departure angles | UMi_LOS (TR38.901) @ 6.5 GHz | Measured ASD @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured ASD @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μASD (log_10⁡〖ASD/1°〗 ) | 1.16 | 1.04 | 1.15 | 0.98 |
| (degree) | 14.66 | 11.18 | 14.18 | 9.6 |
| σASD | 0.41 | 0.21 | 0.41 | 0.20 |

- As shown in below table, significant difference compared to mean and standard deviation of ASA for UMi scenario in the existing channel model were confirmed

| Azimuth spread of arrival angles | UMi_LOS (TR38.901) @ 6.5 GHz | Measured ASA @ 6.5 GHz | TR38.901 @ 13.5 GHz | Measured ASA @ 13.5 GHz |
| --- | --- | --- | --- | --- |
| μASA (log_10⁡〖ASA/1°〗 ) | 1.65 | 1.15 | 1.64 | 1.11 |
| (degree) | 45.71 | 14.26 | 43.36 | 12.7 |
| σASA | 0.29 | 0.28 | 0.29 | 0.33 |

RAN1 discuss whether the updates of azimuth spread of departure/arrival angles in UMi LOS scenario is needed

- Currently available channel parameters reflect stochastic properties of measurement results focused on both Below 6 GHz band around 3.5 GHz and mmWave bands around 28 GHz. New measurements across 7 – 24 GHz band may exhibit nonlinear traits inconsistent with prior values

RAN1 need to discuss regarding how to replace channel parameter values/model equations (e.g., by aggregating old measurement results and new measurement results, extracting new statistical outcomes)

- When examining frequency dependent properties of channel parameter, dynamic range of the measured data affect to results

RAN1 discuss whether/how to apply dynamic range when it comes to frequency dependent properties for channel parameter

- Even when a visual LOS condition exists between the Tx (BS) and the Rx (UE), most paths can be obstructed by vegetation, resulting in a significant decrease in path loss compared to an environment with pure LOS

RAN1 discuss whether/how to handle the vegetation impact

- Upon plotting the pathloss across distance per frequency bands utilizing these equations, it was observed that beyond a particular distance (i.e., breakpoint), the values tend to converge

RAN1 discuss whether/how to handle the phenomena that converges regardless of the frequency band after the breakpoint in LOS pathloss equation

- Major characteristics of SMa compared to UMa are wider area and surrounding building environment with lower heights. Thus, channel parameters also need to be defined reflecting these features well

RAN1 consider that BS height is 25 m (Option 1) and ISD is 1299 m (Option 1) at least for calibration purpose

The difference between the outcome of current equation and the actual desired angular spread is large only when the large value of scaling factor is used which is not an usual case

When applying large value for scaling factors in Option 3, it shows that desired angular spread can come out differently in CDL-D and -E only

From our understanding, the primary intention of the MIMO extension to CDL is to avoid fixed PMI usage and not to have exact scaling targets

RAN1 consider Option 3 with prohibiting the large value of scaling factor

If exact result for scaling factor is needed, RAN1 clarify the motivation such as how often large value of scaling will be used and what scenarios along with the necessity of changes in scaling operation is considered based on Option 2a, first

- Some readers of channel model might want to know the difference in performance between having the FR1 antenna at the edge versus the corner, which could also apply to the FR2 antenna array. Additionally, whether antenna for new frequency band become antenna elements or arrays remains unknown.

RAN1 do not consider the detailed guidelines for how antenna elements and antenna modules are mapped to antenna location candidates

- UE antennas have many challenges in implementation compared to BS antennas, so even antennas in the same band can be degraded in terms of performance such as gain and radiation pattern

- θ3dB, ϕ3dB:SLAV:Am:GE,max:With varying the value of HPBW, it tends that the larger the HPBW becomes, the weaker the directional properties get in both horizontal and vertical planes.

- With varying SLAv and Am_while maintaining HPBW, the lower bound boundary increases in both horizontal and vertical planes.

- The radiation pattern of the dipole antenna pattern, which is option 2a (E(θ) = cos(cos(θ)*π /2) / sin(θ)), may not be ideal. This is because there is a singularity where the value becomes very large if θ=0 or θ=π in sin⁡(θ).

- When the antenna is mounted on the device (i.e., handheld device), factors such as structure, frame material and placement can reduce the gain in the back lobe direction. Then, the back side radiation component will not radiate as much as the front side radiation component.

Option 3 is suitable for simple evaluation at low frequency bands such as FR1 and has been widely used. However, this does not align with the purpose of this study for modelling realistic UE antenna radiation pattern

Option 2 have patterns similar to omnidirectionality, but they cannot guarantee representing all radiation patterns for various types of antennas

RAN1 consider Option1 as a baseline for UE antenna modelling with the range of parameter values as below:

- θ3dB, ϕ3dB: 90 – 120 degrees

- SLAV: 25 – 15 dB

- Am: 25 – 15 dB

- GE,max: 0 – 5 dB

- Combining method for 3D antenna element pattern: same with TR38.901

UE antenna modelling in existing channel model sets a bearing angle for the rotation of the UE. It is necessary to confirm that each antenna elements for the UE applies the same bearing angle

RAN1 confirm that maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location

RAN1 consider that same bearing angle is applied to all antenna element candidates

References

RP-234018 “New SID: Study on channel modelling enhancements for 7-24GHz for NR,” RAN#102

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”

R1-2402480 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2404129 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2406666 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2405659 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

R1-2409610 “Discussion on channel model validation of TR38.901 for 7-24 GHz”, Samsung

Report ITU-R M.2135-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced, 2009.