3GPP TSG RAN WG1 #120			R1-2500902

Athens, Greece, February 17th – 21st, 2025

Source:	ETRI

Title:	Discussion on specification support for CSI prediction

Agenda item:	9.1.3

Document for:	Discussion

- Introduction

At the RAN#106 meeting, followings were concluded for specification support for CSI prediction [1].

Conclusion at RAN#106 Provide specification support for the following aspects: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI predictison (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In this contribution, we discuss issues on specification support for CSI prediction and provide our views.

Discussion on specification support for CSI prediction

As we mentioned above, in the RAN#106 meeting, it was concluded that from Q1 of 2025, normative work in RAN1 for CSI prediction is started. Hence, in this section, we discuss issues on specification support for CSI prediction.

Issue 1: Consistency of training/inference

Ensuring consistency between training and inference in an AI/ML model is critical for achieving optimal performance. If consistency is not maintained, the AI/ML model may yield inappropriate results, as the inference environment can differ from the training environment. For example, in CSI prediction, consider a scenario where the UE-side AI/ML model is trained under "Network Side Additional Condition – Setting A." If this condition changes to "Setting B," where factors like tilt angle or TXRU mapping differ from those in Setting A, the model's inference results may become unreliable. Because training and inference occur under distinct network conditions, the model inputs for each may have different characteristics. This discrepancy means the trained model might not align well with the input used during inference, potentially leading to mismatched results. Therefore, in our view, ensuring consistency between training and inference is essential for preventing significant performance degradation in CSI prediction for UE-side AI/ML models.

- Observation 1: Ensuring the consistency between training and inference of UE-sided AI/ML model is essential for preventing non-negligible performance degradation.

In the RAN1 #118 meeting, it was agreed to support associated ID to ensure consistency between training and inference for AI/ML based beam management [2].

Agreement at RAN1#118 For UE sided model in beam management, introduce support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

From our perspective, since the support for an associated ID has already been agreed upon, there is no reason not to extend this support to CSI prediction, as it would reduce the specification workload. Therefore, we propose supporting an associated ID for consistency in the UE-side model for CSI prediction. In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

- Proposal 1: For consistency of training/inference of UE sided model in CSI prediction, support associated ID.

- Proposal 2: In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

If an associated ID is supported, the UE should identify the associated ID of received CSI-RS during the data collection process. The UE (or UE server) may categorize collected data based on the associated ID, allowing its use in model training and inference. Therefore, specifying the configuration of the associated ID is essential. In our view, the associated ID can be configured through either the CSI report configuration or the CSI resource configuration. If the associated ID is configured via the CSI report configuration, the UE can assume that the CSI-RS configured within the CSI report configuration shares the same associated ID. Similarly, if the associated ID is configured through the CSI resource configuration, the UE can assume that the CSI-RS configured within the CSI resource configuration shares the same associated ID.

- Proposal 3: For consistency of UE sided model in CSI prediction, RAN1 to study the method for configuration of associated ID.

  - The associated ID can be configured through the CSI report configuration or CSI resource configuration from a higher layer, with the UE assuming that the configured CSI-RS within the configuration shares the same associated ID.

Issue 2: Performance monitoring for UE-sided AI/ML model

In TR 38.843[3], followings are described as potential specification impact of performance monitoring for CSI prediction using UE-sided model.

TR 38.843 v18.0.0 Clause 7.1.2 …(omitted)… Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

For more accurate CSI prediction, performance monitoring of the UE-sided AI/ML model should be conducted. According to TR 38.843, three types of performance monitoring procedures are suggested. In type 1 performance monitoring, the UE calculates a performance metric based on measurements of the received RS. The UE then reports the performance metric output derived from the calculated performance metric. In type 2 performance monitoring, the UE is required to report both predicted and ground-truth data. In type 3 performance monitoring, the UE calculates and reports the performance metric. From our perspective, Type 2 performance monitoring should be deprioritized due to the signalling overhead. Since both the predicted CSI and the corresponding ground-truth are reported in Type 2 monitoring, a large amount of uplink resources is required for UL reporting. For a similar reason, Type 1 monitoring is more efficient than type 3 monitoring in terms of uplink resource utilization. Since the UE only reports the performance metric output in Type 1 monitoring, the signalling burden is lower compared to Type 3 monitoring, which requires reporting the calculated performance metric. However, if Type 1 performance monitoring is supported, specifying the threshold configuration is essential, as the performance monitoring output may be derived from comparing the calculated performance metric with a (pre)configured threshold.

- Observation 2: Type 1 performance monitoring is more efficient than Type 2 / Type 3 performance monitoring in terms of uplink resource utilization.

- Proposal 4: Support Type 1 performance monitoring for UE-side models in CSI prediction.

For performance monitoring, the UE requires both predicted data and ground-truth data to calculate the performance metric. Therefore, the network can configure RS for model inference (i.e., CSI prediction) and RS for channel measurement (i.e., ground-truth data collection). If the network configures RS for channel measurement, it can also allocate UL resources for reporting the performance metric or performance metric output through configuration. In this case, the UE reports only the performance metric or performance metric output, rather than CSI, for the configured RS.

- Proposal 5: The network can configure RS for ground-truth data collection and model inference, as well as uplink resources for reporting the performance metric or performance metric output in a single configuration.

- Proposal 6: For performance monitoring, the UE reports only the performance metric or performance metric output, rather than CSI, for the configured RS.

For performance monitoring, both NMSE and SGCS are appropriate KPI for model evaluation. Specifically, NMSE and/or SGCS between the predicted and ground-truth channel can be used as performance metrics, representing the similarity between the predicted and ground-truth CSI. In Type 1 performance monitoring, the UE has to report the performance metric output based on the calculated performance metric. In our view, the performance metric output is derived from comparing the calculated performance metric with the corresponding threshold. For example, assuming SGCS is used as performance metric, the UE can report a performance metric output for fallback operation if the calculated performance metric falls below a (pre)configured threshold. Therefore, to support Type 1 performance monitoring, specification efforts are required to configure threshold criteria that facilitate UE-side performance monitoring.

Regarding threshold configuration, it can be set alongside the configuration of UL resources for reporting the performance metric or performance metric output, such as via RRC configuration. In our view, supporting two or more threshold values/criteria would provide greater flexibility for implementation. For instance, if SGCS is used as the performance metric, two threshold values, Thr1, Thr2, can be configured (e.g., Thr1=0.9, Thr2=0.5). If the calculated SGCS value exceeds Thr1, the current model is maintained. If the SGCS value falls below Thr2, the UE reports the performance metric output, facilitating a fallback decision at the network. If the calculated SGCS value falls between the configured thresholds, the UE can switch to another model better suited for the current conditions or temporarily deactivate the mode. Additionally, if the calculated performance metric lies between the two threshold values, the UE can decide whether to maintain or fall back on legacy framework, allowing for various implementation options.

- Proposal 7: For Type 1 / Type3 performance monitoring, support both NMSE and SGCS as performance metric.

- Proposal 8: Specify the configuration of threshold criteria to facilitate UE-side performance monitoring for Type 1 performance monitoring.

- Observation 3: For Type 1 performance monitoring, configuring two or more threshold provides greater flexibility in UE and model implementation.

- Proposal 9: For Type 1 performance monitoring, consider the configuration of two or more threshold values.

For Type 1 performance monitoring, the performance metric output is reported based on the calculated performance metric. In our view, the performance metric output can represent a set of UE behaviors for the UE-side model. Therefore, the performance metric output may consist of three states: 1. Maintain the current model, 2. Switch to another model, 3. Deactivate the model and fall back to the legacy CSI framework. The UE reports the performance metric output according to the calculated performance metric and the configured threshold criteria.

- Proposal 10: The performance metric output can represent a set of UE behaviors for the UE-side model, consisting of three states: 1. Maintain the current model, 2. Switch to another model, 3. Deactivate the model and fall back to legacy CSI framework.

- Conclusion

In this contribution, we have discussed issues on specification support for CSI prediction. As a conclusion, we summarize our views as follows:

- Observation 1: Ensuring the consistency between training and inference of UE-sided AI/ML model is essential for preventing non-negligible performance degradation.

- Proposal 1: For consistency of training/inference of UE sided model in CSI prediction, support associated ID.

- Proposal 2: In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID.

- Proposal 3: For consistency of UE sided model in CSI prediction, RAN1 to study the method for configuration of associated ID.

  - The associated ID can be configured through the CSI report configuration or CSI resource configuration from a higher layer, with the UE assuming that the configured CSI-RS within the configuration shares the same associated ID.

- Observation 2: Type 1 performance monitoring is more efficient than Type 2 / Type 3 performance monitoring in terms of uplink resource utilization.

- Proposal 4: Support Type 1 performance monitoring for UE-side models in CSI prediction.

- Proposal 5: The network can configure RS for ground-truth data collection and model inference, as well as uplink resources for reporting the performance metric or performance metric output in a single configuration.

- Proposal 6: For performance monitoring, the UE reports only the performance metric or performance metric output, rather than CSI, for the configured RS.

- Proposal 7: For Type 1 / Type3 performance monitoring, support both NMSE and SGCS as performance metric.

- Proposal 8: Specify the configuration of threshold criteria to facilitate UE-side performance monitoring for Type 1 performance monitoring.

- Observation 3: For Type 1 performance monitoring, configuring two or more threshold provides greater flexibility in UE and model implementation.

- Proposal 9: For Type 1 performance monitoring, consider the configuration of two or more threshold values.

- Proposal 10: The performance metric output can represent a set of UE behaviors for the UE-side model, consisting of three states: 1. Maintain the current model, 2. Switch to another model, 3. Deactivate the model and fall back to legacy CSI framework.

References

- RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- Draft Report of 3GPP TSG RAN WG1 #118b v0.2.0

- 3GPP TR 38.843 v18.0.0