3GPP TSG RAN WG1 #120	R1- 2501005

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:		9.3.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Cross-link interference handling for duplex evolution

Document for:		Discussion and Decision

# Introduction

In RAN #104, a revision of the WID on Duplex Evolution was approved in [1], containing the updates objectives:

For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - [..] - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling

In this document we discuss the remaining issues for handling the co-channel gNB-to-gNB CLI and the UE-to-UE CLI.

# gNB-to-gNB co-channel CLI handling

## UL muting

During RAN1#119, good progress was made on the topic of UL muting. The following discusses the remaining aspects of this feature. A summary of the most relevant agreements reached during RAN#119 is found as follows:

Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM. A comb offset {0, 1} is configured for the up to two UL muting symbols

### Frequency domain configuration of UL muting resources

It has been agreed that the UL resource muting configuration shall include a comb-offset field which takes 0 and 1 values. Moreover, the same comb-offset is assumed for the up-to 2 muted symbols. It is still pending from RAN#119 whether the configurable comb-offset is also applicable to transform precoding, i.e., when using DFT-s-OFDM. Though we think the muting for DFT-s-OFDM need to be further studied in RAN4, supporting for configuration of comb-offset is useful for scheduling flexibility especially for MU-MIMO with CP-OFDM PUSCH.

Support the comb-offset configuration for DFT-s-OFDM

### UL muting with PUSCH repetition

In RAN1#119, the following agreement was reached regarding UL muting and PUSCH repetition:

Agreement For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions

It can be noted that the current agreement only considers Configuration 1, i.e., where the PUSCH repetitions only occur in SBFD slots or non-SBFD slots. In case of Configuration 2, we think that UL muting should only be applied to PUSCH allocations within SBFD symbols, while PUSCH allocations over UL symbols should not be affected. Thus, if one or more PUSCH repetitions are within a UL slot (non-SBFD Slot), the UL muting should not be performed over those PUSCH transmissions. This could also be seen as a rule for conditional UL muting.

For PUSCH repetitions with Configuration 2, do not perform UL muting over PUSCH repetitions within non-SBFD slots.

### Collision between UL muting symbol and PT-RS

On the collision between UL muting symbol and PTRS, discussions differentiate between whether the PUSCH is transmitted using transform precoding or not. When transform precoding is used and muting is supported, time domain signal with half-symbol length is repeated. Because PTRS is multiplexed as pre-DFT manner with certain mapping patterns which are mapped across whole OFDM symbol without repetition in each half-symbol length, PT-RS need to be redesigned to support UL resource muting.

Since PTRS can be multiplexed with every or every second symbols (i.e. LPT-RS∈1,2LPT-RS∈1,2), for DFT-s-OFDM, gNB can configure LPT-RS=2LPT-RS∈1,2 , and UL resource muting is applied only for the symbol without PTRS. Because UL resource muting symbol locations are preconfigured in a slot regardless of TDRA pattern, it is difficult to ensure avoiding overlap with muting symbol and symbol with PTRS. Thus, we propose that UE doesn’t apply resource muting when collide with symbols with PTRS.

If transform precoding is used, UE doesn’t not apply resource muting in the symbol with including PTRS.

### Applicability of UL muting with DFT-s-OFDM

DFT-s-OFDM is used for transmitting data in different time domain samples by using DFT transform precoding. The basic assumption is the time domain signal is transformed to frequency domain signal with DFT-precoding, and this frequency domain signal is multiplexed into OFDM subcarriers in a contiguous PRBs manner. Also, we can find some limitation in TS 38.214, e.g., uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled. There are two different options for supporting UL muting with transform precoding:

- The first option is to puncture REs in the frequency domain. This causes distortion in TX EVM/RX EVM/PAPR of the original signal. It may introduce critical performance degradation.

- The second option is to map frequency domain signal into REs not muted. This is equivalent to use higher SCS for DFT-s-OFDM signal. In this case, in time domain, the same samples are repeated twice, and energy is divided into two samples in half OFDM symbol interval. In gNB receiver, for the muted symbol, NW will perform different demodulation process than the other symbol. This introduces implementation complexity in gNB.

UL resource muting with DFT-s-OFDM introduces higher complexity both in UE transmitter and gNB receiver, because the different DFT-size in TX and different demodulation algorithm need to be supported. Also, the impact from 3dB power boosting to EVM/MPR need to be carefully studied in RAN4.

However, to make progress in RAN1 while RAN4 investigate the RF impact, we are fine to agree with supporting UL muting for DFT-s-OFDM in RAN1.

Support UL muting for DFT-s-OFDM.

### Conditional UL muting

On the condition UL muting, the following set of rules were captured in the discussions during RAN1#118-bis:

Proposal 1-7 (LP) Study the necessity of following conditions to apply UL resource muting for a PUSCH - Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied - Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied - Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied - Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied - Condition 5: UL resource muting is not applied according to the PHY priority of PUSCH - Condition 6: For PUSCH that is a retransmission, UL resource muting is not applied - Condition 7: For PUSCH that is scheduled by DCI format 0_0, UL resource muting is not applied - Condition 8: for PUSCH with repetition type B, UL resource muting is not applied - Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied. - Other conditions are not precluded.

We think that the definition of these rules is needed. The main reason is to keep the size of the TDRA table reasonable. For instance, assume an entry of the TDRA table that configures a SLIV with start = 0 and length = 14. In order to implement, for example Condition 3, two entries with the same SLIV but with UL muting enabled and disabled are needed. Thus, depending on the BW of the allocation one of the two entries would be selected. This can largely increase the size of the TDRA table when accounting for all the possible conditions listed above. Thus, we support the introduction of UL muting and the corresponding set of rules.

- Support conditional UL muting to reduce the number of entries in the TDRA table, while allowing certain flexibility in the applicability of the UL resource muting.

Among all the captured conditions in the proposal above, we support at least Condition 2, Condition 3 and Condition 7.

MCS/BW can be considered for conditions for disabling muting. When MCS index is less than threshold, due to low SNR advanced receiver gain from interference suppression may not be enough.  When BW is small, the number of muted REs for interference measurement may not be enough. Thus, we think MCS and BW threshold can be used for disabling UL resource muting.

For the condition 7, PUSCH scheduled by DCI 0_0 is scheduled with single port with 3 DMRS symbols, where REs in CDM group 1 not used for DMRS sequence are muted without PUSCH multiplexing. There is no need for using additional muting symbol other than DMRS symbols.

Additionally, we think that a condition related to the symbol type is useful, i.e., the UE is not expected to perform UL resource muting in UL symbols.

Support Condition 2 (MCS index), Condition 3 (number of PRBs) and Condition 7 (DCI format 0_0) of the list of conditions captured in the Proposal 1-7.

Introduce a new condition for conditional UL muting that avoid performing UL muting during UL symbols, i.e., UL muting is only applied to SBFD symbols.

# UE-to-UE co-channel CLI handling

In this section, we provide our view about remaining aspects of L1-based UE-to-UE CLI measurement and reporting. The most relevant agreements from RAN1#119 are captured below:

Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

## CLI report setting

In this section, we discussed about the detail of CSI-ReportConfig IE format for L1 CLI reporting. The reporting setting includes measurement resource setting, reporting configuration type, reporting quantity, etc.

### Measurement Resource

During RAN1#119, RAN1 has made following agreements regarding the L1 UE-to-UE CLI resource configuration.

Agreement Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other parameters Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters

As shown above, RAN#1 agreed to re-use the SRS-ResourceConfigCLI-r16 and RSSI-ResourceConfigCLI-r16 for L1 UE-to-UE CLI measurement resources configuration as baseline. We propose also a down-selection of unnecessary L3 measurement resource configuration parameters for L1 measurements. For instance, the parameters highlighted with red-text in Table 2 are deemed not necessary for L1-CLI-RSSI/L1-SRS-RSRP measurement purpose. These parameters can be deleted given that L1 UE-to-UE CLI measurements are intended for intra-cell measurements. Additionally, we think the range of the startPRB-r16 parameter is unnecessarily large and it can be updated in a similar manner as NZP-CSI-RS or CSI-IM resource configuration.

For L1 CLI measurement resource configuration, perform a down-selection from Rel-16 parameters by:

- Removing srs-SCS-r16/rssi-SCS-r16

- Removing refBWP-r16

- Updating startPRB-r16 range INTEGER(0..2169) to INTEGER(0..maxNrofPhysicalResourceBlocks)

- FFS: refServCellIndex-r16

Table 2. Rel-16 UE-to-UE CLI measurement resource configuration

MeasObjectCLI-r16 ::= SEQUENCE { cli-ResourceConfig-r16 CLI-ResourceConfig-r16, ... } CLI-ResourceConfig-r16 ::= SEQUENCE { srs-ResourceConfig-r16 SetupRelease { SRS-ResourceListConfigCLI-r16 } OPTIONAL, -- Need M rssi-ResourceConfig-r16 SetupRelease { RSSI-ResourceListConfigCLI-r16 } OPTIONAL -- Need M } SRS-ResourceListConfigCLI-r16 ::= SEQUENCE (SIZE (1.. maxNrofCLI-SRS-Resources-r16)) OF SRS-ResourceConfigCLI-r16 RSSI-ResourceListConfigCLI-r16 ::= SEQUENCE (SIZE (1.. maxNrofCLI-RSSI-Resources-r16)) OF RSSI-ResourceConfigCLI-r16 SRS-ResourceConfigCLI-r16 ::= SEQUENCE { srs-Resource-r16 SRS-Resource, srs-SCS-r16 SubcarrierSpacing, refServCellIndex-r16 ServCellIndex OPTIONAL, -- Need S refBWP-r16 BWP-Id, ... } RSSI-ResourceConfigCLI-r16 ::= SEQUENCE { rssi-ResourceId-r16 RSSI-ResourceId-r16, rssi-SCS-r16 SubcarrierSpacing, startPRB-r16 INTEGER (0..2169), nrofPRBs-r16 INTEGER (4..maxNrofPhysicalResourceBlocksPlus1), startPosition-r16 INTEGER (0..13), nrofSymbols-r16 INTEGER (1..14), rssi-PeriodicityAndOffset-r16 RSSI-PeriodicityAndOffset-r16, refServCellIndex-r16 ServCellIndex OPTIONAL, -- Need S ... } RSSI-ResourceId-r16 ::= INTEGER (0.. maxNrofCLI-RSSI-Resources-1-r16) RSSI-PeriodicityAndOffset-r16 ::= CHOICE { sl10 INTEGER(0..9), sl20 INTEGER(0..19), sl40 INTEGER(0..39), sl80 INTEGER(0..79), sl160 INTEGER(0..159), sl320 INTEGER(0..319), s1640 INTEGER(0..639), ... }

For semi-persistent measurement resource, new MAC-CE to activate/deactivate the measurement resource should be supported. We think semi-static resource/reporting is useful to reduce unnecessary overhead and UE complexity. gNB may activate semi-persistent resource /reporting only when CLI measurement is required (e.g. UE RSRP level is lower, UE location is CLI sensitive area, noise rise happens, etc.)

Support new MAC-CE for semi-persistent L1-CLI-RSSI measurement resource activation/deactivation.

### Reporting configuration type (time-domain behavior for CLI-reporting)

For L1-RSRP reporting, periodic/semi-persistent and aperiodic reporting are supported. Periodic and semi-persistent measurement and reporting are useful for observing mid/long-term CLI level while aperiodic measurement and reporting are used for short-term/dynamic coordination of CLI with identifying victim and aggressor.

During RAN1 #117, we have agreed to support at least aperiodic reporting. However, there is no reason to preclude other reporting configuration type. There was an argument on the definition of CSI timeline for SRS-RSRP due to symbol timing misalignment. However, in the current specification, the CPU occupation starts from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource. For SRS-RSRP, we can define the first symbol as the first downlink symbol overlap with the first SRS-RSRP measurement resource. So, periodic/semi-persistent can be supported with simple modification. Regarding aperiodic reporting, legacy framework should be followed and therefore reporting on PUSCH is supported. Due to unclear benefits and potentially large specification impact, we prefer to not support aperiodic reporting over PUCCH.

For L1-based CLI reporting for both L1-CLI-RSSI and L1-SRS-RSRP, support periodic, semi-persistent and aperiodic reporting.

### Reporting Quantity

Regarding the reporting quantity, the following agreements were reached during RAN1#119:

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

Based on the agreement above, the reporting of L1 CLI is following the same principle as the L1-RSRP and L1-SINR reporting. There are a couple of FFS that we discuss in the following. First, whether the UE should also support reporting the least interference measured L1-CLI-RSSI or L1-SRS-RSRP. We think this way of reporting is not optimal since it doesn’t inform the gNB about the aggressor UEs. Thus, we prefer to only rely on the most interfered measured resources.

Do not support the reporting of the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

On how to handle the out-of-range measurements, we would like to highlight that current report mapping table for layer-3 SRS-RSRP supports such condition by adding an entry in the table with corresponds to “infinity”. Specifically, the reported value SRS_RSRP_98 indicates that the UE cannot detect SRS due to too strong signal to measure. This can be used for absolute power reporting. For the case of differential reporting, if the largest measurement is out-of-range the UE can simply assume the maximum value in the range for performing the differential reporting.

In case of differential reporting, the UE assumes the highest L1-SRS-RSRP or L1-CLI-RSSI in the reporting range if the most interfered measurement if out-of-the-range.

In addition to this, we have discussed following proposal during RAN1 #118bis.

#### Proposal 2-12 For L1 UE-to-UE CLI measurement and reporting, support the following a new report quantity: - A bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating whether L1-SRS-RSRP or L1-CLI-RSSI above a given threshold - The length of the bitmap is same as the number of SRS-RSRP or CLI-RSSI measurement resources within the set - FFS: Definition of threshold

For CLI handling, gNB can configure one or more CLI measurement resources for the reporting configuration. When gNB wants to check the level of interference from certain aggressor UE(s), gNB can configure CLI reporting similar as L1-RSRP reporting. But, if there is multiple potential aggressors and the number of CLI measurement resources corresponding to a number of potential aggressor UEs, it is also beneficial to report the index of the measurement resource comprising larger interference than a threshold. In this case, to reduce the reporting overhead, only reporting the resource indices is beneficial. Since all measurement resources are configured by RRC, bitmap with the size of the number of measurement resources configured in the reporting configuration can be used.

On the other hand, instead of configuring threshold, UE can be configured to report only the index of K most interfering CLI-measurement resources. Bitmap can be used for this case too.

Support reporting of index/bitmap of resources with strong interference. Consider the following options.

  - Option 1: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating whether L1-SRS-RSRP or L1-CLI-RSSI is above a given threshold

  - Option 2: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating the K most interfering resource.

### CPU occupancy

There is a discussion about timeline for CPU occupancy for SRS-RSRP reporting. In the current specification, the CPU is occupancy is determined as below. The reporting process occupies CPU from the first symbol of the earliest one of each measurement resource. For L1-SRS-RSRP measurement, due to timing misalignment it is unclear where the CPU occupancy starts.

TS 38.214 section 5.2.1.6 CSI processing criteria … For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity not set to 'none', the CPU(s) are occupied for a number of OFDM symbols as follows: - A periodic or semi-persistent CSI report (excluding an initial semi-persistent CSI report on PUSCH after the PDCCH triggering the report and a semi-persistent CSI report on PUSCH configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18') occupies CPU(s) from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource, or each CSI-RS/CSI-IM resource associated with all configured sub-configurations for periodic CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, or each CSI-RS/CSI-IM resource associated with all activated/triggered sub-configurations for semi-persistent CSI report corresponding to a CSI-ReportConfig that contains a list of sub-configurations provided by csi-ReportSubConfigToAddModList, for channel or interference measurement, respective latest CSI-RS/CSI-IM/SSB occasion no later than the corresponding CSI reference resource, until the last symbol of the configured PUSCH/PUCCH carrying the report.

To address this ambiguity, we can consider options. One option is applying a fixed symbol offset from the start symbol of configured L1-SRS-RSRP resource. The offset may cover a constant timing offset (Tc*NTA_offset) as well as some residual timing error as discussed above (Tother). This fixed offset can be either one or two symbols, and this can be configured by NW or can be derived by parameters such as SCS, FRx and UE capability similar as scheduling availability of UE during CLI measurement specified in 9.7.4 of TS 38.133.

For periodic/semi-persistent L1-SRS-RSRP measurement reporting, the CPU occupancy starts from a fixed symbol offset prior to L1-SRS-RSRP measurement resource. FFS: a fixed symbol offset

# Conclusions

This document discusses the CLI handling in SBFD for both gNB-to-gNB and UE-to-UE interference components. The following includes the list of observations and proposals made throughout the text.

Observations and proposals:

Proposal 1:	Support the comb-offset configuration for DFT-s-OFDM

Proposal 2:	For PUSCH repetitions with Configuration 2, do not perform UL muting over PUSCH repetitions within non-SBFD slots.

Proposal 3:	If transform precoding is used, UE doesn’t not apply resource muting in the symbol with including PTRS.

Observation 1:	UL resource muting with DFT-s-OFDM introduces higher complexity both in UE transmitter and gNB receiver, because the different DFT-size in TX and different demodulation algorithm need to be supported. Also, the impact from 3dB power boosting to EVM/MPR need to be carefully studied in RAN4.

Proposal 4:	Support UL muting for DFT-s-OFDM.

Proposal 5:	Support conditional UL muting to reduce the number of entries in the TDRA table, while allowing certain flexibility in the applicability of the UL resource muting.

Proposal 6:	Support Condition 2 (MCS index), Condition 3 (number of PRBs) and Condition 7 (DCI format 0_0) of the list of conditions captured in the Proposal 1-7.

Proposal 7:	Introduce a new condition for conditional UL muting that avoid performing UL muting during UL symbols, i.e., UL muting is only applied to SBFD symbols.

Proposal 8:	For L1 CLI measurement resource configuration, perform a down-selection from Rel-16 parameters by:

-	Removing srs-SCS-r16/rssi-SCS-r16

-	Removing refBWP-r16

-	Updating startPRB-r16 range INTEGER(0..2169) to INTEGER(0..maxNrofPhysicalResourceBlocks)

-	FFS: refServCellIndex-r16

Proposal 9:	Support new MAC-CE for semi-persistent L1-CLI-RSSI measurement resource activation/deactivation.

Proposal 10:	For L1-based CLI reporting for both L1-CLI-RSSI and L1-SRS-RSRP, support periodic, semi-persistent and aperiodic reporting.

Proposal 11:	Do not support the reporting of the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

Proposal 12:	In case of differential reporting, the UE assumes the highest L1-SRS-RSRP or L1-CLI-RSSI in the reporting range if the most interfered measurement if out-of-the-range.

Proposal 13:	Support reporting of index/bitmap of resources with strong interference. Consider the following options.

a.	Option 1: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating whether L1-SRS-RSRP or L1-CLI-RSSI is above a given threshold

b.	Option 2: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating the K most interfering resource.

Proposal 14:	For periodic/semi-persistent L1-SRS-RSRP measurement reporting, the CPU occupancy starts from a fixed symbol offset prior to L1-SRS-RSRP measurement resource. FFS: a fixed symbol offset

# References

- RP-241614, “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei