3GPP TSG RAN WG1 #120						        R1- 2501042

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.11.3

Source: 	LG Electronics

Title: 	Discussion on NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion and decision

- Introduction

In RAN#104, the Rel-19 NTN WI objectives for UL capacity/cell throughput enhancement for FR1-NTN were revised as follows [1]:

2. Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In RAN1 #119 meeting [3], it was agreed to support OCC length 2/4 with inter-slot OCC to multiplex up to 2/4 UEs. Accordingly, this contribution discusses potential specification aspect(s) when supporting inter-slot OCC for DFT-s-OFDM based PUSCH transmission in FR1 NR NTN FDD system.

- Discussion

  - Specification aspects on OCC technique(s)

    - RV cycling

For inter-slot OCC, redundancy version (RV) cycling over PUSCH repetition(s) needs to be changed. In RAN1 #118bis [2], it was agreed to use the same RV within an OCC group. In RAN1 #119 [3], it was agreed to consider the following options for RV cycling for DG-PUSCH with OCC:

Agreement For RV cycling for OCC with DG-PUSCH, the following are considered: - Option 1: RV cycling is used across OCC groups - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length - Option 2: Fixed RV is used across OCC groups - Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

Since all three options have its own benefit (e.g., coding gain, compatibility with legacy UEs, multiplexing of UEs with different OCC lengths, etc.), a solution that can include the above options as much as possible should be considered. In our view, RV cycling based on RV cycling granularity (e.g., L) can be a unified solution that can include the above three options (i.e., Option 1/2/3) where the RV cycling granularity can be configured by the network. For example, assume that there are 8 repetitive PUSCH transmissions with OCC length is 2 (e.g., 4 OCC groups) and RV cycling per RV cycling granularity as in Table 1. When L is set to OCC length, RV cycling is used across OCC groups (i.e., Option 1). When L is set to equal to or larger than the total number of PUSCH repetitions, fixed RV is used across all OCC groups (i.e., Option 2). When L is set to twice the OCC length, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups (i.e., Option 3).

Table 1: Redundancy version with RV cycling granularity

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion (repetition Type A) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ((n-(n mod L))/L ) mod 4 = 0 | ((n-(n mod L))/L ) mod 4 = 1 | ((n-(n mod L))/L ) mod 4 = 2 | ((n-(n mod L))/L ) mod 4 = 4 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

Proposal #1: For PUSCH with OCC, RAN1 support RV cycling based on RV cycling granularity.

- RV cycling granularity is set as a multiple of the OCC length.

- FFS: how to determine RV cycling granularity

    - UCI multiplexing

For inter-slot OCC, the UCI multiplexing rules may need to be changed since both of data and control should be repeated under OCC. In RAN1 #119 [3], the following agreement was made for the UCI multiplexing for PUSCH with OCC:

Agreement If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered: - Option 1: UCI is dropped - FFS: whether all UCI is dropped - Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped. - Option 3: UCI is multiplexed on PUSCH with inter-slot OCC - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC - FFS: which OCC group - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions Note: combination of the above can be considered FFS Details timeline of PUCCH and PUSCH FFS: handling of PUCCH with repetition FFS: handling of different UCI types

Since the main motivation of OCC is to support UE multiplexing, it is necessary to consider UCI multiplexing rules that maintain the orthogonality of OCC while transmitting both of PUSCH and UCI as much as possible. Therefore, we prefer to support Option 3-a, where UCI is transmitted on PUSCH with OCC and multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC. The OCC group should be an OCC group overlapping with the PUCCH. This is because delaying UCI transmission to a non-overlapping OCC group can cause collision problems with other UCIs and complicate UE operations.

When UCI is multiplexed on all PUSCH repetitions within an OCC group, timeline conditions should need to be considered. The timeline conditions of the existing UCI multiplexing operations can be briefly revisited as follows:

- UE expects that the first symbol S0 of the earliest PUCCH or PUSCH, among a group overlapping PUCCHs and PUSCHs in the slot, satisfies the following timeline conditions:

  - S0 is not before a symbol with CP starting after Tproc,1 after a last symbol of any corresponding PDSCH

  - S0 is not before a symbol with CP starting after Tproc,2 after a last symbol of

    - any PDCCH with the DCI format scheduling an overlapping PUSCH, and

    - any PDCCH providing a DCI format with corresponding HARQ-ACK information in an overlapping PUCCH in the slot

For PUSCH with OCC, in addition to the existing timeline condition, the first symbol of the earliest PUSCH with the same OCC sequence as the overlapping PUSCH should be considered. Given that timeline conditions, UCI can be multiplexed within an OCC group when the timeline conditions are met. If the timeline conditions are not met, it can be left up to the UE implementation, like the existing UCI multiplexing rules. Or, the overlapping OCC group can be dropped.

Moreover, UCI overhead reduction can also be considered. It is worth noting that according to the existing UCI multiplexing rules, there already exists beta offset parameter(s) (e.g., βoffsetPUSCH) that can adjust the number of coded modulation symbols of UCI. For example, beta offset parameter(s) can be semi-statically configured or dynamically indicated by DCI to control UCI overhead on PUSCH. Therefore, for PUSCH with OCC, the UCI overhead can be controlled by adjusting the beta offset parameter(s). The beta offset parameter(s) for PUSCH with OCC can be independent from the beta offset parameter(s) for PUSCH without OCC.

Proposal #2: For PUSCH with OCC, RAN1 support following UCI multiplexing rules:

- If the timeline conditions are met,

  - UCI is multiplexed on all PUSCH repetitions within an OCC group that overlaps with PUCCH without repetition.

- Otherwise,

  - It is left up to the UE implementation.

- For the timeline, the first symbol of the earliest PUSCH with the same OCC sequence as the overlapping PUSCH is considered.

Proposal #3: For PUSCH with OCC, RAN1 consider separating the beta offset parameter(s) between PUSCH without OCC and PUSCH with OCC.

Meanwhile, there has been discussion about the case where an OCC group overlap with two or more PUCCH without repetitions. Considering that UCI multiplexing rules for handling multiple UCIs can complicate UE operation and that it is uncommon for two PUCCHs to exist at different points in the OCC length, the above case can be considered an error case and left to the UE implementation.

Proposal #4: For PUSCH with OCC, UE does not expect an OCC group to overlap with more than one PUCCH without repetition.

    - OCC indication/configuration

For inter-slot OCC, OCC parameters such as OCC index and OCC length can be indicated and/or configured by network. For Type 1 configured grant (CG) PUSCH, the OCC parameters such as OCC index and OCC length can be configured via RRC signaling. For dynamic grant (DG) PUSCH or Type 2 CG PUSCH, the OCC parameters such as OCC enabler, OCC index and OCC length can be configured and/or indicated via both of RRC signaling and DCI. Specifically, the OCC index can be implicitly indicated by DM-RS port and the OCC length can be configured/indicated by using the time domain resource allocation (TDRA) list (e.g., pusch-TimeDomainAllocationList) because the OCC length should be jointly considered with the number of PUSCH repetitions.

Proposal #5: For PUSCH with OCC, RAN1 support OCC indication/configuration as follows:

- For Type 1 CG PUSCH,

  - OCC index and OCC length are configured by RRC signaling.

- For Type 2 CG PUSCH and DG PUSCH

  - OCC index is implicitly indicated by DM-RS port

  - OCC length is indicated by DCI among OCC length candidates configured in TDRA list

In addition, it can be further studied how to align OCC groups between different UEs. For example, the network can indicate/configure the start/end of an OCC application within a PUSCH repetition to align OCC sequences between multiple UEs. If this is not considered, the burden of aligning OCC groups between UEs may become purely on the network scheduler, reducing the opportunities for OCC application.

Proposal #6: For PUSCH with OCC, RAN1 consider the followings to align OCC groups between UEs.

- Signaling of offset to determine OCC start

    - OCC sequence

Unlike terrestrial network (TN), which allow cell planning through proper antenna tilting or sectorization in a small area, NTN serves a very large area through multiple footprints and/or satellite beams with a single satellite. Therefore, when receiving uplink transmission(s) (e.g., PUSCH) from NTN UE(s), inter-beam/cell interference can occur due to side-lobe in the receive beam pattern. Therefore, it was studied to avoid/mitigate the inter-cell/beam interference by frequency reuse factor (FRF) and/or RHCP/LHCP in NTN. When OCC is applied to PUSCH, the inter-cell/beam interference can be mitigated by randomization of OCC resource(s) among cells and/or satellite beams. As an example, when applying OCC to Type 1 configured grant (CG) PUSCH, the impact of interference can be reduced by applying different OCC patterns among cells/beams. Through this, when the number of NTN service UE(s) is sufficiently small, orthogonal OCC resources can be used to alleviate interference for PUSCH transmission between adjacent cells/beams.

Proposal #7: For PUSCH with OCC, RAN1 consider OCC randomization to mitigate UL interference among cell(s) and/or satellite beam(s).

    - TB over multiple slots (TBoMS)

For inter-slot OCC, the use of TB processing over multi-slot PUSCH (TBoMS) can be further enabled. When TBoMS is applied, a single TB can be transmitted over multiple slot(s) (e.g., N slots). In addition, if the number of repetitions of TBoMS is given (e.g., K), TBoMS can be transmitted repeatedly K times. Then, OCC can be applied across multiple TBoMS repetition(s). However, because the orthogonality of OCC may be vulnerable to RF impairments such as TO/FO, it may not be desirable to apply OCC over a long period of time on the time axis. Therefore, when applying TBoMS, it may be considered to improve the TBoMS transmission method to be more advantageous in maintaining OCC orthogonality. Specifically, each sub-block of a TB can be mapped to an OCC group and transmitted. For example, when one TB is divided into B0, B1, B2, and B3, each can be transmitted to OCC group 0, OCC group 1, OCC group 2, and OCC group 3, so that a total of 4 OCC groups are transmitted similar to how OCC is applied in multi-tone NB-IoT.

Proposal #8: For PUSCH with OCC, RAN1 consider TBoMS transmission based on OCC group.

- Conclusions

In this contribution, we discussed UL capacity/throughput enhancement by using OCC in Rel-19 NR NTN and the following proposal(s) are made:

Proposal #1: For PUSCH with OCC, RAN1 support RV cycling based on RV cycling granularity.

- RV cycling granularity is set as a multiple of the OCC length.

- FFS: how to determine RV cycling granularity

Proposal #2: For PUSCH with OCC, RAN1 support following UCI multiplexing rules:

- If the timeline conditions are met,

  - UCI is multiplexed on all PUSCH repetitions within an OCC group that overlaps with PUCCH without repetition.

- Otherwise,

  - It is left up to the UE implementation.

- For the timeline, the first symbol of the earliest PUSCH with the same OCC sequence as the overlapping PUSCH is considered.

Proposal #3: For PUSCH with OCC, RAN1 consider separating the beta offset parameter(s) between PUSCH without OCC and PUSCH with OCC.

Proposal #4: For PUSCH with OCC, UE does not expect an OCC group to overlap with more than one PUCCH without repetition.

Proposal #5: For PUSCH with OCC, RAN1 support OCC indication/configuration as follows:

- For Type 1 CG PUSCH,

  - OCC index and OCC length are configured by RRC signaling.

- For Type 2 CG PUSCH and DG PUSCH

  - OCC index is implicitly indicated by DM-RS port

  - OCC length is indicated by DCI among OCC length candidates configured in TDRA list

Proposal #6: For PUSCH with OCC, RAN1 consider the followings to align OCC groups between UEs.

- Signaling of offset to determine OCC start

Proposal #7: For PUSCH with OCC, RAN1 consider OCC randomization to mitigate UL interference among cell(s) and/or satellite beam(s).

Proposal #8: For PUSCH with OCC, RAN1 consider TBoMS transmission based on OCC group.

Reference

[1] RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3,” Thales, CATT, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

[2] RAN1 Chair’s Note, RAN1#118bis.

[3] RAN1 Chair’s Note, RAN1#119.