3GPP TSG-WG RAN1 Meeting #120	 R1-2501057

# Athens, 17th – 21th February, 2025

Title:	Introduction of WI “Time and Frequency Interleaving for LTE-based 5G Terrestrial Broadcast”

Source:	        EBU, SWR, BNE, ORS

Document for:	Decision

Agenda item:	9.13

# 1. Introduction

Time and frequency interleaving (TFI) are crucial components of wireless broadcast standards (i.e. DVB-T2, ATSC 3.0, DTMB, ISDB-T) [1, 2] to increase the transmission robustness in time-varying channels (e.g., mobile reception) and frequency selective channels (e.g., SFN deployments). Physical layer time interleaving provides significant performance benefits especially for high mobility use cases.

TFI for LTE-based 5G Terrestrial Broadcast was proposed in 3GPP during Rel-16 [3]. The basic approach is to reuse the building blocks behind HARQ retransmissions and combining, which allows to spread the coded bits of a transport block (TB) across multiple non-consecutive slots/subframes.The basic approach is to reuse the building blocks behind HARQ retransmissions and combining, which allows to spread the coded bits of a transport block (TB) across multiple non-consecutive slots/subframes.

# 2. Background

During the last RAN#106 meeting, a new WID [4] supporting TFI for LTE based 5G terrestrial broadcast has been approved with the following objective:

For MBMS-dedicated cells, specify time-frequency interleavers [RAN1] and corresponding signaling [RAN2, RAN3]

- For the time-interleaving part of 5G Broadcast TFI, one transport block is mapped to multiple non-consecutive subframes, following the principles of NR TBoMS. The following steps are followed:

  - Scale the TBS by a factor of N, where N is the number of subframes over which the TB is mapped.

    - RAN1 to discuss the details of the scaling, the value(s) for N and the set of subframes over which the TB is mapped, including how to schedule the TBs.

  - Within one subframe, the transmitted rate matched bits are read consecutively from the circular buffer.

    - In a given subframe, the procedure in TS 36.212 Section 5.1.4.1.2 applies (except for the parameter k0k0 which needs to be modified according to point 3 below)

    - Before frequency interleaving (if applied), the mapping from coded bits to RE follows the current specification.

  - Across subframes belonging to the same TB, the starting point for reading from the circular buffer (k0(k0 in TS 36.212 Section 5.1.4.1.2) is to be decided by RAN1

  - NOTE 1: The maximum TBS after scaling and soft buffer size should follow limitations imposed by current LTE categories. RAN1 to discuss how to capture this limitation.

  - NOTE 2: Time interleaving is not applied for PMCH carrying MSI and/or MCCH.

- For the frequency-interleaving part of 5G Broadcast TFI, interleaving is performed via a row-column interleaving with the following steps:

  - Data symbols in an OFDM symbol are written column-wise.

    - RAN1 to decide what is the unit over which data symbols are written (e.g. a single RE, four REs, etc.)

  - The rows and elements in each row may be permuted, with details to be decided by RAN1.

  - Data symbols in an OFDM symbol are read row-wise.

    - RAN1 to decide the number of {rows, columns} of the interleaver.

NOTE: Time-frequency interleaving is not applicable to 0.37 kHz SCS.

# 3. References

[1] R1-1913095, “Information on Time Interleaving in State-of-the-Art Terrestrial Broadcast Systems”, EBU, BBC, IRT, ABS, NHK, 3GPP RAN WG1, #99, Reno, USA, November 2019.

[2] Peter Klenner et al, “Physical Layer Time Interleaving for the ATSC 3.0 System,” IEEE Trans. on Broadcasting, Vol: 62, Issue: 1, pp. 253-262, March 2016.

[3] R1-1912692 “Numerology with 100us CP”, Qualcomm Incorporated, 3GPP RAN WG1, #99, Reno, USA, November 2019.

[4] RP-243290 “New WID on LTE-based 5G Broadcast Phase 2”, by EBU, ABS, BNE,Cellnex, Eutelsat, ORS, TDF, SWR, XGN, Ateme, CUC, Dolby, Enensys, IITB,  NERC-DTV,one2many B.V, Panasonic, Philips, Qualcomm, Rohde & Schwarz, SJTU, UPV/EHU, Xiaomi,  in RAN#106 Madrid, SPAIN, December 2024.