3GPP TSG RAN WG1 #120		             			            	             R1-2501082

Athens, Greece, February 17th –21st, 2025

Agenda Item:	9.7.2

Source:	AT&T

Title:	Discussions on ISAC Channel Modelling

Document for:     Discussion/Decision

# Introduction

A new study item on channel modeling for integrated sensing and communication was agreed in RAN#102 [1].

The study focuses on defining a common channel modeling framework to enable wireless sensing in 5G-advanced. The study item aims to support object detection and tracking use cases defined in SA1 [2], [3]. For those use cases, the following objectives are identified for this study:

- Identify details of the deployment scenarios corresponding to the above use cases.

- Define channel modelling details for sensing using 38.901 as a starting point, and taking into account relevant measurements, including:

- modelling of sensing targets and background environment, including, for example (if needed by the above use cases), radar cross-section (RCS), mobility and clutter/scattering patterns.

- spatial consistency.

In this contribution, we discuss the channel modelling framework for enabling ISAC and subsequently the modifications needed in the 3GPP channel model, following the agreements made in RAN1#119.

# Sensing Target Modelling

In RAN1#119 [4], agreements were reached regarding the framework of modeling the RCS of the sensing targets.

Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

Agreement

Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1 = 0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Further agreements are needed to establish the framework for modeling humans, vehicles, for bistatic and monostatic sensing, as well as differentiate between single and multi-scattering points.

## RCS Modelling

In radar technology, radar cross section (RCS) is used to quantify the scattering behavior of the target. The RCS is a function of the target viewing angle relative to the transmitter and receiver antenna and of the frequency and polarization of the incident electromagnetic wave. The RCS is a measure of how much of the incident wave is intercepted by and reflected from the target, as well as how much of that is directed towards the radar receiver. The RCS depends on the size and shape of the target, and the materials from which the target is made on the outer surface.  RCS is expected to similarly be used to quantify the scattering behavior of the target in the ISAC channel model.

Based on our ISAC measurement campaign, we propose values for the parameters for the different models of vehicle and pedestrian RCS.  Our ISAC measurements campaign considered two different targets, (i) a large vehicle and (ii) a pedestrian target. The details of the measurement campaign are found in Section 2.1.2.

### RCS Findings from Experimental Modelling

To extract RCS modeling from the measurement data, we assume the direct path is formed by a concatenation of the Tx-target and target-Rx channels, and the combined pathloss at the receiver is expressed as a summation of the PL from the Tx-target, and the PL from the target to the Rx plus the effect of the RCS.

We observe that the measured RCS has significant variation as a function of the bistatic angle and the heading angle, for the case of a large vehicle target. The variation is less discernable for the case of pedestrian target. This is consistent with the agreement from RAN1#119 on the vehicle RCS angular dependency captured in B1.

We conducted both bistatic BS-UE and monostatic BS-BS experiments, and we observed the variation of RCS for both bistatic and monostatic experiments. We observed that the RCS distribution varies as a function of the sensing mode.

In our experiments, we collected measurements for a wide range of incident and scattered angles from the targets. For bistatic experiments, we separated the collected data into three bistatic regions: (i) forward scattering, (ii) bistatic, and (iii) pseudo monostatic. We collected monostatic measurements, from collocated Tx/Rx at the gNBs. Our monostatic measurements agreed with the pseudo-monostatic region of the bistatic measurements.

Observation 1: The RCS distribution is different when considering bistatic BS-UE versus monostatic BS-BS sensing.

Observation 2: The RCS distribution in pseudo-monostatic region of the bistatic measurements and the monostatic measurements generally match.

There was also a discussion in RAN1#119 if monostatic to bistatic equivalence formulas in the literature can be used to compute bistatic RCS for the ISAC channel model. These equivalence formulas usually have a wide margin of error as the bistatic angle increases.

Observation 3: Monostatic to bistatic equivalence formulas have a large margin of errors as the bistatic angle increases.

Furthermore, ray tracing is not a good way to compute bistatic RCS, since ray tracers themselves need to be fed with real information to compute the RCS. It is thus preferred to rely on measurements to derive the RCS distribution for bistatic sensing modes.

Proposal 1: The bistatic RCS distribution for the ISAC channel model is derived from measurements.

In our measurements, to compute a distribution of the RCS based on a simplified model, we removed the effect of the forward scattering region from our measurements. For bistatic RCS evaluations, we used the measurements collected from the bistatic, and the monostatic region, to produce the log normal distribution statistics.

Observation 4: Forward scattering region measurements are not used in the bistatic RCS distribution modeling.

Proposal 2: Bistatic RCS derived from measurements excludes the forward scattering region.

Forward scattering region is important to consider/model in ISAC channel modelling. Measurements in this region are challenging to get a reliable understanding of the effect of forward scattering on the RCS. We propose companies to use EM theory or modeling to understand and model the effect of forward scattering on the RCS for sensing, and include the analytical model in the channel model as optional.

Proposal 3: Forward scattering effect on RCS and channel modelling is studied and included as optional in the ISAC channel model.

#### RCS for human target

For a pedestrian target, according to our measurements, for both monostatic and bistatic measurements,

little dependency is observed on the angle between the Tx-target-Rx  (e.g. bistatic angle) or heading angles, and a log normal distribution can be used in the B component, according to the option 1 in the framework agreement in 119.

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1 = 0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

For monostatic sensing, we propose the following RCS modeling for a human target, following the option 1 agreement from RAN1#119.

Proposal 4: For monostatic sensing, to model the RCS of an adult human target with single scattering point

  - A is mean RCS value given by -6.2 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 10.0

For bistatic sensing, we propose the following RCS modeling for a human target.

Proposal 5: For bistatic sensing, to model the RCS of a adult human target with single scattering point

  - A is mean RCS value given -14.4 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 6.7

#### RCS for a large vehicle target

Modeling RCS of a large vehicle target is more controversial than that of a UAV or a human target, since it involves multiple reflection points, and some companies are showing the need to model it as a multi-point scattering target. In RAN1#119, two agreements were made depending on whether the large vehicle is modeled as a multi-scattering or a single scattering point model.

For an outdoor use case, with vehicle detection and tracking, and for the Tx and the Rx sufficiently far from the target, we think modeling a large vehicle as a single scattering point is sufficient. A multi-scattering point can be used optionally for certain use cases where a more accurate representation of the vehicle model is used.

Proposal 6: For the ISAC channel model, to model the sensing target, for object detection and tracking, single scattering point is used by default. Multi-point scattering models can be optionally considered depending on the use case and the type/size of the target.

For a single scatterer model for a large vehicle target, we propose the following RCS modeling framework for A, and B2 based on our measurements.

For monostatic sensing,

Proposal 7:  For monostatic sensing, to model the RCS of a large vehicle with single scattering point

  - A is mean RCS value given by 7.7 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 8.4

For bistatic sensing,

Proposal 8:  For bistatic sensing, to model the RCS of a adult human target with single scattering point

  - A is mean RCS value given -0.1 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 6.1

In the case where multi-point scattering is considered, and the five scattering points are used to model the vehicle, in the front, left, back, right and roof side of the vehicle, the following proposal from RAN1#118bis[4] can be agreed, with modifications.

Proposal 9: If a target is modelled with multiple scattering points,

- The number of scattering points of a target is generated at the beginning of the simulation and is kept unchanged for the rest of the simulation

- The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- The RCS value of the multiple scattering points is individually determined

### Experimental Validation

Outdoor experiments were conducted at ATT’s Spectrum Drive facility using a gNB (h=10 m), a mobile UE (hUT=0.5 m), and two targets: (i) a Ford Transit (Target 1) consumer vehicle and (ii) a pedestrian (Target 2). A 400 MHz OFDM (120 kHz SCS) signal at fc=6.7 GHz was transmitted at +40 dBm EIRP to illuminate all targets on test track. UE was stationary at a fix location while targets were moved around the UE in a predetermined track. The position of gNB, UE and all targets were precisely tracked and time-synchronized for the duration of the experiments using a GPS with 1 cm resolution.

Figure 1 shows the trajectories of target 1(green) and Target 2 (yellow) for each of the two UE positions. For these experiments, a vertically-polarized standard horn with 3dB-BW of 60° in azimuth and elevation antennas was used at gNB which was oriented southward (North is up) . Complex channel impulse responses were measured and were recorded every 1ms. The target scattered power was then evaluated based on our model of the target scattered channel.

Figure 1. Vehicle and pedestrian track for UE position 1 (left) and UE Position 2 (right). Vehicle track is shown in green and pedestrian track is shown in yellow.

Figure 2: Target scattering model geometry

Here, we model target pathloss PLt using the following equation.

PLt=PL1-Gs+PL2 dB

Gs= γ+10log104πλ2

Where γ is dBm2 and λ is the wavelength in m. In our model, PL1 and PL2 are based on free space pathloss (FSPL) as a function of d1 and d2. For monostatic sensing, d1 = d2   . Here, we assume UE is within LOS of target. This was enforced during the measurement campaign.

As is well known in radar community, measured RCS γ has significant variation vs. bistatic angle θB and heading angle θH. Figure 3 shows how measured RCS varies for target 1 in the pseudo-monostatic (small θB) region vs. heading angle θH for bistatic based sensing. In general, variations in RCS are highly dependent on target composition (type) and shape. In this case, the box-like structure of the Ford Transit leads to peaks at multiples of 90.

Figure 3. Heading angle dependence of RCS for vehicular target in pseudo-monostatic (small θB) region. Bright scattering (high RCS) is observed when planar surfaces of vehicle (front, side, rear) are oriented perpendicular TRX pointing direction. Rear of vehicle has more coplanar area than front, resulting in higher RCS for θH≈180° vs θH≈0°.

Figure 4 depicts how the measured RCS varies as a function of the heading angle in mono-static based sensing for targets 1 and 2, respectively.

(a)

(b)

Figure 4: heading angle dependence of RCS for (a) vehicular target (b) pedestrian target for mono-static sensing

(a)

(b)

Figure 5: RCS dependence on bistatic angle for (a) vehicular target, (b) pedestrian target.

Figure 5 shows a snapshot of the RCS for bistatic sensing measurements as a function of bistatic angle θBfor targets 1 and 2, respectively. The difference between the nature and the shape of the target between the vehicle and the pedestrian is noticeable in the distribution of the RCS with respect to the bistatic angle.

To simplify the model, we propose to model γ as a random variable with conservative values (taken from bistatic region of θB). Additional measurements may be necessary to further refine this model.

Using the model described above, we fit γ to a lognormal random variable with parameters shown in Table 1 for bistatic sensing, and Table 2 for monostatic sensing. Figure 6 shows a good fit to measured data that is less optimistic in pseudo monostatic region (large sqrt(d1d2)).

Table 1: Lognormal parameters for two target classes based on measurement data for bistatic sensing

| Target class | μ (dBsm) | σ (dBsm) |
| --- | --- | --- |
| Passenger car | -0.1 | 6.1 |
| Pedestrian | -14.4 | 6.7 |

Table 2: Lognormal parameters for two target classes based on measurement data for monostatic sensing

| Target class | μ (dBsm) | σ (dBsm) |
| --- | --- | --- |
| Passenger car | 7.7 | 8.4 |
| Pedestrian | -6.2 | 10.0 |

Figure 6. Measured pathloss components of total pathloss PL and target-scattered pathloss PLt as a function of geometric mean distance. Simulated target-scattered pathloss PLt is shown in green.

# Background Channel Modelling

There are at least two outstanding issues to be considered for the modelling of the background channel for the ISAC channel model.

One of the issues is related to the modelling of the background channel versus the target channel, and how eventually the two components will be used to assess the performance of sensing, or integrated communication and sensing.

Proposal 10: The background channel is modelled with stochastic clusters following the step wise procedure in TR38.901 as a starting point.

One of the issues raised was the need to revisit the background channel in TR38.901 and the power of the stochastic clusters used to model that channel to make sure that the sensing performance is computed as expected and the sensing KPIs defined in SA are met.

Proposal 11 : When needed, the background channel stochastic clusters generation in TR38.901 are revisited to compute proper KPIs for the ISAC channel.

Another issue in the background channel modelling is related to whether all sensing modes are covered in TR38.901 and other related TRs on channel modelling, at least to be used as a starting point for the ISAC channel. TR38.901 captures the modelling of the channel between a TRP node and a UE node, while other related study items considered extensions of this modelling to TRP-TRP channels as well as UE to UE channels in the case of V2X. The latter channel extensions/modifications have not, however, been calibrated or validated with measurements.

The following agreements were made in RAN1#118 on the background channel modeling.

Agreement

For modeling stochastic cluster in background channel, in order to define the background channel for TRP-UE and UE-TRP bistatic sensing mode,

- The large scale and small scale parameters defined in TR 38.901, TR 37.885, TR 36.777 are used as start point

In order to define the background channel for TRP-TRP and UE-UE bistatic sensing mode,

- RAN1 to study how to model background channel

  - Option 1: The large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as start point

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results

FFS whether/how to do power normalization between target channel and background channel

Agreement

In order to define the background channel for TRP and UE mono-static sensing mode,

- RAN1 to study how to model the background channel

  - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario

    - FFS EO is modelled in the background channel

  - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures

    - FFS EO is modelled in the background channel

  - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations.

  - Other options are not precluded

For TRP-TRP bistatic channels for example, the TRP-UE channel was modified by considering a UE at the same height of the UE, and modifying the distribution of the AoA/ZoA to match that of AoD/ZoD for SFBD evaluations. These channel modifications were never validated with measurements, especially since the TRP locations and scattering around the TRPs are very different from those at the UEs.

For TRP-TRP monostatic sensing, the availability of proper channel models or reuse of channel models in TR38.901 or TR38.558 is even more questionable: the direct path cannot be modeled in the same way as in TR38.901. Indirect paths (for target or background channels) are also affected, especially given the height changes from the UE to the gNB, affecting the power of the clusters and the delays, when modelling using stochastic clusters, at least for the background channel.

Observation 5: The TRP-TRP channel, used in TR38.901, needs verification/validation to be used for sensing use cases

Observation 6: For TRP-TRP monostatic sensing, it is crucial to validate the channel to be used for both direct and indirect paths.

Observation 7: Validation of the TRP-TRP background channel model through measurements, at least for the monostatic sensing case, will be time consuming and might not be feasible within this SI.

Proposal 12: At least for TRP-TRP monostatic model, careful consideration is given to reusing TR38.901 LSPs and SSPs without proper validation through measurements.

# Methodologies and General Framework

For ISAC channel modelling, the modified channel model in TR 38.901 [6] can still follow the step wise procedure detailed in section 7 with modifications to include target 3D location and RCS modelling, target mobility in addition to the transmitter and receiver mobility, and spatial consistency.

The current 3GPP channel model supports spatial consistency when the devices are located within a correlation distance from each other, expecting to see similar channel conditions, and when the devices are mobile, accounting for smoother variations in the channel resulting from variations in the UE positions. For sensing, spatial consistency becomes even more important, and should be included as an integral (non-optional) part of the channel model procedure. When the sensing receiver and the sensing target are located close to each other, or when multiple sensing devices are located close to each other in the case of multi-static sensing (multiple sensing TXs or multiple sensing RXs), it is important for the channels to be spatially consistent to ensure good sensing performance. Furthermore, when the target is mobile, and moves within a given area, the channel variations should reflect smooth variations resulting from target mobility. Spatial consistency for both LSPs and SSPs is thus important to be included in the channel model for ISAC.

The following agreement has been reached in RAN1#117 regarding spatial consistency in the ISAC channel modeling:

Agreement

- Spatial consistency should be supported for ISAC channel

- Spatial consistency should be supported based on movement of sensing Tx, sensing target and/or sensing Rx

    - FFS EO handling

The spatial consistency procedure for ISAC should also make sure that the modelling considers the composition of the ISAC channel between target channel and background channel. Similarly, the spatial consistency procedure will consider how the channel is modelled, e.g. as a concatenation between the Tx-target and target-Rx links. For the latter case, spatial consistency between the Tx-target and target-Rx links should be established.

Furthermore, as a starting point, at least for the TRP-TRP bistatic sensing modes, spatial consistency can reuse the consistency procedure detailed in TR38.901 for the sensing target.

Proposal 13: For ISAC channel modelling, spatial consistency considers

- the characteristics of the background and target channels,

- the correlation between Tx-target and target-Rx links when the channel is modeled as a concatenation between Tx-target and target-Rx links

Proposal 14: For ISAC channel modelling, the spatial consistency procedure in TR38.901 can be used as a starting point, at least for TRP-TRP bistatic sensing mode.

# Conclusion

In this contribution, we discussed the general framework and methodology for developing an ISAC channel model. We made the following proposals:

Observation 1: The RCS distribution is different when considering bistatic BS-UE versus monostatic BS-BS sensing.

Observation 2: The RCS distribution in pseudo-monostatic region of the bistatic measurements and the monostatic measurements generally match.

Observation 3: Monostatic to bistatic equivalence formulas have a large margin of errors as the bistatic angle increases.

Observation 4: Forward scattering region measurements are not used in the bistatic RCS distribution modeling.

Observation 5: The TRP-TRP channel, used in TR38.901, needs verification/validation to be used for sensing use cases

Observation 6: For TRP-TRP monostatic sensing, it is crucial to validate the channel to be used for both direct and indirect paths.

Observation 7: Validation of the TRP-TRP background channel model through measurements, at least for the monostatic sensing case, will be time consuming and might not be feasible within this SI.

Proposal 1: The bistatic RCS distribution for the ISAC channel model is derived from measurements.

Proposal 2: Bistatic RCS derived from measurements excludes the forward scattering region.

Proposal 3: Forward scattering effect on RCS and channel modelling is studied and included as optional in the ISAC channel model.

Proposal 4: For monostatic sensing, to model the RCS of an adult human target with single scattering point

  - A is mean RCS value given by -6.2 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 10.0

Proposal 5: For bistatic sensing, to model the RCS of a adult human target with single scattering point

  - A is mean RCS value given -14.4 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 6.7

Proposal 6: For the ISAC channel model, to model the sensing target, for object detection and tracking, single scattering point is used by default. Multi-point scattering models can be optionally considered depending on the use case and the type/size of the target.

Proposal 7:  For monostatic sensing, to model the RCS of a large vehicle with single scattering point

  - A is mean RCS value given by 7.7 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 8.4

Proposal 8:  For bistatic sensing, to model the RCS of a adult human target with single scattering point

  - A is mean RCS value given -0.1 dBsm

  - B2 is modelled using a log-normal distribution with mean 1 and variance 6.1

Proposal 9: If a target is modelled with multiple scattering points,

- The number of scattering points of a target is generated at the beginning of the simulation and is kept unchanged for the rest of the simulation

- The number and locations of the scattering points are common to each pair of sensing Tx/Rx

- RAN1 assumes no ray is scattered from one scattering point to another scattering point of the same target

- The RCS value of the multiple scattering points is individually determined

Proposal 10: The background channel is modelled with stochastic clusters following the step wise procedure in TR38.901 as a starting point.

Proposal 11: When needed, the background channel stochastic clusters generation in TR38.901 are revisited to compute proper KPIs for the ISAC channel.

Proposal 12: At least for TRP-TRP monostatic model, careful consideration is given to reusing TR38.901 LSPs and SSPs without proper validation through measurements.

Proposal 13: For ISAC channel modelling, spatial consistency considers

- the characteristics of the background and target channels,

- the correlation between Tx-target and target-Rx links when the channel is modeled as a concatenation between Tx-target and target-Rx links

Proposal 14: For ISAC channel modelling, the spatial consistency procedure in TR38.901 can be used as a starting point, at least for TRP-TRP bistatic sensing mode.

# References

[1] RP-240799 Revised SID: Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR, Xiaomi, AT&T.

[2] 3GPP TR22.837 Feasibility Study on Integrated Sensing and Communication

[3] 3GPP TS 22.137 Service Requirements for Integrated Sensing and Communication, Stage 1, Rel. 19

[4] R1-2409224 Session notes for 9.7 Study on channel modelling for Integrated Sensing and Communication (ISAC) for NR, Ad-Hoc Chair (Huawei), RAN#118-bis, October 2024.

[5] 3GPP TR38.901 Study on channel model for frequencies from 0.5 to 100 GHz

[6] R1-2409280 Summary #4 on ISAC channel modelling v49 Moderator, Moderator (Xiaomi), October 2024