3GPP TSG RAN WG1 #120                                   R1-2501093

Athens, Greece, February 17th - 21st, 2025

Source:	Sharp

Title:	Discussion on coding aspects

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

A new work item on Ambient IoT [1] for Rel-19 was approved in RAN#106, with objectives including the following:

- RAN1 scope: - [...] - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this document, we share our views on a few coding aspects of the A-IoT physical layer channel design.

Discussion

Aspects common to R2D and D2R

CRC attachment

Use of CRC-16 can be based on the number of bits (Z) prior to CRC attachment, i.e. when Z > 24 bits, as proposed by companies during the study item phase.

- CRC-16 is used if Z > 24, where Z is the number of bits prior to CRC attachment.

- Use of no CRC or CRC-6 may be a little more complex and should be more carefully considered. For R2D, the decision should only be made after RAN1 decides which option to use for determination of R2D end timing (i.e. either “Option 1: TBS information via implicit/explicit L1 R2D control information”, or “Option 2: Postamble at the end of PRDCH”).

For R2D, use of no CRC or CRC-16 is dependent on which option to use for determination of R2D end timing.

D2R (Device-to-reader)

Line coding and small frequency shift

Since Manchester line code is used in R2D, the same line code is proposed to be used in D2R, such that a single line coding scheme is supported in the device. Regarding the support for small frequency shift, we think the Manchester line code Option 1 is the simplest in terms of specification work and should thus be adopted.

- Manchester line code is always used for a D2R transmission.

- Regarding the support for small frequency shift of a D2R transmission, only Manchester line code Option 1 is supported, i.e. each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length).

- For a number of D2R transmissions multiplexed on a same D2R transmission occasion, FDMA can be achieved by allocating a distinct small frequency shift factor (as defined in the TR [2]) to each D2R transmission, in the D2R grant conveyed in the scheduling R2D transmission. The small frequency shift factors can be pre-defined for the D2R bandwidth used by those D2R transmissions. RAN1 should discuss a maximum amount of small frequency shift value (or, a corresponding smallest D2R chip length) taking into account the processing capability of Device 1.

- Each D2R transmission is associated with a small frequency shift factor which is allocated in the D2R grant conveyed in the scheduling R2D transmission.

- The small frequency shift factor is one of the small frequency shift factors pre-defined for the D2R bandwidth of the D2R transmission.

A maximum amount of small frequency shift value (or, a corresponding smallest D2R chip length) should be discussed and decided in RAN1, taking into account the processing capability of Device 1.

- For a number of FDM’ed D2R transmissions, the respectively allocated small frequency shift factors (and corresponding D2R frequency resources) should be spaced as far apart as possible, in order to ensure proper frequency gaps especially considering the SFO impact.

For a number of FDM’ed D2R transmissions, the respective small frequency shift factors (and corresponding D2R frequency resources) should be allocated such that they are spaced as far apart as possible.

Repetition

Considering the gains of block level repetition (e.g. combining gain, and time diversity gain) when comparing with no repetition and bit level repetition, as captured in the TR [2], it seems natural that block level repetition should be supported for a D2R transmission. The number of repetitions (including the case of no repetition) can be either directly indicated by a scheduling R2D transmission, or can be derived based on the time resource allocated for the D2R transmission.

- Block level repetition is supported for a D2R transmission.

- The case of no repetition (i.e. number of repetitions = 1) is supported.

- Bit level repetition is not supported for any D2R transmission.

- The following options are considered for determination of the number of block level repetitions

- Option 1: The number of block level repetitions is indicated by a D2R grant conveyed in a scheduling R2D transmission.

- Option 2: The number of block level repetitions is derived based on the time resources allocated by a D2R grant conveyed in a scheduling R2D transmission.

Conclusion

In this contribution, we discuss a few coding aspects of the A-IoT physical layer channel design, and make the following observations and proposals.

- For R2D, use of no CRC or CRC-16 is dependent on which option to use for determination of R2D end timing.

A maximum amount of small frequency shift value (or, a corresponding smallest D2R chip length) should be discussed and decided in RAN1, taking into account the processing capability of Device 1.

For a number of FDM’ed D2R transmissions, the respective small frequency shift factors (and corresponding D2R frequency resources) should be allocated such that they are spaced as far apart as possible.

- CRC-16 is used if Z > 24, where Z is the number of bits prior to CRC attachment.

- Manchester line code is always used for a D2R transmission.

- Regarding the support for small frequency shift of a D2R transmission, only Manchester line code Option 1 is supported, i.e. each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length).

- Each D2R transmission is associated with a small frequency shift factor which is allocated in the D2R grant conveyed in the scheduling R2D transmission.

- The small frequency shift factor is one of the small frequency shift factors pre-defined for the D2R bandwidth of the D2R transmission.

- Block level repetition is supported for a D2R transmission.

- The case of no repetition (i.e. number of repetitions = 1) is supported.

- Bit level repetition is not supported for any D2R transmission.

- The following options are considered for determination of the number of block level repetitions

- Option 1: The number of block level repetitions is indicated by a D2R grant conveyed in a scheduling R2D transmission.

- Option 2: The number of block level repetitions is derived based on the time resources allocated by a D2R grant conveyed in a scheduling R2D transmission.

References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), RAN#106.

- TR 38.769, “Study on Solutions for Ambient IoT (Internet of Things)”, v19.0.0.