3GPP TSG RAN WG1 #120			    R1-2501155

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.3.3

Source: 	Qualcomm Incorporated

Title: 	Enhancements for CLI handling

Document for:	Discussion/Decision

# Introduction

In RAN plenary #102, the work item on evolution of NR duplex operation, Sub-band full duplex (SBFD), was approved [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier.

In addition, related to enhancements for CLI handling which include the common solutions for both SBFD and dynamic/flexible TDD scenarios, the main objective of the first three R19 RAN1 meetings is to:

1) Down-select the detailed scheme(s) for gNB-to-gNB co-channel CLI handling scheme(s).

2) Down-select the detailed scheme(s) for UE-to-UE co-channel CLI handling scheme(s).

In RAN1 #117 meeting, RAN1 agrees on the down selection schemes for inter-gNB and inter-UE CLI handling.

Agreement For enhancements for CLI handling, the following are recommended from RAN1's perspective: - Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3] - Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3] - Information exchange of strongest DL beam information [RAN3] - Information exchange of CLI-mitigation request [RAN3] - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3. - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - Note: This feature is subject to UE capability - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - Note: At least wideband reporting is supported - UCI bits generation - Priority rules for multiple CSI reporting - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point. - CLI measurement accuracy requirement [RAN4] - Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

In the RAN plenary #104 meeting, RAN-P approves RAN1 agreed down-selection schemes for CLI handling and revised the WID in RP-241614 [1] accordingly as shown below:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution we discuss the further details for each down selected CLI handling scheme for both inter-gNB and inter-UE CLI for agenda 9.3.3.

# Inter-UE CLI handling schemes

In RAN1 #117 meeting, companies agreed to down select the following inter-UE CLI scheme to specify in R19 work item.

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs. - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource - Measurement reporting - At least aperiodic reporting - Note: Periodic and semi-persistent reporting can be considered - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - Note: At least wideband reporting is supported - UCI bits generation - Priority rules for multiple CSI reporting - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point. - CLI measurement accuracy requirement [RAN4]

## L1 based UE-to-UE CLI measurement and reporting

### L1 CLI measurement resources

In RAN1 #119 meeting, RAN1 agreed that:

Agreement Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other configurations Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters

RAN1 agreed to specify L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework and in particular, specify configuration of periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource.

For SRS-RSRP resource, reusing existing SRS patterns was agreed to minimize the spec impact with the existing SRS comb patterns.

For CLI-RSSI resource, R16 has defined CLI-RSSI pattern for L3 UE to UE CLI measurement, which configures contiguous frequency resources via start PRB and number of PRBs and also configures time resources via start position and number of symbols. Reusing existing contiguous CLI-RSSI pattern was agreed which can minimize the spec impact and reduce the UE implementation complexity.

In addition, in last RAN1 119 meeting, there were proposals as shown of FFS to discuss other parameters such as determination/configuration of frequency reference point and sub-carrier spacing (SCS) for L1 inter-UE CLI SRS-RSRP and CLI-RSSI measurement resources. For the case that UE measures CLI SRS-RSRP and CLI-RSSI measurement resources on the same CC as the CSI-ReportConfig, to align with legacy structure for CSI-RS resource, in our view, SRS-RSRP-MeasurementResourceSet or CLI-RSSI-MeasurementResourceSet can be defined per cell under ServingCellConfig IE under existing csi-MeasConfig. Based on aggressor UE’s SCS configuration, gNB can configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource. In addition, since it is under per cell configuration, then frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement is based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid, which is the same rule as legacy CSI-RS resource.

Proposal 1: Support SRS-RSRP-MeasurementResourceSet or CLI-RSSI-MeasurementResourceSet to be defined per cell under ServingCellConfig IE, under existing csi-MeasConfig.

Proposal 2: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource.

Proposal 3: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

In addition, UE may measure CLI SRS-RSRP and CLI-RSSI measurement resources on a different CC as the CSI-ReportConfig, and in our view, UE can reuse same “carrier” parameter in CSI-ReportConfig IE to indicate the CLI measurement CC. Then similar as CLI measurement on the same CC, support configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource, and support frequency reference point to find the first RB for CLI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid of the carrier/cell that UE measures the CLI.

Proposal 4: Support reuse same “carrier” parameter in CSI-ReportConfig IE to indicate the CLI measurement CC.

Proposal 5: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource for cross CC CLI measurement.

Proposal 6: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid of the carrier/cell that UE measures the CLI for cross CC CLI measurement.

In addition, to measure CLI-RSSI in different time slots, gNB can trigger a time window with one or more CLI-RSSI resources configured per slot in the time window with multiple slots. So that UE can report the strongest or weakest N measurement resources on N slots in the time window, which can be associated with the strongest or weakest N aggressor UEs e.g. if different aggressor UEs transmit in different slots scheduled by gNB. With this, it can measure CLI levels and also differentiate different aggressor UEs via CLI-RSSI measurements over different time occasions.

Proposal 7: RAN1 to discuss AP triggered a CLI measurement time window with one or more CLI-RSSI resources configured per slot in the time window with multiple slots to perform CLI measurement and differentiate different aggressor UEs over different time occasions.

### QCL type D determination and configuration

In RAN1 #118bis and RAN1 119 meeting, RAN1 agreed that:

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic (AP) L1 based UE to UE CLI reporting based on a set of aperiodic, periodic, or semi-persistent CLI measurement resources, RAN1 agreed as above on the configuration and determination of QCL-D.

Then, for aperiodic L1 based UE to UE CLI reporting based on aperiodic CLI measurement resources, RAN1 needs to discuss default beam rules to be defined in spec to handle the case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming. The same issue exists for legacy AP CSI-RS, and therefore, default beam rules are defined in spec in 38.214 for AP CSI-RS. To simply the spec impact, in our view, we support to treat this as an error case that UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming. Alternative solution can be that same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource to handle the similar case when scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

Proposal 8: RAN1 to discuss default beam rules to handle the case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming:

- Option 1: An error case that UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Option 2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

### L1 CLI reporting

In RAN1 119 meeting, RAN1 agreed that:

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

For L1 based UE to UE CLI reporting, new report quantities for CLI reporting was agreed to be configured in the report configuration by the higher layer parameter reportQuantity, either cli-SRS-RSRP or cli-RSSI.

For CLI SRS-RSRP reporting, L1-RSRP range and resolution defined in RAN1 spec can be reused. In RAN1 spec, L1-RSRP is defined that the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value, which is part of the same L1-RSRP reporting instance. In addition, RAN4 spec defined R16 L3 SRS-RSRP with the same range and resolution as L1-RSRP that the reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. In our view, L1 CLI SRS-RSRP can reuse the same range and resolution and therefore, reuse the same number of bits for the absolute value with 7 bits and differential values with 4 bits.

For CLI RSSI reporting, R16 L3 CLI RSSI range and resolution defined in RAN4 spec can be reused. In R16, RAN4 spec defines that UE shall report the CLI-RSSI value as a 7-bit value in the range [-100, -25] dBm with 1dB step size. In our view, L1 CLI RSSI can reuse the same range and resolution and therefore, reuse the same number of bits for the absolute value with 7 bits. With differential reporting, similar as CLI SRS-RSRP, differential values can also use 4 bits with 2 dB step size with a reference to the largest measured CLI-RSSI value.

Proposal 9: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support reuse L1-RSRP and R16 SRS-RSRP same range, resolution, and number of bits for L1 CLI SRS-RSRP reporting, and reuse R16 CLI-RSSI same range, resolution and number of bits for L1 CLI-RSSI reporting.

For SRS-RSRP, a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential SRS-RSRP is quantized to a 4-bit value. The differential SRS-RSRP value is computed with 2 dB step size with a reference to the largest or smallest measured SRS-RSRP value.

For CLI RSSI, a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential CLI RSSI is quantized to a 4-bit value. The differential CLI RSSI value is computed with 2 dB step size with a reference to the largest or smallest measured CLI RSSI value.

Therefore, for CLI reporting, same payload structure and report format as L1 beam reporting e.g. with L1-RSRP as reportQuantity can be reused as shown in below table in spec 38.214. That is, UE reports up to four CLI measurement resources. Four CLI measurement resources can correspond to different UEs, or different Rx beams, or different Tx beams, for CLI measurements.

In addition, a new name different than CRI but serving the same purpose as a local resource index for CLI measurement resources in CLI reporting shall be defined. And a new payload structure and report format table for CLI reporting similar as L1 beam reporting table shall be defined as well.

Figure 2-1 Example payload structure for L1-RSRP reporting.

Proposal 10: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, RAN1 to reuse report format as L1 beam reporting.

- FFS: a new local resource index for CLI measurement resource in CLI reporting

- FFS: a new payload format table for CLI reporting similar as L1 beam reporting table.

In addition, for either CLI SRS-RSRP or CLI RSSI measurement and reporting in UL subband, there can be blocking e.g. from an aggressor UE that are very close to a victim UE, in which case UE can not detect SRS or interference signal due to too strong signal to measure due to receiver saturation. This is related to FFS of the above agreement of last RAN1 meeting that whether/how to report differential value in case the largest measurement is out of range. This case could happen, and in R16, SRS-RSRP measured quantity value table in spec, a bit-code (98) for infinity was defined to report that UE cannot detect SRS due to too strong signal to measure. For R16 with L3 reporting, this case will not cause any issue with all absolute values reported. However, for R19 L1 based reporting, a differential reporting method was agreed in RAN1, and this case will cause an issue to report differential value in case the largest measurement is out of range. Therefore, RAN1 shall discuss how to report CLI when blocking happens, how to report differential value in case the largest measurement is out of range and the corresponding report payload.

In our view, one solution can be that when the largest measurement is out of range, then calculate the differential values based on the largest CLI value in the range which is -44 dBm for CLI SRS-RSRP reporting, so UE will report other CLI resources in the same differential report with refer to -44 dBm as the absolute value. An alternative solution can be when the largest measurement is out of range that blocking from an aggressor UE is detected, then UE will not report other CLI resources in the same report, e.g. other CLI resources reported as dummy value if more than 1 CLI resource is configured for the report.

Figure 2-2 L3 SRS-RSRP measured quantity value table for L3 SRS-RSRP reporting.

Proposal 11: RAN 1 to discuss how to report differential value in case the largest measurement is out of range:

- Option 1: UE calculates the differential values based on the largest CLI value in the range which is -44 dBm for CLI SRS-RSRP reporting.

- Option 2: UE will not report other differential values for other CLI resources in the same report, e.g. other CLI resources reported as dummy value if more than 1 CLI resource is configured for the report.

In last RAN1 meeting, for L1 based UE-to-UE CLI measurement and reporting, RAN1 agreed that the absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported and FFS on other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP.

In our view, UE can report either most interfering or least interfering N CLI resources to aid in gNB scheduling and UE pairing based on different scenarios. For example, if UE reports the least interfering N aggressor UEs, gNB can schedule DL UE and corresponding UL UE chosen from the least interfering N aggressor UEs. For another example, if UE reports the most interfering N aggressor UEs, gNB can avoid scheduling DL UE pairing with corresponding UL UE on the most interfering aggressor UE list.

One drawback for reporting the most interfering CLI resources, is that, if gNB configures to report top two most interfering CLI resources, it could be possible that the third most interfering CLI resources which is not included in the report still causes significant CLI impact to the measurement UE. In this case, if gNB only avoids scheduling DL UE pairing with corresponding UL UE on the most interfering aggressor UE list, it is not enough to avoid the CLI impact with the potential possibility that gNB may schedule the DL UE with the third high CLI UL UE for full duplex operation, which is different than L3 with higher payload size that can report more CLI resources.

Therefore, in our view, least interfering CLI resources in the reportConfig shall also be supported for CLI reporting. In this case, the aggressor UEs triggered for measurements by gNB shall be the UEs that gNB intends to schedule and pair with the victim UE, so that gNB can schedule one of the reported least interfering UE to pair with victim UE based on the victim UE’s CLI reporting. gNB can configure the reporting criteria based on e.g. different deployment scenarios and UE density as either most interfering or least interfering CLI resources to report.

Proposal 12: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support configuration of reporting criteria of {most interfering, least interfering} CLI resources in the reportConfig.

In RAN1 #118bis meeting, RAN1 agreed that:

Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Regarding the FFS on whether available slot offset list is also reused, in our view, available slot offset shall not be considered for CLI measurement and the whole FFS shall be removed. First of all, SRS available slot is designed for SRS transmission by counting only UL/FL slot that can accommodate SRS transmission, and it is not motivated for CLI measurement. Secondly, physical slot counting should be good enough to serve CLI measurement purpose and gNB can ensure alignment between aggressor UE and victim UE for CLI measurement. In addition, all DL reception and measurement, including PDSCH repetition is based on physical slot counting. Lastly, available slot counting may not be that useful for SBFD operation considering UL subband or UL transmission can happen in any SBFD symbol, which is different from U symbol in legacy TDD slot format.

Proposal 13: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, no need to further discuss the FFS: whether available slot offset list is also reused.

### L1 CLI reporting and CSI processing timeline

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to CLI RSSI or SRS-RSRP, same CPU occupation as beam reporting shall be used, that single CPU occupation for CLI reporting.

Proposal 14: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support single CPU occupation for CLI reporting.

Based on current RAN4 spec for L3 UE to UE CLI measurement that when the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. Extension to L1 based UE to UE CLI measurement and reporting, for the purpose of determining the CPU occupation duration especially first symbol of CPU occupation, the first symbol shall be the earliest SRS-RSRP symbol or CLI-RSSI symbol. In our view, for accurate CLI SRS-RSRP measurement in UL subband of SBFD symbols, UE shall use UL timing to have accurate measurement of RSRP, or current L3 CLI measurement as RAN4 agreed in #112bis meeting. For CLI-RSSI measurement in DL subband(s), DL timing was agreed as above. In this case, gNB can schedule PDSCH/PDCCH for the UE on the same OFDM symbols as CLI-RSSI measurement.

Current spec specifies timing requirements for CSI processing to guarantee that a UE has enough time to generate the AP CSI report as illustrated in the figure.

Figure 2-3 Example AP L1-CSI timeline.

Current spec specifies timing requirements for CSI processing to guarantee that a UE has enough time to generate CSI report. Current spec defines three latency classes: low latency class (Z1), high latency class (Z2), and latency for beam-reporting (Z3). When UE is configured to report CLI, the computation delay requirement of the CLI report shall be studied to guarantee a UE has enough time to generate CLI report. Processing time may be clarified for L1-CLI for separate CLI reporting. RAN1 to study UE CLI processing timeline and L1-CSI timeline can be used as baseline, e.g. whether to follow CSI computation delay requirement latency for beam-reporting class (Z3) as a baseline for L1 CLI.

As illustrated in the figure, current spec specifies the minimum Z and Z’ values for different classes and the difference between Z and Z’ is timing needed for the UE to process and start monitoring for AP measurement resource. However, at least for SRS-RSRP measurement, the symbol time shall be based on an uplink timing or a constant offset relative to the downlink timing, and advancing the CLI resource will reduce the gap time between triggering DCI and the AP CLI measurement resource. Moreover, for CLI measurement, victim DL UE needs a gap time to adjust its receiving timing to align with aggressor UL UE’s Tx timing. To achieve enough gap time between triggering DCI and the AP CLI measurement resource, RAN1 shall discuss the minimum gap time or relax the Z value for this purpose. So that the AP CLI timeline can be discussed with reusing beam reporting AP CLI timeline as the baseline but may apply a different value of Z.

Figure 2-4 Example AP L1-CLI timeline.

Proposal 15: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, RAN1 to discuss the AP CLI timeline with reusing beam reporting class as a baseline and the gap time between triggering DCI and the AP CLI measurement resource.

RAN1 agreed that for aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. And for aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

At least for SRS-RSRP measurement in UL subband of SBFD symbols, RAN4 agreed that when the UE measures SRS-RSRP and/or CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Therefore, to switch from DL timing to the CLI SRS-RSRP measurement timing as defined above, UE needs a minimum timing switch to be able to perform CLI measurement. At least some UE implementation timing switching happens at slot boundary, so Z-Z’ gap time between triggering DCI and CLI resource must be equal to or more than a slot. However, this is not guaranteed in current spec in 38.214. Note Z, Z’ can be kept as minimum requirement. Actual value can be larger. Moreover, based on the current spec for aperiodic triggering offset for AP CSI-RS, the slot offset can be [0, 31]. Therefore, in our view, a new spec rule is needed to guarantee the slot offset >= 1 slot.

Therefore, RAN1 shall discuss to the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured equal to or more than a slot.

Proposal 16: Support the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall be more than or equal to 1 slot to compensate for the timing switching.

## SBFD Specific Inter-Subband CLI Measurement and Reporting

### CLI measurement applicable configuration and symbol type

In RAN1 118 and 119 meeting, companies agreed that:

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

In RAN1 #118bis and #119 meetings, RAN1 further agreed that:

Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. Conclusion For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported. Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

RAN1 agreed that the method #1,#2,#3 for CLI measurement in SBFD symbols. RAN1 also agreed on configuration 1 and configuration 2 for SBFD operation in RAN1 agenda 9.3.1. However, RAN1 shall discuss further whether CLI SRS-RSRP or CLI-RSSI measurement can apply with Configuration 1 or Configuration 2 or both, and also whether CLI SRS-RSRP or CLI-RSSI measurement apply with SBFD symbols or non-SBFD symbols or both.

In our view, CLI SRS-RSRP or CLI-RSSI measurement can only apply to Configuration 1 that the measurements are restricted to SBFD symbols only or non-SBFD symbols only. CLI SRS-RSRP or CLI-RSSI measurement can apply to SBFD symbols for SBFD operation or non-SBFD symbols for dynamic/flexible TDD operation. Therefore, an indication of valid symbol type can be signaled in CLI SRS-RSRP or CLI-RSSI measurement resource set IE or CLI reportConfig IE. Alternatively, if it is an AP resource or SP resource, the valid symbol type can be determined by the first reception measurement occasion.

Proposal 17: Support CLI SRS-RSRP or CLI-RSSI measurement can only apply to Configuration 1 that the measurements are restricted to SBFD symbols only for SBFD operation or non-SBFD symbols only for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type for the measurement

### CLI measurement method

In RAN1 118 and 119 meeting, companies agreed to support Method #1, #2, and #3 for L1 L1 UE-to-UE CLI measurement and reporting:

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

RAN1 also agreed on to use DL timing for method #1. Based on current RAN4 spec for L3 UE to UE CLI measurement that when the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. For method #2, RAN4 agreed to apply current L3 CLI measurement timing as RAN4 agreed in #112bis meeting. And RAN1 has not discussed the timing assumption for method #3. To our view, DL timing shall be applied to method #3 for RSSI measurement in UL subband. With this assumption, it would be beneficial for UE supporting simultaneous operations between DL reception in the DL sub-band(s) and Method#3 CLI measurement reception in the UL sub-band with the same timing assumption.

Proposal 18: For L1 CLI-RSSI measurement, DL timing is used for Method #3 (UE measures RSSI within UL subband).

# Inter-gNB CLI handling schemes

In RAN1 #117 meeting, companies agreed to down select the following inter-gNB CLI scheme to specify in R19 work item.

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3] - Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3] - Information exchange of strongest DL beam information [RAN3] - Information exchange of CLI-mitigation request [RAN3] - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3. - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - Note: This feature is subject to UE capability

In this section, we will present the proposed inter-gNB CLI handling schemes from our view.

## UL resource muting for gNB-to-gNB CLI handling

In RAN1 #117 meeting, companies agreed to down select the following inter-gNB CLI scheme to specify in R19 work item.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - Note: No new DCI field / MAC CE is introduced - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212. - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH. - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode. - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols. - Note: No changes on TBS determination for PUSCH. - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - Note: This feature is subject to UE capability

In RAN1 119 meeting, companies agreed that:

Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

To configure semi-static UL resource muting pattern for PUSCH, there could be two options to configure the pattern either per serving cell configuration and/or per PUSCH / BWP configuration. The two options build on existing two DL rate matching design options.

If the UL resource muting pattern is configured per serving cell, it is cell common pattern with configured number of symbols per period, fixed symbol location and a subcarrier spacing (SCS). The configured UL resource muting pattern defined per cell level may apply only to the PUSCH of the same numerology/SCS. However, it saves signaling overhead if UL resource muting needs to be applied to all PUSCH under all BWPs that has a specific SCS.

If the UL resource muting pattern is configured per PUSCH configuration, it is UE dedicated pattern with configured number of symbols per period, fixed symbol location not based on PUSCH location or relative symbol location based on PUSCH location for a specific SCS. Also, it may have more flexibility for uplink resource muting configuration to avoid collision with DMRS and PTRS with configured per PUSCH configuration.

Proposal 19: For UL resource muting for PUSCH, RAN1 to discuss semi-static UL resource muting pattern configuration options:

Per PUSCH configuration and/or Per cell configuration.

The UL resource muting pattern may not apply to all PUSCH configurations. For example, with 4 symbols allocated for PUSCH, if 2 symbols are configured as applied for UL resource muting, UE will use one symbol of DMRS and 2 other symbols with UL resource muted REs. In this case, coding rate for this TB will be high for decoding and correspondingly the failure rate could be higher. Note that TB is calculated based on allocated symbols. Therefore, it makes sense that in time domain, UE applies the UL resource muting pattern when PUSCH allocated with equal or more than a threshold quantity of symbols in a slot and FFS in a mini-slot. Similarly in frequency domain, UE applies the UL resource muting pattern when PUSCH allocated with equal or more than a threshold quantity of RBs.

Proposal 20: For UL resource muting for PUSCH, RAN1 to discuss implicit rule whether to apply UL resource muting pattern based on PUSCH time and frequency allocation.

In RAN1 119 meeting, companies agreed that:

Agreement For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions.

RAN1 agreed UL muting for PUSCH repetition Type A in SBFD symbols with Configuration 1. As noted, RAN1 also agreed on configuration 1 and configuration 2 for SBFD operation in RAN1 agenda 9.3.1. However, RAN1 shall discuss further whether UL muting can apply with Configuration 1 or Configuration 2 or both, and also whether UL muting apply with SBFD symbols or non-SBFD symbols or both.

In our view, UL muting can only apply to Configuration 1 that it is restricted to SBFD symbols only. And the valid symbol type is only SBFD symbol. Alternatively, UL resource muting can only apply to Configuration 1 that the UL muting symbols are restricted to SBFD symbols or non-SBFD symbols. Then an indication of valid symbol type can be defined for UL muting IE.

Proposal 21: Support UL muting can only apply to Configuration 1 that the UL muting symbols are restricted to SBFD symbols or non-SBFD symbols.

- FFS: UL muting is restricted to SBFD symbols only for SBFD operation or UL muting can also apply to non-SBFD symbols for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type if UL muting can apply to SBFD or non-SBFD symbols.

For UL muting applied for UL TBoMS, RAN1 shall discuss how to determine the index of the of starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots of UL TBoMS. In current spec, it is determined by following equation:

k0=k0'+H+τ mod Ncb

where H is the total number of coded bits available for transmission of the transport block in the previous slot within the Ns slots assuming no UCI multiplexing.

In our view, two options can be considered for determination:

  - Option 1 (Motivation: simplicity but larger gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2 (Motivation: more accurate and less gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Proposal 22: RAN1 to discuss how to determine the index of the of starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots of UL TBoMS:

  - Option 1 (Motivation: simplicity but larger gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2 (Motivation: more accurate and less gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

In RAN1 119 meeting, companies agreed that:

Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

However, RAN1 has not made an agreement for the case that an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH. When transform precoding is enabled for PUSCH, as shown for PTRS-symbol mapping table when transform precoding is enabled in 38.211, there are different number of PT-RS groups and number of samples per PT-RS group:

Figure 3-1 PT-RS symbol mapping when transform precoding is enabled.

Given an example of 2 groups with 2 samples per group as shown below in legacy TDD, with an UL muting symbol, the number of complex valued symbols  will be half for the muting symbol compared to unmuted symbols, and RAN1 needs to further discuss how to map the second group of the 2 PTRS samples for pre-DFT insertion corresponding to muted OFDM symbol prior to transform precoding, e.g. whether to ignore the second group of the 2 PTRS samples for pre-DFT insertion when it is mapped to complex valued symbols prior to transform precoding, as shown in figure below. Therefore, there will be spec impact on the PTRS mapping of pre-DFT insertion for UL DFT-s-OFDM when transform precoding is enabled for PUSCH if RAN1 allows an UL resource muting symbol to overlap with a symbol containing PTRS for a PUSCH.

Therefore, in our view, if an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol. And gNB can configure PT-RS as every other symbol, to avoid overlapping with UL muting in symbol level; but in case if it happens, PT-RS shall be prioritized.

Figure 3-2 PT-RS symbol mapping when transform precoding is enabled.

Figure 3-3 PT-RS symbol mapping with reduce number of groups to half on muted symbol when transform precoding is enabled.

Proposal 23: If an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

In RAN1 119 meeting, companies agreed that:

Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled. Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

For PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, support 3dB power boosting for both PUSCH and PT-RS REs for full coherent, partial coherent, and non-coherent PTRS cases to maintain a constant per-symbol power over the PUSCH transmission across symbols.

Proposal 24: 3dB power boosting is assumed for REs in the PUSCH symbol containing PT-RS with UL resource muting that 3dB power boosting for both PUSCH and PT-RS REs.

In RAN1 119 meeting, companies agreed that:

Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM, considering the proposal that if an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol, there is no use case to configure a comb offset {0,1}. A comb offset {0} can be predefined for the up to two UL muting symbols, for DFT-S-OFDM for less spec impact.

Proposal 25: For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

A comb offset of zero is predefined for the up to two UL muting symbols.

In RAN1 119 meeting, companies agreed that:

Agreement If transform precoding is enabled for a PUSCH, study further the following two options: - Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting - Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting Note: MscPUSCH is the total number of subcarriers of the PUSCH. FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

If UL muting is applied when transport precoding is enabled for a PUSCH (not containing PTRS), the complex valued symbols shall be divided into sets, and each set corresponds to one OFDM symbol, where set ‘l’ contains  MscPUSCH-δlMscPUSCH2 where δl=1 when OFDM symbol ‘l’ contains an uplink muting pattern, and otherwise, δl=0.

Proposal 26: When UL muting is applied when transport precoding is enabled for a PUSCH, the complex valued symbols shall be divided into sets, each set corresponds to one OFDM symbol, where set ‘l’ contains  MscPUSCH-δlMscPUSCH2 where δl=1 when OFDM symbol ‘l’ contains an uplink muting pattern, and otherwise, δl=0.

In the above agreement, in our view, Option 2 is preferred to maintain the same DFT size. Although Option 1 and Option 2 can be mathematically equivalent, Option 2 is simpler for spec impact. Therefore, in our view, for a OFDM symbol that UL muting is applied, the set of complex valued symbols x is repeated and mapped to a complex valued symbols that x~(i)=xi modMsc2.

Proposal 27: For a OFDM symbol that UL muting is applied, the set of complex valued symbols is repeated and mapped to complex valued symbols with Option 2.

# RRC Parameters

## RRC Parameters for L1 CLI Measurement and Reporting

RAN1 shall discuss RRC parameters for L1 UE-to-UE CLI measurement and reporting:

| RAN2 parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE or Cell specific |
| --- | --- | --- | --- | --- | --- | --- |
| CSI-ResourceConfig | SRS-RSRP-MeasurementResourceSet | New | A set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement |  | Cell | UE-specific |
| CSI-ResourceConfig | CLI-RSSI-MeasurementResourceSet | New | A set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement |  | Cell | UE-specific |
| SRS-RSRP-MeasurementResourceSet | SRS-RSRP-MeasurementResource | New | The resource contains the parameters for L1 SRS-RSRP measurement |  | Cell | UE-specific |
| SRS-RSRP-MeasurementResourceSet | slot offset | New | The slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter |  | Cell | UE-specific |
| CLI-RSSI-MeasurementResourceSet | CLI-RSSI-MeasurementResource | New | Resource contains the parameters for L1 CLI-RSSI measurement |  | Cell | UE-specific |
| CLI-RSSI-MeasurementResourceSet | slot offset | New | The slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter |  | Cell | UE-specific |
| SRS-RSRP-MeasurementResource | - srs-Resource - srs-SCS - qcl-info | New | - SRS resources to be used for CLI measurements. - Subcarrier spacing for SRS. - Qcl-D optional configured for periodic SRS-RSRP measurementresource |  | Cell | UE-specific |
| CLI-RSSI-MeasurementResource | - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - Subcarrier spacing - Qcl-info | New | - CLI-RSSI Resource index - Starting PRB index of the measurement bandwidth. - Allowed size of the measurement BW. - OFDM symbol location of the CLI-RSSI resource within a slot. - Within a slot that is configured for CLI-RSSI measurement the UE measures the RSSI from start symbol to start symbol + number of Symbols - 1. - Periodicity and slot offset for this CLI-RSSI resource. All values are in "number of slots". - Reference subcarrier spacing for CLI-RSSI measurement. - QCL-D optional configured for periodic CLI-RSSI measurementresource |  | Cell | UE-specific |
| CSI-ReportConfig | reportQuantity-r19 {SRS-RSRP, cli-RSSI} | New | The CSI related quantities to report. |  | Cell | UE-specific |
| CSI-ReportConfig | NumberreportedCLIresources | New | Number of reported CLI measurement resources |  | Cell | UE-specific |
| CSI-AssociatedReportConfigInfo | resourcesForChannel-r19 | New | Configure reference signals for channel measurement or CLI SRS-RSRP or CLI-RSSI resource set for CLI measurements |  | Cell | UE-specific |
| resourcesForChannel-r19 | qcl-info | New | Optionally configure QCL-D where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo |  | Cell | UE-specific |

## RRC Parameters for UL Resource Muting

| RAN1 shall discuss RRC parameters for UL resource muting.RAN2 parent IE | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) | UE or Cell specific |
| --- | --- | --- | --- | --- | --- | --- |
| PUSCH-config | ULResourceMuting | New | UL resource muting pattern |  | UE | UE-specific |
| ULResourceMuting | - Time location - Comb offset | New | - time location of each of one or two UL muting symbols - comb offset of {0,1} |  | UE | UE-specific |
| pusch-TimeDomainAllocation | ULResourceMutingIndicator | New | Indicate whether the semi-statically configured UL muting symbol(s) are applicable or not for the indicated row in the PUSCH TDRA table. |  | UE | UE-specific |

# Conclusion

Proposal 1: Support SRS-RSRP-MeasurementResourceSet or CLI-RSSI-MeasurementResourceSet to be defined per cell under ServingCellConfig IE, under existing csi-MeasConfig.

Proposal 2: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource.

Proposal 3: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

Proposal 4: Support reuse same “carrier” parameter in CSI-ReportConfig IE to indicate the CLI measurement CC.

Proposal 5: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource for cross CC CLI measurement.

Proposal 6: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid of the carrier/cell that UE measures the CLI for cross CC CLI measurement.

Proposal 7: RAN1 to discuss AP triggered a CLI measurement time window with one or more CLI-RSSI resources configured per slot in the time window with multiple slots to perform CLI measurement and differentiate different aggressor UEs over different time occasions.

Proposal 8: RAN1 to discuss default beam rules to handle the case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming:

- Option 1: An error case that UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Option 2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

Proposal 9: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support reuse L1-RSRP and R16 SRS-RSRP same range, resolution, and number of bits for L1 CLI SRS-RSRP reporting, and reuse R16 CLI-RSSI same range, resolution and number of bits for L1 CLI-RSSI reporting.

For SRS-RSRP, a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential SRS-RSRP is quantized to a 4-bit value. The differential SRS-RSRP value is computed with 2 dB step size with a reference to the largest or smallest measured SRS-RSRP value.

For CLI RSSI, a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential CLI RSSI is quantized to a 4-bit value. The differential CLI RSSI value is computed with 2 dB step size with a reference to the largest or smallest measured CLI RSSI value.

Proposal 10: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, RAN1 to reuse report format as L1 beam reporting.

- FFS: a new local resource index for CLI measurement resource in CLI reporting

- FFS: a new payload format table for CLI reporting similar as L1 beam reporting table.

Proposal 11: RAN 1 to discuss how to report differential value in case the largest measurement is out of range:

- Option 1: UE calculates the differential values based on the largest CLI value in the range which is -44 dBm for CLI SRS-RSRP reporting.

- Option 2: UE will not report other differential values for other CLI resources in the same report, e.g. other CLI resources reported as dummy value if more than 1 CLI resource is configured for the report.

Proposal 12: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support configuration of reporting criteria of {most interfering, least interfering} CLI resources in the reportConfig

Proposal 13: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, no need to further discuss the FFS: whether available slot offset list is also reused.

Proposal 14: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support single CPU occupation for CLI reporting.

Proposal 15: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, RAN1 to discuss the AP CLI timeline with reusing beam reporting class as a baseline and the gap time between triggering DCI and the AP CLI measurement resource.

Proposal 16: Support the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall be more than or equal to 1 slot to compensate for the timing switching.

Proposal 17: Support CLI SRS-RSRP or CLI-RSSI measurement can only apply to Configuration 1 that the measurements are restricted to SBFD symbols only for SBFD operation or non-SBFD symbols only for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type for the measurement

Proposal 18: For L1 CLI-RSSI measurement, DL timing is used for Method #3 (UE measures RSSI within UL subband).

Proposal 19: For UL resource muting for PUSCH, RAN1 to discuss semi-static UL resource muting pattern configuration options:

Per PUSCH configuration and/or Per cell configuration.

Proposal 20: For UL resource muting for PUSCH, RAN1 to discuss implicit rule whether to apply UL resource muting pattern based on PUSCH time and frequency allocation.

Proposal 21: Support UL muting can only apply to Configuration 1 that the UL muting symbols are restricted to SBFD symbols or non-SBFD symbols.

- FFS: UL muting is restricted to SBFD symbols only for SBFD operation or UL muting can also apply to non-SBFD symbols for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type if UL muting can apply to SBFD or non-SBFD symbols.

Proposal 22: RAN1 to discuss how to determine the index of the of starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots of UL TBoMS:

  - Option 1 (Motivation: simplicity but larger gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2 (Motivation: more accurate and less gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Proposal 23: If an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Proposal 24: 3dB power boosting is assumed for REs in the PUSCH symbol containing PT-RS with UL resource muting that 3dB power boosting for both PUSCH and PT-RS REs.

Proposal 25: For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

A comb offset of zero is predefined for the up to two UL muting symbols.

Proposal 26: When UL muting is applied when transport precoding is enabled for a PUSCH, the complex valued symbols shall be divided into sets, each set corresponds to one OFDM symbol, where set ‘l’ contains  MscPUSCH-δlMscPUSCH2 where δl=1 when OFDM symbol ‘l’ contains an uplink muting pattern, and otherwise, δl=0.

Proposal 27: For a OFDM symbol that UL muting is applied, the set of complex valued symbols is repeated and mapped to complex valued symbols with Option 2.

# References

- 3GPP TSG RAN Meeting #104, RP-241614 “New WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”.