3GPP TSG RAN WG1  #120			R1-2501167

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.7.2

Source: 	Qualcomm Incorporated

Title: 	Discussion on ISAC Channel modeling

Document for:		Discussion

# Introduction

At RAN1 #119, the following agreements were made related to ISAC channel modelling:

Agreement Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction. Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx Agreement The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model Agreement The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - the recommended five scattering points are located in front, left, back, right and roof side of the vehicle - the following RCS model is supported for each scattering point - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model. Agreement - If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled - FFS whether/how blockage/forward scattering can be modelled in the target channel. Agreement - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft Where, - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mT is the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2 Agreement To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i Agreement The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2. Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k. Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model

# ISAC Channel Modelling: Target & Background channel

  - Basic formulation of the ISAC channel model

With regards to the basic formulation of the ISAC channel model, the following agreement has been made:

Agreement The common framework for ISAC channel model is composed of a component of target channel and a component of background channel, HISAC=Htarget+Hbackground - Target channel Htarget includes all [multipath] components impacted by the sensing target(s). - FFS details of the target channel - Background channel Hbackground includes other [multipath] components not belonging to target channel - FFS details of the background channel - FFS whether/how to model environment object(s), i.e., object(s) with known location, other than sensing target(s) - FFS whether/how to model propagation path(s) between the target(s) and the environment object(s) - FFS whether/how to model propagation path(s) between the target(s) and the stochastic clutter(s) - Note: the notation HISAC can be revised later if needed

Assuming that we have a total of Ntargets sensing targets, such that the i-th target has Mscatter-pointsi scatter-points, we propose the following formulation for the ISAC target channel:

Htargetτ,t=i=1Ntargetsj=1Mscatter-pointsiHtar, i,jτ,t

where the target channel Htar, i,j includes all components impacted by the j-th scatter point of the i-th sensing target and interaction between the scatter points of an target, or between scatter points across targets is not modelled.

Proposal 1: Multiple bounce modelling between the scatter-points of a sensing target, is not supported.

Proposal 2: For the ISAC target channel Htarget, support the following formulation

Htargetτ,t=i=1Ntargetsj=1Mscatter-pointsiHtar, i,jτ,t

where

- Ntargets is the number of sensing targets between a pair of Tx and Rx

- The i-th sensing target is modelled as Mscatter-pointsi scatter point, where both Mscatter-pointsi=1 and Mscatter-pointsi>1 is supported.

  - Target & Background Channel normalization

The following working assumption has been made related to the RCS modelling in which an FFS on the powern normalization has been included:

Agreement - The following options are supported to generate the combined ISAC channel - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed. - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between - Alt 1: Power normalization on both target channel and background channel - Alt 2: Power normalization on background channel only - Alt 3: the target channel of a target will replace one cluster in the background channel - FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2 - FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

We believe that addressing power normalization for both the target channel and the background channel involves multiple perspectives and might require a case-by-case analysis. In some situations, adding more objects to a deployment does not necessarily raise the overall power at the receiver, whereas in other instances, this increase is anticipated and expected. For instance, within an indoor factory or a room, adding more people should not lead to higher power at the receiver. Therefore, both the target and background channels should be normalized to ensure that the long-term average power remains similar to that in the absence of these objects. Conversely, in an outdoor scenario where RIS or smart reflectors are added, one should expect an increase in the long-term average power, making power normalization not accurate.

- Assume HRel18 the channel between a Tx and Rx according to up to Rel-18 channel  model.

- Assume HbISAC is the background channel between a Tx and Rx, and Htarget the target channel prior to power scaling.

We propose the following power scaling: HISAC=α⋅(Htarget+HbISAC), where α is such that E|HISAC​2=E|​HRel182.

Proposal 3: Support the following power normalization between the target and the background channel:

- HISAC=α⋅(Htarget+HbISAC), where α is such that E|HISAC​2=E|​HRel182

# ISAC Target Channel

  - ISAC target channel & Absolute time of arrival

The following agreement was made related to the direct path:

Agreement A single direct path is modeled for a scattering point of target - In each of the Tx-target and target RX links, the first NLOS cluster is generated with same delay as the LOS ray (when the absolute delay modelling of Δτ as in section 7.6.9, TR 38.901 is not applied) and with the same direction as the LOS ray. - FFS how to generate NLOS cluster when Δτ is applied

The following absolute time of arrival modelling has been introduced in 38.901:

7.6.9 Absolute time of arrival To support simulations in which absolute time of arrival is important, the propagation time delay due to the total path length is considered in step 11 of the fast fading model as follows. The impulse response in NLOS is determined using equation (7.6-43) instead of (7.5-27) and the impulse response in LOS is determined using equation (7.6-44) instead of (7.5-30), where c is the speed of light. Δτ is generated from a lognormal distribution with parameters according to Table 7.6.9-1. Δτ is generated independently for links between the same UT and different BS sites. The excess delay in NLOS, Dt, should further be upper bounded by 2L/c, where L is the largest dimension of the factory hall, i.e. L = max(length, width, height). Hu,sNLOSτ,t=n=12i=13m∈RiHu,s,n,mNLOStδτ-τn,i-d3Dc-Δτ+n=3NHu,s,nNLOStδτ-τn-d3Dc-Δτ (7.6-43) Hu,sLOSτ,t=1KR+1Hu,sNLOSτ,t+KRKR+1Hu,s,1LOStδτ-τ1-d3Dc. (7.6-44) Table 7.6.9-1: Parameters for the absolute time of arrival model Scenarios; InF-SL, InF-DL; InF-SH, InF-DH. lgΔτ=log10Δτ1s; μlgΔτ; -7.5; -7.5. σlgΔτ; 0.4; 0.4. Correlation distance in the horizontal plane [m]; 6; 11.

Similar to NR Positioning studies, we support using such modelling as an optional component, and extend it to outdoor cases also, e.g., at least for UMi scenarios.

Proposal 4: Support, as an optional modelling component in TS38.901, the absolute time of arrival modelling for both outdoor and indoor scenarios using Section 7.6.9 as a starting point for each Tx-target and target-Rx link.

- Study the parameters (μlgΔτ, σlgΔτ) and correlation distance at least for the UMi scenario

  - Applicability of the direct path generation for  multi-scattering points objects

Another FFS in the above agreement is related to the multi-scattering point target.

Agreement - For radio propagation Case 1, for modelling the target channel of a target with single scattering point, - To model a direct path, a single LOS ray from Tx to target and a single LOS ray from target to Rx are generated - AoA/ZoA of the direct path at Rx, AoD/ZoD of the direct path at target are generated at least based on the 3D location of target and Rx in the global coordinate system - AoD/ZoD of the direct path at Tx, AoA/ZoA of the direct path at target are generated at least based on the 3D location of Tx and target in the global coordinate system - The Delay of the direct path = (d3D_tx_target + d3D_target_rx)/c - The Doppler of the direct path is generated by spherical unit vectors by AoD/ZoD at Tx, by spherical unit vectors by AoA/ZoA at Rx, and velocity of Tx, target and Rx - The power of the direct path is generated as the product of the power of the LOS ray from Tx to target, the power of the LOS ray from target to Rx, and the effect of RCS - FFS initial phase - FFS how to model RCS, polarization of target - FFS number of direct paths - FFS on detailed modelling of indirect path(s) - FFS applicability of direct path generation to each scattering point when the target is modelled as multiple scattering points

Any agreement related to the ISAC channel modelling for an object with a single scattering point, should also apply to an object with multi-scattering points.

Proposal 5: When modelling one or more target channels of target(s) with one or more scattering points,

- Support applying the direct path generation independently to each scattering point.

  - Indirect paths in ISAC target channel

The following agreement has been made related to the indirect paths:

Agreement In order to generate each of the Tx-target link and target-Rx link in the target channel, the large scale and small scale parameters defined in existing 3GPP TRs, e.g., TR 38.901. TR 36.777, TR 37.885, TR 38.858, TR 38.859, TR 38.802, TR 38.854, etc. are used as starting point

The NLOS clusters between a Tx and an Rx, can vary from as small as 12 clusters, and as high as 25 clusters depending on the scenario of interest. We support keeping these properties the same.

Proposal 6: For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- Reuse the existing number of number of clusters and rays per cluster as in TS38.901.

  - 3.3 Concatenation for indirect path modelling

The following agreement has been related to the topic of concatenation-based modeling for ISAC target channel modelling:

Agreement - To generate indirect paths of NLOS ray + NLOS ray in the target channel - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4 - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4 - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2, - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1 - Other methods are up to company choice for complexity reduction - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration - The power threshold for path dropping is X=[-25] dB - X is relative to the strongest indirect path in the target channel - FFS: further power normalization of target channel is performed after path dropping, - Note: power normalization when target channel and background channel are combined can be discussed separately - FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

With regards to the remaining FFS, we make the following proposal:

Proposal 7: Keep the power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB.

Proposal 8: No further power normalization of target channel is performed after path dropping.

  - 3.4 RCS modelling

With regards to the RCS, the following agreements were made:

Agreement RAN1 strives to define a single option per target per monostatic/bistatic sensing mode from the following two options to generate RCS values/patterns for a scattering point of a target. - Option 2: The RCS=A*B of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - A is [mean] RCS value. FFS value(s) A - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B - B is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS - Option 3: The RCS=A*B=A*B1*B2 of a scattering point can be generated by - The component A is commonly applied to any incident/scattered angles at the scattering point - FFS: A = 1 m2 or [mean] RCS value - Note: Mean RCS value is defined as the mean value of the distribution of RCS - The component B is further split into B1, B2, i.e., B=B1*B2 - B1 is deterministic based on incident/scattered angles - FFS: B1 is defined by a function or by a table - B2 is generated by [log-normal] distribution, the related [log-normal] distribution has mean μ=1 and variance V, FFS σ2 - B2 is separately generated for each direct/indirect path at the scattering point. FFS correlation dependent on the incident/scattered angles of the direct/indirect paths - FFS whether/how power of all generated direct/indirect paths need to be normalized considering impact of RCS Agreement RCS Option 3 is selected to model RCS of UAV with single scattering point for monostatic - B2 of UAV is modelled using log-normal distribution for monostatic - Different mean RCS values can be supported for UAV due to different size, shape, frequency, etc. - For UAV of small size (option 2 for UAV size in UAV parameters table) - B1=1 - A is mean RCS value - For UAV of large size (option 1 for UAV size in UAV parameters table) - B1 have dependency on incident/scattered angles - A is mean RCS value

    - 3.4.1 RCS measurements & modelling for UAV

We performed a measurement campaign for determining the RCS of a Drone (Matrice 300 RTK) with the following characteristics:

- Freq range: 2.5 to 4.5 GHz, 3.5 GHz center freq

- DUT test conditions: Propellers deployed

- Physical size: 430x420x430 mm (L x W x H)

We performed the following measurements scans:

- Az cut = 0 to 360 by 1 deg, theta = 90 deg, H/V pol

- El cut = 0 to 360 by 1 deg, phi = 0 deg, H/V pol

- El cut = 0 to 360 by 1 deg, phi = 90 deg, H/V pol

We calibrated the RCS experimental setup using 12 in diameter metal sphere and a flat metal plate (12” x 12”)

|  | μ(dBsm) | σ(dBsm) |
| --- | --- | --- |
| Az cut = 0 , El=90 degrees, H-pol | -9 | 5.7 |
| Az cut = 0 , El=90 degrees, V-pol | -8.5 | 6.1 |
| Az cut = 0 , El=0 degrees, H-pol | -4.2 | 5.5 |
| Az cut = 0 , El=0 degrees, V-pol | -3.5 | 5.3 |
| Az cut = 90 , El=0 degrees, H-pol | -6.6 | 6.2 |
| Az cut = 90 , El=0 degrees, V-pol | -7.3 | 6.2 |
| Across all measurements and polarizations | -6.5 | 6.2 |

Based on the agreements, we need to make a proposal such that there is a B2 with the following properties: (μB2, σB22) for the log-normal distribution of B2 subject to fixed relation μB2=-ln1020σB22

- Note: this is to follow early agreement that the mean of linear B2 values equals to expln1010μB2+ln210200σB22=1. To do so, we make the following proposal:

Proposal 9: With regards to RCS modelling for a small UAV, for each scattering point, for each polarization,

- AdBsm=-2.7

- μB2dBsm=-4.4

- σB2dBsm=6.17

    - 3.4.2 RCS modelling for a vehicle

The following agreement has been related to the RCS modelling for a vehicle:

Agreement The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A Agreement When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc. - the recommended five scattering points are located in front, left, back, right and roof side of the vehicle - the following RCS model is supported for each scattering point - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A

Shape of a vehicle can be approximated as a rectangular box with N scattering centers [9]. Fig. 1 shows the vehicle in front of the anechoic used to measure vehicle RCS with multiple scattering centers. In [9], a greedy algorithm is proposed to extract the strongest scattering points to model the vehicle RCS. The number of strongest scattering points and their locations can be based on the type of the car, resolution of the sensing device, carrier frequency, and distance. The RCS values of scattering point representing a vehicle varies with distance, as shown in Figure 2, and can be modeled as a single point at far distances [10]. For simplicity of modeling, we propose two options

- Option-1: 5-point scattering model with scattering centers at left, right, front, back, and roof seems with its distinct element pattern. The mean (A) and variance (B) will vary based on distance.

- Option-2: 5-point scattering model at near range, whereas 1-point scattering model at far range. When to use 5-point and when to use 1-point is FFS.

Figure 1. Left figure shows a vehicle in front of the anechoic used to measure vehicle RCS. Center figure shows the greedy algorithm to extract strongest virtual scattering points, and the right figure shows the 10 virtual scattering centers used to model ford focus after using the proposed greedy algorithm [9].

Figure 2. RCS comparison of proposed RCS model for a car that varies with distance [11] and its comparison with other prior works.

In [11], it was shown that the frequency characteristics of RCS of passenger car and mini-van are equivalent to those of trihedral, flat plate, etc. For a trihedral reflector with edge length ‘a’ or flat plate of side length ‘a’, the RCS is proportional to the square of the carrier frequency. Additionally, frequency characteristic of RCS of motorcycle equivalent to those of cylinder. RCS is inversely dependent on wavelength. For a cylinder with diameter ‘a’ and height ‘a’, the RCS is given by σ2=πa3/λ.

Figure 3. Left figure shows the anechoic chamber used for vehicle RCS measurement. Right figure shows the plot of measured vehicle RCS at varying carrier frequencies for different vehicles and polarization.

In [12-13], it has been shown that log-normal can be used to model vehicle RCS.

Proposal 10: With regards to RCS modelling for a vehicle, support one of the two following alternatives:

- Alt. 1: For the 5-point scattering model with scattering centres at left, right, front, back, and roof seems with its distinct element pattern:

  - The mean (A) and variance (B2) can vary based on distance.

- Alt. 2: The 5-point scattering model applies at near range, whereas 1-point scattering model applies at far range.

Proposal 11: With regards to RCS modelling for a vehicle for each scattering point, we propose the following:

- The value of the log-normal distribution parameters will depend on carrier frequency, incident/reflected angle.

  - Mean RCS, A, is [10 to 20] dBsm.

  - Value of B2 is FFS.

  - Mean RCS can be modelled linearly with carrier frequency in logarithmic scale. The proportionality factor can depend on the type of vehicle.

  - With regards to B1, support Alt. 1 (formulated similar as the antenna radiation power pattern in 38.901).

    - 3.4.3 RCS modelling for a human

The following agreement has been related to the RCS modelling for a human:

Agreement The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc. - Model 1 - B1=0 dB - A is mean RCS value - B2 is modelled using log-normal distribution - Model 2 - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below: - Alt 1: formulated similar as the antenna radiation power pattern in 38.901 - Alt 2: a function - Alt 3: Lookup table - B2 is modelled using log-normal distribution - FFS RCS component A - FFS: conditions for using which model

In [14], radar cross section of humans with different heights and pedestrian dummies were measured in an anechoic chamber at 23-28GHz as well as 76-81 GHz bands, as shown in Fig. 1. The measured RCS with different azimuth angles for an adult at an antenna height of 0.8 m in the 23-28 GHz band is shown in Fig. 2. The measured RCS averages for humans and pedestrian dummies for both adult and child for different frequency bands, antenna heights, and clothing are summarized in Fig. 3. It was observed that pedestrian shape properties (such as pedestrian height) and viewing angle (such as due to change of antenna height) can impact RCS averages, especially at high frequency bands. Lower antenna height led to higher RCS.

Figure 1: Left figure shows the anechoic chamber measurement setup. Center and right figure show the measured RCS of an adult at different azimuth angles in 23-28 GHz band.

Figure 2: Average RCS values for adult and child at different carrier frequencies

6dB RCS gap for an antenna height difference of 0.5 m

Figure 3: Average RCS of a child dummy at different antenna in 76-81 GHz band

In [15-16], it is shown that Weibull distribution has the best fit to pedestrian RCS measurements at mmWave band (77-81 GHz). In [16], for example, it was shown that the fitting error evaluated for all ranges and incident angles is the least for Weibull distribution as compared to lognormal and gamma ones for a 1.8 m tall pedestrian, as shown in Table 1.

Table1. Fitting error for different distributions to model a pedestrian

It was also observed in [15163], that azimuth angle can effect RCS value and distribution parameters. Figure 4 shows the average RCS and standard deviation of RCS from different incident angles at range of 100 m for a 1.7m tall walking woman. The averaged RCS has peak values at 0º, 90º, 180º and 270º, are corresponding to the two sides, front and back of the person.

Figure 4. Average and standard deviation of RCS for a pedestrian at different azimuth angles

In [18], RCS was measured at 10, 24, 79GHz GHz. It was observed that RCS of human body can be modeled as log-normal distribution with weak azimuth angular dependence.

Table 2 shows the compilation of mean and standard deviation values proposed by different companies from the previous RAN1 meeting according to the Feature Lead Summary document. From [14]-[17] and the Table 2, we can see that mean doesn’t change much but variance is effected considerably with change in standard deviation.

Table 2. Human monostatic RCS table compilation

|  | polarization | frequency (GHz) | mean (dBsm) | standard deviation (dBsm) | variance (dBsm2) |
| --- | --- | --- | --- | --- | --- |
| Xiaomi | VV | 4.9 | 3.65 |  | 30.18 |
| Xiaomi | HH | 4.9 | 3.46 |  | 29.46 |
| Xiaomi | VH | 4.9 | -27.64 |  | 37.76 |
| Xiaomi | HV | 4.9 | -27.64 |  | 37.76 |
| vivo/BUPT | VV | 20 | -3.9933 | 2.4178 |  |
| vivo/BUPT | VV | 28 | -1.7976 | 2.6103 |  |
| vivo/BUPT | VV | 36 | -0.1655 | 2.1441 |  |
| AT&T |  | 6.7 | -6.2 | 10.0 |  |
| LG |  | 15 | -3 |  |  |

Proposal 12: With regards to RCS modelling for a pedestrian, for each scattering point, for each polarization, use RCS modelling Option 2:

- For model 1,

  - A is the mean with dBsm=[-10 to 0] for adult and [-5 to -15] for child.

  - Value of B2 is FFS.

- For model 2,

  - With regards to the dependency on incident/scattered angles support Alt. 1 (formulated similar as the antenna radiation power pattern in 38.901)

  - support B1 to depend on the elevation.

    - 3.4.3 Other RCS modelling

Upper bound for RCS modelling

The following FFS point has been made related to maximum value that component B2 of RCS can take:

Agreement Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Based on the measurements we have made for the UAV, we make the following related proposal:

Proposal 13: Component B2 of RCS is upper bounded by kσ dB with k=[3 or 4].

  - 3.5 Polarization changes

With regards to the impact of an object to the polarization changes, the following agreement has been made:

Agreement To model the effect of polarization for each direct/indirect path: - Polarization of a direct/indirect path is product of polarization matrix of Tx-target link, the target, and the target-Rx link - Total polarization of a direct/indirect path is CPMtx,sp,rx= CPMsp,rx . CPMsp . CPMtx,sp - For a LOS ray from Tx to target or from target to Rx, CPM=100-1 for CPMtx,sp or CPMsp,rx - For a NLOS ray generated by a stochastic cluster from Tx to target or from target to Rx, CPMtx,sp or CPMsp,rx CPMis generated by XPR ratio  and initial random phases referring to TR 38.901 as start point - FFS how to normalize on CPMtx,sp,rx CPMtx,sp,rx - FFS CPMsp CPMspof a scattering point of the target - FFS: how to model the effect of polarization when EO type-2 is present Agreement To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π] - FFS correlation between αi,1,αi,2,βi,1,βi,2 - FFS specular reflection - FFS: CPM normalization The following options are considered for further study, down select one option from the following - Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution - Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i - Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i - CPMsp,i defined in LCS - Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

From the above 4 options, we support Option 1:

Proposal 14: To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point,:

- Support the initial random phase to be uniformly distributed within -π,π.

- No correlation between αi,1,αi,2,βi,1,βi,2

- Support Option 1

  - 3.6 EO Type-2 modelling

With regards to the LOS condition of the Tx-target link and target-Rx link when the EO type-2 is modelled in the target channel, the following agreement has been made:

Agreement When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link - Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition - FFS changes to the LOS probability defined in existing TRs - FFS details on blockage by EO type-2 - Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model Agreement The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2.

With regards to the above options, we make the following related proposal:

Proposal 15: With regards to the EO Type-2 in the target channel and the determination of the LOS condition, support Option A.

  - 3.7 Doppler modelling: macro & micro modelling

The following agreements have been made related to Doppler modelling:

Agreement - Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula, rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft Where, - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point - rtx,n,mT is the spherical unit vector at transmitter for the link from Tx to the scattering point - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0 - ft is only applicable for indirect path - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray - Support two terms of 2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray - Doppler is separately determined for each of the multiple scattering points of a target - vspt can include macro-Doppler and/or micro-Doppler motion, vspt=vmacrot+vmicro,pt - FFS: maximum speed of moving scatterers - FFS: ratio of moving scatterers among all scatterers

We believe that at least for the following 2 use cases: UAVs and human outdoors (e.g., VRUs) it will be useful to consider micro doppler models in this release. There are several benefits to use micro-Doppler signatures to classify different targets at least for these 2 scenarios.

Proposal 16: With regards to doppler modelling, support specifying a function for micro-doppler modelling for the UAV and human outdoor sensing (e.g. VRUs) scenario in this release.

  - ISAC channel Pathloss modelling

We can model the impact of physical scatterers by cascading multiple hops, and we can reuse existing LoS/NLoS-state modeling separately for each hop. The question then arises whether the same approach extends to pathloss modeling- can we cascade the pathlosses of the hops following existing modeling on each hop? The pathloss PL (in dB) between two locations/nodes is a function of distance ‘d’ between the nodes. Consider two hops of distances d1 and d2. The ‘cascaded approach’ would produce an overall 2-hop pathloss (in dB) given by

- PL2hop(d1,d2)=PL(d1)+PL(d2)+L,

where L is the loss (in dB) from interaction with the object, e.g., a reflection loss, which is a function of the RCS. This model is commonly found in radar literature, especially in the case of free-space LoS pathloss function PL(d) where the linear-domain pathloss is proportional to 1/d2. However, consider the case of free-space line-of-sight propagation into which a blocker such as a dielectric material is introduced to block the LoS path. In this case, the object interaction is a ‘through-transmission with attenuation’ rather than a reflection, and in this case it may be more appropriate to use an overall 2-hop pathloss given by

- PL2hop(d1,d2)=PL(d1+d2)+L

Further, the legacy pathloss models may also need further modification to account for propagation during rainfall.

Proposal 17: The pathloss for Tx-Target-Rx propagation is formulated as follows, for at least some scenarios:

PLsd1,d2=PLd1+PLd2+10log10λ24πσRCS

where

- PLd1 is pathloss between Tx and target, with d1 equal to Tx-to-Target LOS distance.

- PLd2 is pathloss between Rx and target, with d2 equal to Target-to-Rx LOS distance.

- The existing pathloss formula in 3GPP TRs is reused as start point.

- λ is wave-length of sensing signal. σRCS is a large-scale RCS value

- Other scenarios requiring alternative formulations are not precluded.

- Study further the cases that the above formula doesn’t apply, and specifically under which scenarios the pathloss should have the following format:

PL2hop(d1,d2)=PL(d1+d2)+L

where L is the loss in dB caused by interaction with the object/scatter-point, and study further which models apply in which scenarios.

- The presence or not of EO type-2 in the target channel doesn’t change the determined pathloss of the target channel

  - 3.9 Multi-bounce modelling

With multiple small objects, each object can be represented by a scatter-point, and the object (=scatter-point) dropping and channel modeling can be repeated separately for each object. Note that each object may be a sensing target or may represent clutter or non-target objects. This approach does not capture paths bouncing from Tx to Rx via multiple objects, i.e., involving at least 3 hops (Txàobject1àobject2àRx) instead of 2 hops considered in Section 2.4.  Given the multitude of combinations of such multi-bounce paths, with the further possibility of modeling LoS (as in Section 2.4.3) and/or NLoS (as in Section 2.4.4) propagation on each hop in the path, this approach rapidly becomes very complex. For simplicity we propose the following:

Proposal 18: For the ISAC target channel, support to optionally model the interaction between a sensing target and an EO type-2 under the following modelling simplifications:

- EO type-2 is modelled as a specular reflection with Section 7.6.8 as a starting point

- For the Tx – target – EO – Rx scenario:

  - There is a LOS ray in the target to Rx link

  - Use the ground reflection model to add an additional specular ray following Section 7.6.8

- For the Tx – EO – target – Rx scenario:

  - There is a LOS ray in the Tx to target link

  - Use the ground reflection model to add an additional specular ray following Section 7.6.8

  - Multi-point scattering target

Modeling an object with multiple scattering points is a crucial feature to develop further. This is particularly useful for larger objects, such as cars, where a channel model can guide whether to represent the object as a single scattering point or as a multi-point object. To leverage much of the single scattering point modeling, we propose the following simplifications:

- The same number and locations of scattering points apply regardless of sensing Tx/Rx.

- There is no need to model the relative location changes between scattering points. This doesn’t preclude micro-doppler modelling to be included.

- Interactions between the scattering points are not modeled.

- The RCS values of the multiple scattering points are determined individually.

- In the basic formulation of the ISAC channel model, the multiple scattering points of the target are treated similarly to having multiple targets, each with a single scattering point

Proposal 19: For the ISAC target channel, support modelling a target with multiple scattering points with the following simplifications:

- The same number and locations of scattering points apply regardless of sensing Tx/Rx.

- There is no need to model the relative location changes between scattering points.

- Interactions between the scattering points are not modeled.

- The RCS values of the multiple scattering points are determined individually.

- In the basic formulation of the ISAC channel model, the multiple scattering points of the target are treated similarly to having multiple targets, each with a single scattering point.

# ISAC Background Channel

  - 4.1 EO Type-2 in the background channel

Proposal 20: When EO type-2 is included in a deployment, its impact is modelled only in the ISAC target channel.

In addition to the effect introduced by the other objects in the background channel, the stochastic clusters that already exist in 38.901 should be included in the model:

  - 4.2 Bistatic Background channel

Agreement On the background channel for TRP-TRP and UE-UE bistatic sensing mode, the large scale and small scale parameters defined in TR 38.901, TR 38.858, 37.885, 38.859 are used as starting point - Update on values of the LSP/SSP parameters can be discussed based on validation data acquired by measurement or ray-tracing model - FFS The power threshold for removing clusters in step 6, i.e., -25 dB is revised to X<-25 dB. FFS X - FFS whether/how to resolve the inconsistency between TRP-TRP channel according to TR 38.858 and the TRP-target (UAV) channel according to TR 36.777 when UAV and TRP are set to same height

For the bistatic UE-UE ISAC background channel, we make the following proposal:

Proposal 21: For the UE-UE bistatic sensing mode, for the ISAC background channel, the following is supported:

- For a vehicle UE – Vehicle UE: use TR 37.885

- For a non-vehicle UEs, reuse the SL Positioning TR solution:

  - Option 1: BS-to-UE channel model defined in TR 38.901 [11] is revised:

    - The UE parameters in the channel model defined in 38.901 [11], e.g., UE height, antenna model, transmit power are used to replace corresponding parameters for BS. Anchor UE height to be reported by companies, e.g., anchor UE height is the same as TRP.

  - Option 2: D2D channel model from 36.843 A.2.1.2 is used.

- Do not update the threshold

  - 4.3 Monostatic Background channel

The following agreement has been made:

Agreement - In order to define the background channel for TRP and UE mono-static sensing mode, - RAN1 to study how to model the background channel - Option 1: randomly drop at least one virtual Rx, and then the background channel is generated based on the channel generated as TR 38.901 between the real Tx and each virtual Rx for a scenario - FFS EO is modelled in the background channel - Option 2: New channel model based on measurement results or ray-tracing model validated by experimental results or radar literatures - FFS EO is modelled in the background channel - Option 3: the locations of clusters in the target channel are deterministically generated, then the background channel is generated using the clusters with the determined locations. - Other options are not precluded

The following proposal was also discussed:

[H][FL4] Proposal 6.3-1-rev3 Strive to down-select among the solutions below at RAN1#120: Solution A: (previous Option 1) On the background channel for monostatic sensing mode, RAN1 takes revised Option 1 with updates as starting point - drop N reference points following certain distribution, and then the background channel is generated based on the channel generated as in existing TR between the real Tx and each reference points for a scenario. - FFS the distribution and the mobility for reference points dropping - N is determined by validation results for each scenario, FFS value N Solution B (Previously option 2) - drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel - only direct path is modeled for each ground clutter - FFS the number, the distribution, RCS, XPR, etc. for ground clutters

For the monostatic ISAC background channel, we make the following proposal:

Proposal 22: For the monostatic TRP-TRP or UE-UE ISAC channel, support Solution B:

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - only direct path is modeled for each ground clutter

# Precipitation modelling

ISAC sensors at the microwave or millimeter wave bands would be more robust than optical or infrared sensors under precipitation conditions [1]. However, severe precipitation conditions can significantly degrade their performance in deployment scenarios, such as UE monostatic sensing for automotive application [1-3].

Figure 1 Rain simulator showing a reduction of 67% vehicle RCS for all rain layers activated

In [2], a rain simulator was developed to study the rain influence on monostatic mmWave automotive sensor for detecting a vehicle, as shown in Figure 1 (left). The watering area is 2m X 4m. There are four rain layers with each rain layer of length 1m. Figure 1 (right) shows the normalized RCS values of the target vehicle with increasing number of rain layers in a boxplot for rainfall rate of 94.5 mm/h. Normalized RCS includes the received power reduction due to precipitation effects. The first box shows the measured median RCS value of 12.13 dBsm with no rain. The last box shows a reduction of 67% for normalized RCS when all four rain layers are activated as compared to no rain. This would lead to a huge decrease in maximum detection range [3] and poor sensing estimation accuracy [1].

Figure 2 Maximum detection range reduction of vehicle (left) and pedestrian (right)

The impact of rain on detection range of a vehicle and a pedestrian using a mmWave sensor was investigated in [3] using theoretical precipitation model. The simulation results in [3], as shown in Figure 2, depict that for typical detection requirement of SINR = 13 dB, detection range of a vehicle, or a pedestrian drops by around 25% in severe conditions (50 mm/h) and > 50% in extreme conditions (400 mm/h).

In [1], the influence of rainfall on sensing estimation accuracy of lateral distance was evaluated using a 100m outdoor rain simulator with a lane width of 6m, as shown in Figure 3 (left). Several test conditions were performed including maneuvers such as accelerate leaving, accelerate approach, lateral leaving, cut-in, cut-out, separation test, and platoon test. Based on 278 measurement cases, the statistics of the sensing estimation error (defined as the difference between measured and true lateral distance) is depicted using box plots in Figure 3. It can be seen that the number of outliers for mmWave radar increased from 102 in night-dry road to 196 in night-moderate rain conditions. This increase in number of outliers for high rainfall rate is due to the increase in rain backscatter interference.

Figure 3. For high rainfall rate, a greater number of outliers are observed in sensing measurement error

Based on the above findings summarized in observation 1, we propose to model precipitation effects as it is important to understand the impact of precipitation on sensing performance. This is also highlighted in LS received from 5GAA [8],which includes precipitation as one of the sensing objects relevant for automotive scenarios.

Observation 1: Maximum detection range and sensing estimation accuracy can significantly degrade under severe precipitation conditions [1-3].

Proposal 23: Introduce precipitation modelling as an Object creating Hazards on Roads/Railways Sensing Targets.

Figure 4. Left figure shows receiver SNR/SINR of a pedestrian at different ranges for varying rainfall rates [3]. Right figure shows the range-Doppler map of a pedestrian in rain [4].

Precipitation model mainly includes two effects: backscatter and attenuation. To study the applicability of both these effects, the sensing SNR and SINR are compared at different distances between a mmWave sensor and a pedestrian for varying rainfall rates as shown in Figure 4 [3]. It can be observed that the gap between SNR and SINR initially grows and then reduces with rainfall rate. At target distance of 25 m with 100 mm/h rainfall rate, the SINR degradation due to rainfall attenuation is 2.1 and 8.9 dB due to backscatter. The rain backscatter signal power first becomes dominant and stronger and then decreases due to the attenuation. At target distance of 50 m with 100 mm/h rainfall rate, the SINR degradation due to rainfall attenuation is 4 and 3 dB due to backscatter. Therefore, it can be concluded that backscatter is dominant on the radar performance at a short distance, whereas the impact of attenuation grows with distance.

Pedestrian detection using a rain simulator with a length of 50m and a width of 4m was also studied in [4]. Measurements were carried out for heavy rain with an intensity of 96 mm/h. Figure 4 shows the range/Doppler of a pedestrian in rain with a range resolution of 0.075 m and a velocity resolution of 0.06 m/s.  Based on the above findings summarized in observation 4, we propose to model precipitation effects using backscatter model by adding an object type for “precipitation modelling” as well as including an attenuation factor along with their applicability, as described in the proposals below.

Observation 2: Precipitation effects on monostatic automotive sensor can be of two types: backscatter and attenuation. Backscatter has more impact on the sensing performance at a short distance, while the influence of attenuation grows with distance. Additionally, the backscatter effect plays a more significant role in performance degradation for a low mobility target with small RCS.

Proposal 24: Support adding an object type for “precipitation modelling” with the following modelling for backscatter effects at short distances and small velocities:

- Backscatter effect: For a 3-D volume, enclosing the Tx/Rx/Object, support dropping multiple scatter-points, up to N points, where N is FFS.

Proposal 25: Support including an attenuation factor for incorporating precipitation effect that grows with distance

  - 5.1 Precipitation backscatter effect

The total RCS, σp, from precipitation due to volume scattering can be represented by

σp=σoVp

where, σo is the precipitation RCS per unit volume and Vp is the volume of sensing resolution cell. The precipitation RCS per unit volume is a function of precipitation particle’s size distribution, velocity, and dielectric constant.

The volume Vp at range ρ with azimuth beamwidth Δθ, elevation beamwidth Δϕ, and range resolution Δρ is given by

Vp=γρ2ΔθΔϕΔρ

where γ is the constant that depends on the exact shape of the sensing beam projected area. For Gaussian beamshape with elliptical shape of the antenna-beam projected area, γ=π8ln2 [3]. Precipitation RCS due to volume scattering depends on the range, range resolution, and beamwidth of the sensor, unlike a point target model.

The precipitation RCS per unit volume σo depends on the carrier frequency, f, and can be given as

σo=αfrβ         m2/m3

where, r is the precipitation rate in mm/h. When the carrier wavelength much larger than the precipitation particle size, σo, can be modeled using Rayleigh scattering model as. For rain, we can use β=1.6 and αf=7×10-12f4  with carrier frequency f in GHz [5]. An average rain fall rate can be used to estimate the large scale precipitation RCS using the above equation.

In the case of higher carrier wavelengths, where the assumption of Rayleigh scattering does not apply, the definitions of α and β would vary for different wavelengths. For f=33 GHz, αf=0.16 and β=1.23 can be used [6].

  - 5.2 Precipitation attenuation

The precipitation attenuation in dB per km can be modeled as

A=a rb

where, the values of a and b would depend on the carrier frequency. For f=33 GHz, a=0.22 and b=1.08 can be used [6-7].

# Conclusion

We have discussed changes needed to TR38.901 to support sensing studies.

Proposal 1: Multiple bounce modelling between the scatter-points of a sensing target, is not supported.

Proposal 2: For the ISAC target channel Htarget, support the following formulation

Htargetτ,t=i=1Ntargetsj=1Mscatter-pointsiHtar, i,jτ,t

where

- Ntargets is the number of sensing targets between a pair of Tx and Rx

- The i-th sensing target is modelled as Mscatter-pointsi scatter point, where both Mscatter-pointsi=1 and Mscatter-pointsi>1 is supported.

Proposal 3: Support the following power normalization between the target and the background channel:

- HISAC=α⋅Htarget+HbISAC, where α is such that E|HISAC​2=E|​HRel182

Proposal 4: Support, as an optional modelling component in TS38.901, the absolute time of arrival modelling for both outdoor and indoor scenarios using Section 7.6.9 as a starting point for each Tx-target and target-Rx link.

- Study the parameters (μlgΔτ, σlgΔτ) and correlation distance at least for the UMi scenario

Proposal 5: When modelling one or more target channels of target(s) with one or more scattering points,

- Support applying the direct path generation independently to each scattering point.

Proposal 6: For the target channel of a target with single scattering point, when stochastic cluster is used to model an indirect path in the target channel,

- Reuse the existing number of number of clusters and rays per cluster as in TS38.901.

Proposal 7: Keep the power threshold for removing clusters in step 6 in section 7.5, TR 38.901, i.e., -25 dB.

Proposal 8: No further power normalization of target channel is performed after path dropping.

Proposal 9: With regards to RCS modelling for a small UAV, for each scattering point, for each polarization,

- AdBsm=-2.7

- μB2dBsm=-4.4

- σB2dBsm=6.17

Proposal 10: With regards to RCS modelling for a vehicle, support one of the two following alternatives:

- Alt. 1: For the 5-point scattering model with scattering centres at left, right, front, back, and roof seems with its distinct element pattern:

  - The mean (A) and variance (B2) can vary based on distance.

- Alt. 2: The 5-point scattering model applies at near range, whereas 1-point scattering model applies at far range.

Proposal 11: With regards to RCS modelling for a vehicle for each scattering point, we propose the following:

- The value of the log-normal distribution parameters will depend on carrier frequency, incident/reflected angle.

  - Mean RCS, A, is [10 to 20] dBsm.

  - Value of B2 is FFS.

  - Mean RCS can be modelled linearly with carrier frequency in logarithmic scale. The proportionality factor can depend on the type of vehicle.

With regards to B1, support Alt. 1 (formulated similar as the antenna radiation power pattern in 38.901).

Proposal 12: With regards to RCS modelling for a pedestrian, for each scattering point, for each polarization, use RCS modelling Option 2:

- For model 1,

  - A is the mean with dBsm=[-10 to 0] for adult and [-5 to -15] for child.

  - Value of B2 is FFS.

- For model 2,

  - With regards to the dependency on incident/scattered angles support Alt. 1 (formulated similar as the antenna radiation power pattern in 38.901)

  - support B1 to depend on the elevation.

Proposal 13: Component B2 of RCS is upper bounded by kσ dB with k=[3 or 4].

Proposal 14: To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point,:

- Support the initial random phase to be uniformly distributed within -π,π.

- No correlation between αi,1,αi,2,βi,1,βi,2

- Support Option 1

Proposal 15: With regards to the EO Type-2 in the target channel and the determination of the LOS condition, support Option A.

Proposal 16: With regards to doppler modelling, support specifying a function for micro-doppler modelling for the UAV and human outdoor sensing (e.g. VRUs) scenario in this release.

Proposal 17: The pathloss for Tx-Target-Rx propagation is formulated as follows, for at least some scenarios:

PLsd1,d2=PLd1+PLd2+10log10λ24πσRCS

where

- PLd1 is pathloss between Tx and target, with d1 equal to Tx-to-Target LOS distance.

- PLd2 is pathloss between Rx and target, with d2 equal to Target-to-Rx LOS distance.

- The existing pathloss formula in 3GPP TRs is reused as start point.

- λ is wave-length of sensing signal. σRCS is a large-scale RCS value

- Other scenarios requiring alternative formulations are not precluded.

- Study further the cases that the above formula doesn’t apply, and specifically under which scenarios the pathloss should have the following format:

PL2hop(d1,d2)=PL(d1+d2)+L

where L is the loss in dB caused by interaction with the object/scatter-point, and study further which models apply in which scenarios.

- The presence or not of EO type-2 in the target channel doesn’t change the determined pathloss of the target channel

Proposal 18: For the ISAC target channel, support to optionally model the interaction between a sensing target and an EO type-2 under the following modelling simplifications:

- EO type-2 is modelled as a specular reflection with Section 7.6.8 as a starting point

- For the Tx – target – EO – Rx scenario:

  - There is a LOS ray in the target to Rx link

  - Use the ground reflection model to add an additional specular ray following Section 7.6.8

- For the Tx – EO – target – Rx scenario:

  - There is a LOS ray in the Tx to target link

  - Use the ground reflection model to add an additional specular ray following Section 7.6.8

Proposal 19: For the ISAC target channel, support modelling a target with multiple scattering points with the following simplifications:

- The same number and locations of scattering points apply regardless of sensing Tx/Rx.

- There is no need to model the relative location changes between scattering points.

- Interactions between the scattering points are not modeled.

- The RCS values of the multiple scattering points are determined individually.

- In the basic formulation of the ISAC channel model, the multiple scattering points of the target are treated similarly to having multiple targets, each with a single scattering point.

Proposal 20: When EO type-2 is included in a deployment, its impact is modelled only in the ISAC target channel.

Proposal 21: For the UE-UE bistatic sensing mode, for the ISAC background channel, the following is supported:

- For a vehicle UE – Vehicle UE: use TR 37.885

- For a non-vehicle UEs, reuse the SL Positioning TR solution:

  - Option 1: BS-to-UE channel model defined in TR 38.901 [11] is revised:

    - The UE parameters in the channel model defined in 38.901 [11], e.g., UE height, antenna model, transmit power are used to replace corresponding parameters for BS. Anchor UE height to be reported by companies, e.g., anchor UE height is the same as TRP.

  - Option 2: D2D channel model from 36.843 A.2.1.2 is used.

- Do not update the threshold

Proposal 22: For the monostatic TRP-TRP or UE-UE ISAC channel, support Solution B:

- drop N ground clutters following certain distribution, generate the target channel for the N ground clutters, the sum of the N target channels is the background channel

  - only direct path is modeled for each ground clutter

Observation 1: Maximum detection range and sensing estimation accuracy can significantly degrade under severe precipitation conditions [1-3].

Proposal 23: Introduce precipitation modelling as an Object creating Hazards on Roads/Railways Sensing Targets.

Observation 2: Precipitation effects on monostatic automotive sensor can be of two types: backscatter and attenuation. Backscatter has more impact on the sensing performance at a short distance, while the influence of attenuation grows with distance. Additionally, the backscatter effect plays a more significant role in performance degradation for a low mobility target with small RCS.

Proposal 24: Support adding an object type for “precipitation modelling” with the following modelling for backscatter effects at short distances and small velocities:

- Backscatter effect: For a 3-D volume, enclosing the Tx/Rx/Object, support dropping multiple scatter-points, up to N points, where N is FFS.

Proposal 25: Support including an attenuation factor for incorporating precipitation effect that grows with distance.

# References

[1] Li, Hexuan, Nadine Bamminger, Zoltan Ferenc Magosi, Christoph Feichtinger, Yongqi Zhao, Tomislav Mihalj, Faris Orucevic, and Arno Eichberger. "The effect of rainfall and illumination on automotive sensors detection performance." Sustainability 15, no. 9 (2023): 7260.

[2] Hasirlioglu, Sinan, Alexander Kamann, Igor Doric, and Thomas Brandmeier. "Test methodology for rain influence on automotive surround sensors." In 2016 IEEE 19th International Conference on Intelligent Transportation Systems (ITSC), pp. 2242-2247. IEEE, 2016.

[3] Zang, Shizhe, Ming Ding, David Smith, Paul Tyler, Thierry Rakotoarivelo, and Mohamed Ali Kaafar. "The impact of adverse weather conditions on autonomous vehicles: How rain, snow, fog, and hail affect the performance of a self-driving car." IEEE vehicular technology magazine 14, no. 2 (2019): 103-111.

[4] Steinhauser, Dagmar, Patrick Held, Bernhard Thöresz, and Thomas Brandmeier. "Towards safe autonomous driving: Challenges of pedestrian detection in rain with automotive radar." In 2020 17th European Radar Conference (EuRAD), pp. 409-412. IEEE, 2021.

[5] Skolnik, Merrill Ivan. Introduction to radar systems. New York: McGraw-hill, 1980.

[6] Gunn, Kenrich Lewis Stuart, and Thomas William Russell East. "The microwave properties of precipitation particles." Quarterly Journal of the Royal Meteorological Society 80, no. 346 (1954): 522-545.

[7] Olsen, R. O. G. E. R. S., David Rogers, and Daniel Hodge. "The aRb relation in the calculation of rain attenuation." IEEE Transactions on antennas and propagation 26, no. 2 (1978): 318-329.

[8] R1-2405964, “LS on Physical Properties of Sensing Targets in Automotive Scenarios for ISAC”, 5GAA LS Working Group 4

[9] Schuler, Karin, Denis Becker, and Werner Wiesbeck. "Extraction of virtual scattering centers of vehicles by ray-tracing simulations." IEEE Transactions on Antennas and Propagation 56, no. 11 (2008): 3543-3551.

[10] Kärnfelt, Camilla, Alain Péden, Ali Bazzi, Ghayath El Haj Shhadé, Mohamad Abbas, and Thierry Chonavel. "77 GHz ACC radar simulation platform." In 2009 9th International Conference on Intelligent Transport Systems Telecommunications,(ITST), pp. 209-214. IEEE, 2009.

[11] Suzuki, Hiromu. "Measurement results of radar cross section of automobiles for millimeter wave band." In Proceedings of the 7th World Congress on Intelligent Systems. 2000.

[12] Lee, Seongwook, Seokhyun Kang, Seong-Cheol Kim, and Jae-Eun Lee. "Radar cross section measurement with 77 GHz automotive FMCW radar." In 2016 IEEE 27th Annual International Symposium on Personal, Indoor, and Mobile Radio Communications (PIMRC), pp. 1-6. IEEE, 2016.

[13] Cai, Xiuzhang. "Autonomous vehicles: Mmw radar backscattering modeling of traffic environment, vehicular communication modeling, and antenna designs." PhD diss., 2020.

[14] Bands, Automotive Radar. "Radar Cross Section Measurements of Pedestrian Dummies and Humans in the 24/77 GHz Frequency Bands." JRC Scientific and Policy Reports, 2013.

[15] Cai, Xiuzhang. "Autonomous vehicles: Mmw radar backscattering modeling of traffic environment, vehicular communication modeling, and antenna designs." PhD diss., 2020.

[16] Yasugi, Makoto, Yunyun Cao, Kiyotaka Kobayashi, Tadashi Morita, Takaaki Kishigami, and Yoichi Nakagawa. "79GHz-band radar cross section measurement for pedestrian detection." In 2013 Asia-Pacific Microwave Conference Proceedings (APMC), pp. 576-578. IEEE, 2013.

[17] Belgiovane, Domenic, Chi-Chih Chen, Ming Chen, Stanley Y-P. Chien, and Rini Sherony. "77 GHz radar scattering properties of pedestrians." In IEEE Radar Conference, pp. 0735-0738. IEEE, 2014.

[18] Motomura, Toshiki, Kazuki Uchiyama, and Akihiro Kajiwara. "Measurement results of vehicular RCS characteristics for 79GHz millimeter band." In 2018 IEEE Topical Conference on Wireless Sensors and Sensor Networks (WiSNet), pp. 103-106. IEEE, 2018.