3GPP TSG RAN WG1 #120	R1-2501294

Athens, Greece, Feb 17 - Nov 21, 2025

Agenda item:		9.6.1

Source:	Nordic Semiconductor ASA

Title:					On LP-WUS and LP-SS design

Document for:		Discussion and Decision

# Introduction

In RAN#102, work item in RP-234056 has been approved. In this contribution we focus on LP-WUS and LP-SS design aspects.

# Discussion

## Waveform generation

Agreements related to waveform generation and comparison are summarized below.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4 Agreement For the SCS used for LP-WUS and LP-SS, further discuss the following options: - The single SCS is configured by gNB - The single SCS is determined by pre-defined rule Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap

OOK-4 with higher M>1 should be supported to increase the data-rate. On the other hand, larger values of M require better synchronization. For example, with M=2 robustness has been observed to be 3us, with M=4 robustness dropped to 1us already for 30kHz SCS. Tighter synchronization requirements will result in more power consumption and/or more frequent transmission of LP-SS, therefore basic idle-mode LP-WUS should contain only M=1 or 2 for 30kHz SCS carrier and M=1,2 or 4 for 15kHz SCS carrier.

Moreover, one serious issue has been raised during Orlando discussions. With M=1 modulation, 1 slot can accommodate at most 14 chips, 7 physical bits, assuming Manchester coding. If M=1 is configured, it is clear that LP-WUS will require more than one slot. In general, crossing subframe boundary has not been an issue for NB-IoT, where DL TBS can be mapped to multiple resource units spanning more than 1ms. For this case eNB shall guarantee constant EPRE to the NB-IoT UE. With single-RB transmission this seems to be not an issue when NB-IoT is multiplexed with LTE or NR. LP-WUS is 11RB and can be a problem for gNB implementation. Furthermore, it remains open whether beam-sweeping and multiple MOs per beam can by physically accommodated in the cell. Therefore, being pragmatic here, RAN1 should focus on M=2 design as primary/mandatory design for LP-WUS.

Proposal-1: IDLE-mode LP-WUS can be configured in a 15-kHz or 30-kHz DL NR carrier.

- M=2 is mandatorily supported.

- M=1,4 is optionally supported.

In idle mode, LP-WUS SCS follows CD-SSB. Mixed SCS cases where SSB is of different SCS than CORESET#0 are to our knowledge not deployed. The reason is that carrier may not use spectrum around SSB for transmission of other signals and channels of the cell with mixed SCS. This resulting in wasting spectrum. Furthermore, majority of network vendors expressed concerns about mixed SCS for LP-WUS, it is expected that also initial-BWP and Redcap-Initial-BWP will have always the same numerology. Therefore, we suggest concluding the following:

Proposal-2: Case where associated CD-SSB and initial DL BWP and Redcap-specific initial DL BWP (if any) have different SCSs is not supported or deprioritized.

## OFDM overlaid

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainàDFT/LSàIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

In previous meetings ZC was selected as overlaid sequence and framework was established. Now details are missing. The SCS-agnostic BW is 11 PRB = 132SCs. The actual design could follow Table 1, where s denotes a ZC sequence and z denotes vector of zeros

Table 1 Sequences to be specified.

| M | Manchester-coded value | OFDMA overlaid sequence in time domain within 132 SCs |
| --- | --- | --- |
| 1 | 0 or 1 | [z1] |
|  |  | [0 0 s1 0 0 ] |
| 2 | 0 | [z3 s2 0 0 0 0 0 ] |
| 2 | 1 | [0 0 0 0 0 s2 z3 ] |
| 4 | 00 | [z4 s4 0 0z4 s4 0 0] |
| 4 | 01 | [z4 s40 0 0 0 s4z4] |
| 4 | 10 | [ 0 0 s4z4z4 s4 0 0] |
| 4 | 11 | [ 0 0 s4z40 0 s4z4] |
| [-] denotes one OFDMA symbol |  |  |

The following length of sequences are proposed:

- z1 length 132 zeros

- s1 length is Bzc=127,Lzc=128 as per working assumption for M=1

- s2 length is Bzc=61 and z2 is length of 66 for M=2, Lzc=132

- s4 length is Bzc=31 and z4 is length is 33 for M=4, Lzc=132

Above proposal should be consistent with sequence generation of the following alternatives.

Proposal-3: For M>1 support the following alternatives:

  - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by extension of

,

    - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

    - Note it doesn’t preclude any pulse shaping scheme if any.

  - Alt2:

Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

## On support of coding

Agreement Support Manchester coding for LP-WUS - FFS other coding schemes For future meetings: Companies are encouraged to consider [H][FL4] Proposal 3.3-1 in R1-2410838 Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: adding CRC, e.g., - CRC is added to LP-WUS information bitsà Manchester coding - FFS one or multiple CRC size(s) - How to adapt CRC size to meet target requirements under different ISD settings channel coding other than Manchester coding, including rate matching if any, e.g., - LP-WUS information bitsàbased on existing coding scheme in 38.212, e.g., REED-MULLER coding and rate matching if anyà Manchester codingà WUS block mapped to a binary sequence/codeword, include truncation if any, e.g., - LP-WUS information bitsà sequence mapping and truncation if anyà Manchester coding repetition, including rate matching if any, e.g., - bit level repetition - LP-WUS information bità bit level repetitionà Manchester coding - OFDM level repetition - LP-WUS information bits in one OFDM symbol à Manchester coding à OFDM level repetition Block level repetition - LP-WUS information bitsà Manchester coding à block level repetition Note: combination of options above is not precluded Note: after Manchester coding, WUS block would be obtained. Note: assume that full waking up information is provided in single MO Note: assume the maximum number of information bits is e.g., 6, 7, 8 Companies are encouraged to report the number of OFDM symbols in one MO (for evaluation purpose, assume the repetitions in single MO for simplicity) FFS how LP-WUS is transmitted within a MO/MOs

It still remained open whether Manchester is mandatory or optionally applied to LP-WUS. In our opinion, Manchester should be mandatory part of LP-WUS design, this is essential for LP-WUR Rx design.

Proposal-4: Manchester coding is applied to every LP-WUS, i.e. irrespective of M and SCS.

In past meetings discussion has been carried regarding coding/forward-error-correction/redundancy. Some companies presented claims that FAR from AWGN is dominant over the FAR from detecting falsely wake-up codepoint of the other UE. This could be the case if, paging rate within an MO of a subgroup is very low and codepoints with small Hamming distance occur rarely. On the other hand, if paging rate within MO is high, small Hamming distance between codepoints should dominate. Therefore, for robust design, codepoints should include additional redundancy to increase Hamming distance between codepoints of different sub-groups.

Reed-Muller would increase Hamming distance between codepoint of sub-groups, and could enable optional forward error correction as well, depending on implementation. Such coding works for 1-11bits of payload length. For K>2, the code-length is 32, however rate-matching is simple truncation and repetition. For our purpose code-length of 16bits would be more suitable.

Proposal-5: Add redundancy to codepoint to increase Hamming distance.

Observation-1: Small block length coding in NR offers out-of-box solution to support 1-6bits of payload. It provides redundancy as well as possibility for FEC. At the same time, UE may detect just a sequence.

Proposal-6: Consider reuse small block length coding in NR to meet target requirements for LP-WUS reception.

## Payload

Agreement In case of overlaid OFDM sequence not carrying information: - Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. - Note 1: multiple overlaid OFDM sequences are specified. - Note 2: gNB can configure different overlaid OFDM sequence(s) for different cells. Agreement Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/Mos Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4

Table 2 illustrates an example of agreed Option 2-2, where overlaid sequences carry all bits, but single ON-symbol carries only 3bits. If there are 31 dedicated sub-groups and one sub-group dedicated to wake-up all, 5bits are dedicated to carry information codepoint and 3bits provide additional redundancy. With M=2, this resulting in 8 OFDMA symbols per LP-WUS.

Table 2 Allocation of overlaid bits for M=2 supporting 31 subgroups + 1 subgroup to wake-up all

| Manchester symbol | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OOK bit (not chip) | b1 | b2 | b3 | b4 | b5 | b6 | b7 | b8 |
| Overlaid sequences carry all bits one ON-symbol carrying 3-bits (Option 2-2) | b3b4 b5 | b6 b7 b8 | b1b2 b5 | b6 b7 b8 | b1b2 b3 | b4 b7 b8 | b1b2 b3 | b4 b5 b6 |

In previous meeting it was agreed that when M=4, the number ZC sequences carrying information in one ON-chip is 4. In our assumption, this would be single base-ZC sequence with 4 different cyclic shifts. When M=2, the ZC length doubles and thus ZC base sequence can support double amount cyclic shifts with similar detection performance.

Proposal-7: The maximum number of sequences carrying information in a ON-chip is 8 for M=2 and 16 for M=1.

When bits are organized like in Table 3, OFDM receiver may detect only two ON-symbols, and such extract all 8bits transmitted, and when SNR conditions are not favorable, use 4 sub-sequent repetitions, this providing trade-off between power consumption and detection performance.

The next question one should ask is how/weather to support less subgroups than 32. Simply, network could allocate less sub-groups than 32 while reusing L1 design for 32 sub-groups, the amount of redundancy bits would increase without a need for new design. Alternatively, we could add the following design, to support 3+1 sub-groups (as example in Table 3), this resulting in 2 information bits, 2 redundancy bit and LP-WUS length of 4 symbols.

Table 3 Allocation of overlaid bits for M=2 supporting 3 subgroups + 1 subgroup to wake-up all

| Manchester symbol | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| OOK bit (not chip) | b1 | b2 | b3 | b4 |
| Overlaid sequences carry all bits one ON-symbol carrying 3-bits (Option 2-2) | b2b3 b4 | b1b3 b4 | b2b1 b4 | b2b3 b1 |

Proposal-8: Discuss the granularity of payload in LP-WUS. In our opinion, it is not necessary to support all payload sizes, i.e. having separate L1 designs for every number of sub-groups from 1 to 32 sub-groups.

## Synchronization

Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz Agreement For LP-SS periodicity, support at least 320ms - FFS: 80ms,160ms, 640ms,1280ms, 2560ms, 5120ms, 10240ms Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32} Agreement To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy. Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

Only a single value of L per M should be supported to reduce complexity of implementation. And selection should follow L=M*[4,8]. As other’s commented in previous meeting, LP-SS should not cross subframe boundary.

Proposal-9: Length of binary sequence is L=M*[4 or 8], i.e. there are three different sequences, one for each M.

We think it should be left up to gNB irrespective of M, whether overlaid is used or not. OOK-receiver relies on LP-SS, and OFDMA receiver on SSB/SSS. We prefer unified design for all M.

Proposal-10: Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable irrespective of value of M.

Regarding additional sync signal, our preference is Option 1A. In our opinion, 320ms periodicity for M=1 and M=2 should be sufficient, for M=4 if deployed, network would provide lower periodicity. Lower periodicity for M=4, will relax design requirement for LP-SS as discussed in Section 2.1.

Proposal-11: Are preference is Option 1A

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms] for M=4

# Conclusions

Proposal-1: IDLE-mode LP-WUS can be configured in a 15-kHz or 30-kHz DL NR carrier.

- M=2 is mandatorily supported.

- M=1,4 is optionally supported.

Proposal-2: Case where associated CD-SSB and initial DL BWP and Redcap-specific initial DL BWP (if any) have different SCSs is not supported or deprioritized.

Proposal-3: For M>1 support the following alternatives:

  - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by extension of

,

    - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

    - Note it doesn’t preclude any pulse shaping scheme if any.

  - Alt2:

Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Proposal-4: Manchester coding is applied to every LP-WUS, i.e. irrespective of M and SCS.

Proposal-5: Add redundancy to codepoint to increase Hamming distance.

Observation-1: Small block length coding in NR offers out-of-box solution to support 1-6bits of payload. It provides redundancy as well as possibility for FEC. At the same time, UE may detect just a sequence.

Proposal-6: Consider reuse small block length coding in NR to meet target requirements for LP-WUS reception.

Proposal-7: The maximum number of sequences carrying information in a ON-chip is 8 for M=2 and 16 for M=1.

Proposal-8: Discuss the granularity of payload in LP-WUS. In our opinion, it is not necessary to support all payload sizes, i.e. having separate L1 designs for every number of sub-groups from 1 to 32 sub-groups.

Proposal-9: Length of binary sequence is L=M*[4 or 8], i.e. there are three different sequences, one for each M.

Proposal-10: Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable irrespective of value of M.

Proposal-11: Are preference is Option 1A

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms] for M=4