3GPP TSG-RAN WG1 Meeting #120	  R1-2501331

Athens, Greece, February 17 – 21, 2025

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on the LS reply to SA2 on AI/ML Positioning of Case 3b

Document for:	Discussion and Decision

# Introduction

After the SA2#166-AdHoc-e meeting, SA2 has sent a LS [1] to RAN1 to ask questions on current progress and future work plans on data types and procedures defined for Case 3b.

# 1 Overall description The WID AIML_CN defines that TS 23.273 includes that the LMF can perform UE Positioning using an AI/ML Model, this corresponds to case 3b as currently being worked on by RAN WGs in NR_AIML_Air WID. The AIML_CN WID obtained an exception to finalize normative work at SA2#167 that contains some Editor´s Note related to RAN3 work. SA2 asks RAN3 to provide feedback for the purpose to resolve the following Editor´s Notes in Clause 5.18 (“Support for UE positioning based on a ML Model at the LMF “) and Clause 6.22.3 (“Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements”) in TS 23.273 v.19.1.0: Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback. Editor's note: Whether and how the LMF takes the RAN load into account in this procedure is FFS. Please provide a reference to the RAN TS that includes the procedure for data collection by LMF from NG-RAN, so that SA2 can update TS 23.273 and remove these Editor´s Note (or an estimation of the time plan in RAN3). SA2 asks RAN1 to provide current progress and future work plans on data types defined for case 3b. If there is any existing agreement in RAN, please provide a reference. # 2 Actions To RAN3 ACTION: SA2 asks RAN3 to provide feedback on the Editor’s Notes in Clause 5.18 and Clause 6.22.3 in TS 23,273 v.19.1.0 and to provide references to the corresponding procedures in RAN TSs (or an estimation on when those procedures can be available). To RAN1 ACTION: SA2 asks RAN1 to provide current progress and future work plans on data types defined for case 3b. If there is any existing agreement in RAN, please provide a reference.

This contribution discusses the draft reply to the SA2 LS.

# Discussion on the LS reply

In previous RAN1 meetings, the following agreements have been achieved related to Case 3b.

Agreement (RAN1#116) For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement (RAN1#116) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement (RAN1#116bis) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. Agreement (RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption (RAN1#117) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement (RAN1#117) For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement (RAN1#118) For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement (RAN1#118) For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement (RAN1#118bis) For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Agreement (RAN1#118bis) From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement (RAN1#119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Regarding the input type of the 1st Editor’s notes, RAN1 has agreed the input data includes Part A (measurement) and Part B (label). In addition, the existing IE is reused to indicate the quality indicator of timing information in Part A when reported.

In addition, for channel measurement, it includes two types: timing information, and paired timing information and power information. RAN1 has agreed to introduce enhancement to path-based measurement in RAN1#119 as the type of the channel measurement, and RAN1 has agreed to reuse legacy UL SRS-RSRPP as the type of the power information for the channel measurement. Besides, existing IE “Time Stamp” in TS 38.455 can be reused as the time stamp of the channel measurement.

Moreover, RAN1 has agreed that the purpose of the channel measurements, such as for Rel-19 AI/ML based positioning, is not specified for Case 3b. That means how to use the channel measurement is up to LMF implementation.

In our view, how to collect the input data may be left to RAN2.

Regarding the future work plan on the data type, though the principle to generate the channel measurements for Case 3b has been agreed, there are still some FFS points such as the exact values of Nt' and k. In addition, for Part B, there seems no need to have additional spec impact, since LMF can obtain the label of coordinates by implementation, or with legacy positioning methods.

Proposal 1: Reply the SA2 LS on the input types for Case 3b:

- RAN1 has agreed the input data includes Part A (measurement) and Part B (label).

  - Existing IEs are reused to indicate the quality indicator of timing information in Part A when reported.

- For channel measurement, it includes two types: timing information, and paired timing information and power information.

  - Existing IEs are reused to indicate the time stamp of channel measurement.

  - RAN1 has agreed to introduce enhancement to path-based measurement as the type of channel measurement.

  - Note: the purpose of the channel measurements, such as for Rel-19 AI/ML based positioning, is not specified for Case 3b.

- Regarding the future work plan on data type,

  - For channel measurement type, RAN1 will further discuss the FFS issues on the values of some parameters for the enhanced path-based measurement.

  - For label type, there is no need for additional spec efforts.

Proposal 2: Reply the SA2 LS with the related RAN1 agreements in the Annex.

# Conclusions

In this contribution, we discussed the questions on current progress and future work plans on data types for Case 3b and the corresponding LS reply. The proposals are provided in below.

Proposal 1: Reply the SA2 LS on the input types for Case 3b:

- RAN1 has agreed the input data includes Part A (measurement) and Part B (label).

  - Existing IEs are reused to indicate the quality indicator of timing information in Part A when reported.

- For channel measurement, it includes two types: timing information, and paired timing information and power information.

  - Existing IEs are reused to indicate the time stamp of channel measurement.

  - RAN1 has agreed to introduce enhancement to path-based measurement as the type of channel measurement.

  - Note: the purpose of the channel measurements, such as for Rel-19 AI/ML based positioning, is not specified for Case 3b.

- Regarding the future work plan on data type,

  - For channel measurement type, RAN1 will further discuss the FFS issues on the values of some parameters for the enhanced path-based measurement.

  - For label type, there is no need for additional spec efforts.

Proposal 2: Reply the SA2 LS with the related RAN1 agreements in the Annex.

# References

S2-2501341, LS on LMF-based AI/ML Positioning for case 3b, TSG-SA2 Meeting #166-AdHoc-e, 20 – 24 January 2025, E-meeting.

# Annex

Agreement (RAN1#116) For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. Agreement (RAN1#116) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement (RAN1#116bis) For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT. Agreement (RAN1#116bis) For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Working Assumption (RAN1#117) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. Agreement (RAN1#117) For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Agreement (RAN1#118) For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications. Agreement (RAN1#118) For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement (RAN1#118bis) For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information Agreement (RAN1#118bis) From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement (RAN1#119) For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified