3GPP TSG RAN WG1 #120		R1-2501351

Athens, Greece, 17th February – 21st February 2025

Agenda item:	9.4.1

Source:		Indian Institute of Technology Madras [IITM]

### Title:			Discussion on Physical channels design for Ambient IoT

Document for:	Discussion

1. Introduction

In the RAN 106 meeting, the work item was approved, with the RAN1 scope being as follows [1].

- RAN1 scope:

  - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively.

  - R2D and D2R signal(s)

  - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA.

  - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations.

  - R2D transmission supports only the Manchester line code in TR 38.769

  - D2R transmission supports:

    - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and

    - A corresponding small frequency shift method according to the options in TR 38.769.

  - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis).

  - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1.

  - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

In this document we discuss the CP handling at the UE side, modulation and TB size selection for the R2D and the D2R signal.

For CP handling there are two methods discussed in the study item phase and are summarized in the TR 38.769 [2].

Method Type 1:	Removal of CP at device without specified transmit-side.

Method Type 2:	Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n.

For Method Type 1, two ways that CP location/length can be determined are studied:

Alt M1-1:	Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols

Alt M1-2:	Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration

For Method Type 2, two approaches regarding subcarrier orthogonality are studied:

Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol.

Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same.

Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP.

Alt M2-2: Method Type 2 does not retain subcarrier orthogonality.

CP handling:

In method type 1, the reader inserts the CP, and the device removes the CP without any additional information from the reader. In method type 2, the reader needs to insert the CP either by making the first and last chip to be the same (Alt M2-1-1), or the reader needs to ensure that no transition edge occurs during the CP.  To achieve method 2, the chip information needs to be changed, which might incur additional overhead on the device side. Therefore, method type 1 is a better option.

For Alt M1-1, the device assumes that the CP length is the same for each OFDM symbol. Although it results in some performance drop, it has a simple implementation on the device side.

For Alt M1-2, the device requires a counter to count the number of samples and keep track of the transition edges. The device can find out the chip duration from the R-TAS signal.

Proposal 1: For CP handling, Method type 1 should be prioritized since Method type 2 might incur additional complexities on the reader and the device side.

M value selection:

Observation 1: For a given SNR, the performance of the R2D decreases as the value of M increases. Therefore, M should be scheduled based on the link performance.

Proposal 2: In R2D, the number of chips per OFDM symbol should be selected based on the link performance in the previous D2R messages.

For e.g. Initially, M can be selected based on the initial messages sent by the D2R for initial attachment procedure.

TB size selection:

For D2R and R2D, as the TB size increases the large-scale fading effects of the channel affect the received waveform. This might cause an error floor at the receiver. To avoid this, small TBs should be scheduled in cases where the large-scale fading effects are less prominent. The figure below shows the waveform for a D2R signal (TB size = 96 bits) in the presence of large-scale fading effects. In such cases, it’s difficult for the receiver to set a threshold and decode the TB correctly.

Proposal 3: For R2D and D2R signals, the TB size should be chosen based on the channel conditions.

3. References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

- TR 38.769: Study on solutions for ambient IoT (Internet of Things)