3GPP TSG RAN WG1 #120		                                                              R1-2501440

Athens, Greece, February 17th – 21st, 2025

Agenda item:	9.1.1

Source:	Samsung (Moderator)

Title:	FL summary #1 for AI/ML in beam management

Document for:	Discussion and Decision

# Introduction

In RAN#102, Rel-19 work item on “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface” is endorsed. The objective of the work item is as follows.

Provide specification support for the following aspects: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, summarized the contributions in RAN 1 #120 on AI/ML for beam management.

## (FL0) Question 0

- Please consider entering contact info below for the points of contact for this email discussion.

| Company | Contact | Email address |
| --- | --- | --- |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Indian Institute of Technology Madras | Sai Prasad Pirati | venkatasiva@5gtbiitm.in |
| OPPO | Jeffrey (Jianfei) Cao | caojianfei@oppo.com |
| Ericsson | Henrik Ryden | Henrik.a.ryden@ericsson.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| MediaTek | Yu-Jen Ku | yu-jen.ku@mediatek.com |
| TCL | Pu Yuan | pu.yuan@tcl.com |
| CMCC | Yi Zheng Jiazhen Zhang | zhengyi@chinamobile.com zhangjiazhen@chinamobile.com |
| NTT DOCOMO | Haruhi Echigo | haruhi.echigo.fw@nttdocomo.com |
| InterDigital | Youngwoo Kwak | youngwoo.kwak@interdigital.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| NEC | Peng Guan Pravjyot Deogun Zhen He Yi Jiang | Guan_peng@nec.cn pravjyot.deogun@EMEA.NEC.COM he_zhen@nec.cn y-jiang_ct@nec.com |
| ETRI | Yongjin Kwon | yjkwon@etri.re.kr |
| Xiaomi | Mingju | limingju@xiaomi.com |
| ZTE | Wenfeng LIU Xingguang WEI | liu.wenfeng@zte.com.cn wei.xingguang@zte.com.cn |
| Qualcomm | Hamed Pezeshki | hamedp@qti.qualcomm.com |
| Spreadtrum | Shijia Shao | Shijia.shao@unisoc.com Hualei.wang@unisoc.com |
| Panasonic | Henry Tran | xuantuong.tran@sg.panasonic.com |
| CATT | Min Zhu | zhumin@catt.cn |  |
| CEWiT | Ebin Chacko Shiv Shankar | echacko@cewit.org.in shiv@cewit.org.in |  |
| Google | Yushu Zhang | yushuzhang@google.com |  |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |  |
| Futurewei | Zhigang Rong | zrong@futurewei.com |  |
| LG | Seongwon Go | sw.go@lge.com |  |
| Lenovo | Bingchao Liu | Liubc2@lenovo.com |  |
| Fraunhofer HHI | Baris Göktepe | Baris.goektepe@hhi.fraunhofer.de |  |
| KDDI | Taishi Watanabe | ta-watanabe@kddi.com |  |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |  |
| SONY | Chen Sun Yingshuang Bai | chen.sun@sony.com yingshuang.bai@sony.com |  |
| Hw/HiSi | Yuan Li | liyuan3@huawei.com |  |
| Intel | Debdeep Chatterjee | debdeep.chatterjee@intel.com |  |
| Apple | Weidong Yang | wyang23@apple.com |  |
| CAICT | Xiaofeng Liu | Liuxiaofeng1@caict.ac.cn |  |
| China Telecom | Bei Yang Wei Li | yangbei1@chinatelecom.cn liw40@chinatelecom.cn |  |
| Kyocera | Omar Sleem Amit Kalhan | omar.sleem@kyocera.com amit.kalhan@kyocera.com |  |
| Nokia | Keeth Jayasinghe | keeth.jayasinghe@nokia.com |  |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |  |

# Performance monitoring for UE-sided model

Agreement (RAN 1 #117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring Agreement(RAN 1 #118bis) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives Agreement(RAN 1 #119) At least for the monitoring Type 1 Option 2 of UE-side model monitoring(when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 1: For Rel-19 AI/ML-based BM, for BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), there is no need to support other alternatives in addition to the already agreed Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin). - The full set of Set A is configured as the resource set/resources for monitoring, preferably with a much longer period than Set B to reduce overhead. |
| Huawei/HiSi | Proposal 25: For monitoring Type 1 Option 2 of UE-side model, for event-triggered reporting, Event-1 can be prioritized: Event is triggered when the beam ID prediction accuracy metric is lower than a threshold and satisfies a timer/counter, where the beam ID accuracy metric could be the statistical value for a set of monitoring samples. The threshold/timer/counter is configured by NW. Proposal 26: For monitoring Type 1 Option 2 of UE-side model, regarding the calculation of BAI (beam accuracy indicator), for a single predicted instance of BM-Case 1 or BM-Case 2, define the accurate state as “Top-M/K: all the Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among the Top-K predicted beams”. - Note 1: This option is compatible to Option 1 and Option 2 in FL proposal when M and K are configured with specific values. - Note 2: Option 3a and Option 3b in FL proposal may overestimate the accuracy, e.g., in case genie-aided Top-1 beam is NOT among Top-K predicted beams but Top-K predicted beams are among weaker beams of the genie-aided Top M beams, the genie-aided Top-1 beam is missed by prediction yet it will still be counted as an “accurate state”. Proposal 27: For monitoring Type 1 Option 2 of UE-side model, consider a unified solution to report the BAI over N≥1 monitoring samples. - In case N=1, it corresponds to per sample based BAI report; otherwise it corresponds to the report of statistic result. - Preclude invalid monitoring samples among N monitoring samples for BAI calculation. A monitoring sample may be invalid for - Case 1: mismatched prediction result and monitoring resource set in case the monitoring resource set is subset of Set A - Case 2: failed CSI processing of the monitoring CSI report or its linked inference CSI report, e.g., due to unsatisfied CPU occupancy or timeline of the CSI report. Proposal 28: For the monitoring Type 1 (Option 1 and Option 2) of UE-side model monitoring, reuse the P-CSI/SP-CSI/A-CSI report to carry the monitoring results. - For Type 1 Option 1, the report of the per sample measurement result is performed based on the configuration/activation/triggering of P-CSI/SP-CSI/A-CSI report via L1 signaling. - For Type 1 Option 2, the metric can be reported when N≥1 monitored samples have been collected. - For P-CSI/SP-CSI report, the statistic BAI is reported by the last CSI report occasion among N monitoring occasions. - For A-CSI report, the statistic BAI over historic N monitoring occasions is reported based on the indication of DCI. Proposal 29: For the monitoring Type 1 Option 2 of UE-side model, following combinations on time domain behavior between inference CSI report and monitoring CSI report can be considered: - P-CSI inference report can be combined with P-CSI/SP-CSI/A-CSI monitoring report. - SP-CSI inference report can be combined with SP-CSI/A-CSI monitoring report. A-CSI inference report can be combined with A-CSI monitoring report. Proposal 30: For the monitoring Type 1 Option 2 of UE-side model, to indicate the linkage between the monitoring CSI report and the inference CSI report, configure the CSI-ReportConfigId of the associated inference CSI report in the configuration of monitoring reporting. Proposal 31: For the monitoring Type 1 Option 2 of UE-side model, the time instance of the inference CSI report should be no later than the time instance of the corresponding monitoring CSI report. Proposal 32: For the monitoring Type 1 Option 2 of UE-side model for BM-Case 1 and BM-Case 2, for a specific monitoring CSI report, consider following steps to determine the prediction instances and the paired monitoring resources for metric calculation: - Step 1: Find a latest inference CSI report no later than the monitoring CSI report. - Step 2: For each predicted instance subject to the inference CSI report, determine a paired monitoring resource. - For BM-Case 1, the paired monitoring resource is selected to be “closest” to the CSI reference resource of the inference CSI report. - For BM-Case 2, the paired monitoring resource is selected to be “closest” to that predicted instance. - Step 3: Calculate the monitoring accuracy metric based on the pair of monitoring resource and the predicted instance. Figure 5 Procedure to pair the predicted instance and the monitoring resources (BM-Case 2 as example) Proposal 33: For monitoring Type 1 Option 2 of UE-side model, if subset of Set A is used as the monitoring resource set, Alt.1 or Alt.3 are slighted preferred to resolve the mismatch issue: - Alt.1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if Top-K predicted beam(s) are included in the subset of Set A. - FFS the case where all the monitoring sample(s) for metric calculation are invalid. - Alt.3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A. - UE may need to report the supported patterns of the monitoring resource set (i.e., supported beams in Set A to construct monitoring resource set). |
| Spreadtrum, UNISOC | Proposal 12: For UE-sided model, at least for BM-Case1, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information. • How to obtain the ranking information belongs to UE implementation. Proposal 13: Not support Alt.4 for Option 2 (UE-assisted performance monitoring). Proposal 14: For the monitoring Type 1 Option 2 of UE-side model, support the resource set(s) for monitoring is configured in a dedicated report configuration. |
| Kyocera | - For the performance monitoring of a UE side AI/ML model, for the content of the report of type 1 option 1 monitoring, support the following as a starting point: - Model related outputs: - Beam information of the top-K predicted beams. - Predicted RSRP if supported by the AI/ML model. - Probability information of the predicted beams. - Confidence information of the of the RSRP if supported by the AI/ML model. - Performance related measurements from the resource set configured for monitoring: - CRI/SSBRI - L1-RSRP of the top-K beams - For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding: - How to report- assess the impact of adopting the following methods: - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance. - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB. - What to report- Support using the following metrics: - Alt1: Beam prediction accuracy - Alt2: The L1-RSRP difference between the predicted beam and the measured beam from the monitoring resource set. - Alt3: The RSRP difference between the predicted RSRP from the UE-side AI/ML model and the measured L1-RSRP from the monitoring resource set. - For BM Case 1 and Case 2 in a UE side AI/ML model, support Alt1 through Alt3 and deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring. - For a UE-side AI/ML model in BM-Case 1 and BM-Case 2, utilizing option 2 (UE-assisted) performance monitoring, the following aspects should be considered regarding the Beam Accuracy Indicator (BAI): - When the size of the performance monitoring set equals the size of Set A, support the following options for the definition of the sample accuracy value: - Option 1 (Top-1/1) sample accuracy: A binary value that characterizes the event when the largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is Top-1 predicted beam. - Option 2 (1/Top-) sample accuracy: A binary value that characterizes the event when the largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring is among the Top- predicted beams. - Option 3 (Top-/) sample accuracy: A binary value that characterizes the event when at least one of the predicted Top- beams is among Top beams with the largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. - At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring, - The CSI-ReportConfigID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - The resource set(s) for monitoring can be configured for full set of Set A, or a subset of Set A, [or different from Set A]. - For the case that a subset of Set A is configured as the resource set(s) for monitoring, further study the following: - Alt 1: Performance metric is calculated (i.e., as valid instance for metric calculation) only if at least one of the Top-K predicted beam(s) are included in the subset of Set A. - Alt 2: For performance metric calculation, with predefined mapping between the full resources for Set A to the subset of set A (with a multi-to-one mapping so that all resources for Set A has a mapped resource for monitoring), consider using a down-sampling factor as a starting point. - FFS QCL relationship can be used for the mapping. - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A - Support using a mapping from beam information to performance monitoring resources. - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report - Alt 4: If the Top K predicted beam(s) is not covered by the resource set(s) for monitoring, the reported result(s) is counted as inaccurate. - Other alternatives are not precluded. - For a UE-side AI/ML model Type 1, option 2 (UE-assisted) performance monitoring, when the performance monitoring set is configured in a dedicated report separate from the inference report configuration, consider introducing a new IE “BeamCorrespondence” within “NZP-CSI-RS-Resource.” This IE should link a performance monitoring resource to a beam index. - FFS: Other alternatives - At least for the monitoring Type 1 Option 2 of UE-side model monitoring, for the time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report, support the following: - Periodic inference report and either periodic, semi-persistent or aperiodic report for monitoring. - Aperiodic inference report and aperiodic report for monitoring. - Semi-persistent inference report and either semi-persistent or aperiodic report for monitoring. - For UE-side AI/ML performance monitoring, RAN1 should further study the following: - An event is defined as the scenario when the performance metric falls above (or below) a certain threshold. - The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events. - For UE side AI/ML performance monitoring, type 2 monitoring has no specification impact, and it should be left up to the UE vendor implementation. |
| Spreadtrum | Proposal 11: support Alt.2 for Option 2 (UE-assisted performance monitoring) Proposal 14: For the monitoring Type 1 Option 2 of UE-side model, support the resource set(s) for monitoring is configured in a dedicated report configuration. |
| China Telecom | Proposal 11: At least support Beam prediction accuracy related KPIs as the metric for performance monitoring. Proposal 12: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring) Alt 1, the resource set for measurement is configured as the full Set A. Proposal 13: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support Alt. 2, i.e., the L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resource for monitoring. Proposal 14: If the number of AI/ML model failure reaches a predefined threshold, consider performing model switch or fallback to non-AI/ML method. |
| Tejas | Proposal 6: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Type 1 performance monitoring Option 1 (NW-side performance monitoring), L1 signaling can be used to send the measurement results to NW for the calculation of performance metrics at NW. - Note: this does not preclude to use higher layer signaling for Type 1 performance monitoring Option 1. - Note: measurement results refer to “measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report in previous agreement”. Observation 1: The full Set A should be measured if fine-grained accuracy is crucial, while measuring only the predicted beams optimizes resource usage when processing overhead is a concern. ##### Proposal 7: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - The resource set/resources for monitoring in Alt1, either the full Set A can be measured, or only the Top 1 or Top K predicted beams from Set A can be used for efficiency. Proposal 8: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - The resource set/resources for monitoring in Alt2/Alt3, the L1-RSRP difference between predicted and measured beams is computed based on both Set A and Set B resources. Proposal 9: For BAI calculation, considering the following option, - Option 3c: - Where: - is the weight given to predicted beam. - is the indicator determines the accuracy of prediction. - 1 for if the predicted beam is among the Top-M beams with the highest measured L1-RSRP values. for if the predicted beam is among the Top-M beams with the highest measured L1-RSRP values. Proposal 10: For the monitoring Type 1 Option 2 of the UE-side model monitoring, consider that the network will allocate dynamic mapping between Set A and monitoring resources based on UE-side conditions. Proposal 11: For UE-side model monitoring, to trigger the report based on an event, consider Event 1 (predicted beam accuracy of the set of predicted beams being below a threshold accuracy) with statistical results in a given window. |
| CMCC | Proposal 37: Type 2 monitoring of UE-side AI/ML model with gNB indication of the UE’s decision is supported. Proposal 38: The mechanism of Type 2 monitoring, corresponding configuration update and the gNB indication of the UE’s decision, needs further discussion. Proposal 39: Regarding Type 2 monitoring of UE-side AI/ML model, NW can configure a threshold criterion to facilitate UE to make a decision of the model selection / activation/ deactivation / switching/ fallback. Proposal 40: A L1 signaling based data collection for model monitoring can be supported. Proposal 41: Regarding Type 1 performance monitoring Option 1 of UE-side AI/ML model, UE’s reporting can be a single sample based measurement result reporting. The content of the reporting can be the measurement results of the resource set for model monitoring, e.g. L1-RSRP and/or Top 1 RS ID, or Top-K/1 prediction accuracy per sample or per measurement instance. Proposal 42: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, option 2 is preferred for new quantity of beam prediction accuracy. - Option 1: , and the accuracy is . - Option 2: - Where N (N≥1) prediction instance results of inference linked with performance monitoring instance are used for the calculation of accuracy - is the number of the prediction instance results out of Proposal 43: Regarding the calculation of , Option 1 and Option 2 can be combined into one option: - Option 1 (1/Top-K): the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - K >=1, or K is equal to the number of reported predicted beam configured by inference report configuration Proposal 44: Regarding the calculation of , following new option can be considered. - Option 5 (Top-1/1 with margin): the Top-1 beam with largest measured value of the resource set(s) for monitoring and Top-1 predicted beam have a certain QCL relationship or the same QCL source Proposal 45: At least one-to-one linkage between an inference result to one monitoring instance is supported, wherein the inference result of prediction instance that is closest to monitoring instance is linked to monitoring instance. The determination of the closest prediction instance can be up to UE implementation. Proposal 46: With option 2 of new quantity of beam prediction accuracy, if one of linked prediction instance and monitoring instance is after reference resource of CSI-ReportConfig for monitoring, this prediction instance is not calculated in N prediction instances. Proposal 47: The quantification method of the new reporting quantity of beam prediction accuracy needs further discussion. Proposal 48: For measurement overhead reduction, the monitoring set is supported to be a subset of set A. The configuration of monitoring set can be uniformly or non-uniformly down-sampling of set A. Proposal 49: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, when a subset of set A is configured as the resource set for monitoring, Alt3 is supported for the update of definition of beam prediction accuracy - Alt 3: Performance metric is calculated based on the Top K’ predicted beam(s) among the subset of Set A - Note: Top K’ predicted beam(s) among the subset of Set A may or may not be the same as Top K predicted beam(s) reported in the inference report Proposal 50: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, NW can configure an event to trigger the reporting of monitoring KPI. There are two options can be considered： - Option 1: Consecutive N1 instances of KPI or system performance less than a threshold - Option 2: Accumulated N2 instances of KPI or system performance less than a threshold within a time period Proposal 51: Regarding Type 1 performance monitoring Option 2 of UE-side AI/ML model, Alt3 is supported with following modification. - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of Top K predicted beams within monitoring resource set - Note: resources for Set B for monitoring are not precluded. - Note: this is only applicable when the model can predict RSRP Proposal 52: The monitoring set is supported to be a subset of set A. The configuration of monitoring set can be uniformly or non-uniformly down-sampling of set A. Proposal 53: Based on the association of CSI report configuration for inference and CSI report configuration for monitoring, the configuration of monitoring resource set considers following options: - Opt 1: RS ID - Opt 2: CRI - Opt 3: bitmap - Opt 4: pre-configure subset of set A and indicate subset ID as monitoring set |
| Apple | Proposal 3-6: For case 1 and case 2 with UE side model, at least for performance monitoring type 1 option 2, define event driven performance report similar to RLM scheme, where the NW can configure the evaluation window length and threshold to trigger the event. Proposal 3-7: For BM Case 1 and Case 2 with UE side model, at least for performance monitoring type 1 option 2 with periodic or semi-persistent report, separate reporting per prediction instance is indicated on whether the measured best beam index is part of the predicted top-K beams. Proposal 3-8: For BM Case 1 and Case 2 with UE side model, at least for performance monitoring type 1 option 2 with aperiodic report, consider define an evaluation window over which the prediction accuracy is evaluated by the UE. |
|  | Proposal 9: Support using a subset of Set A beams for performance monitoring. Regarding how to use the subset, at least further consider Alt 3 (e.g., use Top K’ predicted beams among the subset) and Alt 2 (e.g., with predefined mapping between the full resources for Set A to the subset of set A) for calculation of the performance metric. Proposal 10: Support at least aperiodic CSI reporting of beam prediction accuracy for performance monitoring. FFS on semi-persistent and periodic. - In the performance monitoring framework, consider monitoring on multiple models associated with one AI/ML functionality. |
| GOOGLE | Proposal 20: For UE-side model monitoring, support the following mechanisms: - UE performs the beam prediction accuracy evaluation periodically - UE counts the number of incorrect beam prediction within a time window - UE transmits a report to the NW to notify the counter is above a threshold - Reuse the SCell BFR based mechanism (SR+MAC CE) to transmit the report Proposal 21: Support the network to configure the association between the DL-RS for monitoring and the Set A RS - If the predicted beam from inference is associated with one of the top K beams for DL-RS for monitoring, UE determines that it is a correct prediction - Otherwise, UE determines that it is an incorrect prediction Proposal 22: Similar to RRM, support performance monitoring relaxation for UE in low mobility and high beam quality state - UE does not need to perform performance monitoring when it is in low mobility and high beam quality state Proposal 23: With regard to dynamic activate/deactivate an inference configuration, support to dynamically activate/deactivate the performance monitoring configuration(s). |
| Intel | - In addition to Alt. 1, also support Alt. 2 and Alt. 3 (and it’s variant Alt 3A) for model monitoring metrics from the agreement from RAN1 #118: - [Already agreed] Alt. 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring. - Alt. 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt. 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Alt. 3A: The RSRP difference information between the predicted RSRP for the predicted Top-1/K beam(s) and the actual measured L1-RSRP for the corresponding beams in the monitoring resource set(s). - For UE-side AI/ML models, for BM-Case 1 and BM-Case 2, for reporting of model monitoring related information for Type 1 monitoring, support: - Option A: Report the measurement results (e.g. L1-RSRP and/or beam information) of a set of beams - Baseline assumption: the set of beams is the set A of beams; FFS: subset of set A or smaller set of beams than set A - Option B: Report the beam prediction accuracy related information within a configured time window, e.g., - Top-K/1 beam prediction accuracy - Extended-Top-K/1 beam prediction accuracy such that the “Extended-Top-K/1 beams” include the Top-K/1 measured beams and any additional beams with L1-RSRP within a specified or configured X dB margin, e.g., X = 1 dB - Option C: Report the RSRP difference information between the measured and predicted beams, e.g., - The L1-RSRP difference between the predicted Top-1/K beam(s) and the measured Top-1/K beam(s) in the set of beams identified for measurements for model monitoring can be reported to the network. - The set of beams can be set A or a subset of set A of beams - If the monitoring resource set is configured to be a subset of set A of beams and does not include any of the predicted Top-1/K beam(s), those inference instances may be skipped from evaluation for the given measurement resource set. - For UE-side AI/ML models, at least for BM-Case 1, for reporting of model monitoring related information for Type 1 Option 2 monitoring based on reporting of beam prediction accuracy related information within a configured time window, consider the following: - BAI = Np or Np/N where Np as the number of the reported inference result(s) linked with performance monitoring instance(s) to (out of N) for which, one (configured to the UE) of the following statement holds: - Option 1 (Top-1/1): the Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam - Option 2 (1/Top-K): the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration - Option 4a: (At least one of) the predicted Top-K beams (is)/are among the set of beams that include the Top-K/1 measured beams and any additional beams of the resource set(s) for monitoring with L1-RSRP within a specified or configured X dB margin relative to the largest measured value of L1-RSRP of the resource set(s) for monitoring; X = 0, 1dB, other values FFS. - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values. - For network-side AI/ML models, consider UE event triggered model monitoring with periodic or aperiodic measurement on set B of beams. - For UE-side AI/ML models, support UE-event-triggered reporting of model monitoring results for Options 1 or 2. - Once triggered, UE requests network for resources to report monitoring results. - Triggering events can be defined based on one or more of the following: - (Event-1) The measured Top-1 or Top K beam(s) of Set A and any additional beams with L1-RSRP values within a specified or configured margin X dB compared to the Top K beams, e.g., X = 1 dB and the predicted Top-1 or Top K beam(s) of Set A are different for more than a threshold number of instances within a defined window. - Set of beams can be same as set A of beams or a subset of Set A of beams - (Event-2) The measured L1-RSRP of one set of beams is lower than a threshold for more than a threshold number of instances within a defined window. - The set of beams includes the Top-1 or Top K predicted beams. - (Event-4) The L1-RSRP difference between the measured Top-1 or Top K beam(s) of Set A monitoring resource set(s) and predicted Top-1 or Top K beam(s) of Set A is larger than a threshold value - Set of beams can be same as set A of beams or a subset of Set A of beams - (Event-5) The L1-RSRP difference between the measured and predicted L1-RSRP values for a given beam or set of beams is larger than a threshold. - Set of beams can be same as set A of beams or a subset of Set A of beams - For model monitoring for BM-Case-2, further consider if the time window for model monitoring should be identical to observation window for model inferencing. - For model monitoring for BM-Cases 1 and 2, both one-to-one and many-to-one linkages between inference result(s) and a Tx occasion of a CSI-RS/SSB resource in the resource set(s) for monitoring (or corresponding CSI reference resource) should be supported. - The case where the monitoring resource set(s) span a subset of set A of beams should be supported. - If the predicted Top-1/K beam(s) is/are not included in the monitoring resource set(s), the corresponding inference result may be skipped from consideration as part of performance evaluation. - Support the UE to report to the NW a metric indicating the fraction of inference results that are being skipped from a (configured or specified) number of inference results. - Such reporting could be periodic, aperiodic (upon request from the gNB), or event-triggered. |
| ZTE | Proposal 29: Support beam prediction accuracy Alt 1 and RSRP prediction accuracy Alt 3 as the primary performance metric for performance monitoring of UE-sided AI/ML model, which are able to justify the quality of reported beam information and reported RSRP, respectively. Proposal 30: Considering the limited payload size and near-real-time latency requirement, support L1 measurement (CSI reporting) for collecting data to enable Type 1 performance monitoring Option 1 (i.e., NW-side performance monitoring). Proposal 31: For Option 2 (UE-assisted performance monitoring), further consider monitoring report initiated by NW and monitoring report initiated by UE. Proposal 32: For monitoring report initiated by NW, the report content can be a calculated performance metric or an indication about whether the calculated performance metric is larger than or equal to a configurable threshold. Proposal 33: Consider UE-initiated monitoring report on the basis of the beam failure recovery mechanisms specified in current specifications. Proposal 34: Support all the combinations on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report. Proposal 35: The time instances associated with the measurement results of the monitoring resource set and the time instances associated with the inference results of the linked inference report should be close enough in the time domain to ensure precise performance monitoring. Proposal 36: Type 2 performance monitoring (i.e., UE-side performance monitoring) can only be supported if the UE is authorized by the NW for functionality or model operations. Proposal 37: For the case that a subset of Set A is configured as the monitoring resource set, further evaluation validation is required before down-selecting the different alternatives given that none of the alternatives have been evaluated before. Proposal 38: For the case that a subset of Set A is configured as the monitoring resource set, consider RSRP difference information or probability information as the performance metric due to the their appealing advantages on RS/measurement overhead reduction. Proposal 39: Model/functionality failure detection should be based on monitoring results of several consecutive times within a predefined monitoring window. Proposal 40: UE reporting based on measurement of Set B can serve as an always-on fallback method to guarantee continuous services quality. |
| Ericsson | For UE-sided model, regarding configuration of set A/B, support - Alternative 3: two CSI-ResourceConfigIds is configured for Set A and Set B separately For UE-sided model inference, enable NW to specify set A beam subset restriction similar to codebook subset restriction (CBSR) that is specified for CSI feedback From RAN1 perspective, for UE-sided model, CSI-ReportConfig can be used for configuring the resources for data collection purpose, with - Two CSI-ResourceConfigIds are configured for Set A and Set B separately - The data collection purpose is indicated via a new field such as “resourcesForDataCollection” that comprises a CSI-ResourceConfigId that indicates either set A or set B (details up to RAN2). - UE performs measurement on both resource sets. - Note: The associated ID may be part of the CSI-ResourceSet in each indicated CSI-ResourceConfigId At least extend the aPeriodic resource configuration to support a resourceSet with 64 NZP CSI-RS resources - FFS on whether to support a resourceSet larger than 64 resources for aPeriodic or periodic measurements For the UE-sided model performance metric, at least support option 2: the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration For the reportConfigType of the UE-sided model performance metric, - Support periodic and semi-persistent reportConfigType if the linked inference report is also periodic or semi-persistent - Support aPeriodic monitoring report only for the scenario where the UE has first been configured with a periodic or semi-persistent monitoring/inference resource config type - to enable the UE to include multiple samples when calculating the performance metric Support joint activation of the monitoring and linked inference report configuration For UE-sided model, to identify the connection between RSs in the resource set(s) for monitoring and Set A beams: - UE can assume the same NW transmission parameters for the resourceIDs (all or a subset) that are present both in the monitoring and set A resourceSet, For UE-assisted performance metric calculation, at least support an event where the UE can early indicate if a functionality is not working properly, for example prediction accuracy is below 50% after collecting N/2 samples |
| Vivo | Proposal 12: For performance monitoring, at least support L1 beam reporting for both UE-side model and NW-side model. Proposal 13: For performance monitoring, for UE-side mode, for BM-case1, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring set and the inference reference point is related to inference report. Proposal 14: For performance monitoring, for UE-side mode, for BM-case1, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set). Proposal 15: For performance monitoring, for UE-side mode, for BM-case1, using monitoring set as granularity for determining monitoring reference point, e.g., the monitoring reference point is the first OFDM symbol of one occasion of the entire monitoring set. Proposal 16: For performance monitoring, for UE-side mode, for BM-case2, support to associate inference reference point to the monitoring reference point, i.e., UE identifies monitoring reference point first, and then identify the associated inference reference point to calculate KPI, where the monitoring reference point is related to monitoring occasion set and the inference reference point is related to inference report. Proposal 17: For performance monitoring, for UE-side mode, for BM-case2, "closest" should be defined as the inference reference point (pertaining to inference report) that is nearest and occurs before the monitoring reference point (pertaining to monitoring set). Proposal 18: For performance monitoring, for UE-side mode, for BM-case2, using occasion set as granularity for determining monitoring reference point and inference reference point, e.g., the monitoring reference point is the first OFDM symbol of the first occasion of the entire monitoring occasion set. Proposal 19: For performance monitoring, for UE-side mode, for BM-case2, each occasion within the monitoring occasion set is uniquely mapped to an occasion in the associated inference occasion set.  FFS：how to associate occasions between two occasion sets, if the number of monitoring occasions in a monitoring occasion set < the number of inference occasions in associated inference occasion set. Proposal 20: For monitoring report, support report overhead reduction for NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1), as well as for NW-side AI model, e.g., report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. Proposal 21: For monitoring report, support to report measured L1-RSRP of indicated beam(s), e.g. current beams indicated by DCI, with measured L1-RSRP of top-k beams for both NW-side performance monitoring of UE-side AI model (i.e. type 1 option 1) and NW-side AI model. Proposal 22: For monitoring metric, support Option 1 (Top-1/1), Option 2 (1/Top-K), Option 3b (Top-K/M) and Option 4 for BAI calculation. Proposal 23:For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case1, support to report the number of correct predictions as BAI result. - X (X≥1) monitoring occasions of a given CSI-ReportConfig with monitoring usage are configured to UE - UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions - BAI field size in monitoring report is equal to - Proposal 24: For the monitoring Type 1 Option 2 of UE-side model monitoring, for BM-Case2, support to report the number of correct predictions as BAI result for each monitoring occasion within a monitoring occasion set. - X (X≥1) monitoring occasion of a given CSI-ReportConfig with monitoring usage are configured to UE - UE reports Y (0≤Y≤X) correct predictions from X associated monitoring occasions - Each BAI field size in the monitoring report is equal to - The number of BAI field is equal to the number of monitoring occasions within a monitoring occasion set - Proposal 25 For performance monitoring, for UE-side model, support following options for further study on what/how to report, - Opt1: report metric - Opt2: report event - Opt3: based on event, report event and metric(s) - Other options cannot be precluded |
| OPPO | - For both BM-Case1 and BM-Case2 with UE-side model, specify a rule for the linkage between prediction instance and monitoring instance as 1-on-1 mapping. - The linked prediction instance is the closest instance prior to the measurement instance in time domain. Proposal 14: For the metric of beam prediction accuracy, support Option 3b (Top-K/M): at least one of the Top-K predicted beams is among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. • K and M are configurable Proposal 15: For the cases when monitoring resource set is different from or a subset of Set A, study and discuss to reuse the QCL rule to calculate beam prediction accuracy. Proposal 16: For Type 1 Option 2 performance monitoring, support the probability of model output (Alt.4) as one performance metric. scheme. Proposal 17: For Type 1 Option 1 and Type 2 performance monitoring, discuss and specify (if necessary) LCM events. Proposal 18: For performance monitoring, study the event-based LCM with performance metric of beam prediction accuracy (Alt.1) and probability (Alt.4). Proposal 19: For performance monitoring with UE-side model, support Type 2 (indication/request/report from UE to gNB) performance monitoring. |
| Fujitsu | - Regarding UE-assisted performance monitoring for UE side model, if the resource set for monitoring is the full set of Set A, then implicit mapping between the RS in the resource set for monitoring and Set A beams could be defined, e.g., one-to-one mapping. If the resource set for monitoring is subset of Set A, then explicit mapping between the RS in the resource set for monitoring and Set A beams should be configured to the UE. - Regarding UE-assisted performance monitoring for UE side model, at least periodic report and semi-persistent report should be supported for the inference report and the monitoring report. RAN1 can further discuss whether aperiodic report should be supported or not. - Regarding UE-assisted performance monitoring for BM Case-1 with UE side model, the time interval between the Set B transmission and the transmission of the resource set for monitoring should be within certain threshold, e.g., X symbols/slots, to guarantee the accuracy of performance metric calculation. For BM Case-2 with UE side model, the time interval between the transmission of the resource set for monitoring and the corresponding prediction instance should be within certain threshold, e.g., X symbols/slots. - Regarding performance monitoring for UE side model, a subset of Set A could be configured for measurement to reduce the overhead. The performance metric, e.g., beam prediction accuracy could be determined based on the subset of Set A. - Regarding performance monitoring of UE side model, the performance information calculation over a time window should be considered. - For performance monitoring of UE side model, besides beam prediction accuracy, the L1-RSRP difference could also be used as performance metric, i.e., the RSRP difference information between the predicted RSRP and measured L1-RSRP. |
| CATT | Proposal 20: For Type 1 performance monitoring of Option 1, support per-sample based reporting with CSI framework as the baseline. Proposal 21: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, the reporting quantity of the CSI report for monitoring is a quantized value of , where is the number of “successful inference instances”, and N denotes the number of performance monitoring instances. - the successful inference instance is defined as - one of the predicted Top-K beams is the Top-1 beam from the resources for monitoring - one of the predicted Top-K beams is within X dB margin of the Top-1 beam from the resources for monitoring Proposal 22: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Type 1 Option 2, further support the following metrics: - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set for monitoring - Alt2-1：The L1-RSRP difference between the largest L1-RSRP measurements of the Top K predicted beam(s) and the largest L1-RSRP measurements of a resource set for monitoring - Alt2-2：The mean of the absolute difference of L1-RSRP between the actual L1-RSRP measurements of the Top K predicted beam(s), and the Top-K largest L1-RSRP measurements of a resource set for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - The corresponding beams can be predicted Top-K beam(s) or Set B Proposal 23：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, study how to configure the resources when the number of beams is larger than the max number of CSI-RS resources can be configured to UE for L1-RSRP measurement. Proposal 24：For Type 1 performance monitoring of Option 2, for resource configuration of Set A, reuse the enhancement in Rel-19 MIMO to transmit the aperiodic resources configured in a RS set within two consecutive slots. - Study whether there is a need to trigger the aperiodic RS transmitted in more than two slots. Proposal 25: For the case that the resource set(s) for monitoring is configured for a subset of Set A, the performance metric calculation based on valid time instance needs to further solve the following aspects: - How to ensure the number of valid time instance used for a performance metric calculation; - How to ensure the accuracy of the performance metric; - How to define the mapping relation between the resource of Set A and the resource of monitoring resource set. Proposal 26: For the case that the resource set(s) for monitoring is configured for a resource set different from Set A, support using predefined mapping between the full resources for Set A and the monitoring resources when calculating the performance metric： - The mapping between Set A and the monitoring set can be explicitly indicated to UE or derived from the QCL relations. Proposal 27: For Type 1 Option 2 of UE-sided model monitoring, on the time domain behavior of the monitoring report and the linked inference report, support the following combinations: - A periodic performance monitoring report linked to a periodic inference report; - A semi-persistent performance monitoring report linked to a periodic or semi-persistent inference report; - An aperiodic performance monitoring report linked to a periodic, semi-persistent, or aperiodic inference report. Proposal 28: For Type 1 Option 2 of UE-sided model monitoring, for the timing related issue, consider one-to-one linkage with the following alternatives: - Alt 1: One-to-one linkage between the inference report instance and the transmission occasion of monitoring resource; - Alt 2: One-to-one linkage between the transmission occasion of measurement resource (Set B) of the inference report and the transmission occasion of monitoring resource. Proposal 29: For Type 1 Option 2 of UE-sided model monitoring, the inference report instance linked with the transmission occasion of the monitoring resource meets the following conditions: - For BM-Case 1, the linked inference report instance is the one whose CSI reference resource being closest to the transmission occasion of the monitoring resource; - For BM-Case 2, the linked inference report instance is the one whose predicted slot intervals include the transmission occasion of the monitoring resource, or a certain slot of the predicted slot intervals of the inference report instance is being closest to the transmission occasion of the monitoring resource; - The report time of the linked inference report instance is no later than the CSI reference resource of the monitoring report. Proposal 30: For Type 1 performance monitoring of Option 2, support event-based reporting, and the specific event can be defined as: - The accumulated instances of performance metric (e.g., Top-K/1 beam prediction accuracy) lower than or larger than the threshold within a time period exceed a configured value. Proposal 31: For Type 1 performance monitoring of Option 2, support event-based reporting based on CSI framework, and the uplink resource carrying the report can be pre-configured or requested by UE. Proposal 32: Support Type 2 performance monitoring for UE-sided model, allowing UE to send the request signaling for performance monitoring. |
| Lenovo | Proposal 17: For NW-side AI/ML model performance monitoring, support Tx beam repetition for the UE to report the best L1-RSRP of a Tx beam among all its Rx beams. Proposal 18: For UE-side AI/ML inference, support aperiodic beam measurement for performance monitoring and dynamic beam updating within the beam set associated with the aperiodic trigger state for beam measurement. - In case that full Set A is used for monitoring, the same CSI-ResourceConfig can be configured for Set A in both inference report configuration and monitoring report configuration. - In case that a subset of Set A is used for monitoring, RS resources of the CSI-ResourceConfig in monitoring report configuration is a subset of RS resources of the CSI-ResourceConfig for Set A in inference report configuration. - In BM-case1, linkage of RS resource for monitoring and inference result can be determined based on the following options - Option 1: The relationship between the linked RS transmission occasion in the resource set(s) for monitoring and CSI reference resource corresponding to the CSI report for inference. - Option 2: The relationship between the linked RS transmission occasion in the resource set(s) for monitoring and the CSI report carrying the inference result. - Option 3: The relationship between a RS transmission occasion in the resource set(s) for monitoring and a transmission occasion of RS resource for inference. - In BM-case2, linkage of RS resource for monitoring and prediction time instance can be determined as that the linked RS transmission occasion has the shortest time interval to the prediction time instance. - Additional support Alt 2 and Alt 3 as the performance metric(s) of AI/ML model monitoring. - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring. - For a monitoring sample, the association of the beam measurement for Set B and the beam measurement for benchmark/reference should be ensured. - Support event triggered beam report for hybrid performance monitoring for UE-side AI/ML model |
| Sony | - : Support the configuration of a subset from Set A for monitoring and further investigate the mapping rules between the subset and the full set. - : Support further investigation into the scenario where the subset does not include the Top K predicted beams during monitoring resource configuration. - : For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), we support Alt 2 and Alt 3, - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - : Support for defining event(s) to trigger reporting for monitoring, - when the measured RSRP differs from the predicted RSRP for the same beam in BM-Case 1. - when the multiple prediction results for the same future time instance vary significantly in BM-Case2. - : For BM-Case2, Further study is needed to reduce measurement overhead based on the RS resource set configured by NW in model monitoring. - : Model failure is determined by the performance metrics that are calculated multiple times over a specific time period. |
| InterDigital | Proposal 22: For UE-assisted performance monitoring, Support TCI-state based activation of RS resource sets for performance monitoring. Proposal 23: For UE-assisted performance monitoring, for the case when Top-K beams are included in the monitoring resource set, down select from one of the following performance metrics: - Option 2 (1/Top-K): The Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - Option 3 (Top-K/M): The Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. - Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largest measured value of L1-RSRP of the resource set(s) for monitoring Proposal 24: For UE-assisted performance monitoring, for the case when Top-K beams are not included in the monitoring resource set, support a performance metric based on difference between measured RSRPs and predicted RSRPs. Proposal 25: For UE-assisted performance monitoring, support the following options for activation of performance monitoring at UE: - Based on the reception of RSs associated to Set A beams. - Based on a periodic timer which starts/stops performance monitoring procedure based on gNB signaling or model performance. Proposal 26: Support a UE based indication of AIML model validity. |
| Panasonic | Proposal 11: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support to define a beam accuracy indicator (BAI) as a metric of beam prediction accuracy Proposal 12: For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, support to define a monitoring window for BM-Case1 and BM-Case2 such that BAI can be calculated within the monitoring window - FFS: Details on the monitoring window Proposal 13: Group-based beam reporting can be enhanced to support performance monitoring for NW-sided model. |
| Nokia | Proposal 5: For BM-Case1 and BM-Case2, considering NW-sided performance monitoring for beam prediction related CSI reporting, conclude the following, - A different CSI report can be used to support NW-sided performance monitoring. - RS resource set: The NW can use a different CSI report to get beam measurements/reporting for a monitoring RS resource set (as preferred by the NW) within the legacy CSI reporting framework. - L1-RSRP and RS index of Top-M beams of monitoring RS set is supported as the content of the report - Note: Spec impact may only expect if M is defined separately for monitoring purpose. Proposal 6: For BM-Case1 and BM-Case2, considering Alt. 1: Top 1 or Top K beam prediction accuracy, - Accuracy metric is calculated using multiple monitoring instances (monitoring window) - Configure the monitoring window in the report configuration for monitoring - Accuracy metric, (N-E)/N, shall be determined based on, - N is the number of valid performance monitoring instances within the monitoring window. - A valid performance monitoring instance shall carry Top 1 or Top K predicted beams in the monitoring RS resource set. - E is the number of errors (E) within the number of valid performance monitoring instances. - Best L measured beams of the valid performance monitoring instance are considered in the accuracy determination. - For L >= K, Error may be counted when Top 1 or Top K predicted beams are not among the best L measured beams of the valid performance monitoring instance. - FFS: L for Top-1 and Top-K cases - For L < K, Error may be counted when best L measured beams of the valid performance monitoring instance are not among the Top 1 or Top K predicted beams. - Note: L = 1 and K = 1 provides the Top-1 beam accuracy. - To derive an error, UE shall link an inference instance (e.g., beam prediction report) to a performance monitoring instance (measurement instance of monitoring RS resource set) - FFS: how to define the linking - Support all the combinations on time domain behaviour of the reportConfigType for monitoring. |
| Ruijie | Proposal 4: At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support Option 2 for the configuration for monitoring: - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. |
| Samsung | Proposal 18. For the monitoring Type 1 Option 2 of UE-side model monitoring, introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring - The value of BAI Np (0≤Np≤N) is the number of the reported inference result(s) linked with performance monitoring instance(s) for which the following statement holds - Option 1 (Top-M/K): at least one of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams, where K>M and M, K are RRC configurable - Option 2 (Top-K/M): the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring, where K≤M and M, K are RRC configurable - The size of CSI field associated with the BAI is - N is the number of performance monitoring instance(s) configured by RRC Proposal 19. For the monitoring Type 1 Option 2 of UE-side model monitoring, beam prediction accuracy is generated based on inference result(s) subjected to the inference report(s) with the CSI-ReportConfigID and the linked performance monitoring occasion(s) - In time domain, at least one-to-one linkage is supported between an inference result to one transmission occasion of the CSI-RS/SSB resource in the resource set(s) for monitoring, wherein - for BM-Case 1, the linked transmission occasion to an inference result is a transmission occasion of the CSI-RS/SSB resource which is closest to the CSI reference resource corresponding to the CSI report which carries the inference result - for BM-Case 2, the linked transmission occasion to an inference result is a transmission occasion of the CSI-RS/SSB resource which is closest to the predicted time instance associated with the inference result |
| Transsion | Proposal 8:For performance monitoring of NW-side model, configuration of Set B and/or Set A and the reporting of measurement results of Set B and/or Set A need to be specified. Proposal 9: Regarding Option 2 of UE-sided AI/ML model, support to configure a window for calculating the metric and the following alternatives can be considered to define a window: - Alt-1: The window for calculating the metric is defined based on the uplink slot for the report - Alt-2: The window for calculating the metric is defined based on the CSI reference resource corresponding to the report Proposal 10: Regarding the resource set/resources for monitoring for Option 2 of UE-sided AI/ML model, support to use full set of Set A for measurement. Proposal 11: For Option 2 of Type 1 performance monitoring, the following alternatives can be considered to report the metric: - Alt-1: Quantization of performance metric; - Alt-2: The statistic values of the performance metric; - Alt-3: A specific event. |
| ETRI | Proposal 1: For UE-assisted performance monitoring, CSI report information that includes performance metrics can be included in the report configuration for either performance monitoring or inference. Proposal 2: For UE-assisted performance monitoring, performance metrics can be reported by being measured over a monitoring window. Proposal 3: For UE-assisted performance monitoring, information for the monitoring window can be included in the CSI Report configuration for performance monitoring. Proposal 4: For UE-assisted performance monitoring, the beam prediction accuracy can be expressed as Ns/N, where Ns is the number of times the prediction was successful. Proposal 5: For UE-assisted performance monitoring, support the L1-RSRP difference between predicted and measured RSRP (Alt 3) as a performance metric. Proposal 6: For UE-assisted performance monitoring, support the inclusion of a linkage ID within the CSI-ReportConfig IE. Proposal 7: For UE-assisted performance monitoring, support to report linkage ID within the CSI report to distinguish the performance metrics. Proposal 8: For performance monitoring, the UE can provide information to assist in selecting the subset of Set A for monitoring. Proposal 9: For performance monitoring, the UE can identify the connection between resources in monitoring set and Set A by using QCL information. Proposal 10: For UE-assisted performance monitoring, the monitoring event can be triggered when the counter reaches a specific number - The counter is incremented if the calculated performance metrics fail to meet predefined performance criteria Proposal 11: For Type 2 performance monitoring, the UE can transmit information regarding the activation/deactivation, switching, or fallback of the functionality. Proposal 12: For UE-side performance monitoring, support the use of Monitoring IDs to differentiate monitoring processes for multiple AI/ML models. |
| CAICT | Proposal 1: For option 2 of type 1 UE-sided model monitoring, new indicator i.e. beam accuracy indicator (BAI), could be introduced. Proposal 2: The definition of BAI should be the ratio of correct prediction number to total monitoring number during a time window, where correct prediction could be the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams. Proposal 3: The time window for BAI should be equal to the monitoring result report period by default Proposal 4: No need to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report. Proposal 5: Both periodical or event triggered monitoring report could be considered and periodical monitoring report should be considered as baseline. Proposal 6: Event-1 could be used for the triggered event for Option 1 and/or Option 2. |
| DOCOMO | Proposal 11: Not support type 2 performance monitoring. Proposal 12: Which inference results and measurements are used for performance metric calculation should be specified so that NW and UE can have the common interpretation on reported performance metrics. Proposal 13: Each metric can be derived per measurement occasion for monitoring, where the inference result whose prediction time instance is the closest to each measurement occasion should be used for each metric calculation. Proposal 14: Support reporting of statistical performance metric value over multiple samples in UE-assisted performance monitoring. Proposal 15: Restrict the same time domain behavior of inference result reporting and performance monitoring reporting with the following restriction to alleviate the specification efforts. - When inference result reporting and performance monitoring reporting are both periodic, periodicity of performance monitoring reporting and measurement occasions for performance monitoring should be X times as large as one of inference reporting - when inference reporting and performance monitoring reporting are both aperiodic/semi-persistent, inference reporting and performance monitoring reporting should be triggered jointly via one DCI/MAC CE Proposal 16: Support Alt 3 as the performance metrics to check the accuracy of predicted RSRP value in the actual field. Proposal 17: No need to support Alt 2 as the performance metrics, as Alt 1 is already supported for the same purpose. Proposal 18: Specify one of the following solutions toward the issue that performance metrics cannot be calculated in performance monitoring based on the subset of Set A. - performance metric is calculated only when at least one of top-K predicted beam is included in resources for performance monitoring - measured resource for performance monitoring is determined based on the inference results (e.g., predicted top-K beams are measured for performance monitoring) - performance metric is defined so that it is applicable to performance monitoring based on the subset of Set A Proposal 19: NW should obtain the ratio of the number of accurate beam predictions to the number of valid beam predictions via Alt1 performance metric, where the valid beam prediction is the beam prediction whose accuracy can be checked. Proposal 20: Performance metric from Alt1 can be the ratio of the number of accurate beam predictions to the number of valid beam predictions. Proposal 21: Support the following triggering mechanism of UE-assisted performance monitoring. - Based on NW configuration/indication - When performance metric satisfies some conditions, such as larger/lower than thresholds |
| Sharp | - For BM-Case 1 and BM-Case 2 with a UE-sided AI/ML model, for option 2 (UE-assisted performance monitoring), the beam prediction accuracy-based metric is defined as follows: - , where - N is the number of configured performance monitoring occasions by network, - represents the count of accurately predicted beams on monitoring occasion i, where on a given monitoring occasion i, if a beam of the predicted Top 1/K beams is actually the Top 1 measured beam or one of the Top-K measured beams based on the measurements, the beam is counted in ; otherwise, the beam is not counted in . - For UE-side AI/ML model, additional contents beyond the measurement results is unnecessary for option 1 of Type 1 performance monitoring. - For UE-side AI/ML model, in option 2 of Type 1 performance monitoring, support event-driven performance metric reporting. - Event: a calculated beam prediction accuracy is below a threshold accuracy. - For BM-Case 1 and BM-Case 2 with a UE-sided AI/ML model, for option 2 (UE-assisted performance monitoring), a new quantity (i.e., beam accuracy indictor, BAI) for beam prediction accuracy is introduced in the CSI report configuration for monitoring and the new quantity BAI is defined as , where - N is the number of configured performance monitoring occasions by network, - represents a number of performance monitoring occasions out of the N performance monitoring occasions, that satisfies the following conditions - Option 3a (Top-K/M): the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= 1, FFS other values - M is configured - K<=M - Option 3b (Top-K/M): at least one of the Top-K predicted beams is among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - M is configured - A performance monitoring occasion without a linked inference result is not counted in the . - For UE-side AI/ML model, additional contents beyond the measurement results is unnecessary for option 1 of Type 1 performance monitoring. - For UE-side AI/ML model, in option 2 of Type 1 performance monitoring, support event-driven performance metric reporting. - Event: the value of BAI is below a threshold accuracy. |
| Qualcomm | Proposal 12 For beam prediction for UE-side AI/ML models, performance monitoring metrics should be defined for scenarios in which the performance monitoring set for each performance monitoring instance is a subset of the prediction set, i.e., Set A. • Note: if metrics are defined for such scenarios, they would naturally be applicable to the scenarios in which the performance monitoring set is equal to Set A, but not the other way round. #### Proposal 12 For UE-side beam prediction, with regards to Alt 1 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, define N as the total number of performance monitoring instances, and define as the number of performance monitoring instances (out of ) for which the following statement holds: - Top-K beam prediction accuracy without L1-RSRP margin: best measured beam ID from performance monitoring set is among the Top-K predicted beams, for a given performance monitoring instance. - Top-K beam prediction accuracy with L1-RSRP margin: The highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set is within dB of measured L1-RSRP of best measured beam ID from performance monitoring set, for a given performance monitoring instance. - can be configured by gNB. Define the ratio as Top-K beam prediction accuracy (without or with margin based on which of the above statements holds). - Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance. #### Proposal 13 For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, at least for Option 2 (UE-assisted performance monitoring), support Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring. #### Proposal 14 For UE-side beam prediction, with regards to Alt 2 of performance monitoring alternatives for UE-assisted performance monitoring, in which the performance monitoring set may be a subset of Set A or full Set A for each performance monitoring instance, support the following metric for performance monitoring: - The L1-RSRP difference between measured L1-RSRP of Top-1 measured beam ID from the performance monitoring set and highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set, per performance monitoring instance. Note: “performance monitoring set” is the set of RSs that are to be measured for performance monitoring, per performance monitoring instance. #### Proposal 15 For UE-side beam prediction, and for UE-assisted performance monitoring, study details of performance monitoring reports (contents, frequency of report, carrier), at least as a function of performance monitoring metrics. #### Proposal 16 For BM-Case1 and BM-Case2 with a UE-side AI/ML model, and for Type 1 performance monitoring, study the following events for event-triggered performance monitoring report for both Option 1 and Option 2. - Event 1: c1 predicted beam is NOT within 𝑋 dB of Top-1 measured L1-RSRP, from performance monitoring set. Note: FFS on triggering condition: whether based on per-instance event occurrence or based on number of event occurrences within a window. |
| Indian Institute of Tech (M), IIT Kanpur | Proposal 3: For performance monitoring of UE-sided models, UE can report RSRP difference between true and predicted value for Top 1 or Top K beams along with confidence information of each RSRP difference. Proposal 4: For performance monitoring of UE-sided models, UE can report Top K beams with confidence information/probability information. Proposal 5: To address the report overhead issue, the report quantities, a) RSRP difference of true and predicted Top K beams with confidence information b) Top K beams with probability information can be formulated as Events and can be triggered by UE to report the model performance. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 1: For monitoring UE-sided models, support 2-phase monitoring with varying frequencies and reporting detail. Proposal 2: For monitoring, support monitoring of only a subset of Set A beams. Proposal 3: Consider indication-based and event-based switching into a validation phase. Events may be defined based on agreed performance metrics. Proposal 4: For the activation of monitoring, support Event-1, Event-4 and Event-5. Proposal 7: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting. Proposal 8: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models. Proposal 9: Study the support for radio link monitoring and link recovery procedures considering spatial and temporal beam prediction at the UE-side. Proposal 9: Explore the possibility of using the resources configured for radio link monitoring for model monitoring purposes with respect to CSI beam reporting. Proposal 10: Support a model monitoring configuration that allows for collecting data for model training and monitoring of inactive models. |
| ITL | Proposal 22: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, for Type 1 performance monitoring Option 1 (NW-side performance monitoring), L1 signalling can be used to send the measurement results to NW for the calculation of performance metrics at NW. Proposal 23: For UE-side model monitoring of Option 2 of Type 1, it is supported to use Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring. Proposal 24: It is proposed to support event-triggered UE reporting for UE-sided Type 1 performance monitoring. Proposal 25: For Type 2 performance monitoring of UE-side model, it is proposed to define new report quantity including no reporting (e.g. ‘none’) and the monitoring decision (e.g. activation, deactivation or fallback) in CSI-ReportConfig. |
| KDDI | Proposal 4: Alt 1 and Alt 2 require measuring all beams in Set A to obtain information about the correct Top 1/Top K beams. Proposal 5: Alt 3 enables performance monitoring by measuring only the beams of the monitoring resources predicted by AI/ML based on Set B. Proposal 6: If the monitoring resources for Alt 3 are set to the Top 1/Top K beams, monitoring is possible based on the information obtained during the inference operation. Proposal 7: For Alt 4, the use of probability information in monitoring should be discussed and the difference from Alt 1 should be clarified. |
| Xiaomi | Proposal 6-1: Both of the following two Benchmark/reference for performance comparison should be supported. Proposal 6-2: In addition to beam prediction accuracy, Support Alt 3, i.e., the L1-RSRP difference information between the measured RSRP and predicted RSRP as performance metric for performance monitoring. Proposal 6-3: For Option 2 performance monitoring (UE-assisted performance monitoring), Support to define a window for calculation of performance metrics. Proposal 6-4: For Option 2 performance monitoring (UE-assisted performance monitoring), Support to report the ratio to indicate the Top 1 or Top K beam prediction accuracy. Proposal 6-5: For performance monitoring of UE-side AI/ML model, support to link the inference results and the transmission occasion of the resource set for monitoring which is ‘closest’ to the inference results. Proposal 6-6: For performance monitoring of UE-side AI/ML model, identify the connection between RSs in the resource set for monitoring and Set A beams by CRI/SSBRI. Proposal 6-8: For performance monitoring of UE-side AI/ML model, support full set of Set A for performance monitoring as baseline. Proposal 6-9: For Type 1 performance monitoring of UE-side AI/ML model, both NW-side initiated and UE-side initiated performance monitoring can be supported. NW-side initiated can be based on measurement/report configuration via RRC and UE-side initiated can be based on SR and UL MAC CE with the preferred resource configuration of set B and set A. Proposal 6-10: If the performance metric is the L1-RSRP difference between the measured RSRP and predicted RSRP, not use the L1-RSRP of beams in set B for metric calculation. Proposal 6-11: For UE-side AI/ML model, support Type 2 performance monitoring and it can be initiated by UE. Proposal 6-12: For UE-side AI/ML model with Type 2 performance monitoring, it is better to indicate UE’s decision to NW for consistency of the NW-side additional condition for the new applied UE-side model. Proposal 6-13: For UE-side AI/ML model with Type 2 performance monitoring, configure an event with a threshold to assist UE to make the decision. Proposal 6-14: For performance monitoring of network-side AI/ML model, support to report measurement results of set B and set A separately. Set B can be reported based on beam report, and set A can be reported by MAC CE or RRC with multiple samples. Proposal 6-15: For performance monitoring for network-side AI/ML model, support an event-triggered report if the indicated TCI state is different from the best beams obtained by measurements. |
| Kyocera | Proposal 17: For the performance monitoring of a UE side AI/ML model, for the content of the report of type 1 option 1 monitoring, support the following as a starting point: - Model related outputs: - Beam information of the top-K predicted beams. - Predicted RSRP if supported by the AI/ML model. - Probability information of the predicted beams. - Confidence information of the of the RSRP if supported by the AI/ML model. - Performance related measurements from the resource set configured for monitoring: - CRI/SSBRI - L1-RSRP of the top-K beams Proposal 18: For type 1, option 2, performance monitoring of a UE side AI/ML model, regarding the FFS of: - How to report- assess the impact of adopting the following methods: - Sample based performance metric reporting in order to provide the NW with frequent updates about the AI/ML model performance. - Statistical based performance metric reporting in order to reduce the UL overhead on the expense of less information being available at the gNB. - What to report- Support using the following metrics as a starting point: - The error between the predicted RSRP and the measured L1-RSRP, where the predicted RSRP is based on AI/ML output. - The beam predication accuracy of the top-K beams, where the beam prediction accuracy is the percentage of the beam determined from RS measurements for performance monitoring to be one of the top-K predicted beams by the AI/ML model. Proposal 19 For BM Case 1 and Case 2 in a UE side AI/ML model, deprioritize Alt 4 as a measure for Type 1 option 2 performance monitoring. Proposal 20 Regarding the size of the performance monitoring set of a UE-side AI/ML model, support that the the size of the performance monitoring set is less than or equal to the size of Set A and defined by the network. Proposal 21 For Type 1 option 2 performance monitoring of a UE-side AI/ML model, support using a mapping from beam information to performance monitoring resources. This mapping information can be provided to the UE though SIB or any other gNB to UE dedicated message. Proposal 22 For UE-side AI/ML performance monitoring, RAN1 should further study the following: • An event is defined as the scenario when the performance metric falls above (or below) a certain threshold. • The concept of triggering a report based on specific events is not applicable to option 1. It is, however, relevant to option 2, where the UE calculates performance metrics and can detect events. Proposal 23 For UE side AI/ML performance monitoring, type 2 monitoring has no specification impact, and it should be left up to the UE vendor implementation. Proposal 24 For a NW side AI/ML model performance monitoring, there is no specification impact is needed, and it can be left up to the NW vendor implementation. |
| LGE | Proposal #19: For Option 2 (UE-assisted performance monitoring) of Type 1 performance monitoring, support at least full set A for the dedicated resource set for monitoring. Proposal #20: When an inference report is activated/triggered, and if the report is linked to a monitoring report, the monitoring report should be activated/triggered within a certain time limit or a buffering window for the inference results should be specified. Proposal #21: Regarding supported combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for performance monitoring report, support the following combination: - Periodic report of inference + P/SP/AP report of performance monitoring - Semi-persistent report of inference + SP/AP report of performance monitoring - Aperiodic report of inference + AP report of performance monitoring Proposal #22: Support event-triggered UE reporting for UE-sided AI/ML performance monitoring. - Further consider UE report via UCI or SR to request change of Set A configuration, fallback to legacy beam report, holding the report for a while, etc. |
| NVIDIA | Proposal 5: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model configuration, model activation/deactivation, model recovery/termination, and model selection. Proposal 6: For AI/ML based beam prediction in spatial/time domain, introduce specification support for assistance signalling and procedure for model performance monitoring and model update/tuning. |
| NEC | - For the configuration of monitoring Type 1 Option 2 of UE-side model monitoring, further study: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference. - Clarify that multiple reportConfigIDs of model inference reports can be associated with a single reportConfigID of performance monitoring. The reportConfigType configurations of the two report settings for performance monitoring and model inference should be restricted: AP monitoring report can be based on P/SP/AP inference report, SP monitoring report can be based on P/SP inference report and periodic monitoring report can only be based on periodic inference report - Study monitoring behaviors corresponding to the monitoring report when its associated inference report is stopped or dropped during monitoring. - For both BM-Case1 and BM-Case2, performance monitoring is only needed when the legacy procedures like BFD, RLM detected a physical layer problem. - Support event-trigger/UE-initiated performance monitoring of current AI/ML model if beam failure instance indication or out-of-sync indication is received. - For BM-Case2, when prediction accuracy is used as performance monitoring metric and a model inference result includes predictions for F future time instances, if the first time instance prediction is inaccurate, the whole model inference result is inaccurate. - For RSRP difference as the performance metric, the same Rx for predicted RSRP (or Top 1 predicted beam) and measured RSRP (or Top 1 measured beam) needs to be assured. - At least for regression-based model (or the model can predict RSRP), support Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring. - When the model can output probability, support Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam. - Support UE to report probability(ies) of predicted Top K beam(s) based on some pre-defined interval or threshold/criterion. Otherwise, if the probability is not reported, the probability should be used as one of the determining factors when reporting predicted beams at least for classification model. - For Alt 1 and Alt 3, the performance metric can be calculated either per sample or per set of samples (or monitoring window). - To support performance monitoring based on a set of samples the NW may need be informed of the specific samples included in the metric calculation. - The performance metric for a set of samples ( monitoring instances) can be , where is the prediction accuracy at the sample ( monitoring instance); is the total number of monitoring instances. And can be determined as follows - Alt 1: If the AI/ML model predicts all the beams correctly at the monitoring instance, then can be considered as 1, otherwise as 0. - Alt 2: If the AI/ML model predicts lesser number of beams compared to total predicted beams correctly at the monitoring instance, then - can be ; where is the total number of correct predicted beams at the monitoring instance, is the total number of predicted beams at the monitoring instance; is the total number of monitoring resources. - can be 1 if the value above a threshold value, otherwise 0. - For monitoring resources for Type 1 Option 2 of UE-side model monitoring, full set of Set A as baseline and further study subset of Set A. - UE can measure a subset of monitoring resource set which consists resources configured with the same qcl-info as the predicted Top K beams. - For performance monitoring of UE-side model, support to assess performances for multiple Set Bs to balance beam measurement overhead and performance. - Study simultaneous performance monitoring for multiple candidate models, including how to inform the NW the inactive candidate models and how to request resources and configurations for performance monitoring of the inactive candidate models. - Study the calculation of performance metric when a subset of Set A is configured as monitoring RS, if different performance matrices will be supported for full Set A as monitoring RS and a subset of Set A as monitoring RS. |
| MTK | Proposal 6: For performance monitoring of the UE-sided model, for Type1 Option1, further study how to facilitate NW to correctly monitor the performance of a UE-sided model under the assumption that UE is not always reporting all the predicted best beams according to the model output. Proposal 7: For performance monitoring of the UE-sided model, support both full set of Set A and a subset of Set A to be the measurement set for monitoring. Proposal 8: For performance monitoring of the UE-sided model, at least for Type1 Option2, when the measurement set is a subset of Set A, the performance metric for Alt1 can be defined as the heuristic Top-1 and Top-K beam prediction accuracy, by comparing the beams with the best probabilities among the subset of Set A and the beams with the best measurement among the subset of Set A. Proposal 9: For performance monitoring of the UE-sided model, at least for Type1 Option2, support beam prediction ranking accuracy as one alternative, which compares the ranking sequence of K beams with the best probabilities among the subset of Set A and the best K beams with the best measurement among the subset of Set A. Proposal 10: For performance monitoring of the UE-sided model, Alt. 4 cannot be used directly as the performance metrics. Proposal 11: For performance monitoring configuration of the UE-sided model, at least for Type1 Option2, support Option1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference. Proposal 12: For performance monitoring configuration of the UE-sided model, at least for Type1 Option2, when the resource set(s) for monitoring and report configuration for monitoring are configured within CSI report configuration used for inference, consider using report sub-configuration under the inference report configuration for each monitoring configuration. Proposal 13: For Type 1, option2, UE-assisted performance monitoring, consider the following conditions to trigger a UE report: - A report triggered/activated by NW - UE initiated performance monitoring reporting when an event happens (FFS on event) Proposal 14: For Type 1, option2, UE-assisted performance monitoring, consider the following two alternatives of the reported content: - UE reports the performance metric(s) - UE reports the performance metric(s) and LCM decisions (model switching/activating/deactivating request) Proposal 15: For Type 1, option2, UE-assisted performance monitoring, to facilitate UE to detect a monitoring event for reporting, considering NW signaling to UE the following aspects to define an event, - The performance metrics monitored for the event - The threshold of the performance metrics for determining the occurrence of the event - The number of samples of the occurrence instances required for determining the occurrence of the event, where the occurrence instances are the monitoring samples that the monitored metrics falls below a threshold - The number of monitoring samples required for determining the occurrence of the event - The frequency of each monitoring samples Proposal 17: Set A should always be configured with physical resources (i.e., have corresponding resource ID/resource set ID, ResourceConfig ID), for the following reasons: - Set A is required for monitoring/data collection, and can be configured with longer periodicity - Set A is required for legacy UE Proposal 18: For the design of csi-ReportConfig for UE-sided model, Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value. |
| KT | Proposal 7. RAN1 support Option 2 for configuration for monitoring. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring Proposal 5. For Alt 1 of UE-assisted performance monitoring, support configuring the full set of Set A as the resource set for monitoring. Proposal 6. Support Alt 3 for Type 1 Option 2 of UE-sided model monitoring: - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring. Proposal 7. For option 2 of performance monitoring type 1, UE consider Set A as RSs for monitoring if there is no resource set(s) for monitoring in CSI report configuration for monitoring. Proposal 8. For option 2 of performance monitoring type 1, support configuring a dedicated resource set for monitoring consisting of K CSI-RS resources. - Where K is equal to the number of reported Top K beam. - The reported K beams are dynamically mapped to the CSI-RSs for monitoring. |
| Meta | Proposal 5: For UE side model with monitoring Type-1, Option-1, measurement report with L1-RSRP and RS index is sufficient and other contents may not be needed. Consider configuration of monitoring resource set and time window or filtering based reporting for additional reliability of performance monitoring. Proposal 6: For UE sided model with monitoring Type-1, Option-2, beam prediction accuracy and RSRP difference reporting may be supported as configurable reporting contents but confidence information and/or probability information may be optional based on UE model capabilities. Proposal 7: For reporting RSRP difference information, the baseline can be for the current indicated beam where UE can measure and predict the RSRP. For Top-1/K beams which are predicted but not in set B, further discuss how to evaluate RSRP difference Proposal 8: For UE sided model with monitoring Type-1, Option-2, consider supporting UE report of fallback to non-AI/ML methods Proposal 9: Support Type 2 performance monitoring of UE sided models with reporting for AI/ML model switching/activation and indication of non-AI/ML fallback Proposal 10: For UE sided model with Type-1 performance monitoring, support event triggered monitoring where the gNB configures one or more events to the UE Proposal 11: For UE sided model with Type-1 performance monitoring, support at least Events 1,2 and 3 Proposal 12: For UE sided model with Type-2 performance monitoring, support event driven indication of fallback or model switching/activation. Proposal 13: Consider UE assisted performance monitoring for NW sided models. |
| ASUSTeK | Proposal 1: For a linkage between CSI-ReportConfig for inference and CSI-ReportConfig for monitoring, reportConfigType for infernece report and the reportConfigType for monitoring report should be configured as same Type (as a baseline) - FFS: Whether to allow to a subset of combinations of different Type Observation 1: Due to linkage between monitoring and inference, if there is problem when generating an inference report resulting in no inference report, how UE generates a linked monitoring report needs to be specified. Proposal 2: RAN1 discuss the issue that when there is no inference report transmitted before the CSI reference timing associated with a monitoring report, how UE generates the monitoring report. Observation 2: In TS 38.331, CSI-ResourceConfig comprises a BWP ID to indicate RS set configured by CSI-ResourceConfig belongs to which BWP, however, it’s not clear whether to support set A and set B configured in different BWP. |
| Sharp | - For BM case 1 with a UE-sided AI/ML model, for option 2 (UE-assisted performance monitoring), one performance monitoring occasion is linked to one inference result, where - the transmission occasion of set B where the inference result is derived is the closest in time to the performance monitoring occasion, and - the processing time for performance reporting needs to be ensured for selecting the linked inference results. - For BM case 2 with a UE-sided AI/ML model, for option 2 (UE-assisted performance monitoring), one performance monitoring occasion is linked to one inference result, where - the future time interval associated with the inference result overlaps in time with the performance monitoring occasion. - a performance monitoring occasion is considered to have no linked inference result if there is no time interval associated with inference results overlap in time with the performance monitoring. |
| HONOR | - For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), at least support Alt 2 in the following: - Alt 1: the definition of Top-K(K≥1) beam prediction accuracy is the percentage of “the Top-1 predicted beam is one of the top K measured beams” in all evaluation occasions. - Alt 2: the definition of Top-K(K) beam prediction accuracy is the percentage of “the Top-1 measured beam is one of the Top-K predicted beams” in all evaluation occasions. - The resource set for monitoring is a full set or a subset of Set A, and the Top-K measured beams and Top-K predicted beams for monitoring are defined as follows: - The top K measured beams are the beams with K highest measured RSRPs in the resource set for monitoring. - The top K predicted beams are the beams with the K highest predicted RSRPs or predicted probability in the resource set for monitoring. - For the monitoring Type 1 Option 2 of UE-side model monitoring, support UE to pre-evaluate the quality of predicted beams and report the pre-evaluated results, based on which gNB can decide whether to trigger a performance monitoring using a full Set A. |
| meta | - For UE side model with monitoring Type-1, Option-1, measurement report with L1-RSRP and RS index is sufficient and other contents may not be needed. Consider configuration of monitoring resource set and time window or filtering based reporting for additional reliability of performance monitoring. - For UE sided model with monitoring Type-1, Option-2, Alt-2/3 (RSRP difference reporting) should be supported as configurable reporting contents but confidence information and/or probability information may be optional based on UE model capabilities. - For reporting RSRP difference information, the baseline can be for the current indicated beam where UE can measure and predict the RSRP. For Top-1/K beams which are predicted but not in set B, further discuss how to evaluate RSRP difference - For UE sided model with monitoring Type-1, Option-2, consider supporting UE report of fallback to non-AI/ML methods - Support Type 2 performance monitoring of UE sided models with reporting for AI/ML model switching/activation and indication of non-AI/ML fallback - For UE sided model with Type-1 performance monitoring, support event triggered monitoring where the gNB configures one or more events to the UE - For UE sided model with Type-1 performance monitoring, support at least Events 1,2 and 3 - For UE sided model with Type-2 performance monitoring, support event driven indication of fallback or model switching/activation. |
| RUIJIE | Proposal 1: Regarding how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams, the ID of an inference report configuration can be potentially used by UE to identify the connection or the relationship between RSs in the resource set(s) for monitoring and Set A beams. Proposal 2: Support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report. |

### Issue #1.1 Metric and Report content for Type 1 option 2 performance monitoring

Proposal from #119

For BM-Case1 and at least for BM-Case2 with single time instance in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring,

- BAI = , the accuracy corresponds to the BAI is .

  - FFS: the size of CSI field associated with the BAI is

  - Where  as the number of the reported inference result(s) linked with performance monitoring instance(s) to (out of ) for which, FFS the following statement holds with potentially downselection (at least for the case that full set of Set A can be measured)

    - Option 1 (Top-1/1): the Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam

    - Option 2 (1/Top-K): the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams

      - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration

    - Option 3 (Top-K/M): the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

      - K= 1, FFS other values

      - M is configured

      - K<=M

    - Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largested measured value of L1-RSRP of the resource set(s) for monitoring

      - X=0, 1, FFS on whether to support other values

      - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values

    - Further study on whether to update some definition of the description of above options.

      - Note: the above options at least with the assuming that Top-K predicted beam are included in monitoring set(s).

    - Other options are not precluded.

    - Note: The above options are at least for discussion purpose, the specification

  - Where N is the number of the reported inference result(s) linked with performance monitoring instance(s)

    - FFS on how to link an inference result (e.g., in the inference report) to a performance monitoring instance

    - FFS on whether to exclude some linked reported inference result(s)/monitoring instance(s)

##### (FL0) Proposal 1.1 (metric)

At least for BM-Case1, in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring,

- Note: The resource set(s) for monitoring may or may not be the full set of Set A

- beam accuracy indicator (BAI) is  Np, (0 ≤ Np ≤ N)

  - N is the number of performance monitoring instance(s), configured by gNB in configuration of CSI report for monitoring.

    - N is configured in report configuration for monitoring.

      - Both N=1 and N>1 are supported, FFS values

    - the size of CSI field associated with the BAI is .

  - Where  as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection:

    - Option 2a (Top-M/K): at least one of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams

      - K= 1, [2, 4, 5] is configured

      - M=1, [2, 4] is configured

      - K>=M

      - FFS: K>1

      - FFS: M>1

    - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

      - K= 1, [2, 4, 5] is configured

      - M=1, [2, 4, 8] is configured

      - K<=M

      - FFS: K>1

    - FFS: whether/how Top-M/K or Top M/M with margin is allowed

| Companies | Y/N | Comments |
| --- | --- | --- |
| FL |  | To simplify the defination of beam prediction accuracy, down to 2 options. Please share whether to support M>1, M>1 in each options. Regarding on the payload for BAI, there is no motivation to define quantized % for BAI. N does not expected to be too large, if N is large, then it requires quite long time to obtain one report. e.g., P =80ms, N = 32, => 2650ms per report. Maybe no need to use L1 signaling. => 5 bits for Alt 1 for current design. If N is smaller, the payload can be easily reduced |
| Huawei, HiSilicon | Y | In general OK with the direction. Some comments in below: 1) For Np, it should be restricted to be valid monitoring samples from the overall N monitoring samples. The invalid cases includes, e.g., i) mismatched prediction result and monitoring resource set in case the monitoring resource set is subset of Set A (as will be discussed in a later proposal), ii) failed CSI processing of the monitoring CSI report or its linked inference CSI report, e.g., due to unsatisfied CPU occupancy or timeline of the CSI report. These exceptional cases can be FFS. 2) For Option 2a, the “at least one of Top M” should be replaced with “all of Top M”, since “at least one”may overestimate the accuracy, e.g., if the genie-aided Top-1 beam is NOT among Top-K predicted beams but another weaker genie-aided Top beam (assuming M>1) is among Top-K predicted beams, it will still be counted into Np. 3) We are open to discuss Option 3a, but can proponents clarify the applicable case of 3a? In our view, Option 3a may overestimate the accuracy, e.g., in case Top-K predicted beams cannot cover Top-1 genie-aided beam but are among weaker beams of the genie-aided Top M beams (assuming M>1), the genie-aided Top-1 beam is missed by prediction yet it will still be counted into Np. Changes: - Where as the number of the valid (FFS on the definition of validity) reported inference result(s) linked with performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection: - Option 2a (Top-M/K): at least one all of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams - K= 1, [2, 4, 5] is configured - M=1, [2, 4] is configured - K>=M - FFS: K>1 - FFS: M>1 |
| NTT DOCOMO |  | This proposal ignores performance monitoring based on the subset of Set A. If performance monitoring is a subset of Set A, performance metric is judged as not accurate due to not only the prediction accuracy but also the failure of the performence metric calculation, such as none of predicted top-K beams are induclded in monitoring resource. The beam accuracy metric NW needs is the number of accurate metric instances / the number of valid metric instances, where valid metric instance is the instance whose metric is calculatable. So even if UE report the number of accurate metric instances, NW cannot calculate the beam accuracy metrics without the number of valid metric instances. If inference result reporting is always decoded at NW side without any error, NW can obtain the number of valid metric instances based on inference result reporting and RRC configuration of monitoring. However, this is not reasonable assumption. Hence, we recommend either one of the following. ・UE reports the number of accurate metric instances and the number of valid metric instances ・UE reports the ratio of the number of accurate metric instances to the number of valid metric instances |
| Nokia | Y | In general, we are OK with the direction. Also, agree with the comment from HW on valid performance monitoring instances. On the metric itself, we think that RAN1 has not evaluated impacts on LCM actions (and overall performance) for any performance monitoring metric. Given this is the first release and we do not have actual implementations yet, we shall provide the freedom for companies who implement this to fine-tune exact parameter setting for their needs. In other words, we think that restricting to accuracy for intermediate KPIs may be restrictive. Few edits as below, At least for BM-Case1, in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, - Note: The resource set(s) for monitoring may or may not be the full set of Set A - beam accuracy indicator (BAI) is, (0 ≤ ≤ N) - N is the number of performance monitoring instance(s), configured by gNB in configuration of CSI report for monitoring. - N is configured in report configuration for monitoring. - Both N=1 and N>1 are supported, FFS values - the size of CSI field associated with the BAI is . - Where as the number of the reported inference result(s) linked with valid performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection: - The UE consider Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring for determine Np. M is configured (1, 2, 4, 8) - The UE consider valid performance monitoring instances, where validity is determined - when there is at least one inference report instance that is linked with the monitoring instance, and - when the inference report (Top-K beams) is measured in the performance monitoring instance - For M <= K, Option 2a (Top-M/K): at least one of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams - K= Equal to the configured K value for inference 1, [2, 4, 5] is configured - M=1, - K>=M - FFS: K>1 - FFS: M>1 - For M > K, Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= Equal to the configured K value for inference - K= 1, [2, 4, 5] is configured - M=1, [2, 4, 8] is configured - K<=M - FFS: K>1 - FFS: whether/how Top-M/K or Top M/M with margin is allowed |
| OPPO | Y | We are in general okay to define the BAI as a number rather than a ratio. It could somehow save the efforts in quantization of the ratio of beam prediction accuracy in RAN4 spec, which can be calculated by NW implementation anyway. Secondly, if we include the case of total number N = 1, then it seems to check whether the beam prediction is correct or not on only one instance. Then the beam prediction accuracy would be either 100% or 0%. We don’t know whether such 1 or 0 information can help NW to make LCM decisions, rather than the accuracy in-between 0 and 1, such as 75% or 35%, etc. Finally, on selection of metric based on Option 2a and 3a, we believe Option 3a makes much more sense. Literally, the intention of this metric is to check whether the predicted beam, i.e. Top-K is among the true best beam, i.e. Top-M, but not the other way around. In addition, there could be next round of beam sweeping after beam prediction, so we don’t have to be strigent to require all Top-K to be among Top-M, once this is a good one, it can be found in follow-up beam sweeping with less overhead. Hence, we suggest to allow one, but not all as following change. - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring But anyway, given the current situation, we are fine to have a down selection in near future. |
| Xiaomi |  | We still prefer a ratio than a number since the candidate values and the payload size can be fixed for different value of N. We can’t see the benefit of report a number even the quantization, since if the value of N is not large, the ratio and the Np can have a one-to-one mapping. And if the resource set for monitoring is the subset of set A, some time instance may be a invalid instance for metric calculation. In this case, it doesn’t work if only report the number holding the statement. In addition, if the resource set for monitoring is different from set A, i.e., resource set for monitoring is a set with wide beam, the statement should be revised accordingly. So we suggest to update the note as below: Note: At least for the case that the resource set(s) for monitoring is the full set of Set A. Regarding Option2a and 3a, we prefer 2a. And support K>1. While M=1 can be the baseline. |
| Fujitsu |  | We don’t see the need to support N=1. The performance monitoring should be based on measurement within certain period. The metric calculation is meaningless if it is based on one shot measurement, as commented by OPPO. In addition, the time domain behavior of the resource set for monitoring should be clarified, i.e., it is periodic/semi-persistent. |
| TCL | Y | Firstly, we supprot Option 3a. We agree with OPPO that Option 3a checks whether the predicted beam is among the true best beam which better caputures the objective of model evaluation. To address Huawei’s concern, possible ways are to set some restrictions on Option 3a, for example, - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. Or, - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M one beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. Or, - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring. - Top-1 measured beams must be within the Top-K predicted beams. |
| ZTE |  | Generally, Option 2a and Option 3a are aligned with the Top-K/1 beam prediction accuracy and Top-1/K beam prediction accuracy in TR 38.843, respectively, and Option 2a should be the baseline as R18 does. M>1 in Option 2a and K>1 in Option 3a are reasonable and can be configured by NW. For Option 2a, Top-K beam sweeping is assumed. M>1 implies that the AI model targets at predicting one of the Top M beams, which corresponds to relaxed model capability. For Option 3a, we assume that there is no Top-K beam sweeping. Then all of the Top-K predicted beams should be among the Top M genie-aided beams, and thus the randomly beam selection at gNB side will not cause much performance degradation. For the definition of BAI, the number of correct prediction is preferred since it can avoid the additional spec efforts for the quantization design and for the differentiation between valid and invalid monitoring occasions. |
| vivo |  | We are okay for the proposal. Some further comments: - The size of BAI should not change. Hence N should be the number of configured monitoring occasions, not the valid occasions as commented by some companies. Further, as UE can just report the number of correct predictions Np as given in the proposal, there is no need to have any definition on validity. - For Np, current description is not clear, we suggest the following simplification, - Where as the number of the reported inference result(s) linked with performance monitoring instance(s) correct predictions from N monitoring instances, - For the metric definition based on option 3a, share smilier view with OPPO, we prefer at least on one of the the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring |
| QC |  | The definition for Alt. 1 for which this proposal is trying to introduce a precise metric is beam prediction accuracy with or without margin. It is not clear based on which technical ground and justification the FL has already concluded to not even include beam prediction accuracy with margin as an option. If anything, in our view, it does make more sense compared to Option 3a in FL’s proposal, which, as brought up by us and Huawei before, may misrepresent beam prediction performance. We have provided detailed reasoning in our contribution as to why a beam prediction accuracy with margin is needed, that we include here again for reference: From 2501144: On the necessity of defining Top-K beam prediction accuracy with L1-RSRP margin: Let us consider the following observations from TR 38.843: For spatial beam prediction with measurements of fixed Set B of beams that of 1/8 of Set A of beams: Table 3 Top-1 beam prediction accuracy with and without margin for spatial beam prediction Top-1 DL Tx beam prediction accuracy; Top-1 DL Tx beam prediction accuracy with 1dB margin. # of sources; Beam prediction accuracy (%); # of sources; Beam prediction accuracy with 1dB margin (%). 7; 50; 7; 70-80. 4; 60-70; 1; 80-90. 5; 70-80; 5; More than 90. 4; More than 80. As we see from the above observations, defining margins even as low as 1 dB may have a significant impact on the beam prediction accuracy, while not impacting the overall performance of AI/ML-based beam prediction. With no margin, the reported beam prediction accuracy metrics may be misleading to the NW as the beam prediction may be very reasonable for an instance with very small L1-RSRP delta compared to the best beam from the performance monitoring set. For such a reasonable prediction, Top-K beam prediction accuracy with no L1-RSRP margin counts this as a failure, and hence the reported metric may not be very useful to NW. Additionally, the Top-K beam prediction accuracy with L1-RSRP margin is inclusive of the one with no margin once the margin is set to 0 dB. During discussions it was argued that if needed, Top-K beam prediction accuracy with L1-RSRP margin can be implemented via NW-side performance monitoring, but there’s a significant difference in terms of the report overhead. As an example, consider 16 performance monitoring instances. Even if UE reports only Top-4 beams from the performance monitoring set to the NW per instance, then the total overhead will be , whereas the overhead for the Top-K beam prediction accuracy with L1-RSRP margin would be 3 or 4 bits reported at the end of 16 instances. So, the overhead difference is significant, and this clearly motivates definition of Top-K beam prediction accuracy with L1-RSRP margin. Also agree with Huawei and Nokia on the fact that it would be beneficial to count for “valid” performance monitoring instances. Main issue with current version of Option 2a is that it may be the case that the L1-RSRP difference between the Top-1 beam and Top-2 beam with largest and 2nd largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring may be large, in which case Top-K may include the Top-2 beam and this condition is satisfied, but this may result in poor performance. With that said, we suggest the following modifications: Updated Proposal 1.1 At least for BM-Case1, in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, - Note: The resource set(s) for monitoring may or may not be the full set of Set A - beam accuracy indicator (BAI) is, (0 ≤ ≤ N) - N is the number of performance monitoring instance(s), configured by gNB in configuration of CSI report for monitoring. - N is configured in report configuration for monitoring. - Both N=1 and N>1 are supported, FFS values - the size of CSI field associated with the BAI is .FFS: size of the CSI field - Where as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection: - Option 2a (Top-1M/K without L1-RSRP margin): at least one of the Top M beam(s)Top-1 beam with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are is among Top-K predicted beams - K= 1, [2, 4, 5] is configured - M=1, [2, 4] is configured - K>=M - FFS: K>1 - FFS: M>1 - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= 1, [2, 4, 5] is configured - M=1, [2, 4, 8] is configured - K<=M - FFS: K>1 - FFS: whether/how Top-M/K or Top M/M with margin is allowed - Option 4: (Top-1/K with L1-RSRP margin): The highest measured L1-RSRP of Top-K predicted beams that are within the performance monitoring set is within X dB of measured L1-RSRP of Top-1 measured beam ID from performance monitoring set. X can be configured by gNB in configuration of CSI report for monitoring. |
| Google |  | We suggest changing the words “among“ in option 2a/3a into “associated with“, since we agreed the resource for monitoring does not have to be the full set A. |
| CMCC |  | First we needs clarification on the 1st note. It seems precluding the case that the beams other than set A configured in for the monitoring set. The modification (one another note) to the current proposal is as follows. - Note: The resource set(s) for monitoring may or may not be the full set of Set A - Note: The resource set(s) for monitoring including the resources other than that of Set A can be discussed separately. FFS how to determine the correct prediction for this case. We do not see much difference between reporting the number Np and the ratio of correct predictions. But on the other side, do we really needs to know the exact number of the correct predictions. We prefer a ratio than a number Np since a ratio has fixed payload size and less bit number with appropriate quantization. If UE reports a ratio, there is no ambiguity between UE and gNB. But if UE reports Np, there is no benefit in reducing payload. And there may be ambiguity in calculating prediction accuracy if gNB does not decode inference results correctly and does not know the number of valid monitoring occasions. On the other side, we are still confused the reason to introduce valid monitoring occasions. our understanding is that, if the monitor set is a subset of set A, with a proper selection of the beam for monitoring and correct prediction, at least one of the predicted Top K beams would be fall into the monitor set. Introducing the concept of valid monitor occasions would introduce misalignment for the calculation of the ratio. Regarding value of N, N=1 is not supported since monitoring result per instance is not accurate. It needs further clarification on the benefits of per instance monitor reporting compared with the statistical results over a period. Option 2a is preferred than option 3a. We agree with HW that “at least one of the Top M beam(s)” will overestimate prediction accuracy. It is suggested to be modified as “all of the Top M beam(s)”. |
| Sony |  | We are generally okay with it. However, we still have some concerns about options 2a and 3a. If the top-K predicted beams encompass all M beams with the largest M measured L1-RSRP values of the resource sets for monitoring, we believe this is reasonable. This is because during the second measurement of the K predicted beams, the optimal beam can still be selected. However, consider a case where the actual optimal top-1 beam (i.e., the one with the highest measured L1-RSRP values of the resource sets for monitoring) is not among the top-K predicted beams, would this situation be considered a correct prediction or a failed prediction? It needs to be clarified on whether this should be classified as a successful prediction or a failure. |
| InterDigital |  | We also prefer to indicate a ratio rather indicating the number of instances. Support the direction from CMCC to separately discuss the case that the monitoring resources do not include Top-K resources. |
| ETRI |  | We are generally support this proposal. However, as a same view as HW, valid monitoring sample could be considerecd especially when monitoring set is a subset of set A. |
| Panasonic |  | Firstly, we share similar views as DOCOMO and Xiaomi that a BAI should be defined as a ratio, instead of a number. It helps to maintain a fixed payload of reporting. Secondly, we cannot see a motivation to support N=1 because performance mornitoring is not required to be checked for every inference instance. |
| Spreadtrum |  | We think it makes more sense to report the ratio rather than the number of instances that satisfy the condition. First, consider that the configuration of monitoring set may cause some instances to fail to compute metric (for example, monitoring set is subset of set A). Alternatively, the association between monitoring instance and inference result instance fails, causing some instances to fail to calculate the metric. These all result in the number of valid instances being less than the number of performance monitoring instances (s). In this case, the Np value reported is also based on the number of valid instances that meet the condition. Therefore, the actual ratio cannot be obtained simply by reporting the number of instances meeting the conditions. The UE also needs to report an additional number of valid performance monitoring instances. Therefore, it is more reasonable to report the ratio between the number of qualified instances and the number of valid performance monitoring instances. For Option 2a and 3a, we prefer option 2a. Because the purpose of this metric should be to determine whether the Top M with largest measured value(s) of L1-RSRP is in the predicted Top K beams. Because as long as this condition is met, NW can find the optimal beam through the second round of beam sweeping. So option2a makes more sense. |
| LG | Y | Generally fine with FL’s direction, and prefer Nokia’s revision. We share similar view as OPPO that Option 3a seems more reasonable to evaluate beam prediction accuracy according to whether or not predicted Top-K beam(s) is included in Top-M actual measured beam(s) up to NW’s configuration/allowance regarding M value. It should be further discussed regrading “valid“ performance monitoring instance, considering time domain linkage between inference report instance and monitoring report instance(FL proposal 1.2a). |
| Sharp |  | Regarding the definition for Np, we think that N is the configured monitoring occasions and the Np should be the monitoring ocasions out of the N monitoring occasions, whose linked inference resutls can satisfy witht the conditions. - Where as the number of performance monitoring instance(s), out of N performance monitoring instaces(s), linked with the reported inference result(s) linked with performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection: |
| Ericsson |  | In general fine with the direction, some comments: - N is the number of performance monitoring instance(s), configured by gNB in configuration of CSI report for monitoring. For periodic inference and monitoring, would not the NW configure in the CSI report setting the periodicity of the performance monitoring report (not the exact number). Hence, this would be a better formulation: - N is the number of performance monitoring instance(s), - FFS if N is explicitly configured by gNB, or explicitly configured via resource and report periodicities in configuration of CSI report for monitoring. Regarding option 3a, it is not clear what such options adds, option 2a where at least one of the top-K beams from inference is among the strongest is sufficient. Also, we are supportive of a that there can be a margin of x dB to the strongest as evaluated in the evaluations. - Option 2a (Top-M/K): at least one of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams - K= Equal to the configured K value for inference - Top-M beams are the M beams that are within x dB of the top-1 beam - X is NW configured - X= 0,1,[2,3,4…] - 1, [2, 4, 5] is configured - M=1, - K>=M - FFS: K>1 - FFS: M>1 |
| CATT |  | We are general ok with the proposal. We share same view as DOCOMO, xiaomi and Panasonic that the BAI shoul be defied as a ratio. Because the number of “successful inference instances”cann’t always correctly reflect beam prediction accuracy and the network and UE may have different understanding on the number of valid performance monitoring instances. For example, if the monitoring resource set is the subset of Set A and the network misses the inference results, the number of valid performance monitoring instances for network is smaller than the number of valid performance monitoring instances for UE. Then the network may overestimate beam prediction accurracy for a UE-sided model. Thus, we prefer the UE reports the ratio of Np and N. For the counting of Np, Option 2a with M=1 is our preferred since it has been evaluated in Rel-18 SI phase and the evaluation results provided by company can be found in TR 38.843. For the case of Option 3a that M is more than one, e.g. M=2, if the predicted Top-1 beam is the second best measured beam, this can be regarded as a successful inference, which will lead network cann’t find genie-aided Top-1 beam. Option 3a has negative impact on the network. |
| NEC |  | We prefer to report a ratio, not the number. But even it is to report the number, we don’t need log2(N+1) bits because it is not useful to report very small or very large Np, better to quantize it to a range and report quantization index. - In addtion, the performance metric calculated based on all samples within the monitoring window may not be sufficient to reflect the (future) performance of the model. Moreover, we have some comments on Option 3a as follows. - Option 3a (Top-K/M): [FFS: at least one or all of] the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= 1, [2, 4, 5] is configured - M=1, [2, 4, 8] is configured - K<=M - FFS: K>1 |
| Futurewei |  | We are in general ok with the direction of the proposal and prefer Option 2a with M=1 as baseline. We also agreed with other companies that the counting of Np should be based on the valid performance monitoring instances. |

##### (FL0) Proposal 1.1b (metric)

At least for BM-Case1, in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring,

- Note: The resource set(s) for monitoring may or may not be the full set of Set A

- Note: The resource set(s) for monitoring including the resources other than that of Set A can be discussed separately. FFS how to determine the correct prediction for this case.

- beam accuracy indicator (BAI) is  Np (0 ≤ Np≤ N), and the corresponding beam prediction accuracy is NpN, 0≤ NpN≤ 1).

  - Where N is the number of performance monitoring instance(s),

    - FFS if N is explicitly configured by gNB, or N is determinated by the UE based on the resources and reports in configuration of CSI report for monitoring.

    - N is configured in report configuration for monitoring.

    - Both N=1 and N>1 are supported, FFS values

    - the size of CSI field associated with the BAI is log2(N+1).

  - Where Np as the number of the reported inference result(s) linked with performance monitoring instance(s), for which, the following statement holds, consider the following with potential down selection:

    - Option 2a (Top-M/K): at least one of the Top M beam(s) with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams

      - Where K is the number of predicted beams in the corresponding inference report

      - M=1

      - FFS: M>1, where K>=M

    - Option 2 (Top-1/K):  the beam with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring are among Top-K predicted beams

      - Where K is the number of predicted beams in the corresponding inference report

    - Option 3a (Top-K/M): the Top-K predicted beams are within Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

      - Where K is the number of predicted beams in the corresponding inference report

      - M=1, [2, 4, 8] is configured

      - K<=M

    - Option 4: (Top-1/K with L1-RSRP margin): The highest measured L1-RSRP of Top-K predicted beams is within X dB of measured L1-RSRP of with largest measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring

### Issue #1.2: For UE sided model, resource configuration for performance monitoring

##### (FL0) Proposal 1.2a

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring,

- for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to [the CSI reference resource or the transmission occasion of the resource]

- for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource with is “closest” to the time instance for prediction

- FFS on definition of “closest” and any other condition for the linking

| Company | Y/N | Comments |
| --- | --- | --- |
| FL0 |  | Q1: Referece for the linkage of an inference result should be “a CSI reference resource or a transmission occasion of the resource“? Q2: How to define of “closest“ and how to define N? - A is closest before B or after B or either before or after B. (N for size determination of BAI refers to the number of the inference report) - B is closest before A. (N for size determination of BAI is the number of RS occasion) - Where Ais a CSI reference resource or a transmission occasion of the resource - B is a transmission occasion of the CSI-RS/SSB resource Q3: Any special condition or defination of “closest“? - FFS on closest: before only, after only, or either, whether how close needs to be defined. "no larger than a certain distance" - E.g., within a certain time unit - E.g., whether to define distance in OFDM symbol level, e.g. the first OFDM symbol each transmission occasion |
| Huawei, HiSilicon |  | Comment to Q1: The intention of using “CSI reference resource” is clear, while the intention of “Transmisison of RS” is not clear: the applicable time instance of the prediction result for BM-Case 1 is not equal to the Set B. We may use “applicable time instance of the inference result” to make it generic. Comment to Q2: Regarding “closest”, we prefer “either before or after B” – assuming P-CSI report based inference with prediction result applicable at 10ms, 30ms, 50ms..., and P-CSI report based monitoring with monitor RS at 15ms, 35ms, 55ms,..., then for the inference result at 30ms, pairing with 35ms monitor RS is more accurate than pairing with 15ms monitor RS. Assuming A-CSI report based inference with prediction result applicable at 10ms, and A-CSI report based monitoring with monitor A-CSI-RS at 15ms, then it is natural to pair them also, while the measurement for monitor A-CSI-RS before 10ms may be a long time distance ago and expired. Regarding the definition of N, for BM-Case 1, one inference report with one inference result (Top-K beams) is counted as one sample of N; for BM-Case 2, one predicted instance is counted as one sample of N – if there are multiple prediction instances, there should be multiple N, each of which is individually calculated. Comment to Q3: As analyzed in Q2, Regarding “closest”, we prefer “either before or after B”. Regarding the distance, it needs to be clarified, since it is our understanding that the CSI buffer for a CSI report config ID is updated with a new CSI occassion, while otherwise the CSI (e.g., inference result) is always stored in the CSI buffer. So as long as the inference report is earlier than monitoring report, the inference result is always available. However, the inference report should not be later than the monitoring report, since the inference results are not available for metric calculation. Regarding the time granularity, we are OK with the “ first OFDM symbol each transmission occasion”. Changes: - for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for based on Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to the applicable time instance of the inference result (FFS applicable time instance) [the CSI reference resource or the transmission occasion of the resource] - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource with is “closest” to the time instance for prediction - FFS on definition of “closest” and any other condition for the linking - Time instance of the inference report for the inference result should be no later than the Time instance of the monitoring CSI report corresponding to the CSI-RS/SSB resource |
| NTT DOCOMO |  | Support the principle of proposal. Q1: the latest transmission occasion of the CSI-RS/SSB resource no later than CSI reference resource can be the reference. Q2: no need to restrict either before or after, when we find the closest one. N is the number of transmission occasions for moniotirng, where the transmisison occasion should be no later than CSI reference for monitoring reporting and the trasnmission occasions used for the previous monitoring reporting instance should not be counted. Q3: ok to introduce the maximum distance to satisfy the closest condition. |
| Nokia | Y | OK with clarifying this. We think that a simple solution shall be considered on this. We did not fully understand the above proposal. We suggest the below, At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring associated, linking inference results and monitoring instance (associated with the inference CSI reporting configuration and monitoring CSI reporting configuration), - A relative time window from an inference result instance (reporting slot for the inference result in BM-Case1, reference time for the temporal domain predictions of each inference result in BM-Case2) is considered, where the relative time window may be defined or configured to the UE. - If there are monitoring RS measurement instances occur prior to the inference result instance within the relative time window, the latest monitoring RS measurement instance that is received prior to the inference result instance is considered for performance monitoring. - Otherwise, within the relative time window, the earliest monitoring RS measurement instance that is received after the inference result instance is considered for performance monitoring. - for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource which is “closest” to [the CSI reference resource or the transmission occasion of the resource] - for BM-Case 2, for an inference result for a time instance for prediction, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource with is “closest” to the time instance for prediction - FFS on definition of “closest” and any other condition for the linking |
| OPPO |  | A1: we think the reference for the linkage of an inference result should be a CSI reference resource. The reason lies in the fact that by the time of last RS transmission occasion, the model cannot predict Top-K beams immediately. In legacy CSI reporting (including beam reporting), the CSI reference time is introduced for both sides to have a common understanding on when the reported CSI is applicable. A2: we need a rule to set the closest in time domain. Both A before B and A after B can serve. It can make sure UE only compare the closest inference results and measurement results once. Otherwise, UE may calculate twice inference results and measurement results due to dynamic scheduled prediction and/or measurements, which seems not necessary. N to calculate BAI could be reused here. A3: Similar answer to A2. If we have to make a choice, we would like to pick A is before B only. It aligns the procedure that the model predicts first (A), then UE comes to see the measurements (B) and finally makes a comparison. There seems no need to further restrict how many symbols are in between, since the beam change is not so fast in symbol levels for low-to-mediate speed UE. |
| Xiaomi |  | Q1: we aren’t sure whether there is a reference resource for monitoring after each transmission occasion of the monitoring resource set. Since the monitoring report will be reported after N transmission occasion, there may be only one reference resource after the Nth transmission occasion. Thus, we prefer to use ‘a transmission occasion of the resource’ and suggest to use the last resource in the resource set for monitoring. Q2: we are fine with before or after. And N can be the number of the transmission occasion after the configuration/activation/triggering of the monitoring report and before the reference resource before the monitoring report. Q3: we think the transmission occasion of the resource set is a long time, it is better to define it in OFDM symbol level of one resource, e.g., the last resource. Regarding the certain time unit, we think it should be less than the half of the periodicity of the resource set for monitoring. |
| Fujitsu |  | Q1: we think it should be the transmission occasion of the resource which is more clear. Q2: We think the resource for montoring should be the closest after the resource for Set B, since the inference operation should be performed firstly. Q3: We think it should be within a certain time window, otherwise the monitoring accuracy can not be guaranteed. |
| TCL |  | Q1: Referece for the linkage of an inference result should be “a CSI reference resource or a transmission occasion of the resource“? A transmsission occasion of the resource. Q2: How to define of “closest“ and how to define N? - A is closest before B or after B or either before or after B. Q3: Any special condition or defination of “closest“? - Either before or after is fine. - OK with the symbol level definition of distance. |
| ZTE |  | Q1. It depends on the time instance of the inference result in the CSI report. Consider that during the evaluation of AI beam prediction, one-shot prediction is always assumed, implying that Set B for model input and Set A for model output (as well as CSI reporting content) correspond to the same time instance. Therefore, reference for the linkage of an inference result should be a transmission occasion of the resource, i.e., Set B occasion. Q2. Either before or after is OK. The key is that the inference result and measurement result of monitoring resource set should correspond to the same or similar time instance, regardless of before or after. Additionally, N for size determination of BAI should be the number of monitoring RS occasion. Apparently, the transmission of monitoring resource set is more frequent than that of inference report. N being the number of monitoring RS occasion can guarantee that only one inference report is associated with each monitoring occasion. Conversely, if N is defined as the number of inference report, multiple inference reports may be linked with the same monitoring occasion which doesn't make sense. Q3: The definition of ‘closest’ should be within a certain time unit. Otherwise, in case that there is a significant time gap between the monitoring resource set occasion and the time instance associated with the inference report, the measurement result and inference result would correspond to distinct beam information for quite different time instances, and thus the comparison at those instances loses relevance and cannot reflect the model performance. |
| vivo |  | Q1: We agree that the reference of inference resource is the reference resource. However, regarding how to associate the inference resource and the monitoring RS occasion, there would be two options. - Option 1: Associate the monotring RS occasion to the inference reference resource, i.e., UE identifies inference reference resource first, and then identify the associated monitroing RS occasion to calculate KPI - Option 2: Associate the inference reference resource to the monitoring RS occasion, i.e., UE identifies the monitoring RS occasion first, and then identify the associated inference resource resource to calculate KPI Using the two options may lead to different linking between inference reference resources and monitoring RS occasions. Given the operation is on monitoring, we think opion 2 is a more natural way, i.e., we shall associate the inference reference resource to the monitoring RS occasion, i.e., UE identifies the monitoring RS occasion first, and then identify the associated inference resource resource to calculate KPI. Q2: We think a direction is needed to define closest, i.e., we shall define it is the closest before, or closest after. Otherwise there will be ambiguity. - There could be two occasions which is closest in two directions if we don’t define it clearly, as in the figure below. - There could be two monitoring occasions linked to a same inference reference resource if we don’t define it clearly. Further, Even if we define a direction, e.g., closest before, NW can make this one to be the real cloest one by proper configuration. Hence we don’t see the concern from NW vendors. How to define N for BM Case 2: We think in the proposal, more discussion is needed for Case2, esp. the issue of the granularity to define N. Utilizing an association method that employs a monitoring occasion and an inference occasion as granularity, akin to the approach in spatial domain prediction, can result in different monitoring occasions within the same monitoring occasion set being linked to disparate inference occasion sets. - Occasion level granularity for determining reference points for BM-Case2 can result in different monitoring occasions within the same monitoring occasion set being linked to disparate inference occasion sets. - Occasion set level granularity for determining reference points for BM-Case2 is more clear. Thus, N is defined as the number of monitoring set and the number of occasion set configured in CSI-reportConfig with usage of performance monitoring for BM-Case1 and BM-Case2, respectively, Q3: Similar to Q2. But as long as it is clear on before or after, there should be no other issues. |
| QC |  | Answers to questions: A1: CSI reference resource A2: closest either before or after A3: We believe we should define a threshold value for the aforementioned distance. Details of how to define such threshold value can be FFS. |
| Google |  | We suggest reusing the timeline for the linked CSI report agreed in R19 MIMO. |
| CMCC |  | With current wording, it seems for each inference result, we will find a linked monitoring occasion. If periodicity of monitoring set is large and the time gap between some inference result and monitoring occasion is large, we still calculate prediction with linked pair. Is it more reasonable to find a linked inference result for each monitoring occasion ? For Q1, the transmission occasion of the resource is fine since the inference result for BM case 1 is for the same instance of transmission occasion of set B. For Q2, A is closest before B or after B. For both BM case 1 and 2, considering the applicable time instance of the inference result, another restriction needs to be added is that the CSI reference resource of inference result should before the CSI reference resource of monitoring report. |
| Panasonic |  | The proposal is not clear to us. We think it is necessary to define a time domain window (TDW) for peformance monitoring first, then we can determine which interference results are located within the TDW, so that it can be used to determine a linkage between inference results and monitoring instance in Q1 and determine a “closest“ in Q2 and Q3 if any. |
| Spreadtrum |  | Q1: we think it should be the transmission occasion of the resource. When the monitoring report is based on multiple inference results and there is only one CSI reference time, only one inference result that is closest to the CSI reference time for monitoring can be found. This is obviously not reasonable. Therefore, it is more reasonable to choose the transmission occasion of the resource. Q2: no need to restrict either before or after, we only need find the closest one. Q3: We think a certain time window should be considered, to ensure the validity of the calculated metric. |
| LG |  | For BM-case 2, we are fine. For BM-case 1, I think that the following revision is needed? At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), for the configuration for monitoring, - for BM-Case 1, for an inference result with [a CSI reference resource or a transmission occasion of the resource] for Set B, the linked transmission occasion is a transmission occasion of the CSI-RS/SSB resource for monitoring which is “closest” to [the CSI reference resource or the transmission occasion of the resource] And provide our answers regarding FL’s questions: Q1: CSI reference resource looks more reasonable to us. Q3: the time gap between a reference of inference result report and the transmission occasion of the CSI-RS/SSB resource for monitoring should be within X symbols/ms, where X is up to UE capability reporting. |
| Sharp |  | General question for clarification: We consider that inference reporting occurs more frequently than monitoring occasions/measurements, which means there are more inference results than monitoring occasions. The motivation behind the current proposal is to determine whether each inference result should be linked to a monitoring occasion, or if only a subset should be selected for performance evaluation. Q1: either is fine give both could decide a specific linkage determinaton. Q2: We prefe “ before and after” to find a more inference result time-related to monitoring occasion. For Case 2 where mulitple inferecne results are applied for tuture time instances respectively, “within a predicted time interval” could be a better clarification to “closest”. As in the following illustrarive Figure, we need to clairfy which inference result is used to compare with the measured beams on monitoring occasion. Regarding how to define N, the N monitoring occasions refers to latest N monitoring occasions prior to the reporting instance of BAI. Q3: Additinal conditions can be considered to define the “closest”. To accurately evaluate the prediction accuracy, “ a certain distance” could be further used. For BM Case2, “within a predicted time interval” can be considered as illustrated in Q2. |
| Ericsson |  | Support to agree on a definition. Given the above comments, this is a more challenging question that just agree on the closest. The monitoring resource set should be selected that best represent the UE inference channel conditions and its predictions. We should further outline options for an exact definition. |
| Fraunhofer |  | Q1: We believe the CSI reference resource should be the reference since the main purpose of the reference is to have alignment between the UE and the NW. That can be achieved with the CSI reference resource and is unrelated to the issue of the prediction horizon. Q2: We are fine with either. Q3: A maximum distance should be defined. |
| CATT |  | - Regarding Q1, for BM-Case 1, the reference for the linkage of an inference results should be“a CSI reference resource. And for BM-Case 2, the reference for the linkage of an inference results should be “ the slot interval of the prediction of an inference report instance “. In Rel-18 CSI prediction in MIMO, the reported PMI indicates the precoder matrices for N4 consecutive slot intervals, each with duration of d slots. This definition can be adapted for BM-Case2, where the prediction time is the N consecutive slot intervals. - Regarding Q2, A is closest before B or after B or either before or after B. (N for size determination of BAI refers to the number of the inference report) For one-to-one link between an inference reslut and transmission occasion(s) of monitoring resource,The UE can first determine the transmission occasion(s) of monitoring resource (B) for obtaining the measurement result(s)/ ground-truth, and then determines the inference report instance (A) linked with each measurement result. Regarding Q3, the linked inference report instance needs to meet the condition that the prediction time of the linked inference report instance is closest (including before and after)to the transmission occasion of the monitoring resource in the time domain. Additionally, the report time of the inference report instance should be no later than the CSI reference resource of the monitoring report. |
| NEC |  | About Q1: Not sure the difference between a CSI reference resource and a transmission occasion of the resource. The definition of a CSI reference resource is necessary. About Q2: Regarding “closest”, both either before or after are fine but the certain time interval (unit) between A and B need to be defined. - About Q3: It seems this question is covered by Q2. |
| Futurewei |  | Q1: CSI reference resource. Q2: Closest either before or after. Q3: A threshold on the distance is needed. |

Reference resource of each inference report

Transmission occasions of RS

for monitoring

Linked inference resource to transmission occasion

For BAI calculation

Ericsson

Figure 2: aPeriodic report using periodic or semi-persistent resources

For the reportConfigType of the UE-sided model performance metric,

  - Support periodic and semi-persistent reportConfigType if the linked inference report is also periodic or semi-persistent

  - Support aPeriodic monitoring report only for the scenario where the UE has first been configured with a periodic or semi-persistent monitoring/inference resource config type

    - to enable the UE to include multiple samples when calculating the performance metric

- Sharp

Figure 2: Linkage between performance monitoring occasion and inference result for BM Case 2.

### Issue #1.3(on hold): For UE sided model, FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

Potential contents in RAN 1 #117:

- Event-1: Predicted beam accuracy of the set of predicted beams being below a threshold accuracy/The measured Top-1 or Top K beam(s) of Set A and the predicted Top-1 or Top K beam(s) of Set A are different

  - FFS on whether one shot or statistical results in a given window is used

  - Comments from FL: One shot is not reliable. I think in a window should be considered.

  - Supported by: Intel (Xdb margin), Nokia, Sharp, samsung

- Event-2: The measured L1-RSRP of one set of beams is lower than a threshold.

  - Comments from FL: isn’t this similar as BFD?

  - Supported by: Intel (Xdb margin)

- Event-3: The probability information of Top-1 or Top K beam of Set A is lower than a threshold, or difference comparing to previous is larger than a threshold

  - FFS on how to define the probability information

    - #1: The probability information of predicted Top 1

    - #2: The probability information of each or sum of predicted Top Top-K beams.

  - Comments from FL: this may be straightforward.

  - Supported by: OPPO,Samsung

- Event-4: The L1-RSRP difference between the measured Top-1 or Top K beam(s) of Set A and predicted Top-1 or Top K beam(s) of Set A are larger than a threshold value

  - FFS on RSRP difference information: e.g., RSRP difference, whether RSRP difference is higher than a threshold, all or part of RSRP difference

  - FFS on whether/how define the associated beams for RSRP difference information, e.g.,

    - #1: of a set of beams configured by NW

      - FFS on whether/how to handle the case if the configured beams are not the predicted Top 1 or Top K

    - #2: of predicted Top 1 or Top K beams

      - UE is not required to report the RSRP difference information if the configured beam is not predicted Top 1 or Top K beams

    - #3: RSRP difference between predicted Top 1 or Top K beams, and Top 1 or Top K beams of a set of beams (e.g., full or subset of Set A, same or different as predicted Top 1 or Top K beams)

  - FFS on how to configure resources to obtain the measured L1-RSRP

  - Comments from FL: Similar as Option C for type 1 performance monitoring

  - Supported by: Intel ,Samsung

- Event-5: consecutive N1 times of KPI/BLER less than threshold, or accumulated N1 times of KPI/BLER less than threshold within a time period.

  - Supported by: CMCC, Nokia

Event-6: Predicted L1-RSRP of the set of predicted beams being below a threshold value.

  - Supported by: Nokia

- Event-7: The Counter is incremented if the predicted Top K beam results differ from the actual measured results and/or if the L1-RSRP difference exceeds a predefined value

  - Supported by: ETRI

# Configuration for UE-sided model

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Futurewei [1] | Proposal 5: For Rel-19 AI/ML-based BM, for data collection for UE-sided AI/ML model of BM-Case1 and BM-Case2, support that NW provides/signals multiple possible configurations of DL RS transmission to the UE and the UE reports its supported/preferred one(s) out of the multiple configurations. Proposal 6: For Rel-19 AI/ML-based BM, for data collection for AI/ML model of BM-Case1 and BM-Case2, support using RS ID as implicit indication of beam ID and reusing L1-RSRP reporting as much as possible. Proposal 7: For UE sided model in beam management, confirm the following working assumption [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) |
| ZTE | Proposal 41: For UE sided model in beam management, UE may assume the similar properties of a DL Tx beam set/list associated with the same associated ID. Proposal 42: There is no necessity to further define similar properties of a DL Tx beam or beam set/list regarding UE assumption upon repeatedly receiving the same associated ID. Proposal 43: The configuration of associated ID is optional and can be configured by the NW if necessary. |
| Spreadtrum, UNISOC | Proposal 11: The associated ID should be configured in CSI-ReportConfig |
| Tejas | Proposal 11: For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - The reference time for these N instances should be aligned with the latest available measurement report from Set B. Observation 3: The UE is in a high-mobility environment, the prediction window should be shorter to maintain accuracy, while a more stable environment may allow for a longer window. Proposal 12: For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - The duration for which predictions can be made should be dynamically configurable, depending on network conditions and UE mobility. Observation 4: Set B is sufficient for inference results; Set A could remain unused in inference results. Supporting only Set B can reduce the complexity since the gNB does not need to transmit additional resource sets if they are not being utilized. Proposal 13: For UE-sided model at least for BM case-1, support only one resource set (Set B) is configured for CSI report generation and inference results reporting. Proposal 2: The associated ID should be configured for data collection (training) and inference. - If the associated ID is not configured for either data collection (training) or inference, UE may report it is not applicable. |
| Ofinno | - Support UE requests to initiate/terminate data collection for a UE-sided model: - RAN1 to discuss conditions to trigger UE requests to initiate data collection (e.g., for training a new UE-sided model to potentially replace the current UE-sided model deployed for inference). - RAN1 to discuss events to trigger UE requests stopping data collection (e.g., indicating the model training is completed in case UE performs the training (local refining)). - If Option 2 is considered, the CSI report timing should occur before the earliest time instance of the N time instances. |
| GOOGLE | Proposal 1: Support to configure AP-CSI-RS for BM as RS for Set B for BM-Case 2. Proposal 2: Support to configure SSB as RS for Set B for both BM-Case 1 and BM-Case 2. - Support both legacy SSB and on-demand SSB introduced in Rel-19 NES Proposal 3: Support to report a confidence level indicator for L1-RSRP report to indicate the maximum L1-RSRP measurement error for each beam. Proposal 4: Support to introduce a beam subset configuration indicating a subset of beams that the UE should always report in an L1-RSRP report - Support to define one L1-RSRP state to indicate the reported beam is in an invalid state (UE is unable to measure the L1-RSRP that can meet the L1-RSRP measurement requirement) Proposal 5: Support L1-RSRP report retransmission to facilitate the NW-side beam prediction for BM case 2 Proposal 18: Support to configure aperiodic CSI-RS as set A RS, which is triggered by a group-cast DCI. Proposal 19: For Set A RS, support the NW configures 1-port CSI-RS and UE shall not expect the NW multiplex any other DL RS in the same symbol with the set A RS. - The NW transmits the set A RS based on IFDMA structure for fast UE beam refinement |
| Ruijie | Proposal 1: For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support Option 3: - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource |
| CMCC | Proposal 4: Regarding data collection of UE-side model for training, UE can report or request gNB for the preferred set B (pattern). Proposal 5: Regarding data collection of UE-side model, the configuration method of set A for NW-side model can be reused. - Option 1: The UE capability of the maximum number of the RS per resource set for the RSRP measurement can be enhanced/enlarged. - Option 2: the configuration of set A can contain multiple resource sets in one CSI-ResourceConfig Proposal 6: For training data collection of UE-sided model, two CSI-ResourceConfigId s are configured for Set A and Set B separately. . Proposal 7: The indication of the association between Set A and Set B can be based on RS ID or bitmap. Proposal 8: Regarding training data collection for UE-side model, the configuration of multiple set B can be supported. |
| Intel | For a UE-side AI/ML model, for BM-Case 1/2, explicit configuration of set A and set B should be supported and the configuration may be based on UE capability and any UE-side conditions related to supported model and input/output types. Proposal 4: For a UE-side AI/ML model, for inference results reporting for BM-Case 1/2, support: • Approach 1: One CSI-ResourceConfigId each is configured for Set B, Set A respectively. • Approach 2: One CSI-ResourceConfigId is configured for Set B only and Set A is configured using a new IE as a set of NZP-CSI-RS-ResourceId or SSB-Index and the UE can derive the TCI states based on the TCI state list configuration where the resources in set A can be QCL sources. • Set B may be configured via an association to set A or independently. Proposal 5: For a UE-side AI/ML model, for BM-Case 2, UE may be configured with an observation/measurement window by the network. • The observation/measurement window can be provisioned to a UE based on configuration of a number of time instance(s) corresponding to a number of slots or CSI measurement occasions for Set B of beams. Proposal 6: For a UE-side AI/ML model, for BM-Case 2, the prediction window configuration should be subject to reported values on the length of the window that the UE-side model can support per UE capability/UAI. Proposal 7: For a UE-side AI/ML model, for BM-Case 2, the reference time for the start of the prediction window, i.e., of the earliest time instance for the predicted results is defined relative to the UL slot for the report. - For a UE-side AI/ML model, for BM-Case 2, the prediction window configuration, including duration of an instance and the number of instances for prediction, should be subject to reported values on the length of the window that the UE-side model can support per UE capability/UAI. - For a UE-side AI/ML model, for BM-Case 2, the reference time for the start of the prediction window, i.e., of the earliest time instance for the predicted results is defined relative to the UL slot for the report. |
| ZTE | For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support Option 3 (i.e., based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report) due to the associated configuration simplicity. Proposal 17: If Set A and Set B are different, resources for Set A and resources for Set B are configured as separate resource sets, and the association between Set A and Set B can be established based on the same CSI report setting. Proposal 18: If Set B is a subset of Set A, only resources for Set A is configured, and resources for Set B is indicated as a subset of Set A based on assistance information provided by the NW, such as the mapping between Set A and Set B in the form of bitmap. |
| Ericsson | For UE-sided model for BM-Case 2, for reference time of inference results report, support “Option 1: Based on the uplink slot for the report” |
| Vivo | Proposal 15: For inference, for UE-side model, support UE to recommend some preferred Set B patterns which were trained during the UE-side model training phase. Proposal 16: For inference, for UE-side model, support gNB to configure a resource list including multiple Set B patterns and activate resources of one Set B pattern from the list by signaling during the inference phase. Proposal 17: For UE-side model, support to configure full and/or subset of Set A associated with predicted beam report to address interference issue. Proposal 18: For inference, for UE-side model, Set A or resources in Set A should be indicated as virtual Set or virtual resource, which does not require measurement on these resources for the report. |
| OPPO | Proposal 16: For BM-Case2 with UE-side model, support to configure two resource sets for Set A and Set B separately in a CSI-ReportConfig. Proposal 17: For UE-side model training/inference/monitoring of BM-Case2, specify the measurement window and prediction window in time domain. Proposal 14: For BM-Case2 with UE-side model, support to adopt the CSI reference resource as the reference time of the earliest time instance for predicted results. Proposal 23: For data collection of UE-side model training, support to configure Set B (as model inputs) and Set A (as model labels) using existing CSI framework. Proposal 24: For data collection of UE-side model training, support to configure an associated ID along with the configuration of Set B and Set A. |
| Fujitsu | Proposal 5: - For training data collection, the reference signals same as Set B should be configured to obtain the model input data. And the reference signals same as Set A should be configured to obtain the ground truth data. - If Set B is subset of Set A, then only one resource set, i.e., Set A is configured. - If Set B is different from Set A, then two resource sets should be configured, one resource set for Set A and the other resource set for Set B. Proposal 6: - RAN1 to further discuss whether the same or separate reference signal configuration among training data collection, model inference and performance monitoring could be applied. - Regarding training data collection, RAN1 to further discuss the reference signal configuration for different sub-use cases. - Regarding the configuration of Set A and Set B for inference with UE side model, RAN1 can discuss whether new RRC parameter is needed or some existing RRC parameter could be updated to indicate that the resource set for Set A is not measured by the UE, and separate resource settings are configured for Set A and Set B. - For BM Case-2 with UE side model, support aperiodic CSI-RS for inference. RAN1 can further discuss the details on the configuration of aperiodic CSI-RS. - For BM Case-2 with UE side model, RAN1 to further discuss how to configure the observation window and prediction window for periodic CSI-RS and semi-persistent CSI-RS for inference. - For BM Case-2 with UE side model, regarding the reference time for the predicted results, Option 1 is preferred, i.e., based on the uplink slot for the report. |
| CATT | Proposal 7: For inference of UE-sided model, at least for BM-Case1, the following alternatives can be further supported for the CSI-ReportConfig of inference results reporting: - Alt 1: one CSI-ResourceConfigId is configured for Set B, and Set A can be determined from the associated id. Proposal 8: For UE-sided AI/ML model inference, for BM-Case2, AP CSI-RS can be supported for CMR configuration at least for Set B. Proposal 9: For UE-sided AI/ML model inference of BM-Case2, for periodic or semi-static CSI-RS configuration, one RS set can be configured as CMR for the inference report and the separation between two consecutive measurements is the periodicity of RS set. Proposal 10: For UE-sided AI/ML model inference of BM-Case2, for aperiodic CSI-RS configuration, consider two potential options as follows: - Option 1: One resource set with the resource of Set B, the transmission number of the RS set, and the separation between two consecutive transmissions can be configured for one triggering instance. Multiple transmissions of the resource set are triggered by one triggering instance. - Option 2: Multiple resource sets can be configured for one trigger state and each resource set can be configured with trigger offset separately. Multiple resource sets with different slot offsets are triggered by one triggering instance. Proposal 11: For UE-sided model for BM-Case2, for inference results report, the following option can be considered for determining the reference time of the earliest time instance for the predicted results: - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource. Proposal 12: For UE-sided model for BM-Case2, the UE is expected to report the predicted results for slot intervals, for, where is the earliest predicted time instance, is the number of the consecutive predicted slot interval(s) and is duration of each slot interval. Proposal 13: For UE-sided model for BM-Case2, for the duration values of the slot interval(s) that can be predicted support at least 320ms /480ms/640ms. Proposal 14: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported beam information can be the pre-defined beam index of the predicted Top K beam(s). - the pre-defined beam index can be 0 to M-1 , where M is the number of beams in Set A |
| Lenovo | - For UE-sided model, at least for BM Case-1, support only configure resource set for Set B for inference results report. Proposal 3: For UE-sided model at least for BM Case-1, support to only configure resource set for Set B for inference results report. Proposal 8: Considering the prediction window for BM-Case2 with the following modes: - Mode 1: - Mode 2: is the CSI reference resource for the beam report in slot n and is a value configured by RRC. Which mode is used can be configured by RRC according to UE capability. Proposal 10: RS configuration should be enhanced to support discontinuous RS transmission and adaptation for BM-Case 2. |
| Interdigital. | Proposal 5: For the configuration of Set A and Set B, no configuration method is supported other than configuration of two CSI-ResourceConfig Ids for Set A and Set B separately. Proposal 6: Support Set A for both no transmission and transmission with longer periodicity. Proposal 7: Support CSI resource configs with/without physical RS transmission parameters (e.g., resourceMapping, CSI-ResourcePeriodicityAndOffset and etc.). Proposal 8: Support configuration of RS resources to estimate QCL-parameters for a RS resource without transmission in Set A. Proposal 13: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B. Proposal 16: Support “Option 2: Based on the CSI reference resource corresponding to the report” for determination of reference time. Proposal 17: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration). Proposal 20: Indicating configuration information associated with Set B to UE should be supported. Proposal 21: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams). |
| Panasonic | Proposal 4: For UE-sided model inference, support that a measurement window can be configured with the measurement resource set. Proposal 6: For BM-Case 2 of UE-side model, support that a reference time of the earliest time instance for the predicted results is determined based on an offset and the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource (i.e., Option 3) - FFS: Details on offset between the latest transmission occasion of the CSI-RS/SSB resource in Set B and the first-time instance for the predicted results |
| Nokia | Proposal 2: For BM-Case2, for inference results report, support - N = 1, 2, and 4 - Duration values of the N time instance(s) that can be predicted = 40ms, 80ms, 160ms, 320ms, 640ms - Support Option 1 where reference time is the slot carrying the inference report - Support AP-CSI reporting with AP CSI-RS (for Set B) - FFS: Other overhead reduction options to apply when K and N values are large. Proposal 3: For UE-sided BM Case-2, CSI report configuration shall indicate the number of measurements (N) for Set B. Proposal 4: For UE-sided BM-Case 2, at least for inference, support repetition of Aperiodic (AP) CSI-RS resource set (for Set B) with a dedicated Trigger State (DCI carrying this only triggers the Set B measurements for a corresponding AP-CSI Report) and the AP-CSI report may be triggered separately (second DCI to get the AP-CSI-Report in PUSCH). - FFS: further details on triggering AP-CSI-RS repetition for Set B and linking to the corresponding report. |
| Samsung | Proposal 7. For UE-sided model at least for BM Case-1, for the configuration of inference results reporting using CSI-ReportConfig, support the configurability between Alt 1 and Alt 3. - Alt 1: one CSI-ResourceConfigId is configured for Set B - the size of Set A is configured by the CSI-ReportConfig. - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - The resources configured in Set B can be periodic, semi-persistent or aperiodic. - FFS: For BM-Case2, the configuration of aperiodic resources for Set B to facilitate measurements for multiple past time instance. Proposal 8. For UE-sided model at least for BM Case-1, for the association between Set A and Set B, introduce DL Tx IDs for the identification of downlink spatial domain transmission filter. - Each beam in Set A is associated with an DL Tx ID - Each beam in Set B is associated with an DL Tx ID - Note: UE assumes the beams corresponding to the same DL Tx ID shares the same downlink spatial domain transmission filter. - FFS: the relationship between DL Tx ID and associated ID. Proposal 16. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following: - UE to report the information on the supported/preferred associated ID. - FFS: Other information along with the report of the association ID. Proposal 17. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following: - NW configuration of associated ID in a CSI-ReportConfig Proposal 18. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, and for the definition of similar properties of a DL Tx beam or beam set/list - At least in a cell level, the UE assumes consistent physical beams characteristics on DL Tx beams (e.g., same downlink spatial domain transmission filters) of Set A and Set B, as well as a consistent mapping, e.g., consistent order of measurement resources Proposal 7. For data collection of UE-side AI/ML model, support a CSI-ReportConfig with reportQuantity set to ‘none’. - the CSI-ReportConfig can be configured with one or two CSI-ResourceConfigId for measurement - E.g. For time domain predication only, one CSI-ResourceConfigId is configured; otherwise, two CSI-ResourceConfigId s are configured for Set A and Set B separately - the CSI-ReportConfig can be configured with an associated ID |
| Transsion | Proposal 6: Regarding UE-sided model for BM Case-2, for the reference time to determine the earliest time instance from the future time instances, support Option 1(2nd) and Option 3(1st): - Option 1: Based on the time domain resource for the report. - Option 3: Based on the last transmission occasion of the CSI-RS/SSB resource in Set B for the report. |
| DOCOMO | Proposal 3: UE may request Set A and Set B measurement for training in Step 4 of applicability reporting, when UE is ready for data collection of training the corresponding functionality. Proposal 4: Two CSI resource related configurations should be configured for Set A and Set B for the data collection of UE side model training. Proposal 8: Following approaches can be applied to obtain measurements at multiple time instances for BM-Case2 ・Periodic and semi-persistent consecutive CSI-RS transmission occasions are used as measurements at multiple time instances ・Number of transmission occasions per aperiodic CSI-RS resource is configured to enable measurements at multiple time instances Proposal 9: For BM-case2, UE should report only if receiving multiple CSI-RS transmission occasions no later than CSI reference resource after CSI report (re)configuration Proposal 10: Support the following payload overhead reduction on the reporting of predicted results at multiple time instances. ・Absolute RSRP representation for the combination of CRI/SSBRI and time instance achieving the largest RSRP. ・Differential RSRP representation from the above absolute RSRP for the remaining combinations of CRI/SSBRI and time instance. |
| Sharp | - For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, the existing IE resourcesForChannelMeasurement configure Set B and a new IE is introduced to configure Set A in the CSI report configuration. - For BM Case-2, at least for inference, at least for Set B, support Aperiodic (AP) CSI-RS for CMR. - For UE-side model for BM Case 2, for the reference time of the earliest time instance for the predicted results, consider the following option 3: - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resources. - For UE-side model for BM Case 2, for the reference time of the earliest time instance for the predicted results, the starting location of the N future time instance is the slot , where - Slot is a slot in which the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement is located, - is a configurable value by gNB based on UE’s reported values via capability signalling. - For UE-side model for BM Case 2, duration of each time instance can be configured by the gNB from the supported duration values that are reported by the UE via UE capability signalling. - For UE-side model for BM Case 2, a timing pattern can be configured to indicate the specific timing that the inference results are to be included in the inference report. |
| Qualcomm | #### Proposal 9 For UE-sided beam prediction, use CSI-ReportConfig to configure resources for data collection purpose, with the following further details: - reportQuantity is set to none. - Associated ID is provided within CSI-ReportConfig. - Two CSI-ResourceConfigIds are configured for Set A and Set B separately. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 5: For BM-Case2, for inference, the reference time of the predicted time instances shall be determined based on the CSI reference resource or the transmission occasion of Set B resources. Proposal 6: For BM-Case2, for UE-sided models, introduce a configuration that allows the exclusion of certain past measurements for the inference. Proposal 12: For the configuration of set A and set B of beams for UE-side BM-Case1, the relationship in use-cases where set B of beams is a subset of set A of beams shall be exploited for overhead reduction of CSI resource and/or report configuration. |
| ITL [31] | Proposal 18: For the reference time of the time instance(s) in the UE-side model for BM-Case 2, it is proposed to down-select that the CSI reference resource corresponding to the report is used to determine the earliest time instance (or each time instance) from the N future time instances. Proposal 19: For UE-sided model inference in BM-Case2, - the earliest time instance can be determined by applying at least an additional offset value based on the reference time - each of N future time instances can consist of at least one slot - the time stamp of future time instance(s) can be implicitly indicated to NW. |
| Huawei/HiSi | Proposal 24: For BM-Case 2 with a UE-side model, for the reference time to determine the earliest time instance of the prediction window, consider the following options with higher priority. - Option 1: Based on the time domain resource for the report. - Option 2: Based on the CSI reference resource corresponding to the report. - Note: for Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B, it needs to clarify how to handle the case when the predicted instances are earlier than the CSI report. Proposal 23: For BM-Case 2 with a UE-side model, consider enhancement on the CPU rule to handle the processing of the long observation window. - For P/SP CSI-RS resource type, consider efficient CPU occupation criteria to process the observation instances to avoid long CPU occupancy along with the whole observation window. - A-CSI-RS resource type may not be applicable as it causes long scheduling delay. Proposal 7: For training data collection of a UE-side model, consider separate CSI-ReportConfigs for measurement of Set A and Set B, respectively. Proposal 8: For training data collection of a UE-side model, regarding the training data collection related parameters: - The needed number of samples for training data collection can be reported by UE. - RS configuration related information (e.g., Set A and Set B) is determined by gNB. |
| Xiaomi | Proposal 3-1: For UE-sided model inference, support to configure one resource set with resources in more than one measurement time instance for configuration of set B in BM Case 2. Proposal 3-2: For UE-sided model inference, support to take the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report as the reference time to indicate the time domain location of predicted time instance in BM Case 2, i.e., support Option 3. Proposal 3-6: For the configuration of inference results reporting for UE-sided model, support to configure the RS ID of resources in Set A. Proposal 3-7: For data collection for UE-side AI/ML model training, consider the following two configurations of resources for Set B and Set A for down-selection. - One resource set with additional indication of the number/location of resources in Set B. - Set B: the first N resources or the resources with corresponding bit value ‘1’ in a bitmap - Set A: - All resources except the resources in Set B, if the QCL source of the remaining resources are configured as the resources in Set B respectively. - Else, All resources in the resource set. - Two CSI-ResourceConfigid Proposal 3-7: Both explicit and implicit association between set B and set A can be supported for data collection for UE-side AI/ML model training. - Implicit association: - With same resourceConfig ID or reportConfig ID, resource set with lower set ID(s) is(are) for set B, the last one is for set A. - With same resource set ID, resources with lower IDs are for set B and others are for set A. - Explicit association: - Introduce resourceforModelInput and resourceforModelOutput in reportConfig. - Introduce data set/association ID for each resource set. - With same resource set ID, introduce indication to indicate which resources are for set B. Proposal 4-3: For CSI-ReportConfig for the configuration of inference results reporting for UE-sided model at least for BM Case-1, support Alt 1 and Alt 4. - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A Proposal 4-4: Support periodical, semi-persistent and aperiodic report for AI inference results. Proposal 4-5: For UE-sided model inference, support to configure one resource set with resources in more than one measurement time instance for configuration of set B in BM Case 2. Proposal 4-5: For UE-sided model inference, support to configure the periodicity and offset for set A, or configure the time gap between last measurement time instance of set B and predicted time instance of set A to indicate the time domain location of predicted time instance in BM Case 2. Proposal 4-5: For UE-sided model inference, support to configure one resource set with resources in more than one measurement time instance for configuration of set B in BM Case 2. Proposal 4-6: For UE-sided model inference, support to take the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report as the reference time to indicate the time domain location of predicted time instance in BM Case 2. Proposal 4-7: Support to report the measured beam information of the history measurement time instance for UE side model inference in BM Case 2 if set B equals to set A. Proposal 4-8: If the measured beam information of the last history measurement time instance is reported, support to report the predicted beam information together for UE-side model inference in BM Case 2. Proposal 4-9: For UE-side model inference in BM-case 2, indicate the time instance with the strongest L1-RSRP, and then report beams in the time instance with the strongest L1-RSRP first and other time instance with ascending order. |
| Kyocera [35] | Proposal 2: For a NW-sided AI/ML model for BM-Case1 and BM-Case2, for the configuration of inference results, RAN1 should consider the following options: Set A: No configuration for Set A is required for the UE, as the inference process is conducted transparently from the UE. The gNB should configure reporting-related parameters, including the report size and the number of beams that the UE should report. Explicit configuration of Set B is necessary, with resources for Set B explicitly defined for the UE. Proposal 2: For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where: • Set A is virtually configured. It is not intended to provide measurements to the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret. • Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model. Proposal 3: For UE-side AI/ML models, the association provides the UE with information about the beams that belong to Sets A and B. To facilitate this association, an IE is required. This IE could be defined within the CSI-ResourceConfig or within a new AI/ML specific configuration. |
| LGE | Proposal #3: Consider extending sub-configuration based Rel-18 NES mechanism for Set B beam measurement and reporting. - Different Set A and/or Set B may be associated with each sub-configuration - Different Set B patterns for a specific Set B may be associated with each sub-configuration Proposal #6: For UE-sided model, support to configure only resource set for Set B for CSI-ResourceConfig which is used for the configuration of inference results reporting (i.e., Alt 1), considering huge resource overhead reduction obtained from not configuring Set A. Proposal #7: In order to support only resource set for Set B for CSI-ResourceConfig (Alt 1) for Set A and Set B configuration, assistance information on relation/association between Set A beams and Set B beams should be provided to UE for the UE-side AI/ML model training and inference. To represent beams in Set A and/or Set B while preserving sensitive proprietary information, consider following exemplary methods: - Set A beams are represented by linear combining coefficients of Set B beams - Tx beam directions are represented as ordered numbers on a 2D or 3D coordinate Proposal #16: For UE-side AI/ML model, for BM-case 2, do not support AP CSI-RS for set B configuration for inference. Proposal #17: For UE-side AI/ML model, for BM-case 2, at least for inference, support only P CSI-RS for Set A configuration. |
| NEC | The configured P/SP NZP-CSI-RS resources for Set A should be available for other channel/signal (e.g., PDSCH) at least during model inference. The resourceType configurations of the two resource settings for Set B and Set A should be the same in model inference configuration. Support AP NZP-CSI-RS resources for BM Case2.. For BM-Case2, at least for P/SP beam report, observation window and prediction window should be configured. Furthermore, beam measurement and reporting is needed in observation window, but is not needed in prediction window. For BM-Case2, at least for P/SP beam report, the duration of the N time instance(s) that can be predicted should be a multiple of the periodicity of the P/SP beam report. For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, Option 1 and Option 3 are preferred. |
| MTK | Proposal 12: For the discussion of Set A/B configuration in CSI-ReportConfig, RAN1 makes a working assumption that Set A is assumed to be always configured with physical resources (i.e., have corresponding resource ID/resource set ID, ResourceConfig ID), for the following reasons: - Set A is required for monitoring/data collection, and can be configured with longer periodicity - Set A is required for legacy UE Proposal 13: For the design of csi-ReportConfig for UE-sided model, at least for BM Case-1, consider the following alternative for Set A/B configuration: - Alt1(Set A and Set B are in the same csi-ResourceConfigId): Set A can be configured by the existing IE csi-RS-ResourceSetList. Set B can be configured by a new IE, in the form of either a bitmap, bitmap ID, a set of RS IDs, or ResourceSet ID - Alt2(Set A and Set B are in the separate csi-ResourceConfigId(s)): Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value. Proposal 19: For UE-side model inference BM-Case2, for periodic Set B RS resources, support NW configures the periodicity of future time slot to UE if the periodicity of future time slot is not the same as the Set B periodicity. Proposal 20: For UE-side model inference BM-Case2, for aperiodic Set B RS resources, support NW configures the periodicity of future time slot to UE. Proposal 21: For UE-sided model, to save RS overhead, Set B is determined by NW from RAN1 perspective. Proposal 22: For UE side data collection, Set B information needs to be configured to UE. Proposal 23: For UE side data collection, for report and resource configurations, consider Set A and Set B resources are configured in one report configuration with separate csi-ResourceConfigId: Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value. Proposal 24: To indicate whether a report configuration is for UE side model inference or UE side data collection, NW configures different reportQuantity for each purpose: - reportQuantity = “predicted-CRI” or “predicted-CRI-RSRP”, UE measures Set B, and reports the predicted beam ID/RSRP among csi-ReportConfig::resourcesForChannelMeasurement (i.e., Set A) - reportQuantiy = “cri-RSRP” or “none” (existing values) but Set B is configured in csi-ReportConfig. UE reports as a legacy beam report, and perform data collection (i.e., model input is Set B measurements, model output is measurement of Set A in resourcesForChannelMeasurement) |
| Apple | Proposal 3-4: data collection can be initiated/triggered by configuration from NW; or is requested from UE and then may be configured by NW at NW’s discretion. |
| KT | Proposal 8. Discuss how to indicate the resource sets for Set A and Set B with the following alternatives: - Alt 1. Explicit indication indicating whether the resource set is configured for measurement. - Alt 2. Implicit indication performing the measurement for the resource set with the lowest resource set ID Proposal 9. Discuss how to associate the resource set for monitoring (Set C) with Set B and/or Set A. |
| Meta[42] | - For configuration of set A for UE sided models, support Alt-1 and Alt-4, i.e., set A is not explicitly configured using CSIReportConfig. |
| QUECTEL | Proposal 3: To ensure consistent and reliable data collection for accurate AI/ML predictions in UE-sided models, UE can autonomously trigger the collection process and report the relevant configurations Proposal 4: Regarding data collection of UE-side model, UE can request gNB for the preferred set B (pattern). while the gNB pre-configures multiple Set B patterns and selects the appropriate one to minimize RS overhead during both training and inference phases Proposal 5: Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report one CSI-ResourceConfigId is configured for both Set A and Set B. Proposal 6: Aperiodic (AP) CSI-RS can be support in BM-case2 . Proposal 7: Periodic Reporting ，semi-persistent CSI report，Aperiodic Reporting can be implemented in different scenarios in BM-case1 and BM-case2. Proposal 8: For inference reports include predicted beam IDs and L1-RSRP values for the Top-K beams, with the option to report differential RSRP values, enabling more precise and informed decisions on beam selection based on signal strength and prediction accuracy. Proposal 9: For time-based predictions in the inference report, allowing the network to anticipate beam performance over multiple time instances, particularly in high-mobility environments, to ensure seamless and adaptive beam management. Proposal 10: The existing CSI-ReportConfig structures to support AI/ML-based beam management in BM-Case1 by configuring the UE to report the top-K predicted beams from Set A,. Proposal 11: For BM-Case2, extending the CSI-ReportConfig to include time-based predictions of the top-K beams, along with time instance information and prediction confidence levels, allowing the network to anticipate beam performance over future time periods and optimize resource allocation in dynamic, high-mobility scenarios. |
| China Telecom | - Proposal 3: For UE-sided model, data collection can be triggered by the UE or initiated by network. - Proposal 4: For UE-sided model, it needs to discuss how to indicate the purpose of measurement configuration. - Proposal 8: For UE-sided model at least for BM Case-1, for inference results report, not support to configure only one resource set for Set B. Proposal 10: For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support Option 3 (i.e., Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource). |
| ETRI | Proposal 22: For BM-Case2, support the configuration of multiple Resource Sets for Set A. |
| ASUSTeK | Proposal 3: Set A and set B configured in CSI-ReportConfig for inference should be configured in same BWP Observation 3: For an AI/ML model, for BM-case2 (i.e., temporal prediction), insufficient time instance of set B will result in low confidence when generating an inference report. Proposal 5: RAN1 confirms that signalling overhead for CSI-RS (i.e., set B) reduction in temporal domain is needed for BM case-2. Proposal 6: RAN1 study whether/how to leverage monitoring pattern of set B or monitoring window to skip a subset of time instances of set B. |
| NVIDIA | Proposal 1: For BM-Case 1, introduce specification support for associating Set A of beams with Set B of beams. |
| HONOR | - For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), support both of Alt 1 and Alt 2, and Alt 2 is preferred. - Alt 1: the definition of Top-K(K≥1) beam prediction accuracy is the percentage of “the Top-1 predicted beam is one of the top K measured beams” - Alt 2: the definition of Top-K(K) beam prediction accuracy is the percentage of “the Top-1 measured beam is one of the Top-K predicted beams” |
| Sony | - : For BM-Case 2 of UE-side models, for the reference time of the earliest time instance for the predicted results, support Option 3, i.e., based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource. - : For the UE-side model in BM-Case 2, the duration D_f of each future time instance can be configured as part of the CSI reporting configuration, and at least support: - Periodic Configuration: Initial configuration via RRC signaling to define fixed D_f value. - Dynamic Adjustment: Real-time updates via L1 signaling to dynamically adjust D_f ​ based on real-time requirements. |
| RUIJIE | Proposal 3: For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support Option 3: - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource |

### Summary from contributions on associated ID

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 8: For UE sided model in beam management, confirm the following working assumption [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) |
| Huawei/HiSi | Proposal 11: For the applicable area of the associated ID for the UE-side model, study associated ID subject to cell specific manner. Proposal 13: For UE assumption of associated ID, the impact of inference configurations related to the UE capability (e.g., model input/output related parameters) could be excluded from the “property” implied by the associated ID. Proposal 13: Associated ID is provided in CSI-ReportConfig and applicable to a pair of Set A and Set B. • For model inference and training data collection, the associated ID may be configured in the CSI-reportConfig of Set B. o The paired Set A applicable to the associated ID can be identified based on the indicated linkage between Set A and Set B. • For model monitoring, there is no need to provide associated ID in the CSI-reportConfig for Set A (or monitoring resource set). Proposal 14: If associated ID is provided for data collection/inference, UE may assume the similar properties for a pair of Set A and Set B if the pair of beam sets is with the same associate ID. • No need to define the physical implication (e.g., beam angle, codebook, beam order, etc.) of the associated ID. • If the associated ID is absent, UE may assume the similar properties for a pair of Set A and Set B associated with the same cell. |
| Tejas | Proposal 1: For UE sided model in beam management, - Associated ID should be configured as part of the CSI-Report Config and mapped to the resources in Set A (predicted beams) and Set B (measured beams). Proposal 2: For UE sided model in beam management, consider the following options for mapping between Set A and Set B resource and CSI RS configuration - Option 1: One CSI-Resource Config ID is configured for both Set A and Set B. The associated ID can link the UE's resource usage in training and inference. - Option 2: Two CSI-Resource Config IDs are configured separately for Set A and Set B, with the associated ID ensuring consistency across both sets. Proposal 4: For UE sided model in beam management, to ensure consistency between model training and inference, consider the Associated ID as Associated ID = Proposal 5: For UE sided model in beam management, The DL Tx beam or beam set/list associated with the same associated ID should be consistent in terms of transmission properties (e.g., spatial domain characteristics such as beam shape). |
| Kyocera | - For UE-sided model, the associated ID can be configured for each Set A and Set B within CSI framework, - Configured within nzp-CSI-RS-ResourceSet and csi-SSB-ResourceSet - FFS: Only Set B is configured during inference, - Whether a connection between the associated IDs for the Set A and Set B pairs should be defined, or - No further specification is needed. - RAN1 should prioritize finalizing the discussion regarding what can be assumed by the UE with the same associated ID before addressing the applicability of the associated ID across different cells. - Further study whether to support the following cases for configuration of the associated ID: - Case 1: The associated ID is configured for data collection. - Case 1-0: The associated ID is configured for inference/step 3 - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID. - Case 1-1: The associated ID is not configured for inference/step 3. - UE may report the associated ID applicability. - AI/ML model selection is left up to the UE’s implementation. - Case 2: The associated ID is not configured for data collection. Either it is configured or not for inference: - Case 2-0: The UE cannot assume anything and only uses its trained AI/ML model. - Case 2-1: Depending on the UE’s capability, the UE may report that the associated ID is applicable or not. |
| Spreadtrum | Proposal 12: For inference for UE-side models, to ensure consistency between training and inference, option 1 should be considered. Proposal 2: The associated ID should be configured for data collection (training) and inference. - If the associated ID is not configured for either data collection (training) or inference, UE may report it is not applicable. Proposal 11: The associated ID should be configured in CSI-ReportConfig |
| China Telecom | Proposal 15: Confirm the following working assumption. For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s). Proposal 16: For the case that associated ID configured in CSI framework for UE-sided model, associated ID is configured per CSI report configuration. |
| ZTE | Observation 2: Associated ID based approach to align NW-side additional conditions across training and inference can be challenging due to the following concerns. • Matching ID granularity and model capability • Signaling overhead for ID exchange • Proprietary information disclosure risks Proposal 35: Confirm the working assumption that the associated ID at least can be configured within CSI framework. Proposal 36: For associated ID based consistency approaches, first clarify the scope of the associated ID before touching upon the configuration details. Proposal 37: For UE sided model in beam management, UE may assume the similar properties of a DL Tx beam set/list associated with the same associated ID. Proposal 38: There is no necessity to further define similar properties of a DL Tx beam or beam set/list regarding UE assumption upon repeatedly receiving the same associated ID. Observation 3: Performance monitoring-based approach can be useful for the alignment of NW-side additional conditions and communication environment changes with minor spec effort foreseen. Proposal 41: For the case that associated ID configured in CSI framework for UE-sided model, support associated ID to be configured per CSI report configuration. |
| vivo | Proposal 1: For inference, for UE-side model, the associated ID can be configured in CSI-reportConfig. Proposal 2: For inference, for UE-side model, the associated ID can be configured in inference parameter set prior to CSI-reportConfig to address the consistency issue. Proposal 3: For inference, for UE-side model, associated ID should be mandatory configured within both inference parameter set and CSI framework. Proposal 4: For inference, for UE-side model, pattern ID can be configured for Set B pattern to address consistency of Set B. Proposal 5: For inference, for UE-side model, similar properties of a list can be assumed that same resource IDs or through other rules (e.g. same ordering in a list) in Set A/Set B is consistent from training to inference if UE receives the same associated ID. Proposal 6: For inference, for UE-side model, an indicator should be configured within the CSI-ReportConfig to clearly specify whether following processing follows option A or option B. |
| CMCC | Proposal 25: The associated ID can be optionally configured for inference. Proposal 26: When the associated ID is configured for data collection/inference, the determination of applicable functionality is up to UE instead of depending on the matching of associated ID. Proposal 27: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is either applicable or not applicable. Proposal 28: When the associated ID is configured for data collection but not configured for inference (Case 1-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections. Proposal 29: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is either applicable or not applicable. Proposal 30: When the associated ID is not configured for data collection and not configured for inference (Case 2-1), the UE can report it is applicable and have the same assumption of gNB’s additional conditions as that for data collections. Proposal 31: As in the procedure of Option A, the associated ID can be configured within the inference configuration. Proposal 32: As in the procedure of Option B, the associated ID can be configured as part of inference related parameters and out of CSI-ReportConfig. Proposal 33: The associated ID can be configured within the configuration of data collection. Proposal 34: If associated ID is introduced within CSI framework, it can be configured per CSI-ReportConfig. Proposal 35: The content of inference related parameters at least includes: - associated ID - associated ID is optional - Set A related information (e.g. RS type, set A size) - Set B related information (e.g. RS type, set B size, association of set A and set B) - report content related information (e.g. CRI, RSRP, probabilities) - For BM-Case 2, - number and interval of time instances for measurement - number and interval of time instances for prediction |
| Ericsson | For UE-sided model, the associated ID is configured separately for Set A and Set B in the corresponding ResourceSet |
| Interdigital | Proposal 1: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework. Proposal 2: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID. |
| xiaomi | Proposal 2-1: BM Case 1 and BM Case 2 can be considered as different conditions for different functionalities. Proposal 2-2: For BM case 1, different association/mapping between beams within set B and beams within set A can be considered as different conditions for different functionalities. - Condition 1: Set A and Set B are different (Set B is NOT a subset of Set A) - Condition 2: Set B is a subset of Set A (Set A and Set B are not the same) Proposal 2-3: For BM Case 2, different repeat window can be considered as different conditions for different functionality. Proposal 2-4: For each repeat window in BM case 2, different association/mapping between beams within set B and beams within set A can be considered as different conditions for different functionalities. - Condition 1: Set A and Set B are different (Set B is NOT a subset of Set A) - Condition 2: Set B is a subset of Set A (Set A and Set B are not the same) - Condition 3: Set A and Set B are the same Proposal 2-5: Define different ranges of number of beams in set B and/ or set A as different conditions for different functionalities. Proposal 2-6: Different content of model output can be considered as different conditions for different functionalities. Proposal 2-7: Different performance metric and performance monitoring type can be considered as different conditions for different functionalities. Proposal 2-8: Confirm the WA and support to introduce associated ID within CSI framework per CSI-reportconfig, per Resourceconfig or per resource set. Proposal 2-9: Support cell group specific associated ID. Proposal 3-4: Support one associated ID for the consistency of both Set B and Set A. |
| Google | Proposal 21: Support to configure the associated ID per CSI report configuration - Support dynamic activation/deactivation of CSI report configuration |
| CATT | Proposal 1: For UE sided model in beam management, no need to define the similar properties of a DL Tx beam or beam set/list with the associated ID. Proposal 2: The associated ID can be configured within CSI framework of inference reporting. Proposal 3: For configuring the associated ID within CSI framework, support to configure associated ID at CSI-Report level. Proposal 4: For the consistency of training and inference in different cells, performance monitoring based method can be supported. |
| Fujitus | Proposal 27: - RAN1 to further discuss whether/how to apply associated ID across different cells. |
| Lenovo | Proposal 4: Support the consistency between training and inference for UE side model by combining an associated ID and performance monitoring. |
| Transsion | Proposal 11: Regarding the consistency of NW-side additional condition across training and inference for UE-side model, support to confirm the working assuption: - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) |
| OPPO | Proposal 18: To ensure consistency between Set A and Set B across training and inference, confirm the working assumption that the associated ID can be configured within CSI framework. Proposal 19: To ensure consistence across training and inference, study the UE assumption on similar property of a DL Tx beam or beam set/list. Proposal 20: For UE-side model inference, support that each associated ID corresponds to a Set B and a Set A as a triple of {associated ID, Set B, Set A}. Proposal 21: For UE-side model, to ensure the flexibility on NW-side additional condition, support to configure a group of {associated ID, Set B, Set A}. Proposal 23: For UE-side model training, support to configure an associated ID along with the configuration of Set B and Set A. |
| NEC | Proposal 6: For the consistency of NW-side additional conditions across training and inference for UE-sided model for BM-Case 1 and BM Case 2 based on associated ID, where the NW-side additional conditions may at least impact UE assumption on beams of Set A/Set B. − Associated ID shall at least indicate the site/cell specific variables used for model training like antenna configuration. − NW-side additional conditions with the same associated ID can be consistent across multiple cells. FFS for which cells NW-side additional conditions can be assumed consistent. Proposal 7: Support a reference beam pattern to ensure the consistency. |
| ITL | Proposal 20: The working assumption for the associated ID can be confirmed without other signal(s) and/or in other procedure(s)/framework(s). |
| NTU | Proposal 1: We propose the following for the FFS on “similar properties” of DL Tx beams with the same associated ID - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - The “similar properties” description is from UE CSI report generation and base station processing report perspective, i.e., base station may use DL Tx beams with different properties than the associated ID indicated, as long as base station and process the report correctly to derive the best predicted beams. Proposal 2: The proposed collaborative approach requires the reporting interface with both best beams and measured beams to enable the BS side model to correct codebook mismatch, i.e., the network is allowed to request UE reporting both the best beam based on its prediction and a subset of measured beams. |
| KT | Proposal 1. Support that NW-side additional conditions with the same associated ID are consistent for multiple cells or a cell group. Proposal 2. Support configuring one associated ID for two resource sets for Set A and Set B. Proposal 3. Support both performance monitoring-based approach and associated ID-based approach for ensuring consistency of NW-side additional conditions. Proposal 4. Discuss how to indicate the resource configurations for Set A and Set B with the following alternatives: - Alt 1. Explicit indication indicating whether the resource configuration is configured for measurement or not. - Alt 2. Implicit indication not performing the measurement for the resource configuration with the lowest CSI-ResourceConfigId. |
| Meta | The associated ID should imply that the spatial filters used of the transmission of the DL reference signals for measurement during data collection for model training are identically mapped to the reference signals for measurement/reporting during inference phase |
| CAICT | Proposal 7: Associated ID should be configured in CSI framework and per-cell based design could be considered as starting point. |
| Ruijie | Proposal 3: For the case that associated ID configured in CSI framework for UE-sided model, support Alt 2 - Alt 2: Associated ID is configured separately for Set A and Set B - Note: this means the associated ID can be different for Set A and Set B. |
| LGE | Proposal #19: Confirm the following working assumption: - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) Proposal #20: Define similar properties of a DL Tx beam set (for Set A and/or Set B) that at least the same downlink spatial domain transmission filters are maintained for each beam in different transmission instances. |
| Intel | Proposal 32: For a UE-sided AI/ML model, for consistency between training and inference, Associated ID can be assigned to set A/B configuration. • UE may assume similar properties of DL Tx beams in Sets A and/or B between training and inference configurations upon reception of the same Associated ID. • The similar properties correspond to characteristics relevant to model training for the given functionality/sub-use-case and includes at least: (i) QCL (at least QCL-D) assumptions for the beams configured in the set and (ii) their ordering within the set are consistent when the same ID is used. • To avoid excessive signalling overhead, Associated ID can be configured within CSI framework but at the level of configurations of Sets A and/or B, and NOT at the resource-level granularity. Proposal 33: For a UE-sided AI/ML model, for consistency between training and inference, as a baseline, the scope of an Associated ID is limited to within a serving cell. • FFS: Whether the assumptions can be extended to a list of cells provisioned to the UE by the network. - For a UE-sided AI/ML model, if a UE is NEITHER provided with an associated ID during training data collection NOR during inference stages, the UE may assume that at least within the cell, the network-side additional conditions are consistent, i.e., similar properties of DL Tx beams for sets A/B apply between training and inference. - For a UE-sided AI/ML model, if a UE is provided with an associated ID during training data collection but NOT during inference, the UE may NOT assume that the network-side additional conditions are consistent, i.e., similar properties of DL Tx beams for sets A/B apply, between training and inference. - Accordingly, the UE may indicate that it does not have applicable models ready for inferencing. - For a UE-sided AI/ML model, for consistency between training and inference, performance monitoring-based approaches should be deprioritized. |
| InterDigitial | Proposal 2: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework. Proposal 3: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID. |
| Qualcomm | Proposal 1: For beam prediction for UE-side AI/ML models, consider the following aspects to ensure consistency between training and inference regarding NW-side additional conditions (with regards to Set A, Set B consistency) for inference at UE - Order/indexing consistency: consistency in ordering of resources (e.g., resource index consistency) for Set B beams and Set A beams, across training and inference. - Beam shape consistency: For each Set A resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set A resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set A resource during inference should be under predefined tolerances. Similarly, for each Set B resource, the difference between pointing direction and beamwidth of the physical beam associated with that Set B resource during training compared to pointing direction and beamwidth of the physical beam associated with that same Set B resource during inference should be under predefined tolerances. Proposal 2: For UE-side beam prediction, for the consistency of NW-side additional conditions across training and inference, with regards to FFS on what can be assumed by UE with the same associated ID across training and inference: - Based on spatial Tx filter: For the same associated ID across training and inference, for each Set A resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. Similarly, for each Set B resource, UE can assume that the same spatial TX filter has been utilized by gNB, across training and inference. - Note: a certain tolerance level can be considered for the spatial TX filter used in inference versus training. - Based on QCL relationship: For the same associated ID across training and inference, for each Set A resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship. Similarly, for each Set B resource, UE can assume that the gNB Tx beams used across training and inference have a certain QCL relationship. - FFS: definition of such QCL relationships. Observation 1: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, ensuring consistency based on performance monitoring is not feasible, because of the following reasons: - Extra burden at the UE for monitoring before the functionality is activated for inference - Note that UE has to monitor the performance of a significantly large number of models to determine the best model for current network-side additional conditions. - Monitoring-based model selection needs to be performed every time network side additional condition changes. Furthermore, the network may be required to signal to the UE when network-side additional conditions change. - Will always be a hit/fail method - UE cannot determine with certainty the consistency, as without the information about the network-side additional condition, the UE may not be able to download the relevant models - Note that UE may only have limited memory to store the models. It may require frequent download/discard of the model to support monitoring-based consistency. - UE may need to monitor the performance for a significantly large amount of time before UE can determine a suitable model. This will result in a significant delay in determining applicable functionality at the UE. Proposal 3: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, deprioritize Opt2: Performance monitoring based. Proposal 4: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, UE must receive associated ID and inference configuration from NW, to determine whether it can enable inference operation for beam prediction. - FFS: details of inference configuration Proposal 5: For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, study mechanisms to ensure consistency across different cells. #### Proposal 7 For UE-side beam prediction, for the consistency of NW-side additional condition across training and inference, the NW-side additional conditions with the same associated ID should be consistent within a PLMN. #### Proposal 18 In order for UE to determine applicability of AI/ML functionality for beam prediction, associated ID must be indicated from NW to UE during both training and inference, and without associated ID determining applicability of AI/ML functionality for beam prediction is not feasible. #### Proposal 7 For UE-side beam prediction, regarding where to configure associated ID within CSI framework, support configuring associated ID within CSI-ReportConfig. - FFS: details of how to assign associated ID to configurations that include subsets of Set B and/or Set A and also different Set B patterns. - FFS: details of where to configure associated ID within CSI-ReportConfig. |
| DoCoMo | Proposal 2: Associated ID should be configured within one resource set for Set A/B, where the similar properties of Tx beam can be assumed for each RS within the corresponding resource set. Proposal 2: For only certain UE with the capability, consider performance monitoring based approach for consistency across training and inference due to UE burden brought by performance monitoring. |
| MTK | Proposal 2: For configuring the associated ID, RAN1 consider the following two directions when the associated ID is configured within the CSI-framework - AltA: associated ID is configured in CSI report configuration - AltB: associated ID is configured when NW triggers/activates the CSI reports - Note: whether/how to prevent multiple associated IDs are configured/activated at the same time Proposal 3: To save the number of configuration IDs, for configuring the associated ID within the CSI framework, RAN1 does not consider configuring the associated ID within the CSI Resource/ResourceSet/ ResourceConfig configurations. Proposal 3: Support the specification of a model switch time, which is defined as the time required for UE to switch to a new model after receiving a new associated ID Proposal 4: RAN1 makes a working assumption that each NW vendor has its own exclusive set of associated IDs, which can be managed by PLMN. Proposal 5: For associated ID, considering - First part is a PLMN-specific ID, different NW vendors may not use the same ID under one PLMN - Second part is a cell/gNB-determined ID |
| Kyocera | Proposal 13 For ensuring consistency in network-side additional conditions across training and inference phases for UE-side models in BM Case 1 and BM Case 2, where network-side conditions may impact UE assumptions on beams of Set A/Set B, the following assumptions should be considered by a UE with the same associated ID during both training and inference stages: • Coverage assumptions: o Beam properties: Beam shape, width, and side lobe levels should remain consistent. o Set Sizes: The sizes of Sets A and B should be similar. • Resource assumptions: The arrangement/indexing of resource indices should be consistent during training and inference stages. Proposal 14 To ensure consistency of NW-side additional conditions across training and inference for UE-sided models in BM-Case 1 and BM-Case 2, where these conditions may impact UE assumptions on beams of Set A/Set B, the associated ID(s) is introduced within the current CSI framework. Specifically, introduce the associated ID(s) as a new IE within the RRC messaging of nzp-CSI-RS-ResourceSet and csi-SSB-ResourceSet. Proposal 15 RAN1 should prioritize finalizing the discussion regarding what can be assumed by the UE with the same associated ID before addressing the applicability of the associated ID across different cells. Proposal 16 For a NW-side AI/ML model, to ensure consistency between training and inference, study the feasibility of the following mechanisms: • The UE reports the receive beam index to the NW. • To eliminate the UL reporting overhead, the best receive beam is assumed by the NW. The NW configures the QCL relationship, and the UE uses the receive beam identified for the QCL source for performing the measurements. |
| APPLE | Proposal 6-1: The associated ID in assisted information needs to be PLMN unique, and core network or O&M is involved in assigning/managing the associated ID. Proposal 6-2: The assistance information/associated ID (e.g., dataset ID/model ID), if assigned by higher layer is embedded as part of reference signal configuration. Proposal 6-3: the introduction of “associated ID” should provide a means for UEs to collect training data and for the UE side to train AI/ML models with reasonable effort. Beyond PLMN and PLMN-specific-ID, consider a Tag field to further reduce data collection/data collection effort. Proposal 6-4: Confirm the working assumption from RAN1 #118 regarding associated ID. Proposal 6-5: For CSI report of UE side model inference output, SSBRI/CRI based on set A configuration for training or performance monitoring is reported. The set A RS configuration with an associated ID during inference report and performance monitoring, should have QCL-type D relationship with the set A RS configuration with the same associated ID during training. |
| Panasonic | Proposal 7: Support to apply concept of “associated IDs” for multiple cells for ensuring consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. Proposal 8: Support to determine “associated IDs” within a NW operator (or a MNO) to preserve proprietary information. |
| Nokia | Proposal 7: For beam prediction use cases, considering the configuration of associated ID, - Confirm the working assumption “The associated ID at least can be configured within CSI framework” - Associated ID shall be configured separately for Set A and Set B (i.e., per SSB resource set or CSI-RS resource set) - Assigning identifiers (associated ID) shall be left to NW-implementations. |
| ETRI | Proposal 14: For the UE-sided model, the UE assumes that the same Associated ID represents a beam with the same direction. Proposal 14: For the UE-sided model, the UE can identify the connection between resources in Set A and Set B by using QCL information. Proposal 15: For the UE-side model, the Associated ID can be configured in the following alternatives: - CSI-ReportConfig - CSI-ResourceConfig - CSI-ResourceSet Proposal 16: Support categorizing Associated IDs into cell-specific areas and areas that can be used across multiple cells. Proposal 17: The NW configures the N time instances to the UE based on the UE's capability. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 7: Confirm the following working assumption. - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) Proposal 8: For UE-sided models, configure the associated ID at a configuration level, e.g., CSI-ReportConfig or CSI-ResourceConfig or CSI-ResourceSet. |
| meta | - The associated ID should imply that the spatial filters used of the transmission of the DL reference signals for measurement during data collection for model training are identically mapped to the reference signals for measurement/reporting during inference phase |
| RUIJIE | Proposal 4: For UE-sided model, the associated ID can be configured within CSI framework, CSI-ReportConfig - At least one associated ID for the consistency of both Set A and Set B is supported. - Optionally support another associated ID for Set B |
| Samsung | Proposal 16. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following: - UE to report the information on the supported/preferred associated ID. - FFS: Other information along with the report of the association ID. Proposal 17. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, support the following: - NW configuration of associated ID in a CSI-ReportConfig Proposal 18. For the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, and for the definition of similar properties of a DL Tx beam or beam set/list - At least in a cell level, the UE assumes consistent physical beams characteristics on DL Tx beams (e.g., same downlink spatial domain transmission filters) of Set A and Set B, as well as a consistent mapping, e.g., consistent order of measurement resources |

### Issue #2.1: Associated ID configuration for Set A/B RS

##### Summary of companies’ view and

##### (FL1) Proposal 2.1b (associated ID)

For UE-sided model, the associated ID can be configured within CSI framework, down select from:

- Alt 1: One associated ID is configured in CSI-ReportConfig for both Set A and Set B

  - FFS: For AP-CSI report configuration, same associated ID for different resource set

  - FFS: whether to additionally introduce pattern ID for Set B

  - Supported by: Huawei/HiSi?, Spreadtrum/UNISOC, CATT, China Telecom, CMCC, MediaTek, Qc, OPPO, Samsung, vivo, google, xiaomi

- Alt 2: the associated ID is configured separately for Set A and Set B, i.e. NZP-CSI-RS-ResourceSet, or CSI-SSB-ResourceSet

  - Supported by: Kyocera Nokia, Ericsson, ZTE, DCM, Fraunhofer

| Companies | Comments |
| --- | --- |
| FL0 | Benifit declared by Alt 2 proponents is that, UE can obtain data whenever (within or outside CSI framework) the corresponding RS set are enabled. But I think it the linage of Set A and Set B cannot be resolved so that data may not be easily collected outside of configuration for data collection (if we supported) The issue I can see for alt 2 is, if a CSI RS resourceset/ or CSI-SSB resourc set has one associated ID. When/How UE can assume the similar properties are not clear. For example, for AP CSI, the associated TCI states can be changed from time to time, it may be conflict with UE’s assumption on similar properties. If the valid time is only when associated with inference configuration or data collection configuration or monitoring configuration, why not just configure the associated ID into configuration for inference/data collection/monitoring? Therefore, the I proposed the proposals in 2.1a. If there is any other arguemnet, pls feel free to comment. |
| Huawei, HiSilicon | We prefer Alt 1. For Alt 2, for the following two cases, it is not clear whether one associated ID is assigned to Set B, or separate associated IDs are assigned to Set B: Case 1: assuming a pair of Set B (wide beams) and Set A (narrow beams) has common rotation of 90°, i.e., changing the pattern from (a) to (b), the data property of relationship between Set B and Set A does not change. Then, Set B#1 and Set B#2 should be the same associated ID or separate associated IDs? Case 2: For the same Set A {#1,#2,#3,...}, Set B#1 is one subset of Set A {#1,#4,#7,...}, while Set B#2 is another subset of Set A {#2,#5,#8,...}. Then, Set B#1 and Set B#2 should be the same associated ID or separate associated IDs? They are with the same NW side additional condition (e.g., beam codebook), but different selected beams from the same super set of Set A. |
| InterDigital | First of all. We still do not support configuration of associated ID within CSI-ReportConfig. When applicable, associated ID is already indicated to the UE during the data collection. In this case, rather indicating the associated ID, indication of data collection related configuration used for training would be a better solution and the data collection related configuration will indicate the associated ID as well. In addition, in the case of Option B for the applicability, as we already have the set of inference configurations, indication of the set of inference configurations associated with the CSI report config should be a better solution rather than just indicating the associated ID. In addition, we don’t see the need to separately configure different associated IDs for Set A and Set B. |
| NEC | Prefer Alt.1. |

##### (FL0) Proposal 2.1a (associated ID)

For UE-sided model, associated ID(s) can be configured in CSI-ReportConfig

- The associated ID can be used for the consistency of Set A or both Set A and Set B

- The associated ID-B for Set B can be additionally configured for the consistency of Set B.

- If the associated ID-B for Set B is absent, the associated ID is used for the consistency of both Set A and Set B.

- For AP-CSI report, at least different resource sets (NZP-CSI-RS-ResourceSet, CSI-SSB-ResourceSet) in one CSI resource setting (i.e., CSI-ResourceConfig) associated to one CSI report configuration are associated to the same associated ID.

| Companies | Comments |
| --- | --- |
| FL0 | Potential way forward, let’s see whether this can be lived with or not. |
| Huawei, HiSilicon | OK with the direction of Alt 1, i.e., applicable to a pair of Set A and Set B. But for the 2nd and 3rd bullet, the intention needs to be clarified. Why do we need to introduce two associated Ids for Set A only and Set A+Set B? For the 4th bullet, our understanding is, the associated ID is applicable to the actually triggered resource set (rather than all the resource sets), since data is obtained only for the actually transmitted RS set. |
| Nokia | Not OK. Wes shall first discuss data collection configuration and where this associated ID is configured. Otherwise, this is not a useful parameter to introduce in rel-19 as there may not be any consistency check that can be supported by associated ID. |
| OPPO | Thanks to FL for the analysis on this issue, we agree the associated ID should be configured for the pair of Set A and Set B. If this is acceptable to majority, we don’t have to additionally introduce the so-called associated ID-B or associated ID-A to complicate the problem. Separating Set B and Set A for data collection may bring up more issues, such as how to pair Set B and Set A measurements collected in different time instances. Additionally, for the 4th bullets, it seems we are getting into next level of details. Perhaps it will be more agreeable to stay on the same level. We understand the intention from FL to compromise for both sides, but we think the very basic of associated ID usage should be supported as simple as possible. So we would like to suggest a simple version as below. For UE-sided model, associated ID(s) can be configured in CSI-ReportConfig - The associated ID can be used for the consistency of Set A or both Set A and Set B - The associated ID-B for Set B can be additionally configured for the consistency of Set B. - If the associated ID-B for Set B is absent, the associated ID is used for the consistency of both Set A and Set B. - For AP-CSI report, at least different resource sets (NZP-CSI-RS-ResourceSet, CSI-SSB-ResourceSet) in one CSI resource setting (i.e., CSI-ResourceConfig) associated to one CSI report configuration are associated to the same associated ID. |
| Xiaomi | We prefer associated ID for both set A and set B. We aren’t clear about the meaning of the last sub-bullet. Different resource sets means set B and set A? Why only for AP-CSI report? |
| Fujitsu | We don’t see the need to introduce associated ID-B. Configuration of associated ID in CSI-ReportConfig is sufficient. For AP-CSI report, why configure different resource sets in one CSI resource setting? For inference, we agreed that Set A and Set B are configured via different resoruce setting. Another question is whether this proposal is applicable for the inference, or it is also applicable for training data collection? |
| TCL | Agree the 1st bullte with modification: The associated ID can be used for the consistency of Set A or both Set A and Set B |
| ZTE | From the first and second bullets, if associated ID and associated ID-B are configured for the consistency of Set A and Set B, respectively, why don't we just have the associated ID configured at resource set level (i.e., Alt2 in above). It seems just a naming issue. For the last bullet, the restriction is too strong, especially considering that the TCI states of the different resource sets are dynamically indicated by the trigger state. Actually, for different AP resource sets associated with potentially various TCI states as well as various number of resources, different associated IDs should be assigned. Besides, the AP related issue is due to the conflict between the semi-static configuration of CSI report/resource set by RRC and the dynamic indication of TCI states/trigger state by DCI signaling, which applies to both Alt1 and Alt2. |
| vivo | OK with FL’s proposal. |
| QC | In our view, the direction of the two alternatives identified above (Alt. 1 and Alt. 2) should be discussed and later one of them should be prioritized based on the technical reasoning. Not sure why we are debating whether associated ID should be for consistency of Set A, Set B pair or Set A only. Naturally associated ID should ensure consistency of both Set B and Set A, but whether we go with Alt. 1 or Alt. 2 should be discussed (we believe Alt. 1 is the proper way). So, in our view, let’s first discuss identifying the alternatives and then converging to the one that makes the most sense. |
| MediaTek | We agree with OPPO, we support one ID for Set A and Set B pair. |
| Google | What is the information conveyed by associated ID-B? |
| CMCC | We do not support configure associated id separately for set A and set B. As we discussed in the our contribution, the NW side additional condition is same/aligned for the set A and Set B. There is no need to configure the associated id separately for Set A and Set B. Once AP-CSI report is triggered, gNB will work under the same additional condition, thus all resource sets in one CSI report configuration are associated to the same associated ID. |
| ETRI | We are OK to first bullet. But, we need some clarification the need of associated ID-B. |
| Panasonic | We think it is sufficient to support associated ID for both Set A and Set B. For any other case, let’s put it as FFS point. |
| Spreadtrum | We assume that the associated ID is used to represent the NW additional condition, so only one associated ID is needed for both set A and set B. So the second and third sub-bullet are not needed. The last sub-bullet semms not be a part of the current associated ID discussion. Therefore, it is recommended to remove this sub-bullet. |
| LG | At least this should be discussed after the definition of “similar properties of DL Tx beam a DL Tx beam or beam set/list“ is defined. We cannot decide the granularity of associated ID without clear definition/assumption from UE side. |
| Sharp | Model training is done using both a pair of Set A and Set B. One associted ID is enought for the pair of the Set B and Set B. For inference, it is straightforward to configure the associated ID in CSI report configuration. |
| Tejas | Ok with the proposal. Associate ID can be used for both set A and set B. |
| Ericsson | We don’t support the proposal. Having the associated ID in the CSI report setting is misleading. The use of CSI report setting is to instruct the UE on what to report, for example the report quantity, reporting timeline or the frequency-granularity of the report. It would be in line with the existing framework that we put information on the resources under the resource settings. For example, already today we have some information on the NW spatial filter under resource sets. For example: NZP-CSI-RS-ResourceSet ::= ………. repetition Indicates whether repetition is on/off. If the field is set to off or if the field is absent, the UE may not assume that the NZP-CSI-RS resources within the resource set are transmitted with the same downlink spatial domain transmission filter…… … Our view is that they should be seperately configured for A and set B. |
| CATT | We think the associated ID can represent the physical properties of beams in Set A and Set B, as well as the association between Set A and Set B. Thus, only a single associated ID is required for an association of Set A and Set B. It is unnecessary to configure the associated ID at resource set or resource level. Considering that AI/ML based beam reporting and legacy beam reporting can be associated with the same CSI-ResourceConfig, and that the associated ID is not needed for legacy beam reporting, we think it is more suitable to configure associated ID at CSI-Report level. The monitivation to configure seperate association IDs for Set A and Set B are unclear. |
| Futurewei | We prefere associated ID is for both Set A and Set B. In our view, associated-ID B is not needed. |

### Issue #2.1A (on hold): whether association of Set A and Set B needs additionally explicit indication?

| Companies | Comments |
| --- | --- |
| ZTE | Proposal 17: If Set A and Set B are different, resources for Set A and resources for Set B are configured as separate resource sets, and the association between Set A and Set B can be established based on the same CSI report setting. Proposal 18: If Set B is a subset of Set A, only resources for Set A is configured, and resources for Set B is indicated as a subset of Set A based on assistance information provided by the NW, such as the mapping between Set A and Set B in the form of bitmap. |
| Huawei/HiSI | Proposal 9: For UE-side model, regarding the association for Set A and Set B, - For inference, the CSI-ResourceConfigId of Set A is indicated along with the CSI-ResourceConfigId of Set B in the CSI-ReportConfig for the inference report. - For training data collection, the association can be indicated by referring the CSI-ReportConfigId for the CSI report of Set A measurement in the CSI-ReportConfig for the CSI report of Set B measurement. - For monitoring, the association can be indicated by referring the CSI-ReportConfigId for the inference report (corresponding to Set B) in the CSI-ReportConfig for the monitoring report (corresponding to Set A). Proposal 10: For UE-side model under BM-Case 1/BM-Case 2, the mapping between Set B and Set A can be indicated for the case when Set B is a subset of Set A, e.g. based on a bitmap indication, or implicitly indicated using RS IDs. |

### Issue #2.1B (on hold): whether Set B only can be supported for inference?

Agreement(RAN 1 #118)

For UE-sided model at least for BM Case-1, for inference results report

  - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

    - FFS whether support only resource set for Set B is configured

  - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

  - The beam information in the inference report refers to the resource set for Set A

| Company | views |
| --- | --- |
| CATT | Proposal 7: For inference of UE-sided model, at least for BM-Case1, the following alternatives can be further supported for the CSI-ReportConfig of inference results reporting: - Alt 1: one CSI-ResourceConfigId is configured for Set B, and Set A can be determined from the associated id. |
| China Telecom | - Proposal 8: For UE-sided model at least for BM Case-1, for inference results report, not support to configure only one resource set for Set B. |
| CMCC | Proposal 8: Regarding training data collection for UE-side model, the configuration of multiple set B can be supported. Proposal 4: Regarding data collection of UE-side model for training, UE can report or request gNB for the preferred set B (pattern). Proposal 17: For UE-sided model, the indication of the association between Set A and Set B can be based on RS ID or bitmap. |
| Panasonic | Proposal 8: Support mapping/association of beams within Set A and beams within Set B based on QCL relationship. |

### Issue #2.2: Configuration for data collection for UE-sided model

| Company | Proposal |
| --- | --- |
| Huawei/HiSi | Proposal 7: For training data collection of a UE-side model, consider separate CSI-ReportConfigs for measurement of Set A and Set B, respectively. Proposal 8: For training data collection of a UE-side model, regarding the training data collection related parameters: - The needed number of samples for training data collection can be reported by UE. - RS configuration related information (e.g., Set A and Set B) is determined by gNB. |
| Vivo | Proposal 27: For data collection, for UE-side model, CSI-ReportConfig can used for configuring the resource for data collection purpose  reportQuantity shall be set to none  Associated ID can be configured in CSI-ReportConfig  Two CSI-ResourceConfigIds are configured for Set A and Set B separately |
| Nokia: | Observation 2: Enhancements related to UE data collection should be made taking into account the legacy beam measurement and reporting frameworks. Considerations on signalling aspects related to assistance information can be discussed under the section related to information and/or indication on NW-side additional conditions. |
| OPPO | Proposal 23: For data collection of UE-side model training, support to configure Set B (as model inputs) and Set A (as model labels) using existing CSI framework. Proposal 24: For data collection of UE-side model training, support to configure an associated ID along with the configuration of Set B and Set A. |
| Fujitsu | Proposal 5: - For training data collection, the reference signals same as Set B should be configured to obtain the model input data. And the reference signals same as Set A should be configured to obtain the ground truth data. - If Set B is subset of Set A, then only one resource set, i.e., Set A is configured. - If Set B is different from Set A, then two resource sets should be configured, one resource set for Set A and the other resource set for Set B. Proposal 6: - RAN1 to further discuss whether the same or separate reference signal configuration among training data collection, model inference and performance monitoring could be applied. - Regarding training data collection, RAN1 to further discuss the reference signal configuration for different sub-use cases. |
| Samsung | Proposal 7. For data collection of UE-side AI/ML model, support a CSI-ReportConfig with reportQuantity set to ‘none’. - the CSI-ReportConfig can be configured with one or two CSI-ResourceConfigId for measurement - E.g. For time domain predication only, one CSI-ResourceConfigId is configured; otherwise, two CSI-ResourceConfigId s are configured for Set A and Set B separately - the CSI-ReportConfig can be configured with an associated ID |
| Transsion | Proposal 4: Two CSI resource related configurations should be configured for Set A and Set B for the data collection of UE side model training. |
| Qualcomm | #### Proposal 9 For UE-sided beam prediction, use CSI-ReportConfig to configure resources for data collection purpose, with the following further details: - reportQuantity is set to none. - Associated ID is provided within CSI-ReportConfig. Two CSI-ResourceConfigIds are configured for Set A and Set B separately. |
| xiaomi | Proposal 3-7: For data collection for UE-side AI/ML model training, consider the following two configurations of resources for Set B and Set A for down-selection. - One resource set with additional indication of the number/location of resources in Set B. - Set B: the first N resources or the resources with corresponding bit value ‘1’ in a bitmap - Set A: - All resources except the resources in Set B, if the QCL source of the remaining resources are configured as the resources in Set B respectively. - Else, All resources in the resource set. - Two CSI-ResourceConfigid |
| MTK | Proposal 22: For UE side data collection, Set B information needs to be configured to UE. Proposal 23: For UE side data collection, for report and resource configurations, consider Set A and Set B resources are configured in one report configuration with separate csi-ResourceConfigId: Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value. Proposal 24: To indicate whether a report configuration is for UE side model inference or UE side data collection, NW configures different reportQuantity for each purpose: - reportQuantity = “predicted-CRI” or “predicted-CRI-RSRP”, UE measures Set B, and reports the predicted beam ID/RSRP among csi-ReportConfig::resourcesForChannelMeasurement (i.e., Set A) reportQuantiy = “cri-RSRP” or “none” (existing values) but Set B is configured in csi-ReportConfig. UE reports as a legacy beam report, and perform data collection (i.e., model input is Set B measurements, model output is measurement of Set A in resourcesForChannelMeasurement |

##### (FL0) Proposal 2.2a (data collection)

From RAN 1 perspective, for UE-sided model, CSI-ReportConfig can used for configuring the resource for data collection purpose:

- reportQuantity [can be or is] set to none.

- The associated ID can be configured in [CSI-ReportConfig configuration or in CSI-ResourceSet].

- FFS on whether the associated ID can be absent and corresponding UE assumption without the associated ID

- FFS on the following two options with potential down selection:

- Alt 1: Two CSI-ResourceConfigId can be configured for Set A and Set B separately.

- UE performs measurement on both resource sets

- FFS on the condition for UE to perform measurement on both resource sets,

  - e.g., if reportQuantity is set to none,

  - e.g., explicitly configured to UE that Set A can be measured in the CSI-ReportConfig for inference

- Alt 2: One resource set (for Set A and/or Set B) is configured

- FFS whether or not to manage the link for Set A and Set B

- FFS: how UE knows whether this is Set A or Set B

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Note: "for UE-sided model", "for data collection purpose" are not expected to be captured in RAN 1 spec.

| Companies | Y/N | Comments |
| --- | --- | --- |
| FL0 |  | Q1: whether to only set reportQuantity =none for the purpose of data collection? Q2: whether to support both Alternatives? Or down select to one, which one? Comments from FL: Since for inference configuration, Set A is not expected to be measured. We need configuraiton to enble data collection for UE-sided mode, which is not about delivery part, which is still under study by RAN 2. |
| Huawei, HiSilicon |  | Comment to Q1: We agree that UE is not supposed to report the measurement for training data collection. But the reportQuantity can consider a different notation other than legacy ‘none‘ to distinguish from Rx tuning behavior. Comment to Q2: We prefer to only keep Alt 2. Alt 2 is similar to monitoring, so how to indicate the linkage between Set A CSI report configuration and Set B CSI report configuration under monitoring can be reused. For Alt 1, it is different from inference CSI report, since both Set A and Set B are really transmitted. The CPU rule of including two CSI resource sets (with potentially different time positions and periodicities) within one CSI report needs to be discussed which may be complex – as one example, how to count the CPU occupancy duration for the two resource sets (jointly counted or separately counted, and how to handle the overlapped case)? As another example, how to handle the case if the CPU is exceeded at the start of one resource set (but not exceeded for another resource set)? |
| NTT DOCOMO |  | Q1: Instead of none, another reportQuantity dedicated for UE side data collection can be considered. If some enhancements dedicated to this data collection are introduced such as CPU calculation, a parameter indicating that this CSI reporting is for UE side data collection needs to be configured. Q2: For UE side training, UE should know which one is Set A and which one is Set B before inference reporting. To facilitate this distincion, Alt1 should be supported. Also, we think associated ID can be configured in CSI resource set instead of CSI reportConfig. This can be discussed in Issue #2.1 |
| Nokia |  | The first step is to introduce associated ID in the existing beam reporting configurations, where only one RS resource set is configured. We do not think Rel-19 can otherwise support data collection at the UE. |
| OPPO |  | A1: for UE-side data collection, it differs from legacy beam reporting, so the reportQuantity should be set to none. Though the UE-side data may go to CN or OAM considering the RAN2 solutions, i.e. Option 2 and Option 3, it comes in higher layer signaling, rather than the L1 signaling. A2: Down select Alt.1 It aligns with the RS configuration for inference. Moreover, for Alt.2, if Set B is different from Set A, how could UE measure one resource set (for either Set A or Set B) to collect both model input and label? It seems infeasible to us now. |
| Xiaomi |  | Q1: fine with ‘none‘ Q2: in order to align with configuration for inference, Alt 1 is preferred. Is it a typo, “ e.g., explicitly configured to UE that Set A can be measured in the CSI-ReportConfig for training“? and we think the condition in the first example is better. |
| Fujitsu |  | We think we can start from BM Case-1 and then further discuss BM Case-2. Regarding the two alternatives, we think both Alt 1 and Alt 2 could be supported. Alt 1 could be applicable for the case that Set A is different from Set B. Alt 2 could be applicable for the case that Set B is subset of Set A. |
| TCL |  | Q1: Agree with DOCOMO that may dedicated reportQuantity is needed for data collection. Q2: Support Alt. 1. Alt 2 causes the issues that UE confuses Set A and Set B. The extra work to solve this issue needs extensive but unnecessary. The extra work to solve this issue are extensive but unnecessary. |
| ZTE |  | Q1: No, there should be no mandatory behaviour for gNB configuration for the report quantity. Additionally, if the report quantity is allowed to be configured as none for the purpose of data collection, the CSI processing criteria (e.g., CPU occupation, timeline) as well as UE behaviour should be clarified since the setting of reportQuantity = none is only configured for P3 Rx beam sweeping in the current spec. Q2: We prefer Alt 2 to have separate CSI report configuration for Set A measurement and Set B measurement, respectively. Considering the similarity between training data collection and monitoring (i.e., both Set A and Set B need to be transmitted and measured), the monitoring report configuration should be reused here for Set A measurement and developing the linkage between Set A and Set B occasion. Otherwise, for Alt 1, the linkage between Set A and Set B occasion in the time domain should be additionally defined. |
| vivo |  | Q1: We think report quantity shall be set to none. UE shall not report its collected data to the NW as the basic principle for UE data collection. Q2: Either seems doable while we prefer Alt 1, which seems simpler. For Alt 2, if it is one set (either set A or set B), it seems we need to distinguish the report/resource configuration for set A and the one for set B in the configuration. |
| QC |  | Q1: reportQuantity =none is OK Q2: Down-select to one alternative: Alt. 1 Reasoning: It would be efficient and beneficial to have a cohesive design for data collection for training and inference. As we have already agreed to utilize CSI-ReportConfig for configuring Set B and Set A during inference the natural choice would be to also utilize CSI-ReportConfig for the purpose of configuring Set B and Set A during data collection for training. This way, we can have a unified design and have minimal specification effort to differentiate the configuration for data collection for training versus inference. To achieve this goal, similar to inference, we can configure two CSI-ResourceConfigIds for Set A and Set B separately. Another similar aspect that can be leveraged for the CSI-ReportConfig across training and inference is associated ID. The discussions are still ongoing on where to signal associated ID within the CSI framework during inference, and after we agree on where the associated ID is signalled for inference, we can leverage that design for data collection purpose as well. As mentioned earlier, we believe signalling associated ID within CSI-ReportConfig (jointly for Set A and Set B pairs) would be the most efficient way with minimal specification impact, while ensuring that the linkage between Set A and Set B resources, and also ensuring consistency of NW-side additional conditions across training and inference. So far, we talked about the similarities of configurations for data collection and inference. Now, let us look into what should be different. Let us consider the following agreement first: Agreement (RAN1 #118) For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A. As mentioned above, for inference, UE performs measurement on the resource set for Set B, and UE is not expected to measure resource set for Set A. Conversely, for the report configuration for data collection for training, UE performs measurements on the resources for both Set B and Set A. |
| Google |  | OK. |
| CMCC |  | Q1: Yes Q2: Alt1 is preferred. This is data collection for UE side. Differentiation between set A and Set B can provide more information to the UE for training and inference. Set B is determined by gNB. UE can know cell-specific set B with Alt1. |
| InterDigital |  | In our view, we need to firstly discuss CSI report config is an appropriate framework to provide data collection related configuration for training. For the two options, we don’t see the need of configuring just one resource set. Prefer to delete Alt 2. |
| ETRI |  | Q1: We are OK to ‘None’ Q2: We prefer Alt 1. It provides more clear information to UE. |
| Panasonic |  | Q1: We are open to discuss if a dedicated reportQuantity for data collection is needed. Q2: We think Alt. 1 is applicable if set A and set B are difference and set A is a subset of set B, while Alt. 2 is applicable if set A is a subset of set B. With such understanding, we are fine to support only Alt. 1 as a unified solution. |
| Spreadtrum |  | Q1: One dedicated reportQuantity should be introduced for data collection. Although UE is not required to report measurement results, for AI, the number of measurements required by UE may be much larger than the existing standard configuration, and there is no need to do Rx sweeping. So introducing new parameters is much clearer. Q2: We agree with Fujitsu. Both Alt 1 and Alt 2 can be supported. |
| LG |  | Prefer to defer the discussion after the RAN2 decision whether the data collection for UE-side model is triggered by NW or requested by UE. |
| Sharp |  | Q1: OK. Q2: Alt.1 is preferred. UE needs to measure both Set A and Set B for training data collection. |
| Tejas |  | Ok. We prefer Alt1 |
| Ericsson |  | Similar view as HW,ZTE. The data collection is very similar to the monitoring, except that the UE does not produce any inference results. Hence the data collection configuration can be similar to how the monitoring configuration. We can link to an inference config in the data collection report configuration. The data collection report configuration could be set to report quantity None, but it is not mandatory. Q1: Optional set to None. Q2: Alt 2. |
| Fraunhofer |  | Q1: reportQuantity = none is sufficient. Q2: A single alternative should be specified. In particular, we believe Alt2 has less issues regarding timing of the CSI resource sets. |
| CATT |  | Q1: Since thenetwork doesn’t set reportQuantity =none legacy beam reporting, it’s ok to only set reportQuantity =none for the purpose of data collection and it can be distinguished from the legacy beam reporting. But for not seting reportQuantity =none, the UE cann’t distinguishe the legacy beam reporting from the data collection. Thus, for data collection purpose, reportQuantity [is] set to none. Q2: Both alt1 and alt 2 can be supported. Alt.2 can be used for the caset of Set B is subset of Set A for the BM-Case 1. If we down-select to Alt 1, for the case of Set B is subset of Set A for the BM-Case 1, the UE will measure the beams of Set B for twice which is a waste of RS measurement for the UE. |
| Futurewei |  | Q1: We are ok with reportQuantity=none or defining a new quantity. Q2: We prefere Alt 2, which can reuse the legacy CSI report configuration (with one resource set) as much as possible. |

##### (FL1) Proposal 2.2b (data collection)

From RAN 1 perspective, for UE-sided model, for configuring the resource for data collection purpose, consider the following options with potential down selection:

- Option A: CSI-ReportConfig can used for configuring the resource for data collection purpose where reportQuantity is set to none.

  - Two CSI-ResourceConfigId are configured for Set A and Set B separately. And UE performs measurement on both resource sets

- Option B: Explicitly parameter to configured to UE that Set A can be measured in the CSI-ReportConfig for inference

- Option C: CSI-ReportConfig for inference linked with CSI-ReportConfig for monitoring can be used for data collection at least for when full set A is configured for monitoring

Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model.

Note: "for UE-sided model", "for data collection purpose" are not expected to be captured in RAN 1 spec.

### Issue #2.3: Configuration for BM-Case 2, Reference time and window

Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

#### Summary from Tdoc

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

  - Supported by(9): Huawei/HiSi, Nokia, CMCC, NEC, TCL, LG, Ericsson, Intel, Transsion Fujistu

- Option 2: Based on the CSI reference resource corresponding to the report

  - Supported by(5): Huawei, TCL, LG, OPPO, IDC, Tejas, ITL

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

  - Supported by(12): ZTE, xiaomi, DCM, NEC, Panasonic, CATT/CBN, Samsung, Tejas, Ruijie, Transsion, China Telecom, Sharp, MediaTek, Sony

|  | Reference RS |  | Opt 1 |
| --- | --- | --- | --- |

ZTE:

Ericsson:

Figure 1 BM-case 2 with P/SP CSI-RS resources and an aperiodic CSI report

From Figure 1, the slot offset for each option in the agreement above for the reference time could be calculated as below:

- Option 1: slot offset is equal to  since slot  is the reference time in this option.

- Option 2: slot offset is equal to  since the CSI reference resource (i.e., slot  in the figure) is the reference time in this option.

- Option 3: slot offset is equal to  since slot  in the figure is the reference time in this option.

It should be noted that one of the candidate values for  is  according to Ts 38.214.  Hence, Option 2 is a special case of Option 1.

FL’s note:  is related to UE capability, which cannot be changed. Then NW has to proper select report slot. Which gives unnecessary constrain for NW scheduling

Ofinno

For example, it is undesired that the CSI report carries a time instance (e.g., the earliest time instance) occurring before the CSI report timing, which makes the time instance become obsolete at the reception.

- If Option 2 is considered, the CSI report timing should occur before the earliest time instance of the N time instances.

##### (FL0)Proposal 2.3a (reference time for BMCase2)

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

FFS: whether only the inference result(s) of time instance(s) no earlier than the inference report is reported or not.

| Companies | Commnets |
| --- | --- |
| FL0 | In AI/ML, the time gap between Tp to T3 (the last measurement occasion) is fixed (related to the trained model) The Tp is not related to the time instance for the report. I still think Option 3 is the right way to go. |
| Huawei, HiSilicon | If we go with Option 3, the FFS part should be the necessary condition. From gNB perspective, when it schedules the CSI report, it desires to receive the predicted instances for N certain future instances, while the historic information earlier than the CSI report is not useful. As shown in the following figure, assuming gNB schedules N=1 predicted instance @80ms so that the latest Set B is @40ms; for Case 2, if the first predicted instance #1 (@80ms, i.e., with ) as the model output is earlier than the CSI report, then it needs to report the second predicted instance #2 (@120ms, i.e.,) to gNB; otherwise gNB has no information to perform the scheduling of data @120ms. If we go with Option 1, we do not think it should impose restriction on gNB scheduling – the first predicted instance has fixed time distance to the CSI report instance, and flexible time distance to the latest Set B. Change: - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource FFS: whether only the inference result(s) of N predicted time instance(s) no earlier than the inference report is are reported or not. |
| NTT DOCOMO | Support. |
| Nokia | It is unclear why Option 3 is the right way to go. CSI prediction in Rel-18 already consider Option 1. The UE model assumed at the UE is irrelevant as the UE can always map the output to respective timelines set by the spec/configuration. |
| OPPO | We understand the motivation to propose Option 3 as reference time in order to make the same time gap among measurement instances and prediction instances. That could somehow facilitate data collection for both input and label. However, if it comes to NW-side data collection, the time budget introduced by CSI reference resource and CSI reporting can be considered as well. The model can be trained with this particular time gap, e.g. or . To align with legacy, we would support either Option 1 or Option 2, as companies mentioned that Option 2 could be a special case of Option 1. |
| Xiaomi | Support |
| Fujitsu | Slightly prefer with Option 1 but we could be open. |
| TCL | Agree. |
| ZTE | Support |
| vivo | OK |
| QC | OK |
| MediaTek | Support |
| Google | Support option 1 |
| CMCC | Similar view as oppo and Nokia, we still prefer option 1. For option 3, the latest transmission occasion of set B for measurement is known by UE but unknown to gNB. gNB actually transmit the RS for all the UEs and will not know which transmission occasion is related to which specific UE. For option 3, UE still needs to find the latest transmission occasion for the reporting and then determine which DL RS transmission occasion is the reference time. gNB and UE may have ambiguity in the understanding of time gap between reference time and earliest predicted time instance. |
| Panasonic | Support. |
| Spreadtrum | We prefer option 1. |
| Sharp | Support. |
| Ericsson | We prefer option 1. |
| CATT | Support. For Option 3, the reference time is the latest CSI-RS/SSB occasion of Set B for inference no later than the CSI report, and the candidate values of time offset can be some fixed values, such as 40 ms or 80 ms, which are subject to the UE-sided AI/ML model. If Option 1 or Option 2 is supported, to ensure the flexibility of configuration/activated/triggered, the number of candidate values for slot offset should be a large set of multiple values for different SCSs. |
| NEC | OK. We are also OK with Option 1. |
| KDDI | Support. |
| Futurewei | Our preference is Option 1 but we are open to Option 3 for progress. |

##### (FL1)Proposal 2.3b (reference time for BMCase2)

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

FFS: whether only the inference result(s) of time instance(s) no earlier than the inference report is reported or not,

### Issue #2.4(on hold): Configuration for BM-Case 2, configuration of time information of inference results

Agreement (RAN 1 #118) For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

| Company | Proposals |
| --- | --- |
| CATT | Proposal 12: For UE-sided model for BM-Case2, the UE is expected to report the predicted results for slot intervals, for, where is the earliest predicted time instance, is the number of the consecutive predicted slot interval(s) and is duration of each slot interval. Proposal 13: For UE-sided model for BM-Case2, for the duration values of the slot interval(s) that can be predicted support at least 320ms /480ms/640ms. |
| Nokia | 40ms, 80ms, 160ms, 320ms, 640ms |
| Samsung | Proposal 13. For BM-Case 2 of UE-side model, for the determination of N future time instance(s), support the following: - Each of the N time instance(s) is consist of P (P≥1) consecutive slots - An offset between the reference time slot and the first slot of the earliest time instance is D1 - For N>1, the offset between the first slots of two adjacent time instances is D2 - FFS: the determination and the values for P , D1 and D2 |

### Issue #2.5: Configuration for BM-Case 2, support of AP CSI-RS

Agreement For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS

| Company | Proposals |
| --- | --- |
| ZTE | For temporal beam prediction at the NW/UE side, study enhanced resource indication method to enable efficient resource activation in the measurement window and deactivation in the prediction window. |
| CATT | Proposal 8: For UE-sided AI/ML model inference, for BM-Case2, AP CSI-RS can be supported for CMR configuration at least for Set B. Proposal 9: For UE-sided AI/ML model inference of BM-Case2, for periodic or semi-static CSI-RS configuration, one RS set can be configured as CMR for the inference report and the separation between two consecutive measurements is the periodicity of RS set. Proposal 10: For UE-sided AI/ML model inference of BM-Case2, for aperiodic CSI-RS configuration, consider two potential options as follows: - Option 1: One resource set with the resource of Set B, the transmission number of the RS set, and the separation between two consecutive transmissions can be configured for one triggering instance. Multiple transmissions of the resource set are triggered by one triggering instance. - Option 2: Multiple resource sets can be configured for one trigger state and each resource set can be configured with trigger offset separately. Multiple resource sets with different slot offsets are triggered by one triggering instance. |
| CMCC | Proposal 12: For BM-Case 2 of NW-side model, for inference, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS |

##### (FL0) Proposal 2.5a (support of AP CSI-RS)

For UE-side AI/ML model, for BM-Case 2, at least for inference, for Set B, aperiodic (AP) CSI-RS is supported at least for the case with 1 time instance for measurements of Set B of beams.

- FFS whether/how to support BM-Case2 with multiple time instances for measurement of Set B of beams.

| Companies | YN | Commnets |
| --- | --- | --- |
| FL0 |  | In SI phase, companies reported reuslts with 1 time instance to predict future results. And the performance is good. The issue is not about AP CSI-RS itself, but because how to obtain RSs from multiple time instances as inputs for AI. Let’s see whether some easy case can be agreed first. |
| Huawei, HiSilicon |  | For AP-CSI, yes, the size of observation window could be UE capability and RRC configuration, e.g., when either prediction window size > 1 or observation window size > 1, it is subject to BM-Case 2; when prediction window size = 1 and Observation window size = 1, it falls back to BM-Case 1. Thus, prediction window size > 1 and observation window size = 1 could be a specific combination of RRC parameters. However, I doubt it is worth spending effort on this specific combination, given that A-CSI report with P/SP-CSI-RS works well. |
| Nokia |  | OK. But, agree with HW that this goes together with how we differentiate BM-case1 and BM-case2 with 1 prediction. |
| OPPO |  | Procedure-wise, if the triggering of multiple instances of Set B measurements can be addressed perfectly, then we are okay to support the AP CSI-RS as CMR. But now if only one measurement instance of Set B is supported, we don’t know whether it could be outperformed by P/SP CSI-RS with multiple time instances measurement as Set B. In other word, we should address the FFS first and then come back to support the AP CSI-RS for BM-Case2 as a whole solution. |
| Xiaomi |  | We share same view as OPPO. It is better to solve the problem in the case of multiple history time instance first for BM Case 2. If it can’t be solved, the benefit for only one history time instance may be not essential. |
| Fujitsu |  | Generally fine with the proposal. |
| TCL |  | Agree. |
| QC |  | The meaning and necessity of the proposal is unclear to us. Further elaboration is needed. |
| MediaTek |  | We are OK with this proposal |
| Google |  | OK. But the AP CSI-RS should be AP CSI-RS for BM. |
| CMCC |  | Support to solve the problem of multiple history time instance first for BM Case 2. |
| Panasonic |  | Fine with the proposal. |
| Spresdtrum |  | We do not think that the case of 1 time instance for measurements of Set B should be discussed separately. If the 1 time instance for measurements of Set B supports AP CSI-RS, multiple time instances for measurement of Set B do not support AP CSI-RS, which adds more complexity to the configuration. Therefore, whether BM-Case 2 supports AP CSI-RS should be discussed entirely. |
| LG |  | Similar view as OPPO. |
| Sharp |  | OK to support aperiodic CSI-RS for BM case 2. |
| Ericsson |  | I guess this could technically work if the UE combines its measurement with movement infomration. However, it is not clear if this is a relevant scenario. |
| CATT |  | We agree with that if the measurement duration is long, it may affect the UL scheduling since the PUSCH out-of-order issue should be avoided. However, if the measurement duration is not long, the impact for UL scheduling will be minimal. In our view, AP CSI-RS can be supported for BM-Case2 with UE capability. It’s too restrictive that only supporting the BM-Case 2 with 1 time instance for measurement. |
| NEC |  | We are supportive to have AP trigger. And for multiple time instances case, it could work if the AP trigger can be used to trigger a burst of CSI-RS. |
| Futurewei |  | This proposal may be considered as low priority. |

# Inference result report for UE-sided model

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction. Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Agreement For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances FFS details

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 3: For Rel-19 AI/ML-based BM, for UE-sided model, at least for BM-Case1, for content in the report of inference results, do NOT support Opt 3 and Opt 4. - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP Proposal 4: For Rel-19 AI/ML-based BM, for report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, support Option B - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. |
| ZTE | Proposal 19: For UE-sided model, at least for BM-Case1, for content in the report of inference results, support the reporting of RSRP in Option 2 and probability information in Option 3 for beam selection at the NW side. • The predicted Top-K beams are the K beams with the highest predicted RSRP (or probability information) • The beam information is CRI or SSBRI for BM-Case1, FFS for BM-Case2 • The reported RSRP is predicted RSRP, if the beam is not configured for corresponding measurement, and measured RSRP if the beam is configured for corresponding measurement, when applicable Proposal 20: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, if both the predicted RSRP and measured RSRP to the same beam are available at the UE side, the measured RSRP should be reported due to its higher reliability. Proposal 21: For content in the report of inference results, at least for Opt 1: Beam information on predicted Top K beam(s) among a set of beams, the ranking information of Top K beams can be conveyed by reporting Top-K beam IDs in a descending order according to the predicted RSRP (or probability information) values. |
| Huawei/HiSi | Proposal 18: For the inference of a UE-side AI/ML model, if the inference output is predicted RSRP, the type of the predicted RSRP should be defined from at least one of the following: - Absolute RSRP, where the predicted RSRP from Set A beams is subject to dBm value. - Normalized RSRP, where the predicted RSRP from Set A beams is subject to normalized value. Proposal 20: For the inference of a UE-side model, for the content in the report of inference results, support Opt 3: Beam information on predicted Top K beam(s) and the corresponding prediction probability information of the predicted Top K beam(s). - The ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information based on the prediction probability. - Support differential report for the prediction probability across K>1 beams. Proposal 21: For AI/ML model at the UE-side under BM-Case 1 and BM-Case 2, RAN1 to investigate the priority rule of AI/ML-based CSI reporting types/quantities subject to different LCM procedures. E.g., differentiate the CSI report priority over the CSI types subject to different LCM procedures (e.g., inference, monitoring). |
| Spreadtrum, UNISOC | Proposal 7: For BM-Case1 and BM-Case2 with a network-side AI/ML model, larger quantization step(s) should not be considered at least for model inference. Proposal 8: For BM-Case2, implicit report of time information should be supported. Proposal 9: For BM-Case2, support option 1 for the reference time of the earliest time instance for the predicted results. Proposal 10: For BM-Case2 with a UE-side AI/ML model, the number of reported beams can adaptively selected by UE for each N instance |
| CMCC | Proposal 29: For UE-sided model of BM-Case 1, for the content of inference results, both Option A and Option B can be supported. Either Option A or Option B is used can be configured by gNB. - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement Proposal 30: For Option B, whether/how to differentiate measured L1-RSRP and predicted L1-RSRP needs further discussion. Proposal 31: For BM-Case 2, regarding differential RSRP reporting over multiple time instances, Option 1 is preferred. - Option 1: Beam information includes CRI of top-K predicted beams per instance, and index of beam with largest RSRP over multiple time instances. - Option 2: Beam information includes the union set of CRI of top-K predicted beams over multiple instances, the indication of the instance that a specific top-K predicted beam belongs to, and the index of beam with largest RSRP over multiple time instances. |
| vivo | Proposal 38: For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams. For inference, for UE-side model, support time stamp information as beam content for BM-Case2. For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions. For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2. For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report. For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report. For inference, for UE-side model, support one reference value for differential quantization across multiple time instances, where reference value also needs to be determined through TRI and the beam index indicator indicated from a unique beam list. |
| Qualcomm | #### Proposal 10 For UE-side beam prediction, for content in the report of inference results, regarding FFS on potential down-selection among Option 3 and Option 4, support Option 4. |
| DoCoMo | Proposal 6: Support either one of the following beam representation for BM-Case 2. ・Always CRI/SSBRI ・Bitmap or CRI/SSBRI that achieves smaller overhead according to the number of reported beams, the number of measured beams, and the number of reported future time instances where specification should not support both alternatives, and separate UE capabilities for bitmap or CRI/SSBRI should not be introduced. Proposal 7: Reported time instance should be determined based on the parameters indicating the following for beam prediction. ・interval between two future time instances. ・offset between the last referred transmission occasion (Opt3) and the first future time instance. |
| Ofinno | - For contents in the report of inference results, consider enhancing Opt.1 and Opt. 2 to allow informing prediction quality (e.g., probability information, confidence information). - Specify RRC configuration of a threshold for prediction quality and discuss report options based on the threshold. - In terms of reporting, consider options (1) to include predicted beams, satisfying the threshold, from top K predicted beams; (2) to indicate whether each predicted beam satisfies the threshold. - Support multiple duration values for BM-Case 2 inference report: - UE select one value among the multiple duration values and inform the value in the report. - Specify UE-initiated inference results report mechanism, to override previously reported inference results. |
| Apple | Proposal 4-1: to control feedback overhead, beam reporting for BM Case-1 consists of - Indication of the strongest beam index - The strongest beam’s RSRP - Bitmap to indicate un-omitted beams - Differential RSRPs for uno-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-2: to control feedback overhead and quantization error for NW-side model, set B beam reporting for BM Case-2 consists of - Indication of the strongest beam index among all measurement occasions - Bitmap to indicate un-omitted/omitted beams - Alt. 1: bitmap size equals to the number of set B beams across measurement occasions - Alt. 2: bitmap size equals to the number of set B beams at a single measurement occasion - The strongest beam’s RSRP - Differential RSRPs for un-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-2A: to control feedback overhead and quantization error for UE-side model, set A beam reporting for BM Case-2 consists of - Indication of the strongest beam index among all prediction occasions - Bitmap to indicate un-omitted/omitted beams - Alt. 1: bitmap size equals to the number of set B beams across prediction occasions - Alt. 2: bitmap size equals to the number of set B beams at a single prediction occasion - The strongest beam’s RSRP - Differential RSRPs for un-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-3: to control feedback overhead for the UE-side model, beam reporting for BM Case-2 consists of - Indicating a subset containing top beams across time instances - Indicating a bitmap of selected top beams at time instances, the bitmap is over the cardinality of the subset by the number of future time instances. |
| CATT, CBN | Proposal 13: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported beam information can be the RS indicator(s) (e.g., legacy CRI/SSBRI) or the pre-defined beam index of the predicted Top K beam(s). Proposal 14: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported RSRP type of predicted Top K beam(s) can be configured by gNB with the following options: - Option A: Predicted L1-RSRP; - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. Proposal 15：For UE-sided mode, at least for BM-Case1, for content in the report of inference results, for Opt 1(only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K>1 can be conveyed by the order of the beam information based on the predicted probability information or the predicted L1-RSRP. Proposal 16: For UE-sided model for BM-Case2, for inference report, support to report largest L1-RSRP among multiple beams over multiple time instances and other differential L1-RSRP of multiple beams over multiple time instances with a reference to the largest L1-RSRP in a pre-defined order. |
| Ericsson | For UE-sided model inference, regarding the FFS on potential down selection of option 3 and 4, - support option 3: probability related information of predicted Top K beam(s) - support option 4, the confidence is for example a prediction interval where the UE prediction with a certain probability resides within (e.g. with 90% confidence, the RSRP prediction is within interval [RSRP1, RSRP2] ) For UE-sided model inference, support that value of K could be adaptive and based on the UE-sided model output (e.g. support that UE report K beams so the probability of one of them being strongest is above a certain threshold) For UE-side AI/ML model inference, for BM-Case2, support that UE can update reported inference results of N future time instances after such report. |
| Nokia | Proposal 1: For BM-Case1, consider the following for a CSI report that enables beam prediction at the UE, - For the supported Opt.1 and Opt.2, - K = 1, 2, and 4. K is configurable to the UE. - RSRP of predicted Top K beam(s) reported similar to legacy L1-RSRP reporting. - For the case of Set B is subset of Set A, RSRP of predicted Top K beam(s) can be corresponding to the measured or predicted L1-RSRP, and NW can determine it based on corresponding CRI. - Support Opt. 3, where reporting content shall be “Top-K Predicted-CRIs, probability info” corresponding to a Set A. - K = 1, 2, and 4. K is configurable to the UE. - Beam information shall refer to CRIs corresponding to Set A - Probability information shall be the probability of the beam to be the Top 1. - FFS: how to report probability information (e.g., quantization method of probability information of predicted Top K beam(s)). Proposal 2: For BM-Case2, for inference results report, support - N = 1, 2, and 4 - Duration values of the N time instance(s) that can be predicted = 40ms, 80ms, 160ms, 320ms, 640ms - Support Option 1 where reference time is the slot carrying the inference report - Support AP-CSI reporting with AP CSI-RS (for Set B) - FFS: Other overhead reduction options to apply when K and N values are large. |
| Kyocera | Proposal 2: For a UE-side AI/ML model, support configuring Sets A and B for inference results reporting, where: • When Set B is different from Set A, o Set B is explicitly configured for measurements for the UE. It enables the UE to perform measurements that derive the UE’s AI/ML model. o Set A can be optionally virtually configured. It is not intended to provide measurements to the UE. Instead, it serves as a reference to map the UE-side AI/ML model output, in the case of classification models, to the corresponding CRIs that the gNB can interpret. • When Set B is a subset of or equal to Set A, it might be sufficient for the gNB to only configure Set A. o FFS: Determination of Set B out of Set A. • When Set B is equal to Set A, there is no need to configure Set B and it is sufficient to only configure Set A. - For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 3. Further study the benefits and gains of adopting option 4, considering the effects of quantization methods on system throughput and considering the additional overhead incurred. - Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition: - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model. - FFS on the details of the calculation of the confidence interval - For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set. - For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation. - For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the top-K beams out of N beams in Set A, the following methods can be adopted: - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the entry corresponds to the strongest beam. The strongest beam can be determined by the beam information. - Bitmap Reporting: The UE should report the bitmap K times, with each instance corresponding to a different beam. For example, the UE would report a matrix with K rows and N columns, where the row contains zeros except for the entry corresponding to the strongest beam. - FFS: The feasibility of the bitmap compared to the CRI vector signaling with respect to the associated UL overhead. |
|  | Proposal 11: Support an overhead reduction mechanism for reporting data collection for inference (e.g., based on channel conditions). Proposal 12: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. |
| MTK | Proposal 31: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consider UE to report various number of Top-K beams in one beam report (K determined by UE, K ≤ nrofReportedRS). |
| NEC | Support selecting Top-K beam(s) according to some pre-defined rules (e.g., a sum probability of being Top 1 or Top K beam higher than a threshold, predicted L1-RSRP higher than a threshold) as the reported predicted beams. For BM-Case1 and BM-Case2, the value of K (i.e., the number of predicted beams to report) should be determined and provided to NW by UE. For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, Option B should be supported, i.e., Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement |
| Spreadtrum | Proposal 12: For UE-sided model, at least for BM-Case1, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information. • How to obtain the ranking information belongs to UE implementation. |
| Interdigital | Proposal 18: Support both ‘Option A: Predicted RSRP’ ‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’ Additionally, support a UE based selection procedure to report according to Option A or Option B. Proposal 14: Information about the time stamp for measurement instances should be supported. Proposal 15: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline. |
| China Telecom | - Proposal 9: For UE-sided model, at least for BM-Case1, the beam information in inference result report is the CRI/SSBRI of Top K beams with the highest predicted RSRP above a certain probability of resource in Set A. |
|  | Proposal 4: Support to introduce a beam subset configuration indicating a subset of beams that the UE should always report in an L1-RSRP report - Support to define one L1-RSRP state to indicate the reported beam is in an invalid state (UE is unable to measure the L1-RSRP that can meet the L1-RSRP measurement requirement) Proposal 5: Support L1-RSRP report retransmission to facilitate the NW-side beam prediction for BM case 2 Proposal 6: For beam report based on UE model inference for SD beam prediction, support the followings on the remaining open issues: - The selection of the “top-K” beams are up to UE implementation - Spec only defines the number of reported beams - The reported RSRP should be defined as the predicted RSRP based on a reference transmission power - Support both option 3 (probability information report) and option 4 (confidence information report) - For BM case 2, the differential RSRP is calculated based on the reported absolute RSRP |
| HONOR | - For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams where the set of beams is Set A, i.e., the beams for UE prediction. - For Rel-19 AI/ML-based BM, for report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, support Option B. - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. |
| xiaomi | Proposal 4-1: For BM-Case 1, support to report the predicted L1-RSRP if the beam is not configured for corresponding measurement, and report the measured L1-RSRP if the beam is configured for corresponding measurement for UE-side model inference. Proposal 4-6: For UE-side model inference in BM-Case 2, indicate the time instance with the strongest L1-RSRP, and then report beams in the time instance with the strongest L1-RSRP first and other time instance with ascending order. |
| Panasonic | Proposal 7: To differentiate between prediction and measurement results, the following options can be considered: - Option 1: To introduce prediction-related metrics as the reporting quantities. - Option 2: To introduce different resource sets in a report configuration for prediction and measurement. |
| ETRI | Proposal 18: For the UE-side model, support the CSI report format for temporal domain beam prediction, including the optimal K beam information along with RSRP information from multiple time instances. Proposal 19: For the UE-sided model, support the following differential L1-RSRP methods among multiple beams over multiple time instances. - Relative to a single absolute RSRP value at each time instance. - Relative to the absolute RSRP values at the first time instance. |
| ITL | Proposal 16: For UE-sided model, at least for BM-Case1, it is not necessary to additionally support probability information of predicted Top K beams or confidence information of the RSRP Proposal 17: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, it is proposed to support both Option A and Option B. Proposal 21: For UE-side model, it can be supported to use beam indication for Set A beams. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 13: At least for BM-Case 1, for UE-side models, adopt a ranking of beam information in the inference report based on the Top-K outputs from the AI/ML model. |
| LGE | Proposal #8: For supported Option 1 and Option 2, support K=4 for the max value. - Considering the case that predicted beam is in Set A but not in Set B, reported beam information can be based on the relation/association between Set A beams and Set B beams Proposal #9: For predicted RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported RSRP. Further study whether the information is per model/functionality, per report, per time instance, or per report parameter. Proposal #10: Support to report inference results of N(N>=1) future time instance(s) in one report. - Maximum value of N can be more than 1 - Maximum value of N can be reported by UE capability, and M(M<=N) value for inference results reporting can be configured by gNB Proposal #11: For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances. - In the inference result report, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator is reported by UE - Other than largest predicted RSRP value across every multiple time instances, predicted RSRP value for each beam is expressed by differential RSRP value (e.g., 4 bit) Proposal #12: Regarding the reference time to determine the earliest time instance for the predicted results in BM-case 2, support to re-use Rel-18 MIMO CSI prediction framework. - The reference time is based on the uplink slot for the report and slot offset value can be configured by NW to determine the earliest time instance, wherein candidate value of slot offset also includes the value to indicate CSI reference resource, e.g., Proposal #13: For temporal DL Tx beam prediction, information on time-variation of RSRP can also be included in the report. Proposal #14: For temporal DL Tx beam prediction with UE-sided models, following beam reporting enhancements can be considered: - Report of beam dwelling time for each future time instance - Report of beam(s) for current time instance for fallback operation Proposal #15: Support Option A or Option C (new) for the RSRP of predicted Top K beam(s) in the report of inference results. - Option A: Predicted RSRP - Option C: Not specify whether to report predicted RSRP or measured L1-RSRP when both RSRPs are available at UE side, i.e., leave this case up to UE implementation |
| Sony | - : For the UE-side model, regarding the reporting of inference content, support beam information for the predicted Top K beams among a set of beams, along with probability-related information for these beams. |
| Fujitsu | - For BM Case-2 with UE side model, the beam with the largest RSRP value should be indicated in the differential RSRP reporting over multiple time instances. |
| Lenovo | - For UCI overhead reduction on inference result reporting for BM-case2, to support that - differential RSRP quantification with reference to the largest value among all future time instances - reporting unique beams of future time instance and corresponding time-stamp indicator. - an indication for informing whether to report all beams or unique beams. |
| Sharp | - For UE-side model, at least for BM-Case 1, for content in the report of inference results, support - Opt 3: beam information on predicted Top K beams among a set of beams and probability related information of predicted Top K beam(s) among a set of beams. - The quantization step size for the probability information can be configurable by the gNB. - For UE-side model, for the report of inference results, support a single L1-RSRP corresponding to largest predicted value across multiple time instances, with the remaining predicated RSRP being reported as differential RSRPs. - Introduce a field to indicate which time instance contains the L1-RSRP. - Differential RSRPs are mapped according to timing order of the future time instances. |

### Issue #3.1: Inference result report for BM-Case 1

##### (FL0)Proposal 3.1a:

For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource set for Set A.

For UE-sided model, at least for BM-Case1, for content in the report of inference results, for Opt 1 (only beam information of predicted Top K beam(s)), the ranking information of the predicted Top K beams for K > 1 is conveyed by the order of the beam information. Wherein the ranking information is based on model output (e.g., probability of each beam in Set A to be the Top-1 beam, predicted RSRPs).

| Companies | Y/N | Commnets |
| --- | --- | --- |
| FL0 |  |  |
| Huawei, HiSilicon |  | If the UE supports reporting the beam probability information or predicted RSRP information along with the beam ID(s) (though gNB configures UE to only report beam ID as Opt 1), then the ranking should be based on the ranking of beam probability or predicted RSRP, respectively. Changes: Wherein the ranking information is based on supported reporting content model output (e.g., probability of each beam in Set A to be the Top-1 beam, predicted RSRPs). |
| OPPO |  | First, for the 1st part, it is straight forward to reuse CRI/SSBRI to represent beam information in inference report. Secondly, we would like to echo Huawei’s comment that the probability information could be kept to provide more information. Otherwise, the ranking would be done by UE with its implementation, which could be somehow less trustworthy by NW. |
| Xiaomi |  | Support |
| Fujitsu |  | The first paragraph is not needed. We already agreed on this in Nov meeting. |
| TCL |  | Agree |
| ZTE |  | Support. The ranking information of Top K beams ordered based on the corresponding predicted RSRP or probability values can facilitate beam selection at the NW side without additional reporting overhead. Note that the first sentence is an agreement made in the last meeting. |
| vivo |  | Ok with the first bullet. On the second bullet: We have discussed this for multiple meetings. What we observe from previous meetings includes - Companies cannot have aligned view on whether there should be definition on the metric to determine the order. At least from our understanding, we don’t agree with Huawei or OPPO that there should be any metric to be defined. How to determine the order is definitely UE implementation, which is highly dependent on the model UE would use. We cannot agree to hardcode UE implementation in the RAN1 standard. - Companies are not aligned on whether the order is important or useful in NW. Hence we don’t know whether we should keep discussing this repeatedly. |
| QC |  | Share similar concerns as mentioned by VIVO |
| MediaTek |  | We agree with VIVO and QC. There may be concerns on UE implementation and whether this information is useful to NW is still not clear. |
| Google |  | OK |
| CMCC |  | The ranking is up to UE implementation. If gNB configures CSI report with report quantity of Top K beam info, gNB trusts the predicted Top K beams of AI model has high accuracy, otherwise gNB can configure report quantity of Top 1 beam info or configure Top K sweeping to find Top 1 beam. |
| Panasonic |  | It seems 2 bullet points are talking 2 difference things. It is better to discuss them separately. - For 1st bullet, we are okay to support. - For 2nd bullet, we can discuss it further as companies commented. |
| Spreadtrum |  | Support |
| LG | Y | First part was already agreed, right? Agreement - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A. Second part is fine for us. |
| Sharp | Y |  |
| Ericsson |  | Fine with the proposal. |
| Fraunhofer |  | In general fine. In particular, the ranking can be based on the multiple Top-1 to Top-K model outputs, which already inherently define an order without referring to any ordering information. |
| CATT |  | We are ok with FL’s proposal. |
| NEC |  | OK with the proporal. And we support Huawei’s changes. |
| Futurewei |  | We are in general fine with the proposal. |

##### \(onhold)Proposal 3.xa:

| For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability related information of predicted Top K beam(s) among a set of beams - Where the probability information is the probability of the beam to be the Top 1 or Top K beam - X bits are used to equally quantize the probability information with a range - FFS on X and the range |  |
| --- | --- |
| Companies | Commnets |
| FL0 |  |
| Xiaomi | We are not sure about the benifit compared to report beam informations with ranking information. |
| ZTE | General fine. |
| Nokia | OK |
| OPPO | Support. |
| New H3C | OK |
| MediaTek | Agree with Xiaomi. Also, if probability-related info is reported, are this information testable? |
| SPRD | We don't know what gain will have if NW get the probability information. |
| TCL | We are open to discuss this, however the probability may not be equally quantized for overhead reduction purpose. |
| vivo | We don’t support to report probability. Given we already supports two options for UE side model inference report, we don’t see the need to have another one. Whether this probability information provides any guidance for NW is also not clear. |
| Sony | Support. |
| Ericsson | Support |
| Sharp | Support. |
| Panasonic | As raised by other companies, we do not see a benefit to report probability related information. |
| Lenovo | Fine |
| Fujitsu | Not support |
| HONOR | Support. |
| Google | Support |
| CATT | We think report the information implicitly （with ranking information）is enough, no need to report the probability. |
| QC | Not support |
| NEC | Support. |

### Issue #3.2: Inference result report for BM-Case 2

| Company | proposals |
| --- | --- |
| ZTE | Proposal 22: For BM-Case2, for the quantization of a RSRP value at least for the report of inference results, support that the largest RSRP of each time instance is selected as the reference RSRP and differential RSRPs of the same instance are computed relative to this reference. |
| CMCC | Proposal 23: For BM-Case 2, regarding differential RSRP reporting over multiple time instances, Option 1 is preferred. - Option 1: Beam information includes CRI of top-K predicted beams per instance, and index of beam with largest RSRP over multiple time instances. - Option 2: Beam information includes the union set of CRI of top-K predicted beams over multiple instances, the indication of the instance that a specific top-K predicted beam belongs to, and the index of beam with largest RSRP over multiple time instances. |
| Samsung | Proposal 14. For UE-side AI/ML model inference and BM-Case2, support the following: - The differential L1-RSRP value is with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. FFS: How to identify the largest measured L1-RSRP value if there are multiple time instance. |
| OPPO | - For inference of BM-Case2 with UE-side model, support to enhance the differential RSRP reporting - UE packs the predicted beams and RSRPs from the 1st to the N-th (last) time instances and reports in a single beam reporting instance - For each instance, differential RSRPs refer to the highest RSRP in the time instance |
| xiaomi | Proposal 4-6: For UE-side model inference in BM-Case 2, indicate the time instance with the strongest L1-RSRP, and then report beams in the time instance with the strongest L1-RSRP first and other time instance with ascending order. |
| LGE | Proposal #11: For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances. - In the inference result report, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator is reported by UE - Other than largest predicted RSRP value across every multiple time instances, predicted RSRP value for each beam is expressed by differential RSRP value (e.g., 4 bit) |
| ETRI | Proposal 19: For the UE-sided model, support the following differential L1-RSRP methods among multiple beams over multiple time instances. - Relative to a single absolute RSRP value at each time instance. - Relative to the absolute RSRP values at the first time instance. |
| Sharp | - For UE-side model, for the report of inference results, support a single L1-RSRP corresponding to largest predicted value across multiple time instances, with the remaining predicated RSRP being reported as differential RSRPs. - Introduce a field to indicate which time instance contains the L1-RSRP. - Differential RSRPs are mapped according to timing order of the future time instances. |
| Fujitsu | - For BM Case-2 with UE side model, the beam with the largest RSRP value should be indicated in the differential RSRP reporting over multiple time instances |

Agreement(RAN 1 #118)

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement(RAN 1 #116bis)

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

##### (FL0) Proposal 3.2a (report for BM-Case 2)

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, down selection one option from:

- Alt 1: the largest RSRP value of each time instance is the reference RSRP, and differential RSRPs of the same instance in the report are computed relative to the reference RSRP of the corresponding time instance.

- Alt 2: the largest RSRP of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

| Companies | Alt1/Alt2 | Commnets |
| --- | --- | --- |
| FL0 |  |  |
| Huawei, HiSilicon |  | We already agreed “ support differential RSRP report among multiple beams over multiple time instances”, so Alt 2 seems to be the natural solution. |
| NTT DOCOMO | Alt2 | Slightly prefere Alt2, considering the paylaod overhead. |
| OPPO |  | From our reading, it may have different interpretations (Alt 1 and Alt 2) on the following agreement. Alt 2 may reduce more overhead due to differential RSRP, but it requires to additionally report the time instance with largest RSRP as reference. Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details We are open for more discussions on both Alts. |
| Xiaomi | Alt 2 | Support Alt 2 for further reduce the overhead |
| Fujitsu |  | We think Alt 2 is more aligned with the existing agreement. |
| TCL |  | Agree. However, the time instance informaiton is nod needed, as the time instance of largest RSRP can be determined by the order. - Alt 2: the largest RSRP of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report. |
| ZTE |  | Alt 1 leads to additional reporting overhead since multiple quantized RSRPs with larger bit width need to be reported as reference. Alt 2 leads to serious quantization errors since the RSRP value of different time instances may be quite different and lose correlations. Therefore, we prefer the following alternative to achieve reduced reporting overhead without sacrificing the quantization precision. - Alt 3: the largest RSRP value of each time instance is the reference RSRP, and differential RSRPs of the same instance in the report are computed relative to the reference RSRP of the corresponding time instance. Additionally, the differential value of the largest RSRP of each time instance are computed relative to the largest RSRP of all time instances. - The time instance information of the beam with the largest RSRP are additionally indicated in the report. |
| vivo |  | We agree that we need to make a decision on this. We support Alt 2. We have concern on Alt 1, which will introduce more overhead . |
| QC | Alt. 2 |  |
| Google |  | Support Alt2 |
| CMCC | Alt2 | Support Alt 2 for further reduce the overhead. |
| ETRI |  | We are open to both options. Alt 2 can reduce more overhead compared to Alt 1. However, only one reference RSRP in all time instances may lead more quantization error compared to Alt 1. |
| Panasonic |  | We share a same view as HW. |
| Spreadtrum |  | Agree with OPPO. How much overhead Alt 2 actually reduces needs further discussion |
| LG | Alt 2 | Prefer Alt 2 for overhead reduction. |
| Sharp | Alt.2 | Alt.2 always results in a smaller payload size compared to Alt.1. |
| Tejas | Alt 2 | Support Alt 2 |
| CATT | Alt2 | Comparing with the case of reporting largest L1-RSRP for each time instance, the reporting overhead can be further reduced via Alt2. |
| Futurewei |  | Support Alt 2. |

##### (FL1) Proposal 3.2b (report for BM-Case 2)

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, down selection one option from:

- Alt 1: the largest RSRP value based on prediction of each time instance is the reference RSRP, and differential RSRPs of the same instance in the report are computed relative to the reference RSRP of the corresponding time instance.

- Alt 2: the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Note: Ericsson, IDC prefer Alt 1

### Issue #3.3: Definition of reported RSRP

Agreement (RAN 1 #117)

Following Working Assumption is confirmed.

Working Assumption

- For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement (RAN 1 #116bis)

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s)in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

Google

- The reported RSRP should be defined as the predicted RSRP based on a reference transmission power

LGE

- Option C: Not specify whether to report predicted RSRP or measured L1-RSRP when both RSRPs are available at UE side, i.e., leave this case up to UE implementation

NEC

- For predicted RSRP, the Tx power is assumed based on the configured powerControlOffsetSS of the resource corresponding to the predicted beam if Set A resources are configured and the Tx power is assumed based on setting powerControlOffsetSS to 0 if Set A resources are not configured.

##### (FL0) Discussion point (predicted RSRP)

Summary

| Options | Supporting companies |
| --- | --- |
| A only | Google |
| B only | Futurewei, ZTE, Huawei/HiSi?,xiaomi, HONOR, NEC, Ericsson |
| Both A and B [configurable?] | CMCC, vivo， Samsung, inteditigal, ITL, Panasonic, CATT |
| No need to differantiate | LGE |

##### (FL1) Proposal 3.3a (predicted RSRP)

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s)in the report of inference results, when applicable, Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for measurement by the corresponding CSI-ReportConfig for inference. for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

| Companies | Comments |
| --- | --- |
| FL0 | Please provie your views in the abvoe table. And explain how to support both A and B, or the wording for no differantiate |
| Huawei, HiSilicon | We prefer configurable between A and B. In case the UE side model can accurately reflect the relative RSRP over beams but fails to match the measured L1-RSRP, gNB can prefer to keep the model running while only allows UE to compare among predicted RSRP for beams. |
| TCL | Both A and B [configurable]. |
| ZTE | Support B due to its higher reliability. For A, there used to be one motivation that the comparison between the predicted RSRP in the inference report and the measurement of monotring resource set can be used for the monitoring of UE-sided model. However, it seems that only beam ID prediction accuracy is supported as the monitoring metric. |
| Vivo | Ok with configurable between A and B. |
| LG | Considering that regression model is used for RSRP prediction, actual measured L1-RSRP of Set B resource(s) and predicted RSRP of the Set B resource(s) would not have big difference, so in our view, it seems that it dosen’t need to be specified. |
| CATT | In our view, both A and B can be supported and which option is used for reporting can be configured by gNB. During the discussion, some companies argued that the predicted L1-RSRP may not be accurate. In our view, if there is large difference between the predicted L1-RSRP and the ideal L1-RSRP, the performance degradation will be detected by performance monitoring procedure. Moreover, the predicted L1-RSRP can be used by network for performance monitoring, and hence the network can configure the type of RSRP for reporting. |

Huawei

Proposal 18: For the CSI report for the inference of a UE-side model, the predicted beam ID(s)/RSRP(s) and the measured beam ID(s)/RSRP(s) need to be differentiated, e.g., by introducing an indicator or a new reportQuantity included in CSI-reportConfig.

Proposal 19: For the inference of a UE-side model, if the inference output is predicted RSRP, the type of the predicted RSRP should be defined from at least one of the following:

- Absolute RSRP, where the predicted RSRP from Set A beams is subject to dBm value.

- Normalized RSRP, where the predicted RSRP from Set A beams is subject to normalized value.

##### (FL0) Proposal 3.3-2a

For report content of inference results for UE-sided model, the RSRP of predicted Top K beam(s) is the RSRP subject to dBm value (as current L1- RSRP).

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

| Companies | Y/N | Commnets |
| --- | --- | --- |
| FL0 |  | Maybe better to clarify it. |
| Huawei, HiSilicon |  | OK |
| NTT DOCOMO |  | Support. |
| OPPO |  | It seems fine. Other than dBm, we have no other unit for L1-RSRP. |
| Xiaomi |  | Support |
| Fujitsu |  | Why this proposal is necessary? |
| ZTE |  | OK |
| vivo |  | OK |
| QC |  | OK |
| MediaTek |  | OK |
| Google |  | OK. We also need to clarify the Tx power assumed for the RSRP report |
| CMCC |  | Ok. |
| ETRI |  | We support |
| LG |  | OK |
| Sharp | Y |  |
| Ericsson |  | Ok |
| Fraunhofer |  | Ok |
| CATT |  | OK |
| NEC |  | We don’t think it has spec impact. |
| Futurewei |  | Ok. |

# CPU or other capability for UE-sided model

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| ZTE | Observation 2: The resource occupation for AI/ML-based CSI processing is impacted not only by the report content, but also by the implementation algorithms at the UE side. Proposal 26: For forward compatibility, support UE to report the number of processing units consumed for each AI/ML-based functionality or CSI report. Proposal 27: The overall CPU can be separately counted between legacy CSI reporting and AI/ML-based CSI reporting, which provide flexibility to accommodate the adoption of hardware other than CPUs for AI/ML processing. Proposal 28: Study whether/how to incorporate the functionality activation latency or inference latency in the CSI processing timeline. |
| Huwei/HiSI | Figure 3 Issue on CSI processing criteria under BM-Case 2 Proposal 22: For BM-Case 2 with a UE-side model, consider enhancement on the CPU rule to handle the processing of the long observation window. - For P/SP CSI-RS resource type, consider discontinuous CPU occupation over the multiple observation instances to avoid overcalculated CPU occupancy over the whole observation window which may last hundreds of ms. - A-CSI-RS resource type may not be applicable as it causes long scheduling delay. Figure 4 Example where predicted instance is earlier than the CSI report Proposal 23: For BM-Case 2 with a UE-side model, for the reference time to determine the earliest time instance of the prediction window, if Option 3 (Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B) is considered, the time gap between the reference time and the earliest reported predicted instance should adapt to ensure the UE always report a certain number of predicted instances after the transmission of CSI report. Proposal 34: Regarding the CPU sharing rule for AI/ML-based CSI processing, focus on the solution where the overall CPU is separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and shared among CSI report related AI/ML functionalities. Observation 7: Activation delay is needed for deploying the model to the AI/ML processing memory before performing inference. - Activation of an AI/ML model for beam management could be achieved in TTI level delay. - To achieve efficient usage of AI/ML processing memory, the model should be flushed from the AI/ML processing memory after inference is completed. Proposal 35: For A-CSI report subject to a functionality, the CSI processing timeline is impacted by the active/inactive state of the functionality. When the A-CSI report is triggered: - If the functionality is subject to “active state”, the CSI processing is subject to a shorter timeline. - If the functionality is subject to “inactive state”, the CSI processing is subject to a longer timeline as it additionally includes the activation delay. Proposal 37: The association of multiple CSI report configurations and/or multiple sets of inference related parameters corresponding to a same functionality should be reported by UE in Step 4. Observation 8: UE may need to deploy multiple functionalities in the AI/ML processing memory, each of which may correspond to an individual required memory storage. Proposal 38: Discuss the memory occupancy alignment mechanism to align the availability of memory storage for updating the AI/ML-based CSI report. - The memory is occupied when it requires computation and is flushed when the CSI report is transmitted. - For A-CSI report, the occupation lasts from DCI to the CSI report. - For P-CSI/SP-CSI report, the occupation lasts for a certain time duration before the uplink symbol of per P-CSI/SP-CSI report. - The UE does not need to update an AI/ML-based CSI report if the unoccupied memory cannot support its required memory. |
| CATT | Proposal 33: For UE side AI/ML-based CSI processing, a new type of processing unit or enhanced processing unit can be introduced if different modules are used for RS measurement and model inference. The enhance CPU is separately counted from legacy CPU pool. Proposal 34: For UE side AI/ML-based CSI processing, the new type or enhanced CPU should be shared among CSI related AI/ML features/functionalities. Proposal 35: For UE side AI/ML-based CSI processing, UE should report the information for determining the occupied processing units. Proposal 36: For UE side AI/ML-based beam reporting, the following aspects regarding CPU/enhance CPU occupation time need to be clarified: - The occupied symbols are determined by the resources for measurement (e.g., Set B resource); - How to specify the symbols during which the enhanced CPU is occupied; - How to specify the symbols during which the CPU is occupied for BM-Case2. Proposal 37: Determine the priority of CSI report for different functionalities, as well as for data collection, model inference, and performance monitoring. - At least define the priority according to the report quantity. |
| CMCC | Proposal 54: For UE-side model, the overall CPU is separately counted between legacy CSI reporting and AI/ML-based CSI reporting. The overall CPU is shared counted among AI/ML features. |
| vivo | Proposal 59: For UE-side model, further study whether to define AI process capability including re-use or modified the existing CSI computation time and CSI processing units. |
| OPPO | - For BM-Case1 and BM-Case2 with UE-side model, the overall CPU should be separately counted between legacy CSI reporting and AI/ML-based CSI reporting. |
| Ofinno | - Define priority rules between AI/ML-based report and non-AI/ML-based report, and among multiple types of AI/ML-based reports, considering: - Different use cases (or sub use cases) such as BM Case-1, BM Case-2, or CSI compression. - Different LCM purposes (e.g., reporting inference results, reporting performance monitoring results, and so on) of an AI/ML model. - Different report quantity types (e.g., predicted CRI/SSBRI, predicted RSRP, beam prediction accuracy, one-shot performance indicator, and so on). - Enhance CPU and CPU occupancy considering a long observation window for performance monitoring and inference. |
| Nokia | Proposal 8: For AI/ML-enabled CSI reporting (including Rel-19 beam prediction use cases), considering the CPUs used for inference reporting, the following enhancements and clarifications are supported, - Consider a separate limit () for simultaneous CSI calculations for AI/ML-enabled CSI reports. - Similar to legacy limit on , define may be defined per component carrier or across all component carriers - Both legacy limit (and AI/ML-specific limit ( shall be considered when clarifying the spec (TS 38.214 Section 5.2.1.6) for CPU occupancy for AI/ML-enabled CSI reports - value(s) (number of CPUs) for a AI/ML-enabled CSI report (at least for beam prediction use cases) shall be clarified. - value for measurements of Set B shall be 1 (same as legacy). - value for inference operation (if that is separated from measurements of Set B) shall be clarified. Proposal 9: For beam prediction use cases, considering CPU occupation time for inference reporting, the following enhancements and clarifications are considered, - For BM-Case1, P/SP/AP-CSI-reports occupies CPUs for measurements of Set B as in the legacy timelines. - FFS: Any changes on determining CPU duration for inference operation (if that is separated from measurements of Set B) - For BM-Case2, further study CPU occupation duration(s) considering the measurements of multiple instances of Set B. Proposal 10: For beam prediction use cases, considering CPU occupation time and number of CPUs for performance monitoring reporting, the following enhancements and clarifications are considered, - value for measurements of monitoring RS resource set (one monitoring instance) shall be 1 (same as legacy). - Further study CPU occupation duration(s) considering the reporting framework of monitoring metric. - Further study value for monitoring metric calculation (if that is separated from measurements of monitoring RS resource set). Proposal 11: For beam prediction use cases, to clarify the active NZP-CSI-RS resources and active CSI-RS ports, RAN1 shall discuss the active time durations of CSI-RS resources in Set A. Proposal 12: For beam prediction use cases, to ensure that that priority rules do provide the necessary policies for the UE to handle beam prediction reports, RAN1 to clarify the following, - How to determine a priority value for a AI/ML-enabled CSI reports (related to reporting of inference results) - Whether there are any changes to the priority values for CSI reports that carrying measurements for NW-sided model inference or NW-sided performance monitoring or data collection. |
| Samsung | Proposal 25. For the CSI report for inference for UE-side model, OCPU is depending on UE capability. - FFS: the granularity of the UE capability, e.g., whether it is use case specific Proposal 26. Consider separately CPU counting between legacy non-AI/ML based CSI report and AI/ML based CSI report. Proposal 27. For CPU occupation time, legacy definition is reused - For BM-Case1, the legacy definition of Rel-15 P/SP/AP CSI is reused - For BM-Case2, the legacy definition of CSI for Rel-18 doppler domain CSI prediction is reused Proposal 28. For the CSI report for inference for UE-side model, support the scaling of legacy Z timeline (i.e., Z3/Z3’) for beam management with a scaling factor - The scaling factor is depending on UE capability - FFS: the granularity of the UE capability, e.g., whether it is use case specific |
| Apple | Proposal 7-1: the CPU occupancy rules for AI/ML BM-1/BM-2 inference support two options: In Option 1: the same CPU occupancy pool/rule applies to conventional CSI/BM reporting and AI/ML BM reporting. In Option 2: the same CPU occupancy rule applies to conventional CSI/BM reporting and AI/ML BM reporting, however, two separate pools are used: the first CPU pool is for conventional CSI/BM reporting, and the second CPU pool is for AI/ML beam management and potentially AI/ML CSI prediction. FFS: whether AI/ML beam management and AI/ML CSI prediction utilize separate CPU pools. |
| Interdigital | Proposal 10: Support a shared CPU counter between legacy and AIML-based CSI reporting. |
| Google | Proposal 7: An inference report for BM-Case 1 should take 1 eCPU and 1 legacy CPU - eCPU is occupied for inference and legacy CPU is occupied for Set B RS measurement - Support the UE to report whether BM-Case 1 and BM-Case 2 can share common eCPUs or not - Performance monitoring for BM-Case 1 should also take 1 eCPU and 1 legacy CPU |
| LGE | Proposal #18: For UE-side AI/ML operations, support to introduce AI/ML processing units(APU) for CSI reports for inference result report. - APU is occupied for all AI/ML inference operations at UE-side, and separately managed from CPU - Further study on APU capability, e.g., supported maximum APUs per UE, required APUs per functionality/sub-use-case. |
| Sharp | - For UE-side model, CPU can be separately counted between the legacy CSI reporting and AI/ML-based CSI reporting. - AI-based CPUs are introduced and shared among the AI/ML features/functionalities. - the required number of CPUs for BM case 1 and BM Case 2 depends on UE capabilities. - For UE-side model for BM Case 2, the CPU(s) are occupied from the first symbol of the earliest CSI-RS/SSB occasion within the measurement window until the last symbol of PUCCH/PUSCH carrying the report. - For UE-side model for BM Case 2, the CPU(s) are occupied from the first symbol of the earliest CSI-RS/SSB occasion within the measurement window until the last symbol of PUCCH/PUSCH carrying the report. |
| HONOR | - Support to define a new processing unit dedicated for model inference, e.g. NPU. - For the CPU and NPU sharing rules for AI/ML-based CSI processing, further study the following options: - Option1: The CPU is shared between legacy CSI and AI/ML-based CSI reporting, and NPU is shared among different AI/ML functionalities/use cases. - Option2: The CPU is shared between legacy CSI and AI/ML-based CSI reporting, and NPU is separately counted among different AI/ML features/functionalities. - For an AI/ML-based CSI report, the NPU occupation timeline should reuse the mechanism of CPU occupation timeline. |
| NEC | Support CPUs separately counted. The CPUs used for inference can be named as ACPUs. And AI/ML based CSI reporting occupies one set of CPUs and the other set of ACPUs. If the AI/ML based CSI report is introduced, define CSI priority rule at least in the following cases: Non-AI/ML based CSI report vs. AI/ML based CSI report AI/ML based CSI report vs. AI/ML based CSI report |
| MTK | Proposal 29: For UE-sided model inference, consider the CPUs are separately counted for legacy CSI reporting and AI/ML-based CSI reporting. |
| Lenovo | - Introduce AI process units for beam report with AI/ML inference at UE-side. - Support UE reporting on APU usage required for an AI/ML-based CSI processing. - Study the mechanism on how to determine the reported beams for beam report with AI/ML inference if there is no available AI/ML model inference processing resource. - For a beam report associated with AI inference, the UE indicates that the reported beams are predicted beams or measured beams in the beam report. - Refine the UE CSI computation time for aperiodic CSI report with AI inference by considering the AI process time. |

Agreement

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

### Issue #4.1: details on CPU mechanism

##### (FL0) proposal 4.1:

For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, study the following options:

- Option 1: The CPU occupied by the CSI report at least for inference and the CPU occupied by the non-AI/ML based CSI report for share the same pool.

- Option 2: Introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU), FFS

  - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols

    - Note: the legacy CPU pool will not impact by the activation of AI/ML functionality

  - Alt 2: Processing of a CSI report at least for inference occupies X number of ACPU(s) for M symbols and Y number of CPU(s) for N symbols

    - FFS on whether M=N, X=Y.

- FFS whether AI/ML based beam management and other AI/ML based features, e.g., AI/ML based CSI prediction, share the same (A)CPU pool or not.

FFS for processing of a CSI report for monitoring

FFS whether AI/ML based beam management and AI/ML based CSI prediction share the same (A)CPU pool or not.

| Companies | Y/N | Commnets |
| --- | --- | --- |
| FL0 |  | Two main directions to facilitate the discussion |
| Huawei, HiSilicon | [Option 2] | Clarification questions: Q1: Option 1 means the AI BM occupies both AI specific CPU and legacy (non-AI) CPU? Q2: Does it mean a separately CPU pools are considered between legacy CSI reporting and AI/ML-based CSI reporting? If that is the intention, then we prefer Option 2. Q3: What does Option 2 Alt 2 mean? What is the difference from Option 1? |
| OPPO |  | Okay with this direction. Considering the fact that the hardware for AI/ML based operations are quite different from legacy CSI processing, we would prefer to introduce dedicated CPU pool, i.e. Option 2. |
| Xiaomi |  | Support and prefer Alt 1 in Option 2. |
| Fujitsu |  | Is the proposal applicable to bothe BM Case-1 and Case-2? It’s better to clarify. |
| TCL |  | Agree with option 2. |
| ZTE |  | Prefer Option 2 Alt 1. ACPU can be used to abstract all necessary resources (e.g., computation, storage, measurement) consumped for the AI/ML related CSI processing, and thus there is no necessity to separately count the resource consumption for measurement and for inference. Besides, AI/ML based beam management and AI/ML based CSI prediction can share the same (A)CPU pool as legacy. |
| vivo |  | We support Option 2, which is a more natural way for AI/ML, esp considering AI and non-AI will use totally different hardware and software modules in UE. After determining Option 2, we can further discuss the remaining details. |
| QC |  | Similar questions as Huawei, needs elaboration. |
| MediaTek |  | Agree with the direction. For Option2, we can expect that both ACPU and legacy CPU are utilized simultaneously since there may coexist legacy CSI reports other than AI BM reports. But to support AI functionality, some hardware capabilities reflected in legacy CPU may be occupied. We think it is essential to clarify whether the legacy CPU pool will change depends on whether ACPU is on use or not (i.e., whether AI BM functionality is activated or not) We suggest to add an FFS for Option2: - Option 2: Introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU), FFS - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols - Alt 2: Processing of a CSI report at least for inference occupies X number of ACPU(s) for M symbols and Y number of CPU(s) for N symbols - FFS on whether M=N, X=Y. - FFS: whether the legacy CPU pool will be impact by the activation of AI/ML functionality |
| Google |  | Support option 2 |
| CMCC |  | Support option 2. Is the attention of Alt2 to define different ACPU for different time duration? |
| Panasonic |  | We are fine with the direction. For Option 2, it is better not to list down Alt. 1 and Alt. 2 as we do not understand them clearly. It can be other possibilities. |
| Spreadtrum |  | OK with this proposal. |
| LG |  | Open to discuss. |
| Sharp |  | Option 2 is preferred. For Alt.2, the motivation to introcude different numbers of CPUs for a same CSI report configuration needs to be clarified. |
| CATT | Option 2 | We think alt 2 can be supported. For AI/ML-based CSI processing, it includes two steps for obtaining the inference results: RS measurement and model inference. UE may use different computing modules to perform RS measurement and model inference. UE may use CPU for RS measurement and use ACPU for model inference. The UE should report the supported number or the limit of the ACPU, which is separately counted from the legacy CPU pool. For an AI/ML-based CSI reporting, it may occupy both legacy CPU and enhanced CPU. Therefore, it should meet the requirements of both types of processing units. |
| NEC |  | Prefer option 2 alt2 as the way forward since to have AI/ML based BM UE needs to perform measurement as legacy plus the inference operation. |
| Futurewei |  | We are open to discuss both options. |

# Configuration of RS for Set A and Set B for NW sided model

Agreement For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Tejas | ##### Proposal 9: For NW sided model, for the reporting of RS for Set A and Set B Support multiple resource sets to associated to one L1 beam report. - Consider the configuration where Set A reports beam IDs while Set B reports L1-RSRP measurements. Observation 2: Introduce flexibility in scheduling through dynamic resource allocation where gNB schedules Set A measurements only when necessary. This could also help reduce UE processing requirements when Set A is not crucial to the current transmission or prediction process. ##### Proposal 10: The gNB may require dynamic control over Set A and Set B measurements to minimize overhead while maintaining beam prediction accuracy. |
| CMCC | Proposal 4: Regarding data collection for NW-side model, following options can be considered for the configuration of Set A: - Option 1: The UE capability of the maximum number of the RS per resource set for the RSRP measurement can be enhanced. - Option 2: Multiple resource sets can be supported in one CSI-ResourceConfig Proposal 5: Regarding training data collection for NW-side model, the configuration of multiple set B or multiple Set B patterns can be supported. Proposal 18: For NW-sided model, for BM-Case 2 without sliding measurement window, CSI report framework needs enhancement to configure the resource set for measurement. Proposal 19: For NW-sided model, for BM-Case 2 with sliding measurement window, measurement window is determined by gNB. |
| Intel | For a network-side AI/ML model, for BM-Case 1/2, implicit configuration of set B and/or set A (if required) for AI/ML model input can be used based on existing CSI-ResourceConfig and CSI-ReportConfig frameworks. Proposal 2: It is not necessary to configure multiple resource sets associated to one L1 beam report for a network-sided model. |
| ZTE | Proposal 5: One or two RS resource sets for beam measurement can be configured to the UE for data collection of NW-side AI/ML model, which are dependent on the beam set construction of Set A and Set B. Proposal 6: For the collection of both model input and model label data at the NW side, one RS resource set can be configured to the UE with indication of necessary assistance information, such as mapping of Set A and Set B. |
| Fujitsu [11] | Proposal 5: - For training data collection, the reference signals same as Set B should be configured to obtain the model input data. And the reference signals same as Set A should be configured to obtain the ground truth data. - If Set B is subset of Set A, then only the reference signals of Set A are configured. - If Set B is different from Set A, then the reference signals of both Set A and Set B should be configured to the UE. Proposal 6: - RAN1 to further discuss whether the same or separate reference signal configuration among training data collection, model inference and performance monitoring could be applied. Proposal 7: - Regarding training data collection, RAN1 to further discuss the reference signal configuration for different sub-use cases. |
| ETRI [23] | Proposal 9: For BM-Case2, support the configuration of multiple Resource Sets for Set A. |
| Xiaomi | Proposal 5-6: Both two separate CSI-ReportConfigs and one CSI-ReportConfig can be supported for set B and set A configuration for data collection for NW-side AI/ML model training. Proposal 5-7: If one CSI-ReportConfig is used for set B and set A configuration for data collection for NW-side AI/ML model training, consider to support more than one reportquantity in one CSI-ReportConfig. |
| LGE [36] | Proposal #3: Consider extending sub-configuration based Rel-18 NES mechanism for Set B beam measurement and reporting. - Different Set A and/or Set B may be associated with each sub-configuration - Different Set B patterns for a specific Set B may be associated with each sub-configuration |
| MTk [39] | Proposal 22: For NW-side model inference BM-Case2, for periodic Set B RS resources, observation window length is configured to UE within CSI-framework, with the following alternatives: - Alt A: explicitly configured in csi-ReportConfig - Alt B: implicitly configured in csi-ReportConfig - P/SP report: indicated through ReportPeriodicityAndOffset of the report and ResourcePeriodicityAndOffset of the associated RS resources of Set B - AP report: indicated by the triggered slot and report slot - Note: the observation window length is how many Set B measurements need to be reported Proposal 23: For NW-side model inference, support reporting multiple time instances of Set B measurements within one report. Further study on whether/how to explicitly and/or implicitly include corresponding time information in the report. Proposal 28: For NW side data collection, support higher layer signaling as baseline. Proposal 29: For content for data collection for training, for both UE-sided model and NW-sided model, at least for BM-Case 1 per instance, further study the following options: Note: the intention is for higher layer report - Opt 1: L1-RSRPs from RS resources from one or two sets of beams configured to UE - Opt 2: L1-RSRPs from RS resources from one set of beams configured to UE, and beam information of Top K from RS resources from another set of beams |
| ITL | Proposal 3: For network-side model data collection for training, it is proposed to at least support higher-layer signaling to convey data collection content. |
| Panasonic | Proposal 1: NW-sided model inference, support to that a measurement window can be configured with the measurement resource set. |
| NEC | Support variable Set B for model inference. Specifically, for a variable Set B selected from a set of pre-configured Set B patterns, multiple Set Bs should be configured; for a variable Set B that is a subset of measured beams Set C, criterions/thresholds for determining the Set B should be defined. |
| Samsung | Proposal 3. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance - Support differential L1-RSRP reporting with larger quantization step(s) and/or smaller range(s) - Consider to reduce the overhead of beam report using two-part CSI Proposal 4. For NW-side AI/ML model inference, for CSI-ReportConfig with the measurements for more than 4 beams in one reporting instance, consider the applicability of the following report quantity: - ‚cri-RSRP‘, ‚ssb-Index-RSRP‘, ‚cri-RSRP-Index‘, ‚ssb-Index-RSRP-Index‘. Proposal 5. For NW-side AI/ML model inference, support CSI-ReportConfig with measurements for each of multiple past time instances in one reporting instance. - FFS: How to identify the multiple past time instances - FFS: The support of P/SP/AP reporting |

### (onhold)Issue #5.1: How to configure the resource sets for Set A/Set B, and the contents in the report for data collection for MDT based solution for BM Case1 and BM Case 2

FL: In general, Set A and Set B shall be mapped. Even for BM Case 1, the actual time window for measurement collection matters! In SI phase, 99% of data for BM-Case 1 are generated in one shot. Somehow, we need to link the report to the resource sets.

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support. - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

# Measurement report for NW-sided model

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 2: For Rel-19 AI/ML-based BM, at least for inference for network-sided AI/ML model of BM-Case1, for the content in a beam report in L1 signaling, - Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and the number of reported beams is indicated in the beam report. - The beam information is CRI/SSBRI. - Increase the maximum value M from the existing “4” to “8” as a starting point. |
| Spreadtrum | Proposal 3: For NW-side model, for reported beam information, only support CRI/SSBRI of a measurement resource set. Proposal 7: For BM-Case1 and BM-Case2 with a network-side AI/ML model, larger quantization step(s) should not be considered at least for model inference. |
| China Telecom | Proposal 1: Regarding data collection for NW-sided AI/ML model, support the following for the contents of collected data: - L1-RSRP - Indices of beams - Timestamp Proposal 2: Enhancement on payload size reduction of measurement reporting needs to be considered, e.g., bitmap indication of beam indices, differential RSRP with larger quantization step and/or smaller range. |
| Tejas | Proposal 14: For NW-sided model, for BM-Case 2, support to report of measurement results of multiple past time instances in one report. Observation 5: Instead of reporting the absolute L1-RSRP values for each time instance, differential values can be reported. This reduces the reporting overhead since only the relative change in RSRP from one time instance to the next needs to be communicated. Proposal 15: For NW-sided model, for BM-Case 2, at least for model inference support to report Differential L1-RSRP. Proposal 16: For NW-sided model, for BM-case 2, at least for model inference support dynamic configuration approach that should be developed to allow the gNB to request data from specific time instances that are most relevant for beam prediction. Observation 6: When low-latency is critical, immediate reporting should be prioritized. For cases involving large sets of data (such as multiple time instances), higher-layer signaling (RRC or MAC layer) should be used to handle the report, as this avoids the scheduling delays associated with lower layers (e.g., PUSCH). Proposal 17: For inference report at NW sided model at least for BM-case 1, consider L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. Proposal 18: For inference report at NW sided model at least for BM-case 1, maximum value of M and X dB gap to the largest measured value of L1-RSRP is configured by gNB. Proposal 19: Consider M = 256 as the starting point which is agreed in study item for NW-sided model inference. Proposal 20: For NW-sided model inference, for content of beam related information, consider CRI along with L1-RSRP as a starting point. Proposal 21: At least for NW-sided model, for differential L1-RSRP reporting, support for introducing a larger quantization step size for differential L1-RSRP reporting. Proposal 22: For data collection for training for NW-sided model, from RAN 1 perspective, • L1 signaling can be used • It is up to RAN2 on whether to use RRC/MDT procedures Proposal 23: For NW-sided model, at least for training data collection, at least for BM-Case 1, For L1 signaling, the content of a beam report should: o Support L1-RSRPs and corresponding beam information for up to M beams within an X dB gap to the highest measured L1-RSRP value, where X and M are configurable by the gNB. o FFS: whether and how to report the number of reported beams. Proposal 24: For NW-Sided Model, Inference Report (BM-Case 1) • Options for Beam Reporting When M < N o Option 1: Report the CRI/SSBRI (Cell Reference Index/SS Block Reference Index) for each beam in the measurement resource set. o Option 2: Utilize a bit map to indicate the reported beams from the measurement resource set and include one beam index (i.e., CRI/SSBRI) for the beam with the largest measured L1-RSRP within the set. |
| GOOGLE | Proposal 2: Support configurable quantization step and range for different L1-RSRP and absolute L1-RSRP report. Proposal 3: Support to report a confidence level indicator for L1-RSRP report to indicate the maximum L1-RSRP measurement error for each beam. Proposal 4: Support to introduce a beam subset configuration indicating a subset of beams that the UE should always report in an L1-RSRP report - Support to define one L1-RSRP state to indicate the reported beam is in an invalid state (UE is unable to measure the L1-RSRP that can meet the L1-RSRP measurement requirement) Proposal 5: Support L1-RSRP report retransmission to facilitate the NW-side beam prediction for BM case 2 |
| Tejas[4] | Proposal 1: For inference report at NW sided model at least for BM-case 1, consider L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. Proposal 5: At least for NW-sided model, for differential L1-RSRP reporting, support for introducing a larger quantization step size for differential L1-RSRP reporting. |
| CMCC | Proposal 3: Regarding data collection for NW-side model, following options can be considered for the configuration of Set A: - Option 1: The UE capability of the maximum number of the RS per resource set for the RSRP measurement can be enhanced/enlarged. - Option 2: The configuration of set A can contain multiple resource sets in one CSI-ResourceConfig Proposal 14: For NW-sided model inference reporting, at least for BM-Case 1, L1-RSRPs and corresponding beam information of beams within X dB gap to the largest measured value of L1-RSRP can be supported. - The X dB gap can be configured by gNB - UL resource allocation for variable number of reported beams can be further discussed. Proposal 15: For NW-sided model inference report of BM-Case 2, temporal compression for overhead reduction can be supported at least for aperiodic report. Proposal 16: The RS associated with TCI state indication should be measured at least once before TCI state indication or application. |
| Intel | It is not necessary to configure multiple resource sets associated to one L1 beam report for a network-sided model. For a network-side AI/ML model, for BM-Case ½, for inference, the network may configure the size of the L1 report based on the measurements from set B. The configuration can be explicit or implicit based on measurement resources configured to the UE. The number of beams to be reported in a single reporting instance may depend on the type of data being collected i.e., it may be configured to be different for training, inference and model monitoring. The container for UE beam reporting should be dependent on the type of collected data i.e., inference/model monitoring data collection should use L1 based reporting, while data collection for training should use RRC or MAC-CE based reporting. For content for data collection for training for NW-sided model, for BM-Case 1 per instance, support Opt. 1: L1-RSRPs from RS resources from one or two sets of beams configured to UE. - The beam information for Top K beams (corresponding to set A of beams, but transparent to the UE) can be determined implicitly from the reported L1-RSRP values for set of beams measured. - The set of beams configured to the UE can be the union of sets A and B of beams (transparent to the UE). For content for data collection for training for NW-sided model, for BM-Case 2, support Opt. 1: One or multiple sets of L1-RSRPs for one or multiple time instances, respectively, from RS resources from one or two sets of beams configured to UE. - The beam information for Top K beams (corresponding to set A of beams, but transparent to the UE) can be determined implicitly from the reported L1-RSRP values for set of beams measured. - The set of beams configured to the UE can be the union of sets A and B of beams (transparent to the UE). - Regarding information on the time instances, study the following approaches including combinations thereof: - implicit determination of time instances for multiple measurements based on configuration of two time windows that are defined in terms of the respective window length and time gap between them. - explicit indication of time information for the time instances via reporting of timestamps associated with each measurement. For a network-side AI/ML model, for BM-Case 2, for measurement reporting for inference (purpose to be transparent to a UE), the network may configure the UE with an observation window and the number of measurements on reference resources related to set B to be reported. - The UE reports one or multiple L1-RSRP measurement results for the beams in the set B corresponding to one or multiple time instances, respectively, in a report with related timestamps of the measurements. Proposal 17: For a network-side AI/ML model, for BM-Case 2, the UE may not need to be configured with a prediction window. |
| ZTE | Support to specify the bitmap based method for the reporting of beam information given the fact that it can greatly reduce the reporting overhead in typical settings without compromising any beam prediction performance. Proposal 10: At least for NW sided model, for the quantization of a reported L1-RSRP value, support differential L1-RSRP reporting with larger quantization step size applicable to the differential L1-RSRP. Proposal 11: At least support L1 signaling for NW-side data collection irrespective of the purpose of data collection (e.g., model training, model inference, and performance monitoring). Proposal 12: For NW-sided model inference, the report content for beam related information comprises beam ID information and L1-RSRP, where the beam ID information can be reported by new beam indexing formats (e.g., bitmap) to reduce the reporting overhead. Proposal 13: For NW-sided model inference, the maximum number of reported beam related information in one report can be configured by the NW based on UE capability indication. Proposal 14: Support enhancements to report information about measurements of multiple past time instances in one reporting instance for BM-Case2, at least from the following aspects. • Indication of the timestamp information • Indication of the reference beam • Indication of the common beam information, e.g., a common/super set of beam IDs to be reported Proposal 15: Support a common framework design for the measurement reporting of multiple past time instances for NW-sided model and prediction reporting of multiple future time instances for UE-sided model. |
| Ericsson | - For NW-sided model, regarding max number of reported beam related information in one report, support reporting all measured beams in a resourceSet. - For NW-sided model inference, support NW configuration for UEs to pre-process set B beams to reduce reporting overhead, via: - Support configuring reporting of only beams within X dB of the strongest beam, - Support configuring reporting of at most N strongest set B beams. - For NW-sided model inference, support methods for UEs to compress the set B temporal domain measurement results to reduce the reporting overhead. For NW-sided data collection, RAN1 studies possible “omission/selection of collected data” by the following aspects as a starting point, - Possibility for UE to avoid signalling “duplicated” samples, - Possibility for UE to avoid signalling data based on certain events, one event can comprise that the UE experienced large channel variation during set A measurements. - Note: RAN2 can use such study when designing data collection procedures |
| Vivo | Proposal 38: For data collection, for NW-side model, support to report UE measurement results via L1-layer signaling and higher-layer signaling. Proposal 39: For data collection, for NW-side model, confirming the agreement from SI phase that more than 4 beams can be reported in a beam report.  The maximum number of reported beam related information in one report is related to UE’s capability. Proposal 40: For data collection, for NW-side model, it is crucial to investigate approaches to minimize the overhead of the report transmitted through L1-layer or higher-layer signaling. Proposal 41: For data collection, for NW-side model, report content supported in current specification can be re-used for BM-Case 1 and BM-Case 2, i.e. L1-RSRPs and corresponding beam information. Proposal 42: For data collection, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 1. Proposal 43: For data collection and inference, for NW-side model, enhancement on overhead reduction should be supported for BM-Case 2, e.g. consider to use time domain data compression to reduce overhead. Proposal 44: For inference, for NW-side model, if M = the size of the measurement resource set, report the L1-RSRP in a predetermined order corresponding to the measurement resource set. Proposal 45: For inference, for NW-side model, support to report L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, as well as the number of reported beams Proposal 46: For inference, for NW-side model, support to introduce a beam index type indicator to specify the meaning of resource indicator, which indicates the reported beams are the beams with highest quality or lowest quality. Proposal 47: For inference, for NW-side model, support to report of measurement results of multiple past time instances in one report for BM-Case 2. Proposal 48: For data collection and inference, for NW-side model, support enhancements on quantization range and quantization step to reduce overhead for measurement results report. Proposal 49: For data collection and inference, for NW-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions. |
|  | - For NW-side model, the beam information could be clarified as beam reporting index (i.e. CRI/SSBRI in NR). - For BM-Case2 with NW-side model, UE reports multiple measurement instances in a single beam reporting instance. - For BM-Case2 with NW-side model, enhance unified TCI framework to facilitate beam indication for multiple future time instances. - For NW-side model training, UE reports the following contents to NW - L1-RSRPs measurements of fixed Set B as model inputs - Top-K L1-RSRP(s) and Top-1 Tx beam index as labels - For BM-Case2 with NW-side model, the temporal domain information of collected data for training could be reported in an implicit manner, i.e. no explicit time stamps needed. - Continue to discuss the content for data collection for NW-side model training without mentioning for higher layer or physical layer report. |
| Fujitsu | - For BM Case-1 with NW-side model, regarding the reporting content for Set B measurement results for inference operation, the beam related information could include CRI/SSBRI. - For BM Case-2 with NW-side model, regarding the reporting content for Set B measurement results, the beam related information could include CRI/SSBRI and corresponding L1-RSRP. - For BM Case-2 with NW-side model, similar beam indication enhancement as BM Case-2 with UE side model could be considered. |
| CATT [12] | Proposal 18: For NW-sided model, for inference report, at least for BM-Case 1, support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB. Proposal 19: For NW-sided model, for inference report, the maximum value of M based on UE capability should be discussed in the UE feature. Proposal 20: For NW-sided model, the following options can be considered for the reported beam information - Opt 1: Legacy CRI/SSBRI of a resource set, and resource set id if multiple resource sets consists set B; - Opt 2：The indicator for largest measured value of L1-RSRP, and a bitmap indicating RS index within a resource set, and resource set id if multiple resource sets consists set B. Proposal 21: For NW-sided model for BM case-2, for inference, support to report largest L1-RSRP from N time instances and other differential L1-RSRP of N time instances within the measurement window in a pre-defined order in a beam report. Proposal 22: At least for NW-sided model, for inference, support to introduce a larger quantization step size for differential L1-RSRP reporting. Proposal 43： For content for data collection for training for NW-sided model via higher layer signaling, for both BM-Case1 and BM-Case2, for each per instance, support the following options: - Opt 1: L1-RSRPs measured based on resource set(s) (for Set A and Set B) configured to UE; - Opt 2: L1-RSRPs measured based on resource set (s) (for Set B) configured to UE, and beam information of Top K measured based on resource set(s) (for Set A) configured to UE; - Differential L1-RSRP reporting is supported with legacy quantization steps and ranges. Proposal 44: For performance monitoring of NW-sided model, support to report the RS indicators corresponding to the beams within X dB margin of ideal Top-1 beam. Proposal 45: For performance monitoring of NW-sided model, the content in a beam report includes the beam information of Top M beam(s) with largest M measured L1-RSRP(s). Proposal 46：For Top-K beam measurements (P2 procedure) of NW-sided and UE-sided model, the following options can be considered: - Option 1: NW can dynamically indicate the active resources for measurement among the pre-configured RS resources associated with the CSI report; - Option 2: NW can dynamically indicate the QCL relationship for the pre-configured RS resources associated with the CSI report. Proposal 47：Introduce a new report quantity (e.g., Top-M beam/RSRP) in the CSI report to facilitate the beam reporting for NW-sided model. Proposal 48：Introduce a new report quantity (e.g., predicted RSRP) in the CSI report to facilitate the inference report of UE-sided model. Proposal 49：For BM-Case2, the value of future time instance(s)/slot interval(s) for which inference results are reported can be configured via RRC. |
| TCL | Proposal 7: The following enhancement on the beam management report should be considered. - Alt. 1 Indicating the model type and/or a bitmap to indicate the selected report quantities. - Alt. 2 Indicating the model type and/or the report type to indicate the selected report quantities. Proposal 8: The report for AI/ML BP may include L1-RSRP and/or post processed RSRP. Proposal 9: RAN1 should consider the following enhancement on the report of AI/ML beam management. - For overhead reduction purpose, study the quantization of report quantities, starting from the enhancement on the RSRP quantization. - Study the two-stage report mechanism using both PUCCH and PUSCH. Proposal 10: For BM-Case2, the following overhead reduction approach can be considered. - The report may be split into multiple groups for latency and overhead reduction, FFS the splitting rule and collision control mechanism. - The selection of predicted beams in the report can be indicated by a reference beam plus a bitmap indicating the appearance of predicted beams within the neighbourhood. |
| LGE | Proposal #1: For NW-sided AI/ML in temporal DL Tx beam prediction, support the following UE reporting enhancements for data collection: - Past/present best N beam(s) per time stamp - Tendency/variance of best N beam(s) Proposal #4. Regarding the report of more than 4 beam related information in L1 signaling for NW-sided model, support CRI/SSBRI for beam information as legacy, e.g., CRI/SSBRI(s) of Top M beam(s) and the corresponding M L1-RSRP(s) are reported. |
| Lenovo [14] | Proposal 14: To Support NW-side AI/ML inference, the gNB can configure one or more CSI reports for the UE to report the L1-RSRPs of all the beams configured in the CMR associated with the CSI report. Proposal 15: Study schemes on differential RSRP report for UCI overhead reduction for larger number of beam reports in a beam report. Proposal 2: Support beam report with variable size at least for NW-side model training. |
| InterDigital | Proposal 11: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP. Proposal 19: For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances. - No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used. - Information on measured past instances (e.g., time stamp) is supported. |
| Panasonic | Proposal 1: NW-sided model inference, support to that a measurement window can be configured with the measurement resource set. Proposal 2: Group-based beam reporting can be enhanced to support performance monitoring for NW-sided model. |
| Nokia | Proposal 12: Consider the following for a CSI report that enables beam prediction at the NW, - For BM-Case1 and BM-Case2, consider enhancements for L1-RSRP quantization, increasing the differential L1-RSRPs in the report to X dB quantization step. - FFS: To reduce UCI reporting overhead, discuss value(s) of X (e.g. X=3 and X=4 larger than legacy X>2dB) configurable to the UE. Proposal 13: For BM-Case1, when M < the size of the measurement resource set, consider the following for a CSI report that enables beam prediction at the NW, - For M>4, consider following options, - Option 1: Beam related information can be M CRIs or M SSBRIs of the measurement resource set. - Note: RAN1 already agreed that selected M is the largest M measured value(s) of L1-RSRP(s) of a measurement resource set - Option 2: Beam related information can be an indication of resource subset of the measurement resource set. - The UE may be configured with multiple resource subsets (e.g., can corresponding to fixed beam patterns considered by the NW) for reporting. - Note: The size of the resource subset = M and the selected subset can contain one or more of the largest M measured values of L1-RSRP(s) of a measurement resource set - Configurable M values can be 4, 6, 8, 12, 16, 32. - Further study overhead reduction, including specification impacts, by considering that the UE only reports beams/RSs that are above a preconfigured threshold for each reporting instance. - For M <= 4, legacy procedures are applied. Proposal 14: For BM-Case2, consider the following for a CSI report that enables beam prediction at the NW, - Support common framework design when reporting beam related information for both BM-Case1 and BM-Case2. - Consider enhancements to report multiple past time instances in one reporting instance - FFS: Number of consecutive measurements of beams/RSs to be made between reporting instances. - FFS: whether time stamp information can be derived from report based on report configuration. |
| Ruijie [20] | Proposal 5: Support larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP. Proposal 6: Do not support smaller range(s) for differential L1-RSRP than the already supported legacy range. Proposal 7: For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, for beam information it is proposed to include at least legacy beam index (i.e., CRI/SSBRI), or a bitmap which indicating RS index of a measurement resource set for overhead reduction purpose. |
| Samsung | Proposal 1. For NW-side AI/ML model data collection for training, support at least the following as data collection content: - L1-RSRP(s) for all beam(s) of Set B - L1-RSRP(s) for all beam(s) of Set A - Top-K Beam ID(s) for Set A - Related timestamp - The information to facilitate model training (FFS details) Proposal 2. For NW-side AI/ML model data collection for training, at least support the enhancement to use high layer signaling to convey data collection content. |
| Transsion | Proposal 1: Regarding content for data collection for training for NW-sided model, support Opt2: - Opt 2: L1-RSRPs from RS resources from one set of beams configured to UE, and beam information of Top K from RS resources from another set of beams Proposal 2: Regarding the signaling for training data collection, support higher-layer signalling to report the contents of training data. Proposal 4: Regarding the quantization of a reported L1-RSRP value, support larger quantization step(s) than the legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP. |
| ETRI | Proposal 23: For the NW-side model, support the method of omitting RSRP values based on differences in RSRP values for measurement report. Proposal 24: For the NW-side model, support the use of a bitmap to indicate the beams included in the CSI report for measurement report. Proposal 25: Support methods for reducing UE reporting overhead during data collection for training when the AI/ML model is located on the NW-side. - The NW limits the maximum number of L1-RSRP values that the UE can transmit through CSI reporting. - The NW specifies an L1-RSRP threshold, requiring the UE to report the L1-RSRP values of Tx beams that exceed this threshold. |
| CAICT | Proposal 9: For subset determination of NW-sided model report, Alt.1 (Top M beam(s) is the beams with largest M measured values of L1-RSRPs) is proposed. |
| DOCOMO | Proposal 18: Consider overhead reduction for more than 4 beam related information in L1 signaling. - Large quantization step size for Set B measurement reporting - Reporting of measurements from multiple time instances in one reporting instance. |
| Sharp | - For NW-side model, for inference report, at least for BM-Case 1, support configuring more than 4 measurement results (e.g., L1-RSRP) in one CSI report as a UE capability. - For NW-side model, for inference report, at least for BM-Case 1, when the size of the measurement resource set is configured for reporting, except for the largest measured value of L1-RSRP, the order of differential L1-RSRP values in the report can be based on the ascending order of beam index. - For NW-side model, for inference report, at least for BM-Case 1, beam information of Top M beam(s) in a beam report support: - Option 1: legacy M CRI/SSBRI fields where the M CRI/SSBRI fields indicate Top M beams. - Option 2: a legacy CRI/SSBRI field and a bitmap where one CRI/SSBRI filed indicates a beam index with largest measured L1-RSRP value and the bitmap indicates remaining M-1 beams. - The choice between option 1 and option 2 can be configured by gNB. - For NW-side model, for BM-Case 2, support to report measurement results from multiple past time instances in a single CSI report configuration. - Differential L1-RSRP reporting over multiple time instances is used. - The number of multiple past time instances can be configured by the network. - The number of Top M beam to be reported per time instances is configured by network. - L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP is considered. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 16: Consider the following approaches to enhance the quantization of differential RSRPs: - Multi-resolution quantization, - Increased step sizes, - Adaptive reference beam for differential RSRPs. |
| ITL | Proposal 5: For NW-sided AI/ML model inference, it can be additionally supported to configure reporting of only beams within X dB gap of the strongest beam. Proposal 4: For NW-sided AI/ML model inference, it is proposed to consider a bit map to indicate the reported beams of a measurement resource set and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set only when 4<M<the size of measurement resource set Proposal 6: For NW-sided model inference, the max number of M (e.g. up to 256) reported beam related information in one report can be configured by the NW based on UE capability signaling. It does not need to be varied based on other reporting content. Proposal 7: For NW-sided AI/ML model inference for BM-case-2, support to report largest L1-RSRP from N time instances and other differential L1-RSRP of N time instances in a predefined order or time stamp-based method in a beam report. Proposal 8: It is recommended to increase the quantization step size beyond what is currently supported in legacy L1-RSRP quantization. Proposal 11: For BM-Case2 with the NW-sided model, it can be considered to reduce the overhead of beam report by omitting some of the beam related information and/or L1-RSRP values in a single report. |
| KDDI | Proposal 1: For the L1-RSRP reporting of the NW-sided model in BM-Case 2, consider supporting the reporting of the difference over time. Proposal 2: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, where X and M are configured by gNB. Proposal 3: The UE should be able to report the number of beams it intends to report, either periodically or based on network-specified events. |
| Huawei/Hisi | Proposal 1: For training/monitoring data collection for both NW-side model and UE-side model, RAN1 to discuss the potential mechanism to enable the UE to perform CSI measurements on larger sizes of beam set(s), including: Alt 1: The beam set(s) for measurement consist of multiple resource sets each with legacy size (up to 64) of resources. Alt 2: The beam set(s) for measurement consist of one resource set with increased size of resources, e.g., up to 256. Proposal 2: For the maximum number of reported beam related information in one report, The max number of reported beams can be the same as the number of beams in the measured beam set(s), e.g., up to 256. Consider BM-Case 1 as a starting point. For BM-Case 2, RAN1 needs to decide on whether or not to include multiple past time instances in one report. Proposal 3: From RAN1 perspective, for NW-side model, for monitoring and/or training, conclude that L1 signaling can also be used. - Note: The conclusion can be interpreted that the agreement from RAN1#116 for the report of more than 4 beam related information in L1 signaling does not need to be limited for inference. - Note: Purpose, such as above “For NW-side model, for monitoring and/or training”, will not be specified in RAN1. Proposal 4: For report of data collection in higher layer for NW-side model for BM-Case 1 and BM-Case 2, each data sample includes L1-RSRPs from RS resources from one set of beams configured to UE for per time instance. - FFS on whether/how any corresponding beam information needs to be reported explicitly or not. - Note 1: For BM-Case 2, the beam information and/or L1-RSRP information over multiple time instances can be derived from multiple data samples. - Note 2: Purpose, such as above “For NW-side model for BM-Case 1, BM-Case 2”, will not be specified from RAN1 perspective. Proposal 5: For measurement report for NW-side AI/ML model, support an adaptive number of beams based on threshold, i.e., L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured L1-RSRP value, X and M are configured by gNB, with M>4. Proposal 6: For the report of measured L1-RSRPs for the NW-side model, reuse the legacy quantization step and range for L1-RSRP, i.e., no need to introduce larger quantization steps or smaller ranges for L1-RSRP. Proposal 7: For the report of beam information for the NW-side model, for Opt 1(w omission): L1-RSRPs and corresponding beam information of Top M beam(s) of a resource set, support at least Opt A: CRI/SSBRI, - For Opt B: bitmap and beam index of strongest RSRP, it can be considered if the needed payload size is smaller than Opt A with respect to the resource set size and number of reported beams. Proposal 18: For reporting overhead reduction of NW-side AI/ML model, regarding data compression under BM-Case 2, the necessity of reporting measurements across multiple past time instances in one report is not clear, considering the larger latency and impact on gNB scheduling it would inflict. |
| Xiaomi | Proposal 5-3: For data collection of NW-side AI/ML model training, regarding to reported beam information, the beam index with largest measured value of L1-RSRP should be reported for differential L1-RSRP reporting. Proposal 5-4: For data collection of NW-side AI/ML model training, regarding to reported beam information, support UE to indicate the RS IDs whose L1-RSRP are not reported because of lower than threshold to reduce overhead. Proposal 5-2: For data collection of NW-side AI/ML model training, support to define a time window or a data size for each report with more than one data sample. |
| Kyocera | - For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signalling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set. - The Following options can be considered for beam information: - Option 1: CRI/SSBRI of a measurement resource set - Option 2: A bit map to indicate the reported beams of a measurement resource set |
| LGE | Proposal #1: For NW-sided AI/ML in temporal DL Tx beam prediction, support the following UE reporting enhancements for data collection: - Past/present best N beam(s) per time stamp - Tendency/variance of best N beam(s) Proposal #4. Regarding the report of more than 4 beam related information in L1 signaling for NW-sided model, support CRI/SSBRI for beam information as legacy, e.g., CRI/SSBRI(s) of Top M beam(s) and the corresponding M L1-RSRP(s) are reported. |
| NVIDIA [37] | Proposal 4: For BM-Case 2, at least introduce specification support for using historical optimal beam index and/or L1-RSPR measurement based on Set B of beams as AI/ML model input. |
| NEC | For data collection for NW-sided model, also support L1 report leveraging the existing CSI report framework. For overhead reduction for BM-Case2, support a larger RSRP quantization step size for the historical results obtained earlier, e.g., with a longer time from measurement to model inference. |
| MTK | Proposal 26: For NW side data collection, support higher layer signaling as baseline. Proposal 27: For content for data collection for training, for both UE-sided model and NW-sided model, at least for BM-Case 1 per instance, further study the following options: Note: the intention is for higher layer report - Opt 1: L1-RSRPs from RS resources from one or two sets of beams configured to UE - Opt 2: L1-RSRPs from RS resources from one set of beams configured to UE, and beam information of Top K from RS resources from another set of beams |
| APPLE [40] | Proposal 3-3: L1 beam reporting for performance monitoring for NW-side model is supported. Proposal 4-1: to control feedback overhead, beam reporting for BM Case-1 consists of - Indication of the strongest beam index - The strongest beam’s RSRP - Bitmap to indicate un-omitted beams - Differential RSRPs for uno-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-2: to control feedback overhead, beam reporting for BM Case-2 consists of - Indication of the strongest beam index among all occasions - Bitmap to indicate un-omitted/omitted beams - Alt. 1: bitmap size equals to the number of set B beams across occasions - Alt. 2: bitmap size equals to the number of set B beams at a single occasion - The strongest beam’s RSRP - Differential RSRPs for un-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-3: to control feedback overhead for the UE-side model, beam reporting for BM Case-2 consists of - Indicating a subset containing top beams across time instances - Indicating a bitmap of selected top beams at time instances, the bitmap is over the cardinality of the subset by the number of future time instances. Proposal 5-1: for AI/ML beam management, the effective time for beam reporting has the CSI reference source as the reference instead of beam report time. |
| Meta | Proposal 3: For BM Case 2 measurement report, multiple time instances with a configured number of beams per time instance should be supported and the absolute value of time instances can be derived by NW based on periodicity of measurement RS transmissions. Proposal 4: For training data collection for NW sided model, support L1-RSRP reporting for a configured set of beams using higher layers. |
| HONOR | - For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support: - L1-RSRPs and corresponding beam information up to M beams within X dB gap to the largest measured value of L1-RSRP if the number of beams within X dB gap to the largest measured value of L1-RSRP is larger than N, - and otherwise L1-RSRPs and corresponding beam information of Top N beam(s) with largest N measured value(s) of L1-RSRP(s) of a measurement resource set. M, N and X are configured by gNB, with N<M <the size of measurement resource set. - For NW-sided model, for the report of beam information, support both of Option 1 and Option 2, the choice of which can depend on the number of reported beams and the size of resource set for measurement. - Option 1: CRI/SSBRI of a measurement resource set - Option 2: a bit map to indicate the reported beams of a measurement resource set and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set |
| RUIJIE | Proposal 5: For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, - Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, where X and M are configured by gNB - FFS: whether/how to report number of reported beams |
| meta | - For BM Case 2 measurement report, multiple time instances with a configured number of beams per time instance should be supported and the absolute value of time instances can be derived by NW based on periodicity of measurement RS transmissions. - For training data collection for NW sided model, support L1-RSRP reporting for a configured set of beams using higher layers. |

### Issue #6.0 Max number of reported beam & Max number of measured RS

| Company | proposals |
| --- | --- |
| Futurewei | Proposal 2: For Rel-19 AI/ML-based BM, at least for inference for network-sided AI/ML model of BM-Case1, for the content in a beam report in L1 signaling, - Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and the number of reported beams is indicated in the beam report. - The beam information is CRI/SSBRI. - Increase the maximum value M from the existing “4” to “8” as a starting point. |
| Huawei/HiSi | Proposal 1: For training/monitoring data collection for both NW-side model and UE-side model, RAN1 to discuss the potential mechanism to enable the UE to perform CSI measurements on larger sizes of beam set(s), including: - Alt 1: The beam set(s) for measurement consist of multiple resource sets each with legacy size (up to 64) of resources. - Alt 2: The beam set(s) for measurement consist of one resource set with increased size of resources, e.g., up to 256. Proposal 2: For the maximum number of reported beam related information in one report, - The max number of reported beams can be the same as the number of beams in the measured beam set(s), e.g., up to 256. - Consider BM-Case 1 as a starting point. For BM-Case 2, RAN1 needs to decide on whether or not to include multiple past time instances in one report. |
| Ericsson | (UE sided model) At least extend the aPeriodic resource configuration to support a resourceSet with 64 NZP CSI-RS resources - FFS on whether to support a resourceSet larger than 64 resources for aPeriodic or periodic measurements |
| Nokia | - Configurable M values can be 4, 6, 8, 12, 16, 32. |
| Interdigital | For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances. |
| ITL | Proposal 6: For NW-sided model inference, the max number of M (e.g. up to 256) reported beam related information in one report can be configured by the NW based on UE capability signaling. It does not need to be varied based on other reporting content. |

maxNrofAP-CSI-RS-ResourcesPerSet INTEGER ::= 16

maxNrOfCSI-AperiodicTriggers INTEGER ::= 128 -- Maximum number of triggers for aperiodic CSI reporting

maxNrofReportConfigPerAperiodicTrigger INTEGER ::= 16 -- Maximum number of report configurations per trigger state for aperiodic reporting

maxNrofNZP-CSI-RS-Resources INTEGER ::= 192 -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources

maxNrofNZP-CSI-RS-Resources-1 INTEGER ::= 191 -- Maximum number of Non-Zero-Power (NZP) CSI-RS resources minus 1

maxNrofNZP-CSI-RS-ResourcesPerSet INTEGER ::= 64 -- Maximum number of NZP CSI-RS resources per resource set

maxNrofNZP-CSI-RS-ResourcesPerSet-1-r18 INTEGER ::= 63 -- Maximum number of NZP CSI-RS resources per resource set minus 1

maxNrofNZP-CSI-RS-ResourceSets INTEGER ::= 64 -- Maximum number of NZP CSI-RS resource sets per cell

maxNrofNZP-CSI-RS-ResourceSets-1 INTEGER ::= 63 -- Maximum number of NZP CSI-RS resource sets per cell minus 1

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16 -- Maximum number of resource sets per resource configuration

maxNrofNZP-CSI-RS-ResourcesPerConfig INTEGER ::= 128 -- Maximum number of resources per resource configuration

maxNrofZP-CSI-RS-Resources INTEGER ::= 32 -- Maximum number of Zero-Power (ZP) CSI-RS resources

maxNrofZP-CSI-RS-Resources-1 INTEGER ::= 31 -- Maximum number of Zero-Power (ZP) CSI-RS resources minus 1

maxNrofZP-CSI-RS-ResourceSets-1 INTEGER ::= 15

maxNrofZP-CSI-RS-ResourcesPerSet INTEGER ::= 16

maxNrofZP-CSI-RS-ResourceSets INTEGER ::= 16

Agreement

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement (RAN 1 #117)

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

  - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

  - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRPof a measurement resource set

  - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

  - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

  - FFS on beam information

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

##### Proposal 6.0-1a:

For both NW-side model and UE-side model,

- Alt 1: extend the maxNrofNZP-CSI-RS-ResourcesPerSet (i.e., Maximum number of NZP CSI-RS resources per resource set) to 256.

- Alt 2: At least for UE-sided model, for monitoring and data collection (when applicable), up to 4 resource sets can be configured for Set A in one CSI report configuration.

| Companies | Y/N, Alt1 vs Alt 2 | Commnets |
| --- | --- | --- |
| FL0 |  | Let’s see whether this is needed or not. |
| Huawei, HiSilicon | Either Alt1 and Alt2 is OK. | Given large size of overall beams at gNB, it is essential to extend the number of measured beams to 256 for training data collection. Given less spec effort, Alt 1 is slightly preferred. In addition, Alt 2 should be also applicable to NW side model, right? Change: - Alt 2: At least for UE-sided model, for monitoring and data collection (when applicable), up to 4 resource sets can be configured for Set A in one CSI report configuration. |
| OPPO |  | We are open for the discussion, but it would be better to clarify the purpose of configuring large number of beams, i.e. 256 in this case. Is it for training, inference and/or monitoring? From our memory, it is obviously higher than the values evaluated in Rel-18 SI phase. |
| Fujitsu |  | Support Alt 1. And it should be applicable for periodic/semi-persistent/aperiodic CSI-RS. |
| TCL |  | Either is fine. |
| vivo |  | It seems Alt 1 is more straight-forward. If Alt 1 and Alt 2 serve the same purpose, one Alt is sufficient. |
| QC | N | Not needed |
| MediaTek | N | We don’t think this is an issue limited to AI BM, but the overall CSI framework. We doubt this can be resolved in this WI. |
| Google |  | We do not see strong need for the proposals. |
| Spreadtrum |  | Open to disscuss. |
| Ericsson |  | Prefer Alt 1. The most limiting today is the maximum of 16 resources in the aPeriodic resource set. Hence, we suggest discussing the following proposal: Proposal At least extend the aPeriodic resource configuration to support a resourceSet with 64 NZP CSI-RS resources. |
| CATT |  | Considering the spec impact, Alt 1 is our first preference. There is still an issue for the case that the number of Set A beams is less than the max number of resources configured to measure L1-RSRP, but greater than the number of resources to measure L1-RSRP within a slot, it may need to configure the RS for Set A in several slots. In legacy L1-RSRP measurement, for periodic RS or semi-persistent RS, the periodicity and slot offset of each RS in a RS set can be configured in resource level. The RS set for Set A can consist of resources with different slot offsets, and legacy P/SP resource configuration can be reused. For aperiodic RS used for performance monitoring, the slot offset (aperiodicTriggeringOffset) is configured in resource set level. The resources for Set A may need to be transmitted in consecutive slots since the number of beams in Set A may be large. Rel-19 MIMO has supported to transmit the resources configured in a RS set within two consecutive slots for resource aggregation to attain more than 32 ports. This approach can be reused for aperiodic RS configuration for Set A. |
| Futurewei |  | We prefer Alt 1. |

##### Proposal 6.0-2a:

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, maximum value of M of reported beam in one report is configurable and can be 4, 6, 8, 12, 16, [32, 64]

| Companies | Y/N | Commnets |
| --- | --- | --- |
| FL0 |  | Let’s see whether this is needed or not. |
| Huawei, HiSilicon |  | M can be derived from a configurable ratio, namely R, to the number of measured beams. Change: For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set with L beams, maximum value of M of reported beam in one report is derived from M=, where R is configurable and can be 1/2, 1/4, 1/8, FFS other values. configurable and can be 4, 6, 8, 12, 16, [32, 64] |
| OPPO |  | If that’s for BM-Case2 and NW-side inference, we think it’s okay to report multiple time instances of Set B measurements to NW. Otherwise, we would like to have more discussion on its use case. |
| Fujitsu |  | Let’s start from inference firstly and keep for inference in the proposal, since the existing agreement is for inference. |
| QC | N | Not needed at this stage. Can come back when discussing UE capability. |
| Google |  | We also think it is better to clarify the purpose first. |
| Spreadtrum |  | Agree with Fujitsu. Keep for inference in the proposal to make the discussion more clearly. |
| Ericsson |  | Support. |
| Futurewei |  | We are in general fine with the proposal. |

### Issue #6.1 Beam information

##### (on hold)Proposal 6.1a:

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, the following options for beam information when M<the size of measurement resource set are considered:

- Option 1: CRI/SSBRI of a measurement resource set

  - Supported by: FW, Spreadtrum/UNISOC, CMCC, LGE, Sharp(configurable), HONOR, Fujistu

- FFS: Option 2: a bit map to indicate the reported beams of a measurement resource set and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

  - Note: combination of option 1 and option 2 are not precluded and condition (if any) can be studied

  - Supported by: ZTE, Huawei/HiSi, CMCC, ETRI, Sharp(configurable), HONOR

Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

| Companies | View |
| --- | --- |
| FL | To be discussed after undertand the max RS and Max M But pls feel free to show your prefernce |
| CATT | We also support Option2. If the CRI/SSBRI size is large due to large number of the RS form Set B, a bitmap indicating RS index of Set B can reduce the required bit number for beam information of a beam report. |

### Issue #6.2: Quantization and L1-RSRP omission

##### (on hold)(FL0)Proposal 6.2:

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling,

- Y dB (FFS: Y=3 and/or 4) quantization step(s) than the already supported legacy quantization step for differential L1-RSRP

- Y can be configured by network

- Note: smaller range(s) and larger quantization step(s) for absolute L1-RSRP in differential L1-RSRP reporting are not precluded and can be studied separately.

- Supported by: ITL

- Note supported by(Legacy): Huawei/HiSi, Spreadtrum/UNISOC

| Companies | View |
| --- | --- |
| FL | But pls feel free to show you view |

### Issue #6.3: L1-RSRP omission

##### (on hold)(FL0) Proposal 5.6:

For NW-sided model, for inference report, at least for training data collection, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, where X and M are configured by gNB

  - FFS: whether/how to report number of reported beams

| Companies | View |
| --- | --- |
| FL | Most of companies support data collection via L1. There is clear motivation for quantization step relaxation base on SI output. Why not? |
| NTT DOCOMO | Same comments as Proposal 5.5 in the previous meeting. We have concerns on the overhead reduction gain and large specificatoin impacts from the adaptive number of reported beams. |
| ZTE | Support. |
| HW/HiSi | This report is generally applicable, regardless of whether it is for inference or training, BM Case 1 or BM-Case 2. Suggested update For NW-sided model, for inference report, at least for training data collection, at least for BM-Case 1, the content in a beam report in L1 signaling, support - Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, where X and M are configured by gNB - FFS: whether/how to report number of reported beams |
| NEC | We have the same view with HW/HiSi. |
| CATT | Support. If the proposal can be supported, the UE can adapt dynamically the number of beams in one report based on the measurement results. Compared with the reporting with fixed number of beams, it can avoid reporting redundant beams when channel state is poor and reporting insufficient beams when channel state is good. Also, in Rel-18 SI, it has been observed that if the reported measurement within 14dB~20dB gap to the largest L1-RSRP, there is only about 2%~6% beam prediction accuracy degradation. |

### Issue #6.4: Data collection for training for NW sided model for BM-Case1

#### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Futurewei | Proposal 4: For Rel-19 AI/ML-based BM, for UE-sided model, at least for BM-Case1, for content in the report of inference results, do NOT support Opt 3 and Opt 4. - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP |
| Huawei/HiSi | Proposal 3: From RAN1 perspective, for NW-side model, for monitoring and/or training, conclude that L1 signaling can also be used. - Note: The conclusion can be interpreted that the agreement from RAN1#116 for the report of more than 4 beam related information in L1 signaling does not need to be limited for inference. - Note: Purpose, such as above “For NW-side model, for monitoring and/or training”, will not be specified in RAN1. Proposal 4: For report of data collection in higher layer for NW-side model for BM-Case 1 and BM-Case 2, the data format can be summarized as the following two types: - Type A: All L1-RSRPs of measured beams subject to a configured resource set are reported by UE. - Type B: Beam information and corresponding L1-RSRPs of beams subject to a configured resource set are reported by UE. - Note 1: NW can obtain the model input (Set B) and label (Set A) from two separate CSI report configurations, and pair them by implementation. - Note 2: Purpose, such as above “For NW-side model for BM-Case 1 and BM-Case 2”, will not be specified from RAN1 perspective. |
| Speadtrum, UNISOC | Proposal 4：For the content for data collection for training for NW-sided model via higher layer signaling, all three types of contents for each instance should be supported. |
| China Telecom | - Proposal 7: For UE-sided model at least for BM Case-1, for inference results report, not support to configure only one resource set for Set B. - Proposal 8: For UE-sided model, at least for BM-Case1, for content in the report of inference results, support Option 2, i.e., Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams The predicted Top-K beams are the K beams with the highest predicted RSRP above a proper probability. The beam information is beam index related information. - Proposal 9: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, the reported RSRP is the predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. |
| Tejas | Proposal 25: For the UE-sided model in BM-Case 1, the ranking information of the predicted Top K beams is conveyed through the order of reported information. The report should be structured such that the beam information is ranked based on the predicted RSRP or the probability of being the Top 1 or Top K beam. • The first beam listed should be the one with the highest predicted RSRP or the highest probability of being the Top 1 beam. |
| GOOGLE | Proposal 3: Support to report a confidence level indicator for L1-RSRP report to indicate the maximum L1-RSRP measurement error for each beam. Proposal 6: For beam report based on UE model inference for SD beam prediction, support the followings on the remaining open issues: - The selection of the “top-K” beams are up to UE implementation - Spec only defines the number of reported beams - The reported RSRP should be defined as the predicted RSRP based on a reference transmission power - Support both option 3 (probability information report) and option 4 (confidence information report) - For BM case 2, the differential RSRP is calculated based on the reported absolute RSRP |
| CMCC | Proposal 1: From RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, type 1 and type 3 for each instance can be supported: - Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE - Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE Proposal 2: L1 signaling can be supported for NW-sided training data collection. |
| Transsion | Proposal 5: Regarding UE-sided model for BM Case-2, for absolute RSRP and differential RSRP report over multiple time instances, time instance information of largest RSRP can be reported explictly. Proposal 6: Regarding UE-sided model for BM Case-2, for the reference time to determine the earliest time instance from the future time instances, the following options can be considered: - Option 1: Based on the time domain resource for the report. - Option 2: Based on the CSI reference resource corresponding to the report. - Option 3: Based on the last transmission occasion of the CSI-RS/SSB resource in Set B for the report Proposal 7: Regarding configuration for the measurements for BM-Case 2, study to trigger aperiodic resource set in multiple time instances. |
| Intel | Proposal 3: For a UE-side AI/ML model, for BM-Case 1/2, explicit configuration of set A and set B should be supported and the configuration may be based on UE capability and any UE-side conditions related to supported model and input/output types. Proposal 4: For a UE-side AI/ML model, for inference results reporting for BM-Case 1/2, support: • Approach 1: One CSI-ResourceConfigId each is configured for Set B, Set A respectively. • Approach 2: One CSI-ResourceConfigId is configured for Set B only and Set A is configured using a new IE which lists the resources which form the QCL sources for DL Tx beams which are mapped to the output of the UE-side model, e.g., o Alt. A: Set A is defined as a set of NZP-CSI-RS-ResourceId or SSB-Index and the UE can derive the TCI states based on the TCI state list configuration where the resources in set A can be QCL sources. O Alt. B: Set A is defined as a TCI-State list where each TCI state corresponds to a DL Tx beam and the qcl-info in the TCI state contains a source RS which provides the QCL source (Type D). This source RS can be an NZP-CSI-RS-ResourceId or SSB-Index. • Set B may be configured via an association to set A or independently. Proposal 5: For a UE-side AI/ML model, for BM-Case 2, UE may be configured with an observation/measurement window by the network. • The observation/measurement window can be provisioned to a UE based on configuration of a number of time instance(s) corresponding to a number of slots or CSI measurement occasions for Set B of beams. Proposal 16: For model inference of UE-side AI/ML model, for reporting predicted beams and related RSRP (Opt-2), differentiation between measured L1-RSRP and predicted RSRP from a model is needed. It can be based on an additional bit of information in the report per reported beam. Proposal 17: Opt. 3 and Opt. 4 identified during RAN1 #116 towards defining contents of inference results reporting for UE-sided models are discussed in context of performance monitoring and discussions are not duplicated in context of contents of report with inference results. Proposal 18: For a UE-side AI/ML model, for BM-Case 1/2, the number of best predicted beams may be configured to the UE by the network subject to UE capability. Accordingly, the L1 report from UE to gNB after inference may be defined based on the configured value. |
| ZTE | For content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, both three types of contents can be considered to serve for different training strategies at the NW side. Proposal 5: For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB. Proposal 6: For the reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, the legacy two parts encoding method for CSI reports should be reused as much as possible. Proposal 7: For overhead reduction, support to specify threshold based beam reporting method with configurable minimum and maximum number of reported beam related information in a single report. Proposal 8: For overhead reduction, support specification enhancements for data omission among samples (e.g., according to data quality). |
| Ericsson | - Proposal 7 For UE-sided model inference, regarding the FFS on beam information, conclude that such information at least comprises the CSI-RS resource indicator (CRI) and SSB resource indicator (SSBRI) - Proposal 8 For UE-sided model inference, regarding the FFS on potential down selection of option 3 and 4, - • support option 3: probability related information of predicted Top K beam(s) - • support option 4, the confidence is for example a prediction interval where the UE prediction with a certain probability resides within (e.g. with 90% confidence, the RSRP prediction is within interval [RSRP1, RSRP2] Proposal 9 For UE-sided model inference, conclude that value of K could be adaptive and based on the UE-sided model output (e.g. support that UE report K beams so the probability of one of them being strongest is above a certain threshold) Proposal 10 For UE-side AI/ML model inference, for BM-Case2, support that UE can update reported inference results of N future time instances after such report. |
| Vivo | - Proposal 27: For inference, for UE-side model, support to report predicted L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest predicted value of L1-RSRP, as well as the number of reported beams. - Proposal 26: For inference, for UE-side model, when UE reports L1-RSRP of Top-k beams predicted from Set A, and for those that belongs to Set B, the UE reports the actually measured RSRP corresponding to the beam. - Proposal 27: For inference, for UE-side model, support time stamp information as beam content for BM-Case2. - Proposal 28: For inference, for UE-side model, support time domain compression of beam resource indication to further reduce report overhead with a report including results of multiple occasions. - Proposal 29: For inference, for UE-side model, support beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams per predicted instance for content in the report for BM-Case 2. - Proposal 30: For inference, for UE-side model, support new report quantity to differentiate between AI-based predicted report with legacy measured report. - Proposal 31: For inference, for UE-side model, support to report predicted L1-RSRP of indicated beam(s), e.g. current beam indicated by TCI, with predicted L1-RSRP of top-k beams in one prediction report. - Proposal 34: For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions. |
| OPPO | Proposal 8: For UE-side model, support UE to report Alt.3 for inference, i.e. the beam information on predicted Top K beam(s) and probability information of predicted Top K beam(s). Proposal 9: For UE-side model, clarify the beam information of predicted Top-K beam(s) as SSBRI/CRI associated with Set A. - For BM-Case2 with UE-side model inference, support to enhance the differential RSRP reporting - UE packs the predicted beams and RSRPs from the 1st to the N-th (last) time instance and reports in a single beam reporting instance - For each time instance, differential RSRPs refers to the highest RSRP in the time instance - For BM-Case2 with UE-side model inference, support to indicate multiple unified TCI states for up to N future time instances with one-shot beam indication. |
| Fujitsu | Proposal 1: - For training data collection, at least the following information should be included: - Reference signal ID - Beam quality, e.g., L1-RSRP Proposal 2: - For training data collection, the model input data and the ground truth data should be included. Whether and how to map/associate the model input data with the ground truth data could be further discussed per sub-use case for beam management. Proposal 3: - Regarding training data collection for NW-side model, the UE could report all the L1-RSRP for all the beams in Set A if Set B is subset of Set A. And the UE could report all the L1-RSRP for all the beams in Set B and Set A if Set B is different from Set A. Proposal 4: - For training data collection, RAN1 to further discuss the quantization for the model input data and ground truth data. High-resolution quantization and non-differential RSRP could be considered. |
| CATT | Proposal 13: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported beam information can be the RS indicator(s) (e.g., legacy CRI/SSBRI) or the pre-defined beam index of the predicted Top K beam(s). Proposal 14: For UE-sided model, at least for BM-Case1, for report content of inference results, the reported RSRP type of predicted Top K beam(s) can be configured by gNB with the following options: - Option A: Predicted L1-RSRP; - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. Proposal 15: For UE-sided model for BM-Case2, for inference report, support to report largest L1-RSRP among multiple beams over multiple time instances and other differential L1-RSRP of multiple beams over multiple time instances with a reference to the largest L1-RSRP in a pre-defined order. |
| Lenovo [14] | Proposal 6: Specify a common beam report configuration to support both BM-Case1 and BM-Case2 for UE-side inference. Proposal 7: Support periodic beam report, semi-persistent beam report on PUCCH, semi-persistent beam report on PUSCH, aperiodic beam report with AI prediction. Proposal 8: Considering the prediction window for BM-Case2 with the following modes: - Mode 1: - Mode 2: is the CSI reference resource for the beam report in slot n. And is a value configured by RRC. Which mode is used can be configured by RRC according to UE capability. Proposal 9: For UCI overhead reduction on inference result reporting for BM-case2, to support that - differential RSRP quantification with reference to the largest value among all future time instances - reporting unique beams of future time instance and corresponding time-stamp indicator. - an indication for informing whether to report all beams or unique beams. Proposal 13: For a beam report associated with AI inference, the UE indicates that the reported beams are predicted beams or measured beams in the beam report. Proposal 17: Study mechanism to adapt/change the Set B configuration or the beam selection for AI/ML model input for RS and UCI report overhead reduction. |
| Sony | Proposal 1 : For BM-Case 2, the network can dynamically indicate the time window size for data collection and output to the UE separately, or the UE can dynamically report its preferred time window size to the network. Proposal 2 : For BM-Case 2, the number of consecutive slots within each future time instance can be dynamically indicated by the NW, or the UE can dynamically report its preferred number of consecutive slots to the network. Proposal 3 : For the UE-side model, considering the contents of the report of inference results, we support Options 1, 2, and 3. |
| Panasonic | Proposal 1: For content of data collection for NW-sided model training via higher layer signaling, for both BM-Case 1 and BM-Case2, for each per instance, support L1-RSRPs measured based on one resource set (for Set A and Set B) configured to UE - Differential L1-RSRP reporting is supported with legacy quantization steps and range - FFS: Details on reporting configuration |
| Hyundai [18] | Proposal #2 - Discuss whether not only predicted RSRP but also measured L1-RSRP are considered to select Top K beams for report content of inference results for UE-sided model BM-Case1. Observation #1 - The difference between measured L1-RSRP and predicted L1-RSRP can be beneficial for NW to monitor channel condition and UE’s inference. Proposal #3 - Discuss whether/how UE reports difference between measured L1-RSRP and predicted RSRP or assistance information if the beam is configured for corresponding measurement instead of reporting measured L1. |
| Nokia [19] | Proposal 16: For BM-Case1 and BM-Case2, RAN1 to discuss whether NW-sided data collection (for training) for beam prediction is supported via CSI reporting, and further discuss the following options, - Option 1: Use CSI report to measure L1-RSRPs for all resources in one or two measurement resource sets. - NW assumptions on Set B and Set A combinations may decide whether it is one or two sets. - Option 2: Use CSI report to measure L1-RSRPs for all resources in a first measurement resource set, and Top-1/K beam information in a second measurement set. - FFS: assumption on Rx beams to measure L1-RSRPs for the CSI report. - “best” or “Quasi-optimal” Rx beam should be selected by the UE and adopted for the measurements. |
| Samsung | Proposal 9. For UE-sided model, at least for BM-Case1, for the definition of the beam information on predicted Top K beam(s) among Set A, consider the following options: - Option 1. The beam information is predicted SSBRI/CRI. - Option 2. The beam information is predicted beam indicator. Proposal 10. For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, support both Option A and Option B. Proposal 11. For UE-sided model, at least for BM-Case1, for content in the report of inference results, support: - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam Proposal 12. For UE-sided model, at least for BM-Case1, for content in the report of inference results, further study the method to convey the ranking information of the predicted Top K beams in case of K > 1. Proposal 13. For UE-sided model inference, for the reporting of predicted Top K beam(s) among Set A, - K is configured by CSI-ReportConfig and the maximum configurable value of K is subject to UE capability. Proposal 14. For UE-side AI/ML model inference, for BM-Case2, to report inference results of N future time instance(s) in one report - Each of the N future time instance(s) consists of P (P≥1) consecutive slots - FFS: How to determine P - For the reference time to determine the earliest time instance from the N future time instance(s), consider the following options: - Option 1: Based on the time domain resource for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the transmission occasion of the CSI-RS/SSB resource in Set B for the report - FFS: whether the above options are also applicable to the time instance(s) other than the earliest one - FFS: If N>1, whether the time domain separation between two adjacent time instance(s) from the N future time instances are the same - FFS: How to define measurement window(s) for the inference results of the N future time instance(s) Proposal 15. For UE-side AI/ML model inference and BM-Case2, support the following: - The differential L1-RSRP value is with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. - FFS: How to identify the largest measured L1-RSRP value if there are multiple time instance. |
| ETRI | Proposal 13: For the UE-side model, support the CSI report format for temporal domain beam prediction, including the optimal K beam information along with RSRP information from multiple time instances. Proposal 14: For the UE-sided model, support the following differential L1-RSRP methods for temporal domain beam prediction. - Relative to a single absolute RSRP value at each time point. - Relative to the absolute RSRP values at the first time point. Proposal 15: Support the indication field for model inference when an AI/ML model is located on the UE-side. Proposal 16: Support the CSI report format with only the predicted beam indices without L1-RSRP for model inference when an AI/ML model is located on the UE-side. |
| DOCOMO | Proposal 5: Support either one of the following beam representation ・Always CRI/SSBRI ・Bitmap or CRI/SSBRI that achieves smaller overhead according to the number of reported beams, the number of measured beams, and the number of reported future time instances where specification should not support both alternatives, and separate UE capabilities for bitmap or CRI/SSBRI should not be introduced. Proposal 6: Reported time instance should be determined based on the parameters indicating the following for beam prediction. ・interval between two future time instances. ・offset between the last referred transmission occasion and the first future time instance. Proposal 8: For BM-case2, UE should report only if receiving multiple CSI-RS transmission occasions no later than CSI reference resource after CSI report (re)configuration Proposal 9: Support the following payload overhead reduction on the reporting of predicted results at multiple time instances. ・Absolute RSRP representation for the combination of CRI/SSBRI and time instance achieving the largest RSRP. ・Differential RSRP representation from the above absolute RSRP for the remaining combinations of CRI/SSBRI and time instance. Proposal 7: Beam information on predicted top K beam(s) should be represented by CRI/SSBRI to follow the existing specification. Proposal 8: Since the probability of predicted top K beam(s) and the confidence of predicted RSRP represent the performance metric of beam prediction, the necessity of these information should be discussed in performance monitoring not in inference result reporting. Proposal 9: Reported time instance should be determined based on the parameters indicating the interval between two future time instances and the offset between the last referred transmission occasion and the first future time instance for AI/ML. Proposal 11: For BM-case2, UE reports only if receiving multiple CSI-RS transmission occasions no later than CSI reference resource after CSI report (re)configuration Proposal 12: Support the following payload overhead reduction on the reporting of predicted results at multiple time instances. ・For the combination of CRI/SSBRI and time instance achieving the largest RSRP, absolute RSRP representation ・For the remaining combinations of CRI/SSBRI and time instance, differential RSRP representation from the above absolute RSRP Proposal 13: Enhancements of CSI processing units should be considered for beam prediction. |
| Qualcomm | ##### Proposal 6 For UE-side beam prediction, regarding how to configure associated ID within CSI framework, support configuring associated ID for Set A and Set B pairs (jointly) rather than Set A and Set B separately. #### Proposal 7 For UE-side beam prediction, regarding where to configure associated ID within CSI framework, support configuring associated ID within CSI-ReportConfig. - FFS: details of how to assign associated ID to configurations that include subsets of Set B and/or Set A and also different Set B patterns. - FFS: details of where to configure associated ID within CSI-ReportConfig. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 15: The use of a predicted beam that is not measured/received by the UE for beam indication is supported. Proposal 16: Beam indication for one or more future time instances is not supported. Proposal 17: For UE-sided models, for inference, study the UE reporting its inference time to the gNB. |
| ITL | Proposal 12: It can be considered the reporting the preferred DL RS configurations for the data collection for UE side training when requesting training via UE signaling. Proposal 13: For data collection for UE side model inference, consider UE to send a request for preferred DL RS configuration and/or DL RS transmission Proposal 14: For UE side model inference, existing specifications should be the baseline for the configuration or triggering CSI-RS/SSB of Set B Proposal 15: For UE side model inference, it is proposed to support that both resources for Set A and resources for Set B are configured as two separate resources - FFS on additional signaling to indicate the association Proposal 16: For UE-sided model, beam information for predicted beam (e.g., model ID and CRI, SSBRI) can be reported by a UE based on the existing framework for CSI reporting as baseline. The predicted Top-K beams can be determined at least based on L1-RSRP(s) and a threshold. Proposal 17: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, it is proposed to support both Option A and Option B. Proposal 18: For the reference time of the time instance(s) in the UE-side model for BM-Case 2, it is proposed to assume that the CSI reference resource corresponding to the report is used to determine the earliest time instance (or each time instance) from the N future time instances as the starting point of the discussion. Proposal 19: For UE-sided model inference in BM-Case2, - the earliest time instance can be determined by applying at least an additional offset value based on the reference time - each of N future time instances can consist of at least one slot - the time stamp of future time instance(s) can be implicitly indicated to NW. Proposal 21: For UE-side model, it can be supported to use beam indication for Set A beams. |
| Huawei/HiSi [33] | Proposal 19: For the CSI report for the inference of a UE-side AI/ML model, the predicted beam ID(s)/RSRP(s) and the measured beam ID(s)/RSRP(s) need to be differentiated, e.g., by introducing an indicator included in CSI-reportConfig. Proposal 20: For BM-Case 2 with a UE-side model, the model output for N future time instances can be sent in one report. Overhead reduction techniques can be considered, e.g. model output compression with differential RSRP over temporal domain. The time stamp of the reports can be derived implicitly from the order of the prediction instances. Proposal 21: For BM-Case 2 with a UE-side model, investigate the supported resource types for the observation window: At least P/SP CSI-RS resources can be considered. Study whether/how to define the observation window. Study the CSI processing criteria to measure/process/store the observation instances. Aperiodic CSI-RS resources may not be applicable due to long observation window. Proposal 22: For BM-Case 2 with a UE-side model, for the reference time to determine the earliest time instance of the prediction window, consider Option 1 and Option 2 with higher priority. Option 1: Based on the time domain resource for the report. Option 2: Based on the CSI reference resource corresponding to the report. Time domain separation between two adjacent time instance(s) from the N future time instances are the same. Proposal 23: For AI/ML model inference at the UE-side under BM-Case 1 and BM-Case 2, support to report the predicted beam Ids/RSRPs (i.e., Max value of K) of more than 4 beams in one reporting instance, because It improves the beam prediction accuracy. It improves the generalization performance. It makes the functionality symmetric with the capabilities of a NW-side model. Proposal 24: For AI/ML model inference at the UE-side under BM-Case 1 and BM-Case 2, to reduce the reporting overhead, consider to report a selected number of beams (i.e., Top-K value) determined by the UE based on output RSRP/probability threshold. |
| Xiaomi | Proposal 4-1: Support to report the predicted L1-RSRP if the beam is not configured for corresponding measurement, and report the measured L1-RSRP if the beam is configured for corresponding measurement for UE-side model inference. Proposal 4-7: If the measured beam information of the last history measurement time instance is reported, support to report the predicted beam information together for UE-side model inference in BM Case 2. Proposal 4-7: Support to report the measured beam information of the history measurement time instance for UE side model inference in BM Case 2 if set B equals to set A. Proposal 4-9: For UE-side model inference in BM-case 2, indicate the time instance with the strongest L1-RSRP, and then report beams in the time instance with the strongest L1-RSRP first and other time instance with ascending order. |
| Kyocera [35] | Proposal 6 For the UE-side AI/ML model, regarding the content of the report inference results, support using options 1 through 4. Further study the benefits and gains of adopting options 3 and 4, considering the effects of quantization methods on system throughput and considering the additional overhead incurred. Proposal 7 Regarding the definition of the confidence information, for option 4 of a UE side AI/ML model report content for inference, consider the following definition: - The confidence information represents a range within which the predicted RSRP of a beam in Set A is expected to fall a certain percentage of the time if the same input sample is reused to derive the AI/ML model. FFS on the details of the calculation of the confidence interval. Proposal 8 For a UE-side AI/ML model report content for inference, further study the applicability of adopting Option 4, including its pros and cons, in terms of the associated UL overhead and its potential to provide the gNB with information that could assist in configuring the performance monitoring set. Proposal 7: For a UE side AI/ML model, for the FFS on beam information, support using CRI/SSBRI as a starting point. Proposal 10 For a UE-side AI/ML model, the definition of the predicted top-K beams is related to the definition of the data set labels and the method of label collection. If the model output is specified (e.g., beam information such as CRI/SSBRI), it may be unnecessary to define the top-K predicted beams, and how they are generated during training, explicitly, and this could be left to implementation. Proposal 12: For a UE-side AI/ML model inference report on beam ranking, where the UE is required to report the top-K beams out of N beams in Set A, the following methods can be adopted: - CRI-based Reporting: The UE should report the beam information in a ranked manner using a CRI vector. This involves reporting a K-dimensional vector of CRIs, where the entry corresponds to the strongest beam. - Bitmap Reporting: The UE should report the bitmap K times, with each instance corresponding to a different beam. For example, the UE would report a matrix with K rows and N columns, where the row contains zeros except for the entry corresponding to the strongest beam. FFS: The feasibility of the bitmap compared to the CRI vector signaling with respect to the associated UL overhead Proposal 11: For a UE side AI/ML model probability information reporting, support one decimal precision probabilities where only 4 bits (bit width) are required to represent the probability information per beam. The choice of rounding up/down the probability values is left up to the UE’s implementation. |
| LGE [36] | Proposal #9: For supported Option 1 and Option 2, support K=4 for the max value. - Considering the case that predicted beam is in Set A but not in Set B, reported beam information can be based on the relation/association between Set A beams and Set B beams Proposal #10: For predicted RSRP report, confidence/probability information may be helpful for NW to decide whether/how to use the reported RSRP. Further study whether the information is per model/functionality, per report, per time instance, or per report parameter. Proposal #11: Support to report inference results of N(N>=1) future time instance(s) in one report. - Maximum value of N can be more than 1 - Maximum value of N can be reported by UE capability, and M(M<=N) value for inference results reporting can be configured by gNB - Further consider enhancement on RSRP quantization for UCI overhead reduction Proposal #12: For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances. - In the inference result report, an indicator for the time instance containing the largest predicted RSRP in addition to the best beam indicator is reported by UE - Other than largest predicted RSRP value across every multiple time instances, predicted RSRP value for each beam is expressed by differential RSRP value (e.g., 4 bit) Proposal #12: For temporal DL Tx beam prediction, information on time-variation of RSRP can also be included in the report. Proposal #14: For temporal DL Tx beam prediction with UE-sided models, following beam reporting enhancements can be considered: - Report of beam dwelling timefor each future time instance - Report of beam(s) for current time instance for fallback operation Proposal #14: Support Option A or Option C (new) for the RSRP of predicted Top K beam(s) in the report of inference results. - Option A: Predicted RSRP - Option C: Not specify whether to report predicted RSRP or measured L1-RSRP when both RSRPs are available at UE side, i.e., leave this case up to UE implementation Proposal #15: For UE-sided model inference, consider reporting of performance monitoring related information such as actual Set A measurement or performance monitoring output/result together with predicted beam information on Set A, where the performance monitoring related information may be reported with longer periodicity. |
| NEC | Proposal 16: Support selecting Top-K beam(s) according to some pre-defined rules (e.g., a sum probability of being Top 1 or Top K beam higher than a threshold, predicted L1-RSRP higher than a threshold) as the reported predicted beams. Proposal 17: For BM-Case1 and BM-Case2, the value of K (i.e., the number of predicted beams to report) should be determined and provided to NW by UE. Proposal 18: For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, Option B should be supported, i.e., Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement. |
| MTK | Proposal 32: For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study how to facilitate UE to report various number of Top-K beams in one beam report (K ≤ nrofReportedRS) as AI/ML model output. |
| Rakuten | Proposal 5: In a UE-sided model, whenever some beams have to be excluded from prediction, the gNB indicates the list of beams to be excluded from prediction, along with the time duration. |
| InterDigital | Proposal 12: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B. Proposal 13: Information about the time stamp for measurement instances should be supported. Proposal 14: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline. Proposal 15: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration). Proposal 16: Support both ‘Option A: Predicted RSRP’ ‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’ Additionally, support a UE based selection procedure to report according to Option A or Option B. Proposal 19: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams). Proposal 18: Indicating configuration information associated with Set B to UE should be supported. |
| Apple | Proposal 4-2A: to control feedback overhead and quantization error for UE-side model, set A beam reporting for BM Case-2 consists of - Indication of the strongest beam index among all prediction occasions - Bitmap to indicate un-omitted/omitted beams - Alt. 1: bitmap size equals to the number of set B beams across prediction occasions - Alt. 2: bitmap size equals to the number of set B beams at a single prediction occasion - The strongest beam’s RSRP - Differential RSRPs for un-omitted beams except the strongest beam - Indication of the number of un-omitted beams Proposal 4-3: to control feedback overhead for the UE-side model, beam reporting for BM Case-2 consists of - Indicating a subset containing top beams across time instances - Indicating a bitmap of selected top beams at time instances, the bitmap is over the cardinality of the subset by the number of future time instances. |
| Sharp | For UE-side model, at least for BM-Case 1, for content in the report of inference results, support Opt 3: beam information on predicted Top K beams among a set of beams and probability related information of predicted Top K beam(s) among a set of beams. The quantization step size for the probability information can be configurable by the gNB. For UE-side model, for the report of inference results, support a single L1-RSRP corresponding to largest predicted value across multiple time instances, with the remaining predicated RSRP being reported as differential RSRPs. Proposal 15 For UE-side model for BM Case 2, a timing pattern can be configured to indicate the specific timing that the inference results are to be included in the inference report. |

##### (on hold)For #120 Proposal C

Send LS to RAN 2, to inform RAN 2 that from RAN 1 perspective, for content for data collection for training for NW-sided model via higher layer signaling, for BM-Case1, the following three types of contents for each instance can be considered:

- Type 1: All L1-RSRPs of measured based the resources (for Set A/B) configured to UE

- Type 2: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information of K beams based on some resources (for Set A) configured to UE

- Type 3: All L1-RSRPs measured based on resources (for Set B) configured to UE, and beam information and L1-RSRPs of K beams based on some resources (for Set A) configured to UE

- Wherein the K beams are the beams with the largest K L1-RSRPs based on the resources (for Set A) in one instance

- K is configured by NW.

Note: RAN 1 does not intend to specify the intention of the resources, i.e., (for Set A), (for Set B), (for Set A/B). It is up to RAN 2 on whether to specify the purpose for data collection, i.e. “collection for training for NW-sided model”, “for BM-Case1”. In addition, RAN 1 has no consensus on whether/how to down select the Types.

Also, for BM-Case2, which is more complicated.

(FL0) Proposal

For content for data collection for training for NW-sided model via higher layer signaling,

- Support L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, where X and M are configured by gNB

  - FFS: whether/how to report number of reported beams

### Issue #6.5: Measurement report for BM-Case2

##### (onhold) Proposal 6.1:

For NW-sided model, for BM-Case 2, support to report the measurement results of multiple transmission occasions of resources for channel measurement at least for the RS types including P CSI-RS, SP CSI-RS and SSB in one L1 report.

- Differential L1-RSRP reporting over multiple time instances is used

- FFS on how to determine the multiple occasion(s)

- FFS whether include the time stamp information explicitly or implicitly in the report

- FFS on the max number of RS resources to be reported for one transmission occasion

- FFS on the max number of the multiple transmission occasions.

- Note: Purpose, such as above “For NW-sided model, for BM-Case 2”, will not be specified in RAN 1 specifications

# Beam indication

Agreement(RAN 1 #116) - For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

### Summary from the contributions

| Companies | Proposals |
| --- | --- |
| Huawei/HiSi | Proposal 15: For indicating the second round measurement of the predicted Top-K beams which may vary over time, study the following alternatives: - Alt.1: Reuse legacy A-CSI indication of the selected CSI report with pre-configured resource set for second round measurement of the Top-K beams. - Alt.2: Dynamic indication of RS resources which constitute the resource set of the Top-K beams. - Alt.3: Dynamic indication of the QCL relationship for the pre-configured RS resources second round measurement of the Top-K beams. Observation 4: In legacy, switching delay is needed for a TCI state to switch from unknown state to known state which has a corresponding testing requirement to UE, e.g., to perform Rx beam refinement. Observation 5: For UE-side model, regarding the TCI state of a predicted beam which is subject to unknown state: - If the model is testable at RAN4, it is up to RAN4 to decide whether to introduce a new TCI state/timeline for the predicted beam along with UE requirement. - If the model is not testable at RAN4, there will be no performance requirement for the predicted beam. - From RAN1 perspective, regardless whether it is testable or not, it does not preclude the gNB to schedule the unknown state TCI predicted by UE. Proposal 16: For UE-side model, the existing definition/timeline of TCI states can be reused from RAN1 perspective as it does not cause ambiguity between UE and gNB on scheduling the predicted beam. Proposal 17: For BM-Case 2, do not support to extend the Rel-17 TCI state activation/indication signaling methods to activate/indicate N TCI states which are corresponding to N future time instances, because - Potential benefit of overhead saving (if any) is insignificant. - Overhead saving cannot be achieved under the following typical cases: - For Top-K>1, second round beam sweeping would be anyway needed before the future time instance. - PDSCH subject to the future time instance is scheduled by the same DCI indicating the TCI. - gNB may need to update/override the TCI state that is previously predicted before each corresponding future time instance. - Substantial impact on implementation complexity and RAN4 impact (e.g. an increased number of active TCI states). |
| Spreadtrum | Proposal 6 For BM-Case2, TCI indication framework should be reused by gNB, e.g., beams from multiple time instance can be indicated to UE by multiple beam indications respectively. |
| vivo | For inference, for UE-side model, support to report TRI (time resource indicator) instead of direct predicted beam resource indication scheme with implicit time stamp. TRI indicates where each of the unique reported beams locates in the future time occasions. |
| Ofinno | - Further discuss beam sweeping to address QCL assumption for predicted beams. If adopted, consider flexible indication of beams to be swept based on the gNB configuration/indication. - Consider an approach where gNB indicates spatially correlated Tx beams from Set B for each Tx beam of Set A: - UE may synthesize QCL properties of an unmeasured beam from the QCL properties of its spatially correlated measured beams. - FFS how to indicate the spatially correlated measured beams of an unmeasured beam. - Support to extend unified TCI framework to allow single beam indication for N future time instances. |
| GOOGLE | Proposal 9: Since the activated/indicated TCI based on SD beam prediction is usually an unknown TCI state, to reduce the latency for TCI activation/indication based on SD beam prediction, support the NW to trigger aperiodic CSI-RS resources QCLed with the SSB/CSI-RS configured as the QCL source in the TCI state. - UE measures time/frequency offset and Rx beam based on the aperiodic CSI-RS resources - UE can also measure the pathloss based on the aperiodic CSI-RS resources Proposal 10: To differentiate the TCI state for legacy beam indication and TCI state for beam prediction, support to configure separate TCI state pools for legacy beam indication and TCI state for beam prediction. Proposal 11: Support to configure the action delay for the TCI state for beam prediction. Proposal 12: For temporal beam prediction, the beam quality for current beam from an indicated TCI can be used for performance validation, and if none of the predicted beam(s) can provide better beam quality than current beam, the predicted beam(s) are assumed to fall to pass the performance validation. Proposal 13: Support UE feedback before the beam action time for performance validation for predicted beam in addition to the ACK/NACK for the TCI update signaling for temporal beam prediction. |
| CMCC | Proposal 13: Top K beam sweeping procedure can be introduced and is configurable by gNB. Proposal 14: The indication of Top K beam set with low signaling overhead needs further discussion. Proposal 20: The RS associated with TCI indication should be measured at least once before TCI application. TCI indication associated without RS in set A is not supported. |
| Intel [6] | Proposal 19: For BM-Case 1 and 2, RAN1 should consider beam indication of predicted beams which have TCI states that are not part of the set of MAC-CE activated TCI states. |
| ZTE | Proposal 24: For BM-Case2 (both UE-sided and NW-sided model), support to extend the Rel-17 TCI state activation/indication signalling methods to activate/indicate N TCI states which are corresponding to N future time instances. Proposal 25: Study enhanced QCL indication method for aperiodic RS resources for sweeping over the predicted Top-K beams. |
| Ericsson | - For enabling a P2 sweep based on predicted Top-K beams, RAN1 considers the following solutions: - Option 1: NW indicates that the corresponding beam IDs/TCI states for the P2 ResourceSet follows the UE predicted Top-K beams, - Option 2: NW configures a number of aPeriodicTriggerStates, and the UE provides Top-K predictions restricted to the valid TCI states matching one of the aPeriodicTrigger states, - Option 3: For both UE/NW sided model, NW configures dynamically as part of the MAC CE the beam IDs or TCI states that are part of the Top-K beam sweep. FFS on details. - Option 4: The NW uses legacy signaling, and in a best-effort manner configure a set of aPeriodicTriggerStates - Note 1: Option 2 and 4 would imply reduced performance since P2 cannot support all Top-K combinations - Note 2: NW indicating beam IDs could be used during monitoring - RAN4 is discussing the known/unknown TCI states (see R4-2420335). Since the impact on RAN1 is unclear regarding known/unknown TCI states, RAN1 should await input from RAN4. |
| Tejas | Proposal 37: For BM-Case 2, to extend the Rel-17 TCI state activation/indication signaling methods: - Maximum number (N) of joint TCI states: N can vary based on system capability, but initial studies suggest values between 2 and 4 are reasonable, considering complexity and signaling overhead. - Depending on traffic patterns and mobility scenarios, a higher N could be explored, but efficiency may degrade due to larger signaling loads and interference management. Proposal 38: For BM-Case 2, to extend the Rel-17 TCI state activation/indication signaling methods: - Time periods should align with UE mobility and channel coherence time. Proposals suggest dynamically adapting this based on real-time radio conditions using AI models to predict optimal timing windows, e.g., based on Doppler shifts, historical data, or network feedback. Proposal 39: For TCI state configuration in BM-Case 2, for Training and inference: - Use historic TCI state for predicting N future TCI states, along with Set B historic measurements. - TCI state are configured along with Set A, Set B and Associated ID. Proposal 40: For the beam indication, to ensure valid TCI states for Top-K - To ensure valid TCI states for Top-K measurements during inference, prioritize the Top-K measurements based on historical data and real-time conditions. A dynamic ranking of TCI states can be done, ensuring the highest probability of correct beam activation. |
| OPPO [10] | Proposal 3: For BM-Case2 with NW-side model, support that UE can be indicated with multiple TCI states (more than 1 joint TCI state or a pair of DL and UL TCI states) for multiple future instances. |
| Fujitsu | - For BM Case-2 with UE side model, RAN1 to discuss beam indication enhancement, for example, TCI states of multiple time instances could be indicated via one DCI. |
| CATT , CBN | Proposal 17: From RAN1 perspective, the existing TCI state switching delay requirements can be reused for the predicted RS resource of UE sided-model. Proposal 18: For TCI indication of BM-Case2, there is no need to extend the Rel-17 TCI state activation/indication signaling methods to activate/indicate TCI state indication of multiple future time instances. Proposal 29：Further study how to measure the predicted Top-K beams in the CSI framework. - Option 1: Extend the maximum number of existing trigger state. - Option 2: Introduce a dynamic QCL relations indication scheme for predicted Top-K beams. |
| TCL | Proposal 6: The following enhancement on the TCI framework should be considered to support the AI/ML BM. - Additional TCI state ID dedicated for AI/ML BP should be introduced. - New QCL types is indicated in TCI state to associate the RS sets corresponding to Set A and Set B beams. - At least for BM-Case2, timing information related to different predicted beams should be configured to the UE using RRC signaling, e.g., included in the TCI state information. |
| InterDigital | Proposal 9: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern. |
| Panasonic | Proposal 3: For NW-sided model, support to extend Rel. 17 TCI state activation signaling methods to activate TCI states of K predicted beams for N future time instances in BM-Case 2. The following 2 options can be considered: - Option 1: The TCI states of K predicted beams for N future time instances are included in a combined set of TCI states together with that of legacy BM. - Option 2: The TCI states of K predicted beams for N future time instances are included in a separate set of TCI states, compared to that of legacy BM. |
| Nokia | Proposal 4: For BM-Case1 with the UE-sided model, consider following enhancements/limitations/changes related to the applicability of the beam indication. - The applicability of the TCI indication for a channel/signal may be depended on whether the TCI indication is associated with a measured RS resource or predicted RS resource. E.g., extend followUnifiedTCI-State. - The UE considers a TCI indication associated with a predicted RS resource as known TCI state. - Check the feasibility of this with RAN4 Proposal 5: For BM-Case2 with the UE-sided model, extend the Rel-17 TCI state activation/indication signalling methods to activate/indicate N TCI states which are corresponding to future time N instances. - FFS: maximum number for N - FFS: Time periods that each indicated TCI state is applicable. Proposal 6: For BM-Case2 with the UE-sided model, consider enhancements/limitations/changes related to applicability of the beam indication. - The applicability of the TCI indications for a channel/signal may be depended on whether the TCI indications are associated with measured RS resources or predicted RS resources. E.g., extend followUnifiedTCI-State. - The UE considers TCI indications associated with predicted RS resources as known TCI state. - Check the feasibility of this with RAN4 Proposal 21: For BM-Case2 with the NW-sided model, consider extending the Rel-17 TCI state activation/indication signalling methods to activate/indicate N TCI states corresponding to future time N instances. |
| Samsung | Proposal 19. For UE-side AI/ML model, support beam indication for Set A beams. |
| Sharp | Proposal 14 For BM-Case 2, NW-sided model and UE-sided model, enhance unified TCI state framework to support beam indication of multiple future time instances. |
| Qualcomm [27] | #### Proposal 7 For UE-side beam prediction, regarding FFS on beam information on predicted Top-K beams, conclude that such information includes beam indices from Set A. - FFS: how UE reports such beam indices, considering the fact that Set A beams may not be based on RS’s that are actually transmitted. |
| Fraunhofer HHI, Fraunhofer IIS | Proposal 17: The use of a predicted beam that is not measured/received by the UE for beam indication is supported. Proposal 18: Beam indication for one or more future time instances is not supported. |
| ITL | Proposal 9: For beam/TCI indication, consider using Set B beams of which UE can measure and maintain it Rx beam for P-3, if the gNB directs a beam within Set A that is unknown to the UE as the TCI state. Proposal 10: For beam/TCI indication of BM-Case2(NW side model), consider extending the existing TCI direction method to multiple beams with the associated timestamp information for future time N instances. |
| Xiaomi | Beam indication Proposal 7-1: Support following two TCI state indication mechanism for TCI state indication of more than one predicted time instance. Option 1: reuse legacy TCI state indication with multiple MAC CE or DCI and each MAC CE or DCI indicates TCI state of one time instance. Option 2: enhance TCI state indication to indicate TCI state of more than one time instance and the application time gap between two adjacent TCI states can be configured semi-statically. |
| NEC | To enhance unified TCI state to indicate the predicted beam, support to configure RS in associated Set A as the QCL reference signal in the TCI state. If the predicted beam is not measured, NW can configure a reference signal with a measured beam pattern similar to the predicted beam as the QCL reference signal. For BM-Case 2, support to use one MAC CE or DCI to activate/indicate multiple (future) TCI states, and corresponding time period. Discuss how to reduce the signaling overhead of UE performing measurements of predicted beams to determine the suitable Rx beam. |
| MTK | Proposal 25: For AI/ML-based BM, at this stage, there is no further enhancement needed for beam indication based on unified TCI state framework. |
| Meta | Proposal 14: Consider enhancements to the unified TCI framework for indication of predicted beams which are not part of activated TCI states. |

### (on hold) Issue #7.1: Whether and how to support beam indication for multiple further time instances

Proposal 7.1:

For BM-Case2 (both UE-sided and NW-sided model), study on whether/how to extend the Rel-17 TCI state activation/indication signaling methods to activate/indicate N [joint] TCI states which are corresponding to N future time instances

- FFS: maximum number for N

- FFS: Time periods that each indicated TCI state is applicable.

| Supported companies | FL |
| --- | --- |
| Companies | View |
| FL | Only one companies expressed “NO” explicitly. But the proposal is for study whether and how. |
| NTT DOCOMO | Support the proposal. |
| OPPO | Fine with this direction. One minor comment on activated/indicated TCI state for N future time instances: for DL Tx beam prediction, it would be applicable to use joint or DL TCI states in our view. If that’s the case, could we suggest to add DL along with joint in the bracket? to activate/indicate N [joint/DL] TCI states which are corresponding to N future time instances |
| HW/HiSi | Not support this proposal. From our observation, more than just one company do not support this proposal. In addition to HW/HiSi, there are also for example MTK: “Proposal 25: For AI/ML-based BM, at this stage, there is no further enhancement needed for beam indication based on unified TCI state framework.” and Spreadtrum: “Proposal 8: For BM-Case 1 and BM-Case 2, support UE to report the measurement results of up to 16 beams in one reporting instance.” In our paper, we have analyzed and studied the aspects related to activation/indication of multiple future time instances and come to the conclusion that it should not be introduced for the following reasons: - The sole potential benefit with this proposal would overhead saving of DL control signaling. But this overhead saving would be very small: - As an example, assume that BM-Case 2 predicts the Top-K beams for 4 predicted future time instances (N=4), where each instance has a duration of 80ms. Then, rather than using legacy and to send one DCI prior to each future time instance, it is proposed to use one common DCI that would indicate all 4 future TCI states. For this example, 3 DCIs during a period of 4*80ms=320ms could be saved. However, this is a very small DCI overhead saving considering a long prediction window and applicable only to limited cases, while the resulting specification effort and implementation complexity would be substantial. - The overhead saving would only occur in corner cases, if no PDSCH would be scheduled, or if there is infrequent DL traffic, since otherwise a DCI is anyway transmitted in which the TCI state can be indicated. - In the TCI mechanism, only activated TCI states can be indicated by DCI. But legacy only supports 8 active TCI states, which may be too little if multiple future instances should be indicated from the active TCI state list. But an increased number of active TCI states would severely impact the UE complexity and needs RAN4 efforts. In MIMO Rel-18, this issue was addressed for multi-TRP and could not be agreed for the same UE complexity reasons. - The gNB may not always want to keep all the future TCI states it has predicted. E.g., the TCI state indicated to a UE for a future time instance may not only depend on the predicted beam of this single UE but also depend on other UE(s) to be paired with it in that future time, which cannot be predicted. Especially when the prediction window is long, the gNB may anyway need to send additional DCI to override the previously predicted TCI state. The mechanism of overriding also brings potential spec impact. - If the model output is Top-K>1 beams for each future time instance, which is a general case from the performance point of view, a second round beam sweeping would be needed immediately before each predicted instance. This diminishes the usefulness of indicating multiple future time instances even further. It is therefore our view that the associated cost is too high and outweighs the potential benefit to support the indication of multiple future TCI states in one DCI. |
| Fujitsu | Generally fine with the FL proposal. Ok with suggestion from OPPO to add DL TCI state. |
| TCL | Agree. New TCI state should be introduced to support the AI-purposed configuration. |
| Xiaomi | Fine with OPPO’s update. |
| ZTE | Fine |
| Ericsson | Not support. The only scenario where the above would make sense is_ - Top-1 beam is predicted for N time instances - Top-1 beam RSRP is predicted for N time instance (no measurement is needed for CQI) - There is no need to update the Top-1 beam prediction in time frame 1…N Even if this scenario should be supported, the overhead reduction is minimal. The current method for activating TCI states for data reception via DCI is sufficient. However, it is not clear how to configure the UE with the correct TCI states for Top-K beam measurements (issue 7.3). |
| LG | Support. |
| CATT | Not support. The benefit and necessity should be discussed firstly. We think the benefit of extension rel-17 TCI is limited in case of having PDSCH transmission since anyway gNB should send a DCI scheduling PDSCH. And there is also TCI flexibility issue and the issue of how to overwrite the TCI state before it applies. At a result, we do have concern on extension rel-17 TCI to multiple time instances. |
| Fraunhofer | We believe this is not required. |
| Nokia | Ok |
| Panasonic | Support. |
| MediaTek | We agree with HW. Somehow our views are not listed here. We have studied the pros and cons for this feature and concluded that this feature is not essential, and the corresponding benefit is not clear and very limited, with the following observations: (a) the overhead saving would only occur if model output is Top-1. If the model output is Top-K>1 beams for each future time instance, a P2 beam sweeping would be needed at each future time instance. (c) RAN1 may need to specify another mechanism to handle the case when NW want to change a pre-indicated beam, and a new recover mechanism when the pre-indicated beam fails. (d) it may potentially limit the capability of NW for choosing a different Tx beam based on newly reported beam information from other UEs during the future N time instances |
| Spreadtrum | Not support. Using multiple indication, gNB can select a more appropriate beam based on real-time channel changes. For example, if gNB predicts Top 3 beams for two future time instance (e.g., t1 and t2), gNB can execute beam sweeping before t1 for the first Top 3 beams, and then execute beam sweeping before t2 for the second Top 3 beams. Otherwise, gNB can only select the beam for t1/t2 only based on the prediction result |
| CMCC | Not support. TCI state activation/indication for each future instance is enough. |
| Sharp | Support. However, before determining whether to enhance the TCI indication for BM Case 2, we think time granularity of beam indication for BM Case 2 should be determined first. If we have to indicate TCI state for each slot, it is hard to meet the time requirement considering that HARQ-ACK should be transmitted first to confirm the reception of beam activation command. Thus, if the time granularity is quite small for beam indication, the enhancement for the beam indication should be applied to BM Case2. |
| Futurewei | Not support. The legacy unified TCI framework works and it is unclear about the benefit of the proposal as commented by other company. |
| NEC | Support |
| Kyocera | Support |
| Lenovo | Not Support. Firstly, the benefit is not clear for us because the TCI state can be indicated by a PDCCH without DL assignment when there is no PDSCH scheduling. Secondly, if multiple TCI states are indicted by a DCI for multiple future time instances, the additional timeline for the different beam application should be specified and RAN4 may need to be involved as well. Finally, do the indicated multiple TCI states for future time instances should be activated before indication? |
| Apple | We don’t support this proposal |
| Tejas | Support |
| NEC | Support |

### Issue #7.2: whether any enhancement is needed for P2?

##### (FL0) proposal 7.2-1a:

- For enabling a P2 sweep based on predicted Top-K beams, considers at least the following solutions:

  - Option 1: NW indicates that the corresponding beam IDs/TCI states for the P2 ResourceSet follows the UE predicted Top-K beams,

  - Option 2: NW configures a number of aPeriodicTriggerStates, and the UE provides Top-K predictions restricted to the valid TCI states matching one of the aPeriodicTrigger states,

  - Option 3: For both UE/NW sided model, NW configures dynamically as part of the MAC CE the beam IDs or TCI states that are part of the Top-K beam sweep. FFS on details.

  - Option 4: The NW uses legacy signaling, and in a best-effort manner configure a set of aPeriodicTriggerStates

  - Note 1: Option 2 and 4 would imply reduced performance since P2 cannot support all Top-K combinations

  - Note 2: NW indicating beam IDs could be used during monitoring

| Companies | Y/N | View |
| --- | --- | --- |
| FL |  | But pls feel free to show you view |
| Huawei, HiSilicon |  | Option 1 and Option 2 are not unified solutions, because NW side model may also face the same problem of P2 sweeping, with the difference that the model output is nor reported by UE. In addition, Option 1 is problematic in case of UCI missing. Option 3 or Option 4 is preferred. For Option 3, dynamically indicating TCI/QCL(e.g., to the SSB) is preferred than indicating beam IDs, as the number of SSB beams is smaller than CSI-RS beams, so indicating TCI/QCL benefits the overhead. |
| NTT DOCOMO |  | We think the following option is more useful, as it does not require additional signaling and CSI-RS is usually trasnmitted not only for one UE but for multiple UE in commercial deployments. ・Option5: NW configures all Set A measurement resources, and UE is expected to measure only predicted top-K beams after each inference result. |
| OPPO |  | For P2 beam sweeping, we think it’s okay to reuse Option 4, i.e. legacy signalling. Though the Top-K beam combinations are limited due to the number of configured triggering states, it could anyway be up to NW implementation to test Top-K beams. |
| Fujitsu |  | We don’t see the strong need for this kind of enhancement. |
| TCL |  | Option4 is preferred. |
| ZTE |  | OK with the direction and prefer dynamic indication of the TCI states for the pre-configured K RS resources in Option 3. Option 1 mandates gNB to sweep over the UE predicted Top-K beams and thus cannot reflect any gNB preference. Option 2 and 4 would imply reduced performance as noted. |
| vivo |  | We are okay to discuss this issue. In our understanding, we think either Opt 1 or Opt 3 can be candidate solutions, where Opt 1 means the beam for measurement follows the reported beams from UE, and Opt 3 means NW uses more dynamic signaling to change the beams for measurement. These can be two general directions for further discussion. |
| CMCC |  | Option 2 and 4 will degrade inference performance due to limited combination of Top K beams and increase RRC overhead. Option 1 and 3 are preferred with higher flexibility of Top K beam indication. |
| Panasonic |  | We think this proposal can be deprioritized to be discussed later as it is not urgent as for now. |
| LG |  | Fine with the spirit of the proposal, and we think that P3 procedure for predicted Top-K beam(s) should also be considered in this discussion for UE to determined Rx spatial domain filter. |
| Ericsson |  | Support all 4 options and for companies to provide views on the options for next meeting. The intention of beam IDs is for the UEs to use such information in the monitoring or P3 procedure. |
| Fraunhofer |  | In general fine. Option 1 requires the least configuration effort and the least delay compared to the other solutions. In any case, the NW may still decide whether it wants to follow the UE report or whether it wants to perform a different P2 beam sweep, which is still possible by the legacy procedures. |
| CATT | Option 1 or Option 3 | The maximum number of trigger states for aperiodic CSI reporting in current specification is 128, which is not only for the purpose of beam reporting, thus it’s impossible for network to configure all possible combinations of K beams in Set A, and it also brings complexity for network to trigger second round beam sweeping for Top-K beams. So we think Option 2 and Option 4 cann’t satify the requirement of P2 sweeping measurement. |
| Futurewei |  | Ok with the direction of the proposal and we prefere Option 3. |

# Proposal for online

##### (FL1) Proposal 3.2b (report for BM-Case 2)

For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, down selection one option from:

- Alt 1: the largest RSRP value based on prediction of each time instance is the reference RSRP, and differential RSRPs of the same instance in the report are computed relative to the reference RSRP of the corresponding time instance.

- Alt 2: the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP.

  - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

Note: Ericsson, IDC prefer Alt 1

##### (FL0) Proposal 3.3-2b

For report content of inference results for UE-sided model, where the largest RSRP value is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential RSRP is quantized to a 4-bit value with 2 dB step size.

Note: the model output is UE implementation and it doesn’t have to be RSRP subject to dBm value.

##### (FL0) Proposal 3.3a

For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Note: how to capture it in the spec is a separate discussion.

##### (FL0) proposal 4.1:

For UE-side model, at least for AI/ML based beam management, for processing of a CSI report at least for inference, study the following options:

- Option 1: The CPU occupied by the CSI report at least for inference and the CPU occupied by the non-AI/ML based CSI report for share the same pool.

- Option 2: Introduce a dedicated CPU pool as AI/ML-based CSI processing unit (ACPU), FFS

  - Alt 1: Processing of a CSI report at least for inference only occupies a number of ACPU(s) for a number of symbols

    - Note: the legacy CPU pool will not impact by the activated AI/ML functionality

  - Alt 2: Processing of a CSI report at least for inference occupies X number of ACPU(s) for M symbols and Y number of CPU(s) for N symbols

- FFS whether AI/ML based beam management and other AI/ML based features, e.g., AI/ML based CSI prediction, share the same (A)CPU pool or not.

##### (FL1)Proposal 2.3b (reference time for BMCase2)

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, support

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

FFS: whether only the inference result(s) of time instance(s) no earlier than the inference report is reported or not

#### Summary from Tdoc

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

  - Supported by(13): Huawei/HiSi, Nokia, CMCC, NEC, TCL, LG, Ericsson, Intel, Transsion, Fujistu, Google, OPPO, Futurewei

- Option 2: Based on the CSI reference resource corresponding to the report

  - Supported by(7): Huawei/HiSi, TCL, LG, OPPO, IDC, Tejas, ITL

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

  - Supported by(17): ZTE, xiaomi, DCM, NEC, Panasonic, CATT/CBN, Samsung, Ruijie, Transsion, China Telecom, Sharp, MediaTek, Sony, Qualcomm, KDDI, [Huawei/Hisi], [Futurewei]

# 10 Reference

R1-2500050	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2500066	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2500089	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2500159	Discussion on AIML for beam management	Spreadtrum, UNISOC

R1-2500201	Discussion on AI/ML-based beam management	CATT

R1-2500254	Discussion on AI/ML for beam management	China Telecom

R1-2500274	Discussion on specification support for beam management	CMCC

R1-2500337	Specification support for beam management	vivo

R1-2500390	Specification Support for AI/ML for Beam Management	Kyocera

R1-2500391	AI/ML for beam management	Ericsson

R1-2500404	Specification support for beam management	Tejas Networks Limited

R1-2500465	On specification for AI/ML-based beam management	OPPO

R1-2500512	Discussion on AI/ML for beam management	Ofinno

R1-2500529	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2500545	AI/ML based Beam Management	Google

R1-2500555	Discussion on AIML beam management	TCL

R1-2500560	Discussion on specification support for beam management	Panasonic

R1-2500565	Discussions on AI/ML for beam management	LG Electronics

R1-2500590	Discussion on specification support for beam management	NEC

R1-2500635	AI/ML specification support for beam management	Lenovo

R1-2500642	Discussion on Specification Support for Beam Management	Sony

R1-2500669	Discussion on specification support for beam management	Ruijie Networks Co. Ltd

R1-2500686	Specification support for AI-enabled beam management	NVIDIA

R1-2500710	Discussion on AI/ML for beam management	Xiaomi

R1-2500766	Enhancements for AI/ML enabled beam management	Apple

R1-2500834	Discussion for supporting AI/ML based beam management	Samsung

R1-2500877	Specification support for beam management	KDDI Corporation

R1-2500900	Discussion on specification support for beam management	ETRI

R1-2500925	Discussion on specification support on AI/ML for beam management	Fujitsu

R1-2500962	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2500970	AI/ML for Beam Management	Nokia

R1-2501013	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2501085	AI/ML for Beam Management	Meta

R1-2501103	Specification support for beam management	Intel Corporation

R1-2501104	Discussion on AI/ML for beam management	HONOR

R1-2501130	Discussions on specification support for beam management	Sharp

R1-2501144	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2501190	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2501226	Discussion on AI/ML based beam management	KT Corp.

R1-2501235	Discussion on AIML based beam management	ASUSTeK

R1-2501262	Discussions on AI/ML for beam management	CAICT

R1-2501270	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2501333	Specification support for AI/ML beam management	ITL

# 11 Previous agreements

## 11.1 Agreements in RAN 1 #116

Agreement

For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling

- Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications

- FFS on the report content for beam related information

- FFS on max number of reported beam related information in one report

Agreement

For UE-sided model, at least for BM-Case1, for content in the report of inference results, support

- Opt 1: Beam information on predicted Top K beam(s) among a set of beams

- Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams

- At least K=1 and more, FFS on max value

- FFS on beam information

- FFS on the definition of predicted Top K beam(s)

- FFS on definition of reported RSRP when applicable

- FFS on other information in the report with potential down selection among the following options

  - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams

    - FFS on the quantization method of probability information

    - Probability information is the probability of the beam to be the Top 1 or Top K beam

  - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP

    - FFS on definition of reported RSRP

    - FFS on the definition and quantization method of confidence information

  - Other options are not precluded.

where the set of beams is Set A, i.e., the beams for UE prediction.

Agreement

- For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework

- FFS on whether/how potential enhancement is needed

Conclusion

For UE sided model at least for inference, for measurement, the configuration of Set B,

- take the current CSI framework as the starting point

## 11.2 Agreements in RAN 1 #116b

Agreement

For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report

- wherein information of inference results of one time instance is as in one report for BM-Case 1

  - Note: overhead reduction is not precluded

- FFS on details

Agreement

For network-sided AI/ML model for BM-Case1 and BM-Case2,

- support using existing CSI framework for configuration of Set A as the starting point

- support using existing CSI framework for configuration of Set B as the starting point

- Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

Agreement

For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options:

- Option A: Predicted RSRP

- Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement

- Where the predicted RSRP is based on AI/ML output

- Note: Support both Option A and Option B is not precluded.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output

Agreement

For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting

- FFS on the details in the CSI-ReportConfig, at least considering:

  - Alt 1: one CSI-ResourceConfigId is configured for Set B

    - FFS: how UE can determine the information about set A

  - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B

    - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig

  - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately

  - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId

    - FFS: how to configure/indicate separate resource set(s) for Set A

  - Note: separate CSI-ReportConfig for Set A and Set B are not precluded.

  - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig

  - FFS on the association between Set A and Set B with or without additional IE

  - Other necessary configuration are not precluded.

Agreement

Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B:

- Opt1: Based on associated ID (Referring to AI 9.1.3.3)

  - FFS on what can be assumed by UE with the same associated ID across training and inference

  - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework

- Opt 2: Performance monitoring based

  - FFS details

- Other options are not precluded.

## 11.3 Agreements in RAN 1 #117

Agreement

For BM-Case1 and BM-Case2 with a UE-side AI/ML model:

- Support Type 1 performance monitoring, including the following two options:

  - Option 1 (NW-side performance monitoring):

    - UE sends a report to NW (for the calculation of performance metric at NW)

      - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report

      - FFS on other contents

    - The report is at least configured/triggered by NW

    - Note: this may or may not have additional spec impact

  - Option 2 (UE-assisted performance monitoring):

    - UE calculates performance metric(s)

      - FFS how to report and what to report

  - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2

- FFS Type 2 performance monitoring

Agreement

At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support

- Support differential L1-RSRP reporting with legacy quantization step and range

  - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP

  - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Agreement

Following Working Assumption is confirmed.

Working Assumption

For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s) in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

Agreement

For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support

- L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB

- If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set

- FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams

- FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents)

- FFS on beam information

- Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

## 11.4 Agreements in RAN 1 #118

Agreement

For UE sided model in beam management, introduce support associated ID

- [Working Assumption]

  - The associated ID at least can be configured within CSI framework

    - FFS on details

    - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s)

- UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID

  - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

Agreement

For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support

- Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting

  - For BM-Case 1, support differential RSRP report among multiple beams

  - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances

    - FFS details

Agreement

For UE-sided model at least for BM Case-1, for inference results report

- Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report

  - FFS whether support only resource set for Set B is configured

- UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference,

- The beam information in the inference report refers to the resource set for Set A

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including:

- Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring

- Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring

- Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring

  - Note: resources for Set B for monitoring are not precluded and can be study.

  - Note: this is only applicable when the model can predict RSRP

- Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam

  - Note: this is only applicable when the model can generate probability information

- FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including

  - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference.

- For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

Agreement

For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable

- FFS: how to determinate reference time for the time instance(s)

- FFS: duration values of the N time instance(s) that can be predicted.

## 11.5 Agreements in RAN 1 #118bis

Agreement

Answer to Q2 in R1-2407604 as below:

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Agreement

For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring),

- At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring

  - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation

  - FFS: on other details including how to configure the resource set/resources for monitoring, including

    - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric

- FFS other alternatives

Agreement

For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection:

- Option 1: Based on the uplink slot for the report

- Option 2: Based on the CSI reference resource corresponding to the report

- Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

Agreement

For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing.

- FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities.

- FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

Agreement

For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- Aperiodic (AP) CSI-RS

For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR:

- Periodic (P) CSI-RS

- Semi-persistent (SP) CSI-RS

- FFS: Aperiodic (AP) CSI-RS

Note: above CSI-RS resource refers to that used for beam management.

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring:

- Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference

  - FFS: the resource set(s) for monitoring

  - UE measures the resource set(s) for monitoring.

  - FFS how/when to report the monitoring results.

- Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The dedicated report configuration used for monitoring links to an inference report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - UE measures the resource set(s) for monitoring.

  - FFS when to report the monitoring results.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

Agreement

For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report.

Agreement

For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

Agreement

Incorporating below text into the general part of the LS

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality“ of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-specific FGs

- The concept/terminology “ functionality“ of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters or information/parameters indicated by UE

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality“for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same, depends on certain option in RAN1, and the discussion is still ongoing

Agreement

Answer to Q1 in R1-2407604 as below,

In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-specific FGs (including components and corresponding value ranges). These AI/ML-specific UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

## 11.6 Agreements in RAN 1 #119

Agreement

- For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- Two CSI-ResourceConfigId s are configured for Set A and Set B separately

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

# 12.1 UE-sided model

Conclusion(RAN 1 #119) For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

## 12.1.0 Consistency and applicability

Agreement(RAN 1 #116bis) Further study, for the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2, where the NW-side additional condition may at least impact UE assumption on beams of Set A/Set B: - Opt1: Based on associated ID (Referring to AI 9.1.3.3) - FFS on what can be assumed by UE with the same associated ID across training and inference - FFS on how associated ID is introduced, e.g., within CSI framework, or outside of CSI framework - Opt 2: Performance monitoring based - FFS details - Other options are not precluded. Agreement(RAN 1 #118) For UE sided model in beam management, introduce support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list Agreement(RAN 1 #119) - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Conclusion (RAN 1 #119) For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

## 12.1.1 Data collection

## 12.1.1.1 Configuration

## 12.1.2 Model inference

## 12.1.2.1 Configuration

Conclusion (RAN 1 #116) For UE sided model at least for inference, for measurement, the configuration of Set B, - take the current CSI framework as the starting point Agreement (RAN 1 #116bis) [replaced by later agreement] For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A andSet B inCSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded. Agreement(RAN 1 #118) For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A Agreement(RAN 1 #118bis) For UE-side AI/ML model, for BM-Case1, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - Aperiodic (AP) CSI-RS For UE-side AI/ML model, for BM-Case 2, at least for inference, at least for Set B, support the following CSI-RS resource types for CMR: - Periodic (P) CSI-RS - Semi-persistent (SP) CSI-RS - FFS: Aperiodic (AP) CSI-RS Note: above CSI-RS resource refers to that used for beam management. Agreement(RAN 1 #119) For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately

### BM-Case 2 specific

Agreement (RAN 1 #118) For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted. Agreement(RAN 1 #118bis) For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

## 12.1.2.2 Inference report

Agreement (RAN 1 #116) For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction. Agreement (RAN 1 #116bis) For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s)in the report of inference results, when applicable, further study the following options: - Option A:Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded. Agreement(RAN 1 #118) For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement (RAN 1 #118bis) For UE-side model, for beam management, for inference report, support periodic CSI report, aperiodic CSI report, and semi-persistent CSI report. Agreement(RAN 1 #119) - For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

### BM Case 2 specific

Agreement(RAN 1 #116bis) For UE-side AI/ML model inference, for BM-Case2, support to report inference results of N(N>=1, FFS on N) future time instance(s) in one report - wherein information of inference results of one time instance is as in one report for BM-Case 1 - Note: overhead reduction is not precluded - FFS on details Working Assumption(RAN 1 #116bis) For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output Agreement (RAN 1 #117) Following Working Assumption is confirmed. Working Assumption For report content of inference results for UE-sided model for BM-Case 2, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output.

## 12.1.3 Performance monitoring

Agreement (RAN 1 #117) For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring

## 12.1.3.1 Configuration

Agreement(RAN 1 #118bis) (replaced by later agreement) At least for the monitoring Type 1 Option 2 of UE-side model monitoring(when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. Agreement(RAN 1 #119) At least for the monitoring Type 1 Option 2 of UE-side model monitoring(when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

## 12.1.3.2 Monitoring report

Agreement(RAN 1 #118) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRPof one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) Agreement(RAN 1 #118bis) For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

## 12.1.4 Others

## 12.1.4.1 CPU counting

Agreement (RAN 1 #118bis) For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

## 12.1.4.2 others

Agreement(RAN 1 #118bis) For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.

# 12.2 NW-sided model

Agreement (RAN 1 #117) At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

## 12.2.1 Data collection

## 12.2.1.1 Configuration

## 12.2.1.2 Content

## 12.2.2 Model inference

## 12.2.2.1 Configuration

Agreement (RAN 1 #116bis) For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications

## 12.2.2.2 Inference report

Agreement (RAN 1 #116) For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report Agreement (RAN 1 #117) For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRPof a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications

12.2.3 Performance monitoring

12.2.3.1 Configuration

## 12.2.3.2 Monitoring report

## 12.2.4 Others

## 12.2.4.1 beam indication

Agreement(RAN 1 #116) - For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

# Appendix

### 8.15.2	Known conditions for downlink TCI state

The downlink TCI state is known if the following conditions are met:

-	During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state

-	Downlink TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement

-	The UE has sent at least 1 L1-RSRP report for the target downlink TCI state before the downlink TCI state switch command

-	The target downlink TCI state remains detectable during the downlink TCI state switching period

-	The SSB associated with the downlink TCI state remain detectable during the downlink TCI switching period

-	SNR of the downlink TCI state ≥ -3dB

-	The SSB can be associated with either the serving cell PCI or a PCI different from serving cell PCI.

Otherwise, the downlink TCI state is unknown.

8.15.3	MAC-CE based downlink TCI state switch delay

The requirements in this clause shall apply for DL TCI state switch using separate DL TCI state or joint TCI state of unified TCI state switch framework.

In case that source RS in DL TCI state or joint TCI state is associated with a PCI different from that of the serving cell, the requirements in this clause shall apply if the cell with different PCI satisfies the known cell condition defined in 8.15.1. If the known cell condition is not met, longer delay may be expected.

In case of joint TCI state switch, if the target PL-RS is not maintained, UE is not expected to receive on DL based on the target TCI state before UE completes the DL and UL TCI state switch.

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + + TOk*(Tfirst-SSB + TSSB-proc) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ +  where THARQ (in slot) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3];

-	Tfirst-SSB is time to first SSB transmission after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state

-	TSSB-proc = 2 ms;

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH/PDCCH, 0 otherwise.

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE activation command in slot n, UE shall be able to receive UE-dedicated PDCCH/PDSCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + + (TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc)) / NR slot length. The UE shall be able to receive UE-dedicated PDCCH/PDSCH with the old TCI state until slot n+ THARQ +  .

Where

-	T L1-RSRP = 0 in FR1 or when the TCI state switching not involving QCL-TypeD in FR2. Otherwise,

-	T L1-RSRP is the time for Rx beam refinement in FR2, defined as

-	TL1-RSPR_Measurement_Period_SSB for SSB as specified in clause 9.5.4.1,

-	with the assumption of M=1

-	with TReport = 0