3GPP TSG-RAN WG1 Meeting #120	R1-250xxxx

Athens, Greece, February 17 – 21, 2025

Agenda Item:	9.3.3

Source:	Moderator (Huawei)

Title:	Summary #1 of CLI handling

Document for:	Discussion and Decision

- Introduction

This document summarizes the contributions submitted to RAN1#120 on CLI handling.

- Proposals for online sessions

  - Tuesday (February 18th)

- Proposal 1-2

For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, down-select from the following alternatives:

- Alt.1: A single configuration of up to two UL muting symbols is provided per UE, i.e., one configuration for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH

- Alt.2: Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH

  - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported

- Alt.2: Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH, Type 2 CG PUSCH and Type 1 CG PUSCH

  - Separate configuration for each of multiple Type 1 PUSCH or Type 2 CG PUSCH configurations is supported

Note: The above only applies to DG PUSCH scheduled by a DCI format other than DCI format 0_0 and Type 2 CG PUSCH activated by a DCI format other than DCI format 0_0.

- Proposal 1-11

For PUSCH that is scheduled by DCI format 0_0 or Type 1 CG PUSCH activated by DCI format 0_0, UL resource muting is not applied.

- Proposal 1-3

Confirm the following working assumption

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

- Proposal 2-1

Conclusion

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’

- carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources are to be found. If the field is absent, the resources are on the same serving cell as this report configuration.

- resourcesForChannelMeasurement provides the ID of a CSI-ResourceConfig that contains configuration of CLI-RSSI measurement resources or SRS-RSRP measurement resources (agreed in RAN1#118bis)

- bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in.

- Proposal 2-1b

The UE is not expected to measure L1 SRS-RSRP with a subcarrier spacing other than the one configured for the active BWP confining the L1 SRS-RSRP measurement resource.

The UE is not expected to measure L1 CLI-RSSI with a subcarrier spacing other than the one configured for the active BWP confining the L1 CLI-RSSI measurement resource.

- Proposal 2-2

Update the following agreement in RAN1#118 (in red)

Agreement

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband or a UL subband

- Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

FFS: UE behavior for measurement

UE does not expect to be configured with Measurement resource type#1 outside the DL or UL usable PRBs.

- Proposal 2-5

For L1 based UE-to-UE CLI measurement and reporting, periodic CLI reporting and semi-persistent CLI reporting are not supported.

- Proposal 2-8

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, new reporting criteria considering the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP are not supported.

- Proposal 1-7

Update the following agreement in RAN1#118bis as follows (marked in red)

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

  - The MscPUSCH/2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH/2, and the result is mapped to the MscPUSCH/2 unmuted scheduled subcarriers.

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

  - Option 2a: The block of MscPUSCH/2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH/2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

  - Option 2b: The MscPUSCH/2 modulated symbols are block-wise spreading by OCC [+1,+1] or [+1,-1] depending on whether the comb offset is 0 or 1 into a length-MscPUSCH sequence of modulated symbols before DFT. After DFT, the result is mapped to the MscPUSCH scheduled subcarriers.

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

Note: Option 1 and Option 2a are mathematically equivalent. It is up to UE implementation to choose either option.

- Proposal 1-1

For the configuration of UL resource muting, down-select from the following alternatives

- Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

- Alt.2: A new IE ZP-SRS-Resource is introduced to define a zero-power SRS resource which occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP. Up to two ZP-SRS resources can be configured.

- gNB-to-gNB CLI handling

  - UL resource muting for PUSCH

The WI objectives related to UL resource muting are listed below.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability

    - Indication/determination of UL resource muting

      - Signaling mechanism

- Issue 1: Configuration of UL resource muting resource

Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH - Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI - Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH. FFS: details of semi-static configuration and dynamic indication (if supported) FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

It was agreed that “the time location of each of one or two UL muting symbols is semi-statically configured”. However, it was unclear how the time location of each of one or two UL muting symbols is configured. Based on the contributions submitted to RAN1#120, there are two alternatives:

- Alt 1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

  - Support: CATT, Spreadtrum, vivo, ZTE, Samsung, OPPO, Nokia(?), Tejas Networks (?)

PUSCH-ULMutingResources information element

PUSCH-ULMutingResources ::=               SEQUENCE {

symbolPos   SEQUENCE (SIZE(1..maxNrOfULMutingSymbols)) OF INTEGER(0..13)   OPTIONAL,

combOffset  ENUMERATED {0, 1}                                              OPTIONAL,

}

maxNrOfULMutingSymbols INTEGER ::= 2 -- Maximum number of UL muting symbols for PUSCH within a slot

- Alt.2: A new IE ZP-SRS-Resource is introduced to define a zero-power SRS which occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP.

  - Support: Ericsson, InterDigital, LG, NEC, Panasonic, Sharp

ZP-SRS-Resource information element

ZP-SRS-Resource ::=                     	SEQUENCE {

transmissionComb                        	CHOICE {

n2                                      	SEQUENCE {

combOffset-n2                           	INTEGER (0..1)

},

...

},

resourceMapping                     		SEQUENCE {

startPosition                       		INTEGER (0..13),

nrofSymbols                         		ENUMERATED {n1, ...},

...

},

...

}

To the moderator’s understanding, both alternatives are feasible. In terms of specification impact, it was argued that the merit of Alt.2 is that the “Mapping to physical resources” section in 38.211 for a legacy SRS (38.211 Section 6.4.1.4.3) can be reused almost completely “as is” for defining the muting patterns. On the other hand, with Alt.1, PUSCH RE-level rate matching resource similar to PDSCH rate matching resource needs to be defined. However, this does not seem to be complicated since only the symbol location and comb offset are required to define such resource. Overall, both alternatives are technically feasible and there is no significant merit for either of the two alternatives. It is just a matter of how to capture the feature into the specification.

Companies’ detailed proposals or discussions are listed below.

CATT: For the signaling mechanism of UL muting pattern configuration, support new UL muting pattern configuration.

Ericsson: For configuration of UL resource muting, support a new IE ZP-SRS-Resource in 38.331 for defining a zero-power SRS. A ZP-SRS resource occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP.

Interdigital: For the semi-static configuration of the time location of the UL muting resources, support RS-based pattern indication of the RE to be muted, e.g., based on a ZP comb-2 DMRS configuration with an addiction RRC indication of the RS to be a ZP-RS.

NEC: Support signaling of UL muting pattern to the UE using zero-power SRS resource.

OPPO: Introduce a new IE to explicitly configure UL resource muting pattern.

Sharp: Support a configuration of a ZP SRS for one or two symbols within a slot, where the resource configuration indicates a comb pattern on even or odd REs.

- Issue 2: Enhancement to PUSCH TDRA table

The second issue is how to enhance the PUSCH TDRA table so that for DG PUSCH and Type 2 CG PUSCH, muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI. This issue is related to Issue 1/2/3 in Section 3.1.1.2.

The detailed proposals and discussions are listed below.

Huawei: Extend PUSCH-Allocation-r16 to include an indicator which specifies whether semi-statically configured UL muting symbol(s) is enabled for each row.

LG: For dynamic ON/OFF indication of UL resource muting, single bit of TDRA with fixed location in DCI is used.

Samsung: The time location of each of one or two UL muting symbols is semi-statically configured in the PUSCH TDRA table.

      - Time location

For indication of UL resource muting for PUSCH, the following agreements were made in RAN1#119.

Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions

- Issue 1: UL resource muting for Type 1 CG PUSCH

In RAN1#118bis, it was agreed that “To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH.”

Considering that UE can be configured with multiple Type 1 CG PUSCH(s), it is unclear whether the time location of each of one or two UL muting symbols are configured for each Type 1 CG PUSCH separately or for all configured Type 1 CG PUSCH(s).  ZTE proposes that for the UL muting pattern is configured separately for each of the multiple Type-1 CG PUSCH configurations. Samsung has a similar proposal.

Ericsson proposed to support semi-static configuration of a list of one or two ZP-SRS resources in PUSCH-Config, for DG-PUSCH and both Type-1 and Type-2 CG-PUSCH, i.e., a single UL resource muting pattern for all PUSCH(s).

Huawei proposed to introduce an explicit ON/OFF indicator in rrc-ConfiguredUplinkGrant which specifies whether semi-statically configured UL muting symbol(s) is enabled for this Type 1 CG PUSCH. However, if UL resource muting is configured separately for each Type CG PUSCH, there seems no need to configure such indicator.

The detailed proposals and discussions are listed below.

ZTE: The UL muting pattern should be configured separately for each of the multiple Type-1 CG PUSCH configurations.

Samsung: For CG Type 1 PUSCH, the UL muted symbol positions can be added to the ConfiguredGrantConfig RRC IE.

Ericsson: For DG-PUSCH and both Type-1 and Type-2 CG-PUSCH, support semi-static configuration of a list of one or two ZP-SRS resources in PUSCH-Config, each corresponding to a different OFDM symbol location in a slot (parameter startPosition).

Huawei: An explicit ON/OFF indicator is introduced in rrc-ConfiguredUplinkGrant which specifies whether semi-statically configured UL muting symbol(s) is enabled for this Type 1 CG PUSCH.

Sharp: Consider dedicating a DCI message for the case of CG PUSCH Type 1 to communicate the TDRA field for turning the UL resource muting ON/OFF.

- Issue 2: UL resource muting for PUSCH repetition with Configuration 2

For UL resource muting for PUSCH repetition with Configuration 2, most companies are supportive of the following proposal

For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

Regarding whether UL resource muting can be applied in non-SBFD symbols, most companies are fine not to apply UL resource muting in non-SBFD symbols while QC points out that UL resource muting can also be applied in non-SBFD symbols for dynamic/flexible TDD operation.

The detailed proposals and discussions are listed below.

CATT: For the UL resource muting determination for PUSCH repetition Type A and Type B in SBFD symbols with Configuration 2, adopt the following method.

- For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all the PUSCH repetitions in SBFD symbols.

- For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

CMCC: For UL transmissions across different slots with Configuration 2, define a rule to restrict UL resource muting only apply to SBFD symbol(s).

Ericsson: For multi-PUSCH scheduling with single DCI and PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH(s)/PUSCH repetitions that overlap SBFD symbols. No muting is applied for PUSCH(s)/PUSCH repetitions that overlap non-SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions that overlap SBFD symbols. No muting is applied for actual repetitions that overlap non-SBFD symbols.

Nokia: For PUSCH repetitions with Configuration 2, do not perform UL muting over PUSCH repetitions within non-SBFD slots.

QC: Support UL muting can only apply to Configuration 1 that the UL muting symbols are restricted to SBFD symbols or non-SBFD symbols.

- FFS: UL muting is restricted to SBFD symbols only for SBFD operation or UL muting can also apply to non-SBFD symbols for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type if UL muting can apply to SBFD or non-SBFD symbols.

vivo: To determine the time location of UL muting symbol(s) in a slot for a PUSCH,

- For PUSCH repetition Type A with Configuration 2, the same time location of none, one or two UL muting symbols is applied for all PUSCH repetitions in SBFD symbols.

- For PUSCH repetition Type B with Configuration 2, the same time location of none, one or two UL muting symbols per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

- Issue 3: UL resource muting for multi-PUSCH scheduled by a single DCI

To determine the time location of UL muting symbol(s) in a slot for multi-PUSCH scheduled by a single DCI, two options are proposed

- Option 1: The same time location of none, one or two UL muting symbol(s) is applied for all PUSCH transmissions in SBFD symbols. UL resource muting is not applied for PUSCH transmissions in non-SBFD symbols.

  - Support: Panasonic, Ericsson, vivo

- Option 2: The same time location of none, one or two UL muting symbol(s) is applied per PUSCH transmission. UL resource muting is not applied for PUSCH transmissions in non-SBFD symbols.

  - Support: CATT, Huawei, HiSilicon, Samsung (?), Spreadtrum, Tejas

The difference between the two options is whether UL resource muting is enabled or disabled for all PUSCHs or separately for each PUSCH.  Panasonic provides the following figure to illustrate the two options. This would impact the RRC parameter design. If Option 1 is selected, the ON/OFF indication is added to multiple SLIVs while the ON/OFF indication will be for each SLIV separately.

The detailed proposals and discussions are listed below.

CATT: For multi-PUSCH scheduled by a single DCI, support separate UL muting ON/OFF configuration in pusch-TimeDomainAllocationListForMultiPUSCH.

Ericsson: For multi-PUSCH scheduling with single DCI in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCHs

For multi-PUSCH scheduling with single DCI and PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH(s)/PUSCH repetitions that overlap SBFD symbols. No muting is applied for PUSCH(s)/PUSCH repetitions that overlap non-SBFD symbols.

Panasonic: Regarding the multi-scheduling PUSCH in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) should be applied for all PUSCH transmissions.

Spreadtrum: For multi-PUSCH scheduled by a single DCI in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for each PUSCH.

Tejas: Support the UL resource muting for each of the PUSCH transmission of multi-PUSCH scheduled by a single DCI can be turned ON/OFF using their TDRA field in DCI.

vivo:

- For multiple PUSCHs scheduled by a single DCI, the same time location of none, one or two UL muting symbols is applied for all PUSCHs scheduled by the single DCI in SBFD symbols.

- Issue 5: Configuration granularity

This can be discussed after decision on Issue 1 and Issue 2.

      - Frequency location

For frequency location, the following agreement was made in RAN1#119

Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols

For the frequency location configuration of UL resource muting for DFT-S-OFDM, there is majority support to confirm the working assumption

  - Support: CATT, CMCC, Nokia, ZTE, Samsung, Huawei, HiSilicon, Spreadtrum, Ericsson

There is one company propose to have a predefine comb offset.

  - Support: Qualcomm

Considering that a unified design is preferred, it is proposed to confirm the working assumption.

    - Transform precoding for symbols with UL resource muting

Companies’ views are diverged.

Option 1: Ericsson, vivo, Huawei, HiSilicon

  - Support: Ericsson, vivo, ETRI, Huawei, Google, LG

Option 2:

  - Support: CATT, Xiaomi, QC

Companies’ detailed proposals or discussions are listed below.

CATT: If transform precoding is enabled for a PUSCH, the DFT size is MscPUSCH for symbol(s) with UL resource muting.

CMCC: To generate comb-2 like UL resource muting pattern for DFT-S-OFDM PUSCH, the following two options can be considered.

- Option 1 (Half DFT size based): The DFT size in transform precoding procedure (TS 38.211, Clause 6.3.1.4) is reduced to MscPUSCH2, and the mapping to virtual resource blocks procedure (TS 38.211, Clause 6.3.1.6) is modified to map the MscPUSCH2 complex-valued symbols to virtual PRBs in comb-2 structure with a given comb offset.

- Option 2 (pre-DFT OCC based): The transform precoding procedure (TS 38.211, Clause 6.3.1.4) is modified to support block-wise pre-DFT OCC before transform precoding, and the OCC sequence [+1,+1] and [+1,-1] are corresponding to a comb offset 0 and 1, respectively.

Ericsson: There are at least two possible variants of Option 2 for transform precoding using DFT size MscPUSCH:

- Option 2a: Repetition before the DFT, followed by extraction of even-indexed subcarriers and subsequent mapping to non-muted subcarriers.

- Option 2b: Block spreading by [+1,+1] or [+1,-1] before the DFT, depending on whether odd or even subcarriers should be muted, and the resulting modulated symbols then mapped to all scheduled subcarriers.

For transform precoding for muting, discuss at least Options 1 and 2a and their respective merits.

Options 1 and 2a are mathematically equivalent, and hence, the specifications can be written to be consistent with both. Then the implementing company may choose between DFT size MscPUSCH/2 (Option 1) and DFT size MscPUSCH (Option 2a) depending on which approach is better aligned with their existing hardware chains.

Xiaomi: If transform precoding is enabled for a PUSCH, adopt Option 2.

Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

ETRI: In the case of Option 2 (The DFT size is MscPUSCH for symbol(s) with UL resource muting), the implementation complexity increases due to higher memory requirements, the need for power normalization, and incremental phase adjustments required to achieve different comb-2 offsets.

If transform precoding is enabled for a PUSCH, we support Option 1 (Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting).

Google: For the DFT size with UL resource muting, support DFT size of MscPUSCH2 or use a UE capability signaling to indicate the supported option

Huawei: If transform precoding is enabled for a PUSCH, the DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting and the input of DFT contains MscPUSCH/2 symbols including PUSCH data symbols and PT-RS symbols, if any.

LG: If transform precoding is enabled for a PUSCH, the DFT size is modified as MscPUSCH/2 for symbol(s) with UL resource muting.

DCM: If transform precoding is enabled for a PUSCH, support option 1.

- The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

QC: For a OFDM symbol that UL muting is applied, the set of complex valued symbols is repeated and mapped to complex valued symbols with Option 2.

Samsung:

Table 1: Differences between Option 1 and Option 2

|  | Preprocessing | DFT size | Resource mapping (after DFT block) | How to apply comb offset {0,1} |
| --- | --- | --- | --- | --- |
| Option 1 | Not needed | MscPUSCH2 for UL muting symbol(s) and MscPUSCH for non-UL muting symbol(s) | Outputs of DFT block are mapped to every other input of IDFT block for UL muting symbol(s) Outputs of DFT block are mapped to all inputs of IDFT block for non-UL muting symbol(s) | By resource mapping (every even REs every odd REs) |
| Option 2 | OCC block is needed for UL muting symbol(s). | MscPUSCH for all symbols (legacy) | Outputs of DFT block are mapped to all inputs of IDFT block for all symbol(s) (legacy) | By OCC spreading {[1 1] or [1 -1]} |

From the implementation point of view, we slightly prefer to use OCC based signal generation (Option 2). This does not require a change of the DFT size between PUSCH symbols and when the new Rel-19 resource mapping is applied. The required update is then the OCC block before the DFT.

Note that the OCC block before the DFT spreading is already used in PUCCH Format 4 where the OCC size is 2 or 4 for inter-UE multiplexing. So, the signal generation equation used already for PUCCH format 4 can be reused for PUSCH with UL muting.

For example, the following equation can be used where w0 w1=[1 1] or [1-1] and Pl is the starting index of PUSCH symbol l.

y0Pl+k=1MscPUSCHi=0MscPUSCH-1w(i /(MscPUSCH/2))xPl+i mod MscPUSCH/2e-j2πik/MscPUSCH

It must also be considered that from the implementation point of view, the UE does not support all DFT sizes. The number of PRBs in a PUSCH should correspond to 2α2∙3α3∙5α5. If the signal generation is done according to Option 1, the number of PRBs is effectively reduced to 2α2-1∙3α3∙5α5 for the UL muting symbol. If α2=0 (i.e., odd number of PRBs are scheduled), a new/different DFT implementation is then required. Option 2 simply avoids such an additional complexity.

Spreadtrum: If transform precoding is enabled for a PUSCH, study the following two options:

- PUCCH format 4 OCC like scheme can be performed before DFT

- UL resource muting is not applied to symbol with PTRS when transform precoding is enabled

Tejas: As the implementation complexity of the Option 1 is less than Option 2. Hence, we support Option 1 to reflect in the RAN1 specification.

If transform precoding is enabled for a PUSCH,

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

vivo: If transform precoding is enabled for a PUSCH, the DFT size is MscPUSCH for symbol(s) with UL resource muting.

ZTE: The DFT size should be modified as MscPUSCH/2 for the OFDM symbols with muting resource if transform precoding is enabled for PUSCH.

    - Handling of collision between UL muting resource and PT-RS for DFT-S-OFDM

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, there are two alternatives

- Alt.1: If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

  - Support: Nokia, NTT DOCOMO, OPPO, QC, Samsung, Spreadtrum, vivo, xiaomi

- Alt.2: If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, for PT-RS insertion prior to transform precoding

  - Support: CMCC, Ericsson, Huawei

It should be noted that the PT-RS insertion for a PUSCH when transform precoding is enabled is before DFT operation. Strictly speaking, there will be never a collision between PT-RS REs and muted REs as pointed by Ericsson. The main argument to support Alt.1 is that PT-RS samples need to be adjusted since there is only MscPUSCH2 modulation symbols which potentially brings some additional complexity. However, this does seem to be an issue since the PT-RS samples can still be determined based on the existing procedures.

The detailed proposals or discussions are listed below.

Nokia: If transform precoding is used, UE doesn’t not apply resource muting in the symbol with including PTRS.

DCM: If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and transform precoding is enabled, PTRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

OPPO: Disabling UL resource muting in the symbols containing PTRS for DFT-s-OFDM is a simple way to avoid inaccurate phase tracking and compensation.

Huawei: If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, the PT-RS samples prior to transform precoding in OFDM symbol l is determined as follows

- The PT-RS group pattern are determined based on the number of scheduled RBs for the PUSCH as legacy

- The index m of PTRS samples is determined based the existing mechanism but only the PT-RS samples within the first MscPUSCH2 samples are kept

CMCC: If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, for PT-RS insertion prior to transform precoding, support Option 2.

- Option 2: The PT-RS sample positions are determined using MscPUSCH in table 6.4.1.2.2.2-1 of TS38.211 and the PT-RS sample positions larger than MscPUSCH2 are dropped

Ericsson: For RE muting in a DFT-s-OFDM PUSCH symbol, the UE determines the number of PTRS groups and the number of samples per group using NRB/2 and the locations of the PTRS samples using MscPUSCH/2 according to existing specs. There is no need for any other specs changes than applying the acutal frequency domain resource size with existing procedures.

QC: If an UL resource muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Samsung: If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and when transform precoding is enabled for PUSCH, PTRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Spreadtrum: When transform precoding is enabled, PTRS is prioritized, i.e., the UE applies UL resource muting only within the symbol that PTRS not is not applied to.

vivo: If an UL resource muting symbol overlaps with a symbol containing PTRS for a PUSCH and transform precoding is enabled for the PUSCH, PTRS is prioritized, i.e., the UE does not apply UL resource muting in the symbol.

Xiaomi: If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH when transform precoding is enabled, UE does not apply UL resource muting in the symbol.

    - PUSCH and PTRS EPRE determination in a symbol with UL resource muting

Regarding the PUSCH and PT-RS EPRE determination in a symbol with UL resource muting, the following was agreed at RAN1#118bis

Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

On the FFS bullet, most companies support to perform same power boosting for both PUSCH and PT-RS REs such that total transmit power is not significantly changed between UL symbols w/ and w/o UL resource muting. Several companies have some analysis on the power difference for PUSCH w and w/o PT-RS as well as w and w/o UL resource muting. It was observed that in legacy NR (no UL resource muting), the total energy across the allocated PUSCH bandwidth may be less by a fraction of a dB in a symbol with PTRS compared to a symbol without PTRS depending on the configured PTRS boosting factor (UL-PTRS-power = ’00’ or ‘01’). For UL-PTRS-power = ’00’, the energy difference depends on the PTRS frequency domain density K and the number of layers L.

Table 1: Difference (in dB) between the total energy in a symbol with PTRS vs. a symbol without PTRS assuming Qp=1 PTRS port and non-coherent UEs in 38.211 Table 6.2.3.1-3

| UL-PTRS-power | PTRS Frequency Domain Density (K) | Difference (in dB) between the total energy in a symbol with PTRS vs. a symbol without PTRS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | L = 1 | L = 2 | L = 3 | L = 4 |
| 00 | 2 | 0 | -0.091 | -0.12 | -0.14 |
|  | 4 | 0 | -0.046 | -0.061 | -0.068 |
| 01 | 2 | 0 | 0 | 0 | 0 |

If 3 dB boosting of both PTRS and PUSCH REs is used in an symbol with UL resource muting, there can be a slightly larger difference in the total energy across the PUSCH bandwidth between a symbol with PTRS and a symbol without PTRS compared to the difference observed for legacy NR with no UL muting. The maximum difference is -0.28 dB compared ot legacy NR where the maximum is -0.14 dB.

Table 2: Difference (in dB) between the total energy in a symbol with muting + PTRS vs. a symbol with muting and no PTRS when applying 3 dB boosting of PTRS and PUSCH REs (ρ2=2) assuming Qp=1 PTRS port and non-coherent UEs in 38.211 Table 6.2.3.1-3.

| UL-PTRS-power | PTRS Frequency Domain Density (K) | Difference (in dB) between the total energy in a symbol with muting + PTRS vs. a symbol with muting and no PTRS |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | L = 1 | L = 2 | L = 3 | L = 4 |
| 00 | 2 | 0 | -0.19 | -0.25 | -0.28 |
|  | 4 | 0 | -0.091 | -0.12 | -0.14 |
| 01 | 2 | 0 | 0 | 0 | 0 |
|  | 4 | 0 | 0 | 0 | 0 |

The detailed proposals or discussions are listed below.

CATT: For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, down select from the following two alternatives to determine the value of power boosting.

- Alternative-1: 3dB

- Alternative-2: x=10log10MscPUSCH-kMscPUSCH/2-k(dB)

CMCC: For both CP-OFDM and DFT-S-OFDM PUSCH, keep the PUSCH to PT-RS power ratio per layer per RE (ρPTRSPUSCH) unchanged, and keep the total transmission power (including both PUSCH and PT-RS) unchanged between UL symbols w/ and w/o UL resource muting.

Ericsson: RAN1 to down-select to one of the following boosting approaches for REs in symbol a symbol with muting:

- Alt-1: 3 dB boosting for all unmuted REs (both PUSCH and PTRS, if present), regardless of the PTRS density K and number of PUSCH layers L

- Alt-2: 3 dB boosting for all unmuted REs in a symbol without PTRS and X dB boosting for all unmuted REs in a symbol with PTRS, where X ≥ 3 dB and depends on the configured value of UL-PTRS-power, PTRS density K, and the used number of PUSCH layers L.

Huawei: For PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting regardless of transform precoding is not enabled or not, 3dB power boosting should be assumed.

ETRI: Applying power boosting to PT-RS within muting symbols introduces power inconsistencies that cannot be fully mitigated by receiver-side compensation, leading to degraded phase noise estimation and correction.

3 dB power boosting is not assumed at least for PT-RS REs in the UL muting symbol containing PT-RS when transform precoding is not enabled.

LG: The transmit power of PTRS, EPRE ratio between PTRS RE and PUSCH data RE, is applied by symbol level.

If the transmission power of a specific PTRS RE is too large, the UE applies the actual transmission power as the smaller of the transmission power according to the UE capability and the configured transmission power.

OPPO: 3dB power boosting applied to both unmuted PUSCH and PT-RS REs in the symbols with UL resource muting can help the consistency of the total transmission power among PUSCH symbols.

Panasonic: 3 dB EPRE boosting should be assumed for REs in the PUSCH symbol containing PT-RS with UL resource muting.

QC: 3dB power boosting is assumed for REs in the PUSCH symbol containing PT-RS with UL resource muting that 3dB power boosting for both PUSCH and PT-RS REs.

Tejas: The symbol with 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL resource muting, have higher power as compared to the symbol with PUSCH symbol containing PT-RS with no UL resource muting.

We support the 3dB power boosting for the REs in the PUSCH symbol containing PT-RS with UL muting when transform precoding is not enabled if there is not impact on signal quality or MPR.

vivo: 3dB power boosting is assumed for REs of PUSCH and PTRS in the PUSCH symbol containing PTRS with UL resource muting when transform precoding is not enabled.

ZTE: 3dB power boosting should be assumed for both PTRS RE and the PUSCH RE in an OFDM symbol in case the OFDM symbol has muting RE.

    - Additional restrictions

Companies discussed the necessity of following conditions to apply UL resource muting for a PUSCH.

For those companies supporting additional restrictions to apply UL resource muting for a PUSCH, the following conditions were discussed.

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

  - Support: LG, Samsung

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

  - Support: Nokia

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

  - Support: Nokia

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied

  - Support:

- Condition 5: UL resource muting is not applied according to the PHY priority of PUSCH

  - Support:

- Condition 6: For PUSCH that is a retransmission, UL resource muting is not applied

  - Support:

- Condition 7: For PUSCH that is scheduled by DCI format 0_0, UL resource muting is not applied

  - Support: Nokia

- Condition 8: for PUSCH with repetition type B, UL resource muting is not applied

  - Support:

- Condition 9: For PUSCH carrying certain UCI type with high PHY priority, UL resource muting is not applied.

  - Support: Tejas Network

- Condition 10: UL resource muting is not applied to PUSCH carrying UCI-only payload.

  - Support: Samsung

- Condition 11: Any of DMRS/PTRS collides with resource indicated by UL resource muting.

  - Support: LG

- Condition 12: If the percentage of UL muted REs relative to REs carrying PUSCH modulated symbols after REs carrying UCI bits are multiplexed, is larger than a threshold and the PUSCH has a high L1 priority.

  - Support: Sony

- Other conditions are not precluded.

Companies’ detailed proposals or discussions are listed below.

Ericsson: With the agreement on Option 2, it is not necessary to further discuss conditions on UL resource muting as in Proposal 1-7 from RAN1#118bis.

LG: For the following conditions, RRC configured UL resource muting is disabled.

- For PUSCH repetition type B, number of symbols for actual repetition is less than X.

- Any of DMRS/PTRS collides with resource indicated by UL resource muting

If the BW of ZP SRS includes PUSCH but entire indicated symbols does not include that of the PUSCH, UL resource muting is applied.

When the BW of the PUSCH is not fully indicated by ZP SRS, UL resource muting is not applied to the PUSCH.

Nokia: Support conditional UL muting to reduce the number of entries in the TDRA table, while allowing certain flexibility in the applicability of the UL resource muting.

Tejas Network: The UL resource muting can be turned ON/OFF dynamically using TDRA field in DCI. Hence no conditional check required for following conditions,

- Condition 1: When the number of PUSCH symbols is less than threshold, UL resource muting is not applied

- Condition 2: If the scheduled PUSCH uses a MCS lower than certain reference MCS index, UL resource muting is not applied

- Condition 3: When the number of scheduled RBs is less than threshold, UL resource muting is not applied

- Condition 4: When the number of DMRS symbols is larger than threshold, UL resource muting is not applied.

    - Others

Impact on TBS determination

ETRI: To address the challenges arising from frequently changing muting symbols, it is proposed to fix or semi-statically configure the number of muting symbols for TBS calculation.

Qualcomm: RAN1 to discuss how to determine the index of the of starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots of UL TBoMS:

  - Option 1 (Motivation: simplicity but larger gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2 (Motivation: more accurate and less gap)  𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

UL muting on PUSCH symbols that can carry UCI

Samsung: Support of UL muting on PUSCH symbols that can carry UCI is an optional UE capability in Rel-19 SBFD.

When the UE does not indicate support for UL muting on PUSCH symbols that can carry UCI, and the UE determines presence of UCI on a configured/indicated UL muting symbol, the UE does not apply the UL resource muting in the symbol.

Spreadtrum: Clarification of resource elements available for transmission of data, i.e.  in section 6.2.7, TS 38.212 is needed.

Configured with DMRS bundling

LG: UL resource muted PUSCH is considered as a semi-static event when it is configured with DMRS bundling.

Effective code rate

Vivo: For PUSCH with UL resource muting, effective code rate is defined for PUSCH, and when the effective code rate is larger than a certain threshold, UE will not transmit the PUSCH with initial transmission.

- The effective channel code rate is defined as the number of UL-SCH information bits (including CRC bits) divided by the number of physical channel bits on PUSCH

    - 1st round proposals (Open)

- Proposal 1-1

For the configuration of UL resource muting, down-select from the following alternatives

- Alt.1: A new IE PUSCH-MutingResources is introduced to define the time location and frequency location of UL muting resources. The UL muting pattern configures up to two symbols within a slot and a configurable comb offset.

- Alt.2: A new IE ZP-SRS-Resource is introduced to define a zero-power SRS resource which occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP. Up to two ZP-SRS resources can be configured.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support Alt.1 | ZTE, Tejas Networks, vivo, Samsung, Spreadtrum, CMCC, QC, DOCOMO, Nokia |
| Support Alt.2 | Ericsson, IDC, Sony, LG |

| Companies | Views |
| --- | --- |
| Ericsson | We prefer Alt-2 since it is based on the same principle as ZP-CSI-RS defined in current specs for PDSCH rate matching. Moreover, it allows reuse of legacy SRS specification text in 38.211 which already defines the time/frequency resource mapping for a comb-2 with configurable comb offset. The only updates needed to 38.211 are to state that for a ZP-SRS resource, the sequence values are zero, and that the SRS bandwidth is at least as large as the PUSCH bandwidth, e.g., full BWP, since only the RBs that overlap with the PUSCH bandwidth need to be considered for PUSCH rate matching. Regarding some companies’ concern about too many unused parameters, we have done a careful analysis of 38.211 and found that for any parameter not defined in the new IE ZP-SRS-Resource, there is a default value defined in 38.211, and the default is appropriate for ZP-SRS, e.g., when b-hop and b-SRS are not configured, they default to 0 which further means that frequency hopping for ZP-SRS will be disabled since b-hop=b-SRS. The only parameters that need to be defined for in the new IE for ZP-SRS are as follows: ZP-SRS-Resource ::= SEQUENCE { transmissionComb CHOICE { n2 SEQUENCE { combOffset-n2 INTEGER (0..1) } }, resourceMapping SEQUENCE { startPosition INTEGER (0..13), nrofSymbols ENUMERATED {n1, ...} } } A key aspect of Alt-2 is that it should allow reuse of existing 38.211 spec text which means that the new ZP-SRS-Resource IE should reuse the same RRC parameter names as a legacy SRS resource, hence we think Alt-2 should be updated as follows: - Alt.2: A new IE ZP-SRS-Resource is introduced to define a zero-power SRS resource which occupies a single OFDM symbol in a slot with configurable symbol location. The ZP-SRS resource has a comb-2 RE pattern with a configurable comb offset of 0 or 1, and occupies all RBs of the BWP. Up to two ZP-SRS resources can be configured. To allow reuse of 38.211 spec text, the new IE ZP-SRS-Resource IE reuses the same RRC parameter names as a legacy SRS resource for the following: trasmissionComb (fixed to n2), combOffset-n2, startPosition, nrofSymbols (fixed to n1). Finally, we note that it was already agreed in RAN1#116 that an existing reference signal time-frequency resource pattern shall be used for UL resource muting. At that time the two candidates were comb-2 SRS and PT-RS. The latter was ruled out based on the RAN1#117 agreement to support comb-2 muting, thus comb-2 SRS remains: Agreement (RAN1#116bis) If non-transparent UL resource muting is supported for interference covariance matrix measurement for gNB-to-gNB CLI handling, the following are recommended to be specified - Definition and indication of UL resource muting pattern - Collision with DMRS/PTRS - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination - Power allocation in symbols with muted REs considering potential impact to phase continuity - TB size determination Note: The existing reference signal time-frequency resource pattern, e.g., PT-RS, comb-2 SRS, are the candidates for the UL resource muting pattern. Note: Consider pattern without adverse impact on PAPR Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4. Note: The above does not apply for PUSCH transmission during random access procedures. |
| IDC | Zero-power RS type of muting indication is an efficient way, analogous to existing ZP-CSI-RS. |
| ZTE | We support the direction of the Alt.1. We fail to see the need to introduce the ZP-SRS. UL muting is different from DL rate matching from resource perspective. For DL rate matching, the gNB actually transmitted signal on the muting RE, which is CSI-RS. So the rate matching is performed around the CSI-RS. That is the reason why ZP CSI-RS is used as reserved resource. However, for UL muting, nothing is transmitted on the muting RE. There is no relationship between ZP SRS and the muting resource. There is no need to use ZP SRS for muting resource configuration. Regarding the spec impact, we think there is no big difference. In addition, for ZP SRS, we need to clarify which mandatory parameter of SRS resource is not applied, which require huge spec impact. |
| Tejas Networks | We think adding new IE for time location and frequency location of UL muting resources is a simple and easer solution. Hence, we support Alt.1. |
| vivo | We think Alt 1 is clearer considering there are so many other parameters in SRS configuration are not needed for UL resource muting. |
| Spreadtrum | Option 1 is more straightforward and has less impact on specification. There is no need to introduce one/two zero-power SRS resource to configure UL muting resources. If legacy SRS resources are used to configure the UL muting resource, there are too many unused parameters in legacy SRS resource configuration and it needs to discuss which parameters applied to UL resource muting. From our perspective, legacy startPosition and nrofSymbols may not suitable for UL resource muting. So for Option 2, it also needs to introduce new resourceMapping parameters to indicate the UL muting symbol, e.g., resourceMapping-r19. So Option 2 has no advantage compared Option 1. So we support Option 1. |
| CMCC | Share similar view with ZTE, and Alt.1 is preferred. |
| DOCOMO | Regardless of how to configure the UL muting pattern, UE behavior on UL muting should be the same. A way with less impact on other parts is preferred. We think Alt-1 is a cleaner solution, since Alt-2 is related with SRS and may need some revisit on SRS behavior. |
| Nokia | We are fine with either option. Option 1 is flexible to be applicable for any PUSCH BW. Option 2 is good for reusing existing design. The benefit of Option 1 is clearer, but good to check other benefit from option 2. |
| LG | For the simplicity and following current specification, we support Alt. 2. |

- Proposal 1-2

For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, down-select from the following alternatives:

- Alt.1: A single configuration of up to two UL muting symbols is provided per UE, i.e., one configuration for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH

- Alt.2: More than one configuration of up to two UL muting symbols can be provided per UE, e.g., separate configurations for DG PUSCH and Type 1 CG PUSCH

  - FFS: Separate configuration for each of multiple Type 1 CG PUSCH configurations

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support Alt.1 |  |
| Support Alt.2 | Ericsson (Support the direction of Alt-2, but further discussion needed), Xiaomi (separate configuration for Type 1 CG PUSCH and DG PUSCH/Type 2 CG PUSCH), IDC, Sony, ZTE, vivo, Samsung, Spreadtrum, CMCC, DOCOMO (separate configuration for type 1 CG PUSCH and DG/type 2 CG PUSCH), NEC, Nokia, LG |

| Companies | Views |
| --- | --- |
| Ericsson | We support the direction of Alt-2, but some further discussion is needed. We think separate configuration is beneficial for DG-PUSCH and CG-PUSCH, since CG-PUSCH is constrained to have only one SLIV, hence the muting positions that may work for DG-PUSCH might not be good ones for CG-PUSCH. Why is Alt-2 limited to CG-PUSCH Type-1? Shouldn’t it apply to Type-2 as well? Also, we think some discussion is needed on whether or not UL resource muting should apply to PUSCH scheduled by DCI 0_0? By analogy, ZP-CSI-RS used for PDSCH rate matching is not applicable to PDSCH scheduled by DCI 1_0 (see 38.214 Section 5.1.4.2) |
| Xiaomi | Based on the following agreements, our understanding is that the muting symbols are separately configured for Type 1 CG PUSCH and DG PUSCH/Type 2 CG PUSCH. Furthermore, for Type 1 CG PUSCH, it is natural that it is configured per Type 1 CG PUSCH configuration. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH - Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI - Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH. FFS: details of semi-static configuration and dynamic indication (if supported) FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI |
| Sony | Similar view with Ericsson, Alt-2 is also applicable for Type 2 CG-PUSCH. |
| ZTE | We think Alt-2 is more aligned with the previous agreement. We have agreed a configuration for CG-PUSCH and then have another configuration for DG/Type-2 CG PUSCH. Therefore, separate configuratoin should be supported. |
| Spreadtrum | Separate UL muting symbols should be configured for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH. |
| QC | Not support this proposal and Alt 2 is not clear. Single UL muting pattern shall be configured for DG PUSCH, Type 1 CG PUSCH and Type 2 CG PUSCH. With agreed single pattern for DG PUSCH and Type 2 CG PUSCH, there is no use case to have different muting patterns for Type 1 CG PUSCH. However, a configuration in Type 1 CG PUSCH RRC configuration shall be ok that “To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH.” |
| Nokia | We support Alt 2, we don’t think CG PUCSH type 1 should be configured the same muting pattern as the other types. |

- Proposal 1-3

Confirm the following working assumption

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Ericsson, Xiaomi, IDC, Sony, ZTE, Tejas Networks, vivo, Samsung, Spreadtrum, CMCC, NEC, Nokia, LG |
| Not support | QC |

| Companies | Views |
| --- | --- |
| Ericsson | We support to confirm the WA; from the perspective of having consistency between CP-OFDM and DFT-s-OFDM. We see no reason to have less flexibility for DFT-s-OFDM than CP-OFDM. |
| ZTE | The unified solution should be applied to CP-OFDM and DFT-S-OFDM. |
| QC | Different from CP-OFDM, in which case the configured comb offset can help with UL muting symbol overlapping with PTRS symbol, for DFT-s-OFDM, there is no use case to configure comb offset {0,1}. A predefine comb offset {0} can simply the spec impact and simply UE implementation for UL muting for DFT-s-OFDM. |
| DOCOMO | If Proposal 1-8 is agreed, UL muting is not applied on symbol containing PTRS REs, the motivation to have configured comb offset value is not clear. We think a predefined comb offset value is enough. |
| Nokia | With this agreement, we prefer half-DFT sized implementation in the specification for better reading. |
| LG | It is our understanding that this is only targeting for whether UL muting for a PUSCH is supported or not, and we have separate proposal for the case whether it impacts to PTRS or not. |

- Proposal 1-4

A UE can be configured to applied UL resource muting in SBFD symbols, non-SBFD symbols or both SBFD symbols and non-SBFD symbols.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support |  |
| Not support | Xiaomi, Tejas Networks, CMCC, DOCOMO, NEC, Nokia, LG |

| Companies | Views |
| --- | --- |
| Xiaomi | The intention to have additional configuration is not clear to us. |
| ZTE | We don’t see the need to apply UL muting in non-SBFD symbols as there is no gNB-to-gNB CLI in the UL symbols. We prefer to apply the UL muting only in the SBFD symbols. This also align with Proposal 1-5. |
| Tejas Networks | We are not clear why do we need to configure the UL resource muting in non-SBFD symbols. |
| vivo | The reason to apply UL resource muting is non-SBFD symbols is not clear to us |
| Samsung | Can support if majority of companies supports. |
| Spreadtrum | Whether UL muting can be applied to Configuration 1 or Configuration 2 should be discussed first. |
| CMCC | For both SBFD Deployment Case 3 and SBFD Deployment Case 1, UL resource muting is used to estimate interference covariance matrix from gNB-gNB intra/inter-subband CLI in SBFD symbol(s). Therefore, it is reasonable to only apply UL resource muting for SBFD symbol(s), and there is no need to apply UL resource muting for non-SBFD symbol(s). |
| QC | The intention of the proposal is not clear. If the network operates in SBFD operation, then UL muting is restricted to SBFD symbols only. |
| DOCOMO | The motivation of such configuration is not clear. |
| Nokia | We don’t think muting is necessary for non-SBFD symbol. |
| LG | Considering the primary assumption of SBFD is that having aligned SBFD configurations among gNBs, we don’t think UL resource muting in non-SBFD symbol is needed. |

- Proposal 1-5

For PUSCH repetition Type A in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions in SBFD symbols.

For PUSCH repetition Type B in SBFD symbols with Configuration 2, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions in SBFD symbols.

Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Ericsson, Xiaomi, Sony, tejas Networks, vivo, Samsung, Spreadtrum, Panasonic, CMCC, QC, DOCOMO, Nokia, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | We support the proposal, but suggest a change in wording to be a little more precise: change “in that overlap SBFD symbols” |
| Xiaomi | Regarding the proposed change from Ericsson, is the intention to consider a repetition partially overlaps with SBFD symbols? |
| ZTE | We support this proposal with the same comments above. |
| Spreadtrum | We prefer original proposal. |

- Proposal 1-6

For multi-PUSCH scheduled by a single DCI in SBFD symbols, down-select from the following alternatives

- Alt.1: The same time location of none, one or two UL muting symbol(s) is applied for all PUSCH transmissions in SBFD symbols.

  - A new parameter ul-MutingIndicator is introduced to PUSCH-TimeDomainResourceAllocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled.

- Alt.2: The same time location of none, one or two UL muting symbol(s) is applied per PUSCH transmission in SBFD symbols.

  - A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled.

Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support Alt.1 | Xiaomi (1st preference),Spreadtrum, Panasonic, QC, DOCOMO, LG |
| Support Alt.2 | Xiaomi (2nd preference), ZTE, Nokia |

| Companies | Views |
| --- | --- |
|  | - We don’t understand Alt-2. Why would one configure the muting flag per PUSCH, when the same time locations are applied to all PUSCHs anyway? For Alt-1, the proposal as written is not clear on whether the new parameter ul-MutingIndiator is configured per row of the TDRA table. We think it should be configured per row, such that muting applies to all PUSCHs that overlap SBFD at least SBFD symbols. Furthermore, we think there should be a separate proposal for ul-MutingIndicator that applies to both a multi-PUSCH TDRA table and a single-PUSCH TDRA table rather than bundling this in the proposal only for multi-PUSCH. Hence we think it is sufficient to discuss the following simpler proposal: - Proposal 1-6 For multi-PUSCH scheduled by a single DCI in SBFD symbols, down-select from the following alternatives - Alt.1: The same time location of none, one or two UL muting symbol(s) is applied for all PUSCH transmissions in that overlap SBFD symbols. - A new parameter ul-MutingIndicator is introduced to PUSCH-TimeDomainResourceAllocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled. - Alt.2: The same time location of none, one or two UL muting symbol(s) is applied per PUSCH transmission in SBFD symbols. - A new parameter ul-MutingIndicator is introduced to PUSCH-Allocation-r16 to indicate whether the semi-statically configured UL muting symbol(s) is enabled. Note: whether UL resource muting is also applied in non-SBFD symbols is discussed separately |
| Xiaomi | We understand that for multi-PUSCH scheduling, TDRA is separately configured for PUSCHs in different slot, so it is possible to configure whether to enable UL muting independently for PUSCHs in different slots, i.e., Alt.2. Alt. 2 is more flexible but the question is whether such flexibility is really needed. |
| Sony | We agreed not to introduce new DCI field to enable/disable UL muting. We should use TRDA to perform this indication as agreed. |
| ZTE | We support Alt-2. The multiple PUSCH scheduing is to reduce the DCI size. It does not mean that all the scheduled PUSCH should have the same indication for UL muting. Note they are in the multiple slots, which should have the separate indication. This is the same as the single DCI scheduling. |
| vivo | Alt 2 is more flexible. But alt 1 may be sufficient. |
| Samsung | Same view as Ericsson |
| Spreadtrum | For multi-PUSCH scheduled by a single DCI, it is not necessary to turned ON/OFF the UL muting symbols separately for each PUSCH. |
| Panasonic | We support Alt.1 since Alt.1 is a simpler design than Alt.2. |
| QC | Similar view as Ericsson that support Alt 1. And 1) no need to mention the name of the new parameter in sub bullet; 2) Sub bullet is common for single PUSCH as well. |
| Nokia | Support Alt 2, multi-PUSCH scheduling includes independent PUSCH-Allocation-r16, and we think muting indicator is included in each PUSCH-Allocation-r16. |

- Proposal 1-7

Update the following agreement in RAN1#118bis as follows (marked in red)

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

  - The MscPUSCH/2 modulated symbols are transformed to the frequency domain with a DFT of size MscPUSCH/2, and the result is mapped to the MscPUSCH/2 unmuted scheduled subcarriers.

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

  - Option 2a: The block of MscPUSCH/2 modulated symbols is repeated before DFT. After DFT, the MscPUSCH/2 non-zero even-indexed subcarriers are extracted and then mapped to the unmuted scheduled subcarriers.

  - Option 2b: The MscPUSCH/2 modulated symbols are block-wise spreading by OCC [+1,+1] or [+1,-1] depending on whether the comb offset is 0 or 1 into a length-MscPUSCH sequence of modulated symbols before DFT. After DFT, the result is mapped to the MscPUSCH scheduled subcarriers.

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

Note: Option 1, Option 2a and Option 2b are mathematically equivalent.

FFS: Whether Option 1, Option 2a and Option 2b enforce gNB/UE implementation choices in practice.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi, Sony, Tejas Networks, Samsung (except Note/FFS), Spreadtrum, CMCC (Option 2b is preferred), DOCOMO, Nokia, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | Please note that Option 2b is not mathematically equivalent to Option 1 & 2a. We think the discussion should be limited to Option 1 and 2a since they are mathematically equivalent. We don’t think it is necessary to specify OCCs. We think it is fine to leave the selection of Option 1 or 2a up to UE implementation. Both of these options do have spec impact, but we think that Option 1 is easier to specify (see Appendix B in our contribution R1-2500751). To allow implementation freedom at the UE, we think it should be straightforward to add some wording to the specifications to say that an implementation based on block repetition followed by extraction of even-indexed samples mapped to unmuted carriers (i.e., Option 2a) is mathematically equivalent to the specified Option 1. |
| Xiaomi | We think it is a very good proposal to make the details clearer. According to our understanding, Option 2b is mathematically equivalent to other options and it is our first preference. In addition, we are wondering whether it is common understanding that regardless of which option is adopted, it needs to be reflected in RAN1 spec. |
| ZTE | We support Option 1. Option 2a is the most complicated method. It includes repeating the sequence prior to the DFT operation, selecting the even-indexed symbols, and mapping to the unmuting resources and skipping the muting resources. It should not be adopted. Option 2b introduces addition OCC operation, which is not included in the discussion before. |
| Tejas Networks | We think Option-1 is easy and simple to implement and specify in the standard. We are also fine with adding Option 2a in the standard to provide implementation freedom as mentioned by Ericsson. |
| vivo | We want to add option 2c for discussion: The block of MscPUSCH/2 modulated symbols is extended to MscPUSCH modulated symbols by inserting ‘0’ after every modulated symbol. After DFT, the first MscPUSCH/2 subcarriers are extracted and then mapped to the unmuted scheduled subcarriers. As discussion in our tdoc, for option 2c, the sequence after DFT is a repetitive sequence. the first MscPUSCH/2 subcarriers in option 2c are same as that of the result in option 2a. We support to leave this as UE implementation. |
| Samsung | We need to first discuss whether to support half DFT size for UL muting symbol. If the same DFT size is kept, we need to down-select between 2a and 2b. Our understanding is that options 2a/2b are not mathematically the same. For the last FFS, our understanding is the equation is a reference for signal generation. Actual implementation can be up to gNB/UE vendor. |
| CMCC | We prefer to Option 2b. For Option 1, both Clause 6.3.1.4 (transform precoding procedure) and Clause 6.3.1.6 (mapping to virtual resource blocks procedure) in TS 38.211 need modification. And additional scheduling restriction is needed to avoid odd number of PRBs to guarantee the DFT size as 2α2∙3α3∙5α5. For Option 2a, both Clause 6.3.1.4 and Clause 6.3.1.6 in TS 38.211 need modification. For Option 2b, only TS 38.211 may be modified to support block-wise pre-DFT OCC before transform precoding, and the OCC sequence [+1,+1] and [+1,-1] are corresponding to a comb offset 0 and 1, respectively. |
| QC | We think Option 1 and Option 2a is good enough. No need to mention Option 2b in the proposal. Option 2b is also not mathematically equivalent to Option 1 & 2a. We think from spec impact simplicity point of view, Option 2a should be the specified option. RAN1 shall make a conclusion to conclude that Option 1 and Option 2a is mathematically equivalent and leave UE implementation flexibility on either option. |
| Nokia | Support Alt 1. To support comb offset {0,1}, alt 1 is the better way without confusion. |
| LG | Considering the options are provided to minimize the impact on the UE implementation, introducing OCC is not a good way to go. On top of that, as many other companies mentioned, it is not mathematically equivalent to option 1 and 2a. It is our understanding that option 2a requires more UE complexity since UE needs to reorder the input bits for DFT and extract only part of them. If the half size of DFT is supported by the UE, option 1 is straightforward and will not impact UE behaviour that much. |

- Proposal 1-8

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and when transform precoding is enabled for the PUSCH, PT-RS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi, Tejas Networks, vivo, Samsung, Spreadtrum, QC, DOCOMO, NEC, Nokia, LG |
| Not support | Ericsson, CMCC |

| Companies | Views |
| --- | --- |
| Ericsson | We have strong concerns with this proposal. In RAN1#119, we raised a concern about potentially forbidding the configuration of PTRS time domain density L = 1 (i.e., PTRS in every OFDM symbol) in conjuction with UL resource muting. This concern also applies to DFT-s-OFDM PUSCH since we identify FR2 deployments as the more suitable targets for SBFD technologies. For FR2 deployments, it is important to be able to configure PTRS in every symbol to achieve good phase tracking/compensation performance. It is also important to use DFT-s-OFDM PUSCH to improve UL coverage. Mitigating CLI is needed in FR2, for example, for street level deployment for more urban areas and also for critical use cases such as industrial deployments. We also point out that, as captured in this FL summary (see Section 3.1.3), there is in fact, not even a real collision between RE muting and PTRS. Hence we strongly feel that a solution needs to be found to allow the combination of PTRS every symbol (L = 1) and UL resource muting. We are open to discuss a viable solution – companies have listed several in their contributions, e.g., using only the first half of the PTRS pattern. We think an even simpler solution with minimal UE complexity is the following: Proposal: For RE muting in a DFT-s-OFDM PUSCH symbol, the UE determines the number of PTRS groups and the number of samples per group using N_RB/2 and the locations of the PTRS samples using MscPUSCH according to existing specs. There is no need for any other specs changes than applying the acutal frequency domain resource size with existing procedures. With this proposal, we point out that the UE uses legacy PTRS patterns and that the UE inserts the PTRS samples in a vector that is half the length prior to the DFT (using Option 1 in FL Proposal 1-7) or in the block of samples prior to repetition (using Option 2a in FL Proposal 1-7). |
| ZTE | Not support. We don’t understand why the PTRS and UL muting resource cannot be in the same symbol. For CP-OFDM, we have agreed that they can be in the same OFDM symbol and the network can avoid the collision in the frequency domain. For DFT-S-OFDM, the modulation symbols are only mapped to the unmuting resource as discussed in Proposal 1-7. There is never collision. If we agree this proposal, I think we need to revisit the collision handling between PTRS and and muting resource for CP-OFDM. And the same method should be applied. |
| Samsung | The motivation of the following objective is not to change any RS patterns by introducing UL muting. For CP-OFDM, PT-RS pattern is not changed and thus no implementation impacts for transmitting/receiving the PT-RS on UL muting symbol. However, DFT-S-OFDM, it clearly changes PT-RS position since all PT-RS samples are multiplexed to all subcarriers after DFT spreading. • UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol And, the UL muting is not only solution. gNB can use empty symbols/RBs to estimate gNB-gNB CLI without changing PT-RS patterns |
| Spreadtrum | If UL resource muting is applied to symbol with PTRS, PTRS can only be inserted in PUSCH with the size of MscPUSCH/2. Therefore, the PTRS pattern in symbols with UL resource muting is different from that in symbols without UL resource muting which makes joint estimation impossible. So when transform precoding is enabled, PTRS is prioritized. |
| CMCC | Not support. There will be no collision between PT-RS REs and muted REs when transform precoding is enabled since the PT-RS insertion for a PUSCH is before DFT operation. |
| Nokia | When transform precoding is enabled, reducing the DFT size is difficult, because the PTRS patterns is different with different DFT size. When PN is higher, CLI may not be the dominant interference source. We prefer to prioritize PTRS over UL resource muting. |
| LG | Technically speaking, UL resource muting will not impact on PT-RS when the PT-RS insertion is modified. However, it will change the PT-RS position which ends up with increased gNB complexity, which is not desirable. |

- Proposal 1-9

For the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled, down-select from the following two alternatives

- Alt.1: 3 dB power boosting for both PUSCH and PTRS, if present), regardless of the PTRS density K and number of PUSCH layers L

- Alt.2: X dB boosting for all unmuted REs in a symbol with PTRS, where X ≥ 3 dB and depends on the configured value of UL-PTRS-power, PTRS density K, and the used number of PUSCH layers L.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Ericsson, Xiaomi, IDC, Sony, ZTE, Tejas, vivo, Samsung, Panasonic, CMCC, QC, DOCOMO, Nokia, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | We support the proposal, and we prefer Alt-1 due to its simplicity. We don’t think Alt-2 is needed, since X is 3 dB in many combinations of UL-PTRS-power, PTRS density K, number of PUSCH layers L. Only in a few cases, is X larger than 3 dB, and by at most 0.14 dB. This results in negligible variation of the total Tx power across symbols of a PUSCH allocation. |
| ZTE | We support Option 1. It is simple and align with the WID. There is no need to define another value. |
| Tejas Networks | We support the proposal with Atl. 1. |
| Samsung | Alt 1 is preferred. |
| Nokia | We prefer simple solution. Alt 1 is preferable. |

- Proposal 1-10

For UL muting applied for UL TBoMS, the index of the starting coded bit in a slot, except the first slot, within the 𝑁𝑠 slots allocated for the transmission of TB processing over multiple slots is determined as follows

k0=k0'+H+τ mod Ncb

and H is down-selected from one of the following options

  - Option 1: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and assuming no muted REs.

  - Option 2: 𝐻 is the total number of coded bits available for transmission of the transport block in the previous slot within the 𝑁𝑠 slots assuming no UCI multiplexing and considering the muted REs

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Samsung, Spreadtrum, QC |
| Not support |  |

| Companies | Views |
| --- | --- |
| ZTE | First, we should have the common understanding on whether the UL muting can be applied to TBoMS. In addition, we prefer the option without spec impact. |
| vivo | We think to determine the index of the starting coded bit in a slot, for UL resource muting, it can follow that of UCI multiplexing. i.e. option 1 is preferred. |
| Samsung | We prefer Option 1 for less impact/simplicity. |
| Spreadtrum | We prefer Option 2. From our perspective, the UL muted Res is not considered in H determination. It should be clarified in specification. |

  - Others

Timing misalignment issue

ETRI: We propose discussing how/whether to address timing misalignment between CLI-RS and UL signals received by a gNB.

- UE-to-UE CLI handling

The WI objectives related to UL resource muting are listed below.

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

  - L1 UE-to-UE CLI measurement and reporting

    - Measurement resource

      - Configuration of SRS-RSRP resource and CLI-RSSI resource

For configuration of SRS-RSRP resource and CLI-RSSI resource, the following agreements were made until RAN1#119.

Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets Agreement In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI. Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter. Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis Agreement Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement - Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis) - FFS: Other configurations Define SRS-RSRP-MeasurementResource with the following parameters: - Legacy SRS Resource IE - FFS: Other parameters Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters

- Issue#1-2: SRS-RSRP measurement resource configuration

The additional parameters for SRS-RSRP measurement resource are discussed by several companies including rssi-SCS, refBWP and refServCellIndex which are included in L3 based measurement resource SRS-ResourceConfigCLI.

- srs-SCS: CATT, Qualcomm, Xiaomi, ZTE

- refBWP: CATT, Xiaomi, ZTE

- refServCellIndex: Nokia (FFS), Xiaomi

To the moderator’s understanding, unless the intention is to support L1 SRS-RSRP measurement with a different SCS from the active DL BWP (not supported in Rel-16 L3 based SRS-RSRP measurement), srs-SCS is not needed.

refBWP-r16 in SRS-ResourceConfigCLI-r16 is the DL BWP id which is used to derive the reference point of the SRS resource. bwp-Id in CSI-ResourceConfig can be used to indicate the DL BWP where the SRS-RSRP measurement resources are located in. Therefore, refBWP is not needed.

carrier in CSI-ReportConfig can be used to indicate the serving cell where the SRS-RSRP measurement resource are located in hence refServCellIndex is not needed.

The detailed proposals and discussions are listed below.

CATT: Support multiple measurement resources configured for one L1-CLI report.

Define reference DL BWP id in SRS-RSRP-MeasurementResource.

Define reference SCS in SRS-RSRP-MeasurementResource and CLI-RSSI-MeasurementResource.

CEWiT: For definition of measurement resource for L1-based SRS-RSRP following configuration is needed:

- SRS-RSRP-MeasurementResourceSetId - unique identifier for the resource set.

- SRS-RSRP-MeasurementResourceId - unique identifier for individual SRS-RSRP measurement resources within the resource set.

Nokia: For L1 CLI measurement resource configuration, perform a down-selection from Rel-16 parameters by:

- Removing srs-SCS-r16/rssi-SCS-r16

- Removing refBWP-r16

- Updating startPRB-r16 range INTEGER(0..2169) to INTEGER(0..maxNrofPhysicalResourceBlocks)

- FFS: refServCellIndex-r16

NTT DOCOMO:

For L1 CLI measurement and reporting, following parameters are supported in CSI report config:

- Parameters to configure L1 CLI measurement resources:

  - a new IE CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement resource

  - a new IE CLI-RSSI-MeasurementResourceSet for L1 CLI-RSSI measurement resource set

  - a new IE SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement resource

  - a new IE SRS-RSRP-MeasurementResourceSet for L1 SRS-RSRP measurement resource a set

- New report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} for the parameter reportQuantity

- Parameters for triggering aperiodic L1 CLI reporting:

  - a new parameter in CSI-AssociatedReportConfigInfo to select one set from the list of aperiodic CLI-RSSI measurement resource sets or aperiodic SRS-RSRP measurement resource sets

  - a new parameter in CSI-AssociatedReportConfigInfo to indicate a list of TCI state(s) for aperiodic CLI measurement resources

Xiaomi:

Reference serving cell, reference BWP and reference SCS should be included in SRS-RSRP-MeasurementResource.

Reference serving cell and reference SCS should be included in CLI-RSSI-MeasurementResource.

Restrictions to time domain configuration parameters such as Starting symbol and Number of symbols configured in CLI-RSSI-MeasurementResource should be considered.

- CLI-RSSI resource should be within slot boundary of reference SCS and SCS of configured DL BWP

- Starting symbol and Number of symbols are integer multiple of reference SCS divided by configured BWP SCS

ZTE:

The available slot offset should not be supported for aperiodic SRS for CLI measurement.

For the configuration of the SRS resource, the DL BWP ID, the SCS, and the physical cell ID of the aggressor UE should be needed and the reference serving cell should not be needed.

Qualcomm:

Proposal 2: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource.

Proposal 3: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

Proposal 4: Support reuse same “carrier” parameter in CSI-ReportConfig IE to indicate the CLI measurement CC.

Proposal 5: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource for cross CC CLI measurement.

Proposal 6: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid of the carrier/cell that UE measures the CLI for cross CC CLI measurement.

Proposal 13: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, no need to further discuss the FFS: whether available slot offset list is also reused.

Spreadtrum:

The slot offset and available slot offset configured in legacy SRS-ResourceSet is not reused for SRS-RSRP measurement.

Whether the frequency reference point of CLI measurement resource is defined by reference parameters of CLI resource or CIS resource should be discussed.

Ericsson: Conclude that it is not necessary to define a reference BWP, reference serving cell, or reference SCS in the measurement resource configurations for L1-based SRS-RSRP and CLI-RSSI measurement.

- Issue#2: CLI-RSSI measurement resource configuration

The additional parameters for CLI-RSSI measurement resource are discussed. Qualcomm and Xiaomi proposed to configure rssi-SCS, while Nokia thought rssi-SCS should be removed. For refServCellIndex, ZTE thought there is no need to configure it, Nokia proposed FFS, while Xiaomi proposed it should be included. Again, Ericsson had a view that a reference BWP, reference serving cell, or reference SCS in the measurement resource configurations for L1-based SRS-RSRP and CLI-RSSI measurement.

Similar to the analysis for the SRS-RSRP measurement resource above

bwp-Id in CSI-ResourceConfig can be used to indicate the DL BWP where the CLI-RSSI measurement resource are located in. UE can assume that the CLI-RSSI measurement resources have the same SCS with the DL BWP.

The parameter carrier in CSI-ReportConfig can be used to indicate the serving cell where the CLI-RSSI measurement resources are located in hence refServCellIndex is not needed.

ZTE and Nokia proposed to support new MAC-CE for semi-persistent L1-CLI-RSSI measurement resource activation/deactivation.

The detailed proposals and discussions are listed below.

Nokia: For L1 CLI measurement resource configuration, perform a down-selection from Rel-16 parameters by:

- Removing srs-SCS-r16/rssi-SCS-r16

- Removing refBWP-r16

- Updating startPRB-r16 range INTEGER(0..2169) to INTEGER(0..maxNrofPhysicalResourceBlocks)

- FFS: refServCellIndex-r16

Support new MAC-CE for semi-persistent L1-CLI-RSSI measurement resource activation/deactivation.

NTT DOCOMO: For L1 CLI measurement and reporting, following parameters are supported in CSI report config:

- Parameters to configure L1 CLI measurement resources:

  - a new IE CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement resource

  - a new IE CLI-RSSI-MeasurementResourceSet for L1 CLI-RSSI measurement resource set

  - a new IE SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement resource

  - a new IE SRS-RSRP-MeasurementResourceSet for L1 SRS-RSRP measurement resource a set

- New report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} for the parameter reportQuantity

- Parameters for triggering aperiodic L1 CLI reporting:

  - a new parameter in CSI-AssociatedReportConfigInfo to select one set from the list of aperiodic CLI-RSSI measurement resource sets or aperiodic SRS-RSRP measurement resource sets

  - a new parameter in CSI-AssociatedReportConfigInfo to indicate a list of TCI state(s) for aperiodic CLI measurement resources

Qualcomm:

Proposal 1: Support SRS-RSRP-MeasurementResourceSet or CLI-RSSI-MeasurementResourceSet to be defined per cell under ServingCellConfig IE, under existing csi-MeasConfig.

Proposal 2: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource.

Proposal 3: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid.

Proposal 4: Support reuse same “carrier” parameter in CSI-ReportConfig IE to indicate the CLI measurement CC.

Proposal 5: Support to configure a subcarrier spacing per SRS-RSRP-MeasurementResource/CLI-RSSI-MeasurementResource for cross CC CLI measurement.

Proposal 6: Support frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid of the carrier/cell that UE measures the CLI for cross CC CLI measurement.

Proposal 7: RAN1 to discuss AP triggered a CLI measurement time window with one or more CLI-RSSI resources configured per slot in the time window with multiple slots to perform CLI measurement and differentiate different aggressor UEs over different time occasions.

Proposal 13: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, no need to further discuss the FFS: whether available slot offset list is also reused.

ZTE:

Regarding the UE behaviour for CLI-RSSI measurement, there should be no specification impact.

A new MAC CE should be introduced for activating or deactivating the semi-persistent CLI-RSSI measurement resource.

There should be no need to configure the reference serving cell for the CLI-RSSI measurement resource.

Xiaomi:

Reference serving cell and reference SCS should be included in CLI-RSSI-MeasurementResource.

Restrictions to time domain configuration parameters such as Starting symbol and Number of symbols configured in CLI-RSSI-MeasurementResource should be considered.

- CLI-RSSI resource should be within slot boundary of reference SCS and SCS of configured DL BWP

- Starting symbol and Number of symbols are integer multiple of reference SCS divided by configured BWP SCS

Ericsson: Conclude that it is not necessary to define a reference BWP, reference serving cell, or reference SCS in the measurement resource configurations for L1-based SRS-RSRP and CLI-RSSI measurement.

- Issue#3: The maximum number of L1-SRS-RSRP and L1-CLI-RSSI resources

Several companies discussed the maximum number of L1-SRS-RSRP and L1-CLI-RSSI resources.

The detailed proposals and discussions are listed below.

Huawei: For each CSI Reporting Setting, UE can be configured with up to 32 SRS-RSRP resources and 64 CLI-RSSI resources for L1 based CLI measurement depending on UE capability.

The RRC parameters for L1-SRS-RSRP and L1-CLI-RSSI measurement has been proposed in [3]. The values of a couple of variables need to determined including the following

- maxNrofSRS-RSRP-MeasurementResourceSets: -- Maximum number of SRS-RSRP measurement resource sets per cell

- maxNrofSRS-RSRP-MeasurementResources: -- Maximum number of SRS-RSRP measurement resources

- maxNrofSRS-RSRP-MeasurementResourceSetsPerConfig: -- Maximum number of SRS-RSRP measurement resource sets per resource configuration

- maxNrofSRS-RSRP-MeasurementResourcesPerSet: -- Maximum number of SRS-RSRP measurement resources per resource set

- maxNrofAP-SRS-RSRP-MeasurementResourcesPerSet: -- Maximum number of aperiodic SRS-RSRP measurement resources per resource set

- maxNrofCLI-RSSI-MeasurementResourceSets: -- Maximum number of CLI-RSSI measurement resource sets per cell

- maxNrofCLI-RSSI-MeasurementResources: -- Maximum number of CLI-RSSI measurement resources

- maxNrofCLI-RSSI-MeasurementResourceSetsPerConfig: -- Maximum number of CLI-RSSI measurement resource sets per resource configuration

- maxNrofCLI-RSSI-MeasurementResourcesPerSet: -- Maximum number of CLI-RSSI measurement resources per resource set

- maxNrofAP-CLI-RSSI-MeasurementResourcesPerSet: -- Maximum number of aperiodic CLI-RSSI measurement resources per resource set

NTT DOCOMO: Maximum number of L1 CLI-RSSI measurement resources (regardless of type #1 and type 2) is same as the maximum number of L3 CLI-RSSI measurement resources.

CATT: Maximum number of CLI-RSSI measurement resources and CLI-SRS measurement resources are 64 and 32 respectively per report.

Samsung: The maximum configured number of L1 CLI-RSSI measurement resources in CLI-RSSI-MeasurementResourceSet is same as maxNrofCLI-RSSI-Resources-r16.

- The supported number of CLI-RSSI measurement resources is up to UE capability

- One L1 CLI-RSSI Measurement resource type#2 is counted as two L1 CLI-RSSI Measurement resource type#1

The maximum configured number of L1 SRS-RSRP measurement resources in SRS-RSRP-MeasurementResourceSet is same as value of maxNrofSRS-RSRP-Resources-r16.

- The supported number of SRS-RSRP measurement resources is up to UE capability

Spreadtrum: Maximum number of CLI resource set per configuration should be defined in CSI-ResourceConfig.

Maximum number of CLI resource per resource set in Rel-16 can be reused in new CLI resource set.

ZTE: The maximum number of the CLI-RSSI resource within one CSI-ResourceConfig should be 64.

- Others

MediaTek: Support configuration of a new IE (e.g., SRS-CliResourceSet) containing a set of SRS resources to be transmitted solely for the purpose of L1 SRS-RSRP measurement.

Support power scaling of SRS-RSRP measurement results or new SRS-RSRP reporting value range when using enhance SRS power control optimized for dedicated SRS transmission for L1 SRS-RSRP measurement.

      - Determination of ‘typeD’ QCL for CLI measurement resource

For configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource, the following was agreed in RAN1#118bis and RAN1#119.

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. - If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Qualcomm proposed to discuss default beam rules to handle the case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming:

- Option 1: An error case that UE does not expect to be configured with a scheduling offset between the last symbol of the PDCCH carrying the triggering DCI for AP CLI and the first symbol of the aperiodic CLI SRS-RSRP or CLI-RSSI resources is smaller than the UE reported threshold beamSwitchTiming.

- Option 2: Same default beam rules as AP CSI-RS resource defined in 38.214 can be applied to AP CLI SRS-RSRP or CLI-RSSI resource.

InterDigital:

Support UE configuration where the UE is indicated one or more RX beam(s) for CLI resources (e.g., L1-SRS-RSRP), e.g., by indication of a CSI resource indicator (CRI) from the network for semi-persistent and periodic reporting.

Support UE configurations to enable CLI-based beam-sweeping, where the UE may determine and report the most and least favourable beam(s) or beam pairing(s) between the victim and aggressor UEs.

Support the UE determination of beam pairing between (victim) UE and gNB based on directional CLI measurements and reports.

Support methods to restrict UL beam directions for a configured UL transmission at an aggressive UE based on scheduled victim UEs, that is only for the occasions that a respective victim UE is scheduled for DL reception.

Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

    - Measurement reporting

      - SBFD specific UE-to-UE CLI measurement issue

- Issue 1: Frequency resource allocation for Method#3: UE measures RSSI within UL subband

For Measurement behaviour within the active DL BWP, the following agreement was made until RAN1#119

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

CEWiT, NTT DOCOMO, InterDigital proposed to explicitly indicate the subband (UL or DL) for RSSI measurement. NTT DOCOMO and InterDigital also proposed to implicitly determine the subband (UL or DL) for RSSI measurement.

Companies detailed proposals are listed below.

CEWiT: For a CSI report configured with CSI-ReportConfig, where the higher-layer parameter reportQuantity is set to ‘cli-RSSI’, the CSI-ReportConfig should explicitly indicate the subband (UL or DL) for RSSI measurement.

NTT DOCOMO: When reportQuantity is set as ‘cli-RSSI’, whether Method#1 or Method#3 to be used can be determined by explicit parameter configuration, or implicitly determined based on number of PRBs within DL usable PRBs for the configured CLI-RSSI measurement frequency resource.

InterDigital: Support one of the following alternatives for the configuration of the CLI-RSSI measurement resources:

- Alt.1: UE assumes a Method #3 measurement configuration if the configured set of CLI-RSSI-MeasurementResource within a CLI-RSSI-MeasurementResourceSet indicate frequency resources exclusively within the configured UL subband; and Method #1 otherwise.

- Alt.2: A new parameter indicating the Method type is added in CLI-RSSI-MeasurementResource or CLI-RSSI-MeasurementResourceSet and the UE excludes any frequency resources not included within the corresponding usable PRBs.

Xiaomi: For frequency resource allocation of a CLI-RSSI measurement resource, support measurement resource type#3 in addition to agreed measurement resource type#1 and type#2.

- Measurement resource type#3: One CLI-RSSI measurement resource is configured within a UL subband

For frequency resource allocation of a CLI-RSSI measurement resource,

- the frequency resources of CLI-RSSI measurement resource type#2 are derived by excluding the frequency resources outside DL usable PRBs.

- the frequency resources of CLI-RSSI measurement resource type#1 and #3 are the configured PRBs.

- Issue 2: Timing for Method#3: UE measures RSSI within UL subband

For L1 UE-to-UE CLI measurement based on Method#3, Qualcomm and Vivo proposed the reference timing is same as DL reference timing. CMCC proposed to consider reuse the timing for Rel-16 L3 based CLI-RSSI measurement or DL reference timing.

CMCC: For L1 UE-to-UE CLI measurement and reporting, the reference timing for L1 CLI-RSSI measurement in Method#3 should be discussed and following options can be considered:

- Alt. 1: Reuse the timing for Rel-16 L3 based CLI-RSSI measurement, i.e., a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

- Alt. 2: Use the DL timing.

Agreement@119 For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Qualcomm: For L1 CLI-RSSI measurement, DL timing is used for Method #3 (UE measures RSSI within UL subband).

Vivo: For L1 based UE-to-UE CLI-RSSI measurement within UL subband, the reference timing for CLI measurement is same as DL reference timing.

- Issue 3: Others

Qualcomm proposed to support CLI SRS-RSRP or CLI-RSSI measurement can only apply to Configuration 1 that the measurements are restricted to SBFD symbols only for SBFD operation or non-SBFD symbols only for dynamic/flexible TDD operation.

- FFS: An indication of valid symbol type for the measurement

      - Reporting in time domain

- Issue 1: Aperiodic CLI reporting

It has been agreed that aperiodic CLI reporting is supported for L1 UE-to-UE CLI reporting. Companies discussed the detailed mechanism of aperiodic CLI reporting.

Agreement For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter. - FFS: Whether to reuse the legacy SRS resource set - FFS: Whether available slot offset list is also reused For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter. Agreement For aperiodic CLI reporting on PUSCH - Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig - Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report - Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig - Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources. Conclusion There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

Apple : For a CLI-RS resource configured as aperiodic by the higher-layer parameter resourceType, candidate slots in which the triggered CLI-RS resource may be used for CLI measurement is the first SBFD slot that is not earlier than n+k, where n is the slot in which triggering DCI is received, and k is the higher layer configured parameter slotOffset.

Samsung: The existing/legacy CSI-Request field in DCI 0_1, 0_2 or 0_3 can be reused to trigger the aperiodic L1-CLI-RSSI or L1-SRS-RSRP report from the UE.

LG: Reuse aperiodicTriggeringOffset for both of SRS-RSRP and CLI-RSSI

NTT DOCOMO: Parameters for triggering aperiodic L1 CLI reporting:

  - a new parameter in CSI-AssociatedReportConfigInfo to select one set from the list of aperiodic CLI-RSSI measurement resource sets or aperiodic SRS-RSRP measurement resource sets

  - a new parameter in CSI-AssociatedReportConfigInfo to indicate a list of TCI state(s) for aperiodic CLI measurement resources

ZTE: The available slot offset should not be supported for aperiodic SRS for CLI measurement.

- Issue 2: Periodic CLI reporting

Some companies discussed whether or not support periodic CLI reporting. Views are summarized below.

- Support: CATT, CMCC, Google, InterDigital, MediaTek, ZTE, LG, Xiaomi

- Not Support: Ericsson, NTT DOCOMO, Samsung

Companies’ detailed proposals or discussions are listed below.

CMCC: For L1 based UE-to-UE CLI measurement and reporting, support periodic and semi-persistent reporting in addition to aperiodic reporting.

Table 1. Supportable combinations of CLI reporting configurations and measurement configurations for victim UE

| SRS-RSRP/CLI-RSSI measurement resource type | Periodic CLI reporting | Semi-persistent CLI reporting | Aperiodic CLI reporting |
| --- | --- | --- | --- |
| Periodic | Support | Support | Support |
| Semi-persistent | Not support | Support | Support |
| Aperiodic | Not support | Not support | Support |

MediaTek: For periodic CLI reporting

- Network does not need to trigger specific UE to transmit SRS resources for SRS-RSRP measurement, which implies no DL control overhead.

- Victim UE can occasionally update network of the CLI status, without waiting for network instruction for each individual CLI report.

- Issue 3: Semi-persistent CLI reporting

Some companies discussed whether or not support SP CLI reporting. Views are summarized below.

- Support: CATT, CMCC, Google, InterDigital ZTE, LG

- Not Support: Ericsson, MediaTek, NTT DOCOMO, Samsung

Companies’ detailed proposals or discussions are listed below.

CATT: For some scenarios, periodic reporting is beneficial, e.g. periodically monitoring CLI strength from neighbour gNB whose traffic load information is not available for serving gNB. CLI handling can be done in time without delay generated from triggering the aperiodic CLI measurement and reporting when strong CLI is measured.

Ericsson: For measurement reporting for UE-UE CLI mitigation, none of the following is supported: aperiodic reporting on PUCCH, semi-persistent reporting (on PUSCH or on PUCCH), periodic reporting on PUCCH.

InterDigital: Although CLI may quickly change over time, periodic/semi-persistent measurement and reporting for CLI may be used by the network to order regular measurements by the UE and prepare its scheduling accordingly to enable CLI measurements, without increasing the overhead of dynamically signalling each CLI measurement and reporting.

MediaTek: Due to the MAC CE signalling required to activate/deactivate semi-persistent L1 SRS-RSRP and L1 CLI-RSSI reporting, we prefer to deprioritize semi-persistent reporting for L1 UE-to-UE CLI.

LG: Considering the motivation of introducing L1 based CLI measurement and report is mostly reducing the reporting latency, it will induce too much signaling overhead when only aperiodic reporting is supported. That is, every time when gNB needs to aware of on-going CLI situation at the victim UE, triggering indication via DCI and following UCI report should be done. It is better to provide scheduling flexibility at gNB side by supporting periodic/semi-persistent/aperiodic CLI reporting to prevent it.

      - Reporting quantities

For reporting quantity, the following agreement was made in RAN1#118.

Agreement For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity. - FFS: configuration of number of reported CLI resources in the reportConfig - FFS: reporting criteria Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range

- Issue 1: Whether support least interfered measured L1-CLI-RSSI or L1-SRS-RSRP or not

CATT, InterDigital, NEC, Qualcomm proposed to consider smallest CLI resource(s). In addition, Qualcomm also proposed to support configuration of reporting criteria of {most interfering, least interfering} CLI resources in the reportConfig.

- Support: CATT, InterDigital, NEC, Qualcomm

CATT: Reporting X least measured L1-CLI-RSSI value(s) or L1-CLI-RSRP value(s) is supported.

InterDigtial: For L1 based SRS-RSRP and CLI-RSSI reporting, support that the UE report the most X interfering or Y least interfering CLI measurement resources.

- We think it’s beneficial that the UE can be configured with reporting most and least inference resources. If one focuses only on the CLI identification, the most interfered resources may be sufficient for resource management. However, reporting the least interfered CLI measurement resources may also be useful information for the network coupled with beam management, e.g., to enable the network to schedule UEs to associate beam pairs for UEs without CLI. See more details on beam aspects in section 2.1.4.

NEC: For L1 based SRS-RSRP reporting, the following options are considered

•	Option 3: UE is configured to report the X most interfering or Y least interfering SRS-RSRP measurement resources in one report.

•	Option 4: UE is configured to report the X most interfering and Y least interfering SRS-RSRP measurement resources in one report.

For L1 based CLI-RSSI reporting, the following options are considered

•	Option 3: UE is configured to report the X most interfering or Y least interfering CLI-RSSI measurement resources in one report.

•	Option 4: UE is configured to report the X most interfering and Y least interfering CLI-RSSI measurement resources in one report.

Qualcomm: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support configuration of reporting criteria of {most interfering, least interfering} CLI resources in the reportConfig.

- One drawback for reporting the most interfering CLI resources, is that, if gNB configures to report top two most interfering CLI resources, it could be possible that the third most interfering CLI resources which is not included in the report still causes significant CLI impact to the measurement UE. In this case, if gNB only avoids scheduling DL UE pairing with corresponding UL UE on the most interfering aggressor UE list, it is not enough to avoid the CLI impact with the potential possibility that gNB may schedule the DL UE with the third high CLI UL UE for full duplex operation, which is different than L3 with higher payload size that can report more CLI resources.

- Not support: CMCC, Ericsson, Google, Nokia, OPPO, Panasonic, Sony, Vivo, Xiaomi

CMCC: For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, reporting of the least interfered measured L1 CLI-RSSI or CLI-SRS-RSRP is not supported.

Ericsson: Conclude that besides the agreed reporting of the M most-interfered CLI measurements, “other reporting criteria” are not supported.

Google: Don’t Support reporting the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP.

Nokia: We think this way of reporting is not optimal since it doesn’t inform the gNB about the aggressor UEs. Thus, we prefer to only rely on the most interfered measured resources.

OPPO: The motivation to report the least interfered measurement resources is to schedule DL UE pairing with corresponding UL UE chosen from the least interfered measurement resources, but UE-UE CLI is typically burst and dynamically fluctuated, and the frequent switching of UE pairing may bring scheduling complexity. Thus, the necessity and benefit to report the least interfered CLI measurement resource is not strong enough.

Panasonic: However, to avoid the situation of “there are many aggressor UEs with large CLI around the victim UE”, it is possible to detect such a situation in advance, i.e., the situation of "there are many aggressor UEs with large CLI around the victim UE" can be avoided in advance since gNB knows aggressor UE with a large CLI by reported measurement values. Therefore, the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is sufficient for UE-to-UE CLI handling than the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP.

Sony: It isn’t clear what the benefit is, since the UE would report the actual SRS-RSRP to the gNB and the gNB would know whether the RSRP is above any threshold that the gNB wishes to compare with.  This additional report would consume more uplink resources without additional benefit.

Vivo: Reporting Y least interfering CLI measurement resources will complex the design and the motivation to report least interfering CLI measurement resources needs further discussion. Another issue is about the case the largest measurement is out of range. The validity of such case may need further discussion.

Xiaomi: The motivation to report both least measured report and largest measured result is not clear.

- Issue 2: Whether and how to handle the largest CLI measurement is out of range

Some companies discussed whether and how to handle the largest CLI measurement is out of range.

Companies’ detailed proposals or discussions are listed below.

- Support: CATT, CMCC, Ericsson Google, Nokia, Panasonic, Qualcomm, ZTE

CATT: In cases where the largest CLI measurement is out of range, the following alternatives can be considered for reporting:

- Alt 1: Reporting maximum representable level along with out-of-range flag.

- Alt 2: Reporting Differential value with respect to maximum representable level along with out-of-range flag.

CMCC: For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported,

- When the number of reported CLI measurement resources is larger than one, in case the largest measurement value(s) is(are) out of range, upper bound of the value range of L1 CLI-RSSI or CLI-SRS-RSRP is reported instead of the actual largest measurement value(s), and a differential reporting method is used taking the upper bound of the value range of L1 CLI-RSSI or CLI-SRS-RSRP as the reference.

Ericsson: If the largest CLI measurement is out of range, the UE reports “dummy” differential values for the other measurements

Google: If the largest L1-CLI-RSSI or L1-SRS-RSRP measurement is out of range, use the second-highest measured value as a non-differential reference, with all remaining values reported differentially relative to this second-highest value. An explicit indication can be included to signal that the true maximum measurement is out of range.

Nokia: In case of differential reporting, the UE assumes the highest L1-SRS-RSRP or L1-CLI-RSSI in the reporting range if the most interfered measurement if out-of-the-range.

Panasonic: For L1-based SRS-RSRP considering blocking issue, the reporting value that UE cannot detect SRS should be defined.

When UE cannot detect SRS, how to report the differential value needs to be considered.

Qualcomm: RAN 1 to discuss how to report differential value in case the largest measurement is out of range:

- Option 1: UE calculates the differential values based on the largest CLI value in the range which is -44 dBm for CLI SRS-RSRP reporting.

- Option 2: UE will not report other differential values for other CLI resources in the same report, e.g. other CLI resources reported as dummy value if more than 1 CLI resource is configured for the report.

ZTE: If the reported absolute value is '98', then the largest defined value, i.e., -44 dBm should be used as the reference for the differential reporting.

- Not support: Xiaomi, OPPO

Xiaomi: No enhancement is needed when the largest measurement is out of range.

OPPO: RAN1 specification does not specify new UE behaviors to specifically handle the differential value report in case the largest measured L1-SRS-RSRP is out of range.

Vivo: For L1 CLI measurement and reporting, whether/how to report differential value in case the largest measurement is out of range should discussed after the reported value range is defined.

- It is up to RAN4 to define the reported value range for L1 CLI-RSSI and SRS RSRP reporting.

- Other quantities

Measurement Resource indices via bitmap

- Support: CEWiT, CMCC, Google, NEC, Nokia

CEWiT: Support reporting a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices.

CMCC：At least for periodic reporting and semi-persistent CLI reporting, support two additional report quantities {‘cli-RSSI-measurement-resource-bitmap’, ‘cli-SRS-RSRP-measurement-resource-bitmap’} to reportQuantity

- If reportQuantity is set to ‘cli-RSSI-measurement-resource-bitmap’, a bitmap corresponding to a set of CLI-RSSI measurement resource indices indicating whether L1-CLI-RSSI is above a given threshold

  - The length of the bitmap is same as the number of CLI-RSSI measurement resources within the CLI-RSSI measurement resource set

  - The threshold is configured by the gNB

  - The supported number of CLI-RSSI measurement resources within [X] slots is up to UE capability

- If reportQuantity is set to ‘cli-SRS-RSRP-measurement-resource-bitmap’, a bitmap corresponding to a set of SRS-RSRP measurement resource indices indicating whether L1-SRS-RSRP is above a given threshold

  - The length of the bitmap is same as the number of SRS-RSRP measurement resources within the SRS-RSRP measurement resource set

  - The threshold is configured by the gNB

  - The supported number of SRS-RSRP measurement resources within [X] slots is up to UE capability

Support two additional report quantities {‘cli-RSSI-measurement-resource-bitmap-type2’, ‘cli-SRS-RSRP-measurement-resource-bitmap-type2’} to reportQuantity

- If reportQuantity is set to ‘cli-RSSI-measurement-resource-bitmap-type2’, a bitmap (with size = N - M) corresponding to a set of CLI-RSSI measurement resource indices indicating whether L1-CLI-RSSI is above a given threshold

  - The set of CLI-RSSI measurement resource indices correspond to the (N - M) least interfering CLI measurement resources, where N is the total number of CLI-RSSI measurement resources, M is the numbers of elements in a <CLI-RSSI measurement resource ID, cli-RSSI> list for cli-RSSI report

  - The threshold is configured by the gNB

  - The supported number of CLI-RSSI measurement resources within [X] slots is up to UE capability

- If reportQuantity is set to ‘cli-SRS-RSRP-measurement-resource-bitmap-type2’, a bitmap (with size = N - M) corresponding to a set of SRS-RSRP measurement resource indices indicating whether L1-SRS-RSRP is above a given threshold

  - The set of SRS-RSRP measurement resource indices correspond to the (N - M) least interfering CLI measurement resources, where N is the total number of SRS-RSRP measurement resources, M is the numbers of elements in a <SRS-ResourceId, cli-SRS-RSRP> list for cli-SRS-RSRP report

  - The threshold is configured by the gNB

  - The supported number of SRS-RSRP measurement resources within [X] slots is up to UE capability

Google: Support Periodic reporting and semi-persistent CLI reporting using a configured threshold and a bitmap.

If a multiple ports SRS resource is configured for L1-SRS-RSRP measurement, the UE can measure/report the L1-SRS-RSRP for each SRS port.

NEC: For L1 UE-to-UE CLI measurement and reporting, support the following new report quantity:

-	A bitmap corresponding to a list of SRS-RSRP measurement resource indices with L1-SRS-RSRP above a given threshold (Th1), and/or a bitmap corresponding to a list of SRS-RSRP measurement resource indices with L1-SRS-RSRP below a given threshold (Th2)

o	The measurement resource index lists may be reported via separate bitmap or via joint coding.

o	The two thresholds (Th1 and Th2) can be configured via higher-layer parameters.

Nokia: Support reporting of index/bitmap of resources with strong interference. Consider the following options.

  - Option 1: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating whether L1-SRS-RSRP or L1-CLI-RSSI is above a given threshold

  - Option 2: Report a bitmap corresponding to a set of SRS-RSRP or CLI-RSSI measurement resource indices indicating the K most interfering resource.

Reporting based on a given threshold of CSI/CLI measurement result

CATT: Support threshold-based reporting for L1 based UE-to-UE CLI measurement and reporting.

OPPO: A given threshold of CSI/CLI measurement result can be provided for L1 CLI report to reduce signaling overhead.

Conditional CLI handling behaviour based on monitoring the beams at the victim UE side

InterDigital: Support a conditional CLI handling behaviour based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a subband-wise CLI measurement/reporting for a subband switching to avoid the CLI.

Reporting criteria for CLI measurement is not introduced

LG: Reporting criteria for CLI measurement is not introduced in Rel-19.

      - UCI bits generation

Some companies discussed UCI bits generation. Most companies proposed to support same reporting format of existing L3 UE-to-UE CLI measurement reporting for L1-SRS-RSRP or L1-CLI-RSSI reporting, including the value range, step size, bitwidth. For mapping order, most companies propose the reuse design principle of existing L1 beam reporting, i.e., L1-RSRP or L1-SINR for L1-SRS-RSRP or L1-CLI-RSSI reporting.

- Value range and resolution

- Reuse reporting range and resolution of existing L3 UE-to-UE CLI measurement reporting

  - L1-CLI-RSSI: From -100 dBm to -25 dBm with 1 dB resolution

  - L1-SRS-RSRP: From -140 dBm to -44 dBm with 1 dB resolution

  - Support: Huawei, Hisilicon, CATT, Panasonic, Qualcomm, Samsung, Sony, Xiaomi, ZTE

Ericsson: RAN4 to discuss and decide on range and resolution of CLI measurement reports

Panasonic: For value range/resolution and bit width for CLI reporting, the existing definition (e.g., Rel.16 SRS-RSRP/CLI-RSSI and/or L1-RSRP) can be reused.

Qualcomm: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, support reuse L1-RSRP and R16 SRS-RSRP same range, resolution, and number of bits for L1 CLI SRS-RSRP reporting, and reuse R16 CLI-RSSI same range, resolution and number of bits for L1 CLI-RSSI reporting.

For SRS-RSRP, a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential SRS-RSRP is quantized to a 4-bit value. The differential SRS-RSRP value is computed with 2 dB step size with a reference to the largest or smallest measured SRS-RSRP value.

For CLI RSSI, a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential CLI RSSI is quantized to a 4-bit value. The differential CLI RSSI value is computed with 2 dB step size with a reference to the largest or smallest measured CLI RSSI value.

Samsung: L1-SRS-RSRP and L1-CLI-RSSI are reported with the same value range and resolution as the reported L3 CLI measurements.

Sony: The Rel-16 value range for Rel-16 SRS-RSRP and Rel-16 CLI-RSSI can be directly used for L1-SRS-RSRP and L1-CLI-RSSI, i.e. use 7 bits for each reporting quantity.

Xiaomi: For L1 based UE-to-UE CLI measurement and reporting, 7-bit UCI can be used to report the largest measurement results for SRS-RSRP resource and CLI-RSSI resource respectively.

- For SRS-RSRP, a 7-bit value in the range [-140,-44] dBm with 1 dB step size can be starting point.

- For CLI-RSSI, a 7-bit value in the range [-100, -25] dBm with 1 dB step size can be starting point.

ZTE: The defined reporting range for SRS-RSRP and CLI-RSSI in Rel-16 should be reused.

- Mapping order (if, multiple reported CLI resources are supported for a CSI report)

- Reuse same design principle of mapping order of CSI fields of one report for beam reporting

  - Support: Huawei, Hisilicon, CATT, Qualcomm, ZTE, Spreadtrum, Xiaomi

Apple:

Proposal 3: Mapping order of CLI fields of one report are given in Table 2, where for each row, bit width is given by the corresponding field provided by Table 1.

- It is subject to UE capability to be configured with both, or only one of, SRS-ResourceListConfigCLI and RSSI-ResourceListConfigCLI

| Field | CLI fields |
| --- | --- |
| CLI report #n | SRS-RSRP-RI #1 as in Table 1, if reported |
|  | SRS-RSRP-RI #2 as in Table 1, if reported |
|  | SRS-RSRP as in Table 1, if reported |
|  | Differential SRS-RSRP as in Table 1, if reported |
|  | CLI-RSSI-RI #1 as in Table 1, if reported |
|  | CLI-RSSI-RI #2 as in Table 1, if reported |
|  | CLI-RSSI as in Table 1, if reported |
|  | Differential CLI-RSSI as in Table 1, if reported |

Table 2: Mapping order of CLI report fields of one report for CLI-RSSI and/or SRS-RSRP reporting

CATT: For L1-SRS-RSRP and L1-CLI-RSSI, follow the same design principle of current mapping order of CSI fields of one report for beam reporting.

Table 6: Mapping order of CSI fields of one report for SRS ID/L1-SRS-RSRP or CLI-RSSI ID/L1-CLI-RSSI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SRS ID or CLI-RSSI ID #1 as in Table 7, if reported |
|  | SRS ID or CLI-RSSI ID #2 as in Table 7, if reported |
|  | … |
|  | SRS ID or CLI-RSSI ID #K as in Table 7, if reported |
|  | L1-SRS-RSRP or L1-CLI-RSSI #1 as in Table 7, if reported |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #2 as in Table 7, if reported |
|  | … |
|  | Differential L1-SRS-RSRP or L1-CLI-RSSI #K as in Table 7, if reported |

Table 7: CRI, SSBRI, and RSRP

| Field | Bitwidth |
| --- | --- |
| SRS ID | log2(KsSRS) |
| CLI-RSSI ID | log2(KsCLI-RSSI) |
| L1-SRS-RSRP | 7 |
| L1-CLI-RSSI | 7 |
| Differential RSRP | 4 |
| Differential RSSI | 4 |

Qualcomm: For L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, RAN1 to reuse report format as L1 beam reporting.

- FFS: a new local resource index for CLI measurement resource in CLI reporting

- FFS: a new payload format table for CLI reporting similar as L1 beam reporting table.

Samsung: The mapping order of multiple reported L1-SRS-RSRP measurements reuses the mapping order of the CSI fields in one report for L1 beam reporting.

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported.

- the N ≤ 4 most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP are reported in decreasing order.

Spreadtrum: The mapping order and bitwidth of CSI fields for cli-SRS-RSRP and cli-RSSI are provided in Table 2, Table 3, Table 5 and Table 6.

Table 2. Mapping order of CSI fields of one report for cli-SRS-RSRP

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SRI #1 as in Table 5, if reported |
|  | SRI #2 as in Table 5, if reported |
|  | SRI #3 as in Table 5, if reported |
|  | SRI #4 as in Table 5, if reported |
|  | RSRP #1 as in Table 5, if reported |
|  | Differential RSRP #2 as in Table 5, if reported |
|  | Differential RSRP #3 as in Table 5, if reported |
|  | Differential RSRP #4 as in Table 5, if reported |

Table 3. Mapping order of CSI fields of one report for cli-RSSI

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | CLIRI #1 as in Table 6, if reported |
|  | CLIRI #2 as in Table 6, if reported |
|  | CLIRI #3 as in Table 6, if reported |
|  | CLIRI #4 as in Table 6, if reported |
|  | RSSI #1 as in Table 6, if reported |
|  | Differential RSSI #2 as in Table 6, if reported |
|  | Differential RSSI #3 as in Table 6, if reported |
|  | Differential RSSI #4 as in Table 6, if reported |

Table 5. Bitwidth of CSI fields for cli-SRS-RSRP

| Field | Bitwidth |
| --- | --- |
| SRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

Table 6. Bitwidth of CSI fields for cli-RSSI

| Field | Bitwidth |
| --- | --- |
| CLIRI |  |
| RSSI | 7 |
| Differential RSSI | 4 |
| CapabilityIndex | 2 |

Xiaomi: For CSI report associated with SRS-RSRP resource and CLI-RSSI resource, Table 6.3.1.1.2-8 defined in TS38.212 with modification shown below can be considered as starting point.

- CRI and SSBRI is replaced with SRS-RSRP resource index or CLI-RSSI resource index.

- Capability index is crossed out in the table.

- RSSI is added in the table if the CSI report is associated with CLI-SRS resource.

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SRS-RSRP resource index or CLI-RSSI resource index #1, if reported |
|  | SRS-RSRP resource index or CLI-RSSI resource index #2, if reported |
|  | SRS-RSRP resource index or CLI-RSSI resource index #3, if reported |
|  | SRS-RSRP resource index or CLI-RSSI resource index #4, if reported |
|  | RSRP or RSSI #1, if reported |
|  | Differential RSRP or RSSI #2, if reported |
|  | Differential RSRP or RSSI #3, if reported |
|  | Differential RSRP or RSSI #4, if reported |

- Bit width

- 7 bits for largest or smallest value and 4 bits for differential reporting

  - Support: Apple, Huawei, CATT, LG, Xiaomi

Apple: The bitwidth for CRI, SRS-RSRP, CLI-RSSI, differential RSRP, and differential RSSI are provided in the following Table 1, where KsSRS-RSRP is the number of SRS-ResourceConfigCLI resources in the corresponding CLI-RS-ResourceList, and KsCLI-RSSI is the number of RSSI-ResourceConfigCLI resources in the corresponding CLI-RS-ResourceList

| Field | Bit-width |
| --- | --- |
| SRS-RSRP-RI | log2KsSRS-RSRP |
| CLI-RSSI-RI | log2KsCLI-RSSI |
| SRS-RSRP | 7 |
| Differential RSRP | 4 |
| CLI-RSSI | 7 |
| Differential RSSI | 4 |

Table 1: Bit-width for CLI report fields

LG: Bitwidth of CLI reporting is identical to the beam reporting case.

- The Bitwidth of the resource index(es) for SRS-RSRP and CLI-RSSI are log2KsSRS-RSRP and log2KsCLI-RSSI, respectively, where KsSRS-RSRP and KsCLI-RSSI are the number of SRS-RSRP resources and CLI-RSSI resources in the corresponding resource set, respectively.

Xiaomi: In L1 based UE-to-UE CLI measurement and reporting,

- For differential L1-SRS-RSRP, a 4-bit value with 2 dB step size can be starting point.

- For differential L1-CLI-RSSI, send a LS to RAN4 to ask more information.

      - Reference resource for L1 UE-to-UE CLI measurement

- Issue 2: Reference resource for L1 UE-to-UE CLI measurement

In existing CSI framework, UE reports a CSI report only after receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement no later than CSI reference resource and drops the report otherwise. And in the time domain, CSI reference resource for a CSI reporting is defined by a downlink slot offset with downlink slot n which is the corresponding downlink slot of CSI report in uplink slot n’. CSI reference resource is defined in downlink slot because it is used as the reference for CSI-RS which is a downlink signal. Similar as CSI reference resource, a reference resource should be defined to decide whether CLI measurement resource is valid for a CSI report named ‘CLI reference resource’.

Several companies discussed this issue. Views are summarized below.

LG: The CSI reference resource for CLI reporting is determined by NZP CSI-RS resources configured within the triggered report configuration.

- The existing CSI reference resource definition in section 5.2.2.5 of TS 38.214 is reused.

Nokia: For periodic/semi-persistent L1-SRS-RSRP measurement reporting, the CPU occupancy starts from a fixed symbol offset prior to L1-SRS-RSRP measurement resource. FFS: a fixed symbol offset.

Spreadtrum

The timing for SRS-RSRP measurement resource should be discussed first.

Enhancement of CLI reference resource is needed if the timing for SRS-RSRP measurement resource is not downlink timing.

The valid CLI reference resource slot should comprises at least one higher layer configured SBFD or flexible symbol and not fall within a configured measurement gap.

      - CPU occupation and CSI computation time

- Issue 1: CPU occupation

Several companies discussed CPU occupation.

- Issue 1-1: CSI processing unit

Companies’ views are summarized as below.

- Reuse existing CSI processing unit

  - Support: ZTE, Spreadtrum, Samsung

- Issue 1-2: Number of occupied CPU(s)

Companies’ views are summarized as below.

- Reuse the number of occupied CPU(s) for existing beam-reporting

  - Support: Huawei, Hisilicon, Qualcomm, Samsung, ZTE, Spreadtrum, LG, Vivo

- Other values

  - Support: Huawei, Hisilicon

Companies’ detailed proposals or discussions are listed below.

Qualcomm: support single CPU occupation for CLI reporting.

ZTE: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting should be reused for aperiodic L1 UE-to-UE CLI measurement and reporting.

The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting should be reused for periodic and semi-persistent L1 UE-to-UE CLI measurement and reporting.

Spreadtrum: The enhancement on CSI processing unit, CPU occupation rule are unnecessary.

LG: CPU occupancy is 1 and the occupied time for beam reporting is reused.

Samsung: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting is be reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

- Issue 2: CSI computation time

- Issue 2-1: Duration time of occupied CPU of a CLI reporting

In current specification, for a CSI report, the CPU(s) are occupied for a number of OFDM symbols. For a periodic or SP CSI report (excluding an initial SP CSI report on PUSCH after the PDCCH triggering the report), CPU(s) are occupied from the first symbol of the earliest one of each CSI-RS/CSI-IM/SSB resource until the last symbol of the configured PUSCH/PUCCH carrying the report. For an aperiodic CSI report or an initial SP CSI report on PUSCH triggered by PDCCH, CPU(s) are occupied from the first symbol after triggering PDCCH until the last symbol of the scheduled PUSCH carrying the report.

According to discussion in Issue 2-1, for L1 based UE-to-UE CLI measurement and reporting, a constant offset relative to the DL reference timing in the serving cell or UL reference timing is applied. This may have some impact on the starting symbol of CPU occupation.

Companies’ views are summarized as below.

- Reuse same design principle of duration time of occupied CPU of an existing CSI report

  - Support: ZTE, Samsung

- Enhancement based on existing CSI computation delay requirement for beam-reporting class (Z3)

  - Support: Vivo, Nokia, Huawei

Companies’ detailed proposals or discussions are listed below.

ZTE: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting should be reused for aperiodic L1 UE-to-UE CLI measurement and reporting.

Huawei: For a periodic or SP CLI report (excluding an initial SP CLI report on PUSCH after the PDCCH triggering the report), CPU(s) are occupied from one symbol before the symbol of the earliest one of each SRS-RSRP measurement resource until the last symbol of the configured PUSCH/PUCCH carrying the report.

For an aperiodic CSI report or an initial SP CSI report on PUSCH triggered by PDCCH, CPU(s) are occupied from the first symbol after triggering PDCCH until the last symbol of the scheduled PUSCH carrying the report. For these cases, the starting symbol of CPU occupation can be reused.

Nokia: For periodic/semi-persistent L1-SRS-RSRP measurement reporting, the CPU occupancy starts from a fixed symbol offset prior to L1-SRS-RSRP measurement resource. FFS: a fixed symbol offset.

Vivo: For L1 based UE-to-UE CLI measurement and reporting, the number of CPU occupation and timeline can reuse that of existing L1-RSRP report.

For L1-CLI-RSSI measurement within DL subband and aperiodic L1-CLI-RSRP reporting, the starting time of the CPU occupation is same as that of existing CSI measurement and reporting.

For periodic and semi-persistent reporting (if supported) of L1-CLI-RSRP measurement in UL subband, the starting time of the CPU occupation is the X symbols before the first symbol of the CLI-RS for interference measurement. X=1 for SCS=15kHz and X=2 for SCS=30kHz can be considered.

- Issue 2-2: Computation delay of AP CLI reporting

For aperiodic CSI report, the current specification also defines some timing requirement to guarantee a UE can have enough time to generate the CSI report as shown in Figure 4. For the value of Z and Z’, current specification defines three classes: low latency class (Z1), high latency class (Z2), and latency for beam-reporting (Z3).

Figure 4.  Aperiodic CSI processing timeline

Companies’ views are summarized as below.

- Reuse existing CSI computation delay requirement for beam-reporting class (Z3)

  - Support: ZTE, Samsung, CATT, Huawei

- Enhancement based on existing CSI computation delay requirement for beam-reporting class (Z3)

  - Support:CATT, Qualcomm

Companies’ detailed proposals or discussions are listed below.

Samsung: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting is be reused for the aperiodic CLI measurement and reporting in Rel-19 SBFD.

ZTE: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting should be reused for aperiodic L1 UE-to-UE CLI measurement and reporting.

HW: For aperiodic CLI report, reuse the existing CSI computation delay requirement for beam-reporting class (Z3).

CATT: L1-CLI computation delay requirement depends on whether to support configuration of ‘typeD’ QCL assumption for measurement resource.

- Considering the requirement of L1-RSRP relates to beam switching capability, if L1-CLI measurement resource is configured with ‘typeD’ QCL assumption, the requirement of L1-RSRP can be considered; otherwise, the requirement of L1-SINR can be used.

Qualcomm:

- RAN1 to discuss the AP CLI timeline with reusing beam reporting class as a baseline and the gap time between triggering DCI and the AP CLI measurement resource.

- RAN1 to clarify the CLI measurement resource time e.g. as uplink timing to determine the exact AP CLI timeline and the gap time between triggering DCI and the AP CLI measurement resource.

- Support the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured shall be more than or equal to 1 slot to compensate for the timing switching.

      - Others

Active resources

LG: For the CLI measurement and reporting, UE always measures the latest resource to derive a report.

Time restriction for measurement

LG: In order to satisfy the intention for which L1 based CLI measurement and report was introduced, UE always measures the latest resource to derive a report.

Collision between L1-CLI measurement and DL reception

CATT: For collision between L1-CLI measurement and DL reception, consider the handling scheme used for L3 CLI measurement as starting point.

Google: Study the behaviour of the victim UE when the SRS transmission is dropped by the aggressor UE, e.g. due to collision with other UL signals.

If a vicitm UE is to receive a DL signal and is also expecting an SRS transmission from  agressor UE for UE-to-UE CLI measurements based on different QCL -TypeD, it cannot receive both the SRS and the DL signal simultaneously. Some solutions could be specified:

- Dropping SRS

- Dropping DL signal

- Follow priority rules

Timing misalignment issue

ERTI: To ensure accurate RSRP and channel measurement using CLI-RS, it is necessary to consider adjusting the aggressor UE’s transmission timing of CLI-RS and/or victim UE’s reception timing of CLI-RS.

It is necessary to investigate specification impacts for timing alignment issues for UE-to-UE CLI measurement and reporting (e.g., timing synchronization between victim UE and aggressor UE for CLI-RSRP measurement, victim UE’s two different reception timings for CLI-RS and DL)

    - 1st round proposals (Open)

- Proposal 2-1

Conclusion

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’

- carrier in CSI-ReportConfig is used to indicate the serving cell where the CLI-RSSI measurement resources or SRS-RSRP measurement resource are located in

- resourcesForChannelMeasurement provides the ID of a CSI-ResourceConfig that contains configuration of CLI-RSSI measurement resources or SRS-RSRP measurement resources (agreed in RAN1#118bis)

- bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in

  - UE assumes the subcarrier spacing for the CLI-RSSI measurement resources or SRS-RSRP measurement resources is same as the subcarrier spacing for the DL BWP

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Spreadtrum |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | It is unclear to us why this is needed. Isn’t it so that the UE always does the measurement on the serving cell and active BWP? |
| ZTE | For the sub-bullet, we think the SCS should be the same as that for SRS transmitted by the aggressor UE instead of DL BWP. Therefore, it should be configured. For CLI-RSSI, the SCS can also be different from the DL BWP, which is supported in Rel-16. It should be configured by the network. In addition, does this proposal means that there is no other parameters introduced any more? |
| Samsung | Same questions/concerns as Ericsson. |
| Spreadtrum | Whether the frequency location of CLI measurement resource is based on reference serving cell, BWP and SCS defined in CLI measurement resource as Rel-16 or on the serving cell and BWP of CSI resource/CSI report should be discussed. We are OK to reuse the carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig. |
| QC | We want to clarify that carrier parameter can be reused to indicate in which serving cell the CSI-ResourceConfig indicated are to be found. If the field is absent, the resources are on the same serving cell as this report configuration. RAN1 also shall discuss the frequency reference point to find the first RB for CLI SRS-RSRP and CLI-RSSI measurement based on PRB where this CLI resource starts in relation to common resource block #0 (CRB#0) on the common resource block grid. Similar rule as legacy CSI-RS. For the last sub-bullet, we don’t agree. What happened for SRS-RSRP measurement if subcarrier spacing of UL BWP of aggressor UE is different than subcarrier spacing of DL BWP of victim UE? In R16, this is avoided by explicitly configuring subcarrier spacing per CLI resource, and it does not need to match subcarrier spacing of UE DL BWP. |
| DOCOMO | Sams view as Ericsson that UE always performs the L1 CLI measurement on the active BWP of the serving cell. |
| LG | Same view with Ericsson. Since it is using CSI framework therefore the resource and report configuration for CLI is just a special case of CSI resource and report configuration, we do not see the necessity of this. |

- Proposal 2-2

For frequency resource allocation of a CLI-RSSI measurement resource, support measurement resource type#3 in addition to measurement resource type#1 and type#2 agreed in RAN1#118.

- Measurement resource type#3: One CLI-RSSI measurement resource is configured within a UL subband

- UE does not expect to be configured with Measurement resource type#1 outside the DL usable PRBs.

- UE does not expect to be configured with Measurement resource type#3 outside the UL usable PRBs.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi (FFS the last two sub-bullets), IDC, Sony, QC, NEC |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | It is unclear to us why it is needed to define a 3rd measurement resource type, especially since Method #3 operates on a set of contiguous RBs. Isn’t the only valuable thing here that the CLI-RSSI measurement resources do not step outside the usable PRBs? |
| Xiaomi | In-line with the following agreement in RAN1#118, to support Method#3: UE measures RSSI within UL subband, measurement resource type #3 should be supported. However, more discussion is needed for the last two sub-bullets since it seems that it is assumed that measurement resource type is explicitly configured to UE, which is not clear whether necessary to us. Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement |
| IDC | Because of the agreements made for “Method #3 (UE measures RSSI within UL subband)” and “CLI-RSSI resource configuration” including the starting PRB and number of PRB to perform the measurement on, current parameters may be ambiguous for the cases where the CLI-RSSI resources are configured over the UL subbands. Therefore, a clarification is needed, and in our view the proposal seems a simple way and clean, in that the UE is indicated on whether to perform a measurement of CLI-RSSI over the UL subband (Method #3, type#3 as in the proposal) or a CLI-RSSI measurement over DL subband(s) and excluding the non-usable DL PRBs (Method #1, type#2). There is another implicit way for the clarification such that: If a CLI-RSSI resource is exclusively configured within UL usable PRB, the UE considers the CLI-RSSI resource to be for Method #3: UE measures RSSI within UL subband; otherwise, the UE considers that the measurement is for Method #1 and excludes any non-usable DL PRBs from the measurement/reporting if any. However, we think an explicit indication as in the proposal seems simpler and sufficient. |
| ZTE | In our understanding, the resource is configured by the network. Is the intention to limit the network configuration? |
| Samsung | We are not convinced that it is necessary to introduce a measurement resource type #3. It can simply be stated that the UE expects the CLI-RSSI resource within the UL usable PRBs. |
| QC | May not need to introduce type #3, but ok to leave with it. |
| DOCOMO | We are fine with the principle of the first bullet that CLI measurement resource within UL subband is applied for Method#3. But we think maybe the configuration details should be open considering overlapping with SBFD subband boundary. The intention to put the last two bullets here is not clear to us. |
| Nokia | To my understanding, type #3 is for method #3. It is agreeable, but it is unclear, if the following would limit the number of measurement resource in as UL subband by one. - Measurement resource type#3: One CLI-RSSI measurement resource is configured within a UL subband Frankly speaking, it is not very necessary to be agreed. |
| LG | We also think only resource is configured within usable PRBs is sufficient. |

- Proposal 2-3

For L1-CLI-RSSI measurement and reporting, a UE reports the maximum number of supported CLI-RSSI measurement resources of all three measurement types across all CCs (sum of measurement resource type#1, measurement resource type#2 and measurement resource type#3)

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | IDC, QC |
| Not support | Ericsson, Samsung, LG |

| Companies | Views |
| --- | --- |
| Ericsson | Doesn’t this belong in UE capability session? |
| Xiaomi | Agree with Ericsson |
| IDC | Support, but also OK to be discussed in UE capability session. |
| Sony | Share similar views with Ericsson. |
| ZTE | We think it should be discussed in the UE feature. |
| Samsung | Same concern as Ericsson |
| Spreadtrum | We also OK to be discussed in UE capability session. |
| CMCC | Support to discuss this in UE feature session |
| DOCOMO | Agree with Ericsson. |
| Nokia | Let’s discuss in UE capability. |
| LG | Same view with Ericsson |

- Proposal 2-4

For Method #3 (UE measures RSSI within UL subband), reuse the timing for Rel-16 L3 based CLI measurement, i.e., a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | CMCC |
| Not support | Ericsson, Samsung, QC, Nokia, LG |

| Companies | Views |
| --- | --- |
| Ericsson | We disagree with this proposal – not the content of the proposal, but the working procedure. This issue is being discussed actively in RAN4, and RAN1 should not be making separate agreements on the same thing. |
| ZTE | We think this issue can be discussed in RAN4 considering that RAN4 has defined the timing for method #1. |
| vivo | We think DL timing can be used for method# 3 which is same as measuring RSSI in DL subband. |
| Samsung | Same view as Ericsson |
| CMCC | Also fine to wait for RAN4’s conclusion |
| QC | To our view, DL timing shall be applied to method #3 for RSSI measurement in UL subband. With this assumption, it would be beneficial for UE supporting simultaneous operations between DL reception in the DL sub-band(s) and Method#3 CLI measurement reception in the UL sub-band with the same timing assumption. |
| Nokia | We think the main benefit of method #3 over method #2 is using the same DL timing. For RSSI measurement, timing offset doesn’t impact too much in the RSSI accuracy. We prefer using DL timing. |
| LG | Using DL reception timing for every CLI measurement seems good approach to prevent complicated UE behaviour e.g., partial overlap of CLI measurement and DL reception unless there is critical reason to do so. |

- Proposal 2-5

For L1 based UE-to-UE CLI measurement and reporting, periodic CLI reporting and semi-persistent CLI reporting are not supported.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Ericsson, Sony,vivo, Samsung, Panasonic, DOCOMO |
| Not support | Xiaomi, IDC, ZTE, Spreadtrum, CMCC, NEC, Nokia, LG |

| Companies | Views |
| --- | --- |
| Ericsson | We think aperiodic reporting is sufficient, due to the nature of CLI. Single snapshots are more useful. |
| Xiaomi | We think periodic and semi-persistent CLI reporting is useful and should be supported. |
| IDC | Share same views as Xiaomi |
| ZTE | Don’t Support. Periodic and semi-persistent reporting is important and helpful for the gNB to obtain the interference situation. We don’t see any issue to support these two reporting types. |
| Spreadtrum | Share same views as Xiaomi and ZTE. |
| CMCC | We share similar view with Xiaomi and ZTE. From our perspective, periodic or semi-persistent reporting has some advantages. For example, periodic reporting can be performed at the beginning to help the NW timely acquire the characteristics of UE-to-UE CLI, which will further facilitate the NW configuration/triggering of aperiodic reporting of UE-to-UE CLI. |
| Nokia | We think periodic is important for L1-CLI-RSSI measurement. We are fine to preclude periodic L1-SRS-RSRP reporting. Semi-persistent is useful to minimize the overhead, but we can compromise that periodic L1-CLI-RSSI measurement and reporting is supported. |
| LG | Using DL reception timing for every CLI measurement seems good approach to prevent complicated UE behaviour e.g., partial overlap of CLI measurement and DL reception unless there is critical reason to do so. |

- Proposal 2-6

Working assumption

The range and resolution of L3 based CLI measurement report is reused for L1 based CLI measurement

- For SRS-RSRP, a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential SRS-RSRP is quantized to a 4-bit value. The differential SRS-RSRP value is computed with 2 dB step size with a reference to the largest or smallest measured SRS-RSRP value.

- For CLI RSSI, a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential CLI RSSI is quantized to a 4-bit value. The differential CLI RSSI value is computed with 2 dB step size with a reference to the largest or smallest measured CLI RSSI value.

Send an LS to RAN4 about the above working assumption.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi, Sony, ZTE, Samsung, Spreadtrum, Panasonic, QC, DOCOMO, Nokia, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| ZTE | We are fine with this proposal. We are also fine to leave this issue to RAN4. |
| Nokia | We support the proposal. For method #1, we think the interference level may be very low, we are OK that RAN4 can further check if any update is needed. |

- Proposal 2-7

If the number of reported L1-SRS-RSRP measurement resources is M and the number of L1-SRS-RSRP measurements which are out of range is X (X<=M), the following are reported within one report

- An “out of range” indicator

- A list of invalid indication is reported for each of the X-1 L1-SRS-RSRP measurement

- For the M-X L1-SRS-RSRP measurements which are within the reporting range, a differential reporting method is adopted using the highest L1-SRS-RSRP within the reporting range as the reference

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Panasonic, QC |
| Not support | Xiaomi, CMCC, Nokia, LG |

| Companies | Views |
| --- | --- |
| Ericsson | We have concerns on this proposal since it seems like it results in a variable feedback overhead, i.e., the list is sent only if there are out of range values. Then we have a general question about how important it is to send such detailed reporting. It surely does not happen very often that there is an out-of-range value, and if there is, it seems sufficient just to indicate an out-of-range value for all M resources. The UE will still order the report with largest one first, to then the gNB will know which resource has the largest CLI and can try to do something about it. Does it really matter what the precise value is for the other resources? By analogy, we note that there is no special handling for this case in legacy L1-RSRP and L1-SINR measurement reporting. |
| Xiaomi | We do not think special handling is needed here. |
| ZTE | This leads to addition reporting overhead. And no valid information is delivered to the network. We should focus on the solution without additional signaling overhead. We also think this issue can be handled by UE implementation. |
| vivo | We do not think special handling is needed. |
| Samsung | We think the corresponding specification behavior can follow that of previous releases when the reported value is OOR. |
| CMCC | We think a simpler method can be considered instead. In detail, a differential reporting method can also be used in case the largest measurement is out of range. When the largest measurement value(s) is(are) out of range, the upper bound of the value range of L1 CLI-RSSI or CLI-SRS-RSRP can be reported instead of the actual largest measurement value(s), and a differential reporting method can be used for the other measurement values taking the upper bound of the value range of L1 CLI-RSSI or CLI-SRS-RSRP as the reference. |
| QC | Support in general. For the last bullet, suggest to add if most interfering CLI resource is the reporting criteria. |
| Nokia | Maximum value (e.g. -44dBm) and 0 dB differential value can be reported. |
| LG | We don’t think this is critical problem to be solved. |

- Proposal 2-8

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, new reporting criteria considering the interfered measured L1-CLI-RSSI or L1-SRS-RSRP are not supported.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Ericsson, Xiaomi, Sony, vivo, Samsung, Panasonic, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | We support Proposal 2-8 We think that with the current agreement, it is sufficient to support reporting only the M largest (most interfered) CLI measurements. In the last meeting it was discussed to configure reporting of M the smallest (least interfered) measurements, but we think this is not particularly useful, as some of these measurements may be “in the noise” and thus not have particular value to the gNB. |
| ZTE | We think least interfered results should be supported as it can provide additional information to the network so that the network can perform CLI handling better. |
| QC | In our view, least interfering CLI resources in the reportConfig shall also be supported for CLI reporting. In this case, the aggressor UEs triggered for measurements by gNB shall be the UEs that gNB intends to schedule and pair with the victim UE, so that gNB can schedule one of the reported least interfering UE to pair with victim UE based on the victim UE’s CLI reporting. gNB can configure the reporting criteria as either most interfering or least interfering CLI resources to report |
| DOCOMO | Not sure whether “least” is missed from “considering the least interfered”? |

- Proposal 2-9

Support two new report quantities {‘cli-RSSI-measurement-resource-bitmap’, ‘cli-SRS-RSRP-measurement-resource-bitmap’} to reportQuantity

- If reportQuantity is set to ‘cli-RSSI-measurement-resource-bitmap’, a bitmap corresponding to a set of CLI-RSSI measurement resource indices indicating whether L1-CLI-RSSI is above a given threshold

  - The length of the bitmap is same as the number of CLI-RSSI measurement resources within the CLI-RSSI measurement resource set

  - The threshold is configured by higher-layer parameters

- If reportQuantity is set to ‘cli-SRS-RSRP-measurement-resource-bitmap’, a bitmap corresponding to a set of SRS-RSRP measurement resource indices indicating whether L1-SRS-RSRP is above a given threshold

  - The length of the bitmap is same as the number of SRS-RSRP measurement resources within the SRS-RSRP measurement resource set

  - The threshold is configured by higher-layer parameters

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | ZTE, CMCC, Nokia |
| Not support | Xiaomi, Sony, Samsung, LG |

| Companies | Views |
| --- | --- |
| Xiaomi | With the reporting quantities which have already been agreed, it is not clear to us whether the additional reporting quantities are still needed. |
| Sony | This contradicts with Proposal 2-8, which proposes no new reporting criteria is supported. |
| ZTE | Support. This gives a full picture of the CLI measurement results, especially when the number of measurement resources are huge, e.g, in FR2. The benefit is clear. |
| CMCC | There are two motivations for bitmap-based CLI report. - Motivation 1: Provide more most interfered (largest) CLI resources information. For example, as shown in figure below, among N (=8) cli-SRS-RSRP measurement resources, a <SRS-ResourceId, cli-SRS-RSRP>-list of the M (=4) most interfering CLI measurement resources (e.g., corresponding to SRI#1, SRI#3, SRI#7, SRI#2) is configured to be reported. Nevertheless, CLI related to SRI#6 is also strong enough which asks for coordinated scheduling, but it is omitted in the report. The reported UE may suffer from significant DL throughput degradation if the neighbor UE related to SRI#6 transmits a PUSCH simultaneously. - Motivation 2: Provide least interfered CLI resources information in a reduced signalling overhead way. For example, as shown in figure below, only reports of the resources index for the most interfering CLI measurement resources (e.g., SRI#1, SRI#2, SRI#3, SRI#6, SRI#7) with CLI above a big threshold (e.g., Th1) and the least interfering CLI measurement resources (e.g., SRI#4 and SRI#5) with CLI below a small threshold (e.g., Th2) are sufficient for gNB scheduling, wherein, the former information restricts gNB to schedule UE pair with strong CLI and the latter information encourages gNB to schedule UE pair with negligible CLI. |
| Nokia | The proposed method is useful for gNB scheduling. We don’t see the use case of the proposal for SRS-RSRP case. But we can live with. |

- Proposal 2-10

For L1 based SRS-RSRP and CLI-RSSI reporting, reuse same design principle of mapping order of CSI fields of one report for beam reporting.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi, Samsung, Panasonic, DOCOMO, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | We think more detail is needed for this proposal with a first table showing a table with the mapping order and a second table showing the # of bits for the quantities in the first table. Several companies had such tables in their contributions that could be used. |
| Xiaomi | We agree with the principle and the proposal from Ericsson to provide more details is also preferred. |
| ZTE | It is applied to the most inferred measurement results reporting. We suggest the following. For L1 based SRS-RSRP and CLI-RSSI reporting with the most interfered results, reuse same design principle of mapping order of CSI fields of one report for beam reporting. |
| vivo | Agree with the principle |
| Spreadtrum | More details is needed. |

- Proposal 2-11

For L1 CLI measurement, reuse the number of occupied CPU(s) for L1 beam reporting.

- OCPU=1 for a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’, ‘cli-SRS-RSRP’.

Companies are invited to provide views on the above proposal.

|  | Companies |
| --- | --- |
| Support | Xiaomi, Sony, ZTE, vivo, Samsung, Spreadtrum, QC, DOCOMO, Nokia, LG |
| Not support |  |

| Companies | Views |
| --- | --- |
| Ericsson | By L1 beam reporting, we presume this means “L1-RSRP reporting”? If so, in principle we are okay with this; but we think that Proposal 2-5 should be decided first. |
| ZTE | The CLI measurement is the same with or even simpler than BM measurement. |

    - Others

Timing advance techniques

Apple: To assure symbol level alignment at victim UEV, aggressor UEA is indicated to hold two different TAs:

- one TA for symbols on which TRP is doing legacy TDD, another TA for symbols on which TRP is doing SBFD or dynamic TDD.

Configure a second candidate UL beam direction at the aggressor UE

InterDigital: Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

BFR enhancements

NEC:

- Differentiation of the BFR caused by CLI with the beam blockage is needed.

- Eliminating the effect of the CLI on BFR for BFD and NBI should be considered.

- Contact person

Please provide/update the information of the contact person in the following table to facilitate the discussions.

| Company | Name | Email address |
| --- | --- | --- |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| vivo | Na Li | lina5g@vivo.com |
| Sony | Shin Horng Wong | shinhorng.wong@sony.com |
| Ericsson | Narendar Madhavan Stephen Grant | narendar.madhavan@ericsson.com stephen.grant@ericsson.com |
| Xiaomi | Yanping Xing Lei Wang | xingyanping@xiaomi.com wanglei25@xiaomi.com |
| ETRI | Junhyeong Kim | jhkim41jf@etri.re.kr |
| CEWiT | Sonil Baberwal | sonilbaberwal@cewit.org.in |
| Panasonic | Tomohiro Inoue | inoue.tomohiro004@jp.panasonic.com |
| CATT | Li Shupeng | lsp@catt.cn |
| NEC | Frank Zhang Pravjyot Deogun | Zhang_bohang@nec.cn Pravjyot.Deogun@EMEA.NEC.COM |
| LG | Hyunsoo Ko Jaenam Shim | hyunsoo.ko@lge.com jaenam.shim@lge.com |
| Google | Abdellatif Salah Kao-Peng Chou | asalah@google.com nevillechou@google.com |
| Qualcomm | Emily Zhang | qiaz@qti.qualcomm.com |
| Nokia/NSB | Youngsoo Yuk Nhat-Quang Nhan | youngsoo.yuk@nokia.com nhat-quang.nhan@nokia.com |
| NTT DOCOMO | Hiroki Harada Qiping Pi | hiroki.harada.sv@nttdocomo.com piqp@docomolabs-beijing.com.cn |
| ZTE | Shuaihua Kou Xianghui Han | kou.shuaihua@zte.com.con han.xianghui@zte.com.cn |
| Sharp | Majid Ghanbarinejad | ghanbarinejadm@sharplabs.com |
| Tejas Networks | Virendra Singh Rajput Shrinivas Bhat | virendrar@tejasnetworks.com shrinivasb@tejasnetworks.com |
| Samsung | Marian Rudolf | m.rudolf@samsung.com |
| Spreadtrum | Shuai Zhang | shuai.zhang6@unisoc.com |
| Fraunhofer HHI | Priyanka Dey | priyanka.dey@hhi.fraunhofer.de |

# References

R1-2500095	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2500146	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2500168	Discussion on CLI handling	Spreadtrum, UNISOC

R1-2500223	Discussion on CLI handling for NR duplex evolution	CATT

R1-2500251	Discussion on CLI handling	LG Electronics

R1-2500285	Discussion on CLI handling	CMCC

R1-2500316	Discussion on CLI Handling in SBFD system	MediaTek Inc.

R1-2500348	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2500403	Discussion on gNB-gNB CLI handling	Tejas Networks Limited

R1-2500456	CLI handling in NR duplex operation	OPPO

R1-2500522	Discussion on CLI handling for SBFD operation	InterDigital, Inc.

R1-2500588	CLI handling for NR duplex operations	NEC

R1-2500650	SBFD CLI handling	Sony

R1-2500707	Discussion on CLI handling	Panasonic

R1-2500731	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2500751	CLI handling	Ericsson

R1-2500777	Discussion on CLI handling	Apple

R1-2500849	Cross-link interference handling for SBFD	Samsung

R1-2500882	Discussion on CLI handling	CEWiT

R1-2500910	Discussion on CLI handling for SBFD operation	ETRI

R1-2501005	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2501061	Cross-link interference (CLI) handling	Sharp

R1-2501155	Enhancements for CLI handling	Qualcomm Incorporated

R1-2501201	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2501292	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

# Appendix: Agreements

- RAN1#116

Conclusion

For the down-selection of gNB-to-gNB co-channel CLI handling schemes and UE-to-UE co-channel CLI handling schemes, at least the following aspects should be considered:

- Applicable scenario, performance benefits based on analysis and/or demonstrated by evaluations for SBFD

  - Note: Companies are encouraged to provide more simulation results for SBFD to RAN1#116bis based on the simulation assumptions agreed during the SI.

- Specification impact in RAN1 and RAN3.

- gNB/UE implementation complexity.

- Operational details of the scheme including feasibility and practicability.

Agreement

Consider the following candidate gNB-to-gNB co-channel CLI handling schemes for further down-selection

- gNB-to-gNB co-channel CLI and/or channel measurements

- Spatial domain based schemes

  - Beam nulling

  - Beam pairing

- Coordinated scheduling in time and/or frequency

- Power control based schemes

  - gNB Tx power control

  - UE Tx power control

Note: gNB-to-gNB co-channel CLI and/or channel measurements can be the enablers for some of the above CLI handling schemes.

Agreement

Consider the following candidate UE-to-UE co-channel CLI handling schemes for further down-selection

- UE-to-UE co-channel CLI measurement and reporting

- Coordinated scheduling in time and/or frequency

- Spatial domain based schemes

- Power control based schemes

- Note: UE-to-UE co-channel CLI measurement and reporting can be the enablers for some of the above CLI handling schemes.

Agreement

gNB Tx power control based schemes are not considered in the down-selection of gNB-to-gNB co-channel CLI handling schemes.

Agreement

For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Method#3: UE measures RSSI within UL subband

- Method#4: UE measures RSSI within guard band, if guard band exists

Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply.

Note: Method#4 does not imply that guard band is explicitly configured.

For future meetings:

Companies are to refer to Proposal 2-2a (gNB-gNB CLI handling) and Proposal 3-2a (UE to UE CLI handling) in R1-2401635 for future meetings. Companies are encouraged to provide additional details on potential spec impact and operational details of their preferred CLI handling scheme for further down-selection in RAN1#116bis.

- RAN1#116bis

For future RAN1 meetings:

For the down-selection of gNB-to-gNB CLI handling scheme(s) and UE-to-UE CLI handling scheme(s), companies are encouraged to check whether the candidate co-channel CLI handling scheme can be applicable for inter-operator and/or intra-operator adjacent channel CLI handling.

- Note: Whether flexible symbol(s)/slot(s) with SBFD subband configurations can be convert into DL/UL symbols by TDD-UL-DL-ConfigDedicated is discussed under AI 9.3.1.

- Note: Whether UE-specific SBFD subband time domain location indication is supported is discussed under AI 9.3.1.

Agreement

If beam nulling is supported for gNB-to-gNB CLI handling, the following are recommended to be specified

- Information exchange of measurement resource configuration, i.e., periodic NZP CSI-RS

Agreement

If beam pairing is supported for gNB-to-gNB CLI handling, the following are recommended to be specified

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS

- Information exchange of recommended/not-recommended DL beam information and associated resource configuration

Agreement

If non-transparent UL resource muting is supported for interference covariance matrix measurement for gNB-to-gNB CLI handling, the following are recommended to be specified

- Definition and indication of UL resource muting pattern

- Collision with DMRS/PTRS

- PUSCH resource mapping, i.e., rate-matching around the muted REs

- UCI resource determination

- Power allocation in symbols with muted REs considering potential impact to phase continuity

- TB size determination

Note: The existing reference signal time-frequency resource pattern, e.g., PT-RS, comb-2 SRS, are the candidates for the UL resource muting pattern.

Note: Consider pattern without adverse impact on PAPR

Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4.

Note: The above does not apply for PUSCH transmission during random access procedures.

Agreement

If non-transparent UL resource muting is supported for gNB-to-gNB CLI measurement for gNB-to-gNB CLI handling, the following are recommended to be specified

- Definition and indication of UL resource muting pattern

- Collision with DMRS/PTRS

- PUSCH resource mapping, i.e., rate-matching around the muted REs

- UCI resource determination

- Power allocation in symbols with muted REs considering potential impact to phase continuity

- TB size determination

- Exchange of information across gNBs on measurement resources

Note: The existing reference signal time-frequency resource pattern, e.g., CSI-RS, are used to determine the UL resource muting pattern.

Note: Consider pattern without adverse impact on PAPR

Note: The potential impact on transmit signal quality/MPR requirement may need to checked with RAN4.

Note: The above does not apply for PUSCH transmission during random access procedures.

Agreement

Consider the following alternatives for down selection in RAN1#117.

Alt.1:

If L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework are supported for UE-to-UE CLI handling, the following are recommended to be specified

- Measurement resources

  - Periodic, semi-persistent, or aperiodic measurement resource (set) i.e., SRS-RSRP resource or CLI-RSSI resource

- Measurement reporting

  - Periodic, semi-persistent or aperiodic reporting on PUCCH/PUSCH

  - New report quantities: e.g L1-SRS-RSRP, L1-CLI-RSSI and/or RS indexes

  - UCI bits generation

  - UCI omission rule

  - Priority rules for multiple CSI reporting

  - CSI processing unit and CPU occupation rule

  - Timeline and related UE behaviours

- CLI measurement accuracy requirement [RAN4]

Alt.2:

If L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework are supported for UE-to-UE CLI handling, the following are recommended to be specified

- Measurement resources

  - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., CLI-IMR

- Measurement reporting

  - CSI measurement procedure integrating CLI measurement

  - Note: Reuse the existing periodic, semi-persistent and aperiodic reporting on PUCCH/PUSCH

  - Note: Reuse the existing report quantities, i.e., CQI, L1-SINR, and the new measurements on CLI-IMR are included in the interference measurement term for the existing report quantities

Alt.3:

If L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework are supported for UE-to-UE CLI handling, the following are recommended to be specified

- Measurement resources

  - Periodic, semi-persistent, or aperiodic measurement resource (set) i.e., SRS-RSRP resource or CLI-RSSI resource or CLI-IMR

- Measurement reporting

  - Periodic, semi-persistent or aperiodic reporting on PUCCH/PUSCH

  - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or RS indexes

  - UCI bits generation

  - UCI omission rule

  - Priority rules for multiple CSI reporting

  - CSI processing unit and CPU occupation rule

  - Timeline and related UE behaviours

  - CSI measurement procedure integrating CLI measurement

- CLI measurement accuracy requirement [RAN4]

Note: The new measurements on CLI-IMR are included in the interference measurement term for the existing report quantities, i.e., CQI, L1-SINR.

Agreement

UL Tx power control based schemes are not considered in the down-selection of gNB-to-gNB CLI handling and UE-to-UE CLI handling schemes.

- Note: Support of UL Tx power control enhancements can be discussed in AI 9.3.1 and 9.3.2 (for PRACH only).

Agreement

If coordinated scheduling in time and/or frequency is supported for gNB-to-gNB CLI handling and UE-to-UE CLI handling, the following is recommended to be specified

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration

Conclusion

L1 based UE-to-UE CLI measurement and reporting based on event triggered based reporting are not considered for UE-to-UE CLI handling in Rel-19.

- RAN1#117

Agreement

Update the agreement in RAN1#116bis (changes mark in red) as follows

If beam nulling is supported for gNB-to-gNB CLI handling, the following are recommended to be specified

- Information exchange of measurement resource configuration, i.e., periodic NZP CSI-RS

- Information exchange of CLI-mitigation request.

- Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3.

Agreement

If non-transparent UL resource muting is supported for gNB-to-gNB CLI handling, Option 1 is recommended to be specified

- Option 1: Comb-2 for both DFT-S-OFDM and CP-OFDM

- Note: Additional details can be discussed within RAN1#117.

- Note: Power boosting is assumed for REs in the symbol with UL resource muting. FFS: Details.

- Above is subject to separate UE capability (including separate capabilities for DFT-S-OFDM and CP-OFDM).

Agreement

If spatial domain-based schemes are supported for UE-to-UE CLI handling for FR2, the following are recommended to be specified

- Rx beams configuration for UE-to-UE CLI measurement

- Note: The above is subject to a separate optional UE capability.

Agreement

Agree the updated Alt.1 (changes in red) for L1 based UE-to-UE co-channel CLI measurement and reporting

Alt.1:

If L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework are supported for UE-to-UE CLI handling, the following are recommended to be specified

- Measurement resources

  - Periodic, semi-persistent, or aperiodic measurement resource (set) i.e., SRS-RSRP resource or CLI-RSSI resource

    - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns

  - Rx beams configuration for UE-to-UE CLI measurement (if spatial domain coordination is supported)

- Measurement reporting

  - Periodic, semi-persistent or At least Aperiodic reporting on PUSCH

    - Note: Periodic and semi-persistent reporting can be considered

    - Note: Aperiodic reporting on PUCCH can be considered

  - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or RS indexes

    - Note: At least wideband reporting is supported

  - UCI bits generation

  - UCI omission rule

  - Priority rules for multiple CSI reporting

  - CSI processing unit and CPU occupation rule

  - Timeline and related UE behaviours

  - Note: The existing UCI omission rule, CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

- CLI measurement accuracy requirement [RAN4]

Alt3 will not be considered in the downselection (i.e. only Alt 1 and Alt 2 will be considered)

Agreement

For enhancements for CLI handling, the following are recommended from RAN1's perspective:

- Information exchange of semi-static cell-specific SBFD time and frequency location configuration [RAN3]

- Information exchange of measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS [RAN3]

- Information exchange of strongest DL beam information [RAN3]

- Information exchange of CLI-mitigation request [RAN3]

  - Note: No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3.

- UL resource muting for PUSCH, including [RAN1, RAN2, RAN4]

  - Indication/Determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

    - Note: No new DCI field / MAC CE is introduced

  - PUSCH resource mapping, i.e., rate-matching around the muted REs

  - UCI resource determination in symbols with muted REs

    - Note: No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212.

  - Note: UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH.

  - Note: UL resource muting applies only for UEs in RRC_CONNECTED mode.

  - Note: UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol.

  - Note: Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols.

  - Note: No changes on TBS determination for PUSCH.

  - Note: Check impact on transmit signal quality/MPR requirement, if any, with RAN4

  - Note: This feature is subject to UE capability

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4]

  - Measurement resources

    - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource

      - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs.

    - Configuration/Determination of ‘typeD’ QCL assumptions for the CLI measurement resource

  - Measurement reporting

    - At least aperiodic reporting

      - Note: Periodic and semi-persistent reporting can be considered

    - New report quantities: e.g. L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - Note: At least wideband reporting is supported

    - UCI bits generation

    - Priority rules for multiple CSI reporting

    - Note: The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point.

  - CLI measurement accuracy requirement [RAN4]

- Note: The above information exchanges are subject to RAN3’s assessment of feasibility and specification impact.

- RAN1#118

Agreement

For the time location configuration of UL resource muting for a PUSCH, one of the following options is selected

- Option 1: Semi-statically configure the position for each of the up to two UL muting symbols within a slot

- Option 2: Semi-statically configure X (X>=1) possible positions for each of the up to two UL muting symbols within a slot

Note: Above has no implication on how UL muting symbol indication/determination is done.

Note: For both options, there will at most up to 2 UL muting symbols for the PUSCH.

Agreement

For the reference point of time location of UL resource muting for PUSCH, Option 1 is supported

- Option 1: Starting symbol of a slot for both PUSCH mapping type A and type B

Agreement

Send an LS to RAN3 with the following information (cc: RAN2)

From RAN1 perspective, support information exchange among gNBs of measurement resource configuration for both SSB and periodic NZP-CSI-RS.

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic multi-port NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Each resource in the set consist of P ports, where 1 ≤ P ≤ Pmax, and Pmax is the largest number of ports for a CSI-RS resource supported in RAN1 specifications (128 in Rel-19).

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

Agreement

LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation is agreed.

Final LS is in R1-2407533.

Agreement

For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- FFS: Method#3: UE measures RSSI within UL subband

Agreement

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

- Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

- FFS: UE behavior for measurement

Agreement

To determine the value of k in Prii,CSIy,k,c,s for CSI reports carrying L1 UE-to-UE CLI report, the following options are considered

- Option 1-1: Reusing existing k value, k = 0

- Option 1-2: Reusing existing k value, k = 1

- Option 2: Adding a new k value other than 0 and 1.

Agreement

For L1 UE-to-UE CLI measurement and reporting, the following are supported

- Wideband CLI-RSRP reporting

- Wideband CLI-RSSI reporting

- FFS: Subband CLI-RSSI reporting

Agreement

For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity.

- FFS: configuration of number of reported CLI resources in the reportConfig

- FFS: reporting criteria

- RAN1#118bis

Agreement

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement.

- The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic

- Note: No new usage needs to be defined for SRS resource sets

Agreement

For aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured is configured by higher layer parameter.

- FFS: Whether to reuse the legacy SRS resource set

- FFS: Whether available slot offset list is also reused

For aperiodic CLI-RSSP reporting using aperiodic CLI-RSSI measurement resource set, the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured is configured by higher layer parameter.

Agreement

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH

- Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

- Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH.

FFS: details of semi-static configuration and dynamic indication (if supported)

FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI

Agreement

For UCI rate matching, to determine the number of coded modulation symbols per layer for UCI transmission on PUSCH, the following option is adopted

- Option 2: The number of coded modulation symbols per layer for UCI on PUSCH is calculated based on MscUCIl after excluding the UL muting resource.

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement

For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs.

- A single wideband RSSI measurement report is supported

  - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Agreement

For aperiodic CLI reporting on PUSCH

- Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig

- Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report

- Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig

Agreement

- For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

Agreement

In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList:

- In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo

  - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic

- If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource:

  - If unifiedTCI-StateType is not configured

    - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET”

  - If unifiedTCI-StateType is configured

    - “Indicated” DL only/joint TCI state as specified in TS 38.214

  - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources

FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

Agreement

Update previous agreement as follows (update in red).

Agreement

Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement.

- The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic

- The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications.

- Note: No new usage needs to be defined for SRS resource sets

Agreement

Update the following agreement (update in red).

Agreement

For aperiodic CLI reporting on PUSCH

- Reuse CSI-AperiodicTriggerStateList to configure the trigger state associated with one or more aperiodic CSI-ReportConfig

- Reuse the existing DCI field 'CSI request' to trigger an aperiodic CLI report

- Introduce an indication in CSI-AssociatedReportConfigInfo that selects a set from the list of CLI-RSSI measurement resource sets or SRS-RSRP measurement resource sets configured in CSI-ResourceConfig

- Note: As in current specifications, the parameter for indication (e.g., resourceSet) is always present, but only used in the case that the CSI-ResourceConfig is configured with multiple sets of aperiodic CLI measurement resources.

Agreement

Alt 2: To determine the value of k in Prii,CSIy,k,c,s for CSI reports carrying L1 UE-to-UE CLI report, the following options is adopted

- Option 1-2: Reusing existing k value, k = 1

Agreement

If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol.

- RAN1#119

Agreement

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

Agreement

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

Agreement

For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols

Agreement

If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Agreement

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

Agreement

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter

- For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD.

- If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Agreement

For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported

- Method #3: UE measures RSSI within UL subband

- A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Conclusion

There is no RAN1 consensus on the following:

- Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

Agreement

Send an LS to RAN4 with the following contents:

On UL resource muting, RAN1 has made the following agreements

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Note that RAN1 is still discussing the boosting in symbols containing PTRS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

Final LS is in R1-2410889.

Agreement

Send reply LS to RAN4 with following information:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios.

For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations.

However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Final LS is in R1-2410890.

Conclusion

For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Conclusion

There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19

Agreement

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported

- The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4}

- The M CLI measurement resource ID(s) are reported

- The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported

- When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference

- FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

- FFS: whether/how to report differential value in case the largest measurement is out of range

Agreement

For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.