3GPP TSG RAN WG1 #120						  R1-2501498

Athens, Greece, February 17th - 21st, 2025

Agenda item:	8.1

Source:	Moderator (NEC)

Title:              	Summary#2 on Draft CR on TAG for sidelink resource allocation mode 1 in TS 38.214

Document for:	Discussion and Decision

- Introduction

In Rel-18, a serving cell can be configured with two TAGs, which are configured by tag-Id and tag2, respectively. It causes ambiguity in TS 38.214 to determine the timing advance in sidelink resource allocation mode 1 for sidelink dynamic grant and sidelink configured grant type 2. Therefore draft CR [1] has been proposed.

During online and offline discussion [2], some companies support to have a conclusion that in RAN1 there is no agreement on supporting a serving cell configured with 2 TAGs and sidelink at the same time, while some other companies think that a serving cell can be configured with 2 TAGs for sidelink, according to RAN2 spec TS 38.321. Therefore a further discussion is needed.

This paper is organized as follows.

- Section 2 is the draft CR [1].

- Section 3 is the collected round 1 companies’ views.

- Section 4 is for round 2 collection of companies’ views.

- Section 5 is for conclusion.

- Discussion

The reason for change, summary of change and consequences if not approved are provided in draft CR [1].

| Reason for change: | In Rel-18, a serving cell can be configured with two TAGs. It causes ambiguity when determine the timing advance in sidelink resource allocation mode 1 for sidelink dynamic grant and sidelink configured grant type 2. |
| --- | --- |
| Summary of change: | In 8.1.2.1, clarify that TTA is the timing advance value corresponding to the TAG associated with tag-Id of the serving cell. |
| Consequences if not approved: | Wrong timing advance if two TAGs are configured in one serving cell. |

#### 8.1.2.1 Resource allocation in time domain <Unchanged part omitted> In sidelink resource allocation mode 1: - For sidelink dynamic grant, the PSSCH transmission is scheduled by a DCI format 3_0. - For sidelink configured grant type 2, the configured grant is activated by a DCI format 3_0. - For sidelink dynamic grant and sidelink configured grant type 2: - The "Time gap" field value m of the DCI format 3_0 provides an index m + 1 into a slot offset table. That table is given by higher layer parameter sl-DCI-ToSL-Trans and the table value at index m + 1 will be referred to as slot offset KSL. - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot, where TDL is the starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value corresponding to the TAG associated with tag-Id of the serving cell on which the DCI is received and KSL is the slot offset between the slot of the DCI and the first sidelink transmission scheduled by DCI and Tslot is the SL slot duration. - The "Configuration index" field of the DCI format 3_0, if provided and not reserved, indicates the index of the sidelink configured type 2. - For sidelink configured grant type 1: - The slot of the first sidelink transmissions follows the higher layer configuration according to [10, TS 38.321]. <Unchanged part omitted>

- (Round 1) Collected companies’ views

Q1: Please provide your views or suggestions on the draft CR [1].

| Company | Support CR as in [1]? (Y/N) | Comments |
| --- | --- | --- |
| Moderator (NEC) | Y | Without the CR as in [1], when two TAGs are configured in one serving cell, the applied timing advance TTA may be ambiguous in TDL-TTA2+KSL×Tslot. If both tag-Id and tag2 are configured, the simple solution is to just choose one TAG to determine TTA to be applied. Thus we propose to adopt the changes as in [1]. |
| Samsung | N | There is no agreement on supporting a cell configured with 2TA and SL at the same time. Hence, the CR is not needed. |
| Moderator (NEC) |  | For Samsung’s comment, please refer to the following spec in TS 38.321, it seems that at least from configuration perspective, it is possible that 2 TAGs are configured for a serving cell for sidelink, thus the issue in the draft CR is valid and it could be nice to solve it to make our spec rigorous. TS 38.321 V18.4.0 5.22.1.3 Sidelink HARQ operation and SL-PRS transmission 5.22.1.3.2 PSFCH reception <OMITTED> If sl-PUCCH-Config is configured by RRC, the MAC entity shall for a PUCCH transmission occasion: 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and the Serving Cell is not configured with two TAGs; or 1> if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. <OMITTED>. |
| LGE | N | We have similar view with Samsung. We have not discussed the possibility of supporting the combination of these two features simultaneously. First of all, RAN1 needs to discuss the necessity of enabling these two feature simultaneously, and then we can send LS to RAN2 for RAN1’s understanding if necessary. [Mod3: thanks for the comments. It could be a way forward too. Our understanding is that if RAN1 concludes we don’t support these two features simultaneously, we can send LS to RAN2 to ask them to remove the related content as we provided above.] |
| vivo | Comment | The intention of the change may need some clarification: is it intended to always use the tag ID (the one presented from R15) for determining the TA, regardless of whether two TAGs are configured or not? [Mod3: thanks for the comments. Although we think in the end the effect is exactly as you said, it is better not to touch Rel-15 spec. Therefore it is only a Rel-18 CR, which is only meaningful when two TAGs are configured.] |
| Mod3 |  | Based on comments received, there may be two possible way forward: Alt1: adopt the changes in CR [1] Alt2: reach a conclusion in RAN1 that we don’t support these two features simultaneously, then we can send LS to RAN2 to ask them to remove the related content as we provided above. Please share your views on the alternative you prefer. |
| OPPO | N | Since the we have never discussed the possibility of supporting these two features during the WI, it is not appropriate to discuss this during the maintenance. This would also have impacts to Rel-18 RRC, which is too late now. We are open to sending an LS to RAN2, but not sure if any related contents need to be removed in their specs. We could ask them to check and remove if necessary. [Mod4: thanks for the comments. We can first reach the conclusion and then discuss what can be included in the LS.] |
| Mod4 |  | Let us see if the following proposal is acceptable. It is proposed to have the following conclusion and send LS to RAN2 about this conclusion. Proposed Conclusion: From RAN1 perspective, for Rel-18 sidelink, the serving cell cannot be configured with two TAGs. The motivation to send LS is to ask RAN2 to check whether they need to remove the two TAGs related content in TS38.321. We can further discuss the content and wording of LS. If RAN2 confirms that the two TAGs related content can be removed, the draft CR in [1] will not be pursued. |
| Qualcomm | N | We acknowledge that, during the maintenance phase, such a change may appear as an enhancement, but the suggested clarification appears the obvious way to solve it, which likely a UE implementation will support independently of whether we clarify in the spec or not. However, we prefer not to send an LS to RAN2, nor say that the serving cell cannot have 2 TAGs. Given the above comments, a proposal from our side is to say: “From RAN1 perspective, in a serving cell configured with two TAGs, the determination of the TAG id to be used for sidelink resource allocation in time domain is left up to UE implementation.” [Mod5: thanks for the comments. But if we cannot reach conclusion in RAN1 that two TAGs are not support for sidelink, we prefer to have our original CR, compared to ‘left up to UE implementation’, it adds more clarity. On the other hand, based on comments received so, it seems a fact that for sidelink, a serving cell configured with two TAGs is not discussed or supported.] |

(Round 1) According to companies’ input, it is proposed to have the following conclusion and send LS to RAN2 about this conclusion.

Proposed Conclusion:

From RAN1 perspective, for Rel-18 sidelink, the serving cell configured with two TAGs is not discussed or supported.

- (Round 2) Collection of companies’ views

Q1: Please provide your views.

| Company | Comments |
| --- | --- |
| Moderator (NEC) | Based on comments received online and offline, some companies support to have a conclusion that in RAN1 there is no agreement on supporting a serving cell configured with 2 TAGs and sidelink at the same time, while some other companies think that a serving cell can be configured with 2 TAGs for sidelink, according to RAN2 spec TS 38.321. They are contradictory to each other and may lead to different handling of the draft CR, the moderator suggests the following: Sending LS to RAN2 to check whether they do support that a serving cell can be configured with 2 TAGs for sidelink, given no RAN1 discussion on this point. |
| Mod2 | The draft LS can be as follows, please check. 1. Overall description In Rel-18 MIMO, two TAGs can be configured for a serving cell. But for Rel-18 sidelink, in RAN1, the support of two TAGs configured for a serving cell has not been discussed. RAN1 would like to ask RAN2 the following question. Q1: In TS 38.321 V15.4.0, clause 5.22.1.3.2., for sidelink, there is one condition that the serving cell is not be configured with two TAGs and there is the other condition that the serving cell is configured with two TAGs, is it the correct understanding that from RAN2 perspective, two TAGs can be configured for a serving cell for sidelink? TS 38.321 V18.4.0 5.22.1.3 Sidelink HARQ operation and SL-PRS transmission 5.22.1.3.2 PSFCH reception <OMITTED> If sl-PUCCH-Config is configured by RRC, the MAC entity shall for a PUCCH transmission occasion: 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and the Serving Cell is not configured with two TAGs; or 1> if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. <OMITTED>. 2. Actions: To RAN2 ACTION: RAN1 would like to respectfully ask RAN2 to provide answers to the above question and consider potential clarification in RAN2 specification, if any. |
| OPPO | According to the above clause in TS 38.321 pointed out by the moderator, it seems like RAN2 specification supports the operation when 2 TAGs are configured for the serving cell. In this case, there is no point to ask this question to RAN2 in a LS. I think what we need to do in RAN1 is that given this is the situation, what should we do in RAN1. Option 1: No RAN1 specification change, let UE implementation to decide what to do in L1 when 2 TAGs are configured for the serving cell. This could be a conclusion. Option 2: No RAN1 specification change. This may mean in the end SL operation does not support 2 TAGs configured for the serving cell in Rel-18. No need to have a conclusion for this option. Option 3: No RAN1 specification change. RAN1 should send an LS to RAN2 stating that for Rel-18 sidelink, the serving cell configured with two TAGs has not been discussed in the past and we don’t plan to support this operation for SL in L1. It is up to RAN2 to decide whether to remove any of the related parts in RAN2 specification for SL operation. Option 4: Since Rel-18 MIMO and RAN2 specification for SL supports configuration of 2 TAGs for the serving cell, RAN1 should discuss the support of this operation in L1 as Rel-18 maintenance. [Mod3: thanks for the comments and suggestions. It may not be the real intention for RAN2 to support two TAGs for sidelink without any RAN1 discussion, it is likely that they just copy-paste the same wording in 5.3.2.2 for DL-SCH. Thus we think it is better to have LS to check the answer. With this being said, Option 3 above seems very aligned with intention and wording of the proposed LS, because 1) in draft LS description part ‘But for Rel-18 sidelink, in RAN1, the support of two TAGs configured for a serving cell has not been discussed.’, this paragraph is to say ‘for Rel-18 sidelink, the serving cell configured with two TAGs has not been discussed in the past and we don’t plan to support this operation for SL in L1’ 2) in draft LS action part ‘ask RAN2 to provide answers to the above question and consider potential clarification in RAN2 specification’, this paragraph is to say ‘up to RAN2 to decide whether to remove any of the related parts in RAN2 specification for SL operation.’ Hopefully it is ok to go with this direction. We also modify the wording to better to reflect Option 3. Please see the conclusion part.] |
| Samsung | We thank moderator’s response in the first round, and tend to agree with OPPO that it is very clear RAN2 supported this feature in their spec already. Two high level directions can be considered from our perspective: - RAN1 figured out there is issue to support this feature and ask RAN2 to revert the CR, which may need a solid technical reason for supporting this direction. - RAN1 figured out whether it is possible to support this feature without or with minimized RAN1 spec change. [Mod3: thanks for the comments and suggestions. As mentioned in above reply to OPPO, it may not be the real intention for RAN2 to support two TAGs for sidelink without any RAN1 support, it is likely that they just copy-paste the same wording in 5.3.2.2 for DL-SCH. Thus it may not be a serious issue to ask RAN2 to check the situation and of course up to them to remove it or not. With this being said, hopefully it is ok to go with direction 1 for now. If they do reply that it is truly the intention to have two TAGs for a serving cell for side, we can proceed with direction 2. We also modify the wording to better to reflect it. Please see the conclusion part.] |

- Conclusion

Proposed Conclusion [1]:

From RAN1 perspective, for Rel-18 sidelink, the serving cell configured with two TAGs is not discussed or supported.

According to companies’ input, it is proposed to

Send LS to RAN2 to check whether they do support that a serving cell can be configured with 2 TAGs for Rel-18 sidelink, given no RAN1 discussion on this point.

The draft LS includes the following:

1. Overall description

In Rel-18 MIMO, two TAGs can be configured for a serving cell. But for Rel-18 sidelink, in RAN1, the support of two TAGs configured for a serving cell has not been discussed.

RAN1 would like to ask RAN2 the following question.

Q1: In TS 38.321 V15.4.0, clause 5.22.1.3.2., for sidelink, there is one condition that the serving cell is not be configured with two TAGs and there is the other condition that the serving cell is configured with two TAGs, is it the correct understanding intention that from RAN2 perspective, two TAGs can be configured for a serving cell for sidelink, given the situation that the support of two TAGs configured for a serving cell has not been discussed RAN1?

TS 38.321 V18.4.0 5.22.1.3 Sidelink HARQ operation and SL-PRS transmission 5.22.1.3.2 PSFCH reception <OMITTED> If sl-PUCCH-Config is configured by RRC, the MAC entity shall for a PUCCH transmission occasion: 1> if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is stopped or expired, and the Serving Cell is not configured with two TAGs; or 1> if the Serving Cell on which the HARQ feedback is to be transmitted is configured with two TAGs and if the timeAlignmentTimer of the TAG, associated with the TCI state(s) used for transmitting the HARQ feedback, is stopped or expired: 2> not instruct the physical layer to generate acknowledgement(s) of the data in this TB. <OMITTED>

2. Actions:

To RAN2

ACTION: RAN1 would like to respectfully ask RAN2 to provide answers to the above question and consider potential clarification in RAN2 specification, if any.

- References

- R1-2500595, Draft CR on TAG for sidelink resource allocation mode 1 in TS 38.214, NEC, RAN1 #120, Athens, Greece, February 17th - 21st, 2025

- R1-2501447, Summary on Draft CR on TAG for sidelink resource allocation mode 1 in TS 38.214, NEC, RAN1 #120, Athens, Greece, February 17th - 21st, 2025