3GPP TSG RAN WG1 #120			R1-2501592

Athens, Greece, February 17th – 21st, 2025

Source: 	Moderator (CMCC)

Title:	Summary #5 for coding aspects of physical channel design

Agenda:	9.4.2

Document for:	Discussion & Decision

Introduction

In RAN#106 meeting, a new work item on solutions for Ambient IoT (Internet of Things) in NR has been approved [1], in which a new A-IoT technology focusing on deployment scenario 1 with topology 1 (D1T1) for device 1 and with carrier wave (CW) node outside topology (-B transmission case), is recommended for standardization.

The following RAN1-led objectives have been identified within the General Scope described in the WID:

- RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

This contribution provides a summary of the submitted contributions, issues for discussion, and outcomes in RAN1#120 meeting.

Online/offline proposals

Monday online

[H] Proposal 3-1-v2

For R2D Manchester line code, TR 38.769 already gives us:

6.1.1.2 R2D line coding The line codes studied for R2D are Manchester encoding and PIE. For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. A chip corresponds to one modulated symbol. For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}.

[H] Proposal 4-1-v1

For D2R transmission, the following alternatives are for down-selection:

- Alt. 1: Manchester line code is adopted using the bit-to-chip mapping: bit 0 to chips {10}, bit 1 to chips {01}. For D2R transmission with frequency location of small frequency shift R/Tb Hz, each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to a bit (that is before Manchester line code), where R = Tb/(2 × D2R chip length).

- Alt. 2: For D2R transmission with frequency location of small frequency shift is R/Tb Hz, where Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length), the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation; the transmitted 2R chips for information bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

  - Note: It is up to device implementation to use the Manchester line code with codeword repetition or use no line code with a square-wave modulation to generate the transmitted chips.

[H] Proposal 4-2-v2

For a D2R chip, TR 38.769 already gives us:

#### 6.1.2.2 D2R line coding The line codes studied for D2R are Manchester encoding, FM0 encoding, Miller encoding, and no line coding. FM0 encoding is deprioritized. A chip corresponds to one modulated symbol at least for OOK and BPSK.

[H] Proposal 5-1-v1

RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and mother code rate of 1/3 are reused for Ambient IoT Device 1.

[H] Proposal 5-2-v2

For the initial values of the shift register of the CC encoder, down-select from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Option 3: The initial values of the shift register of the CC encoder are set to zero, and (K-1) zero bits are appended to the end, i.e., zero-tail convolutional code (ZTCC).

- Note: K is the constraint length.

[H] Proposal 5-4-v2

For D2R transmission, not using FEC is supported.

- The reader indicates the use or not use of FEC by R2D control for D2R scheduling.

[H] Proposal 6-1-v1

For D2R repetition, at least block level repetition is supported.

- FFS: Whether the repeated blocks are transmitted via a single PDRCH or can be transmitted via different PDRCHs.

- FFS: Necessity to support bit level repetition type 2 for D2R performance improvements.

[H] Proposal 7-1-v1

RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC is reused for Ambient IoT Device 1.

[H] Proposal 7-2-v2

When CRC is attached to a PRDCH or PDRCH transmission,

- When the block size is ≤ X bits, CRC-6 is used. Otherwise, when the block size is > X bits, CRC-16 is used. Down-selections from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 57

- FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

[H] Proposal 7-3-v2

Down-select from the following options to determine when no CRC is used,

- Option 1: A block size threshold Y. When block size is ≤ Y bits, no CRC is used. Down-selections from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

  - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

- Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type.

Tuesday offline

[H] Proposal 5-2-v2

For the initial values of the shift register of the CC encoder, down-select from the following three options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Option 3: The initial values of the shift register of the CC encoder are set to zero, and (K-1) zero bits are appended to the end, i.e., zero-tail convolutional code (ZTCC).

- Note: K is the constraint length.

[H] Proposal 5-4-v2

For D2R transmission, not using FEC is supported.

- The reader indicates the use or not use of FEC by R2D control for D2R scheduling.

[H] Proposal 6-1-v1

For D2R repetition, at least block level repetition is supported.

- FFS: Whether the repeated blocks are transmitted via a single PDRCH or can be transmitted via different PDRCHs.

- FFS: Necessity to support bit level repetition type 2 for D2R performance improvements.

[H] Proposal 7-2-v2

When CRC is attached to a PRDCH or PDRCH transmission,

- When the block size is ≤ X bits, CRC-6 is used. Otherwise, when the block size is > X bits, CRC-16 is used. Down-selections from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 57

- FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

Tuesday online

[H] Proposal 5-1-v1

RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and mother code rate of 1/3 are reused for Ambient IoT Device 1.

[H] Proposal 7-1-v1

RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC is reused for Ambient IoT Device 1.

[H] Proposal 5-2-v3

For the initial values of the shift register of the CC encoder, down-select from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Note: K is the constraint length.

[H] Proposal 6-1-v2

For D2R repetition, at least block level repetition is supported.

[H] Proposal 5-4-v3

For D2R transmission,

- Not using FEC is optionally supported.

- When FEC is used, further study the necessity to support other code rate than the mother code rate by puncturing.

  - Note: Supporting other code rates should provide a sufficiently large performance gap in SNR.

- The reader indicates the use or not use of FEC, or the code rate (if other code rates than the mother code rate are supported), by R2D control.

Wednesday offline

[H] Proposal 7-2-v2

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

- FFS all segments should use the same CRC, if segmentation is applied.

[H] Proposal 7-3-v2

Down-select from the following options to determine when no CRC is used,

- Option 1: An information bit threshold Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

  - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

- Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, , ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type.

[H] Proposal 5-4-v3

For D2R transmission,

- Not using FEC is supported.

- When FEC is used, further study the necessity to support other code rate of CC than the mother code rate 1/3 by puncturing.

  - Note: Supporting other code rates should provide a sufficiently large performance gap in SNR.

- The reader indicates the use or not use of FEC, or the code rate (if other code rates than the mother code rate are supported), by R2D control.

[M] Proposal 4-4-v2

For D2R transmission with small frequency shift +/- R/Tb Hz, for the value R, down-select from the two alternatives:

- Alt. 1: A subset of R = 2n, n = 0, 1,…

- Alt. 2: A subset of R = 1 and 2n, n = 1, 2, …

- FFS the values of n and maximum value of R, depending on at least the device sampling clock, minimum D2R chip duration, number of multiplexed devices, indication overhead.

[M] Proposal 4-2-v2

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be no larger than 1/(640*2 kHz)= 0.78 us and include at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

Thursday online

[H] Proposal 7-2-v3

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS all segments should use the same CRC, if segmentation is applied.

[H] Proposal 7-3-v3

One or both of the following options are considered to determine when no CRC is used,

- Option 1: A threshold of number of information bit threshold Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

- Option 2: Specified condition(s), e.g., device transmits PDRCH for Msg 1 without CRC upon receiving a PRDCH triggering random access for paging transmission. FFS specified condition(s) and/or how the device (to) determines the specified condition(s).

[H] Proposal 5-4-v3

For D2R transmission, not using FEC is supported.

Punctured code rate of 1/2

other punctured code rate of convolutional encoder from the mother code rate 1/3

WF1

For D2R transmission, support multiple code rates, e.g., 1, 1/2, 1/3, 1/6.

WF2

For D2R transmission, support multiple code rates, at least 1, 1/2, 1/3, 1/6.

[H] Proposal 5-x-v3

For D2R transmission,

- When FEC is used, further study the necessity to support other code rate of convolutional encoder than the mother code rate 1/3 by puncturing.

  - Note: Supporting other code rates should provide a sufficiently large performance gap in SNR.

- The reader indicates the use or not use of FEC, or the code rate (if other code rates than the mother code rate are supported), by R2D control.

[M] Proposal 4-4-v2

For D2R transmission with small frequency shift +/- R/Tb Hz, for the value R, a set of values are selected down-select from the two alternatives:

- Alt. 1: A subset of R = 2n, n = 0, 1,…

- Alt. 2: A subset of R = 1 and 2m, m = 1, 2, …

- FFS the values of n, m and maximum value of R, depending on at least the device sampling clock, maximum minimum D2R chip duration, number of multiplexed devices, indication overhead.

[M] Proposal 4-2-v2

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication multiple or a factor of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be comparable to 1/(640*2 kHz)= 0.78 us and include at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

Friday offline

      - [H] Proposal 7-3-v4

One or both of the following options are considered to determine when no CRC is used,

- Option 1: A threshold of number of information bit threshold Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

- Option 2: Specified condition(s), e.g., device transmits PDRCH for Msg 1 without CRC upon receiving a PRDCH triggering random access for paging transmission. FFS specified condition(s) and/or how the device to determine the specified condition(s).

      - [H] Proposal 5-4-v3 (WF1)

For D2R transmission, support multiple code rates r = (1/k) * (1/p) where (1/k) is the code rate of the convolutional code, and p is the number of repetition, at least [1] and 1/3.

- [For code rate = 1, it is achieved by bypassing the convolutional encoder and no repetition.]

      - [H] Proposal 5-4-v3 (WF2)

For D2R transmission, support multiple code rates, at least 1, 1/2, 1/3, 1/6.

      - [M] Proposal 4-4-v3

For D2R transmission with small frequency shift +/- R/Tb Hz, for the value R, a set of values are selected down-select from the two alternatives:

- Alt. 1: A subset of R = 2n, n = 0, 1,…

- Alt. 2: A subset of R = 1 and 2m, m = 1, 2, …

- FFS the values of n, m and maximum value of R, depending on at least the device sampling clock, maximum minimum D2R chip duration, number of multiplexed devices, indication overhead.

      - [M] Proposal 4-2-v2

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication a multiple or a factor of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be comparable to 1/(640*2 kHz)= 0.78 us and include at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

Friday online

      - [H] Proposal 7-3-v4

One or both of the following options are considered to determine when no CRC is used,

- Option 1: A threshold of number of information bit threshold Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

- Option 2: Specified condition(s), e.g., device transmits PDRCH for Msg 1 without CRC upon receiving a PRDCH triggering random access for paging transmission. FFS specified condition(s) and/or how the device to determine the specified condition(s).

      - [H] Proposal 5-4-v3 (WF1)

For D2R transmission, support multiple code rates, r = (1/k) * (1/p) where 1/k is the code rate of the convolutional encoder and p is the number of repetitions.

- For the code rate of the convolutional encoder 1/k, at least 1/3 is supported. FFS other values, e.g., 1, or other punctured code rate.

- FFS the number of repetitions p.

- The reader indicates the code rate by R2D control.

      - [M] Proposal 4-4-v3

For D2R transmission with small frequency shift +/- R/Tb Hz, for the value R, a set of values are selected down-select from the two alternatives:

- Alt. 1: A subset of R = 2n, n = 0, 1,…

- Alt. 2: A subset of R = 1 and 2m, m = 1, 2, …

- FFS the values of n, m and maximum value of R, depending on at least the device sampling clock, maximum minimum D2R chip duration, number of multiplexed devices, indication overhead.

      - [M] Proposal 4-2-v2

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication a multiple or a factor of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be comparable to 1/(640*2 kHz)= 0.78 us and include at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

      - [H] Proposal 5-2-v4

For the initial values of the shift register of the CC encoder, down-select by RAN1#120bis from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Note: K is the constraint length.

[CLOSED] R2D line code

Background

In the WID, RAN1 supports Manchester encoding as R2D line code.

The TR captures the bit-to-chip mapping for Manchester encoding: bit 0→chips{10}, bit 1→chips{01}.

Summary of inputs

Several companies (e.g., China Telecom, Honor, ITL) discuss the bit-to-chip mapping for Manchester encoding. China Telecom and Honor propose the same bit-to-chip mapping that captured in the TR, while ITL propose the bit-to-chip mapping as: bit 0 →chips{01}, bit 1→chips{10}.

FL understands that we need to have an agreement on the bit-to-chip mapping for Manchester encoding in the normative work, and that has already agreed in the study item should be adopted.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 3-1-v1

For R2D Manchester line code, the bit-to-chip mapping is: bit 0 to chips {10}, bit 1 to chips {01}.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Our understanding is that the FFS from the study still applies, i.e. we can consider as necessary different mappings for CP handling. See RAN1#116bis agreement for R2D CP handling for OFDM based OOK waveform, “FFS: Detail of relationship to line code codewords” |
| TCL | Support only one maping for R2R line code. |
| Spreadtrum | We support this proposal with one comments. For both of R2D and D2R, the bit-to-chip mapping of Manchester line code is: bit 0 to chips {10}, bit 1 to chips {01}. However, FL has no similar proposal for D2R Manchester line code, so we proposed a minor modify as follows: [H] Proposal 3-1-v1 For R2D Manchester line code, the bit-to-chip mapping is: bit 0 to chips {10}, bit 1 to chips {01}. |
| OPPO | We support this proposal. And modification from Spreadtrum is fine to us. |
| QC | Ok with current proposal. We think “R2D” needs be captured in the proposal based on previous study outcome. |
| vivo | Support the proposal. |
| ZTE, Sanechips | okay |
| Nokia | OK |
| Panasonic | We support the proposal. |
| Samsung | We are fine with the proposal and would like to capture the same thing for D2R as well. |
| DOCOMO | Support. We are fine to keep R2D (it can be PRDCH more specifically since no line coding for R2D preamble) considering that it is unclear whether Manchester coding should be applied to D2R. |
| HONOR | Support. |
| IDCC | Fine with the proposal. |
| Xiaomi | For Manchester line coding, the bit-to-chip mapping should be explicitly determined and specified. Both R2D and D2R should have the same bit-to-chip mapping definitions to simplify the system design. The mapping definition in TR can be applied for normative phase. |
| Huawei, HiSilicon | The mapping in the TR is fine, and the WID already tells us to take in the decisions of the TR, so no need to repeat. A RAN1 conclusion to that effect will be sufficient. |
| Ericsson | Fine. |
| Fujitsu | OK. |
| FL | FL shares views with companies that this Manchester bit-to-chip mapping has already been captured in the TR, and we don’t need to further open the discussion in RAN1 based on the WID. From spec drafting perspective, FL thinks that a RAN1 conclusion would be enough, and -v2 is updated: - [H] Proposal 3-1-v2 For R2D Manchester line code, TR 38.769 already gives us: 6.1.1.2 R2D line coding The line codes studied for R2D are Manchester encoding and PIE. For R2D reception, the device can use line codes to achieve at least chip-level time tracking by using the transition(s) of the line code. A chip corresponds to one modulated symbol. For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}.. |
| LGE | Support |
| NEC | Support Spreadtrum’s proposal with “R2D” deleted to avoid same discussion for D2R. |
| Apple | OK |

D2R line code and associated small frequency shift (SFS)

[CLOSED] D2R line code and SFS options

Background

In the WID, RAN1 needs to discuss and down-select D2R line code to either Manchester line code or no line code, and corresponding small frequency shift method in the TR:

- For Manchester encoding, the bit-to-chip mapping is bit 0→chips {10}, bit 1→chips {01}. For applying with Manchester line codes with a repetition number R ≥ 1:

  - Option 1: Each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length).

  - Option 2: By multiplying the Manchester codeword with a square wave corresponding to the small frequency shift, the time duration Tb corresponding to each information bit includes R number of square wave periods, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). The multiplication operation is performed as either an XOR or XNOR operation between a Manchester codeword corresponding to the information bit and the square wave for the small frequency shift.

- If no D2R line code is used, by using a square-wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

Summary of inputs

From reviewing the contributions submitted in the meeting, companies’ view on D2R line code and the associated SFS options are summarized as follows:

- Manchester line code (21): Ericsson, Futurewei, ZTE, Huawei, Spreadtrum, China Telecom, CMCC, vivo, Nokia, TCL, OPPO, Xiaomi, Samsung, LGE, Lenovo, Sharp, InterDigital, Honor, ITL, Docomo, IIT Kanpur

  - SFS Option 1 (14): Ericsson, Futurewei, ZTE, Huawei, Spreadtrum, China Telecom, CMCC, vivo, TCL, OPPO, Xiaomi, Sharp, Honor, ITL

  - SFS Option 2 (3): Samsung, LGE, InterDigital

- No line code and use a square wave corresponding to the SFS (7): Ericsson, Futurewei, CATT, TCL, Fujistu, LGE, Qualcomm

Regarding the two Manchester SFS options, although they have similar D2R link performance, many companies mention that Option 2 has a wider spectrum than Option 1 after SFS (the spectrum of Option 2 after SFS doubles that of Option 1 after SFS), as shown below [9]. Therefore, the link budget of Option 1 and Option 2 is different. From D2R coverage perspective, Option 1 is more attractive than Option 2.

Figure: PSD of no SFS, Manchester SFS Option 1 and Manchester SFS Option 2

In addition, regarding the down-selection between Manchester line code or no line code and associated small frequency shift, it should be noted that many companies (e.g., Futurewei, ZTE, HW, CMCC, vivo, NEC, Apple, LGE, Qualcomm, Docomo) state that the time domain waveform of Manchester line code without SFS or with SFS option 1 is equivalent to no line code with square wave modulation, as shown below. Some companies (e.g., NEC, Apple, Qualcomm) suggest specifying the transmitted bit format for these two equivalent solutions and up to device implementation on one of them.

Figure: Time domain waveform for Manchester SFS Option 1 and no line code with a square-wave modulation

FL understands that although these two solutions have the same effect, either we down-select to one, or only specify the output bit form, the potential RAN1 specification impact is different. Since most companies propose Manchester line code with SFS Option 1, FL lists it as one option. Another option is to ignore the down-selection between the two and propose the output bit form.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 4-1-v1

For D2R transmission, the following alternatives are for down-selection:

- Alt. 1: Manchester line code is adopted using the bit-to-chip mapping: bit 0 to chips {10}, bit 1 to chips {01}. For D2R transmission with frequency location of small frequency shift R/Tb Hz, each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to a bit (that is before Manchester line code), where R = Tb/(2 × D2R chip length).

- Alt. 2: For D2R transmission with frequency location of small frequency shift is R/Tb Hz, where Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length), the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation; the transmitted 2R chips for information bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

  - Note: It is up to device implementation to use the Manchester line code with codeword repetition or use no line code with a square-wave modulation to generate the transmitted chips.

| Company | Alt. ½ | Comments |
| --- | --- | --- |
| FUTUREWEI | 1 |  |
| TCL | 1 and 2 | Thanks for FL’s efforts. Support both of them. For Alt.2, the description of ‘the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation’ is just like bit ‘1’ mapping [0 1 0 1..] only and bit ‘0’ mapping [1 0 1 0..] only. Same comment could be applied for BPSK case. Thus, we give one suggestion for Alt.2 that this description above mentioned could be corrected as ‘...the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …]/ and [1 0 1 0 …], respectively, for OOK modulation; the transmitted 2R chips for information bit 1 and bit 0 are [-1 +1 -1 +1 …]/ and [+1 -1 +1 -1 …], respectively, for BPSK modulation.’ Also, it is not clear why transmitted 2R chips could be limited by [0101...] and [1010...] for OOK and [-1 +1 -1 +1...] and [+1 -1 +1 -1..] for BOSK. In our understanding, there is no any motivation to limit transmitted for case of without line code. Thus, we would want to add potential format for other motivations, e.g., interference estimation. Thus, we further suggest to correct this Alt.2 description as ...the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …]/ and [1 0 1 0 …]/other formats if any, respectively, for OOK modulation; the transmitted 2R chips for information bit 1 and bit 0 are [-1 +1 -1 +1 …]/ and [+1 -1 +1 -1 …]/other formats if any, respectively, for BPSK modulation. |
| Spreadtrum | Alt.1 | We support Alt.1. We have some concern for Alt.2 - Alt2 may introduce the complexity of device implementation, as anyway device has to choose to implement the Manchester line code with codeword repetition or no line code with a square-wave modulation or both of them. - Additional, use no line code with a square-wave modulation might require an additional square wave generator, which will the complexity of device. |
| OPPO | Alt 1. |  |
| QC | Alt. 2 | We think Alt2 is better way of capturing the proposed scheme. Given that there is no technical difference in the generated waveform between two schemes, it is straightforward to directly describe the output waveform. There is no Manchester coding in BB info bits (after down shifting) One of the reasons that it should not described in terms of “Manchester coding” in Alt.1 is that in Alt.1 generates the outcome directly multiplying the baseband information bit with square-wave for frequency shift. This means that when the output waveform is “de-frequency shifted”, the generated baseband information bits are no longer Manchester coded. That is, the baseband info has no embedded clocking information (i.e., no transition in the center of symbol). One good reference is Miller encoding in RFID. In Miller encoding case, base band information bits are 1) Miller coded first, and then 2) frequency up-shifted by multiplying square-wave. This means that even after reader down shift the received signal by square-wave, the generated base band bits are still Miller encoded bits. RFID reader can use non-coherent receiver to detect Miller encoded bits by detecting transitions of Miller coding. So, for RFID case, it makes sense to say info bits are Miller encoded with frequency shift. But, this is not the case for A-IoT, since, as discussed above, there is no more Manchester encoding after down-shift. So, we think it is better to describe the scheme as proposed in Alt.2. To avoid confusion with repetition coding Another reason that we should avoid using “repetition” is that repetition is used for other purpose – for improving link performance to complement FEC scheme. We think bit level repetition should be involved with energy gain. So, the term “repetition” should not be used in Alt.1. It does not bring additional energy gain. |
| vivo | Alt 1 | unified line coding design for R2D and D2R. |
| ZTE, Sanechips | 1 | Alt1 use the same descriptions as R2D line code, which is simple and straightforward. |
| Panasonic | Alt. 1 | The implementation of Alt. 1 is simpler for A-IoT devices, without requiring having a square wave generator. |
| Samsung | Alt 2 | We support Alt 2. For Alt 1, its description is too specific to Manchester coding. |
| DOCOMO | Alt.2 | Given that there is no difference in the output of time domain waveform between Manchester option 1 and no line coding which means no difference on reception of D2R at reader side, we think it is not essential to down-select from Manchester option 1 vs no line coding which can be left to device implementation. |
| HONOR | Alt 1 | Support Alt 1 |
| IDCC | Alt 2. |  |
| Xiaomi | Alt. 1 | In WI phase, either Manchester line coding or No line coding should be determined for potential normative work. Device implementation is not in the scope of discussion. From the perspective of device cost/complexity, Alt. 1 is preferred to be supported. |
| Huawei, HiSilicon | Alt 1 | For the purpose of achieving small frequency shifts, when using option 1, the output waveform is the same for Manchester codes and no line codes. However, if no small frequency shift is used, using no line codes have a significant disadvantage because of the high-pass filter used to suppress the CW interference at the reader, which would also suppress the frequency components around DC, resulting in the receiver not being able to recover the original data bits due to an incomplete signal after filtering. |
| Ericsson | Alt. 1 | We support Alt 1. |
| LGE | Alt 2 | Support Alt 2 |
| NEC | Alt1 | Alt.1 offers simple description. |
| Apple |  | OK with either alternative. They are equivalent |

D2R chip length and bandwidth

Background

In the study item, a D2R chip is defined as one modulated symbol for OOK or BPSK. Suppose that an D2R bit length is Tb, the D2R chip length is Tb/(2 × R), where R is the SFS factor.

In addition, D2R transmission bandwidth, Btx,D2R, has been defined in the study item as the frequency resources scheduled by a reader for a D2R transmission from one device. The transmission bandwidth without SFS and with different SFS schemes have also been studied. Taking Manchester SFS Option 1 or no line code with a square wave modulation as an example, the relationship of D2R chip rate and D2R transmission bandwidth are shown in the following figure.

Figure: Relationship with D2R transmission bandwidth, D2R chip rate/length, SFS factor R

Summary of inputs

Based on the contributions submitted in the meeting, several aspects are discussed:

- Definition of D2R chip or chip length: ZTE, CATT. It is proposed that the D2R chip corresponds to one modulated OOK or BPSK symbol, as captured in the TR.

- Relationship of D2R bit length, D2R chip length, SFS factor R, and D2R transmission bandwidth: ZTE, Huawei, CATT, CMCC. For 2SB modulation, it is proposed that the D2R chip length = D2R bit length Tb / (2 × R). The D2R transmission bandwidth, Btx,D2R, is associated with the D2R baseband chip length (i.e., R = 1 without SFS), Btx,D2R = 2 / (D2R chip length × R).

- D2R chip length or chip rate to be specified: ZTE, Huawei, CATT, CMCC, vivo, OPPO, MediaTek, Sharp, Qualcomm, Docomo. The proposed values are quite diverse, and companies’ proposals are from different points of view.

  - ZTE, Huawei, and Docomo propose to define a set of D2R chip/bit length, or chip rate. To be specific, ZTE prefers{xx us}. Huawei suggests that the D2R chip length should include an integer number of samples to avoid non-integer counting errors. Two options of device sampling frequency (1.92 MHz and 2.4 MHz) are discussed, and 2.4 MHz sampling clock is preferred which has higher transmission efficiency, and the corresponding D2R bit lengths are {266.67μs, 133.33μs, 66.67μs, 26.67μs, 13.33μs, 6.67μs, 3.33μs, 1.67μs}.

  - CMCC, vivo, OPPO, MediaTek, and Qualcomm discuss this issue from D2R chip rate or D2R bandwidth perspective. For example, it is proposed that the D2R chip rate is in integer multiple(s) of 15 kHz.

  - Huawei, CMCC, and InterDigital discuss that it is helpful to define the D2R chip/bit length as a multiple of the R2D chip/bit length. InterDigital proposes that the D2R bit duration = 2-n × R2D chip duration.

  - CATT propose that the maximum D2R chip rate is no higher than 120 kHz.

FL understands that some high-level principles should be considered for discussion on the D2R chip length. First, similar as the discussion on R2D M values that the largest M should ensure that the Ambient IoT R2D data rate is competitive, the largest D2R data rate (related to the smallest D2R chip length) should also be competitive, e.g., larger than 320 kbps. Second, for the smallest D2R data rate (related to the largest D2R chip length), during the evaluation in the study item, to satisfy the coverage requirement, the D2R data rate is as low as ~ 1 kbps, which can be considered as a start point. For other D2R chip lengths to be defined, it is helpful to define the D2R chip length as a multiple of R2D chip length, to avoid introducing additional sample errors.

Round 1 discussion

Based on the inputs, the following questions and proposals are formulated.

      - [H] Proposal 4-2-v1

A D2R chip is one modulated symbol for OOK and BPSK.

- The D2R chip length equals Tb / (2 × R), where Tb is the D2R bit length, and R is the small frequency shift factor (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb).

| Company | Comments |
| --- | --- |
| FUTUREWEI | Editorial comment: there is an additional character between the T and subscript. Can it be removed? |
| TCL | For Tb explanation, is this for with FEC or w/o FEC? |
| Spreadtrum | We support with this proposal with an improvement of the wording to eliminate ambiguity: A D2R chip is corresponds to one modulated symbol for OOK and BPSK. - The D2R chip length equals Tb / (2 × R), where Tb corresponds to the time duration of a D2R bit is the D2R bit length, and R is the small frequency shift factor (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb). |
| OPPO | OK |
| QC | We think, in the Figure’s frequency shift should be 30kHz for R=12 (frequency shift = R/Tb = 12*2.5 = 30kHz) instead of 60kHz. We suggest following modification for proposal. The D2R chip duration length is equals to Tb / (2 × R), where Tb is the D2R bit length, and R is the small frequency shift factor (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb). FL: Thanks for the comment, yes right, the update has been implemented! |
| vivo | Generally fine. |
| ZTE,Sanechips | okay |
| Nokia | Both instances of “length” should be replaced with “duration” for clarity and consistency with earlier agreements (chip duration, bit duration) |
| Panasonic | We are fine with the proposal. |
| Samsung | We are fine with the proposal with the following modification: - The D2R chip length equals Tb / (2 × R), where Tbis the duration corresponding to one information bit in D2R, and R is the small frequency shift factor. For the i.e. part, the description of the small frequency shift factor should be included in Proposal 4-1, if needed. |
| DOCOMO | OK |
| HONOR | Ok |
| IDCC | Ok. |
| Xiaomi | OK with the proposal. |
| Huawei, HiSilicon | The TR already explains this, and the WID takes into the WI the decisions of the TR. Although this wording is equivalent (and it is fine), a RAN1 conclusion to repeat the text of the TR is sufficient. |
| Ericsson | Fine. |
| FL | Similar suggestions for R2D line code, for the main bullet, the proposal is updated referring to what we have captured in the TR. For the sub-bullet, FL realized that it is duplicated with Proposal 4-1, so it is removed for now. - [H] Proposal 4-2-v2 For a D2R chip, TR 38.769 already gives us: #### 6.1.2.2 D2R line coding The line codes studied for D2R are Manchester encoding, FM0 encoding, Miller encoding, and no line coding. FM0 encoding is deprioritized. A chip corresponds to one modulated symbol at least for OOK and BPSK.. |
| LGE | We support the modification from QC. |
| NEC | Support. Tb should be restricted to duration of one bit before line code but after channel code(if applied.). |
| Apple | OK with the definition |

      - [M] Question 4-3-v1

Please provide your views on the following principles when RAN1 defines the D2R chip length:

- Rule #1: The D2R chip length is a multiple of the R2D chip length, e.g., a D2R chip length is a multiple division or multiplication of a R2D chip length, to avoid introducing additional counting errors. Since 1 OFDM symbol is 66.67 us, some examples of D2R chip length such as 133.33 us, 66.67 us, 33.33 us, 16.67 us, 11.11 us, etc., can be further considered.

- Rule #2: For the minimum D2R chip length, which is related to the D2R peak data rate, it should be competitive, e.g., larger than 320 kHz. Note that the minimum D2R chip length is limited by device sampling clock, and a D2R chip should at least include 2~4 samples. The minimum D2R chip length can use 2.08 us (peak data rate is 480 kHz if FEC code rate = 1) as a starting point.

- Rule #3: For the maximum D2R chip length, which is related to the lowest D2R data rate, the evaluated values during the study item can be considered so as to meet the coverage requirement. For example, 133.33 us (lowest data rate is ~1.25 kbps if FEC code rate = 1/3) can be considered as a starting point.

| Company | Rule #1/2/3 | Comments |
| --- | --- | --- |
| FL |  | Please indicate if other rules are to be considered. |
| TCL | #1 | It seems OK for rule #1 if SCS=15KHz. |
| OPPO | Comment | We are considering what’s the intention of this Question if Proposal 4-2-v1 is agreed, is it motivated to introduce some limitation for the parameter R? if yes, we suggest to discuss it with Proposal 4-4-v1 together. FL: The intention is that in RAN1, we should define the supported D2R chip duration, similar as what we have for R2D, we have M values regarding to different data/chip rate. I agree that the discussion is related to the determination of R values to some extent, but suggest to have separate discussion. |
| QC |  | It should be noted that following relations hold. - D2R bit duration = Tb - D2R data rate = 1/Tb - D2R frequency shift FS = R/Tb = R×D2R data rate - D2R chip duration Tc = Tb/2R= 1/(2×FS) D2R chip duration Tc should be described in terms of supported maximum frequency shift FS instead of data rate. Since D2R chip duration is a function FS only. Here is suggested proposal. Proposal: Following relations are defined. - D2R bit duration = Tb (sec) - D2R data rate = 1/Tb (bps) - D2R frequency shift FS (Hz) = R/Tb = R×D2R data rate - D2R chip duration Tc (sec) = Tb/(2×R) = 1/(2×FS) The minimum D2R chip duration is determined by max frequency shift of [FFS e.g., 640/960k]Hz. Maximum D2R chip duration can be determined by the minimum data rate of [FFS e.g., 10]kbps. |
| vivo | Comment | We are generally fine with Rule #1. For rule #2, we agree to have comparable data rate as RFID, but we’d like to clarify, in RFID, maximum data rate is 640kHz rather than 320kHz (640kHz BLF with FM0, which can be understood as R=1). We suggest to change the ‘larger than 320kHz’ to ‘no smaller than 640kHz’. And we’d like to better understand why a D2R chip should be at least include 2~4 samples. We agree to have rule to determine maximum D2R chip length, but we don’t agree with Rule #3. Maximum D2R chip length determines smallest small frequency shift. If small frequency shift is too small, resulting in close to CW frequency, D2R suffers undesirable CW interference. Therefore, we think rule #3 should also add ‘minimum small frequency shift with sufficient gap from CW frequency’. |
| ZTE,Sanechips |  | We are okay with the principles of rule 1/2/3. Comments and suggestions are as below. - Rule #1: The D2R chip length is a multiple of the R2D chip length, e.g., a D2R chip length is a multiple division or multiplication of a R2D chip length, to avoid introducing additional counting errors. Since 1 OFDM symbol is 66.67 us, some examples of D2R chip length such as 133.33 us, 66.67 us, 33.33 us, 16.67 us, 11.11 us, etc., can be further considered. [ZTE,Sanechips] Our understanding of “multiple of” doesn’t includes division. The detailed values can be further discussed. So the examples can be removed. - Rule #2: For the minimum D2R chip length, which is related to the D2R peak data rate, it should be competitive, e.g., larger than 640 320 kHz. Note that the minimum D2R chip length is limited by device sampling clock, and a D2R chip should at least include [2~4] samples. The minimum D2R chip length can use 2.08 us (peak data rate is 480 kHz if FEC code rate = 1) as a starting point. [ZTE,Sanechips] Okay with this principle in general. It seems there is a mixture use of “bit duration” and “chip duration”. The statement “The minimum D2R chip length can use 2.08 us (peak data rate is 480 kHz if FEC code rate = 1” assumes that 2.08 us is bit duration for Manchester code. Furthermore, the gap between the peak date rates should be further discussed. So the examples can be removed. - Rule #3: For the maximum D2R chip length, which is related to the lowest D2R data rate, the evaluated values during the study item can be considered so as to meet the coverage requirement. For example, 133.33 us (lowest data rate is ~1.25 kbps if FEC code rate = 1/3) can be considered as a starting point. [ZTE,Sanechips] Same comments as above regarding the mixture use of “bit duration” and “chip duration”. |
| Huawei, HiSilicon |  | For Rule 1, it is helpful that the D2R chip length is a multiple of the R2D chip length and an integer number of samples can be counted for the D2R and R2D chip length. For Rule 2, since the D2R chip length is limited by the device sampling clock, we prefer to use a 2.4 MHz sampling clock that would support a chip length of 0.83 us, which includes 2 samples per chip. |
| Apple |  | For rule #2, need to agree on the supported sampling clock of device 1. The chip rate will be further derived based on sampling clock and bit rate. |

Round 2 discussion

[CLOSED, updates to -v3] Proposal 4-2-v2

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be no larger than 1/(640*2 kHz) us and include at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

| Company | Comments |
| --- | --- |
| Ericsson | According to the proposal, the minimum chip duration is (1/640) us=1.56 ns. Do you mean (1/640 kHz) = 1.56 us? [Q] What is the basis for the peak rate of 320 kHz or 640 kHz? FL: Correct, change is implemented to avoid confusion. 640 kbps is the peak data rate (BLF) in RFID. |
| CATT | Our preference is to separate Proposal 4-2-v2 into two parts: - first provide the general definition of D2R chip, following Proposal 4-2-v1. - Further define the details (e.g., the values) with the consideration of the companies’ feedback on Question 4-3-v1. To “include at least 2 sampling points per chip”, what is. the assumption of the sampling rate? FL: Regarding the sampling rate, my feeling is that when we have the discussion, we should have some basic assumptions on the sampling clock. For example, the clock is around 2 MHz, and when we discuss the particular chip duration, it would be beneficial if it can be applicable for different clock implementations, such as 1.92 MHz or 2.4 MHz or 2.88 MHz, etc. But eventually in the spec, we don’t need to specify what is the device sampling clock. |

      - [M] Proposal 4-2-v3

A D2R chip corresponds to one modulated symbol for OOK and BPSK. RAN1 further defines the D2R chip duration considering at least the following:

- A D2R chip duration is a multiple division or multiplication of a R2D chip duration, to avoid introducing additional counting errors.

- The minimum D2R chip duration should be comparable to 1/(640*2 kHz)=0.78 us and includes at least 2 sampling points per chip.

- The maximum D2R chip duration should be no smaller than 133.33 us.

| Company | Comments |
| --- | --- |
| FL | Some further updates, see -v3 below: - As we are discussing chip duration and 640 kHz is data rate, there was a typo in the previous version, the correct chip duration should be 1/(640*2 kHz) us, which is 0.78 us. - Due to different assumptions on sampling clock, no larger than 1/(640*2 kHz) = 0.78 us and includes at least 2 sample points may not always be true at the same time, so changing “no larger than” to “comparable to.” |
| Huawei, HiSilicon | For the first bullet, there may be some correction is saying the chip duration is a “multiple division or multiplication”. It could be corrected to only “multiple division or multiplication”. |

SFS factor R and frequency resources for FDMA

Summary of inputs

From reviewing the contributions in the meeting, many companies further discuss the values of SFS factor R and candidate frequency resources for D2R FDMA.

Values of R

Regarding the potential values of R, some companies (e.g., Huawei, vivo, OPPO) propose that at least R = 1 is supported, meaning that no SFS is applied. For the maximum R, essentially it is related to the device sampling frequency and the minimum D2R transmission bandwidth (i.e., maximum D2R baseband chip length). Assuming that device sampling frequency is 1.92 MHz, the minimum D2R transmission bandwidth is 15 kHz (correspond to a maximum chip length of 266.67 us), R can be no larger than 128, as proposed by vivo and OPPO. If device sampling frequency is 2.4 MHz, R can be no larger than 160, as proposed by Huawei.

In addition, the potential values of R should also consider D2R FDMA, so as to multiplex as many as devices with satisfactory D2R performance. Companies point out that using Manchester codeword repetition or square-wave modulation to achieve small frequency shift, the odd harmonics are inevitable. Therefore, the selection of R should at least avoid the odd harmonics. Different sets of R values are proposed:

- R = 2n: ZTE, Huawei, vivo, OPPO, Interdigital, Qualcomm

- R = 2n: Huawei, CMCC, Apple

It is straightforward that defining R = 2n can avoid odd harmonics. On top of R = 2n, values satisfying R = 2n are also possible to avoid odd harmonics, which can provide higher multiplexing capability than R = 2n. An illustration of the spectrum of using R = 2n is shown below [9]. It is observed that ideally, the spectrum of R = {2, 4, 6, 8, 10} are orthogonal. Even considering the practical SFO issue (e.g., 10% SFO at the device side), examples such as R = {2, 10}, R ={4, 8}, R ={6, 10}, are also applicable for a given D2R baseband chip rate [9].

Figure: PSD of different R values for BPSK and OOK modulations

Determination and indication of R for FDMA

Several companies (e.g., CMCC, vivo, OPPO, Docomo) further discuss the determination of R values for FDMA and how it is indicated to devices.

- The reader explicitly indicates R values to device: CMCC, vivo, Docomo. In this case, the reader by implementation determines the set of R values taking into account aspects such as D2R baseband chip length, device SFO, odd harmonics to be avoided, etc. RAN1 needs to define the complete set of R values, at least considering the tradeoff between indication overhead and multiplexing capability.

- RAN1 specifies/tabulates set of R values for different cases: CMCC, OPPO. For example, OPPO proposes that for a D2R transmission bandwidth, a set of R values together with the number of multiplexed devices is defined. In this case, the expected RAN1 specification work is large.

- The set of R values are implicitly indicated or derived: CMCC, Docomo. For example, the reader can indicate the maximum R value used and a pre-defined rule of selecting R, and the device can then derive or calculate the set of R values. In this case, the indication overhead is low, but RAN1 needs additional work to define a selection rule applicable to different cases.

In the following, the values of R are first discussed. The determination and indication of R for FDMA can be considered in the next step pending the progress on R values.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [M] Proposal 4-4-v1

For the small frequency shift factor R (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb), support that,

- R = 1 when no small frequency shift.

- For R > 1, down-select from the two alternatives:

  - Alt. 1: R = 2n

  - Alt. 2: R = 2n

  - FFS the maximum value of R, depending on at least the device sampling clock, minimum D2R chip length, number of multiplexed devices, indication overhead.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Ok |
| TCL | We are okay for both of two main bullets. For second bullet, whether the range of n could be limited for low capability device 1. As mentioned in device architecture of TR, there is one limitation for SFS is a few MHz. In our understanding, maximum n may be needed if given one SCS to meet device 1’s limited capability. |
| Spreadtrum | For R > 1, we suggest to take Alt.1 as a baseline first, whether other additional values of R (= 2n) can be adopted depends on the performance evaluation of the main lobe influenced by odd harmonic interference from other devices. Moreover, considering the interference to inter-system (NR), it is up to RAN4 to assess the impact on the ACLR with the other additional values of R (= 2n). |
| QC | R=1 also introduces frequency shift since it is equivalent to multiply information bit with one period of square-wave. Without frequency shift of R=1, the baseband information bit spectrum will be centered around CW frequency. So, R=1 corresponds to a type of frequency shift. We suggest following modification. Proposal For the small frequency shift factor R (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb), support that, - R = 1 when no small frequency shift. - For R > 1, down-select from the two alternatives - Alt. 1: R = 2n for n = 0, 1, 2, … - Alt. 2: R = 1 or 2n for n = 1, 2, … - Alt. 3: subset of Alt.2 FFS details - FFS the maximum value of R, depending on at least the device sampling clock, minimum D2R chip length, number of multiplexed devices, indication overhead. |
| vivo | Generally fine. |
| ZTE, Sanechips | Comments and suggestions are as below. For the small frequency shift factor R (i.e., the number of repeated Manchester codewords within Tb or number of square-wave periods within Tb), support that, - FFS: R = 1 when no small frequency shift. [ZTE, Sanechips] Starting with R=2 is helpful to mitigate self-interference. - For R > 1, down-select from the two alternatives: - Alt. 1: R = 2n - Alt. 2: R = 2n - FFS value(s) of n and the maximum value of R, depending on at least the device sampling clock, minimum D2R chip length, number of multiplexed devices, inter-device interference,indication overhead. [ZTE, Sanechips] Instead of simply supporting all potential n values that meet the requirements of R< max R, we think the exact value/range of n should be evaluated to achieve the tradeoff between reducing the inter-device interference, and max number of multiplexing devices. |
| Panasonic | We support the proposal. |
| IDCC | Fine with the proposal. |
| Xiaomi | The proposal is OK. For R>1 when small frequency shift is applied, down-select from the two alternatives: |
| Huawei, HiSilicon | We need to clarify does “no small frequency shift” mean the spectrum is kept around DC? If so, this case is a problem due to need to filter CW. Hence it seems that “small frequency shift” with a minimum step away from DC is the minimum case. In the FL’s notation, this seems to require R>1 in all cases. For R>1, we are fine to include some Alt 1 where R=2n. However, including some R=2n also achieves avoiding the odd harmonic of the square wave while giving the reader more options for how to use frequency resource via FDMA. |
| Ericsson | Fine. |
| Apple | OK |
| FL | From the inputs of the first round, and also the agreements regarding D2R line code and associated small frequency shift, the proposal is revised. |

Round 2 discussion

      - [M] Proposal 4-4-v2

For D2R transmission with small frequency shift +/- R/Tb Hz, for the value R, down-select from the two alternatives:

- Alt. 1: A subset of R = 2n, n = 0, 1,…

- Alt. 2: A subset of R = 1 and 2n, n = 1, 2, …

- FFS the values of n and maximum value of R, depending on at least the device sampling clock, minimum D2R chip duration, number of multiplexed devices, indication overhead.

| Company | Comments |
| --- | --- |
| Spreadtrum | We suggest to evaluate the exact value of n depends on the main lobe influenced by odd harmonic interference from other devices. |
| CATT | For more efficient use of the spectrum, our preference is not to limit the values of R to either Alt.1 or Alt.2. It may be desirable to the reader more freedom to configure the R properly to minimize the FDM interference and optimize the use of the spectrum with the consideration of various factors in the field. |
| FL | Based on the comments, added to FFS the values of n in the above version, as in red. |
| Huawei, HiSilicon | We think that a combination of values from the Alt 1 and Alt 2 can be supported for the value of R, and a down-selection may not be the right approach. If we assume a sampling clock of 2.4 MHz, the maximum value of R would be 160, and if we go with Alt 1, all values of R=2n can be supported and additionally, some values satisfying 2n can be included. |

D2R FEC

[CLOSED] Generator polynomial

Background

In the WID, D2R supports only convolutional code with generator polynomials as per TS 36.212 (shown below), unless RAN1 decides to use other generator polynomials by RAN1#120bis.

Figure: LTE convolutional code generation with constraint length of 7 and mother code rate of 1/3.

Summary of inputs

Based on the submitted contributions in the meeting, majority views (Ericsson, Futurewei, Huawei, Spreadtrum, CATT, China Telecom, CMCC, vivo, Nokia, OPPO, xiaomi, Apple, Samsung, Fujistu, LGE, Honor, InterDigital, ITL) support to reuse convolutional code with generator polynomials as per TS 36.212. On the other hand, a few companies (ZTE, Qualcomm, Lenovo) propose to discuss other generator polynomials, e.g.,

- Mother code rate lower than 1/3 with constraint length K = 7, additional new polynomial nested to LTE CC.

- Mother code rate ranging from 1/2 to 1/6 with constraint length K < 7 or K > 7, different generator polynomials.

Companies for both sides provide evaluation results and analysis for different designs. It is observed that,

- The link performance of other designs which provides marginal performance gain over LTE CC (no more than 0.5 dB at target BLER) is at the cost of an increased number of registers or XOR gates.

- The link performance of other designs which is similar or inferior to LTE CC, the potential benefits on device complexity are marginal with less 1 or 2 number of registers and more XOR gates.

- The updates of generator polynomials provide additional complexity of reader decoding hardware.

Most companies support reusing generator polynomials as per TS 36.212. For the proposed new polynomials, it either has a very marginal performance gain over LTE CC at the cost of an increased number of registers and XOR gates, or has similar or even inferior performance to LTE CC with no clear reduction on device complexity (with 1 or 2 less number of registers but more XOR gates). Meanwhile, introducing new polynomials inevitably increases the complexity at the reader side to upgrade decoding hardware. FL understands that the necessity and benefits to have new generator polynomials are not well justified, and it is proposed to reuse polynomials of LTE CC.

Round 2 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 5-1-v1

RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and mother code rate of 1/3 are reused for Ambient IoT Device 1.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Ok |
| Spreadtrum | Y |
| OPPO | Yes |
| QC | New polynomials with lower coding rate could be used for both non-block repetition cas and block repetition case. For block repetition case, different polynomials could be used for different transmission providing coding gain plus energy gain. |
| vivo | Yes |
| ZTE, Sanechips | Based on our evaluations, the following alternatives provides benefits than the LTE CC with mother code rate of 1/3, which should be considered. Alt. 1: Nested polynomials, lower mother code rate of 1/4 and constraint length 7. Pros: better performance than LTE CC Alt. 2: Nested polynomials, lower mother code rate of 1/4 and constraint length 8. Pros: better performance than LTE CC Alt. 3: Nested polynomials, lower mother code rate of 1/6 and constraint length 6. Pros: less complexity than LTE CC |
| Nokia | OK |
| Panasonic | Yes |
| Samsung | Support |
| DOCOMO | Support. |
| HONOR | OK |
| IDCC | Ok. |
| Xiaomi | Yes. The mother code rate and final code rate should be 1/3. Puncture and shift register processing are not applied to the mother code rate for different final code rate, since it will increase device’s cost/complexity. |
| Huawei, HiSilicon | We support this proposal. |
| Ericsson | Fine. |
| Fujitsu | OK. |
| LGE | Support |
| NEC | Yes. |
| Apple | OK |
| Round 2 discussion |  |
| FL | From the inputs in the first round, it seems that only 2 companies are not fine with the proposal. Again, from the WID, we should stick to the polynomials defined in TS 36.212, unless there is a strong reason to consider new polynomials. However, the gain on link performance or complexity reduction is too marginal from the evaluation results provided, therefore, the proposal is not changed and further comments are welcomed. |
| Spreadtrum | Support to reuse generator polynomials of LTE CC. |
| Ofinno | Support. |

Initialization of shift register

Summary of inputs

From reviewing the contributions in the meeting, companies discuss the initialization of the convolutional code encoder, and several options are considered:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC).

  - Proposed by: Futurewei, Spreadtrum, CATT, China Telecom, Qualcomm

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end to make sure that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

  - Proposed by: Futurewei, Huawei, CMCC

- Option 3: The initial values of the shift register of the CC encoder are set to zero, and (K-1) zero bits are appended to the end, i.e., zero-tail convolutional code (ZTCC).

  - Proposal by: Futurewei, Fujistu

- Option 4: The initial values of the shift register of the CC encoder are set to zero, i.e., truncation convolutional code (TrCC).

Note that K is the constraint length.

From the discussion, the performance of TBCC and HBCC are the same as both can ensure the initial state and ending state of the shift registers is the same, which outperforms ZTCC (especially for small payload) and TrCC. TBCC is the existing method in LTE, but HBCC can provide additional benefits, for example, to save (K-1) bits register to store the last (K-1) CRC parity bits, and to enable parallel processing of CRC attachment and CC encoder.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 5-2-v1

For the initial values of the shift register of the CC encoder, down-select from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Note: K is the constraint length.

| Company | Option 1/2 | Comments |
| --- | --- | --- |
| FUTUREWEI | 2 | Option 2 can reduce storage needs for FEC encoding of tail-biting. Clarification: in the summary of inputs, please remove Futurewei from option 3 FL: Implemented! |
| Spreadtrum | Option 1 | We support Option 1 to reuse the TBCC method in LTE. Some companies mentioned that the additional benefits of HBCC is to decrease the processing time. However, compared to TBCC, HBCC only saves the processing time of adding CRC. It needs further quantitative analysis on how much proportion of CRC processing time can be saved compared with the whole processing time of the generation of PDRCH procedure. Additionally, for HBCC, the sequence of information bits entering the FEC encoder will be disrupted, the first 6 bit will be attached in the tail of the inputs bits, which may increase the decoder complexity from the decoding side (reader). Additionally, HBCC will increase spec. impact. |
| OPPO | Option 1 | TBCC is applied in LTE and its performance has been verified. If any other method is used in A-IoT, its performance compared to TBCC in all cases should be justified before we agree to do so. Furthermore, the A-IoT is not delay sensitive system, for example, the transmission duration of Msg 1 will be tens of ms, and even more longer for Mgs 3. Therefore, in our view, the reduction of delay of HBCC is marginal. |
| QC | Option 1 | Current LTE CC shift register is initialized with last bits of information bit sequence. CRC generation is quick and there should be enough time gap T_R2D_min between R2D and D2R transmission, which allows enough processing time of CRC and FEC. So, there should be no issue with enough gap T_R2D_min. Option 2 requires redesign of decoder – since tx bit sequence is now different from that of original LTE CC encoder. Option 2 does not reduce memory requirement since original information bits should be stored somewhere before it is sent from device. |
| vivo |  | Before down-selection, we suggest to further align companies understanding on whether following issue of TBCC is acceptable for device 1: - Memory issue. Memory to store the output signal after CRC generation before start of channel coding, e.g., for up to 1000 bits. If the memory to store the whole TB is infeasible for device 1, device 1 may regenerate the TB again for channel coding. - Latency issue. Because parallel processing for CRC generation and channel coding is not possible for TBCC, it results in latency of (TB size/read rate per bit), which may be up to 1ms, for up to 1000 bits TBS. On top of TBCC and HBCC, there is also zero biting CC. Whether the L bits overhead is unacceptable? |
| ZTE, Sanechips | 1 | We don’t see any benefits to support option 2. The response to the proponents of option 2: Benefit-1 provided by proponents of option 2: reduce memory/register numbers Comments: As it is pointed out in the following figure in our Tdoc, after all the information bits (Data) are input, the output of the CRC parity bits in CRC registers of D0~D15 are generated. The initialization bits of CC encoding are kept in the CRC registers (parity bits), and no extra memory/registers is needed to store CRC bits. Moreover, other solution such as using nested polynomial for CRC generations can reduce device registers. Figure 13 CC coding chain in R1-2500125 Benefit-2 provided by proponents of option 2: reduce encoding latency Comments: The time required by encoding/CRC generation is short, for example, a level of microsecond, there is no clear benefits of HBCC to reduce encoding latency. |
| Samsung | Option 1 | TBCC has been used in LTE and has been validated over a long period of time. Therefore, we prefer not to introduce a new technique at the receiver that would require additional verification. Moreover, the A-IoT D2R link does not have delay requirements like high-speed communication. Thus, imposing additional burdens to reduce CRC attachment delay is unnecessary. |
| DOCOMO | Option 1 | The justification to support option 2 is still unclear for us. As companies mentioned, it is not critical to reduce the latency by using HBCC. The main motivation to support TBCC is that we can reuse what we have in LTE which is beneficial from reader implementation perspective and spec impact perspective. Unless the critical issue is identified to is identified for shift register initialization, we don’t see the need to support option 2. |
| IDCC | Option1 |  |
| Xiaomi | Option 1 | Reuse LTE TBCC is the simplest and most efficient way. The potential benefit of option 2 is not clear which might be evaluation for comparison. |
| Huawei, HiSilicon | Option 2 | We support option 2 because of its advantages in terms of the CC and CRC operation taking place concurrently, saving on additional processing time. Another point to be noted is that in option 2, the number of information bits would be the same for both cases, since the K information bits are removed from the input bit stream and appended to the end. |
| Ericsson |  | Fine. |
| Fujitsu |  | Echo vivo’s input that there is still a middle ground between TBCC and HBCC, i.e., encoding with zero-state start and zero-state end. We propose to bring this alternative into consideration as well, particularly for when the block length is long. If the encoder with zero-state start and zero-state end cannot be considered, we prefer TBCC. The reason is because TBCC is mature and more reliable compared wiht HBCC at least in 3GPP relevant applications. |
| LGE | Option 1 | No feasibility concerns on implementing Option 1. No strong motivation for the Option 2. |
| NEC | Option 1 | By tail-biting, after one repetition, the transmitter does not need to clean those registers, and it only need to fill those information bit again, which reduces the implementation complexity. |
| Apple | Option 1 | Support option 1. Need to option 2, which is claimed to save CRC processing time, should be studied, as it was not discussed in the study phase. |
| FL |  | Based on the inputs, added another option proposed by some companies: - [H] Proposal 5-2-v2 For the initial values of the shift register of the CC encoder, down-select from the following two options: - Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212. - Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC). - Option 3: The initial values of the shift register of the CC encoder are set to zero, and (K-1) zero bits are appended to the end, i.e., zero-tail convolutional code (ZTCC). - Note: K is the constraint length. |

Round 2 discussion

[CLOSED, continue to discuss -v3 below] Proposal 5-2-v2

For the initial values of the shift register of the CC encoder, down-select from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Option 3: The initial values of the shift register of the CC encoder are set to zero, and (K-1) zero bits are appended to the end, i.e., zero-tail convolutional code (ZTCC).

- Note: K is the constraint length.

| Company | Option 1/2/3 | Comments |
| --- | --- | --- |
| FL |  | Based on the inputs in the first round, a few companies also want to discuss zero tailing, which has been listed as Option 3. Companies are encouraged to provide further comments. Here are some understandings / clarifications from FL’s perspective: For the three options, clearly that we have already done sufficient investigation, and the performance of zero tailing is worse than tail biting or head biting, especially for a small packet size. For tail biting and head biting, the performance is the same. Most companies prefer Option 1 in the first round, but the proponents of Option 2 actually show valid points for a very low cost device 1. Some concerns on head biting increase the receiver decoding complexity. To FL’s understanding, the only price we pay is that the reader re-orders what it receives and then use the exact same decoding implementations of tail biting. |
| Spreadtrum | Option 1 | The overall gain of HBCC has not been justified, as we explained in Round 1. |
| Ofinno |  | Okay with listing options. Minor comment that down selection is now among 3 options not 2. |
| Ericsson | 1 |  |

      - [H] Proposal 5-2-v3

For the initial values of the shift register of the CC encoder, down-select from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits, i.e., tail biting convolutional code (TBCC) defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same, i.e., head biting convolutional code (HBCC).

- Note: K is the constraint length.

| Company | Comments |
| --- | --- |
| Nokia | OK. It might be better to remove “head” from Option 2 since this was controversial. |
| Huawei, HiSilicon | We support the use of Option 2 for following main reasons: 1. Reduction in processing time. It is important to note that RFID did not use any FEC and since AIoT devices will be using it, it would incur additional processing time which we must strive to minimize. The concurrent operation of the CC and the CRC would help in achieving this. 2. The device does not always reply with a pre-stored EPC-like. It may reply with other data stored in the tag, which is not fixed. No pre-computation can be done in such cases. To extract this data from memory at the speed required to match RFID is infeasible (e.g. to obtain 100 bits from memory in 10 us implies an internal data rate of 100 Mbps!). Therefore we need to eliminate as much possible of the buffering time before FEC computation which is caused by waiting for the legacy tail biting CRC bits to be available. |
| QC | Prefer option 1. |

Rate matching

Summary of inputs

Based on the contributions submitted in the meeting, a few companies (ZTE, HW, CATT, NEC) analyze the device operation for rate matching. ZTE, HW and CATT believe that rate matching operations, such as sub-block interleaving, is too complicated for device 1 which requires large buffer size and frequent write/read to the memory. On the other hand, NEC proposes considering a simple and effective rate matching procedure, but no details are discussed.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [M] Proposal 5-3-v1

Do NOT support rate matching operation, e.g., sub-block interleaving, for Ambient IoT Device 1.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Ok |
| Spreadtrum | Y |
| OPPO | Yes |
| vivo | Agree with proposal. |
| ZTE, Sanechips | Sub-block interleaving is a part of rate matching operation in LTE, we agree it is too complicated for device 1. But whether to support rate matching depends whether to support higher FEC code rate than mother code rate, which doesn’t necessarily need interleaving. Update is as below. Do NOT support rate matching operation, e.g., sub-block interleaving, for Ambient IoT Device 1. |
| Panasonic | We support not to have rate matching operation like NR or LTE including sub-block interleaving. On the other hand, "just coding rate adjustment by certain puncturing the encoded bits" is also general meaning of rate matching operation, and it would be aligned with FL description of " the benefit of having other code rates without changing the polynomials, e.g., by puncturing, is not clear and should be further justified." |
| HONOR | OK |
| Xiaomi | Support. |
| Huawei, HiSilicon | We support the proposal. |
| Ericsson | Fine. Sub-block interleaving can improve convolutional coding performance but it can be challenging for the device. |
| Fujitsu | We think that the rate matching should be further clarified, such as to limit the ‘rate matching’ as LTE/NR rate matching mechanism. This is because rate matching is a definition which is wider than “sub-block interleaving” adopted in LTE/NR. If a traditional rate-matching method for CC is adopted, such as puncturing coded bits according to a predefined puncturing matrix, the complexity additionally required for the encoder would be minor. Regarding whether we need rate matching or not, the more essential consideration may be whether we need code rates which are higher than the code rate provided by the mother code. |
| LGE | Support |
| Apple | Support |
| FL | In LTE, rate matching includes operations such as bit selection, sub-block interleaving, which is different with what we discussed for having other code rates of CC. To avoid ambiguity, let’s focus on the sub-block interleaving, which is commonly not supportable. |

Round 2 discussion

      - [M] Proposal 5-3-v2

Do NOT support sub-block interleaving for Ambient IoT Device 1.

| Company | Comments |
| --- | --- |
| Spreadtrum | Support. |
| Ericsson | Ok |
| CATT | Ok |
| Nokia | OK |
| Huawei, HiSilicon | Yes. |

FEC code rate (incl. use or not use of FEC)

Summary of inputs

From reviewing the contributions in the meeting,

- Huawei discusses to support not using FEC for cases when devices are expected to be close to the reader. In addition, Spreadtrum also mentions that if D2R message size is less than 6, no FEC is used.

- In addition, a few companies (e.g., Futurewei, ZTE, Docomo) propose to support a higher code rate than 1/L (L is the number of polynomials) by puncturing.

FL understands that FEC reusing the generator polynomials as per TS 36.212 (i.e., code rate = 1/3) is the baseline for D2R transmission, no FEC (which is equivalent to code rate = 1) can also be considered, which is helpful to support a competitive D2R peak data rate, e.g., larger than 320 kHz. Besides code rate = 1/3 and code rate = 1, the benefit of having other code rates without changing the polynomials, e.g., by puncturing, is not clear and should be further justified.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 5-4-v1

For D2R transmission, not using FEC (equivalent to code rate = 1) is supported.

- The reader indicates the use or not use of FEC or indicates that code rate = 1/3 or code rate = 1 by R2D control for D2R scheduling.

| Company | Comments |
| --- | --- |
| FUTUREWEI | In addition, R=1/2 should also be added. Depending on overall coding rates supported, R=1/2 can be useful |
| Spreadtrum | We support this proposal with one observation and one comment. From our observation, compared to FEC, no FEC + repetition can also improve D2R link performance; plus, with no FEC + repetition, a flexible code rate can be achieved by adopting the different repetition number. One the other hand, as mentioned in our contribution[R1-2500170], for some small message size, e.g., the total number of bits entering the FEC encoder may less than 6, the FEC processing may ineffective since the initial value of the shift register of the encoder shall be set to the values corresponding to the last 6 information bits in the input stream. The small message size may be the Command Response to an Command case, which may include an acknowledgement and optionally data (e.g. in case of the Read operation). Although CRC and/or repetition can be used to enlarge the bit sequence input to the FEC encoder, given the small data payload, it is unnecessary to use FEC considering the large overhead. For example, the coding rate is 1/3, coupling with the redundancy brought by CRC and/or repetition, the resource utilization is relative low. Therefore, we support to use no FEC for some scenario. |
| OPPO | OK |
| QC | Multiple coding rates should be considered, e.g., R=1, 1/2, 1/3, 1/4, 1/6 Polynomial sets could be in nested structure such that polynomials in higher coding rate is reused for lower coding rate case. This can lower device encoder complexity. |
| vivo | We support FEC is configurable. Regarding configuration details, it can be FFS. |
| ZTE, Sanechips | Not using FEC is not the same as code rate = 1. For example, one can use only one polynomial of CC to generate a encoded bit with length equal to the information bits. So to reduce other unnecessary discussion, we suggest to remove “ (equivalent to code rate = 1)”. Furthermore, whether this code rate considers Manchester code is unclear. For D2R transmission, not using FEC (equivalent to code rate = 1) is supported. - The reader indicates the use or not use of FEC or indicates that code rate = 1/3 or code rate = 1 by R2D control for D2R scheduling. |
| Nokia | Additional coding rates can be considered, so we suggest to add at least an “FFS: additional code rates”. |
| Panasonic | We support the proposal. |
| Samsung | We do not support this proposal. Devices only support the use of FEC. The purpose of FEC is to guarantee performance in the worst-case scenario. Therefore, making FEC usage controllable is an unnecessary optimization that could increase costs. |
| DOCOMO | We don’t see the strong need to support the case where no FEC is applied while it may not be harm to support as one possible option in spec. In SI, it was assumed to apply FEC to PDRCH for coverage evaluation. We are open to discuss to support multiple code rate. |
| HONOR | Support this proposal |
| Xiaomi | Not support this proposal. FEC should always be applied for D2R. FEC is used to guarantee the D2R transmission reliability, which has performance gain than no-FEC. According to the scope of WID, FEC should be used for D2R transmission without no-FEC case. |
| Huawei, HiSilicon | We support the proposal. The reason we want to support this case is to ensure that devices that are close to the reader can achieve high data rates while reducing the inventory completion time. There exist scenarios where devices are inventoried when always close to the reader, and require short inventory completion times, where not using FEC can be advantageous. Since the distribution of devices and readers is known by the network deployment, due to the vertical integration of A-IoT, it does not depend on any kind of dynamic feedback to the reader. Such a reader can always indicate to the devices to not use FEC, in order to achieve a higher data rate and to improve the D2R transmission efficiency. |
| Ericsson | Fine |
| Fujitsu | This proposal seems like the continuation of Proposal 5-3-v1. The previous one may need to be settled down first. |
| LGE | Support. |
| NEC | OK, for rate between 1 and 1/3, for example, rate 1/2, it can be achieved by repetition of results with no FEC. |
| FL | Based on the comments from ZTE, it would be safer to just say use or not use of FEC. If FEC is used, there are companies proposing code rate other than 1/3. Two aspects: - There are some proponents to introduce code rate lower than 1/3 with new generator polynomials, this will be handled by Proposal 5-1 - There are also proposals, even reusing generator polynomials of LTE CC, to achieve other code rates by, e.g., puncturing. Let’s separate the discussion in another proposal. I’ll provide a new proposal in the next round. For the upcoming online, let’s focus on this: - [H] Proposal 5-4-v2 For D2R transmission, not using FEC is supported. - The reader indicates the use or not use of FEC by R2D control for D2R scheduling. |

Round 2 discussion

[CLOSED, merged to Proposal 5-4-v3 below] Proposal 5-4-v2

For D2R transmission, not using FEC is supported.

- The reader indicates the use or not use of FEC by R2D control for D2R scheduling.

| Company | Comments |
| --- | --- |
| Spreadtrum | Support this proposal. |
| Ofinno | Okay. |
| Ericsson | Perhaps it would be clearer to write “For D2R transmission, both using and not using FEC is supported”. |

[CLOSED, merged to Proposal 5-4-v3 below] Proposal 5-5-v1

For D2R transmission, when FEC is used, further study the necessity to support other code rate than the mother code rate by puncturing.

- Note: Supporting other code rates should provide a sufficiently large performance gap in SNR.

| Company | Comments |
| --- | --- |
| Spreadtrum | Other code rates would increase complexity, so we don’t see the necessity to support other code rate than the mother code rate 1/3. Furthermore, companies propose to support a higher code rate by puncturing, while higher code rates than 1/3 can be achieved through no FEC + repetition (e.g., repetition = 2). It may cause redundant to discuss puncturing or other alternative schemes to achieve the same goal. |

      - [H] Proposal 5-4-v3

For D2R transmission,

- Not using FEC is optionally supported.

- When FEC is used, further study the necessity to support other code rate of CC than the mother code rate 1/3 by puncturing.

  - Note: Supporting other code rates should provide a sufficiently large performance gap in SNR.

- The reader indicates the use or not use of FEC, or the code rate (if other code rates than the mother code rate are supported), by R2D control.

| Company | Comments |
| --- | --- |
| FL | Based on the offline discussion, proponents who support not using FEC raised a valid point that we should make Ambient IoT system to be competitive in D2R peak data rate, e.g., larger than 640 kbps, which also shared by many companies. In addition, suggestions have been received to together discussing this with other code rates than 1/3 (note that we have already agreed in today’s online that polynomials as per TS 36.212 is reused, such that other code rates of CC can only be achieved by puncturing) |
| Nokia | OK in principle, but what does “optionally” in “optionally supported” mean here? I think “optionally” can be removed, the point is that the spec supports it. FL: Removed! |
| Huawei, HiSilicon | We support not using FEC in all cases, and find it beneficial in terms on increasing the data rates based on the deployment scenario, where devices close to the reader can be indicated to not use FEC. |
| QC | For now, it would be good enough to say “multiple coding rates” instead of no FEC. High peak rate could be achieved by shortening bit duration. What coding rates to support could be further discussed next meeting. Following modification is suggested For D2R transmission, - Not using FEC is optionally supported. - When FEC is used, further study the necessity to support code rate(s) of CC other than the mother code rate 1/3 are supported. by puncturing. - Note: Supporting other code rates should provide a sufficiently large performance gap in SNR. - FFS: coding rates to support, how to indicate - The reader indicates the use or not use of FEC, or the code rate (if other code rates than the mother code rate are supported), by R2D control. |

[CLOSED] D2R repetition

Background

In the WID, D2R supports physical layer repetition transmission. During the study item, it has been agreed that for D2R, at least block-level and bit-level repetition type 1 and type 2 are studied, with the following definition:

- Block level: All the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times.

- Bit level type 1: Each bit after CRC attachment (if used) is repeated Rbit times.

- Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times.

Summary of inputs

Based on the submitted contributions in the meeting, views for each repetition type are summarized as follows:

- Block level repetition (25): Ericsson, Futurewei, Huawei, Spreadtrum, CATT, China Telecom, CMCC, vivo, Nokia, TCL, OPPO, NEC, Xiaomi, Samsung, Fujitsu, LGE, MediaTek, Lenovo, Sharp, Honor, InterDigital, ASUSTeK, Qualcomm, Docomo, Sequans

  - From reviewing the contributions, companies have different understandings on the block level repetition:

    - Understanding 1-1: The repeated blocks are transmitted via a single PDRCH. The repetition is performed after CRC attachment and after FEC.

    - Understanding 1-2: The repeated blocks are transmitted via a single PDRCH. The repetition is performed after CRC attachment and before FEC.

    - Understanding 2: The repeated blocks can be transmitted via different PDRCH.

It seems that most companies share understanding 1 (1-1 or 1-2), but some companies (e.g., Spreadtrum, CMCC, TCL, Samsung, Fujitsu, ITL) think that understanding 2 can autonomously enable link adaptation and allow forward compatibility of single tone hopping. Regarding the repetition is performed before or after FEC, FL understands that the output bit string is the same (assuming that TBCC or HBCC is used) and can achieve same combination gain at the reader. Therefore, there is no need to clarify which implementation is performed at the transmitter side.

  - In addition, LGE proposes to support block level repetition only for small TBS to avoid large buffer size. Qualcomm proposes to support block level repetition together with polynomial sweeping. FL understands that Rel-19 considers only inventory and command use cases, where the largest payload is probability Msg 3 (carrying ID), as such information is stored in the device NVM, it does not need additional buffer to support block level repetition. For support block level repetition together with polynomial sweeping, it should wait for the progress whether other generator polynomials are considered in RAN1.

- Bit level repetition: NEC and LGE supports bit level repetition but no clear preference for type 1 or type 2.

  - Bit level type 1 (0).

  - Bit level type 2 (9): Ericsson, Futurewei, ZTE, Huawei, China Telecom (2nd preference), CMCC, Apple, Honor, Docomo

    - Ericsson, Huawei, CMCC, and Docomo think that since Manchester SFS Option 1 is supported, which is achieved by repetition within an information bit duration, therefore, bit level repetition type 2 should be supported. On the other hand, vivo thinks that supporting bit level repetition type 2 is duplicated with supporting Manchester SFS Option 1. FL understands that although bit level type 2 can achieve the same effect as Manchester SFS Option 1 with repetition within the same bit duration, it can also repeat using more time domain resources to improve the D2R performance. The reader’s behavior is not the same for the two implementations. For the former, as it achieves small frequency shift, the reader will filter out the signal at the target frequency location, but for the latter, the reader will perform soft combining. In this sense, to simplify the discussion, FL prefers to separate the discussion on bit level type 2 and Manchester SFS Option 1 and only considers bit level type 2 for D2R performance improvement. Since majority views prefer block level repletion, which can acquire more time domain diversity gain than bit level repetition type 2, FL proposes to support block level repetition.

    - ZTE proposes to implement sub-block repetition using bit level repetition type 2, where the output L bits after FEC for an input bit is as a sub-block repeated Rbit times.

- Chip level repetition (2): Xiaomi, IIT-Kanpur

  - FL understands that during the study item, chip level repetition has already been deprioritized since it is equivalent to using a lower D2R chip rate.

Round 2 discussion

Based on the inputs, the following questions and proposals are formulated.

      - [H] Proposal 6-1-v1

For D2R repetition, at least block level repetition is supported.

- FFS: Whether the repeated blocks are transmitted via a single PDRCH or can be transmitted via different PDRCHs.

- FFS: Necessity to support bit level repetition type 2 for D2R performance improvements.

| Company |  | Comments |  |
| --- | --- | --- | --- |
| FUTUREWEI |  | ok |  |
| TCL |  | Ok for this proposal |  |
| Spreadtrum |  | We support this proposal. We agree with the suggestion from FL to separate the discussion on bit level type 2 and Manchester SFS Option 1 and only considers bit level type 2 for D2R performance improvement. Obviously, block level repetition will achieve better D2R performance than bit level repetition type 2. |  |
| OPPO |  | OK |  |
| QC |  | We think block level repetition with polynomial sweeping method can provide additional coding gain in addition to time diversity and energy gain. |  |
| vivo |  | We support block-level repetition, but not with FFS bullets for now. For 1st FFS, we’d like to further understand, is main difference between single PDRCH and different PDRCH the repeation of x-amble ? If it is intention, we prefer to have a separate and more clear discussion for repetition of x-ambles. For 2nd FFS, we think bit level repetition is duplicated with small frequency shift operation. We don’t think any need of such duplication. |  |
| ZTE,Sanechips |  | We are not okay with this proposal. Based on our evaluation, the block level repetition will either increase the device memory(by storing all the repeated information) or increase the encoding latency/power consumption (by repeat the encoding procedure), thus it is not preferred. On the contrary, bit level repetition 2 can improve the performance and no need of the complexity of block-level repetition. FL: Different understandings of the requirement on memory for block level repetition is identified. R19 focuses on the inventory and command use cases, Msg 3 is probability the largest D2R transmission payload, but it carries the device ID related information, which is stored in the NVM. In this sense, the device does not need to buffer them for block level repetition, it can just read them from the NVM. |  |
| Nokia |  | OK |  |
| Panasonic |  | We support the proposal. The block level repetition offers time diversity gain and the possibility early message decoding. It should be realized without the need of physical layer buffer of the whole information bits to be repeated. |  |
| Samsung |  | Support |  |
| DOCOMO |  | Support. |  |
| HONOR |  | OK |  |
| IDCC |  | Ok. |  |
| Xiaomi |  | For block level repetition, it should be clarified that repetition is processed after CRC attachment and FEC. It is because at receiver side, the combination gain between understanding 1-1 and 1-2 are different, in which 1-2 has no combination gain at reader side. FL: To FL’s understanding, and also confirmed by some other companies’ observations, although for understanding 1-1 and 1-2, the implementation is different, but as long as we use tail biting or head biting CC, the output bit string is exactly the same, and no matter which implementation is achieved at the device side, the reader can just simple combine the first and second half of the received signal and acquires gain. |  |
| Huawei, HiSilicon |  | While we are supportive of using block level repetition for performance gains, we believe that bit level type 2 needs to be supported in order to achieve the small frequency shifts for FDMA. We do not need to compare the performance of bit and block level repetition, since they are used for different purposes and should not be down-selected. |  |
| Ericsson |  | Fine. |  |
| Fujitsu |  | Support this proposal and prefer that the block repetition could be handled as being transparent to devices to simplify both the standardization procedure and the device implementation. |  |
| LGE |  | Support |  |
| NEC |  | Ok. |  |
| Apple |  | We would like to clarify FL’s comment. We do not think 1-1 and 1-2 generate the same output bit string. It should be specified either 1-1 or 1-2. “It seems that most companies share understanding 1 (1-1 or 1-2), but some companies (e.g., Spreadtrum, CMCC, TCL, Samsung, Fujitsu, ITL) think that understanding 2 can autonomously enable link adaptation and allow forward compatibility of single tone hopping. Regarding the repetition is performed before or after FEC, FL understands that the output bit string is the same (assuming that TBCC or HBCC is used) and can achieve same combination gain at the reader. Therefore, there is no need to clarify which implementation is performed at the transmitter side.” FL: Please see my response to Xiaomi. |  |
| Round 2 discussion |  |  |  |
| FL |  | From the inputs of the first round for Question 6-2, it seems that both understandings have many proponents. FL thinks that it would be valid to keep the first FFS bullet under Proposal 6-1. This question is closed, and let’s keep discussing Proposal 6-1, which is unchanged from the 1st round, and further comments are welcomed. |  |
| Spreadtrum |  | Support this proposal Further, we only support block level repetition in R19, and understanding 1-1 is correct for us, i.e., the repeated blocks are transmitted via a single PDRCH. The repetition is performed after CRC attachment and after FEC. |  |
| Ofinno |  | We generally support the proposal. One question for clarification: does this block level repetition apply to preamble and PDRCH or only PDRCH? |  |
| FL |  | Please remember that based on the clarification online, we will further discuss the maximum number of repetitions under this agenda item in future meetings. |  |
| QC |  | We think if we have block level repetition and we no longer needs bit level repetition anymore since there is no more benefit than block level repetition. For D2R repetition, at least block level repetition is supported. - FFS: Whether the repeated blocks are transmitted via a single PDRCH or can be transmitted via different PDRCHs. - FFS: Necessity to support bit level repetition type 2 for D2R performance improvements. |  |

[CLOSED] Question 6-2-v1

Please provide your views and preference on the two understandings on block level repetition:

- Understanding 1: The repeated blocks are transmitted via a single PDRCH.

- Understanding 2: The repeated blocks can be transmitted via different PDRCHs.

| Company | Understanding 1/2 | Comments |
| --- | --- | --- |
| FUTUREWEI |  | Both understandings have benefits and drawbacks |
| TCL | Both | Two understandings could be achieved by reader’s scheduling. So we are okay for them. |
| Spreadtrum | Understanding 1 with comments | From our perspective, both of the two understandings are workable and can achieve the time diversity gain. However, it needs to be evaluated whether it is necessary to also apply repetition to the ambles, as it may increase the overhead. We prefer no repetition on the ambles to save the physical resources, i.e., the repeated blocks are transmitted via a single PDRCH. |
| OPPO | Understanding 1 | Regarding understanding 2, if there is potential R2D transmission between 2 repeated PDRCH, it seems like HARQ based retransmission, which is different with intention of repetition. |
| QC | Understanding 1 | 2 is more like retransmission which can be realized by reader’s additional query transmission. |
| vivo | Comment | Is it about 1st FFS above ? If yes, see our comments on that proposal. |
| ZTE, Sanechips | Understanding 1 | Understanding 2 is even more complicated than Understanding 1 with additional definition of the timing interval between two PDRCH, and additional OH for all the xambles in PDRCH. |
| Nokia | Open to both | For Understanding 2, some clarifications may be required: E.g. are the multiple PDRCHs transmitted immediately one after the other, or will there be a gap in time. |
| Panasonic | Both at this point | We prefer to further discuss the benefits of each alternative and then decide one. |
| Samsung | Both | The two options offer different benefits. Transmitting repetitions via a single PDRCH can be advantageous in terms of transmission time, while using different PDRCHs can be beneficial from a device availability perspective. |
| DOCOMO | Both | We prefer to clarify the difference between understanding 1 and understanding 2 more specifically. For example, if there is no time separation is assumed among PDRCHs for understanding 2, it can be equivalent to understanding 1 with midamble for each block and when CRC is separately attached to each blobk. Therefore, assumption on the following aspects should be clarified. For understanding 1, it should be clarified whether CRC is jointly attached for multiple blocks, and whether midamble is assumed for each block. |
| HONOR | Understanding 1 | The gain of understanding 2 should be justified and could be achieved under Reader control. |
| Xiaomi |  | Both understandings are workable, but the related aspects should be discussed in details. So discussion and analysis are needed to support both or down-select 1. |
| Huawei, HiSilicon | 1 | We support the use of a single PDRCH for all the repeated blocks, because otherwise, there would be an increase in the overhead due to the additional D2R preamble and other ambles and CRC for each of the repeated blocks, which does not translate into useful or necessary gains. |
| Fujitsu | 2 | We prefer understanding 2. In this way, the block repetition could be handled as being transparent to devices to simplify both the standardization procedure and the device implementation |
| LGE | Both | Okay to down-select b/w the two after further discussion. |
| NEC | Understanding 1 | Overhead of preamble and postamble(if any) can be minimized. |
| Apple |  | For understanding 1, it needs to clarify 1-1 and 1-2. |
| FL |  | From the inputs of the first round, it seems that both understandings have many proponents. FL thinks that it would be valid to keep the first FFS bullet under Proposal 6-1. This question is closed, and let’s keep discussing Proposal 6-1. |

R2D/D2R CRC

[CLOSED] Generator polynomial

Background

In the WID, D2R supports both without CRC, and with CRC as per the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC, unless RAN1 decides to use other generator polynomials by RAN1#120bis.

Summary of inputs

From reviewing the contributions submitted in this meeting, on whether to reuse generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC:

- Yes (14): Huawei, CATT, China Telecom, CMCC, vivo, Nokia, OPPO, Apple, Fujistu, LGE, Honor, InterDigital, ITL, Xiaomi

- No (2): ZTE, DCM.

  - Propose using nested polynomials for CRC-6 and CRC-16 to reduce the number of registers and XOR gates:

    - For 6-bit CRC, reuse the generator polynomial in TS 38.212, gCRC6D=D6+D5+1

    - For 16-bit CRC, the generator polynomial is gCRC16D=D16+D11+D6+  D12+D5+1

Majority companies prefer to reuse the generator polynomials as per TS 38.212 for CRC-6 and CRC-16. The only benefit of defining new polynomials (for CRC-16) is to use nested polynomials for CRC-6 and CRC-16, the reduction on device complexity is marginal at the cost that the complexity at the reader side is increased to upgrade decoding hardware. Therefore, FL proposes to go with the majority views.

Round 2 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 7-1-v1

RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC is reused for Ambient IoT Device 1.

| Company | Comments |
| --- | --- |
| FUTUREWEI | ok |
| Spreadtrum | Y |
| OPPO | Yes |
| QC | If polynomial is in nested structure device CRC HW complexity can be reduced. We propose to change polynomial for CRC16 to gCRC16D=D16+D11+D6+ D12+D5+1 |
| vivo | yes |
| ZTE,Sanechips | As we analyzed in our Tdoc, the nested polynomial can reduce the number of registers of device, it also maintain the same FAR performance as NR. |
| Nokia | OK |
| Panasonic | Yes |
| Samsung | Support. |
| DOCOMO | For 6-bit CRC, we support to reuse the polynomials in TS 38.212. For 16-bit CRC, we are fine to consider gCRC16D=D16+D11+D6+ D12+D5+1. |
| HONOR | ok |
| IDCC | Ok. |
| Xiaomi | Yes. Please add Xiaomi to the “Yes” companies as the proposal 3 in our contribution support it. FL: Implemented! |
| Huawei, HiSilicon | We support the proposal. |
| Ericsson | Fine. |
| Fujitsu | Support |
| LGE | Support |
| NEC | Yes |
| Apple | OK |
| Round 2 discussion |  |
| FL | Similar as the polynomials of CC, most companies support reusing that from TS 38.212, we should not consider other polynomials only if strong motivation is identified. The proposal is unchanged and further comments are welcomed. |

Case to use which CRC length, or no CRC

Summary of inputs

Based on the submitted contributions in the meeting, cases to use which CRC length or no CRC are discussed, and views are summarized as follows:

Cases to use CRC-6 and CRC-16

- Majority companies discuss a TBS threshold X1 to determine the use of CRC-6 and CRC-16. The selection of X is a balance of CRC overhead and undetected error probability, when TBS ≤ X1 bits, CRC-6 is used and when TBS > X1 bits, CRC-16 is used. The proposed values are,

  - X1 = 24 (15): Ericsson, Huawei, Spreadtrum, CATT, China Telecom, CMCC, vivo, Nokia, Xiaomi, MediaTek, Sharp, Honor, InterDigital, Qualcomm, Docomo

  - X1 = 57 (1): Ericsson

  - X1 = 56 (1): ZTE

  - X1 = 16 (1): CATT

  - X1 = 32 (1): Nokia

- On the other hand, some companies have different considerations:

  - Futurewei proposes to use CRC-16 for each segment if segmentation on D2R link.

  - Spreadtrum proposes that the number of bits after repetition should be used to determine the CRC length.

  - TCL, Apple, and LGE propose to consider CRC-6 or CRC-16 regarding the message or information type. For example, if L1 R2D control is supported, CRC-16 is used. Such discussion is highly related to if L1 R2D control is supported and if separate CRC is used, as summarized below.

  - Samsung proposes to stick to only one length of CRC and the device does not need to implement multiple CRC attachment schemes.

Cases to use no CRC

- Some companies suggest defining a different TBS threshold X2, when the TBS ≤ X2 bits, no CRC is used. As in RAN1, the command type may be transparent to device, and device can determine no use of CRC depending on the TBS. The proposed values are

  - X2 = 8 (2): ZTE, vivo

  - X2 = 16 (1): Huawei

  - X2 = 6 (1): Spreadtrum

- On the other hand, some other companies (e.g., Spreadtrum, CMCC, TCL, OPPO, NEC, MediaTek) propose to use no CRC depending on the message or information type. For example, for a QueryRep-like command, the functionality is to indicate the slot counter decrement, the error detection of such kind of message has no significant damage to the system, and therefore no CRC can be considered to reduce the CRC overhead.

Joint or separate CRC

- Joint/Single CRC (3): CATT, vivo, Nokia

- Separate CRC (7): Spreadtrum, OPPO, Panasonic, Apple, Samsung, LGE, Honor, ASUSTeK

  - Proposals of OPPO, Apple, Samsung, LGE, Honor, and ASUSTeK explicitly mention that if L1 R2D control information is supported, separate CRC attachment is needed. In addition, Samsung also proposes that for each PDRCH segment due to midamble insertion, CRC is attached per segment.

The discussion on joint or separate CRC is pending RAN1 discussion on whether L1 control is supported.

Round 1 discussion

Based on the inputs, the following proposal is formulated.

      - [H] Proposal 7-2-v1

When CRC is attached to a PRDCH or PDRCH transmission,

- When the block size is ≤ 24 bits, CRC-6 is used.

- Otherwise, when the block size is > 24 bits, CRC-16 is used.

- For the block carried by a PRDCH, further down-select from the following pending RAN1 discussion if L1 R2D control is supported,

  - Option 1: If L1 R2D control is not supported, the block size corresponds to the size of higher-layer payload.

  - Option 2-1: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload and L1 control, i.e., joint CRC is used.

  - Option 2-2: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload only, i.e., separate CRC is used.

    - FFS CRC attachment for L1 R2D control.

| Company |  | Comments |
| --- | --- | --- |
| FUTUREWEI |  | For the block size, it should be 57 bits as no justification for 24 bits has been provided. Can we add an FFS for every segment uses the same 16-bit CRC also. |
| TCL |  | One comment for third bullet. As we know, 941 and 943 are also discussing how to attach CRC according to FL summary. We are not sure whether CRC attachment should be discussed in 912. But at least it seems like that associated with block size. In our understanding, the third bullet could be postponed to discuss until 943 give clear conclusion. |
| Spreadtrum |  | This proposal related to three issues: - the first issue is to determine the threshold between CRC-6/16. - the second issue is whether to apply a separate or joint CRC for L1 R2D control and data. - the third issue is the definition and the information of the block size. We think it will complicate the discussion by combining three issues jointly in one proposal at this stage. And we prefer to only keep the first issue for this proposal, i.e., keep the first two sub-bullets, and postpone to discuss the second issue until the L1 R2D control information is confirmed in AI 9.4.4. As for the third issue, the definition and the information of the block size can be decided at later stage together with RAN2. - [H] Proposal 7-2-v1 When CRC is attached to a PRDCH or PDRCH transmission, - When the block size is ≤ 24 bits, CRC-6 is used. - Otherwise, when the block size is > 24 bits, CRC-16 is used. - FFS: the definition and the information of the block size. - For the block carried by a PRDCH, further down-select from the following pending RAN1 discussion if L1 R2D control is supported, - Option 1: If L1 R2D control is not supported, the block size corresponds to the size of higher-layer payload. - Option 2-1: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload and L1 control, i.e., joint CRC is used. - Option 2-2: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload only, i.e., separate CRC is used. - FFS CRC attachment for L1 R2D control. |
| OPPO |  | We propose that the value of X1 is configurable. The reader has the flexibility to determine whether CRC is applied for a specified TBS. |
| QC |  | When CRC is attached to a PRDCH or PDRCH transmission, - When the block size is ≤ 24 bits, CRC-6 is used. - Otherwise, when the block size is > 24 bits, CRC-16 is used. - For the block carried by a PRDCH, further down-select from the following pending RAN1 discussion if L1 R2D control is supported, - Option 1: If L1 R2D control is not supported, the block size corresponds to the size of higher-layer payload. - Option 2-1: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload and L1 control, i.e., joint CRC is used. - Option 2-2: If L1 R2D control is supported, the block size corresponds to the size of higher-layer payload only, i.e., separate CRC is used. - FFS CRC attachment for L1 R2D control. |
| vivo |  | Generally fine. For last sub-bullet, suggest to remove for now, after progress of separate or joint CRC in 944. |
| ZTE,Sanechips |  | We also agree with the comments by FW that the justification regarding the OH and FAR performance should be provided by the threshold determination of CRC usage before we rush to “24”, which is also similar as Proposal 7-3-v1. |
| Nokia | Agree with others that this proposal mixes too many aspects. Fine with QC’s wording. |  |
| Samsung | The threshold, i.e., 24 bits, between CRC-6 and CRC-16 needs further justification. Another aspects is that we need to discuss with the below Proposal 7-3-v1 for the case without CRC. |  |
| DOCOMO | Support the first two bullets in general but we would like to clarify any exception can be further discussed. For example, even though TBS is smaller than 24 bits, 16 bit CRC is applied to a specific message type. |  |
| HONOR | Support the first two sub-bullets |  |
| Xiaomi | OK with the proposal in principle. Similar view with spreadtrum that these issues can be discussed separately in different proposals. |  |
| Huawei, HiSilicon | We are fine with the first 2 bullets of the proposal. Regarding the size of the block carried by the PRDCH, since it depends on whether L1 R2D control signaling is supported or not, we could revisit this once we make more progress on how the control is signaled. |  |
| Fujitsu | Support 24-bit as the threshold between the applications of 6-bit CRC and 16-bit CRC. |  |
| LGE | We can support this proposal. |  |
| NEC | For Msg1 transmission, with only 16-bit information, if a CRC is needed, at least 6 bit additional overhead is attached. But when a device does not find its ID in Msg2, it will know failure of Msg1, and the CRC seems redundant. Therefore, there should be a case for no CRC. |  |
| FL | Seems that companies prefer to separate the discussion on the block size threshold and other things. The third bullets are removed for now. For the threshold 24 bits, some companies have different views, as this is the first meeting we try to determine the value, proposed values are listed for future down-selection. - [H] Proposal 7-2-v2 When CRC is attached to a PRDCH or PDRCH transmission, - When the block size is ≤ X bits, CRC-6 is used. Otherwise, when the block size is > X bits, CRC-16 is used. Down-selections from the following for X considering the balance of overhead and probability of undetected error: - Alt. 1: 24 - Alt. 2: 57 - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported). |  |

      - [H] Proposal 7-3-v1

Down-select from the following options to determine when no CRC is used,

- Option 1: A block size threshold Y. When block size is ≤ Y bits, no CRC is used. FFS Y = 8 bits or 16 bits.

  - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

- Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type.

| Company | Comments |
| --- | --- |
| FUTUREWEI | We should simplify protocol and always use CRC. |
| TCL | - We prefer option 2. If block size is used for determine no CRC attachment, some cases may be limited for RAN 2 discussion. - Whether this proposal is only for R2D case we see the listed command in option 1 and option 2 is only for R2D. - For option 2, we need to clarify whether considering QueryRep-like R2D command and ACK in RAN 2. In our understanding, AIoT paging/ Subsequent AIoT paging/R2D message and corresponding formats could be discussed in RAN 2. |
| Spreadtrum | We support Option 1 and propose Y = 6 bits considering the CRC overhead is less than 50%. In addition, Option 2 and option 1 are not contradictory and the alignment can also be achieved since the length of different message type is currently undefined by RAN2. A joint discussion for both two options can be done, for example, the decision on the threshold Y can be jointly considered when RAN2 provides a clear size for some special message type. There is no necessity to immediately make a down-selection between two options at this stage. |
| QC | Option 1 |
| vivo | We suggest to first focus on what RAN1 could decide, i.e., option 1 based on TBS. For option 2, since the message type is to be discussed in RAN2, it is more proper to wait for RAN2 progress and then RAN1 to check whether any particular message type needs special handling for CRC. |
| ZTE,Sanechips | Option 1 preferred considering the progress.Furthermore, more comments on option 2 are provide as below. - Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type. [ZTE, Sanechips] not sure why only “ Msg 2 corresponds to Msg 1 without Msg 3 scheduling information” is highlighted, what is the difference with “ Msg 2 corresponds to Msg 1 with Msg 3 scheduling information” ? For “ACK corresponds to a R2D command.”, no conclusion in RAN1 to support it. For “FFS whether and/or how to indicate the message type.”, it depends on the discussion in 9.4.4. FL: It is proposed by companies that when Msg 2 carries Msg 3 scheduling information, CRC is needed, that’s why only Msg 2 purely for ACK of Msg 1 is considered for no CRC case. |
| Nokia | OK to consider both options for now. No need to down-select at this early stage. |
| Samsung | We support Option1. When block size is small, CRC-6 can be a non-negligible overhead. |
| DOCOMO | It should be clarified first whether to support the case where CRC is not attached, e.g., depending on the message size or message type. For option 2, “QueryRep-like R2D command” has not defined yet and unclear for us. |
| HONOR | Suggest to wait for RAN2 progress to check whether Option 1 is enough. |
| Xiaomi | CRC is applied to PRDCH and PDRCH by default. CRC is used to improve the transmission reliability through error detection. Option 1 is based on only block size which may have impact on the reliability performance of crucial information transmission such as some short command during inventory and command. Option 2 is actually define the exact message types which are not attached with CRC, but the message size should also be considered and it should also be short enough to achieve transmission reliability. |
| Huawei, HiSilicon | We support Option 1. For Option 2, while we see the benefits of using the message type to determine whether CRC is used or not, the issue is that the PHY layer may not aware of the message type, and that any discussion related to this would have to be in RAN2 since they would have to consider instructing PHY to include/not use CRC. |
| Ericsson | We can support option 2 |
| LGE | Prefer to discuss both options for now. Okay to down-select later. |
| NEC | Support Option2 for Msg1 with no CRC. For Msg1 transmission, with only 16-bit information, if a CRC is needed, at least 6 bit additional overhead is attached. But when a device does not find its ID in Msg2, it will know failure of Msg1 transmission. |
| FL | The proposal is updated based on comments received: - [H] Proposal 7-3-v2 Down-select from the following options to determine when no CRC is used, - Option 1: A block size threshold Y. When block size is ≤ Y bits, no CRC is used. Down-selections from the following for Y: - Alt. 1: 16 - Alt. 2: 8 - Alt. 3: 6 - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported). - Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type. |

Round 2 discussion

[CLOSED, updated to -v3] Proposal 7-2-v2

When CRC is attached to a PRDCH or PDRCH transmission,

- When the block size is ≤ X bits, CRC-6 is used. Otherwise, when the block size is > X bits, CRC-16 is used. Down-selections from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 57

- FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

| Company | Comments |
| --- | --- |
| Spreadtrum | We support this proposal with Alt.1. |
| Ofinno | Support |
| Ericsson | Ok |
| CATT | Ok. We prefer Alt.1 |
| Nokia | OK |

      - [H] Proposal 7-2-v3

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS all segments should use the same CRC, if segmentation is applied.

| Company | Comments |
| --- | --- |
| FL | From the offline discussion, it seems that the blue highlighted part of this following proposal is pretty stable. On top of it, some further offline comments were received that, as RAN2 is discussing segmentation, at least for D2R transmission, it may come across the case that a large TB is segmented to N segments, and the previous (N-1) segments has number of information bits larger than X, but the last segment has number of information bits smaller than X. If go with the 1st sub-bullet, the performance of the whole block will be limited by that of the last segment, which seems not reasonable. Since pending RAN2 discussion, this case may or may not occur, from current stage, FL feels that it would be safer to leave it open and RAN1 can further discuss it. Therefore, an FFS bullet is further added, which is in red, and companies please check and think about it. |
| Huawei, HiSilicon | We do not think RAN1 needs to worry about the case in the FFS. If higher layers choose to have segmentation, the higher layers will know which segments have succeeded/failed, and will request re-transmission of only the failed segment(s).. |

[CLOSED, updated to -v3] Proposal 7-3-v2

Down-select from the following options to determine when no CRC is used,

- Option 1: A block size threshold Y. When block size is ≤ Y bits, no CRC is used. Down-selections from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

  - FFS the block size corresponds to the size of higher-layer payload and/or L1 R2D control (if supported).

- Option 2: Message type, e.g., Msg 2 corresponds to one and/or multiple Msg 1 without Msg 3 scheduling information, QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. FFS whether and/or how to indicate the message type.

| Company | Comments |
| --- | --- |
| Spreadtrum | For option 1, we support Alt. 3. For option 2, we support two message type: QueryRep-like R2D command triggering random access, ACK corresponds to a R2D command. Option 1 and 2 are not mutually exclusive, as explained in our first Round response. |
| Ofinno | Support. |
| Nokia | OK |

      - [H] Proposal 7-3-v3

One or both of the following options are considered to determine when no CRC is used,

- Option 1: An information bit threshold Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

- Option 2: Specified condition(s), e.g., device transmits PDRCH for Msg 1 without CRC upon receiving a PRDCH for paging transmission. FFS specified condition(s) and/or how the device determines the condition(s).

| Company | Comments |
| --- | --- |
| FL | Based on offline discussion and some further off-offline comments received, several points have been raised: - Either down-selection or both can be supported. Some comments received that at the current stage, RAN1 may not fully determine the situation, and it would be safer to consider the possibility of supporting both. For this, FL further updates to “support one or both of the following”. - Although it is valid to think from functionality perspective to use no CRC, concerns received that RAN1 may be transparent to message type, which is RAN2 work. During the discussion, companies propose that Msg 1 can have no CRC, where the device can determine based on it receiving Msg 0, or Msg 2 can have no CRC if no Msg 3 scheduling information is applied, where the device can determine based on it transmitting Msg 1. Considering these and to avoid discussing message type in RAN1, Option 2 is further updated to specified condition(s). In RAN1, we can further discuss: a) what are those condition(s); 2) how the device determines the condition(s), e.g., the higher layer provides when no CRC is used to the physical layer. |

Others

| Source | Observations / Proposals |
| --- | --- |
| TCL | Observation 10: D2R resource related Msg 1/3 repetition number/BLF could be saved if coverage is better. Observation 11: Reader sends one AIoT paging to devices that wait for inventory, and after a while, send again subsequent AIoT paging to inventory remaining device based on access successful rate. Subsequent AIoT paging could include more access resource or indication information if access successful rate is low. Proposal 16: Discuss the definition of coverage level and/or access level to save resource while meeting coverage and access reliability. |
| Panasonic | Proposal 4: Whether the case that the D2R size can be different from the reader's indicated D2R size is supported or not should be concluded in RAN2. |
| NEC | Proposal 7: When applying small frequency shift, the density of ambles, at least midamble, is increased by small frequency shift factor R. Proposal 8: When applying small frequency shift, the number of effective chips after small frequency shift should be multiples of R between nearby ambles. Proposal 9: To avoid increase of amble overhead, the number of chips of each amble should be multiples of R. |
| Lenovo | Proposal 5: Discuss number of power state supported by device 1 as it important to understand whether content such as timers, inventoried state can be preserved: - 2 states – ON, OFF - 3 states – ON, Memory retention state, OFF Proposal 6: Discuss whether the device can goto memory retention state within inventory round to save power Proposal 7: Discuss how to limit device participating in the inventory round, random access round depending on its availability of stored energy Proposal 8: Support energy aware slot selection in an inventory round as opposed to random slot selection. |

Collection of agreements

RAN1#120

Agreement

For R2D Manchester line code, the specification is according to TR 38.769 as follows:

- A chip corresponds to one modulated symbol.

- For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

- Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

Agreement

For D2R repetition, block level repetition from TR38.769 is supported.

Agreement

RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and coding rate of 1/3 from TS36.212 are reused as the mother code for Ambient IoT.

Agreement

RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC are reused for Ambient IoT.

Agreement

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS impact of segmentation, if any

  - Note: impact may not be in RAN1

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106.

AI 9.4.2

- R1-2500034	Coding aspects for Ambient IoT	Ericsson

- R1-2500045	Discussion on coding aspects for A-IoT physical channel	FUTUREWEI

- R1-2500125	Discussion on Ambient IoTcoding	ZTE Corporation, Sanechips

- R1-2500154	Physical channel design on channel coding	Huawei, HiSilicon

- R1-2500170	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT	Spreadtrum, UNISOC

- R1-2500225	Ambient IoT channel coding and small frequency shift	CATT

- R1-2500260	Discussion on physical channels design about line coding, FEC, CRC, repetition aspects for Ambient IoT	China Telecom

- R1-2500288	Discussion on coding aspects of physical channel design	CMCC

- R1-2500350	Discussion on line coding, FEC, CRC and repetition for A-IoT	vivo

- R1-2500395	AIoT Physical channels design - line coding, FEC, CRC, repetition aspects	Nokia

- R1-2500425	Discussion on other aspects for Ambient IoT physical design	TCL

- R1-2500451	Discussion on physical channels design for A-IoT	OPPO

- R1-2500487	Discussion on Physical Channel Designs for A-IoT	Panasonic

- R1-2500611	Physical layer design – line coding, FEC, CRC, repetition aspects	NEC

- R1-2500733	Discussion on line coding, FEC, CRC and repetition aspects for Ambient IoT	Xiaomi

- R1-2500779	On coding aspects for Ambient IoT	Apple

- R1-2500851	Views on Physical channels design – line coding, FEC, CRC, repetition aspects	Samsung

- R1-2500894	Discussion on general aspects of physical layer design of A-IoT	IIT, Kharagpur

- R1-2500937	Discussion on coding aspects	Fujitsu

- R1-2500950	A-IoT PHY layer design - line coding, FEC, CRC and repetition aspects	LG Electronics

- R1-2501017	A-IoT - PHY line coding, FEC, CRC, repetition aspects	MediaTek Inc.

- R1-2501072	Discussion on the Ambient IoT physical layer design aspects for Line coding, FEC, CRC, Repetition 	Lenovo

- R1-2501093	Discussion on coding aspects	Sharp

- R1-2501109	Discussion on Physical channels design for Ambient IoT-other aspects	HONOR

- R1-2501118	Coding aspects of physical channel design for Ambient IoT	InterDigital, Inc.

- R1-2501124	Discussion on A-IoT Physical channels design	ASUSTeK

- R1-2501157	Physical channels design – line coding, FEC, CRC, repetition aspects	Qualcomm Incorporated

- R1-2501203	Discussion on coding and CRC aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

- R1-2501269	Coding aspects for Ambient IoT	ITL

- R1-2501311	Discussion on other aspects of Phy design for AIoT	IIT Kanpur

- R1-2501373	Discussion on Ambient IoTcoding	ZTE Corporation, Sanechips

AI 9.4.1 (on D2R chip length/bandwidth)

- R1-2501372	Discussion on Ambient IoT modulation	ZTE Corporation, Sanechips

- R1-2500224	Ambient IoT physical channel design and modulation	CATT

- R1-2500450	Discussion on modulation aspects of A-IoT	OPPO

- R1-2501016	A-IoT - PHY modulation aspects	MediaTek Inc.

- R1-2501092	Discussion on modulation aspects	Sharp

- R1-2501202	Discussion on modulation aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

AI 9.4.4 (on SFS factor/FDMA candidate resources, repetition)

- R1-2500453	Discussions on other aspects of A-IoT communication	OPPO

- R1-2500481	Discussion on multiple access, scheduling and timing aspects for A-IoT	Tejas Network Limited

- R1-2501159	Discussion on other aspects for Rel-19 Ambient IoT	Qualcomm Incorporated

- R1-2501347	Discussion on aspects for Ambient IoT	Sequans Communications

- Appendix: Summary of proposals

R2D line code

| Source | Observations / Proposals |
| --- | --- |
| China Telecom | Proposal 1: For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}. |
| TCL | Observation 1: LP-WUS considers overlaid OFDM sequences on OOK symbol to reduce detection overhead at LP-WUR side and enhance data rate. Observation 2: Two overlaid methods based on Manchester coding could be used for AIoT paging/R2D message/Msg 2 transmission. Proposal 1: Consider overlaid binary sequences on OOK-4 ‘ON’ chip to transmit AIoT paging/R2D message/Msg 2 based on Manchester coding. |
| Honor | Proposal 1: R2D transmission supports only the Manchester line coding and bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}. |
| ITL | Proposal 1: In Manchester coding, the encoding scheme maps an information bit ‘0’ to ‘01’ and an information bit ‘1’ to ‘10’. |

D2R line code and associated small frequency shift

#1 D2R line code and SFS options

| Source | Observations / Proposals |
| --- | --- |
| Ericsson | Observation 1: OOK modulation without Manchester encoding has several drawbacks, such as synchronization issues, threshold sensitivity, and susceptibility to noise. Proposal 1: Support Manchester line code as specified in TR 38.769 for both R2D and D2R with OOK modulation. Observation 4: Option 2 is applicable to cases with or without Manchester encoding, whereas Option 1 is limited to cases with Manchester encoding. Proposal 6: For a small frequency shift: - If D2R line coding is adopted, Option 1 can be supported. - If D2R line coding is not adopted, Option 2 should be supported. |
| Futurewei | Observation 11: The Manchester encoding scheme requires twice the bandwidth compared to the option no line code. Observation 12: If the reader is a D2R coherent receiver, then the option no line code can be used; otherwise, Manchester encoding is applied for a D2R non-coherent receiver. Observation 13: If the number of Manchester codeword repetitions is the same as the number of square-wave periods (for the no line code technique), then the square-wave period is equal to 2×chip length. Observation 14: For small frequency shifts, multiplying Manchester-encoded waveforms by a square wave requires twice the bandwidth compared to Manchester codeword repetitions or multiplication of no line code with a square wave. Proposal 7: For small frequency shifts in D2R transmission, the Manchester codeword repetition and no line code (multiplying by a square wave) techniques are supported. Proposal 8: In D2R transmission, reader indicates only one small frequency-shift parameter to devices, either R for Manchester codeword repetition or RS for no line code. |
| ZTE | Proposal 1: Manchester line code is proposed for D2R. Observation 1: When bit duration is 33.33us, option 1 has better performance compared with option 2. When bit duration is 133.33us, option 1 only shows worse performance when the number of devices is larger than 2 @ 1%BLER. Proposal 2: Small frequency shift using Manchester code option 1 can be considered as starting point. |
| Huawei | Observation 18: The time domain waveform generated using Manchester code option 1 for small frequency shifts is equivalent to that of the no line code case using square wave modulation. Observation 19: For Manchester coding option 1, the transmission bandwidth does not change with variations in the value of R. Observation 20: For R=1, the time domain waveform generated using Manchester code option 2 for small frequency shifts is equivalent to that of Manchester code option 1 and the no line code case using square wave modulation. Observation 21: For R>1, the time domain waveform generated using Manchester code option 2 for small frequency shifts introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit when compared with Manchester with option 1. Observation 22: For Manchester coding option 2 and R=1, the waveform is equivalent to that of Manchester code option 1 and the no line code case. Observation 23: For Manchester coding option 2 and R>1, the one-side transmission bandwidth (main lobe) of the D2R signal in Manchester Option 2 is twice of that in Option 1 and hence, the frequency spectrum using Manchester line codes by option 1 is more concentrated than option 2 Observation 24: The performance of FDMA with Manchester option 1 and option 2 under SFO up 105 between multiple devices at target BLER 10-1 is comparable. Proposal 7: For small frequency shifts using D2R line codes, Manchester code Option 1 is supported, since the spectrum utilization of Option 1 is twice that of Option 2, and the performance gap between Option 1 and Option 2 is marginal. |
| Spreadtrum | Observation 1: In case of large SFO, the embedded clock information in Manchester line code is beneficial for reader to adjust timing for demodulation. Proposal 1: Manchester line code is used for D2R transmission. Proposal 8: Supporting Option 1 of Manchester line code to achieve SFS. |
| CATT | Observation 2: With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10%. Observation 3: Compared with CC code only, CC concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm. Observation４: With same SFO assumption and channel coding scheme, BLER performance of BPSK has 6dB gain over that of OOK modulation in AWGN channel. Proposal 5: D2R Manchester line code should not be supported for PDRCH transmission. Proposal 6: For small frequency shift in D2R, the time duration corresponding to each information bit should be associated with an integer number of square waves periods. Proposal 7: Support D2R small frequency shit by XNOR/multiplication operation in the square wave modulation corresponding to the small frequency-shift. Observation 5: SFS factor K could be used for A-IoT Device 1 to determine square wave period. Observation 6: Compared with the small frequency shift without line code (Option 1), the small frequency shift with Manchester line code (Option 2) has poor FDM multiplexing performance, as Option 2 requires twice transmission bandwidth of option 1 per device. Proposal 8: Small frequency shifts in D2R using a square-wave modulation and no line codes should be specified for Rel-19 A-IoT. |
| China Telecom | Proposal 2: For Manchester line codes with a repetition number R ≥ 1, support Option 1 to operate the small frequency shifts. |
| CMCC | Proposal 1: For D2R transmission, support Manchester line code, which maps bit 0 to chip {10} and bit 1 to chip {01}. Observation 1: For Small frequency shifts with Manchester line codes, Option 1 has narrower spectrum than Option 2, and less noise is filtered out at the receiver side. Observation 2: For D2R Small frequency shifts for OOK and BPSK, the frequency spectrums of different R {1,2,4,6,8,10} in Option 1 do not overlap, while the frequency spectrums of adjacent R {1,2,4,6,8,10} in Option 2 are largely overlap. Proposal 2: For D2R Small frequency shifts for OOK and BPSK, - For SFS with Manchester line codes, option 1 is supported. |
| vivo | Proposal 7: For D2R, support Manchester line code with small frequency shift option 1 as defined in TR 38.769. |
| Nokia | Proposal 6: RAN1 supports Manchester coding scheme for D2R transmissions. |
| TCL | Observation 4: It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation. Proposal 5: Down select option 2 of D2R line code and support option 1 or no D2R line code (e.g., by using a square-wave corresponding to the small frequency-shift) |
| OPPO | Proposal 6: Manchester line code is supported to D2R transmission. Proposal 7: Regarding small frequency shift operation for Manchester line codes, the following option (corresponding to option 1) is supported - For repetition of the codewords within the same time duration Tb corresponding to an information bit (Option 1), each Manchester codeword is repeated by a codeword repetition number R, where R = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 * chip length). |
| NEC | Proposal 1: Specify a unified design that treat Manchester line coding as a small frequency shift of transmission of no line coding. |
| Xiaomi | Proposal 1: D2R transmission supports Manchester line code. Proposal 2: In Ambient IoT system, (Option 1) Manchester codeword repetition is applied for small frequency shift of D2R transmission. |
| Apple | Observation 1: Manchester line coding with option 1 is equivalent to no line coding with a square wave modulation. Proposal 8: For D2R transmission with frequency shift R/Tb, where Tb is the time duration to transmit one coded bit, and R is the number of square wave period within Tb, bit 1 is transmitted by 2R OOK chip as [0 1 0 1 …] and bit 1 is chips [1 0 1 0 …]. For BPSK, bit 1 is transmitted by 2R BPSK chip as [-1 1 -1 1 ….] and bit 0 is transmitted as [1 -1 1 -1 …]. |
| Samsung | Proposal 1: For D2R transmission, the Manchester line code in TR 38.769 is to be the baseline. Observation 1: For performance of FDMA in TDL-D channel, 2-user and 3-user D2R multiplexing, - Manchester line codes option 1 and 2 have negligible performance loss after applying D2R FDMA. - With 20 bits payload size: - Compared with single user case with Manchester encoding, in 2-user case, Manchester line codes option 1 and option 2 has ~0.6dB SNR loss at 10% BLER and 1% BLER - Compared with single user case with Manchester encoding, in 3-user case, Manchester line codes option 1 has ~0.8dB SNR loss at 10% BLER and ~1.3dB SNR loss at 1% BLER, Manchester line codes option 2 has ~0.8dB SNR loss at 10% BLER and ~0.9dB SNR loss at 1% BLER. - With 96 bits payload size: - Compared with single user case with Manchester encoding, in 2-user case, Manchester line codes option 1 has ~0.4dB SNR loss at 10% BLER and 1% BLER, Manchester line codes option 2 has ~0.3dB SNR loss at 10% BLER and ~0.4dB SNR loss at 1% BLER. - Compared with single user case with Manchester encoding, in 3-user case, Manchester line codes option 1 has ~0.4dB SNR loss at 10% BLER and ~0.6dB SNR loss at 1% BLER, Manchester line codes option 2 has ~0.3dB SNR loss at 10% BLER and ~0.4dB SNR loss at 1% BLER. Observation 2: For performance of FDMA in TDL-A channel, 2-user and 3-user D2R multiplexing, - Manchester line codes option 1 and option 2 have medium performance loss after applying D2R FDM. - With 20 bits payload size: - Compared with single user case with Manchester encoding, in 2-user case, Manchester line codes option 1 has ~4.3dB SNR loss at 10% BLER - Compared with single user case with Manchester encoding, in 3-user case, Manchester line codes option 1 has ~7.2dB SNR loss at 10% BLER. - With 96 bits payload size: - Compared with single user case with Manchester encoding, in 2-user case, Manchester line codes option 1 and option 2 has ~4dB SNR loss at 10% BLER. - Compared with single user case with Manchester encoding, in 3-user case, Manchester line codes option 1 and option 2 have ~7.4dB SNR loss at 10% BLER. Observation 3: For Option 1 and Option 2 for a small frequency shift in D2R from TS 38.769, Option 1 can be a special case of Option 2. Proposal 2: Support Option 2 in TS 38.769 to achieve a small frequency shift for D2R. |
| Fujistu | Proposal 4: No line code is adopted for D2R transmission. |
| LGE | Proposal 3: For D2R small FS, support no line code with square wave multiplication and Manchester line code Option 2. Proposal 4: Based on the chip duration and the FS factor provided by a reader, the amount of frequency shift for D2R can be determined. - FFS how the chip duration and the FS factor are provided by a reader (e.g., by L1 R2D control, R2D payload) |
| Lenovo | Proposal 2: Confirm Manchester line coding for PRDCH and PDRCH with bit-to-chip mapping of 0.5 code rate: bit 0→chips{10}, bit 1→chips{01} |
| Sharp | Proposal 2: Manchester line code is always used for a D2R transmission. Proposal 3: Regarding the support for small frequency shift of a D2R transmission, only Manchester line code Option 1 is supported, i.e. each Manchester codeword is repeated by a codeword repetition number R, within the same time duration Tb corresponding to an information bit, where R = Tb/(2 × chip length), such that the amount of small frequency shift in Hz is R/Tb = 1/(2 × chip length). |
| Honor | Proposal 2: RAN1 support Manchester line code for D2R and Manchester codeword repetitions (option1) for small frequency shift. |
| InterDigital | Proposal 1: Only Manchester line coding is supported for D2R. Observation 1: At 10-1 BLER, Option 2 is slightly better than Option 1 with OOK modulation. Observation 2: At 10-1 BLER, Option 1 and Option 2 perform similarly with BPSK modulation. Proposal 2: Support Option 2 for small frequency shift with Manchester line coding. |
| Qualcomm | Observation 1: The first half of waveform generated by Manchester coding Option 2 is the inverted version of that by Option 1. Observation 2: The second half of waveform generated by Manchester coding Option 2 is the same as that generated by Option 1. Observation 3: D2R square-wave based method and D2R Manchester coding Option 1 generates identical waveforms. Proposal 1: Deprioritize D2R Manchester coding Option 2. Proposal 2: For D2R, adopt Manchester coding Option1 and square-wave based method based on following waveforms for different R values e.g., R=[1,2,4,8, … Rmax]. Waveform amplitude high/low is mapped to different impedance set {Z1,Z2} and {Z1, Z3} for OOK and BPSK respectively. FFS Rmax |
| Docomo | Proposal 1: For Manchester encoding, support the bit-to-chip mapping defined in TR 38.769; bit 0→chips{10}, bit 1→chips{01}. Proposal 7: For small frequency shift scheme, one option should be down-selected considering its spectrum characteristics, performance of small frequency shift and performance combined with FEC. |
| ITL | Proposal 2: For the small frequency shifts of D2R transmission, support Manchester line codes with a repetition number R ≥ 1 with Option 1. |
| IIT-Kanpur | Proposal 1: Manchester coding should be considered for the D2R transmission. Proposal 3: The following bit mapping patterns should be considered to achieve the small frequency shift of the D2R transmission. For coded bits: - 1/2 code rate: bit 0→ chips{01}, bit 1→chips{10} - 1/4 code rate: - A: bit 0→chips{0011}, bit 1→chips{1100} - B: bit 0→chips{0101}, bit 1→chips{1010} - 1/8 code rate: - A: bit 0→chips{00001111}, bit 1→chips{11110000} - B: bit 0→chips{01010101}, bit 1→chips{10101010} |

#2 D2R chip length and bandwidth

| Source | Observations / Proposals |
| --- | --- |
| ZTE | Under 9.4.1 Proposal 11: D2R chip duration can be defined as the time duration of a modulated symbol. Proposal 12: For D2R transmission without double sideband modulation, the chip duration (Tchip) is associated with D2R transmission bandwidth (Btx,D2R), i.e. Tchip=2/Btx,D2R . Observation 13: As captured in TR 38.796, the bandwidth of D2R transmission with small frequency shift is not changed for different R values, which always equals to 2/Tb for double sideband modulation. Proposal 13: Define D2R bandwidth/bit duration to facilitate at least the indication of D2R scheduling information. Proposal 14: Potential impacts of FDM, data rate and clock acquisition on the definition of D2R bit duration Tb, chip duration Tchip and transmission bandwidth Btx,D2R in Table 1 should be considered as the starting point. Table 1 Potential impacts of aspects on D2R Tb, Tchip and Btx,D2R Aspects; Potential impacts on bit/chip duration and corresponding Btx,D2R. FDM; Small frequency shift is used for the purpose of FDMA of D2R. Chip/bit duration According to the formula Tchip =Tb/(2*R), the minimum chip duration is limited by the maximum number of repetition of line-code codewords (R), i.e. maximum range of small frequency shift, for a given transmission bandwidth. Transmission bandwidth Number of multiplexed devices is associated with the minimum transmission bandwidth. In our views, minimum transmission bandwidth of 15 kHz is preferred to support more FDM-ed devices in bad coverage scenarios.. Data rate; In RFID 18000-C specification, the data rate is in a range of 20 to 640kbps, while A-IoT D2R can be designed to support a same or larger range of data rate than RFID. The potential values of raw data bit duration, as well as the chip duration, corresponding to different data rate for the case of 2SB should be considered. Firstly, the potential values of Btx,D2R corresponding to different data rate for the case of 2SB should be considered. Secondly, maximum 2SB transmission bandwidth in UHF RFID is 2.56 MHz. In order to achieve higher transmission efficiency, the maximum transmission bandwidth should not be less than 2.56 MHz.. Clock acquisition; Chip/bit duration The D2R clock calibration is based on the clock acquisition part in R2D time acquisition signal. Because the R2D time acquisition signal is based on R2D M values for DFT-s-OFDM-based OOK waveform. Therefore, the chip duration of R2D for different M values, i.e. {66.67, 33.34, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08} us for M values of {1, 2, 4, 6, 8, 12, 16, 24, 32}, can be considered for the D2R bit duration, as well as D2R chip duration. Transmission bandwidth The D2R clock calibration is based on the clock acquisition part in R2D time acquisition signal. Thus, it is helpful that the D2R chip length is multiple times of R2D chip length and integer of samples are counted for D2R chip and R2D chip. This design principle can avoid the D2R chip error introduced by the non-integer samples which can also lead to the sampling frequency error. In our views, maximum transmission bandwidth should consider the limitation of device clock frequency (e.g. 1.92MHz). Moreover, maximum transmission bandwidth should also consider the minimum D2R bit/chip duration.. Proposal 15: It is proposed that the bit duration Tb and transmission bandwidth Btx,D2R defined by option 2 in Table 3 can be considered as starting point for D2R. Table 2. Bit duration Tb and the corresponding Btx,D2R of Option 1 Index; Transmission bandwidth (double sideband); Bit duration Tb. 0; 15kHz; 266.67 µs. 1; 30kHz; 133.33 µs. 2; 60 kHz; 66.67 µs. 3; 120 kHz; 33.33 µs. 4; 240 kHz; 16.67 µs. 5; 480 kHz; 8.33 µs. 6; 960 kHz; 4.17 µs. 7; 1920 kHz; 2.08 µs. 8; 2880 kHz; 1.39 µs. Table 3. Bit duration Tb and the corresponding Btx,D2R of Option 2 Index; Transmission bandwidth (double sideband); Bit duration Tb. 0; 15kHz; 266.67 µs. 1; 30kHz; 133.33 µs. 2; 60 kHz; 66.67 µs. 3; 180 kHz; 22.22 µs. 4; 360 kHz; 11.11 µs. 5; 720 kHz; 5.56 µs. 6; 1440 kHz; 2.78 µs. 7; 2880 kHz; 1.39 µs. |
| Huawei | Proposal 8: The following D2R information bit lengths is supported: - Based on 2.4 MHz sampling clock, the D2R information bit lengths are {266.67μs, 133.33μs, 66.67μs, 26.67μs, 13.33μs, 6.67μs, 3.33μs, 1.67μs }. |
| Spreadtrum | Proposal 9: For the Btx,D2R of the D2R transmissions, 2SB modulation is adopted for SFS by Manchester with option 1. |
| CATT | Proposal 7: The D2R chip duration should be defined as the duration of each channel coding bit, which is related to the chip rate from the physical layer transmission perspective. Proposal 8: The square-wave chip length should be further defined and with the following relationship, when the SFS is considered. D2R chip length = 2*R* square-wave chip length, where R is the number of square-wave periods corresponding a D2R chip. Proposal 9: For Device 1, the chip rate of PDRCH should not be higher than 240kbps with M=16. Observation 4: If the maximum occupied bandwidth of PDRCH is consistent with that of PRDCH, the FDMA would not be supported or would only support a limited number of SFS factors, when the chip rate is higher than 120kbps. |
| CMCC | Proposal 13: The D2R transmission bandwidth supports values of integer multiple(s) of 15 kHz. |
| vivo | Proposal 8: For D2R, support small frequency shift values chosen from {30, 45, 60, 90, 120, 180, 240, 360, 480, 720} KHz and codeword repetition number R {1,2,4,8,16,32,64,128} with potential down-selection up to Y, where Y=[4], avoiding interference from odd harmonics and providing sufficient gap among adjacent frequency resource occasions. The small frequency shift parameters, e.g., combination of bit or chip duration and R value is indicated/derived by scheduling information carried by PRDCH. |
| TCL | Proposal 12: Consider the follow equation as baseline to further study guard band of D2R signal: ⑦=(②-1/(chip duration×frequency-shif factor))/2±BWSFOmax, where BWSFOmax represents the occupied BW due to SFO 1）Btx, D2R; 2）Bocc, D2R; 3) Btx, CW; 4）Bocc, CW; 3)Gap between two single-tone; 6)Guard BW for CW; 7)Guard BW for D2R signal; 8）BAIoT, system; 9）Guard BW between AIoT and coexistence system |
| OPPO | Under AI 9.4.1 Proposal 9: D2R transmission at least supports the same candidate chip rate/chip length as the R2D transmission, e.g. 14kcps, 28kcps, 56kcps, 112kcps. |
| MediaTek | Under AI 9.4.1 Proposal 3: For D2R transmission, transmission bandwidth of RE level and RB level are supported. |
| Sharp | Under AI 9.4.1 Proposal 8: A set of D2R bandwidths (including a number of values as integer multiple of SCS and other values as integer multiple of PRBs) is pre-defined for backscattering transmissions in Rel-19. |
| InterDigital | Proposal 3: D2R bit duration is determined as a function of the R2D chip duration. - The function is of the form: D2R bit duration = 2-n × R2D chip duration |
| Qualcomm | Proposal 3: D2R supported data rates (in kbps) are X, 2X, 3X, … , where X is defined as nominal minimum data rate, X = [10]. |
| Docomo | Under AI 9.4.1 Proposal 13: For D2R, chip duration definition should be one of a pre-defined set of pulse time durations. - FFS: candidate values - FFS: how to determine the chip length for each D2R transmission - E.g., D2R chip length is implicitly indicated based on the transmission bandwidth, small frequency shift factor, repetition number of line coding codeword/square wave period. - E.g., set of D2R chip lengths are pre-defined and one of them is explicitly indicated. Proposal 14: For D2R, discuss the candidate chip length considering at least small frequency shift factor, the target peak rate for D2R and decoding performance. |

#3 SFS factor R and frequency resources for FDMA

| Source | Observations / Proposals |
| --- | --- |
| ZTE | Proposal 3: The minimum value of R for small frequency shift should be 2 to mitigate self-interference. Observation 2: Due to some adjacent DSBs not satisfying a 2-fold relationship, while R is a power of 2, Table 5 has more chip durations than Table 4. Proposal 4: The maximum number of multiplexed devices is determined based on the Bit time duration, with 6 devices as the starting point for the maximum number of multiplexed devices. Proposal 5: The indicated R(s) for small frequency shift should be discussed, wherein the aspects such as the transmission bandwidth/chip duration, number of multiplexed devices, interference between devices should be considered. |
| Huawei | Proposal 9: RAN1 to select R values in the range from R = 1 … 160, with the maximum value of R FFS. Proposal 10: The values of R include at least some satisfying R=2n, and at least some satisfying R≠2n, e.g. R=2n. - The total set of values of R is chosen to balance overhead with FDMA scheduling flexibility. |
| CATT | Observation 7: Intra-channel interference due to frequency overlapping among D2R channels caused by small frequency shift using square wave modulation, could degrade BLER performance of D2R transmission significantly. Observation 8: D2R transmissions with SFS factor K=2n have similar BLER performance without FDM performance degradation; however, the number of available D2R channels is reduced tremendously. This will reduce the spectrum utilization. Observation 9: Compared to D2R channel(s) with SFS factor K, the BLER performance of D2R channels with SFS factor 3*K, 5*K and 7*K would be degraded by about 3dB, 1.5 dB and 0.8dB, respectively without interference cancellation at the reader. However, reader can perform interference mitigation with simple multi-tap frequency domain filtering to ensure the BLER performance. |
| CMCC | Observation 3: The candidate frequency resources for FDMA correspond to aspects, including the D2R baseband chip rate, the small frequency shift factor, the maximum D2R frequency resources allocated for A-IoT, device SFO, the avoided harmonic components. Observation 4: With the decrease of the baseband chip rate, device SFO and avoided harmonic components, and with the increase of the maximum allocated frequency resources, the available frequency resource candidates and the multiplexing capability increases. Observation 5: FDMA can be achieved by selecting different SFS R value at the same baseband chip rate, where the selected R values should consdier to avoid FDMed harmonic interference. Proposal 3: To indicate the candidate frequency resources for FDMA to devices, consider the following alternatives: - Alt. 1: The candidate frequency resources are explicitly indicated via R2D control information. - Alt. 2: The candidate frequency resources are specified. - Alt. 3: The candidate frequency resources are obtained by rules. |
| vivo | Proposal 8: For D2R, support small frequency shift values chosen from {30, 45, 60, 90, 120, 180, 240, 360, 480, 720} KHz and codeword repetition number R {1,2,4,8,16,32,64,128} with potential down-selection up to Y, where Y=[4], avoiding interference from odd harmonics and providing sufficient gap among adjacent frequency resource occasions. The small frequency shift parameters, e.g., combination of bit or chip duration and R value is indicated/derived by scheduling information carried by PRDCH. |
| TCL | Proposal 6: Consider the impact of M value and RB allocation on repetition factor of D2R Manchester line code Observation 9: Device may select unavailable sub-channel if reader do not indicate that before, which brings resource waste and reduce the reliability of inventory. Proposal 14: Consider backoff scheme for improving the reliability of inventory for D2R FDMA because of sideband modulation/harmonic/intermodulation interference, etc. |
| OPPO | Observation 6: FDMA of D2R transmissions for device type 1 would be based on repetition of line codes, where the number of repetitions that can be supported for 15kHz are {1, 2, 4, 8, 16, 32, 64, 128}. Observation 7: Since the transmission bandwidth exists frequency overlap for repetition of 1 and 2, repetition of 2 should not be supported. Observation 8: The guard band for FDMA of D2R transmission would not need to be defined in RAN1 and RAN4 (i.e., for backscattering device 1 and 2a), since small frequency shifts are achieved by line coding repetition and as long as the repetition numbers {1, 4, 8, 16, 32, 64, 128} would not cause unwanted interferences to each other (e.g., due to harmonics, inter-modulation). Proposal 5: The supported set of codeword repetition number (R) for different D2R transmission bandwidths should be: - For 15kHz D2R bandwidth, R = {1, 4, 8, 16, 32, 64, 128} - For 30kHz D2R bandwidth, R = {1, 4, 8, 16, 32, 64} - For 60kHz D2R bandwidth, R = {1, 4, 8, 16, 32} - For 120kHz D2R bandwidth, R = {1, 4, 8, 16} - For 240kHz D2R bandwidth, R = {1, 4, 8} - For 480kHz D2R bandwidth, R = {1, 4} For 960kHz D2R bandwidth, R = {1} |
| Tejas | Under AI 9.4.4 Proposal 6: The available frequency domain resources (Y) depends on the transmission bandwidth of devices, receiver bandwidth of the reader, and the guard band. Considering channel bandwidth is same as the receiver bandwidth of the reader and the guard band is minimum 20% of the receiver bandwidth, the frequency domain resources for different values of D2R chip length are: Chip rate (kcps); Chip length (µs); D2R transmission bandwidth (kHz); No. of frequency domain resources for D2R receiver bandwidth without guard band. 180 kHz; 360 kHz; 1.08 MHz. 14; 71.4; 15; 9; 19; 57. 28; 35.7; 30; 4; 9; 28. 56; 17.85; 60; 2; 4; 14. 112; 8.92; 120; 1; 2; 7. |
| Apple | Proposal 9: For D2R transmission with frequency shift R/Tb, to maintain orthogonality with large sampling offset, alternative frequency shift can be used. |
| Sharp | Proposal 4: Each D2R transmission is associated with a small frequency shift factor which is allocated in the D2R grant conveyed in the scheduling R2D transmission. - The small frequency shift factor is one of the small frequency shift factors pre-defined for the D2R bandwidth of the D2R transmission. Observation 2: A maximum amount of small frequency shift value (or, a corresponding smallest D2R chip length) should be discussed and decided in RAN1, taking into account the processing capability of Device 1. Observation 3: For a number of FDM’ed D2R transmissions, the respective small frequency shift factors (and corresponding D2R frequency resources) should be allocated such that they are spaced as far apart as possible. |
| InterDigital | Proposal 4: The chip durations corresponding to small frequency shifts are computed as Tc = 2-i× R2D chip duration, i= n+1, n+2, …nmax. |
| QC | Observation 4: Frequency shift FS (Hz)= R * data rate (bps), where R is frequency shift factor, data rate is baseband information bit rate before frequency shift is applied. Under AI 9.4.4 Observation 4: - Considering large SFO, Df=BW and frequency shifts of R*Df with R={2, 4, 8, 16,…} can be considered to have sufficient guard band and less harmonics interference for D2R FDMA. Observation 4: - Large SFO generates inter-device interference for D2R FDMA transmissions and degrades the performance of D2R even if reader can ideally estimate the SFOs of FDMA devices. - With SFO of 105 ppm, inter-device interference caused by SFO causes BLER floor for some frequency shifts and therefore, the devices with the frequency shifts cannot achieve BLER = 1% - With SFO of 104 ppm, the performance loss due to inter-device interference caused by SFO is limited to 1 – 2 dB for BLER = 1% - Comparing two FDMA patterns, - The FDMA pattern of R={1, 4, 8, 16} does not provide better performance than that of R={2, 4, 8, 16} due to more severe harmonics of the signals transmitted at R=1 under fading channels. - The D2R at R=1 is more sensitive to the CW interference received at receiver than that of R=2. Observation 5: - It is possible to increase the data rate at higher frequency to improve the spectrum efficiency. - For SFO of 104 ppm, the device using 2x data rate at R=8 and the device using 4x data rate at R=16 can achieve 1%. - For SFO of 105 ppm, the D2R using 2x data rate at R=8 and the D2R using 4x data rate at R=16 show error floor and can barely achieve 1%. Observation 6: - To enhance the robustness to the SFO and improve the D2R performance, a subset of the frequency shift candidates can be selected, or the TBS may be restricted. Proposal 2: For D2R FDMA, - To have sufficient guard band for large SFO, the frequency shifts at R*Df with Df=BW and R={2, 4, 8, 16, …} can be used. - FFS: selection/indication of the FDMA frequency shifts - To enhance spectrum efficiency, scalable data rates for higher frequency shifts of D2R FDMA can be considered to further improve the spectrum efficiency. |
| Docomo | Proposal 5: For D2R, support small frequency shift at least for D2R transmission of Msg1. Proposal 6: For the small frequency shift for D2R, discuss how to determine the Tb, R and chip length. - Opt.1: One fixed value is specified - Opt.2: Explicitly indicated via R2D from set of values - Opt.3: Implicitly indicated, i.e., calculated based on the other parameters. - FFS: Detailed values for each of Tb, R and chip length. |

D2R FEC

#1 Generator polynomial

| Source | Observations / Proposals |
| --- | --- |
| Ericsson | Proposal 2: D2R supports only convolutional coding with generator polynomials as specified in TS 36.212. |
| Futurewei | Proposal 3: For the Ambient IoT D2R link, support the generator polynomials as per TS 36.212 for the convolutional code. |
| ZTE | Proposal 6: Consider one of following convolutional code polynomials alternatives: Alt. 1: Mother code rate of 1/4 with constraint length 7, polynomials are [133, 171, 165, 123] in octal. Alt. 2: Mother code rate of 1/4 with constraint length 8, polynomials are [255, 363, 227, 337] in octal. Alt. 3: Mother code rate of 1/6 with constraint length 6, polynomials are [65, 57, 71, 73, 75, 53] in octal. |
| Huawei | Observation 1: The complexity of an FEC encoder depends on the number of shift registers and XOR gates used. Observation 2: The complexity of an LTE convolutional encoder is comparable with that of CRC-5 used in RFIDs. Proposal 1: For R2D FEC, LTE convolutional codes using the same generator polynomials as per TS 36.212 is supported, since it provides a good balance between performance and encoding complexity. Observation 3: An increase in the constraint length provides marginal performance gain, but increases the power consumption and the complexity of the device. Observation 4: The nested convolutional code generators with more XOR gates provides marginal performance gain with marginally increased complexity. Observation 5: The convolutional code generator with less registers and more XOR gates is marginally inferior to the baseline on the performance at the cost of marginal complexity. Observation 6: The marginal increase in performance for changing the CC polynomials does not justify the necessity to increase the complexity of the device, while consuming RAN1 time and effort to perform an evaluation campaign to validate these new polynomials. Observation 7: Changes to the CC polynomials prevent using existing hardware designs for LTE CC decoding in the BS reader, due to changes in the number of registers or XOR gates. Proposal 2: Since, compared with the LTE convolutional code, the convolutional code with more registers, more XOR gates, or with less registers but more XOR gates all provide a marginal performance gap which can be negligible while potentially requiring a redesign of the CC decoder at the reader, RAN1 does not further consider any other convolutional code generator polynomials than those in TS 36.212. |
| Spreadtrum | Proposal 2: TBCC with constraint length 7 and coding rate 1/3 in LTE is adopted for D2R transmission. |
| CATT | Proposal 2: Rel-19 A-IoT should only support CC generator polynomials as per TS36.212. |
| China Telecom | Proposal 3: Reuse LTE convolutional code with generator polynomials in TS 36.212 with necessary modifications (if needed). - A tail biting convolutional code with constraint length 7 and coding rate 1/3 as baseline. |
| CMCC | Proposal 4: For D2R FEC, reuse polynomials from TS 36.212 as baseline, including constraint length and coding rate. |
| vivo | Proposal 5: Support reusing LTE CC polynomials in TS 36.212 with constraint length=7 and code rate =1/3. |
| Nokia | Proposal 4: RAN1 supports convolutional code with generator polynomials as per TS 36.212 for D2R transmissions. That is, no other generator polynomials is introduced. |
| OPPO | Proposal 8: The generator polynomials of LTE convolutional code are used for D2R |
| Xiaomi | Proposal 7: In Ambient IoT system, D2R supports only convolutional code with generator polynomial as per TS 36.212. The constraint length is 7, mother code-rate and final code rate is 1/3. |
| Apple | Proposal 6: Convolutional code with generator polynomials as per TS 36.212 is used for PDRCH. |
| Samsung | Proposal 3: The convolutional code from TS 36.212 should be used as is for D2R. Observation 4: Convolutional code with K=7 may not guarantee adequate decoding performance for a maximum TB size of 1,000 bits. Proposal 4: When using convolutional coding for D2R, specify a maximum TB size smaller than 1,000 bits. |
| Fujistu | Proposal 2: Reuse the convolutional code with the generation polynomials given in TS 36.212 and adopt a zero-start & zero-end encoding manner. |
| LGE | Proposal 5: Reuse LTE generator polynomials specified in TS 36.212 for the convolutional code for D2R. |
| Lenovo | Observation 2: Decreasing code rate provides marginal performance improvement as seen from the results captured in the TR 38.769 Proposal 4: For PDRCH coding, support constrained length of (k<6) and only two code rates ½, ¼ as multiple supported code rates increase the overhead of R2D control signal, complexity/power consumption at the device. |
| Honor | Proposal 3: D2R only support convolutional code with generator polynomials as per TS 36.212 with Constraint length 7 and Mother code-rate 1/3. |
| InterDigital | Proposal 5: D2R supports convolutional code with constraint length and mother code rate as per TS 36.212. |
| Qualcomm | Observation 5: For CC with the constraint length K=7 and the code rate R=1/4, the extra polynomial nested to the existing LTE CC (K=7 and R=1/3) could be P = 117, 127 or 137, which can achieve BLER=1% at Eb/N0 around 2.5 dB. Observation 6: For nested CC generators with a constraint length K=6 and a code rate R ranging from 1/2 to 1/6, candidate polynomials [45, 73, 75, 67, 57, 55], [55, 67, 77, 51, 53, 73] and [77, 73, 55, 45, 67, 65] are only marginally inferior to the baseline LTE CC by 0.15 dB at BLER=1%. Observation 7: For a code rate of 1/3, the obtained nested CC with constraint length K=6 is only 0.4 dB inferior to the LTE CC with K=7 at BLER=1%. However, the decoding complexity is halved. Proposal 4: For D2R FEC, support multiple coding rates. Proposal 5: For D2R FEC, support TBCC with K=[6/7], code rate<1/3 with new additional polynomial(s). |
| Docomo | Proposal 8: For FEC on PDRCH, other constraint length than 7 is not necessary. Proposal 9: For FEC on PDRCH, other code rate than 1/3 is not necessary. - Higher coding rate such as 1/2 can be supported by e.g., puncturing, if necessary. |
| ITL | Proposal 3: It is proposed to consider the LTE-based K=7, R=1/3 configuration as the baseline. |

#2 Initialization of shift register

| Source | Observations / Proposals |
| --- | --- |
| Futurewei | Observation 5: Tailed codes can increase the processing load, increase latency, and decoder complexity. Observation 6: A modified tail-biting code offers the benefits of a tail-biting code while have a similar processing timeline as tailed codes. Proposal 4: For the constraint length L convolutional encoder for the D2R link, down-select from the following: - Tailed code where the initial state is 0 and L-1 tail bits are appended - Tail-biting code where the initial state is based on the last L-1 bits of the input sequence - Tail-biting code where the initial state is based on the first L-1 bits of the input sequence |
| ZTE | Observation 3: TBCC encoding does not require additional complexity or registers. Observation 4: The performance of TrCC is inferior to TBCC and ZTCC. Observation 5: The performance of TBCC is slightly better than ZTCC, and for large payload size, their performance is almost the same. Observation 11: The initialization bits of TBCC encoding are kept in the CRC registers, and no extra memory is needed to store these initialization bits. |
| Huawei | Observation 14: For the D2R transmission of Ambient IoT, the performance of the convolutional codes with zero-padding is significantly degraded when compared to the end state of the shift registers being same as the initial state of the shift registers. Proposal 5: For the D2R transmission of Ambient IoT, convolutional codes with the end state of the shift registers being the same as the initial state of the shift registers is supported for D2R FEC. Observation 15: For TBCC, memory is required to store all the information bits before channel encoding. Observation 16: For TBCC, the CRC attachment and the channel coding are required to be processed in the serial mode, which increases the processing time. Observation 17: If initial values of the shift register are the first 6 information bits, CRC attachment and channel coding can be processed in a pipelined manner, not requiring an extra buffer or processing time, unlike TBCC, while not impacting the hardware design of the decoder at the reader side. Proposal 6: For D2R FEC, support that the initial values of the shift register are set to the first 6 information bits in the input payload. |
| Spreadtrum | Observation 2: For ZTCC, additional zero bits are appended to ensure that all the encoder's final state returns to zero, which leads to the overhead increasing. Observation 3: For truncation convolutional code, simply stopping after encoding completing the entire sequence of information bits will impact the decoding performance due to a tail effect. Proposal 2: TBCC with constraint length 7 and coding rate 1/3 in LTE is adopted for D2R transmission. |
| CATT | Proposal 1: Tail-biting convolutional code (TBCC) should be specified for Rel-19 D2R FEC code. |
| China Telecom | Proposal 3: Reuse LTE convolutional code with generator polynomials in TS 36.212 with necessary modifications (if needed). - A tail biting convolutional code with constraint length 7 and coding rate 1/3 as baseline. |
| CMCC | Observation 6: For D2R FEC, trade-off between encoding delay/complexity and decoding performance is needed. Proposal 5: For D2R link, head biting convolutional code (HBCC) can be considered to reduce encoding delay/complexity. |
| Fujistu | Proposal 1: No tail-biting is supported for the convolutional code adopted in D2R link. |
| Qualcomm | Proposal 5: For D2R FEC, support TBCC with K=[6/7], code rate<1/3 with new additional polynomial(s). |

#3 Rate matching

| Source | Observations / Proposals |
| --- | --- |
| ZTE | Observation 6：Interleaving may cause high memory cost for A-IoT devices. Observation 7: Rate matching may cause high complexity for Ambient IoT devices. |
| Huawei | Observation 10: An interleaver requires buffering of the CC-encoded bits, requiring these bits to be written into non-volatile memory, which is power consuming and increases latency for the device. Observation 11: Using a smaller buffer size would result in a smaller interleaving matrix, reducing its effectiveness and impact on the system performance. Observation 12: If a device has to maintain a large buffer, it would have to perform the write operation more than once to perform interleaving, further increasing latency and power consumption. Observation 13: Given the power consumption level of an Ambient IoT device, it is not possible for the device to support an interleaver with a large enough matrix to be effective, since it would be required to perform write operations more than once to buffer the CC-encoded bits, increasing the latency and power consumption beyond what is achievable for such a device. Proposal 4: Sub-block interleaving is not supported by Ambient IoT devices. |
| CATT | Proposal 3: For CC in A-IoT Device 1 - Repetition operation should be integral multiples of CC codeword. - Bit collection should be performed based on natural order of three encoder output parity streams. - Note: - No need to support the sub-block interleaver(s) and puncturing operation of rate matching in LTE-TBCC. |
| NEC | Proposal 3: Specify a simple and effective rate matching procedure for ambient IoT. |

#4 Configurable FEC code rate (incl. use or not use of FEC)

| Source | Observations / Proposals |
| --- | --- |
| Futurewei | Proposal 2: For the convolutional code in the D2R link, support a mother code rate of 1/R and obtain higher code rates by not generating the output for specific polynomials. Observation 10: Fine granularity for the coding rates may not be possible for complexity reasons at the device. Proposal 6: For the D2R link, support a table of overall coding rates and the corresponding convolutional coding rate and number of repetitions. - FFS: The coding rates |
| ZTE | Proposal 7: A code rate of R=1n0 should be support for convolutional code to increase flexibility, wherein, n0 is an integer and 1<n0≤n, where n is the number of polynomials. |
| Huawei | Observation 8: The inventory completion time can be reduced if the data rate can be increased by not using FEC when devices are distributed close to the reader. Observation 9: Based on the deployment scenario, certain readers can indicate to its devices to be inventoried to not use FEC, since the distance between them is very short, in order to achieve a higher data rate. Proposal 3: For D2R, not using FEC is supported, applicable to suitable fixed deployment cases, e.g. when many devices are expected to be close to the reader. |
| Spreadtrum | Observation 4: The length of the bit sequence input to FEC should be larger than 6 to keep the initial value of the shift register set to the values corresponding to the last 6 information bits in the input stream. Proposal 3: For small size data, e.g., bit size smaller than 6, FEC should not be used in D2R. |
| NEC | Proposal 2: In ambient IoT, we need to first discuss whether D2R transmission with number of TBS less than 6 bits is possible. And then how to initialize TBCC shift register if less than 6 bits transmission is allowed. |
| Lenovo | Observation 2: Decreasing code rate provides marginal performance improvement as seen from the results captured in the TR 38.769 Proposal 4: For PDRCH coding, support constrained length of (k<6) and only two code rates ½, ¼ as multiple supported code rates increase the overhead of R2D control signal, complexity/power consumption at the device. |
| Docomo | Proposal 8: For FEC on PDRCH, other constraint length than 7 is not necessary. Proposal 9: For FEC on PDRCH, other code rate than 1/3 is not necessary. - Higher coding rate such as 1/2 can be supported by e.g., puncturing, if necessary. |

D2R repetition

| Source | Observations / Proposals |
| --- | --- |
| Ericsson | Observation 2: Chip-level repetition is equivalent to long chip transmission, i.e., smaller modulation index M. Proposal 3: R2D transmissions are assumed to not use repetitions as baseline. Bit-level repetition can be considered if further coverage enhancement is needed. Proposal 4: For D2R, the following repetition options can be considered: - Bit-level type 1 repetition (in the absence of FEC) and bit-level type 2 repetition (in the presence of FEC) to support small frequency shift Option 1. - Block-level repetition, where all bits received from the higher and/or physical layer are repeated, if device 1 cannot meet coverage target. |
| Futurewei | Observation 7: Block level repetition can mitigate the effects of fading by increasing the temporal distance between the “same bit”. Observation 8: The processing sequence of block level repetition and tail-biting encoding is equivalent to the processing sequence of tail-biting encoding and block level repetition. Observation 9: The combination of block level repetition and tail-biting encoding enables a receiver to combine LLRs corresponding to repeated bits before decoding. Proposal 5: For the repetition types for the A-IoT D2R link, down-select to - Option 1: Block level: where all the bits received from higher layers and/or physical layer (according to what is present) after CRC attachment (if used) are blockwise repeated Rblock times - Option 2: Bit level type 2: Each bit after both CRC attachment (if used) and FEC (if used) is repeated Rbit times |
| ZTE | Observation 8: Block level repetition may lead to an increase in memory complexity and power consumption for frequent writing and reading operations. Observation 9: The performance of bit level type 1 repetition is inferior to the other repetition type. Observation 10: Repeat on a sub-block (corresponding to n0 coded bits for each input information bit) basis, A-IoT devices do not require buffering coded bits, resulting in simpler hardware, wherein code rate is R=1/n0 and n0 is an integer. Proposal 8: It is proposed that n0 coded bits are repeated on a sub-block basis, wherein the n0 coded bits are output bits of CC encoder for each information bit, code rate is 1/n0 and n0 is an integer. For example, the output coded bits for the k-th information bit are repeated as . |
| Huawei | Proposal 11: For D2R, bit level repetition with type 2 is supported for small frequency shifts functionality. Observation 25: For block level repetition, combining gain and time diversity gain is obtained. Observation 26: In combination with bit level type 2 repetitions for small frequency shifts, block level repetition with repetition number 2 performs gains ~3.5dB @10% BLER. Proposal 12: For the D2R transmission of Ambient IoT, block level repetition is supported for improving performance. |
| Spreadtrum | Proposal 7: Support the block level repetition for D2R transmissions. |
| CATT | Observation 10: For bit level repetition type 1, no repetition gain could be achieved as repetition bits are located within the same CC codeword and LLR combination cannot be performed. Observation 11: Compared to block level repetition, bit level repetition type 2 would degrade both power efficiency and BLER performance in D2R transmission. Proposal 9: With CC coding, block level repetition should be used for D2R transmission of A-IoT. |
| China Telecom | Proposal 6: At least support block-level repetition for D2R transmission. Proposal 7: If bit-level repetition is allowed for D2R transmission, bit-level type 2 can be supported. |
| CMCC | Proposal 10: For D2R repetition, bit level type 2 is supported, at least to achieve small frequency shift Manchester Option 1. The R bit times repletion should be within the duration of one information bit. Observation 7: Block-level repetition understanding 2 can enable early termination and is beneficial for link adaptation. Proposal 11: For D2R block-level repetition, consider and clarify the following two understandings: - Understanding 1: After CRC attachment, the bits are blockwise repeated Rblock times and transmit via a single PDRCH. - Understanding 2: After CRC attachment, the bits are blockwise repeated Rblock times, and each repetition can be transmitted via a single PDRCH. Proposal 12: For D2R repetition, block-level repetition is supported and understanding 2 is preferred. |
| vivo | Proposal 6: Support block-level repetition as the single repetition scheme for D2R. Further down-select between following options: - Block-level repetition Type-1: After CRC attachment and FEC, the information bits are blockwise repeated Rblock times. The repetition only applies to PDRCH but not apply preamble. - Block-level repetition Type-2: After CRC attachment and FEC, the information bits are blockwise repeated Rblock times. The repetition also applies preamble. |
| Nokia | Proposal 5: RAN1 supports block level repetition for the transmissions of PDRCH |
| TCL | Observation 5: As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length. Proposal 10: Consider D2R block-level repetition based on following aspects, - Time and frequency resources for each repetition block - Repetition number (R) - How to indicate time/frequency resource and repetition number (R) Observation 6: Limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition. Observation 7：For the number of bit-level repetition, it is up to coverage and access performance and it will impact on D2R frame structure selection. Proposal 11: Consider D2R bit-level repetition based on following aspects, - Time and frequency resources for each repetition block - Repetition number (R) - How to indicate time/frequency resource and repetition number (R) |
| OPPO | Proposal 9: Block level repetition is supported for D2R transmission |
| NEC | Proposal 5: Support study hybrid repetition scheme of bit level repetition and block level repetition as a complement to provide more flexibility on adjusting between performance and complexity. Proposal 6: Support study flipping source bits for different repetitions before mapping to chips when multiple repetitions are applied. |
| Xiaomi | Proposal 8: For D2R repetition in PDRCH generation procedure, the repetition type can be determined as follow: - When multiple repetition types are supported, either dynamic indication or semi-static configuration can be carried by PRDCH to indicate D2R repetition type of corresponding PDRCH(s). - Down-selection from the D2R repetition types: block level, bit level type 1, bit level type 2, or chip level. Observation 1: For D2R repetition, block level repetition has better performance than bit level type 2 and chip level repetition. Proposal 9: In Ambient IoT system, for PDRCH generation, repetition is performed after FEC. Proposal 10: In Ambient IoT system, at least D2R block level and chip level repetition can be considered. |
| Apple | Proposal 7: Bit level type-2 repetition is supported for PDRCH. |
| Samsung | Observation 7: Bit-level repetition requires an interleaver to mitigate the impact of burst errors, which may be burdensome for devices. Proposal 8: For D2R, block-level repetition is used while bit-level repetition is deprioritized. Proposal 9: D2R repetition is configured via R2D transmission, and additional R2D transmission can be transmitted between D2R repetitions. Proposal 10: R2D message that transmits D2R scheduling information should include at least the following information for repetitions: - Number of repetitions - A time gap between repetitions (if any) - Transmission timing information If this information is not provided, the device is expected to wait for an additional triggering R2D transmission. |
| Fujistu | Observation 1: - The block repetition could be transparent to the devices, which can be implemented by the reader without any standardization impact. - The block repetition is realized by the reader’s implementation, which can simplify the design of the devices and save the standardization effort as well. Proposal 5: Support block repetition in D2R transmission by the implementation of the reader. |
| LGE | Proposal 11: For D2R, consider the following alternatives to restrict the buffer size required to support block-level repetitions. - Alt.1) Support block-level repetitions for TBS < X (e.g., X = tens of bits) - Alt.2) Block-level repetitions with the block size smaller than the TBS - Alt.3) Do NOT support block-level repetitions (i.e., support bit-level type 2 and/or type 1) at least for Device 1 |
| MediaTek | Proposal 2: Block-level repetition can be considered for D2R transmission. |
| Lenovo | Observation 1: For PDRCH Repetition before the FEC increases the usage of FEC, thereby increasing the power consumption and complexity. Proposal 3: For PDRCH repetition, support block repetition after the FEC |
| Sharp | Proposal 5: Block level repetition is supported for a D2R transmission. - The case of no repetition (i.e. number of repetitions = 1) is supported. - Bit level repetition is not supported for any D2R transmission. Proposal 6: The following options are considered for determination of the number of block level repetitions - Option 1: The number of block level repetitions is indicated by a D2R grant conveyed in a scheduling R2D transmission. - Option 2: The number of block level repetitions is derived based on the time resources allocated by a D2R grant conveyed in a scheduling R2D transmission. |
| Honor | Observation 2: Block level repetition can provide the similar combination and time domain diversity gain with bit level repetition type 2 which are greater than bit level repetition type 1. Observation 3: Block level repetition can allow Reader to terminate reception early if it successfully decodes the block. Observation 4: Block level repetition is less susceptible to the burst errors caused by the channel fading. Observation 5: Block level repetition may need more buffer which could be solved by well-designed repetition block size. Observation 6: Configuration of D2R repetition can be indicated by reader via L1 control/higher layer configuration in the previous corresponding R2D transmission. Proposal 8: D2R supports both block level and Bit level type 2 physical layer repetitions. |
| InterDigital | Proposal 8: Support block level repetition for D2R transmission. |
| ASusTek | Proposal 3: Block level repetition can be adopted as channel repetition. |
| Qualcomm | Proposal 6: For D2R repetition, deprioritize chip level repetition. Proposal 7: For D2R repetition, Bit level type 1 repetition. Observation 8: Introducing lower coding rate for CC and supporting multiple coding rates would provide better performance than bit level type 2 repetition. Proposal 8: Discuss bit level type 2 D2R repetition with FEC CC enhancements. Observation 9: To get diversity gain by block level repetition in D2R, the timings of repeated transmissions should have enough gap. Observation 10: Back-to-back block level repetitions bring only energy gain. Observation 11: Higher device clock accuracy could help reducing reader rx decoding complexity for combine/decode of D2R repeated transmissions. Proposal 9: Support D2R block level repetition scheme based on polynomial sweeping based TBCC. |
| Docomo | Proposal 10: For D2R, at least block-level repetition should be supported. - FFS: Repetition factor - FFS: Which of Msg1, Msg3 and Step C2 (if present) supports repetition - FFS: Whether preamble should be repeated |
| ITL | Proposal 6: Support block-level repetition for D2R transmission with following additional details: - Whether/how to determine the number of repetitions for D2R transmission via a device side based on various consideration aspects (e.g. received R2D signal power, use case, data QoS and so on) - Whether/how to enable early termination of repetition transmission |
| IIT-Kanpur | Proposal 2: Chip-level repetition should be considered for the D2R transmission. |
| Sequans | Proposal 4: For D2R, at least block-level repetition is supported. - FFS: Tradeoffs of hybrid bit-level and block-level repetition |

R2D/D2R CRC

#1 Generator polynomial

| Source | Observations / Proposals |
| --- | --- |
| ZTE | Observation 12: The nested design of CRCs can reduce the complexity for A-IoT by approximately one-quarter. Observation 13: FAR performance of new designed CRC-16 polynomial is equivalent to that of the CRC-16 polynomial in NR for both short and long payload size. Proposal 9: Nested polynomials for 6-bit CRC and 16-bit CRC can be considered in A-IoT. It is proposed that the following 6-bit and 16-bit CRCs are used in A-IoT: - for a CRC length ; - for a CRC length . |
| Huawei | Proposal 13: The CRC generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC is supported. RAN1 does not further consider other polynomials. |
| CATT | Proposal 11: There is no need to introduce other 6 bits and 16 bits CRC with different polynomials beyond those defined in TS 38.212. |
| China Telecom | Proposal 4: Support CRC and reuse the generator polynomials with 6-bit and 16-bit CRC in TS 38.212. - for a CRC length ; - for a CRC length . |
| CMCC | Proposal 6: For the CRC generator polynomials, reuse 6 bits and 16 bits CRC with polynomials from TS 38.212. |
| vivo | Observation 1: With RFID as reference, the complexity and cost for re-using the non-nested CRC polynomials defined in TS 38.212 for 6-bits and 16-bits CRC generation is acceptable for A-IoT device 1. Proposal 2: Reuse CRC polynomials defined in TS 38.212 for 6-bits and 16-bits CRC generation respectively for A-IoT. |
| Nokia | Proposal 1: RAN1 supports reuse of the existing generator polynomials defined in TS 38.212 to support CRC-6 and CRC-16. |
| OPPO | Proposal 4: The CRC generator polynomials of LTE and NR for CRC-6 or CRC-16 can be reused for A-IoT. |
| Apple | Proposal 1: CRC as per the generator polynomials in TS 38.212 is used for 6-bit CRC and 16-bit CRC. |
| Fujistu | Proposal 3: Reuse the CRC with the generation polynomials of 6-bit and 16-bit given in TS 38.212. |
| LGE | Proposal 6: Reuse NR CRC generator polynomials for the 6-bit CRC and the 16-bit CRC for Ambient IoT. |
| Honor | Proposal 4: For PRDCH and PDRCH, just use the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC. |
| InterDigital | Proposal 6: PRDCH and PDRCH support CRC as per the generator polynomials in TS 38.212. |
| Docomo | Proposal 3: For 6-bit CRC for PRDCH and PDRCH, reuse polynomial for CRC-6 in TS38.212. Proposal 4: For 16-bit CRC for PRDCH and PDRCH, consider the following polynomial. - gCRC16D=D16+D11+D6+D5+1 |
| ITL | Proposal 5: It is proposed to reuse following generator polynomials with 6-bit and 16-bit CRC as specified in 38.212 - for a CRC length ; - for a CRC length . |

#2 Case to use which CRC length, or no CRC

| Source | Observations / Proposals |
| --- | --- |
| Ericsson | Observation 3: The optimal length of the CRC should be determined by factors such as message size, target false alarm rate (associated with the message type), and complexity. - The CRC overhead becomes significant when the selection threshold between CRC-6 and CRC-16, denoted by X, is set to 16. Proposal 5: The selection threshold between CRC-6 and CRC-16, denoted by X, can be down-selected between X = 24 and X = 57 according to target false alarm rate (associated with the message type). |
| Futurewei | Observation 1: For input sequence lengths between 26 and 57 bits, the performance of the gCRC6(D) polynomial is close to the bounds. Observation 2: One polynomial should be specified for a length 6 CRC. Observation 3: For input sequence lengths between 242 and 2048 bits, the performance of the gCRC16(D) polynomial is close to the bounds. reasonable to reuse current gCRC16D for Ambient IoT Observation 4: For input sequence lengths between 8 and 241 bits, the gCRC16(D) polynomial has similar performance as other polynomials. Proposal 1: For segmentation on the D2R link, every segment uses a 16-bit CRC. |
| ZTE | Observation 14: For CRC-6, when payload size is less than or equal to 56, different sizes of transport blocks have almost identical FAR performance for the same BLER; when payload size exceeds 56, the FAR is gradually deteriorated. Proposal 10: The used CRC length depends on the number of bits payload size before CRC, i.e. CRC-6 for payload size <=X bits, while CRC-16 for payload size > X bits, wherein X is equal to 56. Proposal 11: It is proposed that no CRC is used for payload size <=Z bits, wherein Z is equal to 8. |
| Huawei | Observation 27: Higher-layer message type or purpose is not known to the physical layer. Hence attachment of CRC dependent on message type or purpose conditions is a RAN2 decision, and only potentially applicable to some D2R messages. Proposal 14: From the RAN1 perspective, PRDCH and PDRCH are transmitted without CRC when TBS ≤ 16 bits. - If there are other conditions on CRC usage is up to RAN2, and would require specific procedural design for MAC to inform PHY. Observation 28: For the A-IoT system, CRC-6 can realize reasonable undetected error probability performance under TBS=24bits. Proposal 15: For the A-IoT system, CRC 6 for TBS≤X, and CRC-16 for TBS>X with X = 24 bits is supported. |
| Spreadtrum | Observation 5: The determination of the CRC length is related to the factor of TBS, Target false alarm rate, and whether repetition is performed during PRDCH/PDRCH generation. Observation 6: For PDRCH repetition case, the ratio of useful signals is always less than 50%; when TBS is larger than to certain number, e.g. 48bit, the ratio of useful signals is insensitive to the length of CRC (6 or 16 CRC). Proposal 4: For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of bits Z before CRC, while Z=TBS*rep_num, i.e. No CRC for Z<=6, CRC-6 for 6< Z<=24 bits, and CRC-16 for Z > 24 bits. Proposal 5: For PRDCH/PDRCH, no CRC is adopted for the case of bit sequence shorter than 6 bit, or some specific commands, e.g., QueryRep/ACK messages in inventory procedure. |
| CATT | Proposal 10: The following rule of CRC length determination should be adopted: - CRC-16 should be used for information bits length>N, otherwise CRC-6 is applied, where N=16 or 24. |
| China Telecom | Proposal 5: For PRDCH/PDRCH transmissions with CRC, the used CRC length depends on the number of transmitted data bits Z before CRC, i.e. CRC-6 for Z<=24 bits, while CRC-16 for Z >24 bits. |
| CMCC | Proposal 7: For R2D/D2R transmissions with CRC, the used CRC length depends on the number of bits Z before CRC, i.e. CRC-6 for Z<=24 bits, while CRC-16 for Z > 24 bits. Proposal 8: For R2D transmission with QueryRep-like command or ACK, no CRC is supported. FFS: For R2D transmission where Msg-2 contains scheduling information, CRC is supported. Proposal 9: For D2R transmission with Msg-1, CRC is supported. |
| vivo | Proposal 3: RAN1 decides CRC length based on the number of information bits - No CRC for the number of information bits no larger than 8; - 6-bit CRC for the number of information bits larger than 8 and no less than 24; - 16-bit CRC for the number of information bits larger than 24. - FFS any exceptional case(s) for the transmission/receptions with or without CRC during the random access phase, depending on the command/response type to be defined by RAN2. |
| Nokia | Proposal 2: RAN1 supports CRC-16 in case message size is greater than or equal to 24 or 32 bits. Otherwise, CRC-6 or no CRC is used. |
| TCL | Proposal 2: Consider CRC attachment for R2D transmission, including - No CRC or CRC-6 attachment after CAP transmission - CRC-6/CRC-16 attachment for control information and data information, respectively - No CRC for control information and CRC-6/CRC-16 attachment only for data information - FFS how to determine CRC length if attached Proposal 8: Consider CRC attachment for D2R transmission, including - CRC-6/CRC-16 attachment for control information and data information, respectively - No CRC for control information and CRC-6/CRC-16 attachment only for data information - No CRC for data information if not important or urgent data transmission - No CRC attachment might be considered for frame structure with Postamble/Midamble due to Midamble/Postamble could be used for transmission reliability - CRC-6/CRC-16 attachment might be considered for frame structure without Postamble/Midamble to enhance reliability for D2R transmission - FFS the case of not important or urgent data transmission - FFS how to determine CRC length if attached Proposal 9：Discussion on how to indicate CRC attachment and length at least including - Explicit indication by R2D control information - Implicit indication |
| OPPO | Proposal 1: Regarding PRDCH with Msg-2 (including ACK command and detected ID): - If the Msg-2 corresponds to one device, PRDCH without CRC is supported. - If the Msg-2 corresponds to more than one device, PRDCH with CRC is supported. Proposal 2: Regarding CRC for PDRCH: - For inventory use case, CRC should be applied to Msg-1 and Msg-3 (if supported) transmission - For command use case, if ACK corresponding to R2D command is supported, without CRC can be considered Proposal 3: A threshold of TBS can be pre-defined to determine whether CRC-6 and CRC-16 needs to be used. |
| Panasonic | Proposal 5: The CRC design of D2R should be discussed after the content of control information is determined. |
| NEC | Proposal 4: CRC attachment is exempted for the following transmission: - D2R Msg1 transmission; - R2D Msg2 response. |
| Apple | Proposal 2: For PRDCH, CRC-16 is supported for paging message transmission considering more robustness for paging message as it is typically transmitted to multiple devices. Proposal 3: For PRDCH, if L1 R2D control information is transmitted separately from the R2D message, - CRC-16 is used for the L1 R2D control information transmission. - Depending on the R2D message size, either CRC-6 or CRC-16 can be used. A fixed payload threshold can be defined. Proposal 4: For PRDCH, if L1 R2D control information is transmitted as part of the R2D message, CRC-16 is used for PRDCH. Proposal 5: For PDRCH, depending on the payload size, either CRC-6 or CRC-16 can be used. A fixed payload threshold can be defined. |
| Samsung | Observation 5: Different CRC attachment scheme depending on TBS or message type requires a device to implement multiple sets of CRC polynomials as well as determination of the used CRC attachment scheme. This increases device complexity and, thus, it is undesirable. Proposal 5: For PRDCH or PDRCH, a fixed CRC attachment scheme is applied. |
| LGE | Proposal 9: Consider fixed size CRC for L1 R2D control (6 or 16) and variable size CRC for R2D data. - CRC size for R2D data to be determined based on the R2D data size, or to be signaled via L1 R2D control |
| MediaTek | Observation 1: If CRC-16 is used for a TB size larger than 16, it may result in an overhead up to around 50% (e.g., overhead of 48.48% for the TB size of 17 bits). Proposal 1: Regarding the utilization of CRC, support - No CRC for PRDCH/PDRCH transmission without critical information, e.g., control information - CRC-6 for PRDCH/PDRCH transmission with TBS less than or equal to 24 bits - CRC-16 for PRDCH/PDRCH transmission with TBS greater than 24 bits |
| Sharp | Proposal 1: CRC-16 is used if Z > 24, where Z is the number of bits prior to CRC attachment. Observation 1: For R2D, use of no CRC or CRC-16 is dependent on which option to use for determination of R2D end timing. |
| Honor | Observation 1: Messages using CRC or not should consider the overhead and function and importance. Proposal 5: For RA triggering Msg without resource allocation and ACK-like messages, no CRC is needed. Proposal 6: Use threshold X=24 to determine using CRC-6 or CRC-16 for both PRDCH and PDRCH. |
| InterDigital | Proposal 7: CRC-6 is used for payloads less than 24 bits and CRC-16 is used for payloads large than 24 bits. No CRC is used for very small payloads (FFS exact payload size). |
| Qualcomm | Proposal 10: CRC-6 is used for information size of up to 24 bits. Proposal 11: CRC-16 is used for information size of more than 24 bits. Observation 12: For short input sequence without CRC, probability of undetected error is 1. Observation 13: CRC1 or parity bit could be used to reduce Pud of 1 to 0.5 of no CRC case. Proposal 12: Use CRC1 for input sequence length Z≤[6]. |
| Docomo | Proposal 2: For CRC for PRDCH and PDRCH, 6bit CRC should be assumed for TBS<[24]bits, otherwise, 16bit CRC should be applied. - Exception depending on message type, e.g., no CRC attachment or 6/16-bit CRC regardless of TBS, can be further discussed if any. |
| ITL | Proposal 4: For CRC length of R2D/D2R physical channels, the CRC length depends on the number of transmitted data bits Z before CRC. |

#3 Joint or separate CRC for control information and data

| Source | Observations / Proposals |
| --- | --- |
| Spreadtrum | Proposal 6: Separate CRCs are supported for control and data segments in PRDCH. |
| CATT | Proposal 12: Joint CRC should be applied to both R2D control information and R2D data. |
| vivo | Proposal 4: Single CRC is applied for PRDCH with control and data part, and the CRC length is determined based on the total number of bits of control and data. |
| Nokia | Proposal 3: Do not support a separate CRC for control information. |
| OPPO | Proposal 5: If L1 R2D control information is supported & carried by PRDCH, separate CRC is attached to L1 R2D control information and R2D data respectively is preferred. - FFS whether same or different CRC length for R2D control information and data packet |
| Panasonic | Proposal 3: A separate CRC for R2D control information should be used for R2D transmission. |
| Apple | Proposal 3: For PRDCH, if L1 R2D control information is transmitted separately from the R2D message, - CRC-16 is used for the L1 R2D control information transmission. - Depending on the R2D message size, either CRC-6 or CRC-16 can be used. A fixed payload threshold can be defined. Proposal 4: For PRDCH, if L1 R2D control information is transmitted as part of the R2D message, CRC-16 is used for PRDCH. |
| Samsung | Observation 6: Separate CRC attachment to L1 control and payload is preferable for PRDCH to early identify the indented receiver of the transmission and, thereby, to save device’s energy. Proposal 6: For PRDCH, separate CRC is attached to L1 R2D control information and R2D data, respectively. Proposal 7: For PDRCH, if payload is segmented due to insertion of midamble, each payload segment is attached with separate CRC. |
| LGE | Proposal 7: Support L1 control information for PRDCH, i.e. Option 2. Proposal 8: Support separate CRCs for PRDCH, i.e. the first CRC for L1 R2D control information and the second CRC for R2D data which may or may not contain L2 control information. |
| Honor | Proposal 7: Consider to use separate L1 control CRC for some messages, such as Msg2. |
| ASusTek | Proposal 1: For PRDCH generation, L1 R2D control information and R2D data with separate CRC attachment are carried together in single PRDCH. Proposal 2: For PDRCH generation, not define L1 D2R control information and corresponding CRC attachment. |