3GPP TSG RAN WG1 #120	                                   					       												                                   R1-2501601

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.15.2

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR MIMO Phase 5

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [120-R19-UE_features] during RAN1 #120. According to the Chair’s Notes:

[120-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #120 within the scope of [120-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [15].

# Summary of Contributions Submitted to RAN1 #120

The following is the moderator’s summary of contributions submitted to RAN1 #120 in this agenda item.

## UE-initiated/event-driven beam management

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [at least] for Mode A | 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS | FFS | FFS | FFS | [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | According to the above Agreements, the support of Event-7 is optional while RAN1 has not made a similar decision for the support of Event-1. Since Event-1 only requires RSRP measurement of the current beam which is already among the required measurements for the basic feature (Event-2), we think that, similar to Event-2, Event-1 should be a part of the basic feature: Proposal 2.1: For UE initiated/event-driven beam management, the following component is introduced in FG 59-1-1 and 59-1-2: - Support of Event-1 based measurement and report According to the above agreements, the maximum number of the configured RS(s) for new beam in the RS resource set is subject to UE capability. Therefore, the corresponding component should be included in FG 59-1-1 and 59-1-2. Proposal 2.2: For UE initiated/event-driven beam management, the following component should be included in the FG 59-1-1, 59-1-2: - Maximum number of the configured RS(s) for new beam in the RS resource set (remove brackets from component 4 of FG 59-1-1, 59-1-2). For FG 59-1-1 and FG 59-1-2, we think both scheme-1 and scheme-2 based measurements should be supported for any UE that supports ModeA or ModeB report. Therefore, we suggest to remove the brackets from component 5 of these two FGs. Proposal 2.4: For FG 59-1-1 and FG 59-1-2, remove the brackets from component 5. 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. [Maximum number of the configured RS(s) for new beam in the RS resource set] 5. [Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] 6. Support of Event-1 based measurement and report; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-1/Event-2 and Mode A]; FFS; FFS; FR2 only; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] | FG 59-1-1 (UE-initiated/event-driven beam management at least for Mode A) This FG should be the basic FG for UEI/ED beam reporting, instead of Mode A only, thus keeping “at least” should be clear. If a UE indicating it doesn’t support this FG, the whole feature of UEI/ED beam reporting is not supported by the UE. For component 1, based on the following agreement, the configured number M is invalid and not configured if UE just supports the basic FG of UEI/ED beam management. Hence, we suggest removing the wording of “with M=1”. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.. For component 4, this component should be included in the basic FG for UEI/ED beam reporting, thus the brackets can be removed. For component 5, this component should be included in the basic FG for UEI/ED beam reporting, thus the brackets can be removed. According to the following agreement, the basic FG should also support that the first PUCCH and second PUSCH are from the same PUCCH group. Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability. Proposal 1: Support FG 59-1-1 with the following changes: 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] 6. Support the first PUCCH and second PUSCH from the same PUCCH group; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-1-1: - According to the agreement of RAN1 #118, i.e. for the explicitly configured RS(s) for new beam(s), if legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. Maximum number of the configured RS(s) for new beam in the RS resource set should be supported as a component in FG 59-1-1. - According to the agreement of RAN1 #117, regarding RS measurement for the current beam for Event 2, support Scheme-1: RS for current beam is the QCL RS in the indicated TCI state and Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. The current statement for component 5 in FG 59-1-1 is ambiguous and the following modification is suggested: Support of current beam measurement by using QCL RS in the indicated TCI state for Scheme-1 and the corresponding QCL SSB for Scheme-2, respectively. Proposal 42: Regarding FG 59-1-1 for Rel-19 UEIBM, support the followings: - Support component 4, i.e. the maximum number of the configured RS(s) for new beam in the RS resource set as a component in FG 59-1-1. - Modifying component 5 to be: [5. Support of current beam measurement by using QCL RS in the indicated TCI state for Scheme-1 and the corresponding QCL SSB for Scheme-2, respectively.] |
| ZTE Corporation/Sanechips [5] | - Basically, FG 59-1-1 should be the basic UE feature to support UE-initiated/event-driven beam management as per the highlighted parts in the following agreements. Besides, the granularity should be per band to keep alignment of legacy beam measurement and reporting related UE features, and it’s worth noting that UE-initiated/event-driven beam management can only be operated in FR2 as per the statement of WID. Agreement (RAN1#116bis) On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement (RAN1#116bis) On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. FFS: Details. - For FG 59-1-1 of Mode-A based reporting, - Component 2 should capture the definition of Event-2 based measurement. - Component 4 should include {1, 2, ..., 64} as the candidate values as per the legacy. - The prerequisite FG 2-22 (which is for aperiodic beam report on PUSCH) and FG 2-24 (which is for SSB/CSI-RS based beam measurement) are needed. 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report that L1-RSRP of at least one new beam becomes a threshold value better than the current beam based on one event instance within a time window on the new beam 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively]; FFS 2-22, 2-24; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS Per Band; FFS; FFS Applicable only to FR2; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively]; 2-24. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] 6. Number of reported beams; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] Component 6: {1,2,3,4}; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] 6.Support of L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management 7.Support of N reported beams 8.Support of RRC enabled whether current beam is always reported 9.Support values of time window; FFS; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS; FFS; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements [Component 7 candidate values: {1,…,[N}] [Component 8 candidate values: {support, not}]; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management for event-2 Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively]; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; Per band; n/a; n/a; n/a; FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-1a; UE-initiated/event-driven beam management for event -1 Mode A; Support UE-initiated/event-driven beam management for event -1 Mode A; yes; n/a; UE-initiated/event-driven beam management for event -1 Mode A is not supported; Per band; n/a; n/a; n/a; Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold.; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-1b; UE-initiated/event-driven beam management for event -7 Mode A; Support UE-initiated/event-driven beam management for event -7 Mode A; yes; n/a; UE-initiated/event-driven beam management for event -7 Mode A is not supported; Per band; n/a; n/a; n/a; Note: the event definition Event 7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59-1-1; UEI/ED beam reporting; 1. Support of event-2 2. Support of determination of one event-2 instance (with M=1) within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event-2 instance 5. Support of Mode A 6. Supported candidate value(s) of reported beam(s) N in a reporting instance 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 9. Supported candidate value(s) of time window duration; 2-24; UEI/ED beam reporting is not supported; Per band; N/A; Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 6: candidate value(s): {2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: {Scheme-1, Scheme-2, both} Component 9: candidate value(s) is in ms; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-1-1; UE-initiated/event-driven beam management based on Event-2 [at least] for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively]; FFS 2-24, and 23-1-1 or 23-10-1; yes; n/a; UEI/ED beam report is not supported [for Event-2 and Mode A]; FFS Per Band; FFS No; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-1a; UE-initiated/event-driven beam management based on Event-1 for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-1 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively]; 2-24, and 23-1-1 or 23-10-1; yes; n/a; UEI/ED beam report is not supported [for Event-1 and Mode A]; Per Band; No; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}]; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-1b; UE-initiated/event-driven beam management based on Event-7 for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1] 2. Support of Event-7 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] 6. Supported values of Q; 2-24, and 23-1-1 or 23-10-1; yes; n/a; UEI/ED beam report is not supported [for Event-7 and Mode A]; Per Band; No; FFS; FFS; [Component 4 candidate values: {1, 2, … , 64}] Component 6 candidate values: {1, 2, …, 8}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-1-1 - The following component {Support of same evaluation periodicity as periodicity of current beam and new beams} should be added based on current agreement. - For component 5, should be supported. |
| Ericsson [14] | - During RAN1#119, RAN1 agreed to support also event-1 and event-7. The support for these events should be added as components under FG 59-1-1 and FG 59-1-2: Add support for event-1 and event-7 as components in FG 59-1-1 and FG 59-1-2. - The likelihood that the NW will ever configure a feature is greatly increased if the corresponding UE capability is defined per band. This is the usual procedure for beam management features, and we propose to use it also for FG 59-1-1 and FG 59-1-2: FG 59-1-1 and FG 59-1-2 is signalled per band. 59-1-1; UE-initiated/event-driven beam management for Mode A; 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Support to determine the RS of the current beam using the SSB that is the QCLed with the QCL RS in the indicated TCI state 5. Support to determine the RS of the current beam using the QCL RS in the indicated TCI state 6. Maximum number of the configured RS(s) for new beam in the RS resource set 7. Support for window/counter to trigger event-2 8. Support for event-1 based measurement and report 9. Support for window/counter to trigger event-1 10. Support for event-7 based measurement and report 11. Support for window/counter to trigger event-7 12. Supported values for current beam determination for event-7; 1-4 is basic. |

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | [1.Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X] | FFS | yes | n/a | UEI/ED beam report Mode-B is not supported for Event | FFS | FFS | FFS | FFS | [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | According to the above Agreements, the support of Event-7 is optional while RAN1 has not made a similar decision for the support of Event-1. Since Event-1 only requires RSRP measurement of the current beam which is already among the required measurements for the basic feature (Event-2), we think that, similar to Event-2, Event-1 should be a part of the basic feature: Proposal 2.1: For UE initiated/event-driven beam management, the following component is introduced in FG 59-1-1 and 59-1-2: - Support of Event-1 based measurement and report According to the above agreements, the maximum number of the configured RS(s) for new beam in the RS resource set is subject to UE capability. Therefore, the corresponding component should be included in FG 59-1-1 and 59-1-2. Proposal 2.2: For UE initiated/event-driven beam management, the following component should be included in the FG 59-1-1, 59-1-2: - Maximum number of the configured RS(s) for new beam in the RS resource set (remove brackets from component 4 of FG 59-1-1, 59-1-2). For Mode B based beam reporting, it was agreed that beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. We do not think the value of X should be a component of 59-1-2 as it is related to the gNB processing time for the first PUCCH and is not related to any UE capability. Proposal 2.3: For UE initiated/event-driven beam management, remove component 6 for the supported values of X in the FG 59-1-2. For FG 59-1-1 and FG 59-1-2, we think both scheme-1 and scheme-2 based measurements should be supported for any UE that supports ModeA or ModeB report. Therefore, we suggest to remove the brackets from component 5 of these two FGs. Proposal 2.4: For FG 59-1-1 and FG 59-1-2, remove the brackets from component 5. 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; 1. [Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report 4. [Maximum number of the configured RS(s) for new beam in the RS resource set] 5. [Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X] 7. Support of Event-1 based measurement and report; 59-1-1; yes; n/a; UEI/ED beam report is not supported for Event-1/Event-2 and Mode-B; FFS; FFS; FR2 only; FFS; [Component 24 candidate values: {1, 2, … , 64}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] | FG 59-1-2 (UE-initiated/event-driven beam management for Mode B) This FG is should a separate FG for optional support of Mode B, and FG 59-1-1 should be the prerequisite FG of this FG. For components 1, 2, 4, 5, they can be removed since they are already included in FG 59-1-1. Proposal 2: Support FG 59-1-2 with the following changes: 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; [1.Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X]; FG 59-1-1; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; FFS; FFS; FFS; [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; 1. Support of UE-initiated/event-driven beam report based on one event instance [with M=1 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively [6. Supported values of X]; FFS; yes; n/a; UEI/ED beam report Mode-B is not supported for Event-2; FFS; FFS; FFS; FFS; Component 4 candidate values: {1, 2, … , [N]} FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-1-2: - According to the agreement of RAN1 #116bis, Mode A and Mode B are both supported for UE-initiated/event-driven beam reporting. Meanwhile, Event-2 is supported for UE initiated/event-driven beam reporting. Thus, component 1 and component 2 should be confirmed in FG59-1-2. - Since Event-2 is supported for both Mode A and Mode B, similar to the component 4 for Mode A in FG59-1-1, maximum number of the configured RS(s) for new beam in the RS resource set for Mode B should also be supported as a component in FG 59-1-2. - According to the agreement of RAN1 #117, Scheme-1 and Scheme-2 are supported for current beam measurement RS for Event-2. Since event-2 is supported for both Mode A and Mode B. The following modification on component 5 for Mode B should also be supported similarly to component 5 for Mode A in FG 59-1-1. i.e. Support of current beam measurement by using QCL RS in the indicated TCI state for Scheme-1 and corresponding QCL SSB for Scheme-2, respectively. - According to the agreement of RAN1 #118, on beam report transmission procedure for UE-initiated/event-driven beam reporting, for Mode-B, UE initiated beam reporting is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. FG 59-1-2 is a UE capability to support Mode B based beam report. In our opinion, the supported value of X should be a component of FG 59-1-2. Proposal 43: Regarding FG 59-1-2 for Rel-19 UEIBM, support the followings: - Support component 1, i.e., UE-initated/event-driven beam report for Mode B as a component in FG 59-1-2. - Support component 2, i.e., Event-2 based measurement and report for Mode B as a component in FG 59-1-2. - Support component 4, i.e. the maximum number of the configured RS(s) for new beam in the RS resource set as the component in FG 59-1-2. - Modifying component 5 to be: [5. Support of current beam measurement by using QCL RS in the indicated TCI state for Scheme-1 and the corresponding QCL SSB for Scheme-2, respectively.] - Support component 6, i.e., value of X for Mode B as a component in FG 59-1-2. |
| ZTE Corporation/Sanechips [5] | - For FG 59-1-2 of Mode-B based reporting, - Components 2, 4 and 5 should be removed due to the duplication to FG 59-1-1. - Component 6 that specify the guard time gap between first PUCCH and second PUSCH in Mode B is needed as per the following agreement, where the candidate values at least should include {0, 1}. Agreement (RAN1#118) On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel.. - The prerequisite FG 5-19 (which is to support CSI report on Type CG-PUSCH) and FG 59-1-1 are needed. 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management for Mode B; [1.Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. 2. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. 3. Supported values of X] symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH; FFS 5-19, 59-1-1; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS Per Band; FFS; FFS Applicable only to FR2; FFS; [[Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements Component 3 candidate values: {0, 1, ..., X}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; [1.Support of UE-initiated/event-driven beam report with M=1 [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X]; 59-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; [1. Support of UE-initiated/event-driven beam report [based on one event instance with M=1]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X] 7. Number of reported beams; 59-1-1; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; FFS; FFS; FFS; [Component 2 4 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements Component 7: {1,2,3,4}; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; 1.Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Supported values of X; FFS; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; FFS; FFS; FFS; [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; [1.Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X] 7.Support of L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management 8.Support of N reported beams 9.Support of RRC enabled whether current beam is always reported 10.Support values of time window; FFS; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; FFS; FFS; FFS; [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements [Component 7 candidate values: {1,…,[N}] [Component 8 candidate values: {support, not}]; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; 1.Support of UE-initiated/event-driven beam report [based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X]; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; Per band; n/a; n/a; FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-2a; UE-initiated/event-driven beam management for event -1 Mode B; Support UE-initiated/event-driven beam management for event -1 Mode B; yes; n/a; UE-initiated/event-driven beam management for event -1 Mode B is not supported; Per band; n/a; n/a; n/a; Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold.; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-2b; UE-initiated/event-driven beam management for event -7 Mode B; Support UE-initiated/event-driven beam management for event -7 Mode B; yes; n/a; UE-initiated/event-driven beam management for event -7 Mode B is not supported; Per band; n/a; n/a; n/a; Note: the event definition Event 7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59-1-2; UEI/ED beam reporting Mode B; 1. Support of event-2. 2. Support of determination of one event-2 instance (with M=1) within a time window 3. Supported candidate value(s) of the threshold for determining an event-2 instance 4. Support of determining event-2 based on determination of one event instance 5. Supported candidate value(s) of reported beam(s) N in a reporting instance 6. Supported candidate value(s) of reported beam(s) in a reporting instance that satisfies the event-2 condition(s) 7. Supported maximum number of candidate beam RS resources in a candidate beam RS resource set 8. Supported candidate value(s) of X 9. Support of Scheme-1 and/or Scheme-2 for determining current beam RS 10. Supported candidate value(s) of time window duration; X-1-1; UEI/ED beam reporting Mode B is not supported; Per band; N/A; Component 2: one event-2 instance is determined when a candidate beam becomes a threshold value better than the current beam within a time window Component 3: candidate value(s) of the threshold is in dB range Component 5: candidate value(s): {2, 3, 4} Component 6: candidate value(s): {1, 2, 3, 4} Component 7: candidate value(s): {1, 2, …, 128} Component 8: candidate value(s): {0, 1, 2, …} symbols Note: UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel Component 9: {Scheme-1, Scheme-2, both} Component 10: candidate value(s) is in ms; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-1-2; UE-initiated/event-driven beam management Mode B; [1.Support of UE-initiated/event-driven beam report [based on one event instance]] [2. Support of Event-2 based measurement and report] 3. Support of Mode B UE-initiated/event-driven beam report [4. Maximum number of the configured RS(s) for new beam in the RS resource set] [5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [6. Supported values of X]; FFS 59-1-1 or 59-1-1a or 59-1-1b; yes; n/a; UEI/ED beam report Mode-B is not supported for Event; FFS; FFS; FFS; FFS; [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to combine FG 59-1-1 and FG 59-1-2 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-1-2 - Component1/2/4/5 would not be needed because FG59-1-1 is basic feature. - Prerequisite feature groups should be FG59-1-1. |
| Ericsson [14] | - During RAN1#119, RAN1 agreed to support also event-1 and event-7. The support for these events should be added as components under FG 59-1-1 and FG 59-1-2: Add support for event-1 and event-7 as components in FG 59-1-1 and FG 59-1-2. - The likelihood that the NW will ever configure a feature is greatly increased if the corresponding UE capability is defined per band. This is the usual procedure for beam management features, and we propose to use it also for FG 59-1-1 and FG 59-1-2: FG 59-1-1 and FG 59-1-2 is signalled per band. - As always, we should strive to avoid market fragmentation. In the currently proposed FG 59-1-1 and FG 59-1-2, there is a component where the UE should advertise support for Scheme-1 and Scheme-2. To support UE initiated beam management, the UE must support either, and the NW would have to adopt the implementation based on that support. If some UEs support only Scheme-1 and some UEs support only Scheme-2, the NW would have to implement both. This is unfortunate – there should be one basic implementation that all UEs support. Therefore, we propose Scheme-2, i.e., the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state, is supported for all UEs that support FG 59-1-1 or FG 59-1-2. Support for Scheme-1, i.e., RS for current beam is the QCL RS in the indicated TCI state, is advertised by a component in FG 59-1-1 and FG 59-1-2. - We would also be OK if both scheme-1 and scheme-2 were part of the basic capability. 59-1-2; UE-initiated/event-driven beam management Mode B; 1.Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report 4. Support to determine the RS of the current beam using the SSB that is the QCLed with the QCL RS in the indicated TCI state 5. Support to determine the RS of the current beam using the QCL RS in the indicated TCI state 6. Minimum number of symbols between first and second channel [7. Maximum number of the configured RS(s) for new beam in the RS resource set] 8. Support for window/counter to trigger event-2 9. Support for event-1 based measurement and report 10. Support for window/counter to trigger event-1 11. Support for event-7 based measurement and report 12. Support for window/counter to trigger event-7 13. Supported values for current beam determination for event-7; 1-4 is basic. |

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances within a time window. | Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam] | FFS | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS | FFS | FFS | FFS | FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | FG 59-1-3 (Triggering event determination via detecting ≥ M event instances within a time window) According to the following agreement, the counting operation of event instance(s) within a time window is individually applied for each new beam. Hence, we suggest clarifying the wording by adding the description of “for at least one new beam”. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.. Proposal 3: Support FG 59-1-3 with the following changes: 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window for least one new beam [with M>1 per one new beam]; FFS; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; FFS; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-1-3: - According to the agreement of RAN1 #117, if within a time window(which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. This feature is subject to UE capability. So triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window with M >1 per one new beam should be supported as a UE capability. - According to the agreement of RAN1 #117, reporting the number of M is an optional UE capability. Thus, it should be supported as a separated UE feature group. Proposal 44: Regarding FG 59-1-3 for Rel-19 UEIBM, confirm the highlighted part “[with M>1 per one new beam]”. Proposal 45: Do not merge FG 59-1-3 and FG 59-1-1. |
| ZTE Corporation/Sanechips [5] | - For FG 59-1-3 of triggering event determination, - The description of component should be modified based on the following agreement. Agreement (RAN1#117) Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.. 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam], where M > 1; FFS 59-1-1; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS Per Band; FFS; FFS Applicable only to FR2; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; 59-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; 59-1-1 or 59-1-2; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-1-4; UE-initiated/event-driven beam reporting based on Event-1 and Mode-A; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 for Event-1 2. Support of Event-1 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report for Event-1 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. 59. NR_MIMO_Ph5; 59-1-5; UE-initiated/event-driven beam reporting based on Event-1 and Mode-B; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 2. Support of Event-1 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. 59. NR_MIMO_Ph5; 59-1-6; UE-initiated/event-driven beam reporting based on Event-7 and Mode-A; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 for Event-7 2. Support of Event-7 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. 59. NR_MIMO_Ph5; 59-1-7; UE-initiated/event-driven beam reporting based on Event-1 and Mode-B; 1. Support of UE-initiated/event-driven beam report based on one event instance with M=1 2. Support of Event-2 based measurement and report 3. Support of Mode B UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; Per band; n/a; n/a; n/a; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59-1-3; Determination of event-2 based on more than one event-2 instances; 1. Support of determining event-2 based on determination of M>1 event-2 instances within a time window 2. Supported candidate value(s) of M 3. Support of per candidate beam event instance(s) counting; X-1-1; UEI/ED beam reporting based on determination of more than one event-2 instances is not supported; Per band; N/A; Component 2: candidate value(s): { 2, 3, …}; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; FFS 59-1-1 or 59-1-1a or 59-1-1b; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] | UE-initiated beam management for mode A (FG 59-1-1): This feature group represents mode A in UE-initiated beam management, i.e., when the UE send the beam measurements in response to a CSI request from the NW, and if the UE reports this feature group, the UE supports mode A for event-2, based on a single event instance. The feature group will contain components for how many RSs can be configured for new beams. There is also a placeholder for how the UE determines the serving beam quality, i.e., selecting between Scheme-1 (CSI-RS) and Scheme-2 (SSB). UE-initiated beam management for mode B (FG 59-1-1): This feature group represents mode B in UE-initiated beam management, i.e., when the UE send the beam measurements in a pre-configured resource, and if the UE reports this feature group, the UE supports mode B for event-2, based on a single event instance. The feature group will contain components for how many RSs can be configured for new beams, and a component for the minimum time between the first and second channel (X). There is also a placeholder for how the UE determines the serving beam quality, i.e., selecting between Scheme-1 (CSI-RS) and Scheme-2 (SSB). Triggering event determination (FG 59-1-3): This feature represents the UEs ability to use a counter/window mechanism before initiating the reporting procedure (M>1). As this is now a separate feature group, if the UE supports the counter/window mechanism, it will support it for all supported modes (A and/or B). In other words, it is not possible for the UE to support the counter/window for mode A but not for mode B. - For all these three feature groups, there are notes that states that some of the feature groups may be merged. - In our view, we should have only two feature groups for UE initiated beam management: one for mode A and one for mode B. Each of these feature groups would have several components that describe what the UE supports. This means that FG 59-1-3 should be merged into FG 59-1-1 and FG 59-1-2: Merge FG 59-1-3 into FG 59-1-1 and FG 59-1-2. |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | The support of Event-7 is an optional UE feature. We think a new FG should be created for Event-7 with the following two components: 1) Support of Event-7 based measurement and report; 2) Supported values of Q. A pre-requisite of this FG is 59-1-1. Additionally, Event-7 with Mode B report is supported by joint supporting of this FG and 59-1-2. Proposal 2.5: For UE initiated/event-driven beam management, introduce an a new FG for the support of Event-7with the following components: - Support of Event-7 based measurement and report - Supported values of Q - Note: A pre-requisite of this FG is 59-1-1. Additionally, Event-7 with Mode B report is supported by joint supporting of this FG and 59-1-2. 59. NR_MIMO_Ph5; 59-1-4; UE-initiated/event-driven beam management Event-7; 1. Support of Event-7 based measurement and report 2. Supported values of Q; 59-1-1; yes; n/a; Event-7 is not supported for UEI/ED beam report; FFS; FFS; FR2 only; FFS; Component 2 candidate values: {1, 2, … , 8} FFS: Further partitioning of this FG based one existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] | In additional to above FGs, we propose to add three new FGs based on the following RAN1 agreements. Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported - the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group. - (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups - Subject to separate UE capability. Proposal 4: Introduce the following new FGs according RAN1 agreements: 59. NR_MIMO_Ph5; 59-1-4; UE-initiated/event-driven beam report with Event-1; Support of Event-1 based measurement and report; FG 59-1-1; yes; n/a; UE-initiated/event-driven beam report with Event-1 is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; UE-initiated/event-driven beam report with Event-7; - Support of Event-7 based measurement and report - The maximum number of Q; FG 59-1-1; yes; n/a; UE-initiated/event-driven beam report with Event-7 is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-6; First PUCCH and second PUSCH from different PUCCH groups; Support first PUCCH and second PUSCH from different PUCCH groups; FG 59-1-1; yes; n/a; First PUCCH and second PUSCH from different PUCCH groups is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-1-4; Event-1 based UE-initiated/event-driven beam management; Support of Event-1 based measurement and report; FFS; yes; n/a; Event-1 based UE-initiated/event-driven beam management is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-4; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; Event-7 based UE-initiated/event-driven beam management; Support of Event-7 based measurement and report; FFS; yes; n/a; Event-7 based UE-initiated/event-driven beam management is not supported; FFS; FFS; FFS; FFS; FFS: whether to merge FG 59-1-1 and FG 59-1-5; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - According to the following agreements, FG 59-1-4 and FG 59-1-5 need to be newly added to specify UE features of Event-1 and Event-7 based measurement, respectively. Agreement (RAN1#119) On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed: On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. - Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2. Agreement (RAN1#119) On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation: - Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality. - Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality. - Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs Note: For Event-2, we have the following definition for scheme-1 and scheme-2 - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.. 59. NR_MIMO_Ph5; 59-1-4; UE-initiated/event-driven beam management for Event-1 based measurement and report; - Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a certain threshold based on one event instance within a time window - Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively; 59-1-1; yes; n/a; UE-initiated/event-driven beam management for Event-1 based measurement is not supported; Per Band; FFS; Applicable only to FR2; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; UE-initiated/event-driven beam management for Event-7 based measurement and report; - Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance within a time window one the new beam - Maximum number of the configured RS(s) for new beam measurement in the RS resource set - Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively; 59-1-1; yes; n/a; UE-initiated/event-driven beam management for Event-7 based measurement is not supported; Per Band; FFS; Applicable only to FR2; FFS; Component 1 candidate values for Q: {1, 2, ..., 8} Component 2 candidate values: {1, 2, ..., 64}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-1-4; Event-1 based measurement and report for UE-initiated/event-driven beam management; Support of event-1 based measurement and report.; 59-1-1. 59. NR_MIMO_Ph5; 59-1-5; Event-7 based measurement and report for UE-initiated/event-driven beam management; 1. Support of event-7 based measurement and report. 2. Supported values of Q; 59-1-1. 59. NR_MIMO_Ph5; 59-1-6; Cross-CC beam measurement/report; 1. Support of cross-CC beam measurement/report; 59-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-1-4; Event-1 based measurement and report; Support of Event-1 based measurement and report; 59-1-1 or 59-1-2; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; Event-7 based measurement and report based on Q-th best quality; Support of Event-7 based measurement and report based on Q-th best quality; 59-1-1 or 59-1-2; Values of Q {2, 3, 4, 5, 6, 7}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-6; Cross CC beam reporting for Event-2 with same PUCCH group; Support of cross CC beam reporting for Event-2 with first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group; 59-1-1 or 59-1-2; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-6-1; Cross CC beam reporting for Event-2 with different PUCCH group; Support of cross CC beam reporting for Event-2 with first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group; 59-1-6; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam]; FFS; yes; n/a; Triggering event determination via detecting ≥ M event instances within a time window is not supported; FFS 10; FFS; FFS; FFS; candidate values: {2,…,M}] FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; UE-initiated/event-driven beam management by Event-7; 1. Candidate value(s) of Q (i.e., the activated TCI state with the Q-th best quality) [= {1, 2, …, 8}] 2. Support of UE-initiated/event-driven beam report based on M event instance 3. Support of Mode A and/or Mode B UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6.Support of L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management 7.Support of N reported beams [8.Support of RRC enabled whether current beam is always reported] 9.Support values of time window; 59-1-1; yes; n/a; Event-7 is not supported; Per band; n/a; n/a; n/a; Component 1 candidate values: {1,…,8} Component 2 candidate values: {1,…,M} Component 3 candidate values: {Mode A, Mode B, Mode A and Mode B} [Component 4 candidate values: {1,…,[N}] [Component 7 candidate values: {1,…,[N}] [Component 8 candidate values: {support, not}]; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-1-4; Measurement resource for UE initiated beam report; Per slot limitations: - The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report Memory limitations: - The max number of CSI-RS resources across all CCs configured for UE initiated beam report - The max number of aperiodic CSI-RS resources across all CCs configured for UE initiated beam report - The max number of CSI-RS resources in a CSI-RS resource set configured for new beam for UE initiated beam report Other limitations: - Supported density of CSI-RS (CMR); Yes; n/a; UE initiated beam report is not supported; Per band; n/a; n/a; n/a; Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {0, 4, 8, 16, 32, 64} Component 3: Candidate values {8, 16, 32, 64, 128} Component 4: Candidate values {0, 2, 4, 8, 16, 32, 64} Component 5: Candidate values {2, 4, 8, 16, 32, 64} Component 6: Candidate values {'1 only', '3 only', '1 and 3'} Note: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note: For component 3, 4, 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note: For components 1, 2, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-5; Maximum N for UE initiated beam report; Support of UE initiated beam report with maximum N L1-RSRP values; Yes; n/a; Maximum N=1; Per band; n/a; n/a; n/a; Candidate values {1, 2, 4}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-6; Configuration that current beam is always reported for UE initiated beam report; Support RRC to configure that current beam is always reported for UE initiated beam report; Yes; n/a; RRC to configure that current beam is always reported for UE initiated beam report is not supported; Per band; n/a; n/a; n/a; Note: The reported current beam, if configured, is not counted in the N reported beams; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-7; Beam reporting timing for Mode B UE initiated beam report; For Mode B UE initiated beam report, the minimum number of symbols, X, between the last symbol of report notification on the first PUCCH channel and the first symbol of the second UL channel carrying the UE initiated beam report; Yes; n/a; Per band; n/a; n/a; n/a; Candidate values: different X in symbols can be reported for different SCS 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-8; Q-th best quality for UE initiated beam report event 7; For event 7, quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.; Yes; n/a; Per band; n/a; n/a; n/a; Candidate values: {best, worst, both}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-1-9; Different PUCCH group for the first PUCCH and second PUSCH; Support the first PUCCH and the second PUSCH from different PUCCH group for both Mode A and Mode B; Yes; n/a; The first PUCCH and the second PUSCH must be configured from the same PUCCH group for both Mode A and Mode B; Per BC; n/a; n/a; n/a; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59-1-4; Cross-CC beam measurement and report; 1. Support of cross-CC beam measurement and report for Mode A and/or Mode B; X-1-1; Cross-CC beam measurement and report for UEI/ED beam reporting is not supported; Per band; N/A; Optional with capability signalling. 59-1-5; Additional events to event-2; 1. Support of event-1 and/or event-7 2. Supported candidate value(s) of Q (i.e., the activated TCI state with the Q-th best quality); X-1-1; Additional events to event-2 for UEI/ED beam reporting are not supported; Per band; N/A; Component 1: candidate value(s): {event-1, event-7, both} Note: event-1 is when quality of the current beam is worse than a certain threshold. Note: event-7 is when quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality. Component 2: candidate value(s): {1, 2, 4, 8}; Optional with capability signalling. |
| Qualcomm Incorporated [12] |  |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

## CSI enhancements for up to 128 ports

| 59. NR_MIMO_Ph5 | 59-2-1-1 | Enhanced Type-I SP codebook for 64 ports – Scheme-A | Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources | 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for 64 ports – Scheme-A | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook for 64 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources; 2-352-36; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 64 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook for 64 ports – Scheme-A; - Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources - (CSI processing timeline scaling parameter, CPU occupation counting parameter); FFS 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 64 Tx ports; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] | 59-2-1-1: - According to the agreement in RAN1#116, for the Rel-19 Type-I codebook refinement, the number of CSI-RS ports (i.e., total number of CSI-RS ports across aggregated resources) P=64 is supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE capabilities. Therefore, support of P=64 CSI-RS ports across aggregated resources is a component of FG 59-2-1-1. - For Rel-19 Type-I codebook refinement, two schemes (Scheme A and Scheme B) are agreed for Rel-19 Type I SP codebook refinement. Scheme A is a low feedback overhead codebook with lower performance, while Scheme B is a high feedback overhead codebook with better performance. These two schemes are corresponding to different optimization goals, and they will not be used jointly. Therefore, it is preferred that Scheme A is defined as basic UE feature, and Scheme B is supported as a separate UE feature. - Analogous to the Rel-15 Type I SP codebook FG 2-36, maximum number of ports per resource, maximum number of resources and total number of Tx ports across all CCs should be the component of FG 59-2-1-1. - According to the agreement in RAN1#116bis, all K NZP CSI-RS resources shall be located within 1 slot or 2 consecutive slots, and ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be a separate UE feature. - In RAN1#116bis, 117 and 118bis meetings, two UE capabilities have been introduced for Type-I codebook refinements: capability 1 reuse legacy timeline, OCPU = ceil(P/32), ARC=1; capability 2 scales legacy timeline by ceil(P/32), OCPU = 1, ARC=1. In our view, capability 2 is more relaxed and should be the basic UE feature. Capability 1 timeline is a separate UE feature. - The prerequisite feature group for 59-2-1-1 is FG for CSI report framework (i.e., FG 2-35). Proposal 1: Adopt the following for FG 59-2-1-1: - The components are: - Support of enhanced Type-I SP codebook of P=64 CSI-RS ports across aggregated resources; - Support of Scheme A; - Support of maximum number ports per resource, maximum number of CSI-RS resources and total number of Tx ports across all CCs; - Support of all configured CSI-RS resources within one slot; - Support of capability 2 timeline. - The prerequisite feature group for 59-2-1-1 is FG for CSI report framework (i.e., FG 2-35). Proposal 2: Introduce a new FG for the support of Scheme B for Rel-19 Type-I SP codebook refinement. Proposal 3: Introduce a new FG for the support of all configured CSI-RS resources within two slots for Rel-19 Type-I SP codebook refinement. Proposal 4: Introduce a new FG for the support of capability 1 timeline for Rel-19 Type-I SP codebook refinement. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook; [1. Support of enhanced Type-I SP codebook] 2. Supported codebook scheme 3. Supported #. of antenna ports; FFS; yes; n/a; Enhanced Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component-2 candidate values: {scheme-A, scheme-B, scheme-A and scheme-B} Component-3 candidate values: {64 ports, 48 and 64 ports, 64 and 128 ports, 48 and 64 and 128 ports}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook; 1.Support Scheme-A and Scheme-B 2. Number of ports: {48 (optional), 64 (basic), 128 (optional)}; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …); Optional with capability signalling. |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook for 64 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 64 Tx ports; N/A; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook for 64 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; FFS 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 64 Tx ports; Per band and Per BC; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1; Enhanced Type-I SP codebook for 64 ports – Scheme-A; Support of enhanced Type-I SP codebook for Scheme-A with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for 64 ports – Scheme-A; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-1a | Enhanced Type-I SP codebook for 48 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1a; Enhanced Type-I SP codebook for 48 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1a; Enhanced Type-I SP codebook for 48 ports – Scheme-A; - Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(8,3), (6,4), (8,3) and (6,4)} Component 3 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1a; Enhanced Type-I SP codebook for 48 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1a; Enhanced Type-I SP codebook for 48 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1a; Enhanced Type-I SP codebook for 48 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 48 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-1b | Enhanced Type-I SP codebook for 128 ports – Scheme-A | 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1b; Enhanced Type-I SP codebook for 128 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1b; Enhanced Type-I SP codebook for 128 ports – Scheme-A; - Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(16,4), (8,8), (16,4) and (8,8)} Component 3 candidate values: {(1,4), (4,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1b; Enhanced Type-I SP codebook for 128 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1b; Enhanced Type-I SP codebook for 128 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1b; Enhanced Type-I SP codebook for 128 ports – Scheme-A; 1. Support of enhanced Type-I SP codebook for Scheme-A with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 59-2-1-1; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-A for 128 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-1c | Enhanced Type-I SP codebook for 64 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources | 2-35 | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1c; Enhanced Type-I SP codebook for 64 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources; 2-352-36; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1c; Enhanced Type-I SP codebook for 64 ports – Scheme-B; - Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1c; Enhanced Type-I SP codebook for 64 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1c; Enhanced Type-I SP codebook for 64 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1c; Enhanced Type-I SP codebook for 64 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 2-35; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 64 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-1d | Enhanced Type-I SP codebook for 48 ports – Scheme-B | 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1d; Enhanced Type-I SP codebook for 48 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1d; Enhanced Type-I SP codebook for 48 ports – Scheme-B; - Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(8,3), (6,4), (8,3) and (6,4)} Component 3 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1d; Enhanced Type-I SP codebook for 48 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1d; Enhanced Type-I SP codebook for 48 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1d; Enhanced Type-I SP codebook for 48 ports – Scheme-B; 1. Support of enhanced Type-I SP codebook for Scheme-B with 48 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 48 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-1e | Enhanced Type-I SP codebook for 128 ports – Scheme-b | 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-1c | yes | n/a | Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-1e; Enhanced Type-I SP codebook for 128 ports – Scheme-b; 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports; FFS; FFS; FFS; FFS; Component candidate values: 4-bit bitmap corresponding to {SchemeA RI=1-4, SchemeB RI=1-4, SchemeA RI=5-8, SchemeB RI=5-8}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-1e; Enhanced Type-I SP codebook for 128 ports – Scheme-b; - Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(16,4), (8,8), (16,4) and (8,8)} Component 3 candidate values: {(1,4), (4,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-1e; Enhanced Type-I SP codebook for 128 ports – Scheme-b; 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1e; Enhanced Type-I SP codebook for 128 ports – Scheme-b; 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. Supported maximum rank 4. Max # of CSI-RS resource in a resource set 5. Supported processing capability; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 3 candidate value {4, 5, 6, 7, 8} Component 4 candidate value {1:8} Component 5 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-1e; Enhanced Type-I SP codebook for 128 ports – Scheme-b; 1. Support of enhanced Type-I SP codebook for Scheme-B with 128 Tx ports by aggregating multiple NZP CSI-RS resources 2. Max # of reports across all CCs simultaneously; 59-2-1-1c; yes; n/a; Enhanced Type-I SP codebook is not supported for Scheme-B for 128 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Component 2 candidate values: {1,2,3,4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-2 | Enhanced Type-I MP codebook | [Support of enhanced Type-I MP codebook] | FFS | yes | n/a | Enhanced Type-I MP codebook is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] for 64 Tx ports by aggregating multiple NZP CSI-RS resources; FFS2-40; yes; n/a; Enhanced Type-I MP codebook is not supported for 64 Tx ports; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2a; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] for 48 Tx ports by aggregating multiple NZP CSI-RS resources; FFS2-40; yes; n/a; Enhanced Type-I MP codebook is not supported for 48 Tx ports; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2b; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] for 128 Tx ports by aggregating multiple NZP CSI-RS resources; FFS2-40; yes; n/a; Enhanced Type-I MP codebook is not supported for 128 Tx ports; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] 2. (CSI processing timeline scaling parameter, CPU occupation counting parameter); FFS; yes; n/a; Enhanced Type-I MP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values: {(1,ceil(P/32)), (ceil(P/32),1)}, where P is the total number of CSI-RS resources across the aggregated resources.; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook for 64 ports; Support of enhanced Type-I MP codebook for with 64 Tx ports by aggregating multiple NZP CSI-RS resource; FFS; yes; n/a; Enhanced Type-I MP codebook is not supported for 64 Tx ports.; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2a; Enhanced Type-I MP codebook for 48 ports; Support of enhanced Type-I MP codebook for with 48 Tx ports by aggregating multiple NZP CSI-RS resource; FFS; yes; n/a; Enhanced Type-I MP codebook is not supported for 48 Tx ports.; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2b; Enhanced Type-I MP codebook for 128 ports; Support of enhanced Type-I MP codebook for with 128 Tx ports by aggregating multiple NZP CSI-RS resource; FFS; yes; n/a; Enhanced Type-I MP codebook is not supported for 128 Tx ports.; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| CATT [4] | 59-2-1-2: - Analogous to FG 59-2-1-1, the support of P=64 CSI-RS ports across aggregated resources should be a component of FG 59-2-1-2, and those for P=48 and P=128 are supported as two separate UE features. The maximum number of ports per resource, maximum number of resources and total number of Tx ports should be the component of FG 59-2-1-2. All K NZP CSI-RS resources located ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be a separate UE feature. Capability 2 timeline is more relaxed and should be the basic feature, and Capability 1 timeline is a separate UE feature. - The prerequisite feature group for 59-2-1-1 is FG for CSI report framework (i.e., FG 2-35). Proposal 5: Adopt the following for FG 59-2-1-2: - The components are: - Support of enhanced Type-I MP codebook of P=64 CSI-RS ports across aggregated resources; - Support of maximum number ports per resource, maximum number of CSI-RS resources and total number of Tx ports across all CCs; - Support of all configured CSI-RS resources within one slot; - Support of capability 2 timeline. - The prerequisite feature group for 59-2-1-2 is FG for CSI report framework (i.e., FG 2-35). Proposal 6: Introduce a new FG for the support of all configured CSI-RS resources within two slots for Rel-19 Type-I MP codebook refinement. Proposal 7: Introduce a new FG for the support of capability 1 timeline for Rel-19 Type-I MP codebook refinement. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; [1. Support of enhanced Type-I MP codebook] 2. Supported #. of antenna ports; FFS; yes; n/a; Enhanced Type-I MP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component-2 candidate values: {64 ports, 48 and 64 ports, 64 and 128 ports, 48 and 64 and 128 ports}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; 1.Number of ports: {48 (optional), 64 (basic), 128 (optional)}; 2-35; yes; n/a; Enhanced Type-I MP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …); Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook]; FFS; yes; n/a; Enhanced Type-I MP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook for 64 ports; Support of enhanced Type-I MP codebook for 64 ports by aggregating multiple NZP CSI-RS resources; 2-35; yes; n/a; Enhanced Type-I MP codebook for 64 ports is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2a; Enhanced Type-I MP codebook for 48 ports; Support of enhanced Type-I MP codebook for 48 ports by aggregating multiple NZP CSI-RS resources; 59-2-1-2; yes; n/a; Enhanced Type-I MP codebook for 48 ports is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2b; Enhanced Type-I MP codebook for 128 ports; Support of enhanced Type-I MP codebook for 128 ports by aggregating multiple NZP CSI-RS resources; 59-2-1-2; yes; n/a; Enhanced Type-I MP codebook for 128 ports is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; Support of enhanced Type-I MP codebook with 64 Tx ports by aggregating multiple NZP CSI-RS resources; 2-35; yes; n/a; Enhanced Type-I MP codebook is not supported for 64 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2a; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] with 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-2; yes; n/a; Enhanced Type-I MP codebook is not supported for 48 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-2b; Enhanced Type-I MP codebook; [Support of enhanced Type-I MP codebook] with 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-2; yes; n/a; Enhanced Type-I MP codebook is not supported for 128 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; - Support 64 ports - Additionally supported number of aggregated ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported maximum number of panels - Max # of CSI-RS resource in a resource set - Supported processing capability; yes; n/a; Enhanced Type-I MP codebook is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values bitmap {48 ports, 128 ports} Component 3 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 4 candidate value {2, 4} Component 5 candidate value {1:8} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-2; Enhanced Type-I MP codebook; 1.[Support of enhanced Type-I MP codebook] 2. {# of Tx ports in one report, Max # of reports and total # of Tx ports} across all CCs simultaneously 3. Supported number of panels, Ng; 2-35; yes; n/a; Enhanced Type-I MP codebook is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-1-2 - At least support following components for Type-I MP codebook enhancement. - Supported number of panels, Ng - Max number of ports per panel - {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Add a new FG for timeline as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources - For the ‘Type’ of each FG, similar as legacy, ‘per band and per BC’ can be applied. |
| Ericsson [14] | One open issue related to this FG is whether to partition this FG into multiple FGs. It should be noted that the FGs for enhanced single panel Type I codebook are already partitioned according to number of ports (e.g., 48, 64, and 128) and according to schemes (Scheme A vs Scheme B). Along this line, it may be reasonable to also split this FG into multiple FGs according to the number of ports. Split FG 59-2-1-2 (Support of enhanced Type-I MP codebook) into the following FGs: - FG 59-2-1-2: Support of enhanced Type-I MP codebook for 64 Tx ports - FG 59-2-1-2a: Support of enhanced Type-I MP codebook for 48 Tx ports - FG 59-2-1-2b: Support of enhanced Type-I MP codebook for 128 Tx ports |

| 59. NR_MIMO_Ph5 | 59-2-1-3 | Extended Rel-16 eType-II codebook for 64 Tx ports | Support of extended Rel-16 eType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources | 16-3a | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 64 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook for 64 Tx ports; [Support of extended Rel-16 eType-II codebook] for 64 Tx ports by aggregating multiple NZP CSI-RS resources; 16-3a; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 64 Tx ports; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook for 64 Tx ports; [Support of extended Rel-16 eType-II codebook] for 64 Tx ports by aggregating multiple NZP CSI-RS resources 2. (CSI processing timeline scaling parameter, CPU occupation counting parameter); FFS 16-3a; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 64 Tx ports; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] | 59-2-1-3: - According to the agreement in RAN1#116, for the Rel-19 Type-II codebook refinement based on Rel-16 eType II regular codebook, P=64 is supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE features. Therefore, FG 59-2-1-3 is the basic feature for the support of P=64 ports extended Rel-16 eType II codebook. Support of P=48 and P=128 extended Rel-16 eType II codebook can be two separate FGs and 59-2-1-3 is their prerequisite feature group. The prerequisite feature group for 59-2-1-3 is FG for CSI report framework (i.e., FG 2-35). - Analogous to the Rel-16 regular eType II FG 16-3a, maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports and number of PMI per subband R=1 should be the component. In RAN1#116b, 117 and 118b meetings, two UE capabilities have been introduced for Type-II codebook refinements (except based on Rel-18 Type-II Doppler): capability 1 reuse legacy timeline, OCPU = ceil(P/32), ARC=1; capability 2 scales legacy timeline by ceil(P/32), OCPU = 1, ARC=1. In our view, capability 2 is more relaxed and should be the basic feature. According to the agreement in RAN1#116，all legacy Parameter Combinations from Rel-16 eType II (regular) are supported. Therefore, support of parameter combinations 1-6 should be included. Analogous to the Rel-16 regular eType II FG 16-3a, support of rank 1, 2 should be a basic feature. - Analogous to the Rel-16 16-3a-1, a new FG for the support of PMI sub-bands with R=2 should be introduced. Its prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). The component is maximum number of ports per resource, maximum number of CSI-RS resources and total number of ports to support extended Rel-16 eType II codebook for R=2. - Analogous to the Rel-16 16-3a-2, a new FG for the support of parameter combinations 7-8 should be introduced. Its prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). - Analogous to the Rel-16 16-3a-3, a new FG for the support of rank 3, 4 should be introduced. Its prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). - According to the agreement in RAN1#116bis, all K NZP CSI-RS resources shall be located within 1 slot or 2 consecutive slots, and ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be a separate UE feature. Proposal 8: Adopt the following for FG 59-2-1-3: - The prerequisite feature group is FG for CSI report framework (i.e., 2-35). - The components are: - Support P=64 CSI-RS ports across aggregated resources; - Support maximum number ports per resource, maximum number of CSI-RS resources total number of Tx ports for extended Rel-16 eType II codebook, support for PMI subband R=1; - Support of parameter combinations 1-6; - Support of rank 1,2; - Support of capability 2 timeline； - Support of all configured CSI-RS resources within one slot. Proposal 9: Introduce two new FGs for P=48, 128 ports extended Rel-16 eType II codebook, respectively. - The prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3) - The components are: - Support P=48, 128 CSI-RS ports across aggregated resources, respectively; - Support maximum number ports per resource, maximum number of CSI-RS resources, total number of Tx ports for extended Rel-16 eType II codebook, support for PMI subband R=1. Proposal 10: Introduce a new FG for the support of PMI sub-bands with R=2. - The prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). - The component is: - Support maximum number of ports per resource, maximum number of CSI-RS resources and total number of Tx ports to support extended Rel-16 eType II codebook for R=2. Proposal 11: Introduce a new FG for the support of parameter combinations 7-8. The prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). Proposal 12: Introduce a new FG for the support of rank 3,4. The prerequisite feature group is FG for extended Rel-16 eType-II codebook (i.e., FG 59-2-1-3). Proposal 13: Introduce a new FG for capability 1 timeline for extended Rel-16 eType II codebook. Proposal 14: Introduce a new FG for the support of all configured CSI-RS resources within two slots for extended Rel-16 eType II codebook. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook; [1. Support of extended Rel-16 eType-II codebook] 2. Supported #. of antenna ports; FFS; yes; n/a; Extended Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component-2 candidate values: {64 ports, 48 and 64 ports, 64 and 128 ports, 48 and 64 and 128 ports}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook; 1.Number of ports: {48 (optional), 64 (basic), 128 (optional)}; 2-35; yes; n/a; Extended Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …); Optional with capability signalling. |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook for 64 Tx ports; Support of extended Rel-16 eType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources; 16-3a; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 64 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook for 64 Tx ports; - Support of extended Rel-16 eType-II codebook] for 64 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability; FFS 16-3a; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 64 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-3; Extended Rel-16 eType-II codebook for 64 Tx ports; Support of extended Rel-16 eType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources; 16-3a; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 64 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-1-3 - Add a new FG for timeline as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources - For each FG, as usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. - For the ‘Type’ of each FG, similar as legacy, ‘per band and per BC’ can be applied. |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-3a | Extended Rel-16 eType-II codebook for 48 Tx ports | Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 48 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-3a; Extended Rel-16 eType-II codebook for 48 Tx ports; Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 48 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(8,3), (6,4), (8,3) and (6,4)} Component 3 candidate values: {(1,2), (2,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-3a; Extended Rel-16 eType-II codebook for 48 Tx ports; Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 48 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-3a; Extended Rel-16 eType-II codebook for 48 Tx ports; - Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 48 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-3a; Extended Rel-16 eType-II codebook for 48 Tx ports; Support of extended Rel-16 eType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 48 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-3b | Extended Rel-16 eType-II codebook for 128 Tx ports | Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources | 59-2-1-3 | yes | n/a | Extended Rel-16 eType-II codebook is not supported for 128 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-3b; Extended Rel-16 eType-II codebook for 128 Tx ports; Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources - Supported values of (N1, N2) - (CSI processing timeline scaling parameter, CPU occupation counting parameter); 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 128 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(16,4), (8,8), (16,4) and (8,8)} Component 3 candidate values: {(1,4), (4,1)}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-3b; Extended Rel-16 eType-II codebook for 128 Tx ports; Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 128 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-3b; Extended Rel-16 eType-II codebook for 128 Tx ports; 1. Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources - Support of parameter combination 1-6 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=1 - Supported processing capability; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 128 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-3b; Extended Rel-16 eType-II codebook for 128 Tx ports; Support of extended Rel-16 eType-II codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-3; yes; n/a; Extended Rel-16 eType-II codebook is not supported for 128 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-4 | Extended Rel-17 FeType-II codebook with 64 Tx ports | Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources | 23-9-1 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] | 59-2-1-4: - According to the agreement in RAN1#116，for the Rel-19 Type II codebook refinement based on Rel-17 FeType II PS codebook, PCSI-RS=64 is supported as a part of the respective basic feature, while PCSI-RS=48 is supported as a separate UE capability. Therefore, FG 59-2-1-4 with PCSI-RS=64 is the basic feature. Support of PCSI-RS=48 can be a separate FG with 59-2-1-4 as its prerequisite feature group. The prerequisite feature group for 59-2-1-4 is FG for CSI report framework (i.e., FG 2-35). - Analogous to the Rel-17 FG 23-9-1 (basic feature of Rel-17 FeType II PS), maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports and R=1, M=1 should be included as the component. In RAN1#116b,117 and 118b meetings, two UE capabilities have been introduced for Type-II codebook refinements (except based on Rel-18 Type-II Doppler): capability 1 reuse legacy timeline, OCPU = ceil(P/32), ARC=1; capability 2 scales legacy timeline by ceil(P/32), OCPU = 1, ARC=1. In our view, capability 2 is more relaxed and should be the basic feature. - Analogous to the Rel-17 FG 23-9-2 (Support of M=2 and R=1 for FeType II), a new FG should be introduced for the support of M=2 and R=1 for extended Rel-17 FeType II PS codebook. Its prerequisite feature group is FG for extended Rel-17 FeType-II codebook (i.e., FG 59-2-1-4). Maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports and R=1, M=1 should be included as the component. - Analogous to the Rel-17 FG 23-9-4 (Support of R = 2 for FeType II), a new FG should be introduced for the support of R=2 for extended Rel-17 FeType II PS codebook. Its prerequisite feature group is the FG for the support of M=2 and R=1 for extended Rel-17 FeType II PS codebook. Similar as Rel-17 FG 23-9-1, the support of R=2 for extended Rel-17 FeType II PS codebook is also a component in this new FG. Maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports and R=2, M=2 should be included as the component. - According to the agreement in RAN1#116bis，all legacy Parameter Combinations except for Parameter Combination 8 from Rel-17 FeType II PS are supported. Analogous to the Rel-17 FG 23-9-1, Parameter Combinations with M=1 should be supported. Therefore, support of parameter combinations 1-4 should be included in 59-2-1-4. Regarding the new FG for the support of M=2 and R=1 for extended Rel-17 FeType II PS codebook, Parameter Combination 5-7 are supported. - Analogous to the Rel-17 FG 23-9-1, support of rank 1, 2 should be a basic feature. Support of rank 3, 4 is a separate feature with 59-2-1-4 as its prerequisite feature group. - According to the agreement in RAN1#116bis, all K NZP CSI-RS resources shall be located within 1 slot or 2 consecutive slots, and ‘within 1 slot’ should be basic feature and ‘within 2 consecutive slots’ should be a separate UE feature. Proposal 15: Adopt the following for FG 59-2-1-4: - The prerequisite feature group is FG for CSI report framework (i.e., FG 2-35). - The components are: - Support P=64 CSI-RS ports across aggregated resources; - Support maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports for extended Rel-17 FeType-II PS codebook, support for PMI subband R=1, number of FD basis M=1; - Support of parameter combinations 1-4; - Support of rank 1,2; - Support of capability 2 timeline; - Support of all configured CSI-RS resources within one slot. Proposal 16: Introduce a new FG for P=48 ports for extended Rel-17 FeType-II PS codebook. - The prerequisite feature group is the basic FG for extended Rel-17 FeType-II PS codebook (i.e., FG 59-2-1-4). - The component is: - Support P=48 CSI-RS ports across aggregated resources; - Support maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports for extended Rel-17 FeType-II PS codebook, support for PMI subband R=1, number of FD basis M=1. Proposal 17: Introduce a new FG for the support of M=2 and R=1 for extended Rel-17 FeType II PS codebook. - The prerequisite feature group is the basic FG for extended Rel-17 FeType-II PS codebook (i.e., FG 59-2-1-4). - The components are: - Support maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports for extended Rel-17 FeType-II PS codebook, support for PMI subband R=1, number of FD basis M=2; - Support of parameter combinations 5-7. Proposal 18: Introduce a new FG for the support of R=2 for extended Rel-17 FeType II PS codebook. - The prerequisite feature group is the FG for the support of M=2 and R=1 for extended Rel-17 FeType II PS codebook. - The components are: - Support of R=2 for extended Rel-17 FeType II PS codebook; - Support maximum number of ports per resource, maximum number of CSI-RS resources, total number of Tx ports for extended Rel-17 FeType-II PS codebook, support for PMI subband R=2, number of FD basis M=2. Proposal 19: Introduce a new FG for the support of rank 3,4. The prerequisite feature group is 59-2-1-4. Proposal 20: Introduce a new FG for capability 1 timeline for extended Rel-17 FeType II PS codebook. Proposal 21: Introduce a new FG for the support of all configured CSI-RS resources within two slots for extended Rel-17 FeType II PS codebook. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-4; Extended Rel-17 FeType-II codebook; [1. Support of extended Rel-17 FeType-II codebook] 2. Supported #. of antenna ports; FFS; yes; n/a; Extended Rel-17 FeType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component-2 candidate values: {64 ports, 48 and 64 ports, 64 and 128 ports, 48 and 64 and 128 ports}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-4; Extended Rel-17 FeType-II codebook; 1.Number of ports: {48 (optional), 64 (basic)}; 2-35; yes; n/a; Extended Rel-17 FeType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …); Optional with capability signalling. |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-4; Extended Rel-17 FeType-II codebook with 64 Tx ports; Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources; 23-9-1; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports; N/A; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Parameter Combination 8 from Rel-17 FeType-II PS is not supported for extended Rel-17 FeType-II codebook.; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-4; Extended Rel-17 FeType-II codebook with 64 Tx ports; 1. Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources] - Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability; FFS 23-9-1; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-4; Extended Rel-17 FeType-II codebook with 64 Tx ports; Support of extended Rel-17 FeType-II codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resources; 23-9-1; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 64 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-1-4 - Add a new FG for timeline as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources - For each FG, as usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. - For the ‘Type’ of each FG, similar as legacy, ‘per band and per BC’ can be applied. |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-4a | Extended Rel-17 FeType-II codebook with 48 Tx ports | [Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources] | 59-2-1-4 | yes | n/a | Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-4a; Extended Rel-17 FeType-II codebook with 48 Tx ports; Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources; 59-2-1-4; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports; N/A; N/A; N/A; N/A; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-4a; Extended Rel-17 FeType-II codebook with 48 Tx ports; 1. Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources] 2. Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability; 59-2-1-4; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-4a; Extended Rel-17 FeType-II codebook with 48 Tx ports; [Support of extended Rel-17 FeType-II codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resources]; 59-2-1-4; yes; n/a; Extended Rel-17 FeType-II codebook is not supported with 48 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-5 | Extended Rel-18 eType-II Doppler codebook for 64 Tx ports | Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups | 40-3-2-1 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook for 64 Tx ports; [Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups] 2. CSI processing timeline scaling parameter 3. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 4. Value of Y for CPU occupation (OCPU = Y.Kdopp), when A-CSI-RS is configured for CMR; FFS 40-3-2-1; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component 2 candidate values: {1, 2} Component 3 candidate values: {1, 2, 3, 4, 6} Component 4 candidate values: {1, 2, 3, 4, 6}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] | 59-2-1-5: - According to the agreement in RAN1#116, for the Rel-19 Type-II codebook refinement based on Rel-18 eType II Doppler regular codebook, P=64 is supported as a part of the respective basic feature, while those for P=48 and P=128 are supported as two separate UE features. Therefore, FG 59-2-1-5 with P=64 is the basic feature. Support of P=48 and P=128 are two separate FGs with 59-2-1-5 as their prerequisite feature group. The prerequisite feature group for 59-2-1-5 is FG for CSI report framework (i.e., FG 2-35). - Analogous to the Rel-18 FG 40-3-2-1 (Support of Rel-16-based Doppler CSI), the 10 components in 40-3-2-1 should also be included in 59-2-1-5. In component 5, since P=64 is supported for 59-2-1-5, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}. In RAN1#116b and 118 meetings, two UE capabilities have been introduced for Type II codebook refinements based on Rel-18 eType II Doppler: capability 1 reuse legacy Rel-18 eType II Doppler timeline, OCPU is scaled by ceil(P/32) on Rel-18 eType II Doppler OCPU and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8, ARC is the same as Rel-18 eType II Doppler ARC; capability 2 scales legacy Rel-18 eType II Doppler timeline by ceil(P/32), OCPU is the legacy Rel-18 eType II Doppler OCPU and replace K with KDOPP for AP-CSI-RS, ARC is the same as Rel-18 eType II Doppler ARC. In our view, capability 2 is more relaxed and should be the basic capability. Therefore, in component 8, when A-CSI-RS, the value of Y for CPU occupation (OCPU = Y. KDOPP) is configured for CMR. - Analogous to the Rel-18 FG 40-3-2-1b(Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based Doppler measurement), according to the agreement in RAN1#116b, a UE can be configured with KDOPP = {4, 8, 12} CSI-RS resource groups for the purpose of aperiodic CMR. Therefore, a new FG should be introduced for the support of maximum number of aperiodic CSI-RS resource groups that can be configured in the same CSI report setting for extended Rel-18 eType II Doppler measurement. - Analogous to the Rel-18 FG 40-3-2-2(Support R=2 for Rel-16-based Doppler codebook), a new FG should be introduced for the support of R=2 for extended Rel-18 eType II Doppler codebook. Its prerequisite feature group is FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). A list of supported combinations {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously, with R=2 should be included as the component. - Analogous to the Rel-18 FG 40-3-2-8, a new FG should be introduced for the support of L=6 for extended Rel-18 eType II Doppler codebook. Its prerequisite feature group is FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). The component is the support of L=6 for extended Rel-18 eType II Doppler codebook. - Analogous to the Rel-18 FG 40-3-2-9, a new FG should be introduced for the support of rank equals 3 and 4 for extended Rel-18 eType II Doppler codebook. Its prerequisite feature group is FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). The component is the support of rank equals 3 and 4 for extended Rel-18 eType II Doppler codebook. - According to the agreement in RAN1#116bis, the time separation between the first resources from two consecutive groups (=m) can be configured from {1, 2}, the CSI-RS resources in a resource group are located in one slot should be basic feature and the CSI-RS resources in a resource group span two consecutive slots should be a separate UE feature. Proposal 22: Adopt the following for FG 59-2-1-5: - The prerequisite feature group is FG for CSI report framework (i.e., FG 2-35). - The components are: - Support P=64 CSI-RS ports across aggregated resources; - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’); - Support of PMI subband R=1 for extended Rel-18 eType II Doppler codebook; - Support parameter combinations with L=2,4; - Support for rank = 1,2; - A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously; - Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR; - Value of Y for CPU occupation (OCPU = Y. KDOPP), when A-CSI-RS is configured for CMR; - Support for the size of DD-basis, N4=1; - Scaling factor for active resource counting Kp; - Support of capability 2 timeline; - Support of all CSI-RS resources in a resource group are located in one slot. Proposal 23: Introduce a new FG for P=48 ports for extended Rel-18 eType II Doppler codebook. - The prerequisite feature group is the basic FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). - The component is: - A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports } across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. Proposal 24: Introduce a new FG for the support of maximum number of aperiodic CSI-RS resource groups that can be configured in the same CSI report setting for extended Rel-18 eType II Doppler measurement. - The prerequisite feature group is the basic FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). - The component is: - Maximum number of aperiodic CSI-RS resource groups that can be configured in the same CSI report setting for extended Rel-18 eType II Doppler measurement. Proposal 25: Introduce a new FG for the support of R=2 for extended Rel-18 eType II Doppler codebook. - The prerequisite feature group is the basic FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). - The component is: - A list of supported combinations {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously, with R=2. Proposal 26: Introduce a new FG for the support of L=6 for extended Rel-18 eType II Doppler codebook. - The prerequisite feature group is FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). - The component is: - Support of L=6 for extended Rel-18 eType II Doppler codebook. Proposal 27: Introduce a new FG for the support of rank equals 3 and 4 for extended Rel-18 eType II Doppler codebook. - The prerequisite feature group is the basic FG for extended Rel-18 eType II Doppler codebook (i.e., FG 59-2-1-5). - The component is: - Support of rank equals 3 and 4 for extended Rel-18 eType II Doppler codebook. Proposal 28: Introduce a new FG for capability 1 timeline for extended Rel-18 eType II Doppler codebook. Proposal 29: Introduce a new FG for the support of all CSI-RS resources in a resource group span two consecutive slots for extended Rel-18 eType II Doppler codebook. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook; [1. Support of extended Rel-18 Type-II Doppler codebook] 2. Supported #. of antenna ports; FFS; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …) Component-2 candidate values: {64 ports, 48 and 64 ports, 64 and 128 ports, 48 and 64 and 128 ports}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook; 1.Number of ports: {48 (optional), 64 (basic), 128 (optional)}; 2-35; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., Scheme-A and Scheme-B separation, number of ports, …); Optional with capability signalling. |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook for 64 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups; FFS 40-3-2-1; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 64, K = 2 (each resource 32 ports) and 4 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook for 64 Tx ports; - Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups] - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp; FFS 40-3-2-1; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-5; Extended Rel-18 eType-II Doppler codebook for 64 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 64 Tx ports by aggregating multiple NZP CSI-RS resource groups; 40-3-2-1; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 64 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-1-5 - Add a new FG for timeline as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources - For each FG, as usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. - For the ‘Type’ of each FG, similar as legacy, ‘per band and per BC’ can be applied. |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-5a | Extended Rel-18 eType-II Doppler codebook for 48 Tx ports | Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-5a; Extended Rel-18 eType-II Doppler codebook for 48 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups 2. Supported values of (N1, N2) 3. CSI processing timeline scaling parameter 4. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 5. Value of Y for CPU occupation (OCPU = Y.Kdopp), when A-CSI-RS is configured for CMR; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(8,3), (6,4), (8,3) and (6,4)} Component 3 candidate values: {1, 2} Component 4 candidate values: {1, 2, 3, 4, 6} Component 5 candidate values: {1, 2, 3, 4, 6}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-5a; Extended Rel-18 eType-II Doppler codebook for 48 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 48, K = 2 (each resource 24 ports) and 3 (each resource 16 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-5a; Extended Rel-18 eType-II Doppler codebook for 48 Tx ports; 1. Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups - Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-5a; Extended Rel-18 eType-II Doppler codebook for 48 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 48 Tx ports by aggregating multiple NZP CSI-RS resource groups; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 48 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-5b | Extended Rel-18 eType-II Doppler codebook for 128 Tx ports | Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups | 59-2-1-5 | yes | n/a | Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-1-5b; Extended Rel-18 eType-II Doppler codebook for 128 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups 2. Supported values of (N1, N2) 3. CSI processing timeline scaling parameter 4. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 5. Value of Y for CPU occupation (OCPU = Y.Kdopp), when A-CSI-RS is configured for CMR; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports; FFS; FFS; FFS; FFS; Component 2 candidate values: {(16,4), (8,8), (16,4) and (8,8)} Component 3 candidate values: {1, 4} Component 4 candidate values: {1, 2, 3, 4, 8, 12} Component 5 candidate values: {1, 2, 3, 4, 8, 12}; Optional with capability signalling. |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-5b; Extended Rel-18 eType-II Doppler codebook for 128 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports; N/A; N/A; N/A; N/A; For P (or PCSI-RS) = 128, K = 4 (each resource 32 ports); Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-5b; Extended Rel-18 eType-II Doppler codebook for 128 Tx ports; 1. Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups 2. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) - Support PMI subband R=1 - Support parameter combinations with L=2,4 - Support rank = 1,2 - Support 64 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - Supported processing capability - Value of Y for CPU occupation when P/SP-CSI-RS is configured for CMR - Value of Y for CPU occupation when A-CSI-RS is configured for CMR - Support for the size of DD-basis, N4=1 - Scaling factor for active resource counting Kp; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports; Per band and Per BC; n/a; n/a; n/a; Component 6 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} Component 7 candidate value {Capability 1, Capability 2} Component 8 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: For component of processing capability Capability 1: Legacy timeline OCPU = YN4ceil(P/32) ), when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPPceil(P/32)), when A-CSI-RS is configured for CMR Capability 2: Scale the legacy timeline by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = YN4, when P/SP-CSI-RS is configured for CMR OCPU = Y KDOPP, when A-CSI-RS is configured for CMR Note: maximum OCPU is 8 Note: KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports Note: legacy processing timeline is reported in 59-2-5j; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-5b; Extended Rel-18 eType-II Doppler codebook for 128 Tx ports; Support of extended Rel-18 Type-II Doppler codebook for 128 Tx ports by aggregating multiple NZP CSI-RS resource groups; 59-2-1-5; yes; n/a; Extended Rel-18 Type-II Doppler codebook is not supported for 128 Tx ports; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-6 | CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS] | [Support of 2-slot CSI-RS resource aggregation] | FFS | yes | n/a | 2-slot resource aggregation is not supported | FFS | FFS | FFS | FFS | [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration for periodic, semi-persistent, and aperiodic CSI-RS; Support of 2-slot CSI-RS resource aggregation; FFS; yes; n/a; 2-slot resource aggregation is not supported; FFS; FFS; FFS; FFS; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5]; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS]; [Support of 2-slot CSI-RS resource aggregation]; FFS 59-2-1-1, 2, 3, 4, or 5; yes; n/a; 2-slot resource aggregation is not supported, i.e., only 1-slot resource aggregation is supported; FFS; FFS; FFS; FFS; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5]; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS]; [Support of 2-slot CSI-RS resource aggregation]; 59-2-1-1, 2, 3, 4, 5; yes; n/a; 2-slot resource aggregation is not supported; FFS; FFS; FFS; FFS; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5]; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS]; [Support of 2-slot CSI-RS resource aggregation]; FFS; yes; n/a; 2-slot resource aggregation is not supported; FFS; FFS; FFS; FFS; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5]; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-1-6; Aggregated NZP CSI-RS resources within 2 consecutive slots; Support K aggregated NZP CSI-RS resources within 2 consecutive slots; FFS; yes; n/a; K aggregated NZP CSI-RS resources within 2 consecutive slots is not supported; FFS; FFS; FFS; FFS; Note: K aggregated NZP CSI-RS resources within one slot is basic feature; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration; [Support of 2-slot two consecutive slot CSI-RS resource aggregation]; 59-2-1-1, 2, 3, 4, 5; yes; n/a; 2-slot Two consecutive slot resource aggregation is not supported; N/A; N/A; N/A; N/A; 1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration for aperiodic CSI-RS; [Support of 2-slot CSI-RS resource aggregation]; FFS; yes; n/a; 2-slot resource aggregation is not supported; FFS; FFS; FFS; FFS; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 2, 3, 4, 5]; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-6; CSI-RS resource slot offset configuration [for periodic [and/or] aperiodic CSI-RS]; [Support of 2-slot CSI-RS resource aggregation]; 2-33 and {59-2-1-1 or 59-2-1-1c}; yes; n/a; 2-slot resource aggregation is not supported; Per-band and per-BC; N/A; N/A; N/A; [1-slot resource aggregation is a basic feature and incorporated in 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 2, 3, 3a, 3b, 4, 4a, 5, 5a, 5b]; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-1-6 - There is no need to distinguish periodic and aperiodic CSI-RS. 1-slot resource aggregation is the basic feature and 2-slot resource aggregation is the optional UE feature. Thus, we suggest removing the yellow part in the name of the 59-2-1-6. |
| Ericsson [14] | With this FG, we support to confirm the component “[Support of 2-slot CSI-RS resource aggregation]”by removing the brackets. We all support 1-slot resource aggregation as part of basic features in 59-2-1-1, 2, 3, 4, 5. Furthermore, the feature can be applied to periodic, semi-persistent, and aperiodic CSI-RS resources. For FG 59-2-1-6, support the following: - Support 1-slot resource aggregation as part of basic features in 59-2-1-1, 2, 3, 4, 5 - component “Support of 2-slot CSI-RS resource aggregation” by removing the brackets - Support the feature for periodic, semi-persistent, and aperiodic resources |

| 59. NR_MIMO_Ph5 | 59-2-1-7 | Group-specific 3-bit scaling factors for up to 128 ports | Support of group-specific 3-bit scaling factors | FFS | yes | n/a | Group-specific 3-bit scaling factors is not supported | FFS | FFS | FFS | FFS | Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: 3-bit scaling applies only to the Type-I SP codebook | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; Support of group-specific 3-bit scaling factors; FFS; yes; n/a; Group-specific 3-bit scaling factors is not supported; FFS; FFS; FFS; FFS; Component candidate values {RI=1, RI=2, RI= 1 and 2}; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; 1. Support of group-specific 3-bit scaling factors 2. Supported ranks; FFS 59-2-1-1; yes; n/a; Group-specific 3-bit scaling factors is not supported; FFS; FFS; FFS; FFS; [Component-2 candidate values: {’rank 1’, ‘rank 1 and 2’}] Note: 3-bit scaling applies only to the Type-I SP codebook FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate FGs for RI=1 and RI=2; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; Support of group-specific 3-bit scaling factors; 2-35; yes; n/a; Group-specific 3-bit scaling factors is not supported; FFS; FFS; FFS; FFS; [Candidate values: {’rank 1’, ’rank 2’, ‘rank 1 and 2’}] Note: 3-bit scaling applies only to the Type-I SP codebook FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate FGs for RI=1 and RI=2; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit power offset scaling factors for up to 128 ports; Support of group-specific 3-bit power offset scaling factors; FFS; yes; n/a; Group-specific 3-bit power offset scaling factors is not supported; FFS; FFS; FFS; FFS; Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: 3-bit power scaling applies only to the Type-I SP codebook; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; Support of group-specific 3-bit scaling factors; 59-2-1-1; yes; n/a; Group-specific 3-bit scaling factors is not supported; N/A; N/A; N/A; N/A; Candidate values: {’rank1’, ‘rank1 and rank2’} Note: 3-bit scaling applies only to the Type-I SP codebook; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; Support of group-specific 3-bit scaling factors; yes; n/a; Group-specific 3-bit scaling factors is not supported; Per band and Per BC; n/a; n/a; n/a; [Candidate values: {’rank-1’, ‘rank-1 and rank-2’}] Note: 3-bit scaling applies only to the Type-I SP codebook FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate FGs for RI=1 and RI=2; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1-7; Group-specific 3-bit scaling factors for up to 128 ports; Support of group-specific 3-bit scaling factors; 59-2-1-1 or 59-2-1-1c; yes; n/a; Group-specific 3-bit scaling factors is not supported; Per-band and per-BC; N/A; N/A; N/A; Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: 3-bit scaling applies only to the Type-I SP codebook; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-1-8 | SRS Port Grouping | Support of SRS port grouping | FFS | yes | n/a | SRS Port Grouping is not supported | FFS | FFS | FFS | FFS | [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | For UE feature of SRS port grouping, there’s no need to further separate between different number of receiving antennas, because the supported number of receiving antennas is also reported by UE and the SRS port grouping capability is associated with that number. Therefore, the following UE features are proposed: Proposal 3.4.1: Introduce the following change for SRS port grouping as highlighted in yellow. 59. NR_MIMO_Ph5; 59-2-1-8; SRS Port Grouping; Support of SRS port grouping; FFS; yes; n/a; SRS Port Grouping is not supported; FFSPer UE; FFSn/a; FFSn/a; FFSn/a; [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-8a; SRS Port Grouping for xT6R; Support of SRS port grouping; FFS; yes; n/a; SRS Port Grouping is not supported; FFS; FFS; FFS; FFS; [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-8b; SRS Port Grouping for xT8R; Support of SRS port grouping; FFS; yes; n/a; SRS Port Grouping is not supported; FFS; FFS; FFS; FFS; [Only applicable for reportQuantity = ‘cri-RI-CQI’]; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-1-8; SRS Port Grouping; 1.Support of SRS port grouping 2. For xT6R 3. For xT8R; 2-35; yes; n/a; SRS Port Grouping is not supported; FFS; FFS; FFS; FFS; [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-1-8; SRS Port Grouping; Support of SRS port grouping; FFS; yes; n/a; SRS Port Grouping is not supported; FFS; FFS; FFS; FFS; [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-1-8; SRS Port Grouping; Support of SRS port grouping; 2-38; yes; n/a; SRS Port Grouping is not supported; FFS; FFS; FFS; FFS; Note: Only applicable for reportQuantity = ‘cri-RI-CQI’ Candidate values: {xT8R, xT6R}; Optional with capability signalling. |
| Xiaomi [9] | 60. NR_MIMO_Ph5; 59-2-1-8; Timeline for Rel-19 Type-I and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports; Timeline relaxation parameter; 59-2-1-1, 59-2-1-1a, 59-2-1-1b, 59-2-1-1c, 59-2-1-1d, 59-2-1-1e, 59-2-1-2, 59-2-1-2a, 59-2-1-2b, 59-2-1-3, 59-2-1-3a, 59-2-1-3b, 59-2-1-4, 59-2-1-4a, 59-2-1-5, 59-2-1-5a, 59-2-1-5b; Yes; N/A; Relaxed timeline is not supported; Per band and Per-BC; No; N/A; N/A; Component candidate value for Rel-19 Type I codebook : { Z1∙P32, Z1'∙P32} Component candidate value for Rel-19 Type I codebook : { Z2∙P32, Z2'∙P32}; Optional with capability signaling. 61. NR_MIMO_Ph5; 59-2-1-9; CPU occupation for Rel-19 Type-I and Type-II codebook refinements for 48, 64, and 128 CSI-RS ports; CPU occupation parameter,; 59-2-1-1, 59-2-1-1a, 59-2-1-1b, 59-2-1-1c, 59-2-1-1d, 59-2-1-1e, 59-2-1-2, 59-2-1-2a, 59-2-1-2b, 59-2-1-3, 59-2-1-3a, 59-2-1-3b, 59-2-1-4, 59-2-1-4a, 59-2-1-5, 59-2-1-5a, 59-2-1-5b; Yes; N/A; Additional CPU occupation is not supported; Per band and Per-BC; No; N/A; N/A; Component candidate value for Rel-19 Type I /Type II codebook : OCPU =P32; Optional with capability signaling. 62. NR_MIMO_Ph5; 59-2-1-10; CPU occupation for Rel-19 Type-II Doppler codebook refinements for 48, 64, and 128 CSI-RS ports; CPU occupation parameter; 59-2-1-5, 59-2-1-5a, 59-2-1-5b; Yes; N/A; Additional CPU occupation is not supported; Per band and Per-BC; No; N/A; N/A; Component candidate value for Rel-19 Type I /Type II codebook : OCPU = OCPU∙P32; Optional with capability signaling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-8; SRS Port Grouping; Support of SRS port grouping; yes; n/a; SRS Port Grouping is not supported; Per FSPC; n/a; n/a; n/a; Candidate value, bitmap { xT6R, xT8R } FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR MIMO phase 5; 59-2-1-8; SRS port grouping; Support of SRS port grouping; 2-58; YES; N/A; SRS port grouping is not supported; Per-band and per-BC; N/A; N/A; N/A; UE antenna ports are divided as two groups for two-CW reception: • SRS port group 0, corresponding to CW0, comprises the even y/2 out of y ports; and • SRS port group 1, corresponding to CW1, comprises the odd y/2 out of y ports Note: The y SRS ports of one or more sets of SRS resources for antenna switching (xTyR) are indexed in an ascending order according to SRS resource set ID, SRS resource ID within each set, and port number within each SRS resource [Only applicable for reportQuantity = ‘cri-RI-CQI’] FFS: Further partitioning of this FG based on existing and future agreements, e.g., separate SRS port group for xT6R and xT8R related SRS antenna switching; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 59-2-1-8 - We don’t think separate FG is needed for xT6R and xT8R.. |
| Ericsson [14] | For this FG, we support to confirm that this feature is only applicable for CSI reporting with reportQuantity = ‘cri-RI-CQI’. For FG 59-2-1-8, confirm that this feature is only applicable for CSI reporting with reportQuantity = ‘cri-RI-CQI’. |

| 59. NR_MIMO_Ph5 | 59-2-2-1 | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | [1. M={1,2,3,4}] [2. KS={1,2, …, 8}] | FFS | yes | n/a | Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | Proposal 3.2.1: For CRI(s)-based CSI reporting, the following component should be included in the 59-2-2-1, 59-2-2-2: - Maximum number of reported resources in the CSI-RS resource set Based on CRI(s)-based CSI reporting, at least two resources must be configured, KS =1 should be removed in candidate values for maximum number of resources in the CSI-RS resource set. In addition, maximum number of Tx ports in the CSI-RS resource and total number of Tx ports in the CSI-RS resource set are subject to UE capability. So, the corresponding component should be included in FG 59-2-2-1 and 59-2-2-2. Proposal 3.2.2: For FG 59-2-2-1and 59-2-2-2, remove the KS =1 in candidate values of maximum number of resources in the CSI-RS resource set. Proposal 3.2.3: For CRI(s)-based CSI reporting, the following component should be included in the 59-2-2-1, 59-2-2-2: - Maximum number of Tx ports in the CSI-RS resource - Maximum number of resources in the CSI-RS resource set - Total number of Tx ports in the CSI-RS resource set 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}] 1. Support of maximum number of reported resources in the CSI-RS resource set to support Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook 2. {Maximum number of Tx ports in one CSI-RS resource, maximum number of resources and total number of Tx ports in the CSI-RS resource set} to support Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; FFS; Yes; N/A; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not support; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 1 candidate values: {1,2,3,4} Component 2 candidate values: -- maximum number of Tx ports in the resource: {4,8,12,16,24,32} --maximum number of resources in the CSI-RS resource set: {2,3,4,5,6,7,8} - -maximum total number of Tx ports in the CSI-RS resource set: {8, … ,128}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}] 3.Number of ports per resource; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 3 candidate values {4,8,12,16,24,32}; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; Support for maximum number of reported quadruplets M; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; Candidate values: : M∈1,2,3,4 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-2-1: - According to the agreement in RAN1#116，if the number of CSI-RS resource Ks =2,3,4, the maximum number of ports P=32 per resource; if the number of CSI-RS resource Ks=5,6,7,8, the maximum number of ports P=16 per resource. KS and the number of ports per resource are UE capability. - According to the agreement in RAN1#116bis, the value range for M={1,2,…,min(4, Ks)}. The maximum value of M is subject to UE capability. - The prerequisite feature groups for Rel-15 Type I SP codebook based multi-CRI reporting is the FG for Type I single panel codebook (i.e., FG 2-36). Proposal 30: Adopt the following for FG 59-2-2-1: - The components are: - Support maximum number of CSI-RS resources, maximum number of ports per resource, total number of Tx ports; - Support maximum number of CRI/RI/PMI/CQI reporting in one CSI reporting instance. - The prerequisite feature group is the FG for Type I single panel codebook, (i.e., FG 2-36). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}] 1. Support of Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook 2. Supported maximum #. of reported CRIs 3. Supported maximum #. of CSI-RS resources; FFS 2-36; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: {1 to 4} Component-3 candidate values: {1 to 8}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}]; 2-36; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}]; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Candidate value: Maximum M is {1, 2, 3, 4}; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; FFS; FFS; FFS; FFS; Component-1 Candidate values: {1,2,3,4} Component-2 Candidate values: FFS; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; 1. M={1, …, min(4,KS)} 2. KS={1,2, …, 8}; 2-35; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; N/A; N/A; N/A; N/A; KS={2,3, 4} the maximum number of ports per NZP CSI-RS resource is 32. KS={5,6, 7, 8} the maximum number of ports per NZP CSI-RS resource is 16.; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; - Maximum number of Ks configured CSI-RS resources for channel measurement - Maximum number of M reported CRI; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; Per band and Per BC; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4, 5, 6, 7, 8} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-2-1a; Configuration of MR CSI-RS resource for Rel-15 Type-I SP (single panel) codebook enhancement for CRI-based CSI refinement; - Support the NW configuration of MR (<M) of KS CSI-RS resources to be selected as part of reporting the M CRI(s), for A-CSI only, for Rel-15 Type-I SP (single panel) codebook enhancement for CRI-based CSI refinement - Maximum number of MR; Yes; n/a; NW configuration of MR (<M) of KS CSI-RS resources to be selected as part of reporting the M CRI(s) is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {1, 2}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-2-1; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook; [1. M={1,2,3,4}] [2. KS={1,2, …, 8}]; 2-36; yes; n/a; Hybrid BF (CRI-based) with Rel-15 Type-I SP codebook is not supported; Per-band and per-BC; N/A; N/A; N/A; Note: If Ks={2,3,4}, max # ports per resource is 32; If Ks={5,6,7,8}, max # ports per resource is 16 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-2-1 - For Ks, which is the number of CMRs configured for measurement, it is also related to the maximum number of ports per resource. For example, when maximum number of ports per resource is 32, candidate values of Ks include {1,2,3,4}, while when maximum number of ports per source is 16, candidate values of Ks include {1,2,3,4,5,6,7,8}. Thus, we think there should be two components for Ks for two cases, respectively. Correspondingly, there should be two components for M for the two cases, respectively. - A restriction should be added that M <= Ks. - As usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. For the ‘Type’, similar as legacy, ‘per band and per BC’ can be applied.59-2-2-2 - M=2 is supported only when following condition is met: a maximum of 16 ports per resource, R=1 only, and a maximum UCI payload of 1706 bits. Thus, the condition should be added for the case of M=2. - A restriction should be added that M <= Ks. - As usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. For the ‘Type’, similar as legacy, ‘per band and per BC’ can be applied.59-2-2-3 - 0 should be added as one candidate value for MR. - For the ‘Type’, similar as legacy, ‘per band and per BC’ can be applied. |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-2-1, 2, 3, agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-2-2 | Hybrid BF (CRI-based) with Rel-16 eType-II codebook | [1. M={1,2}] [2. KS={1,2,3,4}] | FFS | yes | n/a | Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | Proposal 3.2.1: For CRI(s)-based CSI reporting, the following component should be included in the 59-2-2-1, 59-2-2-2: - Maximum number of reported resources in the CSI-RS resource set Based on CRI(s)-based CSI reporting, at least two resources must be configured, KS =1 should be removed in candidate values for maximum number of resources in the CSI-RS resource set. In addition, maximum number of Tx ports in the CSI-RS resource and total number of Tx ports in the CSI-RS resource set are subject to UE capability. So, the corresponding component should be included in FG 59-2-2-1 and 59-2-2-2. Proposal 3.2.2: For FG 59-2-2-1and 59-2-2-2, remove the KS =1 in candidate values of maximum number of resources in the CSI-RS resource set. Proposal 3.2.3: For CRI(s)-based CSI reporting, the following component should be included in the 59-2-2-1, 59-2-2-2: - Maximum number of Tx ports in the CSI-RS resource - Maximum number of resources in the CSI-RS resource set - Total number of Tx ports in the CSI-RS resource set 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}] 1.Support of maximum number of reported resources in the CSI-RS resource set to support Hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1 2. {Maximum number of Tx ports in the CSI-RS resource, maximum number of resources and total number of Tx ports in the CSI-RS resource set} to support Hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1; FFS; Yes; N/A; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not support; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 1 candidate values: {1,2} Component 2 candidate values: -- maximum number of Tx ports in the resource: {4,8,12,16,24,32} --maximum number of resources in the CSI-RS resource set: {2,3,4} - -maximum total number of Tx ports in the CSI-RS resource set: {8, … ,128}; Optional with capability signalling. |
| MediaTek Inc. [2] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}] 3.Number of ports per resource; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 3 candidate values {4,8,12,16}; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; Support for the maximum number of NZP CSI-RS resources to measure KS and the maximum number of reported quadruplets M.; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; Candidate values: Ks∈1,2,3,4, M∈1,2. FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-2-2: - According to the agreement in RAN1#116 and 116bis, M={1,2} are supported. When M=1, the maximum value of Ks is {1, 2, 3, 4} and subject to UE capability. When M=2, the maximum value of Ks is {1, 2, 3, 4} with maximum of P=16 ports per resource, R=1 only, and a maximum UCI payload of 1706 bits. The maximum value of M is subject to UE capability. - The prerequisite feature group for Rel-16 eType II codebook based multi-CRI reporting is the FG for regular eType II codebook (i.e., FG 16-3a). Proposal 31: Adopt the following for FG 59-2-2-2: - The components are: - Support maximum number of CSI-RS resources, maximum number of ports per resource and total number of Tx ports, support for PMI subband R=1; - Support maximum number of CRI/RI/PMI/CQI reporting in one CSI reporting instance. - The prerequisite feature group is the FG for regular eType II codebook, (i.e., FG 16-3a). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}] 1. Support of Hybrid BF (CRI-based) with Rel-16 eType-II codebook 2. Supported maximum #. of reported CRIs 3. Supported maximum #. of CSI-RS resources; FFS 16-3a; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: {1 to 2} Component-3 candidate values: {1 to 4}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}]; 16-3a; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}]; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Candidate value: {M=1 only, M=1 and 2}; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; 1. The maximal supported number of CRI report M 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; FFS; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; FFS; FFS; FFS; FFS; Component-1 Candidate values: {1,2} Component-2 Candidate values: FFS; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; 1. M={1,2} 2. KS={1,2,3,4}; 16-3a; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; N/A; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements KS={2,3, 4} the maximum number of ports per NZP CSI-RS resource is 32. M=2 with a maximum of 16 ports per resource, R=1 only; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; - Maximum number of Ks configured CSI-RS resources for channel measurement - Maximum number of M reported CRI; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; Per band and Per BC; n/a; n/a; n/a; Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-2a; Configuration of MR =1 CSI-RS resource for Rel-16 eType-II codebook enhancement for CRI-based CSI refinement; - Support the NW configuration of MR =1 of KS CSI-RS resources to be selected as part of reporting the M CRI(s), for A-CSI only, for Rel-16 eType-II codebook enhancement for CRI-based CSI refinement; Yes; n/a; NW configuration of MR =1 of KS CSI-RS resources to be selected as part of reporting the M CRI(s) is not supported; Per band and Per BC; n/a; n/a; n/a; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-2-2; Hybrid BF (CRI-based) with Rel-16 eType-II codebook; [1. M={1,2}] [2. KS={1,2,3,4}]; 16-3a; yes; n/a; Hybrid BF (CRI-based) with Rel-16 eType-II codebook is not supported; Per-band and per-BC; N/A; N/A; N/A; Note: max # ports per resource is 16 FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-2-1, 2, 3, agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-2-3 | Configuration of MR always-reported resources | [1. For Rel-15 Type-I SP, MR={1,2}] [2. For Rel-16 eType-II, MR={1}] | FFS | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | Proposal 3.2.4: For CRI(s)-based CSI reporting, the following component should be included in the 59-2-2-3: - Support MR = {1,2} for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook - Support MR = {1} for hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1 NR MIMO Phase 5; 59-2-2-3; Configuration of MR always-reported resources; [1. For Rel-15 Type-I SP, Support MR={1,2} for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook] [2. For Rel-16 eType-II, Support MR={1} for hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1]; FFS; Yes; N/A; Configuration of MR always-reported resources is not support, i.e. MR=0; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; Support for the maximum number of resources always reported MR.; FFS; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; FFS; FFS; FFS; FFS; Candidate values: For Rel-15 Type I SP, MR∈1,2; for Rel-16 eType-II, MR∈1. FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-2-3: - According to the agreement in RAN1#118, for Rel-15 Type I SP codebook, MR={1, 2}; for Rel-16 eType II codebook, MR={1}. Since the configuration of MR is coupled with codebook type of multi-CRI reporting, it is preferred to split 59-2-2-3 into two separate FGs for Rel-15 Type I SP codebook and Rel-16 eType II codebook, respectively. This is analogous to Rel-18 UE feature separation between FG 40-3-1-15 (dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook) and FG 40-3-1-16 (dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook). Proposal 32: Split current 59-2-2-3 into two FGs, i.e., 59-2-2-3, 59-2-2-4: - 59-2-2-3 is the configuration of MR always-reported CSI-RS resources for Rel-15 Type I SP codebook. The prerequisite feature group is FG for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook (i.e., FG 59-2-2-1). the components are: - Support the configuration of MR always-reported CSI-RS resources for for Rel-15 Type I SP codebook; - Support maximum number of always-reported CSI-RS resources. - 59-2-2-4 is the configuration of MR always-reported CSI-RS resources for Rel-16 eType II codebook. The prerequisite feature group is FG for hybrid BF (CRI-based) with Rel-16 eType-II codebook (i.e., FG 59-2-2-2). the components are: - Support the configuration of MR always-reported CSI-RS resources for for Rel-15 Type I SP codebook. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; [1. For Rel-15 Type-I SP, MR={1,2}] [2. For Rel-16 eType-II, MR={1}] 1. Support of configuration of MR always-reported resources 2. Supported maximum value of MR for Rel-15 Type-I SP 3. Supported maximum value of MR for Rel-16 eType-II; FFS 59-2-2-1 or 2; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: {1, 2} Component-3 candidate values: {1}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; [1. For Rel-15 Type-I SP, MR={1,2}] [2. For Rel-16 eType-II, MR={1}]; 59-2-2-1, 2; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; [1. For Rel-15 Type-I SP, MR={1,2}] [2. For Rel-16 eType-II, MR={1}]; FFS; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; 1. For Rel-15 Type-I SP, MR={1,2}; 59-2-2-1; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; N/A; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Apple [10] |  |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-2-3; Configuration of MR always-reported resources; [1. For Rel-15 Type-I SP, MR={1,2}<M] [2. For Rel-16 eType-II, MR={1}<M]; 59-2-2-1 or 59-2-2-2; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-2-1, 2, 3, agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report | [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] | FFS | yes | n/a | CJTC Dd report is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; 1. AD - Configured quantization range for CJTC Dd reporting. 2. MD- Configured resolution (number of steps) for the quantization alphabet for CJTC Dd reporting.; FFS; yes; n/a; CJTC Dd report is not supported; FFS; FFS; FFS; FFS; AD={0.5CP, CP} MD={32, 64, 128, 256} CP: Cyclic prefix FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-1: - In components part, one of the components of 59-2-3-1 is support of CJTC Dd report. - According to the agreement of RAN1#116bis, NTRP TRS resource sets are used for CJTC Dd report. In our opinion, the maximum number of NTRP should be defined as one of the components of FG 59-2-3-1.According to the agreement of RAN1#116bis, the reference CSI-RS resource set nref is selected by the UE and reported as a part of the CJTC Dd report. The nref report should be supported as one of the components. According to the agreement of RAN1#117 and RAN1#118, the candidate values of the dynamic range AD are {0.5CP, CP}and the candidate values of the resolution parameter MD are {32, 64, 128, 256}. Since there isn’t any agreement that the 1-bit indicator dn associated with the n-th TRS resource set may be omitted, it should be reported once triggering CJTC Dd report. The inside/outside range for the 1-bit indicator dn is equal to [0, CP].The supported maximum values of ADand MD shall be defined as components of FG 59-2-3-1. The inside/outside range for the 1-bit indicator dn associated with the n-th TRS resource set shall be defined as one of the components of FG 59-2-3-1. - According to the agreement of RAN1#117, OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for CJTC Dd report. In our opinion, the values of X should be defined as one of the components of FG 59-2-3-1. - The prerequisite feature group for 59-2-3-1 is FG for CSI report framework (i.e., FG 2-35). Proposal 33: Adopt the following for FG 59-2-3-1: - The components include - Support of CJTC Dd report; - The maximum number of configured TRS resource sets for CJTC Dd report; - Support of the reference CSI-RS resource set nref report; - Support of maximum value of dynamic range AD; - Support of maximum value of resolution parameter MD; - Support of the inside/outside range for the 1-bit indicator dn associated with the n-th TRS resource set; - Value of X for CPU occupation counting for CJTC Dd report (OCPU =X.NTRP). - The prerequisite feature group for 59-2-3-1 is FG for CSI report framework (i.e., FG 2-35). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] 1. Support of CJTC Dd report 2. Supported dynamical range of DO 3. Supported #. of DO quantization levels 4. Supported CPU occupation factor X; FFS; yes; n/a; CJTC Dd report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: bitmap {0.5CP, CP} Component-3 candidate values: bitmap {32, 64, 128, 256} Component-4 candidate values: {1, 2}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}]; 2-35; yes; n/a; CJTC Dd report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}]; FFS; yes; n/a; CJTC Dd report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; 1. Support Delay offset reporting for CJT calibration 2. Supported value of X for OCPU calculation; FFS; yes; n/a; CJTC Dd report is not supported; FFS; FFS; FFS; FFS; Component-2 Candidate values: {1,2} Note：OCPU =X.NTRP; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; - Support delay offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for delay offset report - Maximum number of configured TRS resource sets for delay offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for delay offset report per CC - Maximum number of simultaneously active CSI-RS resources for delay offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - 6. Value of X for CPU occupation (OCPU=XNTRP); yes; n/a; CJTC Dd report is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-1a; Maximum number of TRS resource sets in a report configuration for delay offset report; - Maximum number of TRS resource sets in a report configuration for delay offset report; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {2, 3, 4}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-1b; Maximum number of delay offset report settings per BWP; - Maximum number of delay offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {1, 2, 3, 4}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-1c; Quantization for delay offset report; - Maximum number of quantization points MD for delay offset report - Quantization range AD for delay offset report; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Component 1 candidate values: {32, 64, 128, 256} Component 2 candidate values: bitmap {0.5CP, CP}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-1; CJTC Dd report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}]; 2-35; yes; n/a; CJTC Dd report is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-1/-2/-3 - In components, - Brackets can be removed (i.e., the existing components can be confirmed). - The number of NZP CSI-RS resource sets (with trs-Info configuration), i.e., KTRS, can be reported. Its value range should be {2, 3, 4} as we do not think it makes any sense to consider only 1 TRP for this feature. - “Triplets”, which can be “Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC” can also be reported. - Type - Type for this FG should be “per band and per BC”, analogous to Rel-18 TDCP. - Prerequisite FG - 2-35 |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-3-1, 2, 3, 4, 5 agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report | [1. AF={0.1ppm, 0.2ppm}] [2. MF={16, 32, 256}] | FFS | yes | n/a | CJTC FO report is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; 1. AF- Configured quantization range for CJTC FO reporting. 2. MF- Configured resolution (number of steps) for the quantization alphabet for CJTC FO reporting.; FFS; yes; n/a; CJTC FO report is not supported; FFS; FFS; FFS; FFS; AF={0.1ppm, 0.2ppm} MF={16, 32, 256} ppm: parts per million of the carrier frequency. FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-2: - In components part, one of the components of 59-2-3-1 is support of CJTC FO report. - According to the agreement of RAN1#116bis, NTRP TRS resource sets are used for CJTC FO report. In our opinion, the maximum number of NTRP should be defined as one of the components of FG 59-2-3-2. - According to the agreement of RAN1#116bis, the reference CSI-RS resource set nref is selected by the UE and reported as a part of the CJTC FO report The nref report should be supported as one of the components. - According to the agreement of RAN1#117 and RAN1#118, the candidate values of the dynamic range AF are {0.1ppm, 0.2ppm}and the candidate values of the resolution parameter MD are {16, 32, 256}. The supported maximum values of AFand MF shall be defined as components of FG 59-2-3-2. - According to the agreement of RAN1#117, OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for CJTC FO report. In our opinion, the values of X should be defined as one of the components of FG 59-2-3-2. - The prerequisite feature group for 59-2-3-2 is FG for CSI report framework (i.e., FG 2-35). Proposal 34: Adopt the following for FG 59-2-3-2: - The components include - Support of CJTC FO report; - The maximum number of configured TRS resource sets for CJTC FO report; - Support of the reference CSI-RS resource set nref report; - Support of maximum value of dynamic range AF; - Support of maximum value of resolution parameter MF; - Value of X for CPU occupation (OCPU =X.NTRP). - The prerequisite feature group for 59-2-3-2 is FG for CSI report framework (i.e., FG 2-35). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; [1. AF={0.1ppm, 0.2ppm}] [2. MF={16, 32, 256}] 1. Support of CJTC FO report 2. Supported dynamical range of FO 3. Supported #. of FO quantization levels 4. Supported CPU occupation factor X; FFS; yes; n/a; CJTC FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: bitmap {0.1ppm, 0.2ppm} Component-3 candidate values: bitmap {16, 32, 256} Component-4 candidate values: {1, 2}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; [1. AF={0.1ppm, 0.2ppm}] [2. MF={16, 32, 256}]; 2-35; yes; n/a; CJTC FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; [1. AF={0.1ppm, 0.2ppm}] [2. MF={16, 32, 256}]; FFS; yes; n/a; CJTC FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; 1. Support frequency offset reporting for CJT calibration 2. Supported value of X for OCPU calculation; FFS; yes; n/a; CJTC FO report is not supported; FFS; FFS; FFS; FFS; Component-2 Candidate values: {1,2} Note：OCPU =X.NTRP; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; - Support frequency offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for frequency offset report - Maximum number of configured TRS resource sets for frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for frequency offset report per CC - Maximum number of simultaneously active CSI-RS resources for frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC 6. Value of X for CPU occupation (OCPU=XNTRP); yes; n/a; CJTC FO report is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-2a; Maximum number of TRS resource sets in a report configuration for frequency offset report; - Maximum number of TRS resource sets in a report configuration for frequency offset report; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {2, 3, 4}. 59. NR_MIMO_Ph5; 59-2-3-2b; Maximum number of frequency offset report settings per BWP; - Maximum number of frequency offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {1, 2, 3, 4}. 59. NR_MIMO_Ph5; 59-2-3-2c; Quantization for frequency offset report; - Maximum number of quantization points MFO for frequency offset report - Quantization range AFO for frequency offset report; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Component 1 candidate values: {16, 32, 256} Component 2 candidate values: bitmap {0.1ppm, 0.2ppm}. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-2; CJTC FO report; [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}]; 2-35; yes; n/a; CJTC FO report is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-1/-2/-3 - In components, - Brackets can be removed (i.e., the existing components can be confirmed). - The number of NZP CSI-RS resource sets (with trs-Info configuration), i.e., KTRS, can be reported. Its value range should be {2, 3, 4} as we do not think it makes any sense to consider only 1 TRP for this feature. - “Triplets”, which can be “Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC” can also be reported. - Type - Type for this FG should be “per band and per BC”, analogous to Rel-18 TDCP. - Prerequisite FG - 2-35 |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-3-1, 2, 3, 4, 5 agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-3 | CJTC wideband PO report | [1. Mphi= {16, 32}] | FFS | yes | n/a | CJTC PO report is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; 1. Mphi- Configured resolution (number of steps) for the quantization alphabet for CJTC WB PO reporting.; FFS; yes; n/a; CJTC PO report is not supported; FFS; FFS; FFS; FFS; Mphi= {16, 32} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-3: - In components part, one of the components of 59-2-3-3 is support of CJTC wideband PO report. - According to the agreement of RAN1#116bis, the reference CSI-RS resource nref is selected by the UE and reported as a part of the CJT calibration report. The nref report should be supported as one of the components of 59-2-3-3. - According to the agreement of RAN1#116, the candidate values of resolution parameter M for PO report are {16, 32}. The resolution depends on UE capability. Therefore, the supported maximum value of Mshall be defined as one of the components of FG 59-2-3-3. - According to the agreements of RAN1#116bis and RAN1#117, NTRP NZP CSI-RS resources are used for PO report. The maximum number of NTRP should be defined as one of the components of FG 59-2-3-3. - According to the agreement of RAN1#118bis, OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for CJTC PO report. The values of X should be defined as a one of the components of FG 59-2-3-3. - The prerequisite feature group for 59-2-3-3 is FG for CSI report framework (i.e., FG 2-35). Proposal 35: Adopt the following for FG 59-2-3-3: - The components include - Support of CJTC wideband PO report; - Support of the reference CSI-RS resource nref report; - Support of maximum value of resolution parameter M; - The maximum number of configured CSI-RS resources in one resource set for PO report; - Value of X for CPU occupation (OCPU =X.NTRP) ; - The prerequisite feature group for 59-2-3-3 is FG for CSI report framework (i.e., FG 2-35). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; [1. Mphi= {16, 32}] 1. Support of CJTC wideband PO report 2. Supported #. of PO quantization levels 3. Supported CPU occupation factor X; FFS; yes; n/a; CJTC PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: bitmap {16, 32} Component-3 candidate values: {1, 2}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; [1. Mphi= {16, 32}]; 2-35; yes; n/a; CJTC PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; [1. Mphi= {16, 32}]; FFS; yes; n/a; CJTC PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; 1. Support wideband phase offset reporting for CJT calibration 2. Supported value of X for OCPU calculation; FFS; yes; n/a; CJTC PO report is not supported; FFS; FFS; FFS; FFS; Component-2 Candidate values: {1,2} Note：OCPU =X.NTRP; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; - Support wideband phase offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured CSI-RS resources for phase offset report - Maximum number of configured CSI-RS resources for phase offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for phase offset report per CC - Maximum number of simultaneously active CSI-RS resources for phase offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Value of X for CPU occupation (OCPU=XNTRP); yes; n/a; CJTC PO report is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-3; CJTC wideband PO report; [1. Mphi= {16, 32}]; 2-35; yes; n/a; CJTC PO report is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-1/-2/-3 - In components, - Brackets can be removed (i.e., the existing components can be confirmed). - The number of NZP CSI-RS resource sets (with trs-Info configuration), i.e., KTRS, can be reported. Its value range should be {2, 3, 4} as we do not think it makes any sense to consider only 1 TRP for this feature. - “Triplets”, which can be “Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC” can also be reported. - Type - Type for this FG should be “per band and per BC”, analogous to Rel-18 TDCP. - Prerequisite FG - 2-35 |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-3-1, 2, 3, 4, 5 agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-4 | CJTC subband PO report | [1. Mphi= {16, 32}] [2. Subband size= {1, 2, 4, 8, 16} RB] | FFS | yes | n/a | CJTC subband PO report is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; 1. Mphi- Configured resolution (number of steps) for the quantization alphabet for CJTC SB PO reporting. 2. Subband size- configured subband size for the CJTC subband PO report.; FFS; yes; n/a; CJTC subband PO report is not supported; FFS; FFS; FFS; FFS; Mphi= {16, 32} Subband size= {1, 2, 4, 8, 16} RB RB: Resource Block FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-4: - In components part, features shall be described in words, e.g., one of the components of 59-2-3-4 is support of CJTC subband PO report. - Analogous to the FG 59-2-3-3, the features related to nref report, the maximum number of configured CSI-RS resources, and the maximum value of X are all components of FG 59-2-3-4. - The prerequisite feature group for 59-2-3-4 is FG for CJTC wideband PO report (i.e., FG 59-2-3-3). Proposal 36: Adopt the following for FG 59-2-3-4: - The components include - Support of CJTC subband PO report; - Support of the reference CSI-RS resource nref report; - Support of maximum value of resolution parameter M; - The maximum number of configured CSI-RS resources in one resource set for PO report; - Value of X for CPU occupation (OCPU =X.NTRP) ; - The prerequisite feature group for 59-2-3-4 is FG for CJTC wideband PO report (i.e., FG 59-2-3-3). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; [1. Mphi= {16, 32}] [2. Subband size= {1, 2, 4, 8, 16} RB] 1. Support of CJTC subband PO report 2. Supported #. of PO quantization levels 3. Supported subband size 4. Supported CPU occupation factor X; FFS; yes; n/a; CJTC subband PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: bitmap {16, 32} Component-3: candidate values: bitmap {1RB, 2RB, 4RB, 8RB, 16RB} Component-4 candidate values: {1, 2}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; [1. Mphi= {16, 32}] [2. Subband size= {1, 2, 4, 8, 16} RB]; 2-35; yes; n/a; CJTC subband PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; [1. Mphi= {16, 32}] [2. Subband size= {1, 2, 4, 8, 16} RB]; FFS; yes; n/a; CJTC subband PO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; 1. Support subband phase offset reporting for CJT calibration 2. Supported value of X for OCPU calculation 3. Supported subband size for phase reporting 4. Supported maximal subband number NSB-P; 59-2-3-3; yes; n/a; CJTC subband PO report is not supported; FFS; FFS; FFS; FFS; Component-2 Candidate values: {1,2} Component-3 Candidate values: FFS Component-4 Candidate values: FFS Note：OCPU =X.NTRP; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; - Support subband phase offset report for aperiodic standalone CJT calibration reporting - Maximum number of reported subbands - Minimum subband size; FFS; yes; n/a; CJTC subband PO report is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {2, 4, 6, 8, 10, 12, 14, 16} Component 3 candidate values: {1, 2, 4, 8,6}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-4; CJTC subband PO report; [1. Mphi= {16, 32}] [2. Subband size= {1, 2, 4, 8, 16} RB]; 59-2-3-3; yes; n/a; CJTC subband PO report is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-4 - Prerequisite FG should be 59-2-3-4 - Other than that, the same as for FG 59-2-3-3 can be considered. |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-3-1, 2, 3, 4, 5 agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-5 | CJTC Dd+FO report | [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}] | FFS | yes | n/a | CJTC Dd+FO report is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; 1. AD- Configured quantization range for CJTC Dd reporting. 2. MD- Configured resolution (number of steps) for the quantization alphabet for CJTC Dd reporting. 3. AF- Configured quantization range for CJTC FO reporting. 4. MF- Configured resolution (number of steps) for the quantization alphabet for CJTC FO reporting.; FFS; yes; n/a; CJTC Dd+FO report is not supported; FFS; FFS; FFS; FFS; AD={0.5CP, CP} MD={32, 64, 128, 256} AF={0.1ppm, 0.2ppm} MF={16, 32, 256} CP: Cyclic prefix ppm: parts per million of the carrier frequency. FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-5: - In components part, one of the components of 59-2-3-1 is support of CJTC Dd+FO report. - According to the agreement of RAN1#116bis, NTRP TRS resource sets are used for CJTC Dd+FO report. In our opinion, the maximum number of NTRP should be defined as one of the components of FG 59-2-3-2. - According to the agreement of RAN1#116bis, the reference CSI-RS resource set nref1 and nref2 is selected by the UE and reported as a part of the CJT calibration report. The nref1 and nref2 report should be supported as one of the components. - According to the agreement of RAN1#116bis, in addition to reporting one type of CJT calibration report in one report, support reporting CJT Dd+FO in one report. The CJT Dd+FO report should conclude AD, MD, dn, AF and MF. - According to the agreement of RAN1#116, OCPU =2X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type. In our opinion, the values of X for CPU occupation counting for CJTC Dd+FO report should be defined as a one of the components of FG 59-2-3-2. - In our opinion, if a UE supports CJT Dd+FO report, it should also support CJTC Dd report and CJTC FO report separately. For the prerequisite of 59-2-3-5, it should be FG for CJTC Dd report (i.e., 59-2-3-1) and CJTC FO report (i.e., 59-2-3-2). Proposal 37: Adopt the following for FG 59-2-3-5: - The components include - Support of CJTC Dd+FO report; - The maximum number of configured TRS resource sets for CJTC Dd+FO report; - Support of the reference CSI-RS resource sets nref1 and nref2 report; - Support of maximum value of dynamic range AD; - Support of maximum value of resolution parameter MD; - Support of the inside/outside range for the 1-bit indicator dn associated with the n-th TRS resource set; - Support of maximum value of dynamic range AF; - Support of maximum value of resolution parameter MF; - Value of X for CPU occupation counting for CJTC Dd+FO report (OCPU =2X.NTRP). - The prerequisite of 59-2-3-5 should be FG for CJTC Dd report (i.e., 59-2-3-1) and CJTC FO report (i.e., 59-2-3-2). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}] 1. Support of CJTC Dd+FO report 2. Supported dynamical range of DO 3. Supported #. of DO quantization levels 4. Supported dynamical range of FO 5. Supported #. of FO quantization levels 6. Supported CPU occupation factor X; FFS 59-2-3-1 and 2; yes; n/a; CJTC Dd+FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: bitmap {0.5CP, CP} Component-3 Candidate values bitmap: {32, 64, 128, 256} Component-4 candidate values: bitmap {0.1ppm, 0.2ppm} Component-5 candidate values: bitmap {16, 32, 256} Component-6 candidate values: {1, 2}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}]; 2-35; yes; n/a; CJTC Dd+FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}]; FFS; yes; n/a; CJTC Dd+FO report is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; 1. Support joint report of delay offset and frequency offset for CJT calibration 2. Supported value of X for OCPU calculation; FFS; yes; n/a; CJTC Dd+FO report is not supported; FFS; FFS; FFS; FFS; Component-2 Candidate values: {1,2} Note：OCPU =2X.NTRP; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; - Support joint delay and frequency offset report for aperiodic standalone CJT calibration reporting - Maximum number of configured TRS resource sets for joint delay and frequency offset report - Maximum number of configured TRS resource sets for joint delay and frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC - Maximum number of simultaneously active CSI-RS resources for joint delay and frequency offset report per CC - Maximum number of simultaneously active CSI-RS resources for joint delay and frequency offset report across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC 6. Value of X for CPU occupation (OCPU=2XNTRP); FFS; yes; n/a; CJTC Dd+FO report is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {2, 4, 6, 8, 10, 12} Component 3 candidate values: {2, 4, 6, 8, 12, … 64} Component 4 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32} Component 5 candidate values: {2, 4, 6, 8, 12, 16, 20, 24, 28, 32, …, 64} Component 6 candidate values: {1, 2}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-5; CJTC Dd+FO report; [1. AD={0.5CP, CP}] [2. MD={32, 64, 128, 256}] [3. AF={0.1ppm, 0.2ppm}] [4. MF={16, 32, 256}]; 59-2-3-1, 59-2-3-2; yes; n/a; CJTC Dd+FO report is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-5 - Prerequisite FG should be 59-2-3-1 and 59-2-3-2 - Other than that, the same proposals as for 59.2-3-1/-2 can apply. |
| Ericsson [14] | For these FGs, we propose to agree the components in the 4th column. For FGs 59-2-3-1, 2, 3, 4, 5 agree the components in the 4th column. |

| 59. NR_MIMO_Ph5 | 59-2-3-6 | New CJT QCL assumptions for PDSCH pre-compensation | [1. Scheme-C] [2. Scheme-D] [3. Scheme-E] | FFS | yes | n/a | New QCL assumptions for PDSCH pre-compensation is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; [1. Scheme-C] [2. Scheme-D] [3. Scheme-E]; FFS; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Note 1: Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} Note 2: Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay}; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; 1. Scheme-C: the PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} 2. Scheme-D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay} 3. Scheme-E: the PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay}; FFS; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-6: - According to the agreement of RAN1#116bis, for a UE indicated with two TCI states, regarding QCL assumptions for PDSCH, support QCL assumption Scheme C, Scheme D and Scheme E for PDSCH. The supportive of new CJT QCL assumptions Scheme C, Scheme D and Scheme E for PDSCH pre-compensation should be a basic feature. We do not see the need of a prerequisite for 59-2-3-6. Proposal 38: Adopt the following for FG 59-2-3-6: - The components include - Support of new CJT QCL assumptions for PDSCH pre-compensation. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; [1. Scheme-C] [2. Scheme-D] [3. Scheme-E] 1. Support of new CJT QCL assumptions ofr PDSCH compensation; FFS; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-1 candidate values: {Scheme-C, Scheme-D, Scheme-E}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; [1. Scheme-C] [2. Scheme-D] [3. Scheme-E]; 40-1-4; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; [1. Scheme-C] [2. Scheme-D] [3. Scheme-E]; FFS; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation - Scheme-C; Support new CJT QCL assumptions Scheme-C for PDSCH pre-compensation; FFS; yes; n/a; New QCL assumptions Scheme-C for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-6a; New CJT QCL assumptions for PDSCH pre-compensation - Scheme-D; Support new CJT QCL assumptions Scheme-D for PDSCH pre-compensation; FFS; yes; n/a; New QCL assumptions Scheme-D for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-6b; New CJT QCL assumptions for PDSCH pre-compensation - Scheme-E; Support new CJT QCL assumptions Scheme-E for PDSCH pre-compensation; FFS; yes; n/a; New QCL assumptions Scheme-E for PDSCH pre-compensation is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; Supported QCL assumption for PDSCH with two indicated TCI states; yes; n/a; Per band; n/a; n/a; n/a; Candidate values: bitmap {Scheme C, Scheme D, Scheme E} Scheme C: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift} Scheme D: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {average delay} Scheme E: The PDSCH DMRS port(s) are QCLed with the DL-RS associated with the first TCI state with respect to QCL-TypeA and QCLed with the DL-RS in the second TCI state with respect to QCL-TypeA except for {Doppler shift, average delay}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-6; New CJT QCL assumptions for PDSCH pre-compensation; [1. Scheme-C] [2. Scheme-D] [3. Scheme-E]; 40-1-4; yes; n/a; New QCL assumptions for PDSCH pre-compensation is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-6 - Given the following reasons, we suggest partitioning this FG into three (i.e., each component can be separated): - Scheme C considers the support of 59-2-3-1, while Scheme D considers 59-2-3-2. And the support of them can be orthogonal each other. If we try to merge them into one, the definition of prerequisite FG will be quite complex. - Scheme E can be supported when the UE also supports C and D. Once the support of Scheme C and D is separately defined, it is also straightforward to decouple Scheme E, by having prerequisite FG be both of Scheme C and D. - Type of this FG (and potential separated FG from the above discussion) should be per band. It doesn’t need to follow their prerequisite FG. |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-3-7 | Linkage of CJTC Dd and Rel-18 eType-II CJT | [1. Joint triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication] | FFS | yes | n/a | Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; 1. Joint triggering 2. Separate triggering w/o delay offset compensation (DOC) indication 3. Separate triggering w/ delay offset compensation (DOC) indication; FFS; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-2-3-7: - According to the agreement of RAN1#118bis, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, joint triggering and separate triggering are supported. Since the linkage is introduced to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, both joint triggering and separate triggering with delay offset compensation indication should be the basic feature. Separate triggering without delay offset compensation indication should be a separate feature. - To enable the linkage, CJTC Dd report and Rel-18 eType-II CJT CSI report should be the prerequisite feature group for 59-2-3-7. The prerequisite of 59-2-3-7 should be FG for CJTC Dd report (i.e., FG 59-2-3-1) and FG for Rel-18 eType-II CJT CSI report (i.e., FG 40-3-1-1). Proposal 39: Adopt the following for FG 59-2-3-7: - The components include - Support of joint triggering; - Support of separate triggering with delay offset compensation indication. - The prerequisite of 59-2-3-7 should be FG for CJTC Dd report (i.e., FG 59-2-3-1) and FG for Rel-18 eType-II CJT CSI report (i.e., FG 40-3-1-1). Proposal 40: For linkage of CJTC Dd and Rel-18 eType-II CJT, separate triggering with delay offset compensation indication should be a separate feature. |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-7a; Linkage of CJTC Dd and Rel-18 eType-II CJT w/ joint triggering; [1. Joint triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication] 1. Support of linkage of CJTC Dd and Rel-18 eType-II CJT w/ joint triggering 2. Supported timeline relaxation amount drelax; FFS 59-2-3-1 and 40-3-1-1; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT w/ joint triggering is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component-2 candidate values: {0, Z1’}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-7b; Linkage of CJTC Dd and Rel-18 eType-II CJT w/ sperate triggering; 1. Support of linkage of CJTC Dd and Rel-18 eType-II CJT 2. Supported CJTC Dd buffering time; 59-2-3-1 and 40-3-1-1; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT w/ separate triggering is not supported; FFS; FFS; FFS; FFS; Component-2 candidate values: {2s, infinity}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; [1. Joint triggering only, separate triggering only, joint triggering and separate triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication] 2. For joint triggering, support of reporting the value of Drelax; 40-3-1-1, 59-2-3-1; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements The value of Drelax is taken from {0, drelax}; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; [1. Joint triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication]; FFS; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering; Support joint triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI; FFS; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-7a; Linkage of CJTC Dd and Rel-18 eType-II CJT with separate triggering; Support separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI; 59-2-3-7; yes; n/a; Separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; [1. Joint triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication]; FFS; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; 1. Support of RRC configuration a UE to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset - 2. Supported mode(s); yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values: {Joint trigger, separate trigger, both}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-7; Linkage of CJTC Dd and Rel-18 eType-II CJT; [1. Joint triggering] [2. Separate triggering w/o delay offset compensation (DOC) indication] [3. Separate triggering w/ delay offset compensation (DOC) indication]; 40-3-1-1, 59-2-3-1; yes; n/a; Linkage of CJTC Dd and Rel-18 eType-II CJT is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-7 - Prerequisite FG should be 40-3-1-1 in addition to 59-2-3-1 - Type should be per band |
| Ericsson [14] |  |

| 59. NR_MIMO_Ph5 | 59-2-3-8 | Separate triggering with configuration of 1-bit indicator per CSI trigger state | Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting | FFS | yes | n/a | Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] | 59-2-3-8: - For the prerequisite of 59-2-3-8, considering the supportive of separate triggering is needed before introducing 1 bit indicate per trigger state, the prerequisite of 59-2-3-8 should be FG for linkage of CJTC Dd and Rel-18 eType-II CJT (i.e., FG 59-2-3-7). Proposal 41: Adopt the following for FG 59-2-3-8: - The prerequisite of 59-2-3-8 should be FG for linkage of CJTC Dd and Rel-18 eType-II CJT (i.e., FG 59-2-3-7). |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting; FFS 59-2-3-7b; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting; 59-2-3-7; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting; FFS; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state for delay offset compensation; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting to indicate delay offset compensation at UE; 59-2-3-7a; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; Per band and Per BC; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-3-8; Separate triggering with configuration of 1-bit indicator per CSI trigger state; Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting; 40-3-1-1, 59-2-3-1; yes; n/a; Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported; Per-band and per-BC; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-2-3-8 - Prerequisite FG should be 59-2-3-7. Also, givnen that this feature considers separate triggering, component 2 or 3 should be supported in 59-2-3-7. Therefore, there should be a restriction that “a UE reporting 59-2-3-8 must report 59-2-3-7 component 2 or component 3”, which can be noted. - Type should be per band |
| Ericsson [14] |  |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; [TBD]; Timelines for for up to 128 ports; 1. the timeline for up to 128 ports.; Yes; No; TBD; n/a; n/a; n/a; Component 1 candidate values: {Capability1, Capability 2} Note 1: capability 1 is the legacy Z/Z’ Not 2: legacy Z/Z’ multiplied by ceil(P/32), where P is the total number of ports across all the aggregated CSI-RS resources; Optional with capability signalling. NR MIMO Phase 5; [TBD]; CPU occupations for up to 128 ports; 1. CPU occupations for up to 128 ports; Yes; No; TBD; n/a; n/a; n/a; Component 1 Candidate values: {Capability1, Capability 2} Note 1: Capability 1 is OCPU = ceil(P/32), where P is the total number of ports across all the K aggregated CSI-RS resources. Note 2: OCPU =1; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-2-1-9; Support of associating a CSI-RS resource set for NCB based UL; Support of associating a CSI-RS resource set for NCB based UL; 2-15a; Yes; n/a; Associating a CSI-RS resource set for NCB based UL is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-2-1-9; Rel-18 SD NES Type-1 CB with extended port subset indication; Support of Rel-18 SD NES Type-1 CB with extended port subset indication; [42-1]; yes; n/a; Rel-18 SD NES Type-1 CB with extended port subset indication is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-10; Mapping method from CSI-RS resource/port index to CSI/PMI calculation for Rel-19 Type I and eType II CB; Support of mapping methods from CSI-RS resource/port index to CSI/PMI calculation for Rel-19 Type I and eType II CB; FFS; yes; n/a; Aggregating of multiple NZP CSI-RS resources is not supported; FFS; FFS; FFS; FFS; Candidate values: {’Method 1’, ‘Method 2’, ‘Method 1 and 2’} Note: UE supporting Rel-19 Type I and eType II CB needs to report this capability; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-11; up to 128 ports CSI-RS with K aggregated resources for SRS resource set with usage of 'nonCodebook'; 1. Support association between 48/64/128 ports CSI-RS with K aggregated resources and SRS resource set with usage of 'nonCodebook 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; 2-15; yes; n/a; Association between 48/64/128 ports CSI-RS with K aggregated resources and SRS is not supported; FFS; FFS; FFS; FFS; Component-2: Maximum size of the list is [8]. the candidate values for the max # of Tx port in one resource is {48, 64,128} The candidate value set of the max # of resources is: {from 1 to [16]} The candidate value set of total # of ports is: {from [48] to [256]}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1-12; Reduced timeline for Rel-19 Type-I SP and eType-II codebook refinements; Support timeline Capability 1 for Rel-19 Type-I SP and eType-II codebook refinements; FFS; yes; n/a; Only timeline Capability 2 for Rel-19 Type-I SP and eType-II codebook refinements is supported; FFS; FFS; FFS; FFS; Note: Capability 1: Reuse legacy Z/Z’ values Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-2-2-4; Configuration of MR always-reported resources; For Rel-16 eType-II, MR={1}; FFS 59-2-2-3; yes; n/a; Configuration of MR always-reported resources is not supported, i.e. MR=0; N/A; N/A; N/A; N/A; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-9; CPU for CJTC report; 1.Support candidate values of X for CPU for CJTC DO report 2.Support candidate values of X for CPU for CJTC FO report 3.Support candidate values of X for CPU for CJTC PO report 4.Support candidate values of X for CPU for CJTC Dd+FO report; FFS; yes; n/a; CPU for CJTC report is not supported; FFS; FFS; FFS; FFS; Component 1 candidate values:{1, 2} Component 2 candidate values:{1, 2} Component 3 candidate values:{1, 2} Component 4 candidate values:{1, 2}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-10; Timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT; Support value of Drelax; FFS; yes; n/a; Timeline for joint triggering od CJTC Dd and Rel-18 eType-II CJT is not supported; FFS; FFS; FFS; FFS; candidate values:{0, drelax }; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-2-1-1f; CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports; Support CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports; Yes; n/a; CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1g; Spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports; - Support spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports by extending the bitmap (i.e., port-subsetIndicator) - Processing timeline; Yes; n/a; Spatial domain NES Type-I for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate value {Capability 1, Capability 2} Note: For component of processing timeline Capability 1 timeline: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1h; CSI-RS port mapping for Type-I and Type II codebook enhancement for up to 128 ports; - Supported CSI-RS port mapping schemes for Type-I and Type II codebook enhancement for up to 128 ports; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Candidate value {Mapping method 1, Mapping method 2, Mapping method 3} Note: The CSI-RS port mapping applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-1i; CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports; - Maximum number of slots the K CSI-RS resources shall be configured within for Type-I and Type II codebook enhancement for up to 128 ports; Yes; n/a; No time domain restriction for K CSI-RS resources; Per band and Per BC; n/a; n/a; n/a; Candidate value {1 slot, 2 slots} Note: The supported CSI-RS resource time domain restriction applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3c; PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports; - Support of PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2; Yes; n/a; PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3d; Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports; - Support of parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports; Yes; n/a; Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3e; Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports; - Support of Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports - Support of parameter combinations with M=1 - Support of rank 1-2 - Support R=1 - A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 - Supported processing capability; Yes; n/a; Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-4b; M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports; - Support M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports - Support of parameter combinations with M=2 A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=1; 59-2-4; Yes; n/a; M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 3 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-4c; R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports; - Support R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2; 59-2-4; Yes; n/a; R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-4d; Rank 3,4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports; Support rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports; 59-2-4; Yes; n/a; Rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-5c; N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; - Support for the size of DD-basis, N4>1 - A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously - A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} for one CSI report setting Value of d=m for the DD unit size when A-CSI-RS is configured for CMR; 59-2-5; Yes; n/a; N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {2,3,4 … 64} d. {64, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {48, 64,128} c. {4,8,12} d.{64, …, 256}. 59. NR_MIMO_Ph5; 59-2-5d; DD unit size d=1 when A-CSI-RS is configured for CMR N4>1; Support value of d=1 for the DD unit size when A-CSI-RS is configured for CMR for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR N4>1 is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5e; Maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Supported maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Candidate values: {4, 8, 12}. 59. NR_MIMO_Ph5; 59-2-5f; PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; - Support PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports - A list of supported combinations, each combination is {Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2; 59-2-5; Yes; n/a; PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a; Component 2 candidate values a. {48, 64,128} b. {2,3,4 … 64} c. {64, …, 256}. 59. NR_MIMO_Ph5; 59-2-5g; X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Support X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; X=1 based on first and last slot of WCSI, for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5h; X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Support X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5i; l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Support l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; l = (n – nCSI,ref) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5j; L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Support L=6 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5k; Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Support Rank 3 and 4 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported; Per band and Per BC; n/a; n/a; n/a. 59. NR_MIMO_Ph5; 59-2-5ls; Processing timeline for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; - Aperiodic CSI report timing relaxation, w, Rel-18 Type-II Doppler codebook enhancement for up to 128 ports Aperiodic CSI report timing relaxation Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols: {14*(KP–1)*d, 14*KP*d} Note: Kp is according to Component 12 of FG59-2-5 Note: d=4 (minimum periodicity of periodic CSI-RS) Component 2 candidate values: {CAP1, CAP2} For N4 = 1 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP1 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP2 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m + Z'2, 2Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w + Z'2, 2Z'2) Z2/Z'2 are defined in Table 5.4-2 in TS38.214 KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots. 59. NR_MIMO_Ph5; 59-2-5m; Maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; Supported maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports; 59-2-5; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Candidate values (in slots): {4, 5, 8, 10, 20}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-3a; Maximum number of CSI-RS resources in a report configuration for phase offset report; - Maximum number of CSI-RS resources in a report configuration for phase offset report; 59-2-10; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {2, 3, 4}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-3b; Maximum number of phase offset report settings per BWP; - Maximum number of phase offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID; 59-2-10; Yes; n/a; Per FS; n/a; n/a; n/a; Candidate values: {1, 2, 3, 4}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-3c; Maximum number of quantization points M for phase offset report; - Maximum number of quantization points M for phase offset report; 59-2-10; Yes; n/a; Per band and Per BC; n/a; n/a; n/a; Candidate values: {16, 32}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-7a; Processing time relaxation for joint triggering of CJTC Dd and Rel-18 eType-II CJT; Processing time relaxation for joint triggering of CJTC Dd and Rel-18 eType-II CJT; yes; n/a; No relaxation; Per band and Per BC; n/a; n/a; n/a; Component candidate values: 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448}; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-3-9; RRC configuration of 1 SRS port of antenna switching associated with phase offset report; Support of RRC configuration of 1 SRS port of antenna switching associate with phased offset report; 59-2-10; Yes; n/a; RRC configuration of 1 SRS port of antenna switching associated with phase offset report is not supported; Per band and Per BC; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-2-4; Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH; - Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously; yes; n/a; Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported; Per FS; n/a; n/a; n/a; Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port is {48, 64, 128} The candidate value set of the max # of resources is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59-2-x; Timeline relaxation for CSI-RS aggregation; Support of capability 1 or capability 2; Per band; Capability 1: Reuse legacy Z/Z’ values Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources The legacy timeline Z/Z’ for Type-I corresponds to Z1/Z1’ in Table 5.4-2 of TS38.214 for Type-I WB SP-CSI with at most 4 CSI-RS ports in a single resource without CRI, and Z2/Z2’ for other Type-I cases The legacy timeline Z/Z’ for Type-II corresponds to Z2/Z2’; Optional with capability signalling. 59-2-x; Timeline relaxation for port subset indication for the SD NES Type-1; Support of capability 1 or capability 2; Per band; Capability 1 timeline: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-2-1- XX; CSI processing timeline for Rel-19 Type-I SP and Type-II codebook; Support from two timeline capabilities, for Rel-19 Type-I single-panel codebook; 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a; YES; N/A; BaseStation does not know UE’s CSI processing timeline, and may have to assume a long report latency; Per-band and per-BC; N/A; N/A; N/A; Component 1 candidate values: Capability 1: Reuse legacy Z/Z’ values Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources Note: A UE that supports 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a must signal this FG; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-1-XX; NES SD Type1 timeline for Rel-19 Type-I single-panel codebook; Support NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook; 59-2-1-1, 1a, 1b, 1c, 1d, or 1e; YES; N/A; BaseStation does not know UE’s CSI processing timeline, and may have to assume a long report latency; Per-band and per-BC; N/A; N/A; N/A; Component 1 candidate values: Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-1- XX; CSI-RS port mapping method1 for aggregation of K={2,3,4} CSI-RS resources; Support CSI-RS port mapping method1 (N1-dimension aggregation) for Rel-19 Type-I/II codebooks; 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 5, 5a, or 5b; YES; N/A; For K={2,3,4} CSI-RS resources, UE only supports CSI-RS port mapping method2 (N2-dimension aggregation); Per-band and per-BC; N/A; N/A; N/A; The support of mapping method2 (N2-dimension aggregation) is required for the prerequisite FGs; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-1-XX; {K=3, Q=16} for P = KQ = 48 CSI-RS ports; Support K=3 CSI-RS resources each with Q=16 ports for Rel-19 Type-I/II codebooks; 59-2-1-1a, 1d, 3a, 4a, or 5a; YES; N/A; {K=3, Q=16} is not supported by UE Note: P=48 ports can still be supported by {K=2, Q=24}; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-1- XX; {K=4, Q=16} for P = KQ = 64 CSI-RS ports; Support K=4 CSI-RS resources each with Q=16 ports for Rel-19 Type-I/II codebooks; 59-2-1-1, 1c, 3, 4, or 5; YES; N/A; {K=4, Q=16} is not supported by UE Note: P=64 ports can still be supported by {K=2, Q=32}; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signaling. 59. NR MIMO phase 5; 59-2-1-XX; Both groups of SRS ports for single-CW reception; Support UE antenna ports associated with both groups of SRS ports for single-CW reception; 59-2-1-8; YES; N/A; UE antenna ports associated with only one of the two SRS port groups are used for single-CW reception; Per-band and per-BC; N/A; N/A; N/A; Applicable to reception of PDSCH with single-CW (i.e. <= 4 layers), and non-PMI report hypotheses of rank1-to-4; Optional with capability signaling. 59. NR MIMO phase 5; 59-2-2-XX; CSI-RS resource-specific RI restriction; Support of CSI-RS resource-specific RI restriction for Rel-19 CRI-based report; 59-2-2-1 or 59-2-2-2; YES; N/A; RI restrictions need to be configured same for all CSI-RS resources in the resource set; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signaling. 59. NR MIMO phase 5; 59-2-2-XX; CSI-RS resource-specific triggering offset; Support of CSI-RS resource-specific triggering offset for aperiodic CSI-RS; 59-2-2-1 or 59-2-2-2; YES; N/A; Aperiodic triggering offsets for all CSI-RS resources in the resource set need to be same; Per-band and per-BC; N/A; N/A; N/A; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-3-XX; Scaling parameter X for CJTC DO CPU; Value of X; 59-2-3-1; YES; N/A; Network does not know how much CPU UE would take for DO report; Per-band and per-BC; N/A; N/A; N/A; O_CPU =X * N_TRP; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-3-XX; Scaling parameter X for CJTC FO CPU; Value of X; 59-2-3-2; YES; N/A; Network does not know how much CPU UE would take for FO report; Per-band and per-BC; N/A; N/A; N/A; O_CPU =X * N_TRP; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-2-3-XX; Scaling parameter X for CJTC PO CPU; Value of X; 59-2-3-3; YES; N/A; Network does not know how much CPU UE would take for PO report; Per-band and per-BC; N/A; N/A; N/A; O_CPU =X * N_TRP; Optional with capability signaling. |
| NTT DOCOMO, INC. [13] | 59-2-1-1 - Add a new FG for timeline as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources - Add a new FG for group-based hard CBSR. - For each FG, as usual, the component of following should be added: {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously. - For the ‘Type’ of each FG, similar as legacy, ‘per band and per BC’ can be applied. - For FG 59-1-1 and FG 59-1-1c, it is preferred to make one of them as the default scheme for 64 ports to avoid market fragmentation. And Scheme-A is better to be the default scheme. Thus, suggest adding 59-2-1-1 as the other prerequisite feature group of FG 59-1-1c. 59-2-1-9 - Add a new FG for timeline for combination of Rel-19 Type-I SP codebook and SD NES Type-1 as following two UE capabilities have been agreed. - Capability 1: Reuse legacy Z/Z’ values - Capability 2: Scale the legacy timeline Z/Z’ by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources 59-2-1-10 - Add a new FG of ‘Active CSI-RS resources and ports for mixed codebook types in any slot’. |
| Ericsson [14] |  |

## 3-antenna-port codebook-based transmissions

| 59. NR_MIMO_Ph5 | 59-3-1 | Non-codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH] | FFS | yes | n/a | Non-codebook based PUSCH transmission for 3TX is not supported | FFS | FFS | FFS | FFS | [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] | FG 59-3-1 (Non-codebook based PUSCH transmission for 3TX for single TRP) Following legacy UE capability, support of association between CSI-RS and SRS for non-codebook-based PUSCH transmission is an optional to UE, and it should be captured as a separate FG. Thus, we propose the following: Proposal 7: Introduce separate FG 59-3-1a for association between CSI-RS and SRS, and removing the component [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH] from FG 59-3-1: 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-1a; Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP; Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH; 59-3-1; yes; n/a; Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol 4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; Component 1 candidate values: {1, 2, 3} Component 2 candidate values: {1,2,3} Component 3 candidate values: {1,2,3} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] | 59-3-1: - For non-codebook UL 3Tx, the number of SRS resource per set should be 3. Thus, the component value for component 2, should be 3. - Regarding component 4, it is not necessary for UE supporting non-codebook 3Tx to associate CSI-RS and SRS resource set(s) as a compulsory component for non-codebook 3Tx. It is our view that it can be a new FG or apply FG 2-15a to 3Tx. Proposal 46: The component value for component 2 for FG 59-3-1 is 3. Proposal 47: Delete component 4 of FG 59-3-1 and apply FG 2-15a to 3Tx, or add a new FG for association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH. |
| ZTE Corporation/Sanechips [5] | - For FG 59-3-1, the component ‘Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH’ should be split as a separate FG as legacy, i.e., FG 59-3-1a ‘Association between CSI-RS and SRS for non-codebook based PUSCH transmission for 3TX for signal TRP’. 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-1a; Association between CSI-RS and SRS for non-codebook based PUSCH transmission for 3TX for signal TRP; 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRSresoureset” 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; FFS; yes; n/a; Association between CSI-RS and SRS is not supported for non-codebook based PUSCH transmission for 3TX for signal TRP; FFS; FFS; FFS; FFS; Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; 2-12. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-1-1; Association between CSI-RS and SRS for non-codebook 3Tx PUSCH; 1) Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRSresoureset" 2) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously.; 59-3-1. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; Support of maximum number of 3 PUSCH MIMO layers for non-codebook-based PUSCH; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; Per FSPC; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-1a; Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH; - Support association between NZP-CSI-RS and SRS resource set for non-codebook-based 3Tx PUSCH - A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously; 59-3-2; yes; n/a; Association between CSI-RS and SRS for non-codebook-based PUSCH is not supported; Per band; n/a; n/a; n/a; Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port is {2, 4, 8, 12, 16, 24, 32, 48, 64, 128} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256} Note: For max # of Tx ports, 1 CSI-RS resource is used for <= 32 ports, and more than 1, up to 4 CSI-RS resources, are used for >32 ports; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-1 (Non-codebook based PUSCH transmission for 3TX for single-TRP), - For Component 2 ([2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)]), based on the following conclusion, it is basically following the legacy UE feature which is up to 4. Hence, we would like to make a corresponding candidate value range from 1 to 4. Conclusion in RAN1#118bis To support non-codebook-based precoding by a 3TX UE, for SRI indication, re-use the legacy-based solution according to NSRS ≤ maxNumberSRS-ResourcePerSet.. - For Component 3 ([3. Maximum number of simultaneous transmitted SRS resources at one symbol]), although the candidate value for the above Component 2 can be up to 4, due to 3TX, simultaneous transmission is up to 3. Hence, we would like to make a corresponding candidate value range from 1 to 3. - For Component 4 ([4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]), we don’t see the necessity to re-define this UE capability separately for 3TX UE, because legacy features (FG 2-15a, FG 2-15b) can be re-used. Hence, we would like to delete the Component 4. 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) 2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission) 3. Maximum number of simultaneous transmitted SRS resources at one symbol; 2-15; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; Per FSPC; FFS; FFS; FFS; Component 1 candidate values: {1,2,3} Component 2 candidate values: {1,2,3,4} Component 3 candidate values: {1,2,3} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-3-1; Non-codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of supported layers (non-codebook transmission scheme) [2. Maximum number of SRS resource per set (SRS set use is configured as for non-codebook transmission)] [3. Maximum number of simultaneous transmitted SRS resources at one symbol] [4. Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH]; FFS; yes; n/a; Non-codebook based PUSCH transmission for 3TX is not supported; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-1 - Support components 1,2,3 and the candidate values. - Component 4 can be a separate FG with FG 59-3-1 as its prerequisite. - Type of 59-3-1 can be per FSPC. - Type of new separate FG for component 4 can be per band. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | In the list, the non-codebook and codebook based PUSCH features are currently indexed as FG 59-3-1 and FG 59-3-2, respectively, which deviates from the legacy indexing convention where codebook based PUSCH feature precede non-codebook based PUSCH feature. To align with the legacy convention, the indexing for non-codebook and codebook based features should be swapped. In the current list of UE features, the non-codebook based PUSCH feature (FG 59-3-1) is indexed before the codebook based PUSCH feature (FG 59-3-2). This deviates from legacy convention, where the codebook based feature is indexed prior to non-codebook based PUSCH feature. Swap the indexing for non-codebook and codebook based features for 3 Tx PUSCH for single TRP, i.e., assign FG 59-3-1 to codebook based feature and FG 59-3-2 to non-codebook based feature. In the list, for codebook based PUSCH feature (i.e., FG 59-3-2), Component 3 has been tentatively included to indicate the maximum number of simultaneous transmitted SRS resources at one symbol. However, for codebook based 3 Tx PUSCH, only a single 4 port SRS resource with port 1003 disabled can be transmitted per OFDM symbol. Additionally, in legacy UE features, this component is included only for the non-codebook based PUSCH feature (i.e., FG 2-15), since UE can transmit multiple 1 port SRS resources in one symbol, as described in Section 6.1.1.2 of TS 38.214: The UE shall use one or multiple SRS resources for SRS transmission, where, in one or two SRS resource set(s), the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities.. |

| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | FFS | FFS | FFS | FFS | [Component 1 candidate values: {1, 2,3}] [Component 2 candidate values: {1,2}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2,3}] [Component 2 candidate values: {1,2}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} Component 3 candidate values: {1,2,3} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2,3}] [Component 2 candidate values: {1,2}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; 2-13. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] [Conclusion (from RAN1#116) For a given CC, multiple SRS resources in a set with usage “Codebook” are not expected to be overlapped in time.] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2,3}] [Component 2 candidate values: {1,2}] [Component 3 candidate values: {1,2,3}]; Optional with capability signalling. |
| OPPO [8] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS; FFS; FFS; FFS; [Component 1 candidate values: {1, 2,3}] [Component 2 candidate values: {1,2}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; 2-14; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; Per FS; NO; NO; NO; Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of 3 PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 3. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; Per FSPC; n/a; n/a; n/a; Component 2 candidate values {1, 2}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-2 (Codebook based PUSCH transmission for 3TX for single TRP), - For Component 3 ([3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH]), since multiple SRS resources with usage of codebook cannot be transmitted simultaneously, we would like to delete the Component 3. - Also, only non-coherent UL codebook is used for 3TX codebook based PUSCH transmission, it is needed to add either a note or a separate component like “Support non-coherent codebook subset only”. 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; 2-14; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; Per FSPC; FFS; FFS; FFS; Component 1 candidate values: {1,2,3} Component 2 candidate values: {1,2} Component 3 candidate values: {1,2,3} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-3-2; Codebook based PUSCH transmission for 3TX for single TRP; 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; Codebook based PUSCH transmission for 3TX is not supported; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 1 candidate values: {1, 2, 3}] [Component 2 candidate values: {1,2,3}] [Component 3 candidate values: {1,2,3}] FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-2 - Support components 1,2,4 and the candidate values. - Do not support component 3. - Type: per FSPC. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | In the list, the non-codebook and codebook based PUSCH features are currently indexed as FG 59-3-1 and FG 59-3-2, respectively, which deviates from the legacy indexing convention where codebook based PUSCH feature precede non-codebook based PUSCH feature. To align with the legacy convention, the indexing for non-codebook and codebook based features should be swapped. In the current list of UE features, the non-codebook based PUSCH feature (FG 59-3-1) is indexed before the codebook based PUSCH feature (FG 59-3-2). This deviates from legacy convention, where the codebook based feature is indexed prior to non-codebook based PUSCH feature. Swap the indexing for non-codebook and codebook based features for 3 Tx PUSCH for single TRP, i.e., assign FG 59-3-1 to codebook based feature and FG 59-3-2 to non-codebook based feature. In the list, for codebook based PUSCH feature (i.e., FG 59-3-2), Component 3 has been tentatively included to indicate the maximum number of simultaneous transmitted SRS resources at one symbol. However, for codebook based 3 Tx PUSCH, only a single 4 port SRS resource with port 1003 disabled can be transmitted per OFDM symbol. Additionally, in legacy UE features, this component is included only for the non-codebook based PUSCH feature (i.e., FG 2-15), since UE can transmit multiple 1 port SRS resources in one symbol, as described in Section 6.1.1.2 of TS 38.214: The UE shall use one or multiple SRS resources for SRS transmission, where, in one or two SRS resource set(s), the maximum number of SRS resources which can be configured to the UE for simultaneous transmission in the same symbol and the maximum number of SRS resources are UE capabilities.. Accordingly, Component 3 appears to be a typo and not required for the codebook based UE feature. In the current UE feature list for 3 Tx PUSCH, Component 3 of FG 59-3-2, which indicates the maximum number of simultaneous transmitted SRS resources in one symbol, appears to be a typo and is not required since this capability is normally associated with non-codebook, rather than codebook based operation. Remove Component 3 from the UE feature for codebook based PUSCH transmission for 3 Tx for single TRP. |

| 59. NR_MIMO_Ph5 | 59-3-3 | 3T6R Antenna switching | 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching] | FFS | yes | n/a | 3TX 3T6R antenna switching is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band 3. Support of reporting whether the UL Tx is switched together with UL Tx in another band 4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FGs 59-3-3 and 59-3-3a, the component ‘Maximum 2 SP and 1 periodic SRS sets’ should be split as separate FGs, i.e., FG 59-3-3b ‘Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching’ and FG 59-3-3c ‘Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching’, respectively. - For FG 59-3-3, a new component ‘Downgrade antenna switching configurations’ should be added, and the candidate values should be combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 2T2R, 2T4R, 2T6R, 3T3R}. 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] 2. Downgrade antenna switching configurations [3. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [4. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 2 candidate values: combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 2T2R, 2T4R, 2T6R, 3T3R}; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; 2-55. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. SRS transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [3. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [4. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3-1; Maximum number of supported SRS sets; Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching; 59-3-3. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of SRS antenna switching xTyR with y=6 3. Support of reporting whether the uplink TX switching impact to downlink receiving in a band 4. Support of reporting whether the UL Tx is switched together with UL Tx in another band; 23-8-3; yes; n/a; 3TX 3T6R antenna switching is not supported; Per FS; NO; NO; NO; Component 2 candidate values: a combination from the set {t1r1, t2r2, t1r2, t2r4, t1r4, [t3r3],t2r6, t1r6, t3r6}; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL; yes; n/a; 3TX 3T6R antenna switching is not supported; Per FS; n/a; n/a; n/a; Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-3 (3T6R Antenna switching), - For Component 2 and 3, since the intention is to report the affected bands, we would like to change Component 2 as “Report the entry number of the first-listed band with UL in the band combination that affects this DL” and Component 3 as “Report the entry number of the first-listed band with UL in the band combination that switches together with this UL”, which both have been used in legacy SRS antenna switching UE capabilities. - For Component 4, as in Rel-17, we would like to define this UE capability information as a separate FG. - We also would like to add downgrade indication including {1T1R, 1T2R, 1T4R, 2T2R, 2T4R, 2T6R, 3T3R} supporting the number of TX antennas at least lower than equal to 3 and the number of RX antennas at least lower than equal to 6. Other possible antenna switching configurations can be discussed further. 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL 4. Downgrade antenna switching configurations; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; Per FS; FFS; FFS; FFS; Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: combination (including empty) of {1T1R, 1T2R, 1T4R, 2T2R, 2T4R, 2T6R, 3T3R} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3-1; Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T6R antenna switching; 59-3-3; yes; n/a; Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T6R is supported; Per FS; FFS; FFS; FFS; Note: The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS; yes; n/a; 3TX 3T6R antenna switching is not supported; FFS Per SFBC; FFS NO; FFS NO; FFS NO; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-3 - Support components 1,2,3. - Component 4 can be a separate FG with FG 59-3-3 as its prerequisite. - A new component is needed: downgrade antenna switching configurations, whose candidate value can be combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R, 3T3R, 3T6R}. - Prerequisite: 2-53. - Type: per FS. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | Given the agreements made in RAN1#116-bis, RAN1#118 and RAN1#119 (outlined below), it is clear that a 3 port SRS transmission is supported by reusing a 4 port SRS resource with port 1003 disabled. As a result, for 3T3R and 3T6R antenna switching, the UE can be RRC-configured with one or two 4 port SRS resources with port 1003 disabled, respectively, in the SRS resource set(s) configured for antenna switching, as agreed in RAN1#119. Agreement (RAN1#116-bis) For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, - Option 3: Always a same port is muted, e.g., the 4th port Agreement (RAN1#118) For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled. Agreement (RAN1#119) For a UE operating with 3TX, - the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook. - the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching. - Note: The RRC parameter mentioned in the above two bullets is configured per set A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.. Accordingly, the UE should indicate the capability to support transmission of 4 port SRS resource with port 1003 disabled to facilitate 3T3R and 3T6R antenna switching. Hence, the tentative requirement “with port 1003 disabled when 4 port SRS resources with port disabled are configured to the UE” for Component 1 of FG 59-3-3/3a should be confirmed. Further, for 3T3R and 3T6R, Component 4 has been tentatively included in FG 59-3-3 and FG 59-3-3a, respectively, to indicate the support for maximum 2 semi-persistent and 1 periodic SRS sets for antenna switching. However, in Rel-17, the UE capability to support maximum 2 semi-persistent and 1 periodic SRS sets for antenna switching is included as an independent feature in FG 23-8-4 (shown below), which is applicable for all supported antenna switching xTyR where y<=8. 23. NR_FeMIMO; 23-8-4; Maximum 2 SP and 1 periodic SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching; srs-AntennaSwitching2SP-1Periodic-r17; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated.. Given this, indicating support for a maximum of 2 semi-persistent and 1 periodic SRS sets for 3T3R or 3T6R can be done in two ways: - Indicating FG 23-8-4, or - Indicating support via Component 4 within FG 59-3-3/3a, independent of FG 23-8-4. In RAN1#106 [2], it was agreed that 3T3R does not explicitly introduce a 3 Rx UE, but is instead applicable to 4, 6 and 8 Rx UEs. Proposal: RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1. - where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. - No RAN4 impact.. Furthermore, 3T6R configurations are most likely supported by UEs with 6 and 8 receive (Rx) antennas. Given UEs with 4, 6, or 8 Rx antennas can indicate FG 23-8-4, makes FG 23-8-4 straightforwardly applicable to both 3T3R and 3T6R antenna switching features. Hence, having Component 4 in FG 59-3-3/3a appears redundant, as its functionality is already encompassed by FG 23-8-4. However, there may be some ambiguity regarding whether indicating Rel-17 FG 23-8-4 for condition xTyR with y<=8 explicitly implies support for a maximum of 2 semi-persistent and 1 periodic SRS sets for the Rel-19 3T3R and 3T6R antenna switching UE features. To address this ambiguity, Component 4 in FG 59-3-3/3a could be retained as a note rather than a standalone component, as shown below. This note would clarify that FG 23-8-4 applies to Rel-19 3T3R and 3T6R configurations, ensuring consistent interpretation and implementation across releases. NR MIMO Phase 5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS: Further partitioning of this FG based on existing and future agreements Note: UE supports maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching if FG 23-8-4 reported. NR MIMO Phase 5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS: Further partitioning of this FG based on existing and future agreements Note: UE supports maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching if FG 23-8-4 reported. Given the agreement from RAN1#119, a 3 Tx UE is to be configured with one or two 4 port SRS resources, with port 1003 disabled, within the SRS set with usage ‘antenna switching’ to support 3T3R and 3T6R configurations, respectively. Therefore, the tentative addition ‘with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE’ to Component 1 of FG 59-3-3/3a can be confirmed. The capability to support a maximum of two semi-persistent and one periodic SRS set for antenna switching in 3T3R and 3T6R configuration can be indicated through the Rel-17 FG 23-8-4. However, the condition y<=8 did not address y=3 in Rel-17. Therefore, Component 4 from FG 58-3-3/3a can be removed and instead added as a note clarifying that for Rel-19 a maximum of two semi-persistent and one periodic SRS sets is supported if FG 23-8-4 is reported. Confirm the addition ‘with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE’ in Component 1 of FG 59-3-3/3a. Remove Component 4 from FG 59-3-3/3a and add a clarifying note that a maximum of two semi-persistent and one periodic SRS sets is supported if FG 23-8-4 is reported. |

| 59. NR_MIMO_Ph5 | 59-3-3a | 3T3R antenna switching | 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching] | FFS | yes | n/a | 3TX 3T3R antenna switching is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band 3. Support of reporting whether the UL Tx is switched together with UL Tx in another band 4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FGs 59-3-3 and 59-3-3a, the component ‘Maximum 2 SP and 1 periodic SRS sets’ should be split as separate FGs, i.e., FG 59-3-3b ‘Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching’ and FG 59-3-3c ‘Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching’, respectively. - For FG 59-3-3a, according to the following agreement reached in RAN#106 meeting, a new component ‘Support of 3T3R antenna switching configuration(s) as an allowing downgrading configuration of xT4R, xT6R, or xT8R’, and 3T3R antenna switching should be supported as a downgrading option of combination of {1T4R, 1T6R, 1T8R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R}. Besides, the prerequisite of FG 59-3-3a should be 14-4 ‘SRS Tx switch with allowing downgrading configuration’ or 40-5-4 ‘SRS 8 Tx ports—antenna switching’. Agreement (RAN#106) RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1. - where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. No RAN4 impact.. 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] - Support of 3T3R antenna switching configuration(s) as an allowing downgrading configuration of xT4R, xT6R, or xT8R [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS 14-4 or 40-5-4; yes; n/a; 3TX 3T3R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements Component 4: 3T3R antenna switching as a downgrading option of combination of {1T4R, 1T6R, 1T8R, 2T4R, 2T6R, 2T8R, 4T4R, 4T8R} Note: ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports.; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3b; Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching; 1. Support maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching; 59-3-3; yes; n/a; Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported for 3T6R; FFS; FFS; FFS; FFS; Note: - If UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3c; Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching; 1. Support maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching; 59-3-3a; yes; n/a; Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported for 3T3R; FFS; FFS; FFS; FFS; Note: - If UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; 2-53. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. SRS transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [3. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [4. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3a-1; Maximum number of supported SRS sets; Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching; 59-3-3a. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Support of SRS antenna switching xTyR with y=3 3. Support of reporting whether the uplink TX switching impact to downlink receiving in a band 4. Support of reporting whether the UL Tx is switched together with UL Tx in another band; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; Per FS; NO; NO; NO; Component 2 candidate values: a combination from the set {t1r1, t2r2, t1r2, t2r4, t1r4, t3r3}; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL; yes; n/a; 3TX 3T3R antenna switching is not supported; Per FS; n/a; n/a; n/a; Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-3a (3T3R Antenna switching), - Similar with FG 59-3-3, we would like to revise Component 2 and 3 accordingly. - Similar with FG 59-3-3, we would like to define Component 4 as a separate FG. - Similar with FG 59-3-3, we also would like to add downgrade indication including {1T1R, 1T2R, 2T2R} supporting the number of TX antennas at least lower than equal to 3 and the number of RX antennas at least lower than equal to 3. Other possible antenna switching configurations can be discussed further. 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL 4. Downgrade antenna switching configurations; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; Per FS; FFS; FFS; FFS; Component 2 candidate value: {1,2,…,32} Component 3 candidate value: {1,2,…,32} Component 4 candidate value: combination (including empty) of {1T1R, 1T2R, 2T2R} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-3-3a-1; Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 3T3R antenna switching; 59-3-3a; yes; n/a; Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS for 3T3R is supported; Per FS; FFS; FFS; FFS; Note: The two SP-SRS resource sets are not activated at the same time; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS; yes; n/a; 3TX 3T3R antenna switching is not supported; FFS Per SFBC; FFS NO; FFS NO; FFS NO; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-3a - Support components 1,2,3. - Component 4 can be a separate FG with FG 59-3-3a as its prerequisite. - A new component is needed: downgrade antenna switching configurations, whose candidate value can be combination (including empty) of {1T1R, 1T2R, 1T4R, 1T6R, 1T8R, 2T2R, 2T4R, 2T6R, 2T8R}. - Prerequisite: 2-53. - Type: per FS. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | Given the agreements made in RAN1#116-bis, RAN1#118 and RAN1#119 (outlined below), it is clear that a 3 port SRS transmission is supported by reusing a 4 port SRS resource with port 1003 disabled. As a result, for 3T3R and 3T6R antenna switching, the UE can be RRC-configured with one or two 4 port SRS resources with port 1003 disabled, respectively, in the SRS resource set(s) configured for antenna switching, as agreed in RAN1#119. Agreement (RAN1#116-bis) For a 3TX UE, to support 3-port SRS transmission with reusing a 4-port SRS resource, support the following for muting one of the ports of the configured 4-port SRS resource, - Option 3: Always a same port is muted, e.g., the 4th port Agreement (RAN1#118) For a UE reporting UE capability of 3TX operation, support introduction of a new RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled. Agreement (RAN1#119) For a UE operating with 3TX, - the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook. - the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching. - Note: The RRC parameter mentioned in the above two bullets is configured per set A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.. Accordingly, the UE should indicate the capability to support transmission of 4 port SRS resource with port 1003 disabled to facilitate 3T3R and 3T6R antenna switching. Hence, the tentative requirement “with port 1003 disabled when 4 port SRS resources with port disabled are configured to the UE” for Component 1 of FG 59-3-3/3a should be confirmed. Further, for 3T3R and 3T6R, Component 4 has been tentatively included in FG 59-3-3 and FG 59-3-3a, respectively, to indicate the support for maximum 2 semi-persistent and 1 periodic SRS sets for antenna switching. However, in Rel-17, the UE capability to support maximum 2 semi-persistent and 1 periodic SRS sets for antenna switching is included as an independent feature in FG 23-8-4 (shown below), which is applicable for all supported antenna switching xTyR where y<=8. 23. NR_FeMIMO; 23-8-4; Maximum 2 SP and 1 periodic SRS sets for antenna switching; Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching; srs-AntennaSwitching2SP-1Periodic-r17; Note1: - Applies for all supported xTyR where y<=8. - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS. - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS. - The two SP-SRS resource sets are not activated.. Given this, indicating support for a maximum of 2 semi-persistent and 1 periodic SRS sets for 3T3R or 3T6R can be done in two ways: - Indicating FG 23-8-4, or - Indicating support via Component 4 within FG 59-3-3/3a, independent of FG 23-8-4. In RAN1#106 [2], it was agreed that 3T3R does not explicitly introduce a 3 Rx UE, but is instead applicable to 4, 6 and 8 Rx UEs. Proposal: RAN clarifies that for ‘3T3R’ antenna switching, SRS configuration i.e., SRS resource/set definition for ‘3T3R’ antenna switching can be discussed and supported as part of Rel-19 MIMO in RAN1. - where ‘3T3R’ is only applicable for the UE equipped with 4Rx, 6Rx, or 8Rx antenna ports. - No RAN4 impact.. Furthermore, 3T6R configurations are most likely supported by UEs with 6 and 8 receive (Rx) antennas. Given UEs with 4, 6, or 8 Rx antennas can indicate FG 23-8-4, makes FG 23-8-4 straightforwardly applicable to both 3T3R and 3T6R antenna switching features. Hence, having Component 4 in FG 59-3-3/3a appears redundant, as its functionality is already encompassed by FG 23-8-4. However, there may be some ambiguity regarding whether indicating Rel-17 FG 23-8-4 for condition xTyR with y<=8 explicitly implies support for a maximum of 2 semi-persistent and 1 periodic SRS sets for the Rel-19 3T3R and 3T6R antenna switching UE features. To address this ambiguity, Component 4 in FG 59-3-3/3a could be retained as a note rather than a standalone component, as shown below. This note would clarify that FG 23-8-4 applies to Rel-19 3T3R and 3T6R configurations, ensuring consistent interpretation and implementation across releases. NR MIMO Phase 5; 59-3-3; 3T6R Antenna switching; 1. Support of 3T6R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching]; FFS: Further partitioning of this FG based on existing and future agreements Note: UE supports maximum 2 SP and 1 periodic SRS sets for 3T6R antenna switching if FG 23-8-4 reported. NR MIMO Phase 5; 59-3-3a; 3T3R antenna switching; 1. Support of 3T3R SRS Tx port switching [with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE] [2. Support of reporting whether the uplink TX switching impact to downlink receiving in a band] [3. Support of reporting whether the UL Tx is switched together with UL Tx in another band] [4. Maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching]; FFS: Further partitioning of this FG based on existing and future agreements Note: UE supports maximum 2 SP and 1 periodic SRS sets for 3T3R antenna switching if FG 23-8-4 reported. Given the agreement from RAN1#119, a 3 Tx UE is to be configured with one or two 4 port SRS resources, with port 1003 disabled, within the SRS set with usage ‘antenna switching’ to support 3T3R and 3T6R configurations, respectively. Therefore, the tentative addition ‘with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE’ to Component 1 of FG 59-3-3/3a can be confirmed. The capability to support a maximum of two semi-persistent and one periodic SRS set for antenna switching in 3T3R and 3T6R configuration can be indicated through the Rel-17 FG 23-8-4. However, the condition y<=8 did not address y=3 in Rel-17. Therefore, Component 4 from FG 58-3-3/3a can be removed and instead added as a note clarifying that for Rel-19 a maximum of two semi-persistent and one periodic SRS sets is supported if FG 23-8-4 is reported. Confirm the addition ‘with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE’ in Component 1 of FG 59-3-3/3a. Remove Component 4 from FG 59-3-3/3a and add a clarifying note that a maximum of two semi-persistent and one periodic SRS sets is supported if FG 23-8-4 is reported. |

| NR MIMO Phase 5 | 59-3-4 | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based | 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based | FFS | FFS | FFS | FFS | [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS; FFS; FFS; FFS; Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; 59-3-2; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Xiaomi [9] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-2; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; Per FS; NO; NO; NO; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values; Optional with capability signalling. |
| Apple [10] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; Per FSPC; n/a; n/a; n/a; Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-4 (M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based), - For the description on cyclic mapping in Component 1, we would like to keep the sentence for having consistency with Rel-17 UE capability (FG 23-3-1). - Similar with FG 59-3-2, since 4 port SRS resource for usage of codebook is used with muting the port 1003, it is needed to add a new component like “Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE”. - We would like to delete the note “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” since this is related to PDCCH repetition which is not relevant on FG 59-3-4. NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE.; 59-3-2, 23-3-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; Per FS; FFS; FFS; FFS; Component 3 candidate values: {1,2} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-4; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – codebook based; 1. Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for codebook based; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-4 - Support components 1,2,3 and candidate values. - Prerequisite: 59-3-2 - The “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” is not related to this FG and should be removed. - Type: per FS. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] |  |

| NR MIMO Phase 5 | 59-3-4a | M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based | FFS | FFS | FFS | FFS | [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; FFS; FFS; FFS; FFS; Component 3 candidate values: {1,2,3.4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; Per FS; NO; NO; NO; [Component 3 candidate values: {1,2,3}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values; Optional with capability signalling. |
| Apple [10] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; Per FSPC; n/a; n/a; n/a; Component 3 candidate values: {1,2,3.4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-4a (M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based), - Similar with FG 59-3-4, for the description on cyclic mapping in Component 1, we would like to keep the sentence for having consistency with Rel-17 UE capability (FG 23-3-1-2). - Similar with FG 59-3-4, we would like to delete the note “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” since this is related to PDCCH repetition which is not relevant on FG 59-3-4a. NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1, 23-3-1-2; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; Per FSPC; FFS; FFS; FFS; Component 3 candidate values: {1,2,3.4} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type A for non-codebook based; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-4a - Support components 1,2. - Candidate values of component 3 should be {1,2,3}. - Prerequisite: 59-3-1 - The “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” is not related to this FG and should be removed. - Type: per FS. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | Further, it is tentatively decided that the maximum number of 1 port SRS resources per SRS set to support 3 Tx non-codebook PUSCH for single TRP (S-TRP) could take values from 1-3. Meanwhile, the Rel-15 UE capability parameter for the maximum number of SRS resources in non-codebook based operation allows values from 1-4, with the maximum number of layers being 1, 2, or 4. In this case, the maximum number of SRS resources and layers align. However, it should also be observed that a UE can declare, for example, a maximum 1 layer capability while supporting up to 4 SRS resources in non-codebook based operation. For 3 Tx operation, the maximum number of layers is 3. One possible reason for limiting the 3 Tx non-codebook capability to a maximum of 3 SRS resources is to simplify operation for UEs with 3 Tx chains that do not virtualize their antennas. Such UEs could toggle their Tx chains on or off without needing to switch between antennas or implement virtualization. However, these UEs could alternatively be designed to support at most 3 SRS resources. On the other hand, maximizing the number of SRS resources offers potential diversity benefits, especially when the maximum number of layers is lower. This is in our understanding a primary motivation for having the smaller layer capability than the number of SRS resources discussed above. Furthermore, it was tentatively decided that the maximum number of SRS resources in one SRS set for 3 Tx multiple TRP (M-TRP) non-codebook PUSCH could take values up to 4, as shown below. This decision, however, introduces an inconsistency with the parameter choices made for S-TRP operation. NR MIMO Phase 5; 59-3-4a; M-TRP PUSCH repetition (type A) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type A for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; [Component 3 candidate values: {1,2,3.4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements. There does not seem to be a benefit to reduce the maximum number of SRS resources for non-codebook based operation below the 4 supported in Rel-15. Supporting a maximum of 3 resources would instead limit the potential for diversity gain, and be less consistent with the principle of allowing a maximum layer capability that is less than the number of SRS resources. There seems to be inconsistency in the value for the maximum number of SRS resources in an SRS set for S-TRP and M-TRP 3 TX non-codebook PUSCH. Support maxNumberSRS-ResourcePerSet = 4 for Rel-19 3 Tx non-codebook based operation. |

| NR MIMO Phase 5 | 59-3-5 | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based | FFS | FFS | FFS | FFS | [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; FFS; FFS; FFS; FFS; Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE; 59-3-2; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-2; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; Per FS; NO; NO; NO; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values; Optional with capability signalling. |
| Apple [10] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; Per FSPC; n/a; n/a; n/a; Component 3 candidate values: {1,2} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-5 (M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based), - Similar with FG 59-3-4, for the description on cyclic mapping in Component 1, we would like to keep the sentence for having consistency with Rel-17 UE capability (FG 23-3-1-1). - Similar with FG 59-3-2, since 4 port SRS resource for usage of codebook is used with muting the port 1003, it is needed to add a new component like “Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE”. - Similar with FG 59-3-4, we would like to delete the note “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” since this is related to PDCCH repetition which is not relevant on FG 59-3-5. NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE.; 59-3-2, 23-3-1-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; Per FS; FFS; FFS; FFS; Component 3 candidate values: {1,2} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-5; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for codebook based; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 3 candidate values: {1,2}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-5 - Support components 1,2,3 and candidate values. - Prerequisite: 59-3-2 - The “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” is not related to this FG and should be removed. - Type: per FSPC. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] |  |

| NR MIMO Phase 5 | 59-3-5a | M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based | Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set | FFS | yes | n/a | M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based | FFS | FFS | FFS | FFS | [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; FFS; FFS; FFS; FFS; Component 3 candidate values: {1,2,3,4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; FFS; FFS; FFS; FFS; [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; Per FS; NO; NO; NO; [Component 3 candidate values: {1,2,3}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values; Optional with capability signalling. |
| Apple [10] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; Per FSPC; n/a; n/a; n/a; Component 3 candidate values: {1,2,3,4} Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-3-5a (M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based) - Similar with FG 59-3-4, for the description on cyclic mapping in Component 1, we would like to keep the sentence for having consistency with Rel-17 UE capability (FG 23-3-1-3). - Similar with FG 59-3-4, we would like to delete the note “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” since this is related to PDCCH repetition which is not relevant on FG 59-3-5a. NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; 59-3-1, 23-3-1-3; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; Per FSPC; FFS; FFS; FFS; Component 3 candidate values: {1,2,3,4} FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; FFS; yes; n/a; M-TRP PUSCH repetition is not supported for 3TX PUSCH transmission with type B for non-codebook based; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-5a - Support components 1,2. - Candidate values of component 3 should be {1,2,3}. - Prerequisite: 59-3-1 - The “Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values” is not related to this FG and should be removed. - Type: per FSPC. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] | Further, it is tentatively decided that the maximum number of 1 port SRS resources per SRS set to support 3 Tx non-codebook PUSCH for single TRP (S-TRP) could take values from 1-3. Meanwhile, the Rel-15 UE capability parameter for the maximum number of SRS resources in non-codebook based operation allows values from 1-4, with the maximum number of layers being 1, 2, or 4. In this case, the maximum number of SRS resources and layers align. However, it should also be observed that a UE can declare, for example, a maximum 1 layer capability while supporting up to 4 SRS resources in non-codebook based operation. For 3 Tx operation, the maximum number of layers is 3. One possible reason for limiting the 3 Tx non-codebook capability to a maximum of 3 SRS resources is to simplify operation for UEs with 3 Tx chains that do not virtualize their antennas. Such UEs could toggle their Tx chains on or off without needing to switch between antennas or implement virtualization. However, these UEs could alternatively be designed to support at most 3 SRS resources. On the other hand, maximizing the number of SRS resources offers potential diversity benefits, especially when the maximum number of layers is lower. This is in our understanding a primary motivation for having the smaller layer capability than the number of SRS resources discussed above. Furthermore, it was tentatively decided that the maximum number of SRS resources in one SRS set for 3 Tx multiple TRP (M-TRP) non-codebook PUSCH could take values up to 4, as shown below. This decision, however, introduces an inconsistency with the parameter choices made for S-TRP operation. NR MIMO Phase 5; 59-3-5a; M-TRP PUSCH repetition (type B) of 3-antenna-port PUSCH transmission – non-codebook based; Support of M-TRP PUSCH repetition for 3-antenna-port PUSCH transmission with type B for non-codebook based - sequential mapping for repetitions larger than 2 [- cyclic mapping for 2 repetitions] 2. Support of two SRS resource sets with usage set to 'non-codebook' 3. Supported number of SRS resources in one SRS resource set; [Component 3 candidate values: {1,2,3,4}] Note: Two linked PDCCH candidates are not expected to be associated with different CORESETPoolIndex values FFS: Further partitioning of this FG based on existing and future agreements. There does not seem to be a benefit to reduce the maximum number of SRS resources for non-codebook based operation below the 4 supported in Rel-15. Supporting a maximum of 3 resources would instead limit the potential for diversity gain, and be less consistent with the principle of allowing a maximum layer capability that is less than the number of SRS resources. There seems to be inconsistency in the value for the maximum number of SRS resources in an SRS set for S-TRP and M-TRP 3 TX non-codebook PUSCH. Support maxNumberSRS-ResourcePerSet = 4 for Rel-19 3 Tx non-codebook based operation. |

| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | FFS | FFS | FFS | FFS | [Candidate values: {1,2}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; FFS; FFS; FFS; FFS; [Candidate values: {1,2}]; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; FFS; FFS; FFS; FFS; Candidate values: {1,2}; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; FFS; FFS; FFS; FFS; [Candidate values: {1,2}]; Optional with capability signalling. |
| CMCC [6] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2. |
| vivo [7] |  |
| OPPO [8] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; FFS; FFS; FFS; FFS; [Candidate values: {1,2}]; Optional with capability signalling. |
| Xiaomi [9] |  |
| Apple [10] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; Per FSPC; n/a; n/a; n/a; Component candidate values {1, 2}; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; 59-3-1 or 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; Per Band; FFS; FFS; FFS; Candidate values: {1,2}; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-6; PTRS of 3-antenna-port PUSCH transmission; Number of supported PTRS ports for PUSCH transmission; [59-3-1 or] 59-3-2; yes; n/a; PTRS is not supported for 3TX PUSCH transmission; FFS Per SFBC; FFS NO; FFS NO; FFS NO; [Candidate values: {1,2}]; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-6 - Support the candidate values. - Prerequisite: 59-3-1 or 59-3-2. - Type: per band. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] |  |

| NR MIMO Phase 5 | 59-3-7 | Support of full-power Mode 0 for codebook-based for 3 Tx operation | UL full power transmission mode of fullpower | 59-3-2 | yes | n/a | UL full power transmission mode of fullpower is not supported for 3 Tx operation | FFS | FFS | FFS | FFS | Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | NR MIMO Phase 5; 59-3-7; Support of full-power Mode 0 for codebook-based for 3 Tx operation; UL full power transmission mode of fullpower; 59-3-2; yes; n/a; UL full power transmission mode of fullpower is not supported for 3 Tx operation; FFS; FFS; FFS; FFS; Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | NR MIMO Phase 5; 59-3-7; Support of full-power Mode 0 for codebook-based for 3 Tx operation; UL full power transmission mode of fullpower; 59-3-2; yes; n/a; UL full power transmission mode of fullpower is not supported for 3 Tx operation; FFS; FFS; FFS; FFS; Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] |  |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | NR MIMO Phase 5; 59-3-7; Support of full-power Mode 0 for codebook-based for 3 Tx operation; UL full power transmission mode of fullpower; 59-3-2; yes; n/a; UL full power transmission mode of fullpower is not supported for 3 Tx operation; Per FSPC; n/a; n/a; n/a; Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | NR MIMO Phase 5; 59-3-7; Support of full-power Mode 0 for codebook-based for 3 Tx operation; UL full power transmission mode of fullpower; 59-3-2; yes; n/a; UL full power transmission mode of fullpower is not supported for 3 Tx operation; Per FS; FFS; FFS; FFS; Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-3-7; Support of full-power Mode 0 for codebook-based for 3 Tx operation; UL full power transmission mode of fullpower; 59-3-2; yes; n/a; UL full power transmission mode of fullpower is not supported for 3 Tx operation; FFS Per SFBC; FFS NO; FFS NO; FFS NO; Note: If a UE does not support this FG, Rel. 15 power scaling procedures apply FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-3-7 - Type: per FS. - Need of FDD/TDD differentiation: No. - Need of FR1/FR2 differentiation: No. - Capability interpretation for mixture of FDD/TDD and/or FR1/FR2: No. |
| Ericsson [14] |  |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] |  |
| CMCC [6] | NR MIMO Phase 5; 59-3-8; 3T3R; Support 't3r3' for 3T3R; 2-53. NR MIMO Phase 5; 59-3-9; 3T3R; Support 't3r6' for 3T6R; 2-55. |
| vivo [7] |  |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-3-3b; Maximum 2 semi-persistent and 1 periodic SRS sets for 3 port SRS antenna switching; Support of maximum 2 semi-persistent SRS resource sets and maximum 1 periodic SRS resource set for 3 port SRS antenna switching; 59-3-3, 59-3-3a; Yes; n/a; Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported; Per FS; n/a; n/a; n/a; Note: The two semi-persistent SRS resource sets are not activated at the same time; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] |  |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

## Asymmetric DL sTRP/UL mTRP scenarios

| 59. NR_MIMO_Ph5 | 59-4-1 | PL offset for PUCCH/PUSCH/SRS power control | Support of applying path loss offset for PUCCH/PUSCH/SRS power controls | FFS | yes | n/a | PL offset for PUCCH/PUSCH/SRS power control is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)) | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | FG 59-4-1 (PL offset for PUCCH/PUSCH/SRS power control) Regarding whether to further partition this FG for joint DL/UL TCI(s) and separate UL TCI(s), we don’t see the need since the UE behavior to apply PL offset to UL power control should be no difference in these two modes. Proposal XX: On FG 59-4-1, no need to separate further partition this FG for joint DL/UL TCI(s) and separate UL TCI(s) |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-1; PL offset for PUCCH/PUSCH/SRS power control; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls; one of: 40-1-1 or 23-1-1; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-1a, - The prerequisite FG 23-10-1 or FG 23-10-1m (which is for separate DL/UL TCI state mode) and FG 23-1-1h (which is for association between TCI state and UL PC settings) need to be added. - For FG 59-4-1b, - The prerequisite FG 23-1-1 or FG 23-1-1a (which is for joint TCI state mode) and FG 23-1-1h (which is for association between TCI state and UL PC settings) need to be added. 59. NR_MIMO_Ph5; 59-4-1a; PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state mode; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls under separate DL/UL TCI state mode; FFS 23-10-1 or 23-10-1m, 23-1-1h; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state mode is not supported; FFS Per Band; FFS; FFS no; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-1b; PL offset for PUCCH/PUSCH/SRS power control under joint TCI state mode; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls under joint TCI state mode; 23-1-1 or 23-1-1a, 23-1-1h; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control under joint TCI state mode is not supported; Per Band; FFS; Applicable only to FR1; FFS; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-1; PL offset for PUCCH/PUSCH/SRS power control; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls; 8-3, 23-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-1; PL offset for PUCCH/PUSCH/SRS power control; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls; 59-4-9 or 59-4-10 or 59-4-11 or 59-4-12; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-1; Support of association of a UL TCI state with a PL offset; Support association of a UL TCI state with a PL offset; Yes; n/a; Association of a UL TCI state with a PL offset is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-1a; Support of association of a joint TCI state with a PL offset; Support association of a joint TCI state with a PL offset; Yes; n/a; Association of a UL TCI state with a PL offset is not supported; Per FS; n/a; FR1 only; n/a; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-4-1 (PL offset for PUCCH/PUSCH/SRS power control), since we define separate FGs for joint TCI framework (FG 23-1-1 for Rel-17 and FG 40-1-1 for Rel-18) and separate TCI framework (FG 23-10-1 for Rel-17 and FG 40-1-2 for Rel-18), and joint TCI framework is only supported in FR1 while separate TCI framework is supported both FR1 and FR2 for PL offset indication, defining separate FGs for each of joint/separate is appropriate way. 59. NR_MIMO_Ph5; 59-4-1; PL offset for PUCCH/PUSCH/SRS power control for joint TCI; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint TCI; 23-1-1 and/or 40-1-1; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control for joint TCI is not supported; Per band; FFS; FFS; FFS; Note: FR1 only FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-1a; PL offset for PUCCH/PUSCH/SRS power control for separate TCI; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate TCI; 23-10-1 and/or 40-1-2; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-1; PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint DL/UL TCI state; FFS 23-1-1 or 23-1-1b or 40-1-1 or 40-1-1a; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state is not supported; FFS Per FSPC; FFS NO; FFS FR1 only; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-1a; PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state; Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate DL/UL TCI state; FFS 23-10-1 or 23-10-1b or 40-1-2 or 40-1-2a; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-1 - Partitioning for joint TCI and separate TCI is not necessary. - Prerequisite FG is not necessary. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting pathloss offset for PUCCH/PUSCH/SRS power controls - “FFS: Prerequisite feature groups” - 8-3 basic power control operation - 23-1-1/23-1-1a Rel-17 Unified TCI, or - 40-1-1/40-1-1a Rel-18 unified TCI - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A Two components can be added to reflect the two agreements below, namely, supporting pathloss offset for separate TCI state mode and supporting pathloss offset for joint TCI state mode. Agreement For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1 Agreement For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. Add the following two components to FG 59-4-1: - supporting pathloss offset for PUCCH/PUSCH/SRS power controls associated with separate TCI state - supporting pathloss offset for PUCCH/PUSCH/SRS power controls associated with joint TCI state |

| 59. NR_MIMO_Ph5 | 59-4-2 | Path Loss offset on PDCCH-order PRACH | Support of applying path loss offset on PDCCH-order PRACH | FFS | yes | n/a | Applying path loss offset on PDCCH-order PRACH is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; 59-4-1; yes; n/a; Applying path loss offset on PDCCH-order PRACH is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-2, - The prerequisite FG 59-4-1a or FG 59-4-1b need to be added. 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; FFS 59-4-1a or 59-4-1b; yes; n/a; Applying path loss offset on PDCCH-order PRACH is not supported; FFS Per Band; FFS; FFS no; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; 23-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; 59-4-9 or 59-4-10 or 59-4-11 or 59-4-12; yes; n/a; Applying path loss offset on PDCCH-order PRACH is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; 59-4-1 or 59-4-1a; yes; n/a; Applying path loss offset on PDCCH-order PRACH is not supported; Per FS; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH; 59-4-1 or 59-4-1a; yes; n/a; Applying path loss offset on PDCCH-order PRACH is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-2; Path Loss offset on PDCCH-order PRACH for joint DL/UL TCI state; Support of applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state; FFS 23-1-1 or 23-1-1b or 40-1-1 or 40-1-1a; yes; n/a; Applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state is not supported; FFS Per FSPC; FFS NO; FFS FR1 only; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-2a; Path Loss offset on PDCCH-order PRACH for separate DL/UL TCI state; Support of applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state; FFS 23-10-1 or 23-10-1b or 40-1-2 or 40-1-2a; yes; n/a; Applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 9-4-2 - Partitioning for joint TCI and separate TCI is not necessary. - Prerequisite FG is FG59-4-1, because PL-offset for PDCCH order PRACH uses PL-offset in a TCI state for PUCCH/PUSCH/SRS. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting pathloss offset on PUCCH-order PRACH - “FFS: Prerequisite feature groups” - 8-3 basic power control operation - 23-1-1/23-1-1a Rel-17 Unified TCI, or - 40-1-1/40-1-1a Rel-18 unified TCI - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A Introduce the following two components to FG 59-4-2: - support of applying pathloss offset on PUCCH-order PRACH when Rel-17 unified TCI is configured - support of applying pathloss offset on PUCCH-order PRACH when Rel-18 unified TCI is configured |

| 59. NR_MIMO_Ph5 | 59-4-3 | Two SRS closed-loop power control adjustment states separate from PUSCH | Support of two separate SRS closed loop indices separate from PUSCH | FFS | yes | n/a | Two separate SRS closed loop indexes is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] | FG 59-4-3 (Two SRS closed-loop power control adjustment states separate from PUSCH) Based on the following agreement, 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 should be introduced to support two SRS CLPC adjustment states. This should be added as one component of FG 59-4-3. Agreement For enhancing DCI format 2_3 for indicating TPC command for two SRS CLPC adjustment states, support Alt2: - Alt2: Introduce one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 - This 1-bit closed-loop-indicator indicates the first SRS CLPC adjustment state or the second SRS CLPC adjustment state. Note: this 1-bit indicator is present for the CC where two SRS CLPC adjustment states are configured.. Proposal XX: On FG 59-4-3, add one component as follows: - Support one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH; - Support of two separate SRS closed loop indices separate from PUSCH - Support one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3; FFS; yes; n/a; Two separate SRS closed loop indexes is not supported; FFS; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH SRS closed loop indices; Support of two separate SRS closed loop indices separate from PUSCH; 59-4-1; yes; n/a; Two separate SRS closed loop indexes is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-3a, - The prerequisite FG 23-10-1 or FG 23-10-1m (which is for separate DL/UL TCI state mode) and FG 8-6 (which is for TPC-SRS-RNTI) need to be added. - For FG 59-4-3b, - The prerequisite FG 23-1-1 or FG 23-1-1a (which is for joint TCI state mode) and FG 8-6 (which is for TPC-SRS-RNTI) need to be added. 59. NR_MIMO_Ph5; 59-4-3a; Two SRS closed-loop power control adjustment states separate from PUSCH under separate DL/UL TCI state mode SRS closed loop indices; Support of two separate SRS closed loop indices separate from PUSCH under separate DL/UL TCI state mode; FFS 23-10-1 or 23-10-1m, 8-6; yes; n/a; Two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported; FFS Per Band; FFS; FFS no; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-3b; Two SRS closed-loop power control adjustment states separate from PUSCH under joint TCI state mode; Support of two separate SRS closed loop indices separate from PUSCH under joint TCI state mode; FFS 23-1-1 or 23-1-1a, 8-6; yes; n/a; Two separate SRS closed loop indexes under joint TCI state mode is not supported; Per Band; FFS; Applicable only to FR1; FFS; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH SRS closed loop indices; Support of two separate SRS closed loop indices separate from PUSCH; 23-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH; Support of two separate SRS closed loop indices separate from PUSCH; 59-4-9 or 59-4-9a or 59-4-10 or 59-4-10a or 59-4-11 or 59-4-12; yes; n/a; Two separate SRS closed loop indexes is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH SRS closed loop indices; Support of two separate SRS closed loop indices separate from PUSCH; yes; n/a; Two separate SRS closed loop indexes is not supported; Per FS; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH; Support of two separate SRS closed loop indices separate from PUSCH; FFS; yes; n/a; Two separate SRS closed loop indexes is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-3; Two SRS closed-loop power control adjustment states separate from PUSCH; Support of two separate SRS closed loop indices separate from PUSCH; FFS 23-1-1 or 23-1-1a or 23-1-1b or 23-10-1 or 23-10-1b or 23-10-1m or 40-1-1 or 40-1-1a Or 40-1-2 or 40-1-2a; yes; n/a; Two separate SRS closed loop indexes is not supported; FFS Per FSPC; FFS TDD only; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-3 - Partitioning for joint TCI and separate TCI is not necessary. - Prerequisite FG is not necessary, because prerequisite FG5-3 (Basic power control operation) is mandatory without capability signaling. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting two separate SRS closed loop indices separated from PUSCH - “FFS: Prerequisite feature groups” - 8-3: basic power control operation - 8-6: TPC-SRS-RNTI - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A |

| 59. NR_MIMO_Ph5 | 59-4-4 | Support two TAs enhancement | Support of two TAs without the restriction of multi-DCI based multi-TRP operation | FFS | yes | n/a | Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation; 40-2-1, 59-4-1,; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-4a, - The prerequisite FG 23-10-1 or FG 23-10-1m (which is for separate DL/UL TCI state mode) and FG 40-2-1 or FG 40-2-2 (which is for two TAs enhancement) need to be added. - For FG 59-4-4b, - The prerequisite FG 23-1-1 or FG 23-1-1a (which is for joint TCI state mode) and FG 40-2-1 or FG 40-2-2 (which is for two TAs enhancement) need to be added. 59. NR_MIMO_Ph5; 59-4-4a; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation under separate DL/UL TCI state mode; FFS 23-10-1 or 23-10-1m, 40-2-1 or 40-2-2; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation under separate DL/UL TCI state mode is not supported; FFS Per Band; FFS; FFS no; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4b; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation under joint TCI state mode; 23-1-1 or 23-1-1a, 40-2-1 or 40-2-2; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation under joint TCI state mode is not supported; Per Band; FFS; Applicable only to FR1; FFS; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation; 23-1-1. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation; 59-4-9 or 59-4-9a or 59-4-10 or 59-4-10a or 59-4-11 or 59-4-12; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported; Per FS; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements (see below for further partition); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4; Support of Single-DCI based intra-cell Multi-TRP operation with two TA; Support single-DCI based inter-cell Multi-TRP operation with two TA; Yes; n/a; Single-DCI based inter-cell Multi-TRP operation with two TA is not supported; Per FS; n/a; n/a; n/a; Candidate value: bitmap {PUSCH TDM repetition Type A, PUSCH TDM repetition Type B, PUCCH TDM repetition, PUSCH SDM, PUSCH SFN, PUCCH SFN} Note: Single-DCI based Multi-TRP is the operation without the restriction of “the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs” Note: FG40-2-8, if reported, or supportedNumberTAG” in 38.306 is applied for the maximum number of TAGs; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4a; Support of Single-DCI based inter-cell Multi-TRP operation with two TA; Support single-DCI based inter-cell Multi-TRP operation with two TA; Yes; n/a; Single-DCI based inter-cell Multi-TRP operation with two TA is not supported; Per FS; n/a; n/a; n/a; Candidate value: bitmap {PUSCH TDM repetition Type A, PUSCH TDM repetition Type B, PUCCH TDM repetition, PUSCH SDM, PUSCH SFN, PUCCH SFN} Note: Single-DCI based Multi-TRP is the operation without the restriction of “the UE is provided two coresetPoolIndex values 0 and 1 for first and second CORESETs, or is not provided coresetPoolIndex value for first CORESETs and is provided coresetPoolIndex value of 1 for second CORESETs” Note: FG40-2-8, if reported, or supportedNumberTAG” in 38.306 is applied or the maximum number of TAGs; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement; Support of two TAs without the restriction of multi-DCI based multi-TRP operation; 59-4-1 or 59-4-2; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-4; Support two TAs enhancement for intra-cell beam management or sDCI based intra-cell Multi-TRP operation; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management or sDCI based intra-cell Multi-TRP operation; FFS 23-1-1 or 23-1-1b or 23-10-1 or 23-10-1b or 40-1-1 or 40-1-1a Or 40-1-2 or 40-1-2a; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management or sDCI based intra-cell Multi-TRP operation is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4a; Support two TAs enhancement for inter-cell beam management operation; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management; 23-1-1a or 23-1-1b or 23-10-1m; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-4 - Partitioning for joint TCI and separate TCI or for sTRP and sDCI mTRP is not necessary. - Prerequisite FG is not necessary, because it is applicable to sTRP or sDCI mTRP. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting two TAs without restriction of multi-DCI based multi-TRP operation - “FFS: Prerequisite feature groups” - 40-1-1/40-1-1a/40-1-2/40-1-2a: unified TCI for single-DCI based mTRP - 40-2-3: TAG ID indiation via absolute TA command MAC CE - 40-2-6: RX timing difference larger than CP length - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A |

| 59. NR_MIMO_Ph5 | 59-4-5 | Overlapping UL transmission reduction | Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement | FFS | yes | n/a | Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported | FFS | FFS | FFS | FFS | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; 40-2-9, 59-4-4,; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; Per Band; N/A; N/A; N/A; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-5, - The prerequisite FG 59-4-4a or FG 59-4-4b need to be added. 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; FFS 59-4-4a or 59-4-4b; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; FFS Per Band; FFS; FFS n/a; FFS; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; 59-4-4. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; 59-4-9 or 59-4-9a or 59-4-10 or 59-4-10a or 59-4-11 or 59-4-12; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; FFS; FFS; FFS; FFS; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; 59-4-4; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; Per band; n/a; n/a; n/a; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; 59-4-1 or 59-4-2, 59-4-4; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; Per band; FFS; FFS; FFS; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-5; Overlapping UL transmission reduction; Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions when the UE is with two TA enhancement; FFS 59-4-4; yes; n/a; Reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions is not supported; FFS Per band; FFS NO; FFS NO; FFS n/a; Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-5 - Partitioning for joint TCI and separate TCI or for sTRP and sDCI mTRP is not necessary. - Prerequisite FG can be “FG23-3-1, FG23-3-1-2 (mTRP PUSCH repetition)”. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting reduction of overlapping duration of the later of the two time-domain overlapping UL transmissions when the Ue is with two TA enhancement - “FFS: Prerequisite feature groups” - 40-2-9: Overlapping UL transmission reduction - 59-4-4: two TAs without restriction of multi-DCI based multi-TRP operation - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A |

| NR MIMO Phase 5 | 59-4-6 | MAC-CE update of PL offset value(s) | Support of MAC-CE update of the configured PL offset value(s) | FFS | yes | n/a | MAC-CE update of the configured PL offset value(s) is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); 59-4-1; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-6, - The prerequisite FG 59-4-1a or FG 59-4-1b need to be added. NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); FFS 59-4-1a or 59-4-1b; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; FFS Per Band; FFS; FFS n/a; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| CMCC [6] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); 59-4-1. |
| vivo [7] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); 59-4-9 or 59-4-10 or 59-4-11 or 59-4-12; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); 59-4-1 or 59-4-1a; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; Per FS; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); 59-4-1 or 59-4-2; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | NR MIMO Phase 5; 59-4-6; MAC-CE update of PL offset value(s); Support of MAC-CE update of the configured PL offset value(s); FFS 59-4-1 or 59-4-1a; yes; n/a; MAC-CE update of the configured PL offset value(s) is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-6 - Partitioning for joint TCI and separate TCI or for sTRP and sDCI mTRP is not necessary. - Prerequisite FG is FG59-4-1. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting MAC-CE updated of the configured pathloss offset values - “FFS: Prerequisite feature groups” - 59-4-1: pathloss offset for PUCCH/PUSCH/SRS power controls - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A |

| 59. NR_MIMO_Ph5 | 59-4-7 | Extended value range of starting bit of block in DCI format 2_3 | Support of the extended value range of starting bit of DCI format 2_3 | FFS | yes | n/a | The extended value range of starting bit of DCI format 2_3 is not supported | FFS | FFS | FFS | FFS | Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; 59-4-1; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; Per Band; N/A; N/A; N/A; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios); Optional with capability signalling. |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-7, - The prerequisite FG 8-6 (which is for TPC-SRS-RNTI) needs to be added. 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; FFS 8-6; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; FFS Per Band; FFS; FFS no; FFS; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios); Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; 8-6. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; 59-4-9 or 59-4-9a or 59-4-10 or 59-4-10a or 59-4-11 or 59-4-12; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; FFS; FFS; FFS; FFS; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; 8_5; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; Per FS; NO; NO; NO; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise; Optional with capability signalling. |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; Per FS; n/a; n/a; n/a; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios); Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; FFS; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; Per band; FFS; FFS; FFS; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios); Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-7; Extended value range of starting bit of block in DCI format 2_3; Support of the extended value range of starting bit of DCI format 2_3; FFS; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported; FFS Per band; FFS NO; FFS NO; FFS n/a; Note: The starting bit of value range extends to X=45 for operations in FR1 in shared spectrum or FR2-2 and X = 43 otherwise FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios); Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-7 - Partitioning is not necessary. - Prerequisite FG can be FG8-6 (TPC-SRS-RNTI). - Type can be per band. |
| Ericsson [14] | This is the FG for supporting extended value range of starting bit of block in DCI format 2_3 - “FFS: Prerequisite feature groups” - 59-4-3: two separate SRS closed loop indices separated from PUSCH - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A |

| 59. NR_MIMO_Ph5 | 59-4-8 | DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) | FFS | yes | n/a | DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states” | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] | To address the FFS in 59-4-8, based on the following agreement, we believe that this FG should be split to two FGs: One for “the support of DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another for “the support of DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”. Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.. Proposal 5.1: Split 59-4-8 to two FGs: One for “the support of DCI format 1_1 for TPC command indication for a separate SRS CLPC adjustment state” and another for “the support of DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”. |
| MediaTek Inc. [2] |  |
| Nokia [3] | 59. NR_MIMO_Ph5; 59-4-8; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); 59-4-1; yes; n/a; DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported; Per Band; N/A; N/A; N/A; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. |
| CATT [4] | 59-4-8: - The agreement in RAN1#118bis meeting that support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH was made for the scenarios that the UE is configured with two SRS CLPC adjustment states. Therefore there is no need to separate it to two FGs for indicating TPC command for one separate SRS CLPC adjustment state and indicating TPC command for two separate SRS CLPC adjustment states, respectively. Proposal 48: For FG 59-4-8, do not seperate it to two FGs for indicating TPC command for one separate SRS CLPC adjustment state and indicating TPC command for two separate SRS CLPC adjustment states, respectively. |
| ZTE Corporation/Sanechips [5] | - For FG 59-4-8a and FG 59-4-8b, - The prerequisite FG 23-10-1 or FG 23-10-1m (which is for separate DL/UL TCI state mode) need to be added. 59. NR_MIMO_Ph5; 59-4-8a; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under separate DL/UL TCI state mode; Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under separate DL/UL TCI state mode; FFS 23-10-1 or 23-10-1m; yes; n/a; DCI 1_1 indicating TPC command for separate SRS closed loop index(es) under separate DL/UL TCI state mode is not supported; FFS Per Band; FFS; FFS no; FFS; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-8b; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under joint TCI state mode; Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under joint TCI state mode; 23-1-1 or 23-1-1a; yes; n/a; DCI 1_1 indicating TPC command for separate SRS closed loop index(es) under joint TCI state mode is not supported; Per Band; FFS; Applicable only to FR1; FFS; Optional with capability signalling. |
| CMCC [6] | 59. NR_MIMO_Ph5; 59-4-8; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); 59-4-3. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-8; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); 59-4-9 or 59-4-9a or 59-4-10 or 59-4-10a or 59-4-11 or 59-4-12; yes; n/a; DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] |  |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-8; DCI format 1_1 to indicate 2-bit TPC for separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); yes; n/a; DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported; Per FS; n/a; n/a; n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-8a; Support of DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH; Support DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH; Yes; n/a; DCI format 1_1 to indicate 1-bit SRS close-loop indicator for two SRS CLPC adjustment states separate from PUSCH is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] | Regarding FG 59-4-8 (DCI format 1_1 to indicate TPC for separate SRS closed loop index(es)), based on the following agreement in RAN1#119, two separate FGs are needed. One is supporting SRS TPC command field in DCI format 1_1, and the other is supporting SRS closed-loop indicator field in DCI format 1_1. Obviously, since the second one (supporting SRS closed-loop indicator field in DCI format 1_1) is based on two separate SRS closed loop, FG 59-4-3 can be a pre-requisite for the second one. Agreement Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.. 59. NR_MIMO_Ph5; 59-4-8; SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1; Support SRS TPC command field for SRS CLPC adjustment state(s) separate from PUSCH in DCI format 1_1; FFS; yes; n/a; SRS TPC command field for SRS CLPC adjustment state separate from PUSCH in DCI format 1_1 is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-8a; SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1; Support SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1; 59-4-3, 59-4-8; yes; n/a; SRS closed-loop indicator field for one of two separate SRS CLPC adjustment states in DCI format 1_1 is not supported; Per band; FFS; FFS; FFS; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| Qualcomm Incorporated [12] | 59. NR_MIMO_Ph5; 59-4-8; DCI format 1_1 to indicate TPC for a separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for a separate SRS closed loop index(es); FFS; yes; n/a; DCI 1_1 indicating TPC command for a separate SRS closed loop index(es) is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-8a; DCI format 1_1 to indicate TPC for two separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for two separate SRS closed loop index(es); FFS 59-4-3; yes; n/a; DCI 1_1 indicating TPC command for two separate SRS closed loop index(es) is not supported; FFS Per FSPC; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. |
| NTT DOCOMO, INC. [13] | 59-4-8 - Partitioning is not necessary. - Prerequisite FG is not necessary. - Type can be per band. |
| Ericsson [14] | This is the FG for supporting DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) - “FFS: Prerequisite feature groups” - “FFS: Type 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC)”: Per Band - “FFS: Need of FDD/TDD differentiation”: N/A - “FFS: Need of FR1/FR2 differentiation”: N/A - “FFS: Capability Interpretation for mixture of FDD/TDD and/or FR1/FR2”: N/A Agreement The working assumption part of the following previous agreement is confirmed. Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: • (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: o The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). • (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command o The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured. Introduce the following two components to FG 59-4-8: - support of application of DCI format 1_1 to indicate TPC for one separate SRS closed loop index - support of application of DCI format 1_1 to indicate TPC for two separate SRS closed loop indexes |

Other

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [1] |  |
| MediaTek Inc. [2] |  |
| Nokia [3] |  |
| CATT [4] |  |
| ZTE Corporation/Sanechips [5] | - Generally, as per the following agreement, the basic UE features of PL offset, two SRS closed-loop power control adjustment states separated from PUSCH and two TAs need to be specified under separate DL/UL TCI state mode and joint TCI state mode, respectively. In this sense, the granularity should be per band to keep the alignment. Agreement (RAN1#116) For the asymmetric DL sTRP/UL mTRP deployment scenarios, separate DL/UL TCI state mode of Rel-17/18 unified TCI framework can be configured for both FR1 and FR2. - Joint TCI state mode can be configured at least for FR1. - FG 59-4-9a and FG 59-4-9b are newly added to specify UE features of DCI format 1_1 to indicate one of two separate SRS closed loop indexes according to the following agreement. Agreement (RAN1#119) The working assumption part of the following previous agreement is confirmed. Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.. 59. NR_MIMO_Ph5; 59-4-9a; DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode; Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under separate DL/UL TCI state mode; 23-10-1 or 23-10-1m, 59-4-8a; yes; n/a; DCI 1_1 indicating one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported; Per Band; FFS; no; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-9b; DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode; Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under joint TCI state mode; 23-1-1 or 23-1-1a, 59-4-8b; yes; n/a; DCI 1_1 indicating one of two separate SRS closed loop indexes under joint TCI state mode is not supported; Per Band; FFS; Applicable only to FR1; FFS; Optional with capability signalling. |
| CMCC [6] | Fourthly, based on the working assumption in RAN1#119 meeting, to support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH, supporting of 2-bit TPC command field and 1-bit SRS close-loop indicator field are separate UE capabilities. Agreement@119 The working assumption part of the following previous agreement is confirmed. Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.. 59. NR_MIMO_Ph5; 59-4-8-1; DCI format 1_1 to indicate SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command; Support DCI format 1_1 to indicate SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command; 59-4-8. |
| vivo [7] | 59. NR_MIMO_Ph5; 59-4-9; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell; Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell; 23-1-1; yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-9a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell; Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell; 23-1-1a; Yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-10; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell; Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell; 23-10-1; Yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-10a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell; Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell; 23-10-1m; Yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-11; Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP; Support of asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP; 40-1-1 or 40-1-1a; Yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-12; Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP; Support of asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP; 40-1-2 or 40-1-2a; Yes; n/a; Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| OPPO [8] | a) PL offset Whether UE supports the PL offset configured in or associated with UL or Joint TCI state should be UE capability. b) two CLPC adjustments states for SRS (separated from PUSCH) Whether UE supports two CLPC adjustments states for SRS (separated from PUSCH) should be a very basic UE capability. In addition, whether UE supports DCI format 2_3 for indicating TPC command and/or closed-loop index (CLI) also should be a UE capability. Proposal 7: The UE capabilities in the following table should be introduced for asymmetric multi-TRP scenario, it includes - The UE capabilities clearly listed in previous agreements and - The UE capabilities not clearly listed in previous agreements but to be discussed in Rel-19 MIMO UE features discussion 59. NR_MIMO_Ph5; 59-4-1; Path Loss offset; 1. PL offset in UL TCI state 2. PL offset in joint TCI state; 8-2, 23-1-1; yes; n/a; The function of PL offset is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-1a; Path Loss offset on PDCCH-order PRACH at FR1; 1.Support applying PL offset for PDCCH order PRACH at FR1; 59-4-1; yes; n/a; Applying PL offset for PDCCH order PRACH at FR1 is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-2; Two separate SRS closed loop indices; 1. Support two separate CLPC adjustment states; 8-2, 23-1-1; yes; n/a; The function of two separate SRS closed loop indexes is not supported; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-2a; Extended value range of starting bit of DCI format 2_3; 1. Support the extended value range of starting bit of DCI format 2_3; 8-5; yes; n/a; The extended value range of starting bit of DCI format 2_3 is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-2b; DCI format 2_3 to indicate TPC for separate SRS closed loop index(es); 1. Support DCI format 2_3 to indicate TPC for separate SRS closed loop index. 2.Support DCI format 2_3 to indicate TPC for two separate SRS closed loop indexes.; 8-2; yes; n/a; The function of DCI 2_3 indicating TPC command for separate SRS closed loop index(es) is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-2c; DCI format 1_1 to indicate TPC for separate SRS closed loop index(es); 1. Support DCI format 1_1 to indicate TPC for separate SRS closed loop index. 2.Support DCI format 1_1 to indicate TPC for two separate SRS closed loop indexes.; 8-2; yes; n/a; The function of DCI 1_1 indicating TPC command for separate SRS closed loop index(es) is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-3; Support two TAs enhancement; 1.Support two TAs without the restriction of multi-DCI based multi-TRP operation.; 23-1-1; yes; n/a; The function of 2 TAs for asymmetric DL sTRP/UL mTRP is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. 59. NR_MIMO_Ph5; 59-4-3a; Overlapping UL transmission; 1.Support the overlapping of UL transmission for two TAGs.; 23-1-1; yes; n/a; UE does not expect two UL transmissions associated with different TAGs are overlapped.; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Xiaomi [9] |  |
| Apple [10] | 59. NR_MIMO_Ph5; 59-4-9; Support of including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH; Support including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH; Yes; n/a; PL offset in the calculation of Type 1 PHR is not supported; Per FS; n/a; n/a; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-10; Support of one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH; Support one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH; Yes; n/a; One 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH is not supported; Per FS; n/a; n/a; n/a; Optional with capability signalling. |
| BEIJING SAMSUNG TELECOM R&D [11] |  |
| Qualcomm Incorporated [12] |  |
| NTT DOCOMO, INC. [13] |  |
| Ericsson [14] |  |

# Discussion Items during RAN1 #120

After review of contributions submitted to RAN1 #120 in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #120.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## New, additional FGs for UE-initiated/event-driven beam management

After review of contributions submitted to RAN1 #120 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-1-6 | First PUCCH and second PUSCH from different PUCCH groups | Support first PUCCH and second PUSCH from different PUCCH groups | FG 59-1-1 | yes | n/a | First PUCCH and second PUSCH from different PUCCH groups is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are okay with this FG |
| ZTE2 | Support, it is aligned with the agreement for confirmation of WA that reached in Monday online session. |
| MediaTek | Support |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a certain threshold based on one event instance within a time window 2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported | Per Band | FFS | Applicable only to FR2 | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance within a time window one the new beam 2. Maximum number of the configured RS(s) for new beam measurement in the RS resource set 3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively | 59-1-1 | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported | Per Band | FFS | Applicable only to FR2 | FFS | Component 1 candidate values for Q: {1, 2, ..., 8} Component 2 candidate values: {1, 2, ..., 64} | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We may need to consider mode A and mode B Also for FG59-1-5, component 1 candidate value needs some clarification, i.e., is it related to Q? |
| ZTE2 | Agree, it is inline with the agreement for supporting Event-1 and Event-7 in addition to Event-2. Regarding Mode-A and Mode-B for the two events, it can be reached by FG 59-1-1 and FG 59-1-2, respectively. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-1-6 | Cross CC beam reporting for Event-2 with same PUCCH group | Support of cross CC beam reporting for Event-2 with first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group | 59-1-1 or 59-1-2 |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-6-1 | Cross CC beam reporting for Event-2 with different PUCCH group | Support of cross CC beam reporting for Event-2 with first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group | 59-1-6 |  |  |  |  |  |  |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | This is similar as the FG proposed above |
| ZTE2 | It seems premature due to lack of the corresponding agreements. |
| MediaTek | FG 59-1-6 should be sufficient. These two are not needed. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances within a time window. | Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window [with M>1 per one new beam] | FFS | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS | FFS | FFS | FFS | Component candidate values: [{2,…,M}] FFS: whether to merge FG 59-1-1 and FG 59-1-3 FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are okay. The candidate values can be further discussed |
| ZTE | Per new beam RS should be captured. |
| MediaTek | We suggest the following changes: 59. NR_MIMO_Ph5; 59-1-3; Triggering event determination via detecting ≥ M event instances within a time window.; Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) within a time window for least one new beam [with M>1 per one new beam]. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-1-4 | Measurement resource for UE initiated beam report | Per slot limitations: 1. The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report 2. The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot for UE initiated beam report Memory limitations: 3. The max number of CSI-RS resources across all CCs configured for UE initiated beam report [4. The max number of aperiodic CSI-RS resources across all CCs configured for UE initiated beam report] 5. The max number of CSI-RS resources in a CSI-RS resource set configured for new beam for UE initiated beam report Other limitations: 6. Supported density of CSI-RS (CMR) |  | Yes | n/a | UE initiated beam report is not supported | Per band | n/a | n/a | n/a | Component 1: Candidate values {8, 16, 32, 64} Component 2: Candidate values {0, 4, 8, 16, 32, 64} Component 3: Candidate values {8, 16, 32, 64, 128} [Component 4: Candidate values {0, 2, 4, 8, 16, 32, 64}] Component 5: Candidate values {2, 4, 8, 16, 32, 64} Component 6: Candidate values {'1 only', '3 only', '1 and 3'} Note: The reference slot duration is the shortest slot duration defined for the FR where the reported band belongs Note: For component 3, 4, 5 the configured CSI-RS resources for both active and inactive BWPs are counted Note: For components 1, 2, a SSB/CSI-RS resource is counted within the duration of a reference slot in which the corresponding reference signals are transmitted | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-1-5 | Maximum N for UE initiated beam report | Support of UE initiated beam report with maximum N L1-RSRP values |  | Yes | n/a | Maximum N=1 | Per band | n/a | n/a | n/a | Component candidate values {1, 2, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-6 | Configuration that current beam is always reported for UE initiated beam report | Support RRC to configure that current beam is always reported for UE initiated beam report |  | Yes | n/a | RRC to configure that current beam is always reported for UE initiated beam report is not supported | Per band | n/a | n/a | n/a | Note: The reported current beam, if configured, is not counted in the N reported beams | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-1-7 | Beam reporting timing for Mode B UE initiated beam report | For Mode B UE initiated beam report, the minimum number of symbols, X, between the last symbol of report notification on the first PUCCH channel and the first symbol of the second UL channel carrying the UE initiated beam report |  | Yes | n/a |  | Per band | n/a | n/a | n/a | Candidate values: different X in symbols can be reported for different SCS 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |
| ZTE2 | Regarding candidate value of X in FG 59-1-7, at least {0, 1} should be captured. |
| MediaTek | RAN1 never agree to use aperiodic CSI-RS resource for UEI beam reporting |

## New, additional FGs for CSI enhancements for up to 128 ports

After review of contributions submitted to RAN1 #120 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | 1. Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously |  | Yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port is {48, 64, 128} The candidate value set of the max # of resources is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We can add component 2 A list of supported combinations, each combination is {Max # of Tx ports, Max # of resources, and total # of Tx ports} simultaneously |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-1-XX | NES SD Type1 timeline for Rel-19 Type-I single-panel codebook | Support NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook | 59-2-1-1, 1a, 1b, 1c, 1d, or 1e | Yes | n/a | BaseStation does not know UE’s CSI processing timeline, and may have to assume a long report latency | Per-band and per-BC | n/a | n/a | n/a | Component 1 candidate values: Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by maxi=1MPi, P/32 where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1- XX | CSI processing timeline for Rel-19 Type-I SP and Type-II codebook | Support from two timeline capabilities, for Rel-19 Type-I single-panel codebook | 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a | Yes | n/a | BaseStation does not know UE’s CSI processing timeline, and may have to assume a long report latency | Per-band and per-BC | n/a | n/a | n/a | Component 1 candidate values: Capability 1: Reuse legacy Z/Z’ values Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources Note: A UE that supports 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a must signal this FG | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | For these two feature groups, we need to decide one of the capabilities as baseline capability which should be part of the basic feature. Our preference it to make Capability 1 as the baseline capability. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-1-XX | CSI-RS port mapping method1 for aggregation of K={2,3,4} CSI-RS resources | Support CSI-RS port mapping method1 (N1-dimension aggregation) for Rel-19 Type-I/II codebooks | 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 5, 5a, or 5b | Yes | n/a | For K={2,3,4} CSI-RS resources, UE only supports CSI-RS port mapping method2 (N2-dimension aggregation) | Per-band and per-BC | n/a | n/a | n/a | The support of mapping method2 (N2-dimension aggregation) is required for the prerequisite FGs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-XX | {K=3, Q=16} for P = KQ = 48 CSI-RS ports | Support K=3 CSI-RS resources each with Q=16 ports for Rel-19 Type-I/II codebooks | 59-2-1-1a, 1d, 3a, 4a, or 5a | Yes | n/a | {K=3, Q=16} is not supported by UE Note: P=48 ports can still be supported by {K=2, Q=24} | Per-band and per-BC | n/a | n/a | n/a |  | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-2-1-XX | {K=4, Q=16} for P = KQ = 64 CSI-RS ports | Support K=4 CSI-RS resources each with Q=16 ports for Rel-19 Type-I/II codebooks | 59-2-1-1, 1c, 3, 4, or 5 | Yes | n/a | {K=4, Q=16} is not supported by UE Note: P=64 ports can still be supported by {K=2, Q=32} | Per-band and per-BC | n/a | n/a | n/a |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | For these FGs, suggest to change the reporting gruanlity to Per band. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-1-10 | Mapping method from CSI-RS resource/port index to CSI/PMI calculation for Rel-19 Type I and eType II CB | Support of mapping methods from CSI-RS resource/port index to CSI/PMI calculation for Rel-19 Type I and eType II CB | FFS | Yes | n/a | Aggregating of multiple NZP CSI-RS resources is not supported | FFS | FFS | FFS | FFS | Candidate values: {’Method 1’, ‘Method 2’, ‘Method 1 and 2’} Note: UE supporting Rel-19 Type I and eType II CB needs to report this capability | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | This seems to be duplicate of the other proposals below |
| Ericsson | It seems duplicated with other prosposals. |
| ZTE | Similar view as Apple and Ericsson. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-2-4 | Configuration of MR always-reported resources | For Rel-16 eType-II, MR={1} | 59-2-2-3 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 | N/A | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are okay |
| Ericsson | Ok |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-3-9 | CPU for CJTC report | 1.Support candidate values of X for CPU for CJTC DO report 2.Support candidate values of X for CPU for CJTC FO report 3.Support candidate values of X for CPU for CJTC PO report 4.Support candidate values of X for CPU for CJTC Dd+FO report | FFS | yes | n/a | CPU for CJTC report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values:{1, 2} Component 2 candidate values:{1, 2} Component 3 candidate values:{1, 2} Component 4 candidate values:{1, 2} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We aer okay |
| Ericsson | Ok. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-3-10 | Timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT | Support value of Drelax | FFS | yes | n/a | Timeline for joint triggering od CJTC Dd and Rel-18 eType-II CJT is not supported | FFS | FFS | FFS | FFS | candidate values:{0, drelax } | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer UE to report drelax for each SCS in units of symbols |
| Ericsson | ok |
| ZTE | We prefer drelax = Z1’, then drelax is predefined for different SCS. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-1-1f | CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports | Support CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports |  | Yes | n/a | CBSR for Rel-15 Type-I SP (single panel) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1h | CSI-RS port mapping for Type-I and Type II codebook enhancement for up to 128 ports | 1. Supported CSI-RS port mapping schemes for Type-I and Type II codebook enhancement for up to 128 ports |  | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate value {Mapping method 1, Mapping method 2, Mapping method 3} Note: The CSI-RS port mapping applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5 | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-1i | CSI-RS resource time domain restriction for Type-I and Type II codebook enhancement for up to 128 ports | 1. Maximum number of slots the K CSI-RS resources shall be configured within for Type-I and Type II codebook enhancement for up to 128 ports |  | Yes | n/a | No time domain restriction for K CSI-RS resources | Per band and Per BC | n/a | n/a | n/a | Candidate value {1 slot, 2 slots} Note: The supported CSI-RS resource time domain restriction applies to all of the following, FG59-2-1, FG59-2-2, FG59-2-3, FG59-2-4, FG59-2-5 | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-3c | PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 |  | Yes | n/a | PMI sub-bands with R=2 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3d | Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports |  | Yes | n/a | Parameter combinations 7-8 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3e | Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports | 1. Support of Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports 2. Support of parameter combinations with M=1 3. Support of rank 1-2 4. Support R=1 5. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=1 and R=1 6 Supported processing capability |  | Yes | n/a | Rank 3,4 for Rel-16 eType-II codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 5 candidate values a. {48, 64} b. {1, …, 64} c. {64, …, 256} Component 6 candidate value {Capability 1, Capability 2} Note: For component of processing capability Capability 1: Reuse legacy Z/Z’ values OCPU = ceil(P/32) Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources OCPU = ceil(P/32) | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-4b | M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | 1. Support M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports 2. Support of parameter combinations with M=2 3. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=1 | 59-2-4 | Yes | n/a | M=2 and R=1 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-4c | R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | 1. Support R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is { Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with M=2 and R=2 | 59-2-4 | Yes | n/a | R=2 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64, 128} b. {1, …, 64} c. {64, …, 256} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4d | Rank 3,4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | Support rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports | 59-2-4 | Yes | n/a | Rank 3, 4 for Rel-17 FeType-II PS (port selection) codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-5c | N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one report, Max # of resources and total # of Tx ports} for one CSI report setting 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR | 59-2-5 | Yes | n/a | N4>1 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {1,2,4,8} b. {48, 64,128} c. {2,3,4 … 64} d. {64, …, 256} Component 3 Candidate values a. {1,2,4,8} b. {48, 64,128} c. {4,8,12} d.{64, …, 256} |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-5d | DD unit size d=1 when A-CSI-RS is configured for CMR N4>1 | Support value of d=1 for the DD unit size when A-CSI-RS is configured for CMR for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | Value of d=1 for the DD unit size when A-CSI-RS is configured for CMR N4>1 is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5e | Maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum number of aperiodic CSI-RS resources groups that can be configured in the same CSI report setting for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values: {4, 8, 12} |  |
| 59. NR_MIMO_Ph5 | 59-2-5f | PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Support PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. A list of supported combinations, each combination is {Max # of Tx ports in one report, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously with R=2 | 59-2-5 | Yes | n/a | PMI subband R=2 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a | Component 2 candidate values a. {48, 64,128} b. {2,3,4 … 64} c. {64, …, 256} |  |
| 59. NR_MIMO_Ph5 | 59-2-5g | X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support X=1 based on first and last slot of WCSI for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | X=1 based on first and last slot of WCSI, for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5h | X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | X=2 CQI based on 2 slots for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5i | l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support l = (n – nCSI,ref ) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | l = (n – nCSI,ref) for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5j | L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support L=6 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | L=6 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5k | Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Support Rank 3 and 4 for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a | Rank 3 and 4 for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports is not supported | Per band and Per BC | n/a | n/a | n/a |  |  |
| 59. NR_MIMO_Ph5 | 59-2-5l | Processing timeline for CSI reference slot for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 1. Aperiodic CSI report timing relaxation, w, Rel-18 Type-II Doppler codebook enhancement for up to 128 ports 2. Aperiodic CSI report timing relaxation Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols: {14*(KP–1)*d, 14*KP*d} Note: Kp is according to Component 12 of FG59-2-5 Note: d=4 (minimum periodicity of periodic CSI-RS) Component 2 candidate values: {CAP1, CAP2} For N4 = 1 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP1 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m, Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w, Z'2) For N4 > 1 and CAP2 in component 2 1) For AP CSI-RS: (Z,Z’) = (Z2 + 14*( KDOPP –1)*m + Z'2, 2Z'2) 2) For P/SP CSI-RS: (Z,Z’) = (Z2 + w + Z'2, 2Z'2) Z2/Z'2 are defined in Table 5.4-2 in TS38.214 KDOPP is the number of CSI-RS resource groups configured for channel measurement, and each CSI-RS resource groups contain K CSI-RS resources for aggregating up to 128 ports M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots |  |
| 59. NR_MIMO_Ph5 | 59-2-5m | Maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | Supported maximum periodicity of CMR when configured as periodic CSI-Rs for Rel-18 Type-II Doppler codebook enhancement for up to 128 ports | 59-2-5 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values (in slots): {4, 5, 8, 10, 20} | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-3-3a | Maximum number of CSI-RS resources in a report configuration for phase offset report | 1. Maximum number of CSI-RS resources in a report configuration for phase offset report | 59-2-10 | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-3b | Maximum number of phase offset report settings per BWP | 1. Maximum number of phase offset report settings (CSI-ReportConfig) configured with resourcesForChannelMeasurement linked to a same BWP ID | 59-2-10 | Yes | n/a |  | Per FS | n/a | n/a | n/a | Candidate values: {1, 2, 3, 4} | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3c | Maximum number of quantization points M for phase offset report | 1. Maximum number of quantization points M for phase offset report | 59-2-10 | Yes | n/a |  | Per band and Per BC | n/a | n/a | n/a | Candidate values: {16, 32} | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |
| Ericsson | Ok |
| ZTE | For 59-2-3-3c, prefer to change this FG to ‘Number of quantization points for phase offset report’, the component should be ‘Supported number of quantization points for phase offset report’, and candidate values of this component should be ’16, 32, 16 and 32’. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-2-3-9 | RRC configuration of 1 SRS port of antenna switching associated with phase offset report | Support of RRC configuration of 1 SRS port of antenna switching associate with phased offset report | 59-2-10 | Yes | n/a | RRC configuration of 1 SRS port of antenna switching associated with phase offset report is not supported | Per band and Per BC | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |
| Ericsson | OK |

## New, additional FGs for 3-antenna-port codebook-based transmissions

After review of contributions submitted to RAN1 #120 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| NR MIMO Phase 5 | 59-3-8 | 3T3R | Support 't3r3' for 3T3R | 2-53 |
| --- | --- | --- | --- | --- |
| NR MIMO Phase 5 | 59-3-9 | 3T3R | Support 't3r6' for 3T6R | 2-55 |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | It is not clear to us why the proposed 59-3-9/8 are not already covered by 59-3/3a, but we are open to discuss. |
| Apple | Dupicate of FG59-3-3 and FG59-3-3a |
| ZTE | Similar view as Ericsson. It seems FGs 59-3/3a are sufficient. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-3-3b | Maximum 2 semi-persistent and 1 periodic SRS sets for 3 port SRS antenna switching | Support of maximum 2 semi-persistent SRS resource sets and maximum 1 periodic SRS resource set for 3 port SRS antenna switching | 59-3-3, 59-3-3a | Yes | n/a | Maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS is supported | Per FS | n/a | n/a | n/a | Note: The two semi-persistent SRS resource sets are not activated at the same time | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Ericsson | OK in general. Suggest ’59-3-3 or 59-3-3a’ for clarity (avoids potentially confusing the comma to mean ‘and’). We prefer to discuss Type (i.e. ‘Per Band vs. Per FS’) later; suggest to leave that column blank for now. However, we note for future discussion that SRS switching for < 8T is per band (e.g. 2-55, 23-8-3 are per band). |
| Apple | We are supportive, and we also okay with the change suggested by Ericsson |
| ZTE | Okay. |
| MediaTek | Support |

## New, additional FGs for Asymmetric DL sTRP/UL mTRP scenarios

After review of contributions submitted to RAN1 #120 in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-9a | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under separate DL/UL TCI state mode | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under separate DL/UL TCI state mode | 23-10-1 or 23-10-1m, 59-4-8a | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under separate DL/UL TCI state mode is not supported | Per Band | FFS | no | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9b | DCI format 1_1 to indicate one of two separate SRS closed loop indexes under joint TCI state mode | Support of DCI format 1_1 to indicate TPC for separate SRS closed loop index(es) under joint TCI state mode | 23-1-1 or 23-1-1a, 59-4-8b | yes | n/a | DCI 1_1 indicating one of two separate SRS closed loop indexes under joint TCI state mode is not supported | Per Band | FFS | Applicable only to FR1 | FFS |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Some discussion is needed whether we need to separate separate and joint TCI and the applicability |
| QC | - More discussion on whether to split FG59-4-8 “DCI format 1_1 to indicate TPC for separate SRS closed loop index(es)” to FG59-4-9a and 59-4-9b above based on joint DL/UL TCI state and separate DL/UL TCI states is needed. However, before that, based on the agreements from RAN1#119, FG59-4-8 “DCI format 1_1 to indicate TPC for separate SRS closed loop index(es)” should be split into the following two FGs: - FG59-4-8a: DCI format 1_1 to indicate TPC for a separate SRS closed loop index - FG59-4-8b: DCI format 1_1 to indicate TPC for two separate SRS closed loop index(es) Agreement The working assumption part of the following previous agreement is confirmed. Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH: - (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where: - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this). - (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured. 59. NR_MIMO_Ph5; 59-4-8a; DCI format 1_1 to indicate TPC for a separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for a separate SRS closed loop index(es); FFS; yes; n/a; DCI 1_1 indicating TPC command for a separate SRS closed loop index(es) is not supported; FFS Per band; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-8b; DCI format 1_1 to indicate TPC for two separate SRS closed loop index(es); Support of DCI format 1_1 to indicate TPC for two separate SRS closed loop index(es); FFS 59-4-3; yes; n/a; DCI 1_1 indicating TPC command for two separate SRS closed loop index(es) is not supported; FFS Per band; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., whether to split based on deployment scenario such asymmetric DL sTRP/UL mTRP and other scenarios) and also whether to have one FG is “DCI format 1_1 for TPC command indicatinon for a separate SRS CLPC adjustment state” and another is “DCI format 1_1 for TPC command indication for two separate SRS CLPC adjustment states”; Optional with capability signalling. |
| Ericsson | We agree with QC’s comments above and proposal. This is also what we proposed in our contribution, either two components or two UE capabilites to capture the two different capabilities as agreed in RAN1 agreement. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-2a | Extended value range of starting bit of DCI format 2_3 | 1. Support the extended value range of starting bit of DCI format 2_3 | 8-5 | yes | n/a | The extended value range of starting bit of DCI format 2_3 is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-2b | DCI format 2_3 to indicate TPC for separate SRS closed loop index(es) | 1. Support DCI format 2_3 to indicate TPC for separate SRS closed loop index. 2.Support DCI format 2_3 to indicate TPC for two separate SRS closed loop indexes. | 8-2 | yes | n/a | The function of DCI 2_3 indicating TPC command for separate SRS closed loop index(es) is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In general, we are okay |
| QC | - The FG “Extended value range of starting bit of DCI format 2_3” is already agreed as FG59-4-7 in last RAN1 meeting. FG59-4-2a seems duplicated with the previous agreed 59-4-7. - For FG59-4-2b, we have the following comments: - Component 1 “Support DCI format 2_3 to indicate TPC for separate SRS closed loop index.” is not needed. Since the UE may support DCI format 2_3 to indicate TPC for separate SRS closed loop index while not support DCI format 2_3 to indicate TPC for two separate SRS closed loop indexes. Not support this FG means the UE doesn’t support DCI format 2_3 to indicate TPC for two separate SRS closed loop indexes, but it doesn’t mean the UE doesn’t support DCI format 2_3 to indicate TPC for a separate closed loop index. - The prerequisite FG 8-2 is not needed since it is mandatory without capability signaling. 59. NR_MIMO_Ph5; 59-4-2b; DCI format 2_3 to indicate TPC for two separate SRS closed loop index(es); 1. Support DCI format 2_3 to indicate TPC for separate SRS closed loop index. 2.Support DCI format 2_3 to indicate TPC for two separate SRS closed loop indexes.; 8-2; yes; n/a; The function of DCI 2_3 indicating TPC command for two separate SRS closed loop index(es) is not supported.; Per band; n/a; n/a; n/a; Optional with capability signaling. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-9 | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell | Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell | 23-1-1 | yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for intra-cell is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-9a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell | Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell | 23-1-1a | Yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with joint DL/UL TCI update for inter-cell is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-10 | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell | Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell | 23-10-1 | Yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for intra-cell is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-10a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell | Support of asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell | 23-10-1m | Yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-17 unified TCI with separate DL/UL TCI update for inter-cell is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-11 | Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP | Support of asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP | 40-1-1 or 40-1-1a | Yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-4-12 | Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP | Support of asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP | 40-1-2 or 40-1-2a | Yes | n/a | Asymmetric DL sTRP/UL mTRP with Rel-18 unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | FG59-4-11 and FG59-4-12 seem to be redundant |
| QC | In Rel.17 and Rel.18, there are already existing FGs for the joint or separate DL/UL TCI state update for intra-cell, inter-cell or sDCI based intra-cell multi-TRP, we don’t see the need to introduce new FGs FG59-4-9 to FG59-4-12. |
| ZTE2 | We do not see the necessity of the above FGs in terms of intra/inter-cell or MTRP schemes. |
| MediaTek | These FGs seem not needed |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-1 | Path Loss offset | 1. PL offset in UL TCI state 2. PL offset in joint TCI state | 8-2, 23-1-1 | yes | n/a | The function of PL offset is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-1a | Path Loss offset on PDCCH-order PRACH at FR1 | 1.Support applying PL offset for PDCCH order PRACH at FR1 | 59-4-1 | yes | n/a | Applying PL offset for PDCCH order PRACH at FR1 is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | FG59-4-1a seems to be duplicate of FG59-4-2 |
| QC | - For 59-4-1, in previous meeting, the FG 59-4-1 “PL offset for PUCCH/PUSCH/SRS power control” has been agreed. Is this FG 59-4-1 “Path loss offset” same as previous FG 59-4-1 “PL offset for PUCCH/PUSCH/SRS power control”? - If FG 59-4-1 refers to the “PL offset for PUCCH.PUSCH/SRS power control” as agreed before, we think FG59-4-1 should be split into the following two FGs: 59. NR_MIMO_Ph5; 59-4-1a; PL offset for PUCCH/PUSCH/SRS power control for joint DL/UL TCI state(s); Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for joint DL/UL TCI state(s); 23-1-1; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control under joint DL/UL TCI state(s) is not supported; Per band; NO; FR1 only; n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-1b; PL offset for PUCCH/PUSCH/SRS power control for separate DL/UL TCI state(s); Support of applying path loss offset for PUCCH/PUSCH/SRS power controls for separate DL/UL TCI state(s); 23-10-1; yes; n/a; PL offset for PUCCH/PUSCH/SRS power control under separate DL/UL TCI state(s) is not supported; Per band; NO; NO; n/a; FFS: Further partitioning of this FG based on existing and future agreements (e.g., joint DL/UL TCI(s) and separate UL TCI(s)); Optional with capability signalling. - For FG59-4-1a “Path Loss offset on PDCCH-order PRACH at FR1”, instead of introducing this new FG, we can split the previous agreed FG59-4-2 into the following two FGs, this is because for joint DL/UL TCI state, only FR1 is supported; while for separate DL/UL TCI state, both FR1 and FR2 can be supported. 59. NR_MIMO_Ph5; 59-4-2a; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH for joint DL/UL TCI state(s); 23-1-1; yes; n/a; Applying path loss offset on PDCCH-order PRACH under joint DL/UL TCI state(s) is not supported; Per band; NO; FR1 only; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-2a; Path Loss offset on PDCCH-order PRACH; Support of applying path loss offset on PDCCH-order PRACH for separate DL/UL TCI state(s); 23-10-1; yes; n/a; Applying path loss offset on PDCCH-order PRACH under separate DL/UL TCI state(s) is not supported; Per band; NO; NO; n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. |
| ZTE2 | Share the same to Apple. |
| MedaiTek | We are supportive to QC’s suggestion |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-3 | Support two TAs enhancement | 1.Support two TAs without the restriction of multi-DCI based multi-TRP operation. | 23-1-1 | yes | n/a | The function of 2 TAs for asymmetric DL sTRP/UL mTRP is not supported. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-3a | Overlapping UL transmission | 1.Support the overlapping of UL transmission for two TAGs. | 23-1-1 | yes | n/a | UE does not expect two UL transmissions associated with different TAGs are overlapped. | Per band | n/a | n/a | n/a |  | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | In general, we are okay |
| QC | - For FG59-4-3 “Support two TAs enhancement”, this seems to be the same as the previous agreed FG59-4-4. In this case, we need to split this FG into the following three FGs. This is because the UE may support two TAs for intra-cell BM, but doesn’t support two TAs for inter-cell BM or sDCI based intra-cell mTRP.In addition, the Type can be per FS. In Rel.18 two TAs, the Type of two TAs is per FSPC since Rel.18 two TAs is conditioned on mDCI mTRP. While for Rel.19 per FSPC is not needed since this is not for mDCI mTRP. Per FS seems a good choice. 59. NR_MIMO_Ph5; 59-4-4a; Support two TAs enhancement for intra-cell beam management; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management; FFS 23-1-1; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation for intra-cell beam management is not supported; FFS Per FS; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4b; Support two TAs enhancement for inter-cell beam management operation; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management; FFS 23-10-1; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation for inter-cell beam management is not supported; FFS Per FS; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. 59. NR_MIMO_Ph5; 59-4-4c; Support two TAs enhancement for sDCI based intra-cell Multi-TRP operation; Support of two TAs without the restriction of multi-DCI based multi-TRP operation for sDCI based intra-cell Multi-TRP operation; FFS 40-1-1; yes; n/a; Two TAs without the restriction of multi-DCI based multi-TRP operation sDCI based intra-cell Multi-TRP operation is not supported; FFS Per FS; FFS NO; FFS NO; FFS n/a; FFS: Further partitioning of this FG based on existing and future agreements; Optional with capability signalling. - For FG 59-4-3a, this seems to be the same as the previous agreed FG59-4-5. This FG59-4-5 “Overlapping UL transmission reduction” should be conditioned on two TAs. Therefore, the prerequisite FG should be one of FG59-4-4a, or FG59-4-4b or FG59-4-4c above. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 59. NR_MIMO_Ph5 | 59-4-9 | Support of including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH | Support including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH |  | Yes | n/a | PL offset in the calculation of Type 1 PHR is not supported | Per FS | n/a |  | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-4-10 | Support of one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH | Support one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH |  | Yes | n/a | One 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are supportive |
| QC | - We are fine with FG59-4-9 “Support of including PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH”. - For FG59-4-10, this can be discussed together with 59-4-2b “DCI format 2_3 to indicate TPC for separate SRS closed loop index(es)”. In our view, as long as 59-2-4b “DCI format 2_3 to indicate TPC for separate SRS closed loop index(es)” is supported, the UE needs supports one 1-bit closed-loop-indicator field for each TPC command in DCI format 2_3 when two closed-loop power control adjustment states are configured for SRS separate from PUSCH. Therefore, FG59-4-10 is not needed. |
| ZTE2 | Support. |
| MediaTek | Support FG 59-4-9 Same view as QC, FG 59-4-9 should be merge to FG 59-4-2b |

# Conclusion

Agreements reached during RAN1 #120 as part of this agenda item are summarized in [16].

# References

- R1-2500101, UE features for NR MIMO Phase 5, Huawei/HiSilicon

- R1-2500108, UE features for NR MIMO Phase 5, MediaTek Inc.

- R1-2500110, Views on NR MIMO Phase 5 UE features, Nokia

- R1-2500213, Discussion on UE features for NR MIMO Phase 5, CATT

- R1-2500246, Discussion on UE features for NR MIMO Phase 5, ZTE Corporation/Sanechips

- R1-2500307, Discussion on UE features for NR MIMO Phase 5, CMCC

- R1-2500372, Discussion on UE features for NR MIMO Phase 5, vivo

- R1-2500474, UE features for NR MIMO Phase 5, OPPO

- R1-2500721, Discussion on UE features for NR MIMO Phase 5, Xiaomi

- R1-2500802, Views on UE features for NR MIMO Phase 5, Apple

- R1-2501120, UE features for NR MIMO Phase 5, BEIJING SAMSUNG TELECOM R&D

- R1-2501181, UE features for NR MIMO phase 5, Qualcomm Incorporated

- R1-2501221, Discussion on UE features for NR MIMO Phase 5, NTT DOCOMO, INC.

- R1-2501271, UE features for NR MIMO Phase 5, Ericsson

- R1-2410821, Initial RAN1 UE features list for Rel-19 NR MIMO Phase 5, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2501602, Session Notes of AI 9.15.2, Ad-Hoc Chair (AT&T)