3GPP TSG RAN WG1 #120bis									R1-2501833

Wuhan, China, April 7th – 11th, 2025

Source:          vivo

Title:		 UE features for Rel-19 LP-WUS/WUR

Agenda Item:    9.15.5

Document for:   Discussion and Decision

- Introduction

In this contribution, we provide updates to UE features for Rel-19 LP-WUS/WUR based on the latest agreements made.

- NR_LPWUS

- Proposal with change mark (based on endorsed version in R1-2501397)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK，M=1,2,4 supported 2. The support of LP-SS based RRM measurement M=1,2,4 supportedand/or SSB-based measurement, corresponding to component 1] -3. The support of further RRM relaxation of UE MR- -4. The support of serving RRM measurement offloading from UE MR to LR- -5. The supported wake-up delay- FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM -overlaid- sequence(s) | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence(s), M=1,2,4 2. FFS: The support of LP-SS based RRM measurement X. The support of SSB-based RRM measurement 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OOK signal，M=1,2,4 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. The supported maximum number of codepoints monitored per LP-WUS MO 5 The support of LP-WUS frequency resource outside active DL BWP FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: (Option 1-1, Option 1-2) Component 3 candidate values: FFS Component 4: candidate value: at least 8, FFS which value(s) smaller than 8 Component 5: (False, True) - Note that “True” is optional as agreed in RAN1#120 | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) | 1. LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OFDM overlaid sequence(s), M=1,2,4 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. The supported maximum number of codepoints monitored per LP-WUS MO 5 The support of LP-WUS frequency resource outside active DL BWP FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: (Option 1-1, Option 1-2) Component 3 candidate values: FFS Component 4: candidate value: at least 8, FFS which value(s) smaller than 8 Component 5: (False, True) - Note that “True” is optional as agreed in RAN1#120 | Optional with capability signalling |

- Proposal in clean version.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 62. NR_LPWUS | 62-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK, M=1,2,4 supported. 2. The support of LP-SS based RRM measurement, M=1,2,4 for LP-SS supported 3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | Per Band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence(s) | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM overlaid sequence(s), M=1,2,4 supported. 2. FFS: The support of LP-SS based RRM measurement X. The support of SSB-based RRM measurement3. The support of further RRM relaxation of UE MR 4. The support of serving RRM measurement offloading from UE MR to LR 5. The supported wake-up delay FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported | Per band | n/a | n/a | n/a | Component 5 candidate values: FFS | Optional with capability signalling |
| 62. NR_LPWUS | 62-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OOK signal, M=1,2,4 supported 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. The supported maximum number of codepoints monitored per LP-WUS MO 5 The support of LP-WUS frequency resource outside active DL BWP FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: (Option 1-1, Option 1-2) Component 3 candidate values: FFS Component 4: candidate value: at least 8, FFS which value(s) smaller than 8 Component 5: (False, True) - Note that “True” is optional as agreed in RAN1#120 | Optional with capability signalling |
| 62. NR_LPWUS | 62-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) | 1. LP-WUS operation in CONNECTED mode to trigger PDCCH monitoring based on OFDM overlaid sequence(s), M=1,2,4 supported 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode 4. The supported maximum number of codepoints monitored per LP-WUS MO 5 The support of LP-WUS frequency resource outside active DL BWP FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: (Option 1-1, Option 1-2) Component 3 candidate values: FFS Component 4: candidate value: at least 8, FFS which value(s) smaller than 8 Component 5: (False, True) - Note that “True” is optional as agreed in RAN1#120 | Optional with capability signalling |