3GPP TSG-RAN WG1 Meeting #RAN 120b	R1-2501848

Wuhan, China, 07-11 Apr, 2025

Agenda Item:	9.1.3

Source:	TCL

Title:	Discussion on CSI prediction

Document for:	Discussion and Decision

Introduction

Based on the discussion at the RAN#105 meeting [1], the AI-based CSI prediction using the UE-sided model has been transitioned from SID to WID. The main objectives are highlighted in the yellow section below:

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024. - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025 - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

In this contribution, we provide our views on AI-based CSI prediction using the UE-side model.

CSI prediction using UE-sided model

In this section, our views are provided from the following three aspects: data collection, model inference, and performance monitoring for AI-based CSI prediction using the UE-side model.

Data collection

According to the process of the study on AI-based CSI prediction using the UE-side model, some specification impact on data collection has been proposed by companies as follows:

Data collection: In CSI prediction using UE sided model use case, at least the following aspects have been proposed by companies on data collection, including: - Signalling and procedures for the data collection - Data collection indicated by NW - Requested from UE for data collection - CSI-RS configuration - Assistance information for categorizing the data, if needed - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration

Considering that the data collection for training requires a large amount of sample data, while the data provided by aperiodic CSI measurements is very limited. Thus, from this perspective, we believe that it is unnecessary to support aperiodic CSI - RS resources for data collection during training.

: For data collection for training, aperiodic CSI-RS resource configuration should not be supported.

As mentioned above, the aperiodic CSI-RS resource configuration for data collection during training is unnecessary. Therefore, separately configuring CSI-RS resources used for measurements for model input and CSI-RS resources used for measurements for ground-truth CSI is also unnecessary. This is because, for data collection during training, both the model input and the ground-truth CSI can be obtained using periodic or semi-persistent CSI-RS resource.

: For data collection for training, the model input and the ground-truth CSI can be obtained based on a periodic and semi-persistent CSI-RS resource.

- For data collection during training, the UE is not required to provide feedback on the predicted CSI to the NW. Therefore, the NW can configure a CSI-ReportConfig with the reporting quantity set to ‘none’, and the codebookType is set to ‘AI-based CSI prediction’ or another specified name.

: For data collection for training,  the NW can configure a CSI-ReportConfig with the reporting quantity set to ‘none’.

Model inference

Based on previous research, the potential specification impacts for model inference have been proposed by companies as follows:

Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

In this section, we will present our views on model inference for CSI prediction using the UE-side model.

Update the number of predicted CSI dynamically

For the legacy Rel-18 Doppler codebook, the number of predicted CSIs is configured semi-statically by the RRC. Regardless of whether it is the legacy Rel-18 Doppler codebook or CSI prediction using the UE-side model, the performance of the predicted CSIs changes with variations in the channel environment. However, for CSI prediction using the UE-side model, the performance of the predicted CSIs can be monitored through performance evaluation. Intuitively, the further into the future the predicted CSI, the poorer the prediction performance becomes. Therefore, when the performance of the predicted CSI at longer time distances is detected to deteriorate, the model can reduce the number of predicted CSIs. Based on this, we propose that dynamically updating the number of predicted CSIs should be supported.

:For CSI prediction using UE-side model, dynamically update the number of predicted CSI should be supported.

Inference: For CSI prediction using UE-sided model, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI processing criteria and timeline Agreement For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on - Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting - Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities. - Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates. - Whether legacy framework for active CSI-RS resource and port counting can be reused Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

CSI processing unit and timeline

- As mentioned above, in the last meeting, the question whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting requires further study. In our opinion, this question depends on whether there are some hardware resources necessary and specific for AI/ML-based CSI reporting. For example, if the UE wants to realize some AI-based CSI reporting, the base-band system of  the UE requires to configure high performance CPU, GPU, or NPU. However, by leveraging model lightweight techniques such as pruning, quantization, and knowledge distillation, or through algorithm-hardware co-design approaches like binary neural networks, AI inference can be implemented on existing mobile hardware (CPU/DSP) without requiring additional hardware. For instance, lightweight CNNs or small-scale RNNs may not require NPU support. Therefore, based on the above analysis, we believe that whether the CPU is shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting is decided by the UE capability.

: Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting depends on the UE capability.

- Moreover, considering that AI/ML related features/functionalities are not always performed, if the processing unit is separately counted among related features/functionalities, it may lead to the low effectiveness of the hardware resource. Therefore, we suggest that the processing unit should be subject to shared counting among AI/ML related features/functionalities, which helps to improve the using effectiveness of the hardware resource.

: The processing unit should be subject to shared counting among AI/ML related features or functionalities.

For CSI prediction using the UE-side model, the report quantity varies depending on the operating mode. For example, for model training the UE does not report anything, during model inference the UE compresses and reports the prediction CSIs, and during performance monitoring the UE reports the performance metrics or both the predicted CSIs and performance metrics together. The different report quantity correspond to varying levels of CSI computational complexity. Moreover, the structure of the AI/ML models may also affect the model inference time. Therefore, we suggest that the CSI processing criteria and timeline for CSI prediction should be further discussed for different stages of model operation, e.g model training, model inference, and performance monitoring.

: For CSI prediction using UE-side model, CSI processing criteria and timeline should be further discussed for different stages of model operation.

Performance monitoring

Based on the previous meeting, the potential specification impacts for performance monitoring have been proposed by the companies as follows:

Performance monitoring: For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW; - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/deactivation/switching as defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report - Note: down selection is not precluded. - Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW. For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. - For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action Agreement For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support - Based on intermediate KPI - Down select between SGCS and NMSE by RAN1#120bis - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

In this part, our views on performance monitoring for CSI prediction using UE-sided model are presented.

Performance metric

In the last meeting, it has been agreed that a performance metric should be selected between SGCS and NMSE in this meeting. In our opinion, the range of SGCS belongs to [0,1], and the range of NMSE belongs to [0, +∞), it is clear that SGCS is more convenient for quantization to express the similarly between two matrices. Moreover, the SGCS is not effected by phase rotation. Therefore, we propose supporting SGCS as a performance metric for performance monitoring.

: For performance monitoring, it is proposed to support using SGCS as a performance metric to measure the level of similarity between the predicted CSI and the ground truth.

Performance monitoring for functionality-based LCM

In the previous meeting, three types of performance monitoring procedures were proposed by the companies. From the perspective of UE reporting overhead, Type 2 requires reporting the ground truth CSI. Compared with Type 1 and Type 3, the reporting overhead of Type 2 is significantly larger. Therefore, we believe that Type 2 should be deprioritized. Moreover, from the perspective of feedback information accuracy, Type 3 provides more information than Type 1. Therefore, we suggest that Type 3 should be used as the baseline, while the necessity of Type 1 needs further discussion. Furthermore, the definition and configuration of performance metrics need to be discussed.

: For CSI prediction using the UE-side model, Type 3 should be used as a baseline, and the definition and configuration of performance metrics need further discussion.

CSI-RS configuration for performance monitoring

For the legacy Rel-18 Doppler codebook, when the channel measurement resource is a periodic or semi-persistent CSI-RS resource, the time-domain interval between two adjacent predicted CSIs is equal to the period of the measurement resource. We believe this mechanism will be reused for AI-based CSI prediction, and the periodic and semi-persistent CSI-RS resources used for model inference can also be utilized for performance monitoring.

: For CSI prediction using the UE-side model, the periodic or semi-persistent CSI-RS resource can be used for both model inference and performance monitoring simultaneously.

Performance monitoring reporting

As mention above we consider that the predicted CSI reporting should support periodic, semi-persistent, and aperiodic reporting. Considering the mobility of the UE, in order to ensure the performance of the prediction CSI, we believe that the inference results should be monitored in real time, therefore, the performance metric(s), performance monitoring output, or ground truth CSI(s), if supported, should be reported with periodically, semi-persistently, and aperiodically.

Moreover, with the intention of reducing reporting overhead and improving the efficiency of reporting, we propose that event-driven reporting should be supported for performance metric(s). The definition of the event can be based on the performance monitoring results within one prediction window and/or across multiple prediction windows, depending on the number of predicted CSIs in the prediction window.

: For CSI prediction using the UE-side model, support real-time monitoring of inference result.

: For CSI prediction using the UE-side model, support periodic, semi-presistent, and aperiodic reporting the performance metric(s), performance monitoring output, or ground truth CSI(s), if supported.

:For performance monitoring, support UE reporting of performance metric(s) based on event-driven mechanisms.

Conclusions

This contribution presents our views on AI-based CSI prediction. We have the following proposals:

: For data collection for training, aperiodic CSI-RS resource configuration should not be supported.

: For data collection for training, the model input and the ground-truth CSI can be obtained based on a periodic and semi-persistent CSI-RS resource.

: For data collection for training,  the NW can configure a CSI-ReportConfig with the reporting quantity set to ‘none’.

:For CSI prediction using UE-side model, dynamically update the number of predicted CSI should be supported.

: Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting depends on the UE capability.

: The processing unit should be subject to shared counting among AI/ML related features or functionalities.

: For CSI prediction using UE-side model, CSI processing criteria and timeline should be further discussed for different stages of model operation.

: For performance monitoring, it is proposed to support using SGCS as a performance metric to measure the level of similarity between the predicted CSI and the ground truth.

: For CSI prediction using the UE-side model, Type 3 should be used as a baseline, and the definition and configuration of performance metrics need further discussion.

: For CSI prediction using the UE-side model, the periodic or semi-persistent CSI-RS resource can be used for both model inference and performance monitoring simultaneously.

: For CSI prediction using the UE-side model, support real-time monitoring of inference result.

: For CSI prediction using the UE-side model, support periodic, semi-presistent, and aperiodic reporting the performance metric(s), performance monitoring output, or ground truth CSI(s), if supported.

:For performance monitoring, support UE reporting of performance metric(s) based on event-driven mechanisms.

- References

- RP-242399, Revised WID for NR_AIML_air, Sep 9 – 12, 2024