3GPP TSG RAN WG1 #120bis		                                            R1-2502000

Wuhan, China, April 7th – 11th, 2025

Source:	CATT

Title:	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes

Agenda Item:	9.6.2

Document for:	Discussion and Decision

Introduction

In RAN1#120, the following agreements were achieved for studying low-power wake-up signal and receiver (LP-WUS/WUR) for NR [1]:

Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported) Agreement For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO Agreement The previous agreement in RAN1#119 is updated as follows: - Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. Agreement Confirm the following working assumption with the additional text in red: Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications Existing metrics SS-RSRP and SS-RSRQ are reused for OFDM-based LP-WUR. No separate metrics (LP-SSS-RSRP and LP-SSS-RSRQ) will be introduced in the specifications. - The metrics SS-RSRP and SS-RSRQ are applicable for OFDM-based LP-WUR for RRC_IDLE and RRC_INACTIVE serving cell measurement. - FFS: whether to remove or modify the restriction of using SMTC window for LP-SSS-RSRP/RSSI measurement. Agreement For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - Down-select between 2 and 4 for max number of POs per LO. Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624. Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values). Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32.

This paper discusses the aspects of LP-WUS monitoring procedures and the eDRX configuration for RRC_ IDLE/INACTIVE modes.

Discussion

As discussed in Rel-18 LP-WUS/WUR SI [2], significant UE power saving gain (up to more than 90%) can be achieved by using LP-WUS/WUR to trigger MR monitoring paging PDCCH compared with existing I-DRX operation (with and without PEI), if sufficient relaxation to MR RRM measurement is applied RRC_IDLE/INACTIVE modes UEs. Moreover, significant UE power saving gain and moderate paging latency increasing can be achieved if LP-WUS monitoring is restricted within existing PTW of eDRX. In this section, the procedure of LP-WUS monitoring and eDRX configuration are discussed.

The procedure of LP-WUS monitoring

Generally, the related procedures of LP-WUS monitoring can be described as following:

- Step1: UE capability of the supporting of LP-WUS and the wake-up delay is indicated during UE registration procedure through NAS procedure.

- Step2: gNB broadcasts SIB1 and SIB-X with the LP-WUS configuration.

- Step3: UE determines the PF and PO from SIB1.

- Step4: UE determines its LO based on the parameters in the LP-WUS configuration of SIB-X.

- Step5: UE determines MOs in its LO based on parameters in the LP-WUS configuration of SIB-X.

- Step6: UE derives the codepoint of the LP-WUS sequences based on subgroup/UE-ID and the LP-WUS configuration.

- Step7: UE monitor the PO depends on the LP-WUS indication.

Figure 1: The procedures that UE monitors LP-WUS

Step1 and Step2 are higher layer NAS procedures. Step3 to Step7 are AS higher layer  procedures, in which UE monitors LP-WUS based on LP-WUS configuration and PO based on the LP-WUS indication. The details of each step will be discussed in the following sections.

UE capability of supporting LP-WUS monitoring

In Step1, the network should receive the report from UE whether UE supports LP-WUS and associated paging subgrouping during the NAS registration. If the UE indicates the support of LP-WUS and associated NR paging subgrouping, the UE may include the LP-WUS paging subgrouping  information in the Requested LP-WUSPS(LP-WUS Power Saving) of  the UE assistance information IE in the REGISTRATION REQUEST message. If the UE indicates support of NR paging subgrouping and the network supports the use of the LP-WUSPS for triggering the UE wakeup for paging indication, the network provides the UE with the Negotiated PEIPS assistance information, including the Paging subgroup ID, in the REGISTRATION ACCEPT message or the CONFIGURATION UPDATE COMMAND message. The Paging subgroup ID is used to determine the NR paging subgroup for paging the UE. The UE NAS layer shall indicate the Paging subgroup ID to the access stratum layer. The network shall store the Paging subgroup ID in the 5GMM context of the specific UE [3].

Proposal 1: UE capability of the supporting of LP-WUS can be indicated during UE registration procedure through NAS procedure.

In RAN1#118bis, 3 candidate values for the wake-up delay capability a UE reports was supported. How UE reports the UE capability of its wake-up delay should be discussed and the exact values of the 3 wake-up delays are FFS.

The UE can transfer its UE capability information upon receiving a UECapabilityEnquiry from the network in RRC signaling when UE is in RRC_CONNECTED mode, as shown in Figure 2.

Figure 2: Procedure of UE capability transfer [5]

The network initiates the procedure to a UE in RRC_CONNECTED mode when it needs UE radio access capability information. The UE capability of the wake-up delay can be included in UECapabilityInformation. After receiving UECapabilityInformation, the network will store the wake-up delay capability in the 5GMM context of the UE. When the AMF tends to page a UE, the AMF initiates the Paging procedure by sending the PAGING message to the gNB as shown in Figure 3 [6]. However, if UE would report the UE capability of the wakeup delay to the gNB for the configuration, the UE needs to transition from the RRC_ IDLE/INACTIVE modes to the RRC_CONNECTED mode, which is a complex procedure. Also, when changing the camping cell, the UE needs to report again in the new camping cell.

Figure 3: Paging procedure from AMF to gNB [6]

Observation 1: If the UE capability of the wake-up delay is reported through RRC signaling procedure, UE is required to transition from RRC_ IDLE/INACTIVE modes to RRC_CONNECTED mode.

Alternatively, the UE capability of the wake-up delay can be reported during UE registration procedure through NAS procedure along with the LP-WUS paging subgrouping  information in the Requested LP-WUSPS of  the UE assistance information IE in the REGISTRATION REQUEST message, as discussion above. The network shall store the Paging subgroup ID and the wake-up delay in the 5GMM context of the specific UE.

Proposal 2：The UE capability of the wake-up delay can be reported during UE registration procedure through NAS procedure.

Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624.

In RAN1#120, 3 candidate values for the wake-up delay capability report were agreed. The remaining issue is that whether other SSB periodicity in addition to 20ms should be considered. Currently, the periodicity of SSB is assumed as 20ms for initial BWP. Since the SSB periodicity of 20 ms is assumed by legacy UEs in the IDLE/INACTIVE states, different SSB periodicities would not support legacy UEs in IDLE/ INACTIVE states. There is no benefit on the implementation and complexity to consider SSB periodicities other than 20 ms.

Proposal 3: There is no benefit of assuming SSB periodicities other than 20ms.

LP-WUS configuration information through system information and/or NAS signaling

For RRC_IDLE/INACTIVE modes UEs, the radio resource for the LP-WUS can be configured by Cell-specific signaling to all UEs in a cell by system information. In Rel-17, the resource allocation of PEI monitoring occasion in time domain for UE's PO is determined by a reference point and an symbol-level offset from the reference point to the start of the first PDCCH monitoring occasion. The reference point is the start of a reference frame determined by a frame-level offset from the start of the first PF of the PF(s) associated with the PEI monitoring occasions [4]. UE determines the first PDCCH MO for PEI monitoring occasions based on the frame-level offset and the symbol-level offset, which is broadcasted in SIB-1. If PEI and subgrouping information are configured to the UE, the paging group in the paging occasion can be divided into one or more subgroups. In the DCI format 2_7 field, each bit indicates the early paging indication of one paging subgroup of a PO. The paging subgroup can be either assigned by CN through NAS signaling or derived from UE_ID. A UE supporting CN assigned subgrouping can be assigned a subgroup ID (between 0 to 7) by AMF through NAS signaling. When the paging subgroup is derived from the UE_ID, the subgroup-ID is derived by UE-ID and the number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in SIB-1 or SIB-X.

In Step2, similar to the higher layer procedure of Rel-17 PEI configuration, the LP-WUS configuration information can be broadcasted in the system information.  Based on LP-WUS configuration, UE can derive the codepoint of the associated LP-WUS sequences and monitor the PO based on the LP-WUS wakeup indication. The exact RRC parameters used in each step will be discussed in the following.

Proposal 4: The parameters of the LP-WUS configuration are broadcasted and included in the SIB-1/SIB-X.

The determination of LP-WUS occasion

After receiving the system information of LP-WUS configuration, UE can determine its PO and PF based on legacy procedures as in 38.304 (i.e. Step3). The Step4  (i.e. UE determines its LO based on the parameters in the LP-WUS configuration) is discussed in this section.

Agreement—RAN1#117 It is supported that UEs monitoring the same PO are divided into multiple subgroups, where LP-WUS can provide wake-up indication for each subgroup. Consider the following options: - Option 1: UEs monitoring the same PO monitor the same LO - Option 2: UEs corresponding to different POs monitor the same LO - Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. - Combinations of the above options can be considered. Agreement—RAN1#118bis For the mapping between LO and PO, supports at least Option 1 (UEs monitoring the same PO monitor the same LO). Agreement—RAN1#120 For the LO to PO mapping from network perspective, support Option 2 (UEs corresponding to different POs monitor the same LO). - This should not increase the maximum number of codepoints per LO/LP-WUS compared to Option 1. - FFS conditions/restrictions for mapping multiple POs to one LO - Down-select between 2 and 4 for max number of POs per LO.

In RAN1#118bis, Option 1 UEs  monitoring the same PO monitor the same LO was supported. In RAN1#120, Option 2 was supported without increasing the maximum number of codepoints per LO/LP-WUS compared to that of Option 1. The conditions/restrictions for mapping multiple POs to one LO and the maximum number of POs per LO are FFS. With the increasing number of the configured PO per LO, the maximum number of codepoints and the required payload size are shown in Table 1. It is noted that one codepoint corresponding to all the subgroups indicated by LP-WUS within PO is considered. It can be seen that the maximum required payload size is 32 code points or 5 bits regardless of the number of the configured number of PO(s) per LO. Thus, the maximum number of POs per LO can be 4, and the candidate number of configured PO can be {1, 2, 4}.  Similar to PEI, the number of PO mapping to a LO should be multiple of Ns when the number of PO per LO is larger than Ns, Ns is specified in TS 38.304 [4]. Moreover, the maximum number of PF associated with one LO can be 1 for simplicity.

Table 1: The maximum number of codepoints and the required payload size with the increasing of the configured number of PO per LO

| The configured number of PO(s) per LO | The maximum numebr of subgroup can be configured | The totoal number of needed codepoint | The required payload size |
| --- | --- | --- | --- |
| 1 | 31 | 1*31+1=32 | 5 |
| 2 | 15 | 2*15+2=32 | 5 |
| 4 | 7 | 4*7+4=32 | 5 |

Proposal 5: The maximum number of POs per LP-WUS can be 4:

- The candidate number of configured PO associated with one LO can be {1, 2, 4}

- The number of PO mapping to a LO should be multiple of Ns when the number of PO per LO is larger than Ns, where Ns is the number of paging occasions for a PF.

Proposal 6: The maximum number of PF associated with a LO can be 1 for simplicity.

Option 3 was proposed  to reduce the number of subgroups indicated by LP-WUS within one MO. In this option, UE monitors the (iLO+1)th LO, iLO is the index of LO, where iLO=0,1,…,X-1, X is the number of LO associated with a PO.

From UE’s perspective, the index of the LO can be determined by:

iLO= (iSG mod X),                                                           Equation 1

where,

- iSG is the subgroup-ID, which can be either assigned by CN or formed based on UE_ID

- X can be provided by an RRC parameter (e.g. LO-NumPerPO) in SIBx.

Similar to Option 1, the location of the LO in time domain for a UE is determined by the reference PF, an frame-level offset and a symbol-level:

- The reference point (reference PO/PF) for the frame-level offset is the start of the PF associated with the LO

- The symbol-level offset from a frame to the start of the first LP-WUS MO of the LO, provided by (i_LO+1)th value in an RRC parameter (e.g.  lp_symbol-offset-r19) in SIBx.

In where lp_symbol-offset-r19 is a set of LO locations with size X. For example, X=4 LOs are associated with one PO, iLO=0, 1, 2, 3, as shown in Figure 4. The lp_symbol-offset-r19={symboloffset1, symboloffset2, symboloffset3, symboloffset4} is configured. Based on Equation 1, for UE with subgroup-ID = 5, the index of monitored LO is iLO = (5 mod 4) =1. UE monitors the (iLO+1)th  (i.e. 2nd )  LO and the corresponding symbol-level offset is the 2nd offset value in lp_symbol-offset-r19 i.e. symboloffset2.

Figure 4: The determination of time location for the LO in Option3

From above discussion, we can see that if the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits comparing to that of Option 1.

Observation 2: If the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits.

LP-WUS MO configuration

In Step5, UE determines MOs in its LO based on parameters in the LP-WUS configuration of SIB-X. It was agreed in RAN1#118bis, when K larger than 1, the configuration of multiple LP-WUS MO can be down-selected between Option A and B, as shown in following agreement. In each option, the value of R, the detailed UE monitoring behavior and how the same LP-WUS information is transmitted in the R LP-WUS MOs are FFS.

Agreement—RAN1#118bis When K (K>1) LP-WUS MOs are configured for each beam in an LO, down select between - Option A: K LP-WUS MOs for a beam are divided into M (M >=1) groups of R LP-WUS MOs. A UE monitors all or some of the MO(s) within the K LP-WUS MOs. For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - M = 1 and M > 1 is supported. - FFS: detailed UE monitoring behavior - FFS R=1 or R>= 1 - Option B: K LP-WUS MOs for a beam are divided into G (G >= 1) groups of R*M (M >= 1) LP-WUS MOs. A UE monitors all or some of the MO(s) within one group of R*M LP-WUS MOs based on its subgroup ID. - Each group of R*M LP-WUS MOs is further divided into M groups of R LP-WUS MOs. - For each group of R LP-WUS MOs, the same LP-WUS information is transmitted. - FFS how the same LP-WUS information is transmitted in the R LP-WUS MOs - Different LP-WUS information can be transmitted in different groups of R LP-WUS MOs. - FFS: detailed UE monitoring behavior - M = 1 and M > 1 is supported. - FFS R=1 or R>=1 - Note: this achieves the same purpose as “Option 3: UEs monitoring the same PO are divided into multiple sets of subgroups, with UEs within each set of subgroups monitoring the same LO. Working Assumption—RAN1#119 - If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

For Option A and Option B, the monitoring behavior of UE determines its MOs can be shown as following:

- Option A: UE monitors MO(s) for each beam in an LO. When UE detects a LP-WUS for the wakeup indication of its subgroup within the LO, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the LO.

- Option B: UE monitor MO(s) in the (i_group+1)th group,  when the UE detects a LP-WUS for the wakeup indication of its subgroup within the LO, the UE is not required to monitor the subsequent monitoring occasion(s) associated with the LO. i group is the index of group, where i group =0,1,…,G-1, G is the number of groups of R*M (M >= 1)in the LO. The index of the group is determined by:

i_ group =(iSG mod G),                                             Equation 2

- where iSG is the subgroup-ID; G is provided by an RRC parameter e.g.  Group-NumPerLO in SIBX.

From above discussion, Option B has similar procedure to that of Option 3 for LO to PO mapping with additional specification complexity without benefits comparing with Option A.

Observation 3: If the LP-WUS can support 32 subgroups, Option B has additional specification complexity without benefits.

Thus, the working assumption achieved in RAN1#119 can be confirmed.

Proposal 7: Confirm the following working assumption:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

The aspect of defining R is that whether the LP-WUS are transmitted R times or one out of R LP-WUS monitoring occasions. When R>1, for each group of R LP-WUS MOs, the same LP-WUS information would be transmitted by gNB.  UE is expected to monitor R LP-WUS MOs. If gNB would transmit LP-WUS on every LP-WUS MO, the LP-WUS repetition can be assumed by the UE to improve the detection probability by accumulating the LP-WUS received energies for LP-WUS coverage enhancement. Once the UE detects a LP-WUS to wake up itself, UE is not required to continue monitoring the remaining MO(s) associated with the same LO. If gNB does not transmit the LP-WUS on all R LP-WUS MOs, UE could not assume the LP-WUS transmission on any given LO and accumulates the received energy in the LP-WUS detection. Thus, there is no benefit in configured more than 1 MO (R>1) for UE power saving.

Proposal 8: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO.

Agreement For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO

In RAN1#120, for the offset value between an LO and a reference PO/PF, a frame-level offset was agreed. How to determine the MOs of the LO is FFS. The LP-WUS MO can be determined by configuring a set of locations of LP-WUS MOs, similar to the determination of LP-WUS LO for Option3, as shown in Figure 5. Once the UE determines time location of the first MO of LO, the time location of the subsequence LP-WUS MOs can be derived from the set of the locations.

Figure 5: The configuration of LP-WUS MOs

Proposal 9: The LP-WUS MO can be by configuring a set of locations of LP-WUS MOs.

The derivation of LP-WUS

A UE monitors LP-WUS on an LP-WUS MO. In Step6, UE derives a codepoint of the LP-WUS based on subgroup/UE-ID and the LP-WUS configuration. In RAN1#117, a LP-WUS indicating a codepoint value corresponding to one or all subgroup(s) was supported. In RAN1#120, it was agreed that the maximum number of subgroups per PO supported in Rel-19 is 31. How the UE derivate its associated codepoints is discussed in this section.

Assuming the number of subgroup per PO is NSGPO, provided by subgroupNumber PO_LPWUS, the number of PO per LO is NPOWUS, provided by po-NumPerLP-WUS,

- For a subgroup index iSG,  0≤iSG<NSGPO,  a UE determines its associated codepoint index, icodepoint, 0≤icodepoint<(NSGPO*（NPOWUS+1), by {(iPO*NSGPO+iSG)}, where iPO=UE_IDmodN⋅NS+i_smodNPOWUS is a paging occasion index, and N, NS, iSG, UE_ID and i_s are defined in TS 38.304, which is similar with Rel-17 PEI.

- For all subgroup indexes in a PO, the codepoint corresponding to all subgroups index is {(NSGPO*NPOWUS+iPO)}.

For example, NSGPO=7, iSG=0,1,2,3,4,5,6;  NPOWUS=4, iPO=0, 1,2,3. The total number of needed codepoint is 32 icodepoint=0,1,2,..31. The codepoint corresponding to all subgroups index is 28, 29, 30 and 31 for iPO=0, 1, 2, 3, respectively, as shown in Table 2.

Table 2: The mapping between codepoint index and subgroup ID

| iPO | iSG | icodepoint |
| --- | --- | --- |
| 0 | 0 | 0 |
|  | 1 | 1 |
|  | 2 | 2 |
|  | 3 | 3 |
|  | 4 | 4 |
|  | 5 | 5 |
|  | 6 | 6 |
| 1 | 0 | 7 |
|  | 1 | 8 |
|  | 2 | 9 |
|  | 3 | 10 |
|  | 4 | 11 |
|  | 5 | 12 |
|  | 6 | 13 |
| 2 | 0 | 14 |
|  | 1 | 15 |
|  | 2 | 16 |
|  | 3 | 17 |
|  | 4 | 18 |
|  | 5 | 19 |
|  | 6 | 20 |
| 3 | 0 | 21 |
|  | 1 | 22 |
|  | 2 | 23 |
|  | 3 | 24 |
|  | 4 | 25 |
|  | 5 | 26 |
|  | 6 | 27 |
| 0 | ALL | 28 |
| 1 | ALL | 29 |
| 2 | ALL | 30 |
| 3 | ALL | 31 |

Proposal 10: For all subgroup indexes in a PO, the codepoint corresponding to all subgroups index is {(NSGPO*NPOWUS+iPO)}, where NSGPO is the number of subgroup per PO, NPOWUS is the number of PO per LO, iPO is the PO index, 0≤iPO<NPOWUS.

eDRX configuration

In Rel-18 LP-WUS/WUR SI [2], significant UE power saving gain and moderate paging latency increase is observed if LP-WUS monitoring is restricted within existing PTW of eDRX with reusing of existing paging occasion determination. If the UE is configured with an extended DRX cycle no longer than 1024 radio frames, the DRX cycle equals to the configured eDRX cycle. UE monitors PO per configured eDRX cycle. Otherwise, a UE monitors POs per DRX cycle during a periodic Paging Time Window (PTW). For Rel-17, the PEI-O is associated with the DRX cycle i.e. the UE monitors one PEI occasion per DRX cycle. Similar, LO can also be associated with DRX cycle without additional specification impact.

When the LP-WUS monitoring is performed outside the PTW of eDRX, the paging latency reduction and moderate UE power saving gain are observed from Rel-18 LP-WUS/WUR study. The UE complexity would increase if the LP-WUS monitoring is outside the PTW window.  There is no significant benefit in the support of monitoring the LP-WUS and corresponding paging outside the existing PTW since the eDRX is introduced for the use case which is not sensitive to latency. Thus, latency reduction should not be the first priority for LP-WUS power saving technique.

Proposal 11: When eDRX is configured, LP-WUS is monitored within PTW.

Conclusion

In this contribution, we discuss aspects of UE capability, configuration of LP-WUS, subgrouping, the LP-WUS monition procedures and LP-WUS detection for RRC_IDLE/INACTIVE modes and give the following observations and proposals:

Observation 1: If the UE capability of the wake-up delay is reported through RRC signaling procedure, UE is required to transition from RRC_ IDLE/INACTIVE modes to RRC_CONNECTED mode.

Observation 2: If the LP-WUS can support 32 subgroups, Option 3 has additional specification complexity without benefits.

Observation 3: If the LP-WUS can support 32 subgroups, Option B has additional specification complexity without benefits.

Proposal 1: UE capability of the supporting of LP-WUS can be indicated during UE registration procedure through NAS procedure.

Proposal 2：The UE capability of the wake-up delay can be reported during UE registration procedure through NAS procedure.

Proposal 3: There is no benefit of assuming SSB periodicities other than 20ms.

Proposal 4: The parameters of the LP-WUS configuration are broadcasted and included in the SIB-1/SIB-X.

Proposal 5: The maximum number of POs per LP-WUS can be 4:

- The candidate number of configured PO associated with one LO can be {1, 2, 4}

- The number of PO mapping to a LO should be multiple of Ns when the number of PO per LO is larger than Ns, where Ns is the number of paging occasions for a PF.

Proposal 6: The maximum number of PF associated with a LO can be 1 for simplicity.

Proposal 7: Confirm the following working assumption:

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Proposal 8: If R>=1 is supported, gNB needs to transmit LP-WUS on every LP-WUS MO to allow UE accumulating the received energy for LP-WUS detection for coverage enhancement. Once the UE detects a LP-WUS to wake up at one of multiple MOs, the UE is not required to monitor the remaining MO(s) associated with the same LO.

Proposal 9: The LP-WUS MO can be by configuring a set of locations of LP-WUS MOs.

Proposal 10: For all subgroup indexes in a PO, the codepoint corresponding to all subgroups index is {(NSGPO*NPOWUS+iPO)}, where NSGPO is the number of subgroup per PO, NPOWUS is the number of PO per LO, iPO is the PO index, 0≤iPO<NPOWUS.

References

- Chairman notes, RAN1#120 meeting, Athens, Greece, February 17th – 21st, 2025.

- TR38.869, Study on low-power wake up signal and receiver for NR.

- TR24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS), Stage 3.

- TR38.304, NR User Equipment (UE) procedures in Idle mode and RRC Inactive state.

- TR 38.331, NR Radio Resource Control (RRC) protocol specification.

- TR 38.413, NG-RAN NG Application Protocol (NGAP).